3GPP TSG RAN WG1 #112							          R1-2301809

Athens, Greece, February 27th – March 03rd, 2022

Source:	Moderator (OPPO)

Title:	FL summary #3 for AI 8.11: R17 eSL power saving RA maintenance

Agenda item:	8.11

Document for:	Discussion and Decision

- Introduction

| Issue# | Issue | References | FL initial assessment |
| --- | --- | --- | --- |
| PS-1 | UE reports a full initialized candidate resource set (SA) when performing random resource selection When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with sl-AllowedResourceSelectionConfig including random resource selection, and random resource selection is configured by higher layer, the UE may perform random resource selection, with executing only step 1 and step 4 as follows to report the set SA to higher layers. | [1] | N |
| PS-2 | Re-evaluation and pre-emption checking for the case of aperiodic transmission When the UE is triggered to perform re-evaluation and/or pre-emption checking, performs at least contiguous partial sensing, and if Prsvp_TX=0Prsvp_TX≠0, | [2], [8] | E |
| PS-3 | Sensing occasions for periodic-based partial sensing (PBPS) The UE monitors K sensing occasions determined by sl-Additional-PBPS-Occasion of each selected candidate slot, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) and K=1 if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one and K=2 if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS. | [3] | H |
| PS-4 | For all partial sensing schemes, clarify resource allocation steps are based on PSCCH decoded and RSRP measured When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. | [4] | H |
| PS-5 | Clarification of resource sets ((r0,r1,r2,…)) and ((r0',r1',r2',…)) are in the qth reservation period when re-evaluation and pre-emption checking is triggered - if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption. If Prsvp_TX≠0, the set of resources (r0,r1,r2,…) and the set of resources (r0',r1',r2',…) are in the qth reservation period (q=0,1,2,…, Cresel-1). | [5] | N |
| PS-6 | Clarification to avoid Preserve to take on a zero value The value of Preserve corresponds to sl-PBPS-OccasionReservePeriodList if (pre-)configured, otherwise, the values correspond to all the non-zero periodicity from sl-ResourceReservePeriodList. | [6] | N |
| PS-7 | Clarification on candidate slots for aperiodic transmission in partial sensing When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots except for the other candidate slots when Y' is smaller than Ymin' and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. | [7] | N |
| PS-8 | Correction on the parameter description for Y’ candidate slots in Step 1 - Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate slots Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. | [8] | E |
| PS-9 | Half-duplex issue in re-evaluation and pre-emption checking for partial sensing For periodic transmission - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL except for those in which its own transmissions occur, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL except for those in which its own transmissions occur. For aperiodic transmission - UE performs CPS starting from at least M consecutive logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL except for those in which its own transmissions occur. | [9] | N |

* E = Essential; N = Non-essential; E = Editorial

From reviewing power saving RA related draft CRs submitted to this meeting [1-9], 9 different CRs / issues are raised as listed in the above table. Out of these issues, some of them are already identified as non-critical/non-essential issues in the past meetings, while one of them was discussed in the last meeting but no consensus was reached on its essentiality. On the remaining, PS-2 and PS-8 are identified as editorial, and perhaps PS-3 and PS-4 deserve some discussion time in the group. Do you agree with the above FL’s initial assessment? Please provide comments otherwise to add and/or removal essential issues.

| Company | Agree FL’s initial assessment (Yes / No) | Comments |
| --- | --- | --- |
| DCM |  | Agree for PS-1/PS-2/PS-3/PS-5/PS-6/PS-8/. For PS-4, we are not sure the clarification is essential since anyhow partial sensing UE performs sensing as specified. For PS-7/PS-9, we do not agree it is non-essential. At least why this is not essential should be clarified. Comment for non-essential corrections: PS-1: 38.321 does not use S_A for random selection. PS-5: Why it is required to consider parallel processes is unclear. PS-6: There is no other interpretation (there is no sensing slot for zero periodicity). |
| Intel | Comments | Labeling in the FL assessment column is unclear as E labels “Essential” as well as “Editorial”. With that said, we assume that H represents “Essential”, and E represents “Editorial”. If that is the case, we agree with the assessment of indicating as editorial the issue PS-2, even though we think it is essential to implement this change as it changes the logic. Also, we are OK to implement issue PS-8. Furthermore, it is our understanding that both issue PS-3 and PS-4 only add minor clarification, but we are OK to discussing these further. We also agree with the assessment of all other issues as non-essential. |
| Sharp |  | Agree FL’s assessment for PS-1/-2/-4/-8/-9. PS-3: We tend to think “the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured” with the wording “correspond to” not only implies the number of sensing occasions, also refers to the specific one (i.e. the default sensing occasion). Therefore, we don’t think the change is essential. PS-5: For Rel-16 NR V2X, how to restrict different TBs from being in the same slot was discussed in RAN1#102-e (R1-2007220), which reads Q2-2: How to restrict selection of resources for different TBs to different slots? - Option 1: Add additional time domain exclusion of resources selected for other TBs - Option 2: Do not exclude, but drop one of the resources when transmission is going to happen - Option 3: Leave up to UE implementation, and capture that a UE is not expected to select/ transmit PSCCH/PSSCH for more than one TB in a slot in a BWP This clarifies different resource selection procedures with overlapping RSWs are supported in SL. Therefore, we think it is essential. PS-6: In current specs for PBPS, it is specified as a requirement for UE to monitor the sensing occasions (i.e. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots). For Preserve=0, the requirement cannot be met. Therefore, it is essential. PS-7: As explained in [7], “the other candidate slots” do not have corresponding PBPS sensing results. Again, the corresponding sensing occasions are requirement for the UE. Therefore, it is essential to preclude these slots w/o enough sensing results, i.e. UE does not monitor all the corresponding PBPS sensing occasions. |
| Nokia, NSB | Comments | PS-1: Non-essential PS-2: editorial PS-3: editorial, and may need further discussion for “K=2” PS-4: editorial PS-5: non-essential PS-6: non-essential PS-7: non-essential PS-8: editorial PS-9: non-essential |
| vivo | comments | We assume H means ‘essential’. PS-1/2/3/4/5/6/7/8/9: Agree with FL’s assessment, PS-3/4/8 are to add some clarifications. ‘K=2’ is because that for each candidate slot in Y, there will be two PSO to monitor (i.e., the last one, the one prior before the last one) when additional PSO monitoring is configured. For PS-4. Generally ok, but we are not sure why the proposed change is added in the middle of the text for M determination, it would be clearer to move the change to the end of the paragraph. |
| OPPO | Agree with comments | As for PS-3, we are ok to address the issue of “k” and “K”. However, we think the new added “of each selected candidate slot” is not needed and current spec is already clear enough. |
| Ericsson |  | PS-1: Not essential PS-2: Editorial PS-3: No. Does not add any new information. PS-4: We think it is clear in the specifications but will not object to such an editorial change. PS-5: Not essential PS-6: Not essential PS-7: Not essential PS-8: Editorial PS-9: Not essential |

Summary:

|  | PS-1 | PS-2 | PS-3 | PS-4 | PS-5 | PS-6 | PS-7 | PS-8 | PS-9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FL | N | E | H | H | N | N | N | E | N |
| DCM | N | E | H | N | N | N | H | E | H |
| Intel | N | E | OK | OK | N | N | N | E | N |
| Sharp | N | E | N | H | H | H | H | E | N |
| Nokia, NSB | N | E | E | E | N | N | N | E | N |
| vivo | N | E | H | H | N | N | N | E | N |
| OPPO | N | E | H | H | N | N | N | E | N |
| Ericsson | N | E | N | E | N | N | N | E | N |

FL recommendation:

- Treat issues PS-3 and PS-4 as H (essential issues) in RAN1#112.

- Treat issues PS-2 and PS-8 as E (editorial issues) in RAN1#112.

- Discussions

  - Essential corrections

    - Issue PS-3: Sensing occasions for periodic-based partial sensing (PBPS)

Reason for change [3]:

- When a UE performs periodic-based partial sensing, the UE monitors one PSO if sl-Additional-PBPS-Occasion is not (pre-)configured or two PSO if sl-Additional-PBPS-Occasion is (pre-)configured for ty'SL, where the time slot of a PSO is determined according to ty-k×Preserve''SL. The value of ‘k’ in ty-k×Preserve''SL indicates the period index of PSO to be monitored, and the period index of the last PSO and the next to last PSO can be larger than 1 and 2 respectively due to the processing time Tproc,0SL+Tproc,1SL ,. It is confusing to use the same notation of ‘k’ to indicate both the index of the PSO in “the UE shall monitor slots at ty-k×Preserve''SL” and the number of the PSO in “The UE monitors k sensing occasions…”.

- Editorial error: a space is missing in ‘wheret'y'SL is a slot belonging to the remaining Y candidate slots’

Proposed change [3]:

The UE monitors K sensing occasions determined by sl-Additional-PBPS-Occasion of each selected candidate slot, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) and K=1 if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one and K=2 if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS.

| Company | Comments |
| --- | --- |
| Qualcomm | Ok with the change |
| Sharp | We don’t think to introduce K is necessary. In current specs, it is clear enough if the RRC parameter is not configured, the number of PSO is 1 and 2 if configured. Therefore, the only correction needed is to remove ‘k’ from the 1st sentence. We propose the following change: The UE monitors k sensing occasion(s) determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS. |
| Intel | We are ok with the change. |
| OPPO | We think the new added “of each selected candidate slot” is not needed. In TS 38.214, when the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots. According to the above formula and description in current specification, it is already clear enough that each selected candidate slot will have one or two corresponding periodic sensing occasions. And we are ok with other changes. |
| ZTE,Sanechips | Ok with the change on K, no need of ‘of each selected candidate slot’ |
| CATT/GH | Share the similar view with OPPO and ZTE. |
| Vivo | Support. The ‘of each selected candidate slot’ is added to avoid misinterpretation that UE only monitor one or two slots in total for the whole set of Y slots. But if companies think it is clear even without ‘of each selected candidate slot’, we are ok to remove it. We are also ok with Sharp’s suggestion. |
| Huawei, HiSilicon | We do not think the changes are needed and original spec is clear. Original “k sensing occasions” represents occasions that UE should sense. “…correspond to the most recent sensing occasion…” means UE only sense one occasion which is the most recent one; “additionally includes the value of k corresponding to the last periodic sensing occasion” indicates UE should sense two occasions. Thus the spec is clear and we think no changes is needed. |

FL’s observation and recommendation

Based on the inputs provided, it seems the majority is OK with the changes in [3/vivo] except the part “of each selected candidate slot”. Sharp suggested a simpler change and it seems to be acceptable by the original proponent. I think this change should be fine by Huawei/Hisilicon. Therefore, I recommend the following modified change proposal from Sharp in the following.

<Beginning of propose changes>

The UE monitors k sensing occasion(s) determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS.

<End of propose changes>

Agreement

The following TP is endorsed for TS38.214 section 8.1.4:

<Beginning of propose changes>

The UE monitors k sensing occasion(s) determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS.

<End of propose changes>

    - Issue PS-4: For all partial sensing schemes, clarify resource allocation steps are based on PSCCH decoded and RSRP measured

Reason for change [4]:

- In step 2) of 8.1.4, UE behavior is clearly defined when the UE performs full sensing and periodic based partial sensing, i.e., the UE shall perform the behavior in the following steps based on PSCCH decoded and RSRP measured in these slots. However, this description is absent in the contiguous partial sensing part and thus should be added.

Proposed change [4]:

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31.

When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

| Company | Comments |
| --- | --- |
| Qualcomm | Ok with the change. |
| Sharp | No strong view. We can follow the majority view. |
| Intel | We are ok with the change. |
| OPPO | OK |
| ZTE,Sanechips | OK, but the sentence had better be here If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. |
| CATT/GH | Support. Regarding ZTE’s suggestion, we are open for discussion. The original intention of adding the sentence there is for clear understanding of the definition of “these slots”, since the later sentences are for further restriction of “M”. |
| vivo | We share similar view as ZTE, the proposed change should be moved to the end of the paragraph |
| Huawei, HiSilicon | Ok with the changes. |

FL’s observation and recommendation

It is observed all inputs are fine with the change in principle and there is no concern raised to ZTE’s suggestion to move the proposed change to the end of the paragraph. Therefore, FL recommends to implement ZTE’s suggestion.

<Beginning of propose changes>

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31.

When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

<End of propose changes>

Agreement

The following TP is endorsed for TS38.214 section 8.1.4:

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31.

When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

  - Editorial corrections

    - Issue PS-2: Re-evaluation and pre-emption checking for the case of aperiodic transmission

Reason for change (FL):

- In TS38.214 V 17.3.0, the sentence is “When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0,”.

- In TS38.214 V 17.4.0, the sentence is changed to “When the UE is triggered to perform re-evaluation and/or pre-emption checking, performs at least contiguous partial sensing, and if Prsvp_TX≠0,”.

- However, there was no agreed change proposal to make such a change in the specification from RAN1#111. Therefore, such specification error (editorial) should be revered.

Proposed correction [2]:

When the UE is triggered to perform re-evaluation and/or pre-emption checking, performs at least contiguous partial sensing, and if Prsvp_TX=0Prsvp_TX≠0,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot t'yiSL and ending at the last slot of the Y' candidate slots, where t'yiSL is the first candidate slot starting from slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled

-	UE performs CPS starting from at least M consecutive logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL

-	For minimum size M of the contiguous partial sensing window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n –T0.

| Company | Comments |
| --- | --- |
| Qualcomm | OK with the change |
| Sharp | Support |
| Intel | Support |
| OPPO | OK |
| ZTE,Sanechips | Ok |
| CATT/GH | Support. |
| vivo | ok |
| Huawei, HiSilicon | ok |

FL’s recommendation

To implement the proposed editorial change proposal in [2].

Agreement

Endorse the TP (for TS38.214 section 8.1.4) in section 2.2.1.

    - Issue PS-8: Correction on the parameter description for Y’ candidate slots in Step 1

Reason for change (FL):

- The number Y’ refers to number of candidate slots, but not candidate single slot resources. This is also evident in the same specification paragraph.

Proposed correction [8]:

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate slots Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

| Company | Comments |
| --- | --- |
| Qualcomm | OK with the change |
| Sharp | Support |
| Intel | Support |
| OPPO | OK |
| ZTE,Sanechips | Ok |
| CATT/GH | Support. |
| vivo | ok |
| Huawei, HiSilicon | ok |

FL’s recommendation

To implement the proposed editorial change proposal in [8].

Agreement

Endorse the TP (for TS38.214 section 8.1.4) in section 2.2.2.

- References

- R1-2300105	Correction on description of random resource selection in TS 38.214	Huawei, HiSilicon

- R1-2300315	Correction on the operation of re-evaluation and/or pre-emption checking for partial sensing	Spreadtrum Communications, vivo

- R1-2300422	Correction on the sensing occasions for periodic-based partial sensing	vivo

- R1-2300628	Correction on the operations of partial sensing	CATT, GOHIGH

- R1-2300791	Clarification on pre-emption and re-evaluation for periodic transmission in partial sensing	Sharp

- R1-2300792	Clarification on Preserve for periodic based partial sensing	Sharp

- R1-2300793	Clarification on candidate slots for aperiodic transmission in partial sensing	Sharp

- R1-2301082	Correction on parameter description for sidelink partial sensing in TS 38.214	ZTE, Sanechips

- R1-2301474	Draft CR on half-duplex consideration for SL re-evaluation/pre-emption check	NTT DOCOMO, INC.