3GPP TSG RAN WG1 Meeting #112	R1-2301877

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.3.2

Source:	Moderator (Futurewei)

Title:	FL Summary #1 on SRS enhancements

Document for:	Discussion and decision

# Introduction

In RAN#94-e, a Work Item for Rel-18 on “MIMO Evolution for Downlink and Uplink” was approved, and the motivations, scopes, and objectives were agreed in [1]. Among the objectives, the underlined in the following are related to SRS enhancements, mainly in the aspects of SRS for TDD Coherent Joint Transmission (CJT or C-JT) and 8 Tx operation:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

24 contributions [2-25] have been submitted to Agenda Item 9.1.3.2 of RAN1#112 on SRS Enhancements targeting TDD CJT and 8 Tx operations. Main views and further discussion points based on these contributions are collected in this document. Any additional inputs from any company can also be provided in this document.

# SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT

## Comb offset hopping and cyclic shift hopping

Almost all companies support one or both of the hopping schemes, and a large number of designs have been provided for comb offset hopping and/or cyclic shift hopping in the contributions. Most of the designs are common to both comb offset hopping and cyclic shift hopping, while some other designs, generally on some detailed aspects, are specific to only one of them. So the plan for this meeting is to continue focusing on designs common to both hopping schemes, and in the meantime deciding on potential down-selection/separate/joint schemes as well as further details, including details that are common to both schemes or specific to one scheme.

Regarding the potential down-selection/separate/joint schemes, we had the following agreement from the last meeting:

Agreement

For SRS interference randomization, support one from the following options  (to be decided in RAN1#112):

- Opt. 1: Cyclic shift hopping

- Opt. 2: Comb offset hopping

- Opt. 3: Both cyclic shift hopping and comb offset hopping

  - FFS: details including whether to support separate and/or combined hopping

  - FFS: details on UE capability and signaling

The general positions are:

Option 1: Cyclic shift hopping (only)

Supported by LG, Sharp, vivo

Option 2: Comb offset hopping (only)

Supported by Apple, Qualcomm (FFS cyclic shift hopping), Xiaomi

Option 3: Both cyclic shift hopping and comb offset hopping

Supported by CATT, CMCC, Ericsson, ETRI, Futurewei, Google, Huawei, HiSilicon, Intel, InterDigital, Lenovo, NEC, Nokia, Nokia Shanghai Bell, NTT DOCOMO,  Samsung, Spreadtrum, ZTE

Combined hopping

Supported by CATT, Lenovo, NEC, ZTE

Separate hopping

Supported by CMCC, ETRI, Futurewei, InterDigital, Lenovo, OPPO, Samsung, Sharp

Based on the majority view, we have the following proposal.

Proposal 2.1: For SRS interference randomization, support:

Opt. 3: Both cyclic shift hopping and comb offset hopping

At least support separate hopping for a UE

Only one of the hopping schemes can be configured for a UE at a time

Introduce separate UE capabilities for the hopping schemes

FFS combined hopping

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | We can be ok with this proposal for progress, and given that the benefits of these two schemes are different as we evaluated in our Tdoc, i.e., - comb offset hopping randomizes the set of interfering UEs in RE domain - cyclic shift hopping randomizes the interference pattern from the same set of UEs (while the benefit is a function of the choice of SRS sequences, and for some pairs of SRS sequences, we observed small gain). For combined hopping, we do not support at this time given that no company evaluated the benefit of such combined hopping schemes. |
| Google | - We support opt3. But we failed to see any reason for the restriction “Only one of the hopping schemes can be configured for a UE at a time”. We can introduce a UE capability to report whether a UE supports concurrent configuration. |
| OPPO | Our preference is to only support one of the hopping schemes. If majority companies want to introduce both hopping, we can only accept separate hopping as in Proposal 2.1: separate UE capability and only one can be configured for a UE. |
| Lenovo | Support the proposal in general. For separate UE capability, we think it may increase gNB’s implementation complexity on account of different UEs with different capabilities. If there is not much difference on UE capability requirement for CS hopping and/or Comb hopping, we prefer one UE capability. For combined hopping, we support to enable CS hopping and Comb hopping simultaneously to increase freedom of degree for hopping. CS hopping and Comb chopping can be designed separately as much as possible to reduce standardization effort. |
| Xiaomi | Although we think it is enough that only one hopping scheme is supported, we can live with the proposal for progress. If both schemes are supported at the same time, it is challenged to UE capability. Hence, we prefer only one is supported at a time |
| Apple | We are fine with Proposal 2.1 |
| Nokia/NSB | We are fine with Proposal 2.1 |
| InterDigital | Support Proposal 2.1 |
| Samsung | Support separate hopping only, but not support combined one. |
| ZTE | Support the proposal for progress. Regarding the separate CS / comb offset hopping option, to avoid the case where two SRS ports sharing a same group number v colliding at a same CS, we propose to combine CS hopping with legacy group hopping and co-design their hopping patterns. We propose to perform CS hopping according to the following formula given in our Tdoc nSRScs=fgh+nIDSRS/30 mod nSRScs,max Additionally, we support combined CS / comb offset hopping for better interference randomization and more flexible co-scheduling of legacy and new UEs. |
| LGE | Agree with OPPO. We prefer to support only one of the hopping schemes. |
| MediaTek | We are fine with Proposal 2.1. Agree with the comment made by OPPO, if both cyclic shift hopping and comb offset hopping is agreed, then it should come with its own individual UE capabilities. We also oppose any combined hopping approaches. |
| Intel | Generally fine with the proposal. Support to have the separate hopping. |
| vivo | We are fine with the proposal, if majority companies support both of CS hopping and comb offset hopping. We also don’t support to combine CS hopping and comb offset hopping, since limited gain can be obtained. Moreover, legacy sequence/group hopping can also provide randomization for SRS, thus it is unnecessary to configure CS/comb offset hopping with sequence/group hopping, considering the complexity of gNB and UE. Proposal 2.1: For SRS interference randomization, support: Opt. 3: Both cyclic shift hopping and comb offset hopping At least support separate hopping for a UE Only one of the hopping schemes can be configured for a UE at a time, including sequence hopping and group hoping Introduce separate UE capabilities for the hopping schemes FFS combined hopping |
| FL | Most of companies are generally fine with this proposal, and views on detailed combined/separate hopping schemes/capabilities have been provided. To facilitate progress, my suggestion is to first agree on supporting both hopping schemes, and FFS combined/separate options and UE capabilities. Proposal 2.1A: For SRS interference randomization, support: Opt. 3: Both cyclic shift hopping and comb offset hopping. And FFS the following: At least support separate or combined cyclic shift hopping and comb offset hopping for a UE Separate or combined with SRS sequence group hopping / sequence hopping Associated UE capability Only one of the hopping schemes can be configured for a UE at a time Introduce separate UE capabilities for the hopping schemes FFS combined hopping |
| CMCC | Support Proposal 2.1A. We support Opt. 3 with separate hopping. For UE with capabilities with both Comb offset hopping and CS hopping, only either of them can be configured by RRC signaling. |
| NEC | We prefer joint hopping if both hopping supported. As for comb offset hopping, there are cases hopping can not be applied, e.g. all comb offsets allocated. And regarding sequence/group hopping, we prefer CS/comb offset hopping being based on sequence/group hopping. |
| Sharp | We are fine with Proposal 2.1 and Proposal 2.1A |
| CATT | Ok with the updated proposal. |

### Designs common to comb offset hopping and cyclic shift hopping: NW-configured ID(s)

We had the following FFS from the last meeting:

FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID

The following summarizes the NW-configured ID(s) which is generally common to both comb offset hopping and cyclic shift hopping:

- Reuse the SRS sequence identity nIDSRS.

  - Supported by Ericsson, ETRI, Futurewei, Lenovo, LG, Qualcomm, Sharp, vivo

- Introduce new ID(s).

  - Supported by CATT, Futurewei (separate new IDs), Lenovo, NTT DOCOMO, Qualcomm

- Reuse cell ID nIDcell:

  - Spreadtrum, Xiaomi (or TRP ID)

- Reuse C-RNTI:

  - Spreadtrum

Based on the positions, we have the following proposal:

Proposal 2.1.1: For SRS comb offset hopping and/or cyclic shift hopping, support one of the options:

Option 1: Reuse the SRS sequence identity nIDSRS.

Option 2: Introduce new ID(s).

FFS: the value range, one new ID or two separate new IDs

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | Support the proposal. Just a clarification on the FFS: Does “two separate new IDs” refer to when comb offset hopping versus when CS hopping is configured? If so, I assume these two new RRC params cannot be configured simultaneously. |
| Google | - Support. In our view, option 1 is better. |
| DOCOMO | Support the proposal. Prefer Option 2, since Option 1 will result in difficult inter-cell interference handling. |
| OPPO | Support Option 1. |
| Lenovo | Support the proposal. If new ID(s) are introduced, the actual ID number can be 2 if combined hopping is supported and be 1 if combined hopping is not supported. For two IDs, new ID(s) have to be configured. |
| Xiaomi | We still prefer is to reuse cell ID, which is straightforward to ensure users located at different cell with different hopping sequence. |
| Huawei, HiSilicon | Although we’ve illustrated in our contribution that under certain scenario (independent SRS resource allocation by each TRP, SRS interference comes from sequence difference) adopting CS hopping only is enough for achieving good interference randomization effect, at least there does exist some scenarios (joint orthogonal SRS resource allocation by coordinated TRPs, SRS interference comes from propagation delay difference) where both CS hopping and group hopping should be enabled, which calls for at least one new ID. |
| Apple | We prefer to Option 2. However, in case ID is not explicitly configuration, we may also need to discuss the default ID. |
| Nokia/NSB | Support the proposal with Option 2. The option 1 is targeted for legacy operation and may cause restrictions to enable efficient Rel-18 TDD CJT operation . |
| InterDigital | Support Option 1 |
| Samsung | Support Option 1. |
| ZTE | To make the proposal clearer, we suggest the following modification Proposal 2.1.1: For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. Option 1: The SRS sequence identity nIDSRS. Option 2: New ID(s). FFS: the value range, one new ID or two separate new IDs - Support option 1. We hold a different view with Xiaomi and Huawei, i.e., reusing nIDSRS also enables combined CS and group hopping if their hopping patterns are carefully designed. On the contrary, reusing nIDSRSmakes co-designing CS and group hopping easier, because both of them are based on a common pseudo-random sequence. As example, we give a good co-design of CS and group hopping in section 2.2 of our Tdoc. |
| LGE | Support Option 1. nIDSRS can be SRS resource-specifically configured for a UE, allowing for interference randomization in both intra-cell and inter-cell environments. |
| MediaTek | Support option 1. |
| Intel | Support Option 1. Regarding the FFS, what’s the case to have two separate new IDs? |
| vivo | Support Option 1. |
| FL | Most of companies are generally fine with this proposal. @QC @Intel: This may depend on the outcome of combined/separate hopping schemes, as well as RRC IE designs. For example, if two IEs are used in RRC, there could be two IEs in these two IEs separately or one IE common to both. @Xiaomi: I think there are cases where multiple groups of SRSs exist in one cell and need different IDs, and cases where CJT UEs require the same IE across multiple cells. Proposal 2.1.1A: For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. Option 1: Reuse the SRS sequence identity nIDSRS. Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s) |
| CMCC | Support Option 1. As long as different UEs in the same cell can have different initialization, legacy parameter is enough. |
| NEC | Option 1 preferred. |
| Sharp | Support Option 1. |
| CATT | Support the proposal and support option 2. Reusing the SRS sequence ID would complicate the configuration of this parameter as the sequence hopping and comb/cyclic shift hopping need to be jointly considered. |

### Designs common to comb offset hopping and cyclic shift hopping: Time-domain hopping behavior

The following was agreed in the last meeting:

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

The general positions are:

Support at least hopping based on slot index, OFDM symbol index (Apple, CMCC, Futurewei, Intel, Lenovo, Xiaomi, ZTE)

  - FFS: Use of symbol group based on repetition factor (InterDigital, Lenovo, Qualcomm, Sharp, vivo)

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index (CMCC, Lenovo), inter-slot hopping based on slot index (CMCC, Intel), per occasion of SRS resource (Qualcomm)

  - FFS: Re-initialization periodicity (Futurewei)

  - Others: Per frequency hopping occasion (vivo), SFN (NEC, Futurewei)

There are different considerations for these different preferences:

Some companies are mainly concerned with the multiplexing orthogonality when hopping is enabled, for SRSs that may be configured with different periodicities, symbol locations, etc. They prefer the hopping pattern to be based on the frame/slot/symbol structures instead of UE-specific / SRS-specific configurations. An example of this design is to have the similar hopping behavior as SRS sequence hopping / sequence group hopping, which in the cases of SRS repetition, would still hop to different resources.

Some other companies are mainly concerned with the joint processing simplicity when hopping is enabled, and therefore, on the R symbols with SRS repetition, would use the same resources.

The group can further discuss these different options. The following proposal is suggested as a starting point:

Proposal 2.1.2: For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index l' as l'/R.

Option 2: Within a slot, hopping based on only the symbol index l'.

At least support reinitialization at the beginning of each radio frame.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | A more fundamental question is the unit of hopping: - Per SRS resource (no hopping within a slot) - Per R consecutive symbols (no hopping within R repetitions) - Per SRS symbol Also, in option 1 (assuming that it corresponds to hopping per R consecutive symbols), the part on “symbol index l' as l'/R” may not be very clear or accurate. Is this floor or ceiling operation? What if the symbol index of the first symbol of R repetitions is not a multiple of R? Instead, we think l' in this option should be the symbol index of the first OFDM symbol within R repetitions (within R consecutive OFDM symbols). E.g., if SRS resource with 4 symbols is on symbols 10-13 with R=2, for the first two symbols l'=10, and for the next two symbols l'=12. Another point is that, for the hopping unit, there is no reason to limit to have the same option for comb offset hopping versus CS hopping (e.g., we may or may not end up choosing the same option). |
| Google | - We also think we need to clarify the hopping granularity first. It is better to list the options as QC mentioned for down-selection first. |
| Lenovo | Support the proposal in general. Agree with QC’s comment on l'/R and Google’s comment that we can research the agreement on hopping granularity firstly. |
| Huawei, HiSilicon | Support option 2, which is already adopted by group/sequence hopping. Option1 can be further considered if justified. In terms of the re-initialization granularity, considering the relatively large SRS periodicity supported by the current spec. compared to the radio frame length, forcing the re-initialization granularity to be as small as radio frame will severely restrict the randomization effect and is not preferred. |
| Apple | We now support TDM SRS ports, which needs to be jointly discussed. |
| Nokia/NSB | Agree with QC that further clarification is needed regarding hopping granularity. |
| InterDigital | Support Option 1. Our understanding is that R corresponds to the number of consecutive symbols. |
| ZTE | We prefer option 2, and are open to option 1. |
| LGE | Prefer option 2. |
| Intel | Agree with QC’s comment to list the options of the hopping unit. In addition, we think the inter-slot case should be discussed firstly. |
| vivo | Support option 1. The tradeoff between randomization performance and UE/gNB complexity should be considered when determining the hopping granularity for cyclic shift hopping, thus per R symbols is an appropriate hopping granularity. |
| FL | @QC: I omitted the floor operation and the linear shift by l0 to emphasize the dependent variables and to be more open to potential alternatives, but they are added now for more clarify. @Option 1 proponents: A question: Does Option 1 require all multiplexed SRSs to have the same repetition behavior on overlapping OFDM symbols? Option 2 allows multiplexed SRS1 and SRS2 to be orthogonal even if SRS1 has R=2 and SRS2 has R=3, when both are configured with hopping, but Option 1 does not seem to allow this. This seems to be a big restriction for Option 1. Please clarify. The proposal is updated based on the inputs. Proposal 2.1.2A: For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: Option 1: Within a slot, hopping based on the repetition factor R and symbol index l' as (l'+l0)/R. Option 2: Within a slot, hopping based on only the symbol index l'. Option 3: No intra-slot hopping. FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) At least support reinitialization at the beginning of each radio frame. FFS reinitialization periodicity of N radio frames. |
| CMCC | Prefer Option 2, and we assume the hopping granularity of Option 2 is Per SRS symbol. |
| NEC | If hopping behavior based on slot index and symbol index, it will lead the hopping pattern same from frame to frame, which will limit the randomization. So we also think the pattern of new introduced cyclic shift hopping and/or comb offset hopping should vary in different radio frames, which can significantly improve the interference randomization. There may be two ways to achieve this, one way is the time domain hopping behavior further depends on a new parameter (besides slot index and symbol index), the other way is to reinitialize the hopping sequence based on the new parameter. For example, the system frame number (nf) which is well supported in current spec can be introduced as the parameter. So we prefer: Updated Proposal 2.1.2A: For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: Option 1: Within a slot, hopping based on the repetition factor R and symbol index l' as (l'+l0)/R. Option 2: Within a slot, hopping based on only the symbol index l'. Option 3: No intra-slot hopping. Option 4: Time domain hopping behaviour further depends on a new parameter. - FFS the new parameter, e.g. system frame number or periodicity of N radio frames. FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) At least support reinitialization at the beginning of each radio frame. FFS reinitialization periodicity of N radio frames or reinitialization based on system frame number. |
| Sharp | We are generally fine with Proposal 2.1.2A, and support Option 1. |
| CATT | Support the updated proposal and support option 2. |
| FL | @NEC: Thanks for the suggestions. My understanding is that the “frame” in SFN is the radio frame, so I tried to shorten Option 4 a bit without changing its meaning. Proposal 2.1.2B: For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: Option 1: Within a slot, hopping based on the repetition factor R and symbol index l' as (l'+l0)/R. Option 2: Within a slot, hopping based on only the symbol index l'. Option 3: No intra-slot hopping. Option 4: Time domain hopping behaviour further depends on system frame number (SFN) nf. FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) At least support reinitialization at the beginning of each radio frame. FFS reinitialization periodicity of N radio frames or reinitialization based on system frame number. |

### Designs common to comb offset hopping and cyclic shift hopping: other proposals

The following proposals are also common to both comb offset hopping and cyclic shift hopping:

Enh. 1: Applicable to aperiodic SRS with usage antennaSwitching.

  - Supported by Ericsson, Futurewei, Lenovo

Enh. 2: Applicable to SRS with usages codebook/nonCodebook/beamManagement.

  - Supported by Futurewei

Enh. 3: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping (if supported) / cyclic shift hopping (if supported).

  - Ericsson, Futurewei, Lenovo

Enh. 4: Hopping time-domain behavior also depends on the system frame number (SFN), and with reinitialization periodicity of n SFNs.

  - Supported by NEC, Futurewei

Enh. 5: May combine with sequence hopping + group hopping

  - Supported by: Qualcomm

Enh. 6: Not expected to combine with sequence hopping or group hopping

  - Supported by: vivo, ZTE

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | Enh 1: Depends on the hopping unit. Enh 2: We can be open subject to UE capability Enh 3: It may not be critical at this point (can be discussed later once the basic designs are somewhat more clear) Enh 4: It may not be critical at this point (can be discussed later once the basic designs are somewhat more clear). Enh 5, 6: Just to be clear, our intention was to have a combination of sequence hopping and group hopping (not necessarily combining these with comb offset hopping or CS hopping) |
| Google | - We suggest we discuss the details for the hopping first. Currently we do not see benefit for any of the enhancements above. |
| DOCOMO | We share Google’s argument above. |
| Lenovo | - Enh 1: Support - Enh 2: More discussion and clarification are needed for SRS with usages codebook/ nonCodebook/beamManagement - Enh3: Support - Enh4: It may be related with conclusion of reinitialization periodicity. We are open for the discussion - Enh5 or Enh6: It depends on gNB’s configuration |
| Huawei, HiSilicon | - Enh.1: Support. - Enh.2: Open to discuss. - Enh.3: Fine to study. - Enh.4: Fine to study. - Enh.5: Fine to study. |
| Nokia/NSB | - Share the same view with Google and Docomo, that this can be revisited once details of hopping schemes are agreed. |
| InterDigital | - This can be discussed after the hopping details are finalized. |
| ZTE | Enh 1: Support. Enh 2: Support. Enh 3: Support. Hopping within a subset of CSs / comb offsets simplifies the co-scheduling between legacy and new UEs. Enh 4: Support. Enh 5, 6: Support combined CS and group hopping to enhance interference randomization. |
| LGE | Same view as Google. |
| MediaTek | Same view as Google. |
| Intel | This is the next level of details. It can be discussed after consensus is reached in Section 2.1.1 and Section 2.1.2. |
| vivo | Enh 6 should be considered. Since sequence/group hopping can also provide randomization for SRS, the limited gain can be obtained to combine sequence/group hopping with CS/comb offset hopping. Besides, from the perspective of UE, it would increase the complexity for UE to generate SRS. From the perspective of the network, it would also increase the complexity of SRS channel estimation. |
| CMCC | Agree with Google. These further enhancements can be discussed latter after the details of hopping is finalized. |
| NEC | We support at least Enh 4 and Enh 5. And fine to discuss later. |
| CATT | Support Enh 1. |

### Designs specific to cyclic shift hopping

The following have been proposed specifically for cyclic shift hopping:

- Enh. 1: Combined with enhanced CS configurations, e.g., per-port CS assignment, non-equidistant CS assignment

  - Supported by: Futurewei

- Enh. 2: K times finer time-domain granularity for a CS

  - Supported by: Futurewei, Huawei, HiSilicon

Views can be provided for the above enhancements, and other designs can also be suggested.

| Company | View |
| --- | --- |
| QC | Enh. 1: Depends on the outcome of Proposal 2.4. We can study such combinations later. Enh. 2: It will effectively introduce more cyclic shifts. We do not think it is needed as it goes beyond the currently supported cyclic shifts. |
| Google | Probably we can defer the decision for the enhancements above. We can come back after we see more details on the hopping. |
| OPPO | Agree with Google. |
| Huawei, HiSilicon | Enh. 1: Can be delayed until consensus have been reached in section 2.4. Enh. 2: Support. As illustrated in our contribution, finer hopping granularity can bring significant benefit under both “inter-sequence” scenario (independent SRS resource allocation by each TRP, SRS interference comes from sequence difference) as well as “intra-sequence” scenario (joint orthogonal SRS resource allocation by coordinated TRPs, SRS interference comes from propagation delay difference) and should be supported for CS hopping. |
| Nokia/NSB | Depends on the outcome of Proposals 2.1 and 2.4. |
| ZTE | Enh. 1: Do not support. Per-port CS assignment greatly increases the signaling overhead while does not bring significant benefits. Non-equidistant CS assignment may shorten the length of zero-correlation zone and degrade the channel estimation accuracy. Enh. 2: We are open to discuss this issue. Just for clarification, if our understanding is correct, here “finer granularity” means finer granularity between adjacent CSs rather than finer granularity in time domain. |
| LGE | Agree with Google. |
| MediaTek | Agree with Google, depends on the outcome of Proposals 2.1 and 2.4. |
| Intel | Agree with Google and other companies that it could be discussed later. |
| Lenovo | Enh.1: More discussion can be made for the actual performance benefit and impact on other multiplexed SRS (for other UEs) on the same time-frequency resource. Enh.2: We are fine for further discussion. If there is performance gain and standard impact is small, it is desirable to be specified in R18. |
| vivo | Enh. 1: Don’t support. We don’t see the benefit of the combination, and more overhead would be needed. Enh. 2: Open to discuss it later. |
| CMCC | Enh.1: Do not support. The benefit of Per port CS configuration is not clear. Enh.2: We are open to further study it. |
| CATT | Do not support Enh 1. |

### Designs specific to comb offset hopping

The following have been proposed specifically for cyclic shift hopping:

- Enh. 1: Enhanced comb offset hopping with multiple comb-offset values

  - Supported by: Nokia, Nokia Shanghai Bell

My understanding is that this is already assumed, but please feel free to correct me.

| QC | Is the intention to do randomization in frequency domain (such that in a given symbol, REs may become non-uniform due to hopping unit being an RB group or a subband)? If yes, this fundamentally changes the comb structure of SRS. |
| --- | --- |
| Google | It seems this depends on the hopping granularity decision. |
| Nokia/NSB | @QC, yes, further interference randomization can be obtained with increasing frequency hopping granularity. |
| ZTE | Do not support. Comb offset hopping with multiple comb-offset values will greatly increase implementation complexity and scheduling complexity. |
| LGE | It is more like second-level detail of hopping. Prefer to defer the discussion. |
| MediaTek | Not support. Agree with ZTEs comments. |
| Intel | We don’t see the necessity to have this proposal. As QC mentioned, the comb structure will be changed. |
| Lenovo | We are open for more discussion on the details. |
| vivo | Does it mean the value of comb would be changed when hopping? If yes, we don’t support it, due to the high complexity. |
| CATT | Do not support. Agree with ZTE comments. |

## Per-TRP power control and/or power control of one SRS towards to multiple TRPs

We have achieved the following agreement:

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

The following options have been discussed:

Option 0: For legacy TRP-specific SRS using multiple SRS resource sets; no enhancement in Rel-18

Pros:

No spec impact

Cons:

The number of SRS resource sets is generally at most 2 per current standards

Support by: Samsung, vivo

Option 1: For TRP-common SRS using one SRS resource set

Pros:

Low SRS overhead; low latency

One source shows performance benefit

Cons:

Receive powers at the CJT TRPs are less accurate

Support by: CMCC, ETRI, Futurewei, Lenovo, NEC, Nokia, Nokia Shanghai Bell, Qualcomm, Spreadtrum, ZTE, Google, OPPO, xiaomi, ETRI

Acceptable by: DOCOMO

Option 2: For TRP-specific SRS using one SRS resource set

Pros:

Receive powers at the CJT TRPs are accurate

Cons:

Higher SRS overhead and/or higher latency compared to Option 1

Support by: Ericsson, Futurewei, Google, Intel, InterDigital, KDDI, LGE, Sharp

Acceptable by:

Option 3: For TRP-common and/or TRP-specific SRS using one SRS resource set

Pros:

Flexible; combines both Option 1 and Option 2

Cons:

Higher complexity than Options 1 or 2

Support by: Futurewei, ZTE

Acceptable by:

There are more proponents for Option 1 than Option 2, though not by a large margin. It seems the only way forward is to support Option 1 as in the following initial proposal, unless companies are willing to compromise. Please indicate above if an option is also acceptable (e.g., as a compromise). If no compromise is made, then we will end up with Option 0, i.e., no enhancements in Rel-18.

Proposal 2.2: For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, for an SRS resource set, support Option 1 (for TRP-common SRS using one SRS resource set):

One power control process for all SRS resources of an SRS resource set where the power control process is based on one P0 value and one closed loop state and jointly on M (M > 1)  DL pathloss RSs and/or M alphas.

Each transmission occasion of the SRS resource is towards multiple TRPs.

Please share your view. If you can also accept an option that is not your first preference, please also indicate below.

| Company | View |
| --- | --- |
| QC | Ok with Option 1, but as we mentioned before, no enhancement for power control is also ok for us given that power can be adjusted by CLPC. |
| Google | Support the proposal in principle. |
| DOCOMO | Added DOCOMO as “Option 1 is acceptable”. We share QC’s view. |
| OPPO | We are also fine with no enhancement for power control. |
| Xiaomi | Support the proposal |
| Huawei, HiSilicon | OK with Option 1. |
| Nokia/NSB | Support Proposal 2.2 |
| InterDigital | Don’t support the proposal. |
| Samsung | Not support power control enhancement (i.e., Option 0 in the above), as QC and Docomo. |
| ZTE | Support Option 1 based on the following reasons: - If Option 2 is adopted, gNB cannot acquire the differences among transmitting powers towards different TRPs, hence cannot determine CJT precoder; - Option 2 causes extra latency and slow update of CJT precoder; - Option 2 requires higher SRS overhead and may cause strong inter-cell interference; - If Option 1 is adopted, each TRP should allocate SRS ports to its target UEs (including serving UEs and CJT UEs), hence the power differences among SRSs transmitted from different UEs will not influence the channel estimation accuracy. |
| LGE | Do not support. As discussed in several meetings, option 1 cannot be the optimal solution since PL compensation is not properly performed for any TRP. |
| MediaTek | Support option 1. |
| Intel | Do not support the proposal. We think proper SRS Tx power is important for the TRP to get accurate channel information in coherent JT scenario. As discussed in previous meetings, there could be near-far issue in the scenario of CJT operation. As commented in previous meeting, Option 1 doesn’t help for proper SRS Tx power setting. Since multiple pathloss reference signals and only one power control state is configured, the SRS output power may be derived by averaging over the pathloss from different pathloss RSs. However, this fundamentally does not solve the issue. The TRP that is farther away may not be able to hear the SRS while the SRS still cause severe interference to the TRP that is closer. In addition, for Option 2, since only one SRS resource set is configured, the SRS overhead is not higher compared with Option 1. Therefore, we think Option 2 should be supported. |
| Lenovo | Support the proposal |
| vivo | Don’t support the power control enhancement. It can’t help for reducing the cross-SRS interference for all UEs. It depends on the situation of interferences on SRS resources from different UEs. Enhancement on power control for SRS may be helpful for some UEs in TDD CJT, but it would also cause more cross-SRS interference to other UEs. |
| ZTE2 | One question for TRP-specific power control SRS (or being based on individual SRS resource set): - If having TRP-specific Tx power (UL-Tx AGC/PA-factor is updated from UE side), can we assume that the UE still can maintain phase-consistency for different SRS transmissions for respective TRP? - If not, from NW perspective, the basic assumption for the CJT DL precoding based on TDD reciprocity seems to be broken? Therefore, for Cons in Option-0/2, the following may need to be highlighted: - CJT DL precoding based on TDD reciprocity may fail due to phase inconsistency for respective TRP-specific/individual SRS transmissions by using different UL Tx powers. Then, for FR1, either way, a unified closed loop power control adjustment seems more efficient for handling interference from serving TRPs. |
| CMCC | OK with Option 1. |
| NEC | Fine with the proposal. |
| Sharp | Don’t support the proposal. For Option 1, the averaged pathloss for multi-TRPs results decreasing a pathloss associated with a far TRP. Therefore, to reach the far TRP, the UE needs to consume more UL resources by repetition and more interference is caused. We prefer Option 2 but can accept Option 0. |
| CATT | Ok with the proposal. |
| KDDI | Don’t support the proposal. In Option1, one power control process with multiple PLs cannot solve the near-far problem. Also, in closed loop, multiple TRPs need to decide on one command for power control, but whether to increase or decrease power may be different for each TRP, preventing proper power control. |

## SRS TD OCC

We have achieved the following agreement:

Agreement

For SRS TD OCC for SRS enhancements for TDD CJT, study:

- Comparison against SRS on 1 OFDM symbol

- Comparison against SRS repeated on multiple OFDM symbols

- Study the following aspects: evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

The general situations are:

Also related to 8 Tx SRS design

Pros:

Capacity enhancement (compared to SRS repetition)

Useful for coverage-limited scenarios

Useful for long channel delays

Useful for channels with long delay spread

Two sources show performance benefit

Cons:

High overhead (compared to SRS without repetition)

May not be highly relevant to interference-limited scenarios such as TDD CJT

Issues when SRS is dropped on one of the OFDM symbols

Supported by: ZTE, NEC, Intel, CMCC

The following is suggested:

Proposal 2.3: Support SRS TD OCC for TDD CJT SRS enhancement.

FFS: Dropping rules of collision with other uplink resource, other details.

Views can be provided for the above enhancement.

| Company | View |
| --- | --- |
| QC | Do not support. |
| Google | In our view, TD-OCC can be deprioritized. TD-OCC may potentially introduce a lot of issues for further study, e.g., whether we consider anything like OCC hopping for interference randomization? |
| DOCOMO | Deprioritize, as suggested by Google. |
| Xiaomi | Do not support |
| Apple | WE already have TDM ports, TD-OCC may not be needed |
| Nokia/NSB | Do not support. |
| InterDigital | Do not support |
| Samsung | Not support |
| ZTE | Support TD-OCC based on the following reasons: - Compared with legacy SRS repeated on multiple OFDM symbols - TD-OCC increase SRS capacity by L (L is the length of TD-OCC) times; - TD-OCC reduces the physical resource overhead of SRS by locating more SRS ports on the same OFDM symbols; - TD-OCC provides more flexibility of co-scheduling of UEs, since one comb can be allocated to multiple UEs different frequency resources; - TD-OCC provides higher channel estimation performance than CS multiplexing in cases of large delay spread. - Compared with legacy CDM SRS on single OFDM symbol - TD-OCC provides power boost gain; - TD-OCC provides more flexibility of co-scheduling of UEs, since one comb offsets can be allocated to multiple UEs with different frequency resources; - TD-OCC provides higher channel estimation accuracy than CDM in cases of large delay spread. Regarding the collision issue, there exist two cases: - Case 1: Only partial SRS resources multiplexed by TD-OCC is dropped in partial OFDM symbols. As shown in Figure 10, SRS-1 is replaced by PUSCH-1, and then PUSCH-1 and SRS-2 will be strong interference in OFDM 1 same as Rel 15/16/17. This case should be avoided by gNB scheduling. - Case 2: All SRS resources multiplexed by TD-OCC are dropped in same OFDM symbols, then the multiple SRS resources are transmitted on same number of OFDM symbols. The multiple SRS resources can be transmitted using TD-OCC of another length, such as length 3. Of course, the SRS resource with original fourth TD-OCC [1, -1, -1, 1] can be dropped on all OFDM symbols. It does not impact the legacy UE behavior. Its capacity is same as or even better than legacy. |
| LGE | Agree with Google. Benefit is not clear. Also, we prefer to focus on candidate schemes with majority supports. |
| MediaTek | We don’t support TD-OCC. |
| Intel | Generally fine with the proposal. |
| Lenovo | We are fine with SRS TD OCC in case of multiple symbol SRS repetition. Since SRS TD OCC is not introduced for 8Tx, we are open for more discussion here for CJT. |
| vivo | Don’t support TD-OCC for CJT. It can’t provide more capacity compared with configuring more SRS resources in TDM manner. |
| Sharp | Not support. |
| CATT | Do not support TD-OCC. |

## Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

At least the following potential enhancements have been discussed:

Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

Enh. 1: Configuration of v (sequence index within a group) per SRS resource (e.g., Qualcomm)

Pros:

Improved reused factor to reduce sequence collisions

Supported by: Qualcomm

Enh. 2: New cyclic shift mapping to enable non-uniform cyclic shift allocation within the range [0, n_CS^max - 1] across ports

Configuration of cyclic shift per SRS port per SRS resource, or dividing [0, n_CS^max - 1] in multiple non-overlapping sub-regions.

Pros:

Optimized CS allocation; useful for long channel delays

Useful for channels with long delay spread

Two sources show performance benefit

Cons:

Potential PAPR issue

Supported by: Samsung, Qualcomm, Futurewei

To move forward, the following proposal can be discussed.

Proposal 2.4: Support new cyclic shift mapping, and if yes, support:

- For a multi-port SRS resource, new cyclic shift mapping to enable non-uniform cyclic shift allocation within the range [0, n_CS^max - 1] across ports.

This does not change the comb offset mapping across ports.

E.g., per-port configuration.

E.g., dividing [0, n_CS^max - 1] in multiple non-overlapping sub-regions.

Views can be provided for the above enhancement.

| Company | View |
| --- | --- |
| QC | Support. |
| Google | In our view, we do not need to discuss this proposal since we have achieved the conclusion below in last meeting. This is a kind of “signaling for flexible SRS transmission”. Conclusion No consensus on enhanced signaling for flexible SRS transmission in Rel-18 |
| OPPO | Agree with Google. |
| Xiaomi | Do not support |
| Huawei, HiSilicon | Open to discuss. |
| Apple | It is not high priority. |
| Nokia/NSB | Deprioritize and potentially revisit after defining details of basic hopping schemes. |
| InterDigital | Do not support |
| Samsung | Support. We evaluated this new cyclic shift mapping and it has benefit rather than legacy CS mapping. Also this can be easily combined with CS hopping by distinguishing hopping region. |
| ZTE | Do not support. As fore-mentioned in 2.1.4, non-uniform cyclic shift allocation may shorten the length of zero-correlation zone and degrades the channel estimation accuracy. |
| LGE | Agree with Google. |
| MediaTek | Do not support. |
| Intel | Agree with Google. |
| Lenovo | Same view as Google and OPPO |
| vivo | Do not support |
| CMCC | Agree with Google. |
| Sharp | Not support. |
| CATT | Do not support |

## Precoded SRS for DL CSI acquisition

At least the following potential enhancements have been discussed:

Pros:

Capacity enhancement

Two sources show performance benefit

Cons:

NW does not have full MIMO channel information

UE Tx/Rx calibration complexity

PAPR issue

Supported by: CMCC, Google, Huawei, HiSilicon, ZTE, Nokia, Nokia Shanghai Bell, Futurewei

Given the large number of supporting companies, the following is suggested:

Proposal 2.5: Support precoded SRS for DL CSI acquisition

- Reusing the standardized mechanism for NCB SRS as the baseline design for precoded SRS for DL CSI acquisition and striving to minimize the standard impact.

- Only for UEs supporting NCB SRS

FFS: If any additional UE antenna calibration is needed for precoded SRS for DL CSI acquisition

- For UEs with xTxR

FFS: For UEs with xTyR

- This feature is optional and subject to UE capability.

Views can be provided for the above enhancement.

| Company | View |
| --- | --- |
| QC | Do not support. |
| Google | Support the proposal |
| OPPO | Don’t support the proposal. |
| Xiaomi | Do not support considering its application restrictions. |
| Huawei, HiSilicon | Given the significant performance benefit and the large number of supported companies, precoded SRS should be considered as a promising capacity enhancement scheme in Rel.18. |
| Apple | Prefer to deprioritize |
| Nokia/NSB | Support proposal 2.5. Regarding to concern on PAPR, any proponent can explain the issue? |
| InterDigital | Do not support |
| Samsung | Not support |
| ZTE | We are open to discuss this issue. |
| LGE | Do not support. |
| MediaTek | Do not support. |
| Intel | Do not support. |
| Lenovo | Prefer to treat precoded SRS enhancement as low priority on account of raised concern from calibration and specification impact. |
| vivo | Do not support. |
| Sharp | Not support. |
| CATT | Do not support |

## Randomized transmission of SRS

- The following has been discussed:

Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

Pros: Interference randomization; two sources show performance benefit

Supported by: vivo, Qualcomm

To move forward, the following potential proposal can be discussed.

Proposal 2.6: Support pseudo-random muting of SRS transmission:

- Muting / transmission is per SRS transmission occasion, where a bundle of L transmission occasions is considered together.

- An index corresponding to a binary sequence of length L from a set of binary sequences is determined based on the pseudo-random sequence c(i) as a function of time (e.g., slot number, symbol number) with a certain UE-specific initialization.

Views can be provided for the above enhancement.

| Company | View |
| --- | --- |
| QC | Support. |
| Google | In our view, we do not need to discuss this proposal since we have achieved the conclusion below in last meeting. This is a kind of “signaling for flexible SRS transmission”. Conclusion No consensus on enhanced signaling for flexible SRS transmission in Rel-18 |
| Xiaomi | Do not support considering channel estimation accuracy and its application limitation. |
| Apple | Prefer to deprioritize |
| Nokia/NSB | Do not support due to high SRS overhead. We don’t have much UL resources for SRS transmission. |
| InterDigital | Do not support |
| Samsung | Not support |
| ZTE | Do not support. For the purpose of interference randomization, there is no necessary to further introduce randomized muting beyond CS / comb offset hopping. |
| MediaTek | Not Support. |
| Intel | Do not support. |
| Lenovo | Similar view as Google. |
| vivo | Support. We would like to mention that SRS muting is a time domain hopping scheme, similar to CS/ comb offset hopping, which is not a scheme mainly dependent on enhanced signaling. |
| Sharp | Not support. |
| CATT | Do not support |

## Other SRS capacity enhancements

Several companies described that SRS capacity enhancement is critical to this WI and practical CJT deployment. The following potential enhancements have been discussed, and new proposals on SRS capacity enhancements are welcome.

Enh. 1: Multiplying mask sequence to the legacy SRS sequence

Pros: SRS capacity increase

Cons: Small increase of PAPR

Supported by: Huawei, HiSilicon

Views can be provided for the above enhancements, and new proposals can also be included.

| Company | View |
| --- | --- |
| QC | Do not support. |
| Google | In our view, we should focus on the issues we have already studied for a few meetings. This should be deprioritized. |
| Huawei, HiSilicon | Support. Enh.1 breaks the ZCZ property to pursue interference equalization through multiplying mask sequence to the legacy SRS sequence, which can obtain performance benefit with even lower PAPR through proper sequence selection. |
| Nokia/NSB | Deprioritize |
| InterDigital | Do not support |
| Samsung | Deprioritize and focus on the above candidate schemes |
| ZTE | We are open to discuss this issue. |
| LGE | Same view with Google. We should focus on already agreed one. |
| MediaTek | Not support. |
| Intel | Same view as Google. |
| Lenovo | Prefer to treat this as low priority |
| vivo | Do not support |
| CATT | Do not support |

## Other randomized frequency/code-domain resource mapping for SRS transmission

At least the following potential enhancements have been discussed:

Enh. 1: Further enhancements to frequency hopping, via different bandwidths in FH (e.g., Samsung)

Pros: Interference randomization, SRS capacity enhancement

Cons: Non-uniform SRS sample pattern in time/frequency domain

Views can be provided for the above enhancement.

| Company | View |
| --- | --- |
| QC | Do not support. |
| Google | In our view, we should focus on the issues we have already studied for a few meetings. This should be deprioritized. |
| Nokia/NSB | Do not support. |
| InterDigital | Do not support |
| ZTE | Do not support. For the purpose of interference randomization, there is no necessary to introduce randomized frequency bandwidth mapping beyond CS / comb offset hopping. |
| LGE | Agree with Google. |
| MediaTek | Not Support |
| Intel | Do not support. |
| Lenovo | Prefer to treat this as low priority |
| vivo | Do not support. |
| CMCC | Do not support. Now we have agreed with CS/Comb offset hopping, it should be deprioritized. |
| CATT | Do not support |

## Others

Some other designs for TDD CJT SRS were also discussed:

Applicability to usages other than ‘antennaSwitching’

- These can be further discussed. Companies’ views on the above are collected as follows. Any other views, issues, potential enhancements, and clarifications can also be provided.

| Company | View |
| --- | --- |

# SRS enhancements targeting 8 Tx operation

It is well known that increasing UE Tx antenna ports can significantly improve various performance metrics for UL/DL transmissions. 8 Tx transmissions can be feasible for at least CPE/FWA/vehicle/industrial devices and hence can be beneficial.

Some remaining issues on the number of SRS resources for 8 Tx SRS and the number of SRS resource sets for 8 Tx SRS will be discussed in agenda item 9.1.4.2 covering “SRI/TPMI enhancement for enabling 8 TX UL transmission; To support up to 4 or more layers per UE in UL targeting CPE/FWA/vehicle/industrial devices”.

## Non-TDMed 8 Tx SRS

We will continue to discuss some detailed designs for non-TDM 8 Tx SRS, such as comb and comb offset allocation, cyclic shift allocation, etc., based on legacy non-TDMed port mapping, i.e., one symbol, repetition, FH, RPFS.

### Comb offset

We had the following agreement:

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support:

- For comb 2, support 1 and 2 comb offsets

- For comb 4, support 2 and [4] comb offset

- For comb 8, support 4 comb offsets

We have a remaining issue for the comb 4 SRS, that is, whether 4 comb offsets will be supported. The summary from the contributions is:

For the number of comb offsets for an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ with comb 4 :

4 comb offsets is supported:

Supported by CATT, CMCC, Ericsson, Futurewei, InterDigital, Lenovo, Sharp, vivo, Xiaomi

4 comb offsets is NOT supported:

Supported by Apple, Google, Intel, OPPO, QC, ZTE

As this decision can affect other designs, it is preferable to decide whether to support this in this meeting.

Proposal 3.1.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), select one of the options from below in RAN1#112:

Option 1: For comb 4, support 4 comb offsets.

Option 2: For comb 4, do not support 4 comb offsets.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | We do not think we need this proposal. We have already stated “[4]” in the last agreement. This proposal does not help for progress, but it is like a kind of retransmission of last agreement with a different RV. |
| DOCOMO | Agree with Google. |
| OPPO | Agree with Google, and considering the situation, maybe we can have a conclusion that there is no consequence to support 4 comb offsets for comb 4. |
| Xiaomi | We support option1. |
| Huawei, HiSilicon | Support option 1. |
| Apple | Prefer option 2 |
| ZTE | We prefer option 2. For sake of simplicity, we suggest only ONE comb offset and one set of CS offsets for each comb value i.e., kTC. - For comb = 4, 2 comb offsets and 4 CSs (among max 12 CSs), no more comb offsets are needed besides the already agreed as above. - In addition, a clarification for comb = 2: we support only 2 comb offsets and 4 CSs (among max 8 CSs) for non-TDM 8 ports. - If one comb offset as in the agreement is used for 8 ports in one symbol (i.e., non-TDM 8 ports), all 8 CSs would be occupied, which causes high sensitivity to delay issue. We do not believe it is a practical use case. So we suggest one comb offset can only be used for the case of m>1 symbols with TDM scheme. We suggest to discuss this problem for non-TDM and TDM separately. |
| QC | We don’t see the significance to support 4 comb offsets for comb-4 structure for Rel-18 SRS. We don’t see how why this small issue will impact other design as well. Nothing fails in Rel-18 SRS with this particular SRS configuration being excluded. We support option 2. If no consensus can be reach, we suggest that RAN1 just move on and not revisit this small issue. |
| MediaTek | Support Option 2. |
| Intel | Prefer to Option 2. Agree with other companies and we don’t see the strong need to support 4 comb offsets for comb-4 structure. |
| Lenovo | Support the proposal and option 1. |
| vivo | Prefer option 1. In case of lager delay spread, 4 comb offsets can be configured for SRS. Otherwise, the gap between different CS is only 3. |
| FL | @Google @DOCOMO @OPPO @QC: If 4 comb offsets for comb 4 is not supported, then existing mechanism can be directly used to determine the number of comb offsets when a comb value is configured; but otherwise, we may need a new way to signal the number of comb offsets (see Sec. 3.1.3). There may also be other differences in the spec impact. In any case a conclusion here should be generally helpful. |
| CMCC | Support Option 1. To split the ports over 4 comb offsets could improve robustness to channel delay spread, and to support both 2 comb offsets and 4 comb offsets are also beneficial for network scheduling flexibility. |
| Sharp | We prefer Option 1. the benefit of Option 1 is clear and has low spec impact. This is because using 4 comb offset values and 2 cyclic shift values for 8 ports was agreed for comb 8. |
| CATT | Support Option 1. For 4-port SRS resource(s) in Rel-15, 1 and 2 comb offsets are supported for comb 4, with the minimum cyclic shifts difference between two SRS ports mapping to the same OFDM symbol be 3 and 6, respectively. For 8-port SRS resource(s) mapped to the same OFDM symbol(s), same maximum minimum cyclic shifts difference between two SRS ports shall be kept. Therefore, for comb 4, in addition to 2 comb offsets, 4 comb offsets shall be supported. |

### Port mapping on multiple comb offsets

Several options for 8-port mapping on multiple comb offsets have been provided, as listed in the proposal below.

Proposal 3.1.2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is configured with kTC>1 comb offsets, determine the mapping from the ports to comb offsets as follows:

If kTC=2, select one of the options:

Option 2-1: ports {1000, …, 1003} are mapped on the first comb offset, {1004, …, 1007} on the second comb offset.

Option 2-2: ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, {1001, 1003, 1005, 1007} on the second comb offset

Option 2-3: ports {1000, 1001, 1004, 1005} are mapped on the first comb offset, {1002,  1003, 1006, 1007} on the second comb offset

If kTC=4, select one of the options:

Option 4-1: ports {1000, 1001} are mapped on the first comb offset, {1002, 1003} on the second comb offset, {1004, 1005} are mapped on the third comb offset, and {1006, 1007} on the fourth comb offset.

Option 4-2: ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} are mapped on the third comb offset, and {1003, 1007} on the fourth comb offset.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Support. |
| DOCOMO | Support |
| OPPO | We think one antenna port group can be allocated with the same comb offset. We can wait for the conclusion on antenna port index grouping in 9.1.4.2. |
| Xiaomi | Support in principle. For k_TC=2，we support option2-2; For k_TC=4,we support option 4-2; |
| Huawei, HiSilicon | Support. |
| Apple | Prefer the second option (staggered mapping) which we think is the legacy mapping |
| Nokia/NSB | Agree with Oppo that antenna port indexing is subject to 9.1.4.2. |
| ZTE | We prefer option 2-2 and option 4-2. Generally, 8 ports should be firstly grouped according to antenna layout, considering coherent capability among ports. We agree with Oppo and Nokia to use port indexing for port groups in AI. 9.1.4.2 for port groups, e.g., allocating same comb offset for non-TDM and same OFDM symbol for TDM for at least one port group. Given that this problem has been discussed in AI.9.1.4.2 but not agreed yet, we can discuss this for further discussion on this topic, e.g., to agree to reuse port indexing for port groups in AI. 9.1.4.2 for port groups. Note that we can discuss comb offset/ CS offset /OFDM symbol allocating scheme for ports using local port index in a port group (with same comb offset, or same OFDM symbol), the exact port indexing can be determined later, e.g., as long as before spec drafting. We suggest the following proposal: Proposal 3.1.2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols and/or one or more comb offsets, determine the mapping as follows: Same comb offset for ports of at least one port group, e.g., for non-TDM Same OFDM symbol for ports of at least one port group, for TDM E.g., if Ng=2, there are 2 port groups, ports in the first port group are allocated with a first comb offset, and ports in the second port group are allocated with a second comb offset. |
| LGE | Support option 2-2 and option 4-2, which are similar to the legacy scheme. In 4-port mapping, {1000, 1002} are mapped on the first offset, and {1001, 1003} are mapped on the second offset |
| QC | The options seem a little Ad hoc. We are not sure what is the benefit or technical justification for each option. We understand certain option can make sure that one antenna port group can be allocated with the same comb offset. But we don’t see that is a big advantage. To us, mapping one antenna port group to different comb offset is also fine. We hope the pros and cons of each option can be discussed before we make a decision. So far, it seems there is no big difference among those options. We can see that option 2-2 and 4-2 are the legacy way to do mapping. So, we prefer option 2-2 for kTC=2, and option 4-2 for kTC=4 for now. |
| MediaTek | Support option 2-2 and option 4-2. |
| Intel | In the spec, k_TC is the parameter for comb size. It’s not accurate to say the SRS resource is configured with k_TC>1 comb offsets. Suggest the following changes to make it more accurate. Proposal 3.1.2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is configured with kTC>1 multiple comb offsets, determine the mapping from the ports to comb offsets as follows: If kTC=2 two comb offsets are configured, select one of the options: Option 2-1: ports {1000, …, 1003} are mapped on the first comb offset, {1004, …, 1007} on the second comb offset. Option 2-2: ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, {1001, 1003, 1005, 1007} on the second comb offset Option 2-3: ports {1000, 1001, 1004, 1005} are mapped on the first comb offset, {1002, 1003, 1006, 1007} on the second comb offset If kTC=4 four comb offsets are configured, select one of the options: Option 4-1: ports {1000, 1001} are mapped on the first comb offset, {1002, 1003} on the second comb offset, {1004, 1005} are mapped on the third comb offset, and {1006, 1007} on the fourth comb offset. Option 4-2: ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} are mapped on the third comb offset, and {1003, 1007} on the fourth comb offset. |
| Lenovo | For kTC=2, support option 2-3 For kTC=4, support option 4-2 |
| vivo | Following the legacy principle is preferred, then legacy formula can be reused as much as possible. For kTC=2, support option 2-2. For kTC=4, support option 4-2. |
| ZTE | Sorry for one typo. For kTC=2, support option 2-3 rather than option 2-2. |
| FL | Thank you for all the good inputs. We can continue the discussion and wait a bit (at least for the first few days in this meeting) to see if an agreement can be made in 9.1.4.2. @Intel: I think you refer to KTC while here we are discussing kTC. Sorry for any confusion. |
| CMCC | For comb=2, suppot Option 2-2 For comb=4, postpone the discussion, since whether 4 comb offsets can be supported or not is under discussion. |
| Sharp | Support Option 2-2 and Option 4-2 to reuse legacy principle. |
| CATT | Ok with the proposal. For SRS transmission for partial-coherent PUSCH transmission, we prefer to map antenna ports in the same coherent antenna group into the same comb offset. Therefore the down-selection can be discussed after partial coherent codebook is available. |

### Cyclic shift locations across the comb offset(s) and indication of the number of comb offsets

There are a few options proposed for how to determine the cyclic shift locations for each comb offset and across the multiple comb offsets, which is also related to how the number of comb offsets is determined. For example, CATT, CMCC, NTT DOCOMO, and vivo proposed to extend the legacy mechanism, i.e., there is only one configured cyclic shift location nSRScs, and based on its value compared with nSRScs,max/2, the number of comb offsets is determined. Based on how the existing cyclic shift/comb offset equations are generalized, the cyclic shifts may be aligned across the comb offsets. These details can be subject to further study. Another option is to explicitly configure multiple cyclic shift locations for the multiple comb offsets.

Proposal 3.1.3: For an 8-port SRS resource configured with comb KTC, the number of comb offset(s) and the cyclic shift locations are determined as follows:

If only one number of comb offsets is allowed for the comb KTC, that number of comb offsets is used.

If x comb offset(s) and y comb offsets are allowed for the comb KTC, select from one of the options:

- Option 1: Based on the one RRC configured cyclic shift location nSRScs. FFS details.

  - E.g., for comb 2, if nSRScs<nSRScs,max/2, then 1 comb offset is used, otherwise 2 comb offsets are used, where nSRScs,max is the maximum number of cyclic shifts per comb offset.

  - FFS: how the one configured cyclic shift location nSRScs determines all the cyclic shift locations on multiple comb offsets.

- Option 2: Based on RRC configured cyclic shift location(s) nSRScs,(j), where j∈{0,1,…,x-1} and x is the number of comb offset(s).

FFS cyclic shift location equation on a comb offset based on the one or more configured cyclic shift locations.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | We do not know why we need such a proposal, which seems to be with a complicated formulation. Probably we can simply say “reuse legacy scheme for CS and comb offset configuration”. |
| DOCOMO | @Google, in our understanding, option 1 could be similar to legacy, while option 2 is different from legacy. Support the proposal as a starting point. We prefer option 1, as legacy approach seems no problem so far. |
| Huawei, HiSilicon | We think the comb offset(s) and the cyclic shift location can be decided before the indication method for clearness. |
| Apple | We prefer RRC to explicitly configure the number of Comb offsets, not the legacy Rel-15 way (option 1) |
| ZTE | As stated in section 3.1.1, we suggest only ONE comb offset and one set of CS offsets for each comb value i.e., kTC. So the second part may not be supported. We suggest to discuss this proposal after conclusion is reached in section 3.1.1. |
| LGE | We prefer option 1. In the legacy scheme, the same operation as option 1 is performed when NapSRS=4, kTC=2,4. |
| QC | We don’t think option 2 can work. The problem needs to be solved is finding a method to decide whether use x comb offsets or y comb offsets, i.e., a binary decision. We can option 1 can work by using nSRScs<nSRScs,max/2 to make the binary decision. But we don’t know how option 2 can make the binary decision by configure x comb-offset specific CS shift locations nSRScs,(j), where j∈{0,1,…,x-1}. Maybe we missed something there. But option 2 needs some clarification. We don’t support the proposal before it is clarified. So far, we support option 1. |
| Intel | We think this proposal can be postponed. It can be discussed after consensus is reached in Section 3.1.1 and Section 3.1.2. |
| Lenovo | Support FL proposal. |
| vivo | Support option 1, as what in the current spec. |
| FL | @Proponenets of Option 1: Could you please suggest the CS equations based on the legacy scheme for 2 and 4 comb offsets when comb 4 is configured? Do the CSs on the multiple comb offsets have to be aligned or not? If not, it seems to me that some extra parameters/rules are needed. Please feel free to suggest an updated proposal. @Huawei: Please feel free to suggest an updated proposal. @QC: For Option 2, if 2 CS locations are configured, then 2 comb offsets are used; if 4 CS locations are configured, then 4 comb offsets are used. For example, if {CS0,CS2,CS0,CS1} are configured, then 4 comb offsets are used, and on the first comb offset, the 2 CSs are uniformly spaced with the first local port occuping CS0, etc. |
| CMCC | Support Option 1. For 2 and 4 comb offsets when comb 4 is configured, the CSs on the multiple comb offsets have to be aligned as legacy scheme. |
| CATT | We prefer option 1. It is aligned with the legacy approach. |

## 8 Tx SRS with TDM

We have agreed to support 8 ports SRS with TDM in the past meeting.

Agreement

For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols

- Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM)

- FFS: m can be legacy values, i.e., 2,4,[8,10,12,14].

### TDM factor and number of OFDM symbols

A clarification here is that m in the above agreement is the configured number of OFDM symbols in a slot for the SRS resource, and it should not be confused with the TDM factor, since the TDM scheme may still be enabled together with repetition, frequency hopping, and/or RPFS. To differentiate, denote the TDM factor as s. The following have been proposed by companies:

- The 8 ports are evenly split into s subsets of ports, and each subset has 8/s different ports. Each OFDM symbol used by the SRS carries one subset of the 8/s ports. On each OFDM symbol, the subset of the 8/s ports is mapped to physical resources (i.e., PRB, REs, etc.) based on the legacy mechanisms, and is mapped to cyclic shifts based on the legacy mechanisms with port indexes local to the OFDM symbol.

  - Generally supported by Apple, CATT, CMCC, Ericsson, Futurewei, Huawei, HiSilicon, InterDigital, OPPO, Qualcomm, vivo, ZTE.

- Almost all companies support s = 2, and a large number of companies also support s = 4. Some other companies also support s = 8.

- Which subset of 8/s ports is mapped onto which OFDM symbol(s) will be further discussed in Sec. 3.2.2.

We have the following initial proposal:

Proposal 3.2.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

s = {2, 4}. FFS: s = 8.

m = 2,4,8,10,12,14, and m is a multiple of s.

Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | Support |
| OPPO | We are generally fine with the proposal. However, the relationship between subset and OFDM symbol is unclear in the proposal. We propose the following: roposal 3.2.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. The s subsets are mapped to s different OFDM symbols. - FFS whether the s OFDM symbols need to be adjacent or not. s = {2, 4}. FFS: s = 8. m = 2,4,8,10,12,14, and m is a multiple of s. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. FFS: which subset of 8/s ports are mapped onto each OFDM symbol. FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. For port mapping, when m is not larger than Ng, one antenna port group can be mapped to the same OFDM symbol for CSI accuracy, while different antenna port groups can be mapped to different OFDM symbol. |
| Xiaomi | support |
| Huawei, HiSilicon | Considering reusing the existing resource mapping designed for 8/s ports on each OFDM symbol may lead to multiple mapping patterns with similar effect, further down-selection should be considered. |
| Apple | Does this mean there is no intra-slot frequency hopping? |
| Nokia/NSB | Support proposal 3.2.1 |
| ZTE | Thanks for FL’s good clarification for s and m. We are generally fine with this proposal. On “FFS: which subset of 8/s ports are mapped onto each OFDM symbol.”, as stated in section 3.1.2, we tend to agree to determine port indexing according to port groups same as in AI. 9.1.4.2. |
| LGE | Prefer s = 2 only. Because when s increases, the probability of collision with other UE channels also increases. Then, 8 ports cannot be fully sounded. |
| QC | We support this proposal in general. |
| MediaTek | Support in principle. Agree with the comment made by LGE, we prefer s=2 only. We also do not see the benefit of having such large values of m, i.e., m>8. |
| Intel | We think s=2 is sufficient. s=4 requires more OFDM symbols to be configured. In addition, we think the value of 8, 10, 12, 14 for m should be further studied. Suggest the following change: s = {2, 4}. FFS: s =4, 8. m = 2,4,[8,10,12,14], and m is a multiple of s. |
| Lenovo | Support FL proposal. |
| vivo | Support FL proposal. |
| FL | @OPPO: Your added FFS is already implied in “FFS: which subset of 8/s ports are mapped onto each OFDM symbol”. Anyway I rephrased the proposal to make it more clear. @all: The possible values for ‘s’ and ‘m’ can be further discussed. Proposal 3.2.1A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. s = {2, 4}. FFS: s = 8. m = 2,4,8,10,12,14, and m is a multiple of s. Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs FFS: which subset of 8/s ports are mapped onto each OFDM symbol. FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. |
| CMCC | Support Proposal 3.2.1A. |
| CATT | Fine with the proposal. |
| KDDI | Fine with the proposal. |

### TDMed time-domain mapping

As mentioned above, which subset of 8/s ports is mapped onto which OFDM symbol(s) needs discussion. There are a few proposals in this regard, and they are generally summarized below. Denote the s subsets of ports as subset 1, subset 2, …, subset s.

Some companies suggested the set of 8 TDMed ports should always be mapped on s adjacent (i.e., consecutive, contiguous) OFDM symbols, that is, any set of s adjacent symbols would carry exactly 8 different ports. E.g., when TDM and repetition are configured, repetition is done after the 8 different ports are transmitted. That is, the subsets are transmitted cyclically in the order of {1,2,…,s,1,2,…,s}.

Some companies suggested the set of 8 TDMed ports do not need to be mapped on s adjacent OFDM symbols, e.g., when TDM and repetition are configured, the first R symbols are for repetition of a first subset of 8/s ports, and then the next R symbols are for repetition of a second subset of 8/s ports, etc. That is, the subsets are transmitted sequentially in the order of {1,1,2,2,…,s,s}.

Some companies suggested whether the set of 8 TDMed ports are adjacent to each other in time domain depends on the antenna coherence (fully coherent, partially coherent, or non-coherent), SRS usages (antennaSwitching or codebook), antenna panels, etc.

Some companies suggested the m OFDM symbols can be non-consecutive in time domain.

To proceed, some high-level decisions have to be made. We suggest the following proposal.

Proposal 3.2.2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2:

Select one of the following options regarding the m OFDM symbols:

  - Option 1-1: the m OFDM symbols are adjacent.

  - Option 1-2: the m OFDM symbols can be adjacent or non-adjacent.

Select one of the following options regarding the TDM pattern:

  - Option 2-1: the s subsets of ports are mapped cyclically.

  - Option 2-2: the s subsets of ports are mapped sequentially.

  - Option 2-3: the s subsets of ports are mapped cyclically or sequentially.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | We suggest we send an LS to RAN4 to ask whether any gap is needed. TDMed scheme requires per symbol power change for each antenna port. |
| OPPO | For the first issue, we think the legacy design can be followed. For the second issue, it should be restricted to the case when repetition or intra-slot frequency hopping is configured for the SRS resource. |
| Xiaomi | Support the proposal |
| Huawei | Support Option 1-1 and Option 2-2 to maintain the phase continuity. |
| Apple | Option 1-1 Option 2-2 |
| Nokia/NSB | Support Proposal 3.2.2 with Option 1-2 and FFS: Options 2-1, 2-2 and 2-3. |
| Samsung | Support with Option 1-1 and Option 2-3 since cyclic and sequential mapping have different benefits. Cyclic mapping can have benefit on overlapping case with other UL channel and minimize channel estimation timing difference between TDMed mapped ports. Sequential mapping is beneficial to minimize different port mapping in time domain. Also as OPPO mentioned this cyclic and sequential mapping is only considered with repetition or intra-slot frequency hopping. Regarding gap symbol, we have asked similar question to RAN4 in Rel-17 multi-TRP PUSCH/PUCCH repetition whether the gap symbol is needed or not between two different repetition with different spatial filter. Also the answer in the reply LS (R1-2102297) includes switching among different ports. Based on RAN4 LS, RAN1 made a conclusion that no consensus on switching gap symbol in RAN1#104b-e. Hence, similarly, we think the switching gap is not needed. |
| ZTE | We prefer option 1-1, and option 2-2. For option 1-x, we discuss adjacent and non-adjacent symbols in one slot. If in more than one slot, symbols may not be adjacent. For option 2-x, we prefer all repetitions for a port groups to be done firstly. |
| LGE | For the first issue, same view with OPPO(option 1-1) For the second issue, prefer option 2-1. Regarding this issue, we have clarification question. m is the number of symbols configured for a SRS resource? Then, what is the UE behavior when Repetition factor R>1 for the SRS resource? |
| QC | We don’t support this proposal. It is too high level. For example, on the first bullet, regarding the m OFDM symbol should be adjacent or non-adjacent, we think it should depend on the SRS usage and the coherency of codebook. For coherent codebook, the m OFDM symbols must be adjacent and in one slot (cannot across slot boundary), otherwise the coherence is broken. For non-coherent codebook, the m OFDM symbol can be non-adjacent and can across slot boundary. On the second bullet, we think the formulation is not clear. We suggest at least add a note to clarify what are the definition of cyclic mapping and sequence mapping. |
| MediaTek | Support Option 1-1and Option 2-2 |
| Intel | We suggest to firstly focus on the case of “s=2”. The case of “s>2” could be further discussed once we have clear picture on “s=2”. We think the m symbols should be adjacent. |
| Lenovo | We prefer option 1-1 and option 2-1. |
| vivo | Support Option 1-1. Support Option 2-1, i.e., {1, 2, …, s,1, 2, …, s}. 8 ports should be adjacent as much as possible. Otherwise, due to the Doppler shifts especially in high speed, it would impact on the channel estimation results of the first two port on the first symbol and the last two port on the last symbol, which would degrade the determination of TMPI and CQI. |
| FL | @Google: Could you please elaborate the case when per-port per-symbol power change is required? Please note that the power ramping between 0 and the TxP should be similar to legacy systems and no gap is required at least in RAN1 specs. Please also see Samsung’s reply below and RAN1#104bis-e conclusion: Conclusion With reference to the normative work on NR-feMIMO: Related to the support of switching gap between UL transmissions towards two TRPs in RAN1 specifications, there is no consensus in RAN1 to specify symbol gap(s) for the following cases - PUSCH Type A - PUCCH scheme 1 - PUSCH Type B - PUCCH scheme 3 The above applies for the case included in the LS from RAN4 in R1-2102297. @ZTE: Since it is “regarding the m OFDM symbols”, it is already restricted in one slot. @LGE: This bullet is indeed mainly intended for R>1, and R=1 is trivial. @QC: I think we indeed need to first clarify at this very high level and then move forward. Any suggestions are welcome to make a progress. @all: Coherency is relevant here but we could make the proposal as general/comprehensive as possible; detailed discussions based on different coherency assumptions can be done as a next level of design. Proposal 3.2.2A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2: Select one of the following options regarding the m OFDM symbols: - Option 1-1: the m OFDM symbols are adjacent. - Option 1-2: the m OFDM symbols can be adjacent or non-adjacent. Select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols. - Option 2-3: the s subsets of ports are mapped cyclically or sequentially on the m OFDM symbols. |
| CMCC | Support Option 1-1. In Rel 15-17, adjacent OFDM symbols are used for SRS transmission regardless of antenna coherence, SRS usages, antenna panels, etc. The same scheme can be reused for 8-port SRS. Support Option 2-1. To consider the phase consistence, cyclically transmission can improve the channel estimation accuracy for 8-port SRS. Repetition transmission is used to improve reliability, which can be done after the 8 different ports are transmitted. |
| Sharp | We support Option 1-1 and Option 2-1. |
| CATT | Regarding the m OFDM symbols, support option 1-1. Legacy SRS resource design that adjacent OFDM symbols are allocated to an SRS resource should be reused. Regarding the TDM pattern, we are open to discuss. |

### Further details for TDMed SRS ports

Several companies further detailed enhancements for the TDMed SRS ports, and some of the enhancements may be related to the outcomes of above discussions.

Enh. 1: Power control, power scaling when some ports are transmitted with other UL signal, per-symbol full-power transmission

  - Supported by: CATT, Google, Huawei, HiSilicon, OPPO

Enh. 2: Collision / dropping handling

  - Supported by: OPPO, Qualcomm

Enh. 3: Guard symbol / gap

  - Supported by: Google

Enh. 4: Same number of symbols and TDM pattern for SRS resources for a SRS resource set

  - Supported by: Apple

Views can be provided for the above enhancements, and new details / proposals can also be included.

| Company | View |
| --- | --- |
| Google | In our view, Enh 1/2/3 are not enhancement, but they are to discuss the potential issue on power control for SRS transmission. We need to discuss such potential issues. |
| OPPO | Without Enh.1, there would be no gain to support TDMed SRS. Enh.2 is necessary to be discussed for a complete solution. As least these two issues are essential. |
| Xiaomi | Fine to discuss. |
| Samsung | Regarding Enh.1, power allocation for 8/s port can be re-used when s is TDM factor mentioned in section 3.2.1. Also, Enh. 2 shall be discussed since symbol-level SRS overlapping handling is already in current spec, and we need to define UE behavior when partial symbols of TDMed port mapping based SRS are overlapped with other UL channels. |
| ZTE | These enhancements can be discussed after completion of the above key issues. |
| LGE | We think that a discussion about Enh. 2 is needed. |
| QC | We think enh 2 is needed, otherwise spec is broken. |
| MediaTek | Support Enh. 2 |
| Intel | We think some power control enhancement is needed for TDM-ed 8-port SRS. In the legacy spec, the SRS transmit power is equally split over the configured ports. With the legacy scheme, there is no power boosting gain for the TDMed SRS. We don’t see the needs for Enh 2/3/4. |
| vivo | Open to discuss Enh. 2 |
| FL | Regarding the power control, in 38.213, we have: For SRS, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS. This may already cover the proper power scaling mentioned by the companies. For proponents of Enh. 1, is the intention here to clarify the understanding? E.g., is something like “… a UE splits … equally across the configured antenna ports for SRS on an OFDM symbol” sufficient? |
| CATT | Fine to discuss potential issues. For power control enhancement, it is our view that power scaling when some ports are transmitted with other UL signal and per-symbol full-power transmission are two separate issues. |
| KDDI | We think enh. 1 is needed. |

## Other designs / design parameters for 8 Tx SRS

Some other designs / design parameters for 8 Tx SRS were also discussed:

Non-uniform / non-equidistant distribution of cyclic shifts in a comb offset

Full power mode

Antenna switching downgrade

PAPR issue

- These can be further discussed. Companies’ views on the above are collected as follows. Any other views, issues, potential enhancements, and clarifications can also be provided.

| Company | View |
| --- | --- |

# Conclusions

## For Monday Online:

Proposal 2.1A: For SRS interference randomization, support:

Opt. 3: Both cyclic shift hopping and comb offset hopping. And FFS the following:

Separate or combined cyclic shift hopping and comb offset hopping for a UE

Separate or combined with SRS sequence group hopping / sequence hopping

Associated UE capability

Proposal 2.1.1A: For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs.

Option 1: Reuse the SRS sequence identity nIDSRS.

Option 2: Introduce new ID(s).

  - FFS: the value range, one new ID or two separate new IDs, default ID(s)

Proposal 2.1.2B: For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index l' as (l'+l0)/R.

Option 2: Within a slot, hopping based on only the symbol index l'.

Option 3: No intra-slot hopping.

Option 4: Time domain hopping behaviour further depends on system frame number (SFN) nf.

FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

At least support reinitialization at the beginning of each radio frame. FFS reinitialization periodicity of N radio frames or reinitialization based on system frame number.

Proposal 3.2.1A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

s = {2, 4}. FFS: s = 8.

m = 2,4,8,10,12,14, and m is a multiple of s.

Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Proposal 3.2.2A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2:

Select one of the following options regarding the m OFDM symbols:

  - Option 1-1: the m OFDM symbols are adjacent.

  - Option 1-2: the m OFDM symbols can be adjacent or non-adjacent.

Select one of the following options regarding the TDM pattern:

  - Option 2-1: the s subsets of ports are mapped cyclically {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

  - Option 2-2: the s subsets of ports are mapped sequentially {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

  - Option 2-3: the s subsets of ports are mapped cyclically or sequentially on the m OFDM symbols.

Proposal 3.1.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), select one of the options from below in RAN1#112:

Option 1: For comb 4, support 4 comb offsets.

Option 2: For comb 4, do not support 4 comb offsets.

Proposal 2.3: Support SRS TD OCC for TDD CJT SRS enhancement.

FFS: Dropping rules of collision with other uplink resource, other details.

OR

Conclusion: No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18.

Proposal 2.4: Support new cyclic shift mapping, and if yes, support:

- For a multi-port SRS resource, new cyclic shift mapping to enable non-uniform cyclic shift allocation within the range [0, n_CS^max - 1] across ports.

This does not change the comb offset mapping across ports.

E.g., per-port configuration.

E.g., dividing [0, n_CS^max - 1] in multiple non-overlapping sub-regions.

OR

Conclusion: No consensus to support new cyclic shift mapping to enable non-uniform cyclic shift allocation within the range [0, n_CS^max - 1] across ports in Rel-18.

Proposal 2.5: Support precoded SRS for DL CSI acquisition

- Reusing the standardized mechanism for NCB SRS as the baseline design for precoded SRS for DL CSI acquisition and striving to minimize the standard impact.

- Only for UEs supporting NCB SRS

FFS: If any additional UE antenna calibration is needed for precoded SRS for DL CSI acquisition

- For UEs with xTxR

FFS: For UEs with xTyR

- This feature is optional and subject to UE capability.

OR

Conclusion: No consensus to support precoded SRS for DL CSI acquisition in Rel-18.

Proposal 2.6: Support pseudo-random muting of SRS transmission:

- Muting / transmission is per SRS transmission occasion, where a bundle of L transmission occasions is considered together.

- An index corresponding to a binary sequence of length L from a set of binary sequences is determined based on the pseudo-random sequence c(i) as a function of time (e.g., slot number, symbol number) with a certain UE-specific initialization.

OR

Conclusion: No consensus to support pseudo-random muting of SRS transmission in Rel-18.

# References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#94-e.

R1-2300051, SRS enhancements for TDD CJT and 8TX operation, FUTUREWEI.

R1-2300097, SRS enhancement for TDD CJT and UL 8Tx operation in Rel-18, Huawei, HiSilicon.

R1-2300160, SRS Enhancements for CJT and 8TX UEs, InterDigital, Inc.

R1-2300185, SRS enhancement targeting TDD CJT and 8 TX operation, ZTE.

R1-2300207, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, Spreadtrum Communications.

R1-2300252, SRS enhancement targeting TDD CJT and 8 TX operation, OPPO.

R1-2300393, On SRS Enhancement, Google.

R1-2300440, Further discussion on SRS enhancements, vivo.

R1-2300514, Discussion of SRS enhancement, Lenovo.

R1-2300526, SRS enhancement targeting TDD CJT and 8 TX operation, LG Electronics.

R1-2300549, Discussion on SRS enhancements, xiaomi.

R1-2300654, Discussion on SRS enhancement in Rel-18, CATT.

R1-2300814, Discussion on SRS enhancement, NEC.

R1-2300849, SRS enhancement targeting TDD CJT and 8 TX operation, Sharp.

R1-2300936, Discussion on SRS enhancement in Rel-18, Intel Corporation.

R1-2300986, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, CMCC.

R1-2301039, Discussion on SRS enhancement targeting TDD CJT, ETRI.

R1-2301077, On SRS enhancements targeting TDD CJT and 8 TX operation, Ericsson.

R1-2301251, Views on SRS enhancements, Samsung.

R1-2301317, Views on SRS enhancement targeting TDD CJT and 8 TX operation, KDDI Corporation.

R1-2301333, Views on Rel-18 MIMO SRS enhancement, Apple.

R1-2301399, SRS enhancement for TDD CJT and 8 Tx operation, Qualcomm Incorporated.

R1-2301481, Discussion on SRS enhancement, NTT DOCOMO, INC.

R1-2301646, SRS enhancement for TDD CJT and 8Tx operation, Nokia, Nokia Shanghai Bell.

# Appendix

## Appendix 1: Agreements from RAN1#109-e

Agreement

For SRS EVM, adopt combined relevant parts from Rel-17 SRS EVM and Rel-18 FDD CJT EVM as starting point

- Details are provided in Appendix 3 of R1-2205330 for system-level simulations

- Details are provided in Appendix 4 of R1-2205330 for link-level simulations.

Agreement

For 8 Tx SRS, a starting point of UE antenna configurations can be:

- (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or

- (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ.

- FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items.

Agreement

For SRS EVM, consider additional EVM as follows

- Realistic channel estimation based on sequence generation for SRS modelling, at least for TDD CJT SRS LLS and 8 Tx SRS LLS as baseline

- Evaluation metrics for 8 Tx SRS LLS can be MSE , BLER or throughput

- TDL-C for TDD CJT SRS LLS can be included as optional.

Agreement

Consider the scenario where there exists SRSs sent by a UE and utilized by multiple TRPs for channel estimation, and the pathlosses between the UE and the TRPs differ by at least x dB in Rel-18 SRS study

- x can be {3,6,10}, and other values can be used.

Agreement

Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement

- Randomized frequency-domain resource mapping for SRS transmission

  - E.g., further enhancements to frequency hopping, comb hopping

- Randomized code-domain resource mapping for SRS transmission

  - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence

- Randomized transmission of SRS

    - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

- Per-TRP power control and/or power control of one SRS towards to multiple TRPs

- SRS TD OCC

- Increasing the maximum number of cyclic shifts

  - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts

- Precoded SRS for DL CSI acquisition

- Enhanced signaling for flexible SRS transmission

  - E.g., dynamic update of SRS parameters

- Partial frequency sounding extensions

  - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS

- Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

  - E.g., configuration of v (sequence index within a group) per SRS resource

  - E.g., configuration of cyclic shift per SRS port per SRS resource.

- Resource mapping for SRS transmission based on network-provided parameters or system parameters

  - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index)

Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

Agreement

Study the potential enhancements for SRS of 8T8R with usage antennaSwitching.

Agreement

Study the potential enhancements for SRS for 8 Tx operation

- SRS resource(s) with 8 ports are configured for codebook-based PUSCH

- Up to 8 single-port SRS resources are configured for non-codebook-based PUSCH

Agreement

For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS,

- Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol

- For the next decision point, study

  - Whether to support 8 ports in one or multiple resources

  - Whether to support 8 ports in one or multiple OFDM symbols

  - The maximum number of SRS resource sets.

- Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

| Rel-18 SLS Assumptions for TDD CJT SRS |  |  |  |
| --- | --- | --- | --- |
| Parameter |  |  | Value |
| Duplex, Waveform |  |  | TDD, OFDM |
| Multiple access |  |  | OFDMA |
| Scenario |  |  | Outdoor OptA Companies can simulate from the following 2 layouts. 1) Outdoor (typical 57-sector, or 21-sector, SLS): OptionA: 1 TRP per sector, 3 sectors per site. N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). The N_TRP TRPs can be selected either only from the same site (intra-site - limited to 3 TRPs), or also from other sites (inter-site) - company should describe what is assumed OptionB: N_TRP co-located (at BS) panels per sector - companies describe how the panels are (azimuthally) oriented - Dense Urban (macro only) 200m ISD or Urban Macro 500m ISD 2) Indoor Hotspot: model in TS 38.802 - N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). |
| Frequency Range |  |  | FR1 only, 3.5GHz |
| Inter-BS (site) distance |  |  | Outdoor: 200m or 500m Indoor Hotspot: per TS 38.802 |
| Channel generation model |  |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| Antenna setup and port layouts at gNB |  |  | - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32,64} |
| Antenna setup and port layouts at UE |  |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 |
| BS Tx power |  |  | Dense Urban or Urban Macro: - Per TRP: 44 dBm for 20MHz, 47dBm for 40MHz, 51dBm for 100MHz Indoor: per TRP 24dBm |
| BS antenna height |  |  | Depending on scenarios (cf. table A.2.1-1 of TS 38.802): DU (25m), UMa (25m), Indoor Hotspot (3m) |
| UE antenna height & gain |  |  | Follow TR36.873 |
| UE receiver noise figure |  |  | 9dB |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |  |
|  | SCS | 30kHz |  |
| Number of RBs |  | 52RB for 20MHz, 104RB for 40MHz, 272RB for 100MHz |  |
| Frame structure |  | DSUDD, or companies to state the used frame structure |  |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |  |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers |  |
| Overhead |  | Companies shall provide the downlink overhead assumption |  |
| Traffic model |  | FTP 1 or FTP 3 with 20%, 50% or 70% traffic load |  |
| UE distribution |  | According to TS 38.802 - DU and UMa: 80% indoor (3km/h), 20% outdoor (30km/h) - Indoor Hotspot: 100% indoor (3km/h) |  |
| UE receiver |  | MMSE-IRC as the baseline receiver |  |
| DL Channel estimation |  | Realistic |  |
| Evaluation Metric |  | DL throughput |  |
| Baseline for performance evaluation |  | R17 SRS design |  |
| SRS modeling for UL channel estimation |  | Companies to state the used SRS periodicity. Companies to state the SRS channel estimation modeling Number of ports = 2 or 4 Tx power = 23 dBm |  |

| Rel-18 LLS Assumptions for TDD CJT SRS |  |
| --- | --- |
| Parameter | Value |
| Scenario | N_TRP (#TRPs): 2, 3, 4 |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 40MHz, 100MHz |
| Channel model | CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| UE velocity | 3km/h |
| Antennas at UE | 1T4R, 2T4R, 4T4R |
| Antennas at gNB | 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Rank and MCS | Rank/MCS can be adaptive or fixed. |
| Evaluation metrics | MSE, BLER or throughput |
| Baseline | R17 SRS design |
| Precoding granularity | Fixed: 2, 4 or wideband for DL, wideband for UL. |
| SRS configurations | Companies to state the used SRS periodicity. Frequency hopping：Companies to state whether SRS frequency hopping is enabled and the hopping pattern if so. |
| DL SNR | Companies to state the used difference between DL SNR and UL SNR |

## Appendix 2: Agreements from RAN1#110

Agreement

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

RAN1 should strive to minimize the number of schemes supported in Rel-18

- SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

Target usage includes antenna switching, codebook/non-codebook based SRS

Agreement

For 8 Tx SRS, at least support

- 8 ports in 1 SRS resource for ‘antennaSwitching’;

  - FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

Agreement

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

Agreement

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

## Appendix 3: Agreements from RAN1#110bis-e

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Agreement

For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time-domain granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset / cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if CS hopping and/or comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

Other details are not excluded

Agreement

For SRS TD OCC for SRS enhancements for TDD CJT, study:

- Comparison against SRS on 1 OFDM symbol

- Comparison against SRS repeated on multiple OFDM symbols

- Study the following aspects: evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

Conclusion

The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT.

Conclusion

- No further discussion of increasing the maximum number of cyclic shifts for CJT SRS.

- No further discussion of partial frequency sounding extensions for CJT SRS.

Agreement

For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

Agreement

For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- n can be 8

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

## Appendix 4: Agreements from RAN1#111

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on:

Option 1: The hopping pattern is based on the pseudo-random sequence c(i), initialized with a network-configured ID.

FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID

FFS: The relation between the legacy group / sequence hopping and the new hopping

Agreement

For SRS interference randomization, support one from the following options  (to be decided in RAN1#112):

- Opt. 1: Cyclic shift hopping

- Opt. 2: Comb offset hopping

- Opt. 3: Both cyclic shift hopping and comb offset hopping

  - FFS: details including whether to support separate and/or combined hopping

  - FFS: details on UE capability and signaling

Conclusion

No consensus on enhanced signaling for flexible SRS transmission in Rel-18

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support:

- For comb 2, support 1 and 2 comb offsets

- For comb 4, support 2 and [4] comb offset

- For comb 8, support 4 comb offsets

Agreement

For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols

- Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM)

- FFS: m can be legacy values, i.e., 2,4,[8,10,12,14].