3GPP TSG RAN WG1 #112		                                                      R1-2301935

Athens, Greece, February 27th – March 3rd, 2023

Source:	Moderator (CATT)

Title:	Summary #3 of subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

This document summarizes the inputs and the discussions on subband non-overlapping full duplex in RAN1#112.

- Proposals for online sessions

  - Feb. 28th (Tue)

Proposal 1-5b

Proposed Agreement:

For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

  - Option 1: DL receptions outside semi-statically configured DL subband(s) are not allowed

  - Option 2-1: the symbol can dynamically fallback via group-common signaling to a DL symbol in which UE assumes there are no UL/DL subbands.

  - Option 2-2: dynamic DL receptions outside semi-statically configured DL subband(s) are allowed by scheduling DCI

For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s)   and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

  - Option 1: DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

  - Option 2-1: the symbol can dynamically fallback via group-common signaling to a flexible symbol in which UE assumes there are no UL/DL subbands

    - Note: the flexible symbol refers to one as configured by TDD-UL-DL-ConfigCommon.

  - Option 2-2: the symbol can dynamically change via group-common signaling to a DL symbol in which UE assumes there are no UL/DL subbands

  - Option 2-3: dynamic DL receptions outside semi-statically configured DL subband(s) and dynamic UL transmissions outside semi-statically configured UL subband(s) are allowed by scheduling DCI

Performance of dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD

Proposal 1-4

Proposed Conclusion:

Dynamic indication of frequency location/size of SBFD subbands is not considered.

Proposal 1-1

Proposed Agreement:

Study whether a slot can consist of both SBFD and non-SBFD symbols.

Proposal 1-6a

Proposed Agreement:

An UL subband can be configured in SSB symbols.

  - FFS UE behaviors for SBFD-aware UEs in the SSB symbols.

Proposal 1-11

Proposed Agreement:

Study potential enhancements for SBFD operation in RRC_IDLE and RRC_INACTIVE states.

Proposal 3-1

Proposed Conclusion:

For inter-UE inter-subband CLI measurement, the following two methods are identified:

- Method#1: victim UE measures RSSI/SINR within DL subband;

- Method#2: victim UE measures RSRP of aggressor UE’s CLI-SRS within UL subband.

Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband.

  - Mar. 1st (Wed)

Proposal 1-4

Proposed Conclusion:

Dynamic indication of frequency location/size of SBFD subbands is not considered.

Proposal 1-1a

Proposed Agreement:

Study whether or not a slot can consist of both SBFD and non-SBFD symbols.

Proposal 3-1

Proposed Conclusion:

For inter-UE inter-subband CLI measurement, the following two methods are identified:

- Method#1: victim UE measures RSSI/SINR within DL subband;

- Method#2: victim UE measures RSRP of aggressor UE’s CLI-SRS within UL subband.

Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband.

Proposal 1-12a

Proposed Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, study the following options for SBFD-aware UEs:

  - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

  - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

  - PDSCH/PUSCH/PUCCH repetitions

  - SPS PDSCH/CG PUSCH

  - TBoMS

  - Multi-PUSCH/PDSCH scheduled by a single DCI

  - Periodic/semi-persistent SRS/CSI-RS/PUCCH

  - PDCCH

Proposal 1-6a

Proposed Agreement:

An UL subband can be configured in SSB symbols.

  - FFS UE behaviors for SBFD-aware UEs in the SSB symbols.

Proposal 1-11

Proposed Agreement:

Study potential enhancements for SBFD operation in RRC_IDLE and RRC_INACTIVE states.

  - Mar. 2nd (Thu)

Proposal 1-14a

Proposed Agreement:

For SBFD-aware UEs, study the following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands:

  - Option 1: Partial RBG(s) due to overlapping with RBs outside DL/UL subband at subband boundaries can be used for PDSCH/PUSCH

  - Option 2: Partial RBG(s) due to overlapping with RBs outside DL/UL subband at subband boundaries cannot be used for PDSCH/PUSCH

Proposal 1-16a

Proposed Agreement:

For SBFD-aware UEs, study at least the following issues:

  - [Partial PRG(s) due to overlapping with RBs outside DL subband at subband boundaries with size of 2 and 4]

  - Wideband precoder in case of non-contiguous DL subband(s)

Proposal 1-18a

Proposed Agreement:

Study the non-contiguous resource allocation for CSI-RS for SBFD-aware UEs considering the following options:

- Option 1: Two contiguous CSI-RS resources link to one CSI report

- Option 2: Non-contiguous CSI-RS resource

  - Option 2-1: non-contiguous CSI-RS resource allocation

  - Option 2-2: contiguous CSI-RS resource allocation and non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

- Option 3: new CRB-to-PRB mapping

Proposal 1-15a

Proposed Agreement:

For SBFD-aware UEs, study the following options for CSI report configuration associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

  - Option 1: separate CSI reporting configurations for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting configurations for SBFD symbols and non-SBFD symbols

Proposal 1-9b

Proposed Agreement:

Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols:

  - Whether/how to configure/allocate separate resources

  - Whether/how to configure separate FH parameters

  - Whether/how to configure separate UL power control parameters

  - Whether/how to configure separate beam configurations

Proposal 1-20

Proposed Agreement:

Study how SBFD-aware UE determines whether to transmit or receive in an SBFD symbol, considering the following options.

  - Option 1: UE is semi-statically configured with a transmission/reception direction

  - Option 2: UE is dynamically indicated with a transmission/reception direction

  - Option 3: UE determines to transmit or receive based on configuration/scheduling of transmissions and receptions

- General aspects of SBFD schemes

This section discusses the general aspects of SBFD schemes except self-interference, inter-gNB and inter-UE CLI handling schemes.

  - Summary of input contributions

The inputs from companies’ contributions are summarized below as per moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or are missed. Companies are encouraged to correct/update the summary with revision marks if needed.

    - Subband location indication

The following agreement was made in RAN1#110bis-e that both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs for baseline SBFD operation at least for RRC_CONNECTED state.

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Spreadtrum proposed to study Alt 2 in addition to Alt 4. Qualcomm proposed that Alt 2 and Alt 3 are not considered in Rel-18.

      - Semi-static configuration of subband time and frequency location

The following agreements were made in RAN1#110 to study semi-static configuration of subband time and frequency location as baseline.

Agreement For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

        - Subband time location indication

The following agreements were made in RAN1#110bis-e for semi-static configuration of subband time locations for SBFD operation.

Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

Periodicity of SBFD subband time location

Two options are identified by companies for the periodicity of SBFD subband time location configuration as summarized in [14] and companies’ views are summarized below:

- Option 1: The period is determined by periodicity of the configured TDD pattern

  - Supported by: Spreadtrum, OPPO, ZTE, vivo, CMCC, Ericsson, Qualcomm

- Option 2: The period is determined by a new configured periodicity.

  - Supported by: ZTE, vivo

Granularity of SBFD subband time location

On granularity of SBFD subband time locations, both slot-level and symbol level SBFD subband configurations can be considered.

- Slot-level SBFD subband time location

  - Supported by: Spreadtrum, CATT

    - Considering the DL-UL switching and UL-DL switching [10]

    - If separate PUCCH configuration for SBFD symbols and non-SBFD symbols is supported, it is not clear which configuration should be applied in a slot with both SBFD symbols and non-SBFD symbols [16]

    - Potential transmission/reception across SBFD and non-SBFD symbols leading to complexity, e.g. power control, if separate parameters are applied to SBFD symbols and non-SBFD symbols, it is not clear which set of parameter to apply [16]

    - More gap may be needed considering the DL to UL switching time [16]

  - Not supported by: Intel

    - Limited resource for SBFD operation, e.g. self-contained frame structure which includes both DL symbol and UL symbol (also flexible symbol) in a slot is unavailable for SBFD operation, because UL symbol is always a non-SBFD symbol, or if the symbol for SSB cannot be configured as SBFD symbol, then the whole slot is unavailable for SBFD operation [22]

- Symbol-level SBFD subband time location

  - Supported by: Intel, Samsung

    - Better flexibility for SBFD operation [22][27]

    - forward-compatibility [27]

  - Lower priority: Spreadtrum

Qualcomm observed that from gNB perspective, switching between SBFD and non-SBFD symbols may require a transition guard period to switch the panels, tune filter and adjust timing which disrupt the transmission or reception.

        - Subband frequency location indication

The following agreements were made for semi-static configuration of subband frequency locations for SBFD operation.

Agreement (RAN1#110) For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline. Agreement (RAN1#110bis-e) For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. Agreement (RAN1#111) For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

Flexible subband

Huawei proposed to introduce flexible subband in both symbols configured as DL and flexible in TDD-UL-DL-ConfigCommon which can be used for UL or DL transmissions from gNB perspective.

ZTE identified three potential alternatives of flexible subband as below and thinks that the flexible subband/resource is beneficial but UL subband can serve as the “flexible subband/resource if DL transmission is allowed to be transmitted in the UL subband.

- Alt.1: subband that can be used to transmit UL and DL is considered as flexible subband.

- Alt.2: subband that configured in the flexible symbols is considered as flexible subband.

- Alt.3: part of the resources within the UL subband is configured as flexible resource, within which DL transmission and UL transmission are allowed.

Vivo proposed that transmission direction of subband(s) other than UL subband is flexible in semi-static flexible symbols, i.e. flexible subband(s).

Intel proposed to not introduce flexible subband. Instead, semi-statically configured DL & UL subband with dynamic indication which explicitly/implicitly switches the semi-static DL/UL subband to a temporal UL/DL subband for full UL/DL symbol can be sufficient.

Samsung thinks that DL transmission in UL subband does not require the introduction of a separate ‘flexible’ SBFD subband type.

Explicit/implicit indication of subband(s)

There are basically two options for DL/UL subband(s) and guardband(s) configuration as illustrated below.

Figure 3-1: Options for RB set and guard band configuration in SBFD symbols [20]

- Option 1: Frequency locations of DL subband(s) and UL subband are explicitly configured. Guardband(s) if any are implicitly derived.

  - Supported by: New H3C, TCL, Spreadtrum, ZTE, Intel, Samsung, LG, MediaTek, DOCOMO, Nokia, WILUS

- Option 2: Frequency locations of UL subband and number of RBs for guardband(s) if any are explicitly configured. Frequency locations of DL subband(s) are implicitly derived.

  - Supported by: OPPO, IDC, ZTE, Ericsson, Intel, Qualcomm, DOCOMO, Nokia, CEWiT, Reliance Jio, IIT Kanpur

Huawei proposed to indicate frequency locations of DL subband, flexible subband and guard band as follows considering flexible subband.

- DL subband should be explicitly indicated, at least for the length of DL subband;

- Flexible subband can be implicitly indicated as a set of consecutive of RBs which are not included in UL subband and DL subband;

- Guard band can be implicitly indicated based on scheduling, i.e., gNB will not schedule UL and DL transmission on guard band.

        - Signalling design of subband location indication

Companies discussed whether semi-static time and frequency locations of SBFD subbands are signalled via cell-common signalling or UE-specific signalling and companies’ views are summarized below.

- Cell-common signalling

  - Supported by: Spreadtrum, ZTE, xiaomi, CATT, Fujitsu, Ericsson (frequency location), MediaTek (frequency location), Samsung (if SBFD in RRC_IDLE/INACTIVE is supported), Qualcomm, Nokia

- UE-specific signalling

  - Supported by: ZTE, vivo, Ericsson (frequency location), OPPO (time location), ETRI, Samsung, Nokia (can be considered as additional option)

    - different configurations may be expected by SBFD aware UEs and non-SBFD aware UEs [14]

    - more flexibility and extendibility [14]

    - for potential different guardband sizes for different UEs [20]

      - Dynamic subband location indication

It was agreed to study semi-static configuration of subband time and frequency location as baseline.

Companies discussed dynamic indication of time and frequency location of UL subband for efficient resource utilization and DL/UL traffic adaptation and the views are summarized below.

- Semi-static subband time and frequency locations only

  - Supported by: Ericsson, CATT

- Dynamic subband time location only

  - Supported by: Huawei, ZTE, vivo, Intel, Lenovo, CMCC, LG, Qualcomm, DOCOMO, Nokia

- Dynamic subband time and frequency locations

  - Supported by: New H3C, TCL, Sony?

- Dynamic subband frequency location only

  - Supported by:

    - SBFD operation in DL symbols configured in TDD-UL-DL-ConfigCommon

The following agreement was made in RAN1#111 meeting for SBFD operation in DL symbols configured in TDD-UL-DL-ConfigCommon.

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

One remaining issue is whether to allow DL receptions outside DL subband(s) in DL symbols configured in TDD-UL-DL-ConfigCommon. Companies’ views are summarized below.

DL receptions outside DL subband(s) in a symbol configured as DL in TDD-UL-DL-ConfigCommon:

- Allowed: ZTE (SBFD symbol is converted to DL symbol), vivo, xiaomi, Fujitsu, NEC, Intel, CMCC, LG, Samsung, QC (dynamic indication/update of time location of SBFD subband)?, DOCOMO, Nokia, ASUSTeK

  - inefficient for resource usage and DL/UL traffic adaptation if UL subband semi-statically configured for SBFD symbols means resources in the UL subband are always reserved for UL

- Not allowed: Huawei, CATT, Ericsson

  - deviates from the definition of UL subband and DL subband [7] [20]

  - complicate the specification as well as the SBFD operation in practice [7]

    - SBFD operation in flexible symbols configured in TDD-UL-DL-ConfigCommon

The following agreement was made in RAN1#111 meeting for SBFD operation in flexible symbols configured in TDD-UL-DL-ConfigCommon.

Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

- Option 1:

  - Supported by: New H3C, Spreadtrum, xiaomi, NEC, CATT, CMCC, Ericsson, Panasonic, Samsung, Lenovo, NEC, Apple, Qualcomm?, DOCOMO

    - harmonized operation with ‘D’ symbols [8][20][30]

    - Easy to avoid intra-subband CLI [8]

  - Not supported by:

- Option 2:

  - Supported by: Huawei, OPPO, vivo, Fujitsu, Intel, ETRI, Nokia, ASUSTeK, WILUS

    - Better flexibility [7][14]

  - Not supported by: Spreadtrum, Ericsson,

    - Dynamic scheduling based indication would cause sever CLI since the non-scheduled UEs do not know the conversion [10]

    - Complicated design [10][20]

    - it does not bring tangible benefits compared with dynamic TDD [20]

UL transmissions outside UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon:

- Allowed: Huawei (if DL subband is not configured), OPPO (if the SBFD symbol is indicated as UL), vivo, Fujitsu, ETRI, Nokia, ASUSTeK

- Not allowed: Huawei (if DL subband is configured), NEC, Samsung, Sharp

DL receptions outside DL subband(s) in a symbol configured as flexible in TDD-UL-DL-ConfigCommon:

- Allowed: OPPO (if the SBFD symbol is indicated as DL), ZTE (SBFD symbol is converted to flexible symbol), vivo, xiaomi, NEC, Fujitsu, Samsung, LG, ETRI, Nokia, ASUSTeK

- Not allowed: Huawei, Ericsson

    - SBFD operation in SSB symbols

It was agreed in RAN1#110bis-e to study whether SBFD operation in SSB symbols is supported or not.

Agreement Study whether SBFD operation in SSB symbols is supported or not.

Companies’ views are summarized below:

SBFD operation in SSB symbols is

  - Supported: Huawei, IDC, Samsung, QC

    - Degradation of SBFD performance due to limited symbols for SBFD operation if SBFD operation on SSB symbols are not supported especially when the number of SSB symbols is relatively large [7][12][22][27]

    - Except the UEs which are under initial access or cell reselection, a short SSB measurement period is not necessary and these UEs can perform the SBFD operation on SSB symbols to improve the performance [7]

    - Frequent switch between DL and SBFD symbols if SBFD in SSB is not supported [22]

  - Not supported: Spreadtrum, OPPO, vivo, CATT,NEC, CMCC, NEC, Ericssion, Nokia, DOCOMO (and CORESET#0), MediaTek, CEWiT, Reliance Jio, IIT Kanpur

    - Avoid impact on SSB due to intra-cell/inter-cell UE-to-UE CLI [11][14][16][20][22][28][32][33][36]

    - UL transmissions may not be supported by UE given that UE cannot transmit UL and measure SSB simultaneously and gNB does not know when UE is receiving SSB [10][16]

    - Self-interference from SSB is not easy to be cancelled considering that SSB signaling is a broadcast signaling with high power [11]

    - Transmission/Reception enhancements

The following agreements were made in RAN1#111.

Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured. Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

A transmission/reception across SBFD symbols and non-SBFD symbols in the same slot

There was an FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD symbols in the same slot if configured in the agreement made in RAN1#111.

It is related to the granularity of SBFD subband time location discussed in section 3.1.1.1.1. If only slot-level SBFD subband location is supported, i.e. a slot would consist of either SBFD symbols or non-SBFD symbols and a physical channel would not be mapped to SBFD and non-SBFD symbols in the same slot. However if symbol-level SBFD subband location is supported, i.e. a slot may consist of both SBFD symbols and non-SBFD symbols, it is possible that a physical channel would be mapped to SBFD and non-SBFD symbols in the same slot.

Spreadtrum observed the following issues to map a physical channel to SBFD and non-SBFD symbols in the same slot:

- First, timing advance for CLI handling in SBFD operation and flexible duplex is an important issue, which were discussed by a lot of companies in their contribution, and proposed to use different TA in SBFD and non-SBFD region. This solution would lead to two TA for one PUSCH in slot 3. It is too early to make decision before we have a clear picture for TA enhancement.

- Second, DL-UL switching is another issue would impact the allowance, e.g. the orange box in the slot 1 shows a possible DL-UL switching pattern for DL symbol change to UL subband. Frankly speaking, the DL-UL switching pattern cause trouble for PDSCH covering SBFD and non-SBFD in a slot.

- Third, some enhancements were agreed for SBFD regions, such as different frequency resource, time domain resource, power control, and spatial domain. It is hard to decide which frequency/time/power/spatial domain parameters or indications should be used for these specific channel.

Intel observed that if one PUCCH/PUSCH/PDSCH/PDCCH map to both SBFD and non-SBFD symbols, it would lead to different performance in different symbols within one PUCCH/PUSCH/PDSCH/PDCCH which degrades the performance and complicates link adaptation and the overall transmission/reception procedure. Qualcomm observed that from UE perspective, SBFD-aware UE may need to do filter retuning and UL sampling rate adjustment from SBFD to UL-SB. For the other example of switching from DL to SBFD, SBFD-ware UE may adjust DL filtering between DL symbol and SBFD symbols which interrupts DL reception.

Therefore, Spreadtrum, Intel and Qualcomm proposed to avoid mapping a physical channel to SBFD and non-SBFD symbols. To be more specific, UE doesn’t expect to be dynamically scheduled with a physical channel that is mapped to SBFD and non-SBFD symbols within a slot and UE drops or discard physical channel transmission or reception configured by higher layer that is mapped to SBFD and non-SBFD symbols.

ZTE prefer to allow PUSCH/PUCCH transmission across SBFD and non-SBFD symbols in a same slot for higher efficiency and proposed to study the impact and potential enhancements.

Vivo proposed to allow dynamically scheduled UL transmissions and DL receptions to be mapped to SBFD symbol(s) and non-SBFD symbol(s) to achieve more flexibility, where consistent frequency domain resources are occupied across all symbols. For semi-statically configured UL transmissions and DL receptions, vivo proposed to not allow them to be mapped to SBFD symbol(s) and non-SBFD symbol(s) in the same slot unless consistent frequency domain resources across all symbols that a UL transmission/DL reception is mapped to can be ensured

Samsung proposed that PDSCH, PUSCH or PUCCH allocations for a UE across non-SBFD and SBFD symbols in a slot are supported and configurable by RRC under condition that the same QCL and EPRE assumptions are valid for the non-SBFD and SBFD symbols.

Potential enhancements in time/frequency/spatial/power domain

Based on companies’ inputs and as per moderator’s understanding, potential enhancements on frequency domain resource allocation at least come from the following aspects:

- Transmissions/receptions across SBFD symbols and non-SBFD symbols based on a single FDRA with different available resources, e.g. periodic transmissions, repetitions, multi-PDSCH/PUSCH scheduling etc.

- Non-contiguous resource allocation in DL and unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands

Many companies also considered different interference levels, gNB antenna configurations etc. and proposed to support separate configurations for SBFD symbols and non-SBFD symbols in terms of spatial/power/time-frequency domains.

Various proposals for different channels/signals are summarized below as per moderator’s understanding.

- PDCCH

  - CORESET adaptation

    - Supported by: Lenovo, QC, Nokia, MediaTek, Indian Institute of Tech (M), CEWiT

  - Separate CORESET configurations for a SS

    - Supported by: NEC

  - PDCCH dropping

    - Supported by: ETRI, Sony, Panasonic, Intel, QC

  - No enhancements:

    - Supported by: Spreadtrum, Sharp (except CORESET0), xiaomi

- PDSCH

  - Separate FDRA for SBFD symbols and non-SBFD symbols

    - Supported by: Nokia, vivo, NEC, Intel, QC, CEWiT, Reliance Jio, IIT Kanpur

      - SPS PDSCH: Nokia (separate configuration), NEC, QC, CEWiT, Reliance Jio, IIT Kanpur

      - Multi-PDSCH scheduling: QC

  - PDSCH dropping in SBFD symbols due to overlapping with RBs outside DL subband(s)

    - Supported by: CATT

      - CG PUSCH: CATT

      - PUSCH repetition: CATT

  - RBs for PDSCH are derived by excluding RBs outside DL subband(s)

    - Supported by: Huawei, Spreadtrum, xiaomi, CATT, Intel, ETRI, Lenovo, Sharp, QC, Nokia, WILUS

      - RBG: Huawei(&RA type 1), Spreadtrum, xiaomi, CATT, Lenovo, Sharp, QC, Nokia (&RA type 1), WILUS

      - PDSCH repetition: WILUS, QC

  - RBs for PDSCH in SBFD symbols and non-SBFD symbols are separated derived

    - Supported by: Intel, CMCC

      - PDSCH repetition: CMCC

      - Multi-PDSCH scheduling: CMCC

  - New VRB-to-PRB mapping

    - Supported by: Lenovo, Nokia

  - New RBG size

    - Supported by: Samsung

  - Restrict the PDSCH transmission in one symbol type

    - Supported by: OPPO, CMCC

      - PDSCH repetition: OPPO, CMCC

  - non-continuous PRB-to-CRB mapping

    - Supported by: CMCC

- PUSCH

  - Separate FDRA for SBFD symbols and non-SBFD symbols

    - Supported by: Huawei, NEC, IDC, ZTE, vivo, CATT, Lenovo, Intel, QC, Nokia, MediaTek, CEWiT, Reliance Jio, IIT Kanpur

      - CG PUSCH: NEC, Lenovo(separate configuration), QC, Nokia (separate configuration), MediaTek, CEWiT, Reliance Jio, IIT Kanpur

      - PUSCH repetition: MediaTek

      - PUSCH FH: Huawei, ZTE, CATT

      - TBoMS: Nokia

      - Multi-PUSCH scheduling: QC

  - PUSCH dropping in SBFD symbols due to overlapping with RBs outside UL subband

    - Supported by: CATT, Intel

      - CG PUSCH: CATT

      - PUSCH repetition: CATT

  - RBs for PUSCH are derived by excluding RBs outside UL subband

    - Supported by: ETRI

  - Restrict the PUSCH transmission in one symbol type

    - Supported by: Huawei, OPPO, xiaomi, Samsung, CMCC, Sharp, MediaTek

      - PUSCH repetition: Huawei, OPPO, xiaomi, Samsung, CMCC, Sharp, MediaTek

      - PUSCH FH: xiaomi, MediaTek

      - TBoMS: xiaomi, CMCC

      - Multi-PUSCH scheduling: xiaomi

  - RBs for PUSCH in SBFD symbols and non-SBFD symbols are separated derived

    - Supported by: Lenovo, Sharp, Intel, CMCC, Ericsson, WILUS

      - PUSCH FH: Ericsson, WILUS

      - PUSCH repetition: Lenovo, CMCC, Sharp

      - TBoMS: Lenovo, CMCC

      - Multi-PUSCH scheduling: CMCC

- PUCCH

  - Separate configurations for SBFD symbols and non-SBFD symbols

    - Supported by: CATT, Lenovo, Intel, CMCC, QC, ETRI, WILUS

  - Separate FDRA for SBFD symbols and non-SBFD symbols

    - Supported by: Huawei, IDC, ZTE, vivo, Lenovo, Sharp, MediaTek

      - periodic/semi-persistent PUCCH: MediaTek

      - PUCCH repetition: Lenovo, Sharp, MediaTek

      - PUCCH FH: Huawei, ZTE, MediaTek

  - Restrict the PUCCH transmission in one symbol type

    - Supported by: Huawei, OPPO, CMCC, xiaomi, Samsung, Sharp, MediaTek

      - PUCCH FH: xiaomi, MediaTek

      - PUCCH repetition: Huawei, OPPO, CMCC, xiaomi, Samsung, Sharp

  - RBs for PUCCH in SBFD symbols and non-SBFD symbols are separated derived

    - Supported by: Ericsson, CMCC, QC

      - PUCCH FH: Ericsson

      - PUCCH repetition: CMCC, QC

- CSI-RS

  - Separate FDRA for SBFD symbols and non-SBFD symbols

    - Supported by: Apple, vivo, NEC

  - CSI-RS resources are derived by excluding RBs outside DL subband(s)

    - Supported by: Huawei, Spreadtrum, xiaomi, CATT, Intel, Sharp, Ericsson, QC, Nokia, CEWiT, Reliance Jio, IIT Kanpur, WILUS

  - Non-contiguous CSI-RS resource configuration

    - Supported by: Spreadtrum, Sharp, xiaomi, QC

  - non-continuous PRB-to-CRB mapping

    - Supported by: CMCC

- SRS

  - Separate configurations for SBFD symbols and non-SBFD symbols:

    - Supported by: Lenovo, CATT, CMCC, NEC, Intel, QC

  - Separate FDRA for SBFD symbols and non-SBFD symbols

    - Supported by: vivo, MediaTek

      - Periodic and semi-persistent SRS: MediaTek

  - SRS skipping in SBFD symbols

    - Supported by: MediaTek

      - Periodic and semi-persistent SRS: MediaTek

- Separate time and frequency resources

- Supported by: vivo, CATT, Intel

- Separate FH parameters

- Supported by: Huawei, ZTE, xiaomi, CATT, Intel, CMCC, Samsung, WILUS

- Separate UL power control parameters

- Supported by: vivo, CATT, Intel

- Separate beam configurations

- Supported by: Qualcomm

    - UE collision handling

The following agreement was made in RAN1#110bis-e.

Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step)

There are two types of collision between UL and DL for a SBFD aware UE as below.

- Type A: Collision between UL transmissions and DL receptions in the same SBFD symbol

- Type B: Collision between transmissions/receptions with transmission direction of subbands

For Type A collision, the following cases are discussed by companies.

- Collision between dynamic UL transmissions and dynamic DL receptions

  - ZTE, xiaomi,  ITRI (with different priorities), DOCOMO, Indian Institute of Tech (M), CEWiT

    - Option 1: For same priority, UL transmission in UL subband is prioritized over DL reception in DL subband, and DL reception in UL subband is prioritized over UL transmission in UL subband; for different priorities, DL/UL transmission with higher priority is prioritized [ZTE]

  - Error case: Spreadtrum, vivo, CATT, Intel, CMCC

- Collision between dynamic DL receptions and semi-statically configured UL transmissions

  - Spreadtrum, vivo, ZTE, xiaomi, CATT, OPPO, Intel, CMCC, MTK, Sony, Apple, DOCOMO, Nokia, WILUS, , Indian Institute of Tech (M), CEWiT

    - Option 1: Additional signalling is provided to indicate the transmission/reception direction [Spreadtrum]

    - Option 2: dynamic transmission/reception is prioritized over semi-static transmission/reception [OPPO, ZTE, Sony, Apple, CMCC, MTK]

- Collision between dynamic UL transmissions and semi-statically configured DL receptions

  - Spreadtrum, vivo, ZTE, xiaomi, CATT, OPPO, Intel, CMCC, MTK, Sony, Apple, QC, DOCOMO, Nokia, WILUS, Indian Institute of Tech (M), CEWiT

    - Option 1: Additional signalling is provided to indicate the transmission/reception direction [Spreadtrum]

    - Option 2: dynamic transmission/reception is prioritized over semi-static transmission/reception [OPPO, ZTE, Sony, Apple, CMCC, MTK]

- Collision between semi-statically configured UL transmissions and semi-statically configured DL receptions

  - Spreadtrum, vivo, ZTE, xiaomi, CMCC, CATT, Intel, Lenovo, DOCOMO, Nokia, Apple, QC, WILUS, Indian Institute of Tech (M), CEWiT

    - Option 1: Additional signalling is provided to indicate the transmission/reception direction [Spreadtrum, CMCC (UE-specific RRC signalling or scheduling DCI or SFI)]

    - Option 2: For same priority, UL transmission in UL subband is prioritized over DL reception in DL subband, and DL reception in UL subband is prioritized over UL transmission in UL subband; for different priorities, DL/UL transmission with higher priority is prioritized [ZTE]

    - UL is prioritized [Apple]

In addition, several companies discussed collision between DL reception and PRACH transmissions. It is not clear yet whether PRACH resources can be configured in UL subband and it is related to the discussions in section 3.1.7. So this case is not considered for now in this section. Similarly, for the collision between SSB and UL transmissions, it is related to the discussion in section 3.1.4 and is not considered for now in this section.

For Type B collision, the transmission direction of subband(s) depends on the discussions in section 3.1.2, 3.1.3 and 3.1.4.

    - SBFD operation for initial access

It was agreed to study SBFD operation at least for RRC_CONNECTED state.

Huawei, New H3C, ZTE, CATT, Intel, Samsung, LG, Qualcomm, DOCOMO, Nokia, MediaTek, Indian Institute of Tech (M), CEWiT observed that SBFD operation for initial access may offer the following potential benefits:

  - Increased RACH capacity [7][8][13][16][22][30][32][33]

  - Reduced initial access latency [7][8][13][16][22][27][30][31][32][33][38]

  - Improved UL coverage for Msg 3 and PRACH due to more UL resources [7][30][32] [38]

  - Avoid UL resource fragmentation [27]

  - Reduced CLI if PRACH resource is not used by UE [16][27]

Therefore, the above companies proposed to study potential enhancement of initial access enhancement for SBFD operation.

Ericsson proposed that UEs in IDLE mode are not aware of whether or not symbols/slots are used for SBFD operation.

    - SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

Ericsson does not support misaligned center frequencies between DL and UL BWPs as it doesn't seem to bring any benefits and would likely negatively impact UE complexity.

Qualcomm see useful scenario to support a single configured DL and UL BWP pair with unaligned center frequencies where network configures the UE with narrowband initial UL/DL BWP and then wideband first active BWP after RRC connection. If SBFD is indicated to only RRC connected UE, then during initial access, all UEs will be configured with center aligned narrowband UL/DL BWP as shown on left figure in the example below. This will restrict leveraging the UL-SB resources and hence don’t achieve UL coverage gain and latency reduction. On the other hand, if this restriction is lifted, two narrowband UL/DL BWP could be configured for the new UEs as shown in the right figure in the example below. This can be applicable for default BWP as well where UE could be scheduled only in one of the two DL subbands.

Figure 3-2: Narrowband UL/DL BWP with aligned and non-aligned center frequency [30]

    - SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies

InterDigital, ZTE, CEWiT, Reliance Jio, IIT and Kanpur proposed to study/support BWP-based solution.

Qualcomm raised concerns on SBFD operation using more than one configured DL and UL BWP pair with aligned center frequency where each subband configured as BWP. Instead, it is proposed that there is one UL/DL BWP pair configured for TDD operation and one UL/DL BWP pair configured for SBFD operation and UE would need to switch from one BWP to the other based on some semi-static configuration of the BWP switching. UE knowledge of the semi-static BWP switch may result into optimized BWP switching delay. It is noted that the UL/DL BWP pair configured for SBFD operation is with unaligned center frequencies and DL BWP can be defined as non-contiguous RBs to cover both DL subbands.

Figure 3-3: Multiple configured DL and UL BWP pairs with unaligned center frequencies [30]

KDDI thinks that the BWP pair for SBFD slot can mitigate inter-UE CLI impact by configuring the DL BWP size to match one of the DL subbands, as shown below, and therefore proposed to study the feasibility and potential benefit of SBFD scheme with more than one configured DL and UL pair with aligned/unaligned center frequencies for a DL and UL BWP pair.

- Single DL and UL BWP pair is activated at a time

- DL and UL BWP pairs are semi-statically switched without dynamic indication by network

Figure 3-4: BWP configuration on operation of multiple DL and UL BWP pairs [34]

However, Nokia has the following observations:

- There is substantially no difference between the RB-set based and the advanced BWP based approach in terms of digital filter bandwidth adaptation at the UE.

- Unless new in-band emission requirements are specified, better UL transmission and DL reception filtering should not be considered as an advantage of any of the proposed schemes.

- Operation according to the basic BWP based scheme is not precluded by RB-set based signaling.

Ericsson thinks the SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is sufficient for SBFD operation.

Intel suggested to not further consider BWP-based SBFD in Rel-18 SI phase considering BWP-based SBFD may not work for a UE incapable of multiple BWPs and limited time for any enhancement for BWP to enable efficient SBFD operation based on BWP, e.g.

  - BWP switching delay enhancement

  - DL control overhead and latency reduction due to BWP switching (SPS PDSCH/type-2 CG PUSCH of previous BWP is released and SPS PDSCH/type-2 CG PUSCH in the new active BWP is usable only after new activation DCI is received)

  - DL performance improvement due to HARQ-ACK dropping (BWP switching leads to HARQ-ACK dropping for PDSCHs in previous BWP)

  - Repetition across different BWPs for coverage improvement

Xiaomi proposed that BWP-based SBFD operation is not supported in Rel-18 duplex enhancement.

    - SBFD operation across carriers

It was agreed in RAN1#109-e to at least study SBFD operation within a TDD carrier.

Agreement At least study SBFD operation within a TDD carrier

SBFD operation across carriers achieved by intra-band TDD CA with different TDD configurations is considered by Huawei as illustrated below. Huawei thinks that the support of SBFD operation across carriers does not seem to require any specific standardization effort compared to the support of SBFD operation within a carrier. What is required is to apply the similar collision handling rule from single carrier to carrier aggregation and therefore proposed to potential enhancements to collision handling for intra-band TDD CA with different TDD configurations to enable SBFD operation across carriers.

Figure 3-5: SBFD operation across carriers [7]

Samsung proposed to study potential enhancements to CA-based SBFD operation in FR2-1 at a later stage of the Rel-18 SI. One area for such potential enhancements in FR2-1 is the Rel-16 directional collision handling.

Qualcomm observed that SBFD operation across multiple CCs requires UE supports of CA as prerequisite while CA framework has some inherent UE complexity and compared to single-CC SBFD, CA-based SBFD has some limitation where DL and UL BW is restricted to the component carrier bandwidth while the inter-channel guardband can’t be utilized. While it is considered that CA-based SBFD operation is interesting for e.g. FR 2-1, it is proposed that SBFD operation across multiple components is studied at later stage in Rel-18 after establishing the baseline study of SBFD operation within component carrier.

Intel proposed to not consider CA-based SBFD in Rel-18 SI due to the following reasons:

  - the rule to determine a reference cell and the prioritization between reference cell and other cells when collision happens may not be sufficient for SBFD operation

  - CA based SBFD operation suffers retransmission restriction in the same carrier

  - repetition transmission across carriers is not supported

  - CA is an optional UE feature

Xiaomi proposed that half duplex CA based SBFD operation is not supported in Rel-18 duplex enhancement.

    - SBFD operation in legacy UL sybmol

The following conclusion was made in RAN#96.

Conclusion: UL symbol as second priority is accepted, no intended suspension of continuation of work in WGs

Ericsson proposed to not support SBFD operation in symbols configured as 'U' by TDD-UL-DL-ConfigCommon because it is not motivated to configure symbols configured as 'U' by TDD-UL-DL-ConfigCommon for SBFD operation and it can create strong gNB-gNB cross-link interference to a legacy (static TDD) gNB.

Qualcomm observed that SBFD operation at legacy UL slot is beneficial in multiple deployment scenarios, e.g., greenfield deployment and UL heavy deployment (InH/InF) to reduce DL blockage and improve DL coverage.

Xiaomi proposed that for subband non-overlapping full duplex, it cannot be applied to UL symbols.

  - [Closed] 1st round discussion

Proposal 1-1

Proposed Agreement:

Study whether a slot can consist of both SBFD and non-SBFD symbols.

|  | Company |
| --- | --- |
| Support | Samsung, DOCOMO, Sony, xiaomi, Intel, IDC, ITRI, Panasonic, Huawei, Hisilicon, WILUS, Lenovo, LG, ZTE, Nokia, NSB, QC, ETRI, KDDI,OPPO |
| Not support | TCL |

| Company | Comments |
| --- | --- |
| Spreadtrum | We think the proposal wants to discuss the granularity of SBFD subband time location configured by semi-static configuration, slot-level or symbol level SBFD. But not the actual SBFD symbols pattern in a slot, which depends on UL symbols since UL symbols with DL subband is with lower priority. So for a slot with D-F-U symbols, U symbols are non-SBFD symbols, but D and F can be SBFD symbol. Although this slot can be indicated as SBFD slot, the actual SBFD symbols within the slot are only limited in DL and Flexible symbols. Thus, the proposal may be changed into: Study the granularity of SBFD subband time location configured by semi-static configuration is slot-level or symbol level. |
| New H3C | If TRX switching time is enough, this case should be support. |
| DOCOMO | OK with this proposal. |
| Sony | OK to study the benefits & complexity of having SBFD & non-SBFD symbols in a slot. |
| Xiaomi | The proposal does not only depends on the time domain granularity for UL subband configuration. Even if slot-level granularity is adopted in the end, there is possibility that a slot contains both SBFD symbol and non-SBFD symbol, e.g. special slot indicated as SBFD slot. |
| NEC | We would prefer to keep SBFD granularity as slot level rather than symbol level. However, if strong justification can be provided for enabling symbol level granularity then we can discuss the set of issues for supporting such granularity. |
| Intel | In our view, this is a basic assumption which impacts the overall gain achievable by SBFD operation, and also how to enhance resource allocation (including time, frequency, power, spatial parameter) as well as timing (if needed). |
| IDC | In our opinion, there might be a switching guard time required to switch from SBFD to non-SBFD symbols (for example at the gNB side). However, it might be worth it for a NW with frequent change of traffic to for example accommodate the UL traffic via SBFD symbols and then switch to DL-only symbols for DL traffic. This could result in much more flexible scheduling at NW. |
| Huawei, HiSilicon | Support to study this further. In a “S” slot with DL/flexible/UL symbols, UL subband can be configured in DL and flexible symbols. In this case, the DL and flexible symbols are SBFD symbols and the UL symbols are non-SBFD symbols. |
| TCL | In our view, there is no need to study this proposal. Since this is a common understanding that the SBFD and non-SBFD operations are configured in symbols in a slot. |
| LG Electronics | We are OK to study. But, we think symbol level granularity is definitely required for SBFD symbol. |
| ZTE | Even legacy TDD slot formation configuration allows symbol-level configuration. SBFD pattern is dependent on the legacy TDD slot format configuration, for compatibility, symbol-level SBFD configuration should be supported. In the end, whether a slot containing both SBFD symbol and non-SBFD symbol is applied or not is up to network implementation in the end. |
| Nokia, NSB | We support the proposed study, with the following considerations: A slot can consist of both SBFD and non-SBFD symbols, as long as a channel is not mapped into symbols of different types within the same slot. (covered in Proposal 1-7) |
| QC | For this proposal, we should list some aspects for the study, e.g. transition time or guard period for the switching, benefits for having a slot with SBFD and non-SBFD symbols. In addition, the proposal is related to bigger discussion on whether switching between SBFD and non-SBFD across slot boundary or ca be within a slot. Another related discussion is the time domain granularity of the SBFD location. In general, we prefer that switching is across slot boundary which means that slots are either SBFD or non-SBFD and first or last few symbols of a slot are used as guard period for duplex transition. If companies see benefit, we are okay to study as long as physical channel is not mapped to combination of SBFD and non-SBFD symbols. |
| OPPO | TDD configuration granularity in time domain is symbol-level, similarly, SBFD configuration should be symbol-level. So it is nature to allow that slot consists of both SBFD and non-SBFD symbols |

Proposal 1-2

Proposed Conclusion:

Flexible subband is not considered for SBFD operation.

|  | Company |
| --- | --- |
| Support | Spreadtrum, New H3C, Samsung, DOCOMO, Xiaomi, NEC, Intel, Panasonic, WILUS, Lenovo, LG , Nokia, NSB, QC, KDDI |
| Not support | Sony, IDC, ITRI, Huawei, Hisilicon, TCL, ETRI |

| Company | Comments |
| --- | --- |
| Spreadtrum | We support the proposal. First, it is easy lead to misalignment among gNB and UEs. Since if DL reception in flexible subband indicated by dynamic scheduling of PDSCH/CSI-RS, the target UE and gNB are aware of this link direction, however the other UEs do not aware the link direction change of flexible subband, they still try to transmit UL which is configured by higher layer signalling in the flexible subband. This unwanted UL will be serious interference for the UE which receive the DL. Similarly, the RBs in flexible subband used as UL would have the same interference issue for gNB. Second, there is no doubt that flexible subband would lead to more complex CLI issues, which need more information exchanged between gNB-gNB and UE-UE. Considering flexible symbol in the past three previous releases have not been deployed properly, which is still being discussed in 9.3.3. Due to complex signaling design and CLI mitigation, we suggest to lower flexible subband priority in Rel-18. |
| New H3C | If supported, flexible subband is researched with low priority at least |
| DOCOMO | As pointed out by many companies, the flexibility benefit raised by proponents can be realized without introduction of flexible subband. Flexible subband introduction would lead to much complexity. |
| Sony | Flexible subband is one way to facilitate dynamic SBFD, where we can follow the same mechanism as Flexible symbol, i.e. semi-static flexibly subband can be further changed to DL, UL o remain flexible by SFI or UL/DL grant. |
| Xiaomi | One of the motivation of supporting flexible subband is to introduce flexibility. However, flexibility is already there considering the current dynamic TDD mechanism. Especially, we are discussing whether dynamic adaptation of UL subband is supported or not. If dynamic adaptation of UL subband is supported, we don’t think there is any needs to support flexible subband. |
| IDC | It seems premature to conclude this. We share similar views with Sony. Considering the objectives of SBFD to increase the coverage and lower the latency, the flexible subbands provide more flexibility for the NW to accommodate the traffic. The NW could also avoid the potential CLI by implementation. |
| Huawei, HiSilicon | It is a bit premature to preclude the discussion on flexible subband. First, one should first decide whether DL transmission is allowed in UL subband. In our view, if DL receptions are allowed in UL subband, there is no practical difference between UL subband and flexible subband. We also notice that in proposal 3-1 “Method#2: victim UE measures RSRP of aggressor UE’s CLI-SRS within UL subband.” is identified. This implies that DL reception in UL subband should be allowed in this method. Besides, we should also discuss the UE behaviors in a SBFD symbol configured as flexible in TDD-UL-DL-ConfigCommon. If a UE is configured with a UL subband on flexible symbols, one may need to know whether RBs outside the UL subband are flexible RBs. If so, the set of consecutive flexible RBs constitute a flexible subband. For both cases list above, there is a benefit of resource efficiency since the resources in the flexible subband can be used as either UL or DL. We also would like note that flexible subband is future-proof considering the potential support of subband over-lapping full duplex in the future. |
| TCL | Flexible subband may have many advantages in collision handling, and it is too early to preclude flexible subband from the discussion. |
| LG Electronics | We are fine with the proposal. If the motivation of flexible subband and the reason of necessity of flexible subband are cleary understood, we may study the benefit of flexilbe subband. But, the study of flexible subband is not preferred in this Release. |
| ZTE | As we discussed in our contribution, it seems that companies have different understandings on the definition of flexible subband, e.g., - Alt.1: subband that can be used to transmit UL and DL is considered as flexible subband. - Alt.2: subband that configured in the flexible symbols is considered as flexible subband. - Alt.3: part of the resources within the UL subband is configured as flexible resource, within which DL transmission and UL transmission are allowed. With a clear and common understanding about “flexible subband”, it is impossible to derive what the proposal is trying say. Also, this discussion is related to whether DL transmission is allowed in UL subband. If yes, then we tend to agree that flexile subband is not needed. However, if no, then we see some benefits of introducing flexible subband. |
| Nokia, NSB | We support the proposed conclusion. |
| QC | We prefer to have DL and UL subbands only with fixed transmission direction. Flexible symbol can provide similar functionality and hence no need for flexible subband. |

Proposal 1-3

Proposed Conclusion:

The frequency location of DL subband(s) can be explicitly indicated or implicitly derived, which can be down-selected in work item phase.

  - In case frequency location of DL subband(s) is implicitly derived, the frequency locations of UL subband and number of RBs for guardband(s) if any are explicitly indicated.

|  | Company |
| --- | --- |
| Support | Spreadtrum (with clarification for guard band), New H3C, Samsung, DOCOMO, Sony, NEC, Intel, IDC (with removing the subbullet), ITRI, Panasonic, Huawei, Hisilicon, TCL, Lenovo, LG (with removing the subbullet) , Nokia, NSB, QC, ETRI, KDDI,OPPO |
| Not support | WILUS |

| Company | Comments |
| --- | --- |
| Spreadtrum | Just clarification for guard band when DL subband(s) are explicitly configured, the proposal can be updated as: The frequency location of DL subband(s) can be explicitly indicated or implicitly derived, which can be down-selected in work item phase. - In case frequency location of DL subband(s) is explicitly configured, guardband(s) if any are implicitly derived - In case frequency location of DL subband(s) is implicitly derived, the frequency locations of UL subband and number of RBs for guardband(s) if any are explicitly indicated. |
| DOCOMO | Support. Spreadtrum’s updated version is also fine. |
| Sony | Didn’t we already agreed to this under this agreement? Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol |
| Xiaomi | We don’t think any explicit configuration besides UL subband is needed as shown in our Tdoc. Guard band can be purely guaranteed by gNB implementation. However, if majority view is we need define guard band, we can live with the current proposal. Furthermore, we think it is better to clarify whether a semi-static configured guard band can be used for DL transmission if there is no UL transmission or the UL transmission is assigned to the center of UL subband, wherein there is sufficient GAP between DL transmission and UL transmission already. |
| IDC | Support the main bullet to be discussed in work item phase, but the subbullet is not needed. |
| Panasonic | We share with Spreadtrum's view. The relationship between DL subband and guardband is better to be clarified. |
| Huawei, HiSilicon | We suggest to remove the subbullet. There is no need to discuss further details if the indication of DL subband is to be discussed in normative phase. |
| WILUS | We have concern on the sub-bullet. Companies provided benefits on knowledge of location of guardband(s) at SBFD aware UE. However, it’s a different issue whether to indicate or not guardband(s) location. For example, pre-defined fixed values of guardband(s) determined by RAN4 can be introduced. In this case, the SBFD aware UE can determine the guardband(s) locations without explicit indication regardless of explicit or implicit indication of DL subband(s). |
| LG Electronics | We are fine with the motivation of this proposal. Also, we are fine with the main bullet of the proposal. We are fine with the proposal with removing the sub-bullet because indication of guard band is not acceptable. Rather than indicating guard band, it is better to indicate the frequency region where DL is not permitted. |
| ZTE | Actually, it has been agreed that the frequency location of UL subband is explicitly indicated. Then it seems the sub-bullet is not needed. Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. We agree that the frequency resource of DL subband should be derived in the end either by explicit indication or by implicit way. From our perspective, we prefer to indicate the frequency resource of DL subband explicitly. This is future-proof for the T/F overlapping full duplex as the DL subband and UL subband can be fully overlapped or partial overlapped. |
| Nokia, NSB | We support the proposed conclusion. |
| QC | Agree with the FL to postpone the discussion on the signalling details to the work item phase. |
| OPPO | Considering that flexible SBFD structure, e.g. {DU},{DUD} and flexible UL/DL size, DL subband indication overhead is largely. It is preferred that frequency location of DL subband(s) is implicitly derived. |

Proposal 1-4

Proposed Conclusion:

Dynamic indication of frequency location/size of SBFD subbands is not considered.

|  | Company |
| --- | --- |
| Support | Spreadtrum, Samsung, DOCOMO, xiaomi, NEC, ITRI, Huawei, Hisilicon, WILUS, LG, Nokia, NSB , QC |
| Not support | IDC (WI phase), TCL, Lenovo, ETRI |

| Company | Comments |
| --- | --- |
| New H3C | It is better to decide it during WI. During SI , it isn’t necessary to decide early. |
| IDC | This issue should also be treated in work item phase. From one point of view, the dynamic indication of SBFD subbands allows the NW to have a much more flexibility in scheduling the resources. On the other hand, the NW can (once in a while) use the dynamic changes of subbands as a tool to estimate the potential UE-to-UE or gNB-to-gNB CLI in the cell and to determine potential scheduling strategies accordingly. The dynamic indication can be UE-specific or group-based. |
| TCL | We share similar views with IDC. In our view, dynamic Indication of frequency location of subband may provide gNB the flexibility to avoid several types of collision. In addition, there are also several benefits of dynamic indication which can help gNB to reduce the effect of interference. |
| Lenovo | As IDC mentioned, the SI should study the pro/cons of dynamic freq location/size indication. Whether it is supported is a WI stage discussion. |
| LG Electronics | We are fine with the proposal. In this study item, it seems necessary to focus on static configuration rather than flexible/dynamic adaptation of frequency region for SBFD subband. We can continue this discussion in enhancement of Duplex Evolution. |
| ZTE | This proposal is related to the subband configuration, i.e., cell-common configuration or UE-specific configuration. For cell-common configured subband, we agree dynamic indication of frequency location/size of SBFD subbands is not considered. However, if UE-specific subband configuration is introduced, then dynamic indication of frequency location/size of SBFD subbands per UE is possible. We propose to clarify the cell-common vs UE-specific subband configuration first and then come back to this issue. Overall, the frequency location/size of SBFD subband has serious impact on the gNB implementation. For example, it impacts the subband filter design, slef-interference suppression, etc. Even if UE-specific subband configuration is introduced, the UE-specific subband should be confined within the cell-common subband to avoid implementation issue. |
| Nokia, NSB | We support the proposed conclusion. If the location/size of the SBFD subbands is dynamically changing in frequency domain, it will be more challenging to handle cross-link interference. However, this agreement shouldn’t preclude the dynamic conversion of the symbol into DL-only (for SBFD operation configured in DL or flexible symbols) or UL-only (SBFD operation configured in flexible symbols), so that the gNB can adapt the SBFD operation to current traffic conditions. |
| QC | To control the CLI, it is better to have all SBFD symbols with same frequency resource of the UL subband. |
| KDDI | Flexibility on UL subband size/location in frequency domain can be attractive for potentially accommodating some use cases, however feasibility from gNB/UE implementation perspective and a benefit should be carefully investigated. |
| OPPO | What does “SBFD subband “ mean? SBFD is UL subband or all of UL subband,DL subband and guard band. |

Proposal 1-5

Proposed Agreement:

The SBFD aware UE does not transmit UL outside UL subband or receive DL outside DL subband(s).

Further study dynamic SBFD where semi-statically configured UL/DL subbands can be dynamically disabled in a symbol/slot.

  - If semi-statically configured UL/DL subbands are disabled in a symbol/slot, there are no UL/DL subbands in the symbol/slot and the symbol/slot is not an SBFD symbol/slot.

  - The dynamic signaling is based on a group-common signaling.

At least the following aspects should be considered.

  - Performance gain compared with SBFD with semi-static subbands only/dynamic TDD

  - Implementation complexity at gNB/UE

  - Design complexity/overhead

|  | Company |
| --- | --- |
| Support | Spreadtrum (main bullet), DOCOMO (main bullet and the first sub-bullet), Sony, NEC (with changes), Intel, IDC (in principle), WILUS, TCL, Lenovo, LG |
| Not support | Samsung, xiaomi, Huawei, Hisilicon |

| Company | Comments |
| --- | --- |
| Spreadtrum | We support the main bullet. For the dynamic SBFD part, we are open to discuss. And we share a same view that if dynamic SBFD is supported, a group-common signaling is only one possible way. So we would like to further clarify it a little bit, hope it is common understanding: - The dynamic signaling is based on a group-common signaling by PDCCH, not considering dynamic scheduling. |
| Samsung | For the first part (“SBFD aware UE does not transmit UL outside UL SB…”): We prefer to keep the FFS on the possibility of DL scheduling by the gNB inside the UL SB for either the SBFD-aware UE configured with the UL SB, or for the other UEs in the cell. Otherwise, we would always hard-split DL and UL resources which we have seen in system level evaluations to result in DL TP/SE penalties. We have serious concerns on the second part for dynamic SBFD operation: For SBFD on legacy D symbols, no such dynamic SBFD operation can apply, and this should be clarified in the wording. For legacy F symbols, it would then be possible to disable the SBFD configuration of the SBFD-aware UE, convert F to UL-only and UL schedule the SBFD-aware UE across its entire UL BWP. When considering Option 2 from RAN1#111, this is undesirable because the Rel-19 UE would then need to dynamically update its UL Tx RB and BB settings “on the fly”. We have not fully resolved the open FFS for SBFD on legacy D symbols and Option 1 or Option 2 and the FFS for the legacy F symbols. Deciding on the signaling should come after we have agreed on the principles of SBFD scheduling on the D/F symbols. |
| DOCOMO | We are supportive of the main bullet and the first bullet. For the second bullet, we prefer to not preclude UE specific signaling for now. |
| Sony | The reserved slot format in the existing SFI can be used to indicate changeds to SBFD symbol |
| Xiaomi | We don’t understand why we need to discuss the dynamic indication of DL subband. DL subband is actually legacy DL resource, what is the behavior after disable DL subband? For the second subbullet under the main-bullet, we think it is too much. It can be address in the further discussion. |
| NEC | We are okay with the intention of the proposal however “UL/DL subbands can be dynamically disabled” can be misinterpreted. We would prefer to keep a simpler wording something like below: “Further study dynamic SBFD where semi-statically configured SBFD slot/symbol can be dynamically converted to DL-only slot/symbol.” |
| Intel | We are generally fine with the proposal. Regarding the dynamic signaling, we think it is reasonable to consider at least group-common signaling, meanwhile, we think explicit indication by UE-specific signaling can be considered, e.g., explicit indication of SBFD or non-SBFD operation by DL assignment or UL grant. Therefore, we suggest to replace the 2nd sub-bullet with - The dynamic signaling is based on a group-common signaling or UE specific signaling for explicit indication. |
| IDC | Support the proposal in general. |
| Panasonic | As Samsung pointed out, this proposal is related to SBFD operation in legacy flexible symbol agreed in RAN1#111. We also think whether option 1 or option 2 is applied need to be clarified first. |
| Huawei, HiSilicon | The first bullet is too restrictive since the DL and UL transmissions are confined within DL subband and UL subband respectively. In addition, we suggest to discuss FL proposal 3-1 together, because DL reception within UL subband may be needed to measure UE-UE CLI and UE-UE beam paring for FR2, and UL transmission within DL subband may be needed to measure gNB-gNB CLI, gNB-gNB channel for CBF for FR1, and gNB-gNB beam pairing for FR2. |
| LG Electronics | We are fine first two bullets. But, it is not clear the motivation to capture the one specific indication method in the second sub-bullet. |
| ZTE | We are generally ok with this proposal. However, regarding the 2nd bullet, it is not clear why only “disable” is allowed but “enable” is not allowed. If the SBFD symbol fallbacks to legacy symbol, then it may need to convert to SBFD symbol again based on the system situation. We propose to support the conversion in both directions. |
| Nokia, NSB | We support the main bullet (slightly prefer NEC’s proposal) and the first bullet. We propose to change the second bullet to: “The dynamic signaling is at least based on a group-common signaling”, so that we do not preclude too early other alternatives. We think it is also important to consider feature parity between legacy devices and SBFD devices, when it comes to the possibility of being scheduled in DL within the UL subband in slots semi-statically configured as SBFD. |

Proposal 1-6

Proposed Agreement:

An UL subband can be configured in SSB symbols.

  - FFS whether it is considered as full-DL symbol or SBFD symbol from SBFD-aware UE perspective.

|  | Company |
| --- | --- |
| Support | New H3C, Samsung, Sony, xiaomi, Intel, IDC, ITRI, Huawei, Hisilicon, WILUS, LG, ZTE, QC, ETRI, KDDI |
| Not support | Nokia, NSB |

| Company | Comments |
| --- | --- |
| Spreadtrum | We think the proposal has more relation with SBFD time domain configuration. Two comments for the proposal: - We have agreed that semi-static configuration of subband time locations for SBFD operation. And SBFD operation only include UL subband in DL and flexible symbols. So the main bullet can change into “Semi-static configuration of subband time locations for SBFD operation”, not only UL subband. - Slot level or symbol level has not been decided yet, so the word of overlap is more proper. So the proposal is suggested to update as: Semi-static configuration of subband time locations for SBFD operation can be configured to overlap with SSB symbols. - FFS whether it is considered as full-DL symbol or SBFD symbol from SBFD-aware UE perspective. |
| DOCOMO | SBFD operation in SSB symbol would cause many issues as proposed in many contributions. If majority companies think it would be difficult to ensure no UL subband configured in SSB symbol, we are fine with the main bullet, and we support to consider the symbol as full DL symbol for the FFS bullet. |
| Sony | If we don’t support this, e.g. CLI is an issue then we will have to support SBFD & non-SBFD in a slot so that the SSB symbol is full DL whilst the other symbols in the slot can be SBFD. Otherwise we are limiting ourselves to the number of SBFD slots that can be configured. |
| NEC | We think we are approaching this point from the wrong direction. First question to be answered is whether SBFD operation is allowed in the SSB symbol or not. We have a preference that SBFD operation should not be allowed in SSB symbol. And once the conclusion for this discussion is reached we can discuss the signaling mechanism whether UL subband can be configured within SSB symbol or not because that would comprise stage-3 detail. |
| IDC | Considering the objectives of SBFD operation to increase the coverage and decrease the latency, the SBFD operation needs to be supported in SSB symbols. Regarding the FFS point, it may be considered as SBFD symbol from SBFD-aware UE perspective. |
| Huawei, HiSilicon | SSB is usually transmitted with a short periodicity, such as 5ms to 20ms, in order to ensure a low initial access delay and mobility management performance. Within a SSB period, the SSB duration can be 5ms at most. This means 25%-100% time within one SSB period cannot be used for SBFD operation; it severely degrades the performances of SBFD operation. Except UEs who are performing initial access or cell reselection, a short SSB measurement period is not necessary. So these UEs can perform the SBFD operation on SSB symbols to improve the performance of SBFD operation. |
| LG Electronics | We are fine with the proposal. We think this proposal is very helpful to make the SBFD symbol indication method simpler. |
| ZTE | We support to configure UL subband in the SSB symbols and the UE behavior can be FFS. |
| Nokia, NSB | We do not agree with the proposed agreement. As we wrote in our contribution, there are different aspects to be considered for the configuration of SBFD in SSB symbols, such as the fact that SSBs are being measured by other UEs in this symbol, including IDLE/INACTIVE UEs and legacy UEs, which might suffer with the increased UE-to-UE CLI in case a UL subband is configured there. Additionally, SSBs can be power boosted, which would increase the self-interference at the gNB. We agree with NEC’s comment that we should first reach a conclusion on whether SBFD operation is allowed in SBFD symbols, and later discuss the signaling mechanism. We would like to hear from other companies the motivation behind configuring an UL subband in an SSB symbol if later on it would be agreed to consider this symbol as full-DL from UE perspective. In this case, the UL subband would be unavailable for UL and DL in that symbol. |
| QC | It is important to agree that SBFD symbols can be configured in UL-subband. Then, RAN1 can study trade-off whether to allows UL trnamission in the UL subband of the SBFD symbols |
| OPPO | The requirement of SSB decoding performance is usually large than normal PDSCH, so to avoid inter-subband CLI, UL subband should not be configured in SSB symbols. If UL subband in SSB symbol is to be supported, CLI cancellation enhancement in SSB, e.g. Larger guard band should be studied firstly. |

Proposal 1-7

Proposed Agreement:

UE does not expect to transmit a PUSCH/PUCCH/SRS or receive a PDSCH/PDCCH/CSI-RS overlapping with both SBFD and non-SBFD symbols in the same slot.

|  | Company |
| --- | --- |
| Support | Spreadtrum, xiaomi, Intel , Huawei, Hisilicon, TCL, Nokia, NSB, QC,OPPO |
| Not support | Samsung, DOCOMO, Sony, IDC, ITRI, Lenovo, ZTE, ETRI |

| Company | Comments |
| --- | --- |
| H3C | If oth SBFD and non-SBFD symbols in the same slot is configured, UE should execute transmit a PUSCH/PUCCH/SRS or receive a PDSCH/PDCCH/CSI-RS overlapping with both SBFD and non-SBFD symbols in the same slot. So if we need discuss about proposal 1-1 and then come back to this proposal, |
| Samsung | For SBFD antenna configuration 1, where QCL and EPRE between non-SBFD and BSFD symbols can be expected to differ, we agree. For SBFD antenna configurations where gNB can ensure consistent TCI handling across the symbol types, the possibility to schedule PDSCH, PUSCH, PUCCH across SBFD/non-SBFD symbols should remain allowed and configurable by the gNB under assumption that the SBFD-aware UE can receive/transmit using the same QCL assumptions and EPREs for the PDSCH/PUSCH/PUCCH when allowed. |
| DOCOMO | We feel it a little too early to forbidden any DL/UL channel/signal overlapping with both SBFD and non-SBFD symbols in the same slot now. For example, if the channel/signal scheduled by DCI, the DCI can indicate proper FDRA/power/MCS parameters for the channel/signal to avoid issues proposed by companies. Such limitation may cause much restriction to gNB scheduling. |
| Sony | We need to see the outcome of Proposal 1-1 or rather part of the study in Proposal 1.1 would lead to whether we can agree to Proposal 1-7. |
| NEC | It would be preferable to reach the conclusion for Proposal 1-1 before we discuss this proposal. |
| IDC | Do not support the proposal. The UE can Tx/Rx in both SBFD and non-SBFD modes in the same slot, as long as the UE is aware of the RBs to be used in SBFD and non-SBFD symbols. |
| Huawei, HiSilicon | We prefer not to support transmitting a PUSCH/PUCCH/SRS or receive a PDSCH/PDCCH/CSI-RS overlapping with both SBFD and non-SBFD symbols in the same slot. It will complicate the spec and has less benefits. |
| WILUS | We share the similar view with H3C and Sony. It can be discussed after agreement is made on Proposal 1-1. |
| Lenovo | This is part of the study and related to proposal 1-1. |
| LG Electronics | Further clarification about the situation ‘to transmit a PUSCH/PUCCH/SRS or receive a PDSCH/PDCCH/CSI-RS overlapping with both SBFD and non-SBFD symbols in the same slot’ is necessary. |
| ZTE | This issue depends on lots of aspects: - Whether transition period is needed between SBFD symbol and non-SBFD symbol from gNB/UE side; - Whether the same configuration/resource allocation is applied for the transmission in SBFD symbol and non-SBFD symbol - If different configuration is configured for the transmission in SBFD symbol and non-SBFD symbol, which configuration should be applied a transmission overlapping with both SBFD and non-SBFD symbols happens. Before making this conclusion, it is better to have some discussion on the above aspects to align companies’ views. |
| Nokia, NSB | We support the agreement, the alternative would introduce unnecessary complexity to the system design. We would like to clarify that this agreement shouldn’t preclude: - TBoMS/inter-slot repetitions to be transmitted across slots of different type (SBFD or non-SBFD) - Intra-slots repetitions to be transmitted on symbols of different type (SBFD or non-SBFD) |
| QC | Support. We don’t see the gain for allowing a channel across SBFD and non-SBFD in a symbols. In addition, there are lots of challenges/requirement from both gNB/UE (e.g. filter tunning, panel switching, timing adjustment, beam and power consistency) |
| OPPO | We support the intention of proposal. However, different detailed solutions should be considered for dynamic scheduling and semi-static configuration. For dynamic scheduling, resource overlapping with both SBFD and non-SBFD symbols should be avoided by scheduling. For semi-static configuration, resource overlapping with both SBFD and non-SBFD symbols is not easy to be avoided by scheduling, and it can be dropped. |

Proposal 1-8

Proposed Agreement:

Consider the following options for potential enhancements on frequency domain resource allocation for SBFD-aware UEs:

- Option 1: Single FDRA for SBFD symbols and non-SBFD symbols

  - Option 1-1: A transmission/reception with frequency resources outside UL subband/DL subband(s) is dropped.

  - Option 1-2: Frequency resources of a transmission/reception in SBFD symbols are derived by excluding RBs outside UL subband/DL subband(s).

  - Option 1-3: FDRA is valid only in SBFD or non-SBFD symbols, i.e. invalid in non-SBFD or SBFD symbols

  - Option 1-4: Separate frequency resources are derived for non-SBFD symbols and SBFD symbols based on DL/UL BWP and DL/UL subband(s) in non-SBFD symbols respectively

- Option 2: Separate FDRA for SBFD symbols and non-SBFD symbols

Note: Different options can be applied to different channels/signals and different options can be applied to a channel/signal e.g. for transmissions/receptions with repetition and without repetitions.

|  | Company |
| --- | --- |
| Support | Samsung, Sony, Intel, IDC, WILUS, Lenovo, Nokia, NSB (with update),OPPO |
| Not support | Huawei, Hisilicon |

| Company | Comments |
| --- | --- |
| Spreadtrum | We want clarification for Option 1-3 and 1-4. - How to decide Option 1-3 FDRA is valid only in SBFD or non-SBFD symbols? Need new indication to say one FDRA is only valid in SBFD for example? - What is the different between Option 1-4 and Option 2? |
| New H3C | Support option 2 and need clarify option 1-3 |
| DOCOMO | We also have question on the understanding option 1-3. Regarding option 1-4, we think the intention is that different interpretation of the single FDRA is applied for SBFD and non-SBFD symbols. |
| Sony | Support Option 2, which seems like a cleaner solution. |
| Xiaomi | The proposal mixes UL and DL together. However, the issue for DL and UL are different. For UL, the problem is that the number of contiguous UL RBs in SBFD symbol and non-SBFD symbol is different. For DL, the problem is that the RBs in SBFD slot are not contiguous while in non-SBFD slot are contiguous. Hence, it is more proper to discuss DL and UL separately. |
| NEC | Further clarification is required on following points: Option 1-3: FDRA is valid only in SBFD or non-SBFD symbols, i.e. invalid in non-SBFD or SBFD symbols Does this option imply that a transmission/reception is only performed for either SBFD or non-SBFD symbols but not both? For example if PUSCH repetitions span both SBFD and non-SBFD slots, then PUSCH would be transmitted in either SBFD slots or non-SBFD slots but not both. Option 1-4: Separate frequency resources are derived for non-SBFD symbols and SBFD symbols based on DL/UL BWP and DL/UL subband(s) in non-SBFD symbols respectively How is this option different than Option-2? Functionality vise both Option 1-4 and Option-2 are same. |
| Intel | Since we have not figured out how to support dynamic SBFD switch (proposal 1-5), and whether/how to support dynamic SBFD switch may have impact on options in this proposal, e.g., whether we use dynamically determined or semi-statically determined SBFD/non-SBFD symbol in this proposal, it is better to focus on the case of semi-static configuration for now. So, we suggest to add ‘at least for the case of semi-statically determined SBFD/non-SBFD symbols’ to the main bullet. |
| IDC | Support the proposal and our preference is currently on Option 1-4. Some clarifications may be needed on those options. |
| Panasonic | We also think option 1-3 and 1-4 need to be clarified. For option 1-4, is “DL/UL subband(s) in non-SBFD symbols” intend for “DL/UL subband(s) in SBFD symbols”? |
| Huawei, HiSilicon | We think we should discuss whether or not allow to allocate multiple-slots PUSCH/PUCCH/PDSCH across different slot types with its cons and pros before we discuss this issue, including - Whether or not a PUSCH repetition type A (physical/available slot counting) can be allocated on both SBFD slots and non-SBFD slots - Whether or not a PUSCH repetition type B can be allocated on both SBFD symbols and non-SBFD-slots - Whether or not a TboMS PUSCH (w/ or w/o repetition) can be allocated on both SBFD slots and non-SBFD slots - Whether or not a PUCCH repetition can be allocated on both SBFD slots and non-SBFD slots - Whether or not a single slot PUSCH/PDSCH/PUCCH can be allocated on both SBFD symbols and non-SBFD symbols - Whether or not semi-static scheduling (CG-PUSCH, SPS-PDSCH) has a same behavior as dynamic scheduling If multi-slots PUSCH/PUCCH/PDSCH can be only allocated on the same slot type, option 1 without any enhancement (i.e., option 1-3 if we have a right understanding on this option) is enough; otherwise, we need to further discuss these options including option 1 with sub-options and option 2 |
| Lenovo | Support. Option 1-3, 1-4 should be further clarified. |
| LG Electronics | We are fine with the discussion. |
| Nokia, NSB | We support the proposed agreement with the following change: - Option 1-4: Separate frequency resources are derived for non-SBFD symbols and SBFD symbols based on DL/UL BWP and DL/UL subband(s) in non-SBFD symbols respectively |
| QC | Suggest separating the proposal for UL and DL. Further discussion on options 1-2, 1-3 and 1-4. Option 1-2: We don’t to deviate from 3GPP where only RM is appled to DL. Option 1-3: Does it mean that repetition applicable to SBFD only (or non-SBFD only)? Option 1-4: We don’t like to deviate from 3GPP principles where the FDRA bitifield interpreted based on UL/DL BWP sizes. Another approach is to target this proposal for multi-slot UL/DL transmission and Periodic transmission across SBFD and non-SBFD symbols (instead of narrowing it down to Freq. resources only). - For UL/DL Repetition and TBoMS, Study whether repetitions are allowed across SBFD and non-SBFD symbols or limited to same symbol type. And Impact to available slot counting, frequency hopping and frequency resources. - Whether sperate RRC configuration for SRS, PUCCH, CG-PUSCH and SPS-PDSCH. |
| ETRI | We are fine with HW’s version. |
| OPPO | Option 1 is preferred to avoid large DCI overhead. |

Proposal 1-9

Proposed Agreement:

Study at least the following separate configurations for SBFD symbols and non-SBFD symbols:

  - Separate time and frequency resources

  - Separate FH parameters

  - Separate UL power control parameters

  - Separate beam configurations

|  | Company |
| --- | --- |
| Support | Samsung, DOCOMO, Sony (need clarification), NEC, Intel, Panasonic, TCL, Lenovo, LG Electronics (With adding ‘Separate MIMO configuration’) , Nokia, NSB, QC |
| Not support | Spreadtrum, IDC (first agree on Proposal 1-8) |

| Company | Comments |
| --- | --- |
| Spreadtrum | We support separate frequency domain resource and FH parameters, but for other aspects we suggest to make decisions late. - Not sure why time domain resource need to improve, would some companies elaborate more? - Power domain also need to postpone, because the power domain enhancement is from the benefits of the CLI handling. However, due to it is still in process without sufficient convince, power domain enhancement can start later. - Same as power domain, spatial domain improvement comes from the CLI handling, such as the preferred or non-preferred beam. We can hold it until more information is provided, e.g. the difference requirements of spatial domain in SBFD and non-SBFD regions. |
| New H3C | The motivation of this proposal isn’t clear to us and need clarify it in detail |
| DOCOMO | Fine with the proposal. |
| Sony | On the following sub-bullet: - Separate time and frequency resources What does time & frequency resources refer to? Is this the subband location, which we already agreed to be at least semi-statically configured? Or is this TDRA and FDRA? We are fine to study the other sub-bullets. |
| Xiaomi | We are a little bit confused with the proposal. The intention is specific for UL transmission, right? |
| IDC | This proposal in a special case of the previous proposal (maybe Option 2?). So, we prefer to discuss the Proposal 1-8 first. |
| Panasonic | We also think that the separate time resources need to be clarified. |
| Huawei, HiSilicon | We understand the intention of the proposal. However, the proposal is not clear since it is not clear what the separate configurations corresponds to. In addition, please also see our reply on FL’s proposal 1-8. |
| WILUS | On the 2nd sub-bullet, more clarifications are necessary, such as FH parameters w.r.t. PUCCH or PUSCH. For PUCCH FH, frequency hop locations are determined based on configurations. For PUSCH FH, frequency hop locations can be adjusted within UL subband by enhancing current modulo operation while not introducing additional configurations. Thus, we suggest to modify the 2nd sub-bullet only for PUCCH case. Proposed Agreement: Study at least the following separate configurations for SBFD symbols and non-SBFD symbols: - Separate time and frequency resources - Separate FH parameters for PUCCH - Separate UL power control parameters - Separate beam configurations |
| LG Electronics | The number of the panel / the number of the antenna element, the number of antenna ports is/are included in MIMO related configuration. One clear point is the antenna configuration for SBFD operation is different from the antenna configuration for non-SBFD operation. |
| ZTE | Overall, we support the proposal to study this issue. However, we are not sure whether the main bullet can cover all the potential solutions. For example, lots of companies raised that SBFD symbol and non-SBFD symbol can share the same configuration but an additional offset is introduced for SBFD symbol, e.g., for adjusting the location of frequency hop. One way to address this is to add an FFS below: FFS: explicit configuration or implicit derivation. |
| Nokia, NSB | We support the proposed study. |
| OPPO | The precondition of separate configuration is SBFD known. So before we discuss this proposal, we’d better to clarify how to determine SBFD symbol and non-SBFD symbol, especially for flexible symbol case and dynamic SBFD indication case. |

Proposal 1-10

Proposed Conclusion:

At least the following cases of time domain conflict of UL transmission and DL reception in the same SBFD symbol are identified.

- Dynamic DL reception vs. semi-statically configured UL transmission

- Dynamic UL reception vs. semi-statically configured DL transmission

- Semi-statically configured DL reception vs. semi-statically configured UL transmission

Note: PRACH transmissions and SSB receptions are not included.

|  | Company |
| --- | --- |
| Support | Spreadtrum (with a Note); New H3C, DOCOMO (suggest adding one case), Sony, NEC (with change), Intel, IDC, ITRI, Huawei, Hisilicon, WILUS, TCL, Lenovo, LG Electronics, Nokia, NSB (with update), QC, ETRI,OPPO |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| Spreadtrum | We support the proposal, but want to make more clear about Dynamic and semi-static, the following Note can be included in the proposal: Note: if the UL transmission is transmitted or DL reception is received using resources indicated by a DCI, then it belongs to “dynamic UL/DL transmission”; if the UL transmission is transmitted or DL reception using semi-statically configured resources or other repetitions/transmissions besides the first one of the dynamic UL transmission or DL reception, then it belongs to “semi-static UL transmission/DL reception”. |
| Samsung | We have not fully resolved the open FFS for SBFD on legacy D symbols and Option 1 or Option 2 and the FFS for the legacy F symbols. Deciding on the additional collision handling cases, if any, introduced by SBFD, should come after we have agreed on the principles of SBFD scheduling on the D/F symbols. |
| DOCOMO | Support the listed cases. On the other hand, we suggest to also study dynamic DL and dynamic UL at least when any of them with repetitions. In legacy specification, dynamic DL/UL repetitions overlapping with UL/DL slot/symbols is not precluded. We think DG DL repetitions colliding with DL UL repetitions is also possible. Dynamic DL reception vs. dynamic UL transmission, when at least one of the DL or UL channel/signals with multiple repetitions. |
| Xiaomi | It is better to add a FFS bullet for the controversial cases. For example: FFS: Dynamic UL vs. Dynamic DL with different priority FFS: Broadcast DL vs. UL transmission within UL subband |
| NEC | We don’t understand why network would configure conflicting resource for semi-static DL and UL which can be avoided by proper configuration. Hence, we think that there is no need to study the 3rd point of the proposal: “Semi-statically configured DL reception vs. semi-statically configured UL transmission” |
| IDC | Agree with the proposal in general. We are open to discuss different scenarios for handling the conflicts. |
| LG Electronics | We are fine with the proposal. |
| ZTE | We are generally ok with this proposal. In addition to the above, we think dynamic DL transmission vs dynamic UL transmission conflict is also possible if urgent transmission happens and gNB wants to transmit the urgent transmission and drop the previous dynamically scheduled transmission. |
| Nokia, NSB | We support the current agreement. With the highlighted changes: At least the following cases of time domain conflict of UL transmission and DL reception in the same SBFD symbol are identified. - Dynamic DL reception vs. semi-statically configured UL transmission - Dynamic UL reception transmission vs. semi-statically configured DL transmission reception - Semi-statically configured DL reception vs. semi-statically configured UL transmission Note: PRACH transmissions and SSB receptions are not included. |

Proposal 1-11

Proposed Agreement:

Study potential enhancements for SBFD operation in RRC_IDLE and RRC_INACTIVE states.

|  | Company |
| --- | --- |
| Support | New H3C, Samsung, DOCOMO, Sony, xiaomi, Intel, IDC, ITRI, Huawei, Hisilicon, WILUS, TCL, Lenovo, LG Electronics, ZTE, Nokia, NSB, QC, ETRI, KDDI |
| Not support | Spreadtrum, NEC |

| Company | Comments |
| --- | --- |
| Spreadtrum | It can be discussed later until the signaling type of SBFD time domain location is decided. |
| NEC | The motivation is not clear for us to support study for initial access |
| IDC | We support studying potential enhancements in RRC-Idle and RRC-Inactive, specifically for the concepts that are already being discussed for RRC-Connected mode. |
| LG Electronics | The UL subband can be used for RACH repetition / msg3 PUSCH repetition. That is the advantage to reduce latency of RACH procedure and to enjoy coverage enhancement. |
| ZTE | We support this proposal, especially for RACH in SBFD symbols. |
| Nokia, NSB | We support the proposed study. This has been discussed for many meetings, with the majority of companies supporting the study. There is no reason to further delay the agreement to study, so that companies can bring the potential enhancements to be discussed in the next meeting. Our view is that enabling PRACH transmission in SBFD symbols can increase PRACH capacity as well as reduce initial access delay. Moreover, PRACH coverage can also be enhanced as compared to only allowing PRACH transmissions in UL slots. We also support cell-common signaling of the SBFD subbands in SIB, so that IDLE/INACTIVE UEs can be made aware of the SBFD configuration to support initial access in SBFD symbols. |

  - [Closed] 2nd round discussion

Proposal 1-1

Proposed Agreement:

Study whether a slot can consist of both SBFD and non-SBFD symbols.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, Samsung, DOCOMO, IDC,NEC,OPPO |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Based on the previous discussions, companies are encouraged to provide detailed aspects to be considered to determine whether a slot can consist of both SBFD and non-SBFD symbols. |
| TCL | We are ok in general to study this proposal, but we are not sure about the motivation of this proposal, since this proposal may imply to the slot format. In case of slot format 0 where all the symbols are DL, if all the symbols are configured for SBFD operation then the slot may not consist of both SBFD and non-SBFD operation. On the other hand, a slot format x which may contains both DL and UL symbols may contains both SBFD operation and non-SBFD operation. |
| Sony | During the offline, I recall there were some minor editorial to this proposal, i.e.: Study whether or not a slot can consist of both SBFD and non-SBFD symbols. Aspect to consider is whether a transmission colliding with a subband of the opposite link direction can be moved in the frequency domain. If it is partially moved, how to deal with phase discontinuity. |
| LG Electronics | We are fine with the proposal to study. Sony’s proposal for minor editorial seems better. |
| IDC | Support this proposal to study further. |
| NEC | Same comment as last round and we agree with Qualcomm’s comment in the first round where we need to also consider the guard period applicability for such transitions which may reduce the resource efficiency of SBFD operation. Legacy TDD system do support symbol level transition granularity however such granularity was mainly specified to reduce latency for UL traffic. However, SBFD operation (even with slot level granularity) itself is defined to reduce the UL latency and hence motivation should be properly justified that why symbol level granularity is required for SBFD configuration, as symbol level granularity can reduce network performance. |
| OPPO | TDD configuration granularity in time domain is symbol-level, similarly, SBFD configuration should be symbol-level. So it is nature to allow that slot consists of both SBFD and non-SBFD symbols |

Proposal 1-12

Proposed Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, study the following options for SBFD-aware UEs:

  - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

  - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

  - PDSCH/PUSCH/PUCCH repetitions

  - SPS PDSCH/CG PUSCH

  - TBoMS

  - Multi-PUSCH/PDSCH scheduled by a single DCI

  - Periodic SRS/CSI-RS/PUCCH

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony , Samsung, DOCOMO, LG, IDC,NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | Should we also consider PDCCH or is it in a category by itself? |
| LG | Semi-persistent SRS/CSI-RS needs to be included? |
| IDC | Support the proposal and our preference is Option 2. |
| NEC | Given that we all have the understanding that current physical layer channels/signals may not support Option-2 natively, we need to also specify that further enhancements should be studied for these channels/signals to span different symbol types. We can keep following FFS point: Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols FFS: Enhancements required physical channel/signal configuration/parameters to enable transmission/reception across different symbol types |
| OPPO | Single PUSCH/PDSCH scheduled by a single DCI is not included in proposal. Does it mean that for Single PUSCH/PDSCH scheduled by a single DCI, the case across SBFD and non-SBFD should be avoided by scheduling? |

Proposal 1-13

Proposed Agreement:

For RBG for PDSCH/PUSCH RA type 0, study the following options for RBG size determination for SBFD-aware UEs:

  - Option 1: RBG size is determined based on size of DL/UL BWP

  - Option 2: RBG size is determined based on size of DL/UL subband(s)

|  | Company |
| --- | --- |
| Support | Support, TCL, Sony, Samsung, LG (with clarification), IDC,NEC,OPPO |
| Not support |  |

| Company | Comments |
| --- | --- |
| LG | In my understanding, for the case of option1, it is assumed that single RBG size is applied regardless of types of OFDM symbol (i.e., SBFD symbol, non-SBFD symbol). But, in case of option 2, it seems to be assumed that different RBG size can be applied depending on SBFD symbol and non-SBFD symbol. Is it right understanding? |
| IDC | Support the proposal. In order to select the best option, the concept of UE-specific guard-band sizes with different number of RBs should be considered. If Option 2 is supported, then the RBG size may change due to changes in the guard-band size. However, in Option 1, the RBG size will be the same in case of changes to the UL subband, DL subband, or guard-band sizes. |
| NEC | We understand that there may be benefits in determining RBG size based on UL/DL BWP, as effective BWP size is reduced to subband available during SBFD operation. To study this option and to understand its feasibility we need to consider different aspects e.g., - Compatibility with legacy UE Tx/Rx in the same symbol as SBFD UE - Handling different subband sizes including the case where subband size could be very small based on UL subband size and BWP partitioning. We can also accept to study option 2. |
| OPPO | Support and we prefer to option 2. Due to smaller RBG size allows finer resource allocation and is benefit for system efficiency. |

Proposal 1-14

Proposed Agreement:

For SBFD-aware UEs, study the following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands:

  - Option 1: Partial RBG(s) at subband boundaries can be used for PDSCH/PUSCH

  - Option 2: Partial RBG(s) at subband boundaries cannot be used for PDSCH/PUSCH

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, Samsung, DOCOMO, LG, IDC, NEC(Option 1) |
| Not support |  |

| Company | Comments |
| --- | --- |
| IDC | Support the proposal in general. This proposal can be discussed after agreement is made on the Proposal 1-13. In Proposal 1-14, our preference is Option 1. In our opinion, Option 2 may result in waste of resources that overlap with the RBs at the boundaries. |
| NEC | We prefer using partial RBGs at subband boundaries to ensure that there is minimal radio resource wastage. It would be good to discuss that in which case Option-2 would be applicable |

Proposal 1-15

Proposed Agreement:

For SBFD-aware UEs, study the following options for CSI reporting configuration across SBFD symbols and non-SBFD symbols:

  - Option 1: separate CSI reporting configurations for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting configurations for SBFD symbols and non-SBFD symbols

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, Samsung, DOCOMO, LG, IDC, NEC (prefer Option-1) |
| Not support |  |

| Company | Comments |
| --- | --- |
| IDC | Support the proposal to study this aspect further. In our opinion, in order to support subband-wise CSI reporting or differential CSI reporting, at least some CSI reporting contents or behaviors should be able to be different for SBFD symbols and non-SBFD symbols, but Option 1 vs. Option 2 seem to focus CSI reporting configuration aspects, which may be under second-level details and we are open for further study. |
| NEC | Does this imply that different CSI reporting configurations based on CSI-RS measured during SBFD and non-SBFD symbols or does this proposal mean different reporting configuration based on whether CSI report is sent during SBFD or non-SBFD symbol? Given that antenna panel-based isolation mechanism could be used for SBFD operation, this could result in different number of antenna elements/panels available during DL and SBFD symbols. Hence, it may finally reflect on availability on number of CSI-RS ports or precoder matrix size based on different network implementations between SBFD and DL symbol. Therefore, it seems natural that CSI report should also be differentiated between SBFD and non-SBFD symbols. |

Proposal 1-16

Proposed Agreement:

For SBFD-aware UEs, study the following issues for PRG:

  - Partial PRG(s) at subband boundaries with size of 2 and 4

  - Wideband precoder in case of non-contiguous DL subband(s)

|  | Company |
| --- | --- |
| Support | New H3C, Sony, DOCOMO, LG, IDC,NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| IDC | Support the proposal in general. More clarification is required. |
| NEC | We think the main bullet can be changed to the below description which is more clear. For SBFD-aware UEs, study the following issues for PRG of PRB bundling procedures for PDSCH: |

Proposal 1-9a

Proposed Agreement:

Study at least the following separate configurations for SBFD symbols and non-SBFD symbols at least for SRS and PUCCH:

  - Separate time and frequency resources

  - Separate FH parameters

  - Separate UL power control parameters

  - Separate beam configurations

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, Samsung (in principle), DOCOMO, LG, IDC,NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | Is there a reason why SRS & PUCCH are emphasized here but not for the other channels? |
| Samsung | PUSCH should be included |
| IDC | Support the proposal in general. It seems not clear why should the SRS and PUCCH have different beam configurations in SBFD and non-SBFD symbols? |
| NEC | We should include an FFS whether this is applicable to other configured physical channels/signals like CSI-RS/CG PUSCH |
| OPPO | The precondition of separate configuration is SBFD known. So before we discuss this proposal, we’d better to clarify how to determine SBFD symbol and non-SBFD symbol, especially for flexible symbol case and dynamic SBFD indication case. |

Proposal 1-17

Proposed Agreement:

Study the non-contiguous resource allocation for PDSCH for SBFD-aware UEs considering the following options:

  - Option 1: RBs outside DL subband(s) are rate-matched

    - Further study relaxing the existing rate matching restriction on the DMRS symbols

  - Option 2: new CRB-to-PRB mapping

|  | Company |
| --- | --- |
| Support | New H3C, TCL, IDC, NEC (change in Option-2),OPPO |
| Not support | Sony, Samsung |

| Company | Comments |
| --- | --- |
| Sony | I take it this is for non-contiguous frequency allocation, i.e. FDRA. If that is the case then we would also like to consider enhancement to the FDRA itself, e.g. enhancement to Type 1 FDRA. |
| DOCOMO | We think Sony’s comment is valid. |
| IDC | Support the proposal in general, and the direction with Option 1 (rate matching) seems better and sufficient. |
| NEC | We understand and agree with the intention. However, when we talk about resource allocation for PDSCH we specify frequency resources in terms of RBGs or VRBs. So, we is not intuitive that how a different mapping of CRB-to-PRB can help us alleviate this issue. Perhaps we need to consider how to new mapping for VRB to PRB. Therefore, it may be better to rephrase this option as: - Option 2: new VRB-to-PRB mapping Besides, only for FDRA type-1 contiguous resource allocation for PDSCH has this problem, and FDRA type-0 can reach non-contiguous resource allocation by RBG bitmap. |
| OPPO | Support and slightly prefer to option 1. |

Proposal 1-18

Proposed Agreement:

Study the non-contiguous resource allocation for CSI-RS for SBFD-aware UEs considering the following options:

- Option 1: Two contiguous CSI-RS resources link to one CSI report

- Option 2: Non-contiguous CSI-RS resource

  - Option 2-1: non-contiguous CSI-RS resource allocation

  - Option 2-2: contiguous CSI-RS resource allocation and non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, Samsung, DOCOMO, LG (with adding one option), IDC,NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | I take it this is about frequency resource allocation, so a minor editorial suggestion: Study the non-contiguous frequency resource allocation for CSI-RS for SBFD-aware UEs considering the following options: |
| LG | Similar with Option 2 (new CRB-to-PRB mapping) in Proposal 1-17, we think new CRB-to-PRB mapping can be considered as option 3. |
| IDC | Support to study this further. |
| NEC | We support Option-2 (specifically Option-2-2) as Option-1 may involve addressing further issues like redefining the restriction on number of CSI-RS resources that UE can measure which may ultimately would need to consider different scenarios and hence could easily get very complex from specification perspective. |

  - [Open] 3rd round discussion

Proposal 1-19

Proposed Agreement:

Study UE behaviors for SBFD aware UEs with interaction with TDD-UL-DL-ConfigDedicated.

|  | Company |
| --- | --- |
| Support | Sharp, Nokia, NSB (with update), Panasonic, Sony, TCL, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support to study, as long as the possibility of having cell-specific signalling is not excluded. In our view, UE-specific configuration could be considered as an additional, not an alternative option to cell-specific signaling. We suggest to update the proposed agreement as: Study UE behaviors for SBFD aware UEs interaction with TDD-UL-DL-ConfigDedicated, where TDD-UL-DL-ConfigDedicated may or may not configure SBFD subbands. |

Proposal 1-6a

Proposed Agreement:

An UL subband can be configured in SSB symbols.

  - FFS UE behaviors for SBFD-aware UEs in the SSB symbols.

|  | Company |
| --- | --- |
| Support | QC, Sharp, Panasonic, Sony, WILUS |
| Not support | Nokia, NSB, TCL |

| Company | Comments |
| --- | --- |
| Sharp | We think another option that UL subband can NOT be configured in SSB symbols can be also considered. On the other hand, assuming that the following two options are the same, we are fine with the proposal. - An UL subband is configured in SSB symbols and the UL subband is disabled in the SSB symbols - An UL subband is not configured in SSB symbols |
| Nokia, NSB | Not ok for us. Mostly UE may receive SSB in SSB symbols, although some companies mention that UE may not always need to receive SSB but UE need to track DL T/F and QCL operation, so we should not allow SBFD in SSB symbols. Additionally, as we wrote in our contribution, there are different aspects to be considered for the configuration of SBFD in SSB symbols, such as the fact that SSBs are being measured by other UEs in this symbol, including IDLE/INACTIVE UEs and legacy UEs, which might suffer with the increased UE-to-UE CLI in case a UL subband is configured there. Additionally, SSBs can be power boosted, which would increase the self-interference at the gNB. |
| TCL | UL subband in SSB symbols may bring collision of UL subband and SSB symbols. Since SSB is periodically transmitted and there may be some UE in a cell which requires SSB. |

Proposal 1-14

Proposed Agreement:

For SBFD-aware UEs, study the following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands:

  - Option 1: Partial RBG(s) at subband boundaries can be used for PDSCH/PUSCH

  - Option 2: Partial RBG(s) at subband boundaries cannot be used for PDSCH/PUSCH

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, Samsung, DOCOMO, LG, IDC, NEC(Option 1), QC, Sharp, Nokia, NSB (Option 1), Panasonic, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| IDC | Support the proposal in general. This proposal can be discussed after agreement is made on the Proposal 1-13. In Proposal 1-14, our preference is Option 1. In our opinion, Option 2 may result in waste of resources that overlap with the RBs at the boundaries. |
| NEC | We prefer using partial RBGs at subband boundaries to ensure that there is minimal radio resource wastage. It would be good to discuss that in which case Option-2 would be applicable |
| Nokia, NSB | We are Ok with the proposed agreement capturing the different options, but our preferred option is Option 1, since Option 2 will result in waste of resources. |

Proposal 1-15

Proposed Agreement:

For SBFD-aware UEs, study the following options for CSI reporting configuration across SBFD symbols and non-SBFD symbols:

  - Option 1: separate CSI reporting configurations for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting configurations for SBFD symbols and non-SBFD symbols

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, Samsung, DOCOMO, LG, IDC, NEC (prefer Option-1), QC, Sharp, Nokia, NSB, Panasonic, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| IDC | Support the proposal to study this aspect further. In our opinion, in order to support subband-wise CSI reporting or differential CSI reporting, at least some CSI reporting contents or behaviors should be able to be different for SBFD symbols and non-SBFD symbols, but Option 1 vs. Option 2 seem to focus CSI reporting configuration aspects, which may be under second-level details and we are open for further study. |
| NEC | Does this imply that different CSI reporting configurations based on CSI-RS measured during SBFD and non-SBFD symbols or does this proposal mean different reporting configuration based on whether CSI report is sent during SBFD or non-SBFD symbol? Given that antenna panel-based isolation mechanism could be used for SBFD operation, this could result in different number of antenna elements/panels available during DL and SBFD symbols. Hence, it may finally reflect on availability on number of CSI-RS ports or precoder matrix size based on different network implementations between SBFD and DL symbol. Therefore, it seems natural that CSI report should also be differentiated between SBFD and non-SBFD symbols. |

Proposal 1-16

Proposed Agreement:

For SBFD-aware UEs, study the following issues for PRG:

  - Partial PRG(s) at subband boundaries with size of 2 and 4

  - Wideband precoder in case of non-contiguous DL subband(s)

|  | Company |
| --- | --- |
| Support | New H3C, Sony, DOCOMO, LG, IDC,NEC, QC, Sharp, Nokia, NSB (with changes), Panasonic, TCL |
| Not support |  |

| Company | Comments |
| --- | --- |
| IDC | Support the proposal in general. More clarification is required. |
| NEC | We think the main bullet can be changed to the below description which is more clear. For SBFD-aware UEs, study the following issues for PRG of PRB bundling procedures for PDSCH: |
| Nokia, NSB | We propose to add “at least” in the main bullet, to not exclude other issues now. For SBFD-aware UEs, study at least the following issues for PRG: |

Proposal 1-9a

Proposed Agreement:

Study at least the following separate configurations for SBFD symbols and non-SBFD symbols at least for SRS and PUCCH:

  - Separate time and frequency resources

  - Separate FH parameters

  - Separate UL power control parameters

  - Separate beam configurations

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, Samsung (in principle), DOCOMO, LG, IDC,NEC, QC, Sharp, Panasonic, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | Is there a reason why SRS & PUCCH are emphasized here but not for the other channels? |
| Samsung | PUSCH should be included |
| IDC | Support the proposal in general. It seems not clear why should the SRS and PUCCH have different beam configurations in SBFD and non-SBFD symbols? |
| NEC | We should include an FFS whether this is applicable to other configured physical channels/signals like CSI-RS/CG PUSCH |
| OPPO | The precondition of separate configuration is SBFD known. So before we discuss this proposal, we’d better to clarify how to determine SBFD symbol and non-SBFD symbol, especially for flexible symbol case and dynamic SBFD indication case. |
| Sharp | We also think PUSCH should be included. |
| Nokia, NSB | Considering the transmission are for same channel/signal, where related configuration should be considered, we propose the following update: Study at least the following separate or related configuration, e.g. with offset between the two configurations on SBFD and non-SBFD, Also add It should also be studied how different for the separate or related configuration for SBFD and non-SBFD symbols. |
| Panasonic | We also think PUSCH should be included. |

Proposal 1-17

Proposed Agreement:

Study the non-contiguous resource allocation for PDSCH for SBFD-aware UEs considering the following options:

  - Option 1: RBs outside DL subband(s) are rate-matched

    - Further study relaxing the existing rate matching restriction on the DMRS symbols

  - Option 2: new CRB-to-PRB mapping

|  | Company |
| --- | --- |
| Support | New H3C, TCL, IDC, NEC (change in Option-2),OPPO, QC, Nokia, NSB, Panasonic, WILUS |
| Not support | Sony, Samsung, Sharp |

| Company | Comments |
| --- | --- |
| Sony | I take it this is for non-contiguous frequency allocation, i.e. FDRA. If that is the case then we would also like to consider enhancement to the FDRA itself, e.g. enhancement to Type 1 FDRA. |
| DOCOMO | We think Sony’s comment is valid. |
| IDC | Support the proposal in general, and the direction with Option 1 (rate matching) seems better and sufficient. |
| NEC | We understand and agree with the intention. However, when we talk about resource allocation for PDSCH we specify frequency resources in terms of RBGs or VRBs. So, we is not intuitive that how a different mapping of CRB-to-PRB can help us alleviate this issue. Perhaps we need to consider how to new mapping for VRB to PRB. Therefore, it may be better to rephrase this option as: - Option 2: new VRB-to-PRB mapping Besides, only for FDRA type-1 contiguous resource allocation for PDSCH has this problem, and FDRA type-0 can reach non-contiguous resource allocation by RBG bitmap. |
| OPPO | Support and slightly prefer to option 1. |
| Sharp | We agree with NEC that the wording should be rephrased by VRB-to-PRB mapping. Further, we have a concern about rate-matching of DMRS. In existing specification, rate-matching is specified in 7.3.1.5 of TS38.211 for PDSCH, but nothing related to rate-matching is specified in 7.4.1.1.2 of TS38.211 for PDSCH DMRS. Therefore, we think Option 1 requires large specification impact. Rather, we think new interpretation of FDRA in which a UE assumes that (V)RBs outside of DL subbands are not allocated should be a good option. Therefore, our proposal is to add Option 3: A UE assumes that (V)RBs outside of DL subbands are not allocated. |
| Nokia, NSB | We support the study. |
| Panasonic | Our preference is option 1. For option 2, we agree with NEC and Sharp that option 2 is rephrased by VRB-to-PRB mapping. Otherwise, new VRB-to-PRB mapping can be added as another option. |

Proposal 1-18a

Proposed Agreement:

Study the non-contiguous resource allocation for CSI-RS for SBFD-aware UEs considering the following options:

- Option 1: Two contiguous CSI-RS resources link to one CSI report

- Option 2: Non-contiguous CSI-RS resource

  - Option 2-1: non-contiguous CSI-RS resource allocation

  - Option 2-2: contiguous CSI-RS resource allocation and non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

- Option 3: new CRB-to-PRB mapping

|  | Company |
| --- | --- |
| Support | QC, Sharp, Nokia, NSB (with update), Panasonic |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia | In general we support the study, preferring Option 2-2, but we would like to propose the following update, to allow for other options if companies see the benefit of it: “Study the non-contiguous resource allocation for CSI-RS for SBFD-aware UEs considering at least the following options:” |
| Sony | Shouldn’t there be a option where no enhancement is required. |
| WILUS | Clarification on Option 3 is necessary. Regarding that CSI-RS resource is configured within UE’s DL BWP, does it imply enhancement on BWP operation? |

- gNB self-interference handling schemes

This section discusses gNB self-interference handling schemes for SBFD.

  - Summary of input contributions

Tx/Rx timing misalignment

BS self-interference related to timing and SCS was discussed in RAN4#104bis-e with the following agreement in R4-2217464.

## 4.1 BS self-interference related to timing and SCS Agreement: - For the BS self-interference issue related to timing and SCS of D and U for both BS and UE: - RAN4 understanding is that this issue will be studied in RAN1. - RAN4 has not evaluated the timing and SCS impacts to BS SI. RAN4 for now assumes they are negligible and do not impact RAN4 requirement work. - RAN4 will not consider this issue in the BS SI feasibility study if no request from RAN1.

This issue is discussed by many companies in previous meetings and in this meeting.

In legacy TDD system, a UE is provided a cell-specific TA offset NTA,offset. It can be provided in SIB1 and if not, UE assumes TA offset of 25600 or 39936 defined in Table 7.1.2-2 in 38.133.

The cell-specific TA offset is used to reserve enough time for UL/DL switching as illustrated below.

Figure 4-1: TA offset in legacy TDD [7]

For SBFD operation, gNB would transmit and receive simultaneously and the non-zero TA offset may cause inter-symbol/inter-slot interference as observed by Huawei, ZTE, Ericsson,…

Furthermore, CATT and Ericsson provided some evaluation results in [16] and [20] respectively. CATT observed that the required SIC capability can be reduced by 4 dB by configuring NTA,offset=0  instead of NTA,offset=25600Tc. Ericsson observed that assuming no transmit side impairments, the misalignment creates a dramatic rise in interference received in the 20 MHz UL subband between the two 40 MHz DL subbands compared to the case of no misalignment. However, a very much reduced gap is observed in the presence of Tx side impairments and the combined effect of Tx side impairments and UL subband selection filter come very close to fully masking the misalignment effect.

Huawei proposed to configure NTA, offset=0.

ZTE, Nokia, Panasonic… observed that there may be potential backward compatibility issue since legacy UE may not support NTA, offset=0 in TDD system. In addition, with NTA,offset=0, the gap for UL-to-DL switching may disappear and the gap realized based on scheduling avoidance can be larger.

Alternatively, ZTE, Nokia proposed to consider two TA offset values for SBFD-aware UEs for UL transmissions in non-SBFD symbols and SBFD symbols. There can be potential overlap between UL transmission in a SBFD symbol and next UL transmission in a non-SBFD symbol. Nokia proposed to study how to reduce the increased overhead when switching between TDD DL/UL slots and SBFD slots.

Ericsson proposed that RAN1 should model practical transmitter and receiver components to enable a realistic assessment of the effect of UL/DL timing misalignment at the gNB.

Sony and Intel discussed larger TA value to align the symbol boundary for DL and UL with different symbol index. To be specific, Sony proposed to add a time alignment offset to the overall timing advance for UL transmission in SBFD slots, where time alignment offset is selected such that the 2nd or higher OFDM symbol of the UL transmission aligns with a potential DL interferer at the gNB and also provides at least NTX-RX at the UE between the end of an UL transmission and the start of a DL reception for UL to DL switching as shown below. Intel observed that at least one symbol overhead would be generated.

Figure 4-2: Adding TUL to the overall timing advance to align the 2nd OFDM symbol with the DL interferer [19]

Power control based solution

ZTE observed that the uplink transmissions in the UL subband are subject to different interference levels due to different frequency domain isolation between the uplink transmission and the DL subband as show below and proposed that UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the different levels of inter-subband interference.

Nokia proposed that the potential benefits of boosting the UE Tx power and/or reducing the gNB power in SBFD slots/symbols shall be further investigated as a potential method to boost the UL received SINR in slots/symbols affected by gNB self-interference.

  - [Open] 1st round discussion

Proposal 2-1

Proposed Agreement:

Study impact/potential enhancements of unaligned slot/symbol boundary for DL and UL subbands in SBFD symbols.

|  | Company |
| --- | --- |
| Support | New H3C, Samsung, Sony, xiaomi, NEC, Intel, IDC, Panasonic, Huawei, Hisilicon, TCL, Lenovo, LG Electronics, ZTE, Nokia, NSB, QC |
| Not support |  |

| Company | Comments |
| --- | --- |
| NEC | We agree with the issue of unaligned boundary between DL and UL, but we think that this issue may not be resolved just by disabling TA during SBFD. For instance, if the slot structure of following type: UXD, then although by disabling TA in the second slot “X” we can align the DL and UL alignment for the same slot, but the issue is not fully resolved during transition from X to D (i.e. SBFD to DL). This is because the network will not get sufficient time gap to transition from UL in SBFD slot to DL. Hence, the issue should be properly solved considering different scenarios. |
| Intel | In our understanding, UL timing consists of Nta_offset and Nta. Even with Nta_offset >0, it is still possible to align slot/symbol boundary for DL and UL subbands in SBFD symbols by proper value of Nta. Thus, the study does not only include enhancement but also include the impact if we just reuse existing mechanism to achieve aligned boundary. |
| LG Electronics | We are fine with the study. For eliminate the self-interference, the symbol boundary alignment between DL and UL would be beneficial. But, it seems necessary to apply separate TA depending on symbol types (i.e., SBFD and non-SBFD) |
| ZTE | While we are discussing this issue under the gNB self-interference section, this impact is not limited to gNB self-interference but also inter-UE inter-subband CLI and inter-gNB inter-subband CLI. Therefore, we recommend adding the following note. Note: gNB self-interference, inter-UE inter-subband CLI and inter-gNB inter-subband CLI should be taken into account. |
| Nokia, NSB | We support the proposed study. |
| QC | Minor clarification, the study is not limited to resetting the TA-offset in SBFD symbols? |

- Inter-gNB and inter-UE CLI handling schemes

The guidance from Mr. Chair on discussions of CLI handling in AI 9.3.2 and AI 9.3.3 is as follows. Accordingly, this section discusses the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

Guideline for future meetings - Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. - Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

In addition, according to the guidance from Mr. Chair, L1/L2 based CLI measurement/report for UE-to-UE CLI handling is to be discussed in AI 9.3.3 and exchange of intended subband configurations for SBFD operation across gNBs is to be handled in AI 9.3.2.

  - Summary of input contributions

The inputs from companies’ contributions on inter-gNB and inter-UE CLI handling schemes are summarized below as per moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or missed and encourage companies to correct/update the summary with revision marks if needed.

    - UE-to-UE CLI handling

Potential enhancements to UE-to-UE CLI measurement/reporting

The following agreements were made in RAN1#110.

Agreement (RAN1#110) Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

Non-contiguous CLI-RSSI measurement/report

The following agreement was made in RAN1#110bis-e.

Agreement Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

For non-contiguous CLI-RSSI measurement resource in frequency, gNB can configure separate CLI measurement resources/reports in two DL subbands according to existing specification.

Huawei, Ericsson, Intel, Qualcomm discussed potential enhancements on non-contiguous CLI-RSSI measurement resource configuration/report in frequency. In general, the solutions are similar as those for non-contiguous CSI-RS resource configuration, including:

- Option 1: Two contiguous CLI-RSSI resources configured to be linked to one CLI-RSSI report

- Option 2: Non-contiguous CLI-RSSI resource

  - Option 2-1: non-contiguous CLI-RSSI resource allocation

  - Option 2-2: contiguous CLI-RSSI resource allocation and non-contiguous CLI-RSSI resource derived by excluding frequency resources of UL subband and guardband(s)

CATT think that if the CLI in two DL subbands is considered to be non-symmetric, separate CLI-RSSI measurement resources/reports can be configured in different DL subbands according to existing Rel-16 CLI-RSSI mechanism; otherwise if the CLI in two DL subbands is considered to be symmetric, i.e. CLI in RBs in two DL subbands with the same frequency separation from UL subband is the same, it seems sufficient to measure CLI in only one DL subband. So it is proposed to further discuss the motivation and potential benefits for enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

LG think that the measured strength of CLI may be different depending on the position of DL. But, if the UE-to-UE CLI-RSSI measurement/report is taken non-contiguous measurement resource in frequency into account, the measured strength of CLI is averaged over the non-contiguous measurement resource. Hence, it is hard to determine which part of DL subband is suffered from intra-cell inter-UE CLI. So it is proposed that the use cases and potential benefits for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency should be justified.

CLI measurement in UL subband

ZTE, CMCC considered inter-subband CLI measurement in UL subband, e.g. to identify the aggressor.

Intel discussed in [15] that Rel-16 CLI measurement mechanism supports CLI measurement in UL subband as long as the frequency resources are within DL active BWP, i.e. the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband.

Huawei thinks the feasibility of DL measurement in UL subband in SBFD symbols has not confirmed (there is no agreement yet to support DL reception within an UL subband). More importantly, the usage of such measurement should be clarified especially the benefit if any compared to the Rel-16 SRS-RSRP measurement mechanism.

CLI measurement and reporting based on a finer granularity

CLI measurement and reporting based on a finer granularity are considered by ZTE, Sony, Qualcomm, DOCOMO, considering the CLI is not frequency flat. But it is also pointed out that existing configuration for L3 CLI-RSSI measurement resource already supports multiple resources in different frequency regions, which can be directly reused [22].

Information exchange between gNBs

Huawei thinks for UE-to-UE co-channel CLI measurement, at least the configuration of measurement sig-nals/channels should be exchanged between aggressor gNBs and victim gNBs. In addition, the CLI measurement reports can also be exchanged between aggressor gNBs and victim gNBs. Whether the information exchange needs to be specified can be further discussed.

Similarly, Qualcomm proposed that gNB to exchange information of the CLI resource configurations and/or CLI measurements.

Timing alignment aspects

Nokia proposed to study schemes for measurement and reporting of time difference of arrival between DL RS and UL RS, and potential extensions to the CLI measurement framework to include assistance information consisting of e.g. specific timing offset (with respect to DL timing) to be used when performing a CLI measurement.

Power control based solution

ZTE proposed that UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the inter-subband interference with different levels. As an example shown below, UL subband is divided into three areas in the frequency domain. A higher transmission power can be used for uplink transmission in Area 1 as it is far away from DL subband and a lower interference can be expected. For Area 2 and Area 3, a lower transmission power can be used for uplink transmission in them as they are closed to DL subband.

Figure 5-2: Different allowed maximum power for UL transmission in different areas [13]

Others

ZTE proposed that different frequency densities can be configured for reference signals transmitted in different areas with different interference levels.

InterDigital proposed to consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario. In addition, it is proposed to study measurement resources and reporting configurations for subband-edge CLI measurement and to study a conditional CLI handling behavior based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

    - gNB-to-gNB CLI handling

CMCC proposed to study the following two methods for inter-gNB inter-subband CLI measurement.

Figure 5-3: Inter-gNB inter-subband CLI measurement method#1 & method#2 [19]

In addition, CMCC proposed to support inter-gNB coordination in time-domain, frequency-domain, spatial-domain, and power domain and backhaul signalling enhancement is needed to support inter-vendor cooperation.

- Method#1: victim gNB measures leakage power from aggressor gNB within UL subband;

- Method#2: victim gNB measures RSRP of aggressor gNB’s RS within DL subband.

Furthermore, it is proposed that inter-gNB transmission coordination in orthogonal time-domain, frequency-domain or spatial-domain resources can be supported to identify the strongest aggressor gNB in inter-gNB inter-subband CLI measurement Method#1.

Resource muting

Huawei proposed to study the following aspects about resources muting for gNB-gNB co-channel CLI measurement.

- Feasibility and performance of muting the REs on the DL subband in UL DMRS symbols and the REs on the UL subband in DL DMRS to improve channel estimation and inter-cell interference estimation and suppression.

- Feasibility and performance of specific CLI measurement resources to improve gNB-gNB and UE-UE co-channel and adjacent-channel inter-subband CLI estimation and suppression.

Figure 5-4: Resource muting for gNB-gNB co-channel CLI measurement [7]

Beam nulling between gNBs

Huawei proposed to study the feasibility and performance of beam nulling for gNB-to-gNB CLI suppression with gNB-to-gNB channel, e.g. if the channel between transmitter and receiver can be measured, the DL beamforming weights can be manipulated to avoid transmitting in the direction of the receiver by using coordinated beamforming (CBF).

Figure 5-5: CBF to suppress blocking interference [7]

Interference suppression based on analogue filter

Huawei proposed to study the feasibility and performance of applying filters at both transmitter and receiver sides in SBFD involving RAN4. It is noted that Huawei thinks that the study should be started by RAN4 and the performances should be provided to RAN1 to check the feasibility as well as the performance of SBFD.

Information exchange between gNBs

Spreadtrum proposed to consider subband-level information for gNB-to-gNB’s information exchange. Qualcomm proposed to support subband-based inter-gNB CLI reporting for accurate measurement of CLI leakage in SBFD.

ZTE, Intel, Panasonic, Qualcomm, DOCOMO and Nokia proposed to exchange intended subband configurations for SBFD operation across gNBs.

Vivo proposed to study exchange of subband related configuration and/or information among gNBs, including configured/intended subband settings, interference information, recommended/restricted beam(s), as well as information about resource allocation in frequency domain.

Reference signals with different frequency densities

Similar as ZTE’s proposal for UE-to-UE CLI, ZTE proposed that different frequency densities can be configured for reference signals (e.g., RS for gNB-to-gNB CLI measurement, RS for uplink transmission channel estimation) transmitted in different areas with different interference levels.

  - [Open] 1st round discussion

Proposal 3-1

Proposed Conclusion:

For inter-UE inter-subband CLI measurement, the following two methods are identified:

- Method#1: victim UE measures RSSI/SINR within DL subband;

- Method#2: victim UE measures RSRP of aggressor UE’s CLI-SRS within UL subband.

Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband.

|  | Company |
| --- | --- |
| Yes | New H3C, Samsung, Sony, Intel, IDC, Panasonic, TCL, Lenovo, ZTE, Nokia, NSB, QC |
| No |  |

| Company | Comments |
| --- | --- |
| Xiaomi | We are not clear on method#2. In UL subband, SBFD aware UE performs UL transmission, there is no UE-UE CLI from the other UEs. Could proponents clarify a bit? |
| Huawei, HiSilicon | For Method#2, our understanding is that for a UE to perform such measurement, the UE must be able to receive DL in an UL subband especially when the aggressor UE and the victim UE are in the same cell. Note that this is a bit contradicting to proposal 1-5 “The SBFD aware UE does not transmit UL outside UL subband or receive DL outside DL subband(s).” In addition, this is essentially same to configure the UE with a flexible subband. |
| Nokia, NSB | We agree that these methods are necessary. Study of RSSI measurements within DL subbands should not preclude the case in which the UE measures RSSI/SINR in the DL subband(s) while transmitting in the UL subband. |

Proposal 3-2

Study exchange of intended subband configurations for SBFD operation across gNBs for coordinated scheduling for time/frequency resources between gNBs.

|  | Company |
| --- | --- |
| Support | New H3C, Samsung, DOCOMO, Sony, xiaomi, NEC, Intel, IDC, Panasonic, TCL, Lenovo, ZTE, Nokia, NSB, QC,OPPO |
| Not support | Huawei, Hisilicon |

| Company | Comments |
| --- | --- |
| NEC | It would be essential to support this enhancement to mitigate interference successfully between gNBs. We note that when all gNBs belong to the same operator then such signaling can be accomplished using OAM signaling however, the same mechanism may not work well for multiple vendor gNB deployment. For such a case, it would be essential to have 3GPP signaling framework. Note that we already have the functionality of sharing TDD configuration between gNBs for CLI mitigation and further inter-gNB coordination techniques are being considered in 9.3.3, this proposal is an extension to the given feature. |
| Huawei, HiSilicon | This proposal has been discussed for several meetings. It is still unclear to us what the intended subband configuration for SBFD operation actually means. Is it SBFD pattern rather than SBFD configuration? If so, we may need first decide that SBFD symbols can be dynamically changed to non-SBFD symbols as discussed in 1-5. Even so, it is unclear how SBFD pattern can help coordinated scheduling. |
| Nokia, NSB | We support the proposed study. Our understanding is that this is a simple extension of the exchange of information on the intended TDD frame structure agreed for dynamic TDD in Rel-16. |
| QC | This is already agreed in AI 9.3.3 |

Question 3-3

Do you think BWP-based SBFD operation can potentially reduce inter-UE CLI?

| Company | Comments |
| --- | --- |
| Moderator | KDDI thinks that the BWP pair for SBFD slot can mitigate inter-UE CLI impact by configuring the DL BWP size to match one of the DL subbands and therefore propose to study BWP-based SBFD operation. Companies are welcome to provide your views. |
| New H3C | We already have below agreement : For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair In this case , firts of f all, we need further study benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair and then consider BWP-based SBFD operation for inter-UE CLI. |
| QC | We support to study the benefits of SBFD operation when UE have the capability of supporting multiple BWPs. It has two advantages. First from specification impact compared to SBFD within BWP, SBFD-aware UE behavior is well defined as the UL-SB is configured as UL-BWP and all signals/channels are defined within the BWP. Secondly, from CLI perspectives, it brings improved CLI mitigation as brought up by KDDI as UE filters the CLI in the UL_SB before FFT operation of the DL signal. |

- Offline discussion

  - Feb. 27th (Mon)

Proposal 1-5a

Proposed Agreement:

For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in the symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

  - Option 1: semi-statically configured UL/DL subbands can be dynamically disabled in the symbol/slot.

    - If semi-statically configured UL/DL subbands are disabled in a symbol/slot, there are no UL/DL subbands in the symbol/slot and the symbol/slot is not an SBFD symbol/slot but a DL symbol/slot

  - Option 2: dynamic DL receptions outside semi-statically configured DL subband(s) are allowed by scheduling DCI.

For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options

  - Option 1: semi-statically configured UL/DL subbands can be dynamically disabled in the symbol/slot.

    - If semi-statically configured UL/DL subbands are disabled in a symbol/slot, there are no UL/DL subbands in the symbol/slot and the symbol/slot is not an SBFD symbol/slot but a flexible symbol/slot

  - Option 2: semi-statically configured UL/DL subbands can be dynamically disabled in the symbol/slot.

    - If semi-statically configured UL/DL subbands are disabled in a symbol/slot, there are no UL/DL subbands in the symbol/slot and the symbol/slot is not an SBFD symbol/slot but a DL symbol/slot

  - Option 3: dynamic DL receptions outside semi-statically configured DL subband(s) and dynamic UL receptions outside semi-statically configured UL subband(s) are allowed by scheduling DCI.

Proposal 1-8

Proposed Agreement:

Consider the following options for potential enhancements on frequency domain resource allocation for SBFD-aware UEs:

- Option 1: Single FDRA for SBFD symbols and non-SBFD symbols

  - Option 1-1: A transmission/reception with frequency resources outside UL subband/DL subband(s) is dropped.

  - Option 1-2: Frequency resources of a transmission/reception in SBFD symbols are derived by excluding RBs outside UL subband/DL subband(s).

  - Option 1-3: FDRA is valid only in SBFD or non-SBFD symbols, i.e. invalid in non-SBFD or SBFD symbols

  - Option 1-4: Separate frequency resources are derived for non-SBFD symbols and SBFD symbols based on DL/UL BWP and DL/UL subband(s) in non-SBFD symbols respectively

- Option 2: Separate FDRA for SBFD symbols and non-SBFD symbols

Note: Different options can be applied to different channels/signals and different options can be applied to a channel/signal e.g. for transmissions/receptions with repetition and without repetitions.

Proposal 1-1

Proposed Agreement:

Study whether a slot can consist of both SBFD and non-SBFD symbols.

Proposal 1-4

Proposed Conclusion:

Dynamic indication of frequency location/size of SBFD subbands is not considered.

Proposal 1-7

Proposed Agreement:

UE does not expect to transmit a PUSCH/PUCCH/SRS or receive a PDSCH/PDCCH/CSI-RS overlapping with both SBFD and non-SBFD symbols in the same slot.

  - Feb. 28th (Tue)

Proposal 1-12

Proposed Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, study the following options for SBFD-aware UEs:

  - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

  - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

  - PDSCH/PUSCH/PUCCH repetitions

  - SPS PDSCH/CG PUSCH

  - TBoMS

  - Multi-PUSCH/PDSCH scheduled by a single DCI

  - Periodic SRS/CSI-RS/PUCCH

Proposal 1-18

Proposed Agreement:

Study the non-contiguous frequency resource allocation for CSI-RS for SBFD-aware UEs considering the following options:

- Option 1: Two contiguous CSI-RS resources link to one CSI report

- Option 2: Non-contiguous CSI-RS resource

  - Option 2-1: non-contiguous CSI-RS resource allocation

  - Option 2-2: contiguous CSI-RS resource allocation and non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Proposal 1-14

Proposed Agreement:

For SBFD-aware UEs, study the following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands:

  - Option 1: Partial RBG(s) at subband boundaries can be used for PDSCH/PUSCH

  - Option 2: Partial RBG(s) at subband boundaries cannot be used for PDSCH/PUSCH

Proposal 1-13

Proposed Agreement:

For RBG for PDSCH/PUSCH RA type 0, study the following options for RBG size determination for SBFD-aware UEs:

  - Option 1: RBG size is determined based on size of DL/UL BWP

  - Option 2: RBG size is determined based on size of DL/UL subband(s)

Proposal 1-15

Proposed Agreement:

For SBFD-aware UEs, study the following options for CSI reporting configuration across SBFD symbols and non-SBFD symbols:

  - Option 1: separate CSI reporting configurations for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting configurations for SBFD symbols and non-SBFD symbols

Proposal 1-9a

Proposed Agreement:

Study at least the following separate configurations for SBFD symbols and non-SBFD symbols at least for SRS and PUCCH:

  - Separate time and frequency resources

  - Separate FH parameters

  - Separate UL power control parameters

  - Separate beam configurations

  - Mar. 1st (Wed)

Proposal 1-19

Proposed Agreement:

Study UE behaviors for SBFD aware UEs with interaction with TDD-UL-DL-ConfigDedicated.

Proposal 1-14

Proposed Agreement:

For SBFD-aware UEs, study the following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands:

  - Option 1: Partial RBG(s) at subband boundaries can be used for PDSCH/PUSCH

  - Option 2: Partial RBG(s) at subband boundaries cannot be used for PDSCH/PUSCH

Proposal 1-15

Proposed Agreement:

For SBFD-aware UEs, study the following options for CSI reporting configuration across SBFD symbols and non-SBFD symbols:

  - Option 1: separate CSI reporting configurations for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting configurations for SBFD symbols and non-SBFD symbols

Proposal 1-16

Proposed Agreement:

For SBFD-aware UEs, study the following issues for PRG:

  - Partial PRG(s) at subband boundaries with size of 2 and 4

  - Wideband precoder in case of non-contiguous DL subband(s)

Proposal 1-9a

Proposed Agreement:

Study at least the following separate configurations for SBFD symbols and non-SBFD symbols at least for SRS and PUCCH:

  - Separate time and frequency resources

  - Separate FH parameters

  - Separate UL power control parameters

  - Separate beam configurations

Proposal 1-18a

Proposed Agreement:

Study the non-contiguous resource allocation for CSI-RS for SBFD-aware UEs considering the following options:

- Option 1: Two contiguous CSI-RS resources link to one CSI report

- Option 2: Non-contiguous CSI-RS resource

  - Option 2-1: non-contiguous CSI-RS resource allocation

  - Option 2-2: contiguous CSI-RS resource allocation and non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

- Option 3: new CRB-to-PRB mapping

- Contact person

Please provide/update the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| Sony | Shin Horng Wong | shinhorng.wong@sony.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| Qualcomm | Muhammad Abdelghaffar | mabdelgh@qti.qualcomm.com |
| New H3C | Lei Zhou | zhou.leih@h3c.com |
| New H3C | Lei Kong | Kong.lei@h3c.com |
| vivo | Lihui Wang | wanglihui@vivo.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Xiaomi | Lei Wang | wanglei25@xiaomi.com |
| OPPO | Wenfeng Zhang | zhangwenfeng@oppo.com |
| Ericsson | Stephen Grant | stephen.grant@ericsson.com |
| Spreadtrum | Huan Zhou | Huan.Zhou@unisoc.com |
| CATT | Yanping Xing | xingyanping@catt.cn |
| Panasonic | Hidetoshi Suzuki Tomoya Nunome Quan Kuang | suzuki.hidetoshi@jp.panasonic.com nunome.tomoya@jp.panasonic.com Quan.Kuang@eu.panasonic.com |
| Intel | Yi Wang | yi5.wang@Intel.com |
| ITRI | Jen-Hsien Chen | itriA40175@itri.org.tw |
| Lenovo | Hyejung Jung Vijay Nangia | hyejung@motorola.com vnangia@motorola.com |
| ETRI | Hoondong Noh | hoondong.noh@etri.re.kr |
| ZTE | Xingguang WEI | wei.xingguang@zte.com.cn |
| Samsung | Marian Rudolf Kyungjun Choi | m.rudolf@partner.samsung.com kyungj.choi@samsung.com |
| CMCC | Tuo Yang Fei Wang | yangtuo@chinamobile.com wangfei@chinamobile.com |
| DOCOMO | Qiping Pi | piqp@docomolabs-beijing.com.cn |
| WILUS | David (Geunyoung) Seok | david.seok@wilusgroup.com |
| CEWiT | Priyanka Dey | priyanka@cewit.org.in |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| MediaTek | Mohammed Al-Imari | Mohammed.Al-Imari@mediatek.com |
| LG Electronics | Hyunsoo Ko | hyunsoo.ko@lge.com |
| SK Telecom | Sanghoon Cho | seanc.cho@sk.com |
| KDDI | Masahito Umehara | ma-umehara@kddi.com |
| TCL | Shahid Jan | shahid.jan@tcl.com |
| Fujitsu | Taewoo LEE | lee.taewoo@fujitsu.com |
| Apple | Ali Fakoorian | sfakoorian@apple.com |

- Reference

- RP-220633		Revised SID: Study on evolution of NR duplex operation	CMCC

- R1-2205399		TR 38.858 skeleton for study on evolution of NR duplex operation	Rapporteur (CMCC)

- R1-2205520		Summary #2 of [109-e-R18-Duplex-03] Email discussion on subband non-overlapping full duplex	Moderator (CATT)	RAN1#109-e

- R1-2208122		Summary #5 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#110

- R1-2210317		Summary #4 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#110bis-e

- R1-2212735		Summary #3 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#111

- R1-2300087		Discussion on potential enhancement on subband non-overlapping full duplex	Huawei, HiSilicon

- R1-2300150		Discussion for subband non-overlapping full duplex	New H3C Technologies Co., Ltd.

- R1-2300167		Discussion on subband non-overlapping full duplex	TCL Communication Ltd.

- R1-2300217		Discussion on subband non-overlapping full duplex	Spreadtrum Communications

- R1-2300287		Discussion on subband non-overlapping full duplex	OPPO

- R1-2300331		Discussion on subband non-overlapping full duplex operation	InterDigital, Inc.

- R1-2300340		Discussion of subband non-overlapping full duplex	ZTE

- R1-2300451		Discussion on subband non-overlapping full duplex	vivo

- R1-2300574		Discussion on subband non-overlapping full duplex	xiaomi

- R1-2300678		Discussion on subband non-overlapping full duplex	CATT

- R1-2300750		Discussion on subband non-overlapping full duplex	Fujitsu

- R1-2300763		Discussion on subband non-overlapping full duplex	NEC

- R1-2300873		Considerations on Subband Full Duplex TDD operations	Sony

- R1-2300908		Subband non-overlapping full duplex	Ericsson

- R1-2300918		Discussion on subband non-overlapping full duplex	Panasonic

- R1-2300946		On SBFD in NR systems	Intel Corporation

- R1-2301000		Discussion on subband non-overlapping full duplex	CMCC

- R1-2301044		Discussion on subband non-overlapping full duplex enhancements	ETRI

- R1-2301064		Study on Subband non-overlapping full duplex	LG Electronics

- R1-2301205		Subband non-overlapping full duplex	Lenovo

- R1-2301262		SBFD feasibility and design considerations for NR duplex evolution	Samsung

- R1-2301301		Discussion on subband non-overlapping full duplex	Sharp

- R1-2301344		Views on subband non-overlapping full duplex	Apple

- R1-2301411		Feasibility and techniques for Subband non-overlapping full duplex	Qualcomm Incorporated

- R1-2301491		Discussion on subband non-overlapping full duplex	NTT DOCOMO, INC.

- R1-2301570		On subband non-overlapping full duplex for NR	Nokia, Nokia Shanghai Bell

- R1-2301594		Discussion on subband non-overlapping full duplex for NR	MediaTek Inc.

- R1-2301639		Discussion on subband non-overlapping full duplex	KDDI Corporation

- R1-2301650		Details of subband non-overlapping full duplex	ASUSTeK

- R1-2301692		Discussion on subband non-overlapping full duplex	CEWiT, Reliance Jio

- R1-2301732		Discussion on subband non-overlapping full duplex	WILUS Inc.

- R1-2301745		Discussion on subband non-overlapping full duplex	Indian Institute of Tech (M)

Appendix: Previous agreements of SBFD

RAN1#109-e

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

At least study SBFD operation within a TDD carrier

Conclusion

For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Guideline for future meetings

- Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD.

- Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

RAN1#110

Agreement

Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state.

- SBFD operation Alt 1:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

- SBFD operation Alt 2:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

UE capability discussion is held in work item phase.

Agreement

For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

Agreement

For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline.

Working Assumption

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Agreement

For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes:

- Measurement resource/reporting configuration

- Measurement/reporting details (including UE processing delay)

- Relevant information exchange (between gNBs) if needed

- Usage of measurement at gNB

Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

RAN1#110bis-e

Agreement

For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline.

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Agreement

For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required.

- FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Agreement

Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Agreement

Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE

- If there are, whether/how to avoid/handle such collision cases (as second step)

Agreement

Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

Agreement

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

Agreement

The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier

Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible.

Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance.

Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above.

LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671.

Agreement

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

RAN1#111

Agreement

For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

  - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

- DL receptions within DL subband(s) are allowed in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

Agreement

For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

Agreement

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols

- Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

- Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

Examples of potential enhancements include:

- Resource allocation in frequency domain including frequency hopping

- Resource allocation in time domain

- Power domain

- Spatial domain

FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Agreement

For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs,

Option 1:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Option 2:

- UL transmissions within UL subband are allowed in the symbol

- The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

  - FFS: SBFD aware UE behaviours

  - FFS: Whether or not signalling of guardband(s) is needed

- FFS: Whether or not the symbol can be converted to a DL-only symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

Agreement

Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following:

- RBG for PDSCH RA type 0

- CSI reporting configuration

- CSI-RS resource configuration

- PRG of PDSCH