3GPP TSG RAN WG1#112			R1-2301944

Meeting, February 27th – March 3rd, 2023

Agenda item:	9.1.2

Source: 	Samsung

Title: 	Views on CSI enhancements

Document for:	Discussion and Decision

# Introduction

This contribution provides Samsung’s view on the key issues related to Rel.18 CSI enhancements that are prioritized for RAN1#112 (as announced by the FL). It also discusses other issues that are relevant for further discussions.

# Type II codebook refinement for coherent-JT

## Key issues

    - Issue 1 (W2 quantization, working assumption)

[2] Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Regarding the working assumption on Alt3 for amplitude grouping, the main argument from the proponents of Alt3 is the power imbalance across TRPs in case of inter-site scenarios with large ISD (e.g. 500m), which may require reporting of this power imbalance via different amplitude groups, one per TRP (2N groups in total). In our view, since the SCI is across TRPs, coefficients associated with different TRPs need to be normalized with respect to the strongest coefficient across all TRPs, implying that the W2 coefficients after normalization in some sense shall already capture the power level of different TRPs. This is regardless of the scenarios or ISD values. We verify this via simulation results provided in Section 2.2, wherein we show that there is no gain with Alt3 over Alt1 in inter-site scenarios with small (200m) as well as large (500m) ISDs. We therefore propose to revert the working assumption.

Observation 1: due to one SCI across TRPs, the W2 coefficients for different TRPs (after normalization with the strongest coefficient) capture the different power level across TRPs, implying that there is no need for per-TRP per-polarization reference amplitude reporting (i.e. Alt3)

Proposal 1: Revert the working assumption on the additional support for Alt3 for the amplitude grouping

- Note that the working assumption was included with the understanding that the gain of Alt3 over Alt1 (already agreed to be supported) can be demonstrated – not simply that “as long as Alt3 is not broken, the WA should be confirmed as agreement.”

    - Issue 2 (mode-1: Alt1 vs 2 vs 3)

[1] Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources Offline proposal 1.B.1: On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

We support Alt1. For mode1, independent per-TRP FD basis reporting can be w.r.t. a reference FD basis (of a reference TRP). The reporting of a relative FD basis offset for each TRP ϕn (except the reference, which can be set to 0) can help align FD basis (or the delay profile) of different TRPs. Furthermore, the relative FD basis Wf (hence delay profile) after cyclic shift or alignment can be similar. The main advantage of Alt1 over alternatives (Alt2/3) is that it can unify the functionality of mode 1 and mode 2 (which is a design principle included in the agreement on the support of mode 1 and mode 2), reduce overhead (when compared with Alt2 and Alt3), and reduce UE computational complexity (due to one vs per-TRP Wf). Simulation results comparing these alternatives are provided in Section 2.2. The results demonstrate that there is no clear performance gap between Alt1/2/3.

Observation 2: on FD basis reporting for mode-1, when compared with Alt 2/3, Alt1 can

- unify the functionality of mode 1 and mode 2 (per agreement on support of mode1/2),

- achieve similar performance with lower overhead of FD basis reporting as well as lower complexity of FD basis calculation

Proposal 2: support Alt1 of the offline proposal 1.B.1 for mode-1 FD basis reporting

    - Issue 3 (parameter combination)

[1] Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter , introduce as a candidate value  = 1/8 in addition to the supported value(s) from the legacy specification. - FFS (by RAN1#111): whether additional value 1 can also be added [1] Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter pv, in addition to the supported value(s) from the legacy specification for Rel-16 regular eType-II codebook, introduce as a candidate value - pv = 1/8 for v=1,2 (hence 1/16 for v=3,4) - FFS (by RAN1#111): whether additional value pv = 1/2 for v=1,2,3,4 can also be added [1] Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter pv, support the additional value of pv=1/2 for v=1,2,3,4 with the following condition: Only to be used in combination with other parameter value(s) to limit the increase in PMI overhead comparable to the maximum overhead of the legacy Rel-16/17 Type-II codebooks (exact parameter combination(s) FFS) [1] Agreement On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling - When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations - NL =1 is one of the supported candidate values - FFS: Other supported value(s) of NL, and its respective UE capability - FFS: The supported combinations of values for {L1, ..., LNTRP} - Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of PCSI-RS/2Ln codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2. - The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and - for Rel-17-based refinement, the gNB configures a set of N_L combinations for {alpha1, ..., alphaNTRP} Ln=αnPCSI-RS/2 where αn=12,34, 1 FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints. Offline conclusion 1.C.1: On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) codebook parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv,} combinations - FFS: Whether/how the list of supported {Ln} combinations can be linked with the list of supported {pv,} combinations without introducing a new (not previously agreed) codebook parameter, e.g. via some UE capability

We support the offline conclusion 1.C.1 as it reflects the discussion thus far, especially during the agreement in RAN1#111 in which we agreed that new parameters such as Ltot or Lmax are not to be introduced. In addition, such parameters are redundant as they can be derived from the configured NL combinations. We are open to discuss such parameters in UE feature discussions.

Regarding the parameter combinations, we support two separate tables, one for {Ln} combinations and another for (pυ,β), an example is shown in Figure 1. The two-table setup allows reusing legacy table for (pυ,β) as much as possible and can simplify the specification effort. Regarding the first table, we don’t need to support all possible combinations of {L1,…,LNTRP} since some combinations can be similar (hence redundant) to or worse than other combinations in UPT vs overhead trade-off, as shown later in Section 2.2. So, we can limit the number of supported combinations. A set of (NL≥1) combinations of values for {L1,…,LNTRP} can then be configured from this restricted table. Likewise, for the second table, the number of supported combinations is limited up to 8 (similar to legacy). This can also be based on the UPT vs overhead trade-off, just like the approach adopted during legacy parameter combination discussions. Finally, the configuration from these tables should be via separate RRC parameters.

Figure 1

Since the number of candidates for {Ln} combinations and (pυ,β) are large, a fully exhaustive search to determine the two tables seems challenging, especially when results are provided by multiple companies. We therefore propose a simplified approach as described in Table 1.

Table 1

Metric: according to the agreed EVM, i.e., UPT vs overhead trade-off (just like Rel.16/17) Criterion: similar to Rel. 16 (R1-1908499) - The overhead regime of interest <= max overhead of Rel.16/17 (~1000 bits) - pv, β values and {Ln} combinations performing worse or incurring very high overhead excluded. Candidate values - pv, β: max 8 combinations (similar to legacy) - Candidate pv values: 1/8, 1/4, 1/2 - Candidate β values: 1/8, 1/4, 1/2, 3/4 - {Ln}: minimize the number of combinations as much as possible - Candidate Ln values: {2,4,6} - Ln=6: - Optional in Rel.16 due to complexity (e.g. large size SVD) - For >1 TRPs, the complexity is even more - Ln=6 can incur very high overhead - Therefore, Ln=6 can be excluded from the candidate list To reduce simulation efforts, two-step approach - Step 1: for each {Ln} combination, select up to best 8 pairs pv, β based on the criterion - Step 2: - For each candidate pv, β, count the number of {Ln} combinations that include pv, β as one of the 8 pairs from Step 1 - Select 8 pairs pv, β that have the largest counts from the previous step - From the selected 8 pairs pv, β, prune out (exclude) redundant {Ln} combinations based on the criterion

Observation 3:

- a new parameter Ltot or Lmax is redundant

- two separate table allows reusing legacy table for (pυ,β) and can simplify the specification

- a fully exhaustive search across all possible candidate parameter values is challenging, and a simplified approach is preferable for each of the discussions

Proposal 3: regarding the parameter combinations for CJT codebook

- support the offline conclusion 1.C.1

- support two separate tables for supported {Ln} and supported pv, β, respectively

  - separate RRC configurations of NL≥1 {Ln} combinations and pv, β

  - two tables are determined based on a simplified approach (e.g. Table 1)

    - a metric (UPT vs overhead trade-off),

    - a criterion similar to legacy, and

    - a simplified approach for ease of discussion and simulation efforts

    - Issue 4 (UCI parameter design)

Based on the legacy framework, we can list up UCI parameters for part 1 and part 2 as follows:

- Part 1

  - KNZ: total number of non-zero coefficients across all layers and CSI-RS resources

  - CSI-RS resource selection bitmap: NTRP-bit bitmap (absent if N=NTRP is configured)

  - Indication of {L1,…,LNTRP} selection: log2NL bit indicator (absent if NL=1 is configured)

  - WB/SB CQIs

- Part 2

  - N bitmap per layer: 2MvLn for each CSI-RS resource n

  - SCI: for RI=1, log2KNZ-bit indicator. For RI>1, log22nLn-bit indicator

  - SD basis vector selection indicator: log2N1N2Ln-bit indicator for each CSI-RS resource n

  - FD basis vector selection indicator: for Mode 2, same as legacy. For Mode 1, can be different based on the outcome of Alt1/2/3 of issue 2.

  - Phase coefficient indicator: 4(KNZ-v) bits

  - Amplitude coefficient indicator: one reference amplitude (4-bit) + 3(KNZ-v) bits

  - SD oversampling indicator per CSI-RS resource or common across CSI-RS resources

Proposal 4: follow the legacy framework to divide UCI parameters into CSI part 1 and CSI part 2 for Rel-18 CJT

    - Issue 5 (CBSR)

Our initial view on CBSR is to consider a natural extension of the legacy framework with adding CSI-RS resources. The followings can be a starting point.

- Rank restriction should be TRP-common.

- Beam and amplitude restrictions can be TRP-specific.

Proposal 5: For CBSR, consider an extension of the legacy framework with adding CSI-RS resources, and a joint rank restriction across CSI-RS resources

    - Issue 6 (CSI omission)

Our initial view on CSI (UCI) omission is to consider a natural extension of the legacy framework with adding CSI-RS resources. Regarding G0, G1, and G2 grouping, reusing or extending the legacy framework can be considered.

In the legacy framework, UCI omission priority rule is in order of layerSDFD. For CJT, TRP (CSI-RS resource) can be further considered for UCI omission priority rule in addition to layer, SD, and FD.

For the case where UCI omission is needed, we prefer to have TRP index (CSI-RS resource index) to be the highest priority in the omission rule, i.e., TRPlayerSDFD, since CJT is more beneficial in the joint-SVD operation for mTRP channels, wherein the UE determines and reports the CSI using the composite channel of mTRP.

Proposal 6: For CSI omission, consider an extension of the legacy framework with adding CSI-RS resources, a priority order can be CSI-RS resource  layer  SD  FD.

## Simulation results

We provide system-level simulation (SLS) results on (1) performance comparison for all possible combinations of {Ln} and (pv,β), (2) performance comparison for two alternatives on FD basis selection offset for Mode 1, and Mode 2, and (3) performance comparison for different reference grouping methods, i.e., Alt1, and Alt3, of Issue 1, in inter-site inter-cell scenarios with ISD=500m and 200m. The simulation assumptions are summarized in Appendix A.

Evaluation 1: performance comparison for all possible combinations of {Ln} and (pv,β)

Simulation setup:

We performed SLS simulations to compare Mode 2 performance for all possible pairs of (pv,β) for each {Ln,n=1,⋯,NTRP} combination for each case of NTRP=2,3,4, where Ln∈{2,4} and pv∈18,14,12 and β∈18,14,12,34. Here, pv=x implies pv=x for v∈1,2 and pv=x2 for v∈3,4. We considered both of the scenarios where UE is allowed to select dynamic rank up to 2 and up to 4.

For NTRP=2,3,

- We considered intra-site inter-cell scenario.

For NTRP=4,

- We considered intra-site intra-cell scenario.

Figure 2: Average UPT gain vs overhead w.r.t. {Ln} and (pv, beta), when NTRP=3 and dynamic rank=1,2

Figure 3: Average UPT gain vs overhead w.r.t. {Ln} and (pv, beta), when NTRP=3 and dynamic rank=1,2,3,4

Figure 2 and Figure 3 show the average UPT gain vs overhead w.r.t. all possible combinations of {Ln} and (pv,β) for NTRP=3 for dynamic rank up to 2 and up to 4, respectively. There are 12 different colors, each of which corresponds to each pair of (pv,β). For each same color, there are points each of which corresponds to different {Ln} combination.

As seen in Figure 2 and Figure 3, we can consider the overhead regime of interest (<=1000 bits) and find the best performing points within the overhead regime.

Finding best pairs of (pv,β):

To find the best pairs of (pv,β), we considered the following approach based on the criterion described in section 2.1.3.

- Step a: For each given NTRP and for each combination of {Ln}, find the (≤8) best pairs of (pv,β) that yield the best UPT vs overhead trade-off (most left and upper points)

- Step b: For each candidate (pv,β), count how many times are selected as a best pair (determined in Step a) by checking every case of NTRP and {Ln} combination.

- Step c: Sort the best pairs of pv,β using the counts found in Step b.

By the approach, we have obtained the following statistics shown in Figure 4.

Figure 4: Best-pair frequency for each (pv, beta)

As seen in Figure 3, for example, pv,β=18,18 and 18,14 are selected as a best pair for all cases of (NTRP,{Ln})  and pv,β=14,14 and 12,14 are selected as a best pair for more than or equal to a half of all cases of (NTRP,{Ln}). Here, we considered one case of {Ln} among its possible permutations.

Based on the above approach, we found the following 6 best pairs of (pv, β).

Table 2: Best pairs of (pv, β)

| Index | pυ |  | β |
| --- | --- | --- | --- |
|  | v∈{1,2} | v∈{3,4} |  |
| 1 | 1/8 | 1/16 | 1/8 |
| 2 | 1/8 | 1/16 | 1/4 |
| 3 | 1/8 | 1/16 | 1/2 |
| 4 | 1/4 | 1/8 | 1/4 |
| 5 | 1/4 | 1/8 | 1/2 |
| 6 | 1/2 | 1/4 | 1/4 |

Observation 4: pv, β=18,18, 18,14,18,12, 14,14, 14,12, 12,14 are good candidates to yield better UPT vs overhead trade-off performance than other combinations of pv, β.

Pruning {Ln} combinations performing worse, based on the best pairs of (pv,β)

Once we found the best pairs of (pv, β), we can compare the performance of {Ln} using the best pairs of (pv, β).

Figure 5: Average UPT gain vs overhead for {Ln}=(4,2,2) and (4,4,2) using (pv, β) pairs in Table 2 for NTRP=3.

Figure 6: Average UPT gain vs overhead for {Ln}=(4,4,2,2) and (4,4,4,2) using (pv, β) pairs in Table 2 for NTRP=4.

Figure 5 and Figure 6 show some examples to compare the performance of two {Ln} combinations using (pv, β) pairs in Table 2 for NTRP=3 and NTRP=4, respectively. Based on the results we can make the following observations:

Observation 5:

- Ln=(4,4,2) (and its permutations) perform worse than Ln=(4,2,2), hence they are good candidates to be removed from parameter combination for Ln when NTRP=3.

- Ln=(4,4,2,2) (and its permutations) perform worse than Ln=(4,4,4,2), hence they are good candidates to be removed from parameter combination for Ln when NTRP=4.

On parameter value of pv=12 for v=1,2,3,4

Figure 7: Max overhead comparison for R16 eType-II CSI vs CJT CB Mode 2 with pv=1/2 for v=1,2,3,4

Figure 7 shows max overhead comparison for R16 eType-II CSI vs CJT CB Mode 2 with pv=1/2 for v=1,2,3,4. To gain some insight, we also plot one-slot PUSCH capacity for 10 PRBs w.r.t. MCS index. As seen in Figure 7, max overhead of CJT CB Mode 2 with pv=1/2 for v=1,2,3,4 cannot be supported by any MCS value, under the usage of 40% one-slot PUSCH capacity. On the other hand, max overhead for R16 eType-II CSI can be supported after >11 MCS index within the usage of 40% one-slot PUSCH capacity, which is already huge constrained in use cases. The parameter value of pv=1/2 for v=1,2,3,4 is not desirable and the max overhead for R16 eType-II CSI should be baseline to determine parameter candidates.

Observation 6: max overhead of CJT CB Mode 2 with pv=1/2 for v=1,2,3,4 cannot be supported by any MCS value, under the usage of 40% one-slot PUSCH capacity.

Evaluation 2: performance comparison for two alternatives on FD basis selection offset for Mode 1, and Mode 2

Figure 8: Average UPT gain vs overhead w.r.t. different FD basis selection offset methods, i.e., Mode 1 with Alt1 and Alt2, and Mode 2 in inter-site inter-cell scenario

To compare the performance of Alt1 and Alt2 for Mode 1 (and Mode 2 as a reference), we considered R=2 to satisfy N3>19. The detailed selection schemes of Alt1/Alt2 in our SLS results are as follows:

- Alt1: one Minitial∈{-2Mv+1,-2Mv+2,⋯,0} value for a reference TRP + one FD relative offset in {0,1,,⋯,N3-1} for each of remaining N-1 TRPs + common Mv FD basis vector selection across N TRPs

- Alt2: one Minitial∈{-2Mv+1,-2Mv+2,⋯,0} value for each of N TRPs + independent Mv FD basis vector selection for each of N TRPs

For Mode 1 with Alt1 (as well as Mode 2), we considered the case of joint SVD for mTRP channels, whereas for Mode 1 with Alt2, we considered both the cases of joint SVD and per-TRP SVD for mTRP channels. Note that during the discussion of Mode 1 vs Mode 2, the proponents of Mode 1 strongly argued (despite the absence of evidence that Mode 1 provides performance benefit over Mode 2) that the Alt2 structure lends itself to a simpler PMI calculation as per-TRP PMI search is possible (i.e. per-TRP SVD).

Here, we use the UPT with Alt1 of paraComb=1 as the reference UPT value (which we regard as 100%). As shown in Figure 8, we can make the following observation.

Observation 7:

- Mode 1 with Alt 2 per-TRP SVD (the advocated lower complexity benefit for Alt2) incurs ~4% UPT loss (for the same PMI overhead) over Mode 2.

- Overall, Mode 2 and Mode 1 with Alt 1 and Alt 2 using joint-SVD operation yield similar performance.

- Mode 1 with Alt 2 needs additional UE processing to find per-TRP FD basis vectors for the case of joint-SVD operation, compared to Mode 2 or Mode 1 with Alt 1 – thereby resulting in higher UE complexity

Figure 9: Average UPT gain vs overhead w.r.t. different FD basis selection offset methods, i.e., Mode 1 with Alt1 and Alt2, and Mode 2 in inter-site inter-cell scenario, TRP-common q3 and O3=4

Figure 10: Average UPT gain vs overhead w.r.t. different FD basis selection offset methods, i.e., Mode 1 with Alt1 and Alt2, and Mode 2 in inter-site inter-cell scenario, TRP-specific q3 and O3=4

In addition, we further enhance the performance of Mode 1 Alt1/Alt2 by optimizing the oversampling factor O3 for FD basis selection. Figure 8 and Figure 9 show the SLS results when considering the cases of TRP-common q3 for Mode 1 and TRP-specific q3 for Mode 1, respectively. Note that TRP-common q3 for Mode 1 can be spec-transparently implemented but TRP-specific q3 for Mode 1 needs additional spec impact. As seen in Figure 8 and Figure 9, we can make the additional observations as follows:

Observation A1:

- With TRP-common q3, Mode 1 Alt1 performs slightly better than Alt2, and Mode 1 Alt 1 and Mode 2 perform similarly.

- With TRP-specific q3 (additional spec impact to be needed), the performance of the both Mode 1 Alt1 and Alt2 can be improved and they yield a small gain (~2% average UPT gain) over Mode 2. Regardless, Mode1 Alt1 and Alt2 perform similarly.

Evaluation 3: performance comparison for different reference grouping methods, i.e., Alt1, and Alt3, of Issue 1, in inter-site inter-cell scenarios with ISD=500m and 200m

Figure 11: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1 and 3, in inter-site intercell scenario with ISD = 500m (Mode 1)

Figure 12: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1 and 3, in inter-site intercell scenario with ISD = 500m (Mode 2)

Figure 13: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1 and 3, in inter-site intercell scenario with ISD = 200m (Mode 1)

Figure 14: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1 and 3, in inter-site intercell scenario with ISD = 200m (Mode 1)

To see any benefit of Alt3 for W2 quantization scheme, we considered inter-site inter-cell scenarios with ISD=500m and ISD=200m (detailed assumptions are described in Table 6). As seen in Figure 11 - Figure 14, there is no benefit of Alt3 over Alt1 shown in our SLS results for both mode 1 and mode 2 cases even in the inter-site inter-cell scenarios. The Alt1 scheme sufficiently performs well with the least overhead. Here, we use the UPT for Alt1 scheme with paraComb=1 as the reference UPT value (which we regard as 100%).

Observation A2: there is no benefit of Alt3 over Alt1 shown in our SLS results for both mode 1 and mode 2 cases even in the inter-site inter-cell scenarios.

Type II codebook refinement for high/medium velocities

## Key issues

    - Issue 7 (Rel-16-based vs Rel-17-based Doppler)

[3] Agreement The work scope of Type-II codebook refinement for high/medium velocities includes refinement of the following codebooks, based on a common design framework: - Rel-16 eType-II regular codebook - Rel-17 FeType-II port selection (PS) codebook FFS: Whether to prioritize/down-select from the two

Given limited time/resource constraints and large number of potential Rel.18 enhancements, refining both Rel-16-based and Rel-17-based Type-II codebooks for high/medium velocities is quite challenging, considering substantial simulation work required for codebook design. Between the two, Rel-16-based design therefore should be prioritized.

Proposal 7: prioritize Rel-16-based design for Type II codebook refinement for high/medium UE velocities.

    - Issue 8 (Finalize CQI, X>1 and CQ reference slot/window)

[1] Agreement For the Type-II codebook refinement for high/medium velocities, in one CSI reporting instance, for a given CQI sub-band, at least support including one CQI - FFS: The association of the CQI with PMI(s) and/or slot(s) within one duration of CSI reporting window WCSI - FFS: The support for including more than one CQIs [1] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives: - Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices - Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Alt2B. The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB [1] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported - If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots) Offline proposal 2.B.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrices - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the [middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrices][last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrices]

Regarding CQI, there are two open issues:

- Reference or time instance the CQI is associated with: there are 3 alternatives to down-select from and pros and cons of these alternatives are provided in Table 3. Our main concern with Alt2A and Alt2B is that it is not aligned with the current spec, since the CQI has to be conditioned on PMI in the current spec, and PMI is for the entire CSI reporting window, not just for 1 or d slots. Besides, CQI calculated based only on 1 or d slots will be inaccurate for remaining of the WCSI slots and can be outdated for future slots if the 1 or d slots is in the beginning of the CSI reporting window, which is the case with Alt2A and Alt2B. These will lead to CQI mismatch, resulting in a UPT loss (when compared with Alt1). Due to inaccurate and outdated CQI, the open loop link adaptation (OLLA) may not be able to recover this loss since it gets an inferior reference (staring) CQI for adaption to begin with, and OLLA is a long-term adjustment, which can’t recover losses due to short-term channel variations. We observe the above-mentioned UPT losses in our simulation, as shown in Section 3.3. Based on these results, we support Alt1 as reference for CQI calculation.

- Whether X=2 CQIs is also supported in addition to X=1: when WCSI is large (e.g. 20 slots) and the channel changes at a sufficiently fast rate during WCSI (e.g. to high speed), CQI is likely to vary across slots. The CQI variation, however, is expected to be slower than PMI, implying that the number of CQIs in time domain (X) can be limited. Based on the simulation results provided in section 3.3, we observe that X=2 offers additional avg. UPT gain over X=1 with slight increase in overhead, hence achieves a better UPT vs overhead trade-off than 1 CQI. We therefore propose to support at least X=2 in addition to X=1.

Regarding the offline proposal 2.B.1, as described above, our concern is that if X=1 and Alt2A is supported as a mandatory/basic feature, the gain of the Type II Doppler codebook will vanish (which is already not large when compared with Rel.16 Type II), this would mean that this feature will be in danger of not being able to meet its objective, i.e., codebook refinement for high/medium velocities. We understand the UE complexity concern from other companies with Alt1. However, complexity is an implementation issue and is not easy to quantity. So, our preference remains Alt1 and X=2, but for progress, we can be OK with this proposal as long as it is agreed as is, including both optional features, since it addresses some of our performance related concerns, as demonstrated via simulation (cf. Section 3.3) that two slots (if properly) chosen can achieve UPT vs overhead trade-off, in between Alt1 and Alt2A/2B.

Table 3

|  | Reference | Pros | Cons |
| --- | --- | --- | --- |
| Alt1 | WCSI | Aligned with current spec (CQI conditioned on PMI) | Complexity of CQI calculation during WCSI |
| Alt2A | 1st slot | Similar to legacy (which is for a slot) Lower complexity | Not aligned with the current spec CQI mismatch (hence performance loss) |
| Alt2B | 1st DD/TD unit | Extension of legacy to DD/TD unit Lower complexity | Not aligned with the current spec CQI mismatch (hence performance loss) |

Observation 8: regarding CQI

- Alt2A and Alt2B, when compared with Alt1,

  - are not aligned with the current spec, since CQI is not conditioned on PMI, and

  - incur UPT performance loss due to inaccurate and outdated CQI for future slots

  - provides an inferior reference (starting) CQI value to OLLA, which can’t recover the incurred UPT loss

- X=1 CQI may lead UPT performance loss for fast varying channel due to high/medium UE velocities

Proposal 8: support offline proposal 2.B.1,

- support both X=1 and X=2 CQIs, and the validity window associated with a CQI = W_CSI/X

    - Issue 9 (remaining parameters)

[1] Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, at least Q=2 is supported. - FFS: Whether Q=3 and/or Q=4 are also supported as other candidate value(s), as well as the supported Parameter Combination(s) [1] Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding parameter Q, decide in RAN1#112 whether to support the additional values of 3 and/or 4 [1] Agreement For the Type-II codebook refinement for high/medium velocities, the parameter WCSI (in slots) is determined as follows: WCSI = dN4 [1] Agreement For the Type-II codebook refinement for high/medium velocities, the parameter N4 (length of DFT vector, unit-less) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {1, 2, 4} - FFS: If additional candidate value(s) of N4 are supported, e.g. 3, 5, 6, 8, 10, 16, 32, as well as the supported Parameter Combination(s) [1] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), support 8 as an additional candidate value - FFS (by RAN1#112): Whether any of the following additional candidate values are supported: 3, 5, 16, 32 - The candidate values supported by UE are reported via UE capability (details can be discussed in UE feature). [1] Agreement For the Type-II codebook refinement for high/medium velocities, the parameter K (the number of AP-CSI-RS resources for the CMR) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {4, 8} - FFS: If additional candidate value(s) of K are supported, e.g. 5, 12, 16, also taking into account other use cases (e.g. for training filter coefficients, prediction or performance monitoring) and TDD [1] Agreement For the Type-II codebook refinement for high/medium velocities, the parameter δ (in slots) is gNB-configured via higher-layer (RRC) signalling from a set of the following candidate values: - First candidate value: δ=0, - 2 additional non-zero values of parameter δ are supported - FFS: the non-zero value(s), to be selected from 1, 2, 3, 4, 5, 6, and 8 [1] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), support the additional value of 2 - FFS (by RAN1#112): For the last supported additional value, down select between 1, 3, 4, and 5 [1] Proposal 2.C.5: For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - Support at least the following candidate value: - If the configured CMR is AP-CSI-RS, this candidate value is the configured value of m parameter - FFS: If the configured CMR is P/SP-CSI-RS - FFS: Whether in the above two cases, the number of slots between the last CSI-RS occasion no later than the legacy reference resource and the starting of WCSI window shall be integer multiples of d slots. - FFS: Whether additional candidate value(s) of d are supported, e.g. d<m, d>m, d=1 If more than one candidate values of d are supported, the value of d is gNB-configured via higher-layer (RRC) signalling

Our view regarding the FFs on the remaining codebook parameters are provided below.

- Q: we do not support Q=3,4 since we observe that the UPT vs overhead trade-off for Q=3,4 is worse than that for Q=2, as shown in Section 3.3. This is due to the following:

  - User throughput (UPT): a large Q requires additional (CSI-RS) measurement, which consumes additional DL resources, hence reduces resources for DL data (resulting is reduced UPT when compared with a small Q). Also, ideally (e.g. perfect channel knowledge), increasing Q should increase UPT; but due to prediction error, there is inaccuracy associated with Doppler components, especially for ‘weaker’ components, which implies that increasing Q may not result in ‘desired/similar to the ideal case’ (increased) UPT gain.

  - CSI overhead: increases with increasing Q, mainly due to bitmap and NZC associated with additional Doppler components.

  - Both together implies that the additional coefficients (when Q=3,4) increase CSI overhead, but don’t provide sufficient UPT gain; hence UPT vs overhead trade-off is inferior than Q=2.

- N4: the candidate for the support of additional N4 values are 3, 5, 16, and 32. Since 4 and 8 are already agreed values, values 3 and 5 can be excluded. The value 16 may be used for lower UE speeds (e.g. 10-20 kmph), especially when d=1, since the prediction error is likely to be small at these speeds, as shown in Appendix B. The value 32 may be used for very low speed (e.g. 3-10 kmph). So, at least N4=16 should be supported.

- K: the candidate for the support of additional K values are 5, 12, and 16. Since 4 and 8 are already agreed, the value 5 can be excluded. The remaining values (12 and 16) can incur very large delay (up to 32 slots for measurement if m=2 plus a few slots for CSI processing) between the slot carrying the DCI that triggers the aperiodic CSI report and the slot in which the CSI is reported. We therefore do not support any additional value of K.

- δ: the candidate for the support of additional δ values are 1, 3, 4, and 5. Sine δ determines the first slot (l) of the CSI reporting window, it is preferable to have l as close to the measurement window as possible. This is to ensure that the channel prediction is accurate enough for the CSI reporting. Considering this, we support δ=1 as the last supported additional value.

- d: the candidate for the support of additional d values are as follows.

  - d<m and d>m: this applies only to AP-CSI-RS resources. Since d=m∈{1,2} is already agreed, d>m corresponds to values in {3,4,…}, which can be supported if a SP/P-CSI-RS resource is supported (an FFS point from last meeting), since in that case, d can be equal to the periodicity (p)

of the SP/P-CSIRS resource, and the min supported value of p is 4.

  - d=1: since d=m=1 is agreed, this value is already supported for AP-CSIRS resources. For SP/P-CSI-RS resources, d=1 will require per-slot channel interpolation (between two measurement occasions), which can be an over-burden for a UE.

  - d=p: this applies to SP/P-CSI-RS resources. If SP/P-CSI-RS resource is supported as CMR for Type II Doppler, d=p can also be supported.

Observation 9: regarding remaining parameters,

- UPT vs overhead trade-off for Q=3,4 is worse than that for Q=2

- N4=16 may be used for lower UE speeds (e.g. 10-20 kmph), since the prediction error is likely to be small

- K=12,16 can incur very large delay between the slot carrying the triggering DCI and the CSI reporting slot

- δ is preferable to be small to ensure that the channel prediction is accurate for the CSI reporting

Proposal 9: regarding remaining parameters,

- do not support Q=3,4 and any additional value of K

- support the following additional parameters

  - at least N4=16

  - δ=1 as the last supported additional value

  - d = period of P/SP CSI-RS resource

    - Issue 10 (W2 quantization)

Offline proposal 2.D.1 (from RAN1#111): For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Offline proposal 2.D.2 (from RAN1#111): For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

The Rel-16/17 based W2 reporting mechanism (i.e., SCI, bitmap to indicate indices of NZ coefficients, and amplitude/phase of the NZ coefficients) should be reused for Doppler codebook, considering that the objective of this item is Type II codebook refinement, which implies reusing legacy as much as possible. We hence support both offline proposals.

Proposal 10: support the following

- offline proposal 2.D.1, i.e., reuse legacy (Rel.16 quantization) for N4>1, and

- offline proposal 2.D.2, i.e., similar to legacy, one SCI and one group per polarization for amplitude reporting

    - Issue 11 (NNZC bitmap design)

[1] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support the following: - Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector - The number of selected DD basis vectors is denoted as Q - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors. FFS: Further overhead reduction on bitmap(s) FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector Offline proposal 2.E.1: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector.

The Rel-16/17 based bitmap to indicate indices of NZ coefficients should be reused for Doppler codebook. However, since the coefficient matrix ( W2) comprises 2LMυQ coefficients, the total number of coefficients increases to Q times if Rel-16/17 parameter combinations are reused, which can be too large (for reporting via two-part UCI in one slot). Thus, mechanisms to limit the increase in W2 reporting payload should be considered. There are two possible solutions:

- Alt1: it is a simple extension of legacy bitmap, i.e., Q bitmaps, each 2LMυ bits, but with a smaller β value when Q>1. The rationale behind reducing beta value is that the ‘weak’ coefficients will add to overhead, but may not contribute to the UPT due to prediction and quantization errors. This has been demonstrated via SLS results in Section 3.3. In particular, it has been shown that a lower beta value can achieve better UPT vs overhead tradeoff than legacy Rel.16 beta for Q>1.

- Alt3A/3B/3C: there are two bitmaps, a first bitmap of size X×S bits and a second bitmap of size Y×Z bits, where (X,Y,Z)=(2L, M, Q), (M,2L, Q), and (Q,2L,M) for Alt3A, Alt3B, and Alt3C, respectively. The second bitmap selects S out of Y×Z basis vector pairs, hence is equivalent to a joint basis across two of (SD, FD, DD). Between these alternatives, Alt3B and Alt3C includes a joint basis across (SD, DD) and (SD, FD), respectively, i.e., both alternatives include potentially selecting a subset of the 2L SD basis vectors (corresponding to angular spread of the channel), which is not desired. So, Alt3B and Alt3C should be excluded.

Considering the above, the offline proposal 2.E.1 is OK for further discussion. Simulation results comparing these alternatives are provided in Section 3.3. When compared with Alt1, there is some overhead saving with Alt3A, but the gain in terms of UPT vs overhead trade-off is unclear.

Observation 10: Regarding bitmap

- Alt3B and Alt3C correspond to reducing the number of candidate basis vectors in angular domain (i.e. SD), which is not desired.

- Compared with Alt1, Alt3A can save some overhead, but gains in terms of UPT vs overhead trade-off is unclear.

Proposal 11: Regarding the bitmap design

- support Alt1 of offline proposal 2.E.1 as baseline

- support lower β values than legacy

    - Issue 12 (parameter combination)

[1] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, - The definition and supported values for each of the SD/FD codebook parameters follow the legacy specification. - FFS: The supported parameter combinations considering SD, FD, and DD codebook parameters - For N4=1, the legacy quantization is fully reused

For simplicity, the parameter Q for the Doppler domain can be separate from the parameter combination (L,pυ,β). The legacy table should be a starting point for the discussion on the parameter combinations (L,pυ,β). For further study and discussions, support for lower β values and the need for supporting large L (e.g. L=6) and pυ values (e.g. pυ=12) should be considered.

Proposal 12: Regarding the parameter combination

- Q is a separate from the parameter combination (L,pυ,β)

- Support legacy parameter combination table as the starting point for further discussion, which includes

  - supporting lower β values

  - whether large L and pυ values need to be supported

## Other issues

When the slot(s) associated with the CQI calculation (if Alt1 of CQI calculation is not supported) does not include all of the WCSI slots, the UE behavior about the following needs to be clarified.

- BLER requirement: the UE calculates the CQI for reporting such that it meets the BLER requirement in each slot of the slot(s) associated with the CQI calculation.

- Validity: since the CSI report is expected to be valid for WCSI slots, the calculated CQI is expected to be valid for a validity window comprising WCSI/X slots.

Proposal 13: clarify the following UE behavior,

- BLER requirement is met for each slot of the slot(s) associated with CQI calculation.

- The calculated CQI is valid over a validity window comprising WCSI/X slots.

## SLS results

The SLS results based on the agreed EVM assumptions (Table 7 in Appendix A) are provided in this section.

Evaluation 1: reference or slot(s) associated with CQI

In this evaluation, we first compare UPT vs overhead tradeoffs of the three alternatives (Alt1, Alt2A, Alt2B) of the reference or slot(s) associated with the CQI. The results are shown in Figure 15 for the following simulation setting:

- CSI report window

  - d=2, N4=8  WCSI=dN4=16

  - 1st slot: l=nref

- CSI-RS measurement:

  - K=4, m=1

  - Measurement overhead modelled

- Q=2

- CSI periodicity P = 20 slots

- UE speed = 10 kmph

- OLLA = ON, OFF

We also compare the following alternatives (cf. offline proposal 2.B.1), and the simulation result are shown in Figure 16.

- One slot: 3 alternatives are considered {1st, middle, last}

- Two slots: 3 alternatives are considered {(1st, middle), (middle, last), (1st, last)}

We can make the following observation.

Observation 11:

- Alt1 outperforms Alt2A and Alt2B, i.e., achieves up to 2% and 2.5% avg. UPT gain, respectively.

- Alt1 outperforms Alt2A and Alt2B regardless whether OLLA is ON or OFF

- Among the three alternatives {1st, middle, last} for one slot, last slot is the best, i.e., achieves ~up to 2% avg. UPT gain over 1st or middle slots.

- Among the three alternatives {(1st, middle), (middle, last), (1st, last)} for two slots, (middle, last) or (1st, last) is better than (1st, middle), i.e., achieve ~up to 3% avg. UPT gain over (1st, middle).

Figure 15

Figure 16

Evaluation 2: Number of CQIs

In this evaluation, we compare UPT vs overhead tradeoffs for X=1 CQI and X=2 CQIs for the following simulation setting:

- CSI report window

  - WCSI=24

  - 1st slot: l=nref

- CSI-RS measurement:

  - K=4, m=1

  - Measurement overhead modelled

- Q=2

- CSI periodicity P = 20 slots

- UE speed = 10 kmph

The results are shown in Figure 17. We can make the following observation.

Observation 12: X=2 CQIs achieves better UPT vs overhead trade-off than X=1 CQI, i.e. up to 1.5%% gain in avg. UPT

Figure 17

Evaluation 3: Q value

The simulation setting for comparing different Q values is summarized as follows:

- CSI report window

  - WCSI=24

  - 1st slot: l=nref

- CSI-RS measurement:

  - K=4, m=1

  - Measurement overhead modelled

- CSI periodicity P = 20 slots

- Q=2,3,4

- UE speed = 10 kmph

- (Rel.16) paraComb=1-4 with

  - Rel.16 beta

  - Smaller beta (e.g. Rel.16 beta / Q)

The UPT vs overhead tradeoffs is shown Figure 18. We can make the following observation.

Observation 13:

- UPT vs overhead trade-off for Q>2 is worse than that for Q=2

- Weak coefficients increase overhead, but don’t provide UPT gain ( beta can be small)

Figure 18

Evaluation 4: bitmap

In this evaluation, we compared the four alternatives for the bitmap (Alt1, Alt3A, Alt3B, and, Alt3C).

- Alt1: β and μβ

- Alt3A: S=μMQ

- Alt3B: S=μ2LQ

- Alt3C: S=μ2LM

- Here, β= legacy (Rel.16) and two values μ= 0.5, 0.75 are considered.

The UPT vs overhead tradeoffs is shown Figure 19. We can make the following observation.

Observation 14: there is no noticeable difference between Alt1 and Alt3A/3B/3C for the bitmap;

- Alt1 with lower β value (i.e., μβ) than legacy can be used to achieve similar performance vs overhead as Alt3A/3B/3C with legacy β

- When compared with Alt1,

- there is no gain with Alt3A/3B/3C in high overhead regime (>400 bits), and

- there is very small (<1%) gain with Alt3A/3B/3C in low overhead regime (<400 bits)

Figure 19

TDCP reporting

## Key issues

    - Issue 13 (TDCP parameter)

[1] Agreement For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112: - AltA.1 (Doppler spread) as described in R1-2210523 - AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP) - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,… - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by At,τ=ct,τct,0 , where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. - AltB (TD correlation profile) as described in R1-2210523 Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming: - Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports. - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric. - Other scenarios of the agreed use cases can optionally be simulated - Based on the agreed EVM for sTRP and mTRP Note: Different alternatives may or may not apply to different use cases FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high FFS: TDCP parameter(s) signalled with respect to each alternative

Regarding the TDCP parameter, we focused on the user case of switching between Type-I and Rel-16 eType-II codebooks, compared the three alternatives (AltA.1, AltA.2, AltB), and observed that AltB is better than other alternatives for this use case in terms of UPT performance, reporting overhead, and UE complexity. We therefore support the (auto-)correlation-based parameter, i.e., AltB. Besides, the auto-correlation has other advantages over Doppler-based metric. For instance, it is simpler to calculate (requires less computation), feasible with current UE implementations, is testable (i.e. RAN4 can come up with a test case), and can be used to compute other metric at the gNB, e.g., Doppler profile can be obtained (e.g. taking FFT) from (auto-)correlation values corresponding to ‘dominant’ lag values. Simulations results comparing the alternatives are provided in Section 4.2.

Observation 13: AltB is simpler, easier to calculate/implement, and test; also, Doppler profile can be obtained from the TD correlation

There are a few open points with AltB that require further discussion. For instance,

- Number and indices of lag values: whether NW-configured or UE-reported

- Number of TRS resources: whether one TRS resource is sufficient considering that the min periodicity of a TRS resource is 4 slots, implying the possible lag value can be {0,1,4,5,…}. If the lag values = 2,3 are ‘dominant’, they can be supported with one TRS resource. So, number of TRS resources for TDCP reporting requires further study.

- Number of TRS resource sets: the current spec allows up to 4 TRS resources in one TRS resource set, 2 resources in 2 consecutive slots. Multiple TRS resource sets may be needed for (1) reporting different lag values and (2) for mTRP scenarios.

Proposal 14:

- support AltB (auto-correlation profile) for TDCP parameter

- support TDCP reporting associated with multiple TRS resources

- study TDCP reporting associated wih multiple TRS resource sets

## Simulation results

Two sets of results are provided for the use case of Type I/II switching based on a threshold. The first set of results is based on a simplified SLS setup wherein average UPT vs UE speed performance has been considered as a metric, and the second set of results is based on a LLS, wherein BLER vs UE speed performance has been considered as a metric.

SLS results

SLS setup: 1 ring (21 cells), 1 UE per cell, full buffer, 16 BS ports, 2 UE ports, dynamic rank 1-2, Rel. 16 T2 PC 6, CSI period = 20, TRS period = 5

Observation 16: based on SLS evaluations,

- AltB and AltA.2 achieve similar performance, and AltA.1 is worse than AltA.2 and AltB

- AltB is simpler, easier to calculate/implement, and test; also, Doppler profile can be obtained from the TD correlation

Figure 20

LLS results

Figure 21 : BLER Vs UE speed

In this section, we have provided the LLS results for Type I, Type II and switching between Type I and Type II based on a threshold. Based on our results, the switching point is between Type I and Type II is around ~8 km/hr. The relevant details for AltA.2 and AltB are provided in Table 4, and the simulation assumptions are provided in Table 8.

Table 4

|  | AltA.2 | AltB |
| --- | --- | --- |
| Threshold | 25Hz | 0.96 |
| #TRS lags | 7 | 3 |
| Feedback | 1 Doppler shift | 1 correlation |
| TRS period | 5 slots (2.5ms) | 5 slots (2.5ms) |
| CSI period | 40 slots (20ms) | 40 slots (20ms) |

Observation 17: based on LLS evaluations,

- The BLER performance is almost same for AltA.2 and AltB away from switching threshold speed.

- At switching speed, AltB performs slightly better than AltA.2.

- The number of lags required for AltA.2 computation is higher than AltB

Conclusion

In this contribution, the following observations and proposals are made:

Type II C-JT

Observation 1: due to one SCI across TRPs, the W2 coefficients for different TRPs (after normalization with the strongest coefficient) capture the different power level across TRPs, implying that there is no need for per-TRP per-polarization reference amplitude reporting (i.e. Alt3)

Observation 2: on FD basis reporting for mode-1, when compared with Alt 2/3, Alt1 can

- unify the functionality of mode 1 and mode 2 (per agreement on support of mode1/2),

- achieve similar performance with lower overhead of FD basis reporting as well as lower complexity of FD basis calculation

Observation 3:

- a new parameter Ltot or Lmax is redundant

- two separate table allows reusing legacy table for (pυ,β) and can simplify the specification

a fully exhaustive search across all possible candidate parameter values is challenging, and a simplified approach is preferable for each of the discussions

Observation 4: pv, β=18,18, 18,14,18,12, 14,14, 14,12, 12,14 are good candidates to yield better UPT vs overhead trade-off performance than other combinations of pv, β.

Observation 5:

- Ln=(4,4,2) (and its permutations) perform worse than Ln=(4,2,2), hence they are good candidates to be removed from parameter combination for Ln when NTRP=3.

- Ln=4,4,2,2 (and its permutations) perform worse than Ln=4,4,4,2, hence they are good candidates to be removed from parameter combination for Ln when NTRP=4.

Observation 6: max overhead of CJT CB Mode 2 with pv=1/2 for v=1,2,3,4 cannot be supported by any MCS value, under the usage of 40% one-slot PUSCH capacity.

Observation 7:

- Mode 1 with Alt 2 per-TRP SVD (the advocated lower complexity benefit for Alt2) incurs ~4% UPT loss (for the same PMI overhead) over Mode 2.

- Overall, Mode 2 and Mode 1 with Alt 1 and Alt 2 using joint-SVD operation yield similar performance.

Mode 1 with Alt 2 needs additional UE processing to find per-TRP FD basis vectors for the case of joint-SVD operation, compared to Mode 2 or Mode 1 with Alt 1 – thereby resulting in higher UE complexity

Observation A1:

- With TRP-common q3, Mode 1 Alt1 performs slightly better than Alt2, and Mode 1 Alt 1 and Mode 2 perform similarly.

- With TRP-specific q3 (additional spec impact to be needed), the performance of the both Mode 1 Alt1 and Alt2 can be improved and they yield a small gain (~2% average UPT gain) over Mode 2. Regardless, Mode1 Alt1 and Alt2 perform similarly.

Observation A2: there is no benefit of Alt3 over Alt1 shown in our SLS results for both mode 1 and mode 2 cases even in the inter-site inter-cell scenarios.

Proposal 1: Revert the working assumption on the additional support for Alt3 for the amplitude grouping

- Note that the working assumption was included with the understanding that the gain of Alt3 over Alt1 (already agreed to be supported) can be demonstrated – not simply that “as long as Alt3 is not broken, the WA should be confirmed as agreement.”

Proposal 2: support Alt1 of the offline proposal 1.B.1 for mode-1 FD basis reporting

Proposal 3: regarding the parameter combinations for CJT codebook

- support the offline conclusion 1.C.1

- support two separate tables for supported {Ln} and supported pv, β, respectively

  - separate RRC configurations of NL≥1 {Ln} combinations and pv, β

  - two tables are determined based on a simplified approach (e.g. Table 1)

    - a metric (UPT vs overhead trade-off),

    - a criterion similar to legacy, and

    - a simplified approach for ease of discussion and simulation efforts

Proposal 4: follow the legacy framework to divide UCI parameters into CSI part 1 and CSI part 2 for Rel-18 CJT

Proposal 5: For CBSR, consider an extension of the legacy framework with adding CSI-RS resources, and a joint rank restriction across CSI-RS resources

Proposal 6: For CSI omission, consider an extension of the legacy framework with adding CSI-RS resources, a priority order can be CSI-RS resource  layer  SD  FD.

Type II Doppler

Observation 8: regarding CQI

- Alt2A and Alt2B, when compared with Alt1,

  - are not aligned with the current spec, since CQI is not conditioned on PMI, and

  - incur UPT performance loss due to inaccurate and outdated CQI for future slots

  - provides an inferior reference (starting) CQI value to OLLA, which can’t recover the incurred UPT loss

- X=1 CQI may lead UPT performance loss for fast varying channel due to high/medium UE velocities

Observation 9: regarding remaining parameters,

- UPT vs overhead trade-off for Q=3,4 is worse than that for Q=2

- N4=16 may be used for lower UE speeds (e.g. 10-20 kmph), since the prediction error is likely to be small

- K=12,16 can incur very large delay between the slot carrying the triggering DCI and the CSI reporting slot

- δ is preferable to be small to ensure that the channel prediction is accurate for the CSI reporting

Observation 10: Regarding bitmap

- Alt3B and Alt3C correspond to reducing the number of candidate basis vectors in angular domain (i.e. SD), which is not desired.

- Compared with Alt1, Alt3A can save some overhead, but gains in terms of UPT vs overhead trade-off is unclear.

Observation 11:

- Alt1 outperforms Alt2A and Alt2B, i.e., achieves up to 2% and 2.5% avg. UPT gain, respectively.

- Alt1 outperforms Alt2A and Alt2B regardless whether OLLA is ON or OFF

- Among the three alternatives {1st, middle, last} for one slot, last slot is the best, i.e., achieves ~up to 2% avg. UPT gain over 1st or middle slots.

- Among the three alternatives {(1st, middle), (middle, last), (1st, last)} for two slots, (middle, last) or (1st, last) is better than (1st, middle), i.e., achieve ~up to 3% avg. UPT gain over (1st, middle).

Observation 12: X=2 CQIs achieves better UPT vs overhead trade-off than X=1 CQI, i.e. up to 1.5%% gain in avg. UPT

Observation 13:

- UPT vs overhead trade-off for Q>2 is worse than that for Q=2

- Weak coefficients increase overhead, but don’t provide UPT gain ( beta can be small)

Observation 14: there is no noticeable difference between Alt1 and Alt3A/3B/3C for the bitmap;

- Alt1 with lower β value (i.e., μβ) than legacy can be used to achieve similar performance vs overhead as Alt3A/3B/3C with legacy β

- When compared with Alt1,

- there is no gain with Alt3A/3B/3C in high overhead regime (>400 bits), and

- there is very small (<1%) gain with Alt3A/3B/3C in low overhead regime (<400 bits)

Proposal 7: prioritize Rel-16-based design for Type II codebook refinement for high/medium UE velocities.

Proposal 8: support offline proposal 2.B.1,

- support both X=1 and X=2 CQIs, and the validity window associated with a CQI = W_CSI/X

Proposal 9: regarding remaining parameters,

- do not support Q=3,4 and any additional value of K

- support the following additional parameters

  - at least N4=16

  - δ=1 as the last supported additional value

  - d = period of P/SP CSI-RS resource

Proposal 10: support the following

- offline proposal 2.D.1, i.e., reuse legacy (Rel.16 quantization) for N4>1, and

- offline proposal 2.D.2, i.e., similar to legacy, one SCI and one group per polarization for amplitude reporting

Proposal 11: Regarding the bitmap design

- support Alt1 of offline proposal 2.E.1 as baseline

- support lower β values than legacy

Proposal 12: Regarding the parameter combination

- Q is a separate from the parameter combination (L,pυ,β)

- Support legacy parameter combination table as the starting point for further discussion, which includes

  - supporting lower β values

  - whether large L and pυ values need to be supported

Proposal 13: clarify the following UE behavior,

- BLER requirement is met for each slot of the slot(s) associated with CQI calculation.

- The calculated CQI is valid over a validity window comprising WCSI/X slots.

TDCP reporting

Observation 15: AltB is simpler, easier to calculate/implement, and test; also, Doppler profile can be obtained from the TD correlation

Observation 16: based on SLS evaluations,

- AltB and AltA.2 achieve similar performance, and AltA.1 is worse than AltA.2 and AltB

- AltB is simpler, easier to calculate/implement, and test; also, Doppler profile can be obtained from the TD correlation

Observation 17: based on LLS evaluations,

- The BLER performance is almost same for AltA.2 and AltB away from switching threshold speed.

- At switching speed, AltB performs slightly better than AltA.2.

- The number of lags required for AltA.2 computation is higher than AltB

Proposal 14:

- support AltB (auto-correlation profile) for TDCP parameter

- support TDCP reporting associated with multiple TRS resources

- study TDCP reporting associated wih multiple TRS resource sets

# References

- Chairman’s notes, RAN1#111

- Chairman’s notes, RAN1#110bis-e

- Chairman’s notes, RAN1#110

# Appendix A

Table 5: EVM for Type II C-JT

| Parameter |  | Value (Intra-cell scenario) | Value (Inter-cell scenario) |
| --- | --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM | FDD, OFDM |
| Multiple access |  | OFDMA | OFDMA |
| Scenario |  | RMa (Rural Macro) N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP) for each UE Outdoor1 | Dense Urban (Macro only) N_TRP (#TRPs): 2, 3, (N_TRP is semi-statically chosen based on, e.g. RSRP) Outdoor2 OptA |
| Frequency Range |  | FR1, 700Hz | FR1, 2GHz |
| Inter-BS distance |  | 1.7km | 200m |
| Channel model |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT |
| Number of Rings |  | 2 rings (57 sectors) - Each sector has N TRP as a cooperating mTRP set. | 2 rings (57 sectors): - The three sectors of each site is a cooperating mTRP set. |
| Number of UEs per sector |  | 30 | 30 |
| Antenna setup and port layouts at gNB |  | For each TRP, - 4 ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8)λ - 8 ports: (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ Total #ports in mTRP = N TRP x {4,8} | For each TRP, - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32} |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 46 dBm for 10 MHz | 41 dBm per TRP for 10 MHz |
| BS antenna height |  | 25m | 25m |
| UE antenna height & gain |  | Follow TR36.873 | Follow TR36.873 |
| UE receiver noise figure |  | 9dB | 9dB |
| Modulation |  | Up to 256QAM | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS | 15kHz | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots | Slot Format 0 (all downlink) for all slots |
| MIMO and scheduling scheme |  | MU-MIMO PF scheduling (User Rank 1 or 2 or 3 or 4) | MU-MIMO PF scheduling (User Rank 1 or 2 or 3 or 4) |
| MIMO layers |  | Up to 4 | Up to 12 |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Based on Alt1A/B, Alt2, Rel-16 eType-II | Based on Alt1/B, Alt2B, Rel-16 eType-II |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | RU 30~40% or 70~80% | RU 30~40% or 70~80% |
| UE distribution |  | 50% indoor (3km/h), 50% outdoor (120km/h) | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC | MMSE-IRC |
| Feedback assumption |  | Realistic | Realistic |
| Channel estimation |  | Realistic | Realistic |
| Evaluation Metric |  | User perceived throughput and CSI feedback overhead | User perceived throughput and CSI feedback overhead |

Table 6: EVM for Type II C-JT

| Parameter |  | Value (Inter-cell scenario) |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) N_TRP (#TRPs): 2, 3, (N_TRP is semi-statically or dynamically chosen based on, e.g. RSRP) (*Each same color indicates each collaborating mTRP set) Outdoor2 OptA - Inter-site inter-cell scenario |
| Frequency Range |  | FR1, 2GHz |
| Inter-BS distance |  | 200m or 500m |
| Channel model |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT |
| Number of Rings |  | 2 rings (57 sectors): - The three sectors with each same color above is a cooperating mTRP set. |
| Number of UEs per sector |  | 30 |
| Antenna setup and port layouts at gNB |  | For each TRP, - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32} |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm per TRP for 10 MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO and scheduling scheme |  | MU-MIMO PF scheduling (User Rank 1 or 2 or 3 or 4) |
| MIMO layers |  | Up to 12 |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Based on Alt1/B, Alt2B, Rel-16 eType-II |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | RU 30~40% or 70~80% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | User perceived throughput and CSI feedback overhead |

Table 7: EVM for Type-II Doppler

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. UE speed: 10 kmph Mobility model: - Spatial consistency procedure not modeled - No trajectory is assumed - Doppler spectrum model is not needed Single TRP |
| Frequency Range |  | FR1 only, 2GHz |
| Duplexing gap (b/w DL and UL) |  | 200MHz |
| Inter-BS (site) distance |  | 200m |
| Channel generation model |  | According to the TR 38.901 O2I car penetration loss per TS 38.901 can be assumed |
| Reciprocity model |  | Not applicable |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 20 MHz (10 MHz DL + 10 MHz UL) for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | Dynamic SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 MU layers |
| CSI feedback |  | Baseline scheme: CSI feedback periodicity (full CSI feedback): p1=5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling): D=4 ms eType II Doppler: CSI feedback periodicity : W=p2=10 ms CSI-RS burst: window of B measurement instances with separation d - B=2 - d=1 CSI reporting window: WCSI=B+D+p2=16 Prediction: - gNB-side prediction: DFT-based - UE-side prediction: LMMSE CQI: - gNB-side prediction: 1 CQI (legacy) - UE-side prediction: 1 or 2 CQIs over WCSI |
| Overhead |  | DMRS, CSI-RS (including CSI-RS burst overhead) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% for SU/MU-MIMO with dynamic rank 1-2 adaptation |
| UE distribution |  | 100% outdoor |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 regular eTypeII with CSI feedback periodicity 5 ms |

Table 8: simulation assumptions for TDCP LLS

| Parameter | Value |
| --- | --- |
| Channel Model | CDL-C |
| Delay spread | 300 ns |
| Frequency | 3.5 GHz |
| Subcarrier Spacing | 30 KHz |
| FFT | 1024 |
| No of gNB ports | 32 |
| No of UE ports | 2 |
| Type II parameter combination | 6 |
| CSI periodicity | 40 slots (20 ms) |
| Bandwidth | 20 MHz |
| TRS periodicity | 5 slots(2.5 ms) |
| Autocorrelation lags for Alt. A2 | 7 lags |
| Autocorrelation lags for Alt. B | 3 lags |
| Threshold for Alt. A | 25 Hz |
| Threshold for Alt. B | 0.96 |
| UE Antenna Pattern | Omnidirectional |
| BS Antenna Pattern | According to TR36.873 |
| Channel Estimation | Ideal |
| DMRS estimation | 2DMMSE |
| MIMO mode | SU-MIMO with rank=1 |
| UE pathloss | 0 |
| SNR | -15 dB |

# Appendix B

We used the Rel-16 parameter combination table unless otherwise noted.

# Appendix C (W_CSI)

# Measurement: 10 slots

# Normalized MSE b/w channel HT and predicted channel Ht' where t≤T

# T=1,…,40

# t=1,…,T

# Observation

# MSE period (P) decreases w/ UE speed

# CSI window (W_CSI) should depend on P, e.g. W_CSI=P/2

# If P is small (e.g. @higher speed), W_CSI can be within [n_ref, n]

Figure 22

Figure 23

Figure 24