3GPP TSG RAN WG1 Meeting #112	R1-2301957

Athens, Greece, February 27th – March 3rd, 2023

Source:	Moderator (Samsung)

Title:	Feature lead summary #2 evaluation of AI/ML for beam management

Agenda Item:  9.2.3.1

Document for:	 Discussion and Decision

- Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. The following use cases were identified as the initial set:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

The performance of AI/ML based algorithms for the use cases includes the following aspects:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution summarized the discussions and proposal on evaluation methodology (EVM) and KPIs from contributions submitted to AI 9.2.3.1 for beam management (BM).

Question 0

- Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| Ericsson | Henrik Ryden | Henrik.a.ryden@ericsson.com |
| vivo | Hao Wu | hao.wu@vivo.com |
| Samsung | Jackson Wang Yeon-Geun Lim | h0809.wang@samsung.com yg.lim@samsung.com |
| CATT | Da Wang | wangda@catt.cn |
| Huwei, HiSilicpn | Thorsten Schier | Thorsten.schier@huawei.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Qualcomm | Hamed Pezeshki | hamedp@qti.qualcomm.com |
| OPPO | Jianfei (Jeffrey) Cao | caojianfei@oppo.com |
| Lenovo | Srinivas Kothapalli | vkothapalli@lenovo.com |
| MediaTek | Gyubum Kyung Yu-Jen Ku | gyubum.kyung@mediatek.com yu-jen.ku@mediatek.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| ZTE | Wenfeng Liu | liu.wenfeng@zte.com.cn |
| CMCC | Jun Zuo | zuojun@chinamobile.com |
| NTT DOCOMO | Haruhi Echigo Liu Liu | haruhi.echigo.fw@nttdocomo.com liul@docomolabs-beijing.com.cn |
| LG Electronics | Haewook Park | haewook.park@lge.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| Lenovo | Srinivas Kothapalli | vkothapalli@lenovo.com |

General evaluation assumptions

  - Quantization/Measurement error

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSi [2] | Observation 13: At least for spatial domain beam prediction, the legacy quantization granularity for RSRP as inference input has negligible impact on the prediction performance. - For the AI/ML model with the output type of probability/best beam ID, the labels for training can be the best beam ID(s) and therefore is irrelative to the RSRP quantization granularity. |
| Ericsson [4] | - Proposal 7: Conclude that UEs can only detect beams with SNR higher than -3 dB in the evaluations. - Proposal 8: For beam prediction evaluations consider providing the results with measurement accuracy noise modelled as additive gaussian noise with 95% of the density function within the measurement accuracy range, and/or uniformly distributed noise Thermal noise has significant impact on prediction KPIs in scenarios with indoor UEs, and should therefore be considered in evaluations. - Proposal 9: Consider the following to mitigate the L1-RSRP measurement inaccuracy impact in ML based beam prediction - RAN4 to explore possibility to tighten requirements on L1-RSRP measurement accuracy - Define different UE capability based on their capability in fulfilling a measurement accuracy requirement. - Observation 10 Setting higher reporting accuracy in terms of granularity of reported values cannot improve the AI model performance without setting higher accuracy level on measurement error accuracy. |
| Samsung [21] | Observation # 25: Quantization error has a minor negative effect on the prediction accuracy with classification model. Proposal # 6: Study the impact on quantitation error of L1-RSRP for NW side model for beam management. |

1st round (closed)

Observation 1.1a

- Based on the results provided by [two] companies, beam prediction accuracy (%) is not sensitive to quantization error with 1 or 2dB granularity for AI/ML in beam management.

| Company | Comments |
| --- | --- |
| FL0 | Although this proposal may be premature, this will have impact on discussion of specification impact for L1-RSRP reporting in 9.2.3.2 please share your view. |
| MediaTek | - Don’t agree. Based on our study of quantization error in 9.2.3.2, legacy quantization granularity (1dB quantization error for the highest RSRP and 2dB quantization error for the rest of the RSRPs) will lead to ~10% beam prediction accuracy performance drop compared to use FP16 or FP32 to quantize the RSRP values. Further study is required to check what factors cause the different observations. |
| HW/HiSi | - Agree with FL |
| ZTE | - Prefer to evaluate it further. Before having this observation, both the spatial domain beam prediction and temporal beam prediction need to be evaluated with the quantization error. Besides, whether a classification model or a regression model is used and the associated granularity of the quantization error may also make a difference. |
| CATT | - It’s too early to have such observation only based on two or three companies’ simulation results. Prefer to evaluate it further. |
| Ericsson | - Agree. We have same observation. |
| InterDigital | - We prefer to study and evaluate further. It is too early to draw a conclusion on the impact from quantization error. |
| Google | - We see performance degradation with 5dB error. Probably we can wait for more results at next meeting? |
| Samsung | - We support the observation. - We are also open for further study. |
| FL | - Based on the feedback, the observations were combined to proposal 1.1b |

Proposal 1.1a

- Further study on whether/how to evaluate the performance impact with L1-RSRP measurement accuracy.

| Company | Comments |
| --- | --- |
| FL0 | In FL’s view, the measurement error may be caused by many aspects, e.g., thermal noise, interference, nonlinearity, etc, It may not be correct to use additive aussian noise with the range defined by RAN 4. If the L1-RSRPs estimation error of different beams is correlated, it may or may not have big impact on beam prediction accuracy. Please share your view on this proposal. |
| MediaTek | - We agree with the FL, it may not be a good idea to add i.i.d. aussian noise to each L1-RSRP measurement. But if there is a good way model the L1-RSRPs estimation error, we are happy to further study the performance impact. |
| Xiaomi | - Share same views as FL. |
| HW/HiSi | - Ok |
| ZTE | - Agree with FL’s assessment. |
| NTT DOCOMO | - Support to study more on the measurement error and quantization error. |
| Lenovo | - RSRP measurement errors is an important point to be discussed – How model the error in a simple yet - effective manner, whether modeling it using Gaussian distribution is good enough, would the error be - correlated across all the beam measurements etc. |
| CATT | - Share same views as FL. |
| Ericsson | - We are ok with the proposal. - Note that in our understanding, during the Rel-15 requirement setting for FR2 it was assumed that we cannot ensure the same Rx beam can be used to measure two reference signals for relative L1-RSRP by a UE. This is why in FR2 the relative and absolute RSRP measurement accuracy requirements are equal, i.e. 6.5 (While in FR1, it is assumed that the same Rx chain is used for measuring different RSs at the same time (fully digital Rx)). Hence, we think the inaccuracy in measurement among beams can be independent and modelled as Gaussian noise as a starting point. Regarding interference, this is an additional error source that will be visible in the system-level simulations. Support to study this and hear other companies views. |
| Fujitsu | - Support this proposal. |
| LG Electronics | - Share same views as FL. |
| Google | - Support. We think current RAN4 requirement on L1-RSRP measurement accuracy can be a starting point. |
| Samsung | - We share similar view as FL. |

  - Others

The following proposals were proposed by companies:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSi[2] | Proposal 9: For the evaluation of temporal domain beam prediction, Option 4, i.e., random direction straight-line trajectories for randomly dropped UEs, should be considered as the starting point. |
| InterDigital [11] | Observation 7: As using same beamwidth for all channels and signals is a general implementation within a frequency range, using a subset of Set A as Set B is a reasonable option if Set A and Set B are utilized in a same frequency range. Observation 8: It is difficult to use a subset of Set A considering different beamwidths for beam management between different frequency ranges. Observation 9: Utilization of wide beam information from a low frequency range has great potential as a low frequency range is more reliable and utilization of wide beam requires much less time and frequency resources for beam management. Proposal 8: Support ‘Set B is a subset of Set A’ when Set A and Set B are utilized in a same frequency range for both temporal/spatial domain prediction. Proposal 9: Support ‘Set A and Set B are different’ when Set A and Set B are utilized in different frequency ranges for both temporal/spatial domain prediction. Proposal 10: AI/ML based beam management based on association between different frequency ranges should supported for both between FR1 and FR2-1 and between FR2-1 and FR2-2. Proposal 11: For traffic model, support the following evaluation assumptions: - For beam information related KPIs, no traffic model is needed to be defined as UE is only measuring reference signals not decoding actual PDSCHs. - For system performance related KPIs, FTP traffic should be used to reflect practical traffics for the evaluation. - For FTP traffic model, FTP model 3 is preferred as generating a new UE for each packet (FTP model 1) is not appropriate for evaluating benefits from AI/ML based beam prediction. Proposal 12: For UE distribution, support the following evaluation assumptions: - For FTP traffic model, 10 UEs per cell/sector with 50% and 70% RUs is preferred. - 80% outdoor UEs and 20% indoor UEs for spatial domain beam prediction as defined in TR 38.901 (Option 1). |
| Intel [17] | Proposal 6: UE trajectories with straight line movement without sharp turns should be considered as a first step for evaluation. Proposal 7: For initial evaluations fix UE orientation towards the direction of motion. Proposal 8: Spatially consistent large-scale parameter generation should be used for mobility evaluations. Additionally, only spatial consistency model B in [4] can be used for mobility evaluation. Proposal 9: The UE trajectory should be sampled at least at the minimum decorrelation distance of the large-scale parameters corresponding to the scenario of evaluation. Data generalization Observation 1: For a large portion of the trajectory samples, the number of unique best BS/UE beam is very limited. Observation 2: Ping-pong effect is observed with the best beam index selection among close time-domain samples, but the measured RSRP of best beam pairs during ping-pong effect can be very small. Observation 3: Ping-pong effect for best beam indices can be reduced by thresholding (hysteresis implementation) and/or smoothening the RSRP for best beam selection. Observation 4: The effect of UE speed and measurement interval to the number of unique beams is not significant. Observation 5: The UE rotation mainly affects the distribution of the number of unique best UE beams. Higher UE rotation speed leads to more frequent change of the best beam pair index. Proposal 11: RAN1 should further discuss input sample length and the number of beam changes or beam dwelling time for BM-Case 2 to ensure model performance is not misleading. |
| Qualcomm [23] | Proposal 3: For BM-Case2, consider the scenario in which the UE orientation changes as a function of UE trajectory. FFS: details of this function Observation 1 At least for BM-Case2, AI/ML-based methods will provide an advantage in high-stress scenarios where frequent UE orientation changes lead to rapid changes in the best beams. Observation 2 For BM-Case2 with high UE rotation speeds, the AI/ML-based method (LSTM) strongly outperforms the sample-and-hold baseline, especially in the UE Rx beam prediction and Tx-Rx beam pair prediction use cases. - The rapid rotation leads to significant changes in best-beam RSRPs between measured cycles; the LSTM can predict for these changes, while the sample-and-hold scheme breaks down. |

Question 1.2a

| Company | Comments |
| --- | --- |
| FL0 | Please share you view if you think any of the above aspects is important and requires further agreements for the evaluation. |
| HW/HiSi | Maybe agreeing on a UE trajectory for BM-Case 2 could be useful. But otherwise, we don’t think any of the listed issues above has high priority. |
| NTT DOCOMO | We would like to keep current assumption that the UE orientation is independent of the UE trajectory. |

KPIs on AI/ML in beam management

  - Beam prediction accuracy related KPIs

    - Clarification of Top-1 genie-aided

Agreement - The options to evaluate beam prediction accuracy (%): - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam” - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams” - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams” - Where K >1 and values can be reported by companies. Agreement - For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam considers the following options - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams Agreement - For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair considers the following options: - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Option B: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams

Some observations/proposals were made in the contributions on beam prediction accuracy related KPIs:

| Company | Proposals/observations |
| --- | --- |
| Futurewei[1] | Observation 1: For DL Tx beam prediction, Option A for the definition of Top-1 genie-aided Tx beam better reflects the performance of AI/ML model than Option B. Proposal 1: For DL Tx-Rx beam pair prediction, adopt Option A as the definition for Top-1 genie-aided Tx-Rx beam pair: - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams Proposal 2: For DL Tx beam prediction, keep both Option A and Option B as definition options for Top-1 genie-aided Tx beam and companies may choose which option to adopt, or report both. - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) Observation 2: For spatial-domain DL Tx beam prediction sub use case, when using either fixed Set B beam pattern or variable Set B beam pattern sampling approach, no significant performance difference is observed when the AI/ML model is trained using a pre-identified Rx beam then performs inference using measurements for a different pre-identified Rx beam. Observation 3: For spatial-domain DL Tx beam prediction sub use case, when adopting definition Option A for Top-1 genie-aided Tx beam, the average L1-RSRP difference of Top-K/1 predicted beam(s) is significantly larger compared to the result that adopts definition Option B. Observation 4: There is noticeable performance degradation (evaluated using average L1-RSRP difference of Top-K/1 predicted beam(s)) when predicting Top-1/Top-K DL Tx beam(s) using pre-identified fixed Rx beam(s) based on prior information compared to the L1-RSRP of the true optimal beam over all the available Tx and Rx beam pairs. |
| Huawei/HiSi[2] | Proposal 4: For DL Tx beam prediction, for Option B, it should be clarified that the specific Rx beam is interpreted as the one fixed Rx beam per model input sample. - The applicable case for Option B under DL Tx-Rx beam pair prediction needs to be clarified. Observation 2: The 2 potential AI/ML-model output types are the probability of the best beam IDs and the predicted L1-RSRP values. - The already agreed KPI for the L1-RSRP difference between the ideal L1-RSRP of the predicted beam and the ideal L1-RSRP of the genie-aided best beam is applicable when the AI/ML-model is inferring the probability of the best beam ID. - The not yet agreed KPI of the L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 genie-aided beam is applicable when the AI/ML-model is inferring the predicted L1-RSRP values. |
| ZTE[3] | Proposal 1: For DL Tx beam prediction, the Top-1 genie-aided Tx beam can be defined as the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam. Proposal 2: For DL Tx-Rx beam pair prediction, the Top-1 genie-aided Tx-Rx beam pair can be defined as the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams. |
| Spreadtrum[5] | Proposal 3: The definition of Top-1 genie-aided beam: - For DL Tx-Rx beam pair prediction, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s); - For DL Tx-Rx beam pair prediction, the Top-1 genie-aided Tx-Rx beam pair is the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams. |
| OPPO[6] | Proposal 2: For DL Tx beam prediction, adopt Option B in which Top-1 genie-aided Tx beam is selected over all Tx beams with specific Rx beam(s). Proposal 3: For DL Tx-Rx beam prediction, adopt Option A in which Top-1 genie-aided Tx-Rx beam pair is selected over all Tx and Rx beams. |
| LGE[9] | Proposal 1. For the definition of Top-1 genie-aided Tx beam of DL Tx beam prediction, Option A (based on the largest L1-RSRP over all Tx and Rx beams) is preferred. |
| InterDigital [11] | Observation 5: Option B (specific Rx beam) does not reflect actual benefits from AI/ML based beam prediction and misleads interpretation of evaluation metric for AI/ML based beam prediction. Proposal 6: Support Option A (best Rx beam among all Rx beams) for Top-1 genie-aided Tx-Rx beam pair. |
| CATT [13] | Proposal 1: For DL Tx-Rx beam pair prediction, Option A is selected for the definition of Top-1 genie-aided Tx beam, i.e., the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams. Proposal 2: For DL Tx beam prediction, Option B is selected for the definition of Top-1 genie-aided Tx beam, i.e., the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s). – The specific Rx beam is one DL Rx beam selected from all Rx beams; company can report which specific Rx beam it is. |
| Intel [17] | Proposal 5: For DL Tx and DL Tx-Rx beam pair prediction, the definition of top 1 genie aided beams considers the best UE beam on the best panel as the specific Rx beam |
| Lenovo [20] | Proposal 5 Adopt “Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)" as the baseline for spatial beam prediction. |
| Samsung [21] | Proposal # 1: For both Tx-Rx beam pair prediction, the definition of Top-1 genie-aided beam is defined as - The beam (pair) that results in the largest L1-RSRP over all Tx and Rx beams |

Summary of the views from companies

| Case | Option A | Option B |
| --- | --- | --- |
| DL Tx beam prediction | FutureWei, LGE, InterDigital, Lenovo, Samsung | Spreadtrum, OPPO, CATT, ZTE |
| Tx-Rx beam pair prediction | Futurewei, ZTE, Spreadtrum, OPPO, LGE, InterDigital, CATT, Lenovo, Samsung | Intel (specific panel) |

1st round

Proposal 2.1.1-1a

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as

  - Option A: the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Other options are not precluded and can be reported by companies.

Proposal 2.1.1-2a

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Other options are not precluded and can be reported by companies.

| Company | Comments |
| --- | --- |
| FL0 | For Opt B, even with 100% prediction accuracy, it may not be the actual best beam (pairs), in FL’s view, I don’t think it make sense for Opt B. Please share your view. |
| MediaTek | - Support |
| Xiaomi | - Support these two proposals |
| HW/HiSi | - Agree with 2.1.1-1a and 2.1.1-2a |
| vivo | - We support Opt A for both DL Tx beam prediction and Tx-Rx beam pair prediction. |
| ZTE | - We prefer Option B for DL Tx beam prediction. Whether Opt B is the actual best beam depends on the Rx beam assumption, which is considered separately. As far as we are concerned, the DL Tx beam prediction is used to predict an optimal Tx beam with the assumption of specific Rx beam during one round of inference, which can well match the P2 beam refinement procedure in the current specification. Since there is no sampling or prediction at the Rx beam space, the determination of the Top-1 genie-aided Tx beam for benchmark should also be constrained by the given Rx beam space. In this case, the Top-1 genie-aided Tx beam should be the optimal Tx beam obtained by the traditional exhaustive beam sweeping method, with the assumption of the specific Rx beam. That is, the Top-1 genie-aided Tx beam is defined as the the Tx beam that results in the largest L1-RSRP over all Tx beams with the specific Rx beam. |
| NTT DOCOMO | - For DL Tx beam prediction, the choice of Top-1 genie-aided Tx beam depends on the specific Rx beam assumption for data collection. If not all the Rx beams are assumed for the output data for AI/ML model, we think Option B could be reasonable. - For DL Tx-Rx beam prediction, we support to use Option A. |
| Futurewei | - For DL Tx beam prediction, we think the answer depends on how the measurements are taken and what “over all Tx beams with specific Rx beam” means. As in DL Tx beam prediction, Rx beam may not be known and the measurements available may be based on a specific Rx beam. In the case Set B is a subset of Set A, the model is trained by using a small subset of those available measurements (all with respect to a specific Rx beam) to predict the Top-1 beam among Set A (which contains all the measurements that are with respect to a specific Rx beam). Under such assumption Option B is more reasonable to evaluate the AI/ML model performance as it was trained to predict the best Tx beam among those available measurements which are for a specific Rx beam. |
| Lenovo | - Support both proposal 2.1.1-1a and 2.1.1.-2a. We think Option A would be the correct choice. |
| CATT | - We wonder whether this genie-aided Tx beam is only used for Simulation evaluation purpose. If the genie-aided Tx beam is also used for model monitoring, then we think for DL Tx beam prediction Option B, i.e., the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s), should also need to be considered, since in the real implementation, to sweep all Tx and Rx beam need consume a lot of RS resources and power consumption. |
| Ericsson | - Support. We prefer option A. |
| InterDigital | - Support both proposals and agree with the FL that Top-1 beam/beam pair based on option B is not an actual best beam/beam pair. |
| Fujitsu | - For proposal 2.1.1-1a, it’s assumed a specific Rx beam for Tx beam predication. In this case, the beam prediction accuracy calculation is better to align with such assumption. If option A is as the definition of Top-1 genie-aided Tx beam, it’s better to include the additional Tx beam sweeping for Top-K predicted Tx beam and the Rx beam sweeping when the prediction accuracy is calculated. |
| LG Electronics | - Support |
| Spreadtrum | - We are OK with Proposal 2.1.1-2a. - For Proposal 2.1.1-1a, we think option B will be better. The prediction of Tx beam is based on one or some Rx beams. Instead of predicting for all Tx-Rx beam pairs, the definition of option A is too strict for Tx prediction. If Rx beam is wrong, then the AI model may never predict the optimal Tx. |
| Google | - Support in principle. One minor comment, we suggest changing “For DL Tx beam prediction” into “For the evaluation of DL Tx beam prediction” for both proposals. |
| Samsung | - We support the proposal. |
| Apple | - We don’t support the proposal. We support Option B. Option A and Option B have different reference signal resources assumptions, we don’t think there should be Option A only. |
| OPPO | - We would like to change our preference to Option A for DL Tx beam prediction. Sorry for the misleading by our proposal. - Given above proposals, unified genie-aided beam (pair) definition can be achieved by sweeping both Tx and Rx beams. Thus we are fine with both Proposal 2.1.1.1-1a and 2.1.1.1-2a. |

2nd /3rd round (stable)

Proposal 2.1.1-1b

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as

  - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

  - Note: This is only for evaluation discussion

Proposal 2.1.1-2a

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Other options are not precluded and can be reported by companies.

| Company | Comments |
| --- | --- |
| FL0 | First of all, this is a metric to represent the performance, i.e., how good we can do with AI. For Tx beam prediction, I understand that some companies think using “specific” Rx is more reasonable. However, with Option B, ow to compare the result from difference companies with different assumption of “specific” Rx beam? How to define/align the “specific” Rx beam? In my understanding, Opt A for DL Tx beam prediction provides the information that, with the predicted Tx beam, what is the performance gap with the corresponding best Rx beam, to the best Tx beam with the corresponding Rx beam. Therefore, I strongly suggest companies to take Opt A as baseline. |
| Xiaomi | Support |
| CAICT | Support |
| NTT DOCOMO | We understand the motivation to represent the performance, and support the current proposals. |
| MediaTek | We are OK with the current version of Proposal 2.1.1-1b and Proposal 2.1.1-1a. |
| Futurewei | We understand FL’s point. For DL Tx beam prediction, using Option A will let companies to decide what Rx beam(s) to use (or what assumptions are for Rx beam) to measure for Tx beams’ L1-RSRPs. Contributions from some companies assume best Rx beam at each sampling point or certain percentage of the samples are based on best Rx beams. If we can clarify the assumption for Rx beam when using Option A, then we are ok to use Option A as baseline. |
| Samsung | We share similar view with FL. Also, we don’t think Option A is not practical and one of special cases of Option B. For example, the selected best Tx beam from P2 and P3 beam sweeping falls into Option A since the best Rx beam selected at P3 corresponds to the best Tx beam selected at P2. We even think Option B is not practical in some cases – e.g., random or preconfigured/predefined RX beam is selected before finding best TX beam. |
| LG Electronics | Support |

    - Predicted L1-RSRP difference

Some observations/proposals were made in the contributions on beam prediction accuracy related KPIs:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSi [2] | Observation 2: The 2 potential AI/ML-model outputs are the probability of the best beam IDs and the predicted L1-RSRP values. • The already agreed KPI for the L1-RSRP difference between the ideal L1-RSRP of the predicted beam and the ideal L1-RSRP of the genie-aided best beam is applicable when the AI/ML-model is inferring the probability/best beam ID. • The not yet agreed KPI of the L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 genie-aided beam is applicable when the AI/ML-model is inferring the predicted L1-RSRP values. |
| ZTE[3] | Proposal 3: Support to define the predicted L1-RSRP difference, which is the L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam. |
| Ericsson [4] | - Proposal 2 Define a metric for L1-RSRP predictions for the Top-1 predicted beam. - Proposal 3 Error metric comprises the absolute difference of the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam - Proposal 4 Evaluate more percentiles for the L1-RSRP error (e.g. 95th,99th percentile) |
| Spreadtrum[5] | Observation 2：When the Top-1 predicted beam is different from the Top-1 genie-aided beam，the KPI (Diff to genie-aided beam) may be meaningless. |
| OPPO[6] | Proposal 4: Support another definition of L1-RSRP difference • The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam |
| Xiaomi [10] | Proposal 2: To evaluate the performance of predicted L1-RSRP, other than existing KPI of L1-RSRP difference (ideal RSRP Diff to genie-aided beam), further consider additional KPI as: - (Diff to genie-aided beam): The L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam. Observation 2: the average L1-RSRP difference between the ideal L1-RSRP of Top-1 predicted beam and the predicted L1-RSRP of the Top 1 predicted beam of AI based beam prediction in spatial domain with fixed set B is less than 0.5 dB, when Top 1 predicted beam is same as Top 1 genie-aided beam. |
| InterDigital [11] | Proposal 5: Support beam information related KPIs as optional for temporal measures. - Support average L1-RSRP difference of Top-1 predicted beam in addition to the agreed options for beam prediction accuracy (%). |

1st round

Proposal 2.1.2-1a

- For the AI/ML model with prediction of L1-RSRP as output, further consider the predicted L1-RSRP difference defined as:

  - The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam.

| Supported by | MediaTek, Xiaomi, ZTE, NTT DOCOMO, CATT, Ericsson |
| --- | --- |
| Objected by |  |

| Company | Comments |
| --- | --- |
| FL0 | For the companies that use predicted L1-RSRP as output, it seems make sense to define such KPI. However, in FL’s view, with the above definition, we cannot compare the “L1-RSRP difference” with “predicted L1-RSRP difference”. This newly defined KPIs only provides the predicted L1-RSRP difference with AI/ML. It may not present the whether the beam is correctly predicted or not. |
| MediaTek | Support |
| Xiaomi | Support the proposal since the predicted L1-RSRP can provide some assistance information to gNB if it is predicted with high accuracy. |
| Hw/HiSi | Neutral |
| vivo | Support this proposal. |
| ZTE | Agree with FL. As we already have the beam prediction accuracy related KPIs to evaluate the accuracy of the predicted beam index, the accuracy of the predicted L1-RSRP can also be evaluated as a complement. In this case, the ability of L1-RSRP prediction of the AI/ML model can be well evaluated, and the link quality of each beam can be well indicated. |
| Futurewei | As FL explained, this KPI, if defined, will not directly reflect Top-1 or Top-K prediction accuracy, which may provide misleading performance information to whoever uses it if not correctly understood. However, we understand this KPI may provide some useful information if companies want to consider using the predicted Top-1 beam directly for data transmission (bypassing P2 and/or P3). As a compromise, if majority of the companies insist on defining such a KPI, adding a note to clarify that this KPI may not imply or indicate the performance (and shall not be used to replace) for Top-1 or Top-K prediction accuracy. |
| Ericsson | Regarding FL concern. Correct, it only evaluates the L1-RSRP for the predicted beam, hence, the metric itself is not sufficient. One can envision a two-step approach, a first model predicts the best beam. And a second model predicts the L1-RSRP for said beam. |
| InterDigital | Not support. Agree with the FL. We don’t see benefits by introducing the new KPI. Regardless of predicted RSRP values, the important aspect is which beam is selected as a best beam. |
| LG Electronics | - Agree with FL. |
| Spreadtrum | If this KPI is not an independent indicator but a supplement to other KPIs, we think it is OK |
| Google | We do not see how predicted L1-RSRP can help for beam selection. |
| Samsung | We don’t think this is proper KPI for beam prediction accuracy, but this is for L1-RSRP estimation. We suggest to further discuss the intention of this proposed KPI. |
| OPPO | We are fine for the AI/ML model to output predicted L1-RSRP. Along with already agreed L1-RSRP difference (between real L1-RSRP of predicted beam and real L1-RSRP of genie-aided beam) and the L1-RSRP in Proposal 2.1.2-1a, could we also consider another KPI, i.e. the L1-RSRP difference (between predicted L1-RSRP of predicted beam and real L1-RSRP of genie-aided beam)? That could directly capture the gap between predicted L1-RSRP and the real RSRP of genie-aided beam obtained by beam sweeping, if possible. |

2nd round

Proposal 2.1.2-1b

- For the AI/ML model with prediction of L1-RSRP as output, further consider the predicted L1-RSRP difference defined as:

  - The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam.

- NOTE: this KPI may not imply or indicate the performance for Top-1 or Top-K beam prediction accuracy.

| Company | Comments |
| --- | --- |
| Xiaomi | Support. As for the note, if the AI/ML model output is the L1-RSRP of all beam(pair)s, this KPI will indicate the beam prediction accuracy. If the note is added for compromise, we can accept it. |
| CAICT | Support |
| Fujitsu | Support it. |
| MediaTek | We are OK with the updated version. |
| Futurewei | Thanks, FL, for adding the note. We are ok with the proposal. |
| Ericsson | Support. |
| Samsung | Before define the KPI, we think some meaning evaluation results should be collected regarding whether an AI/ML model can predict L1-RSRP or not. Therefore, to be safe, we suggest to revise the proposal as follows: ##### Proposal 2.1.2-1c - For evaluation of the AI/ML model with prediction of L1-RSRP as output, companies optionally report further consider the predicted L1-RSRP difference defined as: - The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam. - NOTE: this KPI may not imply or indicate the performance for Top-1 or Top-K beam prediction accuracy. |

3rd round

Proposal 2.1.2-1c

- To evaluate of accuracy of predicted L1-RSRP, companies optionally report average/CDF of the predicted L1-RSRP difference (absolute value), where the predicted L1-RSRP difference is defined as:

  - The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam.

  - Note: This can be used for the predicted L1-RSRP reporting for UE side model

| Supported by |  |
| --- | --- |
| Objected/concerns by |  |

| Company | Comments |
| --- | --- |
| OPPO | We are in general fine with this direction. In our offline comment, we suggested to add another definition on difference, that is The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of genie-aided beam But we notice that companies may need some time to digest it. To make this proposal more agreeable, can we suggest to add a note as highlighted. Thank you for considering this. ##### Proposal 2.1.2-1c - To evaluate of accuracy of predicted L1-RSRP, companies optionally report average of the predicted L1-RSRP difference (absolute value), where the predicted L1-RSRP difference is defined as: - The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam. - Note: This can be used for the predicted L1-RSRP reporting for UE side model - Note: other definition of L1-RSRP difference are not excluded and can be optionally reported by companies |
| LG Electronics | Support. Also generally fine with Oppo’s intention. But, the main sentence say L1-RSRP difference is optional. so, “other definition is not excluded” seems sufficient. |

    - Others

Some observations/proposals were made in the contributions on beam prediction accuracy related KPIs:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSI [2] | Proposal 15: For temporal beam prediction evaluation, results for Top-K, K>1 should be presented in addition to Top-1 results. - The Top-1 predicted beam can be derived as the eventual result after the second round sweeping based on the AI/ML inferred Top-K beams. |
| OPPO[6] | Proposal 1: For beam prediction accuracy, adopt the metric of Top-K/1 (%). Proposal 8: For spatial domain beam prediction, select the best beam within Set A via exhaustive beam sweeping (Option 1) as baseline. Proposal 9: For temporal domain beam prediction, select the best beam for T2 within Set A via exhaustive beam sweeping (Option 1a) as baseline. |
| InterDigital [11] | Observation 2: Beam information related KPI is not a measure to evaluate actual performance benefits from AI/ML based beam prediction, however, beam information related KPI can be used as a temporal measure to have an insight as beam information related KPI requires less computational overhead for evaluation. Observation 3: Specification enhancements should be justified based on actual system performance benefits such as throughputs not based on beam related KPIs such as L1-RSRP difference. |
| CMCC [18] | Proposal 2: The definition of beam prediction accuracy (%) with 1 dB margin for Top-K beams is: - The percentage of “the ideal highest L1-RSRP of the predicted Top-K beams is within 1 dB of the L1-RSRP of the Top-1 genie-aided beam” |
| Apple [22] | Proposal 1: The KPI for AI based beam prediction could be the beam prediction accuracy and the L1-RSRP distribution for the AI predicted beam. The KPI with RSRP can be used for making decision/drawing conclusion in the whole Rel-18 study item. |
| Qualcomm [23] | For BM-Case1 and BM-Case2, study the performance of AI/ML models with and without incorporating assistance information and compare the performance. - Study the existing trade-offs including overhead required for signalling of assistance information and corresponding performance benefits. - The agreed KPIs related to beam prediction accuracy and RS overhead reduction can be used for performance evaluation. - Examples of such assistance information: gNB beam shape-related, such as information about gNB beam shape, beam boresight directions, 3dB beamwidth, etc. - Study the impact of assistance information on generalization performance. - In addition to the above-mentioned beam shape-related information, assistance information can be in the form of gNB codebook index, etc. |

  - System performance related KPIs

    - RS overhead reduction for BM-Case2

Agreement - For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on additional measurements - FFS: Option 3: RS OH reduction%=1-NLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction - Companies report the assumption on T1 and T2 patterns - Other options are not precluded and can be reported by companies.

There were several proposals/discussions related to RS overhead[reduction] for BM-Case2:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSI [2] | Observation 5: For BM-Case 2 RS OH reduction, Option 3 is only feasible if the AI/ML-model is inferring the Top-1 beam. Option 2, on the other hand, is generically applicable. Proposal 10: For the evaluation of the overhead for BM-Case2, adopt only Option 2. |
| Ericsson [4] | - Update Option 3 with the text below - FFS: Option 3: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each repeated time instance window - where M is the total number of beams (pairs) to be predicted for each repeated time instance window - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction - Companies to report the assumption on the repeated time-window (e.g. periodicity) - Companies report the assumption on T1 and T2 patterns - Other options are not precluded and can be reported by companies.. |
| Spreadtrum[5] | Proposal 8: For RS overhead reduction in BM-case 2, option 2 is sufficient as KPI, option 3 should be excluded. |
| OPPO [6] | Proposal 6: For BM-Case2, clarify Option 3 on the definition of L to correctly calculate the overhead reduction [%], if needed. Proposal 7: For BM-Case2, support the formula 1 – (T1*N)/((T1+T2)*M) to reflect the overhead reduction [%]. |
| LGE [9] | Proposal 4. For RS overhead reduction of BM-Case 2, option 2 can be a baseline. |
| Xiaomi [10] | Proposal 8: Study the following options on RS overhead reduction for temporal beam prediction: - Option 3-1: RS OH reduction%=1-NLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each history time instance, and the beams (pairs) required for additional measurements after the prediction if applicable - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction - Option 3-2: RS OH reduction%=1-N+i=1LPiLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each history time instance - where Pi the beams (pairs) required for additional measurements after the prediction for each future time instance if applicable. - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction |
| Samsung [21] | Proposal # 2: For BM-Case2, consider both option 2 and option 3 for the evaluation of RS overhead reduction. |

1st Round

Question 2.2.1a

For RS overhead reduction for BM-Case2:

- Option 3a: RS OH reduction%=1-NM

  - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each repeated time instance window

  - where M is the total number of beams (pairs) to be predicted for each repeated time instance window

  - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

  - Companies to report the assumption on the repeated time-window (e.g. periodicity)

- Option 3b: RS OH reduction%=1-N+i=1LPiLM

  - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each history time instance

  - where Pi the beams (pairs) required for additional measurements after the prediction for each future time instance if applicable.

  - where M is the total number of beams (pairs) to be predicted for each time instance

  - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

| Company | Comments |
| --- | --- |
| FL0 | Please share your view on Option 3a and Option 3b. or you think no need to support option 3 at all. |
| MediaTek | We prefer Option 3a |
| Xiaomi | We slightly prefer Option 3b and it can be applicable for the case that predict the beam of the future time instance with short periodicity by the measurement results of history measurement instance with long periodicity. In addition, the beams (pairs) required for additional measurements before/after the prediction if applicable are also included. While for Option 3a, it is better to include the beams (pairs) for additional measurements before/after the prediction if applicable. In addition, if the repeated time window refers to a long periodicity consists of multiple short periodicities, and the beam for future time instance with short periodicity is predicted by the measurement results of history measurement instance with long periodicity, we can also accept it. |
| HW/HiSi | We do not support Option 3a, since it only would be applicable for Top-1 inference. Option 3b seems to solve this issue and could be discussed further if it is workable. But we do not see a strong need to support Option 3 in addition to the already agreed Option 2. It is also fine to skip this study. |
| vivo | We think Opt 3a seems generic enough. With Opt 2 and Opt 3a, there seems no further need to consider 3b. |
| ZTE | Not support. For option 3a, we prefer the previous wording since ‘time instance’ has already been used in many other proposals and can be well understood by companies. Besides, the temporal beam prediction can also be used for one-shot rather than repeatedly. For option 3b, the additional measurements Pi are not mandatory and up to NW. Besides, the calculation of RS OH reduction in option 3b may not be accurate in some certain cases. For example, the total number of beams (pairs) M is transmitted in a cell-specific way (such as SSB based beam sweeping) and the additional measurements Pi is conducted in a UE-specific way. In this case, the number of UEs in the cell should also be included in the RS overhead calculation. |
| NTT DOCOMO | In general, we support to study the pattern where the periodicity of measurement is different from the periodicity of prediction. In this case, all the options could be applied for it could be supportable. Specifically, comparing Option 3a and 3b, since Option 3b explicitly consider the calculation for the possible additional measurement after prediction which is important for the follow-up data transmission, we slightly prefer Option 3b. |
| CATT | Option 2 is generically applicable for BM-Case1 and BM-Case2. We don’t think it’s necessary to define option 3, since Option 2 already covers Option 3. |
| Ericsson | Preferer option A, but we can also be ok with B if majority support it. The intention with the option is to keep it general enough, and it is not only for Top-1 predictions (response to HW/HiS). |
| InterDigital | We don’t see the need to down prioritize one of the options. In our view, we can keep the both options and each company can choose the option that which is applicable to their evaluation scenario. |
| LG Electronics | Agree with CATT that option 2 can be generally applicable for BM-Case 1 and 2. |
| Spreadtrum | We believe that option 2 is sufficient, and if option3 is to be considered further, companies need to give scenarios in which option 2 will fail and option3 can be computed successfully. |
| Google | Slightly prefer option 3A. |
| Samsung | We think this is a good try to revise Option 3a, but the definition of repeated time-window should be clarified. |
| OPPO | We are supportive to the updated Option 3a. In our understanding, Option 3a and Option 3b may have different assumptions on model inputs. The former relies on the time window whereas the latter is based on historical time instances. It would be better that the difference between these two inputs could be clarified, otherwise it may cause confusion, especially for model implementation. |

3rd /2nd Round

Question 2.2.1b

For RS overhead reduction for BM-Case2:

- Option 3a: RS OH reduction%=1-NM

  - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each repeated time instance window

  - where M is the total number of beams (pairs) to be predicted for each repeated time instance window

  - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

  - Companies to report the assumption on the repeated time-window (e.g. periodicity)

- Option 3b: RS OH reduction%=1-N+i=1LPiLM

  - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each history time instance

  - where Pi the beams (pairs) required for additional measurements after the prediction for each future time instance if applicable.

  - where M is the total number of beams (pairs) to be predicted for each time instance

  - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

Summary

Option 3a:

- Support: MediaTek, vivo, Ericsson, IDC, Google(slightly). OPPO

- Not support: Huawei/HiSi

Option 3b:

- Support: Xiaomi, HW/HiS(for FFS), DCM, IDC

- Not support: vivo

Not needed:

- Huawei/HiSi, CATT, LG Electronics

| Company | Comments |
| --- | --- |
| FL0 | Please continue indicate your preference |
| Xiaomi | Prefer Option 3b. For Option 3a, it is better to include the beams (pairs) for additional measurements before/after the prediction if applicable. |
| NTT DOCOMO | Although we slightly prefer Option 3b, we are also fine with both options and leave companies to report the detailed the calculation according their RS pattern assumption. |
| MediaTek | We can accept Option3b if companies think it is necessary to include additional measurements overhead for any case which cannot be covered by the already agreed Option2. |
| Ericsson | Option 3A intends to include all beams measured before the data transmission with the selected beam. We could add a note stating: o where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each repeated time instance window Note: N includes all measurements prior to data transmission with the selected beam from the AI/ML procedure. |

    - Others

RS overhead reduction for BM Case1

There were several proposals/discussions related to RS overhead reduction for BM-Case1:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSI[2] | Proposal 8: For the evaluation of the overhead reduction for BM-Case1, Option 2 is preferred because it takes all related processing for the beam management procedure into account. - If Option 1 is used, Option 2 should be reported as a complement. |
| OPPO[6] | - Proposal 5: For RS overhead reduction [%] of BM-Case1, adopt 1-N/M (Option 1) to briefly reflect the overhead reduction. |
| Xiaomi [10] | Proposal 1: For the evaluation of the overhead for BM-Case1, support Option 1 in Option A for beam pair prediction. |

User throughput

Several companies mentioned that the system performance shall be also evaluated:

| Company | Proposals/observations |
| --- | --- |
| Interdigital[11] | Proposal 3: Support system performance related KPIs as mandatory KPIs. - Support Avg. and 5% UE tput for system performance KPIs. |
| Qualcomm [23] | At least for BM-Case1, consider spectral efficiency CDF for SLS evaluations as a KPI. |
| MediaTek [25] | Proposal 3: To evaluate the system level throughput performance of AI/ML beam management, both average user throughput and cell edge user throughput need to be reported. |

UCI report overhead and RRC signaling overhead

The following was discussed in contributions:

| Company | Proposals/observations |
| --- | --- |
| Vivo [8] | Proposal 3: UCI reporting overhead reduction, including the number of UCI report and UCI payload size, should be considered as basic KPI. Proposal 4: RRC singling overhead can be considered as optional KPI if huge amount of data, such as training data, assistance information, and AI model data, is exchanged via RAN air interference. |
| LGE[9] | Proposal 2. UCI report overhead can be considered as one of KPI for NW sided beam prediction. |
| Interdigital[11] | Observation 4: UCI report overhead is not a metric for evaluation, but assumption for evaluation. Proposal 4: UCI report is reported as assumptions not as KPI. |
| Lenovo[20] | Proposal 3 Consider the number of UCI reports and the size of each UCI report (in bits) as a measure of the reporting overhead. The reporting overhead need to be included into the KPIs. |
| MediaTek [25] | Proposal 2: To define the UCI report overhead, first discuss the number of UCI reports and how the report is quantized. |

Latency reduction

- Lenovo [20]

  - Latency Reduction = 1- Total transmission time of N beamsTotal transmission time of M beams

  - Proposal 4	Consider Beam Prediction Accuracy, Overhead Reduction and Latency Reduction as the key KPIs in evaluating an AI/ML model for beam management and consider adopting the definitions proposed above.

QCL related

| MediaTek [25] | Proposal 1: For QCL relation overhead, first study and list the scenarios when such QCL relation overhead exists, then discuss how to define the KPI. |
| --- | --- |

  - Baseline performance

Some companies provided some analysis on baseline performance for benchmark.

| Company | Proposal/observation |
| --- | --- |
| InterDigital [11] | Observation 1: Legacy beam management with Rel-17 without AI/ML algorithms is not an appropriate baseline as implementation-based AI/ML operation is available for UE and gNB implementations. Proposal 1: ‘No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes’ could be an appropriate baseline to accurately evaluate the benefits of AI/ML with specification enhancements. |
| Intel [17] | Proposal 4: For baseline performance evaluation, Option 2 should correspond to hierarchical beam search where, based on sub-use case being evaluated, set B may be a subset of set A or set B can contain both wide and correlated narrow beams. |
| Lenovo [20] | Proposal 5 Adopt “Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)" as the baseline for spatial beam prediction. |

AI/ML related assumptions

  - Set B of beams(pairs)

RAN 1 #110bis agreed three options of the selection of Set B of beams(pairs).

Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured patterns - Opt B: Set B is randomly changed among pre-configured patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded.

The following proposals were about the section of Set B:

| Company | Proposal/observation |
| --- | --- |
| Futurewei [1] | Proposal 3: For the options on the selection of Set B of beams (pairs) for Option 2, further clarify Opt B and Opt C as following: - Opt B: Set B is randomly changed among pre-configured / pre-known candidates of Set B patterns - Opt C: Set B is randomly changed among candidates of Set B which is some non-pre-configured subsets of Set A beams (pairs) Observation 1: In spatial-domain beam prediction using fixed Set B beam pattern in AI/ML model training phase, performance degrades significantly when Set B beam pattern changes in model inference phase. Observation 2: In spatial-domain beam prediction using fixed Set B beam pattern in AI/ML model training phase, performance degrades significantly when at least one of the beam pairs in Set B is missing during model inference phase. Observation 3: In spatial-domain beam prediction using variable Set B beam patterns in AI/ML model training phase, performance degrades significantly when Set B beam patterns changes in model inference phase. |
| Huawei/HiSi [2] | Observation 4: If multiple pre-configured Set Bs are used for training, it is more realistic for the gNB to select a limited number. Proposal 6: For BM-Case-1 and Case-2, for the selection of Set B, consider Set B is fixed across training and inference as a starting point. - If multiple pre-configured patterns are evaluated, their maximum number should be limited, e.g. to 5. Observation 12: For spatial domain beam prediction, variable Set B patterns selected from a set of 5 pre-configured patterns can achieve close performance to the fixed pattern approach but with better generalization capabilities with respect to different patterns. |
| ZTE [3] | BM-Case1 Observations 4：For Tx-Rx beam pair prediction and DL Tx beam prediction, similar beam prediction performance is achieved with different Set B patterns in the same sampling rate. Observations 5：For DL Tx beam prediction, a sufficient high performance is achieved with Set B patterns 4/5/6 in a sampling rate of 12.5%, where the beam prediction accuracy for Top-5 beams is above 97%. Observations 6：For DL Tx beam prediction, marginal performance loss is observed when the AI model inputs are Top 8 beams or Top 4 beams of all 16 beam measurements. Observations 7：For Tx-Rx beam pair prediction, significant performance loss is observed when the AI model inputs are Top 8 beams or Top 4 beams of all 128 beam pair measurements. Proposal 7: To reduce the overhead of UE feeding back beam measurements for DL Tx beam prediction in BM-Case 1, it is better to only report partial beams from a set of beams for measurement for a NW-side model. BM-Case2 Observation 16: In BM-Case2, compared with the case that Set B is fixed across training and inference, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns. Observation 17: In BM-Case2, compared with the case that the Set B pattern is randomly selected from a total of 4 beam patterns in each past time instance, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns. Observation 18: In BM-Case2, if Top-4 or Top-2 beams of all measured beams are used as model inputs in each past time instance, only a little performance loss is observed compared with the case that all measured beams are used as model inputs. Proposal 10: To reduce UE report overhead in BM-Case 2 for network-side model, it is better to take partial beams from the beam set for measurement as the model input. |
| Ericsson [4] | - Proposal 6: Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/ report/measurement during training and/or inference), Set B is deterministically changed, based on the following alternatives - Pre-configured/pre-known pattern - Pre-processing of measurements (e.g. UE report K strongest beams, NW filters out K beams, etc.) - Combination of a) and b) - #beams in Set B can be fixed or variable - Set B can be a subset of set A - Companies to report how variable set B is selected - Assumptions on pre-configured/pre-known/pre-processing - Companies to report the number of measurements - Observation 6: By allowing variable number of reported beams via UE pre-processing of measurements, the reporting overhead can be substantially reduced with little performance degradation. |
| Spreadtrum[5] | Proposal 1: For both BM-Case1 and BM-Case2, more details about the composition of set A/B should be provided to facilitate the comparison of simulation results. Observation 1：For the composition of set A/B, Alt2 requires more complex operations than Alt1. FL: Alt1: Set A =! Set B, Alt 2= Set B is a Subset of Set A, Proposal 2: For the selection of Set B of beams (pairs). - If AI/ML inference is at NW side, beams in Set B can be determined by NW implementation. - If AI/ML inference is at UE side, beams in Set B can be determined with a fix pattern. |
| OPPO [6] | Observation 1: fixed Set B across training and inference phases has the potential to provide more stable and accurate performance for beam prediction when compared with variable Set B. |
| Vivo [8] | Observation 1: Fixed pattern selection scheme with different fixed patterns brings tremendous performance difference. Observation 2: Better performance gain can be obtained for one fixed pattern selected by well-designed rule or enumerated with predefined searching criterion. Observation 3: The performance with different training and validation fixed patterns is quite poor and not acceptable, i.e., fixed set B selection scheme suffers serious generalization issue. Proposal 5: Unless an excellent generalization performance can be proved in option 1, i.e. a fixed pattern in Set B for training and same fixed pattern in Set B for validation, fixed set B selection scheme should be deprioritized. Observation 5: Random pattern selection scheme, which allows multiple random patterns in training, can improve generalization performance as well as beam management related performance if compared to mismatched pattern with always using one pattern in training. Observation 6: Set B with random beam patterns still suffers tremendous performance deterioration due to huge number of combinations of selecting a target number of beams from total beam pairs. Observation 7: Although certain performance loss can be observed from Set B with pre-configured patterns in comparison with fixed pattern scheme, it has a significant increase compared to random pattern scheme. Observation 8: Beam prediction performance deteriorates along with the increase of the number of pre-configured patterns. Proposal 6: Support pre-configured patterns of Set B with a limitation to the number of best X predefined beam patterns, i.e. option 2B. |
| LGE [9] | Proposal 3. For selection of Set B of beams, Option 1 can be considered as a baseline. |
| Xiaomi [10] | Observation 1: AI based beam prediction in spatial domain can provide good performance. And the performance can be further improved by inputting corresponding beam pair ID in addition to measured L1-RSRP or by inputting L1-RSRP of same beam pair IDs. Observation 3: different fixed set B with continual beam pair ID can provide almost same performance, but different set B in pre-configured set of set B results in some performance loss because of low generalization capability among different fixed set B. Observation 4: different fixed set B with non-continual beam pair ID can provide almost same performance, and different set B in pre-configured set of set B results in only few performance loss because of good generalization capability among different fixed set B. Observation 5: Fixed set B with non-continual bam pair ID can provide better performance than that of fixed set B with continual beam pair ID since non-continual beam pair ID covers more Rx beams. Proposal 4: Different fixed set B consists of L1-RSRP with more Rx beams should be considered with high priority. |
| InterDigital [11] | Observation 6: For different beam pattern type, each beam pattern type has its own pros and cons for performance, flexibility of inference input and range of inputs required for training. Proposal 7: Further study benefits of various beam pattern types. |
| Nokia [12] | Proposal 3: For evaluating various Set B, companies report the number and the generation of pre-configured/pre-known patterns of Set B(s) as well as the number of SetB patterns generated for simulating the Opt2C configuration. Observation 9: Model performances for random SetB (OptC) Tx beam prediction is sensitive to the size of the dataset used for training. (Full Set B) Observation 10: Model performances for OptC are more sensitive to the number of reported beams. (N best Reporting) Observation 11: Reporting 4-best beams may not be sufficient for training and inference at NW side the model for DL Tx beam prediction with Fixed/Pre-configured SetBs. OptC requires reporting of at least 8 beams. (when Set A=64) Proposal 4: For BM-Case1, RAN1 may further investigate to enhance the reporting from 4-best beams to 8-best beams. Proposal 5: For BM-Case1, RAN1 may prioritize the measurements of Fixed/Pre-configured SetBs (Opt1 and Opt2B) to be used on the NW side for input to model training/inference. Proposal 6: For BM-Case1, RAN1 may prioritize the measurements of Random SetB (Opt2C) to be used at UE side for input to model training and the measurements of Fixed/Pre-configured SetBs (Opt1 and Opt2B) to be used at UE side for model inference. Observation 19: In BM-Case1 DL Tx- Rx beam pair prediction, training model with fixed beam number in random Set B outperforms the training model with varied beam number in random Set B. Observation 20: In BM-Case1 DL Tx-Rx beam pair prediction, the use of random SetB provides a nonnegligible performance drop compared to the use of fixed SetB. Top-K beam search may not be sufficient to achieve sufficient intermediate performance KPIs. Proposal 10: RAN1 prioritizes fixed or pre-configured SetB patterns for further investigations of DL Tx-Rx beam pair prediction. |
| CATT [13] | Observation 1: For Beam pair prediction, using pre-configured patterns for training and inference, the Top-1 accuracy can be greater than 93%. Proposal 4: For BM-Case1, random pattern of Opt B, i.e., Set B is randomly changed among pre-configured patterns, can be used to improve beam pair prediction accuracy. Observation 2: For DL Tx prediction with random pattern, using beam ID as additional information can improve prediction accuracy. Observation 3: For DL Tx prediction, the prediction accuracy is greatly reduced when using different pre-configured patterns for training and inference. Proposal 5: The patterns for training and inference should be pre-configured or configured to cover all beams of Set A as much as possible. Observation 6: For DL Tx RSRP prediction with fixed pattern, the prediction accuracy of Top-1 is higher than 93% for DL Tx beam RSRP prediction |
| CAICT [16] | Observation 2: Fixed Set B (Option 1) achieves better performance than randomly chosen pre-configured pattern schemes for BM-Case 1 beam pair prediction. Proposal 1: Fixed Set B (Option 1) could be used as baseline for BM-Case 1 further beam pair comparation. |
| Intel [17] | Proposal 2: The variability of Set B can only be due to updating the L1 measurements corresponding to beams or beam-pairs in Set B at different intervals. The cardinality of the set should not change across training and inference. Proposal 3: Construction of set B patterns should be defined only for model inference and not for training data collection. |
| CMCC [18] | Proposal 1: On selection of Set B of beams (pairs), study the following options when Set B is variable -  Set B is changed among pre-configured/pre-known candidates of Set B -  Set B is changed among Set A beams (pairs) candidates of Set B which is subsets of Set C (a set of beams (pairs) for measurement) It can be seen that when Set B is variable, the AI/ML model can achieve satisfactory performance with the cost of higher model complexity and computational complexity. |
| Lenovo [20] | Observation 1: The beam prediction accuracy performance of AI-based BM is relevant with the Set B pattern selection. |
| Samsung [21] | Observation # 1: Opt A (Fixed Set B) is more suitable for BM-Case2 considering multiple reports at different time instance. Opt A may be also beneficial for BM-Case1. Observation # 2: Opt B (Change with pre-configured pattern) may be reasonable if different Set B is needed for different UEs. Observation # 3: Opt C (randomly changed among Set A) may be reasonable for AI/ML at UE side since different gNB may have different Set B. Observation # 4: Opt D (randomly changed among a pre-configured subset of Set A) require less L1-RSRP report overhead and may be more reasonable for AI/ML at gNB side. Observation # 20: With decent number of beams in Set B, e.g., ¼ beams of Set A, fixed Set B or pre-known different patterns in each time step has similar performance. Observation # 21: Select from pre-known patterns with or without knowing the order has similar performance. Observation # 22: Random Set B (option 2C) has the worst performance comparing with fixed or pre-known Set B patterns. Observation # 31: Using the L1-RSRP of the “best” Rx beam with exhaustive beam sweep as inputs can provide the best performance for the accuracy of Top-1/N beam prediction than fixed one Rx beam or randomly selected one or two Rx beams with fixed or random Tx beams for DL Tx-Rx beam pair prediction for BM-Case 1. Observation # 32: With L1-RSRPs of fixed Rx beam(s) as AI inputs can provide better performance than L1-RSRP of random Rx beam(s) for DL Tx-Rx beam pair prediction for BM-Case 1. Observation # 33: For beam pair prediction for BM-Case 1, AI with inputs as L1-RSRPs of fixed Tx beams and implicit beam ID information can provide better performance than non-AI based approach. Observation # 39: Even if UE reports only half of Set B’, we can observe gain in top K/1 prediction accuracy compared to non-AI. As the number of reporting beams increases, there is a benefit, but it does not increase significantly. Proposal # 3: Support the following options on the selection of Set B of beam(pairs) for evaluation for AI/ML at gNB side - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/ measurement during training and/or inference) - Opt A (baseline for BM Case2): Set B is changed following a set of pre-configured/pre-known candidates of Set B in pre-configured/pre-known order - Opt B (baseline for BM Case1): Set B is changed among pre-configured/ pre-known candidates of Set B - Opt D (optional): Set B is changed among candidates of Set B which is subsets of Set C (a set of beams (pairs) for measurement) - The number of beams(pairs) in Set B can be fixed or variable Proposal # 4: Support the following options on the selection of Set B of beam(pairs) for evaluation for AI/ML at UE side - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/ measurement during training and/or inference) - Opt A (Optional): Set B is changed following a set of pre-configured/pre-known candidates of Set B in pre-configured/pre-known order - Opt B (Optional): Set B is changed among pre-configured/ pre-known candidates of Set B - Opt C (baseline): Set B is changed among candidates of Set B which is subsets of Set A beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable Proposal # 5: For Opt D, adopt the maximum 4 number of reported beams out of Set C as a baseline for evaluation. |
| DoCoMo [24] | Observation 2: For Tx beam prediction with Rx beams assumption Option 1 and Option 2a: - When the Rx beams in Set C is specific beams instead of all Rx beams (Option 2a), the prediction performance deteriorates a bit Proposal 11: Remove the word “randomly” from Opt B and Opt C of Option 2 on the selection of Set B. Further discuss on other modification to the classification and description in Option 2. |
| Qualcomm [23] | Observation 3 For BM-Case1 and the sub use case of “Set B is a subset of Set A”, and for a fixed Set B pattern option, the beam prediction performance largely depends on how Set B is selected. - For instance, if set A includes beams in both azimuth and elevation, selecting Set B beams only in azimuth dimension may adversely impact prediction performance. |
| MediaTek [25] | Observation 9: The selection of beams in Set B will affect the prediction accuracy of the AI/ML-based spatial beam prediction. Proposal 8: For AI/ML-based spatial domain beam prediction evaluation, study the subset selection (number and combination) if Set B is variable (Option2 on the selection of Set B of beams in the RAN1 #110 agreement). Observation 10: The spatial beam prediction by using multi-arm beam design in Set B performs better than using subset beam design in Set B. Observation 11: The spatial beam prediction by using wide beam design in Set B does not outperforms the performance by using subset beam design in Set B. Proposal 9: Study and evaluate a more comprehensive Set B design, including joint designing the number of beams in Set B and their beam shape for spatial beam prediction. |

1st Round

From FL’s observation, companies may have different assumption on AI/ML model inputs. In order to facility the discussion on Assistance information, please consider the following proposals:

Proposal 3.1-1a

- For the evaluation of Option 1: Set B is fixed across training and inference, further down select the following as AI/ML model inputs

  - Alt 1: No explicit or implicit information of Tx beam ID and/or Rx beam ID

  - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID, e.g., use special value to represent the measurements of the beams (pairs) not in Set B

  - Alt 3: The information of Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly

  - Note: information Tx beam ID and/or Rx beam ID is not treated as assistance information

| Companies support Alt1 | MediaTek, Xiaomi, Hw/HiSi, vivo, ZTE, NTT DOCOMO, Futurewei, CATT, Ericsson, LG Electronics, Google |
| --- | --- |
| Companies support Alt2 | Samsung |
| Companies support Alt3 |  |

Proposal 3.1-2a

- For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further down select the following as AI/ML model inputs

  - Alt 1: No explicit or implicit information of Tx beam ID and/or Rx beam ID

  - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID, e.g., use special value to represent the measurements of the beams (pairs) not in Set B

  - Alt 3: The information of Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly

  - Note: information the Tx beam ID and/or Rx beam ID is not treated as assistance information

| Companies support Alt1 | Xiaomi(Opt A &B), NTT DOCOMO, Ericsson |
| --- | --- |
| Companies support Alt2 | MediaTek, ZTE, Futurewei, CATT, Samsung |
| Companies support Alt3 | MediaTek, Xiaomi(Opt C), Hw/HiSi, vivo, Futurewei |

Please share your view on proposals

| Companies | comments |
| --- | --- |
| FL0 | For fixed Set B, based on FL’s observation, no company report explicit input the IDs For random Set B, although some companies report the results with no information of beam(pair)ID, I don’t think it is worth to study the case due to the poor performance at least for OptC (random among Set A) |
| Xiaomi | For fixed set B, Alt 1 is preferred. For random set B, Alt 1 is preferred for Opt A and Opt B. While for Opt C, Alt 3 is preferred. |
| HW/HiSi | For our understanding: for the purpose of evaluation, does it really make a difference which Alternative is selected? Isn’t this question more related to spec impact? |
| ZTE | Agree with FL that implicit or explicit beam ID information is needed for AI/ML model input if random set B is used. Whether to use implicit or explicit beam ID information as model input is up to implementation. |
| NTT DOCOMO | According to options in Proposal 3-1-3a, different assistance information should be expected. Hence, we suggest to discuss it per Option for Proposal 3-1-3a. |
| Lenovo | In our opinion, companies can report the AI/ML model inputs and we need not be specifying what should be the model inputs. |
| CATT | We wonder what’s the different between “implicit information of Tx beam ID and/or Rx beam ID” in Alt1 and Alt2? Alt2 is already covered by Alt1? |
| Ericsson | We don’t think the ID per-se needs to be inputted to the model. However, beam IDs can be used to select the order of the beam measurement input for the first layer ([RSRP_beamID1, RSRP_beamID2,….,, RSRP_beamIDN]). What is important is that the beam measurements are inputted to the model in the same order each time. For variable beam predictions, the non-measured beams can be set to a fixed value, where such fixed-value is also inputted to the model instead of the measured value, to preserve the order of the beam measurement input. Also, for the spec. impact discussions, IDs of the beams are needed to understand what the order of beam measurements is to be inputted to the model. |
| InterDigital | We support Alt 3 for both proposals as we believe that reporting beam IDs is essential. Especially in practical implementation, reporting all the RSRP values may require huge overhead as each RSRP reporting requires 7 bits unless differential RSSP is used. So, we prefer to study beam ID reporting even for the fixed Set B case. |
| Fujitsu | Support either explicit or implicit information of Tx beam ID and/or Rx beam ID. But for the Alt.2, we are not clear for the motivation of “use special value to represent the measurements of the beams (pairs) not in Set B”. Could FL make a clarification on what’s kind of special value and why beams(pairs) are not in Set B? |
| Spreadtrum | For Proposal 3.1-1a, we felt there is one point that needed clarification. If set B is input into the AI model in a certain order, does this carrying some implicit beam ID information? For Proposal 3.1-2a, beam ID information should be provided as supplementary information. |
| Samsung | We think Alt 2 also should be consider how to work OptC in practice. |
| OPPO | As for Alt.1, could we understand the order of a tensor (containing the measurement results of Set B) as no implicit input? If that’s the case, we are fine with Alt.1 in Proposal 3.1-1a. Otherwise, we understand the order of tensor as Alt.2, i.e. the implicit input. Hope this could be clarified. |

Proposal 3.1-3a

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is randomly changed among pre-configured patterns candidates of Set B patterns

    - Opt C: Set B is randomly changed among some non-pre-configured subsets of Set A beams (pairs)

    - Opt D: Set B is a subset of measured beam (pair) Set C, e.g. Top-K beams of Set C

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

| Supported by | MediaTek, NTT DOCOMO, Futurewei, CATT, Ericsson, Google, Samsung |
| --- | --- |
| Objected by |  |

Please share your view on proposals

| Companies | comments |
| --- | --- |
| MediaTek | We are OK with the current version. Just one typo: - Opt B: Set B is randomly changed among pre-configured patterns candidates of Set B patterns |
| Xiaomi | We are not clear about the use case of Opt D, the motivation to take Top-K beams (pairs) of Set C as input is to avoid the case that the worst beams(pairs) may reduce the prediction accuracy of AI/ML model? Which is not aligned to the Observation 1.1 a. Change some small typos as below - Opt B: Set B is randomly changed among pre-configured patterns candidates of Set B patterns - …… - Opt D: Set B is a subset of measured beam (pair) Set C, e.g. Top-K beams (pairs) of Set C |
| HW/HiSi | Support Option 1. Support Option 2B. For 2B, the number of pre-configured beams should not be too large, e.g. 5. One reason is performance that degrades steadily the more sets are used and another reason is the beam sweeping pattern is elaborately designed in general, and the gNB would not arbitrarily change the beam sweeping set in the realistic network, especially considering Set B also takes the role of beam sweeping over legacy UEs for backward compatibility. |
| ZTE | Support. Opt. D can be used to reduce reporting overhead for a NW-side model. In this case, only partial beams, such as Top 4 or Top 8 beam of all beam measurements in Set C are reported and used as model input. According our simulation results, only marginal performance loss is observed compared with reporting all beams in Set C for measurement. |
| Lenovo | In RAN1 #111 we have made the following agreement. Agreement - Companies report the pattern of Set B. - Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B). As it was decided that “companies report the pattern of Set B”, we do not understand the need to discuss the selection of Set B again. |
| Ericsson | Support |
| LG Electronics | Agree with Lenovo. |
| Spreadtrum | Support Option 1. And agree with Xiaomi's typo modification for option 2. |

Proposal 3.1-4a

- For DL Tx beam prediction, study the following pattern design of Set B of beams, at least consider the following options:

  - Alt1: Uniformed distributed in Set A

  - Alt2: Based on statistic information, FFS on feasibility and details

  - Other alternatives are not precluded.

Proposal 3.1-5a

- For Tx-Rx beam pair prediction, study the following pattern design of Set B of beam pairs, at least consider the following options:

  - Alt1: Sparely uniformed distributed among both Tx beams and Rx beams

  - Alt2: Uniformed distributed among Tx beams for all Rx beams

  - Alt3: Based on statistic information, FFS on feasibility and details

  - Other alternatives are not precluded.

Please share your view on proposal 3.1-4a and proposal 3.14-5a

| Companies | comments |
| --- | --- |
| FL0 | The two proposals are to facility the further discussion to compare the performance of different Set B. For example, if the Set B designed based on statistic information provides better performance, some additional work might be needed during data collection. In addition, we can further compare the performance between fixed/variable Set B, the assumption of patterns of Set B matters. @Xiaomi, can you elaborate more on “continual beam pair ID” and “non continual beam pair ID” with Tx beam and Rx beam other than beam pair ID since we don’t have common understanding on how to index for beam pair. |
| MediaTek | Support proposal 3.1-4a. We don’t support 3.1-5a, with the same reason we state in Question 3.7a. |
| Xiaomi | For proposal 3.1-4a, in our understanding, Alt 1 can only be applied for the case that set B is a subset of set A and it can’t be applied for the case that set B is different from set A. in addition, the meaning of Alt 2 is not very clear for us. For proposal 3.1-5a, the following figure provides the association between beam pair ID and Tx/Rx beam ID in R1-2300569. It means that for each Rx beam ID, the beam pair ID for 32 Tx beam is contiguous. If set B consists of continual beam pair ID, it means that only a few Rx beams are selected. If set B consists of non continual beam pair ID, it means that more Rx beams are selected. For Alt 1 in the proposal, could FL provide an example? Which TxRx beam pairs will be selected according to our rule for beam pair ID? For Alt 2, it is clear and we support it. For Alt 3, same comments as in proposal 3.1-4a. |
| HW/HiSi | For our understanding, the 2 proposals above seem to address that Set B is a subset if Set A. We don’t think that the intention here is to preclude Set B being different from Set A? In we are right with our understanding, it could be helpful to update the main bullets in both proposals like: - For DL Tx beam prediction, when Set B is a subset of Set A, study the following pattern design of Set B of beams, at least consider the following options: - For Tx-Rx beam pair prediction, when Set B is a subset of Set A, study the following pattern design of Set B of beam pairs, at least consider the following options: ##### For Proposal 3.1-4a, Alt1 can be studied. For Alt2 it is not clear to us if the Set B based on statistic information imply that it is a totally variable Set B? Does it mean that a pre-configured Set B can be selected based on the statistic information? We think that somehow this proposal is tightly related to the fixed vs variable Set B comparison? For Proposal 3.1-5a, we think that Alt1 and Alt2 can be studied. For Alt 3 we are not clear similar to proposal 3.1.-4a. |
| vivo | For our understanding, this is only for fixed set B study, correct? For various set B, as the pattern of Set B is flexibly changed within a set, it seems no need to have such pattern definition. |
| ZTE | We agree with Xiaomi and HW that Alt 1 is only suitable to the case that Set B is a subset of Set A. Besides, the statistic information in the proposal should be clarified by proponents. |
| NTT DOCOMO | We support to study different alternatives on the design of Set B. |
| Lenovo | The current wording of these proposals appears to be again for selecting Set B. It is not clear ho these proposals “facilitate” further discussion to compare the performance of different Set B choices. |
| CATT | For these two proposal, we have the following two questions: What’s meaning of statistic information and how to get it, since different scenarios and configurations may have different statistic information? For Proposal 3.1-5a, for Alt1, what’s meaning of “Sparely uniformed distributed among both Tx beams and Rx beams”? |
| Ericsson | We assume that the statistic information in Alt2 is based on the collected dataset for all potential beams, prior to determine set A and B. Support in general, but needs clarification. |
| InterDigital | We are not sure that why we need to introduce different alternatives for DL Tx beam prediction and beam pair prediction. Especially, what’s the difference between Alt1 and Alt2 for beam pair prediction? |
| Fujitsu | For pattern design of Set B of beams(pairs), it’s not clear how to work on the design based on statistic information. We support alt.1 for proposal 3.1-4a and alt.1/2 for proposal 3.1-5a. |
| Google | Support |
| Samsung | We suggest FL to clarify the motivation. |
| OPPO | We are in general fine with the direction. But for Tx-Rx beam pair prediction, the Rx beams somehow belongs to UE implementation and it seems possible that not all the Rx beam patterns of Set B can enable uniformly distributed. In addition, between Alt.1 (sparely uniformed) and Alt.2 (uniformed), can the difference be clarified a little? Thanks in advance. |

2nd Round

Proposal 3.1-1c

- For the evaluation of Option 1: Set B is fixed across training and inference, no explicit information of Tx beam ID and/or Rx beam ID is used as AI/ML inputs

Proposal 3.1-2b

- For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study for potential down selection the following options as AI/ML model inputs

  - Alt 1: No explicit or implicit information of Tx beam ID and/or Rx beam ID

  - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID, e.g., use special value to represent the measurements of the beams (pairs) not in Set B

  - Alt 3: The information of Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly

  - Note: information the Tx beam ID and/or Rx beam ID is not treated as assistance information

| Companies | comments |
| --- | --- |
| Xiaomi | Proposal 3.1-1c In my understanding, for fixed set B, there is no explicit or implicit information of Tx beam ID and /or Rx beam ID. The L1-RSRP of beam(pair)s for input is sorted in ascending order based on beam (pair)s ID. It means there is implicit information on relative value difference between beam ID, not on absolute value of beam ID. Proposal 3.1-2b To align with proposal 3.1-1c, alt 1 and alt 2 can be combined to new Alt 1 “no explicit information of Tx beam ID and/or Rx beam ID is used as AI/ML inputs”. It means the following update proposal: ##### Proposal 3.1-2b - For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study for potential down selection the following options as AI/ML model inputs - Alt 1: No explicit or implicit information of Tx beam ID and/or Rx beam ID - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID, e.g., use special value to represent the measurements of the beams (pairs) not in Set B - Alt 23: The information of Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly - Note: information the Tx beam ID and/or Rx beam ID is not treated as assistance information |
| CAICT | Fine with proposal 3.1-1c and 3.1-2b |
| NTT DOCOMO | We support Proposal 3.1-1c and Proposal 3.1-2b. |
| Fujitsu | support proposal 3.1-1c. and for 3.1-2b, support alt2 and alt3. |
| MediaTek | We are fine with these two proposals. |
| Futurewei | We are ok with Proposal 3.1-1c and 3.1-2b and we are ok if Alt 1 and Alt 2 are combined. |
| Ericsson | Proposal 3.1-1c Share same view as Xiaomi, The L1-RSRP of beam(pair)s for input is ordered based on beam (pair)s ID. When the input to the model is fixed, the order of the beam measurements also needs to be fixed (potentially via some beam identification). In general, we are not sure of why we need these proposals. We don’t see the benefit to distinguish if the IDs are used as input to the model. The IDs are mainly used in the pre-processing step to create representative input to the model. For example, IDs are used to order the input. Or distinguish which beams IDs that are non-hearable, and we need to impute a missing value for such ID. |
| Samsung | We think implicit and explicit information should be further clarified. In our understanding, no explicit or implicit information means how to map measurements in Set B into beam IDs as AI/ML model inputs is unknown. Implicit information means how to map measurements in Set B into beam IDs as AI/ML model inputs is known from CRI/SSBRI. Explicit information means how to map measurements in Set B into beam IDs as AI/ML model inputs is known from CRI/SSBRI and dedicated information from NW for UE-side model or UE for NW-side model. |

Proposal 3.1-3b

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference),

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is changed among pre-configured patterns candidates of Set B patterns

      - For the evaluation, the number of candidates of Set B patterns is no larger than [5]

    - Opt C: Set B is changed among some non-pre-configured subsets of Set A beams (pairs)

    - Opt D: Set B is a subset of measured beams (pairs) Set C, e.g. Top-K beams(pairs) of Set C

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

| Supported by | MediaTek, NTT DOCOMO, Futurewei, CATT, Ericsson, Google, Samsung, CAICT, Fujitsu |
| --- | --- |
| Objected by |  |

Please share your view on proposals

| Companies | comments |
| --- | --- |
| Xiaomi | For Opt D, the motivation to take Top-K beams (pairs) of Set C as input is to avoid the case that the worst beams(pairs) may reduce the prediction accuracy of AI/ML model? If not, why must be the Top-K beams (pairs) of Set C, why not any K beams (pairs) of Set C? |

Proposal 3.1-4b

- For DL Tx beam prediction, when Set B is a subset of Set A, study the following pattern design of Set B of beams when Set B is fixed, at least consider the following options:

  - Alt1: Uniformed distributed in Set A

  - Alt2: Based on statistic information from the collected dataset, FFS on feasibility and details

  - Other alternatives are not precluded.

Proposal 3.1-5a

- For Tx-Rx beam pair prediction, study the following pattern design of Set B of beam pairs, at least consider the following options:

  - Alt1: Sparely uniformed distributed among both Tx beams and Rx beams

  - Alt2: Uniformed distributed among Tx beams for all Rx beams

  - Alt3: Based on statistic information, FFS on feasibility and details

  - Other alternatives are not precluded.

| Companies | comments |
| --- | --- |
| Xiaomi | Proposal 3.1-4b Support Alt 1. While for Alt 2, does it mean to determine some pre-configured patterns for set B base on collected dataset? Proposal 3.1-5a Compared to Alt 2, does Alt 1 mean it is possible that there is no L1-RSRP of at least one Rx beam to be input to the AI/ML model? |
| CAICT | For proposal 3.1-4b and 3.1-5a, uniformed distributed (Alt1) is preferred and fine with the two proposals. |
| Fujitsu | Generally, support these two proposals. But for pattern design of Set B based on statistic information, it needs more clarification on how to do it. |
| Ericsson | Support, the two design alternatives can be summarized into: - Domain knowledge (e.g. uniformly over set A) - Data driven (determine set B after collecting the dataset of set A) |

3rd round

Proposal 3.1-2b

- For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study for potential down selection the following options as AI/ML model inputs

  - Alt 1: No explicit or implicit information of Tx beam ID and/or Rx beam ID

  - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID

  - Alt 3: The information of Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly

  - Note: The information of Tx beam ID and/or Rx beam ID is not treated as assistance information

| Support to remove Alt1 |  |
| --- | --- |
| Must keep Alt 1 |  |
| Support the proposal |  |

| Companies | comments |
| --- | --- |
| FL | For fixed Set B, companies are encouraged to describe the inputs format. For variable Set B, in FL’s view, it does not work well with Alt 1. Please indicate whether Alt 1 can be removed or not. |
| OPPO | For variable Set B, we don’t know how Alt.1 (no information for beam IDs) is going to work. |

Proposal 3.1-3b

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference),

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is changed among pre-configured patterns candidates of Set B patterns

      - For the evaluation, the number of candidates of Set B patterns is no larger than [5]

    - Opt C: Set B is changed among some non-pre-configured subsets of Set A beams (pairs)

    - Opt D: Set B is a subset of measured beams (pairs) Set C, e.g. Top-K beams(pairs) of Set C

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

| Supported by | MediaTek, NTT DOCOMO, Futurewei, CATT, Ericsson, Google, Samsung, CAICT, Fujitsu, LG Electronics |
| --- | --- |
| Objected by |  |

Proposal 3.1-4b

- For pattern design of Set B, further study the performance for the following options:

  - Option 1: Domain knowledge (e.g. uniformly over set A)

  - Option 2: Data driven (determine set B after collecting the dataset of set A)

  - Companies report the patterns and design principle of Set B, if applicable.

| Supported by |  |
| --- | --- |
| Objected by |  |

| Companies | comments |
| --- | --- |
| OPPO | Could we consider the last bullet (pasted as below) as Option 3? - Companies report the patterns and design principle of Set B, if applicable. |

  - Rx beams assumption

Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per model input sample - Option 2b: Measurements of specific Rx beam(s) for all model input sample - FFS how to select the specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample - Other options are not precluded and can be reported by companies.

| Company | Proposals |
| --- | --- |
| FUTUREWEI [1] | Observation 2: For spatial-domain DL Tx beam prediction sub use case, when using either fixed Set B beam pattern or variable Set B beam pattern sampling approach, no significant performance difference is observed when the AI/ML model is trained using a pre-identified Rx beam then performs inference using measurements for a different pre-identified Rx beam. |
| Huawei/HiSi[2] | Observation 6: For the Rx beam selection in case of AI/ML-based DL Tx beam prediction there are various implementation options to optimize the Rx beam to trade latency, overhead and RSRP performance, e.g., - Option 1: Fixed Rx beams is used for inference during P-1/P-2 and the Rx beam sweeping is performed to determine the Rx beam in P-3. - Option 2: A quasi-optimal DL Rx beam can be identified by measuring the always-on SSB beams at P-1 and used for Tx beam prediction at P-2. - Option 3: Exhaustive Rx beam sweeping is swept over multiple P-1/P-2 rounds each of which predicts the best Tx beam for a specific Rx beam. Proposal 12: For AI/ML based DL Tx beam prediction BM-Case1 and BM-Case2, it should be studied how to select a quasi-optimal Rx beam without [substantially] increasing the overhead and its performance should be evaluated. Observation 11: Using prior information (e.g., by measuring the always-on SSB beams) on the Rx beam selection to derive a quasi-optimal Rx beam can come very close in performance to always using the genie-aided best Rx beam obtained from an exhaustive sweep. - If the determination of the quasi-optimal Rx beam is of 90% accuracy, the prediction accuracy is 96.1% (of Top-5) vs 97.8% for genie-aided best Rx beam and the L1-RSRP is -0.11dB vs -0.05dB for genie-aided best Rx beam. - If the determination of the quasi-optimal Rx beam is of 80% accuracy, the prediction accuracy is 94.3% (of Top-5) vs 97.8% for genie-aided best Rx beam and the L1-RSRP is -0.18dB vs -0.05dB for genie-aided best Rx beam. |
| ZTE[3] | - Case 1-2-1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample. - Case 1-2-2: Measurements of a specific Rx beam per model input sample, i.e., the specific Rx beam is changed from the first Rx beam to the fourth Rx beam on the first UE panel as the input sample changes, and followed by recycling. - Case 1-2-3: Measurements of a specific Rx beam per model input sample, i.e., the specific Rx beam is changed from the first Rx beam to the eighth Rx beam on the two UE panels as the input sample changes, and followed by recycling. - Case 1-2-4: Measurements of a specific Rx beam for all model input samples, i.e., the first Rx beam on the first UE panel is always used. Observations 3：For DL Tx beam prediction, the best performance is achieved by measurements of the “best” Rx beam (i.e., Option 1) among all options of Rx beam assumptions. Besides, Option 2a (i.e., measurements of specific Rx beam(s) per model input sample) and Option 2b (i.e., measurements of specific Rx beam(s) for all model input samples) have similar performances, which is slightly lower than that of Option 1. |
| Ericsson [4] | - Observation 13 The information of UE Rx beam plays a vital role on prediction KPIs in DL Tx beam prediction. Therefore, it is important to address the impact of Rx beam selection For DL Tx beam prediction, Option 3 with random Rx beam per model input sample achieves very poor performance. |
| Spreadtrum [5] | Proposal 4：For DL Tx beam prediction, - For option 1, how to determine the “best” Rx beam with less overhead should be further considered. - For option 2, the definition of “specific Rx beam(s)” needs to be further explained. |
| OPPO[6] | Proposal 10: For DL Tx beam prediction, adopt the “best” Rx beam (Option 1) with exhaustive beam sweeping for each model input sample. |
| Vivo [8] | Observation 14: Beam pair prediction provides obvious performance improvement in KPI of beam prediction accuracy for top 4/1 beam, while similar performance can be observed in other KPIs for beam pair prediction and DL Tx beam prediction with multiple Rx beams where each Tx beam uses its own best Rx beam. Proposal 13: Further study the benefit of DL Tx beam prediction with multiple Rx beams where each Tx beam uses its own specific Rx beam. Observation 15: A considerable performance deterioration can be found in DL Tx beam prediction with a fixed specific Rx beam for all model input samples compared to beam pair prediction scheme. Proposal 14: DL Tx beam prediction with a randomly selected specific Rx beam for all model input samples shall be deprioritized. Observation 16: The performance of DL Tx beam prediction with the best Rx beam searched from total beam pairs provides considerable improvement, as decreased prediction difficulty from predicting 256 beam pairs to 32 beam pairs by acquiring precise best Rx beam of each sample. Observation 17: Zero or near zero beam prediction accuracy can be found in DL Tx beam prediction with 2nd best Rx beam or worst Rx beam searched from total beam pairs. Proposal 15: Study on how to search best Rx beam from total beam pairs for each model input sample and its feasibility in real network. Observation 18: Both beam pair prediction and DL Tx beam prediction with a specific Rx beam searched from a specific Tx beam for each model input sample has its own application targets. Observation 19: Similar performance can be observed for DL Tx beam prediction cases with best Rx beam searched from different Tx beams. Proposal 16: Support both beam pair prediction scheme and DL Tx beam prediction scheme for different AI based beam prediction targets. Proposal 17: Support DL Tx beam prediction with best Rx beam searched from a Tx beam for each model input sample, and how to define this Tx beam can be FFS. |
| Xiaomi [10] | Observation 9: AI model for DL Tx beam prediction in spatial domain with one specific Rx beam results in large performance loss compared to exhaustive beam pair sweeping. Proposal 5: Not support DL Tx beam prediction with specific Rx beam considering the low beam prediction accuracy. Observation 10: AI model for DL Tx beam prediction in spatial domain with the best Rx beam provide better performance compared to exhaustive beam pair sweeping because of the low model complexity. Proposal 6: Support DL Tx beam prediction with the best Rx beam considering the high beam prediction accuracy. |
| Nokia [12] | Observation 16: Selecting the Rx beam by following the configured QCL-D info may or may not be the optimum choice for the beam pair prediction. Proposal 7: Advanced Rx beam selection procedure other than following the configured QCL-D info should be considered for beam pair prediction. |
| CATT [13] | Observation 5: The performance of randomly selecting Rx beam is worse than that of selecting the best Rx beam, but selecting the best Rx beam requires additional selection steps, which will increase the complexity of UE implementation. |
| Fujitsu [15] | Proposal 1: For the selection of Set B of beams, option 1 is suggested as starting point for evaluation. Proposal 2: Regarding the Opt A and Opt B of variable Set B, Opt B is suggested for BM-case 1 and Opt A is suggested for BM-case 2. Proposal 3: Regarding the Opt C of variable Set B, it’s suggested to further study on how to construct the training dataset. |
| CMCC [18] | Observation 3: For BM-Case1, DL Tx beam prediction with the “best” Rx beam achieves higher prediction accuracy than that of DL Tx beam prediction with specific Rx beam. |
| Samsung [21] | Proposal # 12: Companies report the time domain RS assumption for both spatial domain and temporal beam prediction. Figure 2 Example to obtain the best Rx beam with SSB Figure 3 Example to obtain the best Rx beam with CSI-RS in one shot Observation # 23: Using the L1-RSRP of the “best” Rx beam with exhaustive beam sweep as inputs can provide the best performance for the accuracy of Top-1/N beam prediction than fixed or randomly selected one or two Rx beams with fixed or random Tx beams for BM-Case 1. Observation # 24: With L1-RSRPs of fixed Rx beam(s) as AI inputs can provide better performance than L1-RSRP of random Rx beam(s) for DL Tx beam prediction for BM-Case 1. Proposal # 10: Consider the following cases for specific Rx beam(s) selection. - Case 1: One fixed Rx beam with given index. - Case 2: One random Rx beam - Case 3: Two fixed Rx beam from one panel - Case 4: One fixed Rx beam from each panel Proposal # 11: For the evaluation assuming measurement with best Rx beam, RS overhead to obtain the best Rx beam is reported by companies. |
| DoCoMo [24] | Proposal 1: Study the Rx beam(s) determination methods for the Set B measurement for Tx beam prediction. Simulation Case 0: Tx beam prediction with best Rx beam for the best Tx beam within Set B. Simulation Case 1: Tx beam prediction with the best Rx beam for each Tx beam within Set B Simulation Case 3: Tx beam prediction with the best Rx beam among specific Rx beams for each Tx beam within Set B Observation 1: For Tx beam prediction with Rx beams assumption Option 1: - The beam prediction accuracy as well as average L1-RSRP are obviously better than baseline. - The “best” Rx beam for the best Tx-Rx beam pair in Set C provides better performance than the “best” Rx beam for each Tx beam in Set C. |

1st round

Proposal 3.2-1a

- To obtain the measurements by the “best” Rx beam, further study the following cases:

  - Case 0: the best Rx beam for the best Tx beam within Set B.

  - Case 1: the best Rx beam for each Tx beam within Set B

  - Case 3: the best Rx beam among specific Rx beams for each Tx beam within Set B

| Companies | comments |
| --- | --- |
| FL0 | For Case 3, what kind of information do we expect? Performance with limited Rx beam sweeping? |
| MediaTek | We are fine with the proposal, but we prefer Case 1. |
| Xiaomi | For Case 0, we would like to clary the definition for “the best Tx beam” first. Opt 1 is that it is the best Tx beam with a specific Rx beam. Opt 2 is that it is the best Tx beam among all Tx Rx beams. the RS overhead for determination of the best Rx beam with Opt 1 is lower than that of Opt 2. And the RS overhead for determination of the best Rx beam with Opt 2 is same as that in case 1. For Case 3, we would like to clarify the difference between it and Case 2. How to define the specific Rx beams? it may depend on UE implementation. |
| vivo | We think there should be another case to be added, which means the best Rx beam that results in the largest RSRP among all the Tx-Rx beam pairs. It can be a baseline as it is ideally the best Rx beam. Case 4: The best Rx beam among all the Tx-Rx beam pairs. |
| NTT DOCOMO | Support. Case 3 considers the case where the limited Rx beam sweeping is performed instead of exhaustive sweeping. When all Rx beam may not be available for certain time due to UE implementation (e.g., certain panel is not available) or RS resources are not sufficiently configured to perform beam sweeping with all Rx beams, Rx beam would be limited for Rx beam sweeping. Case 3 can evaluate the performance even in that scenario. In addition, if Rx beams are assumed in the Tx-Rx beam sweeping in P1 to obtain the measurement results for the input of AI/ML model. In this case, the additional Rx beam sweeping will be required after the beam prediction. Case 3 covers that scenario as well. |
| CATT | We also have the same comments on Case 3 as FL. Seems Case 3 is a special case of Case 1. |
| Ericsson | Prefer case 1. |
| InterDigital | We prefer to have better understanding and provide the following questions. Q1. What would be the intention of this proposal? Is it to align the evaluation assumptions or to decide a method for deciding the best Rx beam? Q2. Is Case 0 practical? How does the UE decide the best Tx beam without deciding the best Rx beam? Maybe, the best Tx beam is a previously predicted best Tx beam? |
| LG Electronics | Fine with the proposal. |
| Spreadtrum | We are a little confused about case 1 and case 3. If you are looking for best Rx beam for each Tx beam, multiple best Rx beams may appear at this time. What are the advantages of finding an optimal Rx beam (case 0), we hope the proponents to clarify |
| Google | OK in principle. But we think we do not need to spend too much effort studying Rx beam, since this is anyway transparent. |
| Samsung | We prefer Case 1 |
| Apple | Is the clarification for Option 1 from the previous meeting? |

Proposal 3.2-2a

- At least for the “best” Rx beam, considering the following options with potential down selection:

  - Opt A: “beam” Rx beam is obtained by measuring the always-on SSB

    - 20ms periodicity of each SSB burst is used for the evaluation

  - Opt B: “beam” Rx beam is obtained by measuring the CSI-RS

    - [1ms] periodicity of CSI-RS with repetition-on is assumed for the evaluation.

  - Opt C: “beam” Rx beam is obtained ideally among measurements generated in one shot

| Companies | comments |
| --- | --- |
| FL0 | Based on some companies’ evaluation, the assumptions to obtain the “Best” Rx beam matter the performance. Opt C seems not practical, but Opt B may provide similar performance as Opt C. Moreover, in FL’s understanding, if the way to obtain “best” Rx beam has performance difference, the assumption to obtain the measurements may lead to different performance for both Tx beam and Tx-Rx beam pair prediction. Please share your views. |
| MediaTek | We prefer Opt B. Also, we think “beam” should be “best”, maybe it is a typo. |
| Xiaomi | We agree that the way to obtain the best Rx beam may lead to difference performance. And we think it can be reported by companies as well as the RS overhead for obtaining best Rx beam. |
| HW/HiSi | Option A and Option B can be studied further. For Option C we need to understand it better. Does it mean that all measurements on all Tx beams are carried out in one shot? |
| vivo | OK to study this. |
| ZTE | Not sure the intention of this proposal. Prefer to let companies report. |
| NTT DOCOMO | Opt A and Opt B may have different beam prediction performance. Opt B and Opt C may have different results on RS OH reduction and/or latency reduction. We support Opt A and Opt B to fairly compare both the performance and overhead reduction. Regarding Opt A, even within one SSB burst, the exact measurement occasions are different according to SSB. In our view, the time gap between SSB is trivial so it is fine to ignore the point (assuming all SSB are measured at the same time resource within one SSB burst). However, it would be better if companies can have the same assumption even on that point. |
| CATT | We prefer Opt B. We agree FL comment that we need to select one option for the best Rx beam. We also agree Option C is not practical. Since in AI based beam management, the aim is to predict the beam of CSI-RS. Thus, the best Rx beam should be obtained by measuring the CSI-RS, not SSB. |
| Ericsson | Ok to study. |
| InterDigital | As well as the previous proposal, we failed to understand the motivation of this proposal. If it is for evaluation assumption, then using reported method by each company should be enough. If it is not, then please clarify your intention. |
| LG Electronics | Prefer option B. |
| Google | OK in principle. But we think we do not need to spend too much effort studying Rx beam, since this is anyway transparent. |
| Samsung | We think opt A and opt B are reasonable assumption for the evaluation. |
| Apple | We don’t support the proposal as the assumption for Option B is not realistic (too frequent measurement) and Option B needs clarification (“CSI-RS” here is not clear what it exactly refers to, with repetition on or without repetition on? And a key thing is the measurement overhead, then the RS overhead needs to be clarified) |
| OPPO | Slightly prefer Option B. It relies on the existing CSI-RS based beam sweeping to find the best Rx beam. As for Option A, it takes much longer delay (20*X) to find the best Rx beam, where X stands for the Rx beams per panel. For Option C, we agree with FL that it seems impractical. |

2nd round

Proposal 3.2-1b

- To obtain the measurements by the “best” Rx beam for evaluation, further study the following cases:

  - Case 0: the best Rx beam for the best Tx beams within Set B.

  - Case 1: the best Rx beam for each Tx beam within Set B

  - Case 3: the best Rx beam among specific Rx beams for each Tx beam within Set B

  - Case 4: the best Rx beam among all the Tx-Rx beam pairs with in Set A

| Supported by | CAICT,NTT DOCOMO(concern on Case 4), MediaTek (Case4*), Futurewei |
| --- | --- |
| Objected by |  |

Proposal 3.2-2a

- At least for the “best” Rx beam, considering the following options with potential down selection:

  - Opt A: the “best” Rx beam is obtained by measuring the always-on SSB

    - 20ms periodicity of each SSB burst is used for the evaluation

  - Opt B: the “best” Rx beam is obtained by measuring the CSI-RS

    - [1ms] periodicity of CSI-RS with repetition-on is assumed for the evaluation.

  - [Opt C: the “best” Rx beam is obtained ideally among measurements generated in one shot]

| Supported by | CAICT, NTT DOCOMO, MediaTek |
| --- | --- |
| Objected by |  |

| Companies | comments |
| --- | --- |
| FL | @vivo, pls check whether Case 4 is what you want? |
| Xiaomi | Proposal 3.2-1b First we suggest to add “for AI/ML model input” before “for evaluation” like the following ##### Proposal 3.2-1b To obtain the measurements by the “best” Rx beam for AI/ML model input for evaluation, further study the following cases: Second, for Case 0, we would like to clary the definition for “the best Tx beam” first. Opt 1 is that it is the best Tx beam with a specific Rx beam. Opt 2 is that it is the best Tx beam among all Tx Rx beam pairs. the RS overhead for determination of the best Rx beam with Opt 1 is lower than that of Opt 2. And the RS overhead for determination of the best Rx beam with Opt 2 is same as that in case 1. For Case 3, we would like to clarify the difference between it and Case 2. How to define the specific Rx beams? it may depend on UE implementation. For case 4, since it is about the measurements for AI/ML model input, we think it is not necessary to consider the best Rx beam in set A. BTW, the case 0, case 1, case 3, case 4 can be changed to case 0, case 1, case 2, case 3. Proposal 3.2-2a We still prefer to report by companies on how to obtain the best Rx beam. For Opt A, if there are 8 Rx beam, 160ms is needed to obtain the best Rx beam. which leads too long delays. For Opt B, it means to find the best Rx beam for a specific Tx beam? or will find the best Rx beam for each Tx beam? |
| NTT DOCOMO | As the input to AI/ML model, the measurement should be based on Set B. We are not clear about the motivation of the beam sweeping across all Tx-Rx beam pairs, i.e. Set A. |
| Fujitsu | The motivation of proposal 3.2-2a is for study how to select the best Rx beam for Tx beam prediction. We are OK for Opt A and B. But for proposal 3.2-1b, it’s not clear for its motivation. Our understanding is FL want to clarify what’s kind of the measurement results with best Rx beam will input to AI/ML model. In other words, how to construct the set B with best Rx beam. If our understanding is correct, why the measurement results by best Rx beam are from Set B (e.g. , in case 0/1/3, “…… within Set B”)? it seems the measurement results by best Rx beam are from Set A for evaluation if set B is subset of Set A. (e.g., for in case 0/1/3. “…… within Set A). |
| MediaTek | For the newly added Case 4 in Proposal 3.2-1b, current wording will make Case 4 only applicable to beam pair prediction only (Set A consists of beam pairs). We would like to suggest a new wording below: o Case 4: the best Rx beam for the best Tx beams among all the Tx-Rx beam pairs with in Set A With this wording, it covers both what VIVO wants and the DL Tx beam prediction case. The best Rx beam for the best Tx beams in Set A, with the same best Tx beams, automatically formulate the best Tx-Rx beam pairs. |
| Samsung | Regarding Proposal 3.2-1b, we are okay to study Case 1 and Case 2. But it is unclear how to fairly evaluate Case 3 among companies. Also it is unclear for us how to obtain the best Rx beam with in Set A. We suggest to delete Case 3 and 4. Regarding Proposal 3.2-2a (maybe 3.2-2b), since Opt A, Opt B, and Opt C result in different performance owing to different measurement aging during Rx beam sweeping, we need to align the assumption for fair comparison. Thus, we are fine to consider all options for potential down selection. |

3rd round

Proposal 3.2-1d

- For the “best” Rx beam” determination of “Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample”, further study the following cases:

  - Case 0: the best Rx beam for the best Tx beams within Set B.

  - Case 1: the best Rx beam for each Tx beam within Set B

  - Case 3: the best Rx beam among specific Rx beams for each Tx beam within Set B

  - Case 4: the best Rx beam for the best Tx beam among all the Tx-Rx beam pairs with in Set A

| Supported by | CAICT, NTT DOCOMO (concern on Case 4), MediaTek (Case4*), Futurewei |
| --- | --- |
| Support to remove Case 4 |  |
| Objected by |  |

Proposal 3.2-2a

- At least for the “best” Rx beam, considering the following options with potential down selection:

  - Opt A: the “best” Rx beam is obtained by measuring the always-on SSB

    - 20ms periodicity of each SSB burst is used for the evaluation

  - Opt B: the “best” Rx beam is obtained by measuring the CSI-RS

    - [1ms] periodicity of CSI-RS with repetition-on is assumed for the evaluation.

  - [Opt C: the “best” Rx beam is obtained ideally among measurements generated in one shot]

| Supported by | CAICT, NTT DOCOMO, MediaTek |
| --- | --- |
| Objected by |  |

| Companies | comments |
| --- | --- |
| FL | Proposal 3.2-1d is to further define what is the “best Rx beam” in previous agreements of. Please check the updates Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per model input sample - Option 2b: Measurements of specific Rx beam(s) for all model input sample - FFS how to select the specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample Other options are not precluded and can be reported by companies. |
| vivo | Re the current formulation of 3.2-1d, we would like to understand the details on the listed cases. For Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample, our understanding is for each model input sample, there is only one best Rx beam is used per the definition. Then - For Case 1 “the best Rx beam for each Tx beam within Set B”, it means which of the following two. Case 1a: The best Rx beam searched from one Tx beam within Set B. Basically it means we can use one certain Tx beam to do P3 first, and then we use the searched best Rx beam for AI based P2. Case 1b: Each Tx beam in Set B uses its own best Rx beam We think it violates the definition of Option 1 as it will cause multiple Rx beams for each model input sample. It’s better we can clarify this for Case 1. We are okay to list both cases. Then it is better not to define these cases only for Option 1 in previous agreement. - For Case 4, we think it is beneficial to evaluate the upper bound performance for DL Tx beam prediction as well. Then “among all the Tx-Rx beam pairs” shouldn’t be removed. If the concern is DL Tx beam prediction case does not have Tx-Rx pair in Set A, it is okay to remove Set A as it already specifies that this is the best Rx beam among all beam pairs. |

  - Comparison between DL Tx beam and Tx-Rx beam pair

| Company | Observation |
| --- | --- |
| Huawei/HiSi[2] | Observation 15: The comparison of DL Tx beam prediction and DL Tx-Rx beam pair prediction under BM-Case 1 shows that beam prediction has better performance, e.g. - For prediction accuracy (0dB RSRP difference) for Top-5, Tx-Rx beam pair: 93.5%; Tx beam (90% best Rx): 96.5%; Tx beam (best Rx): 97.5%. - 1dB RSRP difference for Top-5, Tx-Rx beam pair: 94.5%; Tx beam (90% best Rx): 97%; Tx beam (best Rx): 98.5%. |
| ZTE[3] | Besides, even with a same sampling rate on the whole beam space, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction. As can be concluded from the simulation results, the number of Rx beams is relatively small and any sampling at the Rx beam space may cause serve loss of the spatial channel characteristic. Thus, in the beam pair prediction, it’s better to take measurement results of all Rx beams as the model input instead of performing any sampling at the Rx beam space Observations 4：For Tx-Rx beam pair prediction and DL Tx beam prediction, similar beam prediction performance is achieved with different Set B patterns in the same sampling rate. Observations 6：For DL Tx beam prediction, marginal performance loss is observed when the AI model inputs are Top 8 beams or Top 4 beams of all 16 beam measurements. Observations 7：For Tx-Rx beam pair prediction, significant performance loss is observed when the AI model inputs are Top 8 beams or Top 4 beams of all 128 beam pair measurements. |
| OPPO[6] | Observation 6: The accuracy of Tx beam prediction is as high as 92.6% and the L1-RSRP difference of it is as small as 0.13dB. Tx beam prediction can achieve even better prediction performance than that of beam pair prediction. |
| Vivo[8] | Observation 14: Beam pair prediction provides obvious performance improvement in KPI of beam prediction accuracy for top 4/1 beam, while similar performance can be observed in other KPIs for beam pair prediction and DL Tx beam prediction with multiple Rx beams where each Tx beam uses its own best Rx beam. Observation 15: A considerable performance deterioration can be found in DL Tx beam prediction with a fixed specific Rx beam for all model input samples compared to beam pair prediction scheme. |
| Nokia [12] | Proposal 8: Investigate the feasibility to measure the top-K predicted beam pairs since it is needed for improving the model performance for the Tx-Rx beam pair prediction. Proposal 9: To support RAN1 comparing DL Tx-Rx beam pair prediction (substitutuing P2-P3) and Tx beam prediction companies may report the assumptions for obtaining top-K predicted/measured Tx-Rx beam pairs with Tx beam prediction. Observation 17: With the same RS resources, the DL Tx beam prediction has better beam performance regarding the model intermediate KPIs. |
| Samsung [21] | Observation # 34: For DL Tx beam prediction in BM-Case 1, L1-RSRPs with implicit Tx beam index as AI inputs and best Tx beam as AI outputs and can provide a better performance than with L1-RSRPs with implicit Tx beam index and Rx beam index as AI inputs and best Tx-Rx beam pair as AI outputs. |

1st round

Proposal 3.3-1a

- For performance comparison between DL Tx beam prediction and Tx-Rx beam pair prediction, take the following options as a starting point:

  - Option 1: Tx-Rx beam pair prediction accuracy with the same RS resources

    - For the case using AI/ML to predict DL Tx beam, P1 or P3 may be needed to obtain the best Rx beam of the predicted Tx beam

    - FFS on other details

  - Option 2: DL Tx beam prediction accuracy with the same RS resources

    - For the case using AI/ML to predict Tx-Rx beam pair, DL Tx beam prediction accuracy only counting the percentage of the predicted Tx beam(s) is correct or not regardless Rx beam

    - FFS on other details

  - Other options are not precluded

| Company | Proposal/observation |
| --- | --- |
| FL0 | Whether/how to compare the DL Tx beam and Tx-Rx beam pair prediction, e.g. - Top 1 DL Tx beam prediction and measurement/RS overhead - RS overhead with DL Tx beam prediction + RS sweeping vs Tx-Rx beam pair prediction - … |
| MediaTek | We don’t support this proposal. We don’t see any reason to compare between DL Tx beam prediction and Tx-Rx beam pair prediction. |
| Xiaomi | It is not fair to use Option 2 since with same RS resource, the DL Tx beam prediction accuracy for DL Tx beam prediction with the best Rx beam for each Tx beam is higher than that of Tx/Rx beam pair prediction by our evaluation. But there is no predicted Rx beam for DL Tx beam prediction. We can consider Option 1 as a starting point. |
| Hw/HiSi | @Mod: Do you mean to compare Option 1 and Option 2 with the same RS resources? We should evaluate model output of Top-K where K is larger than 1 for both approaches. |
| vivo | Support Opt 1. As the final goal of BM is to find the best Tx-Rx beam pair either with a P1, P2+P3 or P3+P2 procedure, evaluation follow Opt 1 can achieve this final goal. |
| ZTE | Not support. The DL Tx beam prediction and beam pair prediction are used in different scenarios. Therefore, we don't need to merge these two cases together or perform any comparison between them. Specifically, the DL Tx beam prediction is used to predict an optimal Tx beam with the assumption of specific Rx beam during one round of inference, which can well match the P2 beam refinement procedure in the current specification. The beam pair prediction is used to predict an optimal Tx-Rx beam pair out of all available Tx-Rx beam pairs, which corresponds to the P1 beam selection procedure. |
| NTT DOCOMO | We slightly prefer Option 1. The point is same Rx beam assumption should be required to compare the Tx beam prediction and Tx-Rx beam pair prediction under the same resources. For example, if Tx{a,b,c,d} * Rx{0,3} are assumed for the Tx-Rx beam pair prediction, the Rx beam for each Tx beam in Tx beam prediction should be from Rx{0,3}. In this case, we think the UE behavior in P1 would be the same for Tx beam prediction and Tx-Rx beam pair prediction. |
| Lenovo | In beam pair prediction, we determine both the Tx beam and the Rx beam. In Tx beam prediction, we determine only the Tx beam, but not the Rx beam. Thus, it would not be fair to compare beam pair prediction with only single beam prediction method. If we want to compare the two, then we should do the following: Assume that the Tx-Rx beam pair prediction consumed certain amount of RS overhead, say X and given us an accuracy of Y in predicting the Tx-Rx beam pair. Then, comparison could be done in one of the two ways: - Using Tx beam prediction and Rx beam prediction separately, with the amount of RS overhead equal to X, determine the accuracy of Tx beam prediction and Rx beam prediction and compare that with Y. To achieve the same prediction accuracy of Y, using Tx beam prediction and Rx beam prediction separately, determine the RS overhead and compare that with X. |
| CATT | We are fine with this proposal. |
| Fujitsu | It’s not clear the motivation for such comparison. If such comparison is necessary, we slightly support Option 1. |
| LG Electronics | Agree with MTK and ZTE. |
| Spreadtrum | Not support. We believe that Tx bean prediction and beam pair prediction are the two types that can be supported by AI models. Due to the different application scenarios, there is no comparison. And the Tx beam prediction depends heavily on the Rx beam selection, so comparing the two may not be meaningful |
| Google | OK with option 2 |
| OPPO | We doubt that the KPI (e.g. beam prediction accuracy) of DL Tx beam prediction and Tx-Rx beam pair prediction are comparable. By default, given the same Tx beams at NW side, it would easier to correctly predict the best Tx beams (not including the Rx beam) than to predict the Tx-Rx beam pair. |

3rd /2nd round

Question 3.3-1a

Do you support the comparison for DL Tx beam prediction and Tx-Rx beam prediction or not?

| support comparison | NTT DOCOMO, Samsung |
| --- | --- |
| Not support comparison |  |

| Company | Proposal/observation |
| --- | --- |
| FL0 | Please indicate whether need to compare the performance of Tx beam and Tx-Rx beam pair. If yes, how? |
| NTT DOCOMO | It is good to observe the performance difference between Tx beam prediction and Tx-Rx beam pair given the same condition on RS resource configuration and UE behavior on measurement. Therefore, we slightly prefer Option 1. |
| Samsung | We think feasibility of Tx-Rx beam pair prediction firstly needs to be checked before the comparison. For example, based on evaluation, we observe DL-Tx beam prediction at NW-side looks feasible in current NR system, while the feasibility of DL-Tx beam prediction at UE-side is still discussing (e.g., variable Set B Opt C). Similar to this, under variable Set B, the feasibility of DL Tx-Rx beam pair prediction at UE-side (i.e., subset of Set B for Tx beam is variable) or NW-side (i.e., subset of Set B for Rx beam is variable) is questionable in current NR system. |

  - Additional assumptions for BM-Case 2

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi [2] | Proposal 13: For temporal domain beam prediction, regarding the relationship between Set A and Set B: - The size of Set B smaller than Set A should be considered as baseline. - Both can be considered in evaluations: Set B is a subset of Set A; Set B contains wide beams with full direction which are different from Set A with narrow beams. - Set B equal to Set A can be optionally used for performance comparison in evaluations. Observation 18: For temporal domain beam prediction, with comparable total overhead for temporal DL Tx beam prediction, longer prediction interval with larger number of Top-K candidates can achieve better performance than shorter prediction interval with a smaller of Top-K candidates. Proposal 15: For temporal domain beam prediction, study the trade-off over different prediction window lengths and different number of inferred Top-K candidates in terms of overhead and performance. |
| Spreadtrum [5] | Proposal 6: For temporal beam prediction, "Set A and Set B are the same" should be used as the baseline for simulation evaluation of subsequent schemes. Proposal 7: For the AI/ML model input of BM-Case 2, - Alt 1 and Alt 3 should be studied with high priority. - Whether to choose Alt 1 or Alt 3 needs further discussion according to the beam pattern selection. - If Set B is fixed, Alt 1 will be selected; - If Set B is variable, Alt 3 will be selected. - For the corresponding beam ID in Alt 3, it should be input in AI model explicitly. - Alt 2 should be clarified which assistance information can be used as AI model input. |
| Xiaomi [10] | Proposal 7: At least for BM-Case 2, consider the following assumptions for evaluation - Periodicity of time instance(s) for prediction: - 20ms, 40ms, 80ms, 160ms, - Note: the periodicity of time instances for prediction can be same as that for measurement/report, or the periodicity of time instance for each measurement/report can be multiple times of that for prediction. - Other values can be reported by companies. - Number of time instances for prediction: - 1, 2, 4 - Other values can be reported by companies. |
| Nokia [12] | Proposal 13: Support RAN1 to further study BM Case-2 considering observation window larger than prediction window, limiting prediction window to relatively short future period (e.g. 1 s). |
| Intel [17] | Observation 6: Using larger training window length for LSTM model training may achieve slightly better performance on Top-1 beam prediction. Observation 7: The LSTM model is more resilient to the increase of measurement periodicity than sample-and-hold method. Observation 8: Increasing the prediction window size leads to worse prediction accuracy for both the LSTM mode and the baseline scheme. |
| Qualcomm [23] | Proposal 2 For BM-Case2, adopt the following notation: MxPy: AI/ML model is given as input L1-RSRP measurements from x contiguous beam management cycles out of every x+y contiguous cycles, then provides predictions for the following y cycles. |
| DoCoMo [24] | Proposal 2: Consider both Pattern A and Pattern B for temporal beam prediction. Figure 8. T1 and T2 patterns in temporal beam prediction. (a) prediction of beam quality between each measurement/reporting (b) prediction of beam quality instead of measurement/reporting. |

Question 3.4a

| Company | Comments |
| --- | --- |
| FL0 | Please share you view if you think any of the above aspects is important and requires further agreements for the evaluation. |
| Xiaomi | First, we prefer to take set B = set A as a baseline for BM Case 2. Second, we prefer to support the following proposal Proposal 7: At least for BM-Case 2, consider the following assumptions for evaluation - Periodicity of time instance(s) for prediction: - 20ms, 40ms, 80ms, 160ms, - Note: the periodicity of time instances for prediction can be same as that for measurement/report, or the periodicity of time instance for each measurement/report can consist of multiple times of that for prediction. - Other values can be reported by companies. - Number of time instances for prediction: - 1, 2, 4 - Other values can be reported by companies. |
| HW/HiSi | It has been seen that the prediction accuracy increases for Top-K when K also increases. At the same time, the overhead increases also due to additional sweeping. It would be interesting to evaluate what is better, having longer prediction periods with a larger K, or shorter prediction periods with smaller K. |
| NTT DOCOMO | In temporal beam prediction, there are two patterns for T1 and T2: prediction of beam quality between each measurement/reporting (pattern A) and prediction of beam quality instead of measurement/reporting (pattern B). Even though both patterns can reduce the overhead of measurement/reporting, one approach can lead to the large periodicity of measurement/reporting, while the other approach could enable skipping of measurement/reporting for a while. As both approaches bring the practical gain and the desired pattern can be different according to NW operation, it is beneficial to study both patterns for T1 and T2. |

  - Assumptions for Set B=! Set A

| Company | Proposal/observation |
| --- | --- |
| Nokia [12] | Proposal 2: For BM-Case1, RAN1 may further study the case of Set A/B are DL Tx and Set B/Set A are different. - Set B is a wide beam codebook and Set A is a refined beam codebook - Advance Set B designs are needed to provide sufficient refined beam prediction performance. |

Question 3.5a

| Company | Comments |
| --- | --- |
| FL0 | Please share you view if you think any of the above aspects is important and requires further agreements for the evaluation. |
| Hw/HiSi | Or our understanding, this is for Set B used wide-beams and Set A narrow beams, right? If yes, then we think that this can be studied further but not sure if any agreements are needed for that. We have them already in place according to our understanding. |
| NTT DOCOMO | We support to study the Set B is a wide beam codebook. |

  - Assistance information

Assistance information were discussed and some observations are summarized:

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi[2] | Proposal 7: For the evaluation of assistance information, - Information that in Rel-17 is regarded as proprietary and/or privacy shall not be considered as a candidate for assistance information, e.g. UE location, UE moving direction, NW-side beam shape information (e.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc.) - The benefits of other potential assistance information should be evaluated in 9.2.3.1 firstly, before a study on potential spec impact is conducted in 9.2.3.2 |
| Ericsson [4] | Observation 3: Using solely location information (i.e. no RSRP measurements), we could achieve almost 90% prediction accuracy of best beam (Top-1). This could be useful in scenarios where RS-transmission is costly, e.g. at high load situations. |
| vivo[8] | Observation 9: Similar performance can be achieved for a fixed pattern in set B with or without assistance information. Observation 10: Compared with Random pattern Set-1, assistance information brings considerable gain in random pattern selection scheme, especially for Tx/Rx beam angle as assistance information. Proposal 7: Assistance information, such as Tx/Rx beam ID or angle in connection with input RSRPs, should be used as AI input at least for random pattern scheme. Observation 11: Such pre-configured patterns in Set B with Tx/Rx beam angle information as input barely suffers performance loss compared with the best beam pattern. Observation 12: Pre-configured beam pattern scheme has potential to approach the performance upper bound, i.e. the best fixed pattern, if the performance of each pattern in top-N best patterns has similar performance of top-1 best pattern. Proposal 8: Support assistance information with both Tx and Rx beam information in pre-configured pattern scheme. Proposal 9: Suggest to use both Tx and Rx beam information as assistance information for further performance improvement for both BM-Case1 and BM-Case2. Observation 13: More flexible AI model deployment for different number of Tx/Rx beams can be achieved through using expected Tx/Rx beam information method with only marginal performance loss. Proposal 10: Study beam pair prediction with expected Tx/Rx beam information as the AI input as one of the solutions for generalization to different number of Tx/Rx beams in BM-Case1. Proposal 11: Further study expected information method in BM-Case2. Proposal 12: Further study multiple expected beam information simultaneously used in AI input. Proposal 19: Further study assistance information, such as beam shape pattern, 3dB beam width, etc., as model input to address performance deterioration for generalization of different beam shapes in BM-Case1. Observation 21: No performance loss can be observed from proprietary protection with mathematical function processing compared to beam prediction using beam angle directly, if a same processing function is maintained for training and inference. Proposal 20: Support proprietary protection mechanism for proprietary/privacy information disclosing issue in BM Case 1. Detailed proprietary protection mechanism can be FFS. Proposal 21: Support to use proprietary processed assistance information as model input to address performance deterioration and sensitive proprietary information disclosure issues in BM-Case1, where a same mapping function is maintained for training and inference. Proposal 25: Further study assistance information, such as beam shape pattern, 3dB beam width, etc., as model input to address performance deterioration for generalization of different beam shapes in BM-Case2. Proposal 26: Suggest to use beam pointing angle or other physical IDs reflecting beam pointing angle information as assistance information for AI model input. Observation 26: For the case using proprietary processed beam angle, beam loss and accuracy degenerate slightly compared to the performance of the case using beam angle directly. Proposal 27: Support proprietary protection mechanism for proprietary/privacy information disclosing issue in BM-Case 2. Detailed proprietary protection mechanism can be FFS. Proposal 28: Suggest to use proprietary processed assistance information as model input to address performance deterioration and sensitive proprietary information disclosure issues in BM-Case2, where a same mapping function is maintained for training and inference. |
| Nokia [12] | Observation 3: For BM-Case1, Set B RSRP may not be sufficient for beam prediction input in certain cases. Observation 4: For BM-Case1, the ML model using as input only RSRP measurements has performances that reduce significantly by changing the number of RSRP measurements from 8 to 4, i.e., further downsampling Set A, from a ratio of ¼ to a ratio of 1/8. Observation 5: For BM-Case1, when the ML model uses the UE angle as the assistance information, it has a better performance than all the other variants. Observation 6: For BM-Case1, the ML model using input RSRP measurements and UE Position has performances that outweigh the performance of the ML model using only RSRP. Observation 7: For BM-Case1, using assistance information like Beam Angle and Beam ID related to the measured beams may not significantly improve the performance of the ML model using as input only RSRP with a fixed pattern. Proposal 1: For BM-Case1, RAN1 further study the use of assistance information at the ML model input. The following assistance information can be prioritized: • the UE position information (for NW side model training/inference). • the UE’s angle relative to a panel array of the gNB (for UE side model training/inference). Observation 31: For the UE speed of 30 Km/h and prediction windows of 40 and 80 ms, the ML model using as input RSRP and assistance info (UE position) does not provide significant gains to the ML model using as input only RSRP. Proposal 14: For BM-Case2, RAN1 further verifies whether there is any use of using assistance information at the input of the ML model for different UE speeds and trajectory. The UE position information can be prioritized for NW side model training/inference. |
| Apple [22] | Observation 1: If explicit Tx beam shape information for different datasets is not available to model trainer, it may be difficult to design AI model to generalize well over different scenarios/configurations. However, acquiring explicit Tx beam shape information at the UE side may be difficult due to concerns on disclosing proprietary information. |
| Qualcomm [23] | For BM-Case1 and BM-Case2, study the performance of AI/ML models with and without incorporating assistance information and compare the performance. - Study the existing trade-offs including overhead required for signalling of assistance information and corresponding performance benefits. - The agreed KPIs related to beam prediction accuracy and RS overhead reduction can be used for performance evaluation. - Examples of such assistance information: gNB beam shape-related, such as information about gNB beam shape, beam boresight directions, 3dB beamwidth, etc. - Study the impact of assistance information on generalization performance. - In addition to the above-mentioned beam shape-related information, assistance information can be in the form of gNB codebook index, etc. Observation 4 For spatial domain beam prediction (BM-Case1), for at least the case in which Set B is a subset of Set A, assistance information from gNB about gNB beam boresight directions and information about gNB antenna array structure is beneficial in boosting spectral efficiency across UEs. #### Observation 9 Signalling of assistance information can have a monumental role in “scenario discovery” and improving model generalization through model switching, for UE-side AI/ML models. |
| MediaTek [25] | Observation 2: Temporal beam prediction by adding additional UE angle information directly to the input of the model did not show significant gains compared to predicting without UE angle information. Proposal 4: Study more scenarios where additional information may improve the temporal beam prediction performance. |

Summary of the proposals/observations on assistance information

| Company | Proposal/observation |
| --- | --- |
| vivo [8] | Tx/Rx beam ID or angle Expected beam information ??? Beam shape pattern, 3dB beam width ,etc |
| Nokia [12] | UE angle UE PoS |
| Qualcomm [23] | gNB beam boresight directions and information about gNB antenna array structure |
| MediaTek [25] | UE angle information no (gain) |

Proposal 3.6-1a

- Further study the needs/benefit with UE position information as the assistance information.

| Supported by | Lenovo, Ericsson |
| --- | --- |
| Objected by | MediaTek, vivo, CATT, Spreadtrum, Samsung |

| Company | Comments |
| --- | --- |
| FL0 | NOTE: Beam ID related information is discussed in Proposal 3.1-1a and proposal 3.1-2a. - Views on Proposal 3.6-1a - Please share your views on eNB beam related information (other than Tx beam ID) as assistance information - Please share your views on UE beam related information (other than Rx beam ID) as assistance information |
| HW/HiSi | - Not support. The UE position is a privacy information that should not be disclosed. - Not support. - Not support. |
| vivo | We generally agree with that privacy information should be carefully protected for assistance information. - We don’t support it. In our view, there is no way that privacy information can be protected if UE positioning is disclosed as the assistance information. 2)&3) We support to use beam shape information (e.g., 3dB beam width) and expected beam information as assistance information to ensure generalization performance and model scalability. Expected beam information is the beam ID or angle which needs to be included in the output. It generally applied to both gNB side and UE side. |
| ZTE | Not support. According to simulation results provided by companies, no obvious performance gain is obtained with UE position information as the assistance information, at least for the current simulation assumptions. Therefore, we don’t need to have this proposal at the current stage. Besides, the introduction of any assistance information needs to consider the proprietary/privacy information disclosure issues, while UE position as well as other beam related information is not expected to be shared with the opposite node. |
| CATT | The beam information and UE position information has proprietary/privacy issue, which can not be disclosed. Moreover, the benefit to use beam information and UE position information in model training and inference is small. Thus, it’s not necessary to study the beam information and UE position information. |
| InterDigital | - No support. As we already have an agreement on this aspect, we don’t see the need to have an agreement for further study. 2) and 3) Support. As UE/gNB needs to predict best beams from not physically transmitted/measured beams, provided beam related information can be a good basis for beam prediction. |
| Google | Do not support the proposal. |
| Apple | Don’t support the proposal. |
| OPPO | Given the high beam prediction accuracy (e.g. 80+%), we don’t there is too much room to improve the KPI by additionally using UE position information. Such assistance information surely would cost much more effort to obtain, e.g. by AI/ML-based approach introduced in 9.2.4.x. |

  - Beam management procedures

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi [2] | Observation 6: For the Rx beam selection in case of AI/ML-based DL Tx beam prediction there are various implementation options to optimize the Rx beam to trade latency, overhead and RSRP performance, e.g., - Option 1: Fixed Rx beams is used for inference during P-1/P-2 and the Rx beam sweeping is performed to determine the Rx beam in P-3. - Option 2: A quasi-optimal DL Rx beam can be identified by measuring the always-on SSB beams at P-1 and used for Tx beam prediction at P-2. - Option 3: Exhaustive Rx beam sweeping is swept over multiple P-1/P-2 rounds each of which predicts the best Tx beam for a specific Rx beam. Figure 3. UE measures the narrow Tx beams with a fixed Rx beam (Option 1) Figure 4. UE measures the wide Tx beams with a quasi-optimal Rx beam (Option2) Figure 5. UE measures the narrow Tx beams for several rounds with different Rx beams (Option 3) |
| ZTE [2] | Proposal 4: In Option 2 (i.e., use AI/ML model to predict Top-K Tx beams in Set A) and Option 4 (i.e., use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A), it is not mandatory to trigger a P2 procedure to select the Top-1 DL Tx beam for DL data transmission, which is up to the NW according to overall NW performance. Proposal 5: For the gNB side AI/ML model in Option 3 (i.e., use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A) and Option 4 (i.e., use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A), the corresponding Rx beam of each Tx beam can be configured/recommended to the UE or obtained by initiating a P3 beam sweeping procedure, which can be further investigated. |
| Ericsson [4] | - Observation 1 For the TX or TX/RX Top-1 beam pair prediction aiming to replace P2 and/or P3 procedure with beam prediction, L1-RSRP/CQI/SINR predictions are needed for link-adaptation to avoid additional measurement - Proposal 5 Consider flowcharts in Figure 1 for the discussion on alternatives for spatial-beam predictions with respect to the P1/P2/P3 procedure TX-beam prediction TX/RX-beam prediction |
| LGE [9] | In our view, defining beam management procedure may be helpful for aligning the simulation between companies and finding potential AI/ML gain for beam management. However, listed options above are one of example of BM case-1 and BM case-2. As another example, it can be simulated both Top-1 / Top-K beam prediction together by adding additional output (e.g. expected RSRP per predicted beam). Also, any assumption on additional measurement (e.g., P1, P2, P3) can be reported by companies. Therefore, any agreement related to above listed options is not necessary. |
| CATT [13] | Proposal 3: For both BM-Case1 and BM-Case2, further study the following options to apply AI/ML in beam management procedure: - Option 1: Use AI/ML model to predict Top-1 DL Tx beam in Set A - For UE side AI/ML, the Top-1 DL Tx beam can be recommended by UE for DL data transmission - For gNB AI/ML, the Top-1 DL Tx beam can be used for DL data transmission - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam - Option 2: Use AI/ML model to predict Top-K Tx beams in Set A - For UE side AI/ML, the Top-K Tx beams can be recommended by UE for P2 - For gNB AI/ML, the Top-K DL Tx beam can be used for P2 - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam - Option 3: Use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A - For UE side AI/ML, - the Top-1 Tx-Rx beam pair can be used for DL data reception - the Tx beam of the Top-1 Tx-Rx beam pair can be recommended by UE for DL data transmission - P3 procedure is not needed - For gNB side AI/ML, - the Tx beam of the Top-1 Tx-Rx beam pair can be used for DL data transmission - the Rx beam of the Top-1 Tx-Rx beam pair can be recommended by gNB for DL data reception - Option 4: Use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A - For UE side AI/ML, - the Top-K Tx-Rx beam pairs can be used for DL data reception - the Top-K Tx beams of Top-K Tx-Rx beam pairs can be recommended by UE for P2 - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation - P3 procedure is not needed - For gNB side AI/ML, - the Top-K Tx beams of the Top-K Tx-Rx beam pairs can be used for P2 - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation - the Rx beam corresponding with the selected DL Tx beam can be recommended by gNB - P3 procedure is not needed - Other options are not precluded. |
| Fujitsu [15] | Proposal 4: Regarding the Tx-Rx beam pairs prediction with a NW-side model, it’s suggested to use P3 procedure on the Tx beams of predicted Top-1/K beam pairs. |
| Samsung [21] | Proposal # 7: For the evaluation of Tx-Rx beam pair prediction, NW side model is deprioritized. Proposal # 8: For the evaluation of DL Tx beam prediction, the RS overhead in P3 needs to be considered. Proposal # 9: For Top-K beam (pair) prediction, the RS overhead in P2 procedure needs to be considered. |

1st round

Question 3.7a

- Option 1: AI/ML can be used to predict a DL Tx beam set for P2 procedure based on sparse Tx beam sweeping (subset of narrow beams or wide beams)

- Option 2: AI/ML can be used to predict Top-1 Tx beam for P3 procedure based on sparse Tx beam sweeping (subset of narrow beams or (FFS) wide beams)

- Option 3: AI/ML can be used to predict a best Tx-Rx beam pair to substituting P2-P3 procedure based on sparse Tx beam sweeping

- Option …

| Company | Comments |
| --- | --- |
| FL0 | It is benefit to identify when/how AI/ML can help beam management with existing or new procedure. Companies are encouraged to provide views. |
| MediaTek | 1. Option1 and Option2 seem to be more promising to us. 2. As many companies pointed out in 9.2.3.1, we don’t think Option3 is a feasible solution based on the UE orientation changing and UE Rx implementation concerns. |
| HW/HiSi | Option 1: Support Option 2: Support Option 3: Ok |
| vivo | Ok to study these options. We prefer Opt 1 and 3, but how to configure these procedures should be up to gNB’s implementation. |
| ZTE | Support to further study. |
| NTT DOCOMO | In our view, the order of P2 and P3 is not pre-defined. P2 is not necessarily performed before P3. Considering that point, the proposal misses the option where AI/ML model can be used to predict top-1 Tx beam substituting P2 procedure based on P1 and P3. This option is basically the same function as Option2. Hence, we prefer to modify Option 2 as follows. Option 2: AI/ML can be used to predict Top-1 Tx beam for P3 procedure to substituting whole P2 or a part of P2 based on sparse Tx beam sweeping (subset of narrow beams or (FFS) wide beams) |
| CATT | For Option 1, AI/ML can be used to predict a DL Tx beam set, which may not be used for P2 procedure. gNB can select one among the DL Tx beam set based on gNB implementation. Thus, we prefer to delete the “for P2 procedure” in Option 1. |
| InterDigital | We support the proposal. One additional bullet can be added with “Other options are not precluded.” |
| Spreadtrum | Support |
| Google | OK with option 1 and 2 |

2nd round

Proposal 3.7b

For the usage of AI/ML in beam management, further considering the following as a starting point:

- Option 1: AI/ML can be used to predict a DL Tx beam set for P2 procedure based on sparse Tx beam sweeping (subset of narrow beams or wide beams)

  - E.g., the predicted DL Tx beam set can be used for P2 procedure

- Option 2: AI/ML can be used to predict Top-1 Tx beam for P3 procedure based on sparse Tx beam sweeping (subset of narrow beams or (FFS) wide beams)

  - E.g., to substituting whole P2 or a part of P2 procedure

- Option 3: AI/ML can be used to predict a best Tx-Rx beam pair to substituting P2-P3 procedure based on sparse Tx beam sweeping

  - E.g., to substituting P2-P3 procedure

- Other options are not precluded and can be reported by companies.

| Support Opt 1 | NTT DOCOMO, MediaTek, Futurewei, Samsung |
| --- | --- |
| Support Opt 2 | NTT DOCOMO, MediaTek, Futurewei Samsung |
| Support Opt 3 | NTT DOCOMO, Futurewei Samsung |
| Objected/concerned to list options |  |

| Company | Comments |
| --- | --- |
| FL | Please indicate supported options, or if you have concern to list options. And any other comment |
| NTT DOCOMO | At least for Option 3, the prediction could be based on the Tx and Rx beam sweeping. Therefore, we propose to delete the “Tx” from the “based on sparse Tx beam sweeping” in the options. |
| Ericsson | We are ok with the options. Minor editiorial comment to option 3. Option 3: • Option 3: AI/ML can be used to predict Top-1 a best Tx-Rx beam pair - E.g., to substituting whole P2-P3 or a part of P2-P3 procedure |

3rd round

Proposal 3.7c

For the usage of AI/ML in beam management, further considering the following as a starting point:

- Option 1: AI/ML can be used to predict a DL Tx beam set based on sparse Tx beam sweeping (subset of narrow beams or wide beams)

  - E.g., the predicted DL Tx beam set can be used for P2 procedure

- Option 2: AI/ML can be used to predict Top-1 Tx beam based on sparse Tx beam sweeping (subset of narrow beams or (FFS) wide beams)

  - E.g., to substituting whole P2 or a part of P2 procedure

- Option 3: AI/ML can be used to predict Top-1 a best Tx-Rx beam pair based on sparse Tx beam sweeping

  - E.g., to substituting whole P2-P3 procedure or a part of P2-P3 procedure

- Other options are not precluded and can be reported by companies.

| Support Opt 1 | NTT DOCOMO, MediaTek, Futurewei, Samsung |
| --- | --- |
| Support Opt 2 | NTT DOCOMO, MediaTek, Futurewei Samsung |
| Support Opt 3 | NTT DOCOMO, Futurewei, Samsung |
| Objected/concerned to list options |  |

| Company | Comments |
| --- | --- |
| FL | Please indicate supported options, or if you have concern to list options. And any other comment |

  - Others

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi [2] | - Observation 1: For the AI/ML-based beam prediction mechanism, Alt.1 (DL Tx beam prediction) may achieve optimized Tx-Rx beam combination by DL Tx beam prediction and legacy Rx beam sweeping. Proposal 1: For the evaluation of AI/ML-based beam prediction mechanism, - Alt.1 (DL Tx beam prediction) should be considered as the starting point. - Both Case A (best Rx beam) and Case B (same specific Rx beam) can be adopted and reported by companies. - Alt.3 (Tx-Rx beam pair prediction) can be also evaluated to justify potential additional performance gain over Alt.1. - Alt.2 (DL Rx beam prediction) can be considered with lower priority. Proposal 2: For evaluation of DL Tx beam prediction, the following two baselines can be considered: - Baseline 1: A lower performance bound obtained by non-AI/ML-based legacy sparse beam sweeping with the same overhead as the AI/ML-based approach. - Baseline 2: An upper performance bound obtained from exhaustive beam sweeping over all available beams in Set A. Proposal 3: For evaluation of DL Tx-Rx beam pair sweeping, the following two baselines can be considered: - Baseline 1: Non-AI/ML solution with legacy Tx beam sweeping plus legacy Rx beam sweeping, to justify the performance gain over non-AI/ML. - Baseline 2: AI/ML-based DL Tx beam prediction plus the legacy Rx beam sweeping, to justify the performance gain over DL Tx beam prediction. |
| Qualcomm [23] | Proposal 1: For BM-Case1 and BM-Case2, consider the following factors for UE-side and gNB-side AI/ML models: - Feasibility and availability of inputs to the AI/ML model at each side - If feasible, study the benefits, impact on system operation, and trade-offs for making a given set of inputs available at each side |
| MediaTek [25] | Proposal 6: For AI/ML-based spatial domain beam prediction evaluation, adopt the RSRP of beams in Set B as the AI/ML model inputs. Additional information to the input of AI/ML model is not excluded. Proposal 7: Adopt one of the following as the output of AI/ML model: (i) beam index of highest RSRP Set A of beams. (ii) RSRPs of all the Set A of beams. Observation 12: The spatial prediction accuracy does not improve much by using UE angles directly as the additional input, at least for the ratio of Set B and Set A sizes is larger than 1/8. |

| China Telecom | Observation 1: Modelling the spatial beam prediction task as a classification model provides better performance with less training overhead. |
| --- | --- |
| MediaTek [25] | Observation 13: The AI/ML model of spatial beam prediction can estimate L1-RSRP of beams in Set A while maintaining similar system level performance in selecting optimal beams. Observation 14: The predicted L1-RSRP difference varies consistently with L1-RSRP difference. It can not only measure the accuracy of RSRP estimation, but also serve as a system performance indicator. Proposal 10: Further study on using the predicted L1-RSRP as the AI/ML BM model output and the performance of the model. |

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi [2] | Observation 3: Using 256 beams in Set A constructed from a dense codebook increases the angular resolution compared to a 64-DFT codebook, while the same sparse sweeping procedure for inference and gNB configurations can be applied. Proposal 5: To assess the RSRP gains achievable with 256 Tx beams compared to Exhaustive 64 beam sweeping, companies are encouraged to evaluate a dense codebook with overlapping beams for the construction of Set A. |
| Nokia [12] | Observation 1: For BM-Case1, a large number of beams in Set B (e.g., 32) may not improve the prediction accuracy and the system throughput. Therefore, ML-based beam selection should consider a Set B with a maximum of 16 beams when Set A has 64 beams, hence Set B should have a max of ¼ of Set A beams. |
| MediaTek [25] | Observation 8: With a greater number of beams in Set B, both models achieve higher Top-K/1 accuracy. However, greater number of beams in Set B requires more beam RSRP measurements. Proposal 5: Study the tradeoff between the beam measurement overhead and prediction accuracy for different number of beams in Set B. |

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi[2] | Observation 7: In case of DL Tx-Rx beam pair prediction, Set A of the AI/ML model consist of beam pairs and is expected to be significantly larger than for DL Tx beam prediction. Proposal 11: If DL Tx-Rx beam pair prediction is evaluated further, since the effort for required impact on signaling between gNB and UE is similar for UE-side and NW-side AI/ML deployment, both UE-side model and NW-side model should be given the same priority during the evaluations. |
| Intel [17] | Proposal 1: For AI/ML evaluation for beam management use cases, including spatial and temporal domain beam management, consider only offline training of AI/ML models. |

Question 3.8a

| Company | Comments |
| --- | --- |
| FL0 | Please share you view if you think any of the above aspects is important and requires further agreements for the evaluation. |
| Hw/HiSi | It would be good if more companies would evaluate the performance of Tx beam prediction when a quasi-optimal Rx beam is used. Different assumption on the quality of the quasi-optimal beam could be mead (for instance 100%, 90%, 80% of the cases the best genie aided beam is chosen). |

AI/ML model Generalization

Agreement The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - The following case for generalization verification, can be optionally considered by companies: - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side

| Company | Proposals/observations |
| --- | --- |
| Vivo [8] | Proposal 2: Support to report generalization-Case2 and generalization-Case3 related assumptions, at least including model input data for training and inference with different configurations and/or scenarios. |
| Xiaomi [10] | Proposal 10: General model can be supported for AI/ML in beam management for each of the following aspects: - Different UE speed - Different number of Rx beam - Different Scenarios: UMa, Umi - Different UE distribution |
| Nokia [12] | Proposal 11: RAN1 prioritizes model generalization studies for case 3 and case 2a. Proposal 12: Support RAN1 to further study fine-tuning (case 2a), including assessing the performance on previously learned scenarios/configurations. |
| Fujitsu [15] | Observation 10: For BM-case 1, the performance of AI/ML model is more sensitive to the different parameters of configuration than that of scenarios. Proposal 5: Regarding generalization performance evaluation of AI/ML model, it’s suggested to have high priority on the configuration related parameters or settings. |
| Intel [17] | Proposal 10: Generalization across different cells of the same deployment in system level simulation should necessarily have different configurations in each cell such that the cells are not statistically identical with respect to generated channels. |
| Lenovo [20] | - Proposal 1 Generalizability of a proposed AI/ML model for beam management is evaluated by computing the agreed KPIs, inclusive of the gains achieved and the costs incurred, by the model for each of the different network conditions/scenarios/parameter values. - Proposal 2 Discuss how to decide on the generalization ability of an AI/ML model based on the KPIs, inclusive of the gains achieved, and the costs incurred, that are evaluated for each of the different network conditions/scenarios/parameter values. Further, consider the threshold-based methods for further study. |
| Apple [22] | Proposal 2: For AI model generalization, generalization performance regarding analog beam design including Set A design, antenna configurations including M/N, antenna spacing and deployment scenario should be considered. - Observation 2: the feasibility to support fine-tuning of a deployed AI model on the UE side needs study. Observation 3: - For generalization performance Case -1, trained AI models perform well, provide better beam prediction accuracy than the conventional approach. - For generalization performance Case -2, trained AI models can perform much worse than that for GP Case-1, they may lead to even worse beam prediction accuracy than the conventional approach. - For generalization performance Case-3, trained AI models can perform worse than that for GP Case-1, even though are in general better than that for GP case-2. the AI performance with set B beam at 8 beams with GP Case-3 is roughly the same as the AI performance with set B at 4 beams with GP Case-1. Proposal 3: As generalization performance can be poor for AI models trained without Tx beam shape information, study NW-trained cell-specific AI models for AI enabled beam management. |
| Qualcomm [23] | Observation 7: The focus of generalization study and analysis for BM use cases should not be solely on a single AI/ML model generalizing to new scenarios/configurations. Other alternatives such as training multiple AI/ML models each tailored to a specific scenario/configuration and switching among those AI/ML models based on the deployed scenario should also be considered. |

  - Generalization for BM-Case1

    - Different deployment scenarios

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| Ericsson[4] | - With identical antenna configuration, initial evaluations indicates that a model trained in one cell is found to be generalized well while the performance heavily depends on the sector is selected for the inference. Generalization results indicate the importance of having model monitoring procedures that detects issues when a model trained in one cell is used in another. |
| OPPO [6] | - Observation 14: Thanks to generalization capability of well-trained AI/ML model, changing scenario from UMa to UMi may not necessarily deteriorate the beam prediction performance. |
| Xiaoma[10] | Observation 6: AI model trained by hybrid data of Uma and Umi for beam prediction in spatial domain can provide good generalization capability for Uma or Umi. While AI model trained by data of only Uma or only Umi provide a little worse generalization capability for different scenario, i.e., Uma model for Umi inference, or Umi model for Uma inference. |
| CATT [13] | Observation 8: When AI/ML model is trained and inferenced with different scenario configurations, e.g., the Set B of training is Uma and inference is Umi, the Top-1 accuracy can be comparable. |
| Fujitsu[15] | Observation 7: For BM-case 1, the performance of AI/ML model is not sensitive to the scenario of Uma/Umi. - The mismatch on the scenario of Uma/Umi between training and inference cause slight performance degradation. |
| Lenovo [20] | Observation 7: The AI model for spatial beam prediction can achieve stable and good performance in different deployment scenarios or different ISDs, e.g., training under UMa scenario and testing under UMi scenario, training with ISD1 and testing with ISD2 with the agreed simulation assumptions. |
| Samsung [21] | Observation # 9: For various deployment scenarios, if data sets for training and inference are from different scenarios (i.e., Case 2), the performance has significate degradation. However, it is still better than non-AI scheme. Observation # 10: For various deployment scenarios, with model finetune (Case 2A) or training with mixed data (Case 3), the performance is slightly lower than training and inference with the same deployment scenario (Case 1). Observation # 19: The settings/parameters that may cause verification of wireless channel will degrade the generalization performance for AI/ML in beam prediction. |
| Qualcomm[23] | Observation 5 Homogeneous inter-site scenarios (e.g., same deployment, different cells) have generally better generalization performance compared to heterogeneous inter-site scenarios (e.g., cells from different deployments). Observation 6 For heterogeneous inter-site scenarios (e.g., cells from different deployments), incorporating datasets from different deployments in the training may improve the heterogenous inter-site generalization without significantly affecting the performance in the seen cells. |

    - Different ISD

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| CATT [13] | Observation 11: When AI/ML model is trained and inferenced with different ISD, e.g., the Set B of training of ISD is 200 and inference is 500, the Top-1 accuracy can be comparable. Proposal 6: For the study of generalization, we should focus on the influence of different patterns or different Set B for generalization. |
| Fujitsu [15] | Observation 8: For BM-case 1, the performance of AI/ML model is not sensitive to the parameters of ISD. - The mismatch on the parameter of ISD between training and inference cause almost no performance degradation. |
| Lenovo [20] | Observation 7: The AI model for spatial beam prediction can achieve stable and good performance in different deployment scenarios or different ISDs, e.g., training under UMa scenario and testing under UMi scenario, training with ISD1 and testing with ISD2 with the agreed simulation assumptions. |

    - Different UE distribution

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| Xiaomi [10] | Observation 7: AI model trained by hybrid data of different UE distribution for beam prediction in spatial domain can provide good generalization capability. While AI model trained by data of only UE distribution Option A provides a little worse generalization capability for UE distribution Option B. |
| InterDigital [11] | Observation 10: Training an AIML model for a certain environment (e.g., UMa) and a certain a UE dropping scenario for a different UE dropping scenario results in degradation in Top-1 accuracy by 4%-11%. Observation 11: Training an AIML model for a certain environment (e.g., UMa) and a mixed data set from different UE dropping scenarios can be generalized for different UE dropping scenarios. |
| Nokia [12] | Observation 21: The ML model trained with only outdoor UEs may NOT generalize well for indoor UEs, whereas the ML model trained with mixed indoor/outdoor UEs may generalize well for both indoor and outdoor UEs. Observation 22: The ML model trained with indoor UEs may be applied also for outdoor UEs without compromising the performance with respect to the ML model trained with mixed indoor/outdoor UEs. |
| CATT [13] | Observation 9: When AI/ML model is trained and inferenced with different UE distributions, e.g., the Set B of training of UE distributions is 0% and inference is 50%, the Top-1 accuracy can be comparable. |
| Fujitus [15] | Observation 9: For BM-case 1, the performance of AI/ML model is not sensitive to the various outdoor/indoor UE distribution. - The mismatch on the various outdoor/indoor UE distributions between training and inference cause almost no performance degradation. |
| Samsung [21] | Observation # 11: For various UE distribution scenarios, if data sets for training and inference are from different scenarios (i.e., Case 2), the performance has significate degradation. However, it is still better than non-AI scheme. Observation # 12: For various UE distribution scenarios, with model finetune (Case 2A) or training with mixed data (Case 3), the performance is slightly lower than training and inference with the same deployment scenario (Case 1). Observation # 13: Various UE distribution scenarios may have different scene complexity, which should be emphasized in the finetune process. Finetune from a complex model to a simple model is easier (Case 2A-1), but harder on the other way around (Case 2A-2). Observation # 14: The scene complexity of various UE distribution scenarios also affects the performance of mix-training. With mix-training, the AI/ML model works well under simple scenarios (Case 3-1), but may be slightly inferior under complex scenarios (Case 3-2). |
| CEWiT [26] | Observation 3: Training on a mix of data from different scenarios can improve the performance of the AI/ML model. |

    - Different numbers/patterns of Set B

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| Futurewei [1] | Observation 1: In spatial-domain beam prediction using fixed Set B beam pattern in AI/ML model training phase, performance degrades significantly when Set B beam pattern changes in model inference phase. Observation 2: In spatial-domain beam prediction using fixed Set B beam pattern in AI/ML model training phase, performance degrades significantly when at least one of the beam pairs in Set B is missing during model inference phase. Observation 3: In spatial-domain beam prediction using variable Set B beam patterns in AI/ML model training phase, performance degrades significantly when Set B beam patterns changes in model inference phase. Observation 4: In spatial-domain beam prediction using variable Set B beam patterns in AI/ML model training phase, when Set B pattern changes in model inference phase, less performance degradation is observed when only a subset of the beam patterns used as input in the inference phase are different from the beam patterns used in the training phase compared to when all the beam patterns used as input in inference phase are different than the training phase. |
| ZTE [3] | Number of Set B Observation 8: The case that model training and testing are performed in mixed datasets with different beam constructions can achieve a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>, in which the latter two cases are evaluated in the same assumption for model training and testing. Observation 9: The AI based Tx beam prediction with the optimal Rx beam achieves slightly better performance than that of the AI based method with a specific (e.g, first) Rx beam. Set B pattern Observation 10: For the model generalization on different Set B patterns, if the Set B pattern of the testing data samples is different from that of the training data samples, a significant performance loss compared with the non-AI case is observed. Observation 11: For the model generalization on different Set B patterns, if the Set B pattern of the testing data samples is included in the beam patterns of the training data samples, or the same Set B pattern is used during the model training phase and testing phase, the AI method obtains significant performance gains over the non-AI case. |
| InterDigital [11] | Observation 12: Training an AIML model with different beam patterns or random beam patterns can help obtain a model that generalizes over multiple beam patterns without degradation in prediction accuracy. |
|  | Observation 12: In BM-Case1 DL TX beam prediction, training model with fixed beam number in random Set B outperforms the training model with varied beam number in random Set B. Observation 13: In BM-Case1 DL TX beam prediction, compared to training model with fixed Set B, training model with random Set B can provide similar performance when ¦Set B¦/¦Set A¦ is large (i.e., 32/64) but the performance will become inferior when ¦Set B¦/¦Set A¦ is small. Observation 14: In BM-Case1 DL TX beam prediction, the top-K beam search is needed for the model trained with random Set B. Observation 15: For training the DL Tx prediction model a Random SetB pattern can be used. Later in the inference stage, the DL Tx prediction model can use measurements from pre-configured/fixed SetB patterns. Proposal 6: For BM-Case1, RAN1 may prioritize the measurements of Random SetB (Opt2C) to be used at UE side for input to model training and the measurements of Fixed/Pre-configured SetBs (Opt1 and Opt2B) to be used at UE side for model inference. |
| CATT [13] | Observation 3: For DL Tx prediction, the prediction accuracy is greatly reduced when using different pre-configured patterns for training and inference. Observation 7: When AI/ML model is trained and inferenced with different Set B, e.g., the Set B of training is 32 and inference is 16, the Top-1 accuracy is degraded. |
| Fujitus [15] | Observation 1: For BM-case 1, the performance of AI/ML model is sensitive to the size of Set B. - The mismatch on the size of Set B between training and inference will cause big performance degradation. Observation 2: For BM-case 1, the training dataset constructed by a set of pre-configured size of Set B will improve the generalization performance of AI/ML model. |
| Lenovo[20] | Observation 8: The beam prediction accuracy and average L1-RSRP difference are affected marginally when the size of Set A and Set B during testing is less than that for the training. |
| DoCoMo[23] | Observation 3: For Tx beam prediction with Rx beam assumptions Option 1 and Option 2a: - The AI/ML model trained with mixed Rx beam assumptions at UE side could provide acceptable generalization performance. Observation 4: For Tx-Rx beam prediction: - The AI/ML model trained with mixed Set B Tx beam configurations could provide acceptable generalization performance. |

    - Different UE Rx assumptions

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| ZTE [3] | [Same UE Rx conf. using L1-RSRP from different Rx] Observation 12: For model generalization on different Rx beam assumptions for Tx beam prediction, compared with the case that the training data samples and testing data samples are generated based on the same Rx beam assumption, only marginal performance loss is observed for the case of different Rx beam assumptions on model training and model testing. Proposal 8: The AI model can provide good generalization capability on different Rx beam assumptions for Tx beam prediction. |
| Xiaomi[10] | Observation 8: AI model for beam prediction in spatial domain can provide good generalization capability among different number of UE Rx beam, e.g., AI model with more Rx beam number can be applied for beam prediction of less Rx beam number. FL: The assumption for beam pair prediction needs to be further clarified since the number of Rx beam changed, the beam pair in Set A will also change. |
| Qualcomm [23] | #### Observation 12 For DL Tx beam prediction for BM-Case1, mismatch of UE array assumptions between training and test datasets (for NW-side models) results in a minor performance loss, e.g., less than 5% loss in Top-1 beam prediction accuracy, which is much less significant than the performance loss due to gNB array mismatch (for UE-side models). |
| DoCoMo[24] | Observation 3: For Tx beam prediction with Rx beam assumptions Option 1 and Option 2a: - The AI/ML model trained with mixed Rx beam assumptions at UE side could provide acceptable generalization performance. |

    - Different gNB antenna setting/different number or pattern of Set A

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| ZTE [3] | Observation 13: For model generalization on different gNB settings, a significant performance loss is observed if the training data samples and the testing data samples are generated under different gNB beam structures. Observation 14: Compared with the case that the same gNB beam structure is used for generating the training data samples and the testing data samples, only marginal performance loss is observed if the training data samples of AI model are mixed data samples. |
| Vivo [8] | Observation 20: As the difference of beam shape pattern increases, the performance loss of both average RSRP difference and beam prediction accuracy increases along with the difference of the antenna configurations between training subset and validation subset. Proposal 18: Further study generalization performance for different antenna configurations and different beam shapes in BM-Case1. Proposal 19: Further study assistance information, such as beam shape pattern, 3dB beam width, etc., as model input to address performance deterioration for generalization of different beam shapes in BM-Case1. FL: based on my reading of vivo’s assumption, vivo actually used different gNB config. Other than a beam shape. |
| Nokia [12] | Observation 23: model's generalization capabilities are poor for Case 2, when the model is tested on a gNB antenna array configuration that is different from the configuration used during training. Observation 24: The model generalizes well in Case 3, when trained with a mix of data from different gNB antenna array configurations. Observation 25: The model generalizes well for Case 2a when a reduced dataset for a different gNB antenna array configuration is used for fine-tuning the model. |
| Fujitus [15] | Observation 5: For BM-case 1, the performance of AI/ML model is sensitive to the size of Set A. - The mismatch on the size of Set A between training and inference will cause big performance degradation. Observation 6: For BM-case 1, the training dataset constructed by a set of pre-configured size of Set A will improve the generalization performance of AI/ML model. |
| Samsung [21] | Observation # 15: For various codebook scenarios, if AI/ML never trained with a given codebook, the performance is worse than non-AI baseline. Observation # 16: Finetune (10%) can improve the generalization performance for different codebook scenarios, but it still has some degradation comparing with training with single codebook (Case 1). Observation # 17: Training with mixed data (Case 3) can provide better performance than finetune (Case 2A), and the performance is close to the performance training with single codebook (Case 1). Observation # 18: Beam correlation related parameters have significant impact on generalization performance for AI/ML in beam prediction. Without finetune or mix-training, the performance may be even worse than non-AI scheme. |
| Apple [22] | Proposal 2: For AI model generalization, generalization performance regarding analog beam design including Set A design, antenna configurations including M/N, antenna spacing and deployment scenario should be considered. Observation 1: If explicit Tx beam shape information for different datasets is not available to model trainer, it may be difficult to design AI model to generalize well over different scenarios/configurations. However, acquiring explicit Tx beam shape information at the UE side may be difficult due to concerns on disclosing proprietary information. |
| Qualcomm [23] | #### Observation 8 A single UE-side AI/ML model trained using a first gNB codebook does not generally generalize well to “unseen” gNB codebooks. #### Observation 10 Using a mixed dataset from multiple gNB codebooks when training a UE-side AI/ML model improves generalization performance if the deployment codebook was in the mixed dataset during training, compared to the case in which the deployment codebook was not encountered during training. #### Observation 11 When using a mixed dataset from multiple gNB codebooks for training a UE-side AI/ML model, signalling of codebook index from gNB to UE and incorporation of this information at the UE side (e.g., as an auxiliary input) can boost the generalization performance. |

    - Different carry frequency

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| CATT [13] | Observation 10: When AI/ML model is trained and inferenced with different carrier frequency, e.g., the Set B of training is 20G and inference is 30G, the Top-1 accuracy can be comparable. |

    - UE speed

| Company | Proposals/observations |
| --- | --- |
| Samsung [21] | Observation # 29: For DL TX beam prediction and beam pair prediction, AI/ML model performs the best when the training and testing dataset are drawn from the same UE speed. However, performance degradation is observed when the training dataset and testing datasets are drawn from different UE speed. Observation # 30: For DL TX beam prediction and beam pair prediction, training a model with a mixture of dataset drawn from a range of UE speeds allows the model to perform well over a range of UE speeds. |

    - Summary for BM-Case 1

The following table summarized the observations from companies

| Scenarios/configurations | Observations |
| --- | --- |
| deployment scenarios (e.g., UMa, UMi) | Case 2 - One of the explanations would be that the model trained under UMa scenario is more adaptable to UMi scenario. (ZTE) - little worse generalization capability (xiaomi) - Comparable (CATT) - Not sensitive (Fujitus) - stable and good performance (Lenovo) - degradation (Samsung) - Homogeneous inter-site scenarios (e.g., same deployment, different cells) have generally better generalization performance compared to heterogeneous inter-site scenarios (e.g., cells from different deployments). (Qualcomm) Case 3: - Mixed data is beneficial for different ISD (Ericsson) - Good performance Umi, Uma(xiaomi) - Slightly lower than Case 1 (Samsung) - incorporating datasets from different deployments in the training may improve the heterogenous inter-site generalization without significantly affecting the performance in the seen cells (Qualcomm) |
| Different ISD | Case 2(different training/testing) - Performance degradation for different ISD (Ericsson) - Comparable for 200m and 500m (CATT) - No degradation (Fujitsu) - stable and good performance (Lenovo) Case 3: - Mixed data is beneficial for different ISD (Ericsson) |
| UE distribution | Case 2 - Challenge using 100% outdoor to model 80/20% in/outdoor but better to use AI trained with 80/20% in/outdoor to test 100% outdoor (Ericsson) - for various UE distribution, the performance varies according to the considered deployment scenario (i.e., UMa or UMi). (Ericsson) - little worse generalization capability (xiaomi) - results in degradation in Top-1 accuracy by 4%-11%. (InterDigital) - 100% outdoor => 80/20% in/outdoor is not good (Nokia) - Indoor =>outdoor is fine (Nokia) - Comparable (CATT) - 80% outdoor =>100% outdoor Not sensitive (Fujitsu) - Degradation (Samsung) - In/outdoor =20/80 => in/outdoor =80/20 is harder than, In/outdoor =80/20 => in/outdoor =20/80 Case 2A - Finetune can improve the performance (Ericsson) - Slightly degradation than Case 1(Samsung) Case 3: - Good performance Umi, Uma (xiaomi) - can be generalized (InterDigital) - Mixing UE distribution generalized good (Nokia) - Slightly degradation than Case 1(Samsung) - Improve the performance (CEWiT) |
| UE speeds | Case 2(different training/testing) - Performance degradation (Samsung) Case 3(mixed) - Better performance (Samsung) |
| Variable set B (number/pattern) | Case 2 (different pattern/number): - Significantly degradation (Huawei/HiSI) - Performance loss when training and testing using different patterns (ZTE) - Performance loss (InterDigital) - Degraded (CATT) - big performance degradation. (Fujitsu) - are affected marginally when Set A/Set B is less then training(Lenovo) Case 3 (mixed) - Good performance with mixed training data for different number of Tx beams in Set B and different patterns (ZTE) - generalizes over multiple beam patterns without degradation in prediction accuracy (InterDigital) - improve the generalization performance (Fujitsu) |
| Carrier frequency | Case 2(different training/testing) - Comparable for 20GHz and 30GHz (CATT) |
| Different number /array of UE Rx beam | Case 2: (different training/testing) - Good generalization with measurement by different Rx (Same UE Rx conf.) (ZTE) - For Case 3 (a half number of Rx beams) and Case 4 (a half number of Tx beams and a half number of Rx beams), we observe slight performance loss due to the generalization issue. The more dramatic change from training set to testing set, the more performance loss can be observed. (OPPO) - Different array a minor performance loss (Qualcomm) Case 3:(mixed) - acceptable generalization performance (DoCoMo) |
| gNB antenna array/ Different number/pattern of Set A | Case 2:(different training testing) - significant performance loss (ZTE) - performance loss (vivo) - poor (Nokia) - Different number of Set A, big performance degradation. (Fujitsu) - Worse than Non-AI for different beambook (pattern of Set A)(Samsung) - does not generally generalize well to “unseen” gNB codebooks (Qualcomm) Case 2A(finetune) - well (Nokia) - Still some degradation for different beambook (patten of SetA) (Samsung) Case 3(mixed) - marginal performance loss (ZTE) - well (Nokia) - Different number of Set A, improve the performance. (Fujitsu) - Better than finetune for different beambook (patten of SetA) (Samsung) - improves generalization performance for multiple gNB codebook (Qualcomm) |

1st round

Observation 4.1-1a (UMa, UMi)

- Based on the preliminary results, at least for BM-Case1, for different deployment scenarios (e.g., UMa, UMi)

  - Generalization performance is not sensitive (Case 2)

  - Training with mixed data may improve the generalization performance (Case 3)

Observation 4.1-2a (Variable Set B)

- Based on the preliminary results, at least for BM-Case1, for different Set B(number and/or pattern)

  - Significantly performance degradation is observed (Case 2)

  - Training with mixed data improve the generalization performance (Case 3)

Observation 4.1-3a (Different gNB antenna conf.)

- Based on the preliminary results, at least for BM-Case1, for different gNB antenna configuration (e.g., antenna array, Tx beam codebook, patterns/numbers of Set A)

  - Significantly degradation is observed (Case 2)

  - Model finetune improve the performance but may still have some degradation (Case 2A)

  - Training with mixed data improve the generalization performance (Case 3)

| Company | Comments |
| --- | --- |
| FL0 | Please provide your view on the above observations. Wording update other than “too early to agree” is appreciated. |
| Xiaomi | Observation 4.1-1a, agree Observation 4.1-2a, we would to clarify that is it for variable set B or fixed set B? |
| HW/HiSi | On first sight the observations are fine and we are supportive with their intention. If there is a the same opinion from most companies then we can check the wording more thoroughly |
| vivo | For 4.1-2a: Based on our observation, training with mixed data can improve generalization performance, but it requires to have beam angle or ID as input. Otherwise performance is not good. For 4.1-3a: Based on our observation for Case 2, if the beam angles in inference dataset are a subset of beam angles in training dataset, and beam angles or IDs are included as input of the AI model, the performance loss is rather small. |
| Lenovo | Consider training for configuration/scenario A and B. If we train with a mixed data set containing data corresponding to both A and B, then the AI/ML model might be able to generalize well over configuration/scenario A and B. But the benefit in generalization should not be at the cost of performance of AI/ML model for only configuration/scenario A or configuration/scenario B. In other words, consider three AI/ML models: 1st model trained with data set corresponding to configuration/scenario A, 2nd model trained with data set corresponding to configuration/scenario B, and the 3rd model trained with mixed data set having data corresponding to configuration/scenario A and B. It can be understood that the difference in performance of 3rd model would be minimum when we switch configuration/scenario from A to B or B to A. We also need to check whether the performance of the 3rd model is same as that of the 1st model for Configuration/scenario A and whether the performance of the 3rd model is same as that of the 2nd model for Configuration/scenario B. |
| CATT | Agree with Observation 4.1-1a and 4.1-2a. For Observation 4.1-3a, further evaluation is needed for fine-tuning and mixed train. |
| Google | Agree with all the observations above |
| Samsung | Observation 4.1-1a looks stable. We see Observation 4.1-2a and 3a are too early to agree. |

3rd/2nd round

Observation 4.1-1b (UMa, UMi)

- Based on the preliminary results, at least for BM-Case1 with fixed Set B, for different deployment scenarios (e.g., UMa, UMi)

  - Generalization performance is not sensitive (Case 2)

  - Training with mixed data may improve the generalization performance (Case 3)

Observation 4.1-2b (Variable Set B)

- Based on the preliminary results, at least for BM-Case1 with fixed Set B, for different Set B (number and/or pattern)

  - Significantly performance degradation is observed (Case 2)

  - Training with mixed data improve the generalization performance (Case 3)

Observation 4.1-3a (Different gNB antenna conf.)

- Based on the preliminary results, at least for BM-Case1, for different gNB antenna configuration (e.g., antenna array, Tx beam codebook, patterns/numbers of Set A)

  - Significantly degradation is observed (Case 2)

  - Model finetune improve the performance but may still have some degradation (Case 2A)

  - Training with mixed data improve the generalization performance (Case 3)

| Company | Comments |
| --- | --- |
| FL | In FL’s view, further study on Case 3 is needed, e.g., comparing the performance difference of Case 3 and Case 1, which can be captured with additional observations |
| Xiaomi | Support the observation 4.1-1b and 4.1-2b with removing (Variable Set B) |
| CAICT | Support |
| Futurewei | We agree with Observation 4.1-2b, which is consistent with our study. |
| Samsung | Observation 4.1-1b, support. Observation 4.1-3a, too early to agree. For model finetune, we think more result is needed to better understand how much is the “some degradation”. whether the degradation can be acceptable or not. Regarding Observation 4.1-2b, our understanding about fixed Set B is Set B is fixed across training and test. Thus, Case 2 and Case 3 generalization with fixed Set B is unclear. If the intention is variable Set B, we think more additional observation should be captured. |

  - Generalization for BM-Case2

    - UE speed

| Company | Proposal/observations |
| --- | --- |
| Huawei/HiSi[2] | Observation 19: For the generalization verification over various UE speeds under temporal domain beam prediction, when trained with 8000 samples: - AI/ML has poor generalization performance when trained with a UE speed of 30 km/h and tested with 90 km/h, or vice versa - In terms of prediction accuracy, e.g., it is slightly above 60% for Top-8 and slightly less than 40% for Top-4. - In terms of prediction accuracy with 1dB margin, e.g., it is slightly below 70% for Top-8 and around 43% for Top-4. - In terms of average L1-RSRP, e.g., it is more then -2dB for Top-8 and more than -4.5dB for Top-4. - AI/ML can achieve moderate performance when trained with a UE speed of mixed 30 km/h and 90 km/h, and tested with either 30 km/h or 90 km/h. - In terms of prediction accuracy, e.g., it is more than 80% for Top-8 and 65-75% for Top-4. - In terms of prediction accuracy with 1dB margin, e.g., it is more than 80-90% for Top-8 and 70-77% for Top-4. - In terms of average L1-RSRP, e.g., it is less than -1 dB for Top-8 and less than -2.2dB for Top-4. |
| ZTE [3] | Observation 20: In BM-Case2, if the AI model is trained with samples of mixed UE speeds, a little performance loss is observed as the UE speed in testing samples increases from 30km/h to 90km/h. Observation 21: In BM-Case2, compared with the case that the same UE speed is used for generating the training data samples and the testing data samples, average performance is observed if both the training data samples and the testing data samples of the AI model are mixed data samples. |
| OPPO[6] | Observation 17: For AI/ML model trained with UE speed 30km/h and tested with UE speed higher than 60km/h, the generalization performance on beam prediction accuracy and avg. L1-RSRP difference drops accordingly. Observation 18: For AI/ML model trained with mixed UE speed (e.g. from 30km/h to 120km/h) and tested with different UE speed, the generalization performance on beam prediction accuracy and avg. L1-RSRP difference outperform the one trained with single UE speed. |
| Xiaomi[10] | Observation 14: AI model for beam prediction in time domain trained by data of 30km/h or only 60 km/h or hybrid can provide good generalization capability to UE speed with both 60km/h and 30km/h. |
| InterDigital [11] | Observation 13: Training an AIML model with examples from different UE speeds can help obtain a model that generalizes for temporal beam prediction for different UE speeds. |
| Nokia [12] | Differently, when the ML model trained at a specific UE speed is applied to a different UE speed (Case #2), the performance degrades significantly, especially if the UE speeds are very different. This is evident for instance looking at the dashed-dot red line, representing the ML model trained at 120 Km/h and tested at 30Kmh and the black dotted line representing the ML model trained at 30 Km/h and tested at 120 Kmh. On the other hand, Figure 2.3-2(b) shows the generalization performance when the model is trained with a dataset containing a mix of scenarios with UE moving at different speed. In this case, the model tested on different speeds generalize well and performance remains high independently by the UE speed. - Proposal 15: Support RAN1 to further study different approaches to improve the ML model generalization for BM-Case2 for different UE speeds. |
| Samsung [21] | Observation # 29: For DL TX beam prediction and beam pair prediction, AI/ML model performs the best when the training and testing dataset are drawn from the same UE speed. However, performance degradation is observed when the training dataset and testing datasets are drawn from different UE speed. Observation # 30: For DL TX beam prediction and beam pair prediction, training a model with a mixture of dataset drawn from a range of UE speeds allows the model to perform well over a range of UE speeds. |
| DoCoMo [24] | Observation 5: For temporal beam prediction Pattern A and Pattern B: - The AI/ML model trained with mixed data from different UE speed could provide acceptable generalization performance. - The AI/ML model trained with data from low speed UE could still provide better performance than baseline method, when it is applied to the data from high speed UE with the same time parameters. |

    - Different numbers/patterns of Set B

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| Samsun [21] | Observation # 40: If the number of Set B is not the same in training and inference phase, the performance is no better than non-AI scheme. Observation # 41: For different size of Set B, training a model with a mixture of dataset obtained from the dataset consists of the maximum size of Set B allows the model to perform than non-AI scheme. However, ~10% of performance degradations are observed for Top-1(%) comparing with the case ¦Set B¦ = ¦Set B’¦. |
| DoCoMo[23] | Observation 4: For Tx-Rx beam prediction: - The AI/ML model trained with mixed Set B Tx beam configurations could provide acceptable generalization performance. |

    - Different gNB antenna setting

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| Vivo [8] | Observation 24: Performance loss can be observed if there is difference in beam shape patterns for training and validation in BM-Case2. Observation 25: For the case using local beam ID as model input, beam loss and accuracy degenerate significantly compared to the performance of AI model training and inference with beam pointing angle. Proposal 24: Further study generalization performance for different antenna configurations and different beam shapes in BM-Case2. |
| Samsung [21] | Observation # 42: For DL TX beam prediction and Tx-Rx beam pair prediction, AI/ML model performs the best when the training and testing dataset are drawn from the same BS antenna configuration. However, performance degradation is observed when the training dataset and testing datasets are drawn from different BS antenna configuration. Observation # 43: For DL TX beam prediction and Tx-Rx beam pair prediction, training a model with a mixture of dataset drawn from various BS antenna configurations allows the model to perform well for generalization. Observation # 44: For DL TX beam prediction and Tx-Rx beam pair prediction, performance degradation is not significant when the training dataset and testing datasets are drawn from different single antenna HPBW configurations. Training a model with a mixture of dataset is not required when there is a scaling change in the Tx beam pattern. |

    - Summary for BM-Case2

The following table summarized the observations from companies

| Scenarios/configurations | Observations |
| --- | --- |
| UE speeds | Case 2(A=>B) - Poor performance (Huawei/HiSi) - Performance drops (ZTE) - Degradation (Samsung) Case 3(Mix=>A or B) - moderate performance for Case 3 (Huawei/HiSi) - Outperform than Case 2(ZTE) - good generalization capability (xiaomi) - speeds can help obtain a model that generalizes (InterDigital) - better performance (Samsung) - acceptable generalization performance. (DoCoMo) |
| Variable set B | Case 2 - No better than non-AI case (Samsung) Case 3 - 10% performance loss (Samsung) - provide acceptable generalization performance (DoCoMo) |
| gNB antenna array | Case 2:(different training testing) - performance loss (vivo) - degradation different antenna conf. (Samsung) - Not significant with different single antenna HPBW configurations (Samsung) Case 3 - allows the model to perform well for generalization different antenna conf. (Samsung) |

1st round

Observation 4.2-1a (Different gNB antenna conf.)

- Based on the preliminary results, at least for BM-Case1, for different UE speed

  - Performance degradation is observed (Case 2)

  - Training with mixed data improve the generalization performance (Case 3)

| Company | Comments |
| --- | --- |
| FL0 | Please provide your view on the above observations. Wording update other than “too early to agree” is appreciated. |
| Xiaomi | “(Different gNB antenna conf.)” may be needed to be deleted. |
| Hw/HiSi | This observation seems correct and it can also be seen for BM-Case 2. |
| ZTE | The title should be ‘Different gNB antenna conf. UE speeds’. |
| Samsung | We think there may be another typo. Is the intention to say‘BM-Case 2“ |

3rd / 2nd round

Observation 4.2-1b (Different UE speed)

- Based on the preliminary results, at least for BM-Case2, for different UE speed

  - Performance degradation is observed (Case 2)

  - Training with mixed data improve the generalization performance (Case 3)

| Company | Comments |
| --- | --- |
| FL0 | Fixed the typo |
| Xiaomi | Support the observation |
| CAICT | Support |

Evaluation results for AI/ML in beam management

Working Assumption For both BM-Case1 and BM-Case 2, the following table is adopted as working assumption for reporting the evaluation results. Table X. Evaluation results for [BM-Case1 or BM-Case2] without model generalization for [DL Tx beam prediction or Tx-Rx beam pair prediction or Rx beam prediction] Company A; ……. Assumptions; Number of [beams/beam pairs] in Set A. Number of [beams/beam pairs] in Set B. Baseline scheme. AI/ML model input/output; Model input. Model output. Data Size; Training. Testing. AI/ML model; [Short model description]. Model complexity. Computational complexity. Evaluation results [With AI/ML / baseline]; [Beam prediction accuracy (%)]; [KPI A]. [KPI B] …. [L1-RSRP Diff]; [Average L1-RSRP diff] …. [System performance]; [RS overhead Reduction (%)/ RS overhead]. [UCI report]. [UPT] …. To report the following in table caption: - Which side the model is deployed Further info for the columns: - Assumptions - Number of beams/beam pairs in Set A - Number of beams/beam pairs in Set B - Baseline scheme, e.g., Option 1 (exhaustive beam sweeping), Option 2(based on measurements of Set B), or baseline described by companies - Other assumptions can be added later based on agreements - Model input: input type(s) - Model output: output type(s), e.g., the best DL Tx and/or Rx beam ID, and/or L1-RSRPs of N beams(pairs) - Dataset size, both the size of training/validation dataset and the size of test dataset - Short model description: e.g., CNN, LSTM - Model complexity, in terms of “number of model parameters” and/or size (e.g. Mbyte)”, and - Computational complexity in terms of FLOPs - Evaluation results: agreed KPIs, with AI/ML / with baseline scheme (if applicable) Note: To report other simulation assumptions, if any.

| Company | Observation |
| --- | --- |
|  | Confirm the working assumption for reporting the evaluation results with minor modification, i.e. Table X. Evaluation results for [BM-Case1 or BM-Case2] without model generalization with generalization-Case1 for [DL Tx beam prediction or Tx-Rx beam pair prediction or Rx beam prediction] Table X. Evaluation results for [BM-Case1 or BM-Case2] with generalization-Case2/generalization-Case3 for [DL Tx beam prediction or Tx-Rx beam pair prediction or Rx beam prediction] Company A; ……. Assumptions; Number of [beams/beam pairs] in Set A. Number of [beams/beam pairs] in Set B. Baseline scheme. AI/ML model input/output; Model input. Model output. Model input data for training at 1st configuration/scenario. Model input data for inference at 2nd configuration/scenario. Data Size; Training. Testing. AI/ML model; [Short model description]. Model complexity. Computational complexity. Evaluation results [With AI/ML / baseline]; [Beam prediction accuracy (%)]; [KPI A]. [KPI B] …. [L1-RSRP Diff]; [Average L1-RSRP diff] …. [System performance]; [RS overhead Reduction (%)/ RS overhead]. [UCI report]. [UPT] …. |

  - Evaluation results for BM-Case 1

    - General observations

| Company | Observation |
| --- | --- |
| Huawei/HiSi [2] | Observation 8: For spatial domain beam prediction, AI/ML-based schemes under the 64-DFT codebook outperform the legacy approach in most of the cases in terms of beam selection accuracy, e.g., - AI/ML-based Top-5 prediction reaches almost the upper performance bound with a prediction accuracy of 94.95% but with an overhead reduction of 67.17%. On the other hand, for the same overhead reduction, the established legacy Baseline approach can only achieve a prediction accuracy of 55.3% - With AI/ML-based Top-3 prediction, the overhead compared to the legacy Baseline approach can be further reduced by another 8%, while the prediction still is much higher (89.2% as opposed to 55.3%) Observation 9: For spatial domain beam prediction, AI/ML-based schemes under the 64-DFT codebook outperform the legacy approach in most of the cases in terms in terms of average L1-RSRP difference, e.g., - For AI/ML-based Top-5 prediction, the L1-RSRP difference compared to genie-aided beam prediction in Exhaustive 64 is as low as 0.03 dB, with an overhead reduction of 67.17%. On the other hand, for the same overhead reduction, the established legacy Baseline approach can only achieve an average L1-RSRP difference of 1.02dB - With AI/ML-based Top-3 prediction, the overhead compared to the legacy Baseline approach can be further reduced by another 8%, while the average L1-RSRP difference is still is much smaller (0.08dB as opposed to 1.02dB) Observation 14: The spatial domain beam prediction based on the Set A with 256 beams provides a considerable gain over the legacy upper bound Exhaustive 64 in achievable L1-RSRP using even less overhead associated with an Exhaustive 64 sweep. Observation 15: The comparison of DL Tx beam prediction and DL Tx-Rx beam pair prediction under BM-Case 1 shows that beam prediction has better performance, e.g. - For prediction accuracy (0dB RSRP difference) for Top-5, Tx-Rx beam pair: 93.5%; Tx beam (90% best Rx): 96.5%; Tx beam (best Rx): 97.5%. - 1dB RSRP difference for Top-5, Tx-Rx beam pair: 94.5%; Tx beam (90% best Rx): 97%; Tx beam (best Rx): 98.5%. |
| Ericsson [4] | In outdoor scenarios, AI/ML can reduce beam spatial-domain beam prediction overhead substantially while maintaining good accuracy for 4x8 (32 beams in Set A). In scenarios with primarily indoor UEs, spatial-domain beam predication is more challenging. With the adopted beam pattern, the conventional scheme could have very good performance which significantly outperforms the baseline schemes and have similar performance as AI/ML schemes. |
| Nokia[12] | - Observation 2: The design of Set A/B together with the ML model design should provide comparable or better sector throughput and cell-edge UE throughput compared to the non-ML baseline. |
| CAICT [16] | Observation 1: AI-based solution could achieve good performance for BM-Case 1 beam pair prediction with same training and validation set configurations. |
| CMCC [18] | Observation 1: Compared with baseline option 1, BM-Case 1 with Tx-Rx beam pair prediction has minor loss of prediction accuracy for Top-3 beam pair but has large beam sweeping overhead reduction. |
| NAVIDA [19] | Observation 2: AI/ML-based algorithms for beam prediction in spatial domain can achieve performance comparable to that of exhaustive beam search, while the reference signal overhead, measurement effort, reporting overhead, and latency can be much reduced. |
| Lenovo [20] | Observation 2: L1-RSRP of top-1 predicted beam from AI-based BM is very close to baseline with difference of 0.24dB. Observation 3: AI-based BM can achieve up to 98.08% Top4/1 beam prediction accuracy. Observation 4: Up to 92.25% and 97.33% beam prediction accuracy can be obtained with 1dB margin and 2dB margin of top-1 predicted beam L1-RSRP difference. Observation 5: AI-based BM is better than that of non-AI based BM and very close to that of the baseline. Observation 6: AI-based BM can achieve 75% RS overhead reduction while system performance loss is marginal, i.e., 0.82% UPT loss. |
| MediaTek [25] | Observation 3: Both AI/ML models can reach less than 10% throughput difference when comparing with Option1 baseline, when the number of beams in Set B is larger than 8. Observation 4: Transformer AI/ML model can achieve 100% throughput ratio with 50% RS overhead reduction. Observation 5: Incorrect beam prediction impacts the throughput more intensively for cell edge users than for all the users. Observation 6: Transformer AI/ML model may achieve >100% throughput ratio for cell-edge users with 50% RS overhead reduction. |
| CEWiT [26] | - Observation 1: For a sufficiently large Set-B size, the AI/ML model can perform as good as the baseline scheme with reduced overhead. - Observation 2: The performance of AI/ML degrades as the number of indoor users increases |

    - Tx-Rx beam pair prediction

| Company | Observation |
| --- | --- |
| Ericsson[4] | - For Set B with SSB beams, the joint TX/RX prediction can give quite good performance while significantly reducing RS overhead compared to measurements of all RX beams for each TX beam in Set B. |
| OPPO[6] | Observation 2: Spatial domain beam pair prediction can yield beam prediction accuracy (at least 80%) while overhead/latency reduction rate is 75%. Observation 3: The system level metric, i.e. spectrum efficiency or throughput, is not sensitive to the L1-RSRP difference introduced by spatial domain beam pair prediction. Observation 4: For 80% of the incorrect spatial domain beam pair prediction cases, the L1-RSRP difference can be kept within 2dB. Observation 5: When beam pair prediction accuracy is high (at least 80%) and L1-RSRP difference is small (within 1 dB), the system-level performance, i.e. spectrum efficiency or throughput, may only provide non-essential insight, therefore focusing on L1-RSRP difference and beam prediction accuracy would be enough. |
| Nokia [12] | Observation 18: The Top-1 beam pair prediction performances are sensible to the number of beam pairs in Set B.A second stage for measuring the Top-K predicted beam pairs can be used to significantly improve the model performance when the number of beam pairs in SetB is reduced. |

    - Set B is different from Set A

| Company | Proposal/observations |
| --- | --- |
| Huawei/HiSi [2] | Observation 10: For spatial domain beam prediction, better prediction accuracy is achieved when Set B is a subset of Set A compared to the case where Set B is a wide beam set, especially when K=1; with the increase of K, the gap between two options becomes narrower. |
| Nokia [12] | Observation 8: For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance. |
| Samsung [21] | Observation # 26 : For spatial domain prediction, AI can provide better performance in terms of beam prediction accuracy than non-AI based scheme with the measurements of a set of wide beams and a subset of narrow beams to select a best beam among a full set of narrow beams. Observation # 27: For spatial domain prediction, AI can predict the best narrow beam based on the measurements of wide beams only with decent performance. Observation # 28: For spatial domain prediction, AI can help gNB to predict the best narrow beam set that including the best narrow beam for UE to measure with high probability. |

  - Evaluation results for BM-Case2

    - General observations

| Company | Observation |
| --- | --- |
| Huawei/HiSi[2] | Observation 16: For temporal beam prediction, AI/ML based methods are more robust than legacy approaches to variations of the UE speed. - When the time interval is 0.08s in the observation and prediction window, for UE speed 30km/h, the AI/ML Top-8 approach is 42% better than for the legacy baseline but for a UE speed of 90 km/h, the AI/ML Top-8 prediction accuracy is 47% better than for the legacy baseline - When the time interval is 0.16s in the observation and prediction window, for UE speed 30km/h, the AI/ML Top-8 approach is 48% better than for the legacy baseline but for UE speed 90 km/h, the AI/ML Top-8 prediction accuracy is 77% better than for the legacy baseline. Observation 17: For temporal beam prediction, lower spatial consistency has more impact on the prediction accuracy achieved by the legacy approach than on accuracy achieved by the AI/ML-based methods. This can be seen from the results when different time instances are evaluated. - For UE at 30km/h, the accuracy of AI/ML Top-8 degrades 3.35% but the baseline degrades 4.8% when stretching the two prediction intervals from 0.08s to 0.16s - For UE at 90km/h, the accuracy of AI/ML Top-8 degrades 0.93% but the baseline degrades 9.56% when stretching the two prediction intervals from 0.08s to 0.16s |
| Ericsson [4] | - Evaluations indicate the possibility to increase the measurement periodicity from 40ms to 80ms, where the prediction-based method used to predict 40ms ahead indicates slight gain over baseline for the worst UEs Temporal beam prediction achieves minor gains over baseline in the considered scenarios. The gains for the 16-beam case are mainly due to the spatial beam prediction. |
| Nokia [12] | Observation 18: The Top-1 beam pair prediction performances are sensible to the number of beam pairs in Set B.A second stage for measuring the Top-K predicted beam pairs can be used to significantly improve the model performance when the number of beam pairs in SetB is reduced. Observation 26: In BM-Case2 with low UE speed configuration, the best beam prediction measured at time t from the reference non-ML method is accurate at least for a certain short period. |
| CMCC [18] | Observation 2: Compared with baseline option 1, BM-Case 1 with DL Tx beam prediction has minor loss of prediction accuracy for Top-2 and Top-3 beam prediction but has large beam sweeping overhead reduction. |
| NAVIDA [19] | Observation 3: AI/ML-based algorithms for beam prediction in time domain can simply use a history of the best beam index to perform the prediction. Observation 4: AI/ML-based algorithms for beam prediction in time domain can help lower reference signal overhead and reduce UE’s measurement requirement. |
| Samsung [21] | Observation # 37: In the case of non-AI, there is almost no performance degradation due to the increase in target predict time. Since the coverage of beams in Set B is wide, Top-1 prediction accuracy of the selected beam in Set B slightly decreases as the target predict time increases. Observation # 38: In the case of AI, the performance is superior to non-AI, but it can be observed that it decreases as the target predict time increases. Due to the narrow coverage of beams in Set A, it would be hard for AI to learn the Top-1 beam after longer time later based on the latest measurement. |
| Qualcomm [23] | Observation 1 At least for BM-Case2, AI/ML-based methods will provide an advantage in high-stress scenarios where frequent UE orientation changes lead to rapid changes in the best beams. Observation 2 For BM-Case2 with high UE rotation speeds, the AI/ML-based method (LSTM) strongly outperforms the sample-and-hold baseline, especially in the UE Rx beam prediction and Tx-Rx beam pair prediction use cases. - The rapid rotation leads to significant changes in best-beam RSRPs between measured cycles; the LSTM can predict for these changes, while the sample-and-hold scheme breaks down. |
| MediaTek [25] | Observation 1: The AI/ML approach does not show much gain for BM-Case2 in terms of average throughput, compared to baseline Option1a and Option2, when UE is reporting the Top-4 beams. |

    - DL Tx beam prediction

| Company | Observation |
| --- | --- |
| OPPO[6] | Observation 11: Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 74.4%) while overhead/latency reduction can be up to 87.5% (for the case of K = 4, F = 4 and Set B = 32 beam pairs, Set A = 128 beam pairs). Observation 12: Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 64.5%) while overhead/latency reduction can be up to 87.5% (for the case of K = 8, F = 8 and Set B = 32 beam pairs, Set A = 128 beam pairs). Observation 13: For spatial and temporal domain beam prediction, the longer beam prediction period (e.g. F = 8 prediction instances), the deeper performance loss can be observed given the same measurement period (e.g. K = 8 measurement instances). |
| Vivo [8] | Observation 23: For BM-Case2, compared with non-AI 2-step scheme, AI based 2-step scheme improves beam prediction accuracy and reduces average L1-RSRP difference significantly. Proposal 23: Further study performance comparison between enhanced beam pair prediction and DL Tx beam prediction with various Rx beam assumptions, such as worst Rx beam, second best Rx beam, random Rx beam per sample, etc., in BM-Case2. |
| Nokia [12] | Observation 27: In BM-Case2, compared to baseline option 2, the ML method shows beam prediction performance improvement in different future time instants even when the testing prediction window is longer than the training prediction window, i.e. [F+1, …, F’]. Observation 28: In BM-Case2, the ML method only using beam RSRP input will be more useful for beam prediction for a relatively short future period. Observation 29: For BM-Case2, the ML model using as input only RSRPs has performance that decreases when Set B is a subset of Set A and if no advanced algorithm is applied for beam selection in Set B. Observation 30: For BM-Case2, the ML model using as input only RSRPs has performance that decreases when increasing the length of the prediction window. |

    - Tx-Rx Beam pair prediction

| Company | Observation |
| --- | --- |
| OPPO[6] | Observation 7: Temporal domain beam pair prediction can provide prediction accuracy (e.g. 81%) while overhead/latency reduction is as large as 50% (for the case of K = 4 and F = 4). Observation 8: Beam pair predication accuracy slightly decreases from 89.1% to 81% (the case of Top-1) when F increases from 1 to 4, but strongly increases from 81% to 98.9% (the case of F = 4) when the number of predicted beam pair increases from Top-1 to Top-4. Observation 9: For 80% of the incorrect temporal domain beam pair prediction cases, the L1-RSRP difference is lower than 3.5dB. |
| Vivo [8] | Observation 22: For BM-Case2, compared with non-AI scheme, beam pair prediction scheme improves beam prediction accuracy and reduces average L1-RSRP difference significantly. Proposal 22: Further study beam pair prediction scheme with expected information as AI input for improving generalization performance in BM-Case2. |
| Xiaomi[10] | Set A=Set B Observation 11: both AI based beam prediction scheme 1 and scheme 2 in time domain can provide good performance. - Scheme 1 assumes same periodicity for history measurement instance and future time instance. - Scheme 2 assumes that periodicity for history measurement instance is N times of future time instance. It can reduce more RS overhead than scheme 1. Observation 12: Set B < set A causes much more performance degradation compared to set B=set A for temporal beam prediction. Observation 13: The performance may degrade when larger N (history measurement instances) is assumed. |

    - Comparison between DL Tx beam and Tx-Rx beam pair

| Company | Observation |
| --- | --- |
| ZTE [3] | - Case 2-2-1: RSRPs of all Tx beams of 5 past time instances are used as the model input and RSRPs of all Tx beams of 5 future time instances are used as the model output, where the optimal Rx beam is assumed for measurement - Case 2-2-2: RSRPs of all Tx-Rx beam pairs of 5 past time instances are used as the model input and RSRPs of all Tx-Rx beam pairs of 5 future time instances are used as the model output, where the optimal Rx beam is assumed for measurement - Case 2-2-3: RSRPs of all Tx beams of 5 past time instances are used as the model input and RSRPs of all Tx beams of 5 future time instances are used as the model output, where the specific (i.e., first) Rx beam is assumed for measurement - Case 2-2-4: RSRPs of all Tx-Rx beam pairs of 5 past time instances are used as model input and RSRPs of all Tx-Rx beam pairs of 5 future time instances are used as model output, where the specific (i.e., first) Rx beam is assumed for measurement Observation 19: In BM-Case2, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction with the assumption of the optimal Rx beam or the specific Rx beam. |
| OPPO[6] | Observation 10: For BM-Case2, the case of Tx beam prediction can slightly outperform that of beam pair prediction in terms of prediction accuracy and L1-RSRP difference. |

  - Other aspects

| Company | Observation |
| --- | --- |
| China Telecom | Observation 1: Modelling the spatial beam prediction task as a classification model provides better performance with less training overhead. |

Proposals for online discussion

Proposal 2.1.1-1b

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as

  - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

  - Note: This is only for evaluation discussion

Proposal 2.1.1-2a

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Other options are not precluded and can be reported by companies.

Proposal 2.1.2-1d

- To evaluate of accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as:

  - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of Top-1[/K] predicted beam.

  - Note: This can be used for the predicted L1-RSRP reporting for UE side model

Proposal 3.1-2b

- For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study for potential down selection the following options as AI/ML model inputs

  - Alt 1: No information of Tx beam ID and/or Rx beam ID

  - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID

  - Alt 3: Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly

  - Note: The information of Tx beam ID and/or Rx beam ID is not treated as assistance information

Proposal 3.2-1e

- At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

  - Case 0: the best Rx beam for the best Tx beam within Set B

  - Case 0a: the best Rx beam searched for one Tx beam within Set B

  - Case 1: the best Rx beam for each Tx beam within Set B

  - Case 3: the best Rx beam among specific Rx beams for each Tx beam within Set B

  - Case 4: the best Rx beam for the best Tx beam with in Set A

  - Note: The best Rx beam may be based on measurements of always-on SSB, or CSI-RS for Set B

Proposal 3.2-2b

- For dataset construction for training and/or inference for AI/ML in BM, consider the following options with potential down selection:

  - Opt A: measurements are obtained by measuring the always-on SSB

    - 20ms periodicity of each SSB burst is used for the evaluation

  - Opt B: measurements are obtained by measuring the CSI-RS

    - [1ms] periodicity of CSI-RS with repetition-on is assumed for the evaluation.

  - Opt C: measurements are obtained ideally among measurements generated in one shot

Agreements

  - Agreements on Monday

Agreement

- Further study the impact of quantization error of inputed L1-RSRP (for training and inference) for AI/ML model for beam management.

  - Existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) is the starting point for evaluation at least for network-sided model.

Agreement

- Further study on whether/how to evaluate the performance impact with L1-RSRP measurement accuracy.

Reference

| [1] | R1-2300046 | Discussion and evaluation of AI/ML for beam management | FUTUREWEI |
| --- | --- | --- | --- |
| [2] | R1-2300110 | Evaluation on AI/ML for beam management | Huawei, HiSilicon |
| [3] | R1-2300173 | Evaluation on AI beam management | ZTE |
| [4] | R1-2300179 | Evaluations of AIML for beam management | Ericsson |
| [5] | R1-2300213 | Evaluation on AI/ML for beam management | Spreadtrum Communications |
| [6] | R1-2300282 | Evaluation methodology and results on AI/ML for beam management | OPPO |
| [7] | R1-2300399 | On Evaluation of AI/ML based Beam Management | Google |
| [8] | R1-2300446 | Evaluation on AI/ML for beam management | vivo |
| [9] | R1-2300532 | Evaluation on AI/ML for beam management | LG Electronics |
| [10] | R1-2300569 | Evaluation on AI/ML for beam management | xiaomi |
| [11] | R1-2300593 | Discussion for evaluation on AI/ML for beam management | InterDigital, Inc. |
| [12] | R1-2300606 | Evaluation of ML for beam management | Nokia, Nokia Shanghai Bell |
| [13] | R1-2300673 | Evaluation on AI/ML for beam management | CATT |
| [14] | R1-2300718 | Evaluation on AI/ML for beam management | China Telecom |
| [15] | R1-2300746 | Evaluation on AI/ML for beam management | Fujitsu |
| [16] | R1-2300843 | Some discussions on evaluation on AI-ML for Beam management | CAICT |
| [17] | R1-2300943 | Evaluations for AI/ML beam management | Intel Corporation |
| [18] | R1-2300992 | Discussion on evaluation on AI/ML for beam management | CMCC |
| [19] | R1-2301180 | Evaluation of AI and ML for beam management | NVIDIA |
| [20] | R1-2301201 | Evaluation on AI/ML for beam management | Lenovo |
| [21] | R1-2301257 | Evaluation on AI/ML for Beam management | Samsung |
| [22] | R1-2301339 | Evaluation for AI/ML based beam management enhancements | Apple |
| [23] | R1-2301406 | Evaluation on AI/ML for beam management | Qualcomm Incorporated |
| [24] | R1-2301487 | Discussion on evaluation on AI/ML for beam management | NTT DOCOMO, INC. |
| [25] | R1-2301589 | Evaluation on AI/ML for beam management | MediaTek Inc. |
| [26] | R1-2301689 | Evaluation on AI/ML for beam management | CEWiT |

Appendix: Agreements

  - Agreements in RAN 1 #109e

R1-2205269	Feature lead summary #1 evaluation of AI/ML for beam management	Moderator (Samsung)

From May 17th GTW session

Agreement

- For dataset construction and performance evaluation (if applicable) for the AI/ML in beam management, system level simulation approach is adopted as baseline

  - Link level simulation is optionally adopted

Agreement

- At least for temporal beam prediction, companies report the one of spatial consistency procedures:

  - Procedure A in TR38.901

  - Procedure B in TR38.901

Agreement

- At least for temporal beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

- For spatial-domain beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

Agreement

- At least for spatial-domain beam prediction in initial phase of the evaluation, UE trajectory model is not necessarily to be defined.

Agreement

- At least for temporal beam prediction in initial phase of the evaluation, UE trajectory model is defined. FFS on the details.

Agreement

- UE rotation speed is reported by companies.

  - Note: UE rotation speed = 0, i.e., no UE rotation, is not precluded.

Agreement

- For AI/ML in beam management evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline.

Conclusion

Further study AI/ML model generalization in beam management evaluating the inference performance of beam prediction under multiple different scenarios/configurations.

- FFS on different scenarios/configurations

- Companies report the training approach, at least including the dataset assumption for training

Agreement

- For evaluation of AI/ML in BM, the KPI may include the model complexity and computational complexity.

  - FFS: the details of model complexity and computational complexity

Agreement

- For spatial-domain beam prediction, further study the following options as baseline performance

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

    - FFS CSI-RS/SSB as the RS resources

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams

    - FFS: Set B is a subset of Set A and/or Set A consists of narrow beams and Set B consists of wide beams

    - FFS: how conventional scheme to obtain performance KPIs

    - FFS: how to determine the subset of RS resources is reported by companies

  - Other options are not precluded.

Decision: As per email decision posted on May 22nd,

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the parameters (if applicable) in Table 1.2-1b for Dense Urban scenario for SLS

Table 1.2-1b Assumptions for Dense Urban scenario for AI/ML in beam management

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 30km/h (baseline), 60km/h (optional) - Other values are not precluded |
| UE distribution | - FFS UEs per sector/cell for evaluation. More UEs per sector/cell for data generation is not precluded. - For spatial domain beam prediction: FFS: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | - [One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline] - [Four panels: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ as optional] - Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | [Panel structure: (M,N,P) = (1,4,2)] - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FFS: - Option 1: Full buffer - Option 2: FTP model Other options are not precluded |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | [40 dBm] |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

Agreement

- For temporal beam prediction, the following options can be considered as a starting point for UE trajectory model for further study. Companies report further changes or modifications based on the following options for UE trajectory model. Other options are not precluded.

  - Option #2: Linear trajectory model with random direction change.

    - UE moving trajectory: UE will move straightly along the selected direction to the end of an time interval, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - UE move straightly within the time interval with the fixed speed.

    - FFS on UE orientation

  - Option #3: Linear trajectory model with random and smooth direction change.

    - UE moving trajectory: UE will change the moving direction by multiple steps within an time internal, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - The time interval is further broken into N sub-intervals, e.g. 100ms per sub-interval, and at the end of each sub-interval, UE change the direction by the angle of A_diff/N.

      - UE move straightly within the time sub-interval with the fixed speed.

    - FFS on UE orientation

  - Option #4: Random direction straight-line trajectories.

    - Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed.

      - The initial UE location should be randomly drop within the following blue area

where d1 is the minimum distance that UE should be away from the BS.

        - Each sector is a cell and that the cell association is geometry based.

        - During the simulation, inter-cell handover or switching should be disabled.

For training data generation

    - For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance.

      - The value of T (or D) can be further discussed

      - The trajectory sampling interval granularity depends on UE speed and it can be further discussed.

    - UE can move straightly along the entire trajectory, or

    - UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length ΔT

      - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°]

    - If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated.

      - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded.

      - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.

    - FFS on UE orientation

- Generalization issue is FFS

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the following assumption for LLS as optional methodology

| Parameter | Value |
| --- | --- |
| Frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Data allocation | [8 RBs] as baseline, companies can report larger number of RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or Non-ideal (Companies explain how is oppler) |
| Channel model | FFS: LOS channel: CDL-D extension, DS = 100ns NLOS channel: CDL-A/B/C extension, DS = 100ns Companies explains details of extension methodology considering spatial consistency Other channel models are not precluded. |
| BS antenna configurations | - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline - Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS antenna element radiation pattern | Same as SLS |
| BS antenna height and antenna array downtile angle | 25m, 110° |
| UE antenna configurations | Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE antenna element radiation pattern | Same as SLS |
| UE moving speed | Same as SLS |
| Raw data collection format | Depends on sub-use case and companies’ choice. |

Decision: As per email decision posted on May 25th,

Agreement

- For UE trajectory model, UE orientation can be independent from UE moving trajectory model. FFS on the details.

  - Other UE orientation model is not precluded.

Agreement

- Companies are encouraged to report the following aspects of AI/ML model in RAN 1 #110. FFS on whether some of aspects need be defined or reported.

  - Description of AI/ML model, e.g, NN architecture type

  - Model inputs/outputs (per sub-use case)

  - Training methodology, e.g.

    - Loss function/optimization function

    - Training/ validity /testing dataset:

      - Dataset size, number of training/ validity /test samples

      - Model validity area: e.g., whether model is trained for single sector or multiple sectors

      - Details on Model monitoring and model update, if applicable

  - Others related aspects are not precluded

Agreement

- To evaluate the performance of AI/ML in beam management, further study the following KPI options:

  - Beam prediction accuracy related KPIs, may include the following options:

    - Average L1-RSRP difference of Top-1 predicted beam

    - Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

      - Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

    - CDF of L1-RSRP difference for Top-1 predicted beam

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

    - the definition of L1-RSRP difference of Top-1 predicted beam:

      - the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

    - Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

  - System performance related KPIs, may include the following options:

    - UE throughput: CDF of UE throughput, avg. and 5%ile UE throughput

    - RS overhead reduction at least for spatial-domain beam prediction at least for top-1 beam:

      - 1-N/M,

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement

        - where (FFS) M is the total number of beams

        - Note: Non-AI/ML approach based on the measurement of these M beams may be used as a baseline

      - FFS on whether to define a proper value for M for evaluation.

    - Other System performance related KPIs are not precluded and can be reported by companies.

  - Other KPIs are not precluded and can be reported by companies, for example:

    - Reporting overhead reduction: (FFS) The number of UCI report and UCI payload size, for temporal /spatial prediction

    - Latency reduction:

      - (FFS) (1 – [Total transmission time of N beams] / [Total transmission time of M beams])

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) in the input beam set required for measurement

        - where M is the total number of beams

    - Power consumption reduction: FFS on details

Final summary in R1-2205641.

  - Agreement in RAN 1 #110

Agreement

The Following updated based on the agreements in RAN 1 #109-e is adopted

| Parameters | Values |
| --- | --- |
| UE distribution | - FFS 10 UEs per sector/cell for system performance related KPI (if supported) [e.g,, throughput] for full buffer traffic (if supported) evaluation (model inference). - X UEs per sector/cell for system performance related KPI for FTP traffic (if supported) evaluation (model inference). - Other values are not precluded - Number of UEs per/sector per cell during data collection (training/testing) is reported by companies if relevant - More UEs per sector/cell for data generation is not precluded. |
| UE Antenna Configuration | - Antenna setup and port layouts at UE: [1,2,1,4,2,1,1], 2 panels (left, right) - [Panel structure: (M,N,P) = (1,4,2)] - panels (left, right) with (Mg, Ng) = (1, 2) as baseline - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |

Agreement

The Following updated based on the agreements in RAN 1 #109-e is adopted

| Parameters | Values |
| --- | --- |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 3km/h(optional), 30km/h (baseline), 60km/h (optional), 90km/h (optional), 120km/h (optional) - Other values are not precluded |
| UE distribution | - For spatial domain beam prediction: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |

Agreement

- If UE orientation is modeled, it can be independently modeled from UE moving trajectory model.

  - This is not precluded that UE orientation coupled with UE moving trajectory model.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

Agreement

- To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead can be further studied as one of KPI options.

  - FFS: number of UCI reports and UCI payload size

  - Agreements in RAN 1 #110bis

Working Assumption

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

      - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Conclusion

- For system performance related KPI (if supported) evaluation (model inference), companies report either of the following traffic model:

  - Option 1: Full buffer

  - Option 2: FTP model with detail assumptions (e.g., FTP model 1, FTP model 3)

Agreement

- BS antenna configuration:

  - antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ

  - Other assumptions are not precluded

- BS Tx power for evaluation:

  - 40dBm (baseline)

  - Other values (e.g. 34 dBm) are not precluded and can be reported by companies

- UE antenna configuration (Clarification of agreement in RAN 1 #110):

  - antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right)

  - Other assumptions are not precluded

Agreement

- For the evaluation of both BM-Case1 and BM-Case2, 32 or 64 downlink Tx beams (maximum number of available beams) at NW side.

  - Other values, e.g., 256, etc, are not precluded and can be reported by companies.

- For the evaluation of both BM-Case1 and BM-Case2, 4 or 8 downlink Rx beams (maximum number of available beams) per UE panel at UE side.

  - Other values, e.g., 16, etc, are not precluded and can be reported by companies.

Agreement

- The options to evaluate beam prediction accuracy (%):

  - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam”

  - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

  - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

  - Where K >1 and values can be reported by companies.

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam considers the following options

  - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

Agreement

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair considers the following options:

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

Agreement

- For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, the set of scenarios/configurations are considered focusing on one or more of the following aspects as a starting point:

  - Scenarios

    - Various deployment scenarios

    - Various outdoor/indoor UE distributions

    - Various UE mobility

  - Configurations

    - Various UE parameters

    - Various gNB settings

    - [Various Set B of beam(pairs)]

  - Other aspects of scenarios/configurations are not precluded

  - The selected scenarios/configurations for generalization verification may consider the AI model inference node (e.g., @UE or @gNB) and use case (e.g., BM-Case1, or BM-Case2)

- Companies to report the selected scenarios/configurations for generalization verification

- Note: other approaches for achieving good generalization performance for AI/ML-based schemes are not precluded.

Working Assumption

For both BM-Case1 and BM-Case 2, the following table is adopted as working assumption for reporting the evaluation results.

Table X. Evaluation results for [BM-Case1 or BM-Case2] without model generalization for [DL Tx beam prediction or Tx-Rx beam pair prediction or Rx beam prediction]

|  |  |  | Company A | …… |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of [beams/beam pairs] in Set A |  |  |
|  |  | Number of [beams/beam pairs] in Set B |  |  |
|  |  | Baseline scheme |  |  |
| AI/ML model input/output |  | Model input |  |  |
|  |  | Model output |  |  |
| Data Size |  | Training |  |  |
|  |  | Testing |  |  |
| AI/ML model |  | [Short model description] |  |  |
|  |  | Model complexity |  |  |
|  |  | Computational complexity |  |  |
| Evaluation results [With AI/ML / baseline] | [Beam prediction accuracy (%)] | [KPI A] |  |  |
|  |  | [KPI B] … |  |  |
|  | [L1-RSRP Diff] | [Average L1-RSRP diff] … |  |  |
|  | [System performance] | [RS overhead Reduction (%)/ RS overhead] |  |  |
|  |  | [UCI report] |  |  |
|  |  | [UPT] … |  |  |

To report the following in table caption:

      - Which side the model is deployed

Further info for the columns:

- Assumptions

  - Number of beams/beam pairs in Set A

  - Number of beams/beam pairs in Set B

  - Baseline scheme, e.g., Option 1 (exhaustive beam sweeping), Option 2(based on measurements of Set B), or baseline described by companies

  - Other assumptions can be added later based on agreements

- Model input: input type(s)

- Model output: output type(s), e.g., the best DL Tx and/or Rx beam ID, and/or L1-RSRPs of N beams(pairs)

- Dataset size, both the size of training/validation dataset and the size of test dataset

- Short model description: e.g., CNN, LSTM

- Model complexity, in terms of “number of model parameters” and/or size (e.g. Mbyte)”, and

- Computational complexity in terms of FLOPs

- Evaluation results: agreed KPIs, with AI/ML / with baseline scheme (if applicable)

Note: To report other simulation assumptions, if any.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is randomly changed among pre-configured patterns

    - Opt C: Set B is randomly changed among Set A beams (pairs)

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

Working assumption

- For the evaluation of the overhead for BM-Case1, further study the following two metrics for potential down selection:

  - Option A: RS overhead reduction, FFS for potential down selection:

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on beam sweeping

    - Option 3: RS OH reduction%=1-N+PM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where M is the total number of beams (pairs) to be predicted

      - FFS the following alternatives consider different targets (e.g., beam or beam pair) for prediction:

        - Alt1: P is the number of Top-K selected beams (pairs) for beam sweeping (if applicable)

        - Alt2: P is the number of Top-K selected beams (pairs) not in Set B for beam sweeping (if applicable)

        - Alt3: P is the number of beams used for beam sweeping to get the best Rx beam (if applicable)

        - Companies report the assumption on beam sweeping

    - Other options can be reported by companies

  - Option B: RS overhead, FFS for potential down selection:

    - Option 1: RS OH = N,

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

    - Option 2: RS OH = N + P

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - FFS the following alternatives consider different targets (e.g., beam or beam pair) for prediction:

        - Alt1: P is the number of Top-K selected beams (pairs) for beam sweeping (if applicable)

        - Alt2: P is the number of Top-K selected beams (pairs) not in Set B for beam sweeping (if applicable)

        - Alt3: P is the number of beams used for beam sweeping to get the best Rx beam (if applicable)

        - Companies report the assumption on beam sweeping

    - Other options can be reported by companies

Agreement

- At least for BM-Case 2, consider the following assumptions for evaluation

  - Periodicity of time instance for each measurement/report in T1:

    - 20ms, 40ms, 80ms, [100ms], 160ms, [960ms]

    - Other values can be reported by companies.

  - Number of time instances for measurement/report in T1 can be reported by companies.

  - Time instance(s) for prediction can be reported by companies.

  - Agreements in RAN 1 #111

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- The following case for generalization verification, can be optionally considered by companies:

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

    - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

    - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side

Agreement

- For the evaluation of the overhead for BM-Case1, adoption the following metrics:

  - RS overhead reduction,

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Companies report the assumption on additional measurements

Agreement

- Companies report the pattern of Set B.

- Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B).

Agreement

For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering

- Various Set B of beam(pairs)

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

- Option 2: Measurements of specific Rx beam(s)

  - Option 2a: Measurements of specific Rx beam(s) per model input sample

  - Option 2b: Measurements of specific Rx beam(s) for all model input sample

  - FFS how to select the specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

- Other options are not precluded and can be reported by companies.

Agreement

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on additional measurements

    - FFS: Option 3: RS OH reduction%=1-NLM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance

      - where M is the total number of beams (pairs) to be predicted for each time instance

      - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

    - Companies report the assumption on T1 and T2 patterns

    - Other options are not precluded and can be reported by companies.