3GPP TSG-RAN WG1 Meeting #112	Tdoc R1-2302170

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.2.4.1

Source:	Moderator (Ericsson)

Title:	Summary #6 of Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#112 for the agenda item 9.2.4.1, Evaluation on AI/ML for positioning accuracy enhancement.

This discussion corresponds to the objectives related to the positioning use case described in RP-213599 (SID) below.

RP-213599 (SID): Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. … Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

# (Closed) Evaluation methodology

## Training dataset size / UE density

In RAN1#111, the following agreement was made to investigate user density/size of the training dataset:

Agreement

Study how AI/ML positioning accuracy is affected by: user density/size of the training dataset.

Note: details of user density/size of training dataset to be reported in the evaluation.

Selected input from companies’ contribution are copied below, with evaluation results shown in section 6.4 and 7.4.

- Nokia (R1-2300608) Observation-2: The availability of good quality data with sufficient diversity of positioning ground truth labels and samples with accurate information for model training, testing/validation, and monitoring is one of the key challenges in AI/ML-based positioning. Observation-3: It is important to note that training dataset size as an indication of user area density is valid only for uniform distribution of UEs within the simulation setting. Proposal-2: For evaluation of AI/ML based positioning, consider additional UE distribution options such as sparse or clustered deployment of UEs to better represent real-world scenarios. Observation-4: For the UE-based positioning method with UE-based AI/ML model training and inference, currently, it is unclear as to how to ensure that the provided training data is utilized in a manner than ensures optimal model performance. Observation-5: The dataset size is not a relevant parameter to indicate the quality of the model training. Proposal-3: For evaluation of AI/M-based positioning, consider Inter-point distance (IPD) metric-based criteria for collection of label data to improve model training/testing/validation. Figure 8 - Positioning accuracy vs. IPD for 18, 9, and 6 TRPs cases. Observation-14: With the same training dataset size but with different spatial distribution, the positioning accuracy degradation can reach 55%. Proposal-8: RAN1 to agree on the importance of studying/evaluating the data diversity in general, and inter-point distance (IPD) in particular.

- Ericsson (R1-2300141) Table 5 Proposed dataset size classes. Train dataset size class; User density [UE/m2]; Number of samples over 60 m × 120 m; Number of samples over 40 m × 100 m; Number of sources reported in RAN1 #111 for direct positioning. Small datasets; ~1.39; ~10,000; ~5,556; [~3]. Medium-size datasets; ~2.79; ~20,000; ~11,111; [~5]. Large datasets; ~5.56; ~40,000; ~22,222; [~3]. Very large datasets; >6.94; >50,000; >27,778; [~7]. Proposal 4 To further 3GPP discussion and preparation of observations/conclusions for the technical report, four train dataset size classes are defined as follows: - Small datasets: density ~1.39 UE/m2 - Medium-size datasets: density ~2.79 UE/m2 - Large datasets: density ~5.56 UE/m2 - Very large datasets: >6.94 UE/m2

## (Closed) 1st round discussion

Checking companies’ input, an important issue to discuss is the evaluation area where the UEs are dropped.

For evaluation methodology, the following was agreed.

| UE horizontal drop procedure | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull. |
| --- | --- |

Specifically, for the agreed baseline hall size of 120x60 m, the baseline (whole hall area) evaluation area is: Aw=120 m × 60 m = 7,200 m2.

The optional (the convex hull) evaluation area is: Ac=100 m × 40 m = 4,000 m2, which is only 55.5% of the baseline evaluation area Aw.

According to the contributions submitted to RAN1#112, most companies used the baseline UE distribution of 120x60 m in their simulation, while some used the smaller area of 100x40 m for convex hull.

- (Baseline) Whole hall area of 120x60 m: CAICT, CATT, Ericsson, InterDigital, MediaTek, Nokia, NVIDIA, Qualcomm, ZTE.

- (Optional) Convex hull of 100x40 m: Apple, China Telecom, OPPO

- Unclear: CMCC, Huawei, Samsung, Xiaomi, vivo

Additionally, MediaTek evaluated the large hall size of 300x150 m for InF-SH, which has a much larger UE distribution area, hence larger training dataset is needed when UE density is the same. Clearly the various UE distribution area used by companies significantly affects user density for a given training dataset size.

For investigation of this issue, the following agreement was made in RAN1#111. Considering the various UE distribution area, user density and training dataset are not equivalent. User density is a much better common metric, while training dataset size was agreed in the templates. To remedy this problem, companies should clearly mark their assumption of evaluation area in the reporting table, especially if the UE distribution area is not the baseline of 120x60 m. Then user density can be easily calculated and compared across companies.

Agreement

Study how AI/ML positioning accuracy is affected by: user density/size of the training dataset.

Note: details of user density/size of training dataset to be reported in the evaluation.

Thus, it is proposed that companies clearly mark it as shown below in the templates for evaluation results.

Proposal 2.2-1

For both direct AI/ML positioning and AI/ML assisted positioning, companies include the evaluation area in their reporting template, assuming the same evaluation area is used for training dataset and test dataset.

Note:

- Default evaluation area = 120x60 m.

- if different evaluation areas are used for training dataset and test dataset, they are marked out separately under “Train” and “Test” instead.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m]

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Model complexity | Computation complexity | AI/ML |

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | Test | Model complexity | Computation complexity | AI/ML |

|  | Company |
| --- | --- |
| Support | CATT, NOK |
| Not support |  |

| Company | Comments |
| --- | --- |
| Hw/HiSi | Ok |
| Fujitsu | 120*60 can be default, only distribution area other than default needs to be reported. |
| NOK | We support the proposal. However, accordingly with our Proposal 2, companies should complement the evaluations using different UE distribution (not only uniform distribution) rather that only “distribution area”. For example, sparse or clustered deployment to better represent real-world scenarios. [Moderator] Only “Uniformly distributed” is agreed as evaluation assumption. Other distribution (e.g., sparse or clustered) need further agreement. Also: interested companies can submit evaluation results for other distribution. |
| Qualcomm | We are fine with the proposal. |
| Mediatek | Support, for generalization of different hall size, we can set hall size in Settings Settings (e.g., drops, clutter param, mix). Train; Test. 120*60; 300*150. And we also proposal to further evaluate performance of AI/ML positioning for non-uniform UE distribution. [Moderator] Edited proposal to reflect. |

# (Closed) KPI

In the following, the remaining issues on KPI are discussed.

## Model complexity and computational complexity

- Ericsson (R1-2300141) Figure 4: Model complexity vs computational complexity for direct positioning models as reported in RAN1 #111 except for three updated Ericsson’s reported values from this contribution. Proposal 1 For 3GPP AI/ML for PHY SI discussion, companies shall report nominal computational complexity values (and not accelerator-optimized computational complexity values). Otherwise, the reported computation complexity value cannot be included for a fair cross-company comparison. Proposal 2 To further 3GPP discussion and preparation of observations/conclusions for the technical report, three model size classes are defined for the positioning use case as follows: - Small models: < 1 M model parameters - Medium-size models: 1 – 8 M model parameters - Large models: > 8 M model parameters Proposal 3 For 3GPP AI/ML for PHY SI discussion, companies shall report model complexity in terms of “number of real-valued model parameters” regardless of underlying model arithmetic.

- Lenovo (R1-2301203) Proposal 3: In addition to FLOP counts, the AI/ML positioning evaluation should also consider the type of data being used as input, training type, e.g., offline vs online, complexity type, e.g., worst-case/average-case.

### 1st round discussion

For reporting of computational complexity values, the following conclusions have been made.

Conclusion (RAN1#110bis) This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP. Conclusion (RAN1#111) Companies describe how their computational complexity values are obtained. - It is out of 3GPP scope to consider computational complexity values that have platform-dependency and/or use implementation (hardware and software) optimization solutions.

It is anticipated that RAN1 need to capture model complexity and computational complexity values from participating companies in TR 38.843. To have fair comparison, it is necessary that all companies report complexity calculated in the same way. Considering RAN1 conclusions above, it is reasonable that all companies report the nominal computational complexity values, i.e., not optimized for hardware platform like GPU or TPU.

Regarding the model complexity, it is useful to clarify model complexity in terms of “number of real-valued model parameters” regardless of whether the model is based on real value arithmetic or complex value arithmetic. This allows easy comparison between companies.

Proposal 3.1.1-1

For AI/ML-based positioning, companies shall report nominal computational complexity values (i.e., not accelerator-optimized computational complexity values). Otherwise, the reported computation complexity value cannot be included in TR 38.843.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | We should first agree on the definition of “nominal computational complexity values”. It seems better to make this generic guidance on complexity in 9.2.1. |
| Fujitsu | A little bit confused, we agreed to use FLOPs as the computational complexity other than FLOPS, the former one should be related to the model only and will not be affected by the platform, if only CPU used for model training or inference it will be very slow but the computational complexity will not be changed. |
| CATT | We think this issue is a common issue, which should be aligned with other use cases. Maybe it’s better to be discussed in 9.2.1. |
| OPPO | We also think it’s better to be discussed in 9.2.1. |
| NOK | Agree with OPPO and HW, discuss based on general aspects (9.2.1) will guide on this topic. |
| Mediatek | Not sure if there is difference of computational complexity values between CPU and GPU. We can follow the discussion in 9.2.1. |
| Moderator | OK to address this issue in 9.2.1. |

Proposal 3.1.1-2

For AI/ML-based positioning, companies shall report model complexity in terms of “number of real-valued model parameters” regardless of underlying model arithmetic.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Hw/HiSi | Probably better to be handled in 9.2.1? |
| Fujitsu | Also bit confused, the AI/ML model only recognize real-value so complex values must be split/transformed to real-value first and then fed into the model, so the number of model parameters can only be real-valued. |
| CATT | Same comments as Proposal 3.1.1-1. |
| OPPO | Fine with the proposal, but maybe it’s better to be discussed in 9.2.1. |
| Qualcomm | We are fine with the proposal. |
| Moderator | The following agreement was made under AI 9.2.4.1. The intention is to update/clarify this agreement. The issue is, some companies (Ericsson) used complex arithmetic to build the model, thus the model parameters (weights, biases) are complex number. To be fair, the number of complex parameters should be converted to real parameters by multiplying by 2. Agreement (RAN1#110) For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of model parameters”. One way is to rephrase the proposal as below. Proposal 3.1.1-2a The agreement made in RAN1#110 AI 9.2.4.1 is updated to the following: For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of real-valued model parameters”. |

Proposal 3.1.1-3

Based on model complexity range, categorize the AI/ML models in three model size classes for the positioning use case:

- Small models: < 1 M model parameters

- Medium-size models: 1 – 8 M model parameters

- Large models: > 8 M model parameters

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Hw/HiSi | Too early to make a categorization. We need maybe first more simulations and observe the model complexity that has been reported. |
| CATT | We don’t see the meaning for categorizing the AI model size. |
| Qualcomm | The motivation and intuition of this proposal is not clear. It is better to cast more light on the need to agree on such categorization. It is also not clear how the boundaries of brackets have been selected. |
| Moderator | Companies’ evaluations report a range of positioning accuracy values @90%. One factor is, the model size affects the positioning accuracy. Larger models tend to achieve better positioning accuracy while incurring higher model complexity and computational complexity. Thus it is not a fair comparison to directly compare 90% accuracy without taking into account the complexity differences. If the models are grouped in n complexity classes, then it can be expected that well-designed models in the same complexity class should achieve comparable positioning accuracy. We can also demonstrate what the complexity-performance tradeoff is for the positioning use case. The boundary of the brackets can be further discussed. The brackets in the proposal intend to approximately evenly cover the range of complexities reported by companies. |

### 2nd round discussion

For Proposal 3.1.1-1, it seems that companies prefer to have it addressed under AI 9.2.1.

For Proposal 3.1.1-2, the issue is resolved with an agreement. The understanding is, if an AI/ML model is a complex-valued model with Nc complex-valued parameters, then the model complexity should be reported as 2xNc (real-valued) parameters.

For Proposal 3.1.1-3, the intention is to groups the models evaluated by companies according to complexity range. Then it’s meaningful to compare performance (90% accuracy) among models of similar complexity. Otherwise, it is not fair to compare performance of a small model (e.g., <1M parameters) to the performance of a large model (e.g., >8M parameters). Since the model complexity reported by companies vary widely, it seems necessary to have a mechanism to group companies’ model for fair comparison. The proposal is revised to the following to reflect the intention.

Proposal 3.1.2-1

For the positioning use case, examine the relationship between the positioning accuracy and the model size.

- Note: the baseline KPI is the horizonal accuracy at CDF=90%.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| NOK | We agree with the direction of the proposal. However, we need some clarifications. Currently companies have provided information related to Model Size and Position Accuracy (CDF=90%). What should be the new outcome of this evaluation compared to previous one that all companies have done ? |
| ZTE | The intention is not clear. Our understanding is that we will not draw conclusions based on model complexity based on the following conclusions. Conclusion (RAN1#110bis) This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP. Conclusion (RAN1#111) Companies describe how their computational complexity values are obtained. It is out of 3GPP scope to consider computational complexity values that have platform-dependency and/or use implementation (hardware and software) optimization solutions. |
| Mediatek | Different models with different design may have similar positioning accuracy performance but the size of these models may vary. Therefore, it may be hard to get an exact relationship between the positioning accuracy and the model size. It is up to each company further studying their model size and related positioning results. |
| Fujitsu | We believe the model size does affect the model performance to some extent, but the relationship between them are not linear or even not positively connected, as an simple example, very basic FNN can be used to perform the AI/ML direct positioning but if the layer numbers increase continuously, the corresponding positioning accuracy will not be improved accordingly, in some case may even get worse. What is more, the accuracy is highly dependant on the input data (e.g., the data coherence or user density) rather than the model size, so it is not clear the purpose on evaluating this point. |

## Intermediate performance metric of assisted AI/ML positioning

For assisted AI/ML positioning, further input are provided by companies as shown below.

- Qualcomm (R1-2301408) Proposal 10: For AI/ML-based soft information reporting approaches, the 90th percentile of the top-K error in ToA is reported as an intermediate KPI. FFS: the value of K to be reported.

- Nokia (R1-2300608) Observation-9: to evaluate the prediction of LOS/NLOS links could not remain only in the accuracy calculation. Other metrics as the false positive probability (FPP) and false negative probability (FNP), could indicate valuable information to the assisted legacy methodology in AI/ML assisted positioning. Observation-10: the F1-score KPI measurement could evaluate the sensitivity of LOS prediction for AI/ML-assisted positioning. Proposal-6: RAN1 to consider accuracy and F1-score as KPIs to measure the quality/sensitivity of LOS prediction for AI/ML assisted positioning. Observation-11: the impact of training a model in an imbalanced dataset (high NLOS rates) and fine-tuning it in a balanced dataset provides high accuracy; however, the f1-score (quality LOS indication) is sensitive to fine-tuning generalization. Proposal-7: RAN1 to consider F1-score as KPI metric for LOS/NLOS classification (AI/ML assisted positioning) for unbalanced dataset scenarios to evaluate LOS indication quality. Proposal-11: For multi-TRP scenarios (i.e., one model for N TRPs), RAN1 is to consider F1-score as KPI metric for AI/ML assisted positioning to measure the LOS prediction quality. Table 10 - LOS/NLOS performance evaluation in both scenarios. Scenario 01 with clutter density of 40%, and Scenario 2 with clutter density of 60%. Classification accuracy; F1score; False positive probability; False negative probability. Scenario 01 (hard selection); 87.9%; 0.8499; 0.059; 0.061. Scenario 02 (hard Selection); 99.4%; 0.659; 0.0017; 0.004. Observation-19: the F1-score, accuracy, false positive probability, and false negative probability are intermediate monitoring metrics for LOS/NLOS classification in AI/ML assisted positioning.

- Fujitsu (R1-2300748) Proposal 3 For the intermediate output without explicit connections to the UE locations, it is suggested to have dynamic model monitoring metrics management framework to study the applicability of different available metrics under certain conditions or configurations.

- LG (R1-2300534) Proposal #2: At least for LOS/NLOS classification of AI/ML assisted positioning, consider also to utilize a soft value of the ML output as a LOS classification accuracy.

### 1st round discussion

Nokia pointed out that it’s useful to include F1-score as an intermediate KPI when the model output include the LOS/NLOS classification of AI/ML assisted positioning. For hard-value LOS/NLOS indicator, the F1-score can be calculated directly using the formula. For soft-value LOS/NLOS indicator, the hard value can be obtained from the soft value (i.e., 0 or 1 depending on if the soft value is greater than 0.5 or not), then the F1-score can be calculated.

Proposal 3.2.1-1

For AI/ML assisted positioning, if LOS/NLOS indicator is a model output, then F1-score is reported as an intermediate KPI, where:

F1 score=2* Precision*RecallPrecision+Recall

|  | Company |
| --- | --- |
| Support | NOK |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Not sure at this stage. Would be great to hear more views from others. |
| Fujitsu | We are OK to incorporate this common KPI which can be used for evaluating all sorts of AI/ML models, but the precision/recall may be more suitable for the True/False classification such as if the patient has been diagnosed as ill or healthy. If one threshold such as 0.5 to be set for the LOS/NLOS indicator, it will be a bit inaccurate, we cannot see there is a big difference for the value with 0.49 and 0.51. |
| NOK | As commented in the contribution, using F1-score is a very good complement to accuracy rate, specially on imbalanced scenarios (Clutter density 60%). |
| Qualcomm | We think the proposal needs also to consider the case when the model output is a soft-timing/angle information. E.g., For AI/ML assisted positioning, if soft-information of timing/angle is a model output, then the top-K error in timing/angle is reported as an intermediate KPI |
| Mediatek | In Nokia R1-2300608 table 4, f1_score of 0 fine tune data is 0.0004 but in table 6 it is 0.66, it is unclear to us how to calculate the f1_score. |
| Moderator | @Nokia Please address companies’ concern on this proposal. |
| NOK | To Mediatek, first of all, thank you for the comment. In R1-2300608 Table 4 and Table 6 (first row) provided results of Scenario 2 (Clutter density 60%) with a large rate of NLos links (99%). As each table was generated with different epochs and the extreme-imbalanced dataset, we expect this difference. Unfortunately, we did not stress this result with many campaigns to evaluate the mean and Standard deviation of the performance, and we neither increasing the computational and model complexity (please check the reported complexity in both tables). However, following the line of Mediatek’s comment, we identified the importance of using F1-score based on the following metric analysis. In both tables. The Accuracy Rate is very high (better than 99%). However, the F1-score indicates the LOS indication is very bad in Table 4, which indicates that Accuracy Rate is not a fair metric for LOS indication compared to F1-score. About the F1-score, it is a very popular KPI for binary classification. To explain the calculation, we prepared the following illustrative figure: If we observe, F1-score evaluates the LOS sensitivity prediction, in both Recall and Precision rates, in both cases TP is always in the numerator. F1 score=2* Precision*RecallPrecision+Recall |
| ZTE | Not an important issue. Intermediate KPI based on accuracy rate is enough. Final KPI is more important. |
| Mediatek | The F1-score is an equal balance between two sides: recall rate and precision. But if a company wants to favor any side of it, F1-score would not be helpful. So, we suggest using F_β score, where F1-score is also included. Moreover, F1-score or F_β-score has a disadvantage that comparing different scenarios with different LOS/NLOS ratios may be problematic. So we suggest to modify the comment as: For AI/ML assisted positioning, if LOS/NLOS indicator is a model output, further study the intermediate KPI, e.g., then F1-score is reported as an intermediate KPI, where: F1 score=2* Precision*RecallPrecision+Recall |
| NOK | We are aligned with Mediatek proposal. |

# (Closed) Model Input and output

Regarding input and output of the ML model, representative proposals submitted in companies’ contributions are copied below.

## Model Input

- Samsung (R1-2301259) Proposal 1: RAN1 shall study the generalization ability for imperfect input/output data and how to model the imperfections. Proposal 2: the formation of the input data (e.g., the normalization of CIR) should be studied for AI/ML for positioning. Fig.6 CDF of positioning errors with normalized or non-normalized CIR Observation 6: the normalized CIR may degrade the inference performance.

- vivo (R1-2300448) Observation 2: Different inputs of AI/ML model will affect the positioning performance for AI/ML based positioning. Time domain channel CIR as the input of AI/ML model obtains the best positioning accuracy. Proposal 1: Capture in the TR that time domain CIR as the model input for direct AI/ML positioning obtains the best performance compared to other model inputs. Proposal 2: Support time domain CIR as the model input at least for direct AI/ML positioning.

- Nokia (R1-2300608) Observation-6: The UE/TRP can report only the timing and RSRP values, and the signaling of CIRs from the UE to the network is not supported. Observation-7: If CIR is agreed as a baseline model input, that would imply that only UE-based direct or AI/ML assisted positioning methods are considered. However, in such scenario, there might be challenges related to acquiring labeled training data from other UEs or from the network. Proposal-4: RAN1 to consider existing measurements as defined in TS 38.215, such as RSRP (DL PRS RSRP and UL SRS RSRP) or RSRPP (i.e., DL PRS RSRPP and UL SRS RSRPP) as a baseline model input for evaluation of direct and AI/ML assisted positioning. Observation-8: For CIR as model input, one key aspect that needs to be taken into account relates to the additional signaling overhead required for collecting CIR-based labeled datasets. Proposal-5: RAN1 to consider overhead for CIR reporting as part of data collection, monitoring, and model inference – for LMF/network-based positioning where the UE would need to report this information frequently to the network. Observation-15: to efficiently exploit both the channel tap gains and phases, an ML model using CIR as input is expected to be more complex than a model using PDP. In other words, to outperform a PDP-based model, the CIR-based model is expected to be larger and require more computational resources. Proposal-9: RAN1 to evaluate the gains of CIR over PDP in terms of generalization (model fine-tuning/update) and trade-off between performance and computational complexity.

- ZTE (R1-2300175) Observation 7: Due to the enriched channel observations from multi-port PRS, positioning performance is improved under a given number of data samples for model training, which can relief the efforts to collect training data in reality. Proposal 4: Study and identify the benefit of AI/ML based positioning when the model input includes channel observations from multi-port PRS.

- Google (R1-2300401) Proposal 1: For CIR/PDP based model input, study the impact from the following Rx schemes - Rx Scheme 1: The CIR/PDP is measured from the Rx port with the strongest RSRP - Rx Scheme 2: The CIR/PDP is averaged over all the Rx ports Proposal 2: For CIR/PDP based model input, study at least the following options for CIR/PDP quantization: - Option 1: The CIR/PDP is quantized based on several DFT bases - Option 2: The CIR/PDP is quantized based on several DCT bases Proposal 3: Study to use L1-SINR from each cell in addition to the CIR/PDP as the input to reflect the potential channel estimation accuracy for the CIR/PDP.

- CEWiT (R1-2301690) Proposal-1: For the evaluation on AI/ML-based Direct positioning results, support both CIR and PDP as the model inputs.

## Model output

- Qualcomm (R1-2301408) Table 11 Evaluation results for the {60%, 6m, 2m} clutter setting, with the AI/ML model deployed on UE-side, without model generalization, using a single-TRP construction with the same model for all TRPs Method; Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Training; test; Model complexity; Computational complexity; AI/ML. Hard decision; CER (1,2, 64); Single value of ToA; 0%; {60%, 6m, 2m}, Drop A; {60%, 6m, 2m}, Drop A; 18000 (1000 UEs * 18 TRPS); 18000 (1000 UEs * 18 TRPS); 22K params; 206K FLOPs per TRP 3.7M FLOPs for 18 TRPs; 25.0. Soft information; CER (1,2, 64); Distribution of ToA; 0%; {60%, 6m, 2m}, Drop A; {60%, 6m, 2m}, Drop A; 18000 (1000 UEs * 18 TRPS); 18000 (1000 UEs * 18 TRPS); 22K params; 206K FLOPs per TRP 3.7M FLOPs for 18 TRPs; 5.1. Table 12 Evaluation results for the {40%, 4m, 2m} clutter setting, with the AI/ML model deployed on UE-side, without model generalization, using a single-TRP construction with the same model for all TRPs Method; Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Training; Test; Model complexity; Computational complexity; AI/ML. Hard decision; CER (1,2, 64); Single value of ToA; 0%; {40%, 4m, 2m}, Drop A; {40%, 4m, 2m}, Drop A; 18000 (1000 UEs * 18 TRPS); 18000 (1000 UEs * 18 TRPS); 22K params; 206K FLOPs per TRP 3.7M FLOPs for 18 TRPs; 14.8. Soft information; CER (1, 2, 64); Distribution of ToA; 0%; {40%, 4m, 2m}, Drop A; {40%, 4m, 2m}, Drop A; 18000 (1000 UEs * 18 TRPS); 18000 (1000 UEs * 18 TRPS); 22K params; 206K FLOPs per TRP 3.7M FLOPs for 18 TRPs; 0.5.

## 1st round discussion

Regarding reporting overhead and specification impact, the model input and output need to be discussed for different cases:

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

For model inference:

- For Case 1: both model input and output do not need to be reported to another entity, thus are up to UE implementation. The only exception is UE location as model output of direct AI/ML positioning, but UE location can be sent via existing LPP signaling when requested, thus no concern of specification impact.

- For Case 2a and 3a (AI/ML assisted positioning):

  - Model input does not need to be specified. Model input for AI/ML assisted positioning is up to implementation. There is no need to have any restrictions on model input, including info type (e.g., CIR, PDP) and size of model input.

  - Model output need to be specified for signaling to LMF.

- For Case 2b and 3b (LMF-side model, direct AI/ML positioning):

  - Model input needs to be specified, since the model input needs to be sent to LMF.

  - Model output is UE location, and no concern of specification impact.

Regarding data collection for model training and model monitoring:

Many issues are open for model training and model monitoring, including where/how data collection is carried out, and what data to collect. Thus the signalling and specification impact are unclear at the moment. FL suggests that the discussion on model input and model output can focus on model inference for now.

The following agreement was made in RAN1#111.

Agreement

For reporting the model input dimension NTRP * Nport * Nt of CIR and PDP, Nt refers to Nt consecutive time domain samples.

- If N’t (N’t < Nt) samples with the strongest power are selected as model input, with remaining (Nt ‒ N’t) time domain samples set to zero, then companies report value N’t in addition to Nt. It is also assumed that timing info for the N’t samples need to be provided as model input.

Agreement

At least for model inference of AI/ML assisted positioning, evaluate and report the AI/ML model output, including (a) the type of information (e.g., ToA, RSTD, AoD, AoA, LOS/NLOS indicator) to use as model output, (b) soft information vs hard information, (c) whether the model output can reuse existing measurement report (e.g., NRPPa, LPP).

To continue the investigation, the following is proposed based on companies’ input.

Proposal 4.3-1

For direct AI/ML positioning with LMF-side model, study the model input at least for model inference, considering the tradeoff among model performance, model complexity and computational complexity, signaling overhead, and potential specification impact.

- The type of information to use as model input, e.g., time-domain CIR, PDP

- The dimension of model input in terms of NTRP, Nt, and Nt’.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Hw/HiSi | There is a typo in the text before the agreements “The following agreement was made in RAN1#1121.” We don’t think that signaling and spec impact needs to be considered in this agenda item. This should be done in 9.2.4.2 based on the evaluations that we are carrying out here. We think what could be studied is model input size and its possible relationship to model performance and complexity. If then the model input is signaled or kept within the same entity depends on the function allocation that is better suited in 9.2.4.2- For the model input size we have presented simulation results in R1-2300112 where it is shown that sub-meter level accuracy can be maintained even when going down to 4 TRPs and 64 samples for CIR and even down-to 32 samples for PDP. Would it be possible to observe/capture these results here, as they are based on the agreements that have achieved during RAN1#111? The proposal could be updated as follows: For direct AI/ML positioning with LMF-side model, study the model input size at least for model inference, considering the tradeoff among model performance, model complexity and computational complexity. , signaling overhead, and potential specification impact. - The type of information to use as model input, e.g., time-domain CIR, PDP - The dimension of model input in terms of NTRP, Nt, and Nt’. [Moderator] First bullet is not about ‘size’ though. |
| Fujitsu | Basically OK for this proposal, though some wording may need to be checked as HW pointed out above. |
| CATT | Generally OK. Also fine with HW’s updates. |
| OPPO | Fine with the intentions. However, could moderator elaborate what’s the additional information of this proposal compared to previous agreements in Agenda item 9.2.4.1 and 9.2.4.2 [Moderator] The intention is to investigate the best tradeoff for model input. Previous agreement 9.2.4.1 only asked companies to report what input was used in their evaluation. |
| NOK | Ok with the direction. |
| Qualcomm | We have a few concerns about this proposal. The proposal needs to be tailored to the intended cases. - First, the direct AI/ML positioning with LMF-sided models and time-CIR/PDP input can be easily realized with Case3b reporting. Case3b incurs much smaller radio resources when compared to Case2b. - Second, we also think the evaluation should first target understanding the benefits/overhead of reporting CIR/PDP to LMF versus reporting existing measurements that are obtained using AI/ML assisted positioning approach. In other words, we need first to understand the performance gains of the following: - Case3a (existing/enhanced measurements) vs Case3b (CIR/PDP both optimized and unoptimized) - Case2a (existing/enhanced measurements) vs Case2b (CIR/PDP both optimized and unoptimized) [Moderator] This proposal will help to understand the comparison between the Cases. |
| Mediatek | Fine with the proposal |
| Moderator | Update the proposal according to Huawei input. Proposal 4.3-1a For direct AI/ML positioning with LMF-side model, study the model input at least for model inference, considering the tradeoff among model performance, model complexity and computational complexity, signaling overhead, and potential specification impact. - The type of information to use as model input, e.g., time-domain CIR, PDP - The dimension of model input in terms of NTRP, Nt, and Nt’. |

Regarding the issue of multi-port PRS, several companies (e.g., vivo, ZTE) have evaluated the benefit of defining multi-port PRS. ZTE pointed out that multi-port PRS reduces the efforts to collect training data. On the other hand, up till now, only single-port PRS is specified in 3GPP. It needs to be discussed whether AI/ML justifies the introduction of multi-port PRS, while conventional positioning methods do not need it thus far.

Question 4.3-2

For AI/ML positioning using DL PRS, does the introduction of AI/ML justify the introduction of multi-port PRS? Please share your view and why.

- Yes

- No

| View | Company |
| --- | --- |
| Yes |  |
| No |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | We think at the moment, this question about justification cannot be answered. All we can do for now is an evaluation of the multi-port PRS. Before we can justify multi-port PRS, we need for example - agree on signaling of measurements and labels for Cases 1-3B - evaluate how many samples are needed in certain conditions to achieve the desired prediction accuracy. For that kind of study we already have agreements in place. We suggest that we move on with this and capture observations firstly. In these kind of observation, for the given cases, multi-port PRS performance might also be shown. We suggest to postpone the question about justification of multi-port PRS until the above aspects have progressed |
| Fujitsu | We think this multi-port PRS evaluation can be left for companies who are interested in it, if more companies show the interests on this, maybe it will be the time for an overall discussion here. |
| CATT | Share the same view with HW. |
| OPPO | We prefer to defer the conclusion to wait for evaluation from more companies since is only limited companies provided simulation results. |
| Qualcomm | The concept of multi-port PRS is not clear. Is it multi-port PRS at TX or RX side? Our understanding is that it is a network implementation to map multiple PRS resources to a given TRP point (e.g., mapping can happen in a way that mimics a multi-port PRS transmission at TX side). We appreciate it if more details and assumptions are provided on how the multi-port PRS has been simulated and evaluated. |
| Moderator | Based on companies’ input above, there is no clear convergence on the multi-port PRS evaluation. Proponents of the multi-port PRS evaluation are encouraged to address companies’ questions above to facilitate the discussion. |

## (Closed) 2nd round discussion

Based on the online discussion, proposal 4.3-1 is updated below.

Proposal 4.4-1

For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input at least for model inference, considering the tradeoff among model performance, model complexity and computational complexity, and model input size.

- The type of information to use as model input. The candidates include at least: time-domain CIR, PDP

- The dimension of model input in terms of NTRP, Nt, and Nt’.

- Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference.

|  | Company |
| --- | --- |
| Support | NOK, Mediatek, Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| NOK | We agree with the proposal. It is clear to obtain numerical values for model performance, model complexity, and computational complexity. However, it is not clear how we provide numerical values for the signaling overhead. Could you provide an example to get this value? |
| ZTE | Prefer to remove signaling overhead. The target is to evaluate whether the current measurements in specification is enough or not. The signaling overhead should be studied in another agenda. In addition, we think model training should also be included. Other, this proposal is more like to evaluate model generalization. |
| Xiaomi | Generally, we are OK. But we think AI/ML assisted positioning should be included in the study as well. Maybe there is no need to deliver the input from one entity to another entity. But it may impact the overhead of model delivery. Small dimension of model input usually correspond to AI model with small size and the overhead to delivery the AI model would become lower accordingly. |
| Fujitsu | From our point of view, the signaling overhead will be introduced by new measurement input or enhanced measurement input, as well as the assistance information for model inference, if so, this will be like a 9.2.4.2 issue? In 9.2.4.2, we also proposed to study something like truncated CIR and so on. |

# (Closed) Other

## Deployment Scenarios and evaluation methodology

- Qualcomm (R1-2301408) Categorization of generalization and robustness for studying AI/ML positioning enhancement: - Type 1: Heterogeneous inter-site (or heterogeneous inter-zone): Performance of AI/ML model on unseen deployment type (e.g., Umi vs. InF scenarios) - Type 2: Homogeneous inter-site (or homogeneous inter-zone): Performance of AI/ML model on unseen deployment of the same type (e.g., trained on drop 1 and tested on drop 2 of the same scenario) - Type 3: Time varying changes: Performance of AI/ML model on unseen time variations within the same site (or zone) (e.g., moving objects, small environment variations over time in a factory, floor, warehouse, etc.) - Type 4: Cross-configuration: Performance of AI/ML models across TX/RX configurations (e.g., training and testing can have different beam or transmit powers/SNR mismatch, synchronization/timing errors, etc.). Proposal 1: For studying the impact of time varying changes (e.g., mobility of clutter objects and blockers in the environment), the following modelling approaches can be considered: - Option1: Modeling approach listed in Appendix 1 - Option2: Blocker Model A or Blocker Model B in TR 38.901 (Section 7.6.4 [2]). Proposal 4: consider the following observations on LCM approaches (i.e., model switching, mixed training dataset, and model finetuning) for AI/ML positioning: - Model switching offers the best positioning accuracy followed by mixed training dataset and model finetuning - Model finetuning with small dataset size can only be feasible for enhancing positioning accuracy for small unseen changes

- Lenovo (R1-2301203) Proposal 1: The evaluation methodology should be designed under a common generalizability framework, including additional considerations such as different UE mobility options, Tx/Rx beam configurations and DL-PRS configurations. Proposal 2: The positioning AI/ML model evaluation methodology should support scenarios evaluating a model's robustness and adaptability, e.g., including how often an AI/ML evaluation model is updated based on a particular evaluation criterion. FFS any other relevant criteria.

## Model switching

- Qualcomm (R1-2301408) Observation 3: Site-specific AI/ML positioning models achieve excellent performance within their intended coverage area (i.e., the trained site). Observation 4: Model switching can help scaling the excellent performance of site-specific AI/ML positioning models across different sites. Proposal 2: Evaluate the model switching as a solution to enhance performance of site-specific AI/ML positioning models across different sites. Proposal 3: To evaluate AI/ML positioning enhancement with model switching, consider multiple sites (e.g., N sites) that have different drop values, clutter settings, and/or deployment scenario. Then conduct evaluation for the two following cases: - No model switching case: Train a single model with L datasets from L sites among the N sites (where L<N), and test on all N sites using the trained model. - Model switching case: Train M models (M>1) with datasets from the N sites, and test on all N sites while switching between the M trained models and picking the right model that fits the testing site. Proposal 8: Study the specification impacts for enabling model switching of direct AI/ML positioning models to enable generalization across different drops (i.e., inter-site generalization). Proposal 9: Prioritize model switching as a solution to enable generalization of direct AI/ML positioning across different drops (i.e., inter-site generalization). Table 10 Evaluation results for AI/ML model deployed on UE-side, with model switching for generalization (Type 2 - different drops), CNN Model input; Model output; Label; Model switching (number of models); Setting ({60%, 6m, 2m}); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Training; Test; Model complexity [parameters]; Computational complexity [FLOPs]; AI/ML. CIR (18,4,400); 2D; 0%; 2; two drops; two drops; 15k; 2k; 1.5M; 1.54G; 2.75. CIR (18,4,400); 2D; 0%; 1; one drop; two drops; 15k; 2k; 1.5M; 1.54G; 9.45.

- Nokia (R1-2300608) Observation-1: Model switching scales the upper bound performance and it was represented in all previous evaluations when the ML testing is performed on the same dataset used for ML training. Thus, RAN1 has no necessity to evaluate model switching scenarios.

- Ericsson (R1-2300141)

Observation 6 Even with small model sizes, models trained with mixed datasets can operate at state-of-the-art performance in multiple substantially different environments with no need of explicit environment identification and model switching. Observation 7 For the alternative approach of keeping multiple models for multiple operating environments, reliability of environment identification becomes the critical point of failure. Observation 8 To operate at state-of-the-art performance in multiple substantially different environments, mixed dataset training is superior to either training multiple models from scratch or storing multiple fine-tuned models with explicit environment identification and model switching.

## 1st round discussion

For model switching, QC propose to “Prioritize model switching as a solution to enable generalization of direct AI/ML positioning across different drops”. On the other hand, Nokia pointed out that evaluation of model switching is not necessary since evaluation results are already available when ML testing is performed on the same dataset used for ML training. Ericsson pointed out that model switching has the additional step of environment identification, and the reliability of environment identification and model switching becomes the critical point of failure.

Companies are invited to share your view whether model switching should be evaluated as a solution to enhance performance of site-specific AI/ML positioning models across different sites.

Question 5.3-1

Should RAN1 evaluate model switching as a solution to enable generalization across different deployment environments? Please provide your reasoning.

- Yes

- No

| View | Company |
| --- | --- |
| Yes |  |
| No | CATT, NOK |

| Company | Comments |
| --- | --- |
| HW/HiSi | No With model switching, the model is simply changed to a new model that has been trained for the new scenario conditions. This is not related to e.g. generalization performance. However, that does not mean that we automatically would oppose to a functionality of model switching. The performance of a model could be monitored and the decision might be to switch to another model. But this is a separate discussion and not part of this evaluation in our view. |
| Fujitsu | We think model switching is one follow-up action after the model monitoring, e.g., from model A to model B. It will be natural to say that we evaluate the performance of both model A and B, respectively, but it is weird to say we evaluate the performance of model switching because it is an instant action which cannot be evaluated. What is more, model switching cannot guarantee the generalization performance since the entities will make decisions of switching based on all kinds of elements such as the monitoring results, additional information among entities and maybe the model capabilities, but it cannot be assured that the decision made will be 100% correct or effective. |
| CATT | Model switching is a procedure issue which can be discussed under 9.2.4.2, which cannot be evaluated. |
| OPPO | We think environment identification is the key issue for model switching, proper monitoring metric is needed for model switching. |
| NOK | As commented in our observation, training and testing an AI/ML model on the same dataset (scenario) represents the upper bound performance in all cases. Thus, for evaluation purposes, we do not get advantage to evaluate model switching because it was already done. |
| Qualcomm | We think the TR needs to capture an observation that model switching when done properly and timely ensures the scalability of site-specific models and solutions. The TR should not exclude obvious options such as model switching for enabling the scalability of AI/ML positioning. We find the other approaches are considerable options, but the TR needs to acknowledge the approaches that have been evaluated. Back to the point on whether model switching is a generalization solution or not. The other two approaches, i.e., mixed dataset training and model fine-tuning, are much likely similar to model switching in terms of considering samples from the testing case/scenario. |
| Mediatek | For companies who are interested in it can provide more evaluation results. RAN1 can start by selecting different models in different environments to demonstrate the benefits of using multiple models, but how to evaluate model switching can be discussed further. |
| Ericsson | As mentioned in our contribution, the evaluation for model switching should be on the mechanism to reliably detect environment change, find the matching model to the new environment, and switch to matching model with minimum delay. It is not yet clear to us how to evaluate such mechanism. |
| Moderator | Based on input above, most companies do not think it is necessary to evaluate model switching by combining performance results of model A trained for environment A, and results of model B trained for environment B. Such results already exist, and this does not reveal how effective a model switching mechanism is. Proponent @QC can address companies’ concern to facilitate further discussion. |
| ZTE | No. Similar views as other companies. It’s a model monitoring issue. And, the procedures for model switching can be discussed in 9.2.4.2. |

## 3rd round discussion

Based on the input in 1st round discussion, it is proposed that the issue can be closed with a conclusion, see below. It is understood that model switching is still a candidate approach to achieve good performance across different scenarios.

Companies can submit evaluation results on the performance (e.g., accuracy, reliability, latency) of the steps in model switching. For example,

- The step to detect the environmental change. This can be evaluated as part of model monitoring.

- The step to identify the proper model to switch to when model drift is detected. Thus far there is no evaluation results on the potential error of switching to a improper model.

Conclusion 5.4-1

No dedicated evaluation is needed for the positioning accuracy performance of model switching. Other evaluation related to model switching steps are encouraged, for example, model monitoring to detect scenario change.

|  | Company |
| --- | --- |
| Support | Fujitsu, NOK, Hw/HiSi |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | We support the conclusion, but it seems that the example has no direct relationship to model switching, companies may want to detect the scenario change for model monitoring, but if model switching, model re-training/fine-tuning or fallback to legacy will be taken as the result of the model monitoring will be another story which may not directly determined by the evaluation result. |
| NOK | We agree with the direction oft he proposal. However, we suggest to delete the last sentence. Thus, the conclusion should be something like: „No dedicated evaluation is needed for the positioning accuracy performance of model switching.“ The deleted sentence is already covered by the agreement on model monitoring approved yesterday. |
| HW/HiSI | Support and also fine with NOK’s suggestion |
| Qualcomm | We are fine with a conclusion that captures what the FL and other companies indicated as model switching being an obvious approach to enhance performance. We suggest the following changes to reflect the understanding: Conclusion Model switching is an obvious candidate approach to achieve good performance across different scenarios. Therefore, no dedicated evaluation is needed for the positioning accuracy performance of model switching. Other evaluation related to model switching steps are encouraged, for example, model monitoring to detect scenario change. Note: the TR 38.843 acknowledges model switching as a candidate approach to achieve good performance across different scenarios/settings. |
| ZTE | Agree with Nokia. We already agree to study model monitoring. It’s not necessary to repeat it again. |

## 4th round discussion

Based on companies‘ feedback, the conclusion is updated to below.

Conclusion 5.5-1

No dedicated evaluation is needed for the positioning accuracy performance of model switching.

- Note: Model switching is considered an candidate approach to achieve good performance across different scenarios.

|  | Company |
| --- | --- |
| Support | NOK, Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| NOK | Agree, only some rewording is suggested in the Note: - Note: Model switching is another candidate approach to achieve good or upper-bound performance across different scenarios. |
| Fujitsu | Agree. |
| HW/HiSi | Ok. Have some concern on the updated wording from Nokia, since also after the model switch it might not be an upper bound. This depends on how well the new model fits to the environment. |

# Performed evaluation of direct AI/ML positioning

In this meeting, a large amount of evaluation work has been performed by companies for direct AI/ML positioning. These valuable results are very important to help RAN1 to make progress.

Selected results submitted by companies are copied below.

## Evaluation without generalization considerations (same setting for training and testing)

- ZTE (R1-2300175) Table.1 Direct AI/ML positioning based on PDP, 1.0 m grid width for data generation, deployed on UE/network side, Model backbone (CNN [5]) Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Training; test; Model complexity; Computation complexity; AI/ML. PDP {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 8}; - D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 7200; 800; 9.50 M; 158.47 M; 1.93. PDP {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 16}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 7200; 800; 9.50 M; 158.47 M; 1.40. PDP {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 32}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 7200; 800; 9.50 M; 158.47 M; 1.75. PDP {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 64}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 7200; 800; 9.50 M; 158.47 M; 0.83. PDP {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 7200; 800; 9.50 M; 158.47 M; 0.71. PDP {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 7200; 800; 9.50 M; 158.47 M; 0.69. Table.3 Direct AI/ML positioning based on PDP, 0.5 m grid width for data generation, deployed on UE/network side, Model backbone (CNN) Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Training; test; Model complexity; Computation complexity; AI/ML. PDP {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 64}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 28800; 1800; 9.50 M; 158.47 M; 0.508. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 64}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 28800; 1800; 9.50M; 158.66 M; 0.421. PDP {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 28800; 1800; 9.50 M; 158.47 M; 0.446. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 28800; 1800; 9.50M; 158.66 M; 0.278. Observation 5: The positioning performance of CIR based model input is better than PDP based model input, which shows that path phase information is beneficial for AI/ML model training. Proposal 3: Study and identify the benefit of path phase information in AI/ML based positioning. Figure. 2 CDFs of positioning errors for multi-port AI/ML positioning Observation 6: When the model input includes channel observation from multi-port PRS, better positioning performance can be observed when compared to single port PRS.

- Apple (R1-2301341) Table 4: Direct AI/ML Positioning: Evaluation results for AI/ML model deployed on UE/network-side, without model generalization, with a CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18 x 256 x 2]; UE coordinates [1x2]; 100% labeled; Drop 1; Drop 1; 47500; 2500; 2.43; 5.12; 1.01m.

- CAICT (R1-2300845) Observation 1: The horizontal positioning accuracy of direct AI/ML positioning could be 0.046m at CDF=90% when all samples in training dataset have ground truth label without model generalization. Observation 2: The horizontal positioning accuracy of direct AI/ML positioning could be 0.493m at CDF=90% when partial samples (1000 samples out of 150000 samples) in training dataset have ground truth label.

- xiaomi (R1-2300571) Table 1 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,277,442; 5.76GFlops; 0.4462. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2}; {0.4，2，2}; 70000; 10000; 21,277,442; 5.76GFlops; 0.7566.

- China Telecom (R1-2300719) Table 3. Evaluation results for AI/ML model deployed on network-side, without model generalization Model input; Model output; Label; Settings (clutter parameter); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. TOA; Predicted UE position; True UE position; [0.6,6m,2m]; [0.6,6m,2m]; 78400 samples; 1600 samples; 75.7k; 75.1k; 0.69. DL-TDOA; Predicted UE position; True UE position; [0.6,6m,2m]; [0.6,6m,2m]; 78400 samples; 1600 samples; 75.7k; 75.1k; 0.73. CIR; Predicted UE position; True UE position; [0.6,6m,2m]; [0.6,6m,2m]; 78400 samples; 1600 samples; 145.9M; 2.8M; 0.54. RSRP+TOA; Predicted UE position; True UE position; [0.6,6m,2m]; [0.6,6m,2m]; 78400 samples; 1600 samples; 184.2k; 182.8k; 0.43. RSRP+DL-TDOA; Predicted UE position; True UE position; [0.6,6m,2m]; [0.6,6m,2m]; 78400 samples; 1600 samples; 184.2k; 182.8k; 0.38.

- CATT (R1-2300675) Observation 1: For directly estimating UE’s positioning with perfect network synchronization, the horizontal accuracy is 0.58m@90%.

- MediaTek (R1-2301591) Observation 19: The evaluation results shown that the positioning performance of 2 transmit antenna ports (by different polarization) is better than the existing 1 antenna port in the spec. Proposal 7 : At least support PDP as model input for direct AI/ML positioning, and further study CIR to check whether the phase part in CIR is useful. Proposal 8 : Study and evaluate the performance of direct AI/ML positioning with multiple transmit/receive antenna port pairs (for example, 2 ports with different polarization).

## Evaluation of generalization aspects (different setting for training and testing)

### Different drops

- Qualcomm (R1-2301408) Table 2 Horizontal positioning error (meters) of RFFP with Type 2 generalizations Train; Test; 50%; 67%; 80%; 90%tile. Drop A; Drop A; 1.41; 1.79; 2.19; 2.77. Drop A; Drop B; 5.98; 7.81; 9.88; 12.33. Classical - Drop A; 14.65; >20; >20; >20. Classical - Drop B; 13.88; >20; >20; >20. Figure 6 CDF of horizontal positioning errors of direct AI/ML positioning (solid plots: Baseline performance; dashed plots: Type 2 generalizations).

- vivo (R1-2300448) Observation 6: Positioning performance of direct AI/ML positioning degrades when the model trained with dataset of one drop is tested with dataset of other drops.

- Apple (R1-2301341) Table 5: Direct AI/ML Positioning: Evaluation results for AI/ML model deployed on UE/network-side, model generalization, with a CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18 x 256 x 2]; UE coordinates [1x2]; 100% labeled; Drop 1; Drop 2; 47500; 2500; 2.43; 5.12; 4.167m. CIR [18 x 256 x 2]; UE coordinates [1x2]; 100% labeled; {60%,6,2}; {40%,2,2}; 47500; 2500; 2.43; 5.12; 4.1935m. CIR [18 x 256 x 2]; UE coordinates [1x2]; 100% labeled; Ideal n/w synch; Non-ideal network sync; 47500; 2500; 2.43; 5.12; 10.88m. CIR [18 x 256 x 2]; UE coordinates [1x2]; 100% labeled; InF-DH; InF-SH; 47500; 2500; 2.43; 5.12; 4.237m.

- ZTE (R1-2300175) Various simulation drops: Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; 1st Drop; N/A; 1st Drop; 28800; N/A; 1800; 984.96K; 44.28 M; 0.26. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; 1st Drop; N/A; 2nd Drop; 28800; N/A; 1800; 984.96K; 44.28 M; 20.42. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; 1st Drop; 2nd Drop; 2nd Drop; 28800; 5000; 1800; 984.96K; 44.28 M; 2.72. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; 1st Drop; 2nd Drop; 2nd Drop; 28800; 10000; 1800; 984.96K; 44.28 M; 2.33. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; Mixed datasets from 1st Drop and 2nd Drop; N/A; 1st Drop; 28800 + 28800; N/A; 1800; 984.96K; 44.28 M; 0.31.

- OPPO (R1-2300284) Observation 4: For the InF-DH scenario, if the training and testing data sets for AI model training and testing are generated from different drops, there will be large performance degradation for AI-based positioning.

- MediaTek (R1-2301591) Observation 20: Performance of direct AI/ML positioning degrades when the model trained with dataset of one drop is tested with dataset of other drops since there is no correlation between multipath realization of different drops. Observation 21: Training the AI model with mixed dataset can be an effective way to improve the performance of direct AI positioning when the model is deployed at different drops.

### Different clutter parameters

- vivo (R1-2300448) Figure 15 CDF of positioning accuracy of clutter parameters {0.6, 6, 2} and {0.4, 4, 2}

- xiaomi (R1-2300571) Table 3 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, without model generalization (different clutter parameter), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.4，2，2}; 70000; 10000; 21,277,442; 5.76GFlops; 7.0914. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2}; {0.6，6，2}; 70000; 10000; 21,277,442; 5.76GFlops; 1.5328.

- ZTE (R1-2300175) Various clutter settings: Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; N/A; {60%, 6m, 2m}; 28800; N/A; 1800; 984.96K; 44.28 M; 0.26. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; N/A; {40%, 4m, 2m}; 28800; N/A; 1800; 984.96K; 44.28 M; 19.31. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m} 1st Drop; {{40%, 4m, 2m}; {40%, 4m, 2m}; 28800; 5000; 1800; 984.96K; 44.28 M; 3.81. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {40%, 4m, 2m}; {40%, 4m, 2m}; 28800; 10000; 1800; 984.96K; 44.28 M; 3.18. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; Mixed datasets from {60%, 6m, 2m} and {40%, 4m, 2m}; N/A; {40%, 4m, 2m}; 28800 + 28800; N/A; 1800; 984.96K; 44.28 M; 0.48.

- OPPO (R1-2300284) Observation 5: For the InF-DH scenario, if the training and testing data sets for AI model training and testing are generated with different clutter settings, there will be large performance degradation for AI-based positioning.

- CATT (R1-2300675) Observation 3: For generalization performance with different assumptions on direct AI/ML positioning, the positioning accuracy is seriously degraded.

### Network synchronization error

- Qualcomm (R1-2301408) Figure 9 CDF of horizontal positioning error for RFFP scheme under different TRPs’ synchronization assumptions (blue plot: TRPs are synchronized; magenta plot: TRPs have random synchronization error within [-10, 10] nanoseconds).

- xiaomi (R1-2300571) Table 9 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, without model generalization (AI/ML model is trained by data set with ideal network synchronization and tested by data set with 100ns network synchronization error), ResNet Model input; Model output; Label; Clutter param & network synchronization error; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2} 0ns error; {0.6，6，2} 100ns error; 70000; 10000; 21,277,442; 5.76GFlops; 12.4486. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2} 0 ns error; {0.4，2，2} 100ns error; 70000; 10000; 21,277,442; 5.76GFlops; 14.5779.

- ZTE (R1-2300175) Various network synchronization errors: Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; Ideal network sync; N/A; Ideal network sync; 28800; N/A; 1800; 984.96K; 44.28 M; 0.26. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; Ideal network sync; N/A; Network sync error = 50 ns; 28800; N/A; 1800; 984.96K; 44.28 M; 10.32. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; Network sync error = 50 ns; N/A; Network sync error = 50 ns; 28800; N/A; 1800; 984.96K; 44.28 M; 0.34.

- OPPO (R1-2300284), Observation 7: For the InF-DH scenario, if the training data set of AI model is generated without NW synchronization error, the AI model performance for the case with NW synchronization error will suffer larger performance loss • In this case, the AI/ML assisted positioning scheme “Indirect: Normalized CIR for all TRPs” achieves similar performance as the traditional non-AI based scheme.

- CATT (R1-2300675) Observation 2: For directly estimating UE’s positioning with perfect network synchronization error, the horizontal accuracy is 0.84m@90%.

- vivo (R1-2300448) Table 21 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; Pos.; 0; 0ns; 0ns; 25k; 1k; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; 0ns; 2ns; 25k; 1k; 1.65M; 22.30M; 1.64. CIR; Pos.; 0; 0ns; 10ns; 25k; 1k; 1.65M; 22.30M; 4.56. CIR; Pos.; 0; 0ns; 50ns; 25k; 1k; 1.65M; 22.30M; 10.18. Table 22 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; Pos.; 0; 0ns; 10ns; 25k; 1k; 1.65M; 22.30M; 4.56. CIR; Pos.; 0; Mix 0ns+10ns; 10ns; 25k+2k; 1k; 1.65M; 22.30M; 1.16. CIR; Pos.; 0; 0ns; 50ns; 25k; 1k; 1.65M; 22.30M; 10.18. CIR; Pos.; 0; Mix 0ns+50ns; 50ns; 25k+2k; 1k; 1.65M; 22.30M; 1.52.

### UE/gNB RX and TX timing error

- Qualcomm (R1-2301408) Observation 9: RFFP can be made robust to network and UE timing errors (e.g., UE clock drift, network synchronization, etc.), by taking timing impairments into the training dataset. Figure 8 CDF of horizontal positioning error for RFFP scheme under different UE clock drift conditions (green plot: RFFP performance in ideal settings when no clock drift present; blue plot: training accounts for UE clock drift and testing includes UE clock drift within [-150,150] nanoseconds).

- ZTE (R1-2300175) Various UE RX timing errors: Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; Without UE Rx timing error; N/A; Without UE Rx timing error; 28800; N/A; 1800; 984.96K; 44.28 M; 0.26. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; Without UE Rx timing error; N/A; UE Rx timing error = 10 ns; 28800; N/A; 1800; 984.96K; 44.28 M; 0.89. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; UE Rx timing error = 10 ns; N/A; UE Rx timing error = 10 ns; 28800; N/A; 1800; 984.96K; 44.28 M; 0.27. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256}; 2-D UE position (1x2); 2-D UE position; UE Rx timing error = 20 ns; N/A; UE Rx timing error = 20 ns; 28800; N/A; 1800; 984.96K; 44.28 M; 0.34.

- MediaTek (R1-2301591) Observation 26: Direct AI/ML model trained with large timing error dataset can be generalized to dataset with small timing error. Observation 27: The positioning performance of direct AI/ML can be improved by mixing dataset with different timing errors at the cost of the training complexity.

#### Discussion

Thus far, only MediaTek (R1-2301591) evaluated and observed that direct AI/ML model trained with large timing error dataset can be generalized to dataset with small timing error. Further evaluation is needed to solidify the understanding.

### Different InF scenarios

- MediaTek (R1-2301591) Table 25. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML InF-SH; AI/ML InF-HH. CIR [18,2,256]; UE pos [x,y]; 0%; InF-DH({60%, 6m, 2m}),small hall; InF-SH/HH, large hall; InF-SH/HH, large hall; 0; 0; 3600; 464.24K; 0.266G; >100; >100. 1200; 9.806; 14.073. 3600; 5.391; 5.972. 7200; 3.815; 5.104. PDP [18,2,256]; UE pos [x,y]; 0%; InF-DH({60%, 6m, 2m}); InF-SH/HH, large hall; InF-SH/HH, large hall; 0; 0; 3600; 463.95K; 0.264G; >100; .>100. 1200; 8.964; 21.755. 3600; 5.186; 14.537. 7200; 4.162; 9.581. Observation 31: Performance of direct AI/ML positioning degrades when the model trained with dataset of InF-DH is tested with dataset of InF-SH/HH with different hall size, and fine-tuning can improve the performance.

- vivo (R1-2300448) Figure 16 CDF of positioning accuracy when training dataset and test dataset are not matched

### SNR mismatch

- Samsung (R1-2301259) Case; Training; Interference; Pos error (m) @90%. 1; -10db SNR level; -10db SNR level; 9.21. 2; -10db SNR level; 10db SNR level; 7.52. 3; 10db SNR level; -10db SNR level; 49.41. 4; 10db SNR level; 10db SNR level; 3.47. Fig.7 CDF of positioning errors with different SNR level Fig.8 CDF of positioning errors with different SNR level (2) Observation 2: the mis-alignment of the SNR may not always degrade the performance, e.g., low SNR model may have better pos accuracy by having high SNR inference input than same low SNR inference input. Proposal 3: the impact of SNR to the AI/ML model training/inference should be studied more comprehensively.

- Qualcomm (R1-2301408) Observation 10: Training direct AI/ML model on higher SNR achieves higher positioning accuracy Observation 11: If training and testing have mismatched SNR (e.g., due to change in transmit power), the positioning accuracy of direct AI/ML positioning can show slight to moderate degradation. Observation 12: If training and testing have mismatched SNR (e.g., due to change in transmit power), training direct AI/ML model on a higher SNR regime can achieve better generalization to unseen SNR settings than training on a smaller SNR regime. Table 6 Evaluation results for AI/ML model deployed on UE-side, with different TX power settings, CNN Model input; Model output; Label; Setting ({60%, 6, 2}); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Train; Test; Model complexity; Computational complexity; AI/ML. CIR (18,4,400); 2D; 0%; Drop 1 (TX power 12 dBm); Drop 1 (TX power 12 dBm); 15k; 2k; 1.5M; 1.54G FLOPs; 2.6. CIR (18,4,400); 2D; 0%; Drop 1 (TX power 12 dBm); Drop 1 (TX power 18 dBm); 15k; 2k; 1.5M; 1.54G FLOPs; 3.12. CIR (18,4,400); 2D; 0%; Drop 1 (TX power 12 dBm); Drop 1 (TX power 21 dBm); 15k; 2k; 1.5M; 1.54G FLOPs; 3.64. CIR (18,4,400); 2D; 0%; Drop 1 (TX power 12 dBm); Drop 1 (TX power 24 dBm); 15k; 2k; 1.5M; 1.54G FLOPs; 3.89. CIR (18,4,400); 2D; 0%; Drop 1 (TX power 18 dBm); Drop 1 (TX power 12 dBm); 15k; 2k; 1.5M; 1.54G FLOPs; 2.91. CIR (18,4,400); 2D; 0%; Drop 1 (TX power 18 dBm); Drop 1 (TX power 18 dBm); 15k; 2k; 1.5M; 1.54G FLOPs; 2.39. CIR (18,4,400); 2D; 0%; Drop 1 (TX power 18 dBm); Drop 1 (TX power 21 dBm); 15k; 2k; 1.5M; 1.54G FLOPs; 2.45. CIR (18,4,400); 2D; 0%; Drop 1 (TX power 18 dBm); Drop 1 (TX power 24 dBm); 15k; 2k; 1.5M; 1.54G FLOPs; 2.56.

#### Discussion

Likely due to how Samsung (R1-2301259) and Qualcomm (R1-2301408) simulated SINR mismatch, the observations from companies are not fully aligned.

### Time varying changes

- Qualcomm (R1-2301408) Observation 7: RFFP shows good robustness to subtle and moderate unseen reflections and multipath components that are different from training. Table 4 Evaluation results for AI/ML model deployed on UE-side, with Type 3-time varying changes, CNN Model input; Model output; Label; Setting ({60%, 6, 2}); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Train; Test; Model complexity; Computational complexity; AI/ML. CIR (18,4,400); 2D; 0%; Drop 1 (odd clusters); Drop 1 (odd clusters); 15k; 2k; 1.5M; 1.54G FLOPs; 2.74. CIR (18,4,400); 2D; 0%; Drop 1 (odd clusters); Drop 1 (odd except clusters 3&5); 15k; 2k; 1.5M; 1.54G FLOPs; 3.16. CIR (18,4,400); 2D; 0%; Drop 1 (odd clusters); Drop 1 (odd with clusters 2&4); 15k; 2k; 1.5M; 1.54G FLOPs; 2.88. CIR (18,4,400); 2D; 0%; Drop 1 (odd clusters); Drop 1 (odd with removal & addition of two random clusters); 15k; 2k; 1.5M; 1.54G FLOPs; 3.46. Observation 8: RFFP method can show improved robustness to slight environment changes such as time-varying blocking when trained on mixture of such changes. Table 5 Horizontal positioning error (meter) for RFFP robustness with Type 3 ‘time varying changes’ (ML model trained on mixture of channel realizations) Train; Test; 50%tile; 67%tile; 80%tile; 90%tile. Mixed clusters; Odd clusters; 1.33; 1.59; 2.19; 2.80. Mixed clusters; Odd except clusters 1&3; 2.41; 3.07; 3.81; 4.87. Mixed clusters; Odd except clusters 1&5; 2.21; 2.97; 3.72; 4.64. Mixed clusters; Odd except clusters 5&7; 1.45; 1.84; 2.42; 3.15. Mixed clusters; Odd with clusters 2&4; 1.39; 1.63; 2.25; 2.89. Mixed clusters; Odd with clusters 6&8; 1.34; 1.60; 2.23; 2.87. Mixed clusters; Remove up to two random odd clusters and add up to two random even ones; 1.50; 1.822; 2.46; 3.17.

- MediaTek (R1-2301591) Table 26. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; without time varying; with time varying; with time varying; 0; 0; 3600; 464.24K; 0.266G; >100. 1200; 11.206. 3600; 6.712. 7200; 5.076. PDP [18,2,256]; UE pos [x,y]; 0%; without time varying; with time varying; with time varying; 0; 0; 3600; 463.95K; 0.264G; >100. 1200; 9.950. 3600; 6.146. 7200; 4.796. Observation 32: Performance of direct AI/ML positioning degrades when there is time varying change between the training data and test data, and fine-tuning can improve the performance. Proposal 10 : Further study modeling of dynamically varying environment on 3GPP InF deployment to investigate the model robustness to time varying changes.

#### Discussion

Likely due to how MediaTek (R1-2301591) and Qualcomm (R1-2301408) simulated SINR mismatch, the observations from companies are not fully aligned.

### Channel estimation error

- MediaTek (R1-2301591) Observation 28: The direct AI Model trained by dataset with large channel estimation error can be generalized to dataset with small channel estimation error. Observation 29: The positioning performance can be improved by mixing dataset with different channel estimation errors at the cost of the training complexity. Proposal 9 : For AI/ML positioning, support better training dataset construction (e.g., mix dataset with different clutter parameters, different timing errors, and different channel estimation errors) for AI/ML model generalization.

- vivo (R1-2300448) Table 20 Evaluation results for AI/ML model deployed on UE or Network side, ViT Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; Pos.; 0; Without interference; 0 interfering TRP (Without interference); 25k; 1k; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; 1 interfering TRP; 25k; 1k; 1.65M; 22.30M; 8.35. CIR; Pos.; 0; 4 interfering TRPs; 25k; 1k; 1.65M; 22.30M; 10.22. CIR; Pos.; 0; 8 interfering TRPs; 25k; 1k; 1.65M; 22.30M; 13.14. Observation 16: The interference from TPRs can dramatically impair the positioning performance of AI/ML model.

#### Discussion

Thus far, only MediaTek (R1-2301591) evaluated and observed that direct AI/ML trained by dataset with large channel estimation error can be generalized to dataset with small channel estimation error. Further evaluation is needed to solidify the understanding.

## Model LCM evaluations

### Evaluation of model fine-tuning

- vivo (R1-2300448) Table 44 Evaluation of model fine-tuning for different cases Cases; Training; Fine-tuning; Testing; Positioning accuracy @90%. Direct AI/ML positioning; {0.6, 6, 2}; {0.4, 2, 2}; {0.4, 2, 2}; 4.40. {0.4, 2, 2}; {0.6, 6, 2}; {0.6, 6, 2}; 3.23. Drop1; Drop2; Drop2; 3.97. InF-DH; InF-HH; InF-HH; 8.78. Sync 0ns; 50ns; 50ns; 2.39. Sync 0ns; 10ns; 10ns; 1.28. Sync 0ns; 2ns; 2ns; 1.11. AI/ML assisted positioning; {0.6, 6, 2}; {0.4, 2, 2}; {0.4, 2, 2}; 0.63. Drop1; Drop2; Drop2; 5.50. InF-DH; InF-HH; InF-HH; 0.17. InF-DH; InF-SH; InF-SH; 0.17. Sync 0ns; 50ns; 50ns; 3.40. Sync 0ns; 10ns; 10ns; 1.78. Sync 0ns; 2ns; 2ns; 1.30. Observation 30: Model fine-tuning is suitable for the following tasks: • The source domain and the target domain are greatly similar, such as with different synchronization error. • The target domain is easy to fit, such as TOA estimation of LOS path. Table 45 Fine-tuning data sample efficiency for different cases Cases; Range of sample size; Data efficiency (@90% per 100 additional samples); Positioning accuracy with sample size N1 for sample range N1~N2(@90%). Train: {0.6, 6, 2} Fine-tuning: {0.4, 2, 2} Testing: {0.4, 2, 2}; 0-500; 0.69; 8.67 (0 samples). 500-1000; 0.16; 5.22 (500 samples). 1000-2000; 0.09; 4.40 (1000 samples). 2000-3000; 0.03; 3.50 (2000 samples). Train: {0.4, 2, 2} Fine-tuning: {0.6, 6, 2} Testing: {0.6, 6, 2}; 0-500; 0.17; 4.77. 500-1000; 0.13; 3.89. 1000-2000; 0.06; 3.23. 2000-3000; 0.01; 2.56. Train: Drop1 Fine-tuning: Drop2 Testing: Drop2; 0-500; 0.26; 6.00. 500-1000; 0.14; 4.69. 1000-2000; 0.06; 3.97. 2000-3000; 0.04; 3.37. Train: DH Fine-tuning: HH Testing: HH; 0-500; 14.98; >>10. 500-1000; 0.34; 10.50. 1000-2000; 0.29; 8.78. 2000-3000; 0.12; 5.84. Train: Sync. Error 0ns Fine-tuning: 50ns Testing: 50ns; 0-500; 1.39; 10.18. 500-1000; 0.16; 3.22. 1000-2000; 0.06; 2.39. 2000-3000; 0.02; 1.73. Train: Sync. Error 0ns Fine-tuning: 10ns Testing: 10ns; 0-500; 0.62; 4.56. 500-1000; 0.03; 1.44. 1000-2000; 0.02; 1.28. 2000-3000; 0.01; 1.06. Train: Sync. Error 0ns Fine-tuning: 2ns Testing: 2ns; 0-500; 0.10; 1.64. 500-1000; 0.001; 1.11. 1000-2000; 0.01; 1.11. 2000-3000; 0.005; 0.95. Figure 51 The curve of positioning error reduction with increasing number of sample size (data efficiency)

- Huawei (R1-2300112) Dimension 2: Pre-trained model with model updating - Table 14. Evaluation results for AI/ML model deployed on network-side, ResNet Model input(NTRP * Nport * Nt); Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; Test; Model complexity; Computation complexity; AI/ML. CIR 18*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 2; {60%, 6m, 2m}, Drop 2; 25000; 1000; 5000; 52.42K; 25.81M; 2.86. CIR 18*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {40%, 2m, 2m}, Drop 1; {40%, 2m, 2m}, Drop 1; 25000; 1000; 5000; 52.42K; 25.81M; 3.1. CIR 18*4*256; UE coordinates; UE coordinates; Without network synchronization error; With network synchronization error @50ns; With network synchronization error @50ns; 25000; 5000; 5000; 52.42K; 25.81M; 8.47. CIR 18*4*256; UE coordinates; UE coordinates; Without UE timing error; With UE timing error@20ns; With UE timing error@20ns; 25000; 5000; 5000; 52.42K; 25.81M; 1.13. Proposal 1: Model updating should be supported to improve the performance under the presence of UE timing errors and for the occurrence of the unlearned channel characteristics, including unseen drops and clutter settings. Observation 23: For direct AI/ML positioning, when the model input is CIR, over different dataset sizes for fine-tuning, the positioning accuracy increases when the size of fine-tuning dataset increases. At least 5000 samples are needed for fine-tuning to maintain sub-meter level accuracy. Observation 24: For direct AI/ML positioning, when the model input is PDP, over different dataset sizes for fine-tuning, the positioning accuracy increases when the size of fine-tuning dataset increases. At least 5000 samples are needed for fine-tuning to maintain meter level accuracy.

- Samsung (R1-2301259) Fig.5 – Test and fine-tune performance on DH662 and DH422 on model trained from SH Fig.6 – Test and fine-tune performance on DH662 and SH on model trained from DH422 Observation 4: When the training dataset is from DH662 small hall size, and the testing/inference dataset generated from different scenario (InF-SH), or different clutter parameter (DH422), or different deployment (large hall size), the performance of all three genralzation cases are degraded severely. Observation 5: When the training dataset is from DH662 small hall size, and 1k training data update and the testing/inference dataset generated from different scenario (InF-SH), or different clutter parameter (DH422), or different deployment (large hall size), the performance of all three generalizations cases are recovered significantly. The larger FH size, the better performance could be achieved. Proposal 5: RAN1 study the update/fine-tuning the model with limited number of data set or targeting generalization case.

- Qualcomm (R1-2301408) Observation 13: Model fine-tuning with small dataset can only offer slight to moderate enhancement to positioning performance of direct AI/ML positioning when tested with different drops (i.e., inter-site generalization). Table 8 Evaluation results for AI/ML model deployed on UE-side, with model finetuning generalization (Type 2 - different drops), CNN Model input; Model output; Label; Settings ({60%, 6m, 2m}); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). train; finetune; Test; Train; Finetune; Test; Model complexity [parameters]; Computational complexity FLOPs; AI/ML. CIR (18,4,400); 2D; 0%; drop A; --; drop B; 15k; 0; 2K; 1.5M; 1.54G; 12.33. CIR (18,4,400); 2D; 0%; drop A; drop B; drop B; 15k; 100; 2K; 1.5M; 1.54G; 10.47. CIR (18,4,400); 2D; 0%; drop A; drop B; drop B; 15k; 240; 2K; 1.5M; 1.54G; 6.92. CIR (18,4,400); 2D; 0%; drop A; drop B; drop B; 15k; 500; 2K; 1.5M; 1.54G; 6.07.

- CMCC (R1-2300994) Table IV. Evaluation results for AI/ML model deployed on UE side, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; Test; Model comp-lexity; Computa-tion comp-lexity; AI/ML. CIR size:18*1*256; UE coordinates; ideal UE coordinates; {60%, 6m, 2m}, Drop1; {60%, 6m, 2m}, Drop 2; {60%, 6m, 2m}, Drop 2; 25000; 500; 2500; 3.71M; 7.42M; 4.14. 1000; 3.35. 2000; 2.74. 3000; 1.95.

- xiaomi (R1-2300571) Table 7 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (with fine-tuning), ResNet Model input; Model output; Label; Clutter parameter; Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.4，2，2}; {0.4，2，2}; 70000; 5000; 10000; 21,277,442; 5.76GFlops; 1.4315. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2}; {0.6，6，2}; {0.6，6，2}; 70000; 5000; 10000; 21,277,442; 5.76GFlops; 0.7185. Table 13 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (with fine-tuning), ResNet Model input; Model output; Label; Clutter param & network synchronization error; Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2} , 0ns error; {0.6，6，2} , 100 ns error; {0.6，6，2} , 100 ns error; 70000; 5000; 10000; 21,277,442; 5.76GFlops; 4.7145. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2} 0ns error; {0.4，2，2} 100 ns error; {0.4，2，2} 100 ns error; 70000; 5000; 10000; 21,277,442; 5.76GFlops; 8.0254.

- NVIDIA (R1-2301182) Figure 6: Positioning accuracy of AI/ML based method under different drops. Figure 8: Positioning accuracy of AI/ML based method under different drops. Observation 8: Compared to finetuning for different drops, finetuning for different clutter parameter sets provides less positioning accuracy improvement with the same finetuning samples. This shows that finetuning is more effective when the first scenario (for which the AI/ML model is originally trained) and the second scenario (that finetuning targets) are more similar.

- CATT (R1-2300675) Observation 7: When AI/ML model is trained with the dataset of clutter parameter {60%, 6m, 2m} and fine-tuned with a small dataset of clutter parameter {40%, 2m, 2m}, the horizontal positioning accuracy is improved obviously compared with AI/ML model without fine-tuning. If the fine-tuning dataset size is larger, the improvement of horizontal positioning accuracy is greater.

- MediaTek (R1-2301591) Observation 22: Fine-tuning the model with samples from a drop can achieve positioning accuracy improvement when the pre-trained model is transferred to a new drop for direct AI/ML positioning. Observation 30: Fine-tuning a model with samples of new parameter setting (e.g., drop, clutter setting, channel estimation error, timing error, scenario) can achieve positioning accuracy improvement when the pre-trained model is transferred to a new parameter setting for direct AI/ML positioning (the more fine-tuning data the better performance), but performance degrades for pre-trained parameter setting.

### Evaluation of training with mixed dataset

- xiaomi (R1-2300571) Table 5 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (different clutter parameter), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6，6，2} {0.4，2，2}; {0.6，6，2}; 35000+35000; 10000; 21,277,442; 5.76GFlops; 0.5419. 18*256*1 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6，6，2} {0.4，2，2}; {0.4，2，2}; 35000+35000; 10000; 21,277,442; 5.76GFlops; 0.7684.

- Ericsson (R1-2300141) Table 8. 90%tile UE 2D positioning errors for small Model I trained with 40,000 samples from {60%, 6m, 2m} and/or {40%, 2m, 2m} and tested on four different environments. Positioning approach; Model complexity [# of parameters]; Training dataset; 90%tile UE 2D positioning errors [m] – small Model I on different test sets. {40%, 2m, 2m}; {60%, 2m, 2m}; {40%, 6m, 2m}; {60%, 6m, 2m}. Dist. Assisted; 0.86 M; {60%, 6m, 2m}; 8.998; 4.435; 0.543; 0.489. {40%, 2m, 2m}; 0.439; 0.642; 2.096; 2.295. {40%, 2m, 2m} {60%, 6m, 2m}; 0.514; 0.564; 0.626; 0.594. Cent. Assisted; 0.73 M; {60%, 6m, 2m}; 8.377; 3.587; 0.451; 0.409. {40%, 2m, 2m}; 0.741; 0.821; 1.562; 1.727. {40%, 2m, 2m} {60%, 6m, 2m}; 0.697; 0.604; 0.470; 0.459. Cent. Direct; 0.73 M; {60%, 6m, 2m}; 8.168; 3.355; 0.478; 0.443. {40%, 2m, 2m}; 0.749; 0.848; 1.574; 1.733. {40%, 2m, 2m} {60%, 6m, 2m}; 0.699; 0.604; 0.488; 0.479.

- CMCC (R1-2300994) Observation 3: If the mixed training dataset comprises the samples of the drop as the test dataset, the positioning accuracy can be improved obviously.

- OPPO (R1-2300284) Observation 6: For the InF-DH scenario, by training AI model based on the mixed data set with different clutter settings, the performance of AI model inference for the data set with one of these clutter settings can be improved.

- CATT (R1-2300675) Observation 4: When AI/ML model is mix-trained with the dataset of clutter parameter {60%, 6m, 2m} and a small dataset of clutter parameter {40%, 2m, 2m}, the horizontal positioning accuracy is improved from 2.64m to 1.77m compared with AI/ML model generalization performance without mix-training. Observation 5: When AI/ML model is mix-trained with the dataset of ideal synchronization and a small dataset of network synchronization error 50ns, the horizontal positioning accuracy is improved from 12.6m to 3.04m compared with AI/ML model generalization performance without mix-training. Observation 6: When AI/ML model is mix-trained with the dataset of InF-DH {60%, 6m, 2m} and a small dataset of InF-SH {20%, 10m, 2m}, the horizontal positioning accuracy is improved from 6.48m to 1.81m compared with AI/ML model generalization performance without mix-training.

- MediaTek (R1-2301591) Observation 23: Performance of direct AI/ML positioning degrades when the model trained with clutter setting ({60%,6m,2m}) is tested with dataset of another clutter setting ({40%,2m,2m}). Observation 24: Training the AI model with mixed dataset can be an effective way to improve the performance of direct AI positioning when the model is deployed at different clutter settings.

## Impact of user density/size of the training dataset

- Qualcomm (R1-2301408) Observation 5: RFFP can demonstrate different performance metrics depending on the UE area density considered for training. It is important to study how UE area density can affect performance as this helps companies decide on data collection strategies and signalling requirements depending on the case of interest. Figure 5 CDF of horizontal positioning error for RFFP under different UE area densities (blue plot: sparse UE area density; black: dense UE area density).

- Ericsson (R1-2300141) Table 4 90%tile 2D positioning accuracy for different model size classes in the {60%, 6m, 2m} InF-DH scenario. Model class; Positioning approach; Model complexity [# parameters]; Computational complexity [FLOPs]; 90%tile 2D positioning error [m] with different train set sizes in {60%, 6m, 2m} InF-DH. 80,000; 40,000; 20,000; 10,000. Small models; Dist. Assist.; 0.86 M; 36 M; 0.379; 0.489; 0.656; 0.977. Cent. Assist.; 0.73 M; 32 M; 0.335; 0.409; 0.568; 0.790. Cent. Direct; 0.73 M; 32 M; 0.366; 0.443; 0.596; 0.830. Medium-size models; Dist. Assist.; 3.37 M; 132 M; 0.219; 0.312; 0.491; 0.802. Cent. Assist.; 2.85 M; 110 M; 0.222; 0.294; 0.413; 0.641. Cent. Direct; 2.85 M; 110 M; 0.250; 0.338; 0.484; 0.763. Large models; Dist. Assist.; 11.2 M; 425 M; 0.167; 0.264; 0.443; 0.761. Cent. Assist.; 11.26 M; 410 M; 0.166; 0.263; 0.374; 0.597. Cent. Direct; 11.26 M; 410 M; 0.203; 0.299; 0.470; 0.743.

- Huawei (R1-2300112) Dimension 1: Different training dataset sizes. Table 12. Evaluation results for AI/ML model deployed on network-side, ResNet, CIR, different sizes of training dataset Model input(NTRP * Nport * Nt); Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. CIR 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 15.43K; 3.34M; 0.85. CIR 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 15000; 5000; 15.43K; 3.34M; 1.14. CIR 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 10000; 5000; 15.43K; 3.34M; 1.3. CIR 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 5000; 5000; 15.43K; 3.34M; 2.75. Table 13. Evaluation results for AI/ML model deployed on network-side, ResNet, PDP, different sizes of training dataset Model input(NTRP * Nport * Nt); Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. PDP 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 13.08K; 2.14M; 0.81. PDP 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 15000; 5000; 13.08K; 2.14M; 0.99. PDP 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 10000; 5000; 13.08K; 2.14M; 1.11. PDP 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 5000; 5000; 13.08K; 2.14M; 1.83. Observation 18: For direct AI/ML positioning, when the model input is PDP or CIR, over different dataset sizes for training, the performance of AI/ML-based fingerprint positioning decreases when the training dataset becomes smaller. In general, less complex models converge faster and need less labels to achieve a given accuracy, e.g., * 4 TRPs with length-128 PDP-based fingerprinting can provide sub-meter accuracy with training 15,000 samples, whereas the more complex CIR based model requires up to 25,000 samples for the same input dimensions.

- InterDigital (R1-2301101) Table 1 Summary of evaluation results of size of training dataset for direct AIML positioning Model Input; Model output; (Percentage of training data set without) Label; Settings (drops, clutter param, network synchronization error); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. RSRP; UE position; 0% (default); {60%, 6m, 2m} , drop A, T1= 0 ns; {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns; 16000; 4000; 332 k; 11.37 M FLOPs; 3.35. RSRP; UE position; 0% (default); {60%, 6m, 2m} , drop A, T1= 0 ns; {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns; 8000; 4000; 332 k; 11.37 M FLOPs; 4.25. RSRP; UE position; 0% (default); {60%, 6m, 2m} , drop A, T1= 0 ns; {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns; 4000; 4000; 332 k; 11.37 M FLOPs; 5.00. RSRP + RSTD; UE position; 0% (default); {60%, 6m, 2m} , drop A, T1= 0 ns; {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns; 16000; 4000; 334 k; 11.41 M FLOPs; 1.69. RSRP+RSTD; UE position; 0% (default); {60%, 6m, 2m} , drop A, T1= 0 ns; {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns; 8000; 4000; 332 k; 11.41 M FLOPs; 1.98. RSRP+RSTD; UE position; 0% (default); {60%, 6m, 2m} , drop A, T1= 0 ns; {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns; 4000; 4000; 332 k; 11.41 M FLOPs; 2.46.

- China Telecom (R1-2300719) Table 4. Positioning Accuracy with different training dataset size Dataset size; 50%; 67%; 80%; 90%. Traditional method; 11.89; 13.62; 14.78; 16.36. 80000; 0.27; 0.36; 0.43; 0.54. 40000; 0.36; 0.47; 0.57; 0.70. 20000; 0.46; 0.61; 0.75; 0.90. 10000; 0.52; 0.67; 0.81; 0.98. 5000; 0.69; 0.88; 1.10; 1.31.

- CMCC (R1-2300994) 2.3.1 Impact of training dataset size and model input type Table II. Evaluation results for AI/ML model deployed on UE side, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. CIR size：18*1*256; UE coordinates; ideal UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 2500; 3.71M; 7.42M; 0.38. {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 5000; 2500; 1.32. {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 2500; 2500; 2.36. CIR (size:18*1*256)+ RSRP (size:18*1); UE coordinates; ideal UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 2500; 3.71M; 7.42M; 0.33. {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 5000; 2500; 0.90. {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 2500; 2500; 1.27.

- xiaomi (R1-2300571) Figure 3 Relationship between size of training data set and positioning accuracy for direct AI-based positioning

- NVIDIA (R1-2301182) Figure 9: Positioning accuracy of AI/ML based method under different user densities.

- MediaTek (R1-2301591) Table 27. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size UE density; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; InF-DH{60%,6m,2m}; InF-DH{60%,6m,2m}; 5UEs/m2; 464.24K; 0.266G; 0.940. 2.5UEs/m2; 1.273. 1.25UEs/m2; 1.839. 0.625UEs/m2; 3.250. PDP [18,2,256]; UE pos [x,y]; 0%; 5UEs/m2; 463.95K; 0.264G; 0.821. 2.5UEs/m2; 1.357. 1.25UEs/m2; 2.039. 0.625UEs/m2; 2.758. Observation 33: Performance of direct AI positioning decreases as the UE density decreases. Table 28. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. PDP [18,2,256]; UE pos [x,y]; 0%; InF-DH{60%,6m,2m}; Left InF-DH; 3600 (left) +14400 (right)=18000; 3600; 463.95K; 0.264G; 1.518. Right InF-DH; 3600; 1.021. Proposal 11 : Further evaluate performance of AI/ML positioning for non-uniform UE distribution.

- Nokia (R1-2300608) Table 8 - Horizontal positioning accuracy at CDF=90% for the evaluation related to dataset density based on IPD. Case; Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; AI/ML. 1; ToA; 2d horizontal position (x, y); 2d horizontal position (x, y); Inf DH (40%); Inf DH (40%); 20% Randomly located; 80%; 2.73. 2; ToA; 2d horizontal position (x, y); 2d horizontal position (x, y); Inf DH (40%); Inf DH (40%); 20% Same size as case 1 but re-arranged (Mostly located on the left side); 80%; 4.24.

## Evaluation of input size reduction

- Huawei (R1-2300112) Dimension 1: Different numbers of CIRs used as model input. Table 7. Evaluation results for AI/ML model deployed on network-side, ResNet, CIR, different number of TRPs Model input(NTRP * Nport * Nt); Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. CIR 18*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 34 K; 25.81M; 0.62. CIR 4*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 15.43K; 6.52M; 0.97. CIR 2*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 7.18K; 3.26M; 2.4. CIR 1*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 7.11K; 1.63M; 5.16. Observation 12: For direct AI/ML positioning, when the AI/ML model input is CIR (one CIR per TRP) and different numbers of TRPs are evaluated, * The positioning accuracy decreases slightly when the number of TRPs is reduced from 18 to 4. But to maintain sub-meter level accuracy, the number of TRPs should be at least 4. * Model and computational complexity decrease significantly with a smaller number of TRPs. The model complexity is reduced by more than 50% and computational complexity about 75% when reducing the number of TRPs from 18 to 4. Dimension 2: Different numbers of CIR samples used as model input. Table 8. Evaluation results for AI/ML model deployed on network-side, ResNet, 18 TRPs, different CIR lengths Model input(NTRP * Nport * Nt); Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. CIR 18*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 52.42K; 25.81M; 0.62. CIR 18*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 34 K; 12.91M; 0.5. CIR 18*4*64; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 34 K; 6.52M; 0.64. CIR 18*4*32; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 34 K; 3.36M; 0.71. Table 9. Evaluation results for AI/ML model deployed on network-side, ResNet, 4 TRPs, different CIR lengths Model input(NTRP * Nport * Nt); Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. CIR 4*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 15.43K; 6.52M; 0.97. CIR 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 15.43K; 3.34M; 0.85. CIR 4*4*64; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 15.43K; 1.68M; 0.85. CIR 4*4*32; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 15.43K; 925K; 1.19. Observation 14: For direct AI/ML positioning, when the model input is CIR, compared to the initial assumptions of 18 TRPs and 256 samples per CIR as model input, the signaling payload could be reduced to 1/18 when going down to 4 TRPs and 64 samples per CIR, while still maintaining sub-meter level accuracy. Dimension 3: Different number of PDPs used as model input Table 10. Evaluation results for AI/ML model deployed on network-side, ResNet, PDP, different number of TRPs Model input(NTRP * Nport * Nt); Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. PDP 18*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 50.07K; 14.97M; 0.56. PDP 18*4*256; UE coordinates; UE coordinates; {40%, 2m, 2m}, Drop 1; {40%, 2m, 2m}, Drop 1; 25000; 5000; 50.07K; 14.97M; 0.65. PDP 4*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 13.08K; 4.11M; 0.84. PDP 2*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 4.82K; 2.05M; 2.8. PDP 1*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 4.76K; 1.03K; 5.79. Table 11. Evaluation results for AI/ML model deployed on network-side, ResNet, 4 TRPs, different PDP lengths Model input(NTRP * Nport * Nt); Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. PDP 4*4*256; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 13.08K; 4.11M; 0.84. PDP 4*4*128; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 13.08K; 2.14M; 0.81. PDP 4*4*64; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 13.08K; 1.08M; 0.95. PDP 4*4*32; UE coordinates; UE coordinates; {60%, 6m, 2m}, Drop 1; {60%, 6m, 2m}, Drop 1; 25000; 5000; 13.08K; 623K; 0.92. Observation 17: For direct AI/ML positioning, when the model input is PDP, compared to the initial assumptions of 18 TRPs and 256 samples per PDP as model input, the signaling payload could be reduced to 1/36 when going down to 4 TRPs and 32 samples per PDP, while still maintaining sub-meter level accuracy.

- xiaomi (R1-2300571) Table 15 Evaluation results for reduced input dimension for direct AI-based positioning and AI-based ToA predication, model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*24*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,277,442; 539.94MFlops; 0.8219. 18*24*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,285,650; 539.95MFlops; 0.8993.

- MediaTek (R1-2301591) Table 29. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Nt; N’t; Train; Test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; 256; 25; 32400; 3600; 464.24K; 0.266G; 0.940. 20; 0.915. 15; 1.017. 64; 25; 243.058K; 0.066G; 0.984. 20; 0.851. 15; 1.097. PDP [18,2,256]; UE pos [x,y]; 0%; 256; 25; 32400; 3600; 463.95K; 0.264G; 0.821. 20; 1.089. 15; 1.133. 64; 25; 242.770K; 0.066G; 0.942. 20; 0.965. 15; 1.057. Observation 34: By selecting appropriate Nt and N’t, the computational complexity, model complexity and signalling overhead can be reduced without significant performance loss. Proposal 12 : Study the trade-off between performance and complexity by choosing the appropriate Nt and N’t for AI/ML positioning evaluation.

- Nokia (R1-2300608) Figure 10 - Horizontal 2D error -90% CDF with CIR as an input parameter of the AI/ML model using the dataset for testing. The CIR is truncated in the first N samples. The dataset used in this evaluation is the Inf-DH with 40% of clutter density. Proposal-10: In case CIR or PDP will be prioritized as input parameters, RAN1 should evaluate the trade-off between complexity and performance among the available Nt input samples.

## Evaluation of noisy ground truth labels

- OPPO (R1-2300284) Observation 10: For the InF-DH scenario, if the label for the training data set of AI model is obtained by traditional NR DL-TDOA scheme, the AI model inference performance for the case will suffer larger performance loss • Compared to the traditional NR DL-TDOA scheme, the AI model doesn’t show obvious performance gain in this case.

- vivo (R1-2300448) Table 23 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; Pos.; 0; Std = 0; 0; 25k; 1k; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; Std = 0.5; 0; 25k; 1k; 1.65M; 22.30M; 1.51. CIR; Pos.; 0; Std = 1; 0; 25k; 1k; 1.65M; 22.30M; 2.17. CIR; Pos.; 0; Std = 2; 0; 25k; 1k; 1.65M; 22.30M; 3.55. Observation 19: The positioning accuracy gradually degrades with the increase of labeling error, but is still acceptable until standard deviation is 1 m (2.17m@90%). The maximum acceptable labeling errors (standard deviation) in the horizontal direction should be less than 1m to achieve 2m@90% positioning accuracy. Observation 20: AI/ML based positioning is robust to label noise to some extent. Proposal 7: According to the requirement of positioning accuracy, the maximum acceptable labeling error should be identified firstly before data collection

## Semi-supervised learning

- ZTE (R1-2300175) Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Training; test; Model complexity; Computation complexity; AI/ML. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; InF-DH {60%, 6m, 2m}; InF-DH {60%, 6m, 2m}; 2000 labeled; 1000; 9.50M; 158.66 M; 3.94. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; InF-DH {60%, 6m, 2m}; InF-DH {60%, 6m, 2m}; 2000 labeled 100 K unlabeled; 1000; 9.50M; 158.66 M; 1.72. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; InF-DH {60%, 6m, 2m}; InF-DH {60%, 6m, 2m}; 4000 labeled; 1000; 9.50M; 158.66 M; 1.67. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; InF-DH {60%, 6m, 2m}; InF-DH {60%, 6m, 2m}; 4000 labeled 100 K unlabeled; 1000; 9.50M; 158.66 M; 0.83. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; InF-DH {60%, 6m, 2m}; InF-DH {60%, 6m, 2m}; 8000 labeled; 1000; 9.50M; 158.66 M; 0.70. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; InF-DH {60%, 6m, 2m}; InF-DH {60%, 6m, 2m}; 8000 labeled 100 K unlabeled; 1000; 9.50M; 158.66 M; 0.51. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; InF-DH {60%, 6m, 2m}; InF-DH {60%, 6m, 2m}; 16000 labeled; 1000; 9.50M; 158.66 M; 0.38. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; InF-DH {60%, 6m, 2m}; InF-DH {60%, 6m, 2m}; 16000 labeled 100 K unlabeled; 1000; 9.50M; 158.66 M; 0.37.

- MediaTek (R1-2301591) Table 30. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 464.24K; 0.266G; 0.987. 0%; 7200 labelled; 7200; 2.018. 80%; 7200 labelled + 21600 unlabelled; 7200; 1.722. PDP [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 463.95K; 0.264G; 0.874. 0%; 7200 labelled; 7200; 2.213. 80%; 7200 labelled + 21600 unlabelled; 7200; 2.070.

- vivo (R1-2300448) Figure 53 Positioning accuracy comparison of semi-supervised learning and supervised learning with different numbers of labeled samples Observation 31: Semi-supervised learning can achieve a more accurate position estimation as compared to supervised learning with less amount of labeled data. Proposal 13: Capture in the TR the benefits of semi-supervised learning for AI/ML based positioning in terms of less data collection for training and more positioning accuracy.

- Ericsson (R1-2300141) Table 117. Evaluation results for semi-supervised learning vs supervised learning. The AI/ML model is deployed on network-side. The model is trained in InF-DH {60%,6m, 2m}, and tested with the same drop. No network synchronization errors or UE/gNB timing errors. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horiz. pos. accuracy, 90% (m), respectively. Training/ validation; test; Model complexity; AI/ML. 18 RSRP values for a target UE; Horizontal position of the target UE; 1% ~2.5% data with ideal label, 99% ~97% data without labels; {60%,6m, 2m}, same drop for training and testing; 100, 200, 400 data points with ideal label, 16000 data-points without label.; 4,000 of 18 RSRP values; 2 M parameters; 9.0 for 100 labelled datapoints 6.3 for 200 labelled datapoints 4.6 for 400 labelled datapoints (semi-supervised learning). 18 RSRP values for a target UE; Horizontal position of the target UE; Ideal; {60%,6m, 2m}, same drop for training and testing; 100, 200, 400 of 18 RSRP values; 4,000 of 18 RSRP values; 0.165 M parameters; 13.8 for 100 labelled datapoints 10.4 for 200 labelled datapoints 6.8 for 400 labelled datapoints (Supervised learning).

## 1st round discussion

As shown above, companies have submitted a large amount of evaluation results on the various generalization aspects, LCM issues, etc.

For generalization aspects, very similar observations are made by participating companies. Therefore, observations can be drawn based on the wealth of simulation results. Considering that direct AI/ML positioning is sensitive to environment change and implementation imperfections that were unseen during training, methods are needed to ensure proper operation of the AI/ML model, including fine-tuning and training with mixed dataset.

Observation 6.8-1

Observation: Evaluation of generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one setting, while tested with dataset of a different setting.

- The generalization aspects include:

  - Deployment environment:

    - Different drops

    - Different clutter parameters

    - Different InF scenarios

    - SINR mismatch

    - Time varying changes

  - Implementation imperfections:

    - Network synchronization error

    - UE/gNB RX and TX timing error

    - Channel estimation error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning.

  - Better training dataset construction: The training dataset is composed of data from multiple settings, which include data from the same setting as the test dataset.

  - Model fine-tuning: the model is re-trained with a dataset from the same setting as the test dataset.

|  | Company |
| --- | --- |
| Support | Fujitsu, CATT, OPPO, NOK |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Support |
| NOK | In general, we agree with the following complementary information highlighted in red. - Better training dataset construction: The training dataset is composed of data from multiple simulation settings (deployment environment), which include data from the same setting as the test dataset. - Model fine-tuning: the model or part of a previous trained model is re-trained l with a dataset from the same setting as the test dataset. [Moderator] OK. Above edits are incorporated in the updated proposal. |
| Qualcomm | For the second bullet point on solutions to improve performance, please add model switching as a third solution. |
| MediaTek | According to our evaluation of the implementation imperfections (e.g., timing error, channel estimation error), the positioning accuracy of direct AI/ML positioning depends more on the baseline setting of the training dataset(R1-2301591). For example, if the model is trained without timing error and tested with timing error, the performance will decrease. However, if the model is trained with a larger timing error and tested with a smaller timing error, the positioning accuracy of direct AI/ML positioning will not decrease. So “the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one setting, while tested with dataset of a different setting” is applicable to the deployment environment but not to the implementation imperfections. [Moderator] Above comment is very good. On the other hand, it seems that the fundamental problem is whether the model has been trained with data to be encountered in testing. For example, if “the model is trained with a larger timing error and tested with a smaller timing error”, the training data (e.g., truncated Gaussian with stdv=50ns) includes data with smaller timing error (e.g., stdv=20ns) already, thus the model won’t fail when tested with 20ns. The Observation is updated to reflect this. |

Observation 6.8-1a

Observation: Evaluation of generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of a first setting, while tested with dataset of a second setting, where the data in the second setting is unseen in the first setting.

- The generalization aspects include:

  - Deployment environment:

    - Different drops

    - Different clutter parameters

    - Different InF scenarios

    - SINR mismatch

    - Time varying changes

  - Implementation imperfections:

    - Network synchronization error

    - UE/gNB RX and TX timing error

    - Channel estimation error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning.

  - Better training dataset construction: The training dataset is composed of data from multiple simulation settings (deployment environment), which include data from the same setting as the test dataset.

  - Model fine-tuning: the model (or part of a previous trained model) is re-trained with a dataset from the same setting as the test dataset.

Proposal 6.8-2

For direct AI/ML positioning, training data collection need to ensure that data from various settings are collected for the training dataset, which include data for the target deployment environments and data for the expected implementation imperfections.

|  | Company |
| --- | --- |
| Support | Fujitsu, Mediatek |
| Not support |  |

| Company | Comments |
| --- | --- |
| Hw/HiSi | Seems fine |
| OPPO | Generally fine. But the motivation of the proposal is not clear. Observation 6.8-1 is enough to describe the construction of training dataset including the methods of mixed dataset or fine-tuning. |
| Qualcomm | Generally ok |

Proposal 6.8-3

For direct AI/ML positioning, support model fine-tuning to adapt a deployed AI/ML model to new or changing deployment scenario.

|  | Company |
| --- | --- |
| Support | Fujitsu, Mediatek |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Support |
| OPPO | There is still on-going discussion in AI 9.2.1 on the feasibility of model updating at UE sided. Thus, it is safer to emphasize NW-side model. There are to other aspect should be clarified: - “Support” is quite vague. We guess the intentions is to study its spec impact - The performance of fine-tuning depends on some prerequisites, e.g. the new training date should have the matched distribution. If the data from a small group of UEs within a small area, the performance can not be ensured. Suggest following wording changes marked in yellow: For direct AI/ML positioning, support to study the potential spec impact of model fine-tuning to adapt a deployed network side AI/ML model to new or changing deployment scenario if the distribution of the new training data is matched to the new or changing deployment scenario. |
| Qualcomm | So far, we have not seen model fine-tuning (with reasonably small dataset size) as a proper solution to adapt the direct AI/ML positioning to new deployment scenario. We prefer to change the wording on the proposal. The model finetuning with reasonable small dataset can work to adapt for mild to moderate changes but not significant or new ones. [Moderator] This is a good point. Updated the proposal to include re-training if the environment change is significant. |

Regarding labelling error, OPPO evaluation results indicate that if the label (i.e., UE position) for the training data set is obtained by traditional NR DL-TDOA scheme, then AI/ML loses the performance advantage compared to the traditional method. Evaluations by vivo indicate that the labelling error need to be smaller than the targeted positioning accuracy, for example, “The maximum acceptable labeling errors (standard deviation) in the horizontal direction should be less than 1m to achieve 2m@90% positioning accuracy.” As suggested by vivo, given the requirement of positioning accuracy, it is useful to identity the maximum acceptable labeling error before data collection.

Proposal 6.8-4

For direct AI/ML positioning, study the impact of labelling error to positioning accuracy, to understand the maximum acceptable labeling error in training data collection.

|  | Company |
| --- | --- |
| Support | Fujitsu, Mediatek |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Can be postponed in our view. For PRUs we think the impact of the labelling error is very small, since the position and labels can be obtained with very high accuracy. In that case this study is not needed. For UEs it might be different. But we still have not agreed in 9.2.4.2 which entities can be used to generate labels. It is better to decide this firstly. If only PRUs will be supported, then it might not be needed to study the impact of the labelling error, but instead to put more effort on other aspects (for example the number of training samples). Studying the impact of a labelling error seems also to require to agree on how to model the labeling error which can result in long discussion that might not be needed in the first place if we only would use PRUs. |
| Fujitsu | We support to evaluate on this point, but first of all how to simulate the label error should be discussed first, e.g., it is useless to use the results of conventional method as labels, and in the real network, what will be the label error distribution is not clear yet, we may obtain labels from UE, PRU, GNSS, LIDAR, WIFI and so on with different error distributions, even the PRU (may be updated from normal UE) may give labels with errors, so how to realize it via simulation coding need further study. [Moderator] This is a good point. A reasonable assumption is to use truncated Gaussian model, similar to network synchronization error and UE/gNB Rx/Tx timing error. That is, the ground truth (UE location) error has: zero mean=0, standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. |
| OPPO | Fine with the proposal. |
| Qualcomm | It is not clear what would be the intention or conclusion to drawn based on this proposal moving forward. We are ok with studying the impact of the quality of label but reaching a conclusion on the maximum tolerable labeling error can depend on many factors and can be different for different real scenarios. We are not sure whether deciding on a maximum acceptable labeling error using TR 38.901 datasets can yield a meaningful conclusion to be scaled to field-based datasets. [Moderator] Proponent @vivo can explain better. Moderator’s understanding is, this can help to decide what methods are appropriate for providing ground truth for training, when an accuracy requirement is given. |

Observation 6.8-5

Observation: Preliminary evaluation results indicate that semi-supervised learning can improve positioning accuracy when compared to supervised learning with the same amount of labeled data.

|  | Company |
| --- | --- |
| Support | Mediatek |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | We think it is a bit early for this observation, and if this observation is proposed, also other observations for which companies have submitted results should be proposed. For example on the achievable positioning accuracy dependent on the input type (CIR or PDP) and input dimensions of the AI model as in R1-230012. Instead of going into semi-supervised learning already now and capturing it separately, we would prefer if it would be compared with other methods when the number of labels provided from PRUs is assumed to be limited, e.g. data augmentation. |
| Qualcomm | We think the improvement that semi-supervised learning offers on the top of supervised learning starts to diminish as more labeled data is included. It is better to capture this limitation in the observation. |

## 2nd round discussion

Observation 6.9-1

Observation: Evaluation of generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of a first setting, while tested with dataset of a second setting, where the data in the second setting is unseen in the first setting.

- The generalization aspects include:

  - Deployment environment:

    - Different drops

    - Different clutter parameters

    - Different InF scenarios

    - SINR mismatch

    - Time varying changes

  - Implementation imperfections:

    - Network synchronization error

    - UE/gNB RX and TX timing error

    - Channel estimation error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple simulation settings (deployment environment), which include data from the same setting as the test dataset.

  - Model fine-tuning/re-training: the model (or part of a previous trained model) is re-trained with a dataset from the same setting as the test dataset.

|  | Company |
| --- | --- |
| Support | NOK, Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| NOK | We propose some rewording: 1st sub-bullet in 2nd Main-bullet: Better training dataset construction: The training dataset is composed of data from multiple simulation settings (deployment environment), which include data from the train dataset (first setting) and test dataset (second setting). |
| ZTE | We have similar observation as MediaTeK that model can have a good generalization capability on different network synchronization errors and different UE/gNB RX and TX timing errors. In addition, we prefer to use ‘ various scenarios’ instead of ‘deployment environment’ to align the same understanding in 9.2.1. In addition, we don’t have generalization evaluation over various configurations that are different from 9.2.2 and 9.2.3. |
| Mediatek | We think a note for “unseen” is needed. Note: when the AI/ML model is trained with dataset of a first setting (e.g., a larger timing error truncated Gaussian with stdv=50ns), while tested with dataset of a second setting (e.g., a smaller timing error stdv=20ns), where the data in the second setting is seen (not unseen) in the first setting. |
| Fujitsu | Just for clarification, from our understanding, the phrase “different settings” means either different deployment environments or different implement imperfections or combined, but in the last second bullet, it seems that the “settings” are only for deployment environment? We think the dataset construction should cover both of the two aspects for the alignment of the following several proposals. |
| Qualcomm | To Moderator: The listed solutions in the second bullet consider data from the testing setting. So, if we want to be precise, these are not solutions to enhance generalization to unseen settings but can be considered as approaches that help extend the scalability of AI/ML positioining to multiple settings. We raised the point of including model switching as a third approach (which is a reasonable and obvious approach), would the Moderator please comment on why model switching does not fit as a solution along with the listed ones? [Moderator] Model switching is understood as a candidate approach to handle different deployment scenarios, but most companies think no evaluation is needed for positioning performance. Please see discussion in section 5.3, which is dedicated to this topic. The Observations are intended to summarize the take-away messages based on all companies‘ extensive evaluation results for generalization aspects. This is why model switching is not part of the observations. |

Proposal 6.9-2

For direct AI/ML positioning, when the target deployment environments and implementation imperfections are known during training data collection, design of training data collection need to ensure that data from various settings are collected for the training dataset, which include data for the target deployment environments and data for the expected implementation imperfections.

|  | Company |
| --- | --- |
| Support | Mediatek, Fujitsu |
| Not support | vivo |

| Company | Comments |
| --- | --- |
| NOK | The text looks like an Observation instead that a Proposal unless we do not see any evaluation aspect on it. Furthermore, we request a clarification: What is the difference with the Observation 6-9.1 (Second bullet of First main bullet), maybe we can add a Note in the previous in Observation 9-9.1 based on this clarification. |
| Fujitsu | For this proposal and the next one, we do not see clear boundaries between the dataset construction and re-training/fine-tuning for offline trainings. It does not matter if these factors are known or unknown to the model, we can simply generate offline simulation data to incorporate all expected scenarios as many as possible to train the model so there is no need for model re-training/fine-tuning. So we assume that FL refer these two proposals for online training data collection only? |
| vivo | We’d like to get clarification on the intention of this proposal. Is this an observation based on evalution results? The wording of this proposal talking about „design of training data collection“ which we think would be more appropriate for agenda 9.2.4.2 if on potention specification impact discussion or even for general framework 9.2.1 as a general design principle. |

Proposal 6.9-3

For direct AI/ML positioning, when the target deployment environments and/or implementation imperfections are unknown during training data collection, evaluation results demonstrate the need to support model fine-tuning/re-training to adapt an AI/ML model to a new or changing deployment scenario.

|  | Company |
| --- | --- |
| Support | Mediatek, Fujitsu |
| Not support | vivo |

| Company | Comments |
| --- | --- |
| ZTE | At least for implementation imperfections, dataset mixture is not always necessary. |
| Fujitsu | Refer to the above comment. |
| vivo | Isn’t this observation already captured in the 2nd bullet of Observation 6.9-1? |

Proposal 6.8-4

For direct AI/ML positioning, study the impact of labelling error to positioning accuracy, to understand the maximum acceptable labeling error in training data collection.

- The ground truth (UE location) can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range.

  - Value L is up to sources. Example values: L=0.25 (m), 0.5 (m).

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | We should discuss which entity is assumed to acquire the label. If the entity is PRU, we think the labeling error will be very small. If the entity is UE, we’re open to further study. |
| Mediatek | For UE Horizontal location (x,y), the labelling error can be modeled in x and y direction separately, for example Lx=0.5 (m) and Ly=0.25 (m) |
| Fujitsu | We have noticed that FL commented for our previous concern. Just for further clarification, is there some companies’ simulation results showing that the proposed truncated Gaussian distribution can reflect the real label error distribution obtained from different sources in the network? Because we think the label sources are multiple with different accuracy levels and randomness, so a more realistic assumption for the label error modelling will be better, anyway we also support FL’s suggestion marked in red as a starting point. |
| Qualcomm | We think it needs more time to conclude on how to model the labeling error. We still also want to hear more inputs regarding the implication of determining the maximum acceptable labeling error. As we discussed before, there can be multiple factors that decide the maximum acceptable labeling error such as the quality associated with request location service, resource configuration used for labeling, etc. A similar discussion also applies to AI/ML assisted apporaches |
| vivo | 1. We think this study also applies to AI/ML assisted positioning. 2. Labelling error may not be only for training data collection. It may impact on the monitoring performance if monitoring based on ground truth label. |

## 3rd round discussion

Proposal 6.10-1

For direct AI/ML positioning, study the impact of labelling error to positioning accuracy, to understand the maximum acceptable labeling error in training data collection.

- The ground truth (UE location) error in x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range and [-2*L, 2*L] range.

  - Value L is up to sources.

|  | Company |
| --- | --- |
| Support | Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | 1. We think this study also applies to AI/ML assisted positioning. 2. Labelling error may not be only impact training data collection. It may impact on the monitoring performance if monitoring based on ground truth label. |
| Fujitsu | We support this proposal as well as the second point VIVO commented. |
| HW/HiSi | We agree with the points raised by QC from the previous round and also think it is too early for this proposal and some more time should be given how to model the label error. We do not support to apply it on assisted poistioning since the labels would be different in these cases (not coordinates). And we do not agree on the second point raised by vivo since for monitoring we need to make some basic progress on the metrics firstly. |
| OPPO | We suggest to study the impact of positioning accuracy when labels are genarated based on existing NR positioning methods. Suggest following changes: For direct AI/ML positioning, study the impact of labelling error to positioning accuracy, to understand the maximum acceptable labeling error in training data collection. - The ground truth (UE location) error in x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range and [-2*L, 2*L] range. - Value L is up to sources. - Study the impact of positioning accuracy when labels are obtained by NR positioning methods. |
| ZTE | Fine with the proposal. For OPPO’s suggestion, we think it’s not necessary. The proposal already gives the concrete distribution for modeling the error, which can be used to evaluate the relationship between the labeling error and performance. If we use NR positioning method, the labeling error is not controllable. |

For the Observation of generalization aspects, representative evaluation results are provided for the Observation. For the rest of the generalization aspects, it seems that more evaluations are needed to align observations between companies, since companies used different simulation assumptions and the observations are not fully aligned yet:

  - SINR mismatch

  - Time varying changes

  - UE/gNB RX and TX timing error

  - Channel estimation error

Observation 6.10-2

Observation: Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one scenario, while tested with dataset of a different scenario.

- The generalization aspects include:

  - Deployment environment:

    - Different drops

    - Different clutter parameters

    - Different InF scenarios

    - Network synchronization error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple scenarios, which include data from the same scenario as the test dataset.

  - Model fine-tuning/re-training: the model (or part of a previous trained model) is re-trained with a dataset from the same setting as the test dataset.

- Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for different drops. InF-DH, {60%, 6m, 2m}

|  | Train and test with dataset of drop 1 | Train with dataset of drop 1, and test with dataset of drop 2 |
| --- | --- | --- |
| Huawei (R1-2300112) | 0.62 | >10 |
| Ericsson (R1-2300141) | 0.443 | 16.755 |
| vivo (R1-2300448) | 0.99 | 6.00 |
| ZTE (R1-2300175) | 0.26 | 20.42 |
| Qualcomm (R1-2301408) | 2.77 | 12.33 |
| MediaTek (R1-2301591) | 1.32 | 9.15 |

- Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for different clutter parameters {60%, 6m, 2m} vs {40%, 2m, 2m}

|  | Train and test with dataset of {60%, 6m, 2m} | Train with dataset of {60%, 6m, 2m}, and test with dataset of {40%, 2m, 2m} |
| --- | --- | --- |
| Ericsson (R1-2300141) | 0.443 | 8.168 |
| vivo (R1-2300448) | 0.99 | 8.67 |
| ZTE (R1-2300175) | 0.26 | 19.31 |
| MediaTek (R1-2301591) | 1.32 | 9.10 |
| xiaomi (R1-2300571) | 0.4462 | 7.0914 |

- Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for network synchronization error

|  | Train and test with dataset without network synchronization error | Train with dataset network synchronization error, and test with dataset with network synchronization error |
| --- | --- | --- |
| Huawei (R1-2300112) | 0.62 | >10 (50ns) |
| Ericsson (R1-2300141) | 0.460 | 0.681 (10ns) 2.654 (25ns) 14.127 (50ns) |
| vivo (R1-2300448) | 0.99 | 1.64 (2ns) 4.56 (10ns) 10.18 (50ns) |
| ZTE (R1-2300175) | 0.26 | 10.32 (50ns) |
| xiaomi (R1-2300571) | 0.4462 | 12.4486 (100ns) |

- Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for different InF scenarios

|  | Train and test with dataset of InF-DH | Train with dataset of one InF-DH scenario, and test with dataset of InF-HH or InF-SH |
| --- | --- | --- |
| vivo (R1-2300448) | 0.99 | >10 |
| MediaTek (R1-2301591) | 1.32 | >100 |

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | We support capturing observations in principle. However, we’re wondering why impact on other aspects such as implementation error are not listed here. Furthermore, why there’re no exemplary evaluation results for mixed-training and/or model fine-tuning? |
| NVIDIA | We would like our results to be captured in the summary as well: - Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for different drops. InF-DH, {60%, 6m, 2m} Train and test with dataset of drop 1; Train with dataset of drop 1, and test with dataset of drop 2. NVIDIA (R1-2301182); 2.3; 14.5. - Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for different clutter parameters {60%, 6m, 2m} vs {40%, 2m, 2m} Train and test with dataset of {60%, 6m, 2m}; Train with dataset of {60%, 6m, 2m}, and test with dataset of {40%, 2m, 2m}. NVIDIA (R1-2301182); 2.3; 17.9. |
| NOK | We agree in general with the proposal. However, we strongly do not agree on indicating a list of Exemplary evaluation results. |
| InterDigital | Thank you very much for compiling the results. We have the following results that are relevant to the network sync error analysis in the contribution submitted in the Nov. 2022 meeting. The results are shown in Table I, row #4 and #6. In addition, the description in the top row in the FL’s table (for NW sync error) may not be accurate. From our reading of contributions, some compaines have presented scenarios where the AIML model is trained without NW sync error and the trained AIML model is trianed in the presence of NW sync error (e.g., Table 7, row 1 & 2 in R1-2300175). Our evaluation below corresponds to the case where the model is trained without NW sync error and the model is tested in the presence of NW sync error. Train and test with dataset without network synchronization error; Train with data set without network synchronization error and test with dataset without network synchronization error. InterDigital (R1-2211715); 2.22; 9.25. |
| HW/HiSi | Agree with vivo. We almost support capturing the main text without the examples. Just one question here for our understanidng: Why is Network Synch error categorized as a deployment environment. Shouldn’t it better belong to implementation imperfection? For the example results, also results for improved performance achieved by enriched training/model-fine-tuining shoudl bext captured. |
| OPPO | Thank you for collecting the results. If evaluation results are captured in the TR, we would like our evaluation results to be captured: - Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for different drops. InF-DH, {60%, 6m, 2m} Train and test with dataset of drop 1; Train with dataset of drop 1, and test with dataset of drop 2. OPPO (R1-2300284); 0.33; 10.11. - Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for different clutter parameters {60%, 6m, 2m} vs {40%, 2m, 2m} Train and test with dataset of {60%, 6m, 2m}; Train with dataset of {60%, 6m, 2m}, and test with dataset of {40%, 2m, 2m}. OPPO (R1-2300284); 0.33; 15.75. - Exemplary evaluation results of horizontal pos. accuracy at CDF=90% (m) for network synchronization error Train and test with dataset without network synchronization error; Train with dataset network synchronization error, and test with dataset with network synchronization error. OPPO (R1-2300284); 0.33; 6.54 (50ns). In our results for network sync error case, the model is trained without NW sync error and tested with network sync error (50ns). Besides, as mentioned by Interdigital, whether the table header of third column in FL’s table and the evaluation results by sources are matched need double check. |
| Qualcomm | - We also share the same view as NOK, we do not see a strong need on indicating a list of evaluation results at this stage. This can be added at the end in the TR to support different observations. - Another minor wording point: just to avoid confusion with In-F scenarios, it is better to replace the (scenario) by (setting). - To Moderator: Back to Moderator‘s response to our comment in the 2nd round (Observation 6.9-1): The current observation lists the candidate solutions to enhance performance. I think the Moderator and other companies indicated that there is no need to evaluate model switching because it is an obvious solution and the good performance is expected. Therefore, there should be no concern in adding it as a candidate solution along with the two solutions listed in this observation. |
| ZTE | Generally fine with the proposal. - Regarding the following text, we think it’s not needed to be listed separately in the bracket. It’s just one way of model fine-tuning. If the concern is re-training, we suggest the following. the model (or part of a previous trained model) is re-trained/fine-tuned with a dataset from the same setting as the test dataset. - Prefer to use ‘deployment scenario ‘ to replace ‘deployment environment’ - Agree with vivo, mixed datasets are missing in the observation. |

Observation 6.10-3

For direct AI/ML positioning, evaluation results demonstrate that training data collection need to ensure that data from various scenarios are collected for the training dataset, which include data for the target scenario and data for the expected implementation imperfections.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Observation 6.10-2 does not capturing any observation w.r.t implementation imperfactions. We suggest adding reference (evalution results from different sources as in observation 6.10-2) to support such statement of this observation 6.10-3. |
| HW/HiSi | To avoid possible misunderstanding: What is meant in this context with “data for the expected implementation imperfections”? Does it mean that the model cannot be implemented pre-trained for all conditions and realistic data needs to be collected from the target scenario to improve the performance? |
| Qualcomm | The intention and implication of this proposal are not clear. It is well-understood that the direct AI/ML positioning approach is scenario-/setting-specific. It is obvious that data needs to be collected from the target scenario along with information on expected imperfections. The (various scenario) wording in the proposal and its implication are not clear. |
| ZTE | Issues regarding mixed datasets should be included in Observation 6.10-2 |

Observation 6.10-4

For direct AI/ML positioning, evaluation results demonstrate the need to support model fine-tuning/re-training to adapt an AI/ML model to a new or changing deployment scenario.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Similiar comment as to observation 6.10-3. We suggest adding reference (evalution results from different sources as in observation 6.10-2) to support such statement of this observation 6.10-4. |
| HW/HiSi | Support |
| Qualcomm | Once again, we think the wording here needs to be more accurate. From the submitted evaluations, model fine-tuning with reasonable dataset size cannot help adapting the performance to new deployment scenarios. Even when fine-tuning is applied to new deployment scenario with a reasonable dataset size, there is still a gap in performance when compared to existing deployment scenario. |
| ZTE | Observation 6.10-2 already covers model fine-tuning/re-training. |

For the impact of user density (or: training dataset size for a given evaluation area) to positioning accuracy, evaluation results for both direct AI/ML positioning and AI/ML assisted positioning show that the positioning accuracy improves as the user density increases. See the representative evaluation results in section 6.4 for direct AI/ML positioning, and section 7.4 for AI/ML assisted positioning.

Observation 6.10-5

For both direct AI/ML positioning and AI/ML assisted positioning, evaluation results show that the positioning accuracy improves as the user density in the training dataset increases.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | We don’t feel this observation is complete and accurate. This sentence could be understood that performance improvement can always be obtained. However, there’re results actually showed a diminishing improvement on positioning accuracy as the increased training dataset size. |
| Fujitsu | As pointed in our contribution, the user desity or number of training dataset samples are not exactly positively connected to the model performance which can be affected by multiple factors under heavy NLOS, so technically if no extra condition apply to this oberservation above, then it may not be absolutely accurate. |
| HW/HiSi | Even if we think that this observation migt be correct in a broader sense, considering the comments above and our evaulation results in R1-2300112 it might be better to observe that the accuracy improves when more training samples are used. Suggested update: For both direct AI/ML positioning and AI/ML assisted positioning, evaluation results show that the positioning accuracy improves as the size of user density in the training dataset increases. |
| ZTE | This observation has no additional information. It’s obvious. We need to further discuss how to capture this aspect. For example, what number of additional data needed to be collected in order to increase positioning performance from meter-level to sub-meter level. |

## 4th round discussion

Proposal 6.11-1

For direct AI/ML positioning, study the impact of labelling error to positioning accuracy, to understand the maximum acceptable labeling error in training data collection.

- The ground truth (UE location) error in x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range.

  - Value L is up to sources.

- FFS: whether/how to study the impact of labelling error to labeled-based model monitoring methods.

- FFS: how to study the impact of labelling error for AI/ML assisted positioning.

|  | Company |
| --- | --- |
| Support | NOK, Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| NOK | To better compare results for the next meeting on this evaluation, we suggest defining the range of Max. value for L. |
| Fujitsu | Support. |
| HW/HiSi | We acknowlegde that there is some interest from some companies to evaluate this aspect, even we think that it is a bit too early. At the current stage, we think it should not be a part of the baseline evaluation methodology but could be reported optionally by companies. We think that this is intention anyway with this proposal. To clarify this we suggest to update the main bullet as follows: For direct AI/ML positioning, optionally study the impact of labelling error to positioning accuracy, to understand the maximum acceptable labeling error in training data collection. |

In the Observation 6.11-2 below: the bullet “Companies have provided evaluation results …” can be removed considering Observation 6.11-3 and 6.11-4.

Note that the exemplary evaluation results are just provided to support the Observations, since some company request it. This does not mean that these tables are to be copied into the TR.

For the data point by InterDigital on network synchronization error, it is not clear what level of synchronization error 9.25 corresponds to. @InterDigital please provide this info.

Observation 6.11-2

Observation: Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario.

- The generalization aspects include:

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

  - Network synchronization error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

  - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

- Exemplary evaluation results of horizontal positioning accuracy at CDF=90% (m) for different drops. InF-DH, {60%, 6m, 2m}

| Sources | Train and test with dataset of drop 1 | Train with dataset of drop 1, and test with dataset of drop 2 |
| --- | --- | --- |
| Huawei (R1-2300112) | 0.62 | >10 |
| Ericsson (R1-2300141) | 0.443 | 16.755 |
| ZTE (R1-2300175) | 0.26 | 20.42 |
| vivo (R1-2300448) | 0.99 | 6.00 |
| NVIDIA (R1-2301182) | 2.3 | 14.5 |
| Qualcomm (R1-2301408) | 2.77 | 12.33 |
| MediaTek (R1-2301591) | 1.32 | 9.15 |
| OPPO (R1-2300284) | 0.33 | 10.11 |

- Exemplary evaluation results of horizontal positioning accuracy at CDF=90% (m) for different clutter parameters {60%, 6m, 2m} vs {40%, 2m, 2m}

| Sources | Train and test with dataset of {60%, 6m, 2m} | Train with dataset of {60%, 6m, 2m}, and test with dataset of {40%, 2m, 2m} |
| --- | --- | --- |
| Ericsson (R1-2300141) | 0.443 | 8.168 |
| ZTE (R1-2300175) | 0.26 | 19.31 |
| vivo (R1-2300448) | 0.99 | 8.67 |
| xiaomi (R1-2300571) | 0.4462 | 7.0914 |
| NVIDIA (R1-2301182) | 2.3 | 17.9 |
| MediaTek (R1-2301591) | 1.32 | 9.10 |
| OPPO (R1-2300284) | 0.33 | 15.75 |

- Exemplary evaluation results of horizontal positioning accuracy at CDF=90% (m) for different network synchronization error

| Sources | Train and test with dataset without network synchronization error (i.e., 0 ns) | Train with dataset network synchronization error, and test with dataset with network synchronization error |
| --- | --- | --- |
| Huawei (R1-2300112) | 0.62 | >10 (50ns) |
| Ericsson (R1-2300141) | 0.460 | 0.681 (10ns) 2.654 (25ns) 14.127 (50ns) |
| vivo (R1-2300448) | 0.99 | 1.64 (2ns) 4.56 (10ns) 10.18 (50ns) |
| ZTE (R1-2300175) | 0.26 | 10.32 (50ns) |
| xiaomi (R1-2300571) | 0.4462 | 12.4486 (100ns) |
| InterDigital (R1-2211715) | 2.22 | 9.25 (50ns) |
| OPPO (R1-2300284) | 0.33 | 6.54 (50ns) |

- Exemplary evaluation results of horizontal positioning accuracy at CDF=90% (m) for different InF scenarios

| Sources | Train and test with dataset of InF-DH | Train with dataset of one InF-DH scenario, and test with dataset of InF-HH or InF-SH |
| --- | --- | --- |
| vivo (R1-2300448) | 0.99 | >10 |
| MediaTek (R1-2301591) | 1.32 | >100 |

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| NOK | We support the Observation. However, we do not agree on adding the exemplary evaluation results at this stage. We share QC view, this list can be added at the end of the TR, not now. |
| InterDigital | We added clarification to the simulation assumption above (with tracked changes). We copy & paste our comments in the previous round here. In addition, the description in the top row in the FL’s table (for NW sync error) may not be accurate. From our reading of contributions, some compaines have presented scenarios where the AIML model is trained without NW sync error and the trained AIML model is trianed in the presence of NW sync error (e.g., Table 7, row 1 & 2 in R1-2300175). Our evaluation below corresponds to the case where the model is trained without NW sync error and the model is tested in the presence of NW sync error. [Moderator] The intention was to gather data for the case when training was done without sync error. For R1-2300175, the row without sync error was copied. Thus the description should be fine. |
| HW/HiSi | We support the observation. The simulations results for both generalization performance and using model updating/enriching the training data set could be added at a later stage. |

Observation 6.11-3

For direct AI/ML positioning, evaluation results demonstrate that training data collection need to ensure that data from various scenarios are collected for the training dataset, which include data for the target scenario.

- Exemplary evaluation results of mixed dataset for different drops

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy @ CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Train | Test | Model complexity | Computation complexity | AI/ML |
| ZTE (R1-2300175) | CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256} | 2-D UE position (1x2) | 2-D UE position | Mixed datasets from 1st Drop and 2nd Drop | 1st Drop | 28800 + 28800 | 1800 | 984.96K | 44.28 M | 0.31 |
| Huawei (R1-2300112) | CIR 18*4*256 | UE coordinates | UE coordinates | {60%, 6m, 2m}, 5 Drops mixed | {60%, 6m, 2m}, Drop 1 (inside the trained Drops) | 25000 (5000/drop) | 5000 | 34 K | 25.81M | 1.28 |

- Exemplary evaluation results of mixed dataset for different clutter parameters

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| vivo (R1-2300448) | CIR | Pos. | 0 | Mix of {0.6, 6, 2} and {0.4, 2, 2} | {0.6, 6, 2} | 25k & 25k | 1k | 1.65M | 22.30M | 0.87 |
| xiaomi (R1-2300571) | 18*256*1 CIR | 2*1 UE coordinates | 2*1 UE coordinates | Mix of {0.6，6，2}, {0.4，2，2} | {0.6，6，2} | 35000+35000 | 10000 | 21,277,442 | 5.76GFlops | 0.5419 |
| Ericsson (R1-2300141) | Time domain CIR, 18x2x256 complex array | 2D UE position estimate | Ideal | {60%, 6m, 2m}, {40%, 2m, 2m} | {40%,2m,2m} | 40,000 each train dataset | 4000 | 0.73 M parameters | 32 M FLOPs | 0.699 |
| MediaTek (R1-2301591) | CIR [18,2,256] | UE pos [x,y] | 0% | {60%, 6m, 2m} +{40%, 2m, 2m} | {40%, 2m, 2m} | 16200+16200 | 3600 | 464.24K | 0.266G | 2.30 |

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Fine. For consistency with the earlier observation, numbers could be included later. |

Observation 6.11-4

For direct AI/ML positioning, evaluation results demonstrate the need to support model fine-tuning/re-training to adapt an AI/ML model to a new or changing deployment scenario.

- Exemplary evaluation results of fine-tuning/re-training for different drops

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Fine-tune | Test | Train | Fine-tune | Test | Model complexity | Computation complexity | AI/ML |
| OPPO (R1-2300284) | Normalized CIR + RSRP | UE coordination | UE coordination | 1 drop, 80,000 UEs per drop {60%, 6, 2} | 2nd drop {60%, 6, 2} | Remaining samples of the 2nd drop | 80,000 | 1,000 | 7,900 | 2.66M | 5.32 MFLOPs | 1.233 |
|  |  |  |  |  |  |  |  | 5000 | 7,500 |  |  | 0.688 |

- Exemplary evaluation results of fine-tuning/re-training for different clutter parameters

| Sources | Model input | Model output | Label | Clutter parameter |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| xiaomi (R1-2300571) | 18*256*1 CIR | 2*1 UE coordinates | 2*1 UE coordinates | {0.6，6，2} | {0.4，2，2} | {0.4，2，2} | 70000 | 5000 | 10000 | 21,277,442 | 5.76GFlops | 1.4315 |

- Exemplary evaluation results of fine-tuning/re-training for network synchronization error

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| vivo (R1-2300448) | CIR | Pos. | 0% | Sync. 0ns | / | 50ns | 25k | / | 1k | 1.65M | 22.30M | 10.18 |
|  |  |  |  |  | 50ns |  |  | 0.5k |  |  |  | 3.22 |
|  |  |  |  |  |  |  |  | 1k |  |  |  | 2.39 |
|  |  |  |  |  |  |  |  | 2k |  |  |  | 1.73 |
|  |  |  |  |  |  |  |  | 3k |  |  |  | 1.47 |

- Exemplary evaluation results of fine-tuning/re-training for different InF scenarios

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| vivo (R1-2300448) | CIR | Pos. | 0% | DH | / | HH | 25k | / | 1k | 1.65M | 22.30M | >10 |
|  |  |  |  |  | HH |  |  | 0.5k |  |  |  | 10.50 |
|  |  |  |  |  |  |  |  | 1k |  |  |  | 8.78 |
|  |  |  |  |  |  |  |  | 2k |  |  |  | 5.84 |
|  |  |  |  |  |  |  |  | 3k |  |  |  | 4.66 |
| MediaTek (R1-2301591) | CIR [18,2,256] | UE pos [x,y] | 0% | InF-DH({60%, 6m, 2m}),small hall | InF-SH/HH, large hall | InF-SH/HH, large hall | 0 | / | 3600 | 464.24K | 0.266G | >100 |
|  |  |  |  |  |  |  |  | 1200 |  |  |  | 9.806(InF-SH) 14.073(InF-HH) |
|  |  |  |  |  |  |  |  | 3600 |  |  |  | 5.391(InF-SH) 5.972 (InF-HH) |
|  |  |  |  |  |  |  |  | 7200 |  |  |  | 3.815 (InF-SH) 5.104 (InF-HH) |

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | We support the main text of the observation. For the evaluations results, if the exemplary results are intended to be kept we would like our results to be listed here as well. In R1-2300112, we have provided results for every of the listed examples i) different drops, ii) different clutter parameters, iii) network synch error, iv) UE timing error, we have evaluated both for model fine-tuning. We have additionally examined the impact on the size of the data set used for fine tuning. Some examples from our contribution: [Moderator] Include in the updated tables. In fact, as explained, these examples are just for supporting the observations. It’s not intended to be exhaustive or go directly to TR. |

For the impact of user density (or: training dataset size for a given evaluation area) to positioning accuracy, evaluation results for both direct AI/ML positioning and AI/ML assisted positioning show that the positioning accuracy improves as the user density increases. See the representative evaluation results in section 6.4 for direct AI/ML positioning, and section 7.4 for AI/ML assisted positioning. Two plots are copied below to illustrate the results.

| - xiaomi (R1-2300571) | - Ericsson (R1-2300141) |
| --- | --- |

Observation 6.15-5

For both direct AI/ML positioning and AI/ML assisted positioning, evaluation results show that with diminishing returns, the positioning accuracy improves as the sample density of the training dataset increases.

- FFS: the range of desirable sample density in training data collection.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSI | Is it really the sample density that we see in the evaluation results or is it the data size size? [Moderator] The intention is the same. Sample density (or user density) is used so that there is no worry of varying size of floor area. |

## 5th round discussion

The following observations were not treated or endorsed at Thursday online session. They are provided below for further discussion. Note that the exemplary evaluation results are not copied below. If interested, please see section 6.11 for the exemplary evaluation results.

Observation 6.12-1

For direct AI/ML positioning, evaluation results demonstrate that training data collection need to ensure that data from various deployment scenarios are collected for the training dataset, which include data for the target deployment scenario.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| NOK | We support this Observation. We suggest the following rewording: For direct AI/ML positioning, evaluation results demonstrate that for generalization purposes, the training data collection need to ensure that data from various deployment scenarios are collected for the training dataset, including at least, data from the target deployment scenario. |
| Hw/HiSi | We support the intention of observation, but the wording is not correct. Even if only training data from the target scenario is included, the performance is good. We support the update from Nokia. |

Observation 6.12-2

For direct AI/ML positioning, evaluation results demonstrate the need to support model fine-tuning/re-training to adapt an AI/ML model to a new or changing deployment scenario, if the existing training dataset and/or trained model cannot adequately cover the new or changing deployment scenario.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Qualcomm | As we discussed in our previous responses, one concern is that using fine-tuning cannot help scaling to new deployment cases. Fine tuning is implicitly considering reasonably small dataset and is hard to achieve comparable performance as you do with new/re-training. We suggest the following changes (blue) to the observation: For direct AI/ML positioning, evaluation results demonstrate the feasibility need to support of model fine-tuning and model re-training to adapt an AI/ML model to a new or for changing and new deployment scenario, respectively, if the existing training dataset and/or trained model(s) cannot adequately cover the new or changing deployment scenario. |
| NOK | Our understanding is that model fine-tuning/re-training should be used under the following conditions: - if the existing training dataset and/or trained model cannot adequately cover the new or changing deployment scenario. AND - if the dataset size is small compared to the dataset used for Model training. If the data size is large, it does not make sense to apply fine-tuning with a sub-set of this dataset because with the availability of a medium/large dataset, it is logical do a regular model training because in this scenario the AI/ML model obtains the best performance. Other reason is that if the model is re-trained with the new medium/large dataset, the probability of catastrophic forgetting in the AI/ML model is very high. Thus, we propose the following rewording: For direct AI/ML positioning, evaluation results demonstrate the necessity to support model fine-tuning/re-training to adapt an AI/ML model to a new or changing deployment scenario when simultaneously the following conditions are met: - If the existing training dataset and/or trained model cannot adequately cover the new or changing deployment scenario. - If the dataset size of the new or changing deployment is small compared to the dataset used for model training. |
| HW/HiSi | We suggest the following change. The secind bullet from Nokia would require further evaluation. For direct AI/ML positioning, evaluation results demonstrate with the the need to support of model fine-tuning/re-training to adapt an the AI/ML model can adapt to a new or changing deployment scenario, if the existing training dataset and/or trained model cannot adequately cover the new or changing deployment scenario. |

For Observation 6.12-3 below,

- The FFS bullet is removed, since some company commented that it is not part of observation.

- Also, a question was raised about “sample density” vs “user density”. “Sample density” is used here based on vivo input, since it more accurately refers to the data samples collected in the training dataset. “User density” can be mis-interpreted as the number of users that run the AI/ML model for positioning.

- Regarding “diminishing return”: this refers to the phenomenon that beyond a certain sample density, collecting more and more training data gives smaller and smaller accuracy improvement. That is, this refers to the flattening curve of “positioning accuracy” vs “training dataset size” that companies have presented.

Observation 6.12-3

For both direct AI/ML positioning and AI/ML assisted positioning, evaluation results show that with diminishing returns, the positioning accuracy improves as the sample density (e.g., #samples/m2) of the training dataset increases.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Qualcomm | Thanks for clarifying the point on diminishing returns. Maybe it is better to leave it as note with more elaboration. Another approach is to break the observation as two bullets where the second bullet can highlight the flattening curve observation that the FL mentioned. |
| HW/HiSi | Ok |

# Performed Evaluation of AI/ML-assisted positioning

In this meeting, a large amount of evaluation work has been performed by companies for AI/ML-assisted positioning. These valuable results are crucial to help RAN1 to make progress.

Representative results submitted by companies are copied below.

## Evaluation of single-TRP construction with same model for N TRP

### Evaluation without generalization considerations (same setting for training and testing)

- Qualcomm (R1-2301408) Proposal 11: For AI/ML-assisted positioning, the single-TRP approach is adopted for evaluation as a baseline. Observation 19: ML-based soft information reporting method provides a significant improvement in positioning accuracy over the classical scheme. The 90th percentile of the horizontal positioning error reduces from >20 m for the classical scheme to 5.10 m.

- Huawei (R1-2300112) Table 17. Evaluation results for AI/ML model deployed on network-side, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity (Single-TRP, same model for N TRPs); Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. PDP 1*4*256; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); {40%, 2m, 2m}; {40%, 2m, 2m}; 18000; 9000; 582; 192K; 0.353.

- vivo (R1-2300448) Table 5 Evaluation results for AI/ML model deployed on UE or Network side Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR w. BS info.; TOA; 0; Drop1; Drop1; 25k*18; 1k*18; 11.92M*1; 23.79M*1; 0.83. CIR w/o. BS info.; TOA; 0; Drop1; Drop1; 25k*18; 1k*18; 11.92M*1; 23.79M *1; 2.57. Table 9 Evaluation results of LOS/NLOS identification accuracy for AI/ML model deployed on UE or Network side, without model generalization, full-connection network Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Accuracy of LOS/NLOS identification. Training; test; Model complexity; Computational complexity; AI/ML. CIR; LOS/NLOS; 0; {0.4, 2, 2}; 25k; 1k; 3.62M*18; 7.24M*18; >99%. R17 [9]; {0.4, 2, 2}; /; 93%. Observation 5: AI/ML based LOS/NLOS identification for positioning has the following advantages: - More accurate LOS/NLOS identification along with a confidence metric - Better compatibility with existing positioning protocol framework. - Great generalization capability. and disadvantages: - Positioning performance could suffer from severe degradation in heavy-NLOS scenarios. - Obtain LOS/NLOS labels is a challenging task for data collection.

- Ericsson (R1-2300141) Table 49 Evaluation results for AI/ML model deployed on network-side, without model generalization investigation. No network synchronization error. Two architectures of the ML model: Model I (6 layers: 3 Conv1D layers, 3 Dense layers) and Model II (9 layers: 6 Conv1D layers, 3 Dense layers) Model; Model input; Model output; Label; Settings; Dataset size; AI/ML complexity; Horiz. pos. accuracy @90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. I; Time domain CIR, 1x2x256 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 5400; 4000; 71 k parameters; 7.3 M FLOPs; 0.109. II; 73 k parameters; 17 M FLOPs; 0.062.

- Apple (R1-2301341) Table 6: AI/ML-assisted Positioning: Evaluation results for AI/ML model deployed on network-side, without model finetuning, with a CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18 x 256 x 2]; ToA [1x1]; 100% labeled; Drop 1; Drop 1; 47500; 2500; 1.6; 3.1; 1.138m.

- InterDigital (R1-2301101) Table 2 Evaluation results for AIML assisted positioning by unobserved RSTD prediction Model Input; Model output; (Percentage of training data set without) Label; Settings (drops, clutter param, network synchronization error); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. RSRP; (estimated) RSTD; 0% (default); {60%, 6m, 2m} , drop A, T1= 0 ns; {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns; 16000; 4000; 332 k; 11.42 M FLOPs; 15.68. RSRP; (unobserved) RSTD; 0% (default); {60%, 6m, 2m} , drop A, T1= 0 ns; {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns; 16000; 4000; 332 k; 11.42 M FLOPs; 10.60.

- CATT (R1-2300675) Observation 19: For single-TRP with the same AI/ML model which is trained with the CIR of 18 TRPs, the horizontal accuracy is 0.58m@90%. Observation 20: For single-TRP with the same AI/ML model which is trained with the CIR of 6 TRPs, the horizontal accuracy is 1.73m@90%. Observation 22: For AI/ML assisted positioning with LOS/NLOS identification, the performance is similar with model inputs between PDP and CIR.

- MediaTek (R1-2301591) Observation 1: The soft-decision approach outperforms the hard-decision approach for AI/ML assisted LOS/NLOS identification positioning. Observation 2: High user density of training dataset provides an improvement in LOS/NLOS identification accuracy over the low user density. Observation 9: The soft-decision approach outperforms the hard-decision approach for AI/ML assisted TOA estimation positioning. Proposal 3 : Support means and variance of TOA as model output to report for AI/ML assisted TOA estimation positioning.

### Generalization aspects (different setting for training and testing)

- Qualcomm (R1-2301408) Observation 20: The ML-assisted soft information reporting method using single-TRP approach generalizes well across inter-site changes with homogeneous clutter settings. Figure 20 CDF of horizontal positioning error for ML-based soft information reporting across drops Observation 21: Training on a mix of clutter settings achieves good accuracy in each setting without the overhead of model switching, while training a separate model for each setting provides better accuracy. - The 90th percentile error increases from 5.10 m to 7.34 m when testing on (60%, 6m, 2m) clutter, and - from 0.53 m to 0.91 m when testing on (40%, 2m, 2m) clutter Observation 22: The ML-assisted soft information reporting using single-TRP approach has good robustness to zone-specific time varying changes. Figure 22. CDF of horizontal positioning error for ML-based soft information reporting across time varying changes in a specific zone.

- Huawei (R1-2300112) Dimension 1: Clutter parameters. Table 18. Evaluation results for AI/ML model deployed on network-side, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Identification rate. Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. PDP 1*4*256; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); InF-DH {40%, 2m, 2m}; InF-DH {40%, 2m, 2m}; 18000; 9000; 582; 192K; 97.2%. PDP 1*4*256; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); InF-DH {40%, 2m, 2m}; InF-DH {60%, 6m, 2m}; 18000; 9000; 582; 192K; 98.6%. PDP 1*4*256; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); InF-DH {40%, 2m, 2m}; InF-DH {40%, 3m, 5m}; 18000; 9000; 582; 192K; 97.7%. Observation 27: When the channel parameters of the inference dataset and the training dataset are different, AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS Identification rate. Dimension 2: InF scenarios. Table 19. Evaluation results for AI/ML model deployed on network-side, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Identification rate. Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. PDP 1*4*256; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); InF-SH {20%, 2m, 10m}; InF-DH {40%, 2m, 2m}; 18000; 9000; 582; 192K; 95.1%. PDP 1*4*256; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); InF-DH {40%, 2m, 2m}; InF-SH {20%, 2m, 10m}; 18000; 9000; 582; 192K; 78%. PDP 1*4*256; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); InF-DH {40%, 2m, 2m}& InF-SH {20%, 2m, 10m} mixed; InF-SH {20%, 2m, 10m}; 18000; 9000; 582; 192K; 97.8%. PDP 1*4*256; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); InF-DH {40%, 2m, 2m}& InF-SH {20%, 2m, 10m} mixed; InF-DH {40%, 2m, 2m}; 18000; 9000; 582; 192K; 97.3%. Observation 28: When the model is trained under the InF-DH scenario but inferred under the InF-SH scenario, the performance deteriorates significantly. But when trained under the InF-SH scenario and inferred under the InF-DH scenario, the AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS Identification rate. Observation 29: When the mixed training dataset consists of samples with the same scenarios as the inference dataset, the performance of the intermediate LOS/NLOS identification rate is improved under both scenarios' inference.

- Ericsson (R1-2300141) Table 50. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {40%, 2m, 2m} and tested with various InF-DH clutter parameters and new drop. No network synchronization error. Two architectures of the ML model: Model I (6 layers: 3 Conv1D layers, 3 Dense layers) and Model II (9 layers: 6 Conv1D layers, 3 Dense layers) Model; Model input; Model output; Label; Settings; Dataset size; AI/ML complexity; Horiz. pos. accuracy @90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. I; Time domain CIR, 1x2x256 complex array; (1). LoS/NLoS classification(2). ToA estimate; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m} new drop; 5400; 4000; 71 k parameters; 7.3 M FLOPs; 0.100. {50%,2m,2m}; 0.264. {60%, 2m, 2m}; 5.340. {60%, 6m, 2m}; 13.476. II; {40%,2m,2m} new drop; 73 k parameters; 17 M FLOPs; 0.061. {50%,2m,2m}; 0.150. {60%, 2m, 2m}; 4.732. {60%, 6m, 2m}; 13.528.

- CATT (R1-2300675) Observation 23: For the AI/ML assisted positioning with LOS/NLOS identification, the generalization performance with different clutter parameters and different scenarios is good.

- MediaTek (R1-2301591) 3.1.2.3 Evaluation of different scenario Observation 3: Performance of AI/ML assisted LOS/NLOS identification positioning degrades when the model is trained with dataset of InF-DH scenario and tested with dataset of InF-SH large hall size scenario. And the performance is improved when mix InF-DH and InF-SH training data. 3.1.2.4 Evaluation of channel estimation error Observation 4: Performance of AI/ML assisted LOS/NLOS identification positioning significantly degrades when the model is trained with small channel estimation error and tested with large channel estimation error. Observation 5: AI/ML model in AI/ML assisted LOS/NLOS identification positioning has robust generalization capability when the model is trained with large channel estimation error and tested with small channel estimation error. Proposal 1 : If generalization over channel estimation error is considered, training data should at least include large channel estimation error for AI/ML positioning. 3.1.2.5 Evaluation of timing error Observation 6: Performance of AI/ML assisted LOS/NLOS identification positioning significantly degrades when the model is trained with small timing error and tested with large timing error. Observation 7: AI/ML model in AI/ML assisted LOS/NLOS identification positioning has robust generalization capability when the model is trained with large timing error and tested with small timing error. Proposal 2 : If generalization over timing error is considered, training data should at least include large timing error for AI/ML positioning.

## Evaluation of single-TRP construction with N models for N TRP

### Evaluation without generalization considerations (same setting for training and testing)

- CATT (R1-2300675) Observation 21: For single-TRP with specific AI/ML model which is trained with the CIR of 6 TRPs separately, the horizontal accuracy is 2.37m@90%.

- vivo (R1-2300448) Table 3 Evaluation results for AI/ML model deployed on UE or Network side Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; Pos.; 0; Drop1; Drop1; 25k; 1k; 1.65M; 22.30M; 0.99. CIR; TOA; 0; Drop1; Drop1; 25k; 1k; 4.26M*18; 8.50M*18; 0.73.

### Generalization aspects (different setting for training and testing)

- vivo (R1-2300448) Table 16 Evaluation results for AI/ML model deployed on UE or Network side Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; TOA; 0; {0.6, 6, 2}; {0.6, 6, 2}; 25k; 1k; 4.26M*18; 8.50M*18; 0.73. CIR; TOA; 0; {0.6, 6, 2}; {0.4, 2, 2}; 25k; 1k; 4.26M*18; 8.50M*18; 3.70. CIR; TOA; 0; {0.4, 2, 2}; {0.4, 2, 2}; 25k; 1k; 4.26M*18; 8.50M*18; 0.32. CIR; TOA; 0; {0.4, 2, 2}; {0.6, 6, 2}; 25k; 1k; 4.26M*18; 8.50M*18; 1.53. Table 18 Evaluation results for AI/ML model deployed on UE or Network side Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; TOA; 0; DH; DH; 25k; 1k; 4.26M*18; 8.50M*18; 0.73. CIR; TOA; 0; DH; HH; 25k; 1k; 4.26M*18; 8.50M*18; >10. CIR; TOA; 0; DH; SH; 25k; 1k; 4.26M*18; 8.50M*18; >10. CIR; TOA; 0; HH; SH; 25k; 1k; 4.26M*18; 8.50M*18; 0.05. Observation 14: For those scenarios whose positioning does not rely on fingerprint features, AI/ML based TOA estimation has better generalization ability than direct AI/ML positioning. Observation 15: AI/ML based TOA estimation has great advantages in positioning performance, deployment flexibility, compatibility with existing positioning protocol framework, and generalization capability.

## Evaluation of multi-TRP construction

### Evaluation without generalization considerations (same setting for training and testing)

- xiaomi (R1-2300571) Table 2 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,285,650; 5.76GFlops; 0.6778. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4，2，2}; {0.4，2，2}; 70000; 10000; 21,285,650; 5.76GFlops; 0.8533.

- vivo (R1-2300448) Table 7 Evaluation results for AI/ML model deployed on UE or Network side Construction; Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Construction 1; CIR; TOA; 0; Drop1; Drop1; 25k; 1k; 4.26M*18; 8.50M*18; 0.73. Construction 2; CIR; TOA; 0; Drop1; Drop1; 25k*18; 1k*18; 11.92M*1; 23.79M *1; 0.83. Construction 3; CIR; TOA; 0; Drop1; Drop1; 25k; 1k; 1.65M; 22.30M; 1.08.

- ZTE (R1-2300175) Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Intermediate results (RSTD errors at CDF=90% (meters)); Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Training; test; Model complexity; Computation complexity; AI/ML. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; 2-D UE position (1x2); 2-D UE position; {60%, 6m, 2m}; {60%, 6m, 2m}; 28800; 1800; 9.50M; 158.66 M; N/A; 0.278 (direct AI/ML positioning). CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; DL RSTD values (1x18); DL RSTD values; {60%, 6m, 2m}; {60%, 6m, 2m}; 28800; 1800; 9.50M; 158.66 M; 0.183; 0.269 (AI/ML assisted positioning).

- CATT (R1-2300675) Observation 10: For AI/ML assisted positioning with perfect network synchronization, the intermediate result of ToA estimating is 1.59ns@90% and the eventual result is 0.655m@90% of CDF percentile of horizontal accuracy. Observation 11: For AI/ML assisted positioning with network synchronization error, the intermediate result of ToA estimating is 1.74ns@90% and the eventual result is 0.7m@90% of CDF percentile of horizontal accuracy.

- MediaTek (R1-2301591) Observation 8: Multi-TRP approach outperforms single-TRP for AI/ML assisted TOA estimation positioning. Observation 10: High user density of training dataset provides an improvement in AI/ML assisted TOA estimation positioning over the low user density. Proposal 4 : Support different user density of training dataset for different requirement on AI/ML positioning.

- Nokia (R1-2300608) Figure 11 - Average accuracy and F1-score for AI/ML assisted positioning with LOS/NLOS in the output model. This prediction corresponds to a multi-TRP scenario with one model for N TRPs used simultaneously. This exercise considered dataset 1 (clutter density 40%). Observation-18: For Assisted AI/ML positioning case, considering LOS/NLOS as the intermediate feature with multi-TRP (i.e., one model for N TRPs), the prediction accuracy is dropped when the number of TRPs (i.e., N) is increased.

- Ericsson (R1-2300141) Table 71 Evaluation results for AI/ML model deployed on network-side, without model generalization investigation. No network synchronization error. Architectures of the ML model: 34 layers complex network. Model; Model input; Model output; Label; Settings; Dataset size; AI/ML complexity; Horiz. pos. accuracy @90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. I; Time domain CIR, 3x2x256 complex array; ToA estimate; Ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 80,000; 4000; 0.86 M parameters; 36 M FLOPs; 0.379. 40,000; 0.489. 20,000; 0.656. 10,000; 0.977. II; 80,000; 3.37 M parameters; 132 M FLOPs; 0.219. 40,000; 0.312. 20,000; 0.491. 10,000; 0.802. III; 80,000; 11.2 M parameters; 425 M FLOPs; 0.167. 40,000; 0.264. 20,000; 0.443. 10,000; 0.761. Table 95. Evaluation results for AI/ML model deployed on network-side, without model generalization investigation. No network synchronization error. Architectures of the ML model: 18 layers complex network. Model; Model input; Model output; Label; Settings; Dataset size; AI/ML complexity; Horiz. pos. accuracy @90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. I; Time domain CIR, 18x2x256 complex array; Direct path ToA estimate; Ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 80,000; 4000; 0.73 M parameters; 32 M FLOPs; 0.335. 40,000; 0.409. 20,000; 0.568. 10,000; 0.79. II; 80,000; 2.85 M parameters; 110 M FLOPs; 0.222. 40,000; 0.294. 20,000; 0.413. 10,000; 0.641. III; 80,000; 11.26 M parameters; 410 M FLOPs; 0.166. 40,000; 0.263. 20,000; 0.374. 10,000; 0.597.

### Evaluation of generalization aspects

- xiaomi (R1-2300571) Table 4 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, without model generalization (different clutter parameter), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2}; {0.4，2，2}; 70000; 10000; 21,285,650; 5.76GFlops; 7.1173. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4，2，2}; {0.6，6，2}; 70000; 10000; 21,285,650; 5.76GFlops; 1.5413. Table 10 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, without model generalization (AI/ML model is trained by data set with ideal network synchronization and tested by data set with 100ns network synchronization error), ResNet Model input; Model output; Label; Clutter param & network synchronization error; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2} 0ns error; {0.6，6，2} 100ns error; 70000; 10000; 21,285,650; 5.76GFlops; 12.7748. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4，2，2} 0 ns error; {0.4，2，2} 100ns error; 70000; 10000; 21,285,650; 5.76GFlops; 15.4699.

- ZTE (R1-2300175) AI/ML assisted RSTD estimation Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Intermediate results (RSTD errors at CDF=90% (meters)); Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; DL RSTD values (1x18); DL RSTD values (1x18); 1st Drop; N/A; 1st Drop; 28800; N/A; 1800; 9.50M; 158.66 M; 0.183; 0.271. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; DL RSTD values (1x18); DL RSTD values (1x18); 1st Drop; N/A; 2nd Drop; 28800; N/A; 1800; 9.50M; 158.66 M; 16.759; 29.365. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; DL RSTD values (1x18); DL RSTD values (1x18); 1st Drop; 2nd Drop; 2nd Drop; 28800; 5000; 1800; 9.50M; 158.66 M; 2.700; 3.491. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; DL RSTD values (1x18); DL RSTD values (1x18); 1st Drop; 2nd Drop; 2nd Drop; 28800; 10000; 1800; 9.50M; 158.66 M; 2.092; 3.136. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; DL RSTD values (1x18); DL RSTD values (1x18); 1st Drop; 2nd Drop; 2nd Drop; 28800; 15000; 1800; 9.50M; 158.66 M; 1.889; 2.896. CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 128}; DL RSTD values (1x18); DL RSTD values (1x18); 1st Drop; 2nd Drop; 2nd Drop; 28800; 20000; 1800; 9.50M; 158.66 M; 1.374; 1.863.

- CATT (R1-2300675) Observation 12: For generalization performance with different assumptions on AI/ML assisted positioning, the positioning accuracy is seriously degraded.

- MediaTek (R1-2301591) 3.1.3.4 Evaluation of different scenario Observation 11: Performance of AI/ML assisted TOA estimation positioning degrades when the model is trained with dataset of one scenario and tested with dataset of another scenario, especially for different hall size scenario. Proposal 5 : Further study the monitor mechanism of different hall size scenario for AI/ML positioning. 3.1.3.5 Evaluation of channel estimation error Observation 12: Performance of AI/ML assisted TOA estimation positioning significantly degrades when the model is trained with small channel estimation error and tested with large channel estimation error. Observation 13: AI/ML model in AI/ML assisted TOA estimation positioning has robust generalization capability when the model is trained with large channel estimation error and tested with small channel estimation error. 3.1.3.6 Evaluation of timing error Observation 14: Performance of AI/ML assisted TOA estimation positioning significantly degrades when the model is trained with small timing error and tested with large timing error. Observation 15: AI/ML model in AI/ML assisted TOA estimation positioning has robust generalization capability when the model is trained with large timing error and tested with small timing error.

- Ericsson (R1-2300141) Table 97. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {60%, 6m, 2m}, then fine-tuned with InF-DH {40%, 2m, 2m}, and tested with various InF-DH clutter parameters. No network synchronization error. Architectures of the ML model: 18 layers complex network Model; Model input; Model output; Label; Settings; Dataset size; AI/ML complexity; Horiz. pos. accuracy @90% (m). Train; Fine-tune; Test; Train; Fine-tune; Test; Model complexity; Computation complexity; AI/ML. I; Time domain CIR, 18x2x256 complex array; Direct path ToA estimates; Ideal; {60%, 6m, 2m}; {40%, 2m, 2m}; {40%,2m,2m}; 40,000; 40,000; 4000; 0.73 M parameters; 32 M FLOPs; 0.672. 20,000; 0.917. 10,000; 1.290. 4,000; 1.943. 2,000; 2.698. 1,000; 4.127.

## Impact of user density/size of the training dataset

- xiaomi (R1-2300571) Figure 4 Relationship between size of training data set and positioning accuracy for ToA-based prediction

- Ericsson (R1-2300141) Table 4. 90%tile 2D positioning accuracy for different model size classes in the {60%, 6m, 2m} InF-DH scenario. Model class; Positioning approach; Model complexity [# parameters]; Computational complexity [FLOPs]; 90%tile 2D positioning error [m] with different train set sizes in {60%, 6m, 2m} InF-DH. 80,000; 40,000; 20,000; 10,000. Small models; Dist. Assist.; 0.86 M; 36 M; 0.379; 0.489; 0.656; 0.977. Cent. Assist.; 0.73 M; 32 M; 0.335; 0.409; 0.568; 0.790. Cent. Direct; 0.73 M; 32 M; 0.366; 0.443; 0.596; 0.830. Medium-size models; Dist. Assist.; 3.37 M; 132 M; 0.219; 0.312; 0.491; 0.802. Cent. Assist.; 2.85 M; 110 M; 0.222; 0.294; 0.413; 0.641. Cent. Direct; 2.85 M; 110 M; 0.250; 0.338; 0.484; 0.763. Large models; Dist. Assist.; 11.2 M; 425 M; 0.167; 0.264; 0.443; 0.761. Cent. Assist.; 11.26 M; 410 M; 0.166; 0.263; 0.374; 0.597. Cent. Direct; 11.26 M; 410 M; 0.203; 0.299; 0.470; 0.743. Figure 5: Positioning accuracy vs training data size according to Table 4.

## Model LCM evaluations

### Evaluation of model fine-tuning

- vivo (R1-2300448) Table 32 Evaluation results of fine-tuning for AI/ML model deployed on UE or Network side, FNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal Pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR; TOA; 0%; Drop1; /; Drop2; 25k; 0; 1k; 4.26M*18; 8.50M*18; 10.37. CIR; TOA; 0%; Drop1; Drop2; Drop2; 25k; 0.5k; 1k; 4.26M*18; 8.50M*18; 5.61. CIR; TOA; 0%; Drop1; Drop2; Drop2; 25k; 1k; 1k; 4.26M*18; 8.50M*18; 5.50. CIR; TOA; 0%; Drop1; Drop2; Drop2; 25k; 2k; 1k; 4.26M*18; 8.50M*18; 5.03. CIR; TOA; 0%; Drop1; Drop2; Drop2; 25k; 3k; 1k; 4.26M*18; 8.50M*18; 4.08. Table 34 Evaluation results of fine-tuning for AI/ML model deployed on UE or Network side, FNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal Pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR; TOA; 0%; DH; /; HH; 25k; 0; 1k; 4.26M*18; 8.50M*18; >10. CIR; TOA; 0%; DH; HH; HH; 25k; 0.5k; 1k; 4.26M*18; 8.50M*18; 0.30. CIR; TOA; 0%; DH; HH; HH; 25k; 1k; 1k; 4.26M*18; 8.50M*18; 0.17. CIR; TOA; 0%; DH; HH; HH; 25k; 2k; 1k; 4.26M*18; 8.50M*18; 0.09. CIR; TOA; 0%; DH; HH; HH; 25k; 3k; 1k; 4.26M*18; 8.50M*18; 0.06. Proposal 10: Further study and confirm the benefits of fine-tuning in terms of model generalization enhancement for AI/ML assisted positioning. Proposal 11: Capture in the TR the benefits of fine-tuning for AI/ML assisted positioning in terms of positioning accuracy for AI model generalization capability. Table 46 Fine-tuning data sample efficiency for different cases Cases; Range of sample size; Data efficiency (@90% per 100 additional samples); Positioning accuracy with sample size N1 for sample range N1~N2(@90%). Train: {0.6, 6, 2} Fine-tuning: {0.4, 2, 2} Testing: {0.4, 2, 2}; 0-500; 0.570; 3.70 (0 samples). 500-1000; 0.044; 0.85(500 samples). 1000-2000; 0.015; 0.63(1000 samples). 2000-3000; 0.001; 0.48(2000 samples). Train: Drop1 Fine-tuning: Drop2 Testing: Drop2; 0-500; 0.952; 10.37. 500-1000; 0.022; 5.61. 1000-2000; 0.047; 5.50. 2000-3000; 0.095; 5.03. Train: DH Fine-tuning: HH Testing: HH; 0-500; 1.99; 20.20. 500-1000; 0.026; 0.30. 1000-2000; 0.008; 0.17. 2000-3000; 0.003; 0.09. Train: DH Fine-tuning: SH Testing: SH; 0-500; 2.042; 20.70. 500-1000; 0.022; 0.28. 1000-2000; 0.007; 0.17. 2000-3000; 0.001; 0.10. Train: Sync. Error 0ns Fine-tuning: 50ns Testing: 50ns; 0-500; 0.896; 8.45. 500-1000; 0.114; 3.97. 1000-2000; 0.043; 3.40. 2000-3000; 0.042; 2.97. Train: Sync. Error 0ns Fine-tuning: 10ns Testing: 10ns; 0-500; 0.002; 2.11. 500-1000; 0.064; 2.10. 1000-2000; 0.021; 1.78. 2000-3000; 0.017; 1.57. Train: Sync. Error 0ns Fine-tuning: 2ns Testing: 2ns; 0-500; 0.054; 1.70. 500-1000; 0.012; 1.43. 1000-2000; 0.001; 1.37. 2000-3000; 0.006; 1.37.

- xiaomi (R1-2300571) Table 8 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, with model generalization (with fine-tuning), ResNet Model input; Model output; Label; Clutter parameter; Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2}; {0.4，2，2}; {0.4，2，2}; 70000; 5000; 10000; 21,285,650; 5.76GFlops; 1.5325. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4，2，2}; {0.6，6，2}; {0.6，6，2}; 70000; 5000; 10000; 21,285,650; 5.76GFlops; 0.8494. Table 14 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, with model generalization (with fine-tuning), ResNet Model input; Model output; Label; Clutter param & network synchronization error; Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2} , 0ns error; {0.6，6，2} , 100 ns error; {0.6，6，2} , 100 ns error; 70000; 5000; 10000; 21,285,650; 5.76GFlops; 4.4925. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4，2，2} 0ns error; {0.4，2，2} 100 ns error; {0.4，2，2} 100 ns error; 70000; 5000; 10000; 21,285,650; 5.76GFlops; 7.5060.

- CATT (R1-2300675) Observation 16: When AI/ML-assisted model is trained with the dataset of clutter parameter {60%, 6m, 2m} and fine-tuned with a small dataset of clutter parameter {40%, 2m, 2m}, the horizontal positioning accuracy is improved obviously compared with AI/ML model without fine-tuning. If the fine-tuning dataset size is larger, the improvement of horizontal positioning accuracy is greater. Observation 17: When AI/ML model is trained with perfect network synchronization and fine-tuned with a small dataset with network synchronization error, the horizontal positioning accuracy is improved obviously compared with AI/ML model without fine-tuning. If the fine-tuning dataset size is larger, the improvement of horizontal positioning accuracy is greater. Observation 18: When AI/ML-assisted model is trained with InF-DH and fine-tuning with a small dataset with InF-SH, the horizontal positioning accuary is improved obviously compared with AI/ML model with fine-tuning. If the fine-tuning dataset size is larger, the improvement of horizontal positioning accuracy is greater.

- MediaTek (R1-2301591) Observation 16: Fine-tuning the model with samples from a scenario can achieve positioning accuracy improvement when the pre-trained model is transferred to a new scenario for AI/ML assisted TOA estimation positioning (the more fine-tuning data the better performance), but performance degrades for pre-trained scenario.

- Ericsson (R1-2300141) Table 97. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {60%, 6m, 2m}, then fine-tuned with InF-DH {40%, 2m, 2m}, and tested with various InF-DH clutter parameters. No network synchronization error. Architectures of the ML model: 18 layers complex network Model; Model input; Model output; Label; Settings; Dataset size; AI/ML complexity; Horiz. pos. accuracy @90% (m). Train; Fine-tune; Test; Train; Fine-tune; Test; Model complexity; Computation complexity; AI/ML. I; Time domain CIR, 18x2x256 complex array; Direct path ToA estimates; Ideal; {60%, 6m, 2m}; {40%, 2m, 2m}; {40%,2m, 2m}; 40,000; 40,000; 4000; 0.73 M parameters; 32 M FLOPs; 0.672. 20,000; 0.917. 10,000; 1.290. 4,000; 1.943. 2,000; 2.698. 1,000; 4.127.

### Evaluation of training with mixed dataset

- xiaomi (R1-2300571) Table 6 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, with model generalization (different clutter parameter), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; Mix of {0.6，6，2} {0.4，2，2}; {0.6，6，2}; 35000+35000; 10000; 21,285,650; 5.76GFlops; 0.6867. 18*256*1 CIR; 18*1 UE TOA; 18*1 UE TOA; Mix of {0.6，6，2} {0.4，2，2}; {0.4，2，2}; 35000+35000; 10000; 21,285,650; 5.76GFlops; 0.7974.

- CATT (R1-2300675) Observation 13: When AI/ML-assisted model is mix-trained with the dataset of clutter parameter {60%, 6m, 2m} and a small dataset of clutter parameter {40%, 2m, 2m}, the horizontal positioning accuracy is improved from 3.11m to 1.51m compared with AI/ML model generalization performance without mix-training. Observation 14: When AI/ML-assisted model is mix-trained with the dataset of ideal synchronization and a small dataset of network synchronization error 50ns, the horizontal positioning accuracy is improved from 12.8m to 1.81m compared with AI/ML-assisted model generalization performance without mix-training. Observation 15: When AI/ML-assisted model is mix-trained with the dataset of InF-DH {60%, 6m, 2m} and a small dataset of InF-SH {20%, 10m, 2m}, the horizontal positioning accuracy is improved from 6.894m to 1.467m compared with AI/ML model generalization performance without mix-training.

- Ericsson (R1-2300141) Table 98. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained with mixed datasets in InF-DH {60%, 6m, 2m} and InF-DH {40%, 4m, 2m} and tested with various InF-DH clutter parameters. No network synchronization error. Architectures of the ML model: 18 layers complex network. Model; Model input; Model output; Label; Settings; Dataset size; AI/ML complexity; Horiz. pos. accuracy @90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. I; Time domain CIR, 18x2x256 complex array; Direct path ToA estimates; Ideal; {60%, 6m, 2m}, {40%, 2m, 2m}; {40%,2m,2m}; 40,000 each train dataset; 4000; 0.73 M parameters; 32 M FLOPs; 0.697. {60%,2m,2m}; 0.604. {40%,6m,2m}; 0.470. {60%,6m,2m}; 0.459.

## Evaluation of input size reduction

- xiaomi (R1-2300571) Table 15 Evaluation results for reduced input dimension for direct AI-based positioning and AI-based ToA predication, model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*24*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,277,442; 539.94MFlops; 0.8219. 18*24*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,285,650; 539.95MFlops; 0.8993.

## Semi-supervised learning

- MediaTek (R1-2301591) Table 15. Evaluation results for UE side model, CNN Model input; Model output; (percentage of training data set without) Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP(256*8); 18TOA; 0%; {40%, 4m, 2m}; 32400UE*18TRP labelled; 3600UE*18TRP; 187k; 15M; 1.56. non-normalized PDP(256*8); 18TOA; 0%; {40%, 4m, 2m}; 3600UE*18TRP labelled; 3600UE*18TRP; 187k; 15M; 5.58. non-normalized PDP(256*8); 18TOA; 50%; {40%, 4m, 2m}; 3600UE*18TRP labelled 3600UE*18TRP un-labelled; 3600UE*18TRP; 187k; 15M; 4.64. non-normalized PDP(256*8); 18TOA; 88.9%; {40%, 4m, 2m}; 3600UE*18TRP labelled 28800UE*18TRP un-labelled; 3600UE*18TRP; 187k; 15M; 4.46. non-normalized PDP(256*8); 18TOA; 0%; {40%, 4m, 2m}; 7200UE*18TRP labelled; 7200UE*18TRP; 187k; 15M; 3.33. non-normalized PDP(256*8); 18TOA; 50%; {40%, 4m, 2m}; 7200UE*18TRP labelled 7200UE*18TRP un-labelled; 7200UE*18TRP; 187k; 15M; 3.06. non-normalized PDP(256*8); 18TOA; 75%; {40%, 4m, 2m}; 7200UE*18TRP labelled 21600UE*18TRP un-labelled; 7200UE*18TRP; 187k; 15M; 2.83.

## 1st round discussion

For AI/ML assisted positioning, companies have submitted a large amount of evaluation results on the various generalization aspects and many other issues as shown above. Similar observations have been made by companies, and the observations below are drawn.

Observation 7.8-1

Observation: For AI/ML assisted positioning using single-TRP with same model for N TRPs,

- A single AI/ML model deployed to all TRPs for LoS classification and/or ToA estimation can generalize to different InF-DH clutter parameters and different drops.

- LoS classification KPIs are robust to network synchronization error and UE/gNB RX and TX timing error.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi |  |
| OPPO | In the proposal, there is no quantification or description on the generalization performance can be obtained by the AI/ML model. Suggest to be more specific to what generalization performance or robustness the AI/ML model can achieve. |
| Mediatek | From our observation, LoS classification performance degrade if training data with 0 ns timing error and test with 50ns timing error. [Moderator] Maybe the observation on LoS classification is not fully converged. Remove it from Observation for now. |

Observation 7.8-2

Observation: For AI/ML assisted positioning using multi-TRP construction, evaluation of generalization aspects show that the positioning accuracy deteriorates when the ML model is trained with dataset of one setting, while tested with dataset of a different setting.

- The generalization aspects include:

  - Deployment environment:

    - Different drops

    - Different clutter parameters

    - Different InF scenarios

    - SINR mismatch

    - Time varying changes

  - Implementation imperfections:

    - Network synchronization error

    - UE/gNB RX and TX timing error

    - Channel estimation error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning.

  - Better training dataset construction: The training dataset is composed of data from multiple settings, which include data from the same setting as the test dataset.

  - Model fine-tuning: the model is re-trained with a dataset from the same setting as the test dataset.

|  | Company |
| --- | --- |
| Support | Fujitsu, CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| OPPO | Fine with the proposal. |
| Qualcomm | Please include model switching as a third solution in the second bullet. |
| Mediatek | From our observations, the positioning accuracy is not always degraded when the ML model is trained with dataset of one setting, while tested with dataset of a different setting. For example training data with 50ns timing error and test data without timing error. [Moderator] Updated the observation to clarify that the data is unseen during testing. |

## 2nd round discussion

Observation 7.8-1

Observation: For AI/ML assisted positioning using single-TRP with same model for N TRPs,

- A single AI/ML model deployed to all TRPs for ToA estimation can generalize to different InF-DH clutter parameters and different drops.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| NOK | As non all companies provided yet results on ToA estimation, we suggest to postpone this Observation for the next meeting. However, we can have a similar Observation during this meeting on LOS/NLOS classification, as it was deployed by many companies. |
| ZTE | From our observation, ToA/RSTD estimation cannot generalize well for InF-DH scenario(e.g., low LOS probability) |
| Mediatek | There are not enough results on the observation. Need to further study. |
| Fujitsu | Agree with NOK, ToA estimation is not mainstream simulation case across companies. |
| vivo | The wording „can generalize” is subject to interpretation. We suggest capturing values instead of such general statement. |

Observation 7.8-2

Observation: For AI/ML assisted positioning using multi-TRP construction, evaluation of generalization aspects show that the positioning accuracy deteriorates when the ML model is trained with dataset of a first setting, while tested with dataset of a second setting, where the data in the second setting is unseen in the first setting.

- The generalization aspects include:

  - Deployment environment:

    - Different drops

    - Different clutter parameters

    - Different InF scenarios

    - SINR mismatch

    - Time varying changes

  - Implementation imperfections:

    - Network synchronization error

    - UE/gNB RX and TX timing error

    - Channel estimation error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning.

  - Better training dataset construction: The training dataset is composed of data from multiple settings, which include data from the same setting as the test dataset.

  - Model fine-tuning: the model is re-trained with a dataset from the same setting as the test dataset.

|  | Company |
| --- | --- |
| Support |  |
| Not support | vivo |

| Company | Comments |
| --- | --- |
| NOK | Aligned with the proposal. Please consider the same rewording proposed in the case of Direct AI/ML. |
| ZTE | Similar comments as Observation 6.9-1. |
| Mediatek | Same with Observation 6.9-1 We think a note for “unseen” is needed. Note: when the AI/ML model is trained with dataset of a first setting (e.g., a larger timing error truncated Gaussian with stdv=50ns), while tested with dataset of a second setting (e.g., a smaller timing error stdv=20ns), where the data in the second setting is not unseen in the first setting. |
| Fujitsu | Similar comments as Observation 6.9-1. |
| Qualcomm | Similar to our comment to Observation 6.9-1 |
| vivo | We don’t know why the observation is only „For AI/ML assisted positioning using multi-TRP construction“. We have evaluated and provided results for single TRP with the same model and single TRP with N models, as well as multi-TRP construction. At least some of the generalization aspects (e.g., InF scenarios) listed actual degrade performance even for single TRP construction. Current proposal does not capture all the observations from results and is not complete nor accurate. |

## 5th round discussion

With the Observation for direct AI/ML positioning agreed, similar observation can be drawn for AI/ML assisted positioning using multi-TRP construction. Thus the following is provided for further discussion.

Observation 7.10-1

Observation: For AI/ML assisted positioning using multi-TRP construction, evaluation of generalization aspects show that the positioning accuracy deteriorates when the ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario.

- The generalization aspects include:

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

  - Network synchronization error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

  - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

- Note: ideal model training and model switching may provide the upper bound of achievable performance when the AI/ML model need to handle different deployment scenarios.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Ok |

# (Closed) Evaluation of model monitoring methods

For the topic of model monitoring, the following methods have been proposed by companies.

## Model monitoring based on inference accuracy

- vivo (R1-2300448) 7.2.1. Ground truth label based model monitoring Proposal 18: The accuracy and quantity of ground truth labels should be considered for ground truth label based model monitoring 7.2.2. Motion sensors assisted model monitoring Observation 36: Motion sensors can be used to assist model monitoring. Proposal 19: Further study how to use motion sensors’ information to assist model monitoring. 7.2.3. Self-monitoring for AI/ML assisted positioning Observation 37: The proposed self-monitoring method can achieve model monitoring for AI/ML assisted positioning.

- Ericsson (R1-2300141) Figure 7: Residual losses from conventional triangulation-based error minimization positioning algorithms. The ML model is trained in the {60%, 6m, 2m} environment and tested in three environments: {60%, 6m, 2m}, {40%, 6m, 2m} and {40%, 2m, 2m}. Observation 11 For AI/ML assisted positioning approaches (e.g., Case 3a), model monitoring metrics can be accurately and reliably provided by the conventional positioning methods (e.g., residual loss). This is an important advantage of AI/ML assisted positioning approaches over the direct AI/ML positioning approach (e.g., Case 3b). Observation 12 For AI/ML assisted positioning approaches, model monitoring leveraging conventional positioning method incurs negligible cost in terms of: signaling overhead, complexity, latency, and power consumption for obtaining a model monitoring sample. Proposal 6 Capture in TR 38.843 that: For AI/ML assisted positioning, model monitoring metrics can be reliably provided by the conventional positioning methods.

## Model monitoring based on input data distribution

- vivo (R1-2300448) 7.1.1. The shift detection of dominant feature distribution Observation 33: Adopting SINR as a dominant feature of CIR is valid for model monitoring Proposal 15: When model input is CIR or PDP, identify these dominant features strongly related to positioning for model monitoring Proposal 16: The metrics that can describe the difference between two distributions mathematically can be reused directly for model monitoring. 7.1.2. AI/ML based adversarial validation 7.1.3. AI/ML based out-of-distribution detection

- LG (R1-2300534) Proposal #1: Consider model monitoring metric based on model input depending on the amount of similarity between input and training data distribution

## Model monitoring based on both input and output

- Ericsson (R1-2300141) Figure 6. 2D position estimate difference using unmodified or modified positioning request data at production in different operating environments for a small centralized direct positioning model trained with {60%, 6m, 2m} dataset samples. Observation 9 For a direct positioning ML model, self-model monitoring can be performed if the model was trained with data augmentation techniques. Otherwise, model monitoring generally requires collecting new ground truth samples during model operation. Observation 10 For both direct and AI/ML assisted positioning methods, self-model monitoring method does not require ground truth label and has no signaling overhead. The complexity, power consumption, and latency for obtaining one model monitoring sample are equal to one round of model inference. Proposal 5 Capture in TR 38.843 that: For both direct and AI/ML assisted positioning methods, self-model monitoring is a candidate solution for model monitoring.

## 1st/2nd round discussion

In RAN1#110bis, the following agreements were made in AI 9.2.1 on model monitoring.

Agreement

Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case:

    - Monitoring based on inference accuracy, including metrics related to intermediate KPIs

    - Monitoring based on system performance, including metrics related to system peformance KPIs

    - Other monitoring solutions, at least following 2 options.

      - Monitoring based on data distribution

        - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc.

        - Output-based: e.g., drift detection of output data

      - Monitoring based on applicable condition

Note: Model monitoring metric calculation may be done at NW or UE

Agreement

Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance

    - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

    - Overhead (e.g., signaling overhead associated with model monitoring)

    - Complexity (e.g., computation and memory cost for model monitoring)

    - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

    - FFS: Power consumption

    - Other KPIs are not precluded.

Note: Relevant KPIs may vary across different model monitoring approaches.

FFS: Discussion of KPIs for other LCM procedures

In RAN1#111, the following agreement was made for model monitoring for AI/ML assisted approach:

Agreement

For AI/ML assisted approach, study the performance of model monitoring metrics at least where the metrics are obtained from inference accuracy of model output.

For this meeting, more model monitoring methods are proposed, including those for direct AI/ML approach.

Some methods require a complementary mechanism to provide ground truth label (or its approximation) during model deployment. Other methods do not require ground truth label (or its approximation), including the methods that monitor based on input data distribution, and the methods that monitor based on both model input and model output.

Based on the preliminary evaluation results, the following are proposed for companies to further investigate the model monitoring methods.

Proposal 8.4-1

For direct AI/ML positioning, study the performance of model monitoring methods, including:

- Inference accuracy based methods, where ground truth label (or its approximation) is provided during model deployment for the calculation of inference accuracy.

- Label-free methods, which do not require ground truth label (or its approximation) during model deployment.

|  | Company |
| --- | --- |
| Support | Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | For positioning, the second bullet is quite important, the only concern is that we should keep aligned with 9.2.1 on the general model monitoring discussion. [Moderator] The agreements from 9.2.1 are copied above now. It’s consistent. |
| NOK | For evaluation purposes, we understand that the inference accuracy-based method was already done in the current evaluations that all companies have done (testing/trained in the same dataset setting). However, label-free method is a new scenario to be evaluated. To avoid many divergency on the evaluation of label-free method, companies should agree a basic benchmark on label-free method. |
| Xiaomi | Generally we are OK. But, we see In section AI 9.2.4.2, the study of performance is categorized as input-based monitoring and output-based monitoring. we would like to suggest to keep consistent categorization of performance monitoring methods across different AIs for better comparison. |

Proposal 8.4-2

For AI/ML assisted approach, study the performance of label-free model monitoring methods, which do not require ground truth label (or its approximation) during model deployment.

|  | Company |
| --- | --- |
| Support | Fujitsu |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | For positioning, this point is quite important, the only concern is that we should keep aligned with 9.2.1 on the general model monitoring discussion. [Moderator] The agreements from 9.2.1 are copied above now. It’s consistent. |
| Qualcomm | Please include inference accuracy as an option (like in the previous proposal for direct AI/ML approach). [Moderator] This has been agreed in last meeting See agreement copied below. Agreement For AI/ML assisted approach, study the performance of model monitoring metrics at least where the metrics are obtained from inference accuracy of model output. |
| Mediatek | For AI/ML assisted approach, we also need to study ground truth label model monitoring methods. For example TOA based on known position as label for TOA assisted positioning. [Moderator] See response above. |
| NOK | Same comment of 8.4-1 |

# Proposals for online discussion

## Proposals for Tuesday online discussion

Proposal 4.4-1

For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity.

- The type of information to use as model input. The candidates include at least: time-domain CIR, PDP.

- The dimension of model input in terms of NTRP, Nt, and Nt’.

- Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference.

Proposal 8.4-1

For direct AI/ML positioning, study the performance of model monitoring methods, including:

- Inference accuracy based methods, where ground truth label (or its approximation) is provided for the calculation of inference accuracy of model output.

- Label-free methods, which do not require ground truth label (or its approximation) for model monitoring.

Proposal 8.4-2

For AI/ML assisted approach, study the performance of label-free model monitoring methods, which do not require ground truth label (or its approximation) for model monitoring.

Proposal 6.8-4

For direct AI/ML positioning, study the impact of labelling error to positioning accuracy, to understand the maximum acceptable labeling error in training data collection.

- The ground truth (UE location) error in x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range and [-2*L, 2*L] range.

  - Value L is up to sources. Example values: L=0.1 (m), 0.25 (m), 0.5 (m).

Observation 6.9-2

For direct AI/ML positioning, evaluation results demonstrate that training data collection need to ensure that data from various scenarios are collected for the training dataset, which include data for the target scenario and data for the expected implementation imperfections.

Observation 6.9-3

For direct AI/ML positioning, evaluation results demonstrate the need to support model fine-tuning/re-training to adapt an AI/ML model to a new or changing deployment scenario.

## Proposals for Thursday online discussion

Conclusion 5.5-1

No dedicated evaluation is needed for the positioning accuracy performance of model switching.

- Note: Model switching is considered a candidate approach to achieve good performance across different scenarios.

Proposal 6.11-1

For direct AI/ML positioning, study the impact of labelling error to positioning accuracy for model inference.

- The ground truth (UE location) error in x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range.

  - Value L is up to sources.

- FFS: whether/how to study the impact of labelling error to label-based model monitoring methods.

- FFS: whether/how to study the impact of labelling error for AI/ML assisted positioning.

Observation 6.11-2

Observation: Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario.

- The generalization aspects include:

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

  - Network synchronization error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

  - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

  - Note: for performance evaluation, ideal model switching provides the upper bound of achievable performance when the AI/ML model need to handle different deployment scenarios.

- Exemplary evaluation results of horizontal positioning accuracy at CDF=90% (m) for different drops. InF-DH, {60%, 6m, 2m}

| Sources | Train and test with dataset of drop 1 | Train with dataset of drop 1, and test with dataset of drop 2 |
| --- | --- | --- |
| Huawei (R1-2300112) | 0.62 | >10 |
| Ericsson (R1-2300141) | 0.443 | 16.755 |
| ZTE (R1-2300175) | 0.26 | 20.42 |
| vivo (R1-2300448) | 0.99 | 6.00 |
| NVIDIA (R1-2301182) | 2.3 | 14.5 |
| Qualcomm (R1-2301408) | 2.77 | 12.33 |
| MediaTek (R1-2301591) | 1.32 | 9.15 |
| OPPO (R1-2300284) | 0.33 | 10.11 |

- Exemplary evaluation results of horizontal positioning accuracy at CDF=90% (m) for different clutter parameters {60%, 6m, 2m} vs {40%, 2m, 2m}

| Sources | Train and test with dataset of {60%, 6m, 2m} | Train with dataset of {60%, 6m, 2m}, and test with dataset of {40%, 2m, 2m} |
| --- | --- | --- |
| Ericsson (R1-2300141) | 0.443 | 8.168 |
| ZTE (R1-2300175) | 0.26 | 19.31 |
| vivo (R1-2300448) | 0.99 | 8.67 |
| xiaomi (R1-2300571) | 0.4462 | 7.0914 |
| NVIDIA (R1-2301182) | 2.3 | 17.9 |
| MediaTek (R1-2301591) | 1.32 | 9.10 |
| OPPO (R1-2300284) | 0.33 | 15.75 |

- Exemplary evaluation results of horizontal positioning accuracy at CDF=90% (m) for different network synchronization error

| Sources | Train and test with dataset without network synchronization error (i.e., 0 ns) | Train with dataset network synchronization error, and test with dataset with network synchronization error |
| --- | --- | --- |
| Huawei (R1-2300112) | 0.62 | >10 (50ns) |
| Ericsson (R1-2300141) | 0.460 | 0.681 (10ns) 2.654 (25ns) 14.127 (50ns) |
| vivo (R1-2300448) | 0.99 | 1.64 (2ns) 4.56 (10ns) 10.18 (50ns) |
| ZTE (R1-2300175) | 0.26 | 10.32 (50ns) |
| xiaomi (R1-2300571) | 0.4462 | 12.4486 (100ns) |
| InterDigital (R1-2211715) | 2.22 | 9.25 (50ns) |
| OPPO (R1-2300284) | 0.33 | 6.54 (50ns) |

- Exemplary evaluation results of horizontal positioning accuracy at CDF=90% (m) for different InF scenarios

| Sources | Train and test with dataset of InF-DH | Train with dataset of one InF-DH scenario, and test with dataset of InF-HH or InF-SH |
| --- | --- | --- |
| vivo (R1-2300448) | 0.99 | >10 |
| MediaTek (R1-2301591) | 1.32 | >100 |

Observation 6.11-3

For direct AI/ML positioning, evaluation results demonstrate that training data collection need to ensure that data from various deployment scenarios are collected for the training dataset, which include data for the target deployment scenario.

- Exemplary evaluation results of mixed dataset for different drops

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy @ CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Train | Test | Model complexity | Computation complexity | AI/ML |
| ZTE (R1-2300175) | CIR {Nport,NTRP, Nt,Nt'} = {1, 18, 256, 256} | 2-D UE position (1x2) | 2-D UE position | Mixed datasets from 1st Drop and 2nd Drop | 1st Drop | 28800 + 28800 | 1800 | 984.96K | 44.28 M | 0.31 |
| Huawei (R1-2300112) | CIR 18*4*256 | UE coordinates | UE coordinates | {60%, 6m, 2m}, 5 Drops mixed | {60%, 6m, 2m}, Drop 1 (inside the trained Drops) | 25000 (5000/drop) | 5000 | 34 K | 25.81M | 1.28 |

- Exemplary evaluation results of mixed dataset for different clutter parameters

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Train | Test | Model complexity | Computation complexity | AI/ML |
| vivo (R1-2300448) | CIR | Pos. | 0 | Mix of {0.6, 6, 2} and {0.4, 2, 2} | {0.6, 6, 2} | 25k & 25k | 1k | 1.65M | 22.30M | 0.87 |
| xiaomi (R1-2300571) | 18*256*1 CIR | 2*1 UE coordinates | 2*1 UE coordinates | Mix of {0.6，6，2}, {0.4，2，2} | {0.6，6，2} | 35000+35000 | 10000 | 21,277,442 | 5.76GFlops | 0.5419 |
| Ericsson (R1-2300141) | Time domain CIR, 18x2x256 complex array | 2D UE position estimate | Ideal | {60%, 6m, 2m}, {40%, 2m, 2m} | {40%,2m,2m} | 40,000 each train dataset | 4000 | 0.73 M parameters | 32 M FLOPs | 0.699 |
| MediaTek (R1-2301591) | CIR [18,2,256] | UE pos [x,y] | 0% | {60%, 6m, 2m} +{40%, 2m, 2m} | {40%, 2m, 2m} | 16200+16200 | 3600 | 464.24K | 0.266G | 2.30 |

Observation 6.11-4

For direct AI/ML positioning, evaluation results demonstrate the need to support model fine-tuning/re-training to adapt an AI/ML model to a new or changing deployment scenario.

- Exemplary evaluation results of fine-tuning/re-training for different drops

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Fine-tune | Test | Train | Fine-tune | Test | Model complexity | Computation complexity | AI/ML |  |
| OPPO (R1-2300284) | Normalized CIR + RSRP | UE coordination | UE coordination | 1 drop, 80,000 UEs per drop {60%, 6, 2} | 2nd drop {60%, 6, 2} | Remaining samples of the 2nd drop | 80,000 | 1,000 | 7,900 | 2.66M | 5.32 MFLOPs | 1.233 |  |
|  |  |  |  |  |  |  |  | 5000 | 7,500 |  |  | 0.688 |  |
| Huawei (R1-2300112) | CIR 18*4*256 | UE coordinates | UE coordinates | {60%, 6m, 2m}, Drop 1 | {60%, 6m, 2m}, Drop 2 | {60%, 6m, 2m}, Drop 2 | 25000 | 1000 | 5000 | 52.42K | 25.81M | 2.86 |  |

- Exemplary evaluation results of fine-tuning/re-training for different clutter parameters

| Sources | Model input | Model output | Label | Clutter parameter |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Fine-tune | Test | Train | Fine-tune | Test | Model complexity | Computation complexity | AI/ML |
| xiaomi (R1-2300571) | 18*256*1 CIR | 2*1 UE coordinates | 2*1 UE coordinates | {0.6，6，2} | {0.4，2，2} | {0.4，2，2} | 70000 | 5000 | 10000 | 21,277,442 | 5.76GFlops | 1.4315 |
| Huawei (R1-2300112) | CIR 18*4*256 | UE coordinates | UE coordinates | {60%, 6m, 2m}, Drop 1 | {40%, 2m, 2m}, Drop 1 | {40%, 2m, 2m}, Drop 1 | 25000 | 1000 | 5000 | 52.42K | 25.81M | 3.1 |

- Exemplary evaluation results of fine-tuning/re-training for network synchronization error

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Fine-tune | Test | Train | Fine-tune | Test | Model complexity | Computation complexity | AI/ML |
| vivo (R1-2300448) | CIR | Pos. | 0% | Sync. 0ns | / | 50ns | 25k | / | 1k | 1.65M | 22.30M | 10.18 |
|  |  |  |  |  | 50ns |  |  | 0.5k |  |  |  | 3.22 |
|  |  |  |  |  |  |  |  | 1k |  |  |  | 2.39 |
|  |  |  |  |  |  |  |  | 2k |  |  |  | 1.73 |
|  |  |  |  |  |  |  |  | 3k |  |  |  | 1.47 |
| Huawei (R1-2300112) | CIR 18*4*256 | UE coordinates | UE coordinates | Without network synchronization error | With network synchronization error @50ns | With network synchronization error @50ns | 25000 | 5000 | 5000 | 52.42K | 25.81M | 8.47 |

- Exemplary evaluation results of fine-tuning/re-training for different InF scenarios

| Sources | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Fine-tune | Test | Train | Fine-tune | Test | Model complexity | Computation complexity | AI/ML |
| vivo (R1-2300448) | CIR | Pos. | 0% | DH | / | HH | 25k | / | 1k | 1.65M | 22.30M | >10 |
|  |  |  |  |  | HH |  |  | 0.5k |  |  |  | 10.50 |
|  |  |  |  |  |  |  |  | 1k |  |  |  | 8.78 |
|  |  |  |  |  |  |  |  | 2k |  |  |  | 5.84 |
|  |  |  |  |  |  |  |  | 3k |  |  |  | 4.66 |
| MediaTek (R1-2301591) | CIR [18,2,256] | UE pos [x,y] | 0% | InF-DH({60%, 6m, 2m}),small hall | InF-SH/HH, large hall | InF-SH/HH, large hall | 0 | / | 3600 | 464.24K | 0.266G | >100 |
|  |  |  |  |  |  |  |  | 1200 |  |  |  | 9.806(InF-SH) 14.073(InF-HH) |
|  |  |  |  |  |  |  |  | 3600 |  |  |  | 5.391(InF-SH) 5.972 (InF-HH) |
|  |  |  |  |  |  |  |  | 7200 |  |  |  | 3.815 (InF-SH) 5.104 (InF-HH) |

Observation 6.15-5

For both direct AI/ML positioning and AI/ML assisted positioning, evaluation results show that with diminishing returns, the positioning accuracy improves as the sample density (e.g., #samples/m2) of the training dataset increases.

- FFS: the range of desirable sample density in training data collection.

Observation 7.8-2

Observation: For AI/ML assisted positioning using multi-TRP construction, evaluation of generalization aspects show that the positioning accuracy deteriorates when the ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario.

- The generalization aspects include:

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

  - Network synchronization error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

  - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

  - Note: for performance evaluation, ideal model switching provides the upper bound of achievable performance when the AI/ML model need to handle different deployment scenarios.

## Proposals for Friday online discussion

Observation 6.12-1

For direct AI/ML positioning, evaluation results demonstrate that for generalization purposes, the training data collection need to obtain data from various deployment scenarios for the training dataset, including at least, data from the target deployment scenario.

Observation 6.12-2

For direct AI/ML positioning, evaluation results demonstrate that with the support of model fine-tuning/re-training, the AI/ML model can adapt to a new or changing deployment scenario, if the existing training dataset and/or trained model(s) cannot adequately cover the new or changing deployment scenario.

Observation 6.12-3

For both direct AI/ML positioning and AI/ML assisted positioning, evaluation results show that with diminishing returns, the positioning accuracy improves as the sample density (e.g., #samples/m2) of the training dataset increases.

- Evaluation results show that the best compromise between sample density and achieved positioning accuracy can be found.

Observation 7.10-1

Observation: For AI/ML assisted positioning using multi-TRP construction, evaluation of generalization aspects show that the positioning accuracy deteriorates when the ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario.

- The generalization aspects include:

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

  - Network synchronization error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

  - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

- Note: ideal model training and model switching may provide the upper bound of achievable performance when the AI/ML model need to handle different deployment scenarios.

# Conclusion

The following agreements were made in RAN1#112 for AI 9.2.4.1.

Agreement

For both direct AI/ML positioning and AI/ML assisted positioning, companies include the evaluation area in their reporting template, assuming the same evaluation area is used for training dataset and test dataset.

Note:

- Baseline evaluation area for InF-DH = 120x60 m.

- if different evaluation areas are used for training dataset and test dataset, they are marked out separately under “Train” and “Test” instead.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m]

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Model complexity | Computation complexity | AI/ML |

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | Test | Model complexity | Computation complexity | AI/ML |

Agreement

The agreement made in RAN1#110 AI 9.2.4.1 is updated by adding additional note:

Note: if complex value is used in modelling process, the number of the model parameters is doubled, which is also applicable for other AIs of AI/ML

Agreement

For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity.

- The type of information to use as model input. The candidates include at least: time-domain CIR, PDP.

- The dimension of model input in terms of NTRP, Nt, and Nt’.

- Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference.

Agreement

For direct AI/ML positioning, study the performance of model monitoring methods, including:

- Label based methods, where ground truth label (or its approximation) is provided for monitoring the accuracy of model output.

- Label-free methods, where model monitoring does not require ground truth label (or its approximation).

Agreement

For AI/ML assisted approach, study the performance of label-free model monitoring methods, which do not require ground truth label (or its approximation) for model monitoring.

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN#96, June 2022.

R1-2300112	Evaluation on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2300141	Evaluation of AI/ML for Positioning Accuracy Enhancement	Ericsson Inc.

R1-2300175	Evaluation on AI positioning enhancement	ZTE

R1-2300284	Evaluation methodology and results on AI/ML for positioning accuracy enhancement	OPPO

R1-2300401	On Evaluation of AI/ML based Positioning	Google

R1-2300448	Evaluation on AI/ML for positioning accuracy enhancement	vivo

R1-2300534	Evaluation on AI/ML for positioning accuracy enhancement	LG Electronics

R1-2300571	Evaluation on AI/ML for positioning accuracy enhancement	xiaomi

R1-2300608	Evaluation of ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

R1-2300675	Evaluation on AI/ML for positioning enhancement	CATT

R1-2300719	Evaluation on AI/ML for positioning accuracy enhancement	China Telecom

R1-2300748	Discussions on evaluation results of AIML positioning accuracy enhancement	Fujitsu

R1-2300845	Some discussions on evaluation on AI-ML for positioning accuracy enhancement	CAICT

R1-2300994	Discussion on evaluation on AI/ML for positioning accuracy enhancement	CMCC

R1-2301101	Evaluation on AI/ML for positioning accuracy enhancement	InterDigital, Inc.

R1-2301182	Evaluation of AI and ML for positioning enhancement	NVIDIA

R1-2301203	Discussion on AI/ML Positioning Evaluations	Lenovo

R1-2301259	Evaluation on AI/ML for Positioning	Samsung

R1-2301341	Evaluation on AI/ML for positioning accuracy enhancement	Apple

R1-2301408	Evaluation on AI/ML for positioning accuracy enhancement	Qualcomm Incorporated

R1-2301591	Evaluation of AIML for Positioning Accuracy Enhancement	MediaTek Inc.

R1-2301690	Evaluation on AI/ML for Positioning Accuracy Enhancement	CEWiT