3GPP TSG RAN WG1 Meeting #112	 R1-2302249

Athens, Greece, February 27th – March 3rd, 2023

Title:	Reply LS to RAN2 on further questions on feMIMO RRC parameters

Response to:	R2-2302295

Release:	Release 17

Work Item:	NR_feMIMO-Core

Source:	RAN WG1

To:	RAN WG2

Cc:	-

Contact Person:

#### Name:	Bo Gao

E-mail Address:	gao.bo1@zte.com.cn

1. Overall Description:

RAN1 would like to thank RAN2 for the question in LS in R2-2302295 on feMIMO RRC parameters and would like to provide the following response.

Question 1

For pathlossReferenceRS-Id, RAN2 considered two possibilities: 1) pathlossReferenceRS-Id is an index referring to a list element in a list configured in the UL BWP and serving cell where the TCI state is applied, regardless if the field cell is configured in IE TCI-State or not, or respectively the field servingCellId in IE TCI-UL-State or not. 2) When the field cell is configured in IE TCI-State(or the field servingCellId in IE TCI-UL-State), pathlossReferenceRS-Id is an index referring to a list element in a list configured in an UL BWP of the serving cell indicated by cell (or servingCellId ). When cell (or servingCellId ) is absent, pathlossReferenceRS-Id is an index referring to a list element in a list configured in the UL BWP and serving cell where the TCI state is applied. In 2), when cell (or servingCellId) ). is absent, it may be necessary to add a new field to indicate in which UL BWP the list is to be found. Question 1 Is 1) or 2) or yet another alternative the correct understanding for pathlossReferenceRS-Id? If it is 2), please indicate how to know the UL BWP when the field cell (or servingCellId ) is present .

Answer to Question 1:

RAN1 confirms that 1) is the correct understanding for pathlossReferenceRS-Id.

Question 2

For ul-powerControl, RAN2 considered two possibilities: 1) ul-powerControl is an index referring to a list element in a list configured in the serving cell where the TCI state is applied, regardless if the field cell (or servingCellId ) is configured or not. 2) When the field cell (or servingCellId ) is configured, ul-powerControl is an index referring to a list element in a list configured in the serving cell indicated by cell (or servingCellId ). When cell (or servingCellId ) is absent, ul-powerControl is an index referring to a list element in a list configured in the serving cell where the TCI state is applied. Question 2 Is 1) or 2) or yet another alternative the correct understanding for ul-powerControl?

Answer to Question 2:

RAN1 confirms that ul-powerControl is an index referring to a list element in a list configured in the serving cell where the TCI state is configured, regardless if the field cell (or servingCellId) is configured or not.

2. Actions:

To RAN2 group.

ACTION: 	RAN1 requests RAN2 to kindly take the above information into account.

3. Date of Next TSG-RAN WG1 Meeting:

TSG-RAN WG1 Meeting #112-bis-e	April 17th – April 26th, 2023		E-meeting

TSG-RAN WG1 Meeting #113 	May 22nd – May 26th, 2023		Incheon, KR