3GPP TSG-RAN WG1 Meeting #112bis-e 	R1-2302382

E-meeting, 17-26 April, 2023

Agenda Item:	9.5.4

Source:	Huawei, HiSilicon

Title:	Discussion on PRS/SRS BW aggregation

Document for:	Discussion and Decision

Introduction

A new WID on positioning was approved in RAN#98-e[1] and it has the following objectives related to bandwidth aggregation, and with RAN1 as the leading group for discussion:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

In this paper, we further provide our views on the PRS BW aggregation and SRS BW aggregation.

Same antenna port assumption

We think that from RAN1 perspective, it should be clear that the aggregated PRS and SRS should be in the same antenna port, according to the following definition of antenna ports in TS 38.211 [2]

### 4.4.1 Antenna ports An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed.

RAN4 may decide from RF perspective to state that they are from the same RF chain, but the current requirement matches RAN1 definition of antenna ports, and it should overrides any other less-stringent requirements such as QCL.

It is also worth noting that in coverage enhancement of TBoMS, the following description with regards to the same antenna port description has already captured in clause 6.2 of TS 38.211.

If DM-RS bundling is applied to PUSCH and/or PUCCH repetitions and/or transport-block processing over multiple slots as described in clause 6.1.7 of [6, 38.214], the UE transmission shall be such that the channel over which a symbol on the antenna port used for uplink transmission is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed if the two symbols are transmitted within the same actual time-domain window.

Therefore, we think aggregated PRS/SRS should be the same antenna port from RAN1 point of view, and it should be captured in the RAN1 specification.

Proposal 1: For the PRS/SRS BW aggregation, the aggregated PRS resources in multiple PFLs and the aggregated SRS resource in multiple carriers should correspond to the same antenna port.

PRS BW aggregation

Common transmission properties for aggregated PRS resources

Agreement To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs

In the RAN1 #112 meeting [1], it was agreed that the aggregated PRS resources shall have same configuration in certain common transmission properties. For example, the aggregated PRS resources shall be transmitted in the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna).

Regarding the TEG between aggregated PRS resources, RAN4 RF already agreed that no TAE requirement will be defined for this, and we do not need to revisit TEG feature [3].

R4-2220544 WF for study on expanded and improved NR positioning Type: other For: Approval Source: Intel Corporation Abstract: [105][100] Main Session; [WF for Approval] Decision: The document was approved. ## TAE for single RF chain architecture Agreement: - No need to discuss TAE for single RF chain architecture - TAE refers to the TAE requirements across carriers in BS specification

Proposal 2: RAN1 should not discuss whether aggregated PRS are from the same TEG or not, as long as they correspond to the same antenna port.

The last meeting agreed the aggregated PRS resources should be transmitted in same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain. The goal of the agreement is to enable the aggregated PRS resources to be transmitted on the same time-domain location so that the UE can jointly process these aggregated PRS resources. Therefore, it is straightforward to configure the aggregated PRS resources with the same periodicity, slot offset, and muting pattern to guarantee the same-time location of aggregated PRS resources. Similarly, the number of PRS resource sets and resources for a TRP should be the same.

As mentioned above, it is ultimately necessary to ensure that the time-domain location of the aggregated resources is the same. And NR-DL-PRS-SFN0-Offset and dl-PRS-ResourceSlotOffset together determine the slot position of the PRS resource. Therefore, from a flexible design point of view, the NR-DL-PRS-SFN0-Offset of the aggregated PRS resources can be different as long as they eventually appear on the same symbol.

Proposal 3: For the PRS BW aggregation, the aggregated PRS resources in multiple PFLs should have the following properties:

The same periodicity and slot offset

The same muting pattern

The same number of PRS resource sets and resources for a TRP

On-demand PRS resource configuration

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

The last meeting agreed to support LMF-initiated/UE-initiated on-demand PRS request and preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations. From RAN1’s perspective, there is nothing need to be further detailed and the signaling detail is up to RAN2.

Proposal 4: The details of LMF-initiated/UE-initiated on-demand PRS request and preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations is up to RAN2.

SRS BW aggregation

RRC state

In the bandwidth aggregation section of the agreed WID, it suggests to specify RRM requirements in RRC_CONNECTED state and RRC_INACTIVE state. Furthermore, SRS transmission for positioning is already supported for UE in RRC_INACTIVE state in Rel-17, where SRS can be transmitted outside the initial UL BWP. Thus, there is no blocking issue to support intra-band contiguous aggregation for SRS resources beyond the UL carrier yet still within the same band in RRC_INACTIVE state.

In addition, allowing SRS BW aggregation in RRC_INACTIVE state can also enable RTT positioning with BW aggregation in RRC_INACTIVE state, since we already agreed that DL measurement via DL-PRS with BW aggregation can be supported in any RRC state.

Therefore, we have the following proposal.

Proposal 5: Support intra-band contiguous SRS bandwidth aggregation for UE both in RRC_CONNECTED state and RRC_INACTIVE state.

In order to support SRS BW aggregation in RRC_INACTIVE state, one or two carriers containing the SRS configuration should be provided in addition to the current “anchor carrier” in the RRCRelease message. In order to identify the carriers, a frequency information for the carrier should be provided as any regular SCell frequency info.

Proposal 6: To support intra-band contiguous SRS bandwidth aggregation for UE in RRC_INACTIVE state, frequency information of one or two additional carriers should be provided by RRCRelease message.

The relationship between UL communication CA and SRS bandwidth aggregation

Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e. SRS outside BWP and across carriers

The last meeting agreed to study the relationship between UL communication CA and SRS bandwidth aggregation. Considering actual application scenarios, some do not require a very high communication rate, but may require very high positioning accuracy, such as AGV navigation, autopilot, etc. Therefore, positioning and communication are independent processes from the perspective of requirements and services, and thus carrier aggregation used for communication and bandwidth aggregation used for positioning should be decoupled so that both UE and network could design the BB/RF tailored to the communication and positioning individually. For example, a UE may not support carrier aggregation for communication, but may support bandwidth aggregation for positioning to reduce the cost of baseband processing and L1/L2 buffering. So, we propose to decouple the positioning bandwidth aggregation with communication carrier aggregation.

For UL-SRS, because the SRS is configured in the BWP of the serving cell, the SRS bandwidth aggregation depends on the communication carrier aggregation. The decoupling of the SRS bandwidth aggregation and communication carrier aggregation include enhancements such as

- A separate UE capability for SRS carrier/BW aggregation should be defined independent from UE communication CA capabilities, e.g. bandwidth class.

- The configuration of SRS BW aggregation should be not limited by the allowed configuration of UE communication CA.

Proposal 7: Support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities.

Proposal 8: Support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA.

Power control

Agreement To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers - In the same slot, in same symbols, from the same antenna, this implies - FFS: The same gNB Rx TEG and the same UE Tx TEG - The same spatial relation - The same startPosition, nrofSymbols - FFS: periodicityAndOffset, and slotOffset - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of SRS resource sets and resources - The same Tx PSD (power per subcarrier) - FFS whether to need the same pathloss RS, Po and alpha - Note: the Tx PSD is not captured in RAN1 specifications - FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones - Phase continuity between aggregated SRS in different carriers Agreement Study potential power control enhancement of simultaneous transmission of SRS for SRS bandwidth aggregation especially in the case when the total uplink transmission power across multiple carriers exceeds P_c,max

To ensure the same Tx PSD across the aggregated carriers, the simpler way is only configuring one set of power control related parameters, i.e. pathloss RS, Po and alpha, for the aggregated SRS resources across the aggregated carriers. It's also beneficial to save signaling overhead.

Proposal 9: For power control of SRS bandwidth aggregation across multiple carriers, support to configure one set of pathloss RS, Po and alpha.

It was agreed to further study the power control mechanism when total uplink transmission power across multiple carriers exceeds the maximum UE transmission power. In this case, the actual power allocation of the aggregated SRS carriers should be scaled down equally to maintain the same PSD without any priority among those carriers.

Proposal 10: The same transmission power reduction of the aggregated SRS carriers should be applied in the case when the total uplink transmission power across multiple carriers exceeds P_c,max.

Conclusion

In this contribution, we provided our views on the solutions to support PRS/SRS bandwidth aggregation. Based on the discussion, we have the following proposals.

Proposal 1: For the PRS/SRS BW aggregation, the aggregated PRS resources in multiple PFLs and the aggregated SRS resource in multiple carriers should correspond to the same antenna port.

Proposal 2: RAN1 should not discuss whether aggregated PRS are from the same TEG or not, as long as they correspond to the same antenna port.

Proposal 3: For the PRS BW aggregation, the aggregated PRS resources in multiple PFLs should have the following properties:

The same periodicity and slot offset

The same muting pattern

The same number of PRS resource sets and resources for a TRP

Proposal 4: The details of LMF-initiated/UE-initiated on-demand PRS request and preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations is up to RAN2.

Proposal 5: Support intra-band contiguous SRS bandwidth aggregation for UE both in RRC_CONNECTED state and RRC_INACTIVE state.

Proposal 6: To support intra-band contiguous SRS bandwidth aggregation for UE in RRC_INACTIVE state, frequency information of one or two additional carriers should be provided by RRCRelease message.

Proposal 7: Support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities.

Proposal 8: Support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA.

Proposal 9: For power control of SRS bandwidth aggregation across multiple carriers, support to configure one set of pathloss RS, Po and alpha.

Proposal 10: The same transmission power reduction of the aggregated SRS carriers should be applied in the case when the total uplink transmission power across multiple carriers exceeds P_c,max.

- Reference

- RAN1, Chair’s Notes RAN1#110 v20, RAN1#112, Athens, Greece, 27th February – 3rd March, 2023.

- TS 38.211, Physical channels and modulation (Release 17).

- R4-2300001, Meeting Report for TSG RAN WG4 meeting: 105, RAN4#105, Toulouse, FRANCE, 14th – 18th November, 2022