3GPP TSG RAN WG1 #112bis                                                   R1-2302546

e-Meeting, April 17th – April 26th, 2023

Source:	OPPO

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

# Introduction

The NR duplex evaluation assumptions, including deployment scenarios, channel model, interference model, traffic model and performance metrics, are almost finalized in the past meetings. In this contribution, we provide some initial evaluation results, with the adopted simulation assumptions given in Appendix.

# Evaluation results

Some key assumptions used in evaluation include:

- The channel bandwidth is 100MHz (273RBs at SCS=30kHz) and SBFD pattern in frequency domain is {DUD} with < ND, NU, NG > = <104, 55, 5>.

- Four slot patterns {DDDSU, XXXXU, XXXXX, DXXXU} are evaluated for low traffic load and medium traffic load, respectively.

- The FTP3 traffic arrival model parameter is determined with Type-2 RU in legacy TDD (DDDSU) case. The traffic packets generation for legacy TDD case and SBFD cases follow the same random seed for generations of traffic arrival times, as well as the same fixed packet size under the given load condition.

- UE-UE co-channel inter-subband CLI, gNB self-interference, co-site inter-sector co-channel inter-subband CLI are modeled, following the CLI modeling details described in Appendix.

More information on general SLS assumptions can be found in Table A-1 in Appendix.

## Throughput and UPT Analysis

Figure 1 Cell-wise DL/UL Throughput

Figure 1 shows that the average cell-wise DL/UL throughputs are similar among the four slot patterns {DDDSU, XXXXU, XXXXX, DXXXU} for a low load case for Indoor office, Urban Macro and Dense Urban Macro scenario. For medium load case, slot pattern XXXXX can make the largest cell-wise throughput shift between DL and UL. One reason is the UE can transmit only UL or only DL in “X” slot, and some UEs are always selected to transmit UL traffic. Edge UEs are found to be mostly impacted under the medium load.

Figure 2 shows that, for UE UPT,

- The setup of SBFD does not change the tendency that the UE DL/UL UPT decreases as traffic load increases.

- If traffic load is low or medium, XXXXX results in the highest UPT among four slot patterns for both DL and UL. This is because the UL subband in each of “X” slot is large enough for small packet size and moderate-to-low traffic load, and the same is for DL subbands in each of “X” slot. It is also observed that the UL UPT’s of XXXXU, XXXXX and DXXXU are far better than that of DDDSU.

Figure 2 UE DL/UL UPT

Observation 1: The setup of UL subband over DL symbols would not have big impact to the average cell throughput under the assumed traffic loads.

Observation 2: The setup of UL subband over DL symbols improves the UL UPT per UE.

Observation 3: With the assumed D/U resource ratio in “X” slot, if the packet size is small (4Kbytes for DL and 1Kbytes for UL) and the traffic load is low-to-medium, {XXXXX, XXXXU, DXXXU} offer better DL/UL UPT than DDDSU in Indoor office, which is however just partially observed in the other two evaluated scenarios.

## RU analysis

Figure 3 Type-2 RU result

Figure 3 shows that,

- XXXXX offers the highest Type-2 UL RU among four slot patterns for both medium and low load cases.

- One of {XXXXU, DXXXU} offers highest Type-2 DL RU among four slot patterns for both medium and low load case. To be more specific, XXXXU wins for Indoor office and Dense Urban Macro scenarios at the cost of giving lowest Type-2 UL RU, while DXXXU wins for Urban Macro scenario.

- The distributions of Type-2 DL RU and Type-2 UL RU are respectively similar between Indoor office scenario and Dense Urban Macro scenario. The difference between these two scenarios and Urban Macro scenario comes from the edge UE, given the coupling loss for edge UEs in Urban Macro is much bigger than in Indoor office and Dense Urban Macro.

Although our evaluations cover low-to-medium traffic loads, in these evaluations we already observed the numerous occurrences that, at the time a UE is picked by scheduler for scheduling in a slot, the simulation already holds both DL to-be-transmitted traffic and UL to-be-transmitted traffic for the UE. Then the scheduler has to prioritize between DL and UL given the UE has to be in half-duplex mode. We expect this kind of DL-UL collision to occur more often in the high traffic load case. This brings a question whether the performance results corresponding to the following two UE distribution assumptions are comparable:

Agreement For any deployment cases where clustering is not used and where M UEs are distributed per direction, - If each UE is either assigned UL traffic or DL traffic (i.e., option 1 of traffic model), there are 2M UEs, wherein, M UEs are assigned with UL traffic, and the other M UEs are assigned with DL traffic. - If each UE is assigned both UL traffic and DL traffic (i.e., option 2 of traffic model), there are M UEs.

The key difference between the two options is that the first option completely removes the half-duplex restriction from the evaluation, which somehow translate the 2M half-duplex UEs into M full-duplex UEs under a 2-to-1 UE-pairing without taking into account the UE locations. This duplex relaxation behind the first option does not match any real field operation in the long run. We think the evaluation results corresponding to these two UE distribution options should not be mixed together to derive study conclusions.

Proposal 1: RAN1 examines the comparability of evaluation results corresponding to the two agreed UE distribution options (i.e., M DL-only UEs and M UL-only UEs vs. M dual-direction UEs) and, if necessary, summarizes the evaluation results corresponding to these two options separately.

# Conclusion

In this contribution, we show our views on evaluation on NR duplex evolution with following proposals:

Observation 1: The setup of UL subband over DL symbols would not have big impact to the average cell throughput under the assumed traffic loads.

Observation 2: The setup of UL subband over DL symbols improves the UL UPT per UE.

Observation 3: With the assumed D/U resource ratio in “X” slot, if the packet size is small (4Kbytes for DL and 1Kbytes for UL) and the traffic load is low-to-medium, {XXXXX, XXXXU, DXXXU} offer better DL/UL UPT than DDDSU in Indoor office, which is however just partially observed in the other two evaluated scenarios.

Proposal 1: RAN1 examines the comparability of evaluation results corresponding to the two agreed UE distribution options (i.e., M DL-only UEs and M UL-only UEs vs. M dual-direction UEs) and, if necessary, summarizes the evaluation results corresponding to these two options separately.

# Reference

- RP-213591 New SI: Study on evolution of NR duplex operation, CMCC

- R1-2208856 Discussion on evaluation on NR duplex evolution, OPPO

- R1-2300286 Discussion on evaluation on NR duplex evolution, OPPO

# Appendix

## Simulation parameters

Table A-1 simulation parameters for SBFD

| Parameters | Value |  |  |
| --- | --- | --- | --- |
|  | Indoor office | Urban Macro | Dense Urban Macro |
| Scenario | InH(2*6 site) | 7 cells, 3 sectors/cell |  |
| Inter-BS distance | 20m | 500m | 200m |
| Carrier frequency | 4GHz |  |  |
| System bandwidth | 100MHz |  |  |
| SCS | 30KHz |  |  |
| TDD pattern | DDDSU / XXXXU / XXXXX |  |  |
| SBFD pattern | {DUD}, < ND, NU, NG > = <104, 55, 5> |  |  |
| BS Antenna Configuration | (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) (dH, dV) = (0.5λ, 0.5λ) | (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;2,8) (dH, dV) = (0.5λ, 0.5λ) |  |
| UE Antenna Configuration | 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  |  |
| Transmit Power | DL: 24dBm UL: 23dbm | DL: 53dBm UL: 23dbm |  |
| Antenna Height | 3 m for BS and 1.5 m for UE | 25 m for BS and 1.5 m for UE | 25 m for BS and 1.5 m for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |  |  |
| UE speed | 3km/h |  |  |
| Scheduling Algorithm | SU-MIMO + PF |  |  |
| Power control for PUSCH | P0=-60dbm, alpha=0.6 | P0=-80dbm, alpha=0.8 |  |

## UE-UE co-channel inter-subband CLI

IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB

where

- IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value).

- PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value)

- CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE

- NULRB is the total number of UL RBs in the UL subband

- 10*log10(ICSUE)=33dBc

- 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein,

  - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored

  - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1

  - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'.

  - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1.

    - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS

    - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS

  - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs)

  - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs.

## gNB self-interference

ISIper-RB=Ptxper-RBαSI*Nused-DL-RBNDLRB

    - Ptxper-RB is DL transmission power of gNB per RB, Ptxper-RB=Ptxmax/NDLRB.

    - Nused-DL-RB is the number of DL RBs allocated for DL transmission.

    - αSI=PtxmaxISImax*NDLRBNULRB

      - with sensitivity degradation of 1dB, ISImax can be computed based on 10*log10ISImax+NN=1dB, where N is the noise floor over the UL subband given by N(dB)=-174 + 10*log10 (20e6) + 5 = -96 dBm, assuming 20MHz UL subband and 5dB noise figure.

## co-site inter-sector co-channel inter-subband CLI

Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB

Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB

- Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB.

- Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale).

- NDLRB is the total number of DL RBs in the DL subbands.

- Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x.

- αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI.

  - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

    - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment.

  - 10*log10ACLRBS=45dB

  - 10*log10ACSBS=46dB

  - spatial isolationdB=75dB