3GPP TSG RAN WG1 #112bis-e		                                                          R1-2302673

e-Meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	Discussion on 32 HARQ process in PDSCH-HARQ-ACK-EnhType3 configuration for the features extending NR operation to 71 GHz

Agenda Item:	7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the corresponding changes needed in TS38.331 in order to support 32 HARQ processes. It is suggested that the maximum number of HARQ processes in PDSCH-HARQ-ACK-EnhType3-r17 should support 32, subject to UE capability.

- Discussion

In RAN1 WG discussion, the following agreement has been reached:

Agreement: For NR FR2-2 at least for 480/960 kHz SCS, support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability. - Note: Up to 32 maximal supported HARQ process number is already agreed in Rel-17 NTN WI. - Working assumption: The same solution to support up to 32 HARQ process number in Rel-17 NTN WI is reused for NR FR2-2.

However, the required changes to support 32 HARQ processes in PDSCH-HARQ-ACK-EnhType3-r17 configuration in TS 38.331 have not been implemented (corresponding text is marked in green as in the following):

PDSCH-HARQ-ACK-EnhType3-r17 ::= SEQUENCE { pdsch-HARQ-ACK-EnhType3Index-r17 PDSCH-HARQ-ACK-EnhType3Index-r17, applicable-r17 CHOICE { perCC SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1), perHARQ SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (16)) }, pdsch-HARQ-ACK-EnhType3NDI-r17 ENUMERATED {true} OPTIONAL, -- Need R pdsch-HARQ-ACK-EnhType3CBG-r17 ENUMERATED {true} OPTIONAL, -- Need S ...

Proposal 1: In order to support 32 HARQ processes, there is a need to update the above corresponding text in TS38.331. It is suggested to send RAN2 LS indicating such correction, which is marked in green as in the following:

PDSCH-HARQ-ACK-EnhType3-r17 ::= SEQUENCE { pdsch-HARQ-ACK-EnhType3Index-r17 PDSCH-HARQ-ACK-EnhType3Index-r17, applicable-r17 CHOICE { perCC SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1), perHARQ SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (32)) }, pdsch-HARQ-ACK-EnhType3NDI-r17 ENUMERATED {true} OPTIONAL, -- Need R pdsch-HARQ-ACK-EnhType3CBG-r17 ENUMERATED {true} OPTIONAL, -- Need S …

- Conclusion

In this contribution, we discuss the corresponding changes needed in TS38.331 in association to support 32 HARQ processes for PDSCH-HARQ-ACK-EnhType3-r17 and one proposal is given:

Proposal 1: In order to support 32 HARQ processes, there is a need to update the above corresponding portion in TS38.331. It is suggested to send RAN2 LS indicating such correction which is marked in green as in the following:

PDSCH-HARQ-ACK-EnhType3-r17 ::= SEQUENCE { pdsch-HARQ-ACK-EnhType3Index-r17 PDSCH-HARQ-ACK-EnhType3Index-r17, applicable-r17 CHOICE { perCC SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1), perHARQ SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (32)) }, pdsch-HARQ-ACK-EnhType3NDI-r17 ENUMERATED {true} OPTIONAL, -- Need R pdsch-HARQ-ACK-EnhType3CBG-r17 ENUMERATED {true} OPTIONAL, -- Need S ...

- References

3GPP TS 38.331 V17.4.0, “NR; Radio Resource Control (RRC) protocol specification”