3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302911

e-Meeting, April 17th – April 26th, 2023

Source:	Fujitsu

Title:	Discussion on channel access mechanism for SL-U

Agenda item:	9.4.1.1

Document for:	Discussion and Decision

Introduction

For Rel-18 sidelink evolution, sidelink on unlicensed band (SL-U) will be standardized. In the WID, the following objective for SL-U has been captured [1].

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102).

Based on the objective, the required changes to support SL-U are mainly on the channel access and the physical channel structure. In the RAN1#112 meeting, some agreements on those aspects have been achieved. In this contribution, we share some further discussions on the part of channel access for SL-U.

Discussion

UE-A can share a COT initiated by UE-B when some conditions are met. For example, one condition is related to the CAPC values of UE-A and UE-B. If the CAPC value of UE-A is equal to or smaller than that of UE-B, UE-A can share the COT initiated by UE-B. In our view, the RSRP or distance between UE-A and UE-B can be considered as an additional condition. The rationale is that UE-to-UE COT sharing is mainly based on the assumption that UE-A and UE-B have the same LBT results. If UE-A is relatively close to UE-B, it is expected that UE-A and UE-B will have the same LBT results. In this case, it would be safer to let UE-A share the COT initiated by UE-B. Therefore, the RSRP or distance between UE-A and UE-B can be leveraged to determine whether to share a COT. If the RSRP is larger than a threshold, or the distance is smaller than a threshold, UE-A can share the COT indicated by UE-B.

For COT sharing, the RSRP or distance between UE-A and UE-B should be considered to determine whether UE-A can share a COT initiated by UE-B.

Figure 1. An example of receiving two COT indicators transmitted by UE1 and UE2.

In the RAN1#112 meeting, the following agreements have been achieved for COT sharing [2].

Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) FFS: all other details and additional restrictions

According to the agreements, COT sharing is supported for unicast, groupcast and broadcast. Generally, a COT sharing UE (UE-A) and a COT initiating UE (UE-B) should be the target receivers of each other. However, for groupcast or broadcast, one UE-A can be the target receiver of multiple UE-Bs, and multiple UE-Bs can be the target receivers of one UE-A. In this case, UE-A may acquire more than one COT, and need to determine which COT to share. An example is shown in Figure 1. For UE1 and UE2, they have initiated COT#1 and COT#2, and thus transmit two COT indicators respectively. For UE-A, it receives two COT indicators from two UE-Bs (UE1 and UE2), and the COT indicators indicate COT#1 and COT#2 respectively. For these two COTs, the COT durations can be different, and the occupied slots within the COTs can be also different. Therefore, UE-A needs to determine whether to share COT#1 or COT#2. To this end, how to determine which COT to share should be further studied.

For COT sharing, it should be studied how to determine which COT to share if more than one COT is identified by a COT sharing UE.

In the RAN1#111 meeting, the following agreement has been achieved for CPE [3]. In the following, we share some further discussions on some FFS points.

Agreement - A CPE is transmitted from a CPE starting position before SL transmission within a COT, select one or both of the two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively - FFS: whether Option 1 and Option 2 are both applicable and the conditions (e.g., Option 1 in case of COT sharing and Option 2 in case of initiating a COT) - FFS: which channel access type(s) is applicable for option 1 and option 2 - FFS: other details - A single CPE starting position for PSFCH - FFS CPE starting position and whether it should be (pre-)configured in each RP, pre-defined or indicated - FFS other details (e.g., indication granularity) - Note: value 0 is a candidate - At least one CPE starting position for S-SSB - FFS CPE starting position should be (pre-)configured, pre-defined or indicated - FFS: Whether multiple CPE starting positions should be (pre-)configured, pre-defined or indicated - FFS CPE starting positions for the R16 S-SSB and the additional S-SSBs - Note: value 0 is a candidate - One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH - When multiple CPE starting positions are (pre-)configured, - FFS whether/how to define a criteria for selecting a default CPE starting position (e.g., according to partial/full RB set allocation, resource reservation information, within or outside of a COT, etc.) - FFS criteria for selecting one of the multiple CPE starting positions (e.g., according to priority level (e.g., CAPC or L1), selected randomly by UE from the (pre-)configured set of CPEs, selected by the UE based on channel access result, determined based on indication from the COT initiating UE, etc.) - FFS other details

Figure 2: An example of CPE alignment via UE adjusting its CPE.

According to the agreement, it has been agreed to support multiple CPE starting positions in a resource pool. On top of it, one of the multiple CPE starting positions is determined and used for transmission, and this part is still FFS. In NR-U, generally the CPE starting position is determined based on the priority. In SL-U, this basic principle should be followed. Additionally, it would be beneficial if a UE can adjust its CPE starting position to align with that of another UE. An example is shown in Figure 2. If determining the CPE starting position based on the priority, UE#1, UE#2 and UE#3 will have CPE#1, CPE#2 and CPE#3 respectively, as shown in the left part of the figure. If these UEs would be multiplexed in a FDM manner, there will be inter-UE blocking due to LBT. For example, a UE with an earlier CPE starting position will occupy the channel first and thus block the access of the other FDMed UEs. To address this, adjustments to the CPE starting position can be considered. Based on SCI decoding, each UE can know the reserved resources and the CPE starting positions of the other UEs. For example, the CPE starting position of a UE is determined by the priority indicated by the SCI transmitted by the UE. To achieve the FDM multiplexing, UE#1 and UE#2 can adjust their CPE starting positions to align with UE#3’s CPE starting position, i.e., the earliest CPE starting position. By this kind of CPE alignment, the FDM multiplexing among UE#1, UE#2 and UE#3 can be achieved.

A first UE can align its CPE starting position with that of a second UE to achieve FDM multiplexing with the second UE.

- The CPE starting position is determined by the priority.

- By decoding SCI of the second UE, the first UE can obtain the priority and thus obtain the CPE starting position of the second UE.

- The first UE can adjust its CPE starting position to align with the CPE starting position of the second UE.

It has been agreed to support multi-consecutive slots transmission (MCSt). By this way, the UE can avoid performing too much LBT and compete fairly with other devices from the coexisting systems. When consecutive slots are used for transmission, the gap (such as the guard symbol) between the slots can be omitted so that no additional LBT is needed. In other words, the guard symbol between the slots can be also used for transmission.

For multi-consecutive slots transmission (MCSt), it should be considered to support that the guard symbol between the slots is used for transmission.

Figure 3: An example of triggering resource selection (Resource #n1) when a COT is obtained.

It has been agreed that the existing mode 2 resource selection will used as the baseline, and it is FFS whether any enhancement is needed. In our view, mode 2 is not designed for sharing an unlicensed spectrum. Therefore, mode 2 may not be a strong competitor in terms of competing for the unlicensed spectrum or achieving an efficient channel access. Potentially, the enhancements on mode 2 can be considered. For example, it can be considered that resource selection is triggered once a COT is obtained. An example is given in Figure 3. If a UE has successfully initiated a COT or shared a COT initiated by another UE, it is triggered to perform resource selection (Resource#n1). The rationale is that the UE can quickly gain access to the channel once there is an opportunity.

For potential mode 2 enhancements, it should be further studied whether resource selection can be triggered when a COT is obtained.

Figure 4: An example of HARQ-ACK retransmission on PSFCH.

In the RAN1#112 meeting, the following agreement has been achieved [2].

Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

According to the agreement, one PSCCH/PSSCH transmission can be associated with more than one PSFCH occasion and the details are still FFS. The purpose is to address PSFCH dropping due to LBT failure. Based on this physical layer structure, the functionality of SL HARQ-ACK retransmission can be further considered for SL-U. In NR-U, HARQ-ACK retransmission has been supported. More specifically, a UE can be indicated to not transmit HARQ-ACK temporarily, but to retransmit HARQ-ACK together with another HARQ-ACK later. This functionality can be also introduced to SL-U. Following the design principle in NR-U, an example of SL HARQ-ACK retransmission is shown in Figure 4. Wherein, HARQ-ACK1 for PSSCH1 is not transmitted on PSFCH1. The reason could be that PSFCH1 is subject to LBT failure, or that PSFCH1 is outside of a COT. However, HARQ-ACK1 has a second chance to be transmitted on PSFCH2. Also, HARQ-ACK1 can be transmitted on PSFCH2 together with HARQ-ACK2.

Considering PSFCH may endure LBT failure or be outside of a COT, SL HARQ-ACK retransmission should be supported for SL-U where an earlier HARQ-ACK1 can be transmitted together with a HARQ-ACK2 later.

Conclusion

In this contribution, we have expressed our views on unlicensed channel access for SL-U. The proposals are summarized as follows.

- For COT sharing, the RSRP or distance between UE-A and UE-B should be considered to determine whether UE-A can share a COT initiated by UE-B.

For COT sharing, it should be studied how to determine which COT to share if more than one COT is identified by a COT sharing UE.

A first UE can align its CPE starting position with that of a second UE to achieve FDM multiplexing with the second UE.

- The CPE starting position is determined by the priority.

- By decoding SCI of the second UE, the first UE can obtain the priority and thus obtain the CPE starting position of the second UE.

- The first UE can adjust its CPE starting position to align with the CPE starting position of the second UE.

For multi-consecutive slots transmission (MCSt), it should be considered to support that the guard symbol between the slots is used for transmission.

For potential mode 2 enhancements, it should be further studied whether resource selection can be triggered when a COT is obtained.

Considering PSFCH may endure LBT failure or be outside of a COT, SL HARQ-ACK retransmission should be supported for SL-U where an earlier HARQ-ACK1 can be transmitted together with a HARQ-ACK2 later.

Reference

- RP-230077, NR sidelink evolution, RAN#99, March 20-23, 2023.

- Chair’s notes, RAN1#112, February 27 – March 3, 2023.

- Chair’s notes, RAN1#111, November 14 – 18, 2022.