3GPP TSG RAN WG1 Meeting #112bis-e	R1-2303775

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.2  Measurements and reporting for SL positioning

Source:	Locaila, Inc

Title:	Solution for Carrier Phase based RTT Positioning

Document for:	Discussion & Decision

# Introduction

In the last 111th meeting, participating companies in TR38.859 presented simulation results showing the superior accuracy of the Carrier Phase Positioning method. The results showed that the carrier phase positioning can achieve accuracy within a centimeter range. Applying the technology to V2X measurements is expected to provide the high precision required by the vehicle industry, even with a regulatory bandwidth of only tens of MHz in the FR1 range.

This contribution presents one method of applying Carrier Phase technology to measure the relative distance between vehicles by utilizing the phase of the reciprocal signal. This approach is similar to RTT (Return Trip Time) but we propose to use the new terminology RTP (Return Trip Phase) as it calculates the signal phase in the frequency domain to determine the distance and latency.

We present detailed mathematical solutions, such as the double-side RTP (i.e. three-way hand-shaking) method, which was not introduced in the previous contribution. Also, note that in our companion document R1-2303774, we introduced additional solutions such as PRS phase synchronization on the RSU side.

(Proposal 1) Introduce new terminology RTP (Return Trip Phase) to avoid any confusion with RTT when discussing the frequency domain method such as the carrier phase positioning.

# Vector representation of PRS carrier phase

Figure 1 below illustrates a single/double-sided RTP method in which the target UE-B communicates a returning signal with anchor UE-A.

Figure 1. Single/Double-sided RTP method between two vehicles

In Figure 1, the internal clock signal of anchor UE-A and target UE-B are generated by unsynchronized local oscillators, which results in their local time being represented as (t-ψA) and (t-ψB) respectively. While the local clock errors ψA and ψB can be changed slightly over time, for convenience, instantaneous time errors are assumed to be fixed constants.

The RTP procedure begins with anchor UE-A periodically broadcasting the PRS signal generated by Equation (1) below. This PRS signal is the same reference signal used in methods such as TDoA or AoA.

(1)

PRSAt=A1ejω1t-ψA+A2ejω2t-ψA+…+Amejωmt-ψA=q=1mAqejωqt-ψA

In Equation (1), w1, w2, … wm represent the angular frequencies of subcarriers that are equally spaced in the frequency domain. The legacy comb-N structure can also be viewed as a form of equal spacing of subcarriers. However, in this contribution, the PRS waveform structure proposed in [6][8] is preferred, rather than the staggered comb-N arrangement. A1, A2, … Am represent the initial phase sequence of each subcarrier that makes up the PRS signal. This initial phase sequence can be a Gold sequence and it can be removed if the receiver knows this sequence.

The PRS signal can be transmitted periodically at k times of a slot interval. If the initial phase sequences A1, A2, … Am for each iteration cycle remain unchanged, we can see that the same PRS waveform repeats every k times the slot length L, as shown in Equation (2).

(2)

PRSAt=PRSAt+kL ,   L=slot length

In the above (2), L denotes the length of a slot, and k is the PRS iteration cycle. The implication is that the phase value of the PRS signal measured at the time (t+kL) for any k value is equal to the phase value measured at time t. In other words, when applying the phase-based measurement method, which operates on a slot basis, all signals measured at different slot times can be considered to be measured at the same time t, hence the phase comparison can be made regardless of the slot location. Therefore, relative time can be measured more flexibly and conveniently than the RTT method.

The baseband PRS signal in Equation (1) above is mixed with the carrier signal of frequency wc at the RF frontend of the transmitter, and modulated into the passband signal in Equation (3) below.

(3)

PRSApasst=ejωct-ψA∙PRSAt=q=1mAqejωc+ωqt-ψA

In the above equation (3), since the local timing error of A is ψA, the same timing offset is applied to the carrier signal and multiplied by (t-ψA).

The passband signal in (3) above reaches target UE-B after the tAB delay. This can be represented by Equation (4) below. To simplify the description, equation (4) does not include a multipath channel expression, but just shows a single transmission delay on a line-of-sight path.

(4)

PRSAcht=e-jωc+ωqτAB∙PRSApasst=q=1mAqejωc+ωqt-ψA-τAB

Looking closely at Equation (4) above, it can be seen that the PRS signal received from UE-B has a signal phase rotated by the propagation delay tAB between A-B. The receiver can measure this phase rotation to obtain propagation delay and distance information.

The target UE-B multiplies the received passband PRS signal by the inverse carrier frequency to down-conversion. In Equation (5) below, the local clock error of UE-B is ψB, so the carrier signal is multiplied by (t-ψB).

(5)

yBt=e-jωct-ψB∙PRSAcht=qAqejωcψB-ψA-τAB+ωqt-ψA-τAB

Target UE-B collects the down-converted baseband signal samples of Equation (5) above and decomposes them using the FFT process. Hence the PRS subcarriers of w1, w2, … wm is recovered. The recovered PRS subcarriers are shown in the vector form of Equation (6).

(6)

YB=N∙A1ejωc+ω1ψB-ψA-τABA2ejωc+ω2ψB-ψA-τAB⋮Amejωc+ωmψB-ψA-τAB+σB

The carrier phase vector YB in Equation (6) above contains both local clock error (𝜓B - 𝜓A) and path delay tAB in the exponent part of the complex number.  σB represents the residual error that remains after the FFT computation.

Note that the carrier phase vector YB includes the initial phase sequence A1, A2, … Am of the PRS signal sent by anchor UE-A. It can be removed if the UE-B knows the sequence.

The Target UE-B rotates the PRSB subcarrier by the amount of received PRSA signal phase and returns it to anchor UE-A.  This can be understood conceptually as the UE-B copies and returns the received PRSA signal phase.  This phase-rotated PRSB signal is referred to herein as the UL_RTPB signal.

The UL_RTPB signal can be expressed in the form of an inner product of the vector (6) with the PRSB vector of UE-B, as shown in Equation (7) below.

(7)

UL_RTPBt=ejωc+ω1ψB-ψA-τABejωc+ω2ψB-ψA-τAB⋮ejωc+ωmψB-ψA-τABT∙B1ejω1t-ψBB2ejω2t-ψB⋮Bmejω3t-ψB

=q=1mBqejωqt-ψA-τBA+ωcψB-ψA-τAB

The UL-RTPB reference signal whose phase is rotated in Equation (7) above is mixed with the carrier frequency wc as shown in Equation (8) below to up-convert to a passband signal at the RF terminal of UE-B. Also, the same local timing offset is applied to the carrier signal and multiplied by (t-ψB).

(8)

UL_RTPBpasst=ejωct-ψB∙q=1mBqejωqt-ψA-τAB+ωcψB-ψA-τAB

=q=1mBqejωc+ωqt-ψA-τAB

If the up-down channel between A and B is the same (i.e reciprocal), the UL_RTPB signal arriving at the anchor UE-A after time tAB can be expressed as (9) below. Again, to simplify the description, equation (9) does not include a multipath channel expression, but just shows a single transmission delay on a line-of-sight path.

(9)

UL_RTPBcht=e-jωc+ωqτAB∙UL_RTPBpasst

=e-jωc+ωqτAB∙qBqejωc+ωqt-ψA-τAB=q=1mBqejωc+ωqt-ψA-2τAB

Anchor UE-A down-converts the received passband UL-RTPB signal. In Equation (10) below, since the local clock error of UE-A is ψA, the carrier signal is multiplied by (t-ψA) with the same timing offset applied.

(10)

yAt=e-jωct-ψA∙UL_RTPBcht=qBqejωc-2τAB+ωqt-ψA-2τAB

The subcarrier vector YA in Equation (11) below is obtained by collecting the demodulated baseband signal samples of Equation (10), applying the FFT, and extracting m subcarrier elements in the frequency domain.

Similar to the case of the PRS reference signal, if the anchor UE-A knows the initial phase sequence B1, B2, … Bm of the UL-RTPB signal, it can be removed.

(11)

YA=N∙B1e-jωc+ω12τABB2e-jωc+ω22τAB⋮Bme-jωc+ωm2τAB+σA

After removing the initial phase sequence value in the vector YA, the carrier phase value is calculated by summing the complex value of each element and applying the arc-tangent function (i.e. angle function) as  (12) below.

(12)

argsumYA≅arge-jωc+ω∆2τAB=-ωc+ω∆2τAB-2πNcA

In above Equation (12), wc is the carrier frequency and wΔ is the median of the subcarriers w1, w2, … wm.  (wc + wΔ) is a very large number (>108), but the arg(..) function returns only values between the range -π ~ +π. To fully recover the lost carrier phase value (i.e cycle number), it is necessary to estimate and compensate for the unknown integer number NcA. This is a well-known Integer Ambiguity problem [5].

# Integer number resolution using subcarrier phase

In the OFDM method, the unknown integer of the carrier phase can be determined and calculated simply by using the relationship between the carrier and subcarrier wavelengths. [7][8] proposed a method of estimating an integer cycle number using a subcarrier wavelength that is much longer than the carrier wavelength.

If the UE receiving the PRS signal extracts an m-length carrier phase vector as shown in Equation (6) above through FFT decomposition, the lower (m-1) and upper (m-1) elements of the vector are separated from this. Then, the vectors are conjugated element by element, and the subcarrier phase vector of the longest wavelength can be obtained.

For example, Equation (13) below is a subcarrier phase vector of (m-1) length calculated using the carrier phase vector in Equation (11) above.

(13)

YA+1YA-1*=N∙e-jωc+ω22τABe-jωc+ω32τAB⋮e-jωc+ωm2τAB⨁N∙ejωc+ω12τABejωc+ω22τAB⋮ejωc+ωm-12τAB

=N2∙e-jω2-ω12τABe-jω3-ω22τAB⋮e-jωm-ωm-12τAB=N2∙e-2jωgτABe-2jωgτAB⋮e-2jωgτAB

In Equation (13) above, YA+1 means the lower (m-1) length subvector of Equation (11), and Y*A-1 means the complex conjugate of the upper (m-1) length subvector.

The carrier phase vector in Equation (11) appears in the form of a product of the carrier frequency (wc + wΔ), but in the subcarrier vector in Equation (13) above, wc is deleted, and the gap frequency of the subcarrier, wg (=wm–wm-1 ) in multiplicative form. This gap frequency wg corresponds to (subcarrier space x N) Hz, which is the subcarrier arrangement interval of com-N PRS. Therefore, determining wg Hz, the subcarrier allocation gap of the PRS reference signal, is an important factor in designing the carrier phase system.

Since all of the (m-1) subcarrier vector elements in Equation (13) have the same result value, the phase measurement value for the gap frequency wg Hz subcarrier can be obtained by applying the arg(…) function by complex summing them all. .

(14)

argsumYA+1YA-1*≅arge-2jωgτAB=-2ωgτAB

∴ τAB=-argsumYA+1YA-1*2ωg

Equation (14) above is the subcarrier phase measured using a long subcarrier with a wavelength of (2𝜋𝕔/wg)  meter, where 𝕔 is the light constant.

If the distance from Anchor UE-A to Target UE-B is less than 1/2 of this subcarrier wavelength, the above subcarrier phase measurement result is always uniquely determined within the range of -𝜋 ~ +𝜋. Therefore, unlike the carrier phase equation in Equation (12), the value of tAB can be determined without integer estimation.

The phase measurement resolution of the above subcarrier phase difference equation (14) is generally no better than that of the carrier phase equation of (12). However, it may be sufficient for estimating an unknown number. Converting Equation (14) above into an equation for the tAB term, and substituting it into Equation (12), and rearranging for the unknown number NcA, the solution to the unknown integer of Equation (15) below can be obtained.

(15)

NcA=roundωc+ω∆ωgargsumYA+1YA-1*2π-argsumYA2π

In the integer ambiguity solution (15) above, round(…) is the integer rounding function.

Finally, the estimated integer number NcA can be applied to (12) and rearranged to calculate the tAB  value of latency in Equation (16) below.

(16)

τAB=argsumYA+2πNcAωc+ω∆

Anchor UE-A may further implement a double-sided RTP procedure by encoding the calculated time delay information tAB into a DL-RTPA signal and delivering it to target UE-B.

Figure 2 below illustrates a timing diagram of the single-sided RTP procedure between anchor UE-A and target UE-B.

Figure. 2 Timing  Diagram of Sing-Sided RTP Process Between Anchor UE and  Target UE

In Figure 2 above, the PRSA signal transmitted by anchor UE-A reaches target UE-B after the tAB time delay.  target UE-B calculates the phase information  (𝜓B - 𝜓A - tAB)   of  the PRSA signal and uses it to configure the UL-RTPB reference signal.  The UL-RTPB signal is delivered to anchor UE-A. By applying equations (11) through (16),  anchor UE-A can finally obtain a time delay tAB that eliminates the local clock error.

# Conclusion

This contribution briefly shows that the carrier phase method can successfully be applied for the sidelink positioning and achieves high accuracy path delay by removing the initial clock error.

The RTP procedure introduced herein is only one example to apply the carrier phase method to the V2X environment. There are other carrier phase applications, and solutions to explore for other use cases and environments such as V2V, V2I location finding, synchronization between Nodes, and for direction finding as with AoA, AoD, etc.

Based on this discussion, we suggest studying further on the feasibility of the carrier phase positioning for sidelink applications.

(Proposal 3) RAN1 should study the feasibility of carrier phase-based positioning for application to sidelink use cases.

## Proposals

(Proposal 1) Introduce new terminology RTP (Return Trip Phase) to avoid any confusion with RTT when discussing the frequency domain method such as the carrier phase positioning.

(Proposal 2) Study new PRS signal efficiency for phase-based sidelink positioning methods such as carrier phase positioning.

(Proposal 3) RAN1 should study the feasibility of carrier phase-based positioning for application to sidelink use cases.

# References

| [1] | draft TR38.859 v0.2.0, Study on expanded and improved NR positioning (Release 18), RAN1, December, 2022 |
| --- | --- |
| [2] | 3GPP R1-2104880 , "Carrier/Subcarrier Phase Based Enhancement for 5G NR Positioning " , Dankook University , RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021 |
| [3] | 3GPP R1-2104844 , "Carrier Phase Based Downlink Angle of Departure Measurement" , Dankook University , RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021 |
| [4] | 3GPP R1-2203634 , "Use cases and applications on Carrier Phase Based Positioning for NR" , Locaila, RAN WG1 #109-e e-Meeting, May 9 – 20, 2022 |
| [5] | Paul de Jonge, Christian Tiberius, “The LAMBDA method for integer ambiguity estimation : implementation aspects,” LGR-Series, Publications of the Delft Geodetic Computing Centre, August, 1996 |
| [6] | 3GPP R1-2203635, "Continuous PRS for improved carrier phase measurement" , Dankook University , RAN WG1 #109-e e-Meeting, May 9th – 20th, 2022 |
| [7] | 3GPP R1-2206227, "Solution for Integer Ambiguity, TRP synchronization and Vertical Positioning" , Locaila, RAN WG1 #110, Aug 22 – Aug 26, 2022 |
| [8] | 3GPP R1-2211259, "Experiment and simulation result on carrier phase based positioning" , Locaila, RAN WG1 #111, Nov 14 – Nov 18, 2022 |