3GPP TSG RAN WG1 #113			R1-2304437

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.1.3.1

Source:	Ericsson

Title:	On increased number of orthogonal DMRS ports for MU-MIMO and 8 Tx UL SU-MIMO

Document for:	Discussion, Decision

# 1	Introduction

During RAN1 #112bis meeting [1], following agreements were made for DMRS enhancements:

Conclusion

Dynamic switching between R15 DMRS port and R18 DMRS port by a scheduling DCI is not supported in Rel-18

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP,

- Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 68 in Table 7.3.1.2.2-2A-X.

  - For row 68, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 68 | 2 | 0,2,3 | 1 |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 60 in Table 7.3.1.2.2-3A-X.

  - For row 60, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 60 | 2 | 0,2,3 |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is also applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 128 in Table 7.3.1.2.2-4A-X.

  - For row 128, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 128 | 2 | 0,2,3 | 1 |

Agreement

Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH:

- To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

  - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

    - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

- Note: The above Working Assumption for CB based PUSCH may be confirmed later.

Agreement

For 8Tx PUSCH, specify the factor related to PUSCH to PTRS power ratio per layer per RE () based on the following principles.

- Principle 1: When the ptrs-Power configures 01, the factor () is 10log10(L), where L is the total number of PUSCH layers.

- Principle 2: When the ptrs-Power configures 00, the factor () is determined as the following

  - Principle 2.1: For fully coherent TPMIs, the factor () is 10log10(L), where L is the total number of PUSCH layers.

  - Principle 2.2: For non-coherent TPMIs, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE.

  - Principle 2.3: For non-codebook PUSCH, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE.

  - FFS: The factor () for partial coherent TPMIs

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 2 CWs,

      - Alt.1: Confirm the working assumption in RAN1#112 with modification (in red).

        - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |
| [4] | [2] | [0,1,2,3,10] |
| [5] | [2] | [0,1,8,2,3,10] |
| [6] | [2] | [0,1,8,2,3,10,11] |
| [7] | [2] | [0,1,8,9,2,3,10,11] |
| [8] | [2] | [0,2,3,8,9] |
| [9] | [2] | [0,1,2,3,8,9] |

FFS: Additional rows (rows 4~9) if there is technical justification.

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 1 CW,

    - Do not support row 21-22

    - FFS: Whether to support row 23

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 21 | [2] | [8-10] |
| 22 | [2] | [8-11] |

Working Assumption

- Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

- FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH.

Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Agreement

For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, support at least the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-2-X.

- FFS: For row 9-11, 24-30, 55-60, and 81-83 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

- FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64.

Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 2 | 0-4 | 2] |
| 1 | 1 | 1 | 1 | [1 | 2 | 0,1,2,3,4,6 | 2] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,4,5,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,4,5,6,7 | 2] |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,10 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| [8 | 2 | 2,3 | 1] | [8 | 1 | 0,1,4,5,8 | 2] |
| [9 | 2 | 0-2 | 1] | [9 | 1 | 0,1,4,5,8,12 | 2] |
| [10 | 2 | 0-3 | 1] | [10 | 1 | 0,1,4,5,8,9,12 | 2] |
| [11 | 2 | 0,2 | 1] | [11 | 1 | 0,1,4,5,8,9,12,13 | 2] |
| 12 | 2 | 0 | 2 | [12 | 2 | 0,1,4,5,8 | 2] |
| 13 | 2 | 1 | 2 | [13 | 2 | 0,1,4,5,8,12 | 2] |
| 14 | 2 | 2 | 2 | [14 | 2 | 0,1,4,5,8,9,12 | 2] |
| 15 | 2 | 3 | 2 | [15 | 2 | 0,1,4,5,8,9,12,13 | 2] |
| 16 | 2 | 4 | 2 | [16 | 2 | 2,3,6,7,10 | 2] |
| 17 | 2 | 5 | 2 | [17 | 2 | 2,3,6,7,10,14 | 2] |
| 18 | 2 | 6 | 2 | [18 | 2 | 2,3,6,7,10,11,14 | 2] |
| 19 | 2 | 7 | 2 | [19 | 2 | 2,3,6,7,10,11,14,15 | 2] |
| 20 | 2 | 0,1 | 2 | [20 | 2 | 0,1, 2,3,10 | 1] |
| 21 | 2 | 2,3 | 2 | [21 | 2 | 0,1,8,2,3,10 | 1] |
| 22 | 2 | 4,5 | 2 | [22 | 2 | 0,1,8, 2,3,10,11 | 1] |
| 23 | 2 | 6,7 | 2 | [23 | 2 | 0,1,8,9,2,3,10,11 | 1] |
| [24 | 2 | 0,4 | 2] | [24 | 1 | 0,1,4,5,12 | 2] |
| [25 | 2 | 2,6 | 2] | [25 | 1 | 0,1,8,4,5,12 | 2] |
| [26 | 2 | 0,1,4 | 2] | [26 | 1 | 0,1,8,4,5,12,13 | 2] |
| [27 | 2 | 2,3,6 | 2] | [27 | 1 | 0,1,8,9,4,5,12,13 | 2] |
| [28 | 2 | 0,1,4,5 | 2] | [28 | 2 | 0,1,4,5,12 | 2] |
| [29 | 2 | 2,3,6,7 | 2] | [29 | 2 | 0,1,8,4,5,12 | 2] |
| [30 | 2 | 0,2,4,6 | 2] | [30 | 2 | 0,1,8,4,5,12,13 | 2] |
| 31 | 1 | 8 | 1 | [31 | 2 | 0,1,8,9,4,5,12,13 | 2] |
| 32 | 1 | 9 | 1 | [32 | 2 | 2,3,6,7,14 | 2] |
| 33 | 1 | 8,9 | 1 | [33 | 2 | 2,3,10,6,7,14 | 2] |
| 34 | 2 | 8 | 1 | [34 | 2 | 2,3,10,6,7,14,15 | 2] |
| 35 | 2 | 9 | 1 | [35 | 2 | 2,3,10,11,6,7,14,15 | 2] |
| 36 | 2 | 10 | 1 | [36 | 2 | 0,2,3,8,9 | 1] |
| 37 | 2 | 11 | 1 | [37 | 2 | 0,1,2,3,8,9 | 1] |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| [40 | 2 | 8-10 | 1] |  |  |  |  |
| [41 | 2 | 8-11 | 1] |  |  |  |  |
| [42 | 2 | 8,10 | 1] |  |  |  |  |
| 43 | 2 | 8 | 2 |  |  |  |  |
| 44 | 2 | 9 | 2 |  |  |  |  |
| 45 | 2 | 10 | 2 |  |  |  |  |
| 46 | 2 | 11 | 2 |  |  |  |  |
| 47 | 2 | 12 | 2 |  |  |  |  |
| 48 | 2 | 13 | 2 |  |  |  |  |
| 49 | 2 | 14 | 2 |  |  |  |  |
| 50 | 2 | 15 | 2 |  |  |  |  |
| 51 | 2 | 8,9 | 2 |  |  |  |  |
| 52 | 2 | 10,11 | 2 |  |  |  |  |
| 53 | 2 | 12,13 | 2 |  |  |  |  |
| 54 | 2 | 14,15 | 2 |  |  |  |  |
| [55 | 2 | 8,12 | 2] |  |  |  |  |
| [56 | 2 | 10,14 | 2] |  |  |  |  |
| [57 | 2 | 8,9,12 | 2] |  |  |  |  |
| [58 | 2 | 10,11,14 | 2] |  |  |  |  |
| [59 | 2 | 8,9,12,13 | 2] |  |  |  |  |
| [60 | 2 | 10,11,14,15 | 2] |  |  |  |  |
| 61 | 2 | 8,10,12,14 | 2 |  |  |  |  |
| 62 | 1 | 0,1,8 | 1 |  |  |  |  |
| 63 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 64 | 2 | 0,1,8 | 1 |  |  |  |  |
| 65 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 66 | 2 | 2,3,10 | 1 |  |  |  |  |
| 67 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| [69 | 1 | 0,1,8 | 2] |  |  |  |  |
| [70 | 1 | 0,1,8,9 | 2] |  |  |  |  |
| [71 | 1 | 4,5,12 | 2] |  |  |  |  |
| [72 | 1 | 4,5,12,13 | 2] |  |  |  |  |
| [73 | 2 | 0,1,8 | 2] |  |  |  |  |
| [74 | 2 | 0,1,8,9 | 2] |  |  |  |  |
| [75 | 2 | 4,5,12 | 2] |  |  |  |  |
| [76 | 2 | 4,5,12,13 | 2] |  |  |  |  |
| [77 | 2 | 2,3,10 | 2] |  |  |  |  |
| [78 | 2 | 2,3,10,11 | 2] |  |  |  |  |
| [79 | 2 | 6,7,14 | 2] |  |  |  |  |
| [80 | 2 | 6,7,14,15 | 2] |  |  |  |  |
| [81 | 2 | 5,8,9 | 2] |  |  |  |  |
| [82 | 2 | 7,10,11 | 2] |  |  |  |  |
| [83 | 2 | 7,12,13 | 2] |  |  |  |  |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-3-X.

- FFS: For rows 9, 10, 20-23, 33, 34, 44-46, 60-62 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | [2 | 3 | 12-16] |
| 3 | 2 | 0 | [3 | 3 | 12-17] |
| 4 | 2 | 1 | 4 | 2 | 0,1,2,3,12 |
| 5 | 2 | 2 | 5 | 2 | 0,1,2,3,12,14 |
| 6 | 2 | 3 | 6 | 2 | 0-3,12-14 |
| 7 | 2 | 0,1 | 7 | 2 | 0-3,12-15 |
| 8 | 2 | 2,3 | [8 | 3 | 0,1,2,3,12] |
| [9 | 2 | 0-2] | [9 | 3 | 0,1,2,3,12,14] |
| [10 | 2 | 0-3] | [10 | 3 | 0-3,12-14] |
| 11 | 3 | 0 | [11 | 3 | 0-3,12-15] |
| 12 | 3 | 1 | [12 | 2 | 0,2,3,12,13] |
| 13 | 3 | 2 | [13 | 2 | 0,1,2,3,14] |
| 14 | 3 | 3 | [14 | 2 | 0,1,12,2,3,14] |
| 15 | 3 | 4 | [15 | 2 | 0,1,12,2,3,14,15] |
| 16 | 3 | 5 | [16 | 2 | 0,1,12,13,2,3,14,15] |
| 17 | 3 | 0,1 | [17 | 3 | 0,1,2,3,14] |
| 18 | 3 | 2,3 | [18 | 3 | 0,1,12,2,3,14] |
| 19 | 3 | 4,5 | [19 | 3 | 0,1,12,2,3,14,15] |
| [20 | 3 | 0-2] | [20 | 3 | 0,1,12,13,2,3,14,15] |
| [21 | 3 | 3-5] |  |  |  |
| [22 | 3 | 0-3] |  |  |  |
| [23 | 2 | 0,2] |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| [33 | 2 | 12-14] |  |  |  |
| [34 | 2 | 12-15] |  |  |  |
| 35 | 3 | 12 |  |  |  |
| 36 | 3 | 13 |  |  |  |
| 37 | 3 | 14 |  |  |  |
| 38 | 3 | 15 |  |  |  |
| 39 | 3 | 16 |  |  |  |
| 40 | 3 | 17 |  |  |  |
| 41 | 3 | 12,13 |  |  |  |
| 42 | 3 | 14,15 |  |  |  |
| 43 | 3 | 16,17 |  |  |  |
| [44 | 3 | 12-14] |  |  |  |
| [45 | 3 | 15-17] |  |  |  |
| [46 | 3 | 12-15] |  |  |  |
| [47 | 2 | 12,14] |  |  |  |
| 48 | 1 | 0,1,12 |  |  |  |
| 49 | 1 | 0,1,12,13 |  |  |  |
| 50 | 2 | 0,1,12 |  |  |  |
| 51 | 2 | 0,1,12,13 |  |  |  |
| 52 | 2 | 2,3,14 |  |  |  |
| 53 | 2 | 2,3,14,15 |  |  |  |
| 54 | 3 | 0,1,12 |  |  |  |
| 55 | 3 | 0,1,12,13 |  |  |  |
| 56 | 3 | 2,3,14 |  |  |  |
| 57 | 3 | 2,3,14,15 |  |  |  |
| 58 | 3 | 4,5,16 |  |  |  |
| 59 | 3 | 4,5,16,17 |  |  |  |
| [60 | 3 | 13,15,17] |  |  |  |
| [61 | 3 | 13,15] |  |  |  |
| [62 | 2 | 13,15] |  |  |  |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-4-X.

- FFS: For rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 3 | 0-4 | 1] |
| 1 | 1 | 1 | 1 | [1 | 3 | 0-5 | 1] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,6,8 | 2] |
| 4 | 2 | 1 | 1 | [4 | 2 | 0,1,2,3,6,7,8 | 2] |
| 5 | 2 | 2 | 1 | [5 | 2 | 0,1,2,3,6,7,8,9 | 2] |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0-3,12-14 | 1 |
| [9 | 2 | 0-2 | 1] | 9 | 2 | 0-3,12-15 | 1 |
| [10 | 2 | 0-3 | 1] | [10 | 3 | 0,1,2,3,12 | 1] |
| 11 | 3 | 0 | 1 | [11 | 3 | 0-3,12,14 | 1] |
| 12 | 3 | 1 | 1 | [12 | 3 | 0-3,12-14 | 1] |
| 13 | 3 | 2 | 1 | [13 | 3 | 0-3,12-15 | 1] |
| 14 | 3 | 3 | 1 | [14 | 1 | 0,1,6,7,12 | 2] |
| 15 | 3 | 4 | 1 | [15 | 1 | 0,1,6,7,12,18 | 2] |
| 16 | 3 | 5 | 1 | [16 | 1 | 0,1,6,7,12,13,18 | 2] |
| 17 | 3 | 0,1 | 1 | [17 | 1 | 0,1,6,7,12,13,18,19 | 2] |
| 18 | 3 | 2,3 | 1 | [18 | 2 | 0,1,6,7,12 | 2] |
| 19 | 3 | 4,5 | 1 | [19 | 2 | 0,1,6,7,12,18 | 2] |
| [20 | 3 | 0-2 | 1] | [20 | 2 | 0,1,6,7,12,13,18 | 2] |
| [21 | 3 | 3-5 | 1] | [21 | 2 | 0,1,6,7,12,13,18,19 | 2] |
| [22 | 3 | 0-3 | 1] | [22 | 2 | 2,3,8,9,14 | 2] |
| [23 | 2 | 0,2 | 1] | [23 | 2 | 2,3,8,9,14,20 | 2] |
| 24 | 3 | 0 | 2 | [24 | 2 | 2,3,8,9,14,15,20 | 2] |
| 25 | 3 | 1 | 2 | [25 | 2 | 2,3,8,9,14,15,20,21 | 2] |
| 26 | 3 | 2 | 2 | [26 | 3 | 0,1,6,7,12 | 2] |
| 27 | 3 | 3 | 2 | [27 | 3 | 0,1,6,7,12,18 | 2] |
| 28 | 3 | 4 | 2 | [28 | 3 | 0,1,6,7,12,13,18 | 2] |
| 29 | 3 | 5 | 2 | [29 | 3 | 0,1,6,7,12,13,18,19 | 2] |
| 30 | 3 | 6 | 2 | [30 | 3 | 2,3,8,9,14 | 2] |
| 31 | 3 | 7 | 2 | [31 | 3 | 2,3,8,9,14,20 | 2] |
| 32 | 3 | 8 | 2 | [32 | 3 | 2,3,8,9,14,15,20 | 2] |
| 33 | 3 | 9 | 2 | [33 | 3 | 2,3,8,9,14,15,20,21 | 2] |
| 34 | 3 | 10 | 2 | [34 | 3 | 4,5,10,11,16 | 2] |
| 35 | 3 | 11 | 2 | [35 | 3 | 4,5,10,11,16,22 | 2] |
| 36 | 3 | 0,1 | 2 | [36 | 3 | 4,5,10,11,16,17,22 | 2] |
| 37 | 3 | 2,3 | 2 | [37 | 3 | 4,5,10,11,16,17,22,23 | 2] |
| 38 | 3 | 4,5 | 2 | [38 | 2 | 0,1,2,3,14 | 1] |
| 39 | 3 | 6,7 | 2 | [39 | 2 | 0,1,12,2,3,14 | 1] |
| 40 | 3 | 8,9 | 2 | [40 | 2 | 0,1,12,2,3,14,15 | 1] |
| 41 | 3 | 10,11 | 2 | [41 | 2 | 0,1,12,13,2,3,14,15 | 1] |
| [42 | 3 | 0,1,6 | 2] | [42 | 3 | 0,1,2,3,14 | 1] |
| [43 | 3 | 2,3,8 | 2] | [43 | 3 | 0,1,12,2,3,14 | 1] |
| [44 | 3 | 4,5,10 | 2] | [44 | 3 | 0,1,12,2,3,14,15 | 1] |
| [45 | 3 | 0,1,6,7 | 2] | [45 | 3 | 0,1,12,13,2,3,14,15 | 1] |
| [46 | 3 | 2,3,8,9 | 2] | [46 | 1 | 0,1,6,7,18 | 2] |
| [47 | 3 | 4,5,10,11 | 2] | [47 | 1 | 0,1,12,6,7,18 | 2] |
| 48 | 1 | 0 | 2 | [48 | 1 | 0,1,12,6,7,18,19 | 2] |
| 49 | 1 | 1 | 2 | [49 | 1 | 0,1,12,13,6,7,18,19 | 2] |
| 50 | 1 | 6 | 2 | [50 | 2 | 0,1,6,7,18 | 2] |
| 51 | 1 | 7 | 2 | [51 | 2 | 0,1,12,6,7,18 | 2] |
| 52 | 1 | 0,1 | 2 | [52 | 2 | 0,1,12,6,7,18,19 | 2] |
| 53 | 1 | 6,7 | 2 | [53 | 2 | 0,1,12,13,6,7,18,19 | 2] |
| 54 | 2 | 0,1 | 2 | [54 | 2 | 2,3,8,9,20 | 2] |
| 55 | 2 | 2,3 | 2 | [55 | 2 | 2,3,14,8,9,20 | 2] |
| 56 | 2 | 6,7 | 2 | [56 | 2 | 2,3,14,8,9,20,21 | 2] |
| 57 | 2 | 8,9 | 2 | [57 | 2 | 2,3,14,15,8,9,20,21 | 2] |
| 58 | 1 | 12 | 1 | [58 | 3 | 0,1,6,7,18 | 2] |
| 59 | 1 | 13 | 1 | [59 | 3 | 0,1,12,6,7,18 | 2] |
| 60 | 1 | 12,13 | 1 | [60 | 3 | 0,1,12,6,7,18,19 | 2] |
| 61 | 2 | 12 | 1 | [61 | 3 | 0,1,12,13,6,7,18,19 | 2] |
| 62 | 2 | 13 | 1 | [62 | 3 | 2,3,8,9,20 | 2] |
| 63 | 2 | 14 | 1 | [63 | 3 | 2,3,14,8,9,20 | 2] |
| 64 | 2 | 15 | 1 | [64 | 3 | 2,3,14,8,9,20,21 | 2] |
| 65 | 2 | 12,13 | 1 | [65 | 3 | 2,3,14,15,8,9,20,21 | 2] |
| 66 | 2 | 14,15 | 1 | [66 | 3 | 4,5,10,11,22 | 2] |
| [67 | 2 | 12-14 | 1] | [67 | 3 | 4,5,16,10,11,22 | 2] |
| [68 | 2 | 12-15 | 1] | [68 | 3 | 4,5,16,10,11,22,23 | 2] |
| 69 | 3 | 12 | 1 | [69 | 3 | 4,5,16,17,10,11,22,23 | 2] |
| 70 | 3 | 13 | 1 |  |  |  |  |
| 71 | 3 | 14 | 1 |  |  |  |  |
| 72 | 3 | 15 | 1 |  |  |  |  |
| 73 | 3 | 16 | 1 |  |  |  |  |
| 74 | 3 | 17 | 1 |  |  |  |  |
| 75 | 3 | 12,13 | 1 |  |  |  |  |
| 76 | 3 | 14,15 | 1 |  |  |  |  |
| 77 | 3 | 16,17 | 1 |  |  |  |  |
| [78 | 3 | 12-14 | 1] |  |  |  |  |
| [79 | 3 | 15-17 | 1] |  |  |  |  |
| [80 | 3 | 12-15 | 1] |  |  |  |  |
| [81 | 2 | 12,14 | 1] |  |  |  |  |
| 82 | 3 | 12 | 2 |  |  |  |  |
| 83 | 3 | 13 | 2 |  |  |  |  |
| 84 | 3 | 14 | 2 |  |  |  |  |
| 85 | 3 | 15 | 2 |  |  |  |  |
| 86 | 3 | 16 | 2 |  |  |  |  |
| 87 | 3 | 17 | 2 |  |  |  |  |
| 88 | 3 | 18 | 2 |  |  |  |  |
| 89 | 3 | 19 | 2 |  |  |  |  |
| 90 | 3 | 20 | 2 |  |  |  |  |
| 91 | 3 | 21 | 2 |  |  |  |  |
| 92 | 3 | 22 | 2 |  |  |  |  |
| 93 | 3 | 23 | 2 |  |  |  |  |
| 94 | 3 | 12,13 | 2 |  |  |  |  |
| 95 | 3 | 14,15 | 2 |  |  |  |  |
| 96 | 3 | 16,17 | 2 |  |  |  |  |
| 97 | 3 | 18,19 | 2 |  |  |  |  |
| 98 | 3 | 20,21 | 2 |  |  |  |  |
| 99 | 3 | 22,23 | 2 |  |  |  |  |
| [100 | 3 | 12,13,18 | 2] |  |  |  |  |
| [101 | 3 | 14,15,20 | 2] |  |  |  |  |
| [102 | 3 | 16,17,22 | 2] |  |  |  |  |
| [103 | 3 | 12,13,18,19 | 2] |  |  |  |  |
| [104 | 3 | 14,15,20,21 | 2] |  |  |  |  |
| [105 | 3 | 16,17,22,23 | 2] |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 1 | 0,1,12 | 1 |  |  |  |  |
| 117 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 118 | 2 | 0,1,12 | 1 |  |  |  |  |
| 119 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 120 | 2 | 2,3,14 | 1 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 122 | 3 | 0,1,12 | 1 |  |  |  |  |
| 123 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 124 | 3 | 2,3,14 | 1 |  |  |  |  |
| 125 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 126 | 3 | 4,5,16 | 1 |  |  |  |  |
| 127 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| [129 | 1 | 0,1,12 | 2] |  |  |  |  |
| [130 | 1 | 0,1,12,13 | 2] |  |  |  |  |
| [131 | 1 | 6,7,18 | 2] |  |  |  |  |
| [132 | 1 | 6,7,18,19 | 2] |  |  |  |  |
| [133 | 2 | 0,1,12 | 2] |  |  |  |  |
| [134 | 2 | 0,1,12,13 | 2] |  |  |  |  |
| [135 | 2 | 6,7,18 | 2] |  |  |  |  |
| [136 | 2 | 6,7,18,19 | 2] |  |  |  |  |
| [137 | 2 | 2,3,14 | 2] |  |  |  |  |
| [138 | 2 | 2,3,14,15 | 2] |  |  |  |  |
| [139 | 2 | 8,9,20 | 2] |  |  |  |  |
| [140 | 2 | 8,9,20,21 | 2] |  |  |  |  |
| [141 | 3 | 0,1,12 | 2] |  |  |  |  |
| [142 | 3 | 0,1,12,13 | 2] |  |  |  |  |
| [143 | 3 | 6,7,18 | 2] |  |  |  |  |
| [144 | 3 | 6,7,18,19 | 2] |  |  |  |  |
| [145 | 3 | 2,3,14 | 2] |  |  |  |  |
| [146 | 3 | 2,3,14,15 | 2] |  |  |  |  |
| [147 | 3 | 8,9,20 | 2] |  |  |  |  |
| [148 | 3 | 8,9,20,21 | 2] |  |  |  |  |
| [149 | 3 | 4,5,16 | 2] |  |  |  |  |
| [150 | 3 | 4,5,16,17 | 2] |  |  |  |  |
| [151 | 3 | 10,11,22 | 2] |  |  |  |  |
| [152 | 3 | 10,11,22,23 | 2] |  |  |  |  |
| [153 | 3 | 7,12,13 | 2] |  |  |  |  |
| [154 | 3 | 9,14,15 | 2] |  |  |  |  |
| [155 | 3 | 11,16,17 | 2] |  |  |  |  |
| [156 | 3 | 9,18,19 | 2] |  |  |  |  |
| [157 | 3 | 18,19,20 | 2] |  |  |  |  |
| [158 | 3 | 21,22,23 | 2] |  |  |  |  |

Conclusion

No consensus to support MAC CE based switching between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH

Agreement

For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH/PUSCH, if Rel.18 eType1/eType2 DMRS ports is configured by RRC, the DCI size of antenna ports field in DCI format 1_1/1_2/0_1/0_2 is increased by at least 1-bit from Rel.17.

- Note: it does not preclude future possibility to support more than 1-bit, if RAN1 agree the necessity.

Agreement

For RAN1#112 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH.

- Support row 7 for rank2, row1 for rank3, row 1 for rank4.

Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 7 | 2 | 9,11 |

Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 1 | 2 | 8-10 |

Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 1 | 2 | 8-11 |

Agreement

For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following.

- Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

- Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - The CW with the higher MCS is selected in case of two CWs.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

- Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

Conclusion

For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

- For PUSCH, there is no restriction.

Agreement

For partial/non-coherent PUSCH with rank=5-8 transmission (i.e. non of the CWs is disabled) with one PTRS port, PTRS-DMRS association for PUSCH is the following.

- The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - The CW with the higher MCS is selected in case of two CWs.

    - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW |
| 1 | 2nd scheduled DMRS port with the CW |
| 2 | 3rd scheduled DMRS port with the CW |
| 3 | 4th scheduled DMRS port with the CW |

Conclusion

For “The CW with the higher MCS” in RAN1#112 agreement of PTRS-DMRS association field for full-coherent PUSCH with rank=5~8 PUSCH with one port PTRS, following is clarified.

- Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

In this contribution we provide our view on antenna port table design principle and proposed port combinations for PDSCH and PUSCH. For MU-MIMO we discuss co-scheduling of Rel-15 and Rel-18 DMRS. We also share our view on PT-RS power control.

# 2	Discussion

## 2.1 Antenna port table design for PDSCH and PUSCH

In this section we provide our view on the principles for antenna port table design and proposed tables for eType1/2 PDSCH and PUSCH.

One of the most important use-cases for extending the number of DMRS ports is to allow co-scheduling of more UEs for MU-MIMO. For MU-MIMO, as a minimum, one-layer and two-layer transmission should be supported. Thus, port combinations allowing for MU-MIMO co-scheduling all combinations of one-layer and two-layer UEs, utilizing all the available ports should be supported. Since, these ports are to be used for MU-MIMO, there should be no scheduling restrictions associated to these port combinations. The principle is thus: Support a set of one-port and two-port port combinations that allow co-scheduling of any combination of multiple UEs scheduled with one or two layers, filling up all available DMRS ports. For PUSCH supporting such combinations will significantly increase the MU-MIMO capacity as there’s no MU-MIMO restriction.

Note: In the proposed tables below the yellow highlight means rows not yet agreed or proposed for PDSCH.

Include antenna port combinations enable 1-layer and 2-layer transmission without assuming scheduling restrictions. Support following rank=2 tables for PUSCH eType1 maxLength=2.

Table 1: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14 | 1 | 8 | 1 |
| 15 | 1 | 9 | 1 |
| 16 | 2 | 8 | 1 |
| 17 | 2 | 9 | 1 |
| 18 | 2 | 10 | 1 |
| 19 | 2 | 11 | 1 |
| 20 | 2 | 8 | 2 |
| 21 | 2 | 9 | 2 |
| 22 | 2 | 10 | 2 |
| 23 | 2 | 11 | 2 |
| 24 | 2 | 12 | 2 |
| 25 | 2 | 13 | 2 |
| 26 | 2 | 14 | 2 |
| 27 | 2 | 15 | 2 |
| 28 | 1 | 8 | 2 |
| 29 | 1 | 9 | 2 |
| 30 | 1 | 12 | 2 |
| 31 | 1 | 13 | 2 |

Table 2: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10 | 1 | 8,9 | 1 |
| 11 | 2 | 8,9 | 1 |
| 12 | 2 | 10,11 | 1 |
| 13 | 2 | 8,10 | 1 |
| 14 | 2 | 9,11 | 1 |
| 15 | 2 | 8,9 | 2 |
| 16 | 2 | 10,11 | 2 |
| 17 | 2 | 12,13 | 2 |
| 18 | 2 | 14,15 | 2 |
| 19 | 2 | 0,2 | 2 |
| 20 | 2 | 1,3 | 2 |
| 21 | 2 | 4,6 | 2 |
| 22 | 2 | 5,7 | 2 |
| 23 | 2 | 8,10 | 2 |
| 24 | 2 | 9,11 | 2 |
| 25 | 2 | 12,14 | 2 |
| 26 | 2 | 13,15 | 2 |
| 27 | 1 | 0,1 | 2 |
| 28 | 1 | 8,9 | 2 |
| 29 | 1 | 4,5 | 2 |
| 30 | 1 | 12,13 | 2 |
| 31 | Reserved | Reserved | Reserved |

Another principle is to prioritize port-combinations that are compatible with legacy ports. Take type 1 DMRS for example, the ports 0 to 7 for eType1 and Type1 are the same. In order to facilitate co-scheduling legacy UEs with Rel-18 UEs, it is beneficial if the Rel-18 DMRS antenna port tables includes rows that are not using the legacy DMRS ports. Note also that the DMRS port used for fallback DCI is always port 0, port combinations don’t use port 0, for example, {8,9,10}, {8,9,10,11} shall be added to eType1 maxLength=2 PUSCH table. Rel-18 UE can be scheduled with such combination and being co-scheduled with legacy UEs using legacy ports for MU-MIMO PUSCH transmission.

Include antenna ports combinations that are compatible with legacy ports to facilitate co-scheduling legacy UEs with Rel-18 UEs.

Super orthogonal ports combinations shall be included to improve the robustness at high delay spread. To achieve super-orthogonality for port combinations with more than two ports, it’s therefore necessary to utilize more than one CDM group. For PDSCH the UE may assume that no other UE has been co-scheduled within the same CDM group. One such example is rank 6 combination {0,1,2,3,8,9} for eType1 where the first codeword with 3 layers uses antenna ports {0,1,2}, the second codeword with 3 layers uses antenna ports {3,8,9}. As is illustrated in Figure 1, see simulation details in [4], the performance of {2}, {3} is shown with the blue curve; the performance of {0}, {1}, {8}, {9} is shown with the black/green curve. {2}, {3} gives better performance than {0}, {1}, {8}, {9}, and for each codeword one better layer is used. Comparing with the agreed row {0,1,2,3,8,10} which uses the black/green ports with higher channel estimation error and lower throughput, {0,1,2,3,8,9} provides much better performance.

Figure 1 Chest and Throughput comparison for antenna port {0}, {1}, {8}, {9} and antenna ports {2}, {3}.

The antenna port combination {0,1,2,3,8,9} shall be supported for PDSCH and PUSCH. For PUSCH as there’s no MU-MIMO restriction, another combination {2,3,8,9,10,11} can be used which even enables co-scheduling of legacy UEs using antenna ports 0 and 1.

Include antenna port combinations, such as {0,1,2,3,8,9}, that are super orthogonal to improve the robustness at high delay spread

Combinations using as few CDM groups as possible for higher rank shall also be included such that DMRS can be power boosted which can achieve better performance. Especially entries with “CDM without data equal to 1” shall be included to improve the spectrum efficiency when there’s few UEs in the network. One such example is combination {0,1,4,5,8,9,12,13} with eType1 maxLength=2, where only one CDM group is needed to schedule 8 layers.

Include antenna ports combinations using as few CDM groups as possible for higher ranks.

Considering these design principles and different application scenarios, our proposals for the eType1 tables are the following.

Support at least these combinations for eType1 PUSCH/PDSCH tables with maxLength=1, rank = 5,6,7,8.

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 3,8,9,10,11 or [8,9,3,10,11] |
| 2 | 2 | 2,3,9,10,11 |

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9 |
| 1 | 2 | 2,3,8,9,10,11 |
| 2 | 2 | 1,3,8,9,10,11 |

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10 |
| 1 | 2 | 1,2,3,8,9,10,11 or [1,8,9,2,3,10,11] |

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10,11 or [0,1,8,9,2,3,10,11] |

Support at least these combinations for eType1 PUSCH tables with maxLength=2, rank = 3,4,5,6,7,8:

- Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 8-10 | 1 |
| 4 | 2 | 8-10 | 2 |
| 5 | 2 | 12-14 | 2 |
| 6 | 1 | 0,1,8 | 1 |
| 7 | 2 | 0,1,8 | 1 |
| 8 | 2 | 2,3,10 | 1 |
| 9 | 2 | 0,1,8 | 2 |
| 10 | 2 | 2,3,10 | 2 |
| 11 | 2 | 4,5,12 | 2 |
| 12 | 2 | 6,7,14 | 2 |
| 13 | 1 | 0,1,8 | 2 |
| 14 | 1 | 2,3,10 | 2 |

- Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4 | 2 | 8-11 | 1 |
| 5 | 1 | 0,1,8,9 | 1 |
| 6 | 2 | 0,1,8,9 | 1 |
| 7 | 2 | 2,3,10,11 | 1 |
| 8 | 2 | 0,1,8,9 | 2 |
| 9 | 2 | 2,3,10,11 | 2 |
| 10 | 2 | 4,5,12,13 | 2 |
| 11 | 2 | 6,7,14,15 | 2 |
| 12 | 2 | 8,10,12,14 | 2 |
| 13 | 1 | 0,1,8,9 | 2 |
| 14 | 1 | 4,5,12,13 | 2 |

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,8 | 1 |
| 1 | 2 | 3,8,9,10,11 or [8,9,3,10,11] | 1 |
| 2 | 2 | 2,3,9,10,11 | 1 |
| 3 | 1 | 0,1,4,5,8 | 2 |
| 4 | 2 | 0,1,4,5,8 | 2 |
| 5 | 2 | 2,3,6,7,10 | 2 |

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9 | 1 |
| 1 | 2 | 2,3,8,9,10,11 | 1 |
| 2 | 2 | 1,3,8,9,10,11 | 1 |
| 3 | 1 | 0,1,4,5,8,9 | 2 |
| 4 | 2 | 0,1,4,5,8,9 | 2 |
| 5 | 2 | 2,3,6,7,10,11 | 2 |

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10 | 1 |
| 1 | 2 | 1,2,3,8,9,10,11 or [1,8,9,2,3,10,11] | 1 |
| 2 | 2 | 0,1,4,5,8,9,12 | 2 |
| 3 | 1 | 0,1,4,5,8,9,12 | 2 |
| 4 | 2 | 2,3,6,7,10,11 | 2 |
| 5 | 2 | 9,10,11,12,13,14,15 | 2 |

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10,11 or [0,1,8,9,2,3,10,11] | 1 |
| 1 | 2 | 8,9,10,11,12,13,14,15 | 2 |
| 2 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 3 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 4 | 2 | 2,3,6,7,10,11,14,15 | 2 |

Support at least these combinations for eType1 PDSCH tables with maxLength=2

- Table 7.3.1.2.2-x: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,9 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0,1,4,5,8 | 2 |
| 9 | 2 | 0-2 | 1 | 9 | 2 | 0,1,4,5,8,9 | 2 |
| 10 | 2 | 0-3 | 1 | 10 | 2 | 0,1,4,5,8,9,12 | 2 |
| 11 | 2 | 0,2 | 1 | 11 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 12 | 2 | 0 | 2 | 12 | 2 | 2,3,6,7,10 | 2 |
| 13 | 2 | 1 | 2 | 13 | 2 | 2,3,6,7,10,11 | 2 |
| 14 | 2 | 2 | 2 | 14 | 2 | 2,3,6,7,10,11,14 | 2 |
| 15 | 2 | 3 | 2 | 15 | 2 | 2,3,6,7,10,11,14,15 | 2 |
| 16 | 2 | 4 | 2 | 16 | 1 | 0,1,4,5,8 | 2 |
| 17 | 2 | 5 | 2 | 17 | 1 | 0,1,4,5,8,9 | 2 |
| 18 | 2 | 6 | 2 | 18 | 1 | 0,1,4,5,8,9,12 | 2 |
| 19 | 2 | 7 | 2 | 19 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | 1 | 8 | 1 |  |  |  |  |
| 32 | 1 | 9 | 1 |  |  |  |  |
| 33 | 1 | 8,9 | 1 |  |  |  |  |
| 34 | 2 | 8 | 1 |  |  |  |  |
| 35 | 2 | 9 | 1 |  |  |  |  |
| 36 | 2 | 10 | 1 |  |  |  |  |
| 37 | 2 | 11 | 1 |  |  |  |  |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| 40 | 2 | 8-10 | 1 |  |  |  |  |
| 41 | 2 | 8-11 | 1 |  |  |  |  |
| 42 | 2 | 8,10 | 1 |  |  |  |  |
| 43 | 2 | 8 | 2 |  |  |  |  |
| 44 | 2 | 9 | 2 |  |  |  |  |
| 45 | 2 | 10 | 2 |  |  |  |  |
| 46 | 2 | 11 | 2 |  |  |  |  |
| 47 | 2 | 12 | 2 |  |  |  |  |
| 48 | 2 | 13 | 2 |  |  |  |  |
| 49 | 2 | 14 | 2 |  |  |  |  |
| 50 | 2 | 15 | 2 |  |  |  |  |
| 51 | 2 | 8,9 | 2 |  |  |  |  |
| 52 | 2 | 10,11 | 2 |  |  |  |  |
| 53 | 2 | 12,13 | 2 |  |  |  |  |
| 54 | 2 | 14,15 | 2 |  |  |  |  |
| 55 | 2 | 8,12 | 2 |  |  |  |  |
| 56 | 2 | 10,14 | 2 |  |  |  |  |
| 57 | 2 | 8,9,12 | 2 |  |  |  |  |
| 58 | 2 | 10,11,14 | 2 |  |  |  |  |
| 59 | 2 | 8,9,12,13 | 2 |  |  |  |  |
| 60 | 2 | 10,11,14,15 | 2 |  |  |  |  |
| 61 | 2 | 8,10,12,14 | 2 |  |  |  |  |
| 62 | 1 | 0,1,8 | 1 |  |  |  |  |
| 63 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 64 | 2 | 0,1,8 | 1 |  |  |  |  |
| 65 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 66 | 2 | 2,3,10 | 1 |  |  |  |  |
| 67 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| 68 | 2 | 0,1,8 | 2 |  |  |  |  |
| 69 | 2 | 0,1,8,9 | 2 |  |  |  |  |
| 70 | 2 | 2,3,10 | 2 |  |  |  |  |
| 71 | 2 | 2,3,10,11 | 2 |  |  |  |  |
| 72 | 2 | 4,5,12 | 2 |  |  |  |  |
| 73 | 2 | 4,5,12,13 | 2 |  |  |  |  |
| 74 | 2 | 6,7,14 | 2 |  |  |  |  |
| 75 | 2 | 6,7,14,15 | 2 |  |  |  |  |

The above proposals emphasizing on a few antenna port combinations which are particularly important to achieve good SU-MIMO performance and compatible MU-MMIMO co-scheduling with legacy UEs. For real network deployment, as the SU-MIMO and MU-MIMO traffic mode varies rapidly from slot to slot, supporting more antenna port combinations for PDSCH/PUSCH gives possibility for more flexible scheduling options and therefore better performance. During the release 18 DMRS discussion we noticed there’s different initiative from UE/gNB side to promote certain type/row of combinations more than the other, and there’s contractional proposals being driven by different companies. In our understanding, release 18 may be the last DMRS enhancement for NR, we shall try to complete the DMRS design with affordable effort but also future proof. For those controversial combinations, it would be unfortunate that we discard certain combinations only because of the limitation of current UE implementation, however UE and gNB implementation complexity also need to be considered. We would like to support antenna port combinations as many as possible to fulfill the requirement of different MU-MIMO scenarios. For the raised concern on UE implementation complexity, in our understanding we could support such combinations based on different UE capabilities.

For the remaining combinations in the brackets in PDSCH antenna port tables, support as many as possible to fulfill the requirement of different MU-MIMO scenarios and different UE implementations.

Introduce basic and new UE capabilities to reflect different UE implementation of Rel-18 DMRS.

## 2.2 Co-scheduling of Type1/2 DMRS with eType1/2 DMRS

One discussion during the RAN1#112 meeting was about co-scheduling between a Rel-18 UE configured with Rel-15 DMRS ports and a Rel-18 UE configured with Rel-18 DMRS ports. Three different alternatives were discussed in the feature lead summary, which are copied below:

  - For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following.

    - Alt.1: UE does not expect such MU-MIMO in a CDM group.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

    - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction).

In Alt.1, the idea is to restrict co-scheduling of a Rel-18 UE indicated with Rel-15 DMRS ports and a Rel-18 UE indicated with new Rel-18 DMRS ports. The main argument for this alternative is that a Rel-18 UE configured with Rel-15 DMRS can re-use its Rel-15 DMRS receiver implementation.

In Alt.2, the network can indicate to a Rel-18 UE configured with Rel-15 DMRS if another UE may be co-scheduled with a new Rel-18 DMRS ports or not. The main motivation for this is that the Rel-18 UE configured with Rel-15 DMRS can adapt its DMRS receiver based on this information, which will improve the DMRS channel estimation quality for the UE (e.g. if the UE is indicated, using e.g. RRC configuration, that other UEs will not be co-scheduled with new Rel-18 DMRS ports, the UE can apply a DMRS receiver based on FD-OCC code length 2 instead of FD-OCC code length 4, which will make the channel estimation quality more robust towards delay spread in the channel).

In Alt.3, it is up to gNB implementation whether to co-schedule a Rel-18 UE configured with Rel-15 DMRS ports with new Rel-18 DMRS ports or not. One drawback with this solution is that if a network deployment does not support Rel-18 DMRS, then the Rel-18 UEs configured with Rel-15 DMRS will have reduced performance compared to Rel-15 UEs (due to that Rel-18 UEs have to apply DMRS channel estimation based on FD-OCC length 4 instead of FD-OCC length 2 when being co-scheduled with other UEs), which is not acceptable. In addition, in Alt.3 it says that no specification impact is needed. However, in NR Rel-15 it already says that co-scheduling between different types of DMRS ports are no supported, so Alt.3 will not work without updating the specification, hence the whole Alternative is not valid by definition.

Based on the above discussions, we propose to support Alt.2 with both RRC based and DCI based indication.

For co-scheduling of eType1/2 and Type1/2 DMRS, support Alt.2, where the UE can be indicated that there may be another Rel.18 UE1 with Rel.18 New ports in the same CDM group, and where the indication can be signalled with both RRC signalling and DCI signalling

## 2.3 PTRS for up to 8-layer PUSCH

In RAN1#112-bis meeting we’ve made the following agreement:

Agreement

For 8Tx PUSCH, specify the factor related to PUSCH to PTRS power ratio per layer per RE () based on the following principles.

- Principle 1: When the ptrs-Power configures 01, the factor () is 10log10(L), where L is the total number of PUSCH layers.

- Principle 2: When the ptrs-Power configures 00, the factor () is determined as the following

  - Principle 2.1: For fully coherent TPMIs, the factor () is 10log10(L), where L is the total number of PUSCH layers.

  - Principle 2.2: For non-coherent TPMIs, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE.

  - Principle 2.3: For non-codebook PUSCH, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE.

  - FFS: The factor () for partial coherent TPMIs

And in RAN1#112 meeting we´ve made the following agreement:

Agreement

For partially coherent uplink precoding by an 8TX UE codebook,

- When Ng=2

  - Precoding design is based on Rel-15 UL 4TX codebook,

    - Full-coherent precoders are used

      - FFS whether partial-coherent precoders are needed

- When Ng=4, down-select from,

  - Alt1:

    - Precoding design is based on Rel-15 UL 2TX codebook,

      - Full-coherent precoders are used

  - Alt2:

    - Precoding design is based on Rel-15 UL 4TX codebook,

      - Partial-coherent precoders are used

For partially coherent codebook the power boosting might depend on if for example only fully coherent precoders are allowed per antenna port group or if both fully coherent and partially coherent precoders are allowed (where for example a first layer is transmitted with a fully coherent precoder from a first antenna port group simultaneously as a second layer is transmitted with a partially coherent precoder from the same antenna port group). It also not agreed yet if a partially coherent UE can be configured with both partially coherent codebook and non-coherent codebook or only a partially coherent codebook. If a UE can be configured with a both partially coherent codebook and non-coherent codebook, the question is if it will be allowed that port selection precoder is used for an antenna port group for a first layer at the same time as fully/partially coherent precoder is used for a second layer from the same antenna port group.

Since these discussions are still ongoing, we propose to wait with determining the PTRS to DMRS power boosting for partially coherent precoders until the TPMI for partially coherent precoders have been agreed.

For partially coherent precoders the PTRS power boosting is more complicated and can depend on the AI9.1.4.2.

Wait with determining the PTRS to DMRS power boosting for partially coherent precoders until the TPMI for partially coherent precoders have been agreed.

For the agreement on PTRS-DMRS association from RAN1#112bis meeting, we propose to support Alt.1 for its simplicity.

Agreement

For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following.

- Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

- Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - The CW with the higher MCS is selected in case of two CWs.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

- Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

For PTRS-DMRS association with up to 8 layers, support Alt.1, i.e. using 4-bit in DCI format 0_1/0_2.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For partially coherent precoders the PTRS power boosting is more complicated and can depend on the AI9.1.4.2.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include antenna port combinations enable 1-layer and 2-layer transmission without assuming scheduling restrictions. Support following rank=2 tables for PUSCH eType1 maxLength=2.

Proposal 2	Include antenna ports combinations that are compatible with legacy ports to facilitate co-scheduling legacy UEs with Rel-18 UEs.

Proposal 3	Include antenna port combinations, such as {0,1,2,3,8,9}, that are super orthogonal to improve the robustness at high delay spread

Proposal 4	Include antenna ports combinations using as few CDM groups as possible for higher ranks.

Proposal 5	Support at least these combinations for eType1 PUSCH/PDSCH tables with maxLength=1, rank = 5,6,7,8.

Proposal 6	Support at least these combinations for eType1 PUSCH tables with maxLength=2, rank = 3,4,5,6,7,8:

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 3

Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 4

Proposal 7	Support at least these combinations for eType1 PDSCH tables with maxLength=2

Table 7.3.1.2.2-x: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

Proposal 8	For the remaining combinations in the brackets in PDSCH antenna port tables, support as many as possible to fulfill the requirement of different MU-MIMO scenarios and different UE implementations.

Proposal 9	Introduce basic and new UE capabilities to reflect different UE implementation of Rel-18 DMRS.

Proposal 10	For co-scheduling of eType1/2 and Type1/2 DMRS, support Alt.2, where the UE can be indicated that there may be another Rel.18 UE1 with Rel.18 New ports in the same CDM group, and where the indication can be signalled with both RRC signalling and DCI signalling

Proposal 11	Wait with determining the PTRS to DMRS power boosting for partially coherent precoders until the TPMI for partially coherent precoders have been agreed.

Proposal 12	For PTRS-DMRS association with up to 8 layers, support Alt.1, i.e. using 4-bit in DCI format 0_1/0_2.

# References

Chair’s Notes RAN1_112bis-e

Chair’s Notes RAN1_112

Chair’s Notes RAN1_111

R1-2207135 On DMRS enhancement in Rel-18, RAN1#110 meeting, Toulouse, August 2022