3GPP TSG RAN WG1 #113			R1-2304438

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Panasonic

Title: 	Discussion on general aspects of AI/ML framework

Agenda Item:		9.2.1

Document for:	Discussion

- Introduction

A study item “Study on Artificial Intelligence and Machine Learning for NR air-interface” has been approved for Rel.18 [1].

This document describes our view on general aspects of AI/ML framework. The agreement in the past meeting is described in Annex.

- Discussion

Expression of model applicability

When the AI/ML model is trained is same NW or UE side with the selection of AI/ML model before the activation, the selection of AI/ML model can take into account the condition where the AI/ML model is trained. In this case, for example, there is no need to define what is indoor or outdoor. There is also no need to what UE speed can be applicable as what AI/ML model. It is difficult to define what is the boundary between indoor and outdoor and the boundary among multiple speeds.

On the other hand, when the AI/ML model is trained is different NW or UE side with the selection of AI/ML model, in what condition, what AI/ML model to be used need to be defined. Although one way is to define/standardize what is boundary between indoor and outdoor and what is the boundary of UE speed to be used to change the AI/ML model, we don't think it is realistic. It also prevents optimization for future enhancement as such boundary condition is required to be fixed as it is related to how to efficiently utilize multiple of AI/ML models in various operating conditions. Instead, what data is used for the training itself can be used for the expression of AI/ML model. In such case, in addition to the training data, the expected output of AI/ML model is also required for the identification as just training input data does not ensure anything to work.

As we don't expect the selection of AI/ML model located in NW is selected by UE as realistic use case, the more concrete different NW or UE side is the AI/ML model is trained in UE side and the selection of AI/ML model is NW.

Based on the discussion, we propose following.

Proposal 1:  When the AI/ML model is trained is different NW or UE side with the selection of AI/ML model, the AI/ML model is expressed by the training data and expected output. This AI/ML model is logical AI/ML model. More concrete case is the AI/ML mode is trained in UE side and the selection of AI/ML model is NW side.

Functionality based LCM and model-ID-based LCM

The boundary between functionality based LCM and model-ID-based LCM has been discussed. In the last RAN1 meeting, it was agreed that the functionality is feature/FG enabled by configuration(s), where the configuration(s) is(are) supported based on conditions indicated by UE capability. It was also agreed that AI/ML model-ID-based LCM is AI/ML-enabled feature/FG plus additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

The UE capability can indicate the usage purpose like CSI compression, beam management but it is difficult to express what scenarios or what sites are suitable for certain AI/ML functionality or model. Possible examples of the difference are described below.

•	CSI compression

–	Functionality based LCM

•CSI compression without limiting specific site.

–	Model-ID-based LCM

•CSI compression for specific site or antenna deployment like antenna radiation pattern and/or channel condition like indoor/outdoor

•	Beam management

–	Functionality based LCM

•beam management without limiting specific site

–	Model-ID-based LCM

•beam management for specific site and/or antenna deployment like antenna radiation pattern and/or channel condition like indoor/outdoor

In functionality based LCM, it is up to UE implementation what AI/ML model is selected and used in what specific site or specific conditions. These selections of AI/ML model can be based on AI/ML or can be traditional approach i.e. other than AI/ML model. In addition, not only the selection, some of the situation processing itself may be carried out by other than AI/ML model.

In model-ID- based LCM, the NW identifies the AI/ML model itself even it can be logical model. Therefore, its function is limited to what AI/ML model can manage them but which AI/ML model can be NW side even if the model inference is carried out by UE side. In logical model, our view is the certain input and output relation are defined as test in order to define the algorithm. The test can be RAN4 test or bi-lateral test among terminal and NW. In case physical model, NW determines physical model itself including the parameters.

Based on the discussion, we propose following.

Proposal 2:

- In functionality based LCM, UE determines what physical AI/ML model is used for what conditions/site. UE may use other than AI/ML model for the specific condition and/or the selection of the condition.

- In logical model-ID-based LCM, NW determines what logical AI/ML model is used for what conditions/site. UE further determines what physical model is used.

- In physical model-ID-based LCM, NW determines what physical AI/ML model is used for what condition/site. The AI/ML model may be only the parameters management.

Validation

For functionality based LCM, the validation of the function itself can be similar to current non AI/ML model usage. How to use AI/ML model is up to UE implementation and the adaptation of the specific conditions and/or environment is UE side responsibility. RAN4 test may be specified in the certain conditions only but UE has the responsibility to work properly in the other conditions.

For model-ID-based LCM, NW has the function to select logical or physical model. NW can use the information what data was used for the training in the case of logical or physical models. In spite that the training data is known by NW in logical or physical model, how much the current real deployment situation is similar to the training data requires the evaluation. This evaluation of the AI/ML model is before the commercial deployment and it can be deployed to the specific UE. This is a kind of test trial. Such test trial can be skipped when the same AI/ML model is used for the same manufacture's same model in the same software version.

Such test trial is required even for the model trained by NW because it needs to check the impact caused by the compilation, quantization and alignment to the UE internal processing related to AI/ML operating environment.

Based on the discussion, we propose following.

Proposal 3:  For functional LCM, validation may be similar to non-AI/ML operation as all adaptation to the real deployment is rather UE side responsibility. For model-ID-based LCM, the validation using specific UE is required as a kind of test trial. This test trial can be skipped when the same AI/ML model is used for the same manufacture's same model in the same software version. This test trial is required even if the model is trained by the NW as the compilation, quantization and alignment to the UE internal processing related to AI/ML operating environment.

Clarification of model identification and model update

The terminology of model update and model parameter update were agreed as working assumption. On the other hand, there is not clear boundary between what part is called as "model structure" and what part is called as "model parameters". Some of model structure itself can be modified by the parameter itself. Therefore, we don't see the specific need to confirm the working assumption of the term "model parameter update". Regardless of the change is whole or some of parameters or structure, these can be categorized as just model update.

The working assumption of the model update is "process of updating the model parameters and/or model structure of a model". The change of the parameters can be the same or different usage/purpose/applicable condition. The same usage/purpose/applicable condition may aim to achieve better performance or reduced power consumption and so on. Our view is one model ID can be assigned with specific usage/purpose/applicable condition. When the usage/purpose/applicable condition is same, it is just the version difference and it is categorized as model update. If the usage/purpose/applicable condition is different, it should be interpreted as new model as usage/purpose/applicable condition itself is different.

Proposal 4: Not to confirm working assumption of model parameter update.

Proposal 5: Model update is restricted to the process updating the model parameters and/or model structure of a model of the same usage/purpose/applicable condition. It is same meaning with version update. When different usage/purpose/applicable condition is applied, it is new model.

By restricting a model update to the same usage/purpose/applicable condition, we propose model identification as the meaning to identify the specific usage/purpose/applicable condition. Current working assumption of model identification without the knowledge of usage/purpose/applicable condition is not so meaningful from NW perspective as NW cannot use AI/ML model without such information.

Proposal 6: Model identification is "a process/method of identifying an AI/ML model for the common understanding of the usage/purpose/applicable condition between the NW and the UE".

The relation between cases and AI/ML models

In RAN1#112, following cases y to z5 were agreed to be studied. Depending on the cases, the actual usage of AI/ML models are different. Our view is following described in "applicable AI/ML models". For case y, the training is carried out by NW side and the others are different usage. Therefore, we distinguish them as y1 and y2. We didn't analyze z5 as it is not realistic case.

Figure 1. The relation between cases and AI/ML models

| Case | Model delivery/transfer | Model storage location | Training location | Applicable AI/ML models |
| --- | --- | --- | --- | --- |
| y1 | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / neutral site | - Storage and training are via binary or source code model - NW indication is via logical model |
| y2 | model delivery (if needed) over-the-top | Outside 3gpp Network | NW-side | - Storage, training and NW indication are via source code model. |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site | - Storage, training and NW indication are via binary or source code model. |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side | - Not sure the standardization feasibility of this option as NW needs to aware proprietary format. It can be bi-lateral agreement between UE and NW. |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site | - Storage, training and NW indication are via binary or source code model. |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side | - Storage, training and NW indication are via source code model. |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side | - Not realistic |

With above table, we observed following points.

Observation 1: The models stored in the specific location are always physical model. It can be binary or source code model.

Observation 2: When the model stored is source code model, some compilation processing is required. It can be within UE or other network entity for the compilation.

Observation 3: When the training location is NW-side, only source code model can be trained as NW doesn't know UE proprietary implementation and NW cannot manage the UE internal switching functions among physical models. The storage location is within 3GPP network does not influence the control of AI/ML models by the NW.

Observation 4: When the training location is UE-side or neutral side but the storage location is 3gpp network, physical model stored in the 3gpp network is uploaded to 3gpp network beforehand from UE, UE-side or neutral site. Then, depending on the usage/purpose/applicable condition, NW can transfer specific physical models to UE.

- Conclusion

We propose following.

Proposal 1:  When the AI/ML model is trained is different NW or UE side with the selection of AI/ML model, the AI/ML model is expressed by the training data and expected output. This AI/ML model is logical AI/ML model. More concrete case is the AI/ML mode is trained in UE side and the selection of AI/ML model is NW side.

Proposal 2:

- In functionality based LCM, UE determines what physical AI/ML model is used for what conditions/site. UE may use other than AI/ML model for the specific condition and/or the selection of the condition.

- In logical model-ID-based LCM, NW determines what logical AI/ML model is used for what conditions/site. UE further determines what physical model is used.

- In physical model-ID-based LCM, NW determines what physical AI/ML model is used for what condition/site. The AI/ML model may be only the parameters management.

Proposal 3:  For functional LCM, validation may be similar to non-AI/ML operation as all adaptation to the real deployment is rather UE side responsibility. For model-ID-based LCM, the validation using specific UE is required as a kind of test trial. This test trial can be skipped when the same AI/ML model is used for the same manufacture's same model in the same software version. This test trial is required even if the model is trained by the NW as the compilation, quantization and alignment to the UE internal processing related to AI/ML operating environment.

Proposal 4: Not to confirm working assumption of model parameter update.

Proposal 5: Model update is restricted to the process updating the model parameters and/or model structure of a model of the same usage/purpose/applicable condition. It is same meaning with version update. When different usage/purpose/applicable condition is applied, it is new model.

Proposal 6: Model identification is "a process/method of identifying an AI/ML model for the common understanding of the usage/purpose/applicable condition between the NW and the UE".

Observation 1: The models stored in the specific location are always physical model. It can be binary or source code model.

Observation 2: When the model stored is source code model, some compilation processing is required. It can be within UE or other network entity for the compilation.

Observation 3: When the training location is NW-side, only source code model can be trained as NW doesn't know UE proprietary implementation and NW cannot manage the UE internal switching functions among physical models. The storage location is within 3GPP network does not influence the control of AI/ML models by the NW.

Observation 4: When the training location is UE-side or neutral side but the storage location is 3gpp network, physical model stored in the 3gpp network is uploaded to 3gpp network beforehand from UE, UE-side or neutral site. Then, depending on the usage/purpose/applicable condition, NW can transfer specific physical models to UE.

- Reference

[1]	RP-213599, “New SI: Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface,”	Qualcomm (Moderator), RAN#94e.

[2] R1- 2304054, "Final Summary of General Aspects of AIML Framework ", Moderator (Qualcomm), RAN1#112bis-e

- Past agreements

Agreements in RAN1#109:

Agreement

Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

Table: Working list of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

Agreements in RAN1#110:

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

  - Note: some of them to be refined

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

Agreement

The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Performance

  - Intermediate KPIs

  - Link and system level performance

  - Generalization performance

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of data collection

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity of model inference: FLOPs

  - Computational complexity for pre- and post-processing

  - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

- Training complexity

- LCM related complexity and storage overhead

  - FFS: specific aspects

- FFS: Latency, e.g., Inference latency

Note: Other aspects may be added in the future, e.g. training related KPIsNote: Use-case specific KPIs may be additionally considered for the given use-case.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

Note:

Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

Agreements in RAN1#110bis:

Working Assumption

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z

- Clarifying note: Level y includes cases without model delivery.

Agreement

Clarify Level x/y boundary as:

- Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE.
(Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

FFS: Detailed discussion of model ID with associated information and/or model functionality.

FFS: usage of model ID with associated information and/or model functionality based LCM procedure

FFS: whether support of model ID

FFS: the detailed applicable AI/ML operations

Agreement

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

Conclusion

Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact.

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Agreement

Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects:

-	Procedure and assistance signaling for the AI model switching and/or selection

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training).

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case:

    - Monitoring based on inference accuracy, including metrics related to intermediate KPIs

    - Monitoring based on system performance, including metrics related to system peformance KPIs

    - Other monitoring solutions, at least following 2 options.

      - Monitoring based on data distribution

        - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc.

        - Output-based: e.g., drift detection of output data

      - Monitoring based on applicable condition

Note: Model monitoring metric calculation may be done at NW or UE

Agreement

Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance

    - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

    - Overhead (e.g., signaling overhead associated with model monitoring)

    - Complexity (e.g., computation and memory cost for model monitoring)

    - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

    - FFS: Power consumption

    - Other KPIs are not precluded.

Note: Relevant KPIs may vary across different model monitoring approaches.

FFS: Discussion of KPIs for other LCM procedures

Agreement

Study various approaches for achieving good performance across different scenarios/configurations/sites, including

    - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

    - Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

      - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.]

    - Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Agreement

The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Clarification on inference complexity

  - Note: Inference complexity includes complexity for pre- and post-processing.

- LCM related complexity and storage overhead

  - Storage/computation for training data collection.

  - Storage/computation for training and model update

  - Storage/computation for model monitoring.

  - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation.

- FFS: Power consumption, latency (e.g., Inference latency)

Conclusion

This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP.

Agreements in RAN1#111:

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Funtionality

- For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

Working Assumption

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion,

| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

Agreements in RAN1#112:

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Agreement

For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

Agreements in RAN1#112bis:

Agreement

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

  - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

  - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

    - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

    - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

    - FFS: Other aspects that may constitute Functionality

  - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

- For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

  - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

  - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

  - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s).

  - FFS: relationship between functionality-based LCM and model-ID-based LCM

- Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

Conclusion

From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation.

- When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model.

Agreement

- Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities.

- Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

Working Assumption

The definition of ‘AI/ML model transfer’ is revised (marked in red) as follows:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

Working Assumption

| Model selection | The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation |
| --- | --- |