3GPP TSG-RAN WG1 #113	R1-2304597

Incheon, Korea, May 22nd – May 26th, 2023

Title: 	Discussion of enhancements on dynamic/flexible TDD

Source: 	ZTE, China Telecom

Agenda item:	9.3.3

Document for:	Discussion/Decision

Introduction

In RAN#96 meeting, a revised SID [1] on NR duplex evolution has been endorsed, in which enhancements on dynamic/flexible TDD is one of the objectives. During RAN1#109-e meeting, discussion scope division among different agenda items were discussed with guideline for future meetings [2]. More specifically, the potential gNB-to-gNB and UE-to-UE CLI handling schemes that are specific for dynamic/flexible TDD and schemes that are common for both SBFD and dynamic/flexible TDD will be handled under AI 9.3.3.

In this contribution, we provide our further analysis on potential enhancements on dynamic/flexible TDD in Rel-18.

Overview

According to the discussion of AI 9.3.1 in RAN1#109-e meeting, HetNet is an important scenario for dynamic/flexible TDD in Rel-18, in which Urban Macro and Indoor office are deployed in the same carrier. Macro gNBs use DL dominant static TDD UL/DL configuration, e.g., DDDSU, while Indoor gNBs is configured with UL dominant frame structure, e.g., DSUUU, by either static TDD UL/DL configuration or dynamic/flexible TDD UL/DL assignment. And, there are mainly two types of inter-cell co-channel interference, i.e., gNB-to-gNB inter-cell co-channel interference and UE-to-UE inter-cell co-channel interference as also defined in our contribution [3].

Figure-1: Inter-cell co-channel interference under a typical deployment

During NR Rel-16, UE-to-UE CLI and RIM handling were standardized for TDD.

- For UE-to-UE CLI, L3-based wideband CLI measurement/reporting on SRS-RSRP and CLI-RSSI are defined for evaluating interference strength among UEs. In addition, for UE which performs the SRS-RSRP measurement, there is an offset between the SRS reception timing and the DL reception timing. However, the offset is derived by UE implementation in Rel-16 CLI. Further, semi-static DL/UL configuration can be exchanged between gNB for facilitating network coordination.

- For RIM, the related design focuses on mitigating the impact of gNB-to-gNB remote interference, which is caused by atmospheric ducting phenomenon. As agreed in RAN1#110bis-e meeting, potential enhancement to Rel-16 RIM for gNB-to-gNB co-channel CLI handling should not be further discussed.

Conclusion - No further discussion for potential enhancement to Rel-16 RIM for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD.

Overall, the Rel-16 UE-to-UE CLI should be considered as a starting point for any further enhancements to avoid having duplicated features. But some enhancements specific for dynamic TDD are still needed. The detailed enhancement directions will be discussed in section 3 and section 4.

Proposal 1: Take the existing CLI handling schemes defined in the Rel-16 as a starting point for Rel-18 enhancements on dynamic/flexible TDD.

gNB-to-gNB CLI handling

CLI measurement

In previous RAN1 meetings, the following agreements about DL channel(s)/signal(s)/measurement resource(s) used for gNB-to-gNB co-channel CLI measurement were made [4][7][8]:

Agreement(RAN1#111) For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline in RAN1 study. FFS: Whether SSB is CD-SSB or NCD-SSB In the study RAN1 assumes that exchange of configuration for NZP CSI-RS /SSB can be an enabler for gNB-to-gNB CLI measurement and/or channel measurement. Agreement(RAN1#111) For gNB-to-gNB co-channel CLI handling, beam level (i.e., based on measurement result per SSB resource and/or per CSI-RS resource) CLI measurement can be considered for study. Agreement(RAN1#112) For the study of gNB-to-gNB co-channel interference measurement, it is assumed that both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement. Agreement(RAN1#112bis-e) For the gNB-gNB co-channel CLI measurement, both RSRP and RSSI can be used as measurement metric for evaluation purposes only.

As agreed in RAN1#112bis-e, both RSRP and RSSI can be considered as measurement metric for gNB-to-gNB CLI measurement.

- RSRP measurement

Compared with RSSI measurement, it has the advantage of identifying interference sources, but the disadvantage is that it can only be used for intra-frequency CLI measurement, e.g., intra-subband CLI measurement. As agreed in RAN1#111 meeting, at least periodic NZP CSI-RS/SSB can be considered.

Regarding SSBs, they are transmitted periodically through sweeping mode, making them naturally suitable for obtaining per SSB interference. Further, the frequency domain location and time domain patterns are well designed, which benefits the information exchange of measurement resources between gNBs.

Regarding NZP-CSI-RS, it has the advantage of more flexible frequency resource configuration, so it can serve as an effective supplement to SSBs as a CLI measurement RS. In addition, it can also be used for channel measurement among gNBs for spatial domain coordination.

- RSSI measurement

For RSSI measurement, it can be based on a measurement resource. For example, the RSSI measurement for Rel-16 CLI is defined by a starting RB/symbol and a number of RBs/symbols together with a time-domain pattern given by periodicity/offset and SCS. And the measurement result represents received signal strength on the measurement resource. The interference source cannot be distinguished unless different measurement resources are associated with different aggressors. However, the advantage is that it can be used for both of intra-frequency CLI and adjacent-frequency CLI measurement, e.g., inter-subband CLI measurement.

Proposal 2: For gNB-to-gNB co-channel CLI measurement, both RSRP measurement and RSSI measurement can be considered.

- The existing measurement resource configuration for SSB/CSI-RS can be applied as baseline for gNB-to-gNB co-channel RSRP measurement.

- The existing configuration of RSSI measurement resource, e.g., defined by a starting RB/symbol and a number of RBs/symbols together with a time-domain pattern given by periodicity/offset and SCS, can be applied as baseline for gNB-to-gNB co-channel RSSI measurement.

Channel measurement

It has been agreed to consider periodic NZP CSI-RS and SSB as the baseline for gNB-to-gNB co-channel CLI measurement and/or channel measurement. Once the aggressor base station obtains the channel station information between aggressor base station and victim base station, the aggressor can perform CLI handling mechanism (e.g., beam nulling) to address the CLI. The accuracy of channel measurement between aggressor and victim directly impacts the CLI handling. The more accurate channel measurement results the aggressor get, the better performance of the CLI handling we can obtain.

According to the latest NR specification, NZP CSI-RS can be configured with up to 32 ports. However, lots of macro base stations in the field are equipped with 64 antenna ports and 192 antenna elements.

Observation 1: The existing CSI-RS can be configured with up to 32 ports, which is not sufficient for the gNB-to-gNB co-channel channel measurement for gNBs equipped with 64 antenna ports in the practice.

To get the full picture of the channel between aggressor base station and victim base station, the existing NZP CSI-RS design with up to 32 ports is not sufficient. The following alternatives can be considered for addressing this issue.

Table-1: Comparison of different alternatives

| Alternatives | Pros and Cons |
| --- | --- |
| Alt.1 | Solution: Aggressor virtualizes the 64 antenna ports into 32 CSI-RS ports and obtains the 32-port CSI between aggressor and victim. Pros: No spec impacts. Cons: If the aggressor obtains the DL channel information from its UE via SRS, the aggressor generates the DL precoding matrix W64xN, where N is the rank number. However, aggressor base station only obtains the 32-port CSI between aggressor and victim. In order to perform beam nulling at the aggressor side, the 32-port CSI is not compatible with the DL precoding matrix W64xN and impacts the accuracy of beam nulling. |
| Alt.2 | Solution: Define NZP CSI-RS with up to 64 ports. Pros: Aggressor can obtain accurate channel information between aggressor and victim. Cons: Lots of spec impacts, e.g., new resource mapping, new port mapping and etc. |
| Alt.3 | Solution: Two 32-port CSI-RS resources are grouped together to measure the CSI between aggressor and victim, which is similar to the CSI-RS pairing defined in Rel-17 Multi-TRP CSI. Pros: Aggressor can obtain accurate channel information between aggressor and victim with minor spec impacts via reusing CSI-RS pairing defined in Rel-17 Multi-TRP CSI. Cons: Aggressor needs to combine the CSI reports (e.g., PMI) from victim together in order to obtain the full CSI between aggressor and victim. |

Proposal 3: In order to perform the gNB-to-gNB co-channel channel measurement for CLI handling for gNBs equipped with 64 antenna ports, consider the following potential alternatives:

- Alt.1: Aggressor virtualizes the 64 antenna ports into 32 CSI-RS ports and obtains the 32-port CSI between aggressor and victim.

- Alt.2: Define NZP CSI-RS with up to 64 ports.

- Alt.3: Two 32-port CSI-RS resources are grouped together to measure the CSI between aggressor and victim, which is similar to the CSI-RS pairing defined in Rel-17 Multi-TRP CSI.

Tx/Rx timing adjustment for gNB-to-gNB measurement

In RAN1#112bis-e meeting, the following agreement about the issue of Tx/Rx timing adjustment for gNB-to-gNB CLI and channel measurement was made [8].

Agreement For gNB-gNB co-channel CLI measurement and channel measurement, study the impact on system performance because of CLI measurement inaccuracy at victim gNB due to misalignment between UL timing at victim gNB and DL reception timing at victim gNB of CLI measurement resource transmitted from one or more aggressor gNB. - Including potential impact on UL performance

When the aggressor transmits reference signal (e.g., CSI-RS) to victim, the victim will measure the reference signal and feedback measurement report as well. When measuring the reference signal, the victim has to consider the timing difference between aggressor and victim. Otherwise, the accuracy of channel measurement will be seriously impacted.

A field test has been carried out to show the issue of timing difference. A reference signal occupying symbol 9/10/11/12 is transmitted from the aggressor to victim. As shown in Figure-2, when the victim performs reception, it is clear that the signal is not aligned with the symbol boundary of the victim base station and the gap is larger than the cyclic prefix.

Figure-2: Timing difference showed via a field test

Observation 2: Based on the field test, a clear timing difference is observed between the symbol boundary and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

The timing difference is mainly caused by three parts, i.e., T1, T2 and T3.

- T1: DL Tx timing difference between the aggressor and the victim due to the timing alignment accuracy. As defined in [5], the maximum absolute deviation in frame start timing between any pair of TDD cells on the same frequency that have overlapping coverage areas is 3 µs, i.e., T1 ≤ 3us.

- T2: Transmission delay between the aggressor and the victim. About 1.67 µs is required under the deployment scenario of Urban Macro with 500 m inter-BS distance.

- T3: The UL symbol is shifted by NTA_offset (13 us) in the TDD system in order to allow some time for gNB to perform UL-to-DL transition without impacting the UL/DL transmission.

Taking Figure-3 as an example, it is assumed that T1 is 1us and T2 is also 1us. The exact timing difference between the DL symbol of aggressor gNB and the UL symbol of the victim gNB is T1+T3=14 us. Meanwhile, since the propagation delay is 1ms, the gap between the start of UL symbol of victim gNB and the arrival time of the CSI-RS is T1+T2+T3=15us.

Figure-3: Timing difference between different gNBs

The length of CP is 4.69 us, 2.34 us, 1.17 us and 0.57 us for 15KHz, 30KHz, 60KHz and 120KHz SCS, respectively. It is clear that timing difference caused by T1, T2 and T3 probably will be larger than the length of CP. Considering that one victim gNB may have several aggressor gNBs, then there may be different values of T1+T2 values between different victim-aggressor pairs. In order to obtain the accurate channel state information, the following methods focusing on the main part of timing difference, i.e., NTA_offset can be considered.

- Method#1: gNB sets the NTA_offset  as 0us since NTA_offset is the main contributor of the timing difference. In this case, gNB may need to reserve one symbol as the transition gap for each UL-to-DL switch.

- Method#2: Victim extracts the samples for the reference signal by deferring the starting point by 13us by implementation. This will impact the reception of signal from UE on the first symbol after the end of the reference signal, i.e., symbol 13 in Figure-2, because UE is not expected to change its UL transmission timing. Thus, one additional symbol after the reference signal needs to be muted.

- Method#3: Introduce extended CP to cover the maximum time difference.

Proposal 4: RAN1 further discusses the potential issue and solution for the timing difference observed between the symbol boundary of the victim gNB and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

Muting resource determination

During previous RAN1 meetings, the following agreements about further study UL rate matching/cancellation/muting operation were made [6][8]:

Agreement(RAN1#110bis-e) For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not. Agreement(RAN1#112bis-e) For enhancement of gNB-to-gNB co-channel CLI measurement and/or channel measurement, following options are studied for UL resource muting. - Option 1: Transparent UL resource muting method (e.g., avoid the scheduling on measurement resource) - Option 2: Non-transparent UL resource muting method (e.g., define UL resource muting pattern with one or more RE/RB muting patterns)

If the UE transmits UL signal on the measurement resources, the CLI and/or channel measurement result may be affected. Therefore, it is better not to perform the uplink transmission on the measurement resources to avoid to interfere the measurement. And a mechanism similar as DL rate matching can be introduced for UL transmission. That is, a UE can be configured with some resource of CLI and/or channel measurement resources, and the UE shall assume the resource elements occupied by resource are not available for the UL transmission.

Alternatively, UL cancellation mechanism defined for UL inter-UE multiplexing can be considered for avoiding UL interference to gNB-to-gNB CLI and/or channel measurement. The potential problem is that the resource granularity indicated by the current UL cancellation is slightly rough, and some enhancement of indication accuracy can be studied.

Proposal 5: UL resource muting should be supported for more accurate gNB-to-gNB co-channel CLI and/or channel measurement.

Regarding whether a non-transparent method should be specified or not, in our understanding, the benefit of Option 2 with at least RB level muting pattern is clear. It can improve the resource efficiency and reduce scheduling restrictions. In addition, Option 1 and Option 2 can be used for different scenarios. For example, the Option 1 can be used for PUCCH, SRS and PRACH, while Option 2 can be used for PUSCH. Both options should be viable solutions.

Considering that there is no spec impact for Option 1, the study should be focused on the Option 2.

Proposal 6: Both of transparent-based and non-transparent-based UL resource muting methods should be considered.

- For non-transparent method, at least RB level muting pattern should be supported.

- FFS: RE level muting pattern.

The muting resource for uplink transmission can be determined according to the measurement resources described in section 3.1 and 3.2. Furthermore, considering the uplink transmission on the adjacent resources of the measurement resources may also cause interference to the gNB-to-gNB co-channel CLI measurement due to leakage from the adjacent RB and the unaligned symbol boundary between the downlink signal and uplink signal at the victim gNB, so a certain guard bands need to be reserved around the measurement resources for determining the resource of UL muting resource.

Figure-4: UL muting resource determination

Proposal 7: Regarding UL resource muting, the muting resource for uplink transmission can be determined according to the measurement resources.

- The measurement resource contains resource for gNB-to-gNB CLI and channel measurement.

- FFS whether a certain guard bands need to be reserved around the measurement resources for avoiding adjacent frequency interference (e.g., leakage from the adjacent RB).

Spatial domain coordination

In previous RAN1 meetings, the following agreement about spatial domain coordination method for gNB-to-gNB co-channel CLI handling were made [4][7]:

Agreement(RAN1#111) For spatial domain coordination, the exchange of beam related information among gNB(s) (e.g., victim gNB(s) and aggressor gNB(s)) can be an enabler for inter-gNB co-channel CLI management. - For example 1 (from aggressor gNB to victim gNB), DL beam indication from aggressor gNB(s) - For example 2 (from victim gNB to aggressor gNB), preferred/restricted DL beam and associated resource configuration, beam based inter-gNB co-channel CLI measurement result from victim gNB - FFS: how to define DL beam indication - FFS: how to define DL beam Note: The above examples are only provided as starting point for further discussions Agreement(RAN1#112) For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs. Agreement(RAN1#112) For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs

Based on the discussions in previous meetings, different companies may have different understanding of the overall framework of spatial domain interference coordination, which directly affects the content of interaction information and specific mechanism for CLI handling. So first of all, we should have a common understanding of the overall framework of spatial domain coordination. And based on that, further discussion of the details.

Proposal 8: A common understanding of the overall framework of spatial domain gNB-to-gNB CLI coordination should be made firstly.

As a typical deployment described in section 2, gNB-to-gNB CLI may be asymmetric according to specific frame structure configuration, i.e., only from a Macro gNB to Micro gNB. In this scenario, it is highly likely that the Macro gNB and Micro gNB are from different vendors. Therefore, it is necessary to define a unified framework for CLI handling among these gNBs.

Regarding the framework of gNB-to-gNB CLI handling, the following requirements should be considered:

- Victim identifies interference sources and information required for CLI handling effectively

- Victim feedbacks the information required by aggressor for CLI handling

- Victim feedbacks the CLI mitigation effect of different CLI handling schemes

As depicted in Figure-5, an enhanced framework can be considered for Rel-18 dynamic/flexible TDD for CLI handling between victim and aggressor. The interaction procedures for the proposed framework are summarized below.

Figure-5: Proposed Framework for Rel-18 gNB-to-gNB CLI handling

- Step 0: Interference identification.

The victim identifies gNB-to-gNB CLI based on measurement of transmission (e.g., SSB, CSI-RS or other measurement resource) from the aggressor. It is worth noting that due to different power levels between Macro gNB and Micro gNB, the measurement RS should be transmitted by the aggressor (i.e., Macro gNB), and measured by the victim (i.e., Micro gNB) but not the other way around.

The main purpose of interference identification is to determine the high-interference gNB (aggressor) as well as high-interference beam pair. Further, channel state information of the interference channel can also be obtained through the estimation of measurement RS.

- Step 1: Victim indicates interference information (e.g., index of high-interference beam, channel state information for the interference channel, etc) identified from Step 0 to aggressor.

As the victim may be a Micro gNB with lower transmit power than aggressor (e.g., a Macro gNB). So, the interference information may not be indicated via air interface (e.g., RS) from victim to aggressor reliably. Considering that victim and aggressor are adjacent cells in the Rel-18 typical scenario. Thus, it is likely that there are backhaul links between them. Therefore, another possible option is to transmit the interference information via backhaul directly.

- Step 2: Aggressor and/or victim start to perform CLI handling schemes.

- Step 3: Aggressor transmits RS-2, which is used to assist the victim to decide whether/which solution can mitigate the gNB-to-gNB interference effectively. And the victim starts to monitor RS-2 from the aggressor once it performs the CLI handling.

- Step 4: Potential feedback can be transmitted by victim via air interface or backhaul in this step. For example, indicating whether/which solution can mitigate the gNB-to-gNB interference effectively.

Proposal 9: Rel-18 dynamic/flexible TDD can consider the following framework for CLI management,

- Step 0: The victim identifies gNB-to-gNB CLI based on measurement of reference signal from the aggressor (e.g., SSB, CSI-RS or other measurement resource);

- Step 1: The victim indicates interference information identified from Step 0, e.g., index of high-interference beam, channel state information for the interference channel, etc, to the aggressor via either air interface or backhaul;

- Step 2: The aggressor and/or victim start to perform CLI handling schemes;

- Step 3: The victim measures the reference signals sent by the aggressor to evaluate the CLI handling effect;

- Step 4: The victim feedbacks the CLI mitigation effect of the different CLI handling schemes.

                - Regarding the potential CLI handling solution in spatial domain, it can be performed by aggressor and/or victim. For example, resources to be used by the aggressor for downlink Tx and resources to be used by the victim for uplink Rx are determined according to the preset (or preconfigured) time domain pattern. More specifically, Figure-6 shows an example for resource utilization pattern defined for high-interference Tx/Rx beam pair. The high-interfering downlink Tx beam from the aggressor are only allowed to be used on part of resources(e.g., part 1) with different D/U attributes between the aggressor and the victim. And, the high-interfered uplink Rx beam of the victim will be restricted on these resources. In other words, the high-interfered uplink Rx beam of the victim can only be used in the remaining part of resource, i.e., part 2. Then the pairs of high-interference Tx and Rx beams are avoided to be used simultaneously. As another example, the aggressor may adjust the beamforming of high-interference Tx beam by considering the channel state information for the interference channel. In this way, the impact on uplink victim reception can be reduced.

Figure-6: Resource utilization pattern for high-interference Tx/Rx beam pair

Regarding related information exchange, similar mechanism as specified in Rel-17 IAB on beam information exchange for spatial domain coordination was proposed by companies. That is, preferred/non-preferred beams and associated resources are exchanged between gNBs. However, from our point of view, the terms of preferred/non-preferred beam are not suitable. Different with IAB, adjacent gNBs in Rel-18 full duplex do not have a master-slave relationship. It is more appropriate to report high-interference Tx beam index and interference strength objectively. The aggressor may take reported beam and interference strength from all the victims into account for the interference handling in the future.

Proposal 10: Spatial domain coordination can be considered by aggressor gNB and/or victim gNB for handling gNB-to-gNB CLI, e.g.,

- Some spatial domain information related to interference channel can be exchanged from victim to aggressor, such as, index of high-interference beam, channel state information,

- Resources to be used by the aggressor for high-interfering downlink Tx beams and resources to be used by the victim for high-interfered uplink Rx beams are determined according to the preset (or preconfigured) time domain pattern.,

- Adjusting the beamforming of the DL transmission by considering the channel state information of the interference channel, e.g., beam nulling.

Power control based solution

In RAN1#112bis-e meeting, the following agreement about power domain solution for gNB-to-gNB co-channel CLI handling were made [8]:

Agreement Study the effect on DL performance and the UL performance of DL Tx power adjustment to evaluate the feasibility of such scheme to overcome the gNB-to-gNB co-channel CLI. Agreement Study the effect on DL/UL performance and specification impact of applying separate open-loop/closed-loop power control parameters with cochannel CLI and without cochannel CLI for the uplink power control of a UE

As an example shown in Figure-7, two co-channel deployed neighbouring cells, i.e., cell 1 and cell 2, are configured with frame structures ‘DSUUU’ and ‘DDDSU’, respectively. gNB-to-gNB co-channel CLI will be generated in case that the UL transmissions falls on the resources configured with different attributes among neighboring cells, e.g., resources in red dotted line box. Reasonably, to overcome the impact of the gNB-to-gNB CLI to ensure the UL performance, a larger power is expected when the UL transmission is scheduled or configured on these resources. On the contrary, if UL transmission falls within the green dotted line box, gNB-to-gNB CLI will not be generated. Therefore, a lower power is sufficient to ensure the UL pereformance. It can also reduce the UE power consumption and the interference to the others.

Figure-7: Different frame structure configurations among neighboring cells

In Rel-16 URLLC, two values of P0 can be dynamic indicated to UE for PUSCH transmissions. This mechanism can be utilized to control different DG-PUSCHs to use different P0 values to fit into the different interference level (i.e., interference with or without gNB-to-gNB CLI). But for other types of UL transmissions, such as, CG-PUSCH, PUCCH, SRS and PRACH, etc., such mechanism is not supported. Power adjustment under a same closed loop control power state is not sufficient to meet the power change requirements caused by the presence or absence of CLIs. Therefore, some similar enhancements should be considered for these types of UL transmissions.

For example, configuring separate sets of power control parameters, such as, target received power(P0), pathloss compensating factor(α), closed power control loop states, configured maximum output power(), etc, for UL transmission in different resources can be considered.

Proposal 11: Regarding gNB-to-gNB CLI handling in power domain, it should be supported to configure separate sets of power control parameters, such as, target received power(P0), pathloss compensating factor(α), closed power control loop states, configured maximum output power(), etc, for UL transmission in different resources with/without gNB-to-gNB CLI.

Another way to reduce gNB-to-gNB CLI is to reduce the downlink transmission power on resource that with different attributes among neighboring cells. Then, one main issue is that the DL coverage can not be guaranteed. In addition, one question to be discussed further is whether to allow power change for measurement reference signals such as SSB/CSI-RS. If the change is supported, path loss calculation or measurement filtering of the UE is affected, and the UE needs to know the power changes of SSB/CSI-RS transmission on different time domain resources.

On the other hand, if the power of measurement reference signal remains the same, only the power of the data transmission is changed. In this case, the power offset (e.g., powerControlOffset under NZP-CSI-RS-Resource) between the PDSCHs and the CSI-RS are different in different time domain resources, and multiple sets of power offset need to be maintained, which may also complicate UE processing.

Observation 3: Regarding DL Tx power adjustment, DL coverage can not be guaranteed. And downlink measurement and power configuration may be affected.

Proposal 12: It should be de-prioritized  for mitigating gNB-to-gNB CLI through DL Tx power adjustment.

Coordinated scheduling for time/frequency resources between gNBs

In RAN1#112 meeting, the following agreement about coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB CLI handling was made [7]:

Agreement Study the benefit of knowledge among gNBs of configurations such as - SBFD time/frequency configuration

With the introduction of dynamic TDD and SBFD, D/U resources with different attributes will overlap between different gNBs, which result in gNB-to-gNB co-channel interference. The gNB-to-gNB CLI can be accurately measured and effectively coordinated only after the related configuration (e.g., SBFD time/frequency, dynamic TDD) of the neighbouring gNB is obtained. For example, measurement results of interference on SBFD and non-SBFD resources need to be separately reported. Power control parameters may also be independently configured and maintained based on SBFD and non-SBFD resources. And time-frequency resource coordination should also be performed on SBFD resources.

Observation 4: The gNB-to-gNB CLI can be accurately measured and effectively coordinated only after the related configuration (e.g., SBFD time/frequency, dynamic TDD) of the neighbouring gNB is obtained.

Proposal 13: The related configuration (e.g., SBFD time/frequency, dynamic TDD) should be exchanged among gNBs for more accurate CLI measurement and more effective CLI handling

UE-to-UE CLI handling

From our point of view, most of mechanisms defined in Rel-16 CLI can be reused for UE-to-UE CLI in Rel-18 dynamic/flexible TDD. SRS-RSRP and CLI-RSSI measurement with enhancement of L1-based reporting can be used for UE-to-UE CLI in Rel-18. Considering the difference between typical deployments in Rel-16 and Rel-18 dynamic/flexible TDD, another potential enhancement may be timing alignment for the measurement RS transmission. In addition, power control based solution can also be further considered.

L1-based CLI measurement and reporting

Timing for CLI measurement

In RAN1#112bis-e meeting, the following agreement about Tx/Rx timing adjustment for UE-to-UE CLI measurement and channel measurement was made [4]. It is

Agreement For UE-to-UE co-channel CLI measurement, study the impact on system performance because of CLI measurement inaccuracy at victim UE due to misalignment between DL reception timing at victim UE of DL channel/signal transmitted from serving gNB and DL reception timing at victim UE of CLI measurement resource transmitted from aggressor UE(s).

SRS-RSRP and CLI-RSSI measurement defined in Rel-16 CLI can be reused. One potential enhancement is about reception timing determination of SRS from aggressor UE for SRS-RSRP measurement. As shown in Figure-8, UL transmission from UE1 under Micro cell will interfere DL reception of UE2 under Macro cell by assuming the UL and the DL transmission are overlapped.

Figure-8: Timing issue for measurement RS for UE-to-UE CLI

According to mechanism in Rel-16 CLI, the UE1 transmits SRS according to its normal UL transmission timing. The SRS arrives to the UE2 earlier than the DL signal from the Macro gNB and there is a time offset between the SRS reception timing and the DL reception timing at the UE2. In Rel-16 CLI, the time offset is derived by UE implementation. However, UE2 cannot derive the time offset accurately by itself, especially in the typical deployment of Rel-18 dynamic/flexible TDD. Therefore, potential solutions on timing alignment should be reconsidered in Rel-18.

Observation 5: The UE is difficult to derive the reception timing accurately for UE-to-UE CLI measurement without any information exchange, especially in the typical deployment, e.g., HetNet, of Rel-18 dynamic/flexible TDD.

Proposal 14: Timing alignment solution on measurement RS transmission for UE-to-UE CLI should be considered in Rel-18.

- For example, exchange timing related information for reception of measurement RS.

L1-based CLI reporting

In previous RAN1 meetings, the following agreements about L1/L2 based UE-to-UE CLI measurement and reporting were made:

Agreement(RAN1#110bis-e) For UE-to-UE co-channel CLI handling, study L1/L2 based UE-to-UE CLI measurement and reporting - Note: Accounting for UE processing/reporting delay – companies to share their assumptions - Note: Proponents are encouraged to provide the mechanism of L1/L2 based CLI measurement and reporting, and to provide the benefits of L1/L2 based CLI measurement and reporting compared with existing L3 CLI/CSI measurement and report with evaluation result - Note: Accounting for information exchange delay between gNBs (if applicable) Agreement(RAN1#111) For the purpose of UE-to-UE CLI mitigation, consider the following potential enhancements: - For L1/L2 UE-to-UE CLI reporting, periodic, semi-persistent, aperiodic reporting. - FFS: Event triggered reporting. - For L1/L2 UE-to-UE CLI measurement, periodic, semi-persistent, or aperiodic measurement resource. Companies are encouraged to bring additional details and evaluation results to determine the benefit of the above potential enhancements. Agreement(RAN1#112) For L1/L2 based UE-to-UE CLI measurement, SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics. Agreement(RAN1#112) For the study of L1/L2 based UE-to-UE co-channel CLI measurement, measurement resource for CLI-RSSI measurement as defined in Rel-16 and SRS resource for SRS-RSRP measurement as defined in Rel-16 can be considered. Enhancement of measurement resource can be studied. Agreement(RAN1#112) For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the following measurement and report framework. - Use existing CSI framework as the baseline. - Others are not precluded.

In Rel-16 CLI, the L1 reporting and L3 reporting for CLI were discussed. Finally, the L3 reporting for CLI was supported due to the limited time budget. For the L3 reporting for CLI, the RRM measurement and reporting mechanism are reused. The network configures measurement resources and the report configuration for the UE. The UE may measure the configured resources to obtain the measurement results. Then the UE may report the measurement results periodically or triggered by an event by using RRC signaling after performing the L3 filtering for the measurements results.

Compared with L3 reporting, there is no L3 filtering for the measurement results for L1 reporting. Therefore, L1 reporting has the low reporting latency. It can reflect the short-term interference and the interference change better. In addition, the L1 reporting is more frequent than L3 reporting generally. From this perspective, L1 reporting is more suitable for dynamic/flexible TDD operation since the interference changes dynamically in dynamic/flexible TDD operation.

For L1 reporting for CLI, the mechanism of CSI reporting can be reused as agreed in RAN1#112. More specifically, the resources for channel measurement and interference measurement and the reporting configuration are configured by the network for CSI report. To minimize the specification efforts, the current resources configuration for L3 reporting configured by the network can be reused for L1 reporting for CLI. The network just needs to configure a reporting configuration for CLI for the UE and further associate the reporting configuration and the configured resources. Most of the reporting configuration for CSI reporting can be reused. Then the UE can report the SRS-RSRP or CLI-RSSI on the PUCCH or PUSCH.

In current specification, aperiodic reporting is supported for CSI reporting. It can reduce the reporting resource overhead and measurement effort since the gNB can trigger the reporting only when it is needed. If CSI reporting mechanism is reused for CLI, aperiodic CLI reporting can also be considered to reduce the resource overhead. When the gNB want to schedule a UE or get the interference state, it can trigger the aperiodic CLI reporting.

Another way to reduce CLI reporting overhead is to define some reporting conditions. For example, if the CLI measurement result is higher than a predefined threshold, the result will be reported. From our perspective, it can effectively reduce UE reporting overheads, and the reported information is more meaningful for UE data scheduling. Therefore, it should also be supported in Rel-18 L1-based CLI reporting.

In addition, some issues should be resolved if L1 reporting for CLI is supported, e.g., whether/how the L1 reporting and L3 reporting for the CLI co-exist with each other.

Proposal 15: L1-based reporting for UE-to-UE CLI should be considered for Rel-18 dynamic/flexible TDD.

- Reporting according to defined conditions should be supported to reduce the reporting overhead and measurement effort.

- FFS: whether/how the L1 reporting and L3 reporting for the CLI co-exist with each other.

In Rel-16 CLI, only wideband CLI measurement and reporting is supported. Considering that interference in Rel-18 full duplex may have a non-uniform feature, wideband CLI measurement and reporting may fail to reflect the changes of interference in different frequency resources. Therefore, it is necessary to introduce a finer frequency domain measurement granularity, for example, subband CLI measurement and reporting.

Observation 6: Wideband CLI measurement and reporting may fail to reflect the changes of inter-subband interference in different frequency resources.

For supporting subband CLI measurement and reporting, the current mechanism of subband CQI/PMI measurement and reporting can be considered as a starting point. A further research direction is impact of introduction of the UL subband on existing mechanism, for example, configuration and determination of the measurement subband size, and measurement reporting overheads reduction, etc.

Proposal 16: Further study subband CLI measurement and reporting for UE-to-UE CLI handling, e.g., configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc.

Enhancement for CQI reporting

Accurate CQI and CLI report are essential for network’s scheduling since gNB performs scheduling based on UE’s CQI and CLI report. When UE performs CQI measurement, the CLI will impact the measurement accuracy. For example, in case of strong CLI, UE may report low CQI to the gNB to indicate the realistic channel state. However, if gNB can avoid/mitigate the CLI to some extent, a better CQI will be used by the gNB and gNB expects UE to report a CQI in case of no CLI. From this perspective, both of the CQI without CLI and CQI with CLI are beneficial to the network. Take the following figure as an example, UE#1 performs CSI-RS measurement in slot 2 and slot 3, in which UE#1 experiences CLI from UE#2. In slot#2, UE#2 can perform UL muting to the corresponding CSI-RS resource of UE#1. In slot#3, there is no UL muting for UE#2 thus UE#1 can measure the CQI with CLI. After the measurement, UE#1 will send the CQI report without CLI (i.e., CQI measured in slot#2) and CQI report with CLI (i.e., CQI measurement in slot 3) to the gNB.

Figure-9: CQI with CLI and CQI without CLI

Proposal 17: Both the CQI with CLI and CQI without CLI (e.g., CQI measured in case of aggressor’s muting) are reported to the gNB.

Power control based solution

In RAN1#111 meeting, the following agreement about power domain solution for UE-to-UE co-channel CLI handling was made [4]:

Agreement For UE-to-UE co-channel CLI handling, study whether/how to enhance UL power control mechanism. - Existing UL power control mechanism is baseline

Similarly as discussed in section 3.6 for gNB-to-gNB CLI handling in power domain, the enhanced UL power control solution can also be supported for UE-to-UE CLI handling, through which the gNB can flexibly perform gNB-to-gNB CLI or UE-to-UE CLI handling.

Proposal 18: The unified UL power control solution applied to both of gNB-to-gNB CLI and UE-to-UE CLI handling can be considered.

Conclusion

In this contribution, we provide our analysis for potential enhancements on Rel-18 dynamic/flexible TDD and make the following observations and proposals.

Overview

Proposal 1: Take the existing CLI handling schemes defined in the Rel-16 as a starting point for Rel-18 enhancements on dynamic/flexible TDD.

gNB-to-gNB CLI

Proposal 2: For gNB-to-gNB co-channel CLI measurement, both RSRP measurement and RSSI measurement can be considered.

- The existing measurement resource configuration for SSB/CSI-RS can be applied as baseline for gNB-to-gNB co-channel RSRP measurement.

- The existing configuration of RSSI measurement resource, e.g., defined by a starting RB/symbol and a number of RBs/symbols together with a time-domain pattern given by periodicity/offset and SCS, can be applied as baseline for gNB-to-gNB co-channel RSSI measurement.

Observation 1: The existing CSI-RS can be configured with up to 32 ports, which is not sufficient for the gNB-to-gNB co-channel channel measurement for gNBs equipped with 64 antenna ports in the practice.

Proposal 3: In order to perform the gNB-to-gNB co-channel channel measurement for CLI handling for gNBs equipped with 64 antenna ports, consider the following potential alternatives:

- Alt.1: Aggressor virtualizes the 64 antenna ports into 32 CSI-RS ports and obtains the 32-port CSI between aggressor and victim.

- Alt.2: Define NZP CSI-RS with up to 64 ports.

- Alt.3: Two 32-port CSI-RS resources are grouped together to measure the CSI between aggressor and victim, which is similar to the CSI-RS pairing defined in Rel-17 Multi-TRP CSI.

Observation 2: Based on the field test, a clear timing difference is observed between the symbol boundary and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

Proposal 4: RAN1 further discusses the potential issue and solution for the timing difference observed between the symbol boundary of the victim gNB and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

Proposal 5: Regarding UL resource muting, it should be supported for more accurate gNB-to-gNB co-channel CLI and/or channel measurement.

Proposal 6: Both of transparent-based and non-transparent-based UL resource muting methods should be considered.

- For non-transparent method, at least RB level muting pattern should be supported.

- FFS: RE level muting pattern.

Proposal 7: Regarding UL resource muting, the muting resource for uplink transmission can be determined according to the measurement resources.

- The measurement resource contains resource for gNB-to-gNB CLI and channel measurement.

- FFS whether a certain guard bands need to be reserved around the measurement resources for avoiding adjacent frequency interference (e.g., leakage from the adjacent RB).

Proposal 8: A common understanding of the overall framework of spatial domain gNB-to-gNB CLI coordination should be made firstly.

Proposal 9: Rel-18 dynamic/flexible TDD can consider the following framework for CLI management,

- Step 0: The victim identifies gNB-to-gNB CLI based on measurement of reference signal from the aggressor (e.g., SSB, CSI-RS or other measurement resource);

- Step 1: The victim indicates interference information identified from Step 0, e.g., index of high-interference beam, channel state information for the interference channel, etc, to the aggressor via either air interface or backhaul;

- Step 2: The aggressor and/or victim start to perform CLI handling schemes;

- Step 3: The victim measures the reference signals sent by the aggressor to evaluate the CLI handling effect;

- Step 4: The victim feedbacks the CLI mitigation effect of the different CLI handling schemes.

Proposal 10: Spatial domain coordination can be considered by aggressor gNB and/or victim gNB for handling gNB-to-gNB CLI, e.g.,

- Some spatial domain information related to interference channel can be exchanged from victim to aggressor, such as, index of high-interference beam, channel state information,

- Resources to be used by the aggressor for high-interfering downlink Tx beams and resources to be used by the victim for high-interfered uplink Rx beams are determined according to the preset (or preconfigured) time domain pattern.,

- Adjusting the beamforming of the DL transmission by considering the channel state information of the interference channel, e.g., beam nulling.

Proposal 11: Regarding gNB-to-gNB CLI handling in power domain, it should be supported to configure separate sets of power control parameters, such as, target received power(P0), pathloss compensating factor(α), closed power control loop states, configured maximum output power(), etc, for UL transmission in different resources with/without gNB-to-gNB CLI.

Observation 3: Regarding DL Tx power adjustment, DL coverage can not be guaranteed. And downlink measurement and power configuration may be affected.

Proposal 12: It should be de-prioritized for mitigating gNB-to-gNB CLI through DL Tx power adjustment.

Observation 4: The gNB-to-gNB CLI can be accurately measured and effectively coordinated only after the related configuration (e.g., SBFD time/frequency, dynamic TDD) of the neighbouring gNB is obtained.

Proposal 13: The related configuration (e.g., SBFD time/frequency, dynamic TDD) should be exchanged among gNBs for more accurate CLI measurement and more effective CLI handling

UE-to-UE CLI

Observation 5: The UE is difficult to derive the reception timing accurately for UE-to-UE CLI measurement without any information exchange, especially in the typical deployment, e.g., HetNet, of Rel-18 dynamic/flexible TDD.

Proposal 14: Timing alignment solution on measurement RS transmission for UE-to-UE CLI should be considered in Rel-18.

- For example, exchange timing related information for reception of measurement RS.

Proposal 15: L1-based reporting for UE-to-UE CLI should be considered for Rel-18 dynamic/flexible TDD.

- Reporting according to defined conditions should be supported to reduce the reporting overhead and measurement effort.

- FFS: whether/how the L1 reporting and L3 reporting for the CLI co-exist with each other.

Observation 6: Wideband CLI measurement and reporting may fail to reflect the changes of inter-subband interference in different frequency resources.

Proposal 16: Further study subband CLI measurement and reporting for UE-to-UE CLI handling, e.g., configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc.

Proposal 17: Both the CQI with CLI and CQI without CLI (e.g., CQI measured in case of aggressor’s muting) are reported to the gNB.

Proposal 18: The unified UL power control solution applied to both of gNB-to-gNB CLI and UE-to-UE CLI handling can be considered.

Reference

- RP-221352, “Revised SID: Study on evolution of NR duplex operation”, CMCC (Moderator)

- 3GPP TSG RAN WG1 #109-e, “RAN1 Chair’s Notes”

- R1-2208483, “Discussion of evaluation on NR duplex evolution”, ZTE

- 3GPP TSG RAN WG1 #111, “RAN1 Chair’s Notes”

- 3GPP TS 38.133, Requirements for support of radio resource management_v17.6.0”, 2022-06

- 3GPP TSG RAN WG1 #110bis-e, “RAN1 Chair’s Notes”

- 3GPP TSG RAN WG1 #112, “RAN1 Chair’s Notes”

- 3GPP TSG RAN WG1 #112bis-e, “RAN1 Chair’s Notes”