3GPP TSG RAN WG1 #113		       R1-2304890

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.4

Source:	          	    Xiaomi

Title:	Discussion on UE features for NR NCR

Document for:	Decision

- Introduction

In last meeting, great progress has been made on UE features for NCR [1]. In this contribution, we present our views on the remaining issues of UE features for NR NCR.

- Discussion

The basic NCR features were agreed as FG 43-1, one remaining issue is whether this feature is supported with capability signaling or not. As the NCR cannot be supported if those basic features are not supported, those features are mandatory for NCR and no capability signaling needed for the basic features.

The features of periodic beam indication, aperiodic beam indication and semi-persistent beam indication were supported for beam indication in access link, one remaining issue is which type of indication should be mandatory supported and merged with FG 43-1. Since the aperiodic beam indication is the most flexible method, FG 43-3 as aperiodic beam indication merge with FG 43-1 is preferred.

For MAC CE updating the beam index of semi-persistent beam indication, whether this feature should be merged with FG 43-4 is still pending. Based on the agreements below [2], a separate FG is preferred to capture this feature as it is agree that the MAC-CE can be optionally provided. It should be possible for NCR that semi-persistent beam indication is supported while the MAC CE based beam index update is not supported. Therefore, FG 43-4a is not merged with FG 43-4. Another remaining issue for MAC CE updating the beam index of semi-persistent beam indication is whether the temporary should be emphasized in the description. In our understanding, as RAN1 did not discussed the duration of validation of MAC CE updated beam index, it is more straightforward that the updated beam index is valid until a new MAC CE override the previous one or new semi-persistent beam indication configured. Therefore, it is clear enough with saying that the feature is for supporting MAC CE override of RRC configuration of the beam index without mentioning temporary override.

Agreement For semi-persistent beam indication: - Alt-0: - RRC configures Y list of forwarding resource, the yth list is consist of Zy 1≤Zy≤Zmax forwarding resources, and each forwarding resource is defined by {beam index, time resource}. - The periodicity and reference SCS is configured as part of the RRC signaling for each list of forwarding resource - MAC-CE activate/de-activate one of Y list, and all the Zy forwarding resources in this list are selected. - MAC-CE can optionally provide update for Zy beam index

Proposal 1: FG 43-1 (Basic NCR support) is supported without capability signalling.

Proposal 2: Merge FG 43-3 (aperiodic beam indication for access link) with FG43-1 (Basic NCR support).

Proposal 3: FG 43-4a (Beam index updates for semi-persistent beam indication) is not merged with FG 43-4(Semi-persistent beam indication for access link).

Proposal 4: Delete the temporary in the description of FG 43-4a (Beam index updates for semi-persistent beam indication).

- Conclusion

In this contribution, we present our views on the remaining issues on UE features for NR NCR, we have the following proposals.

Proposal 1: FG 43-1 (Basic NCR support) is supported without capability signalling.

Proposal 2: Merge FG 43-3 (aperiodic beam indication for access link) with FG43-1 (Basic NCR support).

Proposal 3: FG 43-4a (Beam index updates for semi-persistent beam indication) is not merged with FG 43-4(Semi-persistent beam indication for access link).

Proposal 4: Delete the temporary in the description of FG 43-4a (Beam index updates for semi-persistent beam indication).

Reference

[1] R1-2304227, Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e, April 17th – 26th, 2023

[2] Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0, Athens, Greece, 27th February – 3rd March 2023