3GPP TSG-RAN WG1 Meeting #113	R1-2305057

22 – 26 May 2023, Incheon , KR

Agenda item:	9.17

Source:	Vodafone, vivo

Title:	Open issues in Higher Layer Parameters for BWP without restrictions

Document for:	Decision, Discussion

# 1	Introduction

This contribution is aiming to address the open issues on the Higher Layer Parameters for the “Complete the specification support for BandWidth Part operation without restriction in NR” Work Item approved in [1], with the following objectives:

- For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on using SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD-SSB and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to Interoperability Device Testing (IoDT) availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

This contribution presents our views both as a Rapporteur and Operator and it is based on the discussion held in previous meetings.

# 2	UE features for BWP without restrictions discussion

As it can be seen in the most updated Higher Layer Parameters list after RAN1#112-bis-e in R1-2304221 [2], the following topic is still under discussion for Options B-1-1 and B-1-2:

Issue 1: Explicit or Implicit configuration of Options B-1-1 and B-1-2

## 2.1 Issue 1

From our point of view, an explicit configuration for Options B-1-1 and B-1-2 is preferrable as it provides a clearer alignment of the gNB configuration respecting the reported UE capabilities accordingly. This seems to be aligned with the RAN2 principle that discourages implicit gNB configuration based on the reported UE capabilities. Nonetheless, this issue should be evaluated in RAN2 and thus we propose to keep the rows as “Unstable”, and in the LS to be sent to RAN2 by the end of the meeting, indicate that RAN2 should make the final decision on this issue.

Proposal 1: Explicit RRC configuration of Options B-1-1 and B-1-2 is preferrable as it provides a clearer alignment of the gNB configuration with respect to the reported UE capabilities. Final decision should be done by RAN2 on this issue.

# 4	Conclusions

In this contribution, the following proposal is made:

Proposal 1: Explicit RRC configuration of Options B-1-1 and B-1-2 is preferrable as it provides a clearer alignment of the gNB configuration with respect to the reported UE capabilities. Final decision should be done by RAN2 on this issue.

# References

- RP-2300805, “Complete the specification support for Bandwidth Part operation without restriction in NR”, Moderator (Vodafone), vivo, 3GPP RAN#99, 20-23 March 2023.

- R1-2304221, “Collection of updated higher layers parameter list for Rel-18  NR”, Moderator (Ericsson), 3GPP RAN1#112-bis-e, 17-26 April 2023.

- R1-2304220, “Summary of Email discussion on Rel-18 RRC parameters for LS to RAN2/RAN3”, Moderator (Ericsson), 3GPP RAN1#112-bis-e, 17-26 April 2023.