3GPP TSG RAN WG1 #113		 R1-2305333

Incheon, KR, May 22nd – May 26th, 2023

Agenda item:	9.2.4.2

Source: 	Qualcomm Incorporated

Title: 	Other aspects on AI-ML for positioning accuracy enhancement

Document for:		Discussion and Decision

1 Introduction

At RAN #94, a new study on artificial intelligence/machine learning for NR air interface has been approved [1] with the following goals briefly summarized as below.

- For the initial set of use cases CSI feedback enhancements, beam management, positioning accuracy improvements for different scenarios were considered

- For each of the above use cases, a representative set of sub use cases for characterization and baseline performance evaluation to be finalized by RAN#98.

- AI/ML model, terminology, and description to identify common and specific characteristics for framework investigations including

  - Characterize the defining stages of AI/ML related algorithms and associated complexity

  - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases

  - Characterize life cycle management of the AI/ML model

  - Dataset(s) for training, validation, testing and inference

  - Identify common notation and terminology for AI/ML related functions, procedures, and interfaces

- For the use cases under consideration:

  - Evaluate performance benefits of AI/ML models for the agreed use cases

    - Evaluation methodology for link and system level simulations

    - KPIs and corresponding requirements

  - Assess potential specification impact, specifically for the targeted use cases for a common framework

    - PHY layer aspects

    - Protocol aspects

    - Interoperability and testing aspects

In this document, we further elaborate on various aspects and specification impacts related to AI/ML positioning model development, training, data collection, LCM (Life Cycle Management), inference, and monitoring. We provide updated evaluations in our companion paper [2]. Our previous discussions on other aspects related to AI/ML positioning can also be found in  [3] [4] [5] [6] [7] [8].

2 Scope of study and representative sub use cases for AI/ML positioning

In previous RAN1 meeting (RAN1-111), companies agreed to choose direct AI/ML and assisted AI/ML positioning approaches as representative sub use cases for AI/ML positioning. We consider the following flavours for AI/ML positioning according to the selected sub use cases.

For Direct AI/ML positioning, we consider RF (Radio Frequency) Fingerprinting (RFFP) in which the UE learns a direct relationship from the channel observations to the UE position. RFFP can have multiple flavours as illustrated in Appendix 1, including UE-based, UE-assisted, and NG-RAN node assisted methods.

For AI/ML assisted positioning, we consider ML-based soft information reporting in which the UE derives the distribution of the LOS (Line of Sight) delay and angle from the channel observations of a given TRP, transforms it into a likelihood function and computes the UE position by combining the likelihoods across all the TRPs. See Appendix 2 for further details.

3 Discussion on general AI/ML positioning framework

## 3.1 AI/ML positioning deployment cases

Agreement (RAN1-110be)

- Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Agreement (RAN1-110be)

- Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

In the previous meeting (RAN1-110be), companies agreed on five cases for mapping AI/ML positioning to network entities and positioning type.

With the understanding that these use cases have been defined from the perspective of how inference is performed at deployment, we illustrate our understanding of the above listed cases in Figure 1.

Figure 1: Cases of AI/ML positioning (D-AIML: Direct AI/ML positioning, A-AIML: AI/ML assisted positioning).

From the figure, we note that in Case 1 and Case 2a, the AI/ML model inference runs at the UE side, while for the other cases, i.e., Case 2b, Case 3a, and Case 3b, the AI/ML positioning model runs at the network side. Within the network, in Case 2b and Case 3b, the model runs at the LMF (Location Management Function) while in Case 3a, the model may run at the gNB where the UL (uplink) measurements are collected and processed.

We first note that each of these cases is not exclusive, and one or more types can be run to achieve a certain positioning function. For example, the 3gpp network may choose to run a combination of Case 2a and Case 3a for multi-RTT settings. Similarly, the network may run a combination of Case 3a and Case 3b to reduce overhead of signalling from the gNB/TRP to the LMF.

Observation 1: For AI/ML positioning, a combination of the defined cases (i.e., Case1 to Case3b) may be used to achieve improved positioning in a defined setting.

We find that these cases are sufficient to study specifications needed to enable of AI/ML positioning on both UE and network sides. Other new cases can also be discussed unless they can be realized by implementation, or they would not have additional specification impacts beyond the agreed ones.

## 3.2 AI/ML positioning model development

Our companion paper  [9] lays out the various considerations in the model training process in a detailed fashion, and we briefly summarize in this section some aspects relevant to the positioning use case. Training data may be initially generated at the UE or the network and may be subsequently sent to one or more data collection entities. Once large-scale field data is collected at the data collection entity, the data should be made available to the vendors responsible for model development. The decisions on the model structure, device specific optimizations and the number of models to develop (generalizable vs. specific models) and the rate at which models should be updated depend on the field data, the network deployment parameters (e.g., direct vs. AI/ML assisted, synchronized vs. non-synchronized, etc.), availability of ground truth and the expected changes in the environment. Model development also requires careful implementation consideration for power consumption, hardware area, latency, and concurrency with other PHY/MAC functionalities and require extensive testing. Therefore, the model development is best to be done by the vendor who implement the device where the AI/ML model inference runs.

Observation 2: For AI/ML positioning model development, the development of the model needs to be done by the vendor who implements the device where the AI/ML model inference runs.

For Case1 and Case2a, where the inference is at UE side and based on PRS (position reference signal), it is best if the model training is performed at the UE side and similarly for Case2b, Case3a and Case3b where the inference is based on a UE’s report or UE transmission, the data collection and training can naturally happen at the network.

Proposal 1: For AI/ML positioning model training, consider model training and development at the side for which the inference of model is expected to be performed:

- Case1: Model training and inference at UE side

- Case2a: Model training and inference at UE side

- Case2b: Model training and inference at network side

- Case3a: Model training and inference at network side

- Case3b: Model training and inference at network side

For UE side, training can happen at UE vendor/chip OTT (Over-The-Top) server or a third-party server, and similarly, for network side, the model training can happen at a network entity or network vendor OTT server.

Proposal 2: For AI/ML positioning model training, modes of operation in which model training and inference happen at different sides are deprioritized for the current AI/ML positioning study.

## 3.3 AI/ML positioning feature, conditions, configurations, and functionality/model LCM aspects

In the general AI/ML framework, a working assumption considered functionality and model identification for sake of easing discussion on model management and LCM.

### 3.3.1 AI/ML positioning functionalities and model identification

Agreement RAN1-112-9.2.1

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Agreement RAN1-112-9.2.1

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Our companying paper [9] provides more discussion on functionality and model identification. In this section, we discuss the applicability of functionality and model identification to AI/ML positioning. In functionality identification, the UE and LMF can identify an AI/ML positioning functionality. It is also possible to have multiple functionalities under the umbrella of AI/ML positioning. We find identifying functionality and/or model between UE and LMF can be beneficial to LCM operation of AI/ML positioning.

The functionality identification can be helpful to manage AI/ML positioning on a UE basis, activating/deactivating a functionality depending on functionality condition at UE side. The functionality LCM can also be helpful to lightly letting LMF manage AI/ML positioning at UE side without micromanaging UE-sided models. Companies can discuss the need to consider any granularity for functionalities and capabilities/conditions and their mapping to the different cases.

For model identification, the UE or LMF can exchange model IDs for AI/ML positioning. For example, the model identification may help LMF apply finer granularity for controlling positioning at UE side, especially when the model maps to some dynamic operations such as operation on multiple sites or multiple operation settings (e.g., multiple frequency bands). The model identification can also help manage AI/ML positioning for a group of UEs sharing the same model ID whereby LCM operation can be applied on a model basis and UEs using the same model can be managed through a global LCM operation.

As a starting point, the first key step can be to identify the AI/ML positioning functionalities between UE and LMF. Identifying model ID between parties can be taken as a subsequent stage if needed. The same discussion above can also be considered for functionality and model identification between LMF and gNB/TRP.

### 3.3.2 AI/ML positioning conditions and configurations

Agreement: RAN1-110be-9.2.4.2

Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement

- Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc.

- Model capability, e.g., positioning accuracy quality and model inference latency

- Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information

- Note: other aspects are not precluded

Agreement RAN1-112be-9.2.4.2

Regarding LCM of AI/ML based positioning accuracy enhancement, at least for Case 1 and Case 2a (model is at UE-side), further study the following aspects on information related to the conditions

- What are the conditions for functionality-based LCM

  - which aspects should be specified as conditions of a Feature/FG available for functionality

- What are the conditions for model-ID-based LCM

  - Which aspects should be considered as additional conditions, and how to include them into model description information during model identification

Agreement: RAN1-110be-9.2.4.2

Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement

- Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc.

- Model capability, e.g., positioning accuracy quality and model inference latency

- Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information

- Note: other aspects are not precluded

Agreement RAN1-112be-9.2.4.2

Regarding LCM of AI/ML based positioning accuracy enhancement, at least for Case 1 and Case 2a (model is at UE-side), further study the following aspects on information related to the conditions

- What are the conditions for functionality-based LCM

  - which aspects should be specified as conditions of a Feature/FG available for functionality

- What are the conditions for model-ID-based LCM

  - Which aspects should be considered as additional conditions, and how to include them into model description information during model identification

To enable AI/ML positioning, the UE side can indicate a capability that can be mapped to functionalities and/or models for UE-sided AI/ML positioning, e.g., as supported feature groups for positioning. The indication of capabilities can be mapped to conditions and additional conditions, where the additional conditions can correspond to supporting AI/ML positioning for temporary or local operation settings.

The UE may indicate conditions that help the LMF identify potential functionalities and models supported at the UE side. In previous meetings, companies agreed to study and provide inputs for conditions that can describe UE capabilities for AI/ML positioning. Companies also agreed to provide inputs regarding conditions/additional conditions that can be helpful for functionality and model-ID LCM. We think conditions for functionality and model can be conveyed using UE features and feature groups (FGs). As a starting point, the conditions need to consider resource and measurement report components needed for AI/ML positioning in a manner that is equivalent to the style and content of FGs considered for legacy positioning. The conditions can map directly to component description of FGs. The conditions can cover details on supported DL PRS resource capabilities, DL PRS measurement reporting, generalization capabilities and assumptions, etc. Once the LMF receives the FGs for AI/ML positioning, it can configure the AI/ML positioning functionality or model as suitable.

Observation 3:  For legacy positioning, UE capabilities for legacy positioning methods are conveyed as part of the UE feature and feature group (FG) listing, and LMF uses these FGs/components to configure legacy positioning.

Proposal 3: For AI/ML positioning functionality and model identification, as a starting point, consider the content of existing legacy positioning FGs to define conditions for functionality and model operation.

Proposal 4: For AI/ML positioning functionality and model identification, consider at least the following aspects for defining conditions:

- DL PRS resource conditions

- DL PRS measurement report conditions

- Generalization conditions

Additional conditions can also be considered to identify dynamic support for AI/ML positioning. For example, the UE moves across sites and additional conditions can be exchanged between LMF and UE to signal these changes and indicate scenarios of interest. Hence, the UE can indicate additional conditions to explicitly tell the LMF that it may have an AI/ML positioning functionality/model for a scenario. The LMF may also send the UE indicators for additional conditions to help or trigger UE to indicate any additional conditions that can be available. The additional conditions can be bidirectional and may be also mapped to model ID. The LMF can then send additional configurations to activate/deactivate and/or select and switch AI/ML functionalities and models, respectively. Examples of additional conditions can be validity area for AI/ML positioning (expressed as zone info [lat./long./alt.] or cell-ID [phy./global]).

Observation 4:  For AI/ML positioning functionality and model identification, the additional conditions can be bidirectional and sent back and forth between the UE and LMF.

Observation 5:  For AI/ML positioning functionality and model identification, the additional conditions can relate to a scenario and/or model ID.

We also want to mention that the above discussion can also be considered for identifying AI/ML positioning functionality and model between LMF and gNB/TRP.

### 3.3.3 AI/ML positioning functionality-based and model-based LCM

Agreement RAN1-110be-9.2.1

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

Agreement RAN1-112-9.2.1

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement RAN1-110be-9.2.1

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

Agreement RAN1-112-9.2.1

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

The general AI/ML framework also discussed options for deciding on LCM aspects by UE and network.

We find that mapping of decision to different LCM aspects for AI/ML positioning functionalities depends on the deployment case (i.e., Case1 to Case3b).

For UE-sided operation (Case1 and Case2a), when it comes to activate, deactivate, or fallback a given functionality, we consider the decision to be taken by LMF. It is possible that this to be initiated by either UE-side or network-side. For example, if UE side believes that the model it supports for a given functionality is outdated, then it can request the LMF to deactivate the functionality and offer a fallback option. If UE also finds it can provide excellent performance using one of the supported functionalities (e.g., Case2a with RSTD reporting), it can request the LMF to get this functionality enabled. This can be dynamically announced by the UE as it moves into a new cell (e.g., as part of dynamic condition indication). The same argument applies to gNB/TRP-sided functionalities (Case3a). The LMF can decide on activation, deactivation, or fallback for gNB/TRP-sided positioning functionalities but it is also possible for gNB/TRP to initiate the process. For LMF sided functionalities (Case2b and Case3b), we find the decision on activation, deactivation, and fallback is also LMF-centric. Though we still find it possible to let UE side and gNB/TRP side initiate the process of activation, deactivation, and falling back. This can be helpful when UE or gNB/TRP can monitor changes in the environment based on the measurements. For example, if the UE monitors measurements and believes the environment has significantly changed, then it can suggest the LMF to deactivate a running LMF-sided positioning functionality. The above discussion can also be considered similarly to model LCM decisions and operations.

Observation 6:  For AI/ML positioning functionality-based LCM (Case1 and Case2a), it is beneficial for letting both LMF and UE initiate the process for activation, deactivation, switching, and falling back of a functionality.

Observation 7:  For AI/ML positioning model-based LCM (Case1 and Case2a), it is beneficial for letting both LMF and UE initiate the process for activation, deactivation, selection, switching of a model.

## 3.4 AI/ML positioning collaboration levels

As we indicated before, for the UE-sided models (Case1 and Case2a), the model development happens at the UE side, and with functionality-based LCM, the UE-side can handle model deployment and update in a transparent fashion to the network. Therefore, there is no strong need to make model deployment and delivery be part of collaboration between the UE and network (e.g., LMF). The same concept applies for gNB/TRP sided models. The model delivery, deployment, and update can happen in a transparent manner to the LMF. For LMF-sided models, it is not expected that UE or gNB/TRP to deliver or update models at LMF side. Therefore, there is no need to consider any collaboration between these parties beyond Level y.

Proposal 5: For collaboration in AI/ML positioning (Case1, Case2a, and Case2b), consider Level-x and Level-y collaboration between UE and LMF.

Proposal 6: For collaboration in AI/ML positioning (Case3a and Case3b), consider Level-x and Level-y collaboration between gNB/TRP and LMF.

4 Discussion on AI/ML model training & data collection

## 4.1 Generating ground truth labels for different cases

Agreement: RAN1-111-9.2.4.2

Regarding data collection for AI/ML model training for AI/ML based positioning,

- The following options of entity and mechanisms to generate ground truth label are identified for further study

  - For direct AI/ML positioning, ground truth label is UE location

    - PRU with known location

    - UE generates location based on non-NR and/or NR RAT-dependent positioning methods

    - LMF generates UE location based on positioning methods

    - LMF with known PRU location

    - Note: user data privacy needs to be preserved

  - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output

    - PRU generates label directly or calculates based on measurement/location

    - UE generates label directly or calculates based on measurement/location

    - Network entity generates label directly or calculates based on measurement/location

- The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study

  - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b)

    - PRU

    - UE

  - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b)

    - TRP

  - Note: other options of entity to generate other training data are not precluded

- Note: Existing PRU definition is in 38.305

Agreement: RAN1-111-9.2.4.2

Regarding data collection for AI/ML model training for AI/ML based positioning,

- The following options of entity and mechanisms to generate ground truth label are identified for further study

  - For direct AI/ML positioning, ground truth label is UE location

    - PRU with known location

    - UE generates location based on non-NR and/or NR RAT-dependent positioning methods

    - LMF generates UE location based on positioning methods

    - LMF with known PRU location

    - Note: user data privacy needs to be preserved

  - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output

    - PRU generates label directly or calculates based on measurement/location

    - UE generates label directly or calculates based on measurement/location

    - Network entity generates label directly or calculates based on measurement/location

- The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study

  - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b)

    - PRU

    - UE

  - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b)

    - TRP

  - Note: other options of entity to generate other training data are not precluded

- Note: Existing PRU definition is in 38.305

Companies identified initial listing of entities for collecting training data, including ground truth labelling and measurement for direct AI/ML and AI/ML assisted positioning. Other entities are for further studies.

Agreement RAN1-112-9.2.4.2

Regarding training data generation for AI/ML based positioning,

- The following options of entity and mechanisms to generate ground truth label are identified

  - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a)

  - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label

    - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods

    - Network entity generates ground truth label based on positioning methods

- The following options of entity to generate other training data (at least measurement corresponding to model input) are identified

  - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b)

    - PRU

    - UE

  - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b)

    - TRP

- Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study

Agreement RAN1-112-9.2.4.2

Regarding training data generation for AI/ML based positioning,

- The following options of entity and mechanisms to generate ground truth label are identified

  - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a)

  - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label

    - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods

    - Network entity generates ground truth label based on positioning methods

- The following options of entity to generate other training data (at least measurement corresponding to model input) are identified

  - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b)

    - PRU

    - UE

  - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b)

    - TRP

- Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study

The previous agreements considered multiple options for entities to collect ground truth labels for direct and AI/ML assisted positioning approaches with proper mapping to different cases. Other remaining entities that also need to be considered is the UE to generate ground truth using NR and non-NR positioning methods with potential LMF assistance. In previous meeting, few companies had concerns about feasibility of training with labels generated using NR RAT-dependent positioning methods. While we partially share the concern that some labels obtained using NR RAT-dependent positioning methods may be too noisy in extreme NLOS conditons, it may not be the case under LOS and mild NLOS settings. Our understanding is that other companies also showed performance enhancements when considering semisupervised learning with unlabeled samples. By carefully labelling samples using NR RAT-dependent positioning methods, it should in principle provide at least comparable performance to semisupervised methods.

Observation 8: For AI/ML positioning data collection, labels generated using NR RAT-dependent positioning methods and captured in LOS or mild NLOS conditions can help enhance training performance and reduce the burden of expensive labelling.

Proposal 7: For AI/ML positioning data collection, consider the following additional entities and mechanisms to generate ground truth label:

- For direct AI/ML positioning, ground truth label is UE location

  - Case1

    - UE generates location based on non-NR and/or NR RAT-dependent positioning methods with network assistance (as applicable).

- For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameters(s) corresponding to AI/ML model output

  - Case1/Case2a

    - UE generates label directly or calculates based on measurement/location with network assistance.

## 4.2 Assistance for data collection

The transfer of training data from the data source entities (e.g., UE, PRU, gNB/TRP, and LMF) to training entities or any data repository is out of the scope of RAN1 and can be left to other groups (e.g., RAN2/3 and SA groups).

Observation 9: For AI/ML positioning data collection, transfer of training data (i.e., measurements, ground truth labels, assistance information) from source entities to training entities is out the scope of RAN1.

### 4.2.1 Signalling for requesting labelling assistance

Computing high quality ground truth labels using NR positioning methods requires information about anchor locations, beam angle information, timing errors, etc. In addition, computing high quality ground truth labels for AI/ML assisted positioning also requires knowledge of target location. Such information can be readily available at LMF side using existing LPP and NRPPa procedures. While such information for computing ground truth label can be available at LMF side, it may not be always available at UE, PRU, and TRP sides. Therefore, there is a need for LMF to provide some signalling for labelling assistance to UE and gNB/TRP sides.

Observation 10: For AI/ML positioning data collection (Case2b and Case3b), LMF can leverage information obtained using existing LPP and NRPPa procedures to compute ground truth labels.

Observation 11: For AI/ML positioning data collection (Case1, Case2a and Case3a), UE and gNB/TRP require assistance from LMF to compute ground truth labels.

For Case1 and Case2a, the LMF can compute ground truth labels using NR positioning method and send it back to UE. The UE can also provide the LMF with an initial estimate of its location (e.g., using a non-NR positioning method), then the LMF utilizes this initial estimate to obtain a refined ground truth label for Case1. For Case2a, since the LMF can have access to TRP locations, beam angle information, and timing errors, the LMF can leverage the reported initial UE location and compute ground truth label for AI/ML assisted positioning, and then send it back to the UE. The LMF may also provide the UE with information needed to compute the ground truth label at UE side in a way like UE-based positioning methods.

For Case3a, the ground truth label corresponds to AI/ML assisted positioning. Computing the ground truth label requires knowledge about both TRP locations and target location. Due to privacy and security concerns, it can be difficult to share target location with the gNB/TRP. Instead, the LMF can compute the label for AI/ML assisted positioning and send it back to gNB/TRP. For example, the LMF may compute target location using NR positioning method (or rely on known PRU location), and then leverage the computed target location (or PRU location) to find the ground truth label. For computing the target location, the LMF may rely on measurements reported by UE, PRU (e.g., PRS measurements) or other gNBs/TRPs (e.g., SRS measurements).

Proposal 8: For AI/ML positioning data collection, consider signalling for labelling assistance from LMF in Case1, Case2a, and Case3a.

### 4.2.2 Other assistance information for data collection

Companies also agreed on a working assumption that lists information that can be helpful for data collection and have potential specification impact. We discuss other information that can also be beneficial for AI/ML positioning data collection and training.

Working Assumption RAN1-112be-9.2.4.2

Regarding data collection at least for model training for AI/ML based positioning, at least the following information of data with potential specification impact are identified.

- Ground truth label

  - At least for model training

  - Report from the label data generation entity

- Measurement (corresponding to model input)

  - At least for model training

  - Report from the measurement data generation entity

- Quality indicator

  - For and/or associated with ground truth label and/or measurement at least for model training

  - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity

- RS configuration(s)

  - At least for deriving measurement

  - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP

  - Note1: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement

- Time stamp

  - At least for and/or associated with training data for model training

    - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities

  - Report from data generation entity together with training data and/or as LMF assistance signaling

  - Note2: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement

- FFS other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) for data collection

- Note3: whether the above information can be applied to other aspects of AI/ML LCM (e.g., updating, monitoring, etc.) can also be discussed

- Note4: transfer of data from the entity generating data to a different entity is not precluded from RAN1 perspective

Working Assumption RAN1-112be-9.2.4.2

Regarding data collection at least for model training for AI/ML based positioning, at least the following information of data with potential specification impact are identified.

- Ground truth label

  - At least for model training

  - Report from the label data generation entity

- Measurement (corresponding to model input)

  - At least for model training

  - Report from the measurement data generation entity

- Quality indicator

  - For and/or associated with ground truth label and/or measurement at least for model training

  - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity

- RS configuration(s)

  - At least for deriving measurement

  - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP

  - Note1: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement

- Time stamp

  - At least for and/or associated with training data for model training

    - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities

  - Report from data generation entity together with training data and/or as LMF assistance signaling

  - Note2: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement

- FFS other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) for data collection

- Note3: whether the above information can be applied to other aspects of AI/ML LCM (e.g., updating, monitoring, etc.) can also be discussed

- Note4: transfer of data from the entity generating data to a different entity is not precluded from RAN1 perspective

Associated information can be beneficial for model training and development, inference, and functionality- & model- LCM. The training entity can leverage associated information to group collected data and have proper training by considering multiple models or consider a mixed dataset training. The model developer can use some of the associated information as input to the model. The model developer can also exploit the associated information to annotate the model with meta information, where this meta information can be leveraged to select the right model or decide switching between models during LCM operation. The associated information can also be useful to set applicability and validity conditions for AI/ML positioning functionality. We list some of the remaining assistance information to be included.

Indication of how PRS resources map to physical anchor location/angles: The mapping between positioning resources and TRPs along with beam configurations is another important associated information for training and inference. The positioning ML model can learn weights that are TRP and beam specific. During inference, the UE needs to know how resources map to TRPs and beams, when compared to the time of training. This helps the UE better map measurements to ML model inputs. It is possible that network configurations change over time in which the mapping of positioning resources to TRPs and beams get changed. Therefore, it is quite important to indicate this mapping as part of associated information reported within assistance data or positioning response.

Indications of timing errors: While UE and network devices (i.e., gNB/TRP) are expected to be well-calibrated, it is common in practice to have some residual calibration errors, and these residual errors can negatively affect the precise positioning performance. Thus, it is beneficial to include expected timing errors at network and UE side as part of the associated information. This helps the model developer to professionally train the model while accounting for the expected timing errors in existing deployments. Examples of timing errors include TRP TX/RX timing error, UE TX/RX timing errors, and inter-TRP synchronization errors. Our understanding is that some of the above timing errors are already supported as optional features in specifications. In AI/ML positioning, indicating such information to UE side can help achieve a better training strategy at the UE side and can also help the UE side select the right model that fits the expected timing error ranges during inference.

Indication of LOS/NLOS maps for each TRP: The network may provide the UE with LOS/NLOS maps for each TRP, which it has aggregated using information from multiple UEs over time. This enables the UE to selectively process TRPs, reduce outliers and improve positioning accuracy.

Observation 12: In AI/ML positioning data collection, the following information can be associated with data collected at UE/PRU side:

- Indication of how PRS resources map to physical anchor location/angles: UE side need to know how different PRS resource sets/resources are mapped to physical anchor location and beam angles.

- Indication of timing errors at network side: UE side can benefit from knowing expected ranges/distributions/indexing of timing errors at network side (e.g., TRP TX timing errors, inter-TRP synchronization errors) for better training and model development.

- Indication of LOS/NLOS maps for each TRP: UE side can benefit from knowing the map of potential LOS/NLOS states for each combination of TRP and PRS resource (if available from LMF side).

Proposal 9: In AI/ML positioning data collection, consider the following associated information for data collection at UE/PRU (Case1 and Case2a):

- Indication of how PRS resources map to physical anchor location/angles

- Indication of timing errors at network side: E.g., TRP TX timing errors, inter-TRP synchronization errors.

- Indication of LOS/NLOS maps for each TRP: E.g., map of potential LOS/NLOS states for each combination PRS/TRP (if available from LMF side).

5 Discussion on AI/ML positioning monitoring

Agreement: RAN1-111-9.2.4.2

- Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects

- At least the following are identified for further study as potential data for calculating monitoring metric

  - If monitoring based on model output

    - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning

  - If monitoring based on model input

    - E.g., measurement corresponding to model inference input

  - Note1: other type of potential data for model monitoring is not precluded

  - Note2: combination of one or more type of potential data for monitoring is not precluded

- If a given type of data is necessary for calculating monitoring metric, study whether and if so

  - How an entity can be used to provide the given type of data for calculating monitoring metric

    - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case

  - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric

  - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric

  - Potential UE-network interaction

    - E.g., model monitoring decision indication between UE and network

Agreement: RAN1-111-9.2.4.2

- Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects

- At least the following are identified for further study as potential data for calculating monitoring metric

  - If monitoring based on model output

    - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning

  - If monitoring based on model input

    - E.g., measurement corresponding to model inference input

  - Note1: other type of potential data for model monitoring is not precluded

  - Note2: combination of one or more type of potential data for monitoring is not precluded

- If a given type of data is necessary for calculating monitoring metric, study whether and if so

  - How an entity can be used to provide the given type of data for calculating monitoring metric

    - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case

  - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric

  - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric

  - Potential UE-network interaction

    - E.g., model monitoring decision indication between UE and network

In the previous meeting, companies agreed to a listing for entities to conduct monitoring and study the feasibility, benefits, and potential specification impacts for different aspects of monitoring.

Agreement RAN1-112-9.2.4.2

Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects

- Entity to derive monitoring metric

  - UE at least for Case 1 and 2a (with UE-side model)

    - FFS PRU for Case 1 and 2a

  - gNB at least for Case 3a (with gNB-side model)

    - FFS gNB for Case 3b (with LMF-side model)

  - LMF at least for Case 2b and 3b (with LMF-side model)

  - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b)

- If model monitoring does not require ground truth label (or its approximation).

  - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc.

  - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc.

  - report of the calculated metric and/or model monitoring decision

- If model monitoring requires and is provided ground truth label (or its approximation)

  - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc.

  - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc.

  - report of the calculated metric and/or model monitoring decision

- Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded

Agreement RAN1-112be-9.2.4.2

Regarding monitoring for AI/ML based positioning, at least the following entities are identified to derive monitoring metric

- UE at least for Case 1 and 2a (with UE-side model)

- gNB at least for Case 3a (with gNB-side model)

- LMF at least for Case 2b and 3b (with LMF-side model)

Agreement RAN1-112be-9.2.4.2

Regarding monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s), feasibility, necessity and potential specification impact for each case (Case 1 to 3b)

- Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring

- Assistance signaling from UE/PRU for network-side model monitoring

- Model monitoring based on provided ground truth label (or its approximation)

  - Monitoring metric: statistics of the difference between model output and provided ground truth label

  - Provisioning of ground truth label and associated label quality

- Model monitoring using at least statistics of measurement(s) without ground truth label

  - Monitoring metric: e.g., statistics of measurement(s) compared to the statistics associated with the training data

  - Note1: the measurement(s) may or may not be the same as model input

- Note2: other monitoring methods (e.g., based on statistics of model output without ground truth label, based UE motion sensor and/or jointly based on multiple monitoring metrics) are not precluded

Agreement RAN1-112-9.2.4.2

Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects

- Entity to derive monitoring metric

  - UE at least for Case 1 and 2a (with UE-side model)

    - FFS PRU for Case 1 and 2a

  - gNB at least for Case 3a (with gNB-side model)

    - FFS gNB for Case 3b (with LMF-side model)

  - LMF at least for Case 2b and 3b (with LMF-side model)

  - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b)

- If model monitoring does not require ground truth label (or its approximation).

  - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc.

  - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc.

  - report of the calculated metric and/or model monitoring decision

- If model monitoring requires and is provided ground truth label (or its approximation)

  - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc.

  - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc.

  - report of the calculated metric and/or model monitoring decision

- Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded

Agreement RAN1-112be-9.2.4.2

Regarding monitoring for AI/ML based positioning, at least the following entities are identified to derive monitoring metric

- UE at least for Case 1 and 2a (with UE-side model)

- gNB at least for Case 3a (with gNB-side model)

- LMF at least for Case 2b and 3b (with LMF-side model)

Agreement RAN1-112be-9.2.4.2

Regarding monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s), feasibility, necessity and potential specification impact for each case (Case 1 to 3b)

- Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring

- Assistance signaling from UE/PRU for network-side model monitoring

- Model monitoring based on provided ground truth label (or its approximation)

  - Monitoring metric: statistics of the difference between model output and provided ground truth label

  - Provisioning of ground truth label and associated label quality

- Model monitoring using at least statistics of measurement(s) without ground truth label

  - Monitoring metric: e.g., statistics of measurement(s) compared to the statistics associated with the training data

  - Note1: the measurement(s) may or may not be the same as model input

- Note2: other monitoring methods (e.g., based on statistics of model output without ground truth label, based UE motion sensor and/or jointly based on multiple monitoring metrics) are not precluded

In our companion paper [9], we discuss various aspects related to the overall framework of performance monitoring and testing. We find that real-time monitoring can incur unnecessary overhead and ignite too frequent signalling. A background monitoring, on the other hand, can still address many concerns for positioning use case. The label-free and label-based monitoring approaches can still be considered for background monitoring.

For label-free monitoring, measurements or model outputs (obtained as part of inference) can be collected and shared with the entity handling model monitoring for background monitoring and no strong assistance from LMF to UE side or gNB-side is expected.

For label-based monitoring, measurements and their ground truth labels can be made available at the side handling the model training and management. For example, these monitoring measurements and their labels can be available at LMF side with help of PRUs. LMF may share both measurements and labels (location or intermediate quantities) with the side responsible for model management and LCM (e.g., UE-side or gNB-side). If there are privacy concerns, LMF may at least share measurements and ask UE-side and gNB-side to report back their positioning estimates. The LMF can then compute monitoring metric and feed the monitoring outcome back to UE-side and gNB-side. Our understanding is that the UE-side or gNB-side should not necessary be the UE itself or gNB itself, but it can be the entity that handles model development and management for AI/ML positioning on these sides (e.g., UE vendor OTT server, or gNB vendor OTT server).

Observation 13: For AI/ML positioning monitoring, measurements and their labels can be available at LMF side with help of PRUs. LMF may share both measurements and labels (location or intermediate quantities) with the side handling model management (e.g., UE-side or gNB-side).

Proposal 10: For AI/ML positioning monitoring (Case1 to Case3b), study the following aspect to enable model monitoring:

- Model monitoring based on joint model input and output (ground truth-based monitoring):  Ground truth label and AI/ML model input measurements can be made available at monitoring/model management side.

We envision decision on monitoring to be tightly coupled with the side that handles model development and inference. This allows a more flexible model update and LCM decisions. It is important to study how the monitoring can map to the different AI/ML positioning deployment cases (i.e., Case1 to Case 3b). This helps understanding how monitoring data (e.g., model input and model output and other assistance information) needs to be collected and shared with the monitoring side.

For Case1/Case2a, UE side (or model management entity of UE side) needs at least access to PRU measurements while ground-truth labels can be kept at network side (if proprietary).

Proposal 11: For AI/ML positioning monitoring (Case1 and Case2a), the following are required assistance signalling for label-based monitoring:

- signalling for letting UE side (or model management entity of UE side) request monitoring data from LMF and providing monitoring data from LMF to model management entity of UE side

- signalling for letting UE side (or model management entity of UE side) and LMF communicate monitoring outcome/metric back and forth (if needed).

For Case3a, gNB/TRP (or model management entity of gNB/TRP side) can access model input monitoring measurements by receiving SRS signals from PRU and running them through the model. The gNB/TRP can report model output’s to LMF. Since the LMF can have knowledge about both PRU and gNB/TRP locations, it can calculate the ground truth label and compare it with the value reported by the gNB/TRP side to find the monitoring outcome. The LMF can then update the gNB/TRP side (or model management entity of gNB/TRP side) about the monitoring outcome. If the gNB/TRP cannot measure SRS signals that are sent directly from PRUs, the LMF can provide, for gNB/TRP side (or model management entity of gNB/TRP side), other PRU-based SRS measurements that are captured at a different gNB/TRP. The gNB/TRP side (or model management entity of gNB/TRP side) can then follow the same procedure as before to obtain monitoring outcomes from LMF.

Proposal 12: For AI/ML positioning monitoring (Case3a), the following are required assistance signalling for label-based monitoring:

- Signalling for letting gNB/TRP side (or model management entity of gNB/TRP side) and LMF request/report monitoring measurements

- Signalling for letting gNB/TRP side (or model management entity of gNB/TRP side) and LMF communicate monitoring outcome.

For Case2b and Case3b, the data needed to conduct monitoring can be readily available at LMF using existing signalling and may not require additional specification impact.

Observation 14: For AI/ML positioning monitoring (Case2b and Case3b), LMF can do label-based model monitoring with help of existing measurements and reporting, and it is not expected to incur additional specification impacts.

For label-free monitoring approaches, the UE, gNB/TRP, and LMF can conduct monitoring without strong need for signalling. The UE can rely on observed PRS measurements, gNB/TRP can rely on SRS observed measurements, and LMF can rely on PRS/SRS reported measurements to conduct measurement-based label-free monitoring.

Observation 15: For AI/ML positioning monitoring, label-free monitoring approach can be realized through implementation and may not have strong specification impact.

6 Discussion on AI/ML positioning inference

In the previous meeting, companies agreed to provide inputs regarding necessity to specify model input and outputs for model inference.

Agreement: RAN1-111-9.2.4.2

Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement

- Types of measurement as model inference input

  - new measurement

  - existing measurement

- UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b

  - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b)

- For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a)

- Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model

  - New and/or enhancement to existing assistance signaling

  - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed

Agreement: RAN1-111-9.2.4.2

Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement

- Types of measurement as model inference input

  - new measurement

  - existing measurement

- UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b

  - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b)

- For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a)

- Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model

  - New and/or enhancement to existing assistance signaling

  - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed

Agreement RAN1-112-9.2.4.2

Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement

- For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead

  - Potential new measurement: CIR/PDP

  - existing measurement: e.g., RSRP/RSRPP/RSTD

  - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied

  - Note2: study the impact of model input for other cases are not precluded

- For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF

  - new measurement report: e.g., ToA, path phase

  - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP

  - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD

- Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model

  - RS configurations

  - Other assistance information is not precluded

Agreement RAN1-112-9.2.4.2

Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement

- For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead

  - Potential new measurement: CIR/PDP

  - existing measurement: e.g., RSRP/RSRPP/RSTD

  - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied

  - Note2: study the impact of model input for other cases are not precluded

- For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF

  - new measurement report: e.g., ToA, path phase

  - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP

  - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD

- Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model

  - RS configurations

  - Other assistance information is not precluded

## 6.1 Model inference input

For Case1 and Case2a, the model runs at the UE side and model input is measured at the UE side. Therefore, from specification impact, we do not see a strong need to specify measurements for model input.

Observation 16: For AI/ML positioning inference (Case1 and Case2a), both measurements and model are obtained at UE side and no strong need to specify model input measurements.

Proposal 13: For AI/ML positioning inference (Case1 and Case2a), no need to specify type of measurements.

For Case2b, the model runs at LMF side and depends on model inputs obtained from the UE. While it is possible to consider new measurement types for LMF-sided model, the new measurements need to consider reasonable measurements (e.g., existing positioning measurements or enhancements to existing positioning measurements). The reporting of any new measurements (e.g., complex CIR or PDP) from UE to LMF incurs high OTA overhead and should be avoided. In our companion evaluation paper [2], we provide extensive evaluation to investigate the trade-off between performance and reporting overhead for Case2b with existing multipath reporting and CIR/PDP measurement reporting. Our conclusion is that Case2b with existing multipath reporting and given reporting overhead assumption can achieve better performance than an optimized CIR/PDP measurement reporting.

Observation 17: For AI/ML positioning inference (Case2b), reporting time-domain CIR/PDP measurements from UE to LMF incurs high OTA reporting overhead and need to be deprioritized.

Observation 18: For AI/ML positioning inference (Case2b), at a given reporting overhead, reporting multipath measurements achieves higher accuracy than reporting an optimized time-domain CIR/PDP measurement.

Proposal 14: For AI/ML positioning inference (Case2b), deprioritize CIR/PDP as new measurements due to their significant reporting overhead and minimal/comparable gain when compared to existing multipath reporting.

Proposal 15: For AI/ML positioning inference (Case2b), type of measurements reported from UE to network are either existing measurements or minor enhancements of existing measurements.

For Case3a, both measurements and model are available at the gNB/TRP side. Therefore, from specification impact, we do not see a strong need to specify measurements for model input.

Observation 19: For AI/ML positioning inference (Case3a), both measurements and model are obtained at gNB/TRP side and no strong need to specify model input measurements.

Proposal 16: For AI/ML positioning inference (Case3a), no need to specify type of measurements.

For Case3b, the model runs at LMF side and depends on model inputs obtained from the gNB/TRP. Model input can consider existing measurements reported from gNB/TRP. It should be noted that reporting from gNB/TRP to LMF is not expected to incur high OTA reporting overhead. Therefore, it is possible to consider new measurements (e.g., complex CIR).

Observation 20: For AI/ML positioning inference (Case3b), reporting has less dependence on OTA resources and can include both existing measurements and new measurements.

Proposal 17: For AI/ML positioning inference (Case3b), study type of measurements while including both existing measurements, enhancements of existing measurements, or new measurements.

## 6.2 Enhanced reporting

In Case2a and Case3a, the UE or gNB/TRP can use AI/ML model to compute the existing parameters to be reported. But they can leverage AI/ML to learn new positioning features that can be helpful to enhance classical positioning methods (e.g., learning a soft value of RSTD, RSRP/RSRPP, RTOA, AoA/AoD. For example, consider the multipath mitigation enhancements in Rel-17 in which the TRP can report multiple candidate UL-AoA values for each path. The TRP may derive a probability distribution of the UL-AoA (e.g., A Gaussian mixture) using ML based algorithms and enhance the report to the LMF using the Rel-17 framework. As discussed in Appendix 2 (ML-based soft information reporting method), the UE may need to report new measurements to realize the gains of ML based positioning especially for the UE-assisted methods. RAN1 may additionally identify some parameters to augment the measurement reporting for existing methods.

Observation 21: For AI/ML positioning inference (Case2a and Case3a), UE/TRP may report new measurement reports such as soft info to timing and angle to help LMF improve positioning accuracy.

Proposal 18: For AI/ML positioning inference (Case2a and Case3a), consider existing/enhanced measurements and new measurements (e.g., soft-info of time/angle)

7 List of observations and proposals

Observation 1: For AI/ML positioning, a combination of the defined cases (i.e., Case1 to Case3b) may be used to achieve improved positioning in a defined setting.

Observation 2: For AI/ML positioning model development, the development of the model needs to be done by the vendor who implements the device where the AI/ML model inference runs.

Observation 3:  For legacy positioning, UE capabilities for legacy positioning methods are conveyed as part of the UE feature and feature group (FG) listing, and LMF uses these FGs/components to configure legacy positioning.

Observation 4:  For AI/ML positioning functionality and model identification, the additional conditions can be bidirectional and sent back and forth between the UE and LMF.

Observation 5:  For AI/ML positioning functionality and model identification, the additional conditions can relate to a scenario and/or model ID.

Observation 6:  For AI/ML positioning functionality-based LCM (Case1 and Case2a), it is beneficial for letting both LMF and UE initiate the process for activation, deactivation, switching, and falling back of a functionality.

Observation 7:  For AI/ML positioning model-based LCM (Case1 and Case2a), it is beneficial for letting both LMF and UE initiate the process for activation, deactivation, selection, switching of a model.

Observation 8: For AI/ML positioning data collection, labels generated using NR RAT-dependent positioning methods and captured in LOS or mild NLOS conditions can help enhance training performance and reduce the burden of expensive labelling.

Observation 9: For AI/ML positioning data collection, transfer of training data (i.e., measurements, ground truth labels, assistance information) from source entities to training entities is out the scope of RAN1.

Observation 10: For AI/ML positioning data collection (Case2b and Case3b), LMF can leverage information obtained using existing LPP and NRPPa procedures to compute ground truth labels.

Observation 11: For AI/ML positioning data collection (Case1, Case2a and Case3a), UE and gNB/TRP require assistance from LMF to compute ground truth labels.

Observation 12: In AI/ML positioning data collection, the following information can be associated with data collected at UE/PRU side:

- Indication of how PRS resources map to physical anchor location/angles: UE side need to know how different PRS resource sets/resources are mapped to physical anchor location and beam angles.

- Indication of timing errors at network side: UE side can benefit from knowing expected ranges/distributions/indexing of timing errors at network side (e.g., TRP TX timing errors, inter-TRP synchronization errors) for better training and model development.

- Indication of LOS/NLOS maps for each TRP: UE side can benefit from knowing the map of potential LOS/NLOS states for each combination of TRP and PRS resource (if available from LMF side).

Observation 13: For AI/ML positioning monitoring, measurements and their labels can be available at LMF side with help of PRUs. LMF may share both measurements and labels (location or intermediate quantities) with the side handling model management (e.g., UE-side or gNB-side).

Observation 14: For AI/ML positioning monitoring (Case2b and Case3b), LMF can do label-based model monitoring with help of existing measurements and reporting, and it is not expected to incur additional specification impacts.

Observation 15: For AI/ML positioning monitoring, label-free monitoring approach can be realized through implementation and may not have strong specification impact.

Observation 16: For AI/ML positioning inference (Case1 and Case2a), both measurements and model are obtained at UE side and no strong need to specify model input measurements.

Observation 17: For AI/ML positioning inference (Case2b), reporting time-domain CIR/PDP measurements from UE to LMF incurs high OTA reporting overhead and need to be deprioritized.

Observation 18: For AI/ML positioning inference (Case2b), at a given reporting overhead, reporting multipath measurements achieves higher accuracy than reporting an optimized time-domain CIR/PDP measurement.

Observation 19: For AI/ML positioning inference (Case3a), both measurements and model are obtained at gNB/TRP side and no strong need to specify model input measurements.

Observation 20: For AI/ML positioning inference (Case3b), reporting has less dependence on OTA resources and can include both existing measurements and new measurements.

Observation 21: For AI/ML positioning inference (Case2a and Case3a), UE/TRP may report new measurement reports such as soft info to timing and angle to help LMF improve positioning accuracy.

Proposal 1: For AI/ML positioning model training, consider model training and development at the side for which the inference of model is expected to be performed:

- Case1: Model training and inference at UE side

- Case2a: Model training and inference at UE side

- Case2b: Model training and inference at network side

- Case3a: Model training and inference at network side

- Case3b: Model training and inference at network side

Proposal 2: For AI/ML positioning model training, modes of operation in which model training and inference happen at different sides are deprioritized for the current AI/ML positioning study.

Proposal 3: For AI/ML positioning functionality and model identification, as a starting point, consider the content of existing legacy positioning FGs to define conditions for functionality and model operation.

Proposal 4: For AI/ML positioning functionality and model identification, consider at least the following aspects for defining conditions:

- DL PRS resource conditions

- DL PRS measurement report conditions

- Generalization conditions

Proposal 5: For collaboration in AI/ML positioning (Case1, Case2a, and Case2b), consider Level-x and Level-y collaboration between UE and LMF.

Proposal 6: For collaboration in AI/ML positioning (Case3a and Case3b), consider Level-x and Level-y collaboration between gNB/TRP and LMF.

Proposal 7: For AI/ML positioning data collection, consider the following additional entities and mechanisms to generate ground truth label:

- For direct AI/ML positioning, ground truth label is UE location

  - Case1

    - UE generates location based on non-NR and/or NR RAT-dependent positioning methods with network assistance (as applicable).

- For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameters(s) corresponding to AI/ML model output

  - Case1/Case2a

    - UE generates label directly or calculates based on measurement/location with network assistance.

Proposal 8: For AI/ML positioning data collection, consider signalling for labelling assistance from LMF in Case1, Case2a, and Case3a.

Proposal 9: In AI/ML positioning data collection, consider the following associated information for data collection at UE/PRU (Case1 and Case2a):

- Indication of how PRS resources map to physical anchor location/angles

- Indication of timing errors at network side: E.g., TRP TX timing errors, inter-TRP synchronization errors.

- Indication of LOS/NLOS maps for each TRP: E.g., map of potential LOS/NLOS states for each combination PRS/TRP (if available from LMF side).

Proposal 10: For AI/ML positioning monitoring (Case1 to Case3b), study the following aspect to enable model monitoring:

- Model monitoring based on joint model input and output (ground truth-based monitoring):  Ground truth label and AI/ML model input measurements can be made available at monitoring/model management side.

Proposal 11: For AI/ML positioning monitoring (Case1 and Case2a), the following are required assistance signalling for label-based monitoring:

- signalling for letting UE side (or model management entity of UE side) request monitoring data from LMF and providing monitoring data from LMF to model management entity of UE side

- signalling for letting UE side (or model management entity of UE side) and LMF communicate monitoring outcome/metric back and forth (if needed).

Proposal 12: For AI/ML positioning monitoring (Case3a), the following are required assistance signalling for label-based monitoring:

- Signalling for letting gNB/TRP side (or model management entity of gNB/TRP side) and LMF request/report monitoring measurements

- Signalling for letting gNB/TRP side (or model management entity of gNB/TRP side) and LMF communicate monitoring outcome.

Proposal 13: For AI/ML positioning inference (Case1 and Case2a), no need to specify type of measurements.

Proposal 14: For AI/ML positioning inference (Case2b), deprioritize CIR/PDP as new measurements due to their significant reporting overhead and minimal/comparable gain when compared to existing multipath reporting.

Proposal 15: For AI/ML positioning inference (Case2b), type of measurements reported from UE to network are either existing measurements or minor enhancements of existing measurements.

Proposal 16: For AI/ML positioning inference (Case3a), no need to specify type of measurements.

Proposal 17: For AI/ML positioning inference (Case3b), study type of measurements while including both existing measurements, enhancements of existing measurements, or new measurements.

Proposal 18: For AI/ML positioning inference (Case2a and Case3a), consider existing/enhanced measurements and new measurements (e.g., soft-info of time/angle)

Proposal 19: Study the specification impact for the reporting of soft information associated with positioning measurements, derived using machine learning.

Appendix 1: RF Fingerprinting – Direct AI/ML based positioning method

## A 1.1 High level overview of RFFP

RF Fingerprinting for positioning (RFFP) is a well-studied ML based positioning method in literature. The key idea in RFFP is to learn a parameterized function between the channel observations made by the UE and TRPs and the current position of the UE. The parameterization is through a NN and the parameters of the NN are trained using a data collection campaign and thus RFFP is an ML based positioning method. Written as an equation, the basic input to output mapping for RFFP methods can be expressed as

fθg(h)=p,

where f is the function which represents the architecture of the NN and is parameterized by the NN parameters (weights) θ. The input to the RFFP network is based on a deterministic function g(h) of the channel observation h. This function captures the common input pre-processing to be done for each channel observation input to the NN and is not specifically dependent on the data realizations used for training the NN model. The output of the RFFP NN, represented by p, is the estimated/predicted position of the UE. In addition, RFFP NN can also be designed to additional quantities such as positioning uncertainty/covariance etc of the predicted positioning, one or more additional hypothesis of the position and their uncertainty etc.

Figure 2:  Illustration of RF Fingerprinting method

The key technical idea behind RFFP is that the entire channel observation is utilized for positioning. All the multipath information from all the channel observations is utilized implicitly in learning the mapping from the input observations to the estimated UE location. One implication of such a method is that there is no assumption that the channel from the TRP to the UE has a LOS component. Thus, this method works in NLOS heavy channels as well. Such a method allows the UE position estimation to be estimated even from a single cell assuming a multipath rich environment. In practice, we would observe channel information from multiple cells and all this information could be used to learn the position.

From literature, we note that RFFP using RSRP/RSSI of the observed PRS/SRS signals at the UE/TRP has been investigated for predicting position of the UE. In the recent years, channel observations (or channel state information (CSI)) have also been used extensively to learn the RFFP NN. We note that CSI based RFFP can achieve cm-level accuracy even in challenging conditions at the cost of more data input while RSRP based methods can achieve accuracy in the order of meters.

## A 1.2 Potential ML architectures to realize RFFP

We discuss various RFFP architectures from the perspective of where positioning inference is performed and with respect to the signals/inputs used for performing the inference.

## A 1.2.1 UE-based RFFP

The RFFP NN is trained and available at the UE for inference. The NN takes as input the channel observations derived from the DL-PRS to determine the UE position. The channel observations from one TRP (single cell RFFP) or from multiple cells/TRP (multi-cell RFFP). Compactly, the UE inference can be represented as fθgHDL, 1HDL2,…,HDLNTRP  = p, where HDL is the channel observed from the TRP.

## A 1.2.2 NG-RAN node assisted RFFP

This method is the counterpart of the UE based RFFP where the network utilizes the UL channel observations from the TRPs to derive the UE position. This procedure can be represented as:

fθgHUL, 1HUL2,…,HULNTRP  = p, or

fθzUL1,zUL2,…,zULN = p, where zi=g(HULi).

In this architecture, the network may directly use the channel observations, or a compressed set of features reported by each TRP to perform inference.

We note that both UE-based and NG-RAN node assisted RFFP (direct AI/ML methods) are already being used for evaluations and further study.

Appendix 2: ML-based soft information reporting – ML enhanced positioning method

Section 3.2.1 outlined scenarios where the performance of classical positioning methods is expected to be degraded. Multipath and NLOS conditions can create ambiguity in determining the true LOS delay from the observed channel profile.

Consider the UE-assisted DL-TDoA positioning method as an example. Current positioning methods require the UE to report an RSTD value for each TRP relative to the reference TRP. Enhancements have been introduced in Rel-17 to improve robustness of the positioning method to multipath and NLOS scenarios by allowing the UE to report RSTD for multiple paths and to send an LOS/NLOS indication.

While such signalling has been defined, it is unclear how to systematically interpret the report and/or the LOS/NLOS indication. What is needed is a way to convey something meaningful that can be directly used to improve positioning accuracy. For example, in addition to a hard estimate of the RSTD, it could be useful to derive soft information such as the uncertainty level associated with it, or a probability distribution associated with it, which the positioning algorithm could make use of. In our proposal, we use a machine learning-based data-driven approach to derive such soft information.

## A 2.1 Combining soft information to improve accuracy in multipath scenarios

In many situations, combining soft information outperforms a hard-decision based approach. If a UE experiences a multipath channel, instead of deciding which of the paths is truly LOS, it would be beneficial if there was a way for the UE to report more than one hypothesis for the RSTD, together with a confidence metric indicating the level of certainty of each hypothesis. More generally, the UE could report a distribution of the measured quantity, e.g., RSTD. Then, the network can combine this soft information with similar information from other measurements made by the UE to derive a more accurate position estimate for the UE.

The figures below illustrate this idea using an example in the context of multi-cell RTT positioning method.

Figure 3: Impact of multipath on positioning accuracy

In Figure 1, there is an over-estimation of RTT with respect to TRP1 due to multipath. As a result, the reports received by the network are not consistent with each other and could result in an error in the position estimate.

Figure 4: ML-based soft information reporting to address multipath scenarios

In Figure 2, the same situation is considered, with the difference that it is possible for the network to determine a distribution of the measured quantity – RTT in this case. For instance, for TRP1, the report could inform the network that the RTT could have two values, together with the relative confidence associated with the values. The network could then perform soft combining of information across reports from all the measurement sources through ML-based soft information reporting. Combining with the reports based on the other two TRPs allows the network to improve the positioning accuracy in this example.

## A 2.2 Machine learning framework for a soft information reporting approach

Machine learning techniques could play a role in the following aspects:

- Derive the distribution of the measured quantity based on the observed channel estimates

- Efficiently combine the likelihood information from multiple sources

The approach of reporting soft information in the form of a distribution of the measured quantity could be applied to any of the existing positioning measurement quantities. For example, the TRP may also apply ML techniques to derive and report a distribution of uplink measured quantities such as UL-TDoA or UL-AoA based on the observed uplink channel features.

In each case, it would be useful to study the framework required to enable this approach. Specifically, we would need to consider aspects such as assistance data to train the ML models, enhancements to the reporting framework, and performance monitoring aspects.

Proposal 19: Study the specification impact for the reporting of soft information associated with positioning measurements, derived using machine learning.

Appendix 3: Overview of previous RAN1 agreements

## A 3.1 Overview of RAN1 109-e agreements

In RAN1 109-e, initial aspects of the framework for positioning were agreed as noted below:

Agreement

Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels.

- Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2.

- Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement.

- Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case

Agreement

For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered.

- Direct AI/ML positioning: the output of AI/ML model inference is UE location

  - E.g., fingerprinting based on channel observation as the input of AI/ML model

  - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation

  - FFS: applicable scenario(s) and AI/ML model generalization aspect(s)

- AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement

  - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement

  - FFS the details of input and output for corresponding AI/ML model(s)

  - FFS: applicable scenario(s) and AI/ML model generalization aspect(s)

- Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement

Agreement

Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement.

- AI/ML model training

  - training data type/size

  - training data source determination (e.g., UE/PRU/TRP)

  - assistance signalling and procedure for training data collection

- AI/ML model indication/configuration

  - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection)

- AI/ML model monitoring and update

  - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning)

- AI/ML model inference input

  - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference)

  - model input acquisition and pre-processing

  - type/definition of model input

- AI/ML model inference output

  - report/feedback of model inference output

  - post-processing of model inference output

- UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring)

- Other aspects are not precluded

- Note: not all aspects may apply to an AI/ML approach in a sub use case

- Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

## A 3.2 Overview of RAN1 110 agreements

In RAN1 110, some aspects of the framework for positioning were agreed as noted below:

Agreement

For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected.

- Direct AI/ML positioning

- AI/ML assisted positioning

- Note 1: the selection does not intend to provide any indication of the prospects of any future normative project.

- Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results

Conclusion

Defer the discussion of prioritization of AI/ML positioning based on collaboration level until more progress on collaboration level discussion in agenda 9.2.1.

Agreement

Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement

- Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report)

  - Partial and/or noisy ground truth label

- Signaling for data collection

- Other aspects are not precluded

Agreement

Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement

- AI/ML model monitoring performance metrics

- Condition of AI/ML model update

- Reference signals and measurement feedback/report

- Other aspects are not precluded

Agreement

Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least

- UE-side or Network-side training

- UE-side or Network-side inference

  - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches

Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity

Conclusion

To use the following terminology defined in TS 38.305 when describe their proposed positioning methods

- UE-based

- UE-assisted/LMF-based

- NG-RAN node assisted

Note: companies are required to clarify their positioning method(s) when their approaches do not fall in one of the above.

## A 3.3 Overview of RAN1 110be agreements

Conclusion

- Defer the discussion of prioritization of online/offline training for AI/ML based positioning until more progress on online vs. offline training discussion in agenda 9.2.1.

Agreement

- Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Agreement

Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement

- Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc.

- Model capability, e.g., positioning accuracy quality and model inference latency

- Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information

- Note: other aspects are not precluded

Agreement

Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects

- Assistance signaling and procedure at least for UE-side model

- Report/feedback and procedure at least for Network-side model

- Note1: study is applicable to both of the following cases

  - Model inference and model monitoring at the same entity

  - Entity to perform the model monitoring is not the same entity for model inference

- Note2: other aspects are not precluded

Agreement

Regarding data collection for AI/ML model training for AI/ML based positioning, at least for each of the agreed cases (Case 1 to Case 3b)

- Study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data

  - Companies are requested to report their assumption of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b)

  - Companies are requested to report their assumption of applicable ground truth label (e.g., location or other information) and/or other training data (e.g., measurement) for each case (Case 1 to Case 3b)

  - Feasibility study on the entity to obtain ground truth label and/or other training data takes into account at least

    - availability of the entity to obtain label and/or other training data

  - Note: further discussion and decision of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) is not precluded based on companies’ input

- Study potential signalling and procedure to enable data collection

  - Potential specification impact on the details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data

  - Potential specification impact on assistance signaling indicating reference signal configuration(s) to derive label and/or other training data

## A 3.4 Overview of RAN1 111 agreements

Agreement: RAN1-111-9.2.4.2

For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

Agreement: RAN1-111-9.2.4.2

Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement

- Types of measurement as model inference input

  - new measurement

  - existing measurement

- UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b

  - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b)

- For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a)

- Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model

  - New and/or enhancement to existing assistance signaling

  - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed

Agreement: RAN1-111-9.2.4.2

Regarding data collection for AI/ML model training for AI/ML based positioning,

- The following options of entity and mechanisms to generate ground truth label are identified for further study

  - For direct AI/ML positioning, ground truth label is UE location

    - PRU with known location

    - UE generates location based on non-NR and/or NR RAT-dependent positioning methods

    - LMF generates UE location based on positioning methods

    - LMF with known PRU location

    - Note: user data privacy needs to be preserved

  - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output

    - PRU generates label directly or calculates based on measurement/location

    - UE generates label directly or calculates based on measurement/location

    - Network entity generates label directly or calculates based on measurement/location

- The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study

  - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b)

    - PRU

    - UE

  - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b)

    - TRP

  - Note: other options of entity to generate other training data are not precluded

- Note: Existing PRU definition is in 38.305

Agreement: RAN1-111-9.2.4.2

Regarding data collection for AI/ML model training for AI/ML based positioning, study benefits, feasibility and potential specification impact (including necessity) for the following aspects

- Request/report of training data

  - Ground truth label

  - Measurement corresponding to model input

  - Associated information of ground truth label and/or measurement corresponding to model input

- Assistance signaling and procedure to facilitate generating training data

  - Reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier

  - Assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc.

  - Note1: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed

- Note2: Study may consider different entity to generate training data as well as different types of training data when applicable

- Note3: study considers both of the following cases when applicable

  - when the training entity is the same entity to generate training data

  - when the training entity is not the same entity to generate training data

Agreement: RAN1-111-9.2.4.2

- Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects

- At least the following are identified for further study as potential data for calculating monitoring metric

  - If monitoring based on model output

    - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning

  - If monitoring based on model input

    - E.g., measurement corresponding to model inference input

  - Note1: other type of potential data for model monitoring is not precluded

  - Note2: combination of one or more type of potential data for monitoring is not precluded

- If a given type of data is necessary for calculating monitoring metric, study whether and if so

  - How an entity can be used to provide the given type of data for calculating monitoring metric

    - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case

  - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric

  - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric

  - Potential UE-network interaction

    - E.g., model monitoring decision indication between UE and network

Agreement: RAN1-111-9.2.4.2

For AI/ML based positioning accuracy enhancement, direct AI/ML positioning and AI/ML assisted positioning are selected as representative sub-use cases.

## A 3.5 Overview of RAN1 112 agreements

Agreement RAN1-112-9.2.4.2

Regarding training data generation for AI/ML based positioning,

- The following options of entity and mechanisms to generate ground truth label are identified

  - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a)

  - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label

    - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods

    - Network entity generates ground truth label based on positioning methods

- The following options of entity to generate other training data (at least measurement corresponding to model input) are identified

  - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b)

    - PRU

    - UE

  - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b)

    - TRP

- Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study

Agreement RAN1-112-9.2.4.2

Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects

- Associated information of training data

  - Quality indicator at least for ground truth label (if needed)

  - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc.

- Assistance signaling and procedure to facilitate generating/collecting training data

  - Potential determination of the UE/PRU/TRP which can provide the training data

  - Configuration of reference signal (for measurement and/or label)

  - Signaling other than above 2 for data collection

    - E.g., requested quality of training data

Agreement RAN1-112-9.2.4.2

Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects

- Entity to derive monitoring metric

  - UE at least for Case 1 and 2a (with UE-side model)

    - FFS PRU for Case 1 and 2a

  - gNB at least for Case 3a (with gNB-side model)

    - FFS gNB for Case 3b (with LMF-side model)

  - LMF at least for Case 2b and 3b (with LMF-side model)

  - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b)

- If model monitoring does not require ground truth label (or its approximation).

  - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc.

  - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc.

  - report of the calculated metric and/or model monitoring decision

- If model monitoring requires and is provided ground truth label (or its approximation)

  - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc.

  - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc.

  - report of the calculated metric and/or model monitoring decision

- Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded

Agreement RAN1-112-9.2.4.2

Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement

- For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead

  - Potential new measurement: CIR/PDP

  - existing measurement: e.g., RSRP/RSRPP/RSTD

  - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied

  - Note2: study the impact of model input for other cases are not precluded

- For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF

  - new measurement report: e.g., ToA, path phase

  - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP

  - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD

- Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model

  - RS configurations

  - Other assistance information is not precluded

Note RAN1-112-9.2.4.2: Companies are encouraged to report their assumption of functionality and their assumption of information element(s) of AI/ML functionality identification for AI/ML based positioning with UE-side model (Case 1 and 2a).

## A 3.6 Overview of RAN1 112bis-e agreements

Agreement RAN1-112be-9.2.4.2

Regarding monitoring for AI/ML based positioning, at least the following entities are identified to derive monitoring metric

- UE at least for Case 1 and 2a (with UE-side model)

- gNB at least for Case 3a (with gNB-side model)

- LMF at least for Case 2b and 3b (with LMF-side model)

Working Assumption RAN1-112be-9.2.4.2

Regarding data collection at least for model training for AI/ML based positioning, at least the following information of data with potential specification impact are identified.

- Ground truth label

  - At least for model training

  - Report from the label data generation entity

- Measurement (corresponding to model input)

  - At least for model training

  - Report from the measurement data generation entity

- Quality indicator

  - For and/or associated with ground truth label and/or measurement at least for model training

  - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity

- RS configuration(s)

  - At least for deriving measurement

  - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP

  - Note1: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement

- Time stamp

  - At least for and/or associated with training data for model training

    - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities

  - Report from data generation entity together with training data and/or as LMF assistance signaling

  - Note2: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement

- FFS other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) for data collection

- Note3: whether the above information can be applied to other aspects of AI/ML LCM (e.g., updating, monitoring, etc.) can also be discussed

- Note4: transfer of data from the entity generating data to a different entity is not precluded from RAN1 perspective

Agreement RAN1-112be-9.2.4.2

Regarding monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s), feasibility, necessity and potential specification impact for each case (Case 1 to 3b)

- Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring

- Assistance signaling from UE/PRU for network-side model monitoring

- Model monitoring based on provided ground truth label (or its approximation)

  - Monitoring metric: statistics of the difference between model output and provided ground truth label

  - Provisioning of ground truth label and associated label quality

- Model monitoring using at least statistics of measurement(s) without ground truth label

  - Monitoring metric: e.g., statistics of measurement(s) compared to the statistics associated with the training data

  - Note1: the measurement(s) may or may not be the same as model input

- Note2: other monitoring methods (e.g., based on statistics of model output without ground truth label, based UE motion sensor and/or jointly based on multiple monitoring metrics) are not precluded

Agreement RAN1-112be-9.2.4.2

Regarding LCM of AI/ML based positioning accuracy enhancement, at least for Case 1 and Case 2a (model is at UE-side), further study the following aspects on information related to the conditions

- What are the conditions for functionality-based LCM

  - which aspects should be specified as conditions of a Feature/FG available for functionality

- What are the conditions for model-ID-based LCM

  - Which aspects should be considered as additional conditions, and how to include them into model description information during model identification

# References

| [1] | RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN Plenary. |
| --- | --- |
| [2] | R1-2305332, “Evaluation on AI-ML for positioning accuracy enhancements,” Qualcomm Incorporated, 2023. |
| [3] | R1-2205029, “Other aspects on AI-ML for positioning accuracy enhancement,” Qualcomm Incorporated, 2022. |
| [4] | R1-2207229, “Other aspects on AI-ML for positioning accuracy enhancement,” Qualcomm Incorporated, 2022. |
| [5] | R1-2209981, “Other aspects on AI-ML for positioning accuracy enhancement,” Qualcomm Incorporated, 2022. |
| [6] | R1-2212113, “Other aspects on AI-ML for positioning accuracy enhancement,” Qualcomm Incorporated, 2022. |
| [7] | R1-2301409, “Other aspects on AI-ML for positioning accuracy enhancement,” Qualcomm Incorporated, 2023. |
| [8] | R1-2303587, “Other aspects on AI-ML for positioning accuracy enhancement,” Qualcomm Incorporated, 2023. |
| [9] | R1-2305327, “General Aspects of AI/ML Framework,” Qualcomm Incorporated, 2023. |