3GPP TSG RAN WG1 #113   	           R1-2305620

Incheon, Korea, May 22nd – May 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for NR NCR

Agenda Item:	9.16.4

Document for: 	Discussion and Decision

Introduction

Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e is captured in R1-2304282 [1]. In this contribution, we provide our views on UE features for NR NCR.

Discussions

FG 43-1

| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 43-1, we have the following suggestion:

- For component 5, we think “support of” should be kept, and candidate value is not needed. We don’t see the motivation to introduce timing offset, so that candidate values for this component is not needed.

- For pre-requisite FG, we don’t see any FG should be pre-requisite of 43-1.

- We think FG 43-1 can be optional without capability signaling. According to RAN2/3 agreement, network is aware that the device is NCR via NCR indication from NCR in Msg5. Since the FG is basic features of NCR, we think capability signaling may not be needed.

FG 43-2, 43-3, 43-4

| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |

For FG 43-2, 43-3, 43-4, we have the following suggestion:

- Among FG 43-2, 43-3, 43-4, if one of them is selected, we think periodic beam indication can be the basic feature of beam indication.

43-4a

| 43. NR_netcon_repeater | 43-4a | Beam index updates for semi-persistent beam indication | 1.Support of [temporary] override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1, 43-4 | N/A | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Yes | FFS: whether to merge this FG with 43-4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 43-4a, we have the following suggestion:

- We think “temporary” is not needed and can be removed.

- We think this feature can be merged to 43-4. It is basic function of semi-persistent beam indication.

FG 43-6, 43-8

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} |

For FG 43-6, 43-8, we have the following suggestion:

- We think candidate value is not needed. In our views, the support of this feature is common for Rel-15/16 and Rel-17 beam indication framework. And whether Rel-15/16 or Rel-17 framework is supported can be known by network based on the report of NCR-MT UE feature which is similar as normal UE.

Summary

Based on the discussions in Session 2, we propose to adopt the following update for NR NCR UE feature.

| Features |  | Index |  | Feature group |  | Components |  | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. |  | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) |  | Need of FDD/TDD differentiation |  | Need of FR1/FR2 differentiation |  | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 |  | Note |  | Mandatory/Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater |  | 43-1 |  | Basic NCR support |  | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. Support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd 7. Support of periodic beam indication for access link 8. Support of Priority flag for periodic indication |  |  |  | N/A |  | Yes |  | NCR is not supported | Per NCR-MT |  | No |  | No |  | Yes |  | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 |  | Optional without capability signaling |  |
| 43. NR_netcon_repeater |  | 43-3 |  | Aperiodic beam indication for access link |  | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] |  | 43-1 |  | N/A |  | Yes |  | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT |  | No |  | No |  | Yes |  | FFS: component 2 candidate values |  | Optional with capability signaling |  |
| 43. NR_netcon_repeater |  | 43-4 |  | Semi-persistent beam indication for access link |  | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3.Support of override of the RRC configuration of the beam index(es) in semi-persistent beam indication |  | 43-1 |  | N/A |  | Yes |  | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT |  | No |  | No |  | Yes |  |  |  | Optional with capability signaling |  |
| 43. NR_netcon_repeater |  | 43-5 |  | Simultaneous UL transmission of backhaul link and C-link |  | 1. Simultaneous UL transmission of backhaul link and C-link |  | 43-1 |  | N/A |  | Yes |  | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT |  | No |  | No |  | Yes |  |  |  | Optional with capability signaling |  |
| 43. NR_netcon_repeater |  | 43-6 |  | Dedicated signalling for backhaul link beam indication |  | 1. Support dedicated signalling for backhaul link beam indication |  | 43-1, 43-8 |  | N/A |  | Yes |  | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT |  | No |  | No |  | Yes |  |  |  | Optional with capability signaling |  |
| 43. NR_netcon_repeater |  | 43-8 |  | Adaptive beam for NCR backhaul link/C-link |  | Support of backhaul link beam determination based on predefined rule |  | 43-1, 2-2,2-4, 2-4a |  | N/A |  | Yes |  | The beam for backhaul link and C-link is fixed. | Per NCR-MT |  | No |  | No |  | Yes |  |  |  | Optional with capability signaling |  |

Reference

[1]	R1-2304282, Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e, Moderators (AT&T, NTT DOCOMO, INC.), Apr. 2023