3GPP TSG RAN WG1 #113	R1-2306228

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary EOM on 9.9.1 Coverage enhancement for NR NTN

Agenda Item:	9.9.1

Document for: 	Discussion and Decision

- Introduction

This document is made for discussion on coverage enhancement for NR NTN. Schedule for discussion is below in UTC time. FL requests companies to consider the schedule.

- Meeting start: Monday 9:00

- 1st offline: Monday 12:00 – 13:00

- 1st online: Monday 17:30 – 19:30

- 2nd offline: Wednesday 8:00 – 9:30

- 2nd online: Thursday 11:00 – 13:00

This topic is mentioned in Rel-18 NR NTN WID as captured in Appendix-1. As discussed/concluded at the previous RAN plenary meeting, we focus on coverage enhancement of PUCCH for Msg4 HARQ-ACK and discussion of DMRS-bundling for PUSCH. Although FL found that several companies propose other mechanisms in their contributions, they will not be summarized/handled since not aligned with the WID description.

In this meeting, FL’s plan is to agree at least the following aspects.

- For PUCCH of Msg4 HARQ-ACK

  - Details of “repetition request or capability report” and LS to RAN2 based on in RAN1#112b-e

  - Design target

  - Dynamic indication details

  - [Down-selection of RSRP threshold]

- For PUSCH DMRS-bundling

  - Information report from UE, e.g., capability report, assistance information, etc. – option list

  - Determination of nominal TDW – decision or option list

  - Determination of actual TDW – decision or option list

FL assumes that the following points are the next meeting plan to be discussed/decided. Note that the next meeting is the last meeting of this work item.

- For PUCCH of Msg4 HARQ-ACK

  - Update/confirmation of working assumption including RSRP threshold

- For PUSCH DMRS bundling

  - Determination of UE information report – down-selection

  - Determination of nominal TDW – corresponding to agreed UE information report

  - Determination of actual TDW – down-selection

In addition, ‘contact information’ in the last section is copied from the summary at the last meeting. Anyone can use/add/update/remove some of the list if necessary.

- Collections of agreements/conclusions in RAN1#113

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- Two-state information is transmitted as ‘repetition request or capability report’ in the existing agreements/working assumptions.

  - The two-state information represents state 1: ‘repetition request or capability report’ or state 2: no indication.

  - How to transmit the two-state information is up to RAN2 when higher layer signaling is used for the transmission.

  - In state 1, only either repetition request or capability report is transmitted from each UE when transmitted, and they are not differentiated in the signaling.

  - Note: repetition request and capability report are defined as in the working assumption reached at RAN1#112.

Agreement

Draft LS to RAN2 in R1-2306085 is endorsed with the following change:

It is noted that an additional working assumption was reached for repetition request or capability report.

Final LS in R1-2306105

Agreement

If PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for PUCCH transmission when dedicated PUCCH resource configuration is not provided:

  - The agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to any PUCCH transmission by using common PUCCH resource

  - The same repetition factor is applied for PUCCH for Msg4 HARQ-ACK and subsequent PUCCH transmissions by using common PUCCH resource

  - Note: It is not precluded for gNB to provide dedicated PUCCH config via Msg4 PDSCH.

Working assumption

For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities and assistance information (if needed).

- i.e., if PUSCH-TimeDomainWindowLength is configured, nominal TDW is determined by PUSCH-TimeDomainWindowLength; otherwise, nominal TDW is determined based on UE capability(ies) signaling.

- FFS: which UE capability(ies) signaling is(are) used

- FFS: whether/how to use UE assistance information, if supported

Agreement

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for actual TDW determination.

- Actual TDW is determined by the existing events and,

  - Alt A: No additional event

    - i.e., no spec impact is assumed for actual TDW determination.

  - Alt B: New event of TA pre-compensation timing dynamically indicated by gNB

    - i.e., TA pre-compensation timing can be dynamically indicated by gNB

    - Note: UE can perform TA pre-compensation update at the indicated timing

    - FFS: detailed indication

  - Alt C: as dynamically indicated by gNB

    - i.e., actual TDW can be dynamically indicated by gNB

    - FFS: detailed indication

  - Alt D: New event based on epoch time

    - FFS details

  - Alt E: New event based on antenna switching

Agreement

For NTN-specific PUSCH DMRS bundling,

- As UE capability report (in addition to FG 44-2), one or more of the following is down-selected.

  - Option 1a: No new capability except for FG44-2

    - Note: FG 30-4 is reported [in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit and without taking TA pre-compensation update into account]

  - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account

    - Note: FG 30-4 is not reported for NTN band

  - Option 1c: Support of antenna switching with DMRS bundling in NTN

  - Option 1d: Max TDW size per NTN platform (e.g., LEO, MEO, GEO) with taking TA pre-compensation update into account

    - FFS details

  - Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account

  - Option 1f: Whether to support actual TDW across pre-compensation segments

    - Segments defined in R17 IoT-NTN is baseline, FFS details

  - Option 1g: Whether to support TA pre-compensation update within an actual TDW that does not violate the phase difference limit

- As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)), one or more of the following is down-selected.

  - Option 2a: No assistance information

  - Option 2b: Max TDW size based on reporting timing

    - FFS which timing is referred

  - Option 2c: TA adjustment timing

  - Option 2d: Antenna switching interval

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Support Alt 1-1d for dynamic indication of repetition factor:

  - Alt 1-1d: DAI field

    - DAI field in DCI format 1_0 with CRC scrambled by TC-RNTI is used for indication.

- Proposals for agreements/conclusions

- Discussion

As in the previous meeting, FL recommends companies to use the following values for discussion in this meeting if any and for future evaluations. Still FL found that some companies are using different values to make observations/proposals. Observations/proposals may be changed dependent on the applied CNR value; thus different values should not be chosen among companies.

| Case | Satellite orbit | Satellite parameter set | Elevation angle [degree] | Frequency [GHz] | UE antenna gain [dBi] | TX: EIRP [dBm] | RX: G/T [dB/T] | No. of PRBs | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | LEO-1200 | 1 | 30 | 2.0 | -5.5 | 18.0 | 1.1 | 1 | 0.18 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -8.1 |
|  |  |  |  |  |  |  |  | 2 | 0.36 |  |  |  |  |  |  | -11.1 |
|  |  |  |  |  |  |  |  | 3 | 0.54 |  |  |  |  |  |  | -12.9 |
|  |  |  |  |  |  |  |  | 4 | 0.72 |  |  |  |  |  |  | -14.1 |
|  |  |  |  |  |  |  |  | 6 | 1.08 |  |  |  |  |  |  | -15.9 |

  - PUCCH enhancements for Msg4 HARQ-ACK

    - [Closed/Highest] Information contents of ‘repetition request or capability report’ + LS to RAN2

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE. - Option B: Higher layer signaling in Msg3 PUSCH Send an LS to RAN2 at RAN1#113 to provide details of “repetition request or capability report”, to ask the feasibility of Option B, and if feasible, to specify the details of Option B.

At the last meeting, the above working assumption was reached. An LS to RAN2 to ask the feasibility of Option B is necessary, but what is the exact contents of ‘repetition request or capability report’ was not decided yet and thus sending the LS was postponed. For sufficient time in RAN2 and correspondingly in RAN1, the LS should be sent as early as possible.

From companies’ contributions, FL observed that most companies (10 or more) believe that only one bit information is assumed, and that repetition factor indication is unnecessary, e.g., according to the following reasons. It is noted that they assume that the only difference between ‘repetition request’ or ‘capability report’ is whether to use RSRP threshold for transmission trigger.

- [20/OPPO]: In Option B, such an indication is already precluded in the previous working assumption.

- [21/Samsung], etc.: no/little gain with large overhead. gNB can decide repetition factor appropriately without such an indication.

- [24/MTK], etc.: gNB RX may depend on DL/UL pathloss difference, interference from other UEs, etc.

Meanwhile, small number of companies (2) believe that repetition factor indication is beneficial.

FL suggests agreeing the majority view quickly and sending an LS to RAN2 with the new agreement.

      - 1st round

Information contents

Proposal 1-1a_v0

For PUCCH repetition for Msg4 HARQ-ACK,

- One bit information is transmitted as ‘repetition request or capability report’ in the existing agreements/working assumptions.

  - Only either repetition request or capability report is transmitted from each UE, and they are not differentiated in the signaling.

Proposal 1-1b_v0 (Note: proposal 1-1a_v0 in this draft will be updated after agreed)

Send an LS to RAN2 as the following.

Title: LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK Response to: - Release: Rel-18 Work Items: NR_NTN_enh-Core Source: RAN WG1 To: RAN WG2 CC: - Contact Person: #### Name: Shohei Yoshioka E-mail Address: syouhei.yoshioka.py@nttdocomo.com Send any reply LS to: 3GPP Liaisons Coordinator: mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: RAN1 has discussed how UE reports information related to PUCCH repetition for Msg4 HARQ-ACK and reached the following working assumption. Working assumption For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE. - Option B: Higher layer signaling in Msg3 PUSCH. It is noted that the following agreement was reached for repetition request or capability report. Proposal 1-1a_v0 For PUCCH repetition for Msg4 HARQ-ACK, - One bit information is transmitted as ‘repetition request or capability report’ in the existing agreements/working assumptions. - Only either repetition request or capability report is transmitted from each UE, and they are not differentiated in the signaling.. 2. Actions To TSG RAN WG2 ACTION: RAN1 respectfully asks RAN2 to provide feedback on the feasibility of Option B, and if feasible, to specify the details of Option B. 3. Date of Next TSG-RAN WG1 Meetings: TSG-RAN1 Meeting #114 Aug. 21st-25th, 2023 Toulouse TSG-RAN1 Meeting #114-bis Oct. 9th-13th, 2023 Xiamen

Q: Do you agree the above two proposals? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO for 1-1a | YES/NO for 1-1b | Comment |
| --- | --- | --- | --- |
| Ericsson | Yes | Yes with comment | RAN2 might not want to specify the details of option B before RAN1 has agreed the working assumption. Therefore, we suggest that RAN1 agrees the working assumption before sending the LS. A note could be added that the agreement is conditioned that RAN2 confirms the feasibility. |
| LG | Yes | Yes |  |
| Vivo |  |  | Either way is fine to us. Note that whether we call repetition request or capability report depends on whether we define RSRP threshold for UE to request the repetition of PUCCH on common PUCCH resource. |
| Panasonic |  |  | How the information is transmitted (e.g. use 1 bit field, use several codepoints, or others) is up to RAN2. But, “one bit information” may give an impression that a specific 1 bit field should be used. Therefore, we should avoid to describe “1 bit” but to describe what content is to be transmitted. Proposal 1-1a_v0 For PUCCH repetition for Msg4 HARQ-ACK, the following two states are informed by ‘repetition request or capability report’ in the existing agreements/working assumptions. - Either of repetition request from UE capable of Msg4 PUCCH repetition or Msg4 PUCCH repetition capability No indication |
| Nokia, Nokia Shanghai Bell | Yes | Yes | Since the gNB will be knowing best which repetition number to use, it will always be sufficient to have single-bit indication from UE to gNB. |
| ZTE | No | No | We think if “repetition request” is to be sent, UE should also support indicating “repetition factor” to network and at least 2-bit information is transmitted. Firstly, if repetition request is sent by UE, it means that UE better knows the performance of PUCCH for msg4 HARQ-ACK. Then UE indicated repetition factor should be beneficial for gNB to better determine which PUCCH repetition factor is better. If it is claimed that gNB can properly determine the repetition factor, then gNB is also able to properly determine whether a repetition is needed or not. Then it is enough to only support UE reporting repetition capability. No need to consider the mechanism of reporting repetition request as well as configuring a RSRP threshold. Secondly, indication of repetition factor has not been precluded. For repetition request/capability report in msg1, there was a concern on whether to support repetition factor indication since this may cause further PRACH fragmentation. However, for report in higher layer signaling of msg3, there is no such problem so that indication of repetition factor is by default feasible instead of precluded. Moreover, only 2-bit information is enough for UE sending the repetition factor as only 4 candidate values are supported. The overhead of additionally reporting 1bit information in msg3 higher layer signaling is really marginal if feasible. |
| Sharp | Yes | Yes |  |
| Huawei, HiSilicon | Yes | Yes |  |
| MediaTek |  |  | We are mainly fine with 1-1a and 1-b with some revisions. UE repetition request seems workable as the UE does not know the level of interference from other UE transmissions at the gNB receiver and there may be some difference between DL and UL path loss. We would propose to revise text ‘repetition request or capability report’ as “UE capability to support MSG4 HARQ Ack repetition” in 1-1a and 1-1b. Then, it is up to gNB to schedule PUCCH repetitions for Msg4 HARQ Ackbased on UE capability. |
| OPPO | Yes | Yes | Agree with Ericsson that we can confirm the working assumption before sending the LS. |

Online session outcome:

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- Two-state information is transmitted as ‘repetition request or capability report’ in the existing agreements/working assumptions.

  - The two-state information represents state 1: ‘repetition request or capability report’ or state 2: no indication.

  - How to transmit the two-state information is up to RAN2 when higher layer signaling is used for the transmission.

  - In state 1, only either repetition request or capability report is transmitted from each UE when transmitted, and they are not differentiated in the signaling.

  - Note: repetition request and capability report are defined as in the working assumption reached at RAN1#112.

Agreement

Draft LS to RAN2 in R1-2306085 is endorsed with the following change:

It is noted that an additional working assumption was reached for repetition request or capability report.

    - [Closed/High] Design target

RAN#99: NTN_enh offline discussion on Tuesday morning coffee break (draft discussion uutcomes)

Dear Nicolas and all, Thank you very much for moderating the discussion, and sorry for sending email just before the deadline. Regarding PUCCH enhancements for Msg4 HARQ-ACK, we had some offline discussion with companies. A number of companies is getting the intention of following Option 1 that it tried to address the coverage hole between 1) Option 2 and 2) PUCCH with dedicated PUCCH resource configuration. As discussed in our contribution RP-230358, if the configuration is not provided via Msg4 PDSCH, feedback of one or more subsequent PDSCH is conveyed on a common PUCCH. The coverage issue should be addressed together. Option 1: PUCCH transmission when dedicated PUCCH resource configuration is not provided Option 2: PUCCH transmission when PUCCH resource is indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI. However, it seems some companies need more time to consider this issue. Also, some other companies have concern whether the corresponding solution would be applied to CONNECTED mode as well if Option 1 is adopted and dedicated PUCCH resource configuration is not provided at all. Considering current situation, we would like to propose to set a checkpoint in RAN#100 whether there are missing cases in which “PUCCH enhancements for Msg4 HARQ-ACK” should address the coverage issue, and if identified, task WGs to address the issue. In 2Q’23, WGs can proceed their work according to the current WID. Proposal: Checkpoint in RAN#100 whether there are missing cases in which “PUCCH enhancements for Msg4 HARQ-ACK” should address the coverage issue, and if identified, task WGs to address the issue Best regards, Shinya NTT DOCOMO, INC.

Dear Shinya, all, Thank you for your response. May be best not to do anything at this meeting (no WID revision, no chair’s note). However, let us discuss how to revise the WID if needed at RAN#100, based on a comprehensive proposal. Best regards Nicolas PS: This doesn’t prevent companies to socialize further the issue at next RAN1 WG meetings

At RAN#99 meeting, it was discussed whether WID should be revised to cover the case where PUCCH transmission when dedicated PUCCH resource configuration is not provided, but the conclusion was pending. Based on this, it was discussed at the last RAN1 meeting whether this WI should cover all common PUCCH transmissions or not. Although most companies believe that this issue is valid and thus all common PUCCH transmissions should be considered, but no consensus unfortunately.

Then FL observed in this meeting that much more companies include this discussion in their contribution, and they indicate that the outcome has impacts on dynamic indication mechanism. Thus, FL suggests having discussion in this meeting again.

The current situation can be summarized as follows:

- Q: Should this WI consider PUCCH transmission after Msg4 HARQ-ACK and before RX of dedicated PUCCH config?

  - YES: 13 companies

  - NO: 1 company

FL believe that supporting companies explained well why all common PUCCH transmissions should be enhanced with repetition and thus FL recommends going with the majority view. Regarding objecting company’s concern, i.e., they believe that Msg4 can include dedicated PUCCH config, some repetition mechanism introduced for subsequent PUCCH transmissions does not exclude dedicated PUCCH config via Msg4. NW side can do that when NW side really wants, though NW does not have any PUCCH-related capabilities other than rep. At least some other NW vendors prefer not to limit repetition only for Msg4 HARQ-ACK. Given that, the following proposal is suggested.

      - 1st round

Design target

Proposal 1-2_v0

PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for:

- PUCCH transmission when dedicated PUCCH resource configuration is not provided.

  - Note: the existing agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to any PUCCH transmission by using common PUCCH resource, except that it is FFS how to determine repetition factor for PUCCH transmission scheduled by DCI format 1_0 with CRC scrambled by C-RNTI, i.e.,

    - The same configuration of PUCCH repetition provided via SIB is applied to any PUCCH transmission by using common PUCCH.

    - The same signaling of repetition request or capability report from UE is used for any PUCCH transmission by using common PUCCH.

    - The same frequency hopping mechanism is applied for any PUCCH transmission by using common PUCCH.

  - Note: It is not precluded for gNB to provide dedicated PUCCH config via Msg4 PDSCH.

Q: Do you agree the above proposal? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Ericsson | Yes |  |
| LG | Yes |  |
| Vivo | Yes. |  |
| Panasonic | YES |  |
| Nokia, Nokia Shanghai Bell | No | This expansion is not needed. If the gNB sees a need for having PUCCH repetitions configured beyond PUCCH for Msg4 HARQ-ACK, the gNB will simply provide the UL-BWP configuration as part of the Msg4, and UE will for later messages be able to provide the needed repetitions. |
| ZTE | Yes |  |
| Sharp | Yes |  |
| Huawei, HiSilicon | Need some addition | We can accept the extension if it can be agreed together that the same repetition of PUCCH indicated by DCI scheduling Msg4 is applied on the subsequent common PUCCH before the dedicated PUCCH is configured. |
| MediaTek | Yes |  |
| OPPO | Yes |  |

Online session outcome:

Agreement

If PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for PUCCH transmission when dedicated PUCCH resource configuration is not provided:

  - The agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to any PUCCH transmission by using common PUCCH resource

  - The same repetition factor is applied for PUCCH for Msg4 HARQ-ACK and subsequent PUCCH transmissions by using common PUCCH resource

  - Note: It is not precluded for gNB to provide dedicated PUCCH config via Msg4 PDSCH.

    - [Closed/High] Dynamic indication

For dynamic indication, the number of companies supporting/objecting each Alt can be summarized as follows:

- Alt 1-1a

  - Support: 9

  - Not support: 7

- Alt 1-1b

  - Support: 7

  - Not support: 8

- Alt 1-1c

  - Support: 3

  - Not support: 9

- Alt 1-1d

  - Support: 15

  - Not support: 3

- Alt 1-1e

  - Support: 2

  - Not support: 11

From the above count, FL suggests deprioritizing at least Alt 1-1c/Alt 1-1e. More companies have concern on less scheduling flexibility.

It seems that Alt 1-1d is the most agreeable one, but at the same time there are multiple comments as this alternative is not applicable if the direction of Proposal 1-2_v0 is agreed. Meanwhile, several companies propose to reuse the indicated repetition factor for Msg4 HARQ-ACK to any subsequent PUCCH transmissions by using common PUCCH. This would be a possible way, but further discussion would be necessary.

For Alt 1-1a, supporting companies believe that DL channel condition is not so good and hence Msg3 rep mechanism can be reused. However, objecting companies believe that DL channel condition may be good and thus higher MCS should be available for Msg4 PDSCH scheduling.

For Alt 1-1b, supporting companies believe that R17 dynamic PUCCH rep indication-like mechanism can be reused. However, objecting companies believe that scheduling flexibility and PUCCH capacity degrades. In addition, regardless of Alt 1-1b, 4 companies pointed out that capacity of common PUCCH is insufficient when rep is introduced.

Besides, it is proposed to define how many bits are used and which bit is used. This aspect should be included in proposal to conclude this topic.

Given the above, FL prepares proposals for both outcomes of agreeing Proposal 1-2_v0 and not.

      - 1st round

Dynamic indication

Proposal 1-3a_v0 (if Proposal 1-2’s direction is NOT agreed)

For PUCCH repetition for Msg4 HARQ-ACK,

- Support Alt 1-1d for dynamic indication of repetition factor.

  - Alt 1-1d: DAI field

- When two repetition factors are configured, 1 MSB bit is used for indication.

- When three or four repetition factors are configured, 2 bits are used for indication.

Proposal 1-3b_v0 (if Proposal 1-2’s direction is agreed)

For PUCCH transmission when dedicated PUCCH resource configuration is not provided,

- Down-select one of the following for dynamic indication of repetition factor:

  - Alt 1-1a: MCS field

  - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - FFS: whether/how to enhance capacity of common PUCCH

  - Alt 1-1d: DAI field

    - DAI field in DCI format 1_0 with CRC scrambled by TC-RNTI is used for indication.

    - The same repetition factor is used for PUCCH transmission scheduled by DCI format 1_0 with CRC scrambled by C-RNTI before dedicated PUCCH config is provided.

- When two repetition factors are configured, 1 MSB bit is used for indication.

- When three or four repetition factors are configured, 2 MSB bits are used for indication if Alt 1-1a or Alt 1-1b is supported, and 2 bits are used for indication if Alt 1-1d is supported.

Q: Do you agree each proposal? If YES for 1-3b, which alt is preferred? If NO, please share the reason and how the proposal should be updated.

| Company | Y/N for 1-3a | Y/N for 1-3b | Alt in 1-3b | Comment |
| --- | --- | --- | --- | --- |
| Ericsson | Y | Y | 1-1b |  |
| LG | Yes | Yes with comment | Alt 1-1d | We think that DAI field can be also used for dynamic indication in case when DCI format 1_0 with CRC scrambled by C-RNTI. This is because, according to the specification 38.213, (If a UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, the UE generates at most one HARQ-ACK information bit.) the UE should generate at most one HARQ-ACK information bit when the codebook information is not provided. That is, considering the normal gNB operation, codebook information would not be provided before providing the dedicated PUCCH resource set. As a result, before receiving the dedicated PUCCH resource set, the UE may not have codebook information, and the UE will not use the DAI field because it must generate only one HARQ-ACK information bit. Therefore, we can add additional possible approach (i.e. possible approach 2) the Alt 1-1d in proposal 1-3b_v0 as followings: - Alt 1-1d: DAI field - Possible approach 1 - DAI field in DCI format 1_0 with CRC scrambled by TC-RNTI is used for indication. - The same repetition factor is used for PUCCH transmission scheduled by DCI format 1_0 with CRC scrambled by C-RNTI before dedicated PUCCH config is provided. - Possible approach 2 DAI field in DCI format 1_0 with CRC scrambled by both TC-RNTI and C-RNTI is used for indication. |
| Vivo | Yes |  |  | Given we have limited time in RAN1, it would be preferred to follow the majority view to support proposal 1-3a. Regarding the number of repetitions, since it’s already agreed to have up to 4 numbers, 2 bits are needed in DCI, no further discussions seems necessary. Agreement For PUCCH transmission for Msg4 HARQ-ACK, supported number of transmissions are 1, 2, 4, 8. - Note: single PUCCH transmission is performed as in the existing specification, and/or (if supported for single PUCCH transmission) according to configuration/indication e.g., in signaling with respect to number of transmissions. - FFS: whether larger number of transmissions is supported - FFS: whether/how single PUCCH transmission can be configured and/or indicated |
| Panasonic | Y | Y | Alt 1-1b | Use of PUCCH resource indicator is straightforward. By PUCCH resource indicator and PDSCH-to-HARQ timing indicator, frequency/code domain and time domain flexibility are available. On the other hand, to extend common PUCCH resource should be allowed. |
| Nokia, Nokia Shanghai Bell |  |  |  | If DAI is being used, we should at least consider whether there would be any impacts on the potential use (or non-ability) to use Type-2 HARQ-ACK codebook. At least, if DAI is determined to be used for this purpose, a note should be made that Type-2 HARQ-ACK codebook is precluded for this scenario. |
| ZTE |  |  | 1-1d | We think even if dedicated PUCCH config is not included in msg4, it will not be configured far from msg4 since the RRC connection should be established as soon as possible after RACH procedure. Therefore, the channel condition of PUCCH transmission before dedicated PUCCH resource configuration should be similar to PUCCH for msg4 HARQ-ACK. In this case, the repetition factor configured for msg4 HARQ-ACK can be reused, which avoids additional spec efforts on designing new configuration mechanism and signaling. |
| Sharp | Y | Y | Alt 1-1d | We agree with LG that DAI field can be used before receiving the dedicated PUCCH resource set with 213 interpretation. Otherwise, we are also OK to use same RF with Msg4 HARQ-ACK. |
| MediaTek |  |  | Alt 1-1d | We may need to wait for progress on proposal 1-2 Alt 1-1d is included in 1-3a and 1-3b, and can be compromise. |
| OPPO | Y | Y | Alt 1-1b, 1d |  |

      - 2nd round

Dynamic indication

Proposal 1-3b_v1

For PUCCH transmission when dedicated PUCCH resource configuration is not provided,

- Support Alt 1-1d Down-select one of the following for dynamic indication of repetition factor:

  - Alt 1-1a: MCS field

  - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - FFS: whether/how to enhance capacity of common PUCCH

  - Alt 1-1d: DAI field

    - DAI field in DCI format 1_0 with CRC scrambled by TC-RNTI is used for indication.

    - The same repetition factor is used for PUCCH transmission scheduled by DCI format 1_0 with CRC scrambled by C-RNTI before dedicated PUCCH config is provided.

- When two repetition factors are configured, 1 MSB bit is used for indication.

- When three or four repetition factors are configured, 2 MSB bits are used for indication if Alt 1-1a or Alt 1-1b is supported, and 2 bits are used for indication if Alt 1-1d is supported.

Q: Do you agree the above proposal? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| LG | Yes with comment | We think second sub-bullet under Alt 1-1d is not necessary. This is because, we have already agreed that “The same repetition factor is applied for PUCCH for Msg4 HARQ-ACK and subsequent PUCCH transmissions by using common PUCCH resource”. So, we don’t need to agree it again. Alternatively, we can go with Proposal 1-3a_v0. Since we have already agreed to apply the same repetition factor when using common PUCCH, we think we only need to determine how to indicate the repetition factor of PUCCH for Msg4 HARQ-ACK. Proposal 1-3a_v0 For PUCCH repetition for Msg4 HARQ-ACK, - Support Alt 1-1d for dynamic indication of repetition factor. - Alt 1-1d: DAI field - When two repetition factors are configured, 1 MSB bit is used for indication. - When three or four repetition factors are configured, 2 bits are used for indication. |
| Xiaomi | Yes | We support this proposal. The proposal could be more concise by deleting the “The same repetition factor is used for PUCCH transmission scheduled by DCI format 1_0 with CRC scrambled by C-RNTI before dedicated PUCCH config is provided” as it has been agreed in another agreements. For PUCCH transmission when dedicated PUCCH resource configuration is not provided, - Support Alt 1-1d Down-select one of the following for dynamic indication of repetition factor: - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - FFS: whether/how to enhance capacity of common PUCCH - Alt 1-1d: DAI field - DAI field in DCI format 1_0 with CRC scrambled by TC-RNTI is used for indication. - The same repetition factor is used for PUCCH transmission scheduled by DCI format 1_0 with CRC scrambled by C-RNTI before dedicated PUCCH config is provided. - When two repetition factors are configured, 1 MSB bit is used for indication. When three or four repetition factors are configured, 2 MSB bits are used for indication if Alt 1-1a or Alt 1-1b is supported, and 2 bits are used for indication if Alt 1-1d is supported. |
| OPPO | Yes | Agree with LG and Xiaomi for the update. |
| Panasonic | YES |  |
| Vivo |  | For the 2nd bullet, could you clarify the intention of using MSB instead of using LSB? In our understanding, the field will not be used for DAI anymore as long as it is used for PUCCH repetition indication. So it would be natural to actually use LSB of the field since the MSB can be 0 when only 2 repetition factors are configured. |
| MediaTek | YES |  |
| Ericsson | Yes | Agree with comments from LG, Xiaomi and vivo. |
| Baicells | YES | Agree with comments from vivo on MSB. |

Dynamic indication

Proposal 1-3b_v2

For PUCCH transmission when dedicated PUCCH resource configuration is not provided,

- Support Alt 1-1d Down-select one of the following for dynamic indication of repetition factor:

  - Alt 1-1a: MCS field

  - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - FFS: whether/how to enhance capacity of common PUCCH

  - Alt 1-1d: DAI field

    - DAI field in DCI format 1_0 with CRC scrambled by TC-RNTI is used for indication.

    - The same repetition factor is used for PUCCH transmission scheduled by DCI format 1_0 with CRC scrambled by C-RNTI before dedicated PUCCH config is provided.

- When two repetition factors are configured, 1 LSB bit is used for indication.

- When three or four repetition factors are configured, 2 MSB bits are used for indication if Alt 1-1a or Alt 1-1b is supported, and 2 bits are used for indication if Alt 1-1d is supported.

Online session outcome:

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Support Alt 1-1d for dynamic indication of repetition factor:

  - Alt 1-1d: DAI field

    - DAI field in DCI format 1_0 with CRC scrambled by TC-RNTI is used for indication.

    - [Closed/Middle] Transmission trigger of UE information (RSRP threshold in working assumption)

For transmission trigger of UE information, i.e., the working assumption reached for configurability of RSRP threshold in RAN1#112 meeting, it was discussed in the last meeting whether the working assumption is valid and which Alt is down-selected for the RSRP threshold configuration. Any agreements were not reached since the validity of the working assumption was controversial.

For this meeting, companies shared in their contribution their views on this working assumption. FL summarizes them as below.

- Confirm the WA

  - Support: 10

  - Not support: 4

- RSRP threshold

  - Alt A: 1

  - Alt B: 15

For confirmation of the WA, it is argued that RSRP threshold is unnecessary since gNB can estimate UL quality, spec becomes complicated without benefit, and RSRP difference within NTN cell is small. Meanwhile, it is mentioned that RSRP threshold is beneficial because different UE types are considered and less UEs perform repetition when a single factor is configured. Besides, several companies point out that WA is not reverted unless any “critical” issue is found. The WA include skipping of configuration of RSRP threshold, thus no critical issue is found. Given that, FL recommends confirming the WA.

For RSRP threshold, the situation is clear, and Alt A is still possible in Alt B by gNB implementation. Besides, if Alt A is taken, gNB shall do repetition for both Msg3 PUSCH and Msg4 HARQ-ACK always simultaneously, which is pointed out by two companies. For the details of Alt B, there are two types of proposals in companies’ contributions. One is an absolute value as in R17 Msg3, and the other is a relative value to R17 Msg3 threshold. FL recommends having discussion of the down-selection. Regarding value X or value range, it will depend on the detailed parameter format; so it is not touched before concluding the point.

      - 1st round

RSRP threshold

Proposal 1-4_v0

Confirm the working assumption at the RAN1#112 meeting, with the following update.

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK

  - FFS: value of X (the maximum configurable value of the RSRP threshold)

  - Down-select one from the following alternatives for the RSRP threshold.

    - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

    - Alt B: New RSRP threshold is introduced.

  - Alt B: New RSRP threshold is introduced.

    - Note: the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation.

    - FFS: whether the definition of the new RSRP threshold is absolute value or relative value to the RSRP threshold in R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17)

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

Q: Please comment ONLY if you find critical issue. (FL’s note: if not critical issue, the proposal will not be updated.)

| Company | Comment |
| --- | --- |
| Ericsson | Support except last FFS. The new threshold should not be defined relative to an optional Rel-17 threshold. According to 38.331, presence of rsrp-ThresholdMsg3 means that both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication are configured in the BWP. If Msg3 repetition is not enabled, or enabled for all UE, the threshold is absent. |
| LG | Support the Proposal 1-4_v0. But, if Proposal 1-5_v0 is agreed, we can remove ‘at least’ in the main bullet. A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. |
| Vivo | We do not see the need to define “X” here. As long as an RSRP threshold is defined, UE would request the repetition only when the measured RSRP is lower than the RSRP threshold. Therefore, more updates are needed. Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Alt B: New RSRP threshold is introduced. - Note: the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation. - FFS: whether the definition of the new RSRP threshold is absolute value or relative value to the RSRP threshold in R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report. |
| Nokia, Nokia Shanghai Bell | We should at least capture that the RSRP value to be used for the selection should be the latest available at the UE side, such that we ensure that cases of NLOS are correctly captured (and UE only provides this indication if/when needed) |
| ZTE | In our view, the introduction of RSRP threshold is aimed to collect UE side information on the channel. If gNB cannot properly determine the configuration of PUCCH repetition, i.e., UE can know the channel better than gNB, UE report of repetition request and repetition factor can be beneficial for determining the repetition configuration. However, if gNB is already able to properly determine the PUCCH repetition, i.e., UE cannot know the channel better than gNB, letting UE to trigger the repetition will degrade the performance due to worse channel estimation. During discussion of required information bit for repetition request/capability report in msg3, a lot of companies claim that 1bit information report is enough since gNB can properly configure the PUCCH repetition factor, i.e., UE does not know the channel better than gNB. If this is the understanding of the group, the introduction of RSRP threshold is redundant and could even degrade the performance due to determining repetition based on worse channel estimation. Hence, we think the WA should not be confirmed. |
| Huawei, HiSilicon | OK. |

      - 2nd round

RSRP threshold

Proposal 1-4_v1

Confirm the working assumption at the RAN1#112 meeting, with the following update.

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

    - Note: the measured RSRP is the latest available value at the UE.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK

  - FFS: value of X (the maximum configurable value of the RSRP threshold)

  - Down-select one from the following alternatives for the RSRP threshold.

    - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

    - Alt B: New RSRP threshold is introduced.

  - Alt B: New RSRP threshold is introduced.

    - Note: the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation.

    - FFS: whether tThe definition of the new RSRP threshold is absolute value or relative value to the RSRP threshold in R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17)

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

Q: Please comment ONLY if you find critical issue.

| Company | Comment |
| --- | --- |
| LG | Support. And, we can remove ‘at least’ in the main bullet. A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. |
| Xiaomi | We are generally fine with the update. The Note that “the measured RSRP is the latest available value at the UE” seems redundant and may cause further problems. For example, if repetition request is carried by Msg3 and Msg3 re-transmission is performed, will UE change the content of Msg3 based on the latest measured RSRP value, if the new measured RSRP value changed to be higher than RSRP-threshold? Hence, we suggest to delete the note the measured RSRP is the latest available value at the UE. |
| OPPO | Support |
| Vivo | For the first sub-bullet, it would be good to use “indicates” instead of using “reports”, considering the capability report will be also performed after RRC connection. For the wording “tThe definition of the new RSRP threshold is absolute value” in Alt B, it should be clarified what the absolute value mean. In our understanding, it means an RSRP definition in the existing RRC spec., i.e. same range as the RSRP threshold defined for requesting Msg3 PUSCH repetition. 38.331: The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. For measurements, integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 156) dBm, except for the IE value 127, in which case the actual value is infinity. RSRP-Range ::= INTEGER(0..127). |
| MediaTek | We are mainly fine with moderator proposal. We do not see a strong need to use a new threshold. |
| Ericsson | Support. We are fine with updates proposed by LG, Xiaomi and Vivo. |
| Baicells | Agree. |

RSRP threshold

Proposal 1-4_v2

Confirm the working assumption at the RAN1#112 meeting, with the following update.

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK

  - FFS: value of X (the maximum configurable value of the RSRP threshold)

  - Down-select one from the following alternatives for the RSRP threshold.

    - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

    - Alt B: New RSRP threshold is introduced.

  - New RSRP threshold is introduced.

    - Note: the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation.

    - FFS: whether tThe definition of the new RSRP threshold is absolute value with same value range as the RSRP threshold for R17 Msg3 repetition or relative value to the RSRP threshold in R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17)

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

Online session outcome: No consensus in this meeting. FL emphasizes that this issue shall be concluded at the next meeting.

    - [Closed/Low] UE behavior when no repetition factor is configured

Although this topic was discussed and not agreed at previous meetings, FL observed that multiple companies propose to clarify the UE behavior. FL’s feeling is that if no rule is agreed, some UEs may assume repetition but other UEs may assume legacy behavior. To avoid misalignment between gNB and UE, clear agreement would be necessary.

7 companies believe that legacy behavior should be applied while 1 company suggests performing repetition if any other repetition-related parameter is configured. From this situation, it can be assumed that at least any repetition-related parameters including repetition factor are not configured, then there is no choice other than legacy behavior. This point can be agreed quickly (probably as conclusion), and the other aspect can be left to RAN2.

      - 1st round

No repetition factor config

Proposal 1-5_v0 (for conclusion)

For PUCCH repetition for Msg4 HARQ-ACK, when no repetition factor is configured via SIB, and if no other parameter related to the PUCCH repetition is configured via SIB, UE follows R15/16/17 specification.

Q: Do you agree the above proposal? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Ericsson | No | There is no need for this conclusion since it is obvious. |
| LG | Yes |  |
| Vivo |  | In our understanding, “when no repetition factor is configured via SIB” means no repetition would be supported, i.e. UE should follow legacy UE behavior in this case. Not sure any other parameters are necessary. |
| Panasonic | YES |  |
| Nokia, Nokia Shanghai Bell | Yes |  |
| ZTE | No | In our view, no other parameter related to the PUCCH repetition is needed to determine whether PUCCH repetition is needed. When no repetitio factor is configured via SIB, network just does not aim to support repetition. Hence, we think the proposal can be updated as follows. Proposal 1-5_v0 (for conclusion) For PUCCH repetition for Msg4 HARQ-ACK, when no repetition factor is configured via SIB, and if no other parameter related to the PUCCH repetition is configured via SIB, UE follows R15/16/17 specification. |
| Sharp | Yes |  |
| Huawei, HiSilicon | Yes |  |
| MediaTek | Yes | This is simplest way with no impact on specifications foreseen |
| OPPO |  | We are not clear what is “other parameter related to the PUCCH repetition” because for Msg4 PUCCH repetition, only 1 or 2 parameters are introduced, including repetition factor and RSRP threshold for Msg4 PUCCH repetition. Considering this issue belongs signaling-related aspects and the RSRP threshold is still under discussion, we prefer to leave it to RAN2 to specify the rule. |

It seems that this kind of proposal may not be essential; let’s skip discussion on this issue unless critical issue can be found. Discussion may be possible in maintenance phase.

    - [Closed/Low] Others

FL would like to ask whether there is any other essential topic by Aug. meeting for PUCCH repetition for Msg4 HARQ-ACK so that R18 NR NTN is declared ‘completed’.

      - 1st round

Q: Please comment ONLY if you find essential topic to be discussed/concluded by Aug. meeting.

| Company | Comment |
| --- | --- |
| LG | According to study item phase, the PUCCH format 1 with 14 OFDM symbols was considered as an enhancement target of PUCCH for Msg4 HARQ-ACK. So, we think explicit agreement or conclusion may be necessary that the NTN UE expects the parameter "pucch-ResourceCommon" to indicate one of indexes 11 to 15 in Table 9.2.1-1 in TS38.213, if the PUCCH repetition for Msg4 HARQ-ACK is configured. |
| Vivo | RAN1 needs to further check and discuss available slot determination and collision handling rules of PUCCH repetition for Msg4 HARQ-ACK. |

FL assumes that the above two issues are not essential. If spec update is really necessary, it can be proposed in maintenance phase. With that, this section was closed.

  - DMRS bundling for PUSCH taking into account NTN-specifics

For discussion of DMRS bundling, the following RAN4 requirements are considered, as stated/used in contributions.

- Timing error limit (Table 7.1C.2-1 in 38.133)

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

- Phase difference limit (Table 6.4.2.5-1 in 38.101-1)

Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p (NOTE 2) | Phase difference between slot 0 and any slot p (NOTE 3) |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [25] degrees | [30] degrees |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots. |  |  |  |

- Frequency error limit (Section 6.4.1 in 38.101-5)

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.] Requirement will be verified for at least two cases of which one has zero Doppler conditions.

    - [Closed/High] UE information report

There are multiple proposals to define new UE information report as new UE capability and as new UE assistance information. Although UE feature session for R18 NR NTN will be started, FL assumes that UE capability aspect should be discussed in this agenda item firstly. With that, UE feature session can decide the details.

FL suggests agreeing a list with all possibilities in this meeting, and at the next meeting we can down-select one or more. Regarding the existing capability, i.e., FG 30-4, it seems that companies have different view on whether this FG can be reused for DMRS bundling in NTN. The key point would be whether this FG can/will include UE behavior of “pre-compensation to keep phase rotation due to timing drift within the phase difference limit”. Clarification of this would be necessary.

| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A | Candidate values for the maximum duration for FDD are {4, 8, 16, 32} Candidate values for the maximum duration for TDD are {2, 4, 8, 16} NOTE: DM-RS bundling is only applicable for UL transmissions with pi/2 BPSK, BPSK, and QPSK modulation orders for the corresponding physical channels. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

      - 1st round

UE information report

Proposal 2-1_v0

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for UE information report.

- As UE capability report,

  - Option 1a: No new capability

    - Note: FG 30-4 is reported in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit

  - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed

    - Note: FG 30-4 is not reported for NTN band

  - Option 1c: Support of antenna switching with DMRS bundling

  - Option 1d: Max TDW size per NTN platform

    - FFS details

  - Option 1e: Max TDW size per elevation angle

- As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)),

  - Option 2a: No assistance information

  - Option 2b: Max TDW size based on reporting timing

  - Option 2c: TA adjustment timing

Q: Do you agree the above proposal? If some update is necessary or other option should be added, please share it. (FL’s note: no down-selection is intended. Please focus on refinement for each option)

| Company | YES/NO | Comment |
| --- | --- | --- |
| Ericsson | Yes with comments | UE capability: We prefer Option 1b. Option 1a (reuse of 30-4) might be ambiguous since a Rel-17 NTN UE without phase pre-compensation capability might report FG 30-4 for an NTN band. Unclear why 1d and 1e would be needed. UE knows the drift rate, and if UE supports digital phase pre-compensation, it should work for any drift rate? UE assistance information: No assistance information is needed. As for IoT NTN, the network can determine the need for TA updates based on ephemeris and configure segment length (TDW) accordingly. |
| Vivo |  | Max TDW size is already supported in TN which can be reused. On top of that, since NTN specific events are needed to support DMRS bundling in NTN, we propose to introduce “NTN DMRS bundling enhancement for PUSCH” as a new UE feature and FG 30-4a/b (DM-RS bundling for PUSCH repetition Type A/B) are the prerequisite FGs. |
| Panasonic | YES | On UE capability report, we think whether TA pre-compensation update to satisfy the timing requirement is taken into account needs also to be clarified. For option 1a and 1b, TA pre-compensation update is not taken into account while for option 1d and 1e, TA pre-compensation update is taken into account in our understanding. - Option 1a: No new capability - Note: FG 30-4 is reported in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit and without taking TA pre-compensation update into account. - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account. - Note: FG 30-4 is not reported for NTN band - Option 1c: Support of antenna switching with DMRS bundling - Option 1d: Max TDW size per NTN platform with taking TA pre-compensation update into account. - FFS details - Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account. Option 1c is not specific to NTN and independent from other options (i.e. add-on feature) in our understanding. It would be better to discuss as separate aspect. We support Option 1a or 1b. gNB needs to know the maximum TDW supported by the UE to configure nominal TDW appropriately. Not sure if the reported value of Option 1a and 1b are different. On UE assistance information, we are ok with the listed options. We support Option 2b. |
| LG | Yes | We support the proposal. Regarding the Option 1 family, we are not sure how Option 1b works. If gNB cannot distinguish when/whether UE performs phase pre-compensation, the information of “max TDW when pre-compensation … is performed” cannot be properly used. If UE always perform phase pre-compensation, then there would be no difference between Option 1a and 1b. We prefer option 1d, which is basically one or more ‘FG 30-4’ per NTN platform types. Regarding Option 2s, we are open and fine to discuss. We slightly prefer Option 2c. |
| Nokia, Nokia Shanghai Bell | Yes | Option 1a is preferred. This feature would lean on top of existing UE feature for coverage enhancements. Option 2b or 2c would be preferred. |
| ZTE |  | We think what really matters is that whether UE can maintain the phase continuity when facing a TA adjustment, which could lead to different UE behaviors on actual TDW determination. If UE cannot maintain the phase continuity, the actual TDW determination need to be determined based on pre-compensation segment (a segment is a period of transmission which use same pre-compensation value, as already defined in R17 IoT-NTN). If UE can maintain the phase continuity, TA adjustment will not have impact on actual TDW determination. Therefore, for proposal 2-1, we suggest to add option 1f Proposal 2-1_v0 For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for UE information report. - As UE capability report, - Option 1a: No new capability - Note: FG 30-4 is reported in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed - Note: FG 30-4 is not reported for NTN band - Option 1c: Support of antenna switching with DMRS bundling - Option 1d: Max TDW size per NTN platform - FFS details - Option 1e: Max TDW size per elevation angle - Option 1f: Whether support actual TDW across pre-compensation segment - As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)), - Option 2a: No assistance information - Option 2b: Max TDW size based on reporting timing - Option 2c: TA adjustment timing Moreover, we think the meaning of “TA adjustment timing” and “Max TDW size based on reporting timing” may need further clarification. Is it meant to directly report a time stamp, or report a segment length of TA adjustment so that network can derive when to update TA? From our point of view, directly indicating a time stamp of TA update can be costing. A stable time interval between TA update should be indicated no matter for reporting or configuration. |
| Huawei, HiSilicon |  | The “option 2d: the antenna switching interval” can be added to enable UE report the antenna interval to assist gNB determine the TDW size. |
| MediaTek |  | Option 1a with re-use Rel-17 FG 30-4 can be baseline. We can support Option 1c with antenna switching as a UE capability. We are not supportive of option 1d and 1e, which we think would have un-necessary impact on the specifications. On UE assistance options, no UE assistance needed (Option 2a). Options 2b and 2c seem to un-necessarily complicate specifications. |
| OPPO |  | UE capability: In our understanding, Option 1a already changed the definition of the existing UE capability (FG 30-4) because the legacy phase offset measurement is based on the phase response of the Tx chain as shown in Annex F.9 of 38.101-1, where NTN-specifics is not involved. In our matter, FG 30-4 should not be changed and be reported even in NTN band. Regarding the UE behavior of DMRS bundling taking into accounting NTN specifics, i.e., pre-compensation to keep phase rotation due to timing drift within the phase difference limit, it should be defined as a new UE capability. In our understanding, the new capability is the output of DMRS bundling enhancement in R18 NTN. However, for Option 1b, we can further discuss the UE capability is defined as the max TDW size or whether the UE can perform the aforementioned pre-compensation because the max TDW size varies with elevation angle. UE assistance information: Regarding the UE assistance information, we also think that the max TDW size varies with elevation angle and can be estimated based on the satellite beam by gNB, the TDW size-related information report is not needed. |

UE information report

Proposal 2-1_v1

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for UE information report.

- As UE capability report,

  - Option 1a: No new capability

    - Note: FG 30-4 is reported in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit and without taking TA pre-compensation update into account

  - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account

    - Note: FG 30-4 is not reported for NTN band

  - Option 1c: Support of antenna switching with DMRS bundling in NTN

  - Option 1d: Max TDW size per NTN platform with taking TA pre-compensation update into account

    - FFS details

  - Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account

- As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)),

  - Option 2a: No assistance information

  - Option 2b: Max TDW size based on reporting timing

    - FFS which timing is referred

  - Option 2c: TA adjustment timing

  - Option 2d: Antenna switching interval

      - 2nd round

Based on agreement at the UE feature session, Option 1a was removed.

UE information report

Proposal 2-1_v2

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for UE information report.

- As UE capability report,

  - Option 1a: No new capability

    - Note: FG 30-4 is reported in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit and without taking TA pre-compensation update into account

  - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account

    - Note: FG 30-4 is not reported for NTN band

  - Option 1c: Support of antenna switching with DMRS bundling in NTN

  - Option 1d: Max TDW size per NTN platform (e.g., LEO, MEO, GEO) with taking TA pre-compensation update into account

    - FFS details

  - Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account

  - Option 1f: Whether to support actual TDW across pre-compensation segments

- As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)),

  - Option 2a: No assistance information

  - Option 2b: Max TDW size based on reporting timing

    - FFS which timing is referred

  - Option 2c: TA adjustment timing

  - Option 2d: Antenna switching interval

Q: Do you agree the above proposal? If some update is necessary or other option should be added, please share it.

| Company | YES/NO | Comment |
| --- | --- | --- |
| LG | Yes | Regarding the Option 1 family, we are not sure how Option 1b works. If gNB cannot distinguish when/whether UE performs phase pre-compensation, the information of “max TDW when pre-compensation … is performed” cannot be properly used. If UE always perform phase pre-compensation, then there would be no difference current FG 30-4. We think Option 1b should be following - Option 1b: Max TDW size for NTN band as like FG 30-4 for TN band. - Note: FG 30-4 is not reported for NTN band We prefer option 1d, which is basically one or more ‘FG 30-4’ per NTN platform types. Regarding Option 2 family, we are open and fine to discuss. We slightly prefer Option 2c. |
| Xiaomi | Yes with comments | Regarding to UE capability report, we support option 1b and 1c. If our understanding of ‘without taking TA pre-compensation’ is correct, it should be TA pre-compensation is not performed during the Max TDW size. For 1d and 1e, it is not clear how many max TDW size should be reported, for example, the elevation angles could be 30 degree, 40 degree or 30 degree , 31 degree… Regarding the UE assistance information, we support option 2d. |
| OPPO | NO | Regarding DMRS bundling enhancement in NTN band, we have agreed that “pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side”, so the UE capability should reflect this directly. In this case, two capabilities can be considered to implement the pre-compensation, including Capability 1 (phase pre-compensation to keep phase rotation due to timing drift within the phase difference limit) and Capability 2 (TA pre-compensation update without violating phase continuity), and further discussed. In addition, in NTN scenario, the max TDW size varies with elevation angle and can be estimated based on the satellite beam by gNB, so the TDW size-related capability/information report is not needed. |
| Panasonic | Yes with comment | Option 1f needs some more clarification, e.g. definition of pre-compensation segments. Instead of defining pre-compensation segment in NR NTN, we would prefer the following description. Option 1f: Whether to support TA pre-compensation update within an actual TDW that does not violate the phase difference limit. On Option 1c and 2d, UE can report max TDW size with taking antenna switching timing into account if the UE supports it. Explicit indication of UE antenna switching capability and interval seems not be needed. |
| MediaTek | NO | We have same view as OPPO.. |
| Ericsson | Yes with comment | Capability: We support Option 1b and 1c. Option 1d and 1e try to capture the fact that more frequent TA precompensation updates are needed when the timing drift is higher, but this is already known by gNB and should be the same for all UE, so it does not have to be captured in the UE capability signaling. It is better (as in option 1b) if the UE reports capability disregarding the need for TA precompensation updates, while gNB configures a shorter nominal TDW than the indicated UE capability when the timing drift is higher (similar to IoT NTN segmented UL transmission). For Option 1f , Panasonic’s updated text is better. Assistance information: We support option 2a (no assistance information). Option 2b, 2c: The benefit of this reporting is limited. gNB can derive this info from ephemeris. Option 2d: The benefit of this reporting is limited. Trade-off between DMRS bundling gain and antenna switching gain can be made by gNB by configuring the nominal TDW. It is enough that gNB knows that UE is capable of antenna switching (and the number of TX antennas). |
| Baicells | Yes with comments | Support Option 1b with the following modification: Option 1b : Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and -without with taking TA pre-compensation update into account. Note: TA pre-compensation is not performed during the Max TDW size |

FL’s note: the following was agreed in 9.16.5.

Agreement: Introduce the following new row/FG for Rel. 18 NR NTN enhancements

| 44. NR_NTN_enh-Core | 44-2 | NTN DMRS bundling enhancement for PUSCH | 1. Support of DM-RS bundling for PUSCH over consecutive slots 2. Support of pre-compensation to keep phase rotation due to timing drift within the phase difference limit [3. Support not to perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.] |
| --- | --- | --- | --- |

UE information report

Proposal 2-1_v3

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for UE information report.

- As UE capability report (in addition to FG 44-2),

  - Option 1a: No new capability except for FG44-2

    - Note: FG 30-4 is reported [in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit and] without taking TA pre-compensation update into account

  - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account

    - Note: FG 30-4 is not reported for NTN band

  - Option 1c: Support of antenna switching with DMRS bundling in NTN

  - Option 1d: Max TDW size per NTN platform (e.g., LEO, MEO, GEO) with taking TA pre-compensation update into account

    - FFS details

  - Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account

  - Option 1f: Whether to support actual TDW across pre-compensation segments

    - FFS: definition of segments

  - Option 1g: Whether to support TA pre-compensation update within an actual TDW that does not violate the phase difference limit

- As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)),

  - Option 2a: No assistance information

  - Option 2b: Max TDW size based on reporting timing

    - FFS which timing is referred

  - Option 2c: TA adjustment timing

  - Option 2d: Antenna switching interval

Online session outcome:

Agreement

For NTN-specific PUSCH DMRS bundling,

- As UE capability report (in addition to FG 44-2), one or more of the following is down-selected.

  - Option 1a: No new capability except for FG44-2

    - Note: FG 30-4 is reported [in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit and without taking TA pre-compensation update into account]

  - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account

    - Note: FG 30-4 is not reported for NTN band

  - Option 1c: Support of antenna switching with DMRS bundling in NTN

  - Option 1d: Max TDW size per NTN platform (e.g., LEO, MEO, GEO) with taking TA pre-compensation update into account

    - FFS details

  - Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account

  - Option 1f: Whether to support actual TDW across pre-compensation segments

    - Segments defined in R17 IoT-NTN is baseline, FFS details

  - Option 1g: Whether to support TA pre-compensation update within an actual TDW that does not violate the phase difference limit

- As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)), one or more of the following is down-selected.

  - Option 2a: No assistance information

  - Option 2b: Max TDW size based on reporting timing

    - FFS which timing is referred

  - Option 2c: TA adjustment timing

  - Option 2d: Antenna switching interval

FL’s note: Companies are encouraged to consider all options and include opinions for each in their contribution for the next meeting. Down-selection shall be done at the next meeting.

    - [Closed/High] Nominal TDW determination

For nominal TDW determination also, FL suggests agreeing a list with all possibilities in this meeting, and at the next meeting we can down-select one or more. Meanwhile, definitely it is better to decide nominal TDW determination in this meeting.

      - 1st round

Nominal TDW determination

Proposal 2-2_v0

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for nominal TDW determination.

- Nominal TDW is determined by,

  - Alt 1: Existing configuration parameter, i.e., PUSCH-TimeDomainWindowLength

  - Alt 2: UE capability signaling, if nominal TDW is not configured

    - FFS: which UE capability signaling is used

  - Alt 3: Dynamic indication

    - FFS: details (e.g., multiple nominal TDWs are configured and one is indicated, nominal TDW size offset is indicated, etc.)

Q: Do you agree the above two proposals? If YES, which alt is preferred? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Alt | Comment |
| --- | --- | --- | --- |
| Ericsson | No | Alt 1+2 | Alt 1 and Alt 2 should be merged since they both describe Rel-17 behavior. We do not think dynamic indication (Alt 3) is justified. Even for LEO, the timing drift varies slowly enough for the nominal TDW to be (re)configured by RRC. |
| Vivo |  | Alt 1 | In our understanding, alt 1 is already agreed in last RAN1 meeting. Agreement For NTN-specific PUSCH DMRS bundling, support Alt 2 for TDW determination. - Alt 2: gNB-centric TDW determination - Nominal TDW is determined based on gNB configuration. - Actual TDW is determined based on gNB configuration/indication. - Note: Alt 2 does not imply that spec impact of actual TDW determination is assumed for NTN. - FFS: details, including UE capability and assistance information reporting. |
| LG | No |  | All alternatives seem not exclusive. We think Alt. 1 + 2 should be baseline based on previous agreement and current specification. |
| Nokia, Nokia Shanghai Bell | Yes | 1 (and 2) | Depending on which UE implementations we allow, we may need to have UE informing the network of its possible window sizes (which may vary as a function of time). Of course our preference is that the UE will simply be able to perform the per-compensation at any point in time and hence use existing parameters. |
| ZTE | Yes | Alt-1 | We think using existing configuration parameter is enough. |
| Huawei, HiSilicon |  |  | Atl.1+2 should be used together. |
| MediaTek |  | Alt-1 | We do not see a need to change specifications for nominal TDW. In Rel-17 NR coverage enhancement, a nominal TDW consists of one or multiple actual TDWs. An actual TDW is terminated in case an event occurs. A new actual TDW is created in response to semi-static events not triggered by DCI or MAC-CE. Whether a new actual TDW is created in response to dynamic events triggered by DCI is subject to UE capability. Frequency hopping and UL beam switching for multi-TRP operation are regarded as semi-static events. The UE shall maintain power consistency and phase continuity within an actual TDW across PUSCH transmissions or PUCCH repetitions. |
| OPPO |  | Alt 1+2 | The existing DMRS bundling procedure and parameter can be reused in NTN scenario. For Alt 3, the dynamic indication provides limited gain and the spec impact is not small, so it can be deprioritized. |

Nominal TDW determination

Proposal 2-2_v1

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for nominal TDW determination.

- Nominal TDW is determined by Alt 1 and Alt 2,

  - Alt 1: Existing configuration parameter, i.e., PUSCH-TimeDomainWindowLength, if configured

  - Alt 2: UE capability signaling, if nominal TDW is not configured

    - FFS: which UE capability signaling is used

  - Alt 3: Dynamic indication

    - FFS: details (e.g., multiple nominal TDWs are configured and one is indicated, nominal TDW size offset is indicated, etc.)

Proposal 2-2_v2

For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities.

- FFS: which UE capability(ies) signaling is(are) used

      - 2nd round

Nominal TDW determination

Proposal 2-2_v3

For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities.

- i.e., if PUSCH-TimeDomainWindowLength is configured, nominal TDW is determined by PUSCH-TimeDomainWindowLength; otherwise, nominal TDW is determined by UE capability(ies) signaling and M as defined in the clause.

- FFS: which UE capability(ies) signaling is(are) used

Q: Do you agree the above proposal or should we still postpone this discussion? If NO or ‘postpone’, please share the reason and how the proposal should be updated.

| Company | YES/NO/Postpone | Comment |
| --- | --- | --- |
| LG | Postpone | It would be better to have time to check whether the current specification work with only the replaced name of the UE capability. |
| OPPO | YES |  |
| Panasonic | YES |  |
| Vivo | YES |  |
| MediaTek | YES |  |
| Ericsson | YES |  |

Nominal TDW determination

Proposal 2-2_v4

For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities.

- i.e., if PUSCH-TimeDomainWindowLength is configured, nominal TDW is determined by PUSCH-TimeDomainWindowLength; otherwise, nominal TDW is determined by UE capability(ies) signaling and M as defined in the clause.

- FFS: which UE capability(ies) signaling is(are) used

- FFS: whether/how to use UE assistance information, if supported

Online session outcome:

Working assumption

For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities and assistance information (if needed).

- i.e., if PUSCH-TimeDomainWindowLength is configured, nominal TDW is determined by PUSCH-TimeDomainWindowLength; otherwise, nominal TDW is determined based on UE capability(ies) signaling.

- FFS: which UE capability(ies) signaling is(are) used

- FFS: whether/how to use UE assistance information, if supported

    - [Closed/High] Actual TDW determination

For actual TDW determination also, FL suggests agreeing a list with all possibilities in this meeting, and at the next meeting we can down-select one or more. It is noted that some UE-centric ways are excluded. Meanwhile, definitely it is better to decide actual TDW determination as well as nominal TDW in this meeting.

      - 1st round

Actual TDW

Proposal 2-3_v0

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for actual TDW determination.

- Actual TDW is determined by the existing events and,

  - Alt A: No additional event

  - Alt B: New event of TA pre-compensation timing indicated by gNB

    - FFS: detailed indication

  - Alt C: New event of actual TDW indicated by gNB

    - FFS: detailed indication

Q: Do you agree the above two proposals? If YES, which alt is preferred? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Alt | Comment |
| --- | --- | --- | --- |
| Ericsson | No | Alt A | It is unclear what Alt C means. Is the actual TDW length configured explicitly by gNB? Alt B and Alt C do not seem to have benefit over gNB defining nominal TDW. |
| Vivo |  |  | In our view, at least RAN1 should discuss whether the indication of epoch time or epoch time itself should be regarded as events which cause power consistency and phase continuity not to be maintained, when DMRS bundling is enabled. |
| LG |  |  | We are fine to discuss, however, it is not clear the meaning of Alt B and Alt C on following aspects. - Dynamic indication is assumed for Alt B and C? - “New event of actual TDW” is too ambiguous. Are there any examples to distinguish from Alt. B? |
| Nokia, Nokia Shanghai Bell | Yes | A |  |
| ZTE | Yes | Alt B | In R17 IoT-NTN, eNB configuration of pre-compensation segment has already been specified, i.e., eNB indicate a segment length and UE will divide the transmission into segments with each segment using same pre-compensation value. It is straightforward to reuse the same mechanism to let gNB determine when to update TA pre-compensation. Then, depending on UE capability, UE will determine actual TDW based on segment or not. |
| Huawei, HiSilicon |  |  | Agree with LG that it is clear on what “indicated by gNB” means. |
| MediaTek |  |  | We do not think new events are needed |
| OPPO | Yes | Alt B | We have agreed that “UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.”, so TA pre-compensation timing should be indicated by gNB to achieve common understanding on actual TDW between the gNB and UE. |

      - 2nd round

Actual TDW

Proposal 2-3_v1

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for actual TDW determination.

- Actual TDW is determined by the existing events and,

  - Alt A: No additional event

  - Alt B: New event of TA pre-compensation timing dynamically indicated by gNB

    - i.e., TA pre-compensation timing can be dynamically indicated by gNB

    - FFS: detailed indication

  - Alt C: New event of actual TDW as dynamically indicated by gNB

    - i.e., actual TDW can be dynamically indicated by gNB

    - FFS: detailed indication

  - Alt D: New event based on epoch time

    - FFS details

Q: Do you agree the above two proposals? If YES, which alt is preferred? If NO, please share the reason and how the proposal should be updated.

| Company | YES/NO | Alt | Comment. |
| --- | --- | --- | --- |
| LG | Yes | Alt. D | We think it would be difficult for gNB to predict TA update timing of UE. Thus, we don’t prefer Alt B and C. Meanwhile, at epoch time, UE should update TA soon. Therefore, it seems clear that TA need to be and will be pre-compensated. Therefore we support Alt. D |
| Xiaomi | Yes | D | We share similar view with LG. |
| OPPO | YES | B | We have agreed that “UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.”, so TA pre-compensation timing should be indicated by gNB to achieve common understanding on actual TDW between the gNB and UE, as segment transmission in R17 IoT NTN. |
| Panasonic | YES | Alt C | Actual TDW needs to be adjusted based on satellite location because UE pre-compensation update period is different depending on the satellite location. Comparing Alt B and C, actual TDW is determined by the gNB indication in both Alts, but Alt C is more generic, i.e. UE’s TA pre-compensation update timing does not necessarily need to follow the indicated timing but up to UE’s implementation. |
| Vivo | YES | Alt B and Alt D (At least Alt D) | To prioritize legacy UE features over DMRS bundling in NTN, events defined by Alt D and Alt B are preferred. |
| Ericsson | YES | Alt A | Alt A is sufficient. Alt B/C: Dynamic indication of actual TDW is not necessary. Even for LEO with quasi-earth fixed cells, the max TDW varies slowly (in the order of minutes) due to varying timing drift. RAN1 has concluded that TDW of 13 slots is feasible at 30° elevation angle while longer TDW (e.g. 20 for VoIP if 20 repetitions are used) will be possible for higher elevation angle. Existing RRC (re)configuration of nominal TDW will work fine. Alt D: TDW length is in the order of ms while epoch time update only occurs when UE reacquires SIB19 which happens at most every 5 seconds (probably less often). The TA update due to new epoch time can wait until the end of the nominal TDW. |
| Baicells | YES | Alt A | Other events mentioned in Alt B/C/D can be considered in the design of Nominal TDW. |

Actual TDW

Proposal 2-3_v1

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for actual TDW determination.

- Actual TDW is determined by the existing events and,

  - Alt A: No additional event

  - Alt B: New event of TA pre-compensation timing dynamically indicated by gNB

    - i.e., TA pre-compensation timing can be dynamically indicated by gNB

    - FFS: detailed indication

  - Alt C: as dynamically indicated by gNB

    - i.e., actual TDW can be dynamically indicated by gNB

    - FFS: detailed indication

  - Alt D: New event based on epoch time

    - FFS details

Online session outcome:

Agreement

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for actual TDW determination.

- Actual TDW is determined by the existing events and,

  - Alt A: No additional event

    - i.e., no spec impact is assumed for actual TDW determination.

  - Alt B: New event of TA pre-compensation timing dynamically indicated by gNB

    - i.e., TA pre-compensation timing can be dynamically indicated by gNB

    - Note: UE can perform TA pre-compensation update at the indicated timing

    - FFS: detailed indication

  - Alt C: as dynamically indicated by gNB

    - i.e., actual TDW can be dynamically indicated by gNB

    - FFS: detailed indication

  - Alt D: New event based on epoch time

    - FFS details

  - Alt E: New event based on antenna switching

FL’s note: Companies are encouraged to consider all options and include opinions for each in their contribution for the next meeting. Down-selection shall be done at the next meeting.

    - [Closed/Low] Others

FL would like to ask whether there is any other essential topic by Aug. meeting for PUSCH DMRS bundling in NTN so that R18 NR NTN is declared ‘completed’.

      - 1st round

Q: Please comment ONLY if you find essential topic to be discussed/concluded by Aug. meeting.

| Company | Comment |
| --- | --- |

No comment was received. This section was closed.

- Contribution summary

  - PUCCH enhancements for Msg4 HARQ-ACK

- Transmission trigger of UE information (RSRP threshold in WA at RAN1#112)

  - Confirm WA

    - YES: [15/CMCC] [17/Apple] [20/OPPO (?)] [23/DCM] [25/Pana] [3/vivo (?)] [5/HW, HiSi] [9/CHT] [10/xiaomi] [13/Ericsson]

      - [20/OPPO] R18 NR NTN considers a large range of UEs with different types, e.g., smartphones and VSAT, thus the RSRP range may be wide according to different UE type with different antenna gains. For this reason, introducing a RSRP threshold for Msg4 PUCCH repetition is beneficial.

      - [25/Pana] critical issues have not been found. As discussed in RAN1#112 in which the working assumption was agreed, reception power differs depending on UE, e.g. blockage/shade, antenna gain. Therefore, RSRP threshold (i.e. repetition request) would be useful especially for cell specific repetition configuration (i.e. only one repetition factor is indicated via SIB) because PRACH/Msg3 reception result can not be used to determine the PUCCH repetition.

      - [3/vivo] In our understanding, if gNB only configures single repetition factor in SIB, all UEs would have to do repetition as long as they have the repetition capability. In this case, RSRP condition would be necessary to prevent some of them from doing repetitions to save both energy and resources. If gNB configures multiple repetition factors, it’s true that gNB can adjust the repetition factors based on the uplink measurement which is normally what a gNB would do. However, it would still be beneficial if some of the UEs are precluded from requesting repetitions in a first step of Msg3 transmission. And the gNB measurement can be applied in a 2nd step to further determine which repetition factors, e.g. 1/2/4/8 repetitions, should be applied for PUCCH repetition after Msg3.

      - [5/HW, HiSi] Firstly, we do not see any reason to revert the WA, since the current WA has no critical issue being identified. To our understanding, it is necessary for UE to perform the repetition request. User terminals in deployment may varies a lot in terms of capability, transmit power, antenna type, etc. Thus the RSRP range may be wide even in a same satellite beam due to different UE type with different EIRP. As for the capability report. The introduced RSRP can also be configured with a value “X” based on gNB implementation to ensure all the capable UEs send the repetition request, which in turn enables the capability report from UE. Therefore, the working assumption already guarantees flexible gNB configuration and no need to be dropped.

      - [13/Ericsson] no critical issues have been identified

    - NO: [14/NEC(?)] [22/ZTE] [2/Nokia, NSB] [7/CATT]

      - [14/NEC] Proposal 1: Support only repetition request signalling for Msg4 HARQ-ACK.

      - [22/ZTE] defining RSRP threshold to configure PUCCH repetition is redundant, which make the specification more complicated without benefits.

      - [2/Nokia, NSB] definition of such a threshold is not necessary for the targeted feature, as before the Msg4 HARQ-ACK the gNB has already received two UL transmissions from the UE (i.e. PRACH and Msg3), from which it can already derive an accurate estimate of the UE UL conditions and assign a proper repetition factor for the subsequent Msg 4 HARQ-ACK PUCCH transmission.

      - [7/CATT] RSRP threshold is not needed for differentiate the PUCCCH repetition request and PUCCH capability report. As commented by other companies, in NTN scenario, RSRP threshold shows small difference compared between cell center and cell edge. Moreover, RSRP threshold and sensitivity are highly depending on UE receiver and weather condition. As a result, in realistic situation, it is difficult to set one suitable threshold to help UE to initiate the repetition request.

    - Comment

      - [18/QC] It should be noticed that the necessity of the RSRP threshold may be questionable in some cases, hence the configuration of RSRP threshold should be optional.

  - RSRP threshold

    - Alt A: [24/MTK]

      - [24/MTK] It is reasonable assumption that the path loss experienced by UE for MSG3 transmission is not expected to change change significantly with MSG4 HARQ ACK transmission.

    - Alt B: [15/CMCC] [17/Apple] [18/QC] [20/OPPO] [21/Samsung] [23/DCM] [25/Pana] [27/ETRI] [3/vivo] [5/HW, HiSi] [6/CCU, NTPU] [8/Intel] [9/CHT] [10/xiaomi] [13/Ericsson]

      - [15/CMCC] Considering that the payload of Msg4 HARQ-ACK is usually smaller than that of Msg3, and the coverage performance is different between PUCCH Msg4 HARQ-ACK and Msg3, it is reasonable that UE decides whether to request PUCCH repetition and Rel-17 Msg3 repetition based on different RSRP threshold.

      - [17/Apple] Proposal 2: The RSRP threshold for Msg4 PUCCH repetition is equal to the RSRP threshold for Msg3 PUSCH repetition minus an offset, where the offset is either pre-defined or configured.

      - [20/OPPO] It can be observed that the performance gap of Msg3 PUSCH is larger than that of Msg4 PUCCH, therefore the need of repetition for Msg3 PUSCH and Msg4 PUCCH does not always go hand-in-hand, so setting a unique threshold for Msg3 PUSCH and Msg4 PUCCH is not meaningful.

      - [21/Samsung] it is more flexible and Alt. B can provide Alt. A by gNB implementation.

      - [25/Pana] As described in section 2.2.1 , not to use this RSRP threshold when the network does not see the need of such usage should be also supported. This can be realized by to set the highest value of RSRP or not to configure RSRP value.

      - [27/ETRI] Proposal 2: The threshold for Msg3 repetition can be reused for PUCCH retransmission for Msg4 HARQ-ACK by adding or subtracting some amount of constant to rsrp-ThresholdMsg3-r17.

      - [3/vivo] Considering early PUCCH repetition feature doesn’t have to be dependent on Msg3 PUSCH repetition feature, the RSRP threshold should be independent from the RSRP threshold of Msg3 repetition. It is always up to network to configure the RSRP threshold, the same value between the new RSRP threshold and the RSRP threshold for R17 Msg3 repetition can be configured by gNB implementation. Thus, Alt A can be included into Alt B via gNB implementation.

      - [6/CCU, NTPU] Proposal 4: For PUCCH repetition for Msg4 HARQ-ACK, the RSRP threshold is indicated by the relative value to the RSRP threshold for Release 17 Msg3 repetition.

      - [8/Intel] [9/CHT] due to different SNR requirements for PUSCH and PUCCH reception at the gNB

      - [10/xiaomi] considering the payload of Msg3 and Msg4 HARQ-ACK are different

      - [13/Ericsson] Rel-17 Msg3 PUSCH repetition and Rel-18 Msg4 HARQ-ACK PUCCH repetition are optional features both for the network and the UE. Support for one of the features should not require support of the other.

  - Value range / Value X

    - Same range / 127: [15/CMCC] [20/OPPO] [21/Samsung] [23/DCM] [3/vivo] [5/HW, HiSi] [9/CHT (?)]

      - [3/vivo] as long as an RSRP threshold is configured for requesting PUCCH repetition for Msg4 HARQ-ACK, it’s a request of PUCCH repetition for Msg4 HARQ-ACK no matter whether X is a maximum allowed RSRP threshold or not.

    - Up to RAN2 for value X

      - [13/Ericsson]

- Information contents of ‘repetition request or capability report’

  - One bit information

    - YES: [15/CMCC] [20/OPPO] [21/Samsung] [23/DCM] [24/MTK (?)] [27/ETRI] [28/Sharp] [5/HW, HiSi] [8/Intel] [10/xiaomi]

      - [15/CMCC] the increased payload should also be avoided considering the NTN UE is more likely suffered from a bad channel condition

      - [20/OPPO] the repetition request or capability report carried in Msg3 PUSCH depends on the RSRP threshold configuration and will not exist simultaneously

  - Repetition factor indication

    - YES: [16/Lenovo] [11/Baicells]

    - NO: [20/OPPO] [21/Samsung] [24/MTK] [25/Pana] [27/ETRI] [28/Sharp] [8/Intel] [13/Ericsson]

      - [20/OPPO] when we are talking about Option B, UE indicating repetition factor has already been excluded as per the following working assumption

      - [21/Samsung] It would have very marginal gain or no benefit at all for a UE to provide the number of repetitions using 2 bits via higher layer signaling in Msg3 PUSCH.

      - [24/MTK] Observation 2: The UE cannot indicate the repetition factor for MSG4 HARQ Ack as the gNB reception may depend on a number of factors – i.e. difference in path loss in DL and UL, interference from other UE transmissions, scheduled MSC.

      - [28/Sharp] Observation 1: Repetition factor determination based on RSRP threshold at the UE side is not affected by PCMAX of UE. Observation 2: The repetition factor determination by using Msg1 based measurement at the gNB side enables to reflect not only pathloss but also the PCMAX of the UE in the determined repetition factor.

  - Send an LS to RAN2 as early as possible: [13/Ericsson]

- Dynamic indication details

  - Alt 1-1a

    - YES: [15/CMCC] [16/Lenovo] [27/ETRI] [3/vivo] [4/Spreadtrum] [5/HW, HiSi] [6/CCU, NTPU] [7/CATT] [9/CHT]

      - [15/CMCC] In the Rel-17 Msg3 repetition mechanism, part of the MCS bit field in RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI is repurposed for indicating Msg3 repetition factor. To avoid significant payload increasing and DCI format spec impact, similar design spirit could be reused

      - [4/Spreadtrum] We have noted that, in NTN scenario, the link budget is low so that the modulation order and the coding rate for transmission of Msg4 in NTN does not need to be large. Therefore, the 3LSB of MCS field maybe enough for Msg4 transmission and the 2MSB can be used for dynamic indication of repetition factors (2,4,8).

      - [5/HW, HiSi] Since the modulation order and the coding rate for transmission of Msg4 in NTN does not need to be large, given the relatively low link budget in appendix B.

      - [6/CCU, NTPU] Considering the low channel quality and ensuring the Msg4 PDSCH can successfully be received by UE, high modulation order and code rate of Msg4 PDSCH in NR NTN are not needed.

      - [7/CATT] MCS for transmission of Msg4 in NTN does not need to be large, due to the relatively bad channel quality and low link budget.

      - [9/CHT] Similar mechanism can be adopted for PUCCH repetition for Msg4 HARQ-ACK

    - NO: [20/OPPO] [22/ZTE] [23/DCM] [25/Pana] [28/Sharp] [8/Intel] [13/Ericsson]

      - [20/OPPO] [25/Pana] [28/Sharp] considering Msg4 PDSCH and Msg4 PUCCH may experience different channel quality, reusing MCS field for indication may cause restriction on Msg4 PDSCH transmission

      - [22/ZTE] considering that DL may have better channel condition, Alt 1-1a may limit the selection of PDSCH MCS, i.e., in some scenarios, higher MCS may be used (e.g. with higher satellite power)

      - [8/Intel] Given that the MCS field is not related to PUCCH and determines the MCS for PDSCH transmission, full range of MCS may be needed due to different DL coverage level comparing to UL coverage

      - [13/Ericsson] Limits PDSCH MCS selection

  - Alt 1-1b

    - YES: [16/Lenovo] [17/Apple] [25/Pana] [2/Nokia, NSB] [9/CHT] [11/Baicells] [13/Ericsson]

      - [17/Apple] Alt 1-1b has the least impact on DCI decoding and interpretation. This approach is applicable in the duration when dedicated PUCCH resource has not been configured after initial access.

      - [25/Pana] Although available PUCCH resources for PUCCH with repetition is fewer compared to PUCCH without repetition, considering that all slots are available for PUCCH transmission in FDD which is assumed in NTN and PDSCH-to-HARQ timing indicator gives additional flexibility, restriction of PUCCH resource indication would not be crucial.

      - [9/CHT] Similar mechanism can be adopted for PUCCH repetition for Msg4 HARQ-ACK

    - NO: [22/ZTE] [23/DCM] [24/MTK] [28/Sharp] [3/vivo] [4/Spreadtrum] [5/HW, HiSi] [7/CATT]

      - [22/ZTE] Alt 1-1b may impact the scheduling flexibility of PUCCH frequency resource, while introducing a new common PUCCH resource table with repetition factor per PUCCH resource will result in less flexibility and more specification impact.

      - [24/MTK] [28/Sharp] [4/Spreadtrum] [5/HW, HiSi] restrict gNB scheduler flexibility un-necessarily

      - [3/vivo] default common PUCCH resource table would be modified with repetition factor configuration per PUCCH resource, or a new common PUCCH resource table with repetition factor per PUCCH resource has to be introduced, which has less flexibility and more specification impact.

      - [5/HW, HiSi] the re-interpretation will have larger impact on the scheduling of PUCCH frequency location

      - [13/Ericsson] Semi-static configuration (in SIB) of repetition factor per PUCCH resource can cause resource segregation which might reduce total capacity of cell-specific PUCCH.

  - Alt 1-1c

    - YES: [23/DCM] [27/ETRI] [11/Baicells]

      - [23/DCM] Observation 1: When PUCCH repetition is applied before dedicated PUCCH config is provided, it is impossible for a single UE to transmit a lot of HARQ-ACK bits in short time duration. Therefore, only small number of DL HARQ processes are used in typical cases.

    - NO: [20/OPPO] [22/ZTE] [24/MTK] [25/Pana] [3/vivo] [4/Spreadtrum] [5/HW, HiSi] [7/CATT] [13/Ericsson]

      - [20/OPPO] [24/MTK] [5/HW, HiSi] scheduling flexibility issue seems to be the major defect.

      - [22/ZTE] Alt 1-1c may impact the flexibility for HPN configuration and cause stalling if the RTT is large.

      - [25/Pana] if PUCCH repetition is supported for PUCCH transmission before dedicated RRC configuration in addition to msg4 HARQ-ACK, shortage of the process number is concerned. Although PDSCH can not be continuously scheduled in case of PUCCH repetition (e.g. need to wait at least 4 slots in case of 4 PUCCH repetitions), only 4 HARQ processes might not be sufficient considering RTT of GEO (i.e. up to 541ms) as the worst case.

      - [3/vivo] HARQ process number is determined by higher layer and the relevant field would be on duty, which should not be changed

      - [4/Spreadtrum] If 2bits are used for dynamic indicator, there are only 4 HARQ process available which would seriously affect the flexibility for HARQ process configuration and throughput of network.

      - [7/CATT] But the random access process is initiated in the RRC reestablishment process and RRC resume request, it is possible of collision with other HARQ process.

      - [13/Ericsson] Reducing number of HARQ processes may cause stalling if the RTT is large.

  - Alt 1-1d

    - YES: [18/QC] [19/LGE] [20/OPPO] [22/ZTE] [23/DCM] [24/MTK] [27/ETRI] [1/Quectel] [2/Nokia, NSB] [3/vivo] [6/CCU, NTPU] [7/CATT] [8/Intel] [11/Baicells] [13/Ericsson]

      - [18/QC] Compared to other bit fields in the DCI, using the reserved DAI bits has the minimum impacts on specification and UE implementation

      - [19/LGE] if two repetition factors are provided via SIB, the MSB bit of DAI field can be used for dynamic indication

      - [22/ZTE] no impact on scheduling flexibility since DAI field is reserved currently

      - [6/CCU, NTPU] The 2 bits DAI field is a reserved field without any use.

    - NO: [25/Pana] [4/Spreadtrum] [5/HW, HiSi]

      - [25/Pana] if PUCCH repetition is supported for PUCCH transmission before dedicated RRC configuration in addition to msg4 HARQ-ACK, this field can not be used for the indication of the repetition factor. Furthermore, in general, to use reserved fields should be avoided as much as possible.

      - [4/Spreadtrum] To keep flexibly

      - [5/HW, HiSi] we prefer not to occupy all the two reserved bits, which should be kept for future purpose if other alternatives, e.g. MCS field,  can already support the indication of PUCCH repetition number.

  - Alt 1-1e

    - YES: [25/Pana] [27/ETRI]

      - [25/Pana] Although available PUCCH resource for PUCCH with repetition is restrictive compared to PUCCH without repetition, there woule be sufficient flexibility with PUCCH resource indicator and PDSCH-to-HARQ timing indicator considering HARQ-ACK can be assigned in any slot for FDD.

    - NO: [20/OPPO] [22/ZTE] [23/DCM] [24/MTK] [28/Sharp] [3/vivo] [4/Spreadtrum] [5/HW, HiSi] [7/CATT] [11/Baicells] [13/Ericsson]

      - [20/OPPO] [24/MTK] [28/Sharp] [4/Spreadtrum] [5/HW, HiSi] [7/CATT] [11/Baicells] [13/Ericsson] scheduling flexibility issue seems to be the major defect.

      - [22/ZTE] Alt 1-1e may constrain UL scheduling and affect scheduling flexibility because candidate “K1” values are strictly mapped to each the repetition factor.

  - Combinations

    - xx + xx:

    - NO:

  - Comment

    - [21/Samsung] Proposal 4: Down-selection on “which DCI field is used for dynamic indication of repetition factor” should be considered after discussing whether or not it can be applicable to “PUCCH transmission when dedicated PUCCH resource configuration is not provided”.

    - [25/Pana] Proposal 8: If only 1 bit is available to indicate Msg4 PUCCH repetition factor considering the loss of flexibility of each field, the following should be considered.

      - Option 1: use relative indication compared to Msg3 PUSCH repetition factor

      - Option 2: use multiple fields to indicate Msg4 PUCCH repetition factor

    - [8/Intel] For configuration of multiple repetition factors in SIB with UE-specific repetition factor indication, four repetition factors {1, 2, 4, 8} are always configured as candidate values for dynamic indication

- Design target

  - Repetition is applied for PUCCH before dedicated config is provided

    - YES: [18/QC] [19/LGE] [20/OPPO] [21/Samsung] [22/ZTE] [23/DCM] [25/Pana] [26/FGI] [28/Sharp] [3/vivo] [5/HW, HiSi] [10/xiaomi] [13/Ericsson]

      - [18/QC] the same repetition factor of the PUCCH for Msg4 HARQ-ACK applies to other PUCCH in response to a transmission scheduled by DCI format 1_0.

      - [19/LGE] Proposal 5. For PUCCH repetition for PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI, support MCS field for dynamic indication of repetition factor from gNB.

      - [19/LGE] the same dynamic repetition factor of PUCCH of Msg4 HARQ-ACK can be reused for another PUCCH repetition

      - [20/OPPO] Proposal 6: For PUCCH repetition after Msg4 PUCCH transmission and before dedicated PUCCH resource configuration, the PUCCH repetition follows the Msg4 PUCCH repetition factor.

      - [21/Samsung] Although “Msg4 PUCCH” is defined in WID, “Msg4” itself is not defined in the specifications and the PDSCH providing contention resolution is not necessarily the first PDSCH after Msg3 PUSCH is correctly received. Also, the issue is applicable in general until a UE is provided dedicated PUCCH resources which is an optional configuration (i.e. a NW is not required by the specifications to provide dedicated PUCCH resources to a UE).

      - [22/ZTE] Proposal 7: For PUCCH transmission when dedicated PUCCH resource configuration is not provided, the indicated repetition factor for PUCCH for msg4 HARQ-ACK will be extended as repetition factor for other common PUCCH transmission.

      - [25/Pana] Proposal 2: PUCCH repetition factor is indicated via DCI scheduling the PDSCH to which the HARQ-ACK is transmitted.

      - [28/Sharp] Observation 8: Since FDD band is used for NTN, payload size of the common PUCCH in FDD band is typically 1 bit, which is the same as Msg4 HARQ-ACK payload size. Proposal 5: The repetition factor determined for the Msg4 HARQ-ACK is also applied to the common PUCCH.

      - [3/vivo] Regarding the overhead, in our view, it’s still under control given the repetition factor is still up to network to configure and can even be dynamically scheduled when multiple repetition factors are configured in SIB1. For simplicity, the same dynamic indication of the repetition factor can be reused in relevant DCI as in DCI scheduling the Msg4 PDSCH.

      - [5/HW, HiSi] Proposal 2: The same repetition factor indicated by the DCI with CRC scrambled by TC-RNTI for PUCCH of Msg4 PDSCH is applied on the subsequent PUCCH transmission until dedicated PUCCH resource is configured.

      - [10/xiaomi] Proposal 2：Reuse the repetition factor of msg4 HARQ-ACK for the other common PUCCH transmission when dedicated PUCCH resource configuration is not provided.

      - [13/Ericsson] During initial access from RRC_IDLE state, the network does not know the UE identity, and therefore not the UE capabilities, when Msg4 is sent, and may decide to wait with configuring dedicated PUCCH until later.

    - NO: [2/Nokia, NSB]

      - [2/Nokia, NSB] if gNB realizes that PUCCH repetitions are needed for a certain UE after the Msg4 HARQ-ACK, the dedicated PUCCH configuration as part of the UL BWP configuration could be indicated to the UE in the Msg4, thereby ensuring that a UE will have the proper configuration for future operation.

  - Comment

    - [23/DCM] Discuss whether PUSCH transmission after Msg3 PUSCH and before dedicated configuration RX has coverage issue in NTN or not.

    - [26/FGI] RAN1 should consider both DCI format 1_0 and DCI format 1_1 with CRC scrambled by C-RNTI when designing PUCCH repetition for common PUCCH resources.

    - [8/Intel] RAN1 to discuss PUCCH for Msg4 HARQ-ACK which corresponds to PUCCH resource indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI. The need of PUCCH enhancements for other cases shall be discussed separately. If the PUCCH coverage enhancements (repetition) is supported for the case when dedicated PUCCH resource configuration is not provided, DCI format 1_0 shall be only considered

- Others

  - WA on container of UE information transmission

    - Confirm: [14/NEC] [9/CHT]

    - If infeasible, PRACH is used: [17/Apple] [19/LGE] [25/Pana] [4/Spreadtrum] [7/CATT]

      - [19/LGE] Observation 1. 4 or 8 LCID code points may be required when higher layer signaling of Msg3 PUSCH is used as container of the repetition request or capability report indicated by UE.

    - [10/xiaomi] Option 1: Using 1 reserved bit for indication, Option 2: Using LCID(s) for indication, Option 2-1: using 4 reserved LCID(s) in CCCH MAC sub-header for indication, Option 2-2: using 1 reserved LCID for new added MAC CE for indication

  - UE behavior when no repetition factor is configured

    - No rep: [14/NEC] [15/CMCC] [19/LGE] [23/DCM] [24/MTK] [6/CCU, NTPU] [11/Baicells]

    - Rep if other repetition-related parameters are configured: [10/xiaomi]

  - UE capability

    - Single: [14/NEC] [23/DCM] [28/Sharp] [13/Ericsson]

      - [14/NEC] In connected mode, PUCCH repetition capability indication applies to all the repetition factors in the set.

      - [23/DCM] UE reporting the capability supports any repetition factor, and both repetition with dynamic indication and repetition without dynamic indication

      - [28/Sharp] If UE does not support part of the repetition factors, the UE may not be guaranteed for continuous connections or access to high-altitude satellites.

      - [13/Ericsson] A single UE capability should be defined for repetition of Msg4 HARQ-ACK PUCCH (and other cell-specific PUCCH) ), covering both the cases of single and multiple repetition factors configured in SIB.

    - [13/Ericsson] Regardless of whether one or more Msg4 HARQ-ACK PUCCH repetition factors are configured in SIB, the UE indicates support for/requests Msg4 HARQ-ACK PUCCH repetition.

  - ‘1’ indication for dynamic indication

    - [15/CMCC] Proposal 4. If no larger number of PUCCH transmission is needed, when multiple repetition factors are configured via SIB, single PUCCH transmission can be indicated by the same way as repetition factor does.

  - Common PUCCH enhancement to solve capacity issue

    - YES: [19/LGE] [23/DCM] [25/Pana] [13/Ericsson]

      - [19/LGE] Proposal 3. Additional PRB offsets can be supported for each PUCCH repetition factors for Msg4 HARQ-ACK.

      - [25/Pana] If the same PUCCH resource is shared between no repeittion and repetition, the PUCCH resource can be collided as PUCCH repetition utilizes more PUCCH resource.

      - [13/Ericsson] If repetition on cell-specific PUCCH resources is configured in the cell, it will cost many PUCCH resources and other UE may not have adequate PUCCH resource or need to wait long time for transmission of PUCCH.

  - Common PUCCH resource set

    - [19/LGE] Proposal 7. The NTN UE expects the parameter "pucch-ResourceCommon" to indicate one of indexes 11 to 15 in Table 9.2.1-1 in TS38.213, if the PUCCH repetition for Msg4 HARQ-ACK is configured.

    - [19/LGE] Proposal 8. It is possible to consider creating a new PUCCH resource set table consisting only of PUCCH format 1 with 14 OFDM symbols for Rel-18 NR NTN UE.

  - Frequency hopping

    - [19/LGE] Proposal 9. Introduce higher payer parameter to indicate the inter-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK.

  - PUCCH format

    - [21/Samsung] Proposal 2: Support at least PUCCH repetitions for PUCCH format 1.

  - Repetition slot counting

    - [2/Nokia, NSB] Proposal 8: Postpone discussion on definition of N_PUCCH^repeat for PUCCH repetition for Msg4 HARQ-ACK to when the feature details are agreed and consolidated.

    - [3/vivo] Proposal 5: RAN1 needs to further check and discuss available slot determination and collision handling rules of PUCCH repetition for Msg4 HARQ-ACK.

    - [8/Intel] Proposal 6: Specification update needs to be considered to support available slot counting based on existing principles (as in section 9.2.6 of TS 38.213) for the repetition of PUCCH in response to Msg4 HARQ-ACK

  - Beam-level repetition

    - [4/Spreadtrum] Proposal 5: Beam-level repetition value configuration of PUCCH for Msg4 HARQ-ACK can be considered.

  - Beam-related

    - [8/Intel] Same Tx beam and same PUCCH resource are applied for all the repetitions of PUCCH transmission with HARQ-ACK in response to Msg4 PDSCH

  - DMRS bundling for PUSCH taking into account NTN-specifics

- Nominal TDW determination

  - Existing way: [15/CMCC] [21/Samsung] [22/ZTE (?)] [23/DCM] [24/MTK (?)] [6/CCU, NTPU] [13/Ericsson]

    - [13/Ericsson] The Rel-17 feature DMRS bundling already supports configuration of transmission segments equivalent to Rel-17 IoT NTN segmented uplink transmission through UE-specific RRC configuration of the nominal TDW length. No enhancement of the configuration is necessary.

    - [13/Ericsson] The gNB can e.g. determine the max Doppler/timing drift rate in a cell based on serving satellite ephemeris, and/or estimate/predict timing drift based on the UL signal from the UE, and configure the UE with a nominal TDW such that pre-compensation updates can be paused within the TDW without exceeding requirements on time/frequency accuracy.

  - Upper bounded based on UE report: [17/Apple]

  - If not configured, determined based on UE report: [17/Apple] [5/HW, HiSi]

  - Discuss how to address information related to NTN circumstances: [19/LGE]

  - Adjusted based on satellite movement: [2/Nokia, NSB]

    - [2/Nokia, NSB] Observation 5: By adjusting the duration of nTDW (via gNB configuration/indication), to account for the timing drift caused by the movement of the satellite or UE specific TA update rate, and assuming that NTN UE pre-compensates its TA at the boundaries of nTDW, no ambiguity remains between gNB and NTN UE for DMRS bundling framework in NTN.

  - TDW size associated with PUSCH repetition factor: [4/Spreadtrum]

    - [4/Spreadtrum] We think that different elevation angles could be associated with different TDW size. Considering that different elevation angel represent different repetition number, we can conclude that there might be a relationship between PUSCH repetition number and TDW size.

  - Multiple nominal TDWs are configured, and one is indicated: [10/xiaomi]

    - [10/xiaomi] The duration of UE can maintain phase continuity increased with the decrease of timing drift correspondingly. In other words, the length of the time window that UE can maintain phase continuity is varying with the elevation angle.

- Actual TDW determination

  - Existing way: [15/CMCC] [20/OPPO] [21/Samsung] [23/DCM] [24/MTK] [6/CCU, NTPU] [13/Ericsson]

    - [20/OPPO]: Proposal 7: For DMRS bundling in NTN, the UE can only perform TA pre-compensation update at the boundary of the configured TDW, and UE reporting information is not needed.

  - An event when the NTN specific information used for deriving the TDW size is updated via DCI: [19/LGE]

  - gNB indication of pre-compensation segment length (FL’s note: timing? Definition of ‘segment’ is unclear…): [22/ZTE] [7/CATT]

  - gNB indication of period for TA adjustment: [12/Hyundai]

  - gNB indication of actual TDW: [25/Pana] [2/Nokia, NSB]

    - [25/Pana] gNB would likely configure the TDW as a conservative value because gNB’s estimation can be not so accurate. In order to use more accurate TDW, one way would be that UE reports the assistance information, e.g., possible TDW considering the satellite elevation angle at the reporting timing, and gNB determines actual TDW based on the assistance information

    - [2/Nokia, NSB] Observation 6: By gNB configuration/indication of the rate or frequency of UE TA update within PUSCH repetitions window (nTDW), both gNB and NTN UE are aware of the instances that aTDW may be broken within an nTDW.

  - NTN TA drift compensation as the new event (FL’s note: how gNB decides it is unclear, and this seems to be not gNB-centric way): [27/ETRI] [3/vivo]

  - The indication or update of epoch time: [3/vivo] [8/Intel]

    - [3/vivo] The validity timer could be started/restarted with configured timer validity duration at the epoch time. That means, the serving satellite ephemeris and common TA related parameters share the same epoch time. If the validity timer is restarted due to the update of epoch time within a TDW, the phase continuity and/or power consistency would not be maintained.

  - gNB configuration for antenna switching interval: [5/HW, HiSi]

  - New common TA and satellite ephemeris application: [8/Intel]

- UE information report

  - Existing capability report: [20/OPPO] [21/Samsung]

    - [20/OPPO] the gNB can provide an appropriate TDW as in segment based transmission in R17 IoT NTN

    - [9/CHT] Whether additional information reporting is necessary depends on whether pre-compensation is taken into account for maxDurationDMRS-Bundling reported by UE. If maxDurationDMRS-Bundling is reported assuming pre-compensation, assistance information on the pre-compensation timing from UE, e.g., time duration to satisfy phase difference limit, is necessary; otherwise, additional information reporting other than maxDurationDMRS-Bundling is not necessary.

  - New capability report for NTN: [15/CMCC] [24/MTK (?)] [5/HW, HiSi] [8/Intel]

    - [8/Intel] consider impact of UE autonomous adjustment of TA and frequency for PUSCH transmission

  - UE capability of “pre-compensation to keep phase rotation within the phase difference limit and keep timing error within the timing error limit”

    - YES: [17/Apple] [18/QC (?)] [23/DCM] [12/Hyundai] [13/Ericsson]

      - Max TDW size with the pre-compensation: [17/Apple] [18/QC (?)]

        - [17/Apple] in which duration a UE can pre-compensate to keep phase rotation within the phase difference limit is up to UE capability

        - [18/QC] Depending on implementation, different maximal TDW duration can be supported.

      - [23/DCM] This capability is mandatory to report support of PUSCH DMRS bundling in NTN.

    - NO: [19/LGE]

      - [19/LGE] To determine proper DMRS bundling window size, it is necessary for UE to provide UE capability report such as maximum TDW size. The performance of phase pre-compensation would be considered to decide the maximum TDW size.

  - UE capability of DMRS bundling + antenna switching

    - Support DMRS bundling with antenna switching: [18/QC]

      - [18/QC] two DMRS bundling windows of 8 slots each provides more than 1.5 dB gain than one DMRS bundling window of 16 slots

    - [1/Quectel] Proposal 3: Antenna switching is supported in actual TDW size of PUSCH DMRS bundling

    - [5/HW, HiSi] Option 1: the antenna switching capability (e.g. switchable antenna number) to the gNB, Option 2: the antenna switching interval

    - [10/xiaomi] Proposal 7: Antenna switching is supported when the TDW size of PUSCH DMRS bundling is lower than a threshold.

  - UE capability based on NTN platform type

    - YES: [19/LGE]

      - [19/LGE] In NTN, the side effect of the phase difference could be dependent on NTN platform, rather than whether the UE can manage it or not. Thus, It is highly recommended to consider UE capabilities regarding NTN platforms and those properties.

  - UE capability on whether to support actual TDW across pre-compensation segment

    - YES: [22/ZTE]

      - [22/ZTE] Obviously, for UE without capability to maintain DMRS bundling across segments, the actual TDW is restricted by segment length. While for UE with the capability, segment length may not be a restriction.

  - Report max TDW length, when needed, based on satellite parameters at the reporting timing

    - YES: [23/DCM] [25/Pana]

      - [23/DCM] If Option 1 is used, so many reporting signalings need to be defined, which leads to large RAN1 workload and also large overhead.

        - Option 1: UE reports several capabilities of max TDW size based on satellite parameters

  - UE capability on max TDW size without assuming timing drift

    - YES: [25/Pana]

      - [25/Pana] because TA pre-compensation timing is up to UE implementation, it is preferable to determine the actual TDW based on the UE assistance information.

  - Information report based on UE’s elevation angle, UE velocity, TA variation, possible TDW length, other information to have impact on the TDW length.

    - YES: [27/ETRI]

  - Report TA adjustment timing

    - YES: [1/Quectel]

      - [1/Quectel] the UE is expected to notify gNB whether the nominal TDW is broken by the new events. An implicit indication is preferred for the notification

  - Dynamic information report

    - YES: [11/Baicells]

  - No assistance information

    - YES: [13/Ericsson]

      - [13/Ericsson] For LEO 1200 with typical beam sizes, a suitable TDW length can be derived by gNB for each beam without assistance information from the UE.

- Others

  - [2/Nokia, NSB] Observation 3: By providing pre-compensation values from gNB to NTN UE that account for accumulated UE TA pre-compensation updates within nTDW, NTN UE can apply one pre-compensation value, at the beginning of PUSCH repetitions, based on its experienced positive or negative timing drift. Therefore, NTN UE is able to withhold its TA update for longer time and not breaking aTDW. Option-1: gNB configuration of pre-compensation values, accounting for potential negative/positive timing drift of an NTN UE within nTDW.

  - [8/Intel] Additional gaps to adjust the UE pre-compensation values are not supported for NR NTN

- References

- R1-2304408	Coverage enhancements for NR NTN	Quectel

- R1-2304434	Coverage enhancements for NR over NTN	Nokia, Nokia Shanghai Bell

- R1-2304496	Discussions on remaining issues of coverage enhancements in NR NTN	vivo

- R1-2304573	Discussion on coverage enhancements for NTN	Spreadtrum Communications

- R1-2304632	Discussion on coverage enhancement for NR NTN	Huawei, HiSilicon

- R1-2304678	Discussion on coverage enhancements for NR NTN	CCU, NTPU

- R1-2304751	Further discussion on UL coverage enhancement for NR NTN	CATT

- R1-2304815	On coverage enhancement for NR NTN	Intel Corporation

- R1-2304863	Discussion on coverage enhancement for NR NTN	China Telecom

- R1-2304916	Discussion on coverage enhancement for NR-NTN	xiaomi

- R1-2304920	Discussion on coverage enhancement for NR NTN	Baicells

- R1-2304965	Discussion on coverage enhancement for NR NTN	Hyundai Motor Company

- R1-2305006	On coverage enhancements for NR NTN	Ericsson

- R1-2305068	Coverage enhancement for NR NTN	NEC

- R1-2305109	Discussion on coverage enhancement for NR NTN	CMCC

- R1-2305212	Coverage enhancements for NR NTN	Lenovo

- R1-2305259	Coverage Enhancement for NR NTN	Apple

- R1-2305352	Coverage enhancements for NR NTN	Qualcomm Incorporated

- R1-2305390	Discussion on coverage enhancement for NR NTN	LG Electronics

- R1-2305436	Discussion on coverage enhancement for NR NTN	OPPO

- R1-2305529	On coverage enhancement for NR NTN	Samsung

- R1-2305556	Discussion on coverage enhancement for NTN	ZTE

- R1-2305611	Discussion on coverage enhancement for NR NTN	NTT DOCOMO, INC.

- R1-2305640	UL coverage enhancements	MediaTek Inc.

- R1-2305699	Discussion on coverage enhancement for NR-NTN	Panasonic

- R1-2305782	Discussion on coverage enhancements for NR NTN	FGI

- R1-2305800	Discussion on coverage enhancement for NR NTN	ETRI

- R1-2305847	Discussions on Coverage enhancement for NR NTN	Sharp

- Appendix-1 (Copy from WID RP-230809)

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the “solutions developed by general NR coverage enhancement” (NR_cov_enh) to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

- Appendix-2 (Outcomes of post meetings)

  - RAN1#109-e

Agreement

For NR NTN coverage enhancement, evaluate only handset terminals as UE type.

- i.e., VSAT is not considered.

Agreement

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

- Step 2: Required SNR of target service is evaluated by LLS

- Step 3: The CNR and the required SNR are compared

Agreement

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

- Note: MEO can be evaluated optionally

Agreement

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

Agreement

For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- Each packet is transmitted within 20 ms, if packet combining is not used.

  - Companies are encouraged to evaluate at least packet transmission without combining

  - Companies are encouraged to report how to apply packet combining, if used.

  - Note: in packet combining, two packets can be combined into a single packet at TX side

    - Companies should report the impact on E2E latency

- VoIP is evaluated only in LEO scenario.

- Note 1: PRB/MCS/TBS determinations are discussed separately

- Note 2: companies should report if HARQ is used in the evaluations, and if evaluations depart from the assumption that each packet is transmitted within 20 ms

Agreement

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, and as in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band.

- In addition, evaluations assuming relevant ITU regulatory limitations on power flux density can be reported in the study phase.

  - Companies should report which value of EIRP density is used and corresponding justification.

Agreement

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 10000 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | As agreed separately |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

Agreement

If corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- For VoIP, max 20 PUSCH repetitions if SCS = 15 kHz and packet combining/HARQ are not applied; otherwise, max 32 PUSCH repetitions with consideration of the impact on E2E latency

- For low-data rate service, max 32 PUSCH repetitions

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

Agreement

For low-data rate service, the following target data rate is assumed.

- For DL, 3 kbps if satellite EIRP density lower than values in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, or values in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band due to ITU regulatory limitations on power flux density is considered; otherwise, 1 Mbps

- For UL, 3 kbps and 100 kbps

  - FFS: which data rate applies for GEO/MEO/LEO

Agreement

For NR NTN coverage enhancement, the following channels/signals can be evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits

- PUCCH format 3 with 11 bits

- PRACH format 0

- PRACH format 2

- PRACH format B4

- PUSCH Msg.3

- PUCCH for Msg.4 HARQ-ACK

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDCCH

- Broadcast PDCCH (PDCCH of Msg.2)

Agreement

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 1 TX, 2TX (optional) / 2 RX with omni-directional antenna element Note: companies should provide their assumption on polarization |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

R1-2205622	[Draft] LS on UE antenna gain for NR NTN coverage enhancement	Moderator (NTT DOCOMO, INC.)

R1-2205623	LS on UE antenna gain for NR NTN coverage enhancement	RAN1, NTT DOCOMO, INC.

Final LS is endorsed in R1-2205623.

Agreement

For coverage performance evaluation, the following elevation angle is assumed.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

  - Note: For GEO-Set 1, channel parameters for 10 deg is used in LLS.

- 30 deg for MEO

- Other elevation angles can be evaluated as optional

- Note: these values are elevation angles at the edge of the edge beam.

Agreement

For NR NTN coverage enhancement, evaluate the following cases.

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | 12.5 | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | 20 | Handset | S-band | Low-data rate service |
| 3 (Optional) | LEO-1200 | 1 | 30 | Handset | S-band | VoIP |
| 4 | LEO-1200 | 2 | 30 | Handset | S-band | VoIP |
| 5 | LEO-1200 | 2 | 30 | Handset | S-band | Low-data rate service |
| 6 (Optional) | LEO-600 | 1 | 30 | Handset | S-band | VoIP |
| 7 | LEO-600 | 2 | 30 | Handset | S-band | VoIP |
| 8 (Optional) | LEO-600 | 2 | 30 | Handset | S-band | Low-data rate service |
| 9 (Optional, with higher priority than case 10) | MEO | 1 | 30 | Handset | S-band | Low-data rate service |
| 10 (Optional) | MEO | 2 | 30 | Handset | S-band | Low-data rate service |

Agreement

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

- Evaluation scenario

  - Rural (LOS/NLOS)

  - Sub-urban (LOS/NLOS) (optional)

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Not assumed

- Frequency offset: 0.1 ppm

Agreement

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 11 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| Repetitions | w/ repetition. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, Format 2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| HARQ configuration | Whether/How is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Other parameters | Reported by companies. |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

  - RAN1#110

Conclusion

For Rel-18 coverage enhancement in NTN, NLOS environment is deprioritized.

Agreement

For NR-NTN coverage enhancement, RAN1 concludes that coverage enhancements specifically for GEO and MEO are de-prioritized in Rel-18.

- Potential enhancements for LEO can also apply to GEO and MEO

Agreement

For NR-NTN coverage enhancement in Rel-18, link budget of parameter set-1 for LEO-1200 operating at LOS is considered as the target to evaluate whether each channel/signal with the existing specification needs to be enhanced or not. The targeted performances are used to evaluate the following services:

- VoIP using AMR 4.75 kbps.

- Low data rate of 3 kbps.

- Potential enhancements for deployments with parameter set-1 can also apply for deployments for parameter set-2

Observation

For PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS,

- Five sources observed that the existing specification can meet the performance requirement

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with at least 0.6 dB gap

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH for Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Three sources observed that the existing specification cannot meet the performance requirement with a gap of 1.8 to 6 dB.

Conclusion

RAN1 concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

Conclusion

RAN1 concluded that enhancement is unnecessary for PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PRACH format 0 with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Eight sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 5.3 dB

For PRACH format 2 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification can meet the performance requirement

- Two sources observed that the existing specification cannot meet the performance requirement with a gap of 1.9 to 8.8 dB

For PRACH format B4 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification cannot meet the performance requirement with a gap of 1.2 to 11.9 dB

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

For PUSCH for VoIP with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement with a margin of 0 to 1.7 dB

  - One company simulated by using 20 repetitions without DMRS bundling

  - Four companies simulated by using 20 repetitions with DMRS bundling

  - One company simulated by using 32 repetitions with DMRS bundling

    - Note: this is the only result using frame combining by application layer

- Nine sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 8.6 dB

  - Eight companies simulated by using 20 repetitions without DMRS bundling

  - Seven companies simulated without frequency hopping

  - One company simulated by using 16 repetitions with DMRS bundling

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

RAN1 concluded that enhancement for PUSCH for VoIP may be needed to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain, when DMRS bundling is not applied.

Observation

For Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with a gap of 1.5 dB.

Conclusion

RAN1 concluded that enhancement is unnecessary for Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

  - RAN1#110bis-e

Agreement

For PUCCH for Msg4 HARQ-ACK,

- Support PUCCH repetition

  - Further discuss the specification impact for at least the following

    - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability indication before Msg4, etc.)

    - Repetition factor

    - Repetition slot counting for FDD

  - Further study whether to enhance or support the following

    - Frequency hopping

    - DMRS bundling

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss the following options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Option 3: UE indicates repetition capability and is dynamically indicated to perform repetition

  - How UE indicates repetition capability before Msg4

Conclusion

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied.

  - FFS: whether specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is needed.

Agreement

For NTN-specific PUSCH DMRS bundling,

- Discuss further the need of enhancement in consideration of at least the following:

  - Phase difference due to timing drift and/or doppler shift.

    - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement and/or recommend RAN4 to update the requirement or UE should pre-compensate phase difference by UE implementation, etc.

  - An event which causes power consistency and phase continuity not to be maintained.

    - e.g., whether the new event is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change or whether such event may not occur depending on implementations, etc.

  - Note: baseline performance for legacy UEs can include antenna switching

Agreement

For PUCCH transmission for Msg4 HARQ-ACK,

- Supported number of transmissions are 1, 2, 4, 8.

  - Note: single PUCCH transmission is performed as in the existing specification, and/or (if supported for single PUCCH transmission) according to configuration/indication e.g., in signaling with respect to number of transmissions.

  - FFS: whether larger number of transmissions is supported

  - FFS: whether/how single PUCCH transmission can be configured and/or indicated

  - RAN1#111

Conclusion

For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1.

Conclusion

RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP

- FFS: How to determine TDW size, including UE capability.

- Note: The above does not mean the performance requirements will be satisfied with DMRS bundling

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- One or more repetition factors may be configured via SIB

  - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor

    - FFS: whether UE requests repetition or indicates repetition capability

  - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB

    - FFS: whether UE requests repetition or indicates repetition capability

    - FFS: whether repetition factor is indicated by UE

  - FFS: UE behavior when repetition factor is not configured via SIB

  - FFS: whether one or more UE capabilities are needed for the above is for further discussion

  - RAN1#112

Observation

For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

- Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed.

  - FFS: whether/how to consider the initial timing error at the beginning

  - FFS: TA pre-compensation update is assumed

- Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed.

- FFS: impact of phase difference limit

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE.

- Option A: PRACH preamble and/or occasion

  - FFS: whether PRACH resource partitioning is needed for indication of [repetition request  or capability report]

  - FFS: whether or not indication of repetition factor is assumed

  - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings

- Option B: Higher layer signaling in Msg3 PUSCH

  - FFS: which signaling is used

  - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2.

- Option C: Physical layer signaling in Msg3 PUSCH

  - FFS: which signaling is used, e.g. DMRS ports

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

  - Alt 1-2: New field with one or two bits

- Alt 2: Field in DCI scheduling Msg3 PUSCH

  - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor

  - Note: it is assumed that there is impact on DCI design

- Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH

  - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc.

- Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK

  - FFS: value of X (the maximum configurable value of the RSRP threshold)

  - Down-select one from the following alternatives for the RSRP threshold.

    - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

    - Alt B: New RSRP threshold is introduced.

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

  - RAN1#112bis-e

Observation

For NTN-specific PUSCH DMRS bundling,

- In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

  - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed.

  - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier.

Working assumption

For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side.

- UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.

  - FFS: how to determine the actual TDW

- FFS: specification impact

- Send an LS to RAN4

Agreement

Final LS is endorsed in R1-2304094 with the following revision to the action:

ACTION: RAN1 respectfully asks RAN4 to take the above RAN1 observations and agreement working assumption into account.

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE.

- Option B: Higher layer signaling in Msg3 PUSCH

Send an LS to RAN2 at RAN1#113 to provide details of “repetition request or capability report”, to ask the feasibility of Option B, and if feasible, to specify the details of Option B.

Agreement

For NTN-specific PUSCH DMRS bundling, support Alt 2 for TDW determination.

- Alt 2: gNB-centric TDW determination

  - Nominal TDW is determined based on gNB configuration.

  - Actual TDW is determined based on gNB configuration/indication.

  - Note: Alt 2 does not imply that spec impact of actual TDW determination is assumed for NTN.

  - FFS: details, including UE capability and assistance information reporting

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, support Alt 1-1 for dynamic indication of repetition factor from gNB. Further discuss which field(s) to be used.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field(s)

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, apply frequency hopping mechanism in R15/16/17 defined for PUCCH transmission for Msg4 HARQ-ACK, in every slot.

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, candidate values of only one repetition factor configuration via SIB are {2, 4, 8}.

- i.e., configuration of only ‘1’ is not supported.

  - RAN1#113

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- Two-state information is transmitted as ‘repetition request or capability report’ in the existing agreements/working assumptions.

  - The two-state information represents state 1: ‘repetition request or capability report’ or state 2: no indication.

  - How to transmit the two-state information is up to RAN2 when higher layer signaling is used for the transmission.

  - In state 1, only either repetition request or capability report is transmitted from each UE when transmitted, and they are not differentiated in the signaling.

  - Note: repetition request and capability report are defined as in the working assumption reached at RAN1#112.

Agreement

Draft LS to RAN2 in R1-2306085 is endorsed with the following change:

It is noted that an additional working assumption was reached for repetition request or capability report.

Final LS in R1-2306105

Agreement

If PUCCH repetition discussed in R18 NR NTN coverage enhancement is supported for PUCCH transmission when dedicated PUCCH resource configuration is not provided:

  - The agreements and working assumptions for PUCCH for Msg4 HARQ-ACK are applied to any PUCCH transmission by using common PUCCH resource

  - The same repetition factor is applied for PUCCH for Msg4 HARQ-ACK and subsequent PUCCH transmissions by using common PUCCH resource

  - Note: It is not precluded for gNB to provide dedicated PUCCH config via Msg4 PDSCH.

Working assumption

For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities and assistance information (if needed).

- i.e., if PUSCH-TimeDomainWindowLength is configured, nominal TDW is determined by PUSCH-TimeDomainWindowLength; otherwise, nominal TDW is determined based on UE capability(ies) signaling.

- FFS: which UE capability(ies) signaling is(are) used

- FFS: whether/how to use UE assistance information, if supported

Agreement

For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for actual TDW determination.

- Actual TDW is determined by the existing events and,

  - Alt A: No additional event

    - i.e., no spec impact is assumed for actual TDW determination.

  - Alt B: New event of TA pre-compensation timing dynamically indicated by gNB

    - i.e., TA pre-compensation timing can be dynamically indicated by gNB

    - Note: UE can perform TA pre-compensation update at the indicated timing

    - FFS: detailed indication

  - Alt C: as dynamically indicated by gNB

    - i.e., actual TDW can be dynamically indicated by gNB

    - FFS: detailed indication

  - Alt D: New event based on epoch time

    - FFS details

  - Alt E: New event based on antenna switching

Agreement

For NTN-specific PUSCH DMRS bundling,

- As UE capability report (in addition to FG 44-2), one or more of the following is down-selected.

  - Option 1a: No new capability except for FG44-2

    - Note: FG 30-4 is reported [in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit and without taking TA pre-compensation update into account]

  - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account

    - Note: FG 30-4 is not reported for NTN band

  - Option 1c: Support of antenna switching with DMRS bundling in NTN

  - Option 1d: Max TDW size per NTN platform (e.g., LEO, MEO, GEO) with taking TA pre-compensation update into account

    - FFS details

  - Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account

  - Option 1f: Whether to support actual TDW across pre-compensation segments

    - Segments defined in R17 IoT-NTN is baseline, FFS details

  - Option 1g: Whether to support TA pre-compensation update within an actual TDW that does not violate the phase difference limit

- As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)), one or more of the following is down-selected.

  - Option 2a: No assistance information

  - Option 2b: Max TDW size based on reporting timing

    - FFS which timing is referred

  - Option 2c: TA adjustment timing

  - Option 2d: Antenna switching interval

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Support Alt 1-1d for dynamic indication of repetition factor:

  - Alt 1-1d: DAI field

    - DAI field in DCI format 1_0 with CRC scrambled by TC-RNTI is used for indication.

- Appendix-3 (Contact information)

| Company | Name | Email |
| --- | --- | --- |
| FL (DCM) | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com syouhei.yoshioka.py@nttdocomo.com |
| Lenovo | Hongmei Liu | Liuhm6@lenovo.com |
| Apple | Chunxuan Ye | Chunxuan_ye@apple.com |
| Apple | Chunhai Yao | Chunhai_yao@apple.com |
| Xiaomi | Min Liu | Liumin10@xiaomi.com |
| Xiaomi | Yajun Zhu | zhuyajun@xiaomi.com |
| vivo | Zhipeng Lin | zhipeng.lin@vivo.com |
| vivo | Yong Wang | wy.wang.5g@vivo.com |
| Nokia | Frank Frederiksen | Frank.frederiksen@nokia.com |
| OPPO | Hao LIN | lin.hao@oppo.com |
| OPPO | Zuomin WU | wuzuomin@oppo.com |
| OPPO | Nande Zhao | zhaonande@oppo.com |
| Huawei, HiSilicon | Xiaolei TIE | tiexiaiolei@huawei.com |
| Huawei, HiSilicon | Ying Chen | chenying18@huawei.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| ZTE | Fangyu Cui | cui.fangyu@zte.com.cn |
| CATT | Deshan Miao | miaodeshan@catt.cn |
| Ericsson | Stefan Eriksson Löwenmark | stefan.g.eriksson@ericsson.com |
| Thales | Mohamed EL JAAFARI | mohamed.el-jaafari@thalesaleniaspace.com |
| Spreadtrum | Zhenzhu Lei | reven.lei@unisoc.com |
| MediaTek | Gilles Charbit | Gilles.charbit@mediatek.com |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Sony | Samuel Atungsiri | Sam.Atungsiri@sony.com |
| Lockheed | Robert Olesen | robert.l.olesen@lmco.com |
| ETRI | Dukhyun You | dhyou@etri.re.kr |
| ETRI | Jung-Bin Kim | jbkim777@etri.re.kr |
| ETRI | Gyeongrae Im | imgrae@etri.re.kr |
| Panasonic | Akihiko Nishio | nishio.akihiko@jp.panasonic.com |
| Samsung | Sungjin Park | sj100.park@samsung.com |
| Samsung | Carmela Cozzo | carmela.c@samsung.com |
| Omnispace | Ron Olexa | rolexa@omnispace.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Ligado | Clive Packer | clive@ligado.com |
| Hughes/EchoStar | Munira Jaffar | Munira.Jaffar@EchoStar.com; munirajaffar@hughes.com |
| Qualcomm | Xiao Feng Wang | wangxiao@qti.qualcomm.com |
| Qualcomm | LiangPing Ma | lpma@qti.qualcomm.com |
| Novamint | Thierry Bérisot | tberisot@novamint.com |
| GateHouse | Robert van der Pool | rvp@gatehouse.com |
| FGI | YenHua Li | danielli@fginnov.com |
| LG | Haewook Park | haewook.park@lge.com |
| LG | Seokmin Shin | seokmin.shin@lge.com |
| LG | Duckhyun Bae | duckhyun.bae@lge.com |
| Baicells | Xiang Yun | yunxiang@baicells.com |
| Baicells | Yong Ding | dingyong@baicells.com |
| Sharp | Toshi Nogami | nogami.toshizoh@sharp.co.jp |
| Sharp | Makoto Kitahara | kitahara.makoto@sharp.co.jp |
| Sharp | Hiro Takahashi | takahashi.hiroki@sharp.co.jp |
| China Telecom | Jianchi Zhu | zhujc@chinatelecom.cn |