3GPP TSG RAN WG1 Meeting #114	R1-2306426

Toulouse, France, August 21-25, 2023

Agenda Item:	9.10.2

Source:	Futurewei

Title:	UE based RACH-less TA determination for inter-cell LTM

Document for:	Discussion and decision

# Introduction

In RAN1 #113 meeting, RAN1 confirmed the working assumption of supporting the UE based RACH-less TA acquisition [1]:

Agreement

Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism

Working Assumption

From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

In RAN2 #122 meeting, RAN2 reached the following agreement for RACH-less initial transmission to the target cell [2]:

- Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell:

  - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch.

  - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

- Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

In this contribution, we pointed out it is essential in Rel-18 mobility enhancement WID to support UE based RACH-less TA determination in inter-DU mobility scenarios where the source and target DUs/cells are generally not precisely synchronized. We also discuss more details on UE based RACH-less LTM to clarify some of the questions raised in last meeting.

# Discussion

## Target TA determination by UE for inter-DU LTM

Figure 1 illustrates that under the inter-DU inter-cell scenario, the timing relation of the reference signals at the source and target cells/TRPs and received at the UE.

Figure 1: Inter-DU mobility source and target cells/TRPs reference timing relationship

Figure 1 illustrates that the UE measured reference signal time difference (RSTD) between the source and target cells includes DU1 and DU2 timing difference (DU2_Offset – DU1_Offset), and DU1 to UE versus DU2 to UE one way delay (OWD) difference (OWD2 – OWD1). The OWD includes the front-haul propagation delay and TRP to UE air propagation delay. In other words, the UE measured DL reference timing offset includes not only propagation delay difference but also at least the asynchronous timing offset between the two DUs. In inter-DU scenario, source DU1 and target DU2 are generally not precisely synchronized. There is timing offset between the (sub-frame boundaries of) data transmission at source DU1 and target DU2, and the timing offset can change over the time due to possible drifting of the local clocks with the DUs. In addition, the fronthaul between the DU and the TRP as well as the air interface may also introduce asymmetric DL and UL propagation delay.

Observation 1: DUs are generally not precisely synchronized. The timing offset between the source and target DUs can change over time due to their clock drifting.

Observation 2: The UE measured DL reference timing offset includes not only the propagation delay to the UE from the source DU versus the target DU, but also the timing offset between the two asynchronous DUs.

Since the UE based TA measurement is to use the source cell TA and the source versus target cells’ propagation delay difference to derive the target cell TA, if in inter-DU scenarios the RSTD is directly used, the timing offset between the DUs can cause large inaccuracy on the derived target cell TA. Therefore, the timing offset between the asynchronous DUs should be remove to allow the UE based TA measurement to be used in inter-DU mobility scenario.

Observation 3: Timing offset between the asynchronous DUs should be removed to allow the UE based TA measurement to be used in inter-DU mobility scenario.

Fortunately, the major factors affecting target TA accuracy, e.g., the timing offset between the source and target DUs, are static or semi-static and they can be easily determined by the network at the mobility preparation and pre-configuration phase or at initial calibration. Then the combined adjustments can be configured to the UE by RRC pre-configuration. In normal mobility scenario with asynchronous source and target nodes, the final TA of the target cell/TRP can be determined at the UE by:

Target_TA = Source_TA + 2*RSTD + TA_NT_Adj_Factor,

where TA_NT_Adj_Factor is the network TA adjustment factor determined by the network which compensates the biases such as timing offset between DUs and DL/UL propagation asymmetry associated with TRPs of the source and target DUs.

Observation 4: The bias of non-ideal synchronization and DL/UL asymmetry on the UE RSTD measurement can be indicated by the network to the UE and eliminated when the UE derives TA.

Revisiting Rel-18 mobility enhancement WID, one can see that the scope of this WID including the intra-CU/inter-DU and the source and target cell are non-synchronized scenarios.:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

…

- Timing Advance management [RAN1, RAN2]

…

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

In order to achieve the objective of minimizing latency and supporting the fast cell switches in LTM as required by this WID, RAN1 agreed to support UE based TA measurement without the need of RACH prior or during a cell switch. To further fulfil the above scope of the WID, UE based TA measurement shall support mobility across inter-DUs which are generally not precisely synchronized. Network assistance with compensation is essential to allow UE based TA measurement to be used in much wider range of inter-DU mobility scenarios.

Observation 5: Network assistance for non-ideal bias compensation is essential to allow UE based TA measurement to be used in Rel-18 inter-DU mobility scenarios.

In previous discussions on the TA network adjustment factor, there was a question on whether the RRC pre-configured network adjustment factor is still valid at the time of cell switch if it is obtained at the earlier mobility preparation phase. The adjustment factor does include the source and target nodes’ (e.g., DUs) timing offset. We consider it semi-static. The timing offset between the DUs is mainly due to their local clock drifting. Clock drifting is normally a much slower process than mobility activities. For accurate clocks, it will take hours for drifting 1us. With low-cost clock it can still take tens of minutes to drift 1us. Before the clock drifting can cause intolerable timing offset, pre-configured cell switch has been completed. The subsequent delta configurations for sequential LTM can update the nodes timing offset in time. Therefore, the network nodes’ timing offset determined at the LTM preparation phase should be valid at the UE execution of the cell change. In fact, the source/target nodes timing offset determined by the CU is per node pair and can be used for many UEs which perform LTM within the same short period.

Observation 6: Normally, the source node and candidate node timing offset change due to clock drifting is much slower than mobility activities. Therefore, the compensation factor determined and configured to the UE at LTM preparation is valid at cell switching.

Based on above discussion, we propose:

Proposal 1: Support network assisted non-ideal bias compensation for UE based TA measurement in non-precisely synchronized inter-DU mobility scenarios.

- The network non-ideal bias compensation factor is RRC pre-configured to the UE.

Since the UE continues moving in the serving cell, the source serving cell TA previously updated and currently maintained at the UE may become imperfect/inaccurate over the time.  The UE based TA measurement and the special case of target TA = source TA rely on the accuracy of the source TA. In order to ensure the accuracy of the target TA derived by the UE with the source TA, it is desirable to update the dynamic change on the source TA at the time of issuing cell switch command.  In cell switch command MAC CE, there is a previously agreed TA field specified for the target cell TA (for PDCCH-ordered RACH), when UE based RACH-less TA measurement is enabled, this field can be indicated to carry the dynamic change of the source TA.

Proposal 2: The cell switch command includes a bit field to indicate whether the TA field contains the target TA or the dynamic change of the source TA, whether the source TA change is used for source TA update for UE based TA measurement, or for directly target TA = source TA.

The detailed procedure of the UE based TA solution can be simply summarized as follows:

- At preparation phase, a TA network adjustment factor is configured to the UE via RRC pre-configuration message for compensation in source/target asynchronous scenarios. The factor is set to 0 for synchronous case.

- At DL synchronization phase, upon acquiring/synchronizing with a candidate reference signal (e.g. a SSB), the UE measures the timing difference between its locally tracked source cell reference signal timing and the candidate reference signal timing, which in fact is the source-vs-candidate RSTD. Then the UE maintains the synchronization and tracks the candidate reference signal timing by updating and storing the RSTD.

- When cell-switch is triggered, the source cell sends the latest dynamic source cell TA change with the cell switch command to the UE. The cell switch command also indicates the target beam.

- Upon the reception of the cell switch command, the UE derives update from current serving cell TA:

Target_TA = Source_TA + delta Source_TA + 2*RSTD + TA_NT_Adj_Factor

- Then the UE adjusts its source TA to the Target_TA and applies it on top of the timing of the UE locally tracked target reference signal for RACH-less UL data transmission to the target cell.

It should be pointed out: the step 2 in the UE based RACH-less procedure is anyway performed for early DL synchronization with candidate cells/beams.

Observation 7: The complexity and impact of UE based RACH-less scheme to standards are moderate.

## Initial UL transmission when RACH-less access target cell

In RAN2 #122 meeting, RAN2 agreed to support (pre-)configured grant (CG) for RACH-less initial transmission to the target cell:

- Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

When RACH-less access to the target cell is enabled, upon received the cell switch command, the UE determines and applies the target TA and performs initial UL transmission to the target cell with CG. The transmission timing of the RACH-less initial UL transmission should follow the received target reference signal timing plus the UE determined target TA.

Observation 8: Upon received cell switch command, the UE performs the initial UL transmission to the target cell following the target cell reference timing and target TA.

Another function with legacy random access is to indicate the best DL beam to be used by the UE. Dedicated preambles corresponding to candidate SSB/CSI-RSs are configured to the UE by RRC mobility configuration. When performing random access, the UE transmits the preamble corresponding to the selected SSB or CSI-RS to indicate the selected target SSB or CSI-RS to the target cell.

For RACH-less access to the target, there is no dedicated preamble to serve the purpose. As indicated in above RAN2 agreement from RAN2#122 meeting, the grant occasion is associated with the selected target beam which serves the purpose of indicating the selected target beam to the target cell. The limitation of this approach is that the CG occasion cycle has to be much shorter than the RACH cycle to ensure a much less latency with RACH-less LTM. As a result, the number of CG occasions will be very small which imposes a limitation on the number of candidate-beams to be pre-configured.

An alternative solution was proposed in [3], that dedicated SRS associated with candidate beams of the candidate cells can be preconfigured to the UE to serve the purpose. Upon a cell switch or target cell access is triggered, the UE performs the initial transmission to the target cell including to transmit SRS corresponding to the selected target beam. In addition, the dedicated SRS can be used for further target TA fine tuning at the target cell. However, the dedicated SRS resources are also limited.

Considering the number of the candidate beams in a candidate cell and more than one UE maybe need to perform LTM at the same time, more resources are need for target beam indication to the target cell. Probably the better solution is to make use of both dedicated SRS and CG occasion resources to support candidate beam association for multiple UEs. For example, if 3 CG occasions and 4 dedicated SRSs are allowed, we have 12 combinations to be used for association with multiple candidate-beams for UEs in LTM.

Proposal 3: Dedicated SRSs associated to each candidate beams are preconfigured to the UE. Upon received cell switch command, the UE uses specific CG occasion and transmits the SRS corresponding to the selected target beam.

## Triggering of UE RACH-less TA measurement

In last meeting, several companies proposed to use low layer command to trigger the early RACH-less TA measurement, then the UE stores the early measured TA similar to PDCCH ordered early RACH. Although it is doable if there is use case, for cell switch, it is not needed to perform the target TA calculation before cell switch command is received. Before cell switch, the early DL synchronization with the candidate cells/beams should be achieved first at the L1 measurement phase. The UE is already tracking the candidate reference signal(s)’ timing and store the timing difference from its local reference. When the cell switch command is received, the UE simply uses the most recent stored target reference signal timing difference to derive the target cell TA. The target TA measured in this way is more accurate with less complexity comparing with network ordered early UE based TA measurement.

Proposal 4: UE based TA measurement is triggered by a cell switch command.

# Conclusions

Based on above discussions, we have the following observations and proposals:

Observation 1: DUs are generally not precisely synchronized. The timing offset between the source and target DUs can change over time due to their clock drifting.

Observation 2: The UE measured DL reference timing offset includes not only the propagation delay to the UE from the source DU versus the target DU, but also the timing offset between the two asynchronous DUs.

Observation 3: Timing offset between the asynchronous DUs should be removed to allow the UE based TA measurement to be used in inter-DU mobility scenario.

Observation 4: The bias of non-ideal synchronization and DL/UL asymmetry on the UE RSTD measurement can be indicated by the network to the UE and eliminated when the UE derives TA.

Observation 5: Network assistance for non-ideal bias compensation is essential to allow UE based TA measurement to be used in Rel-18 inter-DU mobility scenarios.

Observation 6: Normally, the source node and candidate node timing offset change due to clock drifting is much slower than mobility activities. Therefore, the compensation factor determined and configured to the UE at LTM preparation is valid at cell switching.

Proposal 1: Support network assisted non-ideal bias compensation for UE based TA measurement in non-precisely synchronized inter-DU mobility scenarios.

- The network non-ideal bias compensation factor is RRC pre-configured to the UE.

Proposal 2: The cell switch command includes a bit field to indicate whether the TA field contains the target TA or the dynamic change of the source TA, whether the source TA change is used for source TA update for UE based TA measurement, or for directly target TA = source TA.

Observation 7: The complexity and impact of UE based RACH-less scheme to standards are moderate.

Observation 8: Upon received cell switch command, the UE performs the initial UL transmission to the target cell following the target cell reference timing and target TA.

Proposal 3: Dedicated SRSs associated to each candidate beams are preconfigured to the UE. Upon received cell switch command, the UE uses specific CG occasion and transmits the SRS corresponding to the selected target beam.

Proposal 4: UE based TA measurement is triggered by a cell switch command.

# References

Chair's notes RAN1#113 v20

R2_122_ChairNotes EOM

R1-2300057 Discussion of the merits of UE based RACH-less TA acquisition for LTM, Futurewei