3GPP TSG RAN WG1 #114	R1-2306450

Toulouse, France, August 21st - August 25th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Sidelink Carrier Aggregation for NR

Agenda item:	9.4.4

Document for:	Discussion and Decision

# Introduction

In the work item description [1] the following justification is given as background for the objective to support NR SL CA:

Increased sidelink data rate is motivated by applications such as sensor information (video) sharing between vehicles with high degree of driving automation. Commercial use cases could require data rates in excess of what is possible in Rel-17. Increased data rate can be achieved with the support of sidelink carrier aggregation and sidelink over unlicensed spectrum.

The objectives of the SL CA work according to the WID [1] are the following:

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4]

- Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication)

- The work is limited to intra-band CA for the ITS band in FR1 (Band n47).

- No specific enhancements of Rel-17 sidelink features with sidelink CA support.

- This feature is backwards compatible in the following regards

  - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

- Only Mode 2 operation

- Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues

  - Time resources for PSFCH are aligned among the carriers for CA

- No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation)

  - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI)

  - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH

- No consideration for limited transmission and reception capability

- No primary/secondary carrier differentiation

- Reuse the LTE sidelink CA design for the following aspects:

  - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication

- The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18.

- Note: The SL CA work in Rel-18 mainly targets some V2X use cases

In this contribution we discuss the necessary changes to the current specifications to support CA in NR SL.

# Discussion

## Tx power split

According to the WID RAN1 should reuse LTE Tx power splitting mechanism for simultaneous sidelink transmissions. In LTE TS 36.213 the operation is specified in the following way:

In sidelink transmission mode 3 or 4, if a UE's sidelink transmission on a carrier overlaps in time with sidelink transmission on other carrier(s) and its total transmission power exceeds defined in [6], the UE shall adjust the transmission power of the sidelink transmission which has SCI whose "Priority" field is set to the largest value among all the “Priority” values of the overlapped sidelink transmissions such that its total transmission power does not exceed defined in [6]. In this case, calculation of the adjustment to the sidelink transmission power is not specified. If the transmission power still exceeds defined in [6] after this power adjustment, the UE shall drop the sidelink transmission with the largest “Priority” field in its SCI and repeat this procedure over the non-dropped carriers. It is not specified which sidelink transmission the UE adjusts when sidelink transmissions overlapping in time on two or more carriers have the same value for the “Priority” field.

The priority-based power allocation/splitting specified for LTE can be used for NR PSSCH/PSCCH transmissions.

In LTE, SLSS is transmitted on all the synchronization carriers or in one of the synchronization carriers depending on UE capability. For NR S-SSB transmissions, the same operation as in LTE should be supported so that S-SSBs are transmitted in all the synchronization carriers simultaneously and the Tx powers of the S-SSBs are the same.

For PSFCH the current operation of having equal power for all the transmitted PSFCHs in the BWP can be extended so that UE transmits all the PSFCHs in all the aggregated carriers with equal power. According to the current specification a UE may drop some of the PSFCH transmissions associated with lower priority PSSCH transmissions if the number of simultaneously supported PSFCH transmissions is exceeded or if the UE is power limited when DL pathloss based power control is used. The number of simultaneously transmitted PSFCHs can be large in CA but the Rel-16/17 method of splitting the TX power and dropping of the low priority PSFCH transmissions can be modified so that all the simultaneous PSFCH transmissions in all the aggregated carriers are considered in the PSFCH power control procedure.

Proposal 1: For NR PSSCH/PSCCH, when a UE is power limited reuse the power splitting/allocation mechanism specified for LTE.

Proposal 2: When a UE transmits S-SSB simultaneously in multiple carriers, the TX power is split equally to the carriers.

Proposal 3:  For PSFCH, the current Rel-16/17 power control procedure is modified so that the Tx power of each PSFCH is based on the number of simultaneous PSFCH transmissions on all the resource pools on all the aggregated carriers.

## Synchronization of aggregated carriers

In LTE when SLSS based synchronization is used, UE compares SLSS-ids in all the carriers and selects the carrier with the highest priority SLSS as the synchronization carrier. The selected SLSS is used to synchronize all the aggregated carriers. If only lowest priority SLSS is found in the synchronization carriers, it is up to the UE to select the synchronization reference. For NR, the same procedure can be used. If only lowest priority SLSS-ids are found, using S-RSRP as an additional criteria should be considered. A CA UE may be able to find that synchronization in the aggregated carriers is not aligned and selecting the highest RSRP and using it for S-SSB transmissions could speed up the process of finding common synchronization for all the nearby UEs.

Regarding S-SSB transmission, depending on the threshold defined by parameter syncTxThreshOoC UE may skip the S-SSB transmission, if it concludes that S-SSB power level is already high enough. In LTE CA, the determination of the syncTxThreshOoC threshold is done on the selected synchronization carrier and the UE then transmits or skips SLSS in all the aggregated carriers. The consequence of this operation is that the UE may skip SLSS transmissions in some of the synchronization carriers even if there are no SLSS in that carrier. For NR the operation could be improved so that checking of syncTxThreshOoC threshold is done separately for all the carriers where S-SSB is intended to be transmitted.

Proposal 4: If a UE detects two or more synchronization carriers with the same synchronization source priority, then SLSS RSRP criteria is used for the selection of the synchronization carrier.

Proposal 5: The checking of syncTxThreshOoC threshold to determine if NR S-SSB is transmitted, is done separately to all the synchronization carriers.

# Conclusion

In this contribution we have discussed the changes needed to support NR sidelink carrier aggregation. We have the following proposals:

Proposal 1: For NR PSSCH/PSCCH, when a UE is power limited reuse the power splitting/allocation mechanism specified for LTE.

Proposal 2: When a UE transmits S-SSB simultaneously in multiple carriers, the TX power is split equally to the carriers.

Proposal 3: For PSFCH, the current Rel-16/17 power control procedure is modified so that the Tx power of each PSFCH is based on the number of simultaneous PSFCH transmissions on all the resource pools on all the aggregated carriers.

Proposal 4: If a UE detects two or more synchronization carriers with the same synchronization source priority, then SLSS RSRP criteria is used for the selection of the synchronization carrier.

Proposal 5: The checking of syncTxThreshOoC threshold to determine if NR S-SSB is transmitted, is done separately to all the synchronization carriers.

# References

- RP-230077, WID revision: NR sidelink evolution, OPPO