3GPP TSG-RAN WG1 Meeting #114	R1-2306519

Toulouse, France, August 21st-25th, 2023

Agenda Item:	9.10.2

Source:	Huawei, HiSilicon

Title:	Timing advance management to reduce latency

Document for:	Discussion and Decision

Introduction

In this paper, we share our views on the timing advance (TA) management in LTM. The rest of this paper are structured as follows. Potential TA acquisition mechanisms before cell switch command are discussed in section 2. The coexistence of various early TA acquisition methods were discussed in section 3. Conclusions are made in section 4.

TA acquisition mechanisms

Remaining issues on RACH-based solution

- Power control for PRACH

The power control of PRACH was discussed during RAN1#113 and following agreement was made:

Agreement On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS - UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved - whether/how to reset the counter

There are still some FFS bullets on the details of power control of PDCCH-ordered CFRA. In a RACH procedure for serving cell, the power ramping step is provided to UE by powerRampingStep in RACH-ConfigGeneric. The UE determines the power ramping value according to the power ramping counter and the power ramping step as (PREAMBLE_POWER_RAMPING_COUNTER–1) × PREAMBLE_POWER_RAMPING_STEP. Similar mechanism can be adopted for PRACH transmission in candidate cells. When “PRACH retransmission indicator” in DCI 1_0 is set to “0” (i.e. initial transmission), the power ramping counter is reset. And if the “PRACH retransmission indicator” in DCI 1_0 is set to “1” (i.e. retransmission), the power ramping counter will be incremented by 1. This has already been agreed in RAN2 and captured in the CR of 38.321 [2] as copied below:

### 5.1.1 Random Access procedure initialization <omit> 1> set the PREAMBLE_POWER_RAMPING_COUNTER to 1, if the Random Access procedure is not initiated by the PDCCH order for an LTM candidate cell as preamble re-transmission; ### 5.1.3 Random Access Preamble transmission <omit> 1> if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell as preamble re-transmission: 2> increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

Proposal 1: For the power control of PDCCH-ordered CFRA in LTM, UE determines the power ramping value according to power ramping counter and power ramping step defined in TS38.321. The power ramping counter is increased by 1 if “PRACH retransmission indicator” in DCI 1_0 is set to “1”, The power ramping counter is reset When “PRACH retransmission indicator” in DCI 1_0 is set to “0”.

- PRACH transmission overlapped with other transmission

For PDCCH-order based PRACH for candidate cell, the issues on power reduction and prioritization rule were discussed in RAN1#113 with the following agreement for UE not support simultaneous/parallel transmission of PRACH in candidate cell and UL channels and signals in serving cell.

Agreement - For PDCCH-order based PRACH for candidate cell, If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a candidate cell other than current serving cell(including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells - PRACH transmission - PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI - SRS transmission - Any other PUCCH/PUSCH transmission - Down-select the UE behavior in this case - Alt 1: Dropping rule is needed - Alt 2: up to UE implementation

In LTM, the PRACH is triggered by serving cell and NW should be aware of candidate cell’s PRACH resource location in time domain. Serving cell DU can avoid scheduling UL transmission in serving cell on the time resources overlapped with the PRACH resources for candidate cells by implementation. Besides the dynamic scheduling, for the preconfigured UL transmission in the serving cell such as P-SRS and configured grant PUSCH, similar rule as that in Rel-16 DAPS operation in a same frequency band can be adopted. In DAPS, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot overlapping in time with a PRACH transmission to the target MCG. Thus, for UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL in serving cell, UE does not transmit PUSCH/PUCCH/SRS to the serving cell in a slot overlapping in time or overlapping in a time gap below a certain threshold with a PRACH transmission to the candidate cell. It is possible that the PRACH transmission to a candidate cell other than current serving cell overlapped to PRACH transmission to serving cell. Usually, the PRACH transmission in the serving cell is due to some critical events, such as UL out-of-synchronization. So in this case, we suggest to prioritize the PRACH to serving cell and drop the PRACH to candidate.

Proposal 2: If UE capability does not support simultaneous/parallel transmissions PRACH in candidate cell and UL channels and signals in serving cell., the following dropping rule should be adopted:

- PRACH in serving cell > PRACH in candidate cell > PUSCH/PUCCH/SRS in serving cell

As for the case of simultaneous/parallel transmission in source cell and target cell, RAN4 inclined to not support in Rel-18 LTM according to the following agreement in RAN4#106 [3]:

< Agreement>: Discuss what interruption requirements, scheduling restriction and measurement restriction to define instead of discussing whether simultaneous Rx/Tx in source cell and target cell is allowed.

The power reduction rule can be low prioritized in Rel-18 accordingly.

Proposal 3: The discussion on the power reduction rule when UE transmits simultaneously PRACH in candidate cell and UL channels and signals in serving cell can be deprioritized in Rel-18 LTM.

UE based TA acquisition solution

In RAN1#113 meeting, the working assumption on the UE based TA measurement solution was confirmed:

Agreement Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

For UE-based TA measurement, the reference signal timing difference (RSTD) may be derived by the UE based on the measurement on SSBs of serving cell and candidate cell. UE may determine the TA of candidate cell from the serving cell TA and RSTD, and apply to the UL transmission after cell switch directly. There are three sources contributing to TA estimation error for the candidate cell. Firstly, the UE may have DL synchronization errors for both serving cell and candidate cells. In addition, the RSTD defined in positioning is based on PRS which have larger bandwidth to provide better estimation accuracy compare to SSB. Secondly, the estimation of DL Tx timing between serving cell and candidate cells may be difficult. It can be as long as 3 us according to RAN4 spec even for synchronous deployment. It is usually difficult to measure such error by gNB itself. Thirdly, the TA for the candidate cell is derived from the serving cell TA, which has estimation error itself. Thus, further evaluation is required to see whether the accuracy of TA estimation from RSTD can meet the RAN4 requirement on UL synchronization.

Observation 1: Regarding the UE-based TA measurement mechanism, the accuracy of TA estimation by UE should be evaluated/studied considering the UE derived TA will be used for the potential UL data transmission in the new serving cell after cell switch.

It is risky to allow UE directly use the TA derived by itself to transmit UL data. If the aggregated error of the UE derived TA exceed the CP, it may cause UL performance degradation due to ICI/ISI at target cell receiver. To refine the TA acquired by UE before cell switch, an AP-SRS can be transmitted to the target cell after cell switch. The TA derived by UE before cell switch can be used to transmit the SRS considering its robustness to UL asynchronization. Target cell can further update the TA based on the measurement of SRS.

Proposal 4: The TA derived by UE itself before cell switch can be used to transmit AP-SRS after cell switch to further refine the UL synchronization for PUSCH/PUCCH in the target cell.

Besides, the TA of candidate is derived from the RSTD between source cell and candidate cell. The measurement timing and RS resource for RSTD should be configured in advance, such as SMTC and SSB index of candidate cell. If there is DL timing difference between gNBs, it should also be provided to UE.

Proposal 5: SMTC and SSB index of candidate cell should be configured for UE to measure RSTD between candidate cell and source cell. The DL time difference should be provided in asynchronous deployment.

The coexistence of TA acquisition mechanisms

In RAN2#119bis, following agreements on RACH-based cell switch procedures were made:

RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS).

The RACH-based (CFRA, CBRA) procedure after cell switch was also supported in Rel-18 LTM in RAN2, which has not been discussed in RAN1. According to RAN2’s discussion, RACH resource for CFRA for LTM may be provided in RRC configuration or potentially by MAC CE FFS. It is possible that early UL synchronization is performed before CSC, while the RACH resource for target cell after cell switch command is also provided. Moreover, there are several methods to acquire the TA value for the candidate cell before cell switch command. The agreed solutions include PDCCH-ordered RACH before CSC, UE based TA acquisition, LTE RACH-less-like solution (TA=0 or TA is the same as the serving cell indicated by CSC). How UE determines the actual TA value after cell switch when more than one TA acquisition solution are configured should be clarified.

Observation 2: The priority of TAs acquired by different mechanisms should be determined when UE is configured with more than one mechanism.

In LTM, NW may indicate TA in CSC. The TA can be derived by NW from the early PRACH transmitted by UE, or equals to 0 or TA of serving cell based on the NW configuration. In such cases, we believe the TA indication from CSC should have the higher priority than those acquired by UE itself, if configured. UE can also skip the RACH procedure on the pre-configured RACH resource in target cell after cell switch command. If CSC does not have TA indication, and if UE-based TA measurement is performed, UE may apply its derived TA for target cell at least for SRS signals for further TA refinement. The UE may skip the RACH in target cell on the preconfigured resource. If none of the early UL synchronization is performed before CSC, UE shall initialize a random access procedure according to the RRC/MAC CE.

Proposal 6: When more than one TA acquisition solution for target cell are configured for a UE, the following priority should be adopted

- TA indicated in CSC > TA acquired by UE itself > TA acquired by RACH after CSC on the pre-configured resource.

Summary and conclusion

In this contribution, we observe and propose the following:

Observation 1: Regarding the UE-based TA measurement mechanism, the accuracy of TA estimation by UE should be evaluated/studied considering the UE derived TA will be used for the potential UL data transmission in the new serving cell after cell switch.

Observation 2: The priority of TAs acquired by different mechanisms should be determined when UE is configured with more than one mechanism.

Proposal 1: For the power control of PDCCH-ordered CFRA in LTM, UE determines the power ramping value according to power ramping counter and power ramping step defined in TS38.321. The power ramping counter is increased by 1 if “PRACH retransmission indicator” in DCI 1_0 is set to “1”, The power ramping counter is reset When “PRACH retransmission indicator” in DCI 1_0 is set to “0”.

Proposal 2: If UE capability does not support simultaneous/parallel transmissions PRACH in candidate cell and UL channels and signals in serving cell., the following dropping rule should be adopted:

- PRACH in serving cell > PRACH in candidate cell > PUSCH/PUCCH/SRS in serving cell

Proposal 3: The discussion on the power reduction rule when UE transmits simultaneously PRACH in candidate cell and UL channels and signals in serving cell can be deprioritized in Rel-18 LTM.

Proposal 4: The TA derived by UE itself before cell switch can be used to transmit AP-SRS after cell switch to further refine the UL synchronization for PUSCH/PUCCH in the target cell.

Proposal 5: SMTC and SSB index of candidate cell should be configured for UE to measure RSTD between candidate cell and source cell. The DL time difference should be provided in asynchronous deployment.

Proposal 6: When more than one TA acquisition solution for target cell are configured for a UE, the following priority should be adopted

- TA indicated in CSC > TA acquired by UE itself > TA acquired by RACH after CSC on the pre-configured resource.

References

[1] R1-2306317, Introduction of Rel-18 Further NR mobility enhancements, Huawei, RAN1#113, Incheon, Korea, May 22-26, 2023

[2] R2-2306924, 38.321 running CR for introduction of NR further mobility enhancements, Huawei, HiSilicon, RAN2#122, Incheon, Korea, May 22-26, 2023

[3] R4-2303175, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek, RAN4#106, Athens, Greece, 27 February –03 March, 2023