3GPP TSG RAN WG1 #114	 	R1-2306730

Toulouse, France, August 21st – August 25th, 2023

Source:	vivo

Title:	Discussion on DCI field interpretation for multi-PXSCH scheduling during BWP switching in FR2-2

Agenda Item:	7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, for the issue of multi-PUSCH scheduling with CBG-based PUSCH transmission during BWP switching, two interpretations raised during RAN1#112bis-e meeting are discussed, along with related proposal.

- Discussion

During RAN1#112bis-e meeting, an issue for multi-PUSCH scheduling with CBG-based PUSCH transmission during BWP switching was discussed extensively, referring to Issue#5 in [1]. The issue happens when

- CBG-based PUSCH transmission is configured for a serving cell, and

- Multi-PUSCH scheduling is configured for a first BWP in the serving cell, and

- A scheduling DCI indicates BWP switching from the first BWP (as the active BWP) to a second BWP (as indicated BWP), and

- The scheduling DCI indicates TDRA row index X which corresponds to more than one SLIV for the first BWP while corresponds to only one SLIV for the second BWP.

During the discussion, regarding how to understand “the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1” of a DCI format 0_1 scheduling PUSCH transmission(s) when determining if the fields CBG transmission information (CBGTI) and UL-SCH indicator are present or not in the DCI format 0_1, two interpretations were proposed as below.

- Interpretation 1: the scheduled PUSCH(s) implies PUSCH(s) in the active BWP, or, the number of scheduled PUSCHs is determined by TDRA row with index X in TDRA table configured for the active BWP.

- Interpretation 2: the scheduled PUSCH(s) implies PUSCH(s) in the indicated BWP, or, the number of scheduled PUSCHs is determined by TDRA row with index X in TDRA table configured for the indicated BWP.

Whether Interpretation 1 or Interpretation 2 is adopted by the specification was not agreed in the end of RAN1#112bis-e meeting due to limited time. In contrast, a conclusion was achieved in RAN1#112bis-e meeting as below, meaning that the issue should be avoided by gNB implementation, irrespective of whether Interpretation 1 or Interpretation 2 is adopted by the specification.

Conclusion (Issue#5)

If a UE is configured with CBG transmission for a serving cell, the UE expects that CBGTI field in the DCI (that can schedule multiple PUSCHs in the active BWP) is always present, when BWP switching is indicated for the serving cell and the number of scheduled PUSCH indicated by TDRA field for the indicated bandwidth part is equal to 1.

In the following section, we will provide our views on the above two interpretations.

In Rel-15 NR, it was agreed that all DCI bitfields in DCI formats 0_1 and 1_1 detected in USS, as well as DCI sizes of these DCI formats, are determined by current BWP, or the active BWP in the case of BWP switching.

In Rel-16/17, if multi-PUSCH scheduling is configured in the active BWP, there will be always two hypothesises on the arrangement or list of DCI fields, i.e. one for the case of scheduling single PUSCH and the other for the case of scheduling more than one PUSCH. Based on the above principle of Rel-15 NR, for any hypothesis, size of each DCI field is determined based on the active BWP. Besides, for each hypothesis a corresponding DCI size can be determined by the sum of all DCI filed sizes for this hypothesis. Then, the final DCI size is determined by the larger value between the two DCI sizes.  The focus is which hypothesis should be selected, or how to select from the two hypothesises.

Based on our understanding, the scheduled PUSCH(s) naturally implies PUSCH(s) in the indicated BWP, since the scheduled PUSCH(s) will be transmitted actually in the indicated BWP during BWP switching. Therefore, the hypothesis should be selected based on the number of scheduled PUSCHs in the indicated BWP, i.e. Interpretation 2.

On the contrary, if single-PUSCH scheduling is configured in the active BWP, there is only one hypothesis on the arrangement or list of DCI fields, while size of each DCI field is determined based on the active BWP as well. During BWP switching, the single hypothesis in the active BWP is always adopted to interpret a DCI format 0_1 scheduling PUSCH transmission(s).

Proposal 1: For the issue of multi-PUSCH scheduling with CBG-based PUSCH transmission during BWP switching, adopt Interpretation 2, i.e., the scheduled PUSCH(s) implies PUSCH(s) in the indicated BWP, or, whether the number of scheduled PUSCHs is larger than 1 or not is determined by TDRA row with indicated index in TDRA table configured for the indicated BWP.

- Conclusion

In this contribution, for the issue of multi-PUSCH scheduling with CBG-based PUSCH transmission during BWP switching, two interpretations raised during RAN1#112bis-e meeting are discussed, and the following proposal is made.

Proposal 1: For the issue of multi-PUSCH scheduling with CBG-based PUSCH transmission during BWP switching, adopt Interpretation 2, i.e., the scheduled PUSCH(s) implies PUSCH(s) in the indicated BWP, or, whether the number of scheduled PUSCHs is larger than 1 or not is determined by TDRA row with indicated index in TDRA table configured for the indicated BWP.

References

- R1-2304037, Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ), RAN1#112bis-e, 2023.