3GPP TSG RAN WG1 #114	R1-2306769

Toulouse, France, 21 - 25 August, 2023

Source:	vivo

Title:	Evaluation results for R18 LP-WUS/WUR

Agenda Item:	9.11.1

Document for:	Discussion and Decision

- Introduction

In RAN1#113 meeting, evaluation methodologies and evaluation results were discussed, and some agreements were made [1]. In this contribution, we discuss the remaining issues for evaluation methodologies and evaluation results. Design targets are firstly discussed. Then we give our views on the power evaluation for both RRC idle/inactive and connected modes, and the system performance of LP-WUS is provided in section 5 subsequently. At last, LLS and coverage evaluation methodology and results for LP-WUS are discussed.

- Design targets

- Relative power of LP-WUR ON state

The overall target for the power consumption of LP-WUR is to achieve longer battery life of device. For legacy UEs in RRC idle/inactive mode, the average UE power consumption is typically several or tens of mW. To achieve substantial power saving gain, a reasonable target power consumption of LP-WUR ON state needs to be reduced by 100 to 1000 times, corresponding to tens or hundreds of μW. Based on the evaluation results provided in section 3 and 4, to achieve substantial power saving gain (e.g., 80%), less than 1unit relative power of LP-WUR ON state is desirable.

For a general usage, LP-WUS/WUR design should address all traffic characteristics for IoT/Wearable/eMBB. Specifically, for IoT devices with relatively sparse traffic arrival, battery life is critical. Thus, keeping the LP-WUR ON state power consumption as low as possible is important as the results shown in the section 3.

Observation 1: In the case that the relative power of LP-WUR ON state is 0.03~0.5unit, substantial power saving gain (i.e. 80% or higher) can be achieved.

Observation 2: LP-WUS/WUR design should address all traffic characteristics for IoT/Wearable/eMBB, thus keeping the LP-WUR ON state power as low as possible is important for increasing the battery life of the device.

Proposal 1: The target relative power of LP-WUR ON state should be less than 1 unit.

- Latency

For RRC idle/inactive mode, since there are variety of use cases with different latency requirements, it will be difficult to set certain latency targets. For IoT devices, latency in the order of seconds is generally required. Specially, for latency-sensitive IoT devices, target latency may need to be hundreds of milliseconds if necessary. In addition, latency within several seconds is expected for wearable devices.

For RRC connected mode, considering the stringent latency requirement of XR service, the latency by using LP-WUS/WUR should be no more than several milliseconds. For example, continuous LP-WUS monitoring is expected and MR needs to enter micro or light sleep state during LP-WUS monitoring, so as to the latency can be within 3ms (i.e., half of light sleep transition time). Apart from XR use case, the target latency of other eMBB use cases in RRC connected mode can be tens of milliseconds so that discontinuous LP-WUR monitoring and/or deep sleep state of MR can also be considered.

Proposal 2: The latency target for LP-WUS/WUR:

- Latency for IoT and wearable use cases are in the order of second;

- Latency for XR use case is in the order of millisecond;

- Latency for eMBB use case is in the order of tens of milliseconds.

- Data rate

For idle UEs, the maximum data rate for paging can be calculated as R = 48 bits (UE ID length) * 32 (UEs per PO) * 4(POs per radio frame)/10ms = 614kbps. While for LP-WUS, highest data rate may not necessary, e.g., full UE ID delivery may be not required. Hence, around 100kbps data rate design target can be considered.

Proposal 3: Around 100kbps data rate can be considered as design target for LP-WUS.

- Power evaluation for RRC idle/inactive mode

Our power evaluation assumptions for RRC idle/inactive mode are provided as below.

- Baselines: legacy PO monitoring with I-DRX with PEI, eDRX with PEI

- MR sleep state during LP-WUS monitoring: deep sleep or ultra-deep sleep

- LP-WUR (ON power, OFF power) combinations:

  - LP-WUR ON power = 0.01/0.1/1unit, LP-WUR OFF power = 0.001unit

  - LP-WUR ON power = 10/20units, with different LP-WUR OFF power (i.e., 0.001/0.02/0.1unit)

- MR ramp-up time, transition energy = 400ms, 15000units

- MR sync/re-sync time, energy = 100ms, 3340units

  - Note 1: 10ms average continuous detection for SSB at the beginning of sync/re-sync time is assumed

  - Note 2: LP-SS reception are not assumed to assist MR sync/re-sync after wake- up from ultra-deep sleep

- RRM:

- Option 1: RRM measurement is only performed by MR as in I-DRX baseline.

- Option 2: MR performs relaxed RRM measurement every X I-DRX cycles (e.g. X can be 16 or larger). Additionally, LP-WUR perform assistant RRM measurement based on periodic lower power signal e.g., LP-SS. And LP-SS is assumed to be measured during the LP-WUS monitoring window.

- Option 3: MR does not perform RRM measurements and RRM measurement is completely offloaded to LP-WUR.

- The false-alarm rate (FAR) of LP-WUS: [0, 0.1%, 1%]

Note: Alt-2 of FAR target is adopted, i.e. FAR is determined across a reference time duration (e.g., 1.28s) of one or multiple WUS attempts/trials.

- Per UE paging rate: RE=RE, REF= [1%, 0.1%, 0.01% or 0.001%], Y=YREF = 1.28s

- Per group paging rate: RG=1 – (1 – RE)N , N is the number of UEs in the group: [10, 20].

- LP-WUS monitoring:

  - continuously monitoring

  - duty-cycled monitoring

- [200ms, 1280ms] as the cycle periodicity, and within each cycle the ON ratio is [1%, 10%].

- LP-WUR ON-OFF transition time

  - 10ms for LP-WUR ON power>=10unit

  - 1ms for LP-WUR ON power<=1unit.

For the transition from LP-WUR OFF to ON state, since DDR memory and control processor will take most of the transition time for LP-WUR from ON to OFF, if the two components in OFDM based LP-WUR can be drastically simplified, the transition latency from OFDM based LP-WUR ON to OFF may be in the order of milliseconds e.g., 10ms. Given the low circuit complexity and very small difference between power consumption of WUR ON and OFF states for OOK based LP-WUR, 1ms is assumed when WUR ON power is <=1unit.

Based on the agreed evaluation methodologies in previous meetings and the reported assumptions above, the evaluation results and observations are summarized in the following.

- Power consumption and latency comparison for legacy I-DRX, legacy e-DRX and LP-WUS

Figure 1 shows the comparison for I-DRX, e-DRX and LP-WUR/WUS under the configurations in Table 1. Besides, LP-WUS with dynamic PO refers to there is always a PO for the UE, which is located right after the time interval (including ramp-up time and sync time). So, the latency of dynamic PO case is equal to the time interval. And LP-WUS with legacy PO means that UE can only monitor its associated legacy PO after woken up by LP-WUS.

Table 1. Configurations for evaluation with different baselines.

| FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | UE type | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- |
| 0.1% | continuous | 0.001% | eMBB | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |

Figure 1. Power consumption and latency comparison among I-DRX, e-DRX and LP-WUR/WUS

According to the results, the following observations are given:

Observation 3: Compared to I-DRX with PEI, LP-WUR/WUS with continuous monitoring configuration can achieve 53%~98% power saving gain when the relative power of LP-WUR ON state is no more than 1 unit, and ~150ms latency reduction for dynamic PO case or ~500ms latency increase for legacy PO case.

Observation 4: Compared to eDRX with PEI, LP-WUR/WUS with both legacy PO and dynamic PO can significantly reduce the paging latency (i.e., can reduce ~26s), with comparable UE power consumption.

Observation 5: LP-WUR/WUS provides a much better trade-off between latency and power consumption when relative power of LP-WUR ON state is no more than 1unit, compared with legacy I-DRX and eDRX.

Observation 6:  Continuous LP-WUS monitoring cannot provide power saving gain, when the relative power of LP-WUR ON state is higher than 1unit.

- Impact due to different LP-WUR state power

Figure 2 shows the simulation results with different combinations of LP-WUR ON and OFF power under configurations in Table 2.

Table 2. Configurations for the evaluation of different LP-WUR state power.

| Baseline: | duty cycle | duty cycle ratio | FAR of LP-WUS | UE type | Per UE paging rate | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I-DRX with PEI | 1280ms | 1% | 0.1% | eMBB | 0.001% | Ultra-deep sleep, Deep sleep | Option 1 for baseline Option 3 for LP-WUS |

Figure 2.  The performance of LP-WUS/WUR with different combinations of LP-WUR ON and OFF.

Figure 3. The performance of LP-WUS/WUR with different MR sleep state

According to the results, the following observations are given:

Observation 7: Duty cycled LP-WUS monitoring can achieve more than 98% power saving benefit compared to legacy I-DRX with PEI when the relative power of LP-WUR ON is no more than 1unit.

Observation 8: Up to 44% power saving gain will be lost if UE main radio goes into deep sleep rather than ultra-deep sleep during LP-WUS monitoring in RRC idle/inactive.

Observation 9: Compared to LP-WUR ON power >=10unit, 6.4%~15.6% additional power saving gain can be achieved when the relative power of LP-WUR ON is no more than 1unit.

- Impact due to different LP-WUS monitoring duty cycle periodicity and ratio

Figure 4 shows the simulation results for different LP-WUS monitoring duty cycle periodicity and ratio under configurations in Table 3.

- [200ms, 1280ms] as the duty cycle periodicity, and for each cycle the duty cycle ratio is [1%, 10%].

Table 3. Configurations for evaluation with different LP-WUS monitoring configurations.

| Baseline: | FAR of LP-WUS | LP-WUR ON state power | Per UE paging rate | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- |
| I-DRX with PEI | 0.1% | 0.1/1/10/20 units | 0.001% | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |

Figure 4. The power and latency performances of LP-WUS/WUR scheme with different DRX cycle lengths and ratio.

According to the results, the following observations are given:

Observation 10: With fixed duty cycle ratio and LP-WUR “ON” state power, when LP-WUR “ON” state power is no more than 1unit, different LP-WUR duty cycle length have no or marginal impact on UE power consumption.

Observation 11: When the duty cycle ratio raises from 1% to 10%, the achievable UE power saving gain will be largely degraded, e.g., >=40% for 10 or 20 units LP-WUR “ON” power case.

- Impacts due to different RRM schemes

Figure 5 shows the simulation results with different RRM schemes under configurations in Table 4, where the compared three RRM schemes are the same as mentioned before, i.e.

- Option 1 (MR): RRM measurement is only performed by MR as in legacy releases.

- Option 2 (MR+LR): MR performs relaxed RRM measurement every X I-DRX cycles (e.g. X can be 16 or larger). Additionally, LP-WUR may also perform assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

- Option 3 (LR): MR does not perform RRM measurements and RRM measurement is completely offloaded to LP-WUR.

Table 4. Configurations for evaluation with different RRM assumptions.

| Baseline | duty cycle | duty cycle ratio | FAR of LP-WUS | MR sleep state | Per UE paging rate |
| --- | --- | --- | --- | --- | --- |
| I-DRX with PEI | 1280ms | 1% | 0.1% | Ultra-deep sleep | 0.001% |

Figure 5.  The power and latency performances of LP-WUS/WUR scheme with different RRM schemes.

According to the results, the following observations are given:

Observation 12: MR RRM measurement relaxation is critical to achieve power saving gain by LP-WUR/WUS, where the MR RRM measurement relaxation includes MR RRM measurement period by X times, and RRM offloading from MR to LR.

Observation 13: Even with 16x or 32x MR measurement relaxation, less power saving gain e.g., <53% is obtained by duty cycled LP-WUS monitoring (with 1% duty cycle ratio) scheme, when the LP-WUR “ON” power is 10 or 20 units.

Observation 14: Higher UE power saving benefit can be achieved if RRM measurement can be completely offloaded from MR to LR.

Proposal 4: It is recommended to support at least one of the following RRM measurement enhancements to achieve substantial UE power saving benefit by LP-WUR/WUS

- Relax the MR RRM measurement period by X times, where the X should be sufficiently large number, e.g. 16 or larger

- Offload the RRM measurements from MR to LR

- UE type: eMBB or RedCap

Figure 6 shows the simulation results showing power saving benefit for different UE type under configurations in Table 5. It should be noted that 1Rx is assumed for RedCap UEs.

Table 5. Configurations for evaluation with different UE type.

| Baseline: | duty cycle | duty cycle ratio | Per UE paging rate | FAR | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- |
| I-DRX with PEI | 1280ms | 1% | 0.001% | 0.1% | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |

Figure 6. The power saving benefit of LP-WUR/WUS for different UE types (eMBB and RedCap)

Observation 15: RedCap UEs are more sensitive to the LP-WUR ON state power, i.e. compared to eMBB UEs, the power saving gain for RedCap UEs decreases much faster as the LP-WUR ON state power increases.

- Power evaluation for RRC connected mode

  - Potential applying of LP-WUS for XR use case

XR is one of the target use cases for applying LP-WUS. Per the discussion in R17/18 XR, one of the identified characteristics of XR traffic is the unpredictable jitter and its wide range e.g., [-8, +8]ms. LP-WUS/WUR can further reduce the unnecessary PDCCH monitoring due to jitter while guarantee the XR performance (i.e. latency, capacity).

- Existing power saving techniques: UE still has to perform continuous or sparse PDCCH monitoring in the jitter range until the traffic arrival as depicted in Figure 7 (a).

- LP-WUS: LP-WUS triggers the resume of MR PDCCH monitoring from micro/light sleep. Since XR traffic will arrive within the jitter range, continuous LP-WUS monitoring occasions are configured within CDRX onduration which fully align with XR traffic periodicity and cover the whole jitter range. A UE starts LP-WUS monitoring at the beginning of DRX onduration and resumes PDCCH monitoring once LP-WUS is detected in Figure 7 (b).

Observation 16: LP-WUS/WUR can be used to reduce the unnecessary PDCCH monitoring due to jitter while guarantee the XR performance (i.e. latency, capacity) for RRC connected mode.

Figure 7.  Example of jitter handling by using LP-WUS/WUR

  - Power evaluation for LP-WUS in XR use case

Based on the R17 and R18 XR power evaluation methodologies and assumptions, we further evaluate power consumption and system capacity of different cases. As discussed in section 4.1, UE main radio can enter micro or light sleep during LP-WUS monitoring of LR. And the detailed configurations can be founded in Appendix B. Note that for capacity results of LP-WUS/WUR scheme (combined with main receiver light sleep during LP-WUS monitoring of LR) corresponding to the UE satisfaction metric with both 95% and 99% packet successful rate are shown. Evaluations for both low load and high loads are given, corresponding to 5 and 10 UEs per cell, respectively. The detail evaluation methodologies and assumptions can be found in TR 38.838, respectively.

- InH, FR1, DL only XR traffic, 100MHz bandwidth, DDDSU TDD format

Figure 8 and Figure 9 below shows the evaluation results for DL only XR traffic, i.e. 30Mbps and 60FPS. And LP-WUR ON power is assumed 0.1units with always-On monitoring.

Figure 8.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

Figure 9.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

According to the results shown in Figure 8 and Figure 9, compared to always-On scheme (no DRX), R17 PDCCH monitoring adaptation schemes can obtain 11~23% power saving gain, while LP-WUS monitoring with MR enters micro sleep can achieve about 24%~29% power saving gain, showing up to 15% relative power saving gain without capacity performance loss. Besides, LP-WUS with MR enter light sleep will achieve more relative power saving gain i.e., up to 25% compared to the existing power saving schemes, but degrade the satisfied UE rate meanwhile. In addition, different load cases have the similar power saving and capacity observations.

Observation 17: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring with MR micro sleep can bring {7%~15%} relative UE power saving gain with no capacity loss in both low load and high load cases for DL only XR traffic.

Observation 18: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring with MR light sleep can bring {14%~25%} relative UE power saving gain, with acceptable capacity impact at least in low load case for DL only XR traffic.

- InH, FR1, DL (one flow or two flows) and UL (UL pose flow) XR traffics, 100MHz bandwidth, DDDSU TDD format

Figure 10 below shows the evaluation results for XR DL (one or two flows) and UL traffics. The detailed traffic model can be seen in Appendix B. Note that WUR ON power is assumed 0.1unit with continuously monitoring.

Figure 10 Evaluation results for mixed DL (video, audio) and UL (pose) traffics in XR

According to the results shown in Figure 10, due to another one or two XR traffic flows are considered, less relative power saving gain and larger degradation of capacity performance is observed by LP-WUS for DL+UL traffic case compared to that for DL only traffic case.

Observation 19: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring with main receiver micro sleep can bring {8%~12%} additional UE power saving gain with no capacity loss in both low load and high load cases for DL+UL XR traffics.

Observation 20: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring with main receiver light sleep can bring {16%~19%} additional UE power saving gain, with acceptable capacity impact at least in low load case for DL+UL XR traffics.

  - Potential applying of LP-WUS for eMBB use case

LP-WUR/WUS can be also be applied for eMBB use case. In Figure 11, compared to legacy CDRX+DCP scheme, there can be two potential usages of LP-WUS.

- Existing power saving techniques:

- Always-On (baseline): Without adopting any power saving mechanism.

- C-DRX:

| DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) | Note |
| --- | --- | --- | --- |
| 160 | 8 | 100 | Refer to TR 38.840 |
| 40 | 4 | 10 |  |

- C-DRX +DCP (i.e., DCI format 2_6): the time offset between DCP and DRX onduration is 3ms, and the DCP monitoring duration is 1ms as shown in Figure 11 (a).

- R17 PDCCH monitoring adaptation + C-DRX +DCP: PDCCH skipping duration is 100ms, SSSG with PDCCH monitoring every slot.

- LP-WUS scheme:

- LP-WUS replaces DCP to indicate the start of drx-ondurationtimer: UE can monitor LP-WUS instead of DCI2_6 in DCP monitoring window as shown in Figure 11 (b).

- LP-WUS dynamically trigger the resume of PDCCH monitoring: During PDCCH skipping duration, the UE performs continuous LP-WUS monitoring. Once detecting LP-WUS, UE will resume of MR PDCCH monitoring from micro/light/deep sleep with 0/3/10ms wake-up delay. The assumptions on the relative power of LP-WUR “ON” state is 0.01/1/10 units as shown in Figure 11 (c).

For LP-WUS replaces DCP case, UE can save power by LP-WUS instead of DCP monitoring. Besides, the MR of UE can continue sleeping without being woken up if no LP-WUS is separately detected by WUR. However, the scheduling delay by LP-WUS replace DCP still cannot be reduced as shown in Figure 11 (b) and there will be no UPT gain compared to legacy CDRX scheme (with or without DCP). For LP-WUS trigger PDCCH monitoring case, things will be different. It can be observed from the evaluation results shown in later section that when MR is configured to enter micro sleep during LP-WUS monitoring, since UE can be immediately woken up by LP-WUS, the same UPT performance can be achieved as alwayson. Meanwhile, compare to the legacy power saving schemes, some power saving gain will also be lost. Additionally, when assuming UE MR enters deep or light sleep during LP-WUS monitoring, although the ramp up delay will increase to 3 or 10ms, both power saving gain and UPT gain will be obtained compared to legacy power saving schemes. It will be much flexible for the network to balance power saving and UPT performances via configuring different sleep states of MR during LP-WUS monitoring.

Figure 11.  Potential working procedure for LP-WUS/WUR for FTP3 use case

Observation 21: LP-WUS/WUR can be used to improve UE power saving and/or UPT performances for eMBB traffic in RRC connected mode.

Observation 22: There are two potential usages of LP-WUS for RRC connected mode as follows:

- LP-WUS replaces DCP to indicate the start of drx-ondurationtimer;

- LP-WUS dynamically triggers the resume of PDCCH monitoring.

  - Power evaluation for LP-WUS in eMBB use case

Based on FTP model 3 traffic model (200ms mean arrival interval) and CDRX configuration, we further evaluate UE perceived throughput (UPT) and power consumption with the following schemes. The detail evaluation methodologies and assumptions can be found in TR38.840, respectively.

- InH, FR1, DL only FTP3 model, 100MHz bandwidth, DDDSU TDD format

Figure 12. UPT and power consumption of evaluation schemes in InH

Table 8. PSG and UPT loss in InH, 1 UE per cell

| Schemes | LP-WUR “ON” power | PSG (compared to “AlwaysON”) | UPT loss (compared to “AlwaysON”) | PSG (compared to “legacy DCP 40-10-4”) | UPT gain (compared to “legacy DCP 40-10-4”) |
| --- | --- | --- | --- | --- | --- |
| DRX (160-100-8) | - | 56.02% | 53.94% | - | - |
| DRX (40-10-4) | - | 65.61% | 56.15% | - | - |
| legacy DCP(160-100-8) | - | 57.50% | 54.88% | - | - |
| legacy DCP(40-10-4) | - | 69.26% | 63.63% |  |  |
| R17 PDCCHskipping + DRX(160-100-8) + DCP | - | 79.35% | 86.78% | - | - |
| WUS replace DCP(offset = 3ms, 160-100-8) |  | 60.12% | 56.07% | - | - |
| WUS replace DCP(offset = 10ms, 160-100-8) | - | 61.49% | 57.15% | - | - |
| WUS replace DCP(offset = 3ms, 40-10-4) | - | 77.56% | 65.00% | 27.00% | -3.76% |
| WUS replace DCP(offset = 10ms, 40-10-4) | - | 83.76% | 71.64% | 47.17% | -22.00% |
| WUS trigger PDCCH monitoring (MR micro sleep) | 0.1 | 43.85% | 0.00% | -82.64% | 174.99% |
|  | 1 | 42.80% |  | -86.08% |  |
|  | 10 | 33.30% |  | -116.97% |  |
|  | 20 | 22.75% |  | -151.30% |  |
| WUS trigger PDCCH monitoring (MR light sleep) | 0.1 | 68.51% | 25.57% | -2.45% | 104.68% |
|  | 1 | 67.45% |  | -5.88% |  |
|  | 10 | 57.95% |  | -36.77% |  |
|  | 20 | 47.40% |  | -71.10% |  |
| WUS trigger PDCCH monitoring (MR deep sleep) | 0.1 | 86.03% | 56.44% | 54.56% | 19.80% |
|  | 1 | 84.98% |  | 51.13% |  |
|  | 10 | 75.48% |  | 20.23% |  |
|  | 20 | 64.93% |  | -14.09% |  |

- Dense Urban, FR1, DL only FTP3 model, 100MHz bandwidth, DDDSU TDD format

Figure 13. UPT and power consumption of evaluation schemes in DU

Table 9. PSG and UPT loss in DU, 1 UE per cell

| Schemes | LP-WUR “ON” power | PSG (compared to “AlwaysON”) | UPT loss (compared to “AlwaysON”) | PSG (compared to “legacy DCP 40-10-4”) | UPT gain (compared to “legacy DCP 40-10-4”) |
| --- | --- | --- | --- | --- | --- |
| DRX (160-100-8) | - | 55.62% | 53.12% | - | - |
| DRX (40-10-4) | - | 65.93% | 55.30% | - | - |
| legacy DCP(160-100-8) | - | 56.88% | 55.41% | - | - |
| legacy DCP(40-10-4) |  | 69.69% | 64.32% |  |  |
| R17 PDCCHskipping + DRX(160-100-8) + DCP | - | 82.15% | 86.35% | - | - |
| WUS replace DCP(offset = 3ms, 160-100-8) | - | 59.34% | 56.68% | - | - |
| WUS replace DCP(offset = 10ms, 160-100-8) | - | 60.32% | 58.18% | - | - |
| WUS replace DCP(offset = 3ms, 40-10-4) | - | 77.50% | 65.14% | 25.74% | -2.30% |
| WUS replace DCP(offset = 10ms, 40-10-4) | - | 81.27% | 71.33% | 38.19% | -19.64% |
| WUS trigger PDCCH monitoring (MR micro sleep) | 0.1 | 43.85% | 0.00% | -85.26% | 180.27% |
|  | 1 | 42.80% |  | -88.76% |  |
|  | 10 | 33.26% |  | -120.22% |  |
|  | 20 | 22.67% |  | -155.18% |  |
| WUS trigger PDCCH monitoring (MR light sleep) | 0.1 | 68.76% | 26.24% | -3.07% | 106.73% |
|  | 1 | 67.70% |  | -6.57% |  |
|  | 10 | 58.17% |  | -38.03% |  |
|  | 20 | 47.58% |  | -72.99% |  |
| WUS trigger PDCCH monitoring (MR deep sleep) | 0.1 | 86.39% | 57.05% | 55.10% | 20.38% |
|  | 1 | 85.33% |  | 51.60% |  |
|  | 10 | 75.80% |  | 20.14% |  |
|  | 20 | 65.20% |  | -14.82% |  |

Observation 23: With MR entering micro sleep during LP-WUS monitoring, the same UPT performance as always-on scheme can be achieved by the scheme LP-WUR/WUS triggering the resume of PDCCH monitoring, while the UE power consumption will be higher than legacy CDRX scheme.

Observation 24: The scheme LP-WUR/WUS triggering the resume of PDCCH monitoring from light or deep sleep of MR can be superior to the scheme LP-WUS replacing DCP from power saving and/or UPT performance perspective.

Observation 25: When the WUR ON state power is no more than 1unit, the scheme LP-WUR/WUS triggering the resume of PDCCH monitoring from light or deep sleep of MR can achieve the best trade-off between UPT and UE power saving, among all the evaluated UE power saving schemes.

Observation 26: When the WUR ON state power is no less than 10 units, no power saving gain can be achieved by LP-WUS in RRC connected mode.

- System performance

  - System resource overhead of LP-WUS

In order to study the system resource overhead by LP-WUS, we perform the resource overhead calculation with the following assumptions:

- The bandwidth of LP-WUS denoted as BWLP-WUS is 4.32MHz +2RB (30kHz) = 5.04MHz.

- The bandwidth of a serving cell denoted as BWCell is 20MHz or 100MHz.

- The duration of LP-WUS represented by DLP-WUS.

- For RRC idle/inactive mode, the number of beams for LP-WUS transmission is assumed to be 8, which means the LP-WUS is transmitted repeatedly 8 times.

Hence, the resource overhead LP-WUS to the overall system resource can be calculated as below. Table 10 and 11 provide some reference values based on the above assumptions.

- RRC IDLE/INACTIVE mode:

- For RRC idle/inactive mode, paging configuration:

  - Number of pages for a UE per cell per second (NPage_per_cell_second)

  - Number of UEs carried in a LP-WUS is NUE_per_WUS

  - Number of beams is Nbeam

RLP_WUS=BWLP_WUS*DLPWUS*NPagepercellsecond*NUEperWUS*Nbeam BWCell

To match the coverage of Msg.3 PUSCH, the assumed T-F resources of LP-WUS per beam carrying different number of information bits are:

- Alt 1: 5MHz * 1 OFDM symbols (30kHz), carrying 1bit, (Note: Refer to LP-WUS OOK-1Rx 1bit-1symbol in Figure 14);

- Alt 2: 5MHz * 4 OFDM symbols (30kHz), carrying 8bits, (Note: Refer to LP-WUS OOK-1Rx 8bits-4symbols in Figure 14);

- Alt 3: 5MHz * 5 OFDM symbols (30kHz), carrying 8 or 12bits, (Note: Refer to LP-WUS OOK-1Rx 12bits-5symbols in Figure 14).

Figure 14. MIL of LP-WUS for Urban 2.6GHz for non-RedCap UEs

Table 10.  LP-WUS resource overhead ratio to the overall system resource for RRC idle/inactive mode.

| # of info bits carried by LP-WUS | T resources per beam | F resources | SCS | Number of pages UE per second or signal periodicity | Number of UE per group | System configuration, e.g., BW and etc. | Overhead (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 symbol | 5.04MHz | 30kHz | 10 | 1 | 100MHz, 8beams | 0.0150% |
| 1 | 1 symbol | 5.04MHz | 30kHz | 10 | 4 | 100MHz, 8beams | 0.0030% |
| 8 | 4 symbols | 5.04MHz | 30kHz | 10 | 1 | 100MHz, 8beams | 0.0600% |
| 8 | 4 symbols | 5.04MHz | 30kHz | 10 | 4 | 100MHz, 8beams | 0.0100% |
| 8 or 12 | 5 symbols | 5.04MHz | 30kHz | 10 | 1 | 100MHz, 8beams | 0.0750% |
| 8 or 12 | 5 symbols | 5.04MHz | 30kHz | 10 | 4 | 100MHz, 8beams | 0.0130% |
| 1 | 1 symbol | 5.04MHz | 30kHz | 10 | 1 | 20MHz, 8beams | 0.0730% |
| 1 | 1 symbol | 5.04MHz | 30kHz | 10 | 4 | 20MHz, 8beams | 0.0180% |
| 8 | 4 symbols | 5.04MHz | 30kHz | 10 | 1 | 20MHz, 8beams | 0.2900% |
| 8 | 4 symbols | 5.04MHz | 30kHz | 10 | 4 | 20MHz, 8beams | 0.0700% |
| 8 or 12 | 5 symbols | 5.04MHz | 30kHz | 10 | 1 | 20MHz, 8beams | 0.3630% |
| 8 or 12 | 5 symbols | 5.04MHz | 30kHz | 10 | 4 | 20MHz, 8beams | 0.0880% |
| 1 | 1 symbol | 5.04MHz | 30kHz | 39 | 1 | 100MHz, 8beams | 0.0550% |
| 1 | 1 symbol | 5.04MHz | 30kHz | 39 | 4 | 100MHz, 8beams | 0.0150% |
| 8 | 4 symbols | 5.04MHz | 30kHz | 39 | 1 | 100MHz, 8beams | 0.2200% |
| 8 | 4 symbols | 5.04MHz | 30kHz | 39 | 4 | 100MHz, 8beams | 0.0600% |
| 8 or 12 | 5 symbols | 5.04MHz | 30kHz | 39 | 1 | 100MHz, 8beams | 0.2750% |
| 8 or 12 | 5 symbols | 5.04MHz | 30kHz | 39 | 4 | 100MHz, 8beams | 0.0750% |
| 1 | 1 symbol | 5.04MHz | 30kHz | 39 | 1 | 20MHz, 8beams | 0.2800% |
| 1 | 1 symbol | 5.04MHz | 30kHz | 39 | 4 | 20MHz, 8beams | 0.0700% |
| 8 | 4 symbols | 5.04MHz | 30kHz | 39 | 1 | 20MHz, 8beams | 1.1200% |
| 8 | 4 symbols | 5.04MHz | 30kHz | 39 | 4 | 20MHz, 8beams | 0.2800% |
| 8 or 12 | 5 symbols | 5.04MHz | 30kHz | 39 | 1 | 20MHz, 8beams | 1.4000% |
| 8 or 12 | 5 symbols | 5.04MHz | 30kHz | 39 | 4 | 20MHz, 8beams | 0.3500% |
| Note: - For IoT, cases, assuming 1,000,000 user/km2 connection density [ITU M.2412], with packet arrival every 1 message/2 hours/device [ITU M.2412], assuming 500m ISD with 3 sectors. - For eMBB cases, assuming very high population density, e.g., sports event with 5,000 users/cell population density, and per UE paging arrival rate = 1% when paging i-DRX cycle is Tpagingcycle = 1.28s. - The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage. - The above calculation assumes per cell paging. If the tracking area is larger and the base station needs to page UEs in a tracking area covering multiple cells, the LP-WUS overhead may be increased. |  |  |  |  |  |  |  |

- RRC Connected mode:

For RRC connected mode, number of UE configured with LP-WUS per cell NUE_connected can be assumed as 10 (capacity upper bound for XR service in several scenarios). Different UEs can use orthogonal LP-WUS resources. The traffic interval TTrafficInterval can be assumed as 16.67ms (60FPS) for XR and 200ms for eMBB FTP traffic.

RLP_WUS=BWLP_WUS*DLP_WUS*NUE_connected BWCell*TTrafficInterval

As per the agreed assumption of aggregation level (AL) distributions captured in RedCap TR 38.875 below, we note that the distributions of AL1+AL2 accounts for the majority i.e., 70%~90% in typical case. So, LP-WUS matching to PDCCH with AL2 is assumed in our overhead evaluation.

TR 38.875 *** Unchanged text is omitted *** The aggregation level (AL) distributions in Table 6.2-5 are used by different sources for PDCCH blocking rate evaluations. Table 6.2-5: PDCCH AL distributions of AL [1,2,4,8,16], FR1 and FR2 PDCCH AL distributions of AL [1,2,4,8,16]. - Configuration 1 (A1): [0.5, 0.4, 0.05, 0.03, 0.02], assuming majority of the UEs are in is good coverage - Configuration 2 (A2): [0.1, 0.2, 0.4, 0.2, 0.1]: Majority of the UEs are in medium coverage - Configuration 3 (A3): [0.05, 0.05, 0.2, 0.3, 0.4]: Majority of the UEs are in poor coverage - Configuration 4 (A4): [0.3 0.5 0.1 0.06 0.04] - Configuration 5 (A5): [0.4 0.45 0.08 0.04 0.03] - Configuration 6 (A6): [0.2 0.55 0.14 0.06 0.05] - Configuration 7 (A7): [0.4 0.3 0.2 0.05 0.05]. Note: the results for A2/A3 may not represent a typical case, e.g., because of the assumptions of unfavourable channel conditions.. *** Unchanged text is omitted ***

As analysis above, to match the unicast PDCCH AL2-4Rx link budget, we consider the following different examples of T-F resources used for LP-WUS per beam carrying different information bits:

- Alt 1: 5MHz *8 OFDM symbol, carrying 1bit (Note: Refer to unicast LP-WUS OOK-1Rx 1bit-8symbol in Figure 15)

- Alt 2: 5MHz *32 symbols, carrying 8bits (Note: Refer to unicast LP-WUS OOK-1Rx 8bit-32symbol in Figure 15 );

- Alt 3: 5MHz *40 symbols, carrying 12bits (Note: Refer to unicast LP-WUS OOK-1Rx 12bit-40symbol in Figure 15).

Figure 15. MIL of unicast LP-WUS for Urban 2.6GHz for non-RedCap UEs

Table 11.  Resource overhead ratio to the overall system resource for RRC connected mode.

| # of info bits carried by signal | T resources per beam | F resources | SCS | Traffic types | Number of UE per cell | System configuration, e.g., BW and etc. | Overhead (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 (note) | 100MHz, 8beams | 0.8600% |
| 8 | 32 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 | 100MHz, 8beams | 3.4300% |
| 12 | 40 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 | 100MHz, 8beams | 4.2900% |
| 1 | 8 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 100MHz, 8beams | 0.0700% |
| 8 | 32 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 100MHz, 8beams | 0.2900% |
| 12 | 40 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 100MHz, 8beams | 0.3600% |
| Note: The maximum capacity of XR for indoor case is 10 users per cell. |  |  |  |  |  |  |  |

Observation 27: For RRC IDLE/INACTIVE mode, the upper bound of resource overhead used for LP-WUS is less than 1.4% even in highest UE connection density for both IoT and eMBB cases.

Observation 28: For RRC CONNECTED mode, the resource overhead of LP-WUS is

- Less than 4.29% in the cell with maximum XR capacity, i.e. 10 users.

- Less than 0.36% in the cell with 10 users of FTP traffic

| Resource overhead ratio, RLP-WUS | RRC Idle/inactive mode | RRC Connected mode |  |
| --- | --- | --- | --- |
|  |  | XR traffic (10 users per cell, Maximum capacity) | eMBB FTP traffic (10 users per cell) |
| 20MHz, 30KHz SCS | 0.0180%~1.4% | - | - |
| 100MHz, 30KHz SCS | 0.0030%~0.2750% | 0.86%%~4.29% | 0.07%~0.36% |

Proposal 5: Adopting LP-WUS in the network does marginal impact on the system resource overhead for both RRC idle/inactive and connected modes.

  - System resource overhead of LP-SS in RRC idle/inactive mode

LP-SS is beneficial for IDLE/INACTIVE UEs for offloading RRM measurements to LR, and/or synchronization for LP-WUS monitoring. However, there has been some concern regarding system resource overhead and network power consumption due to LP-SS.

We perform the resource overhead calculation of LP-SS with the following assumptions:

- The bandwidth of LP-SS denoted as BWLP-SS is 4.32MHz +2RB (30kHz) = 5.04MHz.

- The bandwidth of a serving cell denoted as BWCell is 20MHz or 100MHz

- The duration of LP-SS represented by DLP-SS is 14 symbols.

- According to the agreed evaluation assumption, the LP-SS periodicity represented by TLP-SS are assumed as 320ms (the most frequent LP-SS transmission), 1280ms (the same as the periodicity of I-DRX paging cycle) and 10240ms (the largest LP-SS periodicity).

- The number of beams for LP-SS transmission is assumed to be 8, which means the LP-WUS is transmitted repeatedly 8 times.

Hence, the resource overhead ratios of LP-SS to the overall system resource can be calculated as below in Table 12.

RLP_SS=BWLP_SS*DLP_SS*Nbeam BWCell*TLP_SS

Table 12.  Resource overhead ratio of LP-SS to the overall system resource in RRC idle/inactive mode

| Resource overhead ratio, RLP-SS | LP-SS periodicity |  |  |
| --- | --- | --- | --- |
|  | 320ms | 1280ms | 10240ms |
| 20MHz, 30KHz SCS | 0.315% | 0.079% | 0.01% |
| 100MHz, 30KHz SCS | 0.063% | 0.0158% | 0.002% |

In addition, LP-SS will not be assumed for RRC connected mode UEs as per the following agreement endorsed in 9.11.3 of the last meeting. Hence, there is no additional resource overhead caused by LP-SS for RRC connected mode.

Agreement - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR.

Observation 29: LP-SS in RRC IDLE/INACTIVE mode consumes no more than 0.32% of the system resource, by considering a dense transmission, e.g., 320ms LP-SS periodicity.

Observation 30: LP-SS is not needed for RRC connected mode UEs enabled with LP-WUR/WUS.

Proposal 6: Adopting LP-SS in the network does marginal impact on the system resource overhead for RRC idle/inactive mode.

  - Network energy consumption

Network energy consumption for transmitting LP-WUS/LP-SS depends on the transmission occasion/periodicity configuration, gNB implementation and cell load etc. In general, there are two types of transmission: periodic transmission of LP-SS, and aperiodic transmission of LP-WUS. Note that the assumptions of different loads follow the agreed definition in R18 NES SI.

As LP-SS is transmitted in periodic manner, the network energy consumption has been a concern by some companies. Due to the sparse traffic arrival rate (e.g., paging rate=[1%, 0.1%, 0.01% or 0.001%]), the network power consumption of LP-WUS transmission is not accounted in our simulation. In the following, the additional network energy consumption due to LP-SS transmission is evaluated, where the baseline assumption and LP-SS configuration is provided in Table 13 and additional evaluation assumptions can be found in Appendix D. The evaluation results for 8 beams and 4 beams LP-SS under different network loads are provided in Table 14 and 15, respectively.

In general, LP-SS can be either FDM or TDM with existing periodic signal/channels, e.g. SSB/SIB1. It is observed that the additional network energy consumption (i.e., <=3.551%) introduced by LP-SS is marginal even for the worst case i.e., 320ms (shortest) periodicity with 8 beams LP-SS TDMed with SSB/SIB1 in zero load. Besides, LP-SS TDMed with SSB/SIB1 will cause more network power consumption than that of LP-SS FDMed with SSB/SIB1.

Table 13. Assumption on baseline and LP-SS configuration

| Parameter | Assumption |
| --- | --- |
| Baseline: SSB and SIB1 transmitted in FDM manner; RACH monitoring | - Periodicity of SSB/SIB1 transmission, RACH monitoring: 20ms - SSB: 4 slots with 2 SSBs in each slot, where 1 SSB occupies 4 OFDM symbols and 20 PRBs - SIB 1: occupies 4 slots and 48 PRBs - RACH: occupies 1 slot - SCS 30KHz |
| LP-SS | - Periodicity of LP-SS: P=320, 640, 1280, 2560, 5120, 10240ms - LP-SS T-F duration is one slot and 11PRBs - 4 or 8 beams assumed for LP-WUS |
| FDM pattern | LP-SS fully FDM with SSB; No time gap between them as shown below |
| TDM pattern | LP-SS is right after SSB as shown below |
| Example of FDM and TDM pattern (assuming 4 beams LP-SS) |  |
| Note: the NW power consumption of LP-WUS transmission is not accounted in the simulation due to the sparse traffic. |  |

Table 14. the Network power consumption caused by 8 beams LP-SS for RRC idle/inactive mode

| Load | Scheme | Case | Cat 1 |  | Cat 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean BS Power consumption | Gain/loss (vs. Baseline) | Mean BS Power consumption | Gain/loss (vs. Baseline) |
| Zero load | FDM with SSB | Baseline | 23.287 |  | 3.466 |  |
|  |  | P=320ms | 23.706 | 1.799% | 3.504 | 1.097% |
|  |  | P=640ms | 23.497 | 0.900% | 3.485 | 0.549% |
|  |  | P=1280ms | 23.392 | 0.450% | 3.476 | 0.274% |
|  |  | P=2560ms | 23.340 | 0.225% | 3.471 | 0.137% |
|  |  | P=5120ms | 23.314 | 0.113% | 3.468 | 0.065% |
|  |  | P=10240ms | 23.302 | 0.061% | 3.467 | 0.031% |
|  | TDM with SSB | Baseline | 23.287 |  | 3.466 |  |
|  |  | P=320ms | 24.114 | 3.551% | 3.538 | 2.076% |
|  |  | P=640ms | 23.701 | 1.776% | 3.502 | 1.038% |
|  |  | P=1280ms | 23.494 | 0.888% | 3.484 | 0.519% |
|  |  | P=2560ms | 23.391 | 0.444% | 3.475 | 0.259% |
|  |  | P=5120ms | 23.332 | 0.192% | 3.470 | 0.124% |
|  |  | P=10240ms | 23.309 | 0.093% | 3.468 | 0.055% |
| Low load | FDM with SSB | Baseline | 27.992 |  | 3.948 |  |
|  |  | P=320ms | 28.364 | 1.328% | 3.984 | 0.912% |
|  |  | P=640ms | 28.168 | 0.627% | 3.966 | 0.459% |
|  |  | P=1280ms | 28.087 | 0.340% | 3.957 | 0.233% |
|  |  | P=2560ms | 28.040 | 0.170% | 3.952 | 0.118% |
|  |  | P=5120ms | 28.017 | 0.087% | 3.950 | 0.057% |
|  |  | P=10240ms | 28.004 | 0.043% | 3.949 | 0.029% |
|  | TDM with SSB | Baseline | 27.992 |  | 3.948 |  |
|  |  | P=320ms | 28.753 | 2.716% | 4.016 | 1.725% |
|  |  | P=640ms | 28.368 | 1.343% | 3.982 | 0.865% |
|  |  | P=1280ms | 28.183 | 0.681% | 3.965 | 0.435% |
|  |  | P=2560ms | 28.090 | 0.350% | 3.956 | 0.222% |
|  |  | P=5120ms | 28.041 | 0.173% | 3.952 | 0.113% |
|  |  | P=10240ms | 28.016 | 0.084% | 3.950 | 0.050% |
| Medium load | FDM with SSB | Baseline | 53.788 |  | 6.649 |  |
|  |  | P=320ms | 54.141 | 0.657% | 6.675 | 0.388% |
|  |  | P=640ms | 53.952 | 0.305% | 6.662 | 0.198% |
|  |  | P=1280ms | 53.875 | 0.162% | 6.656 | 0.105% |
|  |  | P=2560ms | 53.822 | 0.064% | 6.653 | 0.059% |
|  |  | P=5120ms | 53.805 | 0.031% | 6.651 | 0.030% |
|  |  | P=10240ms | 53.796 | 0.015% | 6.650 | 0.015% |
|  | TDM with SSB | Baseline | 53.788 |  | 6.649 |  |
|  |  | P=320ms | 54.367 | 1.076% | 6.698 | 0.742% |
|  |  | P=640ms | 54.137 | 0.648% | 6.674 | 0.375% |
|  |  | P=1280ms | 53.967 | 0.333% | 6.662 | 0.200% |
|  |  | P=2560ms | 53.880 | 0.172% | 6.656 | 0.109% |
|  |  | P=5120ms | 53.831 | 0.080% | 6.653 | 0.053% |
|  |  | P=10240ms | 53.809 | 0.039% | 6.651 | 0.029% |

Table 15. the Network power consumption caused by 4 beams with for RRC idle/inactive mode

| Load | Scheme | Case | Cat 1 |  | Cat 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean BS Power consumption | Gain/loss (vs. Baseline) | Mean BS Power consumption | Gain/loss (vs. Baseline) |
| Zero load | FDM with SSB | Baseline | 23.2873 |  | 3.4662 |  |
|  |  | P=320ms | 23.3048 | 0.075% | 3.46826 | 0.059% |
|  |  | P=640ms | 23.296 | 0.037% | 3.46723 | 0.030% |
|  |  | P=1280ms | 23.2917 | 0.019% | 3.46671 | 0.015% |
|  |  | P=2560ms | 23.2895 | 0.009% | 3.46645 | 0.007% |
|  |  | P=5120ms | 23.2884 | 0.005% | 3.46633 | 0.004% |
|  |  | P=10240ms | 23.2884 | 0.005% | 3.46633 | 0.004% |
|  | TDM with SSB | Baseline | 23.2873 |  | 3.4662 |  |
|  |  | P=320ms | 23.6888 | 1.724% | 3.50218 | 1.038% |
|  |  | P=640ms | 23.488 | 0.862% | 3.48419 | 0.519% |
|  |  | P=1280ms | 23.3877 | 0.431% | 3.47519 | 0.259% |
|  |  | P=2560ms | 23.3375 | 0.216% | 3.47069 | 0.130% |
|  |  | P=5120ms | 23.3124 | 0.108% | 3.46845 | 0.065% |
|  |  | P=10240ms | 23.3124 | 0.108% | 3.46845 | 0.065% |
| Low load | FDM with SSB | Baseline | 27.99219 |  | 3.947543 |  |
|  |  | P=320ms | 28.00813 | 0.057% | 3.949475 | 0.049% |
|  |  | P=640ms | 28.00016 | 0.028% | 3.948511 | 0.025% |
|  |  | P=1280ms | 27.9962 | 0.014% | 3.948034 | 0.012% |
|  |  | P=2560ms | 27.99417 | 0.007% | 3.947789 | 0.006% |
|  |  | P=5120ms | 27.99328 | 0.004% | 3.947671 | 0.003% |
|  |  | P=10240ms | 27.99328 | 0.004% | 3.947671 | 0.003% |
|  | TDM with SSB | Baseline | 27.99219 |  | 3.947543 |  |
|  |  | P=320ms | 28.34794 | 1.271% | 3.981592 | 0.863% |
|  |  | P=640ms | 28.15971 | 0.598% | 3.964701 | 0.435% |
|  |  | P=1280ms | 28.08326 | 0.325% | 3.956271 | 0.221% |
|  |  | P=2560ms | 28.03778 | 0.163% | 3.95196 | 0.112% |
|  |  | P=5120ms | 28.01555 | 0.083% | 3.949791 | 0.057% |
|  |  | P=10240ms | 28.01555 | 0.083% | 3.949791 | 0.057% |
| Medium load | FDM with SSB | Baseline | 53.7879 |  | 6.649008095 |  |
|  |  | P=320ms | 53.79838571 | 0.019% | 6.65037619 | 0.021% |
|  |  | P=640ms | 53.79342857 | 0.010% | 6.649718571 | 0.011% |
|  |  | P=1280ms | 53.79104286 | 0.006% | 6.649399524 | 0.006% |
|  |  | P=2560ms | 53.78975238 | 0.003% | 6.649232381 | 0.003% |
|  |  | P=5120ms | 53.78898095 | 0.002% | 6.649135238 | 0.002% |
|  |  | P=10240ms | 53.78898095 | 0.002% | 6.649135238 | 0.002% |
|  | TDM with SSB | Baseline | 53.7879 |  | 6.649008095 |  |
|  |  | P=320ms | 54.14139048 | 0.657% | 6.67342 | 0.367% |
|  |  | P=640ms | 53.95587143 | 0.312% | 6.661481905 | 0.188% |
|  |  | P=1280ms | 53.87204286 | 0.156% | 6.655593333 | 0.099% |
|  |  | P=2560ms | 53.83029524 | 0.079% | 6.652702857 | 0.056% |
|  |  | P=5120ms | 53.80868571 | 0.039% | 6.651096667 | 0.031% |
|  |  | P=10240ms | 53.80868571 | 0.039% | 6.651096667 | 0.031% |

Observation 31: The additional network energy consumption for periodic LP-SS transmission:

| Network load | LP-SS periodicity | LP-SS duration | Number of beams | Additional NW power consumption |
| --- | --- | --- | --- | --- |
| zero | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 1.097%~3.55% |
| low | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 0.912%~2.716% |
| Medium | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 0.388%~1.076% |

Proposal 7: The additional network energy consumption introduced by periodic LP-SS transmission is marginal.

- Link performance and coverage evaluation results

  - Assumptions in coverage Evaluation

In this section, we provide the evaluation assumptions and coverage evaluation results for LP-WUS, according to agreed evaluation methodologies.

- Frequency error/frequency drift

Working Assumption The following for usage of the clock is assumed for LP-WUR OFF/ON Assumption on LP-WUR OFF power; Assumptions on the clock usage. 0.001; When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 [Note2] (cumulating based on the frequency drift and not exceed maximum frequency error) - The initial frequency offset when LP-WUR switches on can be set to the [FFS: maximum frequency error or a random value within the maximum frequency error] following the parameters of the oscillator option 1/2/3/4[Note2]. - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done.. TBD: value(s); For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 or option 3/4[Note2] (cumulating based on the second value in the value pair and not exceed maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration. [Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power] FFS: Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD2, option 1/2 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD1 Note3: The clock error (of both RTC and LO) could be improved to be less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.

In our evaluation, time offset cumulated in the LP-WUR OFF period can be calculated based on the	 RTC with 20 ppm. frequency error. For time drift, assuming the LP-SS periodicity of 1.28 sec, the time drift ΔT in each of 1.28 sec can be calculated according to the relationship between the maximum frequency error (Fe) and time drift (ΔT) over a time (T), i.e. ΔT(sec)= Fe (ppm)* T(sec) =20 ppm*1.28s=25.6us.

For each LP-WUS monitoring occasion, UE start searching for preamble ΔT time before the actual starting time of the LP-WUS transmission in evaluation. For frequency offset, it is calculated assuming the oscillator opt-1, and we assume the oscillator always work in linear region. For carrier frequency offset(ΔF), max frequency error is assumed, and the frequency displacement in Hz is ΔF(Hz) = ±Fe (ppm) * Fcarrier (Hz)= ±200 ppm*2.6GHz =520kHz.

Figure 18.  Starting detection in advance due to time drifting

- Target FAR and number of attempts across a reference time duration T

In previous meeting, the definition of FAR in evaluation were discussed, and following working assumption were made.

Working Assumption For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials, - UE have N attempts within T, - Company to report (FAR target, T, N) - For example, - if UE makes a single decision based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - if UE performs decoding in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design.

According to above conclusion, when the FAR target and reference time duration T is determined, the greater number of attempts leading to higher FAR, which will conversely limit the MDR performance.

For RRC idle/inactive UEs, always on monitoring of LP-WUS, assume I-DRX=1.28s and 1 slot for each LP-WUS transmission, UE detect LP-WUS with 2560 attempts for the 2560 slots/occasions across the I-DRX cycle. For sequence only LP-WUS, the sequence length should be long enough to achieve both MDR and FAR target. Since the sequence is detected by sliding correlation multiple times in the time duration which covers the time drift, periodical sync to calibrate the timing is needed to reduce the number of correlations within one attempt, to make it easier to achieve the FAR target. For LP-WUS structure with both preamble and payload (with CRC), the FAR should be overall FAR considering both sequence and payload part, and UE start to decode payload part and check CRC only when the correlation of preamble part is higher than certain threshold. With this, the actual number of CRC check per I-DRX cycle is greatly reduced. For example, the FAR of detection preamble part is 1%, UE only need to decode the payload part ~26 times per 1.28 seconds, even with continuous LP-WUS monitoring. To ensure the overall FAR rate lower than 1%, the CRC bits should be long enough, e.g., 12bits.

For RRC connected, main radio (MR) activities are frequent due to CSI/RLM/RRM measurements, etc. Therefore, the accurate timing can be assumed for LP-WUR with the assist from MR. UE only need one attempt for detection of the LP-WUS in each monitoring occasion. In this case, number of attempts is equivalent to number of monitoring occasions. For a typical 60FPS XR traffic with 16.67ms C-DRX cycle, UE need to detect 32 LP-WUS occasions, assuming 1 slot LP-WUS duration and 30kHz SCS, per C-DRX cycle for always on monitoring, and even less times for duty cycled monitoring. In this case, both sequence-only and sequence + payload (with CRC) formats for LP-WUS structure are feasible.

In our evaluation, we assume (FAR target, T, N) = {1%, 1.28sec, 1}. We assume that the periodicity of monitor occasions is 1.28sec, which is I-DRX cycle, and the LP-WUS is transmitted once per 1.28 sec. For detection of each LP-WUS transmission, UE performs multiple correlations for LP-WUS sequence, and these correlations are considered as UE implementation in ONE trial/attempt.

- Noise Figure

In previous meetings, it was agreed that noise figure can be selected from [9, 12, 15, 18, 21, 24] db. One of the above values should be reported together with MIL.

According to [2] [3], 8.7dB and 11dB noise figure can be achieved for LP-WUR, 12dB noise figure is assumed for MIL calculation in this section.

- Assumption for antenna/beamforming gain for LP-WUS and legacy NR channels

The DL coverage is highly dependent on the gNB transmission EIRP. While the EIRP is usually determined based on transmission power and beamforming gain. In Rel-15/Rel-16, for a UE specific downlink transmission, gNB may select beam with better quality to serve a UE through beam management procedure or taking advantage of channel reciprocity, hence large BF gain can be assumed for transmissions or receptions other than DL broadcast channels.

For DL broadcast channels, e.g. SSBs and PDCCH in type 0-2 CSSs, typically these channels are served by fixed and wider beams, the beamforming gain would be lower compared with UE specific channels. To reflect the coverage of the broadcast channels more accurately, the BF gain difference between broadcast channel and unicast channel should be reflected in link budget template. The beamforming gain correction should be considered for both paging PDCCH and LP-WUS received in idle state.

Similarly, for UL channels, difference between broadcast BF gain and unicast BF gain in gNB reception should be reflected. For Msg3 PUSCH, gNB receives the Msg3 using the same beam as SSB which UE selected to initiate the RACH procedure. Hence, the receiving beamforming gain can be same as the SSB beamforming gain, i.e., broadcast beamforming gain. The same correction factor as the correction factor for gNB Tx BF gain for broadcast channel can be assumed for UL channels transmitted in idle state, e.g., Msg3. While for PUSCH eMBB, unicast beamforming gain can be assumed due to beam management procedure.

According to [4], the correction factor for gNB Tx BF gain for broadcast channel and gNB Rx BF gain should be considered in link budget template.

- In FR1, the correction factor is about 8dB for carrier frequency 2.6GHz/4GHz; 0dB for 700MHz.

In the link budget template,

Proposal 8: The antenna correction factor for gNB Tx/Rx BF gain for broadcast DL channel and Msg3, should be considered in link budget template to reflect the difference between broadcast beamforming and unicast beamforming

- The same correction factor should be assumed for broadcast DL channel and Msg3;

- In FR1, the correction factor is about 8dB for 2.6GHz/4GHz; 0dB for 700MHz.

In the link budget template provided in R1-2304151, the correction factors for modelling antenna gain loss including BF gain loss can be provided in the following rows.

(5b) antenna gain correction factor at antenna gain component 2 of transmitter (dB)
Note: zero for uplink

(11bis-b) antenna gain correction factor at antenna gain component 2 of receiver (dB)Note:  zero for downlink

  - Preliminary coverage evaluation results for LP-WUS

In the section, we provide our preliminary coverage evaluation results for LP-WUS. Detailed simulation assumptions are provided in Appendix C and several configurations are listed as following:

-Option-1: sequence+ payload + CRC

preamble length-16 chips, payload-12bits, CRC-8bits, 4.32MHz BW;

preamble length-12 chips, payload-8bits, CRC-6bit, 4.32MHz BW;

- Option-2: sequence only-1 bit,4.32MHz BW;

- Option-3: payload + CRC

payload-12bits, CRC-8bits, 4.32MHz BW;

payload-8bits, CRC-8bits, 4.32MHz BW;

The option1 and option2 channel structure has a preamble, which can be used to resist time error and reduce FAR. Since the option3 channel structure does not have a preamble, it relies on LP-SS to indicate timing information. At this time, a more sophisticated crystal oscillator is required to ensure that the time error is within the tolerance range.

The modulation of the above three options is OOK-4 and OFDM, and in the following figures, we provide the coverage comparison between LP-WUS and legacy NR signals for both  non-RedCap UE and Redcap UE.  In order to meet different coverage requirements, different date rates and number of symbols are used, and the number of resources can be adjusted by power boosting, channel coding, etc. More simulation parameters can be found in Appendix C.

Observation 32: In Urban scenario, for non-RedCap UEs, LP-WUS/LP-SS achieves PUSCH Msg3 coverage, with 1-7 OFDM symbols for carrying 1-12 information bits; LP-WUS/LP-SS achieves broadcast PDCCH AL8-2Rx coverage with 28 OFDM symbols for carrying 1 information bit, but much more resources for carrying 8bits or more

Observation 33: In Urban scenario, for RedCap UEs, LP-WUS/LP-SS achieves PUSCH Msg3 coverage, with 1-7OFDM symbols for carrying 1-12 information bits; LP-WUS/LP-SS achieves broadcast PDCCH AL8-1Rx coverage with 8 OFDM symbols for carrying 1 information bit, and 28-56 OFDM symbols for carrying 8-12 information bits

Observation 34: In Rural scenario, for non-RedCap UEs, LP-WUS/LP-SS achieves PUSCH Msg3 coverage, with 1-7 OFDM symbols for carrying 1-12 information bits; LP-WUS/LP-SS achieves broadcast PDCCH AL8-2Rx coverage with 28 OFDM symbols for carrying 1 information bit, but much more resources for carrying 8bits or more

Observation 35: In Rural scenario, for RedCap UEs, LP-WUS/LP-SS achieves PUSCH Msg3 coverage, with 1-7OFDM symbols for carrying 1-12 information bits; LP-WUS/LP-SS achieves broadcast PDCCH AL8-1Rx coverage with 4 OFDM symbols for carrying 1 information bit, and 16-28 OFDM symbols for carrying 8-12 information bits

Proposal 9: To reduce the MIL gap between LP-WUS and NR PDCCH, coverage enhancement methods for LP-WUS such as power boosting, channel coding and more time/frequency domain resources can be studied.

Figure 17. MIL of LP-WUS for Urban 2.6GHz for non-RedCap UEs(option1&2)

Figure 18. MIL of LP-WUS for Urban 2.6GHz for non-RedCap UEs(option3)

Figure 19. MIL of LP-WUS for Rural 700MHz for non-RedCap UEs(option1&2)

Figure 20. MIL of LP-WUS for Rural 700MHz for non-RedCap UEs(option3)

Figure 21. MIL of LP-WUS for Urban 2.6GHz for Redcap UE(option1&2)

Figure 22. MIL of LP-WUS for Urban 2.6GHz for Redcap UE(option3)

Figure 23. MIL of LP-WUS for Rural 700MHz for Redcap UE(option1&2)

Figure 24. MIL of LP-WUS for Rural 700MHz for Redcap UE(option3)

- Conclusion

In this contribution, design targets and evaluation methodologies for R18 LP-WUS/WUR study are discussed with the following observations and proposals:

Observation 1: In the case that the relative power of LP-WUR ON state is 0.03~0.5unit, substantial power saving gain (i.e. 80% or higher) can be achieved.

Observation 2: LP-WUS/WUR design should address all traffic characteristics for IoT/Wearable/eMBB, thus keeping the LP-WUR ON state power as low as possible is important for increasing the battery life of the device.

Observation 3: Compared to I-DRX with PEI, LP-WUR/WUS with continuous monitoring configuration can achieve 53%~98% power saving gain when the relative power of LP-WUR ON state is no more than 1 unit, and ~150ms latency reduction for dynamic PO case or ~500ms latency increase for legacy PO case.

Observation 4: Compared to eDRX with PEI, LP-WUR/WUS with both legacy PO and dynamic PO can significantly reduce the paging latency (i.e., can reduce ~26s), with comparable UE power consumption.

Observation 5: LP-WUR/WUS provides a much better trade-off between latency and power consumption when relative power of LP-WUR ON state is no more than 1unit, compared with legacy I-DRX and eDRX.

Observation 6:  Continuous LP-WUS monitoring cannot provide power saving gain, when the relative power of LP-WUR ON state is higher than 1unit.

Observation 7: Duty cycled LP-WUS monitoring can achieve more than 98% power saving benefit compared to legacy I-DRX with PEI when the relative power of LP-WUR ON is no more than 1unit.

Observation 8: Up to 44% power saving gain will be lost if UE main radio goes into deep sleep rather than ultra-deep sleep during LP-WUS monitoring in RRC idle/inactive.

Observation 9: Compared to LP-WUR ON power >=10unit, 6.4%~15.6% additional power saving gain can be achieved when the relative power of LP-WUR ON is no more than 1unit.

Observation 10: With fixed duty cycle ratio and LP-WUR “ON” state power, when LP-WUR “ON” state power is no more than 1unit, different LP-WUR duty cycle length have no or marginal impact on UE power consumption.

Observation 11: When the duty cycle ratio raises from 1% to 10%, the achievable UE power saving gain will be largely degraded, e.g., >=40% for 10 or 20 units LP-WUR “ON” power case.

Observation 12: MR RRM measurement relaxation is critical to achieve power saving gain by LP-WUR/WUS, where the MR RRM measurement relaxation includes MR RRM measurement period by X times, and RRM offloading from MR to LR.

Observation 13: Even with 16x or 32x MR measurement relaxation, less power saving gain e.g., <53% is obtained by duty cycled LP-WUS monitoring (with 1% duty cycle ratio) scheme, when the LP-WUR “ON” power is 10 or 20 units.

Observation 14: Higher UE power saving benefit can be achieved if RRM measurement can be completely offloaded from MR to LR.

Observation 15: RedCap UEs are more sensitive to the LP-WUR ON state power, i.e. compared to eMBB UEs, the power saving gain for RedCap UEs decreases much faster as the LP-WUR ON state power increases.

Observation 16: LP-WUS/WUR can be used to reduce the unnecessary PDCCH monitoring due to jitter while guarantee the XR performance (i.e. latency, capacity) for RRC connected mode.

Observation 17: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring with MR micro sleep can bring {7%~15%} relative UE power saving gain with no capacity loss in both low load and high load cases for DL only XR traffic.

Observation 18: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring with MR light sleep can bring {14%~25%} relative UE power saving gain, with acceptable capacity impact at least in low load case for DL only XR traffic.

Observation 19: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring with main receiver micro sleep can bring {8%~12%} additional UE power saving gain with no capacity loss in both low load and high load cases for DL+UL XR traffics.

Observation 20: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring with main receiver light sleep can bring {16%~19%} additional UE power saving gain, with acceptable capacity impact at least in low load case for DL+UL XR traffics.

Observation 21: LP-WUS/WUR can be used to improve UE power saving and/or UPT performances for eMBB traffic in RRC connected mode.

Observation 22: There are two potential usages of LP-WUS for RRC connected mode as follows:

- LP-WUS replaces DCP to indicate the start of drx-ondurationtimer;

- LP-WUS dynamically triggers the resume of PDCCH monitoring.

Observation 23: With MR entering micro sleep during LP-WUS monitoring, the same UPT performance as always-on scheme can be achieved by the scheme LP-WUR/WUS triggering the resume of PDCCH monitoring, while the UE power consumption will be higher than legacy CDRX scheme.

Observation 24: The scheme LP-WUR/WUS triggering the resume of PDCCH monitoring from light or deep sleep of MR can be superior to the scheme LP-WUS replacing DCP from power saving and/or UPT performance perspective.

Observation 25: When the WUR ON state power is no more than 1unit, the scheme LP-WUR/WUS triggering the resume of PDCCH monitoring from light or deep sleep of MR can achieve the best trade-off between UPT and UE power saving, among all the evaluated UE power saving schemes.

Observation 26: When the WUR ON state power is no less than 10 units, no power saving gain can be achieved by LP-WUS in RRC connected mode.

Observation 27: For RRC IDLE/INACTIVE mode, the upper bound of resource overhead used for LP-WUS is less than 1.4% even in highest UE connection density for both IoT and eMBB cases.

Observation 28: For RRC CONNECTED mode, the resource overhead of LP-WUS is

- Less than 4.29% in the cell with maximum XR capacity, i.e. 10 users.

- Less than 0.36% in the cell with 10 users of FTP traffic

| Resource overhead ratio, RLP-WUS | RRC Idle/inactive mode | RRC Connected mode |  |
| --- | --- | --- | --- |
|  |  | XR traffic (10 users per cell, Maximum capacity) | eMBB FTP traffic (10 users per cell) |
| 20MHz, 30KHz SCS | 0.0180%~1.4% | - | - |
| 100MHz, 30KHz SCS | 0.0030%~0.2750% | 0.86%%~4.29% | 0.07%~0.36% |

Observation 29: LP-SS in RRC IDLE/INACTIVE mode consumes no more than 0.32% of the system resource, by considering a dense transmission, e.g., 320ms LP-SS periodicity.

Observation 30: LP-SS is not needed for RRC connected mode UEs enabled with LP-WUR/WUS.

Observation 31: The additional network energy consumption for periodic LP-SS transmission:

| Network load | LP-SS periodicity | LP-SS duration | Number of beams | Additional NW power consumption |
| --- | --- | --- | --- | --- |
| zero | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 1.097%~3.55% |
| low | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 0.912%~2.716% |
| Medium | 320ms | 14symbols per beam, 8 slots in total and 11PRBs | 8 | 0.388%~1.076% |

Observation 32: In Urban scenario , for non-RedCap UEs, LP-WUS/LP-SS achieves PUSCH Msg3 coverage, with 1-7 OFDM symbols for carrying 1-12 information bits; LP-WUS/LP-SS achieves broadcast PDCCH AL8-2Rx coverage  with 28 OFDM symbols for carrying 1 information bit, but much more resources for carrying 8bits or more

Observation 33: In Urban scenario , for RedCap UEs, LP-WUS/LP-SS achieves PUSCH Msg3 coverage, with 1-7OFDM symbols for carrying 1-12 information bits; LP-WUS/LP-SS achieves broadcast PDCCH AL8-1Rx coverage  with 8 OFDM symbols for carrying 1 information bit, and 28-56 OFDM symbols for carrying 8-12 information bits

Observation 34: In Rural scenario , for non-RedCap UEs, LP-WUS/LP-SS achieves PUSCH Msg3 coverage, with 1-7 OFDM symbols for carrying 1-12 information bits; LP-WUS/LP-SS achieves broadcast PDCCH AL8-2Rx coverage  with 28 OFDM symbols for carrying 1 information bit, but much more resources for carrying 8bits or more

Observation 35: In Rural scenario , for RedCap UEs, LP-WUS/LP-SS achieves PUSCH Msg3 coverage, with 1-7OFDM symbols for carrying 1-12 information bits; LP-WUS/LP-SS achieves broadcast PDCCH AL8-1Rx coverage  with 4 OFDM symbols for carrying 1 information bit, and 16-28 OFDM symbols for carrying 8-12 information bits

Proposal 1: The target relative power of LP-WUR ON state should be less than 1 unit.

Proposal 2: The latency target for LP-WUS/WUR:

- Latency for IoT and wearable use cases are in the order of second;

- Latency for XR use case is in the order of millisecond;

- Latency for eMBB use case is in the order of tens of milliseconds.

Proposal 3: Around 100kbps data rate can be considered as design target for LP-WUS.

Proposal 4: It is recommended to support at least one of the following RRM measurement enhancements to achieve substantial UE power saving benefit by LP-WUR/WUS

- Relax the MR RRM measurement period by X times, where the X should be sufficiently large number, e.g. 16 or larger

- Offload the RRM measurements from MR to LR

Proposal 5: Adopting LP-WUS in the network does marginal impact on the system resource overhead for both RRC idle/inactive and connected modes.

Proposal 6: Adopting LP-SS in the network does marginal impact on the system resource overhead for RRC idle/inactive mode.

Proposal 7: The additional network energy consumption introduced by periodic LP-SS transmission is marginal.

Proposal 8: The antenna correction factor for gNB Tx/Rx BF gain for broadcast DL channel and Msg3, should be considered in link budget template to reflect the difference between broadcast beamforming and unicast beamforming

- The same correction factor should be assumed for broadcast DL channel and Msg3;

- In FR1, the correction factor is about 8dB for 2.6GHz/4GHz; 0dB for 700MHz.

Proposal 9: To reduce the MIL gap between LP-WUS and NR PDCCH, coverage enhancement methods for LP-WUS such as power boosting, channel coding and more time/frequency domain resources can be studied.

# References

- Chairman’s Notes of RAN1 #113.

- R1-2304501, “Remaining issues on low power wake-up receiver architecture”, vivo.

- Jaeho Im, Jacob Breiholz, Shuo Li, Benton Calhoun, David D. Wenzloff , “A Fully Integrated 0.2V 802.11ba Wake-Up Receiver with -91.5dBm_Sensitivity”, 2020, IEEE Radio Frequency Integrated Circuits Symposium.

- Renzhi Liu, Asma Beevi K. T., Richard Dorranc, Deepak Dasalukunte, etc. “An 802.11ba 495W -92.6dBm-Sensitivity Blocker-Tolerant Wake-up Radio Receiver Fully Integrated with Wi-Fi Transceiver”, 2019, IEEE Radio Frequency Integrated Circuits Symposium.

# Appendix A – Processing timeline for the schemes in RRC idle/inactive mode

The assumed processing timelines of I-DRX paging, eDRX and LP-WUS/WUR schemes.

# Appendix B – Evaluation assumption for RRC connected mode

The detail configurations for R17 PDCCH monitoring adaptation and LP-WUS/WUR schemes are illustrated as below.

R17 PDCCH monitoring adaptation scheme includes both PDCCH skipping indication and SSSG switching indication, and its key assumptions are listed below:

- Default/sparse SSSG: do PDCCH monitoring in every two slots;

- Dense SSSG: do PDCCH monitoring in every slot.

- Two candidate PDCCH skipping durations: 4ms and 6ms.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

For LP-WUS/WUR scheme, the key assumptions are introduced below:

- The LP-WUS monitoring is performed within DRX onduration.

- The main receiver enters into the micro sleep when doing LP-WUS monitoring.

- The total relative power for LP-WUS monitoring is assumed as 45 or 20 power units (i.e., the main radio enters micro or light sleep).

- And the latency from receiving LP-WUS to be ready for PDCCH monitoring can be assumed to 0ms and 3ms corresponding to MR enters micro or light sleep states during LP-WUS monitoring.

- R17 PDCCH skipping indication is adopted. And three candidate PDCCH skipping durations are 2ms, 4ms and 6ms respectively.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

- System capacity is assumed as the maximum number of users per cell with at least 90% of UEs being satisfied. And a UE can be regarded as a satisfied UE if more than 99% or 95% of packets are successfully transmitted within a given air interface PDB.

Table 16. Simulation assumption for FR1

| Parameter | Indoor Hotspot | Dense Urban |
| --- | --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m | Dense Urban, hexagonal layout with 7, 3 Sectors |
| Channel model | InH | Uma |
| Carrier frequency | 4GHz |  |
| Bandwidth | 100 MHz, 1.72% Guard Band |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ | For 64T(8,8,2,1,1;4,8) , (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |  |
| BS Power | 24 dBm per 20MHz | 44 dBm per 20MHz |
| UE max Power | 23 dBm |  |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| ISD | 20 m | 200m |
| BS height | 3 m | 25m |
| UE height | 1.5 m |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |
| Max MCS | 256QAM |  |
| Device deployment | 100% indoor | 20% outdoor, 80% indoor |
| Down-tilt | 90 degrees | 12 degrees |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| Target BLER | 10% |  |
| UE speed | 3 km/h |  |

Table 17. The DL video traffic models with 60 FPS adopted in R17 XR SI

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| PDB (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 or 5 |  |
| Jitter Range (ms) | [-4, +4], [-6, +6] or [-8, +8] |  |

Table 18. DL audio traffic models of XR

| Traffic model | Packet size | Periodicity (ms) | PDB(ms) | Data rate (Mbps) |
| --- | --- | --- | --- | --- |
| Audio | determined by periodicity and data rate | 10ms | 30 | 0.756 |

Table 19. UL traffic models of XR

| Traffic model | Mean packet size (Bytes) | STD of packet sizes (Bytes) | Min packet size (Bytes) | Max packet size (Bytes) | Packet arrival interval (ms) | PDB (ms) | Jitter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pose/control stream | 100 | 0 | 100 | 100 | 16.67 | 10 | No |

# Appendix C – Evaluation Results on Coverage for LP-WUS

In the evaluation results shown in Appendix D, some common parameters are assumed, provided as follows

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Carrier Frequency | 2.6GHz |  |  |
| Case name | LP-WUS option1 | LP-WUS option2 | LP-WUS option3 |
| Channel structure | Option1： sequence+ payload + CRC 16 chips +12bits+8 bits 12 chips +8bits+6 bits | Option2： Sequence only Sequence only: 1 bit | Option3： payload + CRC 12bits +8 bits 8bits +8 bits |
| Performance metric | FAR <0.1% MDR 1% | FAR < 1% MDR 1% | FAR < 1% MDR 1% |
| Chip rate | 56kcps(M=2)/112kcps(M=4)/224kcps(M=8)/1 symbol 2bit for OFDM |  |  |
| Waveform | OOK -4/OFDM |  |  |
| Coding | 1/2 rate Manchester coding (For information bits and CRC bits) (OFDM no coding) |  |  |
| Time error | RTC 20ppm for Channel structure option1 & option2 |  |  |
| Frequency error | {200 ppm,0.1ppm/s} for Channel structure option1 & option2 (Need sequence to correct time error) {5 ppm,0.1ppm/s} for OOK-4 Channel structure option3 {2 ppm,0.1ppm/s} for OFDM Channel structure option3 |  |  |
| Beacon periodicity | 0.32 sec Note: beacon periodicity is used to calculate the time drift for WUS monitoring |  |  |
| SCS | 30kHz |  |  |
| gNB Channel BW | 20MHz (50 RB) |  |  |
| WUS BW | 4.32MHz |  |  |
| Guard band | 1RB on each side of LP-WUS bandwidth |  |  |
| Filter | 5th Order Butterworth with 4.32MHz bandwidth |  |  |
| ASCI | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH = 1:1. |  |  |
| Sampling Rate | 7.68 MHz |  |  |
| ADC bitwidth | 4 bit ADC |  |  |
| Channel Model | TDL-C 300 |  |  |

Detailed MIL calculation for LP-WUS for above configurations, are provided in the following attachment which is calculated according to the template.

# Appendix D – Evaluation assumptions on network energy consumption

- Assumption on network load

| Zero load | Low load | Light load | Medium load |
| --- | --- | --- | --- |
| NaN | DL FTP3 traffic with 0.5Mbytes packet size and 200ms mean packet interval |  |  |
| 10 UEs per cell | 2 UEs per cell | 6 UEs per cell | 10 UEs per cell |

- Assumption on BS power model

- Reference configuration

|  | Set 1 FR1 |
| --- | --- |
| Duplex | TDD |
| System BW | 100 MHz |
| SCS | 30 kHz |
| Number of TRP | 1 |
| Total number of DL TX RUs | 64 |
| Total DL power level | 55dBm |
| Total number of UL Rx RUs | 64 |

- BS power consumption model

| Power state |  | Relative Power P for Category 1 | Relative Power P for Category 2 |
| --- | --- | --- | --- |
|  |  | Set 1 FR1 | Set 1 FR1 |
| Deep sleep | P1 | 1 | 1 |
| Light sleep | P2 | 25 | 2.1 |
| Micro sleep | P3 | 55 | 5.5 |
| Active DL | P4 | 280 | 32 |
| Active UL | P5 | 110 | 6.5 |

- Assumption on simulation parameters

|  |  | Parameters |
| --- | --- | --- |
|  |  | Set 1 |
| Basic parameters | Channel model | 3D-Uma as in TR 38.901 (low-loss O2I penetration model) |
|  | Percentage of high loss and low loss building type | 100% low loss |
|  | Device deployment | 80% indoor, 20% outdoor |
|  | Inter-site distance | 500m |
|  | Network Topology | 7*3 Sector |
|  | Carrier Frequency | 4.0GHz |
|  | Multiple access | OFDMA |
|  | Duplexing | TDD |
|  | Numerology | 30kHz, 14 OFDM symbol slot |
|  | Guard band ratio on simulation bandwidth | TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth) |
|  | Simulation bandwidth | 100 MHz |
|  | Frame structure | DDDSU (S slot is assumed as 10D:2G:2U) |
|  | UT attachment | Based on RSRP |
|  | Wrapping around method | Geographical distance based wrapping |
| BS parameters | BS antenna height | 25 m |
|  | BS noise figure | 5 dB |
|  | BS Tx power | 55 dBm |
|  | BS antenna element gain | 8 dBi |
|  | Antenna configuration at TRxP | For 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8). based on 38.802 |
| UE parameters | UE power class | 23dBm |
|  | UE noise figure | 9 dB |
|  | UE antenna element gain | 0 dBi |
|  | UE antenna height | Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height |
|  | Antenna configuration at UE | For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| Transmission parameters | Modulation | Up to 256 QAM |
|  | Transmission scheme | SU-MIMO |
|  | SU dimension | For 4Rx: Up to 4 layers |
|  | DL CSI measurement | Precoded CSI-RS based |
|  | DL codebook | non-PMI transmission |
|  | Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
|  | Scheduling | PF |
|  | Receiver | MMSE-IRC |
|  | Channel estimation | Non-ideal |
|  | HARQ scheme | Ideal |
|  | Max HARQ retransmission | 3 |
|  | Target BLER | 10% of first transmission |
|  | Power control parameters | Open loop, P0=-80dBm, alpha=0.8 |
| Common RS | SSB period | 20ms |
|  | SS blocks per SSB burst | Up to 8 |
|  | SSB time resource | 4 symbols for each SSB |
|  | SSB frequency resource | 20 RBs |