3GPP TSG RAN WG1 #114   	                             R1-2306893

Toulouse, France, August 21st – August 25th, 2023

Agenda item:		9.9.2

Source:		LG Electronics

Title:			Discussion on network verified UE location for NR NTN

Document for:	Discussion and Decision

Introduction

Agreement

For network verified UE location in NTN, satellite ephemeris information should be available at the LMF.

Agreement

For network verified UE location in NTN common TA information should be reported at least from gNB to LMF.

Working assumption

In NTN, gNB receive-transmit time difference calculated at uplink time synchronization reference point is reported to the LMF.

In RAN1#113 meeting, following agreements and working assumptions for a single satellite based multi-RTT for network verified UE location were made.

Proposal 2: add in RAN#100 chair’s note

- TSG-RAN tasks RAN1 to focus its network verified UE location work on Alt1 for the combination of “UE and gNB receive-transmit time difference measurements”:

  - Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215.

    - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted

- If the work at RAN1 is not completed at RAN#101, the nwk verified UE location will be dropped from RAN1

Also, in RAN#100, following proposal [1] was endorsed.

This contribution discusses and provides our view on network verified UE location for NR NTN.

Discussions on a single satellite based Multi-RTT positioning

RTT determination

For normative phase of NW verified UE location, it was agreed to prioritize multi-RTT assuming a single satellite in view. Illustration of multi-RTT with a single satellite is depicted in Figure 1.

Figure 1. Illustration of multi-RTT with a single satellite

In TN, in order to determine RTT, gNB Rx-Tx time difference measurement and UE Rx-Tx time difference measurement are defined in TS 38.215. Also, the reference point for gNB Rx-Tx time difference measurement is defined in TS 38.215. For NTN, similar mechanism can be employed for a single satellite based multi-RTT positioning.

For the reference of gNB Rx-Tx time difference measurement, as captured above, uplink time synchronization reference point was agreed as working assumption. Other candidate were on gNB or on satellite. In our view, all options can work well for a single satellite based multi-RTT positioning. The difference is required additional signaling/information at LMF, e.g., Common TA, K_mac, feederlink delay, etc. Therefore, it is preferred to confirm working assumption.

Proposal #1: Confirm following working assumption

- In NTN, gNB receive-transmit time difference calculated at uplink time synchronization reference point is reported to the LMF

Agreement

For RTT determination in NTN, discuss further the accuracy, and reporting details of combinations of the following UE and gNB receive-transmit time difference measurements:

- Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215.

  - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted

- Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215.

  - Note 2: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS

- Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4

FFS: One or multiple SRS can be used in determining the arrival time

FFS: Additional enhancement including additional information to be reported, if justified

Note 3: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

Note 4: The gNB Rx-Tx time difference option in the above alternatives may need updates accordingly based on the outcome of discussion on reference point for the gNB Rx – Tx time difference

In the RAN1#113 meeting, there was intensive discussion to down select combinations of UE and gNB Rx-Tx time difference measurement. However, RAN1 failed to achieve consensus. Instead, there was RAN plenary guideline to focus on Alt-1 as captured above.

For Alt-1, it relies on legacy definition with potential offset values, therefore it may not need to change the definition of UE and gNB Rx-Tx time difference measurement. Alt-2 and Alt-3 employ option 2 for UE Rx-Tx time difference where UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS. For this type of measurement/reporting, more signaling overhead seems to be required compared to that of Alt 1, since the value range can vary depending on which combination of PRS and SRS (pair) is utilized. For gNB Rx-Tx time difference, Alt-3 is based on the option 4 where gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS. Therefore, this alternative has the same issue as UE Rx-Tx measurement based on option 2. Also, if RAN1 fails to achieve consensus, NW verified UE location will be dropped which is undesirable. Therefore, to avoid undesirable situation, support Alt-1 for combination of UE and gNB Rx-Tx time difference measurement.

Proposal #2: For combination of UE and gNB Rx-Tx time difference measurement, support Alt-1 (UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215).

If Alt-1 is supported, following issues need to be further considered. One issue is whether/how to reflect timing error or delay due to processing time in satellite and movement of satellite and/or UE. One way is to let LMF know information such as satellite/UE movement, timing drift, etc. In this way, LMF can handle delay/error caused by satellite and/or UE movement. Also, feeder link delay (e.g., Kmac and/or common TA) needs to be signaled to LMF for the RTT calculation.

Proposal #3: Support UE to report information related to timing error or delay to LMF.

Agreement

Study the following options to resolve the mirror positions ambiguity for multi-RTT positioning:

- Option 1: gNB or LMF implementation to solve the mirror error issue.

  - FFS: whether there is spec impact

- Option 2: Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements

- Option 3: NR NTN UE should report the Doppler calculated on the service link

- Option 4: a VSAT UE should report its beam pointing in respect to satellite beam line of sight

- Option 5: Reporting of cell coverage information (e.g. cell footprint and reference point, or antenna pattern) to the LMF

- Option 6: Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioningOther solutions are not precluded

Mirror position ambiguity for single satellite based multi-RTT

As captured above, potential solutions for mirror position ambiguity for single satellite based multi-RTT were listed in RAN1#112 meeting. Option 1 is based on the gNB or LMF implementation. For example, gNB beamforming be used to resolve this issue. With this option, it has a benefit that no or minimized specification impact is expected. Other options require additional specification impact and/or UE complexity to solve the same ambiguity problem. Therefore, if there are no significant advantages from adopting either option 2 to 6, option 1 should be supported.

Proposal #4: Support option 1 (based on gNB or LMF implementation) for resolving mirror positions ambiguity in single satellite based multi-RTT positioning.

Conclusion

In this contribution, we discussed on network verified UE location for NR NTN. Based on the above discussion, we have the following proposals.

Proposal #1: Confirm following working assumption

- In NTN, gNB receive-transmit time difference calculated at uplink time synchronization reference point is reported to the LMF

Proposal #2: For combination of UE and gNB Rx-Tx time difference measurement, support Alt-1 (UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215).

Proposal #3: Support UE to report information related to timing error or delay to LMF.

Proposal #4: Support option 1 (based on gNB or LMF implementation) for resolving mirror positions ambiguity in single satellite based multi-RTT positioning.

- Reference

[1] RP-231451, Way forward for the Rel-18 WID NR_NTN_enh, Moderator (Thales)