3GPP TSG RAN WG1 #114									R1-2306922

Toulouse, France, 21 August – 25 August 2023

Agenda Item 	:	9.11.3

Source 	:	Sony

Title 	:	On L1 signal design and procedures for LP-WUS

Document for 	:	Discussion and decision

# Introduction

A new release 18 study item on low-power wake-up signal and receiver was approved in the RANP#94e meeting and revised in RANP#97e, with the following objectives

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

During the RAN1#111 meeting, the following agreements were made.

Agreement - Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Note that above does not preclude DFT-S-OFDMA - Study generation and link performance of multi-carrier (MC)-FSK waveforms - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded - For next meeting, companies to provide input on aspects to consider that might impact link performance Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

In RAN#112 meeting, the following agreements were made.

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol) Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2 Agreement For M-bit MC-FSK generation study further the following options - Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view) - Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view) - M >0 - N >1 - Study how to generate segment in time domain, e.g. OOK-1 or OOK-4 - Other options are not precluded. Agreement For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study - FDM/TDM multiplexing with other NR transmissions - link performance - impact to legacy UEs - impact on gNB Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

In RAN#112bis-e the following agreements were made:

Agreement - Capture in TR: From RAN1 perspective, LP-WUS and signals/channels used by MR can be within the same FR1 band. - At least LP-WUS and signals/channels by MR can be on the same carrier in the band - Study further - Whether LP-WUS and signals/channels used by MR can be different carriers in the band - Details on the LP-WUS location within a carrier - Band can be different than band of signals/channels used by MR - LP-WUS association with BWP - LP-WUS can be configurable within guard-band of a band (like NB-IoT) Agreement Update the RAN1#112 agreement as the following: - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] Working assumption: In place of the above deleted bullets: - Alt 1: - average EPRE within the [time]/frequency resources used for LP-WUS (including any guard bands) - time/frequency resources used for LP-WUS (including any guard bands) - Alt 2: - average EPRE within the [time]/frequency resources used for LP-WUS (including any guard bands) - SNR is calculated as average EPRE divided by power of noise [and interference]. - Companies to report whether and how power pooling across and within MR OFDMA symbols is used. - FFS: PAPR applicable to LP-WUS Agreement Replace in RAN1#112 agreement Companies to report - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms with - receiver architecture type and its relative power consumption Agreement - For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g. UE-group, -subgroup or -ID - FFS: cell information - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information - For CONNECTED mode, study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g UE-group, -subgroup or -ID - indication to wake-up to PDCCH monitoring. - Other information candidates are not precluded - Study pros and cons of including above information to LP-WUS. - Note: the information may be explicitly or implicitly indicated. Agreement - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different - In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques. Agreement - Study further following alternatives to carry the LP-WUS information using: - Alt 1: by sequence(s) detection/selection - FFS sequence type - Alt 2: by encoded bits - FFS: what type of encoding scheme - FFS: with or without other bits (e.g. CRC/FCS) - Other alternatives are not precluded - Study whether LP-WUS information needs to be preceded by known one or more sequence(s). Agreement At least for IDLE/Inactive mode, at least one BW-size <=5MHz is recommended to be supported for FR1 - Other BW sizes are not precluded - if additional BW-size(s) are recommended to be supported, BW-size can be up to 20MHz - LP-WUS bandwidth size (including guard-bands) is assumed to be an integer number of PRBs Agreement Study further methods to modulate input signal of the DFT/Least-Square block for OOK-4, and methods to modulate input signal of N SCs for other MC-ASK/FSK schemes - study methods with respect to - improving frequency diversity by flattening the spectrum, frequency repetition and frequency hopping - impact to dynamic range of RE power in frequency domain - FFS: impact to PAPR of generated time domain modulated MC-ASK/FSK symbol - improving robustness to timing error necessary spectrum adjustment for compatibility with CP-OFDM generation Agreement - Study techniques/mechanisms to enhance coverage performance of LP-WUS - Study potential gains available as well as drawback(s) of the technique(s)/mechanisms(s), e.g. system overhead, increased complexity network energy consumption etc… - Study potential issues and corresponding solutions for the case when LP-WUS coverage is insufficient At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.

In RAN1#113, the following agreements were made:

Agreement - For at least RRM serving cell measurement performed by LP-WUR based on reference signal(s), RAN1 identified at least the following metrics for further study and evaluation (including feasibility, complexity, power consumption, etc) - LP-RSSI or Energy detection: linear average of total received power over a RSSI resource. - FFS RSSI resource. - LP-RSRP: linear average of received power of resource of reference signal(s) or signal(s) parts. - FFS resource of reference signal(s) or signal(s) parts - LP-SINR = LP-RSRP/(power of interference and noise) - FFS how to define “power of interference and noise” - LP-RSRQ= [N x] LP-RSRP/LP-RSSI, where N is the factor of resource size difference for evaluation LP-RSRP and LP-RSSI. - Accounting AGC accuracy, ADC of at least 4 bits is required. - Note: Reference signal for performing measurements can be e.g. SSB (PSS/SSS/PBCH DMRS), LP-WUS-waveform sequence, LP-SS - Note: The definition of metrics could be further refined based on future study Agreement Power pooling between OFDM symbols is not assumed for evaluation purposes. Average EPRE is defined per OFDM symbol. Agreement The following observations are to be captured in the TR - At least for LP-WUR that cannot receive existing PSS/SSS, periodic LP-SS signal is beneficial for the following functionality. - RRM measurements by LP-WUR, if supported - at least coarse time synchronization of LP-WUR. - at least coarse frequency synchronization of LP-WUR. - Additional periodic LP-SS system overhead depends on LP-SS periodicity, system BW, # of beams, and resource required to fulfil the target functionality, etc. Periodic signal if used for coarse synchronization may reduce overhead of signal preceding LP-WUS, if any. LP-SS can be designed to be common among UE groups (cell-specific) and such further reduce system overhead. - For LP-WUR that can receive existing PSS/SSS potentially assisted by PBCH DMRS/TRS for synchronization, existing PSS/SSS potentially assisted by PBCH DMRS/TRS may be used for above functionality. - Periodic LP-SS coverage should be equal or better than that of LP-WUS. - For fine time and frequency synchronization, a signal (e.g. preamble) preceding or part of LP-WUS may be used. Agreement - For Idle/Inactive mode, study offloading of RRM measurements of serving cell to LP-WUR under certain conditions, if any, and relaxation of serving/neighboring cell RRM measurements in MR considering - Periodic reference signal(s) is/are used for LR measurements. - FFS: reference signal(s) to measure, e.g. PSS/SSS/PBCH DMRS, LP-WUS waveform sequence, LP-SS - FFS: periodicity, content - MR performs measurements - Alt2: with relaxed periodicity if RRM measurement in MR is relaxed. - FFS: Condition for relaxation if any - Can apply for both neighboring and serving cell - Alt3: only when reference signal(s) based measurements by LP-WUR satisfy certain condition(s), e.g. are below threshold. - FFS threshold. - Above MR measurement under certain conditions can apply for both neighboring and serving cell - Potentially with relaxation methods for MR neighboring cell measurement - Other alternatives are not precluded - FFS: Feasibility of RRM measurements of neighbor cells by LP-WUR Agreement - For waveform generation the following observations are made - Flat spectrum in frequency domain provides robustness against frequency selective fading compared to concentrated energy in frequency domain. - for OOK-4, sequence before DFT/LS with variation in phase via such as ZC, M-sequence or QAM sequence can achieve more flattened spectrum. - Sequences(s) used in LP-WUS symbol generation with different pulse shape or spectral shape may have different performance. - Knowledge of sequence(s) used in LP-WUS waveform generation may improve performance for at least a receiver with I/Q branches - Further discuss the following potential observations for waveform generation: - When DFT is employed in OOK-4 (M>=2), -1/1 alternation in time or frequency shift in frequency domain may be needed to match CP-OFDM generation. - Pre-storing of the generated frequency domain samples at gNB may reduce complexity of waveform generation at gNB with memory requirement depending on number of possible combination. This may be up to gNB implementation. - quantization of generated waveform in frequency domain to existing constellation (e.g. 64QAM) has low impact on performance and reduces complexity. This may be up to gNB implementation. - Repetition of a sequence(s) used in LP-WUS generation in frequency can be used to improve diversity for MC-OOK and robustness against frequency offsets for MC-FSK. For companies to consider for providing evaluation results - Cross-waveform-comparison - OOK-1 M=1 and OOK-4 M=1 (may not need to be simulated, difference can be only in frequency domain sequence used) - OOK-1 with M x higher SCS than NR, and OOK-4 M - M=2,4 - OOK-4 M=2 and OOK-2 M=2 - OOK-3 M=1 and OOK-1 M=1 - OOK-1 and OOK-2 M=2 with further reduced coderate/increased sequence length - OOK-1 and OOK-4 M=2 with further reduced coderate/increased sequence length - FSK1/2 M=1 (1bit per OFDMA symbol) and OOK-1 M=2 - FSK1/2 M=2 (2bits per OFDMA symbol) and OOK-2 M=4 - FSK1/2 M=2 (2bits per OFDMA symbol) and OOK-4 M=4 - OFDMA and other waveforms with roughly matching T-F resources - Note: Above cases should result in same length of LP-WUS in OFDMA symbols and BW for both compared waveforms - Manchester coding 1/2 is applied to OOK for at least encoded bits (payload). - At least time and frequency impairments should be included. - residual time offset 0, 1, 2 and 4 us - residual frequency offset 0, 1, 2, 5 and 10 - optional 50, 100 ppm - showing tolerance higher than above values is not precluded - If further improvement of the signal generation for the agreed waveforms is applied, companies are to provide relevant details - For evaluation of LP-SS accuracy, assume SNR at [-3dB] and LP-WUR noise figure should be reported Agreement - Study the following techniques/mechanisms to enhance coverage performance of LP-WUS - low complex channel coding - FEC - spreading code in time domain - time domain repetition - with combining before or after ED - time-domain interleaving - Note: Also Manchester coding can be considered as channel code - non-contiguous transmission in the frequency domain - frequency domain repetition - frequency-hopping - power-boosting - transmit diversity - study whether any above techniques could be transparent to UE. Agreement - For Idle/Inactive mode, following options for activation and deactivation of LP-WUS monitoring by LP-WUR for a UE can be considered for study - Alt 1a: - gNB transmits legacy paging indication and LP-WUS - UE activation and/or deactivation of LP-WUS WUS monitoring is up to UE implementation. - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient. - Alt 1b: - gNB transmits legacy paging indication and LP-WUS - UE activation and/or deactivation of LP-WUS monitoring is based on preconfigured criteria - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient. - Alt 2: - activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling. - Paging misdetection performance shall not be impacted. Agreement - In RRC CONNECTED mode, study benefit of LP-WUS over existing Rel-15, R16, and R17 power saving techniques for following functionalities: - LP-WUS with similar functionality as R16 DCP. - LP-WUS activates/resumes PDCCH monitoring when LP-WUS is received. - interaction with legacy power saving techniques, if any - other functionalities are not precluded - for evaluation - companies to report - assumption on MR sleep state when LP-WUR is monitoring LP-WUS - deep sleep, - light sleep, - micro sleep - how to activate/deactivate LP-WUS monitoring and deactivate/activate PDCCH monitoring - LP-WUS waveform - In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of - by gNB RRC signaling, with or without UE assistance. - by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance. - based on pre-configured condition(s), such as timer. - LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB. - other options are not precluded.

Our initial evaluation of an OFDM/multi-carrier-OOK based LP-WUS L1 design was provided in [2]. This document provides further details on this OFDM-OOK based design and an OFDM sequence-based LP-WUS design. This document also considers L1 and higher layer procedures required to support LP-WUS operation.

# L1 signal design

The low-power wake-up signal (WUS) should be designed to have certain properties:

- The WUS should avoid causing interference to the existing OFDM transmission – this can be achieved by the signal having a similar mapping as OFDM.

- The WUS should be designed so that it can carry a minimum amount of information.

- The total energy consumption of WUR during WUS detection is minimised.

- The total system overhead is minimized,

- Using the WUS to acquire some degree of synchronization when waking up from sleep mode is advantageous both for the WUS detection and also power consumption. We do not need to decode SSBs before detecting the WUS for example.

- The WUS structure should be configurable to support different conditions, e.g., coverage level, data capacity etc.

- The WUS should have high detection probability and minimum false alarm rate even in poor radio conditions.

- The WUS should be uniquely identifiable as emitted from a particular cell.

- The WUS should identify which UE(s) or group of UEs it is targeted at.

The following sub-sections contain analyses of an OOK-based design for LP-WUS and an OFDM-based design for LP-WUS. Both LP-WUS types should be considered in this study item, allowing conclusions on the most suitable LP-WUS form to be made.

# Interference-free OFDM embedding of OOK Design for LP-WUR

When designing LP-WUS, to ensure low power detection and decoding of the WUS, the following properties should be considered:

- non-coherent modulation to allow for the low-power design of the receiver,

- compensation for the wake-up receiver performance (sensitivity) loss due to extreme low-power design of the LP-WUR and co-channel interference for example.

To tailor LP-WUR/WUS scheme to the NR protocol, the LP-WUS transmission needs to be compliant with the transmission principles of NR, i.e., based on OFDM transmission. In other words, to avoid interfering with the OFDM transmission, the LP-WUS needs to be orthogonal in the same way as OFDM sub-carriers are, despite being based on different modulation principles.

In the following, we discuss LP-WUS structure and content, its transmission scheme, and its base-band processing.

LP-WUS structure and content– To allow for the use of low-power low-complexity WUR designs, a simple non-coherent modulation scheme like on-off keying (OOK), frequency shift keying (FSK), or pulse position modulation (PPM) is often used. Among these simple modulation schemes, we choose to consider Manchester coded OOK modulated sequences in this section as they can be detected at extremely low power consumption levels. Different candidate architectures for the reception of OOK based LP-WUS are proposed, and they are still under discussion. Low-power low-complexity design of a LP-WUR typically leads to a higher noise figure and degraded sensitivity, compared to the main receiver. Depending on the LP-WUR architecture, the level of power consumption vs noise figure and sensitivity can vary. The degraded sensitivity inevitably leads to higher raw bit-error rates (BERs). To compensate for the resulting high raw BER on the channel, we use longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allowing for low detection error, i.e., miss-detection and false-alarm, probabilities. This lengthening of the signals can also be used for carrying additional information, beyond the pure wake-up trigger. In this LP-WUS design, we embed cell identity and a wake-up group identity, which will significantly reduce unnecessary wake-ups. The first part which carries the cell identity can also be used as a preamble, identifying whether there is a WUS on the channel or not. Only after detection of the first part, the UE proceeds with decoding of the second part of the LP-WUS. The sequences are chosen from sequences such as m-sequences with good correlation properties resulting in low miss detection and false-alarm probabilities. Figure 1 shows the general signal structure used in this section of the contribution for an OOK-based LP-WUS containing sequences of length M and L bits, representing cell identity and wake-up group identity, respectively. The address spaces of these two sequences are typically much smaller than the M and L bits in the sequences, providing a degree of spreading that reduces error probabilities. The first part of the LP-WUS can be also used for synchronization purposes.

Figure 1 - low-power wake-up signal (LP-WUS) structure

Proposal 1 – Support Manchester coded OOK modulated LP-WUS as it can be detected at extremely low power consumption levels.

Proposal 2 – For OOK-based LP-WUS, support LP-WUS designs compensating for LP-WUR performance loss and improving LP-WUR coverage using spreading where each information bit is represented by a sequence or several bits.

Proposal 3 – For OOK-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

Transmission scheme – To avoid LP-WUS interfering with the OFDM transmission, the OFDM transmitter can be modified to also transmit OOK modulated sequences designed for non-coherent LP-WUR, on a subset of the carriers, thus embedding the LP-WUS correctly and ensuring orthogonality. This scheme is basically based on the same principle as in “MC-OOK option 4” agreed as one of the options for the generation of MC-OOK waveform in the previous meetings.

Figure 2 – Block diagram of modified OFDM transmitter

Figure 2 shows a block diagram of the modified OFDM transmitter, including the two blocks used to create the embedded WUS. The first block (A) is a sectioning block creating sections of the target LP-WUS and is only used if the length of the LP-WUS becomes longer than one OFDM symbol. The WUS shaping block (B) then shapes each LP-WUS section so that the resulting signal as output from the OFDM modulator becomes as close to the target LP-WUS as possible using the designated sub-carriers. Step (C) to (D) adds CPs to maintain orthogonality in channels with time dispersion.

Comparison of OOK-4 with other OOK schemes - OOK-4 transmission scheme has advantages compared to the other MC-OOK waveform generation candidates in terms of system overhead, latency and low-power consumption. Assume the LP-WUS is L bits long with BWx transmission bandwidth and K the number of transmitted bits in one OFDM symbol. The length of LP-WUS for MC-OOK-4 becomes L/K times lower than the one in MC-OOK-1. This results in lower system overhead, delay and total power consumption in MC-OOK-4 compared to MC-OOK-1 candidate. For MC-OOK-2 and MC-OOK-3 have the same system overhead and delay as the MC-OOK-4 candidate. The LP-WUR for reception of these two waveforms, however, are more complicated and with higher on power consumption. This means MC-OOK-2 and MC-OOK-3candidates results in a higher total average power consumption at the UEs compared to when MC-OOK-4 is used.

Moreover, MC-OOK-4 is well spread over available SCs which automatically makes it robust against frequency fading particularly in comparison to MC-OOK-3 where only a single sub-carrier is used for the transmission and is therefore particularly sensitive to a fading-dip in frequency. MC-OOK-4 only has a single non-zero amplitude level on the time domain signal, making its PAPR low, particularly compared to MC-OOK-1 where all signal energy is maximally concentrated in time due to setting all available sub-carriers to one.

Observation 1 –MC-OOK-4 has lower system overhead and can be detected with shorter delay using lower power LP-WUR, compared to other MC-OOK candidates. It is also more robust to fading, particularly in comparison to MC-OOK-3. Furthermore, MC-OOK-4 only has a single non-zero amplitude level on the time domain signal, making its PAPR low, particularly compared to MC-OOK-1.

Proposal 4 – Support MC-OOK option 4 mechanism to embed LP-WUS within OFDM transmissions without creating interference to other OFDM transmissions.

DBB processing and detection performance– The employed LP-WUR DBB design, consisting of two detector units used to detect the above LP-WUS design, is shown in Figure 3. The detectors contain a cell identity correlator (CIC) of length M and a wake-up group identity correlator (GIC) of length L. First the CIC is active and whenever its output exceeds the threshold, the cell identity is detected, then the next L incoming bits are sent to the GIC for group identity detection. If the output of the GIC also exceeds its threshold we declare LP-WUS detection, and the main receiver is woken up.

Figure 3 – Digital baseband (DBB) block diagram

Based on coverage MIL of ~145 dB, discussed in [3], 240 kbps Manchester-coded OOK symbol rate and a zero-IF architecture with 1-bit ADC, we end up with a LP-WUR power consumption in the range of 100 uW, equivalent to 0.1 unit.  Using the the LP-WUS design described above we simulate the complete LP-WUR architecture including its AFE and the digital baseband processing, based on Figure 3, to find LP-WUS detection characteristics. The simulation is done for both AWGN and TDL-C channels. In this initial performance analysis, Kasami sequences of equal length M and L of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit WUS, which has a total time duration of 0.52 msec. With these parameter settings the maximum outputs from the correlators are equal to the sequence length of 63. The receiver operating characteristics for the cell identity detector when changing threshold γ1 from 0 to 62 and keeping γ2=45, is shown in Figure 4.

Figure 4 – Simulated receiver operating characteristics for LP-WUS

There is one curve for the single cell case and one where the LP-WUS interference from neighbour cells is also included. The two curves are, for this parameter setting, essentially identical. The chosen point of operation is at γ1of 46, which gives us an LP-WUS detection probability of 0.99 and a low false-alarm probability of around 10-5. When performing this simulation, we have taken into account a drift compensation, chosen cautiously to be 10 times that of the ultra-deep sleep of the main receiver. A signal-to-noise ratio of -2dB and 13dB are required for the AWGN and TDL-C channel, respectively, to reach 99% detection and 10-5 false-alarm rate.

| Channel | Required SNR to reach MDR and FAR of 1% (dB) |
| --- | --- |
| AWGN | -2 |
| TDL-C | 13 |

Table 1 - Required SNR for OOK-4 and different channel conditions.

Observation 2 – To compensate for performance loss of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low detection error probabilities.

# OFDM-Sequence-based Design

The OFDM-sequence-based LP-WUS can consist of a preamble sequence optionally followed by a data part as illustrated in Figure 5 below.  The optionality of the data part depends on how much signalling information the WUS is expected to carry. The preamble part can use sequences such as ZC, Gold etc sequences with high autocorrelation properties to maximise detection probability and low cross correlation to minimise false detection probability. Consideration should also be given to PAPR of the LP-WUS signal in the choice of sequences. Further, the preamble part is composed of multiple OFDM symbols modulated with the same preamble sequence. Each LP-WUS OFDM symbol is preceded by a cyclic-prefix preferably longer than is used in transmissions to the main receiver. Using the same sequence across multiple OFDM symbols allows detection of the preamble in the presence of significant frequency offsets. The LP-WUS information symbols carry data in a cyclic shift of the OFDM symbol that provides decoding resilience to co-channel interference and other additive noise during detection of the preamble. Some of the information carried in the LP-WUS information symbols can be used to verify the detected preamble thereby lowering significantly the false alarm rate of the WUS detector.

Figure 5 - OFDM Sequence based LP-WUS design

To modulate the sequences either in-band or out of band of the NR main carrier, elements of the sequence are used to modulate each OFDM subcarrier as illustrated in Figure 6. This example shows an LP-WUS sequence composed of 12*L resource elements which uses L RBs of the OFDM signal. At each end of the LP-WUS signal spectrum, the guard band is made up of D resource elements. The same set of subcarriers are modulated with the same sequence for Np OFDM symbols of the preamble and then a further Nd symbols for the information part. Both Np and Nd depend on the minimum operating SINR that is required whilst Nd additionally depends on how many bits are to be carried in the information part.

Figure 6 - : LP-WUS OFDM modulation

For the information symbols part of the LP-WUS, binary modulation such as QAM can be used but, as this is an amplitude modulation, it is quite susceptible to noise and interference. So instead, the data part can use the same sequence as the preamble part wherein data is carried in a cyclic shift or other more noise-resilient property of the sequence. For example, if b bits of data are to be carried in the LP-WUS data symbols, each symbol is subjected to one of 2b cyclic shifts.

Carrying the data in properties of the sequence changes the data detection into a sequence detection process which is much more immune to noise and interference than amplitude modulations. Sequence detection for the data part requires coherent detection, but the sequences are deployed in a manner that allows easy frequency and time synchronisation at the preamble detection stage. Furthermore, as WUS has to be cell-dependent, different sequences for the WUS preamble can be deployed to different cells.

Observation 3: An OFDMA-based LP-WUS design can easily incorporate time and frequency synchronisation capabilities.

LP-WUS Design Evaluation

We have evaluated an example LP-WUS signal design. In this example, the LP-WUS signal uses one of 32 possible ZC sequences. Each ZC sequence is 143 elements long and deployed on 12 consecutive RBs and is generated with a unique root that corresponds to a targeted group of UEs. To complete the LP-WUS design, the ZC sequence is multiplied by a PN sequence that uniquely identifies the cell that emits the LP-WUS signal for example by initialising its pseudo-random sequence generator with the cell-ID. In the evaluation, each of the Nd data symbols of the LP-WUS carries 5 bits of information. The 5-bits identify the root of the ZC used in the preamble. At the LP-WUR of the UE, this can be used to confirm the detected ZC sequence thereby ensuring that the false alarm rate (wherein the WUR detects an LP-WUS targeted at the UE where, in reality, there was none) is effectively zero. If more bits are transmitted in the cyclic shifts, these can be used to increase the number of targeted UE groups or indeed signal other aspects related to the functioning of the WUR. Table 2 summarises the test conditions.

| Condition | Values | Notes |
| --- | --- | --- |
| (Np, Nd) | (4,8) | Number of preamble and data symbols |
| WUS duration(us) | 500 | WUS of (4,8) symbols @ SCS 30e3 is 500us. |
| Channels | AWGN, TDL-C 300ns @ 3km/h |  |
| SNR (dB) | (-12, -9, -6, -3, 0) | Range for reasonable noise figures |
| LP-WUS SCS | 30kHz |  |
| Main signal SCS | 15kHz |  |
| L | 12 | RBs |
| CP | 2.345us | Dependent on symbol duration @ 30kHz SCS |
| WUS Bandwidth | 4.29MHz |  |
| Guardband width | 1.68MHz | D = 57 REs |
| Component carrier BW | 20MHz |  |
| Receive IF/RF filter | Butterworth 4th order |  |

Table 2: Link-layer simulation conditions for OFDM-based LP-WUS

The OFDM sequence-based design of LP-WUS described in Table 2 was evaluated as part of the design process to gauge the WUS preamble detection capabilities. Simulations were based on a 20MHz component carrier in which RBs not used for the LP-WUS and its guard band were occupied with other DL channels and signals. No LP-WUS symbols were transmitted during the first OFDM symbol of each subframe (containing a longer CP). For SCS of 30kHz comprised of 28 OFDM symbols in each subframe, symbols 0 and 14 have the longer CP. Since the LP-WUS signal comprises only 4 preamble + 8 data (total 12) symbols, these can fit between symbols 0 and 14. Therefore, all LP-WUS OFDM symbols are transmitted with the same CP. The consistent LP-WUS symbol CP eases LP-WUS symbol synchronisation at the LP-WUR. In all the simulations, the FAR is very close to zero because of the confirmation of the LP-WUS sequence transmitted in the data symbols. The LP-WUS was evaluated at different SNR in TDL-C at 3km/h speed. The simulated performance is shown in Figure 7. The ultimate design goal is to minimise both misdetection rate (MDR) and false alarm rate (FAR).

Figure 7 – OFDM-based LP-WUS performance in TDL-C @ 3km/h

At SNR = -6dB, the MDR is below 0.1%. In [3], it is shown that an LP-WUS with this SNR performance easily achieves the NR coverage of the MR.

Observation 4: An OFDMA-based LP-WUS design can operate at SNR = -6dB with MDR and FAR < 0.1%.

In this section, we have studied an OFDMA-based LP-WUS design, according to the agreement made in RAN1#111:

- Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded

Proposal 5 – For OFDMA-based LP-WUS, support LP-WUS designs consisting of N WUS preamble / detection symbols and M WUS information symbols. Values of N and M can be chosen for a suitable SNR operating point.

Proposal 6 – For OFDMA-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

# L1 Procedures: activation / deactivation / reconfiguration of LP-WUS

In this section, we consider issues related to the activation / deactivation or reconfiguration of LP-WUS. These procedures may be required if the coverage and / or reliability of the LP-WUS degrades.

The following agreements were made in RAN1#113:

Agreement - Study the following techniques/mechanisms to enhance coverage performance of LP-WUS - low complex channel coding - FEC - spreading code in time domain - time domain repetition - with combining before or after ED - time-domain interleaving - Note: Also Manchester coding can be considered as channel code - non-contiguous transmission in the frequency domain - frequency domain repetition - frequency-hopping - power-boosting - transmit diversity - study whether any above techniques could be transparent to UE.

Activation / deactivation / reconfiguration of LP-WUS

When the LP-WUR only monitors LP-WUS in IDLE mode, the LP-WUR doesn’t know whether the LP-WUS is being received reliably or not. If the UE relies on the LP-WUS to wake it up and the LP-WUS becomes unreliable (for example, due to coverage degrading), the UE can become unreachable. In such a case, it might be better for the UE to switch to monitoring via the MR. Alternatively, the gNB may reconfigure the LP-WUS to operate with a more robust format, for example by increasing the number of repetitions or transmission power applied to the LP-WUS (or any of the other methods outlined in the above agreement, as appropriate). The reliability of LP-WUS can be determined in various ways, including:

- UE monitoring the number of times that a periodic LP-SS is received, assuming the LP-SS format is based on the LP-WUS format

- The paging response rate of UEs to LP-WUS signalling / paging. The gNB can estimate the reliability of LP-WUS by estimating the percentage of successful pages. If the paging success rate decreases (especially for stationary UEs), the gNB can infer that LP-WUS reliability is degraded

- UE comparing the number of times an LP-WUS is received to the number of times an LP-WUS is transmitted. This would require some indication from the gNB of the number of times (within an interval) that an LP-WUS has been transmitted.

It is apparent that there are various ways in which LP-WUS reliability can be estimated by the UE or gNB. The method of reliability determination can be decided on during a potential work item.

Observation 5: The reliability of LP-WUS can be determined by either the UE or the gNB. RAN1 will need to decide on the method for this reliability determination.

The following agreement was made in RAN1#113:

Agreement - For Idle/Inactive mode, following options for activation and deactivation of LP-WUS monitoring by LP-WUR for a UE can be considered for study - Alt 1a: - gNB transmits legacy paging indication and LP-WUS - UE activation and/or deactivation of LP-WUS WUS monitoring is up to UE implementation. - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient. - Alt 1b: - gNB transmits legacy paging indication and LP-WUS - UE activation and/or deactivation of LP-WUS monitoring is based on preconfigured criteria - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient. - Alt 2: - activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling. - Paging misdetection performance shall not be impacted.

We think that all of the alternatives are workable for the simple case of activation / deactivation of LP-WUS monitoring, whereby the UE switches from using the LP-WUR to the MR. It is however preferable if the LP-WUS coverage can be adapted through reconfiguration in order for it to be reliable throughout the cell. LP-WUS adaptation / reconfiguration requires signalling of the new LP-WUS configuration to UEs. Hence, we prefer Alt. 2.

Proposal 7: The TR observes that “Alt 2 – activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling” also allows for reconfiguration of the LP-WUS to achieve the required LP-WUS reliability.

# RRM measurements and higher layer signalling to support LP-WUR/WUS

In RAN1#113, a set of possible measurements for performing serving measurements was agreed:

Agreement - For at least RRM serving cell measurement performed by LP-WUR based on reference signal(s), RAN1 identified at least the following metrics for further study and evaluation (including feasibility, complexity, power consumption, etc) - LP-RSSI or Energy detection: linear average of total received power over a RSSI resource. - FFS RSSI resource. - LP-RSRP: linear average of received power of resource of reference signal(s) or signal(s) parts. - FFS resource of reference signal(s) or signal(s) parts - LP-SINR = LP-RSRP/(power of interference and noise) - FFS how to define “power of interference and noise” - LP-RSRQ= [N x] LP-RSRP/LP-RSSI, where N is the factor of resource size difference for evaluation LP-RSRP and LP-RSSI. - Accounting AGC accuracy, ADC of at least 4 bits is required. - Note: Reference signal for performing measurements can be e.g. SSB (PSS/SSS/PBCH DMRS), LP-WUS-waveform sequence, LP-SS - Note: The definition of metrics could be further refined based on future study

It should be possible to use one or more of the agreed measurements for serving cell measurements. The choice of which can be decided during any potential work item. The measurements need to be performed on a signal that is repeatedly transmitted, such as an SSB, LP-SS or an LP-WUS waveform sequence that is transmitted at known times (if this is different to the LP-SS or if the LP-SS is not supported).

The LP-WUS waveform sequences referred to above can act as low power reference signals (LP-RS) to support mobility. The gNBs periodically broadcast the low-power reference signal which can be decoded by a LP-WUR. This signal needs to be designed to have the same characteristics as the LP-WUS, i.e., has a lower complexity than the existing reference and synchronization signals.

As described earlier, introducing a new LP-SS with a main functionality of providing synchronization is under study. Another functionality of LP-SS is to use this signal for RRM measurement.  To allow for low-power RRM measurement functionality for both serving cell and neighbour cell measurement, we propose to define a neighbour cell area, referred to as LP neighbour cell area (LP-NCA), which is a preconfigured neighbor cells list, containing information of a group of neighbor cells with the following one or more criteria:

- No cell in the LP neighbor cell area bars a UE with LP-WUR.

- All cells in the area support signaling for LP-WUR operation.

- All cells in the LP neighbor cell area are within the same geographical area.

The UE can roam around in the LP-NCA neighbor cell area by only evaluating LP-SS of the neighbor cells. As long as the UE can camp on one of the cells listed in the LP-NCA when performing cell re-selection, the UE does not need to receive any information related to SIB1-SIB5 preventing from powering up the main receiver. Another advantage of this approach is that the UE does not have to read MIB/SIBs [immediately] when moving to a new cell.

If the UE roams outside the LP neighbor cell area LP-NCA in which it is indicated that all measurements based on LP-RS / LP-SS are below the threshold then the UE activates the main-receiver (M-Rx) to perform cell-reselection using the legacy approach.

Proposal 8 – Support functionality of LP-SS for both serving cell and neighbor cell measurement.

Proposal 9 – Support introducing a neighbor cell area for UEs with LP-WUR, with the following one or more criteria, a) no cell in the LP neighbor cell area bars a UE with LP-WUR, b) all cells in the area support signaling for LP-WUR operation, c) all cells in the LP neighbor cell area are within the same geographical area.

# 5	Conclusion

In this contribution, we have discussed our views on low-power WUS design and aspects related to L1 and higher layer signalling. We have considered both an OOK-based LP-WUS signal design and an OFMDA-based LP-WUS signal design. Our observations and proposals are listed below.

Observation 1 –MC-OOK-4 has lower system overhead and can be detected with shorter delay using lower power LP-WUR, compared to other MC-OOK candidates. It is also more robust to fading, particularly in comparison to MC-OOK-3. Furthermore, MC-OOK-4 only has a single non-zero amplitude level on the time domain signal, making its PAPR low, particularly compared to MC-OOK-1.

Observation 2 – To compensate for performance loss of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low detection error probabilities.

Observation 3: An OFDMA-based LP-WUS design can easily incorporate time and frequency synchronisation capabilities.

Observation 4: An OFDMA-based LP-WUS design can operate at SNR = -6dB with MDR and FAR < 0.1%.

Observation 5: The reliability of LP-WUS can be determined by either the UE or the gNB. RAN1 will need to decide on the method for this reliability determination.

Proposal 1 – Support Manchester coded OOK modulated LP-WUS as it can be detected at extremely low power consumption levels.

Proposal 2 – For OOK-based LP-WUS, support LP-WUS designs compensating for LP-WUR performance loss and improving LP-WUR coverage using spreading where each information bit is represented by a sequence or several bits.

Proposal 3 – For OOK-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

Proposal 4 – Support MC-OOK option 4 mechanism to embed LP-WUS within OFDM transmissions without creating interference to other OFDM transmissions.

Proposal 5 – For OFDMA-based LP-WUS, support LP-WUS designs consisting of N WUS preamble / detection symbols and M WUS information symbols. Values of N and M can be chosen for a suitable SNR operating point.

Proposal 6 – For OFDMA-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

Proposal 7: The TR observes that “Alt 2 – activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling” also allows for reconfiguration of the LP-WUS to achieve the required LP-WUS reliability.

Proposal 8 – Support functionality of LP-SS for both serving cell and neighbor cell measurement.

Proposal 9 – Support introducing a neighbor cell area for UEs with LP-WUR, with the following one or more criteria, a) no cell in the LP neighbor cell area bars a UE with LP-WUR, b) all cells in the area support signaling for LP-WUR operation, c) all cells in the LP neighbor cell area are within the same geographical area.

References

- RP-222644. “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”. RANP#97e. September 2022.

- R1-2210224. “LP-WUS L1 signal design and procedures”. Sony. RAN1#110bis-e. October 2022.

- R1-2306921. “Evaluation of low-power WUS”. Sony. RAN1#114. August 2023.

- R1-2302213. “Summary#3  of discussions on L1 signal design and procedure for low power WUS”. Moderator (Nordic Semiconductor). RAN1#112. February 2023.

- R1-2302861. “Evaluation of low power WUS”. Sony. RAN1#112bis_e. April 2023.

- R1-2302251. “FL summary #4 (final) of evaluation on low power WUS”. Moderator (vivo). RAN1#112 Athens. February 2023.