3GPP TSG RAN WG1 #114							R1-2307170

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	5

Title:	Discussion on RAN2 LS on data collection

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN2#122 meeting, RAN2 sent an LS to RAN1 regarding data collection requirements and assumptions in Rel-18 AI for Air interface [1]. In this contribution, we provide our views on this LS.

- Discussion

In this LS, RAN2 made some working assumptions that need RAN1 confirmation, and identified some aspects that need RAN1 input to facilitate further discussion in RAN2 as below:

  - Part A: Discussion on some assumptions on data collection

RAN2 has the following assumptions on data collection:

Assumption 1: RAN2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - For model inference of the UE-sided model, input data for model inference is available inside the UE. - For UE-side (real-time) monitoring of the UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW. Assumption 2: For the latency requirement of data collection, RAN2 assumes: - For all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - For model inference, when required data comes from other entities, there is a latency requirement for data collection - For (real-time) model monitoring, when required monitoring data (e.g., performance metric) comes from other entities, there is a latency requirement for data collection. Assumption 3: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed. Assumption 4: For the data generation entity and termination entity deployed at different entities, RAN2 made the following assumptions: - For CSI enhancement and beam management use cases: - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE. - For model monitoring at the NW side, performance metrics can be generated by UE and terminated at gNB. - For positioning enhancement use case: - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server. - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB. - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE. - For model monitoring at the NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

    - Assumption 1

Regarding Assumption 1:

- For model inference of the UE-sided model, input data for model inference is available inside the UE.

- For UE-side (real-time) monitoring of the UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW.

For the first bullet, the input data for model inference is generated and available inside UE, for model inference of UE-side model.

However, for the second bullet, there might be some different understanding. Firstly, in current RAN1 discussion, there hasn’t been a terminology about real-time monitoring.

Secondly, based on the following agreements, even for UE-side monitoring of the UE-sided model, if monitoring is based on system performance, including metrics related to system performance KPIs, it still possible that performance metrics can only be available inside NW.

Besides, if to assess/monitor an inactive model/functionality, there might be following several methods, all of which may not available inside the UE.

- Assessment/Monitoring based on the additional conditions associated with the model/functionality

- Assessment/Monitoring based on input/output data distribution

- Assessment/Monitoring using the inactive model/functionality for monitoring purpose and measuring the inference accuracy

- Assessment/Monitoring based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs)

Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE Agreement For the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities (if applicable), study necessity, feasibility and potential specification impact for methods to assess/monitor the applicability and expected performance of an inactive model/functionality, including the following examples: - Assessment/Monitoring based on the additional conditions associated with the model/functionality - Assessment/Monitoring based on input/output data distribution - Assessment/Monitoring using the inactive model/functionality for monitoring purpose and measuring the inference accuracy - Assessment/Monitoring based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs) FFS: Requirements for the assessment/monitoring to be reliable (e.g., sufficient data coverage during evaluation)

From our point of view, for the case of AI/ML positioning, we do have some sub scenarios which may not need RAN2 specification effect. For model inference of the UE-sided model, no strong motivation was identified to enhance the current RAN2 positioning signalling for the case 1(reporting type of target location). But for the case2a, because the reporting information from the UE to the LMF is PRS-based measurement together with some assistance information, it may need some further discussion. For UE-side (real-time) monitoring of the UE-sided model, although UE could generate performance metrics inside the UE itself, assistance information and signalling may be necessary for UE to request or calculate the performance metrics.

Proposal 1: Regarding Assumption 1, following modification is proposed:

RAN2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive):

- For model inference of the UE-sided model, input data for model inference is available inside the UE.

- For UE-side (real-time) monitoring of the UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW.

    - Assumption 2

Regarding assumption 2:

For the latency requirement of data collection, RAN2 assumes:

- For all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection.

- For model inference, when required data comes from other entities, there is a latency requirement for data collection.

- For (real-time) model monitoring, when required monitoring data (e.g., performance metric) comes from other entities, there is a latency requirement for data collection.

For offline model training, data collection is delay-insensitive, there is no latency requirement for data collection. The time stamp is necessary if different parts of the data are generated from different network entities.

For model inference, when required data comes from other entities, there is a latency requirement for data collection. For example, for BM-Case 2, data collection should be finished within T1 for prediction of best beam within T2. Regarding latency requirement for AI based positioning, the requirement in legacy positioning could be a starting point.

For model monitoring, same reason as Assumption 1, there is no clear definition on real-time model monitoring in RAN1. To determine model activation/deactivation/switch/fallback, performance metric is monitored within a period or within a timer. Thus, when required monitoring data (e.g., performance metric) comes from other entities, there is a latency requirement for data collection.

So, Assumption 2 is generally fine, except the “real-time” should be removed.

Proposal 2: Regarding Assumption 2, following modification is proposed:

For the latency requirement of data collection, RAN2 assumes:

- For all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection.

- For model inference, when required data comes from other entities, there is a latency requirement for data collection

- For (real-time) model monitoring, when required monitoring data (e.g., performance metric) comes from other entities, there is a latency requirement for data collection.

    - Assumption 3

Regarding assumption 3:

RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed.

The analysis/selection of the data collection frameworks for CSI enhancement, beam management use cases should focus on the RRC CONNECTED state (for both data generation and reporting), since current CSI report is not supported in idle and inactive mode.

However, considering NR positioning have already enabled DL PRS measurement in RRC_INACTIVE state, we think the analysis/selection of the data collection frameworks should not only focus on the RRC_CONNECTED state (for both data generation and reporting), but also on the RRC_INACTIVE state, at least for some sub cases which may not have too much RAN2 spec impact.

Proposal 3: Regarding Assumption 3, for CSI enhancement and beam management, it can be confirmed; while for AI/ML based positioning, both RRC_CONNECTED and RRC_INACTIVE state should be considered.

    - Assumption 4

Regarding assumption 4:

For the data generation entity and termination entity deployed at different entities, RAN2 made the following assumptions:

- For CSI enhancement and beam management use cases:

- For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server.

- For NW-sided model inference, input data can be generated by UE and terminated at gNB.

- For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE.

- For model monitoring at the NW side, performance metrics can be generated by UE and terminated at gNB.

- For positioning enhancement use case:

- For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server.

- For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB.

- For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE.

- For model monitoring at the NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

For CSI enhancement and beam management use cases:

For UE-side model inference, UE can also measure CSI or L1-RSRP, the input data can be generated by UE and used for inference. Besides, even if UE measures CSI or L1-RSRP by itself, but the assistance information can be generated by gNB and terminated at UE.

For model monitoring at the NW side, according to the following RAN1 agreements on NW-side model monitoring and hybrid model monitoring, the description about monitoring in assumption 4 may not be accurate. For hybrid model monitoring of UE-side model, performance metrics can be generated by UE and terminated at gNB. For NW-side model monitoring of UE-side model, UE reports measured and predicted data for the calculation of performance metric at gNB, and then the performance metrics are generated by gNB.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity: · Configuration/Signaling from gNB to UE for measurement and/or reporting · UE reporting to NW (e.g., for the calculation of performance metric) · Indication from NW for UE to do LCM operations · Other aspect(s) is not precluded · Note1: At least the performance and reporting overhead of model monitoring mechanism should be considered Agreement@113 hybrid monitor For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding performance monitoring, study potential spec impact(s) from the following aspects in addition to those included in previous agreements: · Configuration/Signalling from gNB to UE for measurement and/or reporting · UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) o FFS: definition of an event and the performance metric(s) used to identify it · Indication from NW for UE to do LCM operations

For positioning sub use case, the AI/ML model for positioning can be deployed in LMF, gNB and UE. The LMF, gNB and UE (or their OTT servers) would perform model training in some cases. Based on this understanding, we think the termination entity for training data should include all of these entities.

Proposal 4: Regarding Assumption 4, following modification is proposed:

For the data generation entity and termination entity deployed at different entities, RAN2 made the following assumptions:

- For CSI enhancement and beam management use cases:

- For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server.

- For NW-sided model inference, input data can be generated by UE and terminated at gNB.

- For UE-side model inference, input data/assistance information can be generated by UE/gNB and terminated at UE.

- For model monitoring at the NW side, performance metrics can be generated by UE/gNB and terminated at gNB.

- For positioning enhancement use case:

- For model training, training data can be generated by UE/gNB/LMF and terminated at gNB/LMF/OTT server.

- For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB.

- For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE.

- For model monitoring at the NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

  - Part B: Discussion on aspects of data collection that require RAN1 feedback/inputs

In this subsection, we discuss the following essential aspects of data collection per LCM purpose (i.e., model training, inference and monitoring) per LCM sidedness for each (sub) use case:

- Data content

- Typical data size (value or value range) of the identified data content

- Reporting type (e.g., periodic, event triggered, other) of the identified data content

- Typical latency requirement (value or value range) to transfer the identified data content

To facilitate the discussion on data collection in RAN2 for further progress, RAN2 would like RAN1 to provide feedback/inputs on the following essential aspects: - Data content - Typical data size (value or value range) of the identified data content - Reporting type (e.g., periodic, event triggered, other) of the identified data content - Typical latency requirement (value or value range) to transfer the identified data content RAN2 would require RAN1 feedback/inputs on the data collection requirements per LCM purpose (i.e., model training, inference and monitoring) for each (sub)use case, and the LCM sidedness should also be considered. Besides, RAN2 would also like to know to what extent the data would / should be specified (in detail).

    - CSI enhancement

In previous meetings, we have the following agreements related with data collection for CSI enhancement:

Agreement - In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection - In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection Agreement In CSI compression using two-sided model use case, further study the necessity and potential specification impact of the following aspects related to the ground truth CSI format for NW side data collection for model training:  Scalar quantization for ground-truth CSI  FFS: any processing applied to the ground-truth CSI before scalar quantization, based on evaluation results in 9.2.2.1  Codebook-based quantization for ground-truth CSI  FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - Number of layers for which the ground truth data is collected. And whether UE or NW determine the number of layers for ground-truth CSI data collection. Agreement In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring, including: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - RRC signaling and/or L1 signaling procedure to enable fast identification of AI/ML model performance. - Aperiodic/semi-persistent or periodic ground-truth CSI report.

Data collection for training

Based on the above agreements in previous meetings, for AI based CSI compression, the possible data content for training could include precoding matrix, channel matrix, and some assistance information (e.g., time stamps, and/or cell ID, Assistance information for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator)

Proposal 5: For AI based CSI compression, the possible data content for training could include precoding matrix, channel matrix, and some assistance information (e.g., time stamps, and/or cell ID, Assistance information for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator).

As for the Typical data size, the typical number of training samples during the evaluation in agenda 9.2.2.1 is 200k. And the one sample will consist of the raw channels of 13 subbands, each raw channel will be a matrix with the dimension of 4*32 (the number of Tx ports =32 and the number of Rx ports =4), each element of one raw channel will be a complex with both real part and imaginary part stored in float32. So, the typical data size for training could be 200k * 13 *4*32*2*32 ≈2*1010bits.

Proposal 6: For AI based CSI compression, the typical data size for training could be 1010 bits.

Data collection for inference

Same as data for training, for AI based CSI compression, the possible data content for inference could include precoding matrix, channel matrix, and some assistance information (e.g., time stamps, and/or cell ID, Assistance information for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator), but some of them may be up to implementation, e.g., the input data for model could be precoding matrix or channel matrix, with or without pre-process.

Proposal 7: For AI based CSI compression, the possible data content for inference could include precoding matrix, channel matrix, and some assistance information (e.g., time stamps, and/or cell ID, Assistance information for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator).

In general, the data type for inference could be similar with data for training, while maybe only one sample is needed for inference. So, the typical data size for inference could be 13*4*32*2*32 ≈104~105bits.

Proposal 8: For AI based CSI compression, the typical data size for inference could be 104 bits.

Data collection for monitoring

Based on current agreements, the ground-truth CSI can be used for model monitoring. If the ground-truth CSI is processed with scalar quantization, then the typical data size could be similar to inference data, i.e., 105 bits; while if the ground-truth CSI is processed with codebook-based quantization, then the typical data size could be similar to a normal Type-II codebook-based CSI, i.e., 102 bits. And aperiodic/semi-persistent or periodic ground-truth CSI report could be studied in current SI.

Proposal 9: For AI based CSI compression, the possible data content for monitoring could include ground-truth CSI.

Proposal 10: For AI based CSI compression, the typical data size for monitoring could be 102 ~104 bits.

Proposal 11: For AI based CSI compression, aperiodic/semi-persistent or periodic ground-truth CSI could be possible for model monitoring.

    - Beam management

Data collection for training

In RAN1#112b, the agreement regarding to content of data collection for NW-side AI/ML model has been approved, where the content can be L1-RSRPs of M2 beams, indication of M3 beams and combination of these options.

Agreement Regarding data collection for NW-side AI/ML model, study the following options (including the combination of options) for the contents of collected data, - Opt.1: M1 L1-RSRPs (corresponding to M1 beams) with the indication of beams (beam pairs) based on the measurement corresponding to a beam set, where M1 can be larger than 4, if applicable - FFS: the range of M1 - Opt.2: M2 L1-RSRPs (corresponding to M2 beams) based on the measurement corresponding to a beam set, where M2 can be larger than 4, if applicable - FFS: the range of M2 - Opt.3: M3 beam (beam pair) indices based on the measurement corresponding to a beam set, where M3 can be larger than 4, if applicable - FFS: the range of M3 - FFS: How to select the M1/M2/M3 beam(s) or beam pair(s) - Note: Overhead, UE complexity and power consumption should be considered for the above options

Based on another agreement in RAN1#112b, content of data collection also includes additional information, where the detail of additional information associated with the reported data samples, e.g., timestamps, SNR, data quality, is still FFS.

Agreement Regarding data collection for NW-side AI/ML model, study necessity, benefits and beam-management-specific potential specification impact from RAN1 point of view on the following additional aspects - Mechanism related to the reporting - Additional information for content of the reporting - FFS: Information associated with or configured for the reported data samples, e.g., timestamps, SNR, data quality, etc. - Reporting overhead reduction - Note1: non-3GPP based solution is a separate issue. - Note2: The framework corresponding to higher layer(s) are up to the associated WG(s) - Note 3: Overhead, UE complexity and power consumption should be considered

For UE-side training, the content of data collection is similar to NW-side training. Besides, in RAN1#113 meeting, assistance information for the purpose of differentiating characteristics of data (e.g. Tx antenna configuration ID) is agreed as content of UE-side data collection. This assistance information can be used to achieve better generalization performance, such as training different models for different data categories.

Agreement Regarding data collection for BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the benefits, necessity and potential specification impact of the following aspect on top of those we have agreed in previous meeting: - Assistance information from NW to UE for UE data collection for categorizing the data for the purpose of differentiating characteristics of data. - The assistance information should preserve privacy/proprietary information.

Proposal 12: For AI based beam management, the data content for training could include L1-RSRP, indication of beams, and some assistance information (e.g., timestamps, SNR, data quality, Tx antenna configuration ID, etc.).

If one RSRP is quantified with 4 bits and total 256 beam pairs are considered, typical data size of one sample is about one thousand bits. Typical sample number for training is tens of thousands, then typical data size of training dataset is tens of million bits.

Proposal 13: For AI based beam management, typical data size for training is 107 bits.

Reporting type of training data is probably periodic or semi-persistent since the number of training data is large and periodicity of time instances for measurement is the same. periodic or semi-persistent reporting enables continuous and equal interval data collection with less signaling overhead.

Proposal 14: For AI based beam management, reporting type of training data can be periodic or semi-persistent reporting.

Data collection for inference

For NW-side inference, the content of data collection includes L1-RSRPs of beams in set B and additional information such as timestamps, SNR, data quality. For UE-side inference, the content of data collection is similar to NW-side inference. Proposal 15: For AI based beam management, the data content for inference could include L1-RSRP of beams, and some assistance information (e.g., timestamps, SNR, data quality, Tx antenna configuration ID, etc.).

If one RSRP is quantified with 4 bits and 1/4 of 256 beam pairs are measured, typical data size of the data content is about two hundred bits. Reporting type of inference data can be periodic or semi-persistent.

Proposal 16: For AI based beam management, typical data size for inference is 102 bits.

Proposal 17: For AI based beam management, reporting type of inference data can be periodic or semi-persistent reporting.

Data collection for monitoring

For NW-side monitoring of NW-side model, NW collects not only measurement results but also predicted results, the content of data collection is similar to NW-side training, then NW calculates the performance metric based on these data.

Proposal 18: For AI based beam management, the data content for monitoring could include L1-RSRP, indication of beams, and some assistance information (e.g., timestamps, SNR, data quality, Tx antenna configuration ID, etc.).

Similar to NW-side training, typical data size of one sample is about one thousand bits. Typical sample number for monitoring depends on monitoring KPI. If beam prediction accuracy is KPI, monitoring sample number is several tens, then typical data size of monitoring dataset is tens of thousand bits. Reporting type of monitoring data can be periodic, semi-persistent or event triggered. If an event is defined for triggering model monitoring, when the event happens, UE reports data during a period or within a timer.

Proposal 19: For AI based beam management, typical data size for monitoring is 104 bits.

Proposal 20: For AI based beam management, reporting type of monitoring data can be periodic, semi-persistent or event triggered.

    - Positioning enhancement

To facilitate the AI/ML in real world, it is important to carefully analyse the system payload. During last RAN1 meeting, the effect for the training dataset size was discussed, and the following observation was made.

Observation For AI/ML based positioning, the positioning accuracy is affected by the training dataset size for a given UE distribution area (or equivalently, sample density in #samples/m2), when the UE is distributed uniformly in training data collection. - There exists a tradeoff between the training dataset size and the achievable positioning accuracy. The larger the training dataset size (i.e., higher sample density), the smaller the positioning error (in meters), until a saturation point is reached where additional training data does not bring further improvement to the positioning accuracy. - Note: here a sample refers to the training data collected of one UE at one location. Sample density is equivalent to the density of UEs with data collected in the training dataset.

For the AI/ML based positioning, the more information you put into, the higher accuracy you could expect. But the trade-off between the dataset size and the performance together with the feasibility should be considered. During the SI, CIR is very popular for the input of evaluating the AI/ML based positioning. In a typical scenario, considered the following parameters:

N'TRP = NTRP = 18, Nport = 2, Nt = 256, Breal,CIR = 32

Each one of the samples for model input could be about 73.7 KB, where the size is calculated by:

N'TRP * Nport * Nt * 2 * Breal,CIR

If the train dataset is generated or stored at one network entity and would be transmitted to another network entity for model training, this is really a large air interface payload.

From our point of view, the data content could be classified into two kinds of groups. One group includes the measurement quantities existing in the current NR positioning method, including TDOA, AOA, RTT and so on. The other one contains measurement which haven’t been used in current NR positioning, like CIR, PDP, or anything else.

Proposal 21: For the essential aspects of data collection of positioning use case:

- For the existing NR positioning measurements:

  - RAN1 should study the performance and the feasibility.

- For the new measurements for AI based positioning:

  - RAN1 should study the performance and the feasibility.

- Conclusion

In this contribution, we provide our views on data collection requirements and assumptions for Rel-18 AI for Air Interface. The proposals are summarized as follows:

Proposal 1: Regarding Assumption 1, following modification is proposed:

RAN2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive):

- For model inference of the UE-sided model, input data for model inference is available inside the UE.

- For UE-side (real-time) monitoring of the UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW.

Proposal 2: Regarding Assumption 2, following modification is proposed:

For the latency requirement of data collection, RAN2 assumes:

- For all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection.

- For model inference, when required data comes from other entities, there is a latency requirement for data collection

- For (real-time) model monitoring, when required monitoring data (e.g., performance metric) comes from other entities, there is a latency requirement for data collection.

Proposal 3: Regarding Assumption 3, for CSI enhancement and beam management, it can be confirmed; while for AI/ML based positioning, both RRC_CONNECTED and RRC_INACTIVE state should be considered.

Proposal 4: Regarding Assumption 4, following modification is proposed:

For the data generation entity and termination entity deployed at different entities, RAN2 made the following assumptions:

- For CSI enhancement and beam management use cases:

- For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server.

- For NW-sided model inference, input data can be generated by UE and terminated at gNB.

- For UE-side model inference, input data/assistance information can be generated by UE/gNB and terminated at UE.

- For model monitoring at the NW side, performance metrics can be generated by UE/gNB and terminated at gNB.

- For positioning enhancement use case:

- For model training, training data can be generated by UE/gNB/LMF and terminated at gNB/LMF/OTT server.

- For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB.

- For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE.

- For model monitoring at the NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

Proposal 5: For AI based CSI compression, the possible data content for training could include precoding matrix, channel matrix, and some assistance information (e.g., time stamps, and/or cell ID, Assistance information for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator).

Proposal 6: For AI based CSI compression, the typical data size for training could be 1010 bits.

Proposal 7: For AI based CSI compression, the possible data content for inference could include precoding matrix, channel matrix, and some assistance information (e.g., time stamps, and/or cell ID, Assistance information for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator).

Proposal 8: For AI based CSI compression, the typical data size for inference could be 104 bits.

Proposal 9: For AI based CSI compression, the possible data content for monitoring could include ground-truth CSI.

Proposal 10: For AI based CSI compression, the typical data size for monitoring could be 102 ~104 bits.

Proposal 11: For AI based CSI compression, aperiodic/semi-persistent or periodic ground-truth CSI could be possible for model monitoring.

Proposal 12: For AI based beam management, the data content for training could include L1-RSRP, indication of beams, and some assistance information (e.g., timestamps, SNR, data quality, Tx antenna configuration ID, etc.).

Proposal 13: For AI based beam management, typical data size for training is 107 bits.

Proposal 14: For AI based beam management, reporting type of training data can be periodic or semi-persistent reporting.

Proposal 15: For AI based beam management, the data content for inference could include L1-RSRP of beams, and some assistance information (e.g., timestamps, SNR, data quality, Tx antenna configuration ID, etc.).

Proposal 16: For AI based beam management, typical data size for inference is 102 bits.

Proposal 17: For AI based beam management, reporting type of inference data can be periodic or semi-persistent reporting.

Proposal 18: For AI based beam management, the data content for monitoring could include L1-RSRP, indication of beams, and some assistance information (e.g., timestamps, SNR, data quality, Tx antenna configuration ID, etc.).

Proposal 19: For AI based beam management, typical data size for monitoring is 104 bits.

Proposal 20: For AI based beam management, reporting type of monitoring data can be periodic, semi-persistent or event triggered.

Proposal 21: For the essential aspects of data collection of positioning use case:

- For the existing NR positioning measurements:

  - RAN1 should study the performance and the feasibility.

- For the new measurements for AI based positioning:

  - RAN1 should study the performance and the feasibility.

- References

- R2-2306906, LS out on Data Collection Requirements and Assumptions, TSG-RAN WG2 Meeting #122