3GPP TSG RAN WG1 #114			R1-2307204

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.5.4

Source: 	CMCC

Title: 	Discussion on BW aggregation for positioning measurements

Document for:	Discussion and Decision

Introduction

In RAN#98-e meeting, a new WID on expanded and improved NR positioning was approved [1], in which the application of NR positioning is further expanded to enable sidelink positioning and ranging, improved accuracy, integrity and power efficiency, and positioning for RedCap UEs. To further improve positioning accuracy, the following objectives on bandwidth aggregation were identified:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

In this contribution, we further provide our views on the remaining issues of PRS/SRS bandwidth aggregation for positioning measurements based on the outcomes of previous RAN1 meetings [2].

PRS bandwidth aggregation

- 2.1 Common transmission properties

In RAN1#113 meeting, the following agreement on common transmission properties has been made:

Agreement For PRS bandwidth aggregation between PRS in two or three different PFLs, the following are needed for the aggregated PRS resources for a TRP: - The same periodicity and slot offset - The same muting pattern - The same NR-DL-PRS-SFN0-Offset value - UE expects to be configured with PRS resources that maintain a per-symbol uniformly spaced PRS pattern across aggregated bandwidths in frequency domain (Note: It does not preclude dropping some REs in the guardband between two PFLs). - FFS same antenna port from RAN1 perspective

Based on the agreement, there was a remaining issue that whether same antenna port from RAN1 perspective. This bullet has been intensively discussed for several meetings, but no consensus was achieved. To our understanding, the same antenna port represents that the two or three DL PRS resources from different PFLs pass through the same channel from the same antenna. Therefore, we support to have this condition.

In addition, based on the following agreement in RAN1#112bis-e meeting, it is still open that whether same number of PRS resource sets and/or resources per set for a TRP is needed:

Agreement (RAN1#112bis-e) For PRS bandwidth aggregation between PRS in two or three different PFLs, decide whether one or more of the following are needed for the aggregated PRS resources from a TRP in RAN1#113 meeting: - The same antenna port from RAN1 perspective - Note: this is to achieve phase continuity between PFLs - The same periodicity and slot offset - The same muting pattern - The same number of PRS resource sets and/or resources per set for a TRP - The same NR-DL-PRS-SFN0-Offset value - UE is expected to be configured with PRS resources that maintain a per-symbol uniformly spaced PRS pattern across aggregated bandwidths - FFS: a per-symbol uniformly spaced PRS pattern across aggregated bandwidths does not preclude dropping some REs in the guardband between two PFLs - Others if any

In the last RAN1 meeting, it has been agreed that the linkage of aggregated DL PRS configurations across PFLs is per PRS resource set basis, i.e., the linkage signalling will indicate which DL PRS resource sets are linked per a TRP. Therefore, we think that configuration flexibility can be allowed for the number of PRS resource sets for a TRP. As illustrated in the following figure, we can link resource set #1 and resource set #2 of a TRP, while resource set #3 is not aggregated.

Figure 1. Illustration of common transmission properties for DL PRS BW aggregation

Regarding the number of DL PRS resources per a set, on the other hand, the benefit and necessity to introduce configuration flexibility are not clear, and we prefer to have the same number of DL PRS resources per a set for a TRP.

Proposal 1: To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be further satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs:

- The same number of PRS resources per linked resource set for a TRP

- The same antenna port from RAN1 perspective

In addition, regarding whether same gNB Tx or UE Rx TEG ID in two or three different PFLs should be satisfied, we don’t think it needs to be considered for DL PRS BW aggregation. In Rel-17 positioning, the intention to introduce Tx/Rx TEG ID in assistance data/measurement reporting is to mitigate the Tx/Rx timing errors caused by multi-panel devices; on the other hand, by providing configurations or reporting measurements with TEG ID, it indicates that two different resources/measurements in a PFL associated with the same TEG ID shares the same Rx/Tx timing error margins, but it may not be the same thing for DL PRS resources from different PFLs. Basically, we believe that if conditions are satisfied for DL PRS BW aggregation, it should have a more stringent requirement, and therefore, no need to consider same TEG ID for the aggregated PFLs.

Proposal 2: No need to consider same gNB Tx TEG ID or UE Rx ID across PRS resources in two or three aggregated PFLs.

- 2.2 Measurement reporting

In the last RAN1 meeting, the following agreement was reached on measurement reporting:

Agreement For PRS bandwidth aggregation across PFLs, in a measurement report element, support - Single RSRP or single RSRPP - FFS: the single RSRP/RSRPP is based on aggregated PRS resources across aggregated PFLs - The aggregated reference RSTD - The used PRS resource set IDs for the aggregated measurement which are shared for RSRP/RSRPP and/or timing measurement results

It was supported that single RSRP or single RSRPP are included in a measurement report, but an remaining open issue was to further study whether the single RSRP/RSRPP is based on aggregated PRS resource across aggregated PFLs. From our perspective, we don’t have strong views, but our initial thinking is that the benefit is not clear to mandate a UE reporting a single RSRP/RSRPP based on aggregated DL PRS resources across aggregated PFLs. Basically, the BW aggregation of DL PRS resources are to improve the positioning accuracy of timing-based measurements, including RSTD and Rx-Tx time difference, as the granularity in the time domain can be increase multiple times if two or three PFLs are aggregated. Therefore, it seems not necessary to perform RSRP/RSRPP measurements on the aggregated PFLs. In addition, it is also not clear what is the benefits from the NW side to know whether the RSRP/RSRPP measurement is performed based on aggregated PRS resources. Perhaps it can be up to UE implementation to perform RSRP/RSRPP measurement based on single or aggregated PRS resource(s).

Proposal 3: In a measurement report, for the reported single RSRP/RSRPP, support that:

- Up to UE implementation to perform the RSRPP/RSRPP measurement based on single or aggregated PRS resource(s).

- 2.3 Collision rule

In RAN1#112bis-e meeting, the following agreement was reached on the dropping rule of aggregated DL PRS resources across PFLs:

Agreement For the case when PRS in one of aggregated PFL is dropped, e.g. because of collision with SSB, select one of the following solutions for LMF based positioning - Alt. 1: Drop positioning measurement in all aggregated PFLs in the same symbol(s) - Alt. 2: Still perform positioning measurement based on the remaining PRSs in other PFL(s) - FFS the details and the difference between MG and PPW if PPW is supported - Note: Up to RAN4 to discuss impact on requirements, if any, for such cases

Note that in the last RAN1 meeting, it has already concluded that PPW is not supported in Rel-18, and therefore, for UEs in RRC_CONNECTED state, only MG is considered. In addition, during the discussion in the last meeting, companies prefer to include both RRC_CONNECTED and RRC_INACTIVE states, and we will provide our views on both RRC states.

First, our preference is to have same dropping rule for both RRC_CONNECTED and RRC_INACTIVE states, we don’t see the necessity to differentiate the two for UE behaviour of measuring aggregated DL PRS resources. In addition, for the two alternatives, we slightly prefer Alt. 1. For us, BW aggregation is to improve the positioning accuracy, and if any one of the aggregated DL PRS resources are dropped due to collision of other DL signals/channels, the benefits for aggregation will be lost and hence, the advantages of still performing positioning measurements are not clear. On the other hand, as we prefer to adopt the same rule for both RRC states, in Rel-17, we have specified that DL PRS has lower priority than other DL signals/channels, and therefore, we should drop DL PRS if collision occurs.

Proposal 4: For the case when PRS in one of aggregated PFL is dropped for UEs in RRC_CONNECETD and RRC_IDLE states, support the following:

- Alt. 1: Drop positioning measurement in all aggregated PFLs in the same symbol(s)

SRS bandwidth aggregation

- 3.1 Common transmission properties

On the common transmission properties for SRS BW aggregation, the following agreement was made in RAN1#113 meeting:

Agreement For SRS bandwidth aggregation between SRS in two or three carriers, the following is needed for the aggregated SRS resources - The same periodicityAndOffset, and slotOffset - The configuration of pathloss RS, Po and alpha to ensure the same Tx PSD (power per subcarrier) - The same configuration of Po and alpha. - Note: UE may either perform pathloss RS measurement across CCs and form a single path loss value to apply across CCs or perform pathloss RS measurement in a single CC and apply across CCs

Similar as the discussion in Section 2.1, we propose that same number of SRS resources for a linked SRS resource set should be supported for SRS bandwidth aggregation.

Proposal 5: To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be further satisfied for the aggregated SRS resources across the aggregated carriers:

- The same number of SRS resources per a linked SRS resource set

Regarding the same TEG ID, we have the same views as provided in Section 2.1:

Proposal 6: No need to consider same UE Tx TEG ID or gNB Rx ID across PRS resources in two or three aggregated PFLs.

- 3.2 Measurement reporting

In the last RAN1 meeting, the following agreement was reached on measurement reporting for SRS BW aggregation:

- Agreement For SRS bandwidth aggregation across carriers, support - Single RSRP or RSRPP is reported - FFS: the single RSRP/RSRPP is based on aggregated SRS resources across aggregated carriers - The used SRS resource IDs for the aggregated measurement are shared for RSRP/RSRPP and/or timing measurement results

We have similar views as that presented in Section 2.2, where gNB can indicate whether the single RSRP/RSRPP is based on aggregated SRS resources.

Proposal 7: In a measurement report, for the reported single RSRP/RSRPP for SRS BW aggregation, support that:

- Up to gNB implementation to perform the RSRPP/RSRPP measurement based on single or aggregated SRS resource(s).

- 3.3 Collision rule

In the last RAN1 meeting, on the collision rule of aggregated SRS resources, the agreement on SRS transmissions in RRC_INACTIVE state has been made:

- Agreement For positioning SRS aggregation transmission in RRC_INACTIVE state, reuse Rel-17 prioritization rule of SRS outside initial BWP, i.e. SRS is dropped in the symbol(s) of all aggregated carriers where collision occurs.

For the SRS transmissions in RRC_CONNECTED state, companies views were diverse during the discussion in the last RAN1 meeting and therefore no consensus was made. In our views, similar as the dropping rule of DL PRS, we prefer to have the same collision rule for both RRC states. If we go with Alt. 2, i.e., SRS is still transmitted in other carriers in the same symbol, it should be noted that it would be hard for a UE to maintain the phase continuity of the SRS, and hence the performance cannot be ensured.

Proposal 8: For positioning SRS aggregation transmission in RRC_CONNECTED states, support the following:

- Alt. 1: Stop SRS transmission in all aggregated carriers in the same symbol.

Conclusions

In this contribution, we provide our views on BW aggregation of DL PRS and UL SRS, and the following proposals are made:

Proposal 1: To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be further satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs:

- The same number of PRS resources per linked resource set for a TRP

- The same antenna port from RAN1 perspective

Proposal 2: No need to consider same gNB Tx TEG ID or UE Rx ID across PRS resources in two or three aggregated PFLs.

Proposal 3: In a measurement report, for the reported single RSRP/RSRPP, support that:

- Up to UE implementation to perform the RSRPP/RSRPP measurement based on single or aggregated PRS resource(s)

Proposal 4: For the case when PRS in one of aggregated PFL is dropped for UEs in RRC_CONNECETD and RRC_IDLE states, support the following:

- Alt. 1: Drop positioning measurement in all aggregated PFLs in the same symbol(s)

Proposal 5: To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be further satisfied for the aggregated SRS resources across the aggregated carriers:

- The same number of SRS resources per a linked SRS resource set

Proposal 6: No need to consider same UE Tx TEG ID or gNB Rx ID across PRS resources in two or three aggregated PFLs.

Proposal 7: In a measurement report, for the reported single RSRP/RSRPP for SRS BW aggregation, support that:

- Up to gNB implementation to perform the RSRPP/RSRPP measurement based on single or aggregated SRS resource(s).

Proposal 8: For positioning SRS aggregation transmission in RRC_CONNECTED states, support the following:

- Alt. 1: Stop SRS transmission in all aggregated carriers in the same symbol.

References

- RP-223549, New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson, 3GPP TSG RAN Meeting 98-e.

- R1-2306238, RAN1 agreements for Rel-18 WI on Expanded and Improved NR Positioning, Rapporteur (Intel Corporation), 3GPP TSG RAN WG1 Meeting #113.