3GPP TSG RAN WG1 #114                                       	                                                                              R1-2307367

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.16.16

Source:	Xiaomi

Title:                        UE features for endorsed Rel-18 TEI on pathloss RS for Type 1 CG-PUSCH

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the UE feature for endorsed Rel-18 TEI on pathloss RS for Type 1 CG-PUSCH.

Discussion

In Rel-16 [1][2], the UE feature of MAC CE based pathloss RS updates for PUSCH/SRS was introduced only for Type 2 CG and dynamic grant PUSCH, but not for Type 1 CG PUSCH.

| maxNumberPathlossRS-update-r16 Indicates the maximum number of configured pathloss reference RSs for PUSCH/PUCCH/SRS by RRC that the UE can support for MAC-CE based pathloss reference RS update. | UE | No | No | No |
| --- | --- | --- | --- | --- |

| 16-1e | Pathloss reference RS activation via MAC CE | 1. The maximum number of configured pathloss reference RSs for PUSCH/PUCCH/SRS by RRC for MAC-CE based pathloss reference RS update | 8-3 | maxNumberPathlossRS-Update-r16 | Phy-ParametersCommon | No | No | Candidate values for component (1): {4, 8, 16, 32, 64} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

According to the agreement in RAN1-113 meeting [3], MAC CE based pathloss RS updated for Type 1 CG PUSCH will be supported. Hence, we propose to support the corresponding UE capability in Table 1.

Agreement

Rel-18 TEI proposal on pathloss RS for Type 1 CG-PUSCH is agreed. Relevant TP for clause 7.1.1 in TS 38.213 is endorsed in principle

- Note: Corresponding UE capability and RRC configuration will be introduced and discussed in future meetings.

7.1.1 UE behaviour …… <Unchanged parts are omitted> - For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, - if the UE is provided [enablePL-RS-UpdateForType1CG-PUSCH-SRS], the UE determines a RS resource index qd from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the sri-PUSCH-PowerControlId indicated by the srs-ResourceIndicator value included in rrc-ConfiguredUplinkGrant - if the UE is not provided [enablePL-RS-UpdateForType1CG-PUSCH-SRS], a RS resource index qd is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking - …… <Unchanged parts are omitted>

Table 1, Proposed FG for pathloss RS for Type 1 CG-PUSCH

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. TEI18 | 55-x | Enable MAC CE based pathloss RS updates for Type 1 CG-PUSCH | Support configuration of enablePL-RS-UpdateForType1CG-PUSCH-SRS-r18 | 16-1e | Yes | N/A | MAC CE based pathloss RS updates for Type 1 CG-PUSCH is not supported | Per UE | No | No |  |  | Optional with capability signalling |

Proposal 1: capture the UE capability in Table 1 for pathloss RS for Type 1 CG-PUSCH.

Conclusion

Based on the above discussion on pathloss RS for Type 1 CG PUSCH, we propose the following proposal:

Proposal 1: capture the UE capability in Table 1 for pathloss RS for Type 1 CG-PUSCH.

References

- 3GPP TS 38.306, “User Equipment (UE) radio access capabilities (Release 17),” v.17.4.0, 2023-03.

- 3GPP TR 38.822, “User Equipment (UE) feature list (Release 17),” v.17.0.0, 2023-03.

- RAN1-113 meeting Chairman notes.