3GPP TSG RAN WG1 #114			R1- 2307594

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.1.1.1

Source: 	Moderator (MediaTek. Inc)

Title: 	Summary of pre-meeting offline discussion on extension of unified TCI framework

Document for:	Discussion

For better progress and convergence in RAN1#114, this pre-meeting offline is triggered for the remaining issues from UL PC for UL MTRP operation (Issue 4). I'd appreciate it if you can share your views and comments on these issues and corresponding proposals in this pre-meeting offline. I will keep updating this summary based on your input, and recommended proposals may be provided.

This offline discussion will be closed @Wednesday 9th August, 11:59 pm UTC.

- Discussion

Issue 4 – UL power control for UL MTRP operation

Table 4-1 Summary for Issue 4

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 4.1 | UL PC for S-DCI based STxMP (including SDM/SFN based PUSCH Tx and SFN based PUCCH Tx) | FL note: Based on the following draft CR for UL PC [1], the UE shall determine Tx power for each indicated TCI state by applying the UL PC parameters provided in each indicated TCI state. For the case if there are two indicated TCI states applied to one PUSCH/PUCCH occasion (e.g., for STxMP), the UE shall determine two Tx power from the two indicated TCI states, each based on the procedures in clauses 7.1.1 or 7.2.1, respectively. Draft CR for TS 38.213 – Session 7 Uplink Power control [1] In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, and for each indicated one or two TCI-State or TCI-UL-State of a PUSCH, PUCCH, or SRS transmission occasion as described in [6, TS 38.214] - in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS - in clause 7.1.1, if p0AlphaSetforPUSCH is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State - in clause 7.2.1, if p0AlphaSetforPUCCH is provided, the values of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH associated with the indicated TCI-State or TCI-UL-State - in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set. I’d like to check the group on the following questions: Question 1: If we go with the draft CR for UL PC [1], is it correct understating that a same PCMAX,f,c(i) would be used for determining Tx power on each panel for S-DCI based STxMP? Question 2: Whether the draft CR for UL PC [1] is sufficient to support S-DCI based STxMP? - No to Q1 and Q2. RAN1 specification should reflect at least the RAN4 agreement [2] on “per-panel” configured Tx power (PCMAX) for STxMP - Google, OPPO, QC, Docomo, vivo, Xiaomi, Samsung, CMCC, ZTE, Nokia, Sharp, CATT, Lenovo, Apple, LG, Huawei/HiSilicon, NEC, Panasonic, Intel, IDC, FGI Question 3: If the answer to Question 2 is No, what additional spec change is needed to support S-DCI based STxMP? - “Per-panel” configured Tx power (PCMAX) for STxMP - Google, OPPO, QC, Docomo, vivo, Xiaomi, Samsung, CMCC, ZTE, Nokia, Sharp, CATT, Lenovo, Apple, LG, NEC, Intel, Huawei/HiSilicon, IDC, Panasonic - “Per-panel” UL PC equation - QC, Nokia, Docomo, Lenovo, NEC, Huawei/HiSilicon, Panasonic - Tx power reduction/scaling/prioritization if total power exceeds a limitation (e.g., per-UE total power limitation) on a CC or all CCs - Google, QC, ZTE, CATT, Apple, LG, Huawei/HiSilicon, Intel |
| 4.2 | Type 1 Power headroom report for Rel-18 PUSCH STxMP (including both S-DCI and M-DCI based STxMP) and Rel-17 PUSCH repetition | FL note: Based on the contributions and discussions in RAN1#113, several companies propose to enhance Type 1 PHR at least from the following aspects: - The determination rule of actual/virtual PHR for M-TRP PHR (twoPHRMode) should be modified if it is used for S-DCI based PUSCH STxMP - When M-TRP PHR (twoPHRMode) is to be reported and any one of them is based on a reference PUSCH transmission, the UE shall use the UL PC parameters included in one of the indicated joint/UL-TCI states instead of the default UL PC parameters to determine the virtual PHR. This is applicable for both Rel-17 PUSCH repetition and Rel-18 PUSCH STxMP. Question 1: Whether any enhancement to Type 1 PHR is needed under unified TCI framework extension? Note that issues other than above examples are not precluded. Question 2: If the answer to Question 1 is Yes, please clarify the issue that need to be addressed, especially if it’s not captured by above sub-bullets. - Yes, enhancement to Type 1 PHR is needed - To support “panel-specific” report for STxMP: Google, OPPO, QC, Docomo, Spreadtrum, vivo, Xiaomi, CMCC, ZTE, Sharp, Lenovo, LG, Apple, Huawei/HiSilicon, NEC, Intel, IDC, FGI - To support “panel/TRP-specific” triggering: Apple, Spreadtrum - To support M-DCI based PUSCH+PUSCH STxMP: Google, vivo - No, enhancement to Type 1 PHR is not needed - Samsung |
| 4.4 | How to determine the UL PC parameter setting(s) if one or both indicated joint/UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH | FL note: The following issue has been on the table for several meetings. It would be good if we can conclude it in this meeting. Question 1: If one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUSCH/PUCCH/SRS, which alternative from the followings should be adopted? - Alt1: Support a first and a second UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated, i.e., if the first/second indicated joint/UL TCI state doesn’t include an UL PC parameter setting, the UE should apply the first/second UL PC parameter setting for PUSCH/PUCCH/SRS configured in the corresponding UL BWP - Alt2: No change from Rel-17 unified TCI framework, i.e., if any of the indicated joint/UL TCI states doesn’t include an UL PC parameter setting, the UE shall apply the one single UL PC parameter setting for PUSCH/PUCCH/SRS configured in the corresponding UL BWP - Support Alt1 - OPPO, QC, Docomo, Spreadtrum, vivo, Xiaomi, CMCC, ZTE, Sharp, CATT, Lenovo, Apple, LG, Huawei/HiSilicon, Panasonic, Intel, IDC, FGI - Support Atl2 - Google, Samsung |

Table 5-1 Company inputs for other issue

| Company | Input |
| --- | --- |
| Google | 4.1 Q1: No, this still depends on RAN4’s definition of Pcmax. RAN4 has agreed to define per-panel Pcmax as shown in approved WF in R4-2310268. Q2: No. Q3: Several aspects need to be discussed: - Power sharing across panels. Whether the Pcmax for each panel should be defined in a semi-static manner or a more dynamic manner. - Power scaling. Current power scaling is defined in a per-UE manner, but it should be done in a per-panel manner. 4.2 Q1: Yes. Q2: How to calculate the PH for one PHR and two PHR mode with regard to per-panel Pcmax should be discussed. PHR report for multi-DCI based PUSCH also needs to be discussed. Multi-DCI based PUSCH is a new mode introduced in Rel-18. Currently only PHR for single-DCI and sTRP operation has been defined. 4.3: Alt2 could be sufficient. The pathloss RS could be different for different beams, which is mandatorily configured. UL PC parameters other than pathloss RS do not have to be TRP-specific. |
| OPPO | Issue 4.1 Q1: No. From the CR only, we don’t see a same Pcmax would be used for two panels. Q2: No. If RAN4 completes the per-panel Pcmax, RAN1 specification should be changed accordingly to reflect the per-panel Pcmax. Q3: At least two per-panel Pcmax should be specified in RAN1 specification. Issue 4.2 Q1: Yes. Q2: Since the PHR is calculated based on Pcmax, if per panel Pcmax supported in RAN1 too, then PHR should be per panel as well. Issue 4.3 Q1: Support Alt1. It seems necessary and not harmful to support two default UL PC parameter settings for MTRP. |
| QC | For 4.1 - Q1: No. As mentioned by Google, RAN4 has agreed to define per-panel Pcmax. The remaining options to define per-panel in RAN4 seem not critical to RAN1’s discussion - Q2: No - Q3: For per-panel Pcmax, the following issues may need further clarifications, e.g. to use per-panel Pcmax, instead of legacy Pcmax - Per-panel UL PC equation - Per-panel based power prioritization across CCs - Per-panel based PHR For 4.2 - Q1: Yes - Q2: Per-panel PHR can also be studied For 4.4 - Q1: Support Alt1. Two sets of PC parameters are already supported in R17 mTRP PUCCH/PUSCH. For example, different P0 can better handle different noise level and sensitivity requirements at two TRPs. |
| NTT Docomo | 4.1 Q1: No. With draft CR, it is not clear whether same PCMAX is used for two panels. Q2: No. Q3: Per panel PCMAX, per panel PC equation, per panel PHR is needed. 4.2 Q1: Yes. Q2: per panel PHR is needed for STxMP. 4.4 Q1: support Alt.1. |
| Spreadtrum | Issue 4.1 Q3: It will be more clear if panel-specific power limitation is captured in specification. Issue 4.2 Q1: Yes Q2: TRP-specific PHR could be considered and the issue on PHR triggering and PHR reporting should be discussed. - PHR triggering: - For s-DCI based transmission, the joint triggering condition in Rel-17 could be reused as much as possible. - For m-DCI based STxMP transmission, a separate triggering condition could be considered. - PHR reporting: - For s-DCI based STxMP transmission, the design of PHR reporting in Rel-17 could be reused to STxMP as much as possible. - For m-DCI based STxMP, support separate PHR reporting Issue 4.4 Support Alt 1. For UL MTRP transmission, two PC parameter settings should be provided to different TRPs for path-loss estimation and UL transmission. |
| vivo | Issue 4.1 Q1: No. As RAN4 has agreed to define per-panel Pcmax, panel-specific PCMAX,f,c(i) should be referred in RAN1 spec. Q2: No. Q3: At least introduce the panel-specific PCMAX,f,c(i) into the equations to determine the transmission power for PUCCH/PUSCH in RAN1 spec. Issue 4.2 Q1: Yes. Q2: For SDCI-based STxMP, panel-specific Pcmax should be reported together with two PH values. For MDCI-based STxMP, when overlapped PUSCHs are scheduled to transmit simultaneously from two panels, RAN1 should specify how UE calculates and reports PHR. Issue 4.4 Q1: Support Alt1. |
| Xiaomi | Issue 4.1 Q1: No If only the current per-UE power limitation, PCMAX,f,c(i), will be considered in the power control for S-DCI based STxMP, it would not be used separately for determining Tx power on each panel. First, UE needs to calculate the transmit power for each panel, which can be denoted as P1 and P2, based on the corresponding PC parameters. Then, UE needs perform power reduction if the sum of P1 and P2 exceed the power limitation PCMAX,f,c(i) to decide the exact transmit power for each panel. Q2: No Q3: We need to decide whether any other power limitation in addition to the current per-UE power limitation PCMAX,f,c(i) should introduced first. Issue 4.2 Q1: Yes Q2: per panel PHR and per panel Pcmax are both needed Issue 4.3 Support alt.1. |
| Samsung | Issue 4.1: Q1: No. It just describes UL transmission power is determined per each of indicated TCI state separately Q2: No. Q3: Further clarification on UE panel specific Pc,max needed. Issue 4.2: Q1: No. Rel-17 PHR supports beam specific reporting. We do not sure whether panel specific Pc,max or panel specific PHR needs any further enhancements. Issue 4.4: Q1: No big preference, but slightly prefer Alt 2. STxMP is very complicated operation and we do not sure there should be a case UL PC parameters are not configured. But we are generally O.K. with majorities’ view. |
| CMCC | Issue 4.1 Q1: No. As RAN4 has agreed to define per-panel Pcmax, panel-specific Pcmax should be captured in RAN1 spec. Q2: No. Q3: At least panel-specific Pcmax should be specified in RAN1 specification. Issue 4.2 Q1: Yes. Q2: Panel-specific PHR may be also studied. Issue 4.3 Q1: Support Alt1. A similar rule as Rel-17 can be reused for Rel-18 via configuring two default UL PC parameter settings, and 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting can be determined by a fixed rule. For example, the first indicated joint/UL TCI state is associated with the first configured default UL PC parameter setting, and the second indicated joint/UL TCI state is associated with the second configured default UL PC parameter setting. |
| ZTE | For issue 4.1: Q1: No. Consider RAN4 agreed to define 'per-panel' configured transmitted power for STxMP power control. Per-panel PCMAX,f,c,k (k=0, 1) is to be used for the determination of Tx power on each panel for S-DCI based STxMP, which may be different from the legacy PCMAX,f,c. Q2: No. Q3: Per-panel power limitation should be specified in the determination of Tx power on each panel. Additionally, although per-panel power limitation is to be defined in RAN4, whether both per-UE power limitation and per-panel power limitation are enabled for STxMP power control for a carrier of a serving cell is not clear now. We need to consider the relationship of the two power limitation values and how to perform transmission power reduction in case of exceeding the per-panel and/or per-UE power limitation for a carrier of a serving cell and for CCs. For issue 4.2: Q1: Yes. Q2: Since there is a separate transmission power determination and limitation for each panel, the power headroom should be measured and reported separately for each panel with respect to the panel specific PCMAX,f,c,k. For issue 4.3: Support Alt1. |
| Nokia | For issue 4.1: No. As indicated by other companies, RAN4 agreed to define per-panel Pcmax (see RAN4#107’s WF captured in R4-2310268). The following would then need to be captured for PUSCH and PUCCH: - Adopt the following formula for PUSCH transmission power determination in case of SDM PUSCH transmission: PPUSCH,b,f,c,ti,j,qd,l= minPCMAX,f,c,tiPO_PUSCH,b,f,c,tj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,c,tj∙PLb,f,cqd,t+∆TF,b,f,c, ti+fb,f,ci,lt [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. - Note: Exact detail/definition of PCMAX,f,c,ti is up to RAN4. - FFS the power adjustment parameter ∆TF,b,f,c, ti. - Adopt the following formula for PUSCH transmission power determination in case of SFN PUSCH transmission: PPUSCH,b,f,c,ti,j,qd,l= minPCMAX,f,c,tiPO_PUSCH,b,f,c,tj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,c,tj∙PLb,f,cqd,t+∆TF,b,f,ci+fb,f,ci,lt [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. - Note: Exact detail/definition of PCMAX,f,c,ti is up to RAN4. - Adopt the following formula for PUCCH transmission power determination (in case of SFN PUCCH): PPUCCH,b,f,c,ti,qu,qd,l= minPCMAX,f,c,tiPO_PUCCH,b,f,c,t(qu)+10log102μ⋅MRB,b,f,cPUCCH(i)+PLb,f,c(qd,t)+∆F_PUCCH(F)+∆TF,b,f,ci+gb,f,c(i,lt) [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. - Note: Exact detail/definition of PCMAX,f,c,ti is up to RAN4. For issue 4.2: - We need to first agree that the two-PHR feature is adopted in Rel-18. We think this feature is needed for the same reasons as in Rel-17. - Which transmission scheme to assume for PH calculation and the case where one-PHR mode etc. is configured are aspects to be also considered in the discussions. For issue 4.4: No strong preference, and we would be fine with either Alt.1 or Alt.2. In addition to the above, we think it would be better to explicitly agree that similar to Rel-17, a second TPC field could be configured also for the Rel-18 framework. |
| Sharp | Issue 4.1 Q1: No. If the same Pcmax,f,c(i) is used for determining Tx power on each panel, the total power of 2 panels potentially exceeds the maximum transmission power. Furthermore, Pcmax,f,c,t(i) for each loop t corresponding to a panel is needed because there is no consensus on supporting digital port sharing for STxMP in AI9.1.4.1. For example, in a case that the UE fallbacks from 4 ports for STxMP into 2 ports for single panel, Pcmax,f,c(i) (e.g., 23 dBm) that is configured for the 4 ports cannot be applied to the 2 ports if each port has 17 dBm PA. In this case, the Pcmax,f,c,t(i) should be applied to each of one or two panels when STxMP is enabled. Q2: No. Q3: When STxMP is enabled, Pcmax,f,c,t(i) should be introduced for determining Tx power for each panel. Issue 4.2 Q1: Yes. Q2: Type 1 PHR should provide a difference between the Pcmax,f,c,t(i) and the estimated power for each panel when STxMP is enabled. Issue 4.4 Q1: Support Alt 1. When two TCI states are applied to PUSCH/PUCCH/SRS, different UL PC parameter settings should be used because at least PLs of UE-TRP1 and UE-TRP2 are highly different. |
| CATT | Issue 4.1: Q1: No. Two different PCMAX,f,c(i) might be used if two TCI-states or TCI-UL-States are indicated for determining Tx power on each panel for S-DCI based STxMP. Q2: No. Q3: In our opinion, the maximum value restriction of the sum of two UL Tx power values for PUSCH/PUCCH STxMP on each panel should be discussed and captured. Issue 4.2: Q1: Yes. Q2: For Rel-18 PUSCH STxMP, the number of PHRs (e.g. 1 or 2 PHRs)to be reported for S-DCI and M-DCI based STxMP might need to be discussed. Issue 4.4: Support Alt1. |
| Lenovo | Issue 4.1: Q1: No, according to RAN4 discussion, per TCI state PCMAC can be configured for a cell if two TCI-states or TCI-UL-States are indicated for STxMP transmission. Q2: No. Q3: PPUSCH,b,f,c,ti,j,qd,l should be calculated according to the per TCI state PCMAX, e.g., PCMAX,f,c,ti, t=0 or 1. Issue 4.2: Q1: Yes. Q2: Firstly, the two PH may need to be calculated according to the per TCI state PCMAC. Secondly, per-panel PHR reporting may need to be introduced for STxMP operation. Issue 4.4 Q1: Support Alt.1. Besides, default UL PC parameter settings are also needed when the beam and PL-RS are updated with qnew after receiving the BFRQ response as that in Rel-17. |
| Apple | Issue 4.1 Q1: No. Our view is that the CR does NOT touch upon how to specify the Pcmax,f,c to implement the ‘per-panel’ configured power rule agreed by RAN4, which is subject to RAN1 discussions. Q2: No. Q3: There are many aspects to be discussed in RAN1 e.g., whether to support cross-panel power sharing? Power scaling scheme in power limitation case, and ‘per-panel’ Pcmax,f,c. Issue 4.2: Q1: Yes. Q2: RAN1 needs to disucss e.g.,1) whether enhancement is needed for the condition of PHR report triggering as there are two panels. 2) whether to support ‘per-panel’ PHR report. Issue 4.4: Q1: Alt.1. The function can be supported to provide NW flexibility for power control parameter configuration. If overhead of two default settings is concerned, NW vendor has a full choice to provide PC parameters in each TCI-state and therefore no need to provide these two default settings. |
| LG | Issue 4.1 Q1: No. Regarding the draft CR, it is unclear whether the same Pcmax is applied for panels. Q2: No. Based on RAN4 LS, per-panel Pcmax should be considered for STxMP power control. Q3: Introduction of per-panel Pcmax and the corresponding UL PC equation are required. Also, the method to meet per-UE power limitation when the sum of per-panel power is larger than that should be needed. Issue 4.2 Q1: Needed Q2: Based on per-panel Pcmax, the corresponding PHR is required by including virtual PH value and per panel Pcmax together. Issue 4.3 Support Alt.1 with two default UL PC parameter settings configured in BWP-UplinkDedicated |
| Huawei, HiSilicon | Issue 4.1: Q1: No. PCMAX,f,c,0 and PCMAX,f,c,1 are required for determining per panel Tx power as per RAN4 agreement. Further, UE-level maximum output power PCMAX,f,c must be configured for possible per-panel power adjustment such that the corresponding measured peak EIRP is less than the EIRPmax regulatory requirement. Note that, based on the RAN4 reply LS to RAN1 R4-2303494, “the per-UE power limitation would be applicable at all the time. ‘Limitation’ here applies to regulatory compliance rather than a configured power requirement”. Q2: No. Q3: See answer to Q1. Issue 4.2: Q1: Yes Q2: Support per-panel actual and virtual PHR reports. Issue 4.4: Alt1. |
| NEC | Added our preference in the table. Issue 4.1: Q1: No. Q2: No. Q3: Per panel PCMAX, or dedicated PCMAX,STxMP for STxMP is needed. Or UE needs to report the TCI combinations supported for the cases that both panel can transmit up to PCMAX,f,c(i). Issue 4.2: Q1: Yes. Q2: per panel PHR is needed for STxMP. |
| FGI | Issue 4.1 Q1: No, it seems that the draft CR has not captured per panel Pcmax. Q2: No. Q3: per panel Pcmax, per panel PHR Issue 4.2 Q1: Yes. Q2: per panel PHR should be introduced. Issue 4.4 Support Alt1. |
| Panasonic | Issue 4.1: We agree that two power equations are needed to be specified for the two PUSCH transmissions for STxMP. We also think that each equation should be tied to an SRS resource set, since many parameters in that equation are determined by that. At this stage, we do not agree to any sum power constraints across panels since it is not clear how RAN4 defines spherical coverage, and min peak EIRP and max EIRP requirements for STxMP capable UEs and the consequence of this on per-panel and sum power constraints. But we can discuss further. |
| Intel | Issue 4.1 Q1. No, agree with other companies that per-panel Pc,max is needed Q2. No Q3. RAN1 needs to discuss per panel Pc,max and in addition, how to distribute total power between the two panels Issue 4.2 Q1. Yes Q2. RAN1 should discuss panel specific PHR reporting. PHR for multi-DCI should also be discussed including triggering conditions. Issue 4.3 Support Alt-1 since it seems to be a logical extension to the two TCI case. |
| IDC | We updated our preferences in the above table. Issue 4.1 Q1: No, unclear yet on whether the same Pc,max is applied for each panel. Q2: No. Q3: Per-panel Pc,max and per-panel PHR are needed at least. Issue 4.2 Q1: Yes. Q2: Per-panel PHR including both actual and virtual PHR reports should be discussed. Issue 4.4 Support Alt1. |
| Mod | Issue 4.1: All the companies share the same view that current spec in the draft CR for UL PC [1] is not sufficient to support Rel-18 STxMP UL power control, and corresponding spec impact/change may include the following aspects: - To support “Per-panel” configured Tx power (PCMAX,f,c,ki) for STxMP, as agreed to be supported by RAN4 [2]: - Google, OPPO, QC, Docomo, vivo, Xiaomi, Samsung, CMCC, ZTE, Nokia, Sharp, CATT, Lenovo, Apple, LG, NEC, Intel, Huawei/HiSilicon, IDC - To support “Per-panel” UL PC equation - QC, Nokia, Docomo, Lenovo, NEC, Huawei/HiSilicon - To support Tx power reduction/scaling/prioritization if total power exceeds a limitation (e.g., per-UE total power limitation) on a CC or all CCs - Google, QC, ZTE, CATT, Apple, LG, Huawei/HiSilicon, Intel Since majority prefer to reflect the RAN4 agreement [2] on “per-panel” configured Tx power for STxMP in RAN1 specification, the following recommended proposal will be provided Round 0 summary. Proposal 6.1: On PUSCH/PUCCH Tx power determination for PUSCH/PUCCH transmission occasion i, support PCMAX,f,c,ki for S-DCI based PUSCH/PUCCH STxMP and M-DCI based PUSCH+PUSCH STxMP, and down-select one of the following alternatives for the definition of k: - Alt1: k=0, 1 correspond to the first and second indicated joint/UL TCI states applied to PUSCH/PUCCH transmission occasion i, respectively. - For M-DCI based MTRP operation, the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Alt2: k=0, 1 correspond to the first and second SRS resource sets associated with PUSCH transmission occasion i, respectively. - FFS: The definition of k=0, 1 for PUCCH transmission occasion i - Alt3: This is up to RAN4 decision Issue 4.2: Most companies prefer to enhance Type 1 PHR for Rel-18 PUSCH STxMP. One company think legacy PHR is sufficient. Potential enhancement for Type 1 PHR includes the following aspects: - To support “panel-specific” report for STxMP - Google, OPPO, QC, Docomo, Spreadtrum, vivo, Xiaomi, CMCC, ZTE, Sharp, Lenovo, LG, Apple, Huawei/HiSilicon, NEC, Intel, IDC, FGI - To support “panel/TRP-specific” triggering - Apple, Spreadtrum - To support M-DCI based PUSCH+PUSCH STxMP - Google, vivo Since majority prefer to support “panel-specific” PHR for STxMP, some further questions are raised as follows: - Q1: To support “panel-specific” PHR for S-DCI based STxMP, we should enhance single PHR mode, two PHR mode, or both? - Q2: To support “panel-specific” PHR for M-DCI based STxMP, we should enhance single PHR mode, two PHR mode, or both? Note: For single PHR mode, one PH value for one panel will be included in one report. For two PHR mode, two PH values for two panels will be included in one report, receptively. It would be helpful for progress if companies can provide answers to above questions in your contributions. Issue 4.3: Most of companies support Alt1, and a corresponding proposal will be provided in Round 0 summary. My plan is to conclude this issue in this meeting (even it may be concluded as no consensus). |

- References

| 1 | R1-2306292 | Introduction of MIMO Evolution for Downlink and Uplink | Samsung |
| --- | --- | --- | --- |
| 2 | R4-2310268 | WF on UE RF requirements for NR MIMO evolution | Samsung |

- Appendix: Agreements/conclusions before/in RAN1#113

RAN1#113

Agreement On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the scheduled/activated PDSCH reception - If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception - If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold Agreement On unified TCI framework extension for S-DCI based MTRP: - If a CORESET other than a CORESET with index 0 is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first one, the second one, or both of the indicated joint/DL TCI states to PDCCH reception on the CORESET - If a CORESET other than a CORESET with index 0 is associated at least with CSS sets other than Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET - For a CORESET with index 0: - If the CORESET is associated with SS#0 for Type 0/0A/2 CSS sets, the CORESET is configured by RRC to apply the first one, the second one, or none of the indicated joint/DL TCI state to PDCCH reception on the CORESET - Otherwise, the CORESET is configured by RRC to apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET Note: RAN1 already agrees to use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET in S-DCI based MTRP. Note: There is no consensus in RAN1 on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of above RRC configuration, and whether to reuse followUnifiedTCIstate is up to RAN2 design Agreement On unified TCI framework extension for S-DCI based MTRP, when a 2-bit [TCI selection field] is configured by RRC to be present in a DCI format 1_1/1_2 in a DL BWP: - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to PDSCH reception scheduled/activated by the DCI format 1_1/1_2 based on the Rel-16 rules for mapping legacy TCI states to PDSCH transmission occasions, CDM groups, or non-overlapping frequency domain resource allocations by replacing the first and the second indicated legacy TCI states with the first and the second indicated joint/DL TCI states, respectively - The codepoint "11" of the [TCI selection field] is reserved Agreement On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUSCH transmission - For SDM and SFN based PUSCH Tx schemes, the UE shall apply the first indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the second SRS resource set, respectively. - Note: The association between PUSCH antenna port(s) and an SRS resource set is discussed and defined in STxMP AI Agreement On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUCCH resource/resource group: - For TDM based PUCCH Tx scheme, the UE shall apply two indicated joint/UL TCI states to repetitions of the PUCCH transmission corresponding to the PUCCH resource/resource group based on the Rel-17 rules for mapping spatial settings to the repetitions by replacing the first and second spatial settings with the first and second indicated joint/UL TCI states, respectively. - For SFN based PUCCH Tx scheme, the UE shall apply two indicated joint/UL TCI states to the PUCCH transmission corresponding to the PUCCH resource/resource group Agreement On unified TCI framework extension for S-DCI based MTRP, the following two alternatives are supported for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state Introduce a UE capability on which alternative(s) is supported, and either one of above alternatives can be configured by RRC according to the UE capability Note: In Rel-18, RAN1 has no consensus to support Alt3 - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB Agreement On unified TCI framework extension for S-DCI based MTRP, support the following: - Using RRC configuration to indicate whether the first, second, or both of the indicated joint/DL TCI states is/are applied to PDSCH reception scheduled/activated by DCI format 1_0 - If not configured, the first indicated joint/DL TCI state is applied - Only when the UE is configured with PDSCH-CJT and the UE supports two joint TCI states for PDSCH-CJT or the UE is configured with PDSCH-SFN, the RRC configuration can indicate both indicated joint/DL TCI states are applied. - For PDSCH-CJT and PDSCH-SFN, if the RRC configuration indicates both indicated joint/DL TCI states are applied, the UE shall apply both indicated joint/DL TCI states to PDSCH reception scheduled/activated by DCI format 1_0 on a search space other than Type0/0A/2 CSS on CORESET#0 (FFS: Other search space and/or CORESETs) If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_0 and the scheduled/activated PDSCH reception If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_0 and the scheduled/activated PDSCH reception is equal to or larger than a threshold Agreement On unified TCI framework extension for S-DCI based MTRP, after NW response to TRP-specific BFR request to a BFD-RS set: - If the BFD-RS set is the first BFD-RS set (), QCL assumption/spatial Tx filter/PL-RS corresponding to the first indicated joint/DL/UL TCI state for channel(s)/signal(s) applying the first indicated joint/DL/UL TCI state are updated according to the new beam (qnew) corresponding to the BFD-RS set. - If the BFD-RS set is the second BFD-RS set (), QCL assumption/spatial Tx filter/PL-RS corresponding to the second indicated joint/DL/UL TCI state for channel(s)/signal(s) applying the second indicated joint/DL/UL TCI state are updated according to the new beam (qnew) corresponding to the BFD-RS set. Agreement On unified TCI framework extension for both S-DCI and M-DCI based MTRP operations, if a P/SP/AP SRS resource set for CB/NCB/AS or an AP SRS resource set for BM is configured to follow unified TCI state, an RRC configuration can be provided to the SRS resource set to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the SRS resource set - For M-DCI based MTRP operation, the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - When two SRS resource sets for CB/NCB are configured, the UE does not expect the following - to be configured with the first indicated UL/joint TCI state which is to be applied to the second SRS resource set - to be configured with the second indicated UL/joint TCI state which is to be applied to the first SRS resource set - For M-DCI based MTRP operation, if the RRC configuration is not provided to the SRS resource set and the SRS resource set is an AP SRS resource set triggered by PDCCH on a CORESET associated with a coresetPoolIndex value, the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the SRS resource set How to capture the above is up to the editor Agreement On unified TCI framework extension for M-DCI based MTRP, An RRC configuration can be provided to an aperiodic CSI-RS resource set or a CSI-RS resource in an aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set or to the CSI-RS resource in the aperiodic CSI-RS resource set, if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - The first and the second indicated joint/DL TCI states correspond to the indicated joint/UDL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - Support of ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ RRC configuration is up to UE capability Agreement On unified TCI framework extension for S-DCI based MTRP, if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2: - When the offset between the reception of the scheduling/activation DCI format 1_0/1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2, the UE shall apply the first indicated joint/DL TCI state to the scheduled/activated PDSCH reception Conclusion There is no RAN1 consensus to support the following: On unified TCI framework extension, the following cases for CA operation are supported: - A set of BWP/CCs configured for common TCI state ID activation/update can include BWP/CC(s) operating in STRP and BWP/CC(s) operating in S-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - For the BWP/CCs in above set of BWP/CCs, TCI state ID activation/update MAC-CE can only be sent to a S-DCI based MTRP BWP/CC - A set of BWP/CCs configured for common TCI state ID activation/update can include BWP/CC(s) operating in STRP and BWP/CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - For the BWP/CCs in above set of BWP/CCs, TCI state ID activation/update MAC-CE can only be sent to a M-DCI based MTRP BWP/CC - a CC in the set of CCs operating in S-DCI/M-DCI based MTRP can be configured as the reference CC. - For each CC in the above set of CCs, an RRC parameter is configured to the CC to indicate that the first, the second or both joint/DL/UL TCI states are applied to the CC. Note: “A CC operates in STRP” for above means a CC in which only one joint/UL/DL TCI state is applied Note: “A CC operates in S/M-DCI based MTRP” for above means a BWP/CC operates in Rel-18 unified TCI framework extension for S/M-DCI based MTRP operation. Agreement On unified TCI framework extension for S-DCI based PUSCH/PUCCH STxMP: The UE shall determine a first Tx power for PUSCH/PUCCH transmission occasion i based on the UL PC parameter settings for PUSCH/PUCCH, if any, and the PL-RS included in the first indicated joint/UL TCI state, and a second Tx power for the same PUSCH/PUCCH transmission occasion i based on the UL PC parameter settings for PUSCH/PUCCH, if any, and the PL-RS included in the second indicated joint/UL TCI state

RAN1#112b-e

Conclusion On unified TCI framework extension for S-DCI based MTRP operation, there is no consensus to support dynamic switching between single-TRP operation and multi-TRP operation for channels/signals based on the number of TCI states mapped to the received TCI codepoint in DCI format 1_1/1_2 - FFS: How to switch between Rel-17 sTRP operation and Rel-18 mTRP operation Agreement On unified TCI framework extension, the Rel-17 timeline for updating the indicated joint/DL/UL TCI state(s) is retained, i.e., the indicated joint/DL/UL TCI state(s) applied to the DL reception or UL transmission in each slot is updated based on the Rel-17 beam application time Agreement On unified TCI framework extension for S-DCI based MTRP, the UE shall apply the first indicated joint/UL TCI state to PUSCH transmission(s) scheduled/activated by DCI format 0_0 (including DG and Type2 CG) Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first, the second, or both indicated joint/UL TCI states to the corresponding CG-PUSCH transmission - If the first or the second indicated joint/UL TCI state is applied, the UE shall apply the first or the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If both indicated joint/UL TCI states are applied: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB - FFS: SDM and SFN based PUSCH Tx schemes Agreement On unified TCI framework extension for S-DCI based MTRP, PDSCH-CJT Tx scheme is RRC-configured, and dynamic switching between PDSCH-CJT and other S-DCI based PDSCH Tx schemes is not supported Agreement If the UE is configured with SSB-MTC-AdditionalPCI and receives TCI state activation command (MAC-CE) that activates a set of joint/DL /UL TCI state(s) specific to each coresetPoolIndex value for M-DCI based MTRP in unified TCI framework extension, the activated joint/DL /UL TCI state(s) specific to one coresetPoolIndex value is associated with the serving cell PCI and the activated joint/DL /UL TCI state(s) specific to another coresetPoolIndex value can be associated with a PCI other than the serving cell PCI . - Note: How to implement above in specification is up to spec editor Agreement On unified TCI framework extension for M-DCI based MTRP, after NW response to TRP-specific BFR request to a BFD-RS set associated with a coresetPoolIndex value, QCL assumption/spatial Tx filter/PL-RS for channel(s)/signal(s) that applies the indicated joint/DL /UL TCI state specific to the coresetPoolIndex value are updated according to the new beam (q new ) corresponding to the BFD-RS set. Agreement On unified TCI framework extension for S-DCI based MTRP, the presence of the [TCI selection field] can be RRC-configured per DL BWP - FFS: Whether the presence of the [TCI selection field] can be configured individually for DCI format 1_1 and DCI format 1_2 in the same DL BWP Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint Agreement On unified TCI framework extension for M-DCI based MTRP, support at least Opt2 for PUCCH transmission, and Opt1 is not supported - Note: Opt3 and Opt4 are not precluded Conclusion On unified TCI framework extension for S-DCI based MTRP, there is no consensus in RAN1 on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of the RRC configuration that informs the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET - Above does not impact how RAN2 writes their specifications Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability Agreement On unified TCI framework extension, support the following cases for CA operation: - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in M-DCI based MTRP - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP, CC(s) operating in S-DCI based MTRP, and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case Agreement On unified TCI framework extension for M-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding CG-PUSCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

RAN1#112

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported Agreement On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

RAN1#111

Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH. Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field

RAN1#110b-e

Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support simultaneous configuration of both joint and separate DL/UL TCI modes in a serving cell Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support separate RRC-configured TCI state list(s) for each of TRPs Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - FFS: PL-RS(s), and UL PC parameter setting(s) (including P0, alpha, and closed loop index) for the PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUCCH transmission: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group associated with the same CORESET group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Note: the association indicates whether the UE shall apply the first one, the second one, or both of the joint/UL TCI states indicated by DCI/MAC-CE to a PUCCH resource/group Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 Agreement On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111: - Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Alt2: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) only specific to one of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Note: According to the agreement in RAN1#109-e, support of one additional TCI field or a field associating the TCI field to the TRP(s) is not precluded Note: It has been agreed to use the existing TCI field for TCI state indication for S-DCI based MTRP in RAN1#109e Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS FFS: The behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP

RAN1#110

Agreement On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP - Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1 - Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17 Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings: - Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s) - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions - Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE Switching between multi-TRP and single TRP operation is not precluded Agreement On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, down-selection one alternative from the followings: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - Alt3: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/UL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI format 0_1/0_2 is received in a CORESET group, the indicated joint/UL TCI state(s) associated with the CORESET group is applied to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - FFS: Details of CORESET group(s) FFS: PUSCH transmission scheduled/activated by a DCI format 0_0 and Type-1 CG-PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission, down-selection at least one alternative from the followings: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Alt4: Use DCI to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

RAN1#109e

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Detail of mapping joint/DL/UL TCI state ID(s) to a TCI codepoint, e.g., possible combinations of joint, DL, and/or UL TCI state IDs that can be mapped to a TCI codepoint - FFS: Whether to increase the max number of MAC CE activated TCI codepoints, i.e., more than 8 codepoints - FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits - Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded Note: The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for M-DCI based MTRP, consider the following alternatives for TCI state update: - Alt1: Reuse the same TCI state update scheme for S-DCI based MTRP - Atl2: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate the joint/DL/UL TCI state(s) corresponding to the same CORESETPoolIndex value - Alt3: Use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) to indicate all joint/DL/UL TCI states corresponding to both CORESETPoolIndex values - Study the association between the indicated joint/DL/UL TCI state(s) and a CORESETPoolIndex value - Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value. - Study whether the indicated joint/DL/UL TCI state(s) applies to the channels/signals associated with the same CORESETPoolIndex value or different CORESETPoolIndex value is indicated by DCI Agreement On unified TCI framework extension for S-DCI based MTRP, consider at least the following alternatives to map/associate a joint/DL TCI state to PDCCH reception(s) - Atl1: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt2: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a search space set - Alt3: Use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt4: Use DCI to inform the mapping/association between an indicated joint/DL TCI state and a CORESET or a CORESET group - Alt5: Based on a fixed mapping/association rule, e.g., the first indicated joint/DL TCI state always applies to PDCCH receptions Consider above alternatives for PDCCH repetition, PDCCH-SFN, PDCCH w/o repetition/SFN, and potential support of dynamic switching between S-TRP and M-TRP for PDCCH. It is not precluded to adopt one single alternative or multiple alternatives to support these cases. Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH/PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH/PUCCH (including P0, alpha for PUSCH, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.