3GPP TSG RAN WG1 #114	R1-2307623

Toulouse, France, Aug. 21st –Aug. 25th, 2023

Source:	China Telecom

Title:	Discussion on spatial and power domain CSI adaptation for NES

Agenda Item:	9.7.1

Document for:	Discussion

- Introduction

The new Rel-18 WI of network energy saving NR was approved in RAN#98e [1], and the following objectives are included..

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

The flexible and efficient switch on-off of spatial elements and adjustment of power offset has been discussed in the NES SI, and the major specs impact will be about the CSI enhancement and the related measurement and report procedure. In this contribution, we focus on the network energy saving techniques of spatial and power  and provide some observations and considerations.

- Considerations on spatial domain only adaptation

2.1 Enhancement on CSI-RS resource configuration

In the RAN1#112bis-e meeting, the 2 types of NES spatial domain adaptations and 2 types of NZP CSI-RS resources configuration were defined [3].

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled Agreement Support configurability of NZP CSI-RS resource(s) for channel measurement within one resource setting corresponding to more than one spatial adaptation patterns with at least one of the following - A1-1-revised: a resource set with multiple resources is configured within a resource setting, where each resource is associated with only one spatial adaptation pattern - A1-2-revised: For a resource configured in a resource set within a resource setting, the resource can be associated with more than one spatial adaptation patterns - One or more resources can be configured in the resource set for channel measurement.

2.1.1 The configuration of A-1-2 revised for Type 1

And in last meeting, the agreement on how the two types of configurations adopted to the SD adaptation types was reached. Besides, the sub-configuration in CSI-RS report configuration was introduced, and the restriction on the antenna ports in a resource set was clarified [4].

Agreement Confirm the working assumption with the following update (in blue) - Al-1-revised and A1-2-revised are supported - For Type 1 SD adaptation - A1-2-revised is supported - For Type 2 SD adaptation A1-1-revised is supported. Agreement - For A1-1-revised for Type 2, one or more CSI-RS resources from a CSI-RS resource set for channel measurement can be associated with the same sub-configuration provided in a CSI report configuration - Resources in the resource set for channel measurement have the same number of antenna ports - For A1-2-revised for Type 1, all CSI-RS resource(s) (which can be one or more) in the CSI-RS resource set for channel measurement are associated with each sub-configuration provided in a CSI report configuration - i.e. each CSI-RS resource is associated with all the sub-configurations - Resources in the resource set for channel measurement have the same number of antenna ports - FFS: restriction on total number of CSI-RS resources for channel measurement in a CSI-ReportConfig and/or sub-configuration.

For NZP CSI-RS configuration A1-1-revised, there is no differences from the mechanism in legacy, while for A1-2-revised, how one resource can be associated with multiple spatial adaptation patterns should be further discussed. According to current specific, as it defined shown in in 38.211 Table 7.4.1.5.3-1[5] of the existing CSI-RS patterns, the rows with same cdm-Type and Density but different ports can be used for the multiple association, where the rows with less ports can be seen as a subset of the rows with more ports. With this fundamental, the “nested” structure for achieving the association with multiple spatial patterns within one CSI-RS resource was proposed by many companies in the last few meetings[5]. By adopting such structure, the same configuration parameters can be reused for all the spatial patterns excepts the number of supporting ports.

Observation 1:

In the current specs, the CSI-RS patterns with less ports can be seen as the subset of CSI-RS patterns with more ports but the same “Density” and “cdm-Type”, which can be used for realizing the “nested” structure that can associate one CSI-RS configuration with multiple pattern.

Considering the spatial pattern in current specs, the mapping of CSI-RS resource is configured in the CSI-RS-ResourceMapping IE, when the cdm-Type and density are same, the only configuration needed is to indicate UE the number of ports, i.e., nroPorts. Generally, there are two methods to configure the number of valid CSI-RS ports for each CSI-RS resources.

- Option 1: In the NZP CSI-RS configuration: the nroPortsList can be introduced instead of nroPorts, where multiple numbers of available number of ports can be included. Then all the CSI-RS configuration in a CSI-RS resource set can reuse the same nroPortsList.

- Option 2: In the NZP CSI report configuration: in the NZP CSI-RS configuration, only the maximum number of supported ports is configured. At the same time, a new 1-bit indication can be introduced to distinguish the nroPorts from the maximum number of ports and the number of ports. Then when configuring the sub-configuration in the CSI-RS-ReportConfig, since all the CSI-RS in a sub-configuration have the same number of ports, the number of ports can be configured at the same time.

The first method can be a directly method, but even with the nroPortsList in the CSI-RS resource configuration, the indication of number of ports in the corresponding CSI-RS report sub-configuration is still needed. Therefore, we prefer to adopt Option 2 for configuring multiple spatial adaptation patterns associated with one CSI-RS resource.

Proposal 1:

For A1-2-revisied applied for SD adaptation Type 1, nroPorts should be regarded as the maximum expected number of antenna ports when nested’ structure is adopted

- It should be indicated whether SD adaption Type 1 is adopted in the CSI-RS resource configuration for distinguishing the meaning of nroPorts.

2.1.2 Handle of hybrid cases

Besides, there can be two kind of hybrid cases in one CSI-RS configuration

- Hybrid of A1-2-revised and legal configurability in Rel-17: in one CSI-RS configuration, some CSI-RS resources are associated with multiple SD adaptation patterns, i.e., A1-2-revised configurability, while the others are still associated with only on SD adaptation pattern in Rel-17.

- Hybrid of A1-2-revised and A1-1-revised configurability: in one CSI-RS configuration, some CSI-RS resources are configured by A1-2-revised, while the others are A1-1-revised configured.

For the case 1), as long as which CSI-RS configuration are A1-2-revised is indicated, as suggested in proposal 1, then the enhancement in CSI reporting configuration and the procedure can only adopted for these CSI-RS. E.g., the sub-configuration should not include the CSI-RS not configured as A1-2-revised.

Observation 2:

Explicitly indicating whether SD adaptation Type1 is adopted can avoid the confusion when the Rel-17 CSI-RS configuration is adopted at same time.

For case 2), since the sub-configuration also applies to A1-1-revised, to avoid confusion of sub-configuration for two types SD adaptation, the A-1-2 revised shouldn’t be adopted, i.e., all the CSI-RS configured with A1-1-revised.

Observation 3:

If the SD adaptation Type 1 and Type 2 are supposed to be adopted at the same time, all the CSI-RS should be configured as A1-1-revised.

2.2 Enhancement on CSI reporting configuration

The CSI reporting sub-configuration was introduced in last meeting[5]. The following agreements were reached on the details for sub-configuration.

Agreement@113 For a CSI report configuration with L>1, for Type 1 SD, at least when A1-2-revised is used for the associated codebook configuration, - Only common codebook type for PMI across sub-configurations is supported - Codebook type-1 for PMI is supported Agreement@113 For Type 1 adaptation, for each sub-configuration, - Port subset indication is based bitmap is supported - One bit per port for single panel case (i.e. turning off in a port level) - FFS: One bit per panel for multi-panel case (i.e. turning off in panel level) - Note: It is up to the gNB to ensure the mapping of the bit to a uniform x-pol rectangular array Agreement@113 For Type 1 adaptation, for each sub-configuration, for multi-panel case, - One bit per port based on bitmap is supported - Note: It is up to the gNB to ensure the mapping of the bit to a uniform x-pol rectangular array for each of the activated panel(s). Additionally, if more than one panel is activated, uniformity across panels is ensured by the gNB (i.e., the same N1, N2 across multiple activated panels) Agreement@113 For the sub-configuration(s) in a CSI report configuration with L>1, - for Type 1 SD with A1-2-revised, the following is configured in each sub-configuration - codebook subset restriction, - rank restriction - N1, N2 and Ng - FFS: the case when the number of ports is less than 4 - for Type 2 SD adaptation with A1-1-revised, for each sub-configuration - a list of CSI-RS resource ID - FFS: codebookConfig (including codebookSubsetRestriction/ ri-Restriction) - FFS: CQI table indication - FFS: reportFreqConfiguration - FFS: report quantity

2.2.1 Sub-configuration for Type 1 SD adaptation

For the case when the number of ports is less than 4, where there is no corresponding CSI codebook in specs, so thus the configuration of N1, N2 and Ng. For such case, the configuration is actually not needed since there is only one possible condition of the allocation of activated antenna ports. Then, indicating the exact number of activated antenna ports is enough for UE to understand the configuration. By indicating the number of antenna ports in each sub-configuration explicitly, the configuration of subset indication may also be omitted in some conditions. Therefore, we proposed to indicate the number of antenna ports in each sub-configuration.

Proposal 2:

For Type 1 SD adaptation with A1-2-revisied, for each sub-configuration, support to indicate the number of the antenna ports, and the following configurations can be omitted for the case when the number of ports is less than 4.

- sub port indication

- N1, N2 and Ng

In fact, with the explicit indication of number of antenna ports, the configuration can be further simplified. Revisiting the Table 5.2.2.2.1-2 in TS 38.214 [7], where the supported configuration of (N1,N2) and (O1,O2) are specified, it can be divided into three categories, the cases underlined with the same color are in the same category. Only if the cases are in the same category, the antenna ports configuration with less number of ports can be regarded as a subset of that with more ports. As a result, if A1-2-revisied is adopted using the ‘nested’ structure, as long as the (N1, N2) or (N1, N2, Ng) configuration with the maximum number of ports is given, the exact configuration for other sub cases can be acquired automatically. Therefore, we think only the N1,N2 and Ng for the sub-configuration with the maximum number of antenna ports is needed. Which means, the N1, N2 and Ng may only be needed in the CSI-RS report configuration instead of the sub-configuration.

Observation 4:

For Type 1 SD adaptation with A1-2-revisied, for sub-configuration with less number of ports, the configuration of N1, N2 and Ng is not needed.

Proposal 3:

For Type 1 SD adaptation with A1-2-revisied, the common configuration of N1, N2 and Ng can be used in sub-configuration if the number of antenna ports is indicated.

2.2.2 Sub-configuration for Type 2 SD adaptation

For the type 2 SD adaptation with A1-1-revisied, each resource is associated with only one spatial adaptation pattern, i.e., there is no ‘nested’ structure. Compared with A1-2-revised, in each sub-configuration, the CSI-RS resources have the same number of antenna ports, but have no association with the sub-configuration with more antenna ports. And if some antenna ports are muted, the associated CSI ports may change. It’s obviously that the codebook subset restriction and RI restriction should be configured separately for different CSI report sub-configurations.

No matter which SD adaptation pattern is adopted, the performance target should be the same. Therefore, a common CQI table and report quantity should be configured for all the sub-configurations. For the report frequency of CSI with different ports, gNB may concern more about some of the port numbers. For example, if a 32 ports CSI is adopted by network and the performance is well, then the gNB may want to keep monitoring the channel state with current configuration, but also fine to turn off some antenna elements if possible. In such condition, there should be more CSI based on 32 ports to ensure the performance, reporting the CSI with less antenna ports occasionally is enough. Thus, the reportFreqConfiguration should be configured separately for different sub-configuration.

Proposal 4:

For Type 2 SD adaptation with A1-1-revisied, codebookConfig and reportFreqConfiguration should be configured separately for different report sub-configuration.

Proposal 5:

For Type 2 SD adaptation with A1-1-revisied, the CQI table indication and Report quantity should be common configured for different report sub-configuration.

2.3 Enhancement on CSI reporting

2.3.1 CSI report payload reduction with jointly CSI report

In RAN1#112bis-e and #113, the following agreements were reached.

Agreement@112bis-e - For CSI feedback with CSI overhead/report payload reduction, further study whether/how to report a common value and/or a differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns, at least for the following - CRI - RI - PMI - CQI - FFS: L1-RSRP - Other (new) report quantity, if any - Further study whether/how it is feasible/possible for the UE to skip the evaluations of some sub-configurations/adaptation patterns to reduce the burden at the UE Agreement@113 For both spatial domain NES, when UE reports CSIs corresponding to one or more sub-configurations provided in a CSI report configuration, - At least support baseline: Report CSI for each indicated sub-configuration, according to reportQuantity configuration - FFS: details on how to map CSI(s) in a CSI report - Further enhancement on CSI payload reduction is not precluded

To reduce the measurement complexity and payload reduction, the joint report of the CSI quantity can be considered, where a common value can be adopted for all the CSI report quantity in the same sub-configuration. The foundation of joint report is that there is high correlation between the different CSI-RS. The correlation can exist within a sub-configuration or across multiple sub-configurations. For example, in the figure 2, CSI-RS #1&#2/ CSI-RS #3&#4 are associated with the same CSI-RS resource respectively, while CSI-RS #1&#3/ CSI-RS #2&#4 are in a same sub-configuration respectively. Since CSI-RS #1/#3 is a subset of #2/#4, there is obviously high correlation between the CSI-RSs. Besides, if the difference between the CSI-RS #1 and #3 resource allocation is just the start symbol in a slot, there can also exists a correlation between them. However, the correlation may also not exist if the resource allocations of the different CSI-RS are quite different. Even though the correlation between the CSIs is high, network may not expect UE to report the CSI jointly. Therefore, a new IE can be introduced in the CSI-ReportConfig for indicating whether the CSI can be reported separately.

Figure 2 illustration of CSI report sub-configuration

Proposal 6:

Support to indicate whether the CSI should be reported jointly in the CSI-ReportConfig explicitly.

What’s more, for the CSI-RS with a high correlation, a common value can be considered for joint report. But whether the correlation is high should depend on the specific condition. Therefore, for make the better decision, we think a new field of bitmap can be introduced in CSI-ReportConfig to indicate whether the measurement should be reported jointly or separately. The length of the bitmap is corresponding to the reportQuantity, where each bit is corresponding to one measurement. For example, if the reportQuantity is configured as ‘cri-RI-PMI-CQI’, then the bitmap should be 4 bits, the bits are used for indicating whether the CRI\RI\PMI\CQI can be reported respectively.

Proposal 7:

Support to introduce a bitmap for indicating whether each measurement in the reportQuantity can be reported jointly.

What’s more, the CSI in some sub-configurations may have high correlation and can be jointly reported, while others are not. In such case, which CSIs to be reported jointly can be further indicated. Which CSIs to be reported jointly can be configured with an extra bitmap or just included in a list. Also, a group report ID can be configured for each CSI-RS. The CSI measurement with same group report ID can be reported together.

Proposal 8:

Which CSIs to be jointly reported should be configured in the CSI-ReportConfig.

2.3.2 CSI-RS report sub-configuration selection

The following agreements were reached in RAN1 #113.

Agreement For N>=1 CSI reporting corresponding to N out of L sub-configurations in one reportConfig where each sub-configuration corresponding to an SD adaptation pattern or/[and] a powerControlOffset value, - For A-CSI and SP-CSI on PUSCH report, support DCI-based triggering - For A-CSI-RS, CPU and CSI-RS resource/port counting depend on N indicated sub-configurations - FFS: How to do the counting - FFS: For P-CSI-RS/SP-CSI-RS, CPU and CSI-RS resource/port counting depend on L or N sub-configurations - For SP-CSI on PUCCH report, support MAC-CE-based triggering - FFS: For P-CSI-RS/SP-CSI-RS, CPU and CSI-RS resource/port counting depend on L or N sub-configurations Note: UE complexity reduction is not precluded - For DCI-based triggering, - Alt 1: A triggering state corresponding to N sub-configurations is indicated via the existing CSI request field in DCI. Different triggering states could represent different subsets of L sub-configurations. - The DCI is UE specific (in this case, legacy DCI format applies) - For MAC-CE based triggering - Opt 2: An indication to select to N sub-configurations in a MAC-CE is supported - It is up to RAN2 to decide the signaling designs of the MAC-CE (including whether it is a new MAC CE or an existing MAC CE) - Only one MAC CE is used for this triggering

To reduce the payload and complexity of CSI measurement and report, a mechanism selecting N sub-configurations from L sub-configurations was agreed to introduced in Rel-18. For DCI-based triggering, it has been agreed that the trigger is indicated via the existing CSI request field in DCI. Which N sub-configurations can be further selected according to the CSI-AperiodicTriggerStateList IE. In the current specs [6], CSI-AssociatedReportConfigInfo is included in the CSI-AperiodicTriggerStateList,, where the CSI-reportID is included to indicated which CSI report configuration is configured. While for MAC-CE, the procedure can be similar, which configurations are triggered should also refer to the corresponding RRC configurations.

While if triggering states represents different subsets of N sub-configurations, then which N sub-configurations are selected should be indicated. Therefore, a unique ID for each sub-configuration in a CSI report configuration is needed for representing the sub-configurations and being selected. The ID can be included in the CSI-ReportConfig for each sub-configuration. What’s more, since the different sub-configurations have different number of antenna ports, the number of antenna ports can be used as the ID directly.

Proposal 9:

Each CSI report sub-configuration should have a unique ID so that selection can be indicated.

- The ID can be allocated in the CSI report configuration or reuse the corresponding number of antenna ports.

With such unique, then which CSI report sub-configurations are selected can be further indicated in the CSI-AssociatedReportConfigInfo. The ID of triggered sub-configurations can be listed, or a bitmap can be introduced, where each bit can corresponding to the sub-configurations sorted by a certain order.

Observation5:

The subsets of L sub-configurations can be furthered indicated with a list of sub-configuration ID or a bitmap each bit corresponding to a sub-configuration.

2.3.3 CPU counting

The following agreements were reached in RAN1 #113[5] on CPU counting.

Agreement For spatial domain adaptation or power domain adaptation, for CSIs reporting corresponding to N indicated sub-configurations from L sub-configurations in a CSI report, for the case without CSI payload reduction - OCPU=i=1[N or L]Ksi, where Ksi is the total number of CSI-RS resources corresponding to i-th sub-configuration in the CSI-RS resource set for channel measurement. - the summation is over N for A-CSI RS - This is for CSI processing criteria for NES in Clause 5.2.1.6 of TS 38.214 Agreement Alt 2: For P-CSI reporting from L configured sub-configurations, support: - All L configured sub-configurations are reported in every periodic occasion. - The maximum value of L can be different for A-CSI, SP-CSI, and P-CSI. - OCPU=i=1[N or L]Ksi, where Ksi is the total number of CSI-RS resources corresponding to i-th sub-configuration in the CSI-RS resource set for channel measurement. (N=L in the equation) - FFS: Details on active CSI-RS resource / port counting

It’s generally agreed that the CPU counting should be based on the triggered number of CSI-RS resources in the report sub-configurations without CSI payload reduction. Therefore, for A/SP CSI-RS, the counting should based on N, for P CSI-RS, the counting should be based on L. This calculation can also be adopted for the scenarios where both SD and PD adaptation is adopted.

Proposal 10:

For both SD/PD only adaptation and SD/PD jointly adaptation, without UE complexity reduction,

- For A/SP-CSI reporting, the CPU counting should be based on OCPU=i=1NKsi

- For P-CSI reporting, the CPU counting should be based on OCPU=i=1LKsi

2.3.3 Active CSI-RS resource/antenna ports counting

In the current clause 5.2.1.6 of TS 38.214, the counting of active CSI-RS resource/ports are defined as follow,

If a CSI-RS resource is referred by N CSI reporting settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

From the definition, it can be concluded that for the CSI-RS resource referred in one CSI reporting configuration, the CSI-RS resource/ports only counted once. And the counting is based on the CSI reporting settings. The same principle should be kept for SD/PD adaptation, i.e., the counting of CSI-RS resource should be counted once for each sub-configuration including the CSI-RS configuration. And for each sub-configuration, the counting of antenna ports should be derived from the port subset indication.

Proposal 11:

The counting of CSI-RS resource should be based on the number of sub-configurations including the corresponding CSI-RS resource.

- The counting of antenna ports in a CSI-RS resource should be derived from the port subset indication for corresponding CSI-RS resource.

- Considerations on power domain only adaptation

3.1 CSI-RS resource configuration for PD adaptation

For the PD adaptation, there is no 2 types of adaptation defined as that of SD adaptation. And the following agreement was agreed in RAN1 #112bis-e.

Agreement For power domain adaptation, support the following configuration(s) for CSI-RS resource configuration, - A1-2-power: one or more resources can be configured in a resource set within a resource setting and each resource can be associated with one or more power offset values - FFS: A1-1-power: a resource set with multiple resources is configured within a resource setting, where resources can have different power offset values - FFS: Details of how the different power offset values(s) are configured/indicated.

For A1-2-power, multiple power offset can be configured in a resource set. How to configure the different PO values still need further discussion. The simplest method is to configure a list of PO values for each CSI-RS resource. But if multiple PO values are configured in CSI-RS resource configuration, the power offset are to be indicated again in CSI-ReportConfig. Therefore, to make the procedure simpler, we think a range of PO values can be provided in the CSI-RS resource configuration instead of a list of PO values, then the PO used for CSI measurement can be further configured in the CSI report configuration. For example, in the CSI-RS resource configuration, a central PO POffset and the maximum PO shift ∆PO can be configured, then the available PO value range should be [POffset-∆PO, POffset+∆PO].

Proposal 12: For PD adaptation, a range of available PO values can be configured in the CSI-RS configuration.

3.2 CSI report sub-configuration for PD adaptation

Before discussing the mechanism of sub-configuration of PD adaptation, we need to discuss the definition of sub-configuration for PD adaptation. Within the sub-configuration of SD adaptation, the comprised CSI-RS resources have the same number of antenna ports. And for sub-configuration of SD adaptation for A1-2-revised, all the sub-configurations should be associated with all the CSI-RS resources.

While for the PD adaptation, there is no restriction that all the CSI-RS resources in a CSI-RS resource should have the same PO. Thus, even if the A1-2-power is supported, there shouldn’t be the restriction that all the sub-configuration should be associated with all the CSI-RS resources.

Observation 6:

The restriction that all the sub-configuration associated with all the CSI-RS resources shouldn’t be applied for PD adaptation.

If the sub-configuration for PD is needed, the definition of sub-configuration can be similar to that of SD adaptation, i.e., one or more CSI-RS resources from a CSI-RS resource set with the same available power offset value for channel measurement can be associated with the same sub-configuration provided in a CSI report configuration. In some cases, there may be just on CSI-RS resource in the sub-configuration. This seems like a waste of CSI report configuration. If we look back to why introducing the concept, the motivation is to reduce the configuration times if there are common features among the CSI-RS resources. Without sub-configuration mechanism, the CSI-RS with unique power offset still need to configured alone. Therefore, such condition doesn’t bring extra burden for the CSI report configuration.

Figure 3 illustration of the PD only sub-configuration

Proposal 13:

For PD adaptation, one or more CSI-RS resources from a CSI-RS resource set with the same available power offset value for channel measurement can be associated with the same sub-configuration provided in a CSI report configuration.

Unlike the sub-configuration for SD adaptation, when further think about which parameters should be included in the sub-configuration, we are to find there is actually no more extra parameters except the power offset. Thus, we wonder whether there should be a sub-configuration for PD adaptation. For P-CSI, list all the potential power offset expected to be transmitted in the CSI-ReportConfig is enough. For A/SP-CSI, the power offset can be associated with the CSI-AperiodicTriggerStateList for the triggering of CSI measurement. Thus, we think the sub-configuration for PD adaptation may actually not needed。

Observation 7:

For PD adaptation, without the sub-configuration, the same result can be achieved without extra configuration overhead.

- The sub-configuration for PD adaptation can be formed naturally according to the PO values, for each PO value in CSI-ReportConfig, all the CSI-RS with the same corresponding PO configuration can be associated with the PO.

- Considerations on SD and PD joint adaptation

It was agreed that SD and PD adaptation can be jointly adopted.

Agreement Joint operation of SD and PD adaptation is supported.

When consider the SD and PD adaptation at the same time, the situation can be more complexity.

4.1 SD Type 1 and PD joint adaptation

4.1.1 CSI-RS resource configuration

There may be two options for configuring the SD and PD adaptation at the same time when SD Type 1 is adopted.

- Option 1: for each adaptation pattern associated with CSI-RS resource, multiple PO is configured.

- Option 2: for a CSI-RS resource, multiple PO values are configured.

If Option 1 is adopted, the CSI-RS resource configuration framework should be enhanced and will be complicated. The number or ports associated with the CSI-RS resource must be listed. If Option 2 is adopted, the number of actual CSI-RS configured can be enlarged. For example, CSI-RS resource #1 are associated with both 16 and 32 ports patterns, PO values{x1,x2} are needed for 32 ports pattern, PO values {x3,x4} are needed for 16 ports; then the PO values{x1,x2,x3,x4} should be configured for the whole CSI-RS resource, which means there are actually 4*2=8 kind of configurations. However, without the CSI-ReportConfig, the CSI-RS resource configurations are nothing. With sub-configurations, the number of antenna ports and corresponding PO can be indicated. Thus, for reducing the specs impact on enhancing the CSI-RS resource configuration framework.

Proposal 14:

For SD type 1 and PD jointly adaptation, for each CSI-RS resource configuration, the expected PO values for different SD adaptation patterns can be configured together.

- The indication on specific association of PO values and SD adaptation patterns can be further indicated in the sub-configuration in the CSI-ReportConfig.

4.1.2 CSI Report sub-configuration

If all the combinations of SD and PD adaptations are included in the sub-configurations, there can be too many sub-configurations, which may even exceed the maximum candidate number of AperiodicTriggerState. Since the motivation of SD and PD joint adaptation is to configured multiple CSI with different antenna ports and power offset at the same time. We prefer the sub-configurations based on the subset of all the possible combinations of different number of antenna ports and PO values. The main category of sub-configuration can be based on the number of antenna ports. Then, for each number of antenna ports, several combinations of PO values can be configured, each combination associated with a certain number of antenna ports can be a sub-configuration. For limiting the total number of sub-configurations, there could be a maximum of PO values limitation. In each sub-configuration, the potential PO values for all the associated CSI patterns can be listed together. Then if the sub-configuration is triggered, for each CSI-RS antenna pattern, only need to transmit the CSI-RS with the power corresponding to the RO values indicated in the sub-configuration that has been configured in the CSI-RS resource configuration. The procedure is illustrated in figure 4. In this case, the maximum number of sub-configuration with the same number of antenna ports is limited to 2, then there are 4 sub-configurations in total, while the maximum number of potential sub-configurations is 6.

Figure 4 illustration of sub-configuration for SD Type 1 and PD joint adaptation.

Proposal 15:

For the SD type 1 and PD joint adaptation, the sub-configurations should be a subset of all the possible combinations of number of antenna ports and permutation of PO values corresponding to the CSI-RS included.

- The number of antenna ports should be the main standard for sub-configuration, i.e., there should be at least one sub-configuration corresponding to each number of antenna ports, all the sub-configuration should be associated with all the CSI-RS resources.

- The maximum number of sub-configuration with the same antenna ports should be limited to M.

  - FFS: the value of M.

4.2 CSI report sub-configuration for SD type 2 and PD joint adaptation

While for the SD type 2 and PD joint adaptation, since each CSI-RS resource is only associated with one antenna pattern, the CSI-RS resource configuration is actually the same as PD adaptation.

For the sub-configuration of such condition, the same principle of SD type 1 and PD joint adaptation can be reused. The only difference is that each sub-configurations are not to be associated with all the CSI-RS resources.

Proposal 16:

For SD type 2 and PD joint adaptation,

- The CSI-RS configuration mechanism can be the same as that of PD adaptation.

- The principle of CSI report sub-configuration can be the same as that of SD type1 and PD joint adaptation.

- Beam management and TCI configuration enhancement

In the last meeting, the following options on the BM and TCI were given and to be down-selected in this meeting.

Downselect one of the following for BM enhancements in RAN1#114 - Case 1: Support scaling the threshold of beam failure detection and threshold of candidate beam identification for power domain network energy saving - Case 2: Support UE to send hypothetical beam failure and/or radio link failure (RLF) reports for the indicated hypothetical power offset values. - Case 3: No further work on BM enhancements Downselect one of the following for TCI configuration enhancement in RAN1#114 - Method 1: Configure multiple candidate CSI-RS resources as reference signal for QCL information or for spatial relation information, and switch one of them based on L1/L2 signaling - Method 2: Configure multiple candidate sets of TCI state(s) associated with DL/UL signal/channel and switch one of them based on L1/L2 signaling - Method 3: No further work on TCI configuration enhancement

5.1 Beam management

With PD NES techniques, the beam management should be enhanced since the performance of radio link can be changed due to the variation of transmission power. For case 1, how to scale the threshold of BFD may need further discussion. Besides, it is difficult to distinguish the scale with the change caused by the variation of the wireless channel. While for case2, with UE sending the hypothetical beam failure and RLF reports with the hypothetical PO, the results can be more reasonable since it is acquired according to the current radio link quality. And no more spec impacts on how to define the calculation method is needed. Therefore, we prefer case 2 for the BM enhancement.

Proposal 17:

Support UE to send hypothetical beam failure and/or radio link failure (RLF) reports for the indicated hypothetical power offset values for BM enhancement.

5.2 TCI state

While for TCI state, since the QCL can be changed with SD adaptation, the enhancement is also needed. In the TCI configuration in current specs, each TCI state is associated with multiple QCL type, and each QCL type is associated with one resource. Mechanism of method 1 seems more correspond with the logic of current specs. therefore, we prefer Method 1 for TCI configuration enhancement.

Proposal 18:

Support to configure multiple candidate CSI-RS resources as reference signal for QCL information or for spatial relation information, and switch one of them based on L1/L2 signaling for TCI configuration enhancement.

- Conclusion

In this paper, our views on the issues for network energy saving techniques are given.

Observation 1:

In the current specs, the CSI-RS patterns with less ports can be seen as the subset of CSI-RS patterns with more ports but the same “Density” and “cdm-Type”, which can be used for realizing the “nested” structure that can associate one CSI-RS configuration with multiple pattern.

Proposal 1:

For A1-2-revisied applied for SD adaptation Type 1, nroPorts should be regarded as the maximum expected number of antenna ports when nested’ structure is adopted

- It should be indicated whether SD adaption Type 1 is adopted in the CSI-RS resource configuration for distinguishing the meaning of nroPorts.

Observation 2:

Explicitly indicating whether SD adaptation Type1 is adopted can avoid the confusion when the Rel-17 CSI-RS configuration is adopted at same time.

Observation 3:

If the SD adaptation Type 1 and Type 2 are supposed to be adopted at the same time, all the CSI-RS should be configured as A1-1-revised.

Proposal 2:

For Type 1 SD adaptation with A1-2-revisied, for each sub-configuration, support to indicate the number of the antenna ports, and the following configurations can be omitted for the case when the number of ports is less than 4.

Observation 4:

For Type 1 SD adaptation with A1-2-revisied, for sub-configuration with less number of ports, the configuration of N1, N2 and Ng is not needed.

Proposal 3

For Type 1 SD adaptation with A1-2-revisied, the common configuration of N1, N2 and Ng can be used in sub-configuration if the number of antenna ports is indicated.

Proposal 4:

For Type 2 SD adaptation with A1-1-revisied, codebookConfig and reportFreqConfiguration should be separately configured for different report sub-configuration.

Proposal 5:

For Type 2 SD adaptation with A1-1-revisied, the CQI table indication and Report quantitiy should be commonly configured for different report sub-configuration.

Proposal 6:

Support to indicate whether the CSI should be reported jointly in the CSI-ReportConfig explicitly.

Proposal 7:

Support to introduce a bitmap for indicating whether each measurement in the reportQuantity can be reported jointly.

Proposal 8:

Which CSIs to be jointly reported should be configured in the CSI-ReportConfig.

Proposal 9:

Each CSI report sub-configuration should have a unique ID so that selection can be indicated.

- The ID can be allocated in the CSI report configuration or reuse the corresponding number of antenna ports.

Observation5:

The subsets of L sub-configurations can be furthered indicated with a list of sub-configuration ID or a bitmap each bit corresponding to a sub-configuration.

Proposal 10:

For both SD/PD only adaptation and SD/PD jointly adaptation, without UE complexity reduction,

- For A/SP-CSI reporting, the CPU counting should be based on OCPU=i=1NKsi

- For P-CSI reporting, the CPU counting should be based on OCPU=i=1LKsi

Proposal 11:

The counting of CSI-RS resource should be based on the number of sub-configurations including the corresponding CSI-RS resource.

- The counting of antenna ports in a CSI-RS resource should be derived from the port subset indication for corresponding CSI-RS resource.

Proposal 12:

For PD adaptation, a range of available PO values can be configured in the CSI-RS configuration.

Proposal 13:

For PD adaptation, one or more CSI-RS resources from a CSI-RS resource set with the same available power offset value for channel measurement can be associated with the same sub-configuration provided in a CSI report configuration.

Observation 7:

For PD adaptation, without the sub-configuration, the same result can be achieved without extra configuration overhead.

- The sub-configuration for PD adaptation can be formed naturally according to the PO values, for each PO value in CSI-ReportConfig, all the CSI-RS with the same corresponding PO configuration can be associated with the PO.

Proposal 14:

For SD type 1 and PD jointly adaptation, for each CSI-RS resource configuration, the expected PO values for different SD adaptation patterns can be configured together.

- The indication on specific association of PO values and SD adaptation patterns can be further indicated in the sub-configuration in the CSI-ReportConfig.

Proposal 15:

For the SD type 1 and PD joint adaptation, the sub-configurations should be a subset of all the possible combinations of number of antenna ports and permutation of PO values corresponding to the CSI-RS included.

- The number of antenna ports should be the main standard for sub-configuration, i.e., there should be at least one sub-configuration corresponding to each number of antenna ports, all the sub-configuration should be associated with all the CSI-RS resources.

- The maximum number of sub-configuration with the same antenna ports should be limited to M.

  - FFS: the value of M.

Proposal 16:

For SD type 2 and PD joint adaptation,

- The CSI-RS configuration mechanism can be the same as that of PD adaptation.

- The principle of CSI report sub-configuration an be the same as that of SD type1 and PD joint adaptation.

Proposal 17:

Support UE to send hypothetical beam failure and/or radio link failure (RLF) reports for the indicated hypothetical power offset values for BM enhancement.

Proposal 18:

Support to configure multiple candidate CSI-RS resources as reference signal for QCL information or for spatial relation information, and switch one of them based on L1/L2 signaling for TCI configuration enhancement.

- References

3GPP RP-223540, “New WI: Network energy savings for NR”, Huawei, RAN#98-e, Dec 12th- 16th, 2022.

3GPP TR 38.864 v18.0.0, “Study on network energy savings for NR”, Dec, 2022.

3GPP RAN1 Chairman’s Notes, RAN1 meeting #112bis-e, Apr 17th – 26th, 2023.

3GPP RAN1 Chairman’s Notes, RAN1 meeting #113, May 21st – 26th, 2023.

3GPP TS 38.211, “Physical channels and modulation (Release 17)”.

3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification (Release 17)”.

3GPP TS 38.214, “Physical layer procedures for data (Release 17)”.