3GPP TSG RAN WG1#114															         R1-2307833

Toulouse, France, Aug 21th – Aug 25th, 2023

Source:	Moderator (CMCC), vivo

Title:	draft TP for LP-WUS coverage results for TR38.869

Agenda Item:	9.11.1

Document for:	Discussion and Decision

Change History

| Version | Author | Description |
| --- | --- | --- |
| V001 | Feature lead | https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_113/Inbox/drafts/9.11(FS_NR_LPWUS)/9.11.1/Evaluation%20Results/Coverage%20excel%20sheet/Coverage%20excel%20sheet%20V35_mtk2_SONY.xlsx |
| V002 | Feature lead | https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_113/Inbox/drafts/9.11(FS_NR_LPWUS)/9.11.1/Evaluation%20Results/Coverage%20excel%20sheet/Coverage%20excel%20sheet%20V46_SS3_Ericsson2.zip |
| V004 | Feature lead | https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_113/Inbox/drafts/9.11(FS_NR_LPWUS)/9.11.1/Evaluation%20Results/Coverage%20excel%20sheet/Coverage%20excel%20sheet%20V49_HW%26HiSi2_CATT.xlsx - Main changes include - Incorporate updated results from MTK, HW and CATT - Address comments from companies - Add observation for the cases which cannot achieve comparable coverage - Add back reference MIL for legacy channels - Clarify simulation assumptions in the notes in multiple places |
| V005 | Feature lead | https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_113/Inbox/drafts/9.11(FS_NR_LPWUS)/9.11.1/Evaluation%20Results/Coverage%20excel%20sheet/Coverage%20excel%20sheet%20V52_HW%26HiSi_vivo.xlsx - Main changes include - Added Huawei's Urban redcap, Rural normal UE, and Rural redcap results, and updated observations and bar charts accordingly - Corrected the information in the correction factor table for HUAWEI, and MTK. - Added vivo 2 results, which reflect the information related to X, Y, and array gain in the large table above, as well as in the bar charts of each chapter, but not in the observation - Added the results of ref NR channel Samsung |

Introduction

This document summarize Coverage evaluation results for LP-WUS. The template for evaluation results is

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_113/Inbox/drafts/9.11(FS_NR_LPWUS)/9.11.1/Evaluation%20Results/Coverage%20excel%20sheet

____________________________Start of Text Proposal for TR38.869_______________________________

Evaluation Results

…

Coverage

Comparison between LP-WUS and NR reference channel

<Editor’s Note: Further check correction factor in link budget template>

When summarizing the inputs for coverage evaluation, it is found that the correction factor is not aligned which may impact the compassion between NR channel and LP-WUS. Hence, the moderator suggests companies to further look into it.

- Correction factor for Transmitter Array gain and receiver array gain.

According to TR 38.830(output of R17 coverage enhancement SI), Array gain correction factors are explained as follows

Figure 4.1-1: gNB antenna gain modelling for TDL channel model option 1 Antenna gain component 2 = 10 * log 10(N/k)-1, where 1 is a correction factor to account for various non-idealities impacting the actual gain of antenna gain component 2, and reported by companies. For TDL channel model option 1, option 2 and CDL channel model, the gain of antenna gain components 3 and 4 is expressed by: Antenna gain component 3 + Antenna gain component 4 = Antenna Element Gain + 10 * log 10(M/N) -2. where 2 is a correction factor to account for various non-idealities impacting the actual gain of antenna gain component 3, and reported by companies.

In link budget template, the correction factor should be provided in the following rows

- (4b) antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB)      (corresponding to 2 at transmitter)

- (5b) antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink            (corresponding to 1 at transmitter)

- (11b) antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB)       (corresponding to 2 at receiver)

- (11bis-b) antenna gain correction factor at antenna gain component 2 of receiver (dB) Note:  zero for downlink       (corresponding to 1 at receiver)

However, moderator observes that, paging PDCCH, WUS and Msg3 are not aligned within one company, which may impact the coverage comparison between LP-WUS and reference NR channel. The correction factors for Urban scenario from companies are summarized in the following table.

|  | Urban |  |  |  | Rural |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Company | PDCCH | WUS | Msg3 | PUSCH | PDCCH | WUS | Msg3 | PUSCH |
| E/// | 6 | 6 | 4 | NA | NA | NA | NA | NA |
| vivo | 2.65+8 | 2.65+8 | Vivo: 2.65+8 Vivo2: 2.65+4 | 0 | 2.65 | 2.65 | 2.65 | 0 |
| Huawei | NA | 10.06 | 4.04 | NA | NA | NA | NA | NA |
| Nokia | 5.77 | NA | 5.77 | 5.77 | 5.68 | NA | 5.68 | 5.68 |
| OPPO | 1.5 | 1.5 | NA | 0 | NA | NA | NA | NA |
| Xiaomi | 5.77 | 3 | 5.77 | 5.77 | 3 | 3 | 0 | NA |
| QC | 8.05 | 8 | 3 | NA | 0 | 0 | 0 | NA |
| MTK | 8 | 8 | 4.04 | NA | NA | NA | NA | NA |
| CATT | 8 | 8 | 8 | NA | NA | NA | NA | NA |
| ZTE | 2.95 | 2.95 | 2.95 | NA | 7.55 | 7.55 | 7.55 | NA |
| interDigital | 5.77 | 2.65+8 | 0 | NA | 5.68 | 2.65 | 0 | NA |
| Samsung | 5.77 | 5.77 | 5.77 | NA | NA | NA | NA | NA |

Notes：2.65+8 means 1+2, and both 1 and 2 are provided with none-zero value.

7 companies assume same correction factors for WUS and reference NR channel, while 4 companies use different correction factors.

Moderator suggests companies to further look into it.

Summary of the performance gap between LP-WUS and NR reference channel

The results are summarized in section 8.2.2 and section 8.2.3. A subset of the results are selected to draw observations for comparison between coverage of LP-WUS and reference NR channel. The selection and comparison are performed based on following criterion.

- The relative difference for MIL gap between LP-WUS and NR channel is calculated per each company.

  - If companies only report MIL of NR reference channel or only MIL of LP-WUS, no performance gap can be derived from input from this company.

  - If company only input Msg3 results, and PDCCH results is not provided, only performance gap between LP-WUS and MSG3 can be derived from this company.

- To find whether LP-WUS can achieve similar to or better then MIL of reference NR channels per each companies’ results and then observe the amount of resources required per bit in LP-WUS required for LP-WUS, the following is considered,

- Only when (MIL of LP-WUS – MIL of NR channel)>= -1dB, the LP-WUS sample is considered to have similar to or better than reference NR channels.

- If there are multiple samples fulfil above condition from one company, only the sample with minimum resource per bit is kept to draw conclusion.

- The resource required per bit is provided in terms of X MHz*Symbol/bit. For example, if X= 8.64 and WUS BW is 4.32MHz, it means 2 OFDM symbols are required for LP-WUS to achieve MIL similar to or better then that of NR channel.

- If no samples can fulfil the above condition, the observation is not provided.

The summary of the observation is provided in the following Table 8.2 -1. In each case, only the samples fulfil above condition are collected, where company A: [X (MHz*Symbol/bit), Y (dB)] means company A results show LP-WUS configuration with X MHz*Symbol/bit can achieve MIL similar to or better than MIL of reference channel, and the MIL of LP-WUS – MIL of ref channel is Y dB.

Table 8.2.1 -1 Summary of the performance gap between LP-WUS and NR reference channel

| Company: [X (MHz*Symbol/bit), Y (dB)] |  |  | OOK |  | FSK |  | OFDM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results | Observation | Results | Observation | Results | Observation |
| Urban | Normal UE | AL16,4Rx | - | 8.2.2.1 | - | 8.2.2.5 | - | 8.2.2.9 |
|  |  | AL8,4Rx | ZTE: [46.08, -0.4] | 8.2.2.1 - X = 46.08, Y = -0.4, reported by 1 source. | - |  | ZTE: [34.56, -0.33] SS: [0.31, 2.08] | 8.2.2.9 - X = 34.56, Y = -0.33, reported by 1 source. - X = 0.31, Y = 2.08, reported by 1 source. |
|  |  | AL16,2Rx | - |  | - |  | E///: [17.28, -0.71] SS: [0.31, 2.18] | 8.2.2.9 - X = 17.28, Y = -0.71, reported by 1 source. - X = 0.31, Y = 2.18, reported by 1 source. |
|  |  | AL8,2Rx | E///: [604.8, 0.91] vivo: [28.8, -0.53] | 8.2.2.1 - X = 28.8, Y = -0.53, reported by 1 source. - X = 604.8, Y = 0.91, reported by 1 source. | - | 8.2.2.5 | E///:[17.28, 1.79] SS: [0.31, 5.38] vivo: [18, 0.02] | 8.2.2.9 - X = 0.31, Y = 5.38, reported by 1 source. - X = 17.28~18, Y = 0.02~1.79, reported by 2 sources. |
|  |  | Msg3 | E///: [604.80, 1.94] (Msg3 with 4dB HARQ gain) Interdigital: [8.64, 11.97] Huawei: [4.32, 1.39] MTK: [17.28, 0.06] Samsung: [3.24, 3.23] vivo: [1.8, 2.2] ZTE: [2.88, 0.78] CATT: [8.64, 1.79] Vivo2: [4.32, -0.99] (assumes BS array gain correction for LP-WUS 4dB more than Msg3) Vivo: [0.9, 0.68] (ρ=3dB) Xiaomi: [3.24, 1.24] (ρ=3dB) CATT: [4.32, -0.61] (ρ=3dB) | 8.2.2.3 - X = 1.8~8.64, Y = 0.78~11.97, reported by 6 source, no power boosting for LP-WUS; - X = 17.28, Y = 0.06, reported by 1 sources, assumes antenna element gain of LP-WUR is 3dB worse than MR. - X = 604.80, Y = 1.94, reported by 1 sources, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH and no power boost for LP-WUS. - X=0.9~4.32, Y=-0.61~1.24, reported by 3 sources, assuming 3dB power boosting for LP-WUS. | CATT: [4.32, -0.31] Huawei: [12.96, 0.17] MTK: , [25.92, 0.04] | 8.2.2.7 - X=4.32, Y=-0.31, reported by 1 source. - X= 12.96, Y=-0.17 reported by 1 source. - X= 25.92, Y=-0.04, reported by one source, assumes antenna element gain of LP-WUR is 3dB worse than MR. | E///: [17.28, -0.58] (Msg3 with 4dB HARQ gain) Huawei: [1.8, 0.06] MTK: [1.08,-0.24] Samsung: [0.31, 16.01] vivo: [3.6, 4.84] ZTE: [4.32, 2.2] vivo2: [3.6, 0.84] (assumes BS array gain correction for LP-WUS 4dB more than Msg3) | 8.2.2.11 - X= 0.31~4.32, Y=0.06~16.01 reported by 4 sources. - X= 1.08, Y=-0.24, reported by one source, assumes antenna element gain of LP-WUR is 3dB worse than MR. - X=17.28, Y=-0.58, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH. |
|  | Redcap UE | AL16,2Rx | - |  | - |  | - |  |
|  |  | AL8, 2Rx | - |  | - |  | - |  |
|  |  | AL16,1Rx | E///:[604.8, 1.41] QC: [120.96, 1.48] CATT: [4.32, -0.31] | 8.2.2.2 - X=4.32, Y=-0.31, reported by 1 source. - X = 120.96, Y = 1.48, reported by 1 source. - X = 604.8, Y = 1.41, reported by 1 source. | CATT: [4.32, 2.92] | 8.2.2.6 - X=4.32, Y=2.92, reported by 1 source. | E///: [17.28,2.92] vivo: [7.2, -0.48] SS:[0.43, 1.2] | 8.2.2.10 - X=0.43, Y=1.2, reported by 1 source - X=7.2, Y=-0.48, reported by 1 source. - X= 17.28, Y=-2.92, reported by 1 source. |
|  |  | AL8, 1Rx | E///:[ 60.48, -0.89] MTK: [60.48, -0.92] SS:[12.96, -0.02] vivo: [7.2, -0.7] ZTE:[ 5.76, 0.12] CATT: [4.32, 1.32] | 8.2.2.2 - X=4.32~7.2, Y=-0.7~1.32, reported by 3 sources. - X=12.96~60.48, Y=-0.92~-0.02, reported by 3 sources. | CATT: [4.32, -0.12] MTK:[51.84, 0.18] | 8.2.2.6 - X=4.32, Y=-0.12, reported by 1 source. - X= 51.84, Y=0.18, reported by 1 source. | E///: [17.28, 4.99] MTK: [2.16, 0.48] SS: [0.31, 9.28] vivo: [3.6, -0.33] ZTE: [8.64, 0.51] | 8.2.2.10 - X= 0.31~8.64, Y=-0.33~9.28, reported by 4 sources. - X=17.28, Y=4.99, reported by 1 source. |
|  |  | Msg3 | E///: [604.8, 1.94] (Msg3 with 4dB HARQ gain) MTK: [30.24, 2.36] QC: [120.96, 7.17] SS: [3.24, 1.72] vivo: [1.8, 2.2] ZTE: [2.88, 0.78] CATT: [8.64, 1.79] Huawei: [2.16, 22.48] Vivo2:[2.52, 2.24] (assumes BS array gain correction for LP-WUS 4dB more than Msg3) CATT: [4.32, -0.61] (ρ=3dB) vivo: [0.9, 0.68] (ρ=3dB) xiaomi: [3.24, 1.24] (ρ=3dB) vivo2:[1.68, 2.24] (ρ=3dB) (assumes BS array gain correction for LP-WUS 4dB more than Msg3) | 8.2.2.4 - X=1.8~8.64, Y=0.78~22.48, reported by 5 sources, no power boosting for LP-WUS. - X=30.24, Y=2.36, reported by 1 sources, no power boosting for LP-WUS, assumes antenna element gain of LP-WUR is 3dB worse than MR. - X=120.96, Y=7.17, reported by 1 source, no power boosting for LP-WUS. - X=604.8, Y=1.94, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH. - X=0.9~4.32, Y=-0.61~1.24, reported by 3 sources, assuming 3dB power boosting for LP-WUS. | CATT: [4.32, -0.31] MTK: [25.92, 0.04] Huawei: [2.16, 19.1] | 8.2.2.8 - X=2.16~4.32, Y=-0.31~19.1, reported by 2 sources. - X=25.92, Y=-0.01, reported by 1 source, assumes antenna element gain of LP-WUR is 3dB worse than MR. | E///: [17.28, -0.53] (Msg3 with 4dB HARQ gain) MTK: [1.08, -0.24] SS: [0.31, 18.03] vivo: [3.6, 6.84] ZTE: [4.32, 2.2] Huawei: [1.8, 27.79] Vivo2: [3.6, 2.84] (assumes BS array gain correction for LP-WUS 4dB more than Msg3) | 8.2.2.12 - X= 0.31~4.32, Y=2.2~27.79, reported by 4 sources. - X= 1.08, Y=-0.24, reported by 1 source, assumes antenna element gain of LP-WUR is 3dB worse than MR. - X=17.28, Y=-0.53, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH. |
| Rural | Normal UE | AL16,4Rx | - |  | - |  | - |  |
|  |  | AL8,4Rx | - |  | - |  | - |  |
|  |  | AL16,2Rx | vivo: [69.12, -0.96] | 8.2.3.1 - X =69.12, Y =-0.96, reported by 1 source. | - |  | - | 8.2.3.9 |
|  |  | AL8,2Rx | vivo: [28.8, 0.09] ZTE: [46.08, -0.25] | 8.2.3.1 - X =28.8 ~ 46.08, Y =-0.25 ~ 0.09, reported by 2 sources. | - |  | vivo: [7.2, -0.29] ZTE: [34.56, -0.18] | 8.2.3.9 - X =34.56, Y =-0.18, reported by 1 source. - X =7.2, Y =-0.29, reported by 1 source. |
|  |  | msg3 | vivo: [1.8, 1.07] ZTE: [2.88, 4.71] interdigital: [4.32, 15.37] Huawei: [4.32, 1.95] vivo: [0.9, -0.49] (ρ=3dB) xiaomi: [3.02, -0.04] (ρ=3dB) | 8.2.3.3 - X =1.8 ~ 4.32, Y =1.07 ~ 15.37, reported by 4 sources, no power boosting for LP-WUS. - X =0.9 ~ 3.02, Y=-0.49~-0.04, reported by 2 sources, 3dB power boosting for LP-WUS. | Huawei: [4.32, 0.13] | - X =4.32, Y=0.13, reported by 1 source | vivo: [3.6, 3.8] ZTE: [2.16, 1.5] Huawei: [1.8, 0.63] | 8.2.3.11 - X =1.8 ~ 3.6, Y =0.35 to 3.8, reported by 3 sources. |
|  | Redcap UE | AL16,2Rx | - |  | - |  | - |  |
|  |  | AL8, 2Rx | - |  | - |  | - |  |
|  |  | AL16,1Rx | QC: [120.96, 0.92] | 8.2.3.2 - X =120.96, Y =0.92, reported by 1 source | - |  | vivo: [7.2, -0.28] | 8.2.3.10 - X =7.2, Y =-0.28, reported by 1 source. |
|  |  | AL8, 1Rx | vivo: [3.78, -0.32] ZTE: [11.52, -0.62] | 8.2.3.2 - X =3.78 ~ 11.52, Y =-0.62 ~ -0.32, reported by 2 source. | - |  | vivo: [3.6, -0.21] ZTE: [17.28, 0.25] | 8.2.3.10 - X =3.6 ~ 17.28, Y =-0.21 ~ 0.25, reported by 2 source. |
|  |  | Msg3 | QC: [5.76, 6.65] vivo: [1.8, 1.07] ZTE: [2.88, 4.71] Huawei: [2.16, 14.94] Vivo:[0.9, -0.49] (ρ=3dB) Xiaomi: [3.02, -0.04] (ρ=3dB) | 8.2.3.4 - X =1.8 ~ 5.76, Y =1.07 ~ 14.94, reported by 4 sources, no power boosting for LP-WUS. - X=0.9~3.02, Y= -0.49~-0.04, 3dB power boosting for LP-WUS reported by 2 sources. | Huawei: [2.16, 9.46] | - X =2.16, Y=9.46, reported by 1 source | vivo: [3.6, 3.8] ZTE: [2.16, 1.5] Huawei: [1.8, 18.25] | 8.2.3.12 - X =1.8 ~ 3.6, Y =1.5 to 18.25, reported by 3 sources. |

<Editor’s Note: The text for observation part in section 8.2.2 and 8.2.3 will be updated according to the Table 8.2.1 -1 >

NR Coverage for comparison

The NR coverage for comparison are provided in the following tables

Table 8.2.1 - 2 NR Coverage, Urban, Normal UE

|  | Source | Scenarios | AL16, 4Rx | AL8, 4Rx | AL16, 2Rx | AL8, 2Rx | AL16, 1RX | AL8, 1RX | PUSCH eMBB | PUSCH Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal UE | vivo | Urban 2.6GHz |  |  | 154.94 | 152.33 |  |  | 138.28 | vivo:141.85 vivo2:145.85 |
|  | Nokia | Urban 2.6GHz | 166.27 | 164.28 | 162.86 | 160.74 |  |  | 137.62 | 147.20 |
|  | xiaomi | Urban 4GHz |  |  | 162.65 | 159.84 |  |  | 130.69 | 146.77 |
|  | Huawei | Urban 2.6GHz |  |  |  |  |  |  |  | 149.61 |
|  | OPPO | Urban 2.6GHz |  |  |  | 163.54 |  |  | 135.88 |  |
|  | CATT | Urban 2.6GHz | 158.80 |  |  |  |  |  |  | 147.83 |
|  | Ericsson | Urban 2.6GHz | 159.96 | 157.26 | 156.76 | 154.26 |  |  |  | 153.23 |
|  | Samsung | Urban 4GHz | 168.30 | 162.21 | 162.11 | 158.91 |  |  |  | 146.26 |
|  | Interdigital | Urban 2.6Ghz |  |  | 165.55 |  |  |  |  | 135.46 |
|  | ZTE | Urban 2.6GHz |  | 162.54 |  |  |  |  |  | 151.53 |
|  | MTK | Urban 2.6GHz | 160.81 | 158.31 |  |  |  |  | 142.51 | 148.73 |

Note: “vivo2” assumes UL 4dB correction factor better than DL, “vivo” assumes UL and DL same correction factor in coverage evaluation template. For the figures in section 8.2.2 and 8.2.3, the meanings of 'vivo' and 'vivo 2' refer to this.

Table 8.2.1 - 3 NR Coverage, Urban, Redcap UE

|  | Company Name | Scenarios | AL16, 4Rx | AL8, 4Rx | AL16, 2Rx | AL8, 2Rx | AL16, 1RX | AL8, 1RX | PUSCH eMBB | PUSCH Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Redcap UE | vivo | Urban 2.6GHz |  |  |  |  | 148.72 | 146.02 | 134.96 | vivo:138.85 vivo2: 142.85 |
|  | Nokia | Urban 2.6GHz |  |  | 159.86 | 157.74 | 156.61 | 153.17 | 134.62 | 144.20 |
|  | QC | Urban 2.6GHz |  |  |  |  | 152.96 |  |  | 147.27 |
|  | Intel | Urban 2.6GHz |  |  |  |  | 157.81 |  |  |  |
|  | CATT | Urban 2.6GHz |  |  |  |  | 144.60 | 142.90 |  | 147.83 |
|  | Ericsson | Urban 2.6GHz |  |  |  |  | 153.76 | 151.06 |  | 153.23 |
|  | xiaomi | Urban 4GHz |  |  |  |  | 156.40 | 153.11 | 127.69 | 143.77 |
|  | ZTE | Urban 2.6GHz |  |  |  |  |  | 153.38 |  | 148.53 |
|  | MTK | Urban 2.6GHz |  |  |  |  | 154.61 | 152.11 | 142.51 | 148.73 |
|  | Samsung | Urban 4GHz |  |  |  |  | 155.35 | 152.01 |  | 143.26 |
|  | Huawei | Urban 2.6GHz |  |  |  |  |  |  |  | 145.3 |

Note: “vivo2” assumes UL 4dB correction factor better than DL, “vivo” assumes UL and DL same correction factor in coverage evaluation template. For the figures in section 8.2.2 and 8.2.3, the meanings of 'vivo' and 'vivo 2' refer to this.

Table 8.2.1 - 4 NR Coverage, Rural, Normal UE

|  | Company Name | Scenarios | AL16, 4Rx | AL8, 4Rx | AL16, 2Rx | AL8, 2Rx | AL16, 1RX | AL8, 1RX | PUSCH eMBB | PUSCH Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal UE | vivo | Rural 700MHz |  |  | 154.99 | 152.32 |  |  | 143.97 | 143.60 |
|  | Nokia | Rural 700MHz |  |  | 154.73 | 152.75 |  |  | 138.97 | 140.20 |
|  | xiaomi | Rural 700MHz |  |  | 157.36 | 154.24 |  |  | 134.78 | 140.76 |
|  | Interdigital | Rural 700Mhz |  |  |  |  | 154.35 |  |  | 133.27 |
|  | ZTE | Rural 700MHz |  |  |  | 150.00 |  |  |  | 135.23 |
|  | Huawei | Rural 700MHz |  |  |  |  |  |  |  | 118.88 |

Table 8.2.1 - 5 NR Coverage, Rural, Redcap UE

|  | Company Name | Scenarios | AL16, 4Rx | AL8, 4Rx | AL16, 2Rx | AL8, 2Rx | AL16, 1RX | AL8, 1RX | PUSCH eMBB | PUSCH Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Redcap UE | vivo | Rural 700MHz |  |  |  |  | 149.31 | 146.61 | 140.97 | 140.60 |
|  | Nokia | Rural 700MHz |  |  | 151.73 | 149.75 | 149.45 | 145.48 | 135.97 | 136.09 |
|  | QC | Rural 700MHz |  |  |  |  | 153.72 |  |  | 138.99 |
|  | xiaomi | Rural 700MHz |  |  |  |  | 150.59 | 146.80 | 131.78 | 137.76 |
|  | ZTE | Rural 700MHz |  |  |  |  |  | 143.46 |  | 132.23 |
|  | Huawei | Rural 700MHz |  |  |  |  |  |  |  | 124.67 |

Results for Urban

OOK, Urban, normal UE, PDCCH

Table 8.2.2.1 - 1 Gap with PDCCH AL16, 4RX, Urban, MR is normal UE

No samples with MIL comparable to corresponding NR reference channel is provided.

Observation:

‐	[?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

Table 8.2.2.1 - 2 Gap with PDCCH AL8, 4RX, Urban, MR is normal UE

- Observation

:Observation:

For Normal UE in Urban, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8, 4RX for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X = 46.08, Y  = -0.4, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

Table 8.2.2.1 - 3 Gap with PDCCH AL8, 2RX, Urban, MR is normal UE

Observation:

For Normal UE in Urban, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better)  than PDCCH AL8, 2RX for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X = 28.8, Y  = -0.53, reported by 1 source.

- X = 604.8, Y  = 0.91, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

OOK, Urban, Redcap UE, PDCCH

Table 8.2.2.2 - 1 Gap with PDCCH AL16, 1RX

Observation:

For RedCap UE in Urban, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL16, 1RX for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=4.32, Y=-0.31, reported by 1 source.

- X = 120.96, Y = 1.48, reported by 1 source.

- X = 604.8, Y = 1.41, reported by 1 source.

- [3] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

Table 8.2.2.2 - 2 Gap with PDCCH AL8, 1RX

Observation:

For RedCap UE in Urban, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8, 1RX  for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=4.32~7.2, Y=-0.7~1.32, reported by 3 sources,.

- X=12.96~60.48, Y=-0.92~-0.02, reported by 3 sources.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

OOK, Urban, Normal UE, Msg.3

Table 8.2.2.3 OOK, Urban, Normal UE, Msg.3

Observation:

For Normal UE in Urban, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better)  than PUSCH for Msg.3 when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X = 1.8~8.64, Y  = 0.78~11.97, reported by 6 source, no power boosting for LP-WUS;

- X = 17.28, Y  = 0.06, reported by 1 sources, assumes antenna element gain of LP-WUR is 3dB worse than MR.

- X = 604.80, Y  = 1.94, reported by 1 sources, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH and no power boost for LP-WUS.

- X=0.9~4.32, Y=-0.61~1.24, reported by 3 sources, assuming 3dB power boosting for LP-WUS.

OOK, Urban, Redcap UE, Msg.3

Table 8.2.2.4 OOK, Urban, Redcap UE, Msg.3

Observation:

For Redcap UE in Urban, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PUSCH for Msg.3 when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=1.8~8.64, Y=0.78~22.48, reported by 5 sources, no power boosting for LP-WUS.

- X=4.3230.24, Y=2.36, reported by 1 sources, no power boosting for LP-WUS, assumes antenna element gain of LP-WUR is 3dB worse than MR.

- X=120.96, Y=47.17, reported by 1 source, no power boosting for LP-WUS.

- X=604.8, Y=1.94, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH.

- X=0.9~4.32, Y=-0.61~1.24, reported by 3 sources, assuming 3dB power boosting for LP-WUS.

FSK, Urban, Normal UE, PDCCH

Table 8.2.2.5 - 1 Gap with PDCCH AL16, 4RX

No samples with MIL comparable to corresponding NR reference channel is provided.

Observation:

‐	[?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

Table 8.2.2.5 - 2 Gap with PDCCH AL8, 2RX

No samples with MIL comparable to corresponding NR reference channel is provided.

Observation:

‐	[?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

FSK, Urban, Redcap UE, PDCCH

Table 8.2.2.6 - 1 Gap with PDCCH AL16, 1RX

Observation:

For Redcap UE in Urban, based on the results, FSK based LP-WUS is feasible to achieve comparable coverage (YdB better) than Paging PDCCH AL16 1Rx when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=4.32, Y=2.92, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

Table 8.2.2.6 - 2 Gap with PDCCH AL8, 1RX

Observation:

For Redcap UE in Urban, based on the results, FSK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8 1RX for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=4.32, Y=-0.12, reported by 1 source .

- X= 51.84, Y=0.18, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

FSK, Urban, Normal UE, Msg.3

Table 8.2.2.7 Gap with Msg.3

Observation:

For Normal UE in Urban, based on the results, FSK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PUSCH for Msg.3 when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=4.32, Y=-0.31, reported by 1 source.

- X= 12.96, Y=-0.17 reported by 1 source.

- X= 25.92, Y=-0.04, reported by 1 source, assumes antenna element gain of LP-WUR is 3dB worse than MR.

- [1] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

FSK, Urban, Redcap UE, Msg.3

Table 8.2.2.8 FSK, Urban, Redcap UE, Msg.3

Observation:

For Redcap UE in Urban, based on the results, FSK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PUSCH for Msg.3 when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=2.16~4.32, Y=-0.31~19.1, reported by 2 sources.

- X=25.92, Y=-0.01, reported by 1 source, assumes antenna element gain of LP-WUR is 3dB worse than MR.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

OFDM, Urban, Normal UE, PDCCH

Table 8.2.2.9-1 Gap with PDCCH AL16, 4RX

No samples with MIL comparable to corresponding NR reference channel is provided.

Table 8.2.2.9-2 Gap with PDCCH AL16, 2RX

Observation:

For Normal UE in Urban, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL16 2Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X = 17.28, Y = -0.71, reported by 1 source.

- X = 0.31, Y = 2.18, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

Table 8.2.2.9-3 Gap with PDCCH AL8, 4RX

Observation:

For Normal UE in Urban, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8 4Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =34.56, Y =-0.33, reported by 1 source.

- X = 0.31, Y  = 2.08, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

Table 8.2.2.9-4 Gap with PDCCH AL8, 2RX

Observation:

For Normal UE in Urban, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8 2Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X = 0.31, Y  = 5.38, reported by 1 source.

- X = 17.28~18, Y  = 0.02~1.79, reported by 2 sources.

OFDM, Urban, Redcap UE, PDCCH

Table 8.2.2.10-1 Gap with PDCCH AL16, 1RX

Observation:

For Redcap UE in Urban, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL16 1Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=0.43, Y=1.2, reported by 1 source

- X=7.2, Y=-0.48, reported by 1 source.

- X= 17.28, Y=-2.92, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

Table 8.2.2.10-2 Gap with PDCCH AL8, 1RX

Observation:

For Redcap UE in Urban, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8 1Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X= 0.31~8.64, Y=-0.33~9.28, reported by 4 sources.

- X=17.28, Y=4.99, reported by 1 source.

OFDM, Urban, Normal UE, Msg.3

Table 8.2.2.11 Gap with PUSCH Msg.3

Observation:

For Normal UE in Urban, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PUSCH for Msg.3 when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X= 0.31~4.32, Y=0.06~16.01 reported by 4 sources.

- X= 1.08, Y=-0.24, reported by one source, assumes antenna element gain of LP-WUR is 3dB worse than MR.

- X=17.28, Y=-0.58, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH.

OFDM, Urban, Redcap UE, Msg.3

Table 8.2.2.12 OFDM, Urban, Redcap UE, Msg.3

Observation:

For Redcap UE in Urban, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PUSCH for Msg.3 when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X= 0.31~4.32, Y=2.2~27.79, reported by 4 sources.

- X= 1.08, Y=-0.24, reported by 1 source, assumes antenna element gain of LP-WUR is 3dB worse than MR.

- X=17.28, Y=-0.53, reported by 1 source, assuming 4dB HARQ gain from 2 retransmissions for Msg3 PUSCH.

Results for Rural

OOK, Rural, normal UE, PDCCH

- Gap with AL16, 2Rx

Observation:

For Normal UE in Rural, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL16 2Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =69.12, Y =-0.96, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

- Gap with AL8, 2Rx

Observation:

For Normal UE in Rural, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8 2Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =28.8 ~ 46.08, Y =-0.25 ~ 0.09, reported by 2 sources.

- [1] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

OOK, Rural, Redcap UE, PDCCH

- Gap with AL16, 1Rx

Observation:

For Redcap UE in Rural, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL16 1Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =120.96, Y =0.92, reported by 1 source.

- [?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

- Gap with AL8, 1Rx

Observation:

For Redcap UE in Rural, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8 1Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =3.78~11.52, Y =-0.62 ~ -0.32, reported by 2 source.

- [1] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

OOK, Rural, Normal UE, Msg.3

Observation:

For Normal UE in Rural, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than MSG3 PUSCH when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =1.8 ~ 4.32, Y =1.07 ~ 15.37, reported by 4 sources, no power boosting for LP-WUS.

- X =0.9 ~ 3.02, Y=-0.49~-0.04, reported by 2 sources, 3dB power boosting for LP-WUS.

OOK, Rural, Redcap UE, Msg.3

Observation:

For Redcap UE in Rural, based on the results, OOK based LP-WUS is feasible to achieve comparable coverage (YdB better) than MSG3 PUSCH when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =1.8 ~ 5.76, Y =1.07 ~ 14.94, reported by 4 sources, no power boosting for LP-WUS.

- X=0.9~3.02, Y= -0.49~-0.04, reported by 2 sources, 3dB power boosting for LP-WUS.

FSK, Rural, normal UE, PDCCH

No input

FSK, Rural, Redcap UE, PDCCH

No input

FSK, Rural, Normal UE, Msg.3

Observation:

For Normal UE in Rural, based on the results, FSK based LP-WUS is feasible to achieve comparable coverage (YdB better) than MSG3 PUSCH when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =4.32, Y=0.13, reported by 1 source.

FSK, Rural, Redcap UE, Msg.3

Observation:

For Redcap UE in Rural, based on the results, FSK based LP-WUS is feasible to achieve comparable coverage (YdB better) than MSG3 PUSCH when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X=2.16, Y=9.46, reported by 1 source.

OFDM, Rural, normal UE, PDCCH

- Gap with PDCCH AL16, 2Rx

No samples with MIL comparable to corresponding NR reference channel is provided.

Observation:

‐	[?] sources report none of the simulated configurations of LP-WUS can achieve comparable coverage than the target legacy NR channel.

- Gap with PDCCH AL8 2Rx

Observation:

For Normal UE in Rural, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8 2Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =34.56, Y =-0.18, reported by 1 source.

- X =7.2, Y =-0.29, reported by 1 source.

OFDM, Rural, Redcap UE, PDCCH

- Gap with PDCCH AL16 1Rx

Observation:

For Redcap UE in Rural, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL16 1Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =7.2, Y =-0.28, reported by 1 source.

- Gap with PDCCH AL8 1Rx

Observation:

For Redcap UE in Rural, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than PDCCH AL8 1Rx for paging when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =3.6 ~ 17.28, Y =-0.21 ~ 0.25, reported by 2 source.

OFDM, Rural, Normal UE, Msg.3

Observation:

For Normal UE in Rural, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than MSG3 PUSCH when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =1.8 ~ 3.6, Y =0.35 to 3.8, reported by 3 sources.

OFDM, Rural, Redcap UE, Msg.3

Observation:

For Redcap UE in Rural, based on the results, OFDM based LP-WUS is feasible to achieve comparable coverage (YdB better) than MSG3 PUSCH when the resource occupation for each LP-WUS transmission is at least X MHz*Symbol/bit, where

- X =1.8 ~ 3.6, Y =1.5 to 18.25, reported by 3 sources.