3GPP TSG RAN WG1#114															         R1-23XXXXX

Toulouse, France, Aug 21th – Aug 25th, 2023

Source:	Moderator (CMCC), vivo

Title:	draft TP for LP-WUS overhead evaluation results for TR38.869

Agenda Item:	9.11.1

Document for:	Discussion and Decision

Change History

| Version | Author | Description |
| --- | --- | --- |
| V001 | Feature lead | Based on Others_V006_vivo2_ZTE2.docx |
| V002 | Feature lead | Based on Others_V008_HW_QC.docx |

Introduction

This document summarizes power evaluation results for LP-WUS. The template for evaluation results is

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_113/Inbox/drafts/9.11(FS_NR_LPWUS)/9.11.1/Evaluation%20Results/Other%20results

____________________________Start of Text Proposal for TR38.869_______________________________

Evaluation Results

System overhead

Collection of the results

Evaluation of the LP-WUS and LP-SS overhead are summarized in Table 8.5. And for more results and details, it can be found in Annex B.

RRC IDLE/INACTIVE

LP-WUS

The System overhead of LP-WUS for RRC IDLE/INACTIVE is summarized in Table 8.5.1-1

Table 8.5.1 - 1 Summary of System overhead for LP-WUS for RRC IDLE/INACTIVE

| Source | Signal | # of info bits carried by signal | Overhead (%) | T resources per beam | F resources | SCS | number of pages UE per second or signal periodicity | number of UE per group | System configuration, e.g., BW and etc. | the target channel | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | LP-WUS | 1 | 0.0150% | 1 symbol | 5.04MHz | 30kHz | 10 | 1 | 100MHz, 8beams | Msg.3 | For IoT, cases, Assuming 1,000,000 user/km2 connection density [ITU M.2412], with packet arrival every 1 message/2 hours/device [ITU M.2412], assuming 500m ISD with 3 sectors. The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage. OOK LP-WUS is assumed. |
| vivo | LP-WUS | 1 | 0.0030% | 1 symbol | 5.04MHz | 30kHz | 10 | 4 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 | 0.0600% | 4 symbols | 5.04MHz | 30kHz | 10 | 1 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 | 0.0100% | 4 symbols | 5.04MHz | 30kHz | 10 | 4 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 or 12 | 0.0750% | 5 symbols | 5.04MHz | 30kHz | 10 | 1 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 or 12 | 0.0130% | 5 symbols | 5.04MHz | 30kHz | 10 | 4 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 1 | 0.0730% | 1 symbol | 5.04MHz | 30kHz | 10 | 1 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 1 | 0.0180% | 1 symbol | 5.04MHz | 30kHz | 10 | 4 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 | 0.2900% | 4 symbols | 5.04MHz | 30kHz | 10 | 1 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 | 0.0700% | 4 symbols | 5.04MHz | 30kHz | 10 | 4 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 or 12 | 0.3630% | 5 symbols | 5.04MHz | 30kHz | 10 | 1 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 or 12 | 0.0880% | 5 symbols | 5.04MHz | 30kHz | 10 | 4 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 1 | 0.0550% | 1 symbol | 5.04MHz | 30kHz | 39 | 1 | 100MHz, 8beams | Msg.3 | For eMBB cases, Assuming very high population density, e.g., sports event with 5,000 users/cell population density, and per UE paging arrival rate = 1% when paging i-DRX cycle is Tpagingcycle = 1.28s The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage. OOK LP-WUS is assumed. |
| vivo | LP-WUS | 1 | 0.0150% | 1 symbol | 5.04MHz | 30kHz | 39 | 4 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 | 0.2200% | 4 symbols | 5.04MHz | 30kHz | 39 | 1 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 | 0.0600% | 4 symbols | 5.04MHz | 30kHz | 39 | 4 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 or 12 | 0.2750% | 5 symbols | 5.04MHz | 30kHz | 39 | 1 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 or 12 | 0.0750% | 5 symbols | 5.04MHz | 30kHz | 39 | 4 | 100MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 1 | 0.2800% | 1 symbol | 5.04MHz | 30kHz | 39 | 1 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 1 | 0.0700% | 1 symbol | 5.04MHz | 30kHz | 39 | 4 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 | 1.1200% | 4 symbols | 5.04MHz | 30kHz | 39 | 1 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 | 0.2800% | 4 symbols | 5.04MHz | 30kHz | 39 | 4 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 or 12 | 1.4000% | 5 symbols | 5.04MHz | 30kHz | 39 | 1 | 20MHz, 8beams | Msg.3 |  |
| vivo | LP-WUS | 8 or 12 | 0.3500% | 5 symbols | 5.04MHz | 30kHz | 39 | 4 | 20MHz, 8beams | Msg.3 |  |
| ZTE | LP-WUS | 7 or 24 | <1% | 7 or 14 symbols | 5MHz | 15KHz/30kHz | Paging rate:1%,0.1%,0.01%,0.001%; POs number per PF: 1,2,4; PFs number per cycle: 1,2,4,8,16 | 1,4,8 | 20M, 100M; Beams:1,4 | Msg.3 | Only one case when 20M system bandwidth, 4beams, no grouping, 15KHz, 14OFDM symbols, paging rate 1%, maximum POs per PF, maximum PFs per cycle are configured, the system overhead may reach 1.6%. OOK LP-WUS is assumed. |
| Ericsson | 1-bit OOK WUS | 1 | 0.4500% | 10 slot | 5MHz | 30KHz | 20 Idle mode UEs and paging inter-arrival of 60s seconds |  | 20M, 8beam | Matching with the paging PDCCH link budget (2-OFDM symbol with AL16, 1 Rx UE) |  |
| Ericsson | 48-bit OOK WUS | 48 | 10.0400% | 222 slot | 5MHz | 30KHz |  |  | 20M, 8beam |  |  |
| Ericsson | 1-bit SSS-based WUS | 1 | 0.0100% | 6 symbol | 3.81MHz | 30KHz |  |  | 20M, 8beam |  |  |
| Ericsson | PDCCH AL16, 2 OFDM symbols | - | 0.0028% | 2 OFDM symbols |  | 30KHz |  |  | 20M, 8beam |  |  |
| Ericsson | 1-bit OOK WUS | 1 | 0.3600% | 8 slot | 5MHz | 30KHz |  |  | 20M, 8beam | Matching with the Msg3 PUSCH link budget with 2 retransmissions |  |
| Ericsson | 48-bit OOK WUS | 48 | 8.1400% | 180 slot | 5MHz | 30KHz |  |  | 20M, 8beam |  |  |
| Ericsson | 1-bit SSS-based WUS | 1 | 0.0100% | 5 symbol | 3.81MHz | 30KHz |  |  | 20M, 8beam |  |  |
| Ericsson | PDCCH AL16, 2 OFDM symbols | - | 0.0028% | 2 OFDM symbols |  | 30KHz |  |  | 20M, 8beam |  |  |
| Huawei, HiSilicon | LP-WUS+LP-SS | 24 | 0.1230% | 12 symbols (no repetition) | 5MHz | 30KHz | 50 paging per second |  | 100M, 1 beam |  |  |
| Huawei, HiSilicon | LP-WUS+LP-SS | 24 | 0.2300% | 24 symbols for 2 repetitions | 5MHz | 30KHz | 50 paging per second |  | 100M, 1 beam |  |  |
| Huawei, HiSilicon | LP-WUS+LP-SS | 24 | 0.4440% | 48 symbols for 4 repetitions | 5MHz | 30KHz | 50 paging per second |  | 100M, 1 beam |  |  |
| Huawei, HiSilicon | LP-WUS+LP-SS | 24 | 0.4910% | 12 symbols (no repetition) | 5MHz | 30KHz | 50 paging per second |  | 100M, 4 beams |  |  |
| Huawei, HiSilicon | LP-WUS+LP-SS | 24 | 0.9200% | 24 symbols for 2 repetitions | 5MHz | 30KHz | 50 paging per second |  | 100M, 4 beams |  |  |
| Huawei, HiSilicon | LP-WUS+LP-SS | 24 | 1.7770% | 48 symbols for 4 repetitions | 5MHz | 30KHz | 50 paging per second |  | 100M, 4 beams |  |  |
| Qualcomm | LP-WUS | 6 | 0.0006% | 0.6ms | 5MHz |  | 20 UEs (per UE paging interval is 500s) | 5 | 20MHz |  |  |
| Qualcomm | LP-WUS | 6 | 0.0012% | 1.2ms | 5MHz |  | 20 UEs(per UE paging interval is 500s) | 5 | 20MHz |  |  |
| Qualcomm | LP-WUS | 6 | 0.0030% | 0.6ms | 5MHz |  | 100 UE(per UE paging interval is 500s) | 5 | 20MHz |  |  |
| Qualcomm | LP-WUS | 6 | 0.0060% | 1.2ms | 5MHz |  | 100 UE(per UE paging interval is 500s) | 5 | 20MHz |  |  |
| Qualcomm | LP-WUS | 6 | 0.0004% | 0.6ms | 5MHz |  | 20 UEs(per UE paging interval is 500s) | 5 | 20MHz |  |  |
| Qualcomm | LP-WUS | 6 | 0.0008% | 1.2ms | 5MHz |  | 20 UEs(per UE paging interval is 500s) | 5 | 20MHz |  |  |
| Qualcomm | LP-WUS | 6 | 0.0020% | 0.6ms | 5MHz |  | 100 UE(per UE paging interval is 500s) | 5 | 20MHz |  |  |
| Qualcomm | LP-WUS | 6 | 0.0041% | 1.2ms | 5MHz |  | 100 UE(per UE paging interval is 500s) | 5 | 20MHz |  |  |

LP-SS

The System overhead of LP-SS for RRC IDLE/INACTIVE is summarized in Table 8.5.1-2

Table 8.5.1 - 2 Summary of System overhead for LP-SS for RRC IDLE/INACTIVE

| Source | Signal | Overhead (%) | T resources per beam | F resources | SCS | number of pages UE per second or signal periodicity | System configuration, e.g., BW and etc. | the target channel | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | LP-SS | 0.0630% | 14 symbols | 5.04MHz | 30kHz | 320ms | 100MHz, 8beam | > Msg.3 | The LP-SS coverage is better than msg.3 coverage; The derivation of LP-SS frequency: 4.32MHz +2RB (30kHz) = 5.04MHz |
| vivo | LP-SS | 0.0158% | 14 symbols | 5.04MHz | 30kHz | 1280ms | 100MHz, 8beam | > Msg.3 |  |
| vivo | LP-SS | 0.0020% | 14 symbols | 5.04MHz | 30kHz | 10240ms | 100MHz, 8beam | > Msg.3 |  |
| vivo | LP-SS | 0.3150% | 14 symbols | 5.04MHz | 30kHz | 320ms | 20MHz, 8beam | > Msg.3 |  |
| vivo | LP-SS | 0.0790% | 14 symbols | 5.04MHz | 30kHz | 1280ms | 20MHz, 8beam | > Msg.3 |  |
| vivo | LP-SS | 0.0100% | 14 symbols | 5.04MHz | 30kHz | 10240ms | 20MHz, 8beam | > Msg.3 |  |
| ZTE | LP-SS | 0.125%, | 14 symbols | 5MHz | 15KHz | 160ms | 100M, 4beam | Msg.3 |  |
| ZTE | LP-SS | 0.625%, | 14 symbols | 5MHz | 15KHz | 160ms | 20M, 4beam | Msg.3 |  |
| ZTE | LP-SS | 0.0625%, | 14 symbols | 5MHz | 15KHz | 320ms | 100M, 4beam | Msg.3 |  |
| ZTE | LP-SS | 0.3125% | 14 symbols | 5MHz | 15KHz | 320ms | 20M, 4beam | Msg.3 |  |
| ZTE | LP-SS | 0.0625%, | 14 symbols | 5MHz | 30KHz | 160ms | 100M, 4beam | Msg.3 |  |
| ZTE | LP-SS | 0.3125% | 14 symbols | 5MHz | 30KHz | 160ms | 20M, 4beam | Msg.3 |  |
| ZTE | LP-SS | 0.125%, | 14 symbols | 5MHz | 30KHz | 160ms | 100M, 8beam | Msg.3 |  |
| ZTE | LP-SS | 0.6250% | 14 symbols | 5MHz | 30KHz | 160ms | 20M, 8beam | Msg.3 |  |
| ZTE | LP-SS | 0.03125%, | 14 symbols | 5MHz | 30KHz | 320ms | 100M, 4beam | Msg.3 |  |
| ZTE | LP-SS | 0.1563% | 14 symbols | 5MHz | 30KHz | 320ms | 20M, 4beam | Msg.3 |  |
| ZTE | LP-SS | 0.0625%, | 14 symbols | 5MHz | 30KHz | 320ms | 100M, 8beam | Msg.3 |  |
| ZTE | LP-SS | 0.3125% | 14 symbols | 5MHz | 30KHz | 320ms | 20M, 8beam | Msg.3 |  |
| Huawei, HiSilicon | LP-SS | 0.0160% | 28symbols | 5MHz | 30KHz | 320ms | 100M, 1beam |  |  |
| Huawei, HiSilicon | LP-SS | 0.0630% | 28symbols | 5MHz | 30KHz | 320ms | 100M, 4beam |  |  |
| Qualcomm | LP-SS | 0.1000% | 1ms, 4 repetitions | 5MHz |  | 1000ms | 20MHz |  |  |
| Qualcomm | LP-SS | 0.2000% | 1ms, 4 repetitions | 5MHz |  | 500ms | 20MHz |  |  |
| Qualcomm | LP-SS | 0.5000% | 1ms, 4 repetitions | 5MHz |  | 200ms | 20MHz |  |  |
| Qualcomm | LP-SS | 0.1500% | 1.5ms, 4 repetitions | 5MHz |  | 1000ms | 20MHz |  |  |
| Qualcomm | LP-SS | 0.3000% | 1.5ms, 4 repetitions | 5MHz |  | 500ms | 20MHz |  |  |
| Qualcomm | LP-SS | 0.7500% | 1.5ms, 4 repetitions | 5MHz |  | 200ms | 20MHz |  |  |

      - RRC CONNECTED

The System overhead of LP-WUS for RRC CONNECTED is summarized in Table 8.5.1 - 3

Table 8.5.1 -3 Summary of System overhead for LP-WUS for RRC CONNECTED

| Source | Signal | # of info bits carried by signal | Overhead (%) | T resources per beam | F resources | SCS | traffic types | number of pages UE per second or signal periodicity | number of UE per group | System configuration, e.g., BW and etc. | the target channel | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | LP-WUS | 1 | 0.8600% | 8 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 | 1 | 100MHz, 8beams | unicast PDCCH AL2-4Rx | The coverage of LP-WUS with 8/32/40 symbols is comparable to unicast PDCCH AL2-4Rx coverage. |
| vivo | LP-WUS | 8 | 3.4300% | 32 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 | 1 |  | unicast PDCCH AL2-4Rx |  |
| vivo | LP-WUS | 12 | 4.2900% | 40 symbols | 5.04MHz | 30kHz | XR (16.67ms) | 10 | 1 |  | unicast PDCCH AL2-4Rx |  |
| vivo | LP-WUS | 1 | 0.0700% | 8 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 1 | 100MHz, 8beams | unicast PDCCH AL2-4Rx |  |
| vivo | LP-WUS | 8 | 0.2900% | 32 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 1 |  | unicast PDCCH AL2-4Rx |  |
| vivo | LP-WUS | 12 | 0.3600% | 40 symbols | 5.04MHz | 30kHz | FTP3 (200ms) | 10 | 1 |  | unicast PDCCH AL2-4Rx |  |

Observations

For RRC IDLE/INACTIVE,

- For LP-WUS carrying 1-bit information based on OOK, depending on the time-frequency occupation, traffic inter-arrival time, number of beams, system BW, the system overhead varies,

  - One company reports system overhead is [0.003%~0.055%] for LP-WUS with 8 beams, 100MHz system BW based on varies configurations as described in the Table 8.5.1 - 1

  - Two companies report system overhead is [0.018%~0.45%] for LP-WUS with 8 beams, 20MHz system BW based on varies configurations as described in the Table 8.5.1 - 1

- For LP-WUS carrying multiple bits information based on OOK, depending on the time-frequency occupation, traffic inter-arrival time, number of beams, system BW, the system overhead varies,

  - For LP-WUS carrying 48-bit information,

    - One company reports system overhead is 2.61% (100MHz system BW) and 8.14% ~10.04% (20MHz system BW) for 48-bits LP-WUS with 8 beams;

  - For LP-WUS carrying 7-24 bit information,

  - Three companies report system overhead is [0.0004%~1.4%], for 7~24bits LP-WUS

    - with 1~8 beams, 20MHz system BW;

  - Three companies report system overhead is [0.0100%~1.777%] for 7~24bits LP-WUS

    - with 1~8 beams, 100MHz system BW.

- For LP-WUS carrying 1-bit information based on SSS, one company reports system overhead is 0.01% (20MHz system BW) with 8 beams

- For LP-SS based on OOK with 320ms * X periodicity, 1 slot (30kHz) *Y and 3 - 5MHz * Z time-frequency occupation, with 8 beams*W and 20MHz*T system bandwidth, the system overhead is no more than 0.4% * Y * Z * W / X / T.

- Reference setting for further consideration on LP-SS performance (including sync and/or measurement),

    - Set 1: approximately 5MHz LP-SS BW, 1 slot (30KHz) duration, 320ms periodicity, 8 beams, 20MHz system bandwidth (overhead no more than 0.4%).

    - Set 2: approximately 5MHz LP-SS BW, 1 slot (30KHz) duration, 160ms periodicity, 8 beams, 20MHz system bandwidth (overhead no more than 0.8%).

    - Set 3: approximately 5MHz LP-SS BW, 2 slot (30KHz) duration, 320ms periodicity, 8 beams, 20MHz system bandwidth (overhead no more than 0.8%).

    - Note: the overhead for the above reference sets are derived based on the above formula.

:

For RRC CONNECTED,

- One company reports

For XR (16.67ms, 60FPS) and maximum system capacity,

    - LP-WUS with 1-bit information based on OOK is 0.86%

    - LP-WUS with 8 or 12-bits information based on OOK is 3.43%~4.29%

For FTP3 (200ms mean arrival time) and 10 UEs per cell (RU is 40%),

    - LP-WUS with 1-bit information based on OOK is 0.07%

    - LP-WUS with 8 or 12-bits information based on OOK is 0.29%~0.36%

- Annex B: System Overhead

- B.1 Vivo[R1-xxxxx]

Table 1 Resource overhead of LP-WUS to the overall system resource for RRC idle/inactive mode.

| Source | # of info. bits | Overhead (%) | T-F resources | Number of pages for a UE per cell per second | Number of UE per group | System configuration, e.g., BW and beam number etc. | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | 1 | 0.015% | 1 symbol, 5.04MHz (30kHz SCS) | 10 | 1 | 100MHz, 8beams | For IoT, cases, Assuming 1,000,000 user/km2 connection density [ITU M.2412], with packet arrival every 1 message/2 hours/device [ITU M.2412], assuming 500m ISD with 3 sectors. The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage |
|  | 1 | 0.003% |  | 10 | 4 |  |  |
|  | 8 | 0.06% | 4 symbols, 5.04MHz (30kHz SCS) | 10 | 1 |  |  |
|  | 8 | 0.01% |  | 10 | 4 |  |  |
|  | 8 or 12 | 0.075% | 5 symbols, 5.04MHz (30kHz SCS) | 10 | 1 |  |  |
|  | 8 or 12 | 0.013% |  | 10 | 4 |  |  |
|  | 1 | 0.073% | 1 symbol, 5.04MHz (30kHz SCS) | 10 | 1 | 20MHz, 8beams |  |
|  | 1 | 0.018% |  | 10 | 4 |  |  |
|  | 8 | 0.29% | 4 symbols, 5.04MHz (30kHz SCS) | 10 | 1 |  |  |
|  | 8 | 0.07% |  | 10 | 4 |  |  |
|  | 8 or 12 | 0.363% | 5 symbols, 5.04MHz (30kHz SCS) | 10 | 1 |  |  |
|  | 8 or 12 | 0.088% |  | 10 | 4 |  |  |
|  | 1 | 0.055% | 1 symbol, 5.04MHz (30kHz SCS) | 39 | 1 | 100MHz, 8beams | For eMBB cases, Assuming very high population density, e.g., sports event with 5,000 users/cell population density, and per UE paging arrival rate = 1% when paging i-DRX cycle is Tpagingcycle = 1.28s The coverage of LP-WUS with 1/4/5 symbols is comparable to or better than Msg 3 coverage. |
|  | 1 | 0.015% |  | 39 | 4 |  |  |
|  | 8 | 0.22% | 4 symbols, 5.04MHz (30kHz SCS) | 39 | 1 |  |  |
|  | 8 | 0.06% |  | 39 | 4 |  |  |
|  | 8 or 12 | 0.275% | 5 symbols, 5.04MHz (30kHz SCS) | 39 | 1 |  |  |
|  | 8 or 12 | 0.075% |  | 39 | 4 |  |  |
|  | 1 | 0.280% | 1 symbol, 5.04MHz (30kHz SCS) | 39 | 1 | 20MHz, 8beams |  |
|  | 1 | 0.070% |  | 39 | 4 |  |  |
|  | 8 | 1.12% | 4 symbols, 5.04MHz (30kHz SCS) | 39 | 1 |  |  |
|  | 8 | 0.28% |  | 39 | 4 |  |  |
|  | 8 or 12 | 1.400% | 5 symbols, 5.04MHz (30kHz SCS) | 39 | 1 |  |  |
|  | 8 or 12 | 0.350% |  | 39 | 4 |  |  |

Table 2. Resource overhead of LP-WUS to the overall system resource for RRC connected mode

| Source | # of info. bits | Overhead (%) | T-F resources | Traffic type | Number of UE per cell | Number of UE per group | System configuration, e.g., BW and beam number etc. | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | 1 | 0.86% | 8 symbols, 5.04MHz (30kHz SCS) | XR (16.67ms) | 10 | 1 | 100MHz, 8beams | The coverage of LP-WUS with 8/32/40 symbols is comparable to unicast PDCCH AL2-4Rx coverage. |
|  | 8 | 3.43% | 32 symbols, 5.04MHz (30kHz SCS) |  | 10 | 1 |  |  |
|  | 12 | 4.29% | 40 symbols, 5.04MHz (30kHz SCS) |  | 10 | 1 |  |  |
|  | 1 | 0.07% | 8 symbols, 5.04MHz (30kHz SCS) | FTP3 (mean packet interval= 200ms) | 10 | 1 | 100MHz, 8beams |  |
|  | 8 | 0.29% | 32 symbols, 5.04MHz (30kHz SCS) |  | 10 | 1 |  |  |
|  | 12 | 0.36% | 40 symbols, 5.04MHz (30kHz SCS) |  | 10 | 1 |  |  |

Table 3.  Resource overhead of LP-SS to the system resource for RRC idle/inactive mode

| Source | # of info. bits | Overhead (%) | T-F resources | LP-SS periodicity (ms) | System configuration, e.g., BW and beam number etc. | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Vivo | 1 | 0.063% | 14 symbols, 5.04MHz (30kHz SCS) | 320ms | 100MHz, 8beam | The LP-SS coverage is better than msg.3 coverage; The derivation of LP-SS frequency: 4.32MHz +2RB (30kHz) = 5.04MHz |
|  | 1 | 0.0158% |  | 1280ms | 100MHz, 8beam |  |
|  | 1 | 0.002% |  | 10240ms | 100MHz, 8beam |  |
|  | 1 | 0.315% |  | 320ms | 20MHz, 8beam |  |
|  | 1 | 0.079% |  | 1280ms | 20MHz, 8beam |  |
|  | 1 | 0.01% |  | 10240ms | 20MHz, 8beam |  |

- B.2 ZTE[R1-xxxxxxx]

Table 1. Assumptions for system overhead evaluation

| System bandwidth | 100M,20M |
| --- | --- |
| SCS | 15KHz, 30KHz |
| LP-SS assumption | Periodicity{160ms, 320ms} - Time domain resources:14 symbols - Frequency bandwidth: 5MHz |
| LP-WUS assumption | - Time domain resources:N2 symbols: 7, 14 - Frequency bandwidth 5MHz |
| Assumption for UE specific wake-up | - Number of beams:1,4 for 15KHz and 1,4,8 for 30KHz - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28 |
| Assumption for group specific wake-up | - Number of UEs in a group: 4, 8 |

Table 2. results summary for system overhead

| Source | Signal | Overhead (%) | T-F resources | Inter-arrival time | System configuration, e.g., BW and etc. | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ZTE | LP-SS | 0.125%, 0.625%, | 14 symbols, 5MHz, SCS=15KHz | 160ms | 100M, 20M, 4beam |  |
|  | LP-SS | 0.0625%, 0.3125% | 14 symbols, 5MHz, SCS=15KHz | 320ms | 100M, 20M, 4beam |  |
|  | LP-SS | 0.0625%, 0.3125% | 14 symbols, 5MHz, SCS=30KHz | 160ms | 100M, 20M, 4beam |  |
|  | LP-SS | 0.125%, 0.625% | 14 symbols, 5MHz, SCS=30KHz | 160ms | 100M, 20M, 8beam |  |
|  | LP-SS | 0.03125%, 0.15625% | 14 symbols, 5MHz, SCS=30KHz | 320ms | 100M, 20M, 4beam |  |
|  | LP-SS | 0.0625%, 0.3125% | 14 symbols, 5MHz, SCS=30KHz | 320ms | 100M, 20M, 8beam |  |
|  | LP-WUS | Almost all the cases has <1% system overhead | SCS=15KHz/30kHz Time domain: 7 / 14 symbols Frequency domain: 5M |  | - System bandwidth: 20M, 100M; - Beams:1,4 - Number of UE in a group:1,4,8 - SCS:15KHz,30KHz - OFDM symbols:7,14 - Paging rate:1%,0.1%,0.01%,0.001% - POs number per PF: 1,2,4 - PFs number per cycle: 1,2,4,8,16 | Only one case when 20M system bandwidth, 4beams, no grouping, 15KHz, 14OFDM symbols, paging rate 1%, maximum POs per PF, maximum PFs per cycle are configured, the system overhead may reach 1.6%. |

Table 3. Some example configuration in extreme conditions and system overhead for LP-WUS

| system bandwidth | SCS | grouping | OFDM symbols | beams | UE number | paging rate | PO per PF | PF per cycle | system overhead (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 0 | 1 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 1.6 |
| 20 | 0 | 1 | 7 | 4 | 20.48 | 0.01 | 4 | 16 | 0.8 |
| 20 | 0 | 1 | 14 | 4 | 10.24 | 0.01 | 4 | 8 | 0.8 |
| 20 | 0 | 1 | 14 | 4 | 10.24 | 0.01 | 2 | 16 | 0.8 |
| 20 | 1 | 1 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.8 |
| 20 | 0 | 1 | 7 | 4 | 10.24 | 0.01 | 4 | 8 | 0.4 |
| 20 | 0 | 1 | 7 | 4 | 10.24 | 0.01 | 2 | 16 | 0.4 |
| 20 | 0 | 1 | 14 | 1 | 20.48 | 0.01 | 4 | 16 | 0.4 |
| 20 | 0 | 1 | 14 | 4 | 5.12 | 0.01 | 4 | 4 | 0.4 |
| 20 | 0 | 1 | 14 | 4 | 5.12 | 0.01 | 2 | 8 | 0.4 |
| 20 | 0 | 1 | 14 | 4 | 5.12 | 0.01 | 1 | 16 | 0.4 |
| 20 | 0 | 4 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.4 |
| 20 | 1 | 1 | 7 | 4 | 20.48 | 0.01 | 4 | 16 | 0.4 |
| 20 | 1 | 1 | 14 | 4 | 10.24 | 0.01 | 4 | 8 | 0.4 |
| 20 | 1 | 1 | 14 | 4 | 10.24 | 0.01 | 2 | 16 | 0.4 |
| 100 | 0 | 1 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.32 |
| 20 | 0 | 1 | 7 | 1 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 20 | 0 | 1 | 7 | 4 | 5.12 | 0.01 | 4 | 4 | 0.2 |
| 20 | 0 | 1 | 7 | 4 | 5.12 | 0.01 | 2 | 8 | 0.2 |
| 20 | 0 | 1 | 7 | 4 | 5.12 | 0.01 | 1 | 16 | 0.2 |
| 20 | 0 | 1 | 14 | 1 | 10.24 | 0.01 | 4 | 8 | 0.2 |
| 20 | 0 | 1 | 14 | 1 | 10.24 | 0.01 | 2 | 16 | 0.2 |
| 20 | 0 | 1 | 14 | 4 | 2.56 | 0.01 | 4 | 2 | 0.2 |
| 20 | 0 | 1 | 14 | 4 | 2.56 | 0.01 | 2 | 4 | 0.2 |
| 20 | 0 | 1 | 14 | 4 | 2.56 | 0.01 | 1 | 8 | 0.2 |
| 20 | 0 | 4 | 7 | 4 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 20 | 0 | 4 | 14 | 4 | 10.24 | 0.01 | 4 | 8 | 0.2 |
| 20 | 0 | 4 | 14 | 4 | 10.24 | 0.01 | 2 | 16 | 0.2 |
| 20 | 0 | 8 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 20 | 1 | 1 | 7 | 4 | 10.24 | 0.01 | 4 | 8 | 0.2 |
| 20 | 1 | 1 | 7 | 4 | 10.24 | 0.01 | 2 | 16 | 0.2 |
| 20 | 1 | 1 | 14 | 1 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 20 | 1 | 1 | 14 | 4 | 5.12 | 0.01 | 4 | 4 | 0.2 |
| 20 | 1 | 1 | 14 | 4 | 5.12 | 0.01 | 2 | 8 | 0.2 |
| 20 | 1 | 1 | 14 | 4 | 5.12 | 0.01 | 1 | 16 | 0.2 |
| 20 | 1 | 4 | 14 | 4 | 20.48 | 0.01 | 4 | 16 | 0.2 |
| 100 | 0 | 1 | 7 | 4 | 20.48 | 0.01 | 4 | 16 | 0.16 |
| 100 | 0 | 1 | 14 | 4 | 10.24 | 0.01 | 4 | 8 | 0.16 |
| 100 | 0 | 1 | 14 | 4 | 10.24 | 0.01 | 2 | 16 | 0.16 |

- B.3 Huawei/HiSi [xxxx]

Table 1. Assumptions for system overhead evaluation

| System bandwidth | 100M |
| --- | --- |
| SCS | 30KHz |
| LP-SS assumption | Periodicity: 320ms - Time domain resources: 1ms (28 OFDM symbols) - Frequency bandwidth: 5MHz |
| LP-WUS assumption | - Bit length: 24 - Time domain resources: {12, 24, 48} OFDM symbols, for {1, 2, 4} repetitions - Frequency bandwidth 5MHz |
| Beam sweeping assumption | - {1, 4} beams |
| Paging number per second | - 0 for LP-SS only (i.e. empty load) - 50 for LP-SS and LP-WUS |

- Table 2. results summary for

system overhead

| Number of paging per second | Repetition number of LP-WUS | Beam number | Resource overhead |
| --- | --- | --- | --- |
| 0 (LP-SS only) | - | 1 | 0.016% |
| 0 (LP-SS only) | - | 4 | 0.063% |
| 50 (LP-SS+LP-WUS) | 1 | 1 | 0.123% |
| 50 (LP-SS+LP-WUS) | 2 | 1 | 0.230% |
| 50 (LP-SS+LP-WUS) | 4 | 1 | 0.444% |
| 50 (LP-SS+LP-WUS) | 1 | 4 | 0.491% |
| 50 (LP-SS+LP-WUS) | 2 | 4 | 0.920% |
| 50 (LP-SS+LP-WUS) | 4 | 4 | 1.777% |

- B.4 Qualcomm [xxxx]

| LP-WUS overhead |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note | IDRX, 20UEs, 10kbps | IDRX, 20UEs, 5kbps | IDRX, 100UEs, 10kbps | IDRX, 100UEs, 5kbps | eDRX, 20UEs, 10kbps | eDRX, 20UEs, 5kbps | eDRX, 100UEs, 10kbps | eDRX, 100UEs, 5kbps |
| System config | system BW (MHz) | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
|  | total number of Idle UEs in the cell | 20 | 20 | 100 | 100 | 20 | 20 | 100 | 100 |
| LP-WUS config | # of info bits | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
|  | data rate (kbps) | 10 | 5 | 10 | 5 | 10 | 5 | 10 | 5 |
|  | LP-WUS time duration (ms) | 0.6 | 1.2 | 0.6 | 1.2 | 0.6 | 1.2 | 0.6 | 1.2 |
|  | bandwidth (MHz) | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
|  | Monitoring occasion periodicity (ms) | 1280 | 1280 | 1280 | 1280 | 82900 | 82900 | 82900 | 82900 |
|  | LP-WUS resource per group (ms*MHz) | 3 | 6 | 3 | 6 | 3 | 6 | 3 | 6 |
|  | WUS group size | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
|  | number of LP-WUS groups | 4 | 4 | 20 | 20 | 4 | 4 | 20 | 20 |
| Page Traffic | per UE page inter arrival time (sec) | 500 | 500 | 500 | 500 | 500 | 500 | 500 | 500 |
|  | per group page inter arrival time (sec) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
|  | R_G per group per periodicity | 0.0127 | 0.0127 | 0.0127 | 0.0127 | 0.5635 | 0.5635 | 0.5635 | 0.5635 |
| System Resources | total system resources per LP-WUS periodicity (ms*MHz) | 25600 | 25600 | 25600 | 25600 | 1658000 | 1658000 | 1658000 | 1658000 |
| LP-WUS resources | actually used monitoring occasion resources used for LP-WUS tx per group per periodicity (ms*MHz) | 0.04 | 0.08 | 0.04 | 0.08 | 1.69 | 3.38 | 1.69 | 3.38 |
|  | total LP-WUS resources per LP-WUS periodicity | 0.15 | 0.31 | 0.76 | 1.53 | 6.76 | 13.52 | 33.81 | 67.62 |
|  | LP-WUS overhead (%) | 0.0006% | 0.0012% | 0.0030% | 0.0060% | 0.0004% | 0.0008% | 0.0020% | 0.0041% |

| LP-SS overhead |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note | 10bits, periodicity = 1000ms | 10bits, periodicity = 500ms | 10bits, periodicity = 200ms | 15bits, periodicity = 1000ms | 15bits, periodicity = 500ms | 15bits, periodicity = 200ms |
| System config | system BW (MHz) | 20 | 20 | 20 | 20 | 20 | 20 |
| LP-SS config | # of info bits | 10 | 10 | 10 | 15 | 15 | 15 |
|  | data rate (kbps) | 10 | 10 | 10 | 10 | 10 | 10 |
|  | LP-WUS time duration (ms) | 1 | 1 | 1 | 1.5 | 1.5 | 1.5 |
|  | bandwidth (MHz) | 5 | 5 | 5 | 5 | 5 | 5 |
|  | Monitoring occasion periodicity (ms) | 1000 | 500 | 200 | 1000 | 500 | 200 |
|  | LP-SS resource per periodicity (ms*MHz) | 5 | 5 | 5 | 7.5 | 7.5 | 7.5 |
| Resources | total system resources per LP-WUS periodicity (ms*MHz) | 20000 | 10000 | 4000 | 20000 | 10000 | 4000 |
| LP-SS resources | # of LP-SS repetition per LP-SS burst | 4 | 4 | 4 | 4 | 4 | 4 |
|  | total LP-SS resoruces per periodicity | 20 | 20 | 20 | 30 | 30 | 30 |
|  | LP-SS overhead (%) | 0.10% | 0.20% | 0.50% | 0.15% | 0.30% | 0.75% |