3GPP TSG RAN WG1 #114		                                   					       												                           R1-2307877

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.16.1

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR MIMO evolution

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [114-R18-UE_features-02] during RAN1 #114. According to the Chairman’s Notes:

[114-R18-UE_features-02] Email discussion on UE features for MIMO, positioning, NCR, NR-NTN, IoT-NTN, BWP without restriction, NW energy saving, mobility enhancement – Ralf (AT&T) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed and/or agreed during RAN1 #114 within the scope of [114-R18-UE_features-02]. All proposals are based on the latest RAN1 UE features list for Rel-18 in [1].

# Summary of Contributions Submitted to RAN1 #114

The following is the moderator’s summary of contributions submitted to RAN1 #114 in this agenda item.

| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: separate row for TRP-specific BFR with unified TCI framework FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS that whether separate rows/FGs are needed for intra-cell mTRP and inter-cell mTRP case. To our understanding, new rows are not needed. These FGs are just used to report that UE support unified TCI for mTRP operation. As for whether the unified TCI support is for intra-cell mTRP and inter-cell mTRP, it is subject to whether UE support intra-cell mTRP and inter-cell mTRP. If a UE support both unified TCI for mTRP and intra-cell mTRP operation, it implies that the UE support unified TCI for intra-cell mTRP. Similarly, if a UE support both unified TCI for mTRP and inter-cell mTRP operation, it implies that the UE support unified TCI for inter-cell mTRP. Proposal 2-1: For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is no need to introduce separate rows for intra-cell mTRP and inter-cell mTRP case. For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS on whether a separate row is needed for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. We think that a UE that support unified TCI for single-DCI based mTRP should support the [TCI selection field] in the DCI by default and a separate row is not needed. Proposal 2-2: For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is no need to introduce a row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS on what to do/signal about two default beams. In our view, a new FG should be introduced for supporting two default beams under unified TCI framework since a UE supporting FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a does not necessarily support two default beams. We have added this as FG 40-1-14 in the updated table at the end of this section. Proposal 2-3: Introduce a new FG for the support of two default beams for a UE that supports FG 40-1-1, FG 40-1-1a, FG 40-1-2 or FG 40-1-2a. Proposal 2-4: Support the following components for the maximum number of configured/activated TCI states: - For FG 40-1-1, support following components: - The maximum number of configured joint TCI states per BWP per CC - The maximum number of activated joint TCI states across all CC For FG 40-1-1 and FG 40-1-2, there is an FFS on whether to have a separate row/FG for the supporting of TRP-specific BFR under unified TCI framework. In our view, a UE support unified TCI framework and TRP-specific BFR should naturally support TRP-specific BFR under unified TCI framework. Proposal 2-6: For FG 40-1-1 and FG 40-1-2, there is no need to introduce a separate row for supporting of TRP-specific BFR under unified TCI framework. For FG 40-1-1, there is an FFS on “additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH”. We don’t see the necessity of such signalling. Proposal 2-7: For FG 40-1-1, there is no need to have a UE feature/component on the support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: 1. maximum number of configured joint TCI states per CC per BWP FFS: 2. maximum number of activated joint TCI states across all CCs FFS: separate row for TRP-specific BFR with unified TCI framework FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; 23-1-1; Per band; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {1, 2, 4, 8, 16}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-1-1 - FFS1: We recommend having separate rows for inter-cell and intra-cell m-TRPs - FFS2: We recommend having separate rows for additional support of DCI format 1_1 and 1_2 configured with TCI selection field - FFS3: What to do/signal about two default beams – decisions pending from WI, to be addressed later - FFS4: Candidate values for the maximum number of configured joint TCI states per CC per BWP {8, 12, 16, 24, 32, 48, 64, 128} - FFS5: Candidate values for the maximum number of activated joint TCI states across all CCs {2, 4, 8, 16, 32} - FFS6: We recommend having separate rows for TRP-specific BFR with the unified TCI framework - FFS7: What to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH - decisions pending from WI, to be addressed later 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of configured joint TCI states per CC per BWP – candidate values {8, 12, 16, 24, 32, 48, 64, 128} FFS: maximum number of activated joint TCI states across all CCs – candidate values {2, 4, 8, 16, 32} FFS: separate row for TRP-specific BFR with unified TCI framework FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; 23-1-1. 40. NR_MIMO_evo_DL_UL; 40-1-1b; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC in intra-cell scenario. 40. NR_MIMO_evo_DL_UL; 40-1-1c; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC in inter-cell scenario. 40. NR_MIMO_evo_DL_UL; 40-1-1d; Additional support for DCI format 1_1 and 1_2 with Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC. 40. NR_MIMO_evo_DL_UL; 40-1-1e; Support for TRP-specific BFR with unified TCI framework with Unified TCI with joint DL/UL TCI update. |
| ZTE [4] | In the main feature, FG 40-1-1 should be assumed as a basic feature for the Rel-18 unified TCI extension for S-DCI based multi-TRP operation. Then, regarding the FFS parts, we have the following analysis: - Firstly, we do not have any agreement of enabling inter-cell multi-TRP operation for single-DCI operation, and then the corresponding inter-cell case should be removed. - Then, once having single-DCI based multi-TRP operation, we should mandatory support the [TCI selection field] as in the basic feature. Otherwise, the single-TCI based PDSCH transmission can NOT be scheduled dynamically. - Regarding two default beams, although we can reuse the following UE feature FG 16-2b-0, we prefer to have an independent FG considering that there may be too complicate prerequisite relationship. 16-2b-0; Two default beams for single-DCI based multi-TRP; Support of default QCL assumption with two TCI states; 16-2c. - Meanwhile, we do NOT identify the necessity of a separate FG for ‘additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH’. 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional 1. Support of DCI format 1_1 and 1_2 configured with [TCI selection field] 2. Support of a single default beam for PDSCH with scheduling offset less than a threshold FFS: what to do/signal about two default beams FFS: 3. Maximum number of configured joint TCI states per CC per BWP FFS: 4. Maximum number of activated joint TCI states across all CCs FFS: separate row for TRP-specific BFR with unified TCI framework FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. 40-1-2b; Two default beams for PDSCH with scheduling offset less than a threshold; Support of two default beams for PDSCH with scheduling offset less than a threshold; 40-1-1. 40-1-2c; TRP-specific BFR with unified TCI framework; Support of TRP-specific BFR with unified TCI framework under S-DCI based MTRP operation; 40-1-1. |
| Spreadtrum Communications [5] | For FG 40-1-1 and FG 1-2, a common FFS is whether to separate the intra-cell and inter-cell MTRP into two rows. We think the UE features in Rel-17 [2] can be referred. The same structure could be used, i.e., merge the intra-cell and inter-cell MTRP into a same row. Proposal 1: Do not separate the intra-cell and inter-cell MTRP into two rows in FG 40-1-1 and FG 40-1-2. As for the issues on the maximum number of the TCI states, since there is no consensus on change the maximum number of joint/separate TCI states configured in each CC/BWP or across all CCs, the candidate values of which should be keep the same as those in Rel-17. Proposal 2: The candidate values of the maximum number of joint/separate TCI states configured in each CC/BWP or across all CCs should be keep the same as those in Rel-17. For s-DCI based PDSCH MTRP, two default beams were agreed as a UE capability in Rel-18 and the information should be reported for PDSCH reception. Refer to the UE features in Rel-16, “FG 16-2b-0 Two default beams for single-DCI based multi-TRP” can be reused. Proposal 3: FG 10-2b-0: two default beams for single-DCI based multi-TRP in Rel-16 can be referred. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; Support of DCI format 1_1 and 1_2 configured with [TCI selection field] Prerequisite 23-1-1 FFS: separate rows intra-cell and inter-cell multi-TRP Comment: inter-cell single-DCI based multi-TRP is not supported in Rel-18. FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] Comment: no need. Support of configuration with [TCI selection field] should be mandatory. FFS: what to do/signal about two default beams Comment: can reuse the 16-2c and 16-2b-0. FFS: maximum number of configured joint TCI states per CC per BWP Comment: reuse the values in Rel-17 unified TCI framework 23-1-1. FFS: maximum number of activated joint TCI states across all CCs Comment: reuse the values in Rel-17 unified TCI framework 23-1-1. FFS: separate row for TRP-specific BFR with unified TCI framework Comment: yes. FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra and inter-cell multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: separate row for TRP-specific BFR with unified TCI framework FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; 1. Maximum number of configured joint TCI states per CC per BWP. |
| CATT [9] | 40-1-1: - S-DCI based inter-cell mTRP is supported in Rel-18 unified TCI framework extension, which should not be included in the FG. - Whether the 2-bit [TCI selection field] is present or not in DCI format 1_1 and 1_2 is configured by RRC. The presence of [TCI selection field] in DCI format 1_1 and 1_2 should be supported by all the Rel-18 UEs. Otherwise, without this UE feature, dynamic switching between sTRP and mTRP using DCI for PDSCH could not be achieved, since UE could not discriminate sTRP or mTRP operation based on the number of TCI states corresponding to one codepoint. Therefore, it is not necessary to use separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. - The UE capability of supporting two default beams for S-DCI based MTRP has already been introduced in Rel-16. With the same functionality, it could be reused for Rel-18 unified TCI framework. - The maximum number of configured joint TCI states per CC per BWP has been specified in Rel-17 with candidate value {8, 12, 16, 24, 32, 48, 64, 128}. These values could be reused for Rel-18. - The maximum number of activated joint TCI states across all CCs has been specified in Rel-17 with candidate value {1, 2, 4, 8, 16}. These candidate values are specified for sTRP. Considering Rel-18 mTRP, more TCI states are needed. The candidate values could be {2, 4, 8, 16, 32}. - In the last meeting, TRP-specific BFR with unified TCI framework has been supported for S-DCI based mTRP. According to Rel-17 specification, TRP-specific BFR is an optional UE feature, which should use a separate row from FG 40-1-1. - In MIMO session, it has been discussed that whether specification should restrict that two indicated joint/DL TCI states must be associated with different TRPs. In our opinion, gNB is responsible to indicate appropriate beam(s) for PDCCH and PDSCH/PUCCH/PUSCH. It is not necessary to introduce such restriction. Therefore, we don’t see the need of signaling about additional supportive of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH. The support of different TCI state for different channels should be a basic feature of Rel-18 UE. Proposal 1: S-DCI based inter-cell mTRP should not be included in the FG. Proposal 2: It is not necessary to use separate rows for the Ssupportive of DCI format 1_1 and 1_2 configured with [TCI selection field]. Proposal 3: UE capability of supporting two default beams for S-DCI based MTRP introduced in Rel-16 is reused for Rel-18 unified TCI framework. Proposal 4: One additional UE features should be added for S-DCI based mTRP: 40. NR_MIMO_evo_DL_UL; 40-1-X; TRP-specific BFR with unified TCI framework. |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. This contains several FFSs. We will address them one by one: “FFS: separate rows intra-cell and inter-cell multi-TRP”: Although single-DCI inter-cell multi-TRP is not agreed, inter-cell beam management is supported. However, inter-cell beam management requires the support of more than one activated TCI state. Since only one activated TCI state is supported in FG 40-1-1, inter-cell beam management is not possible. In our understanding, inter-cell support is included in FG 40-1-1a. “FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]”. A separate row is needed. This is added as FG 40-1-1d. “FFS: what to do/signal about two default beams”. This would be a component in FG 40-1-1. This FFS can be removed. “FFS: maximum number of configured joint TCI states per CC per BWP”. Following the Rel-17 solution, this would be a component in FG 40-1-1. “FFS: maximum number of activated joint TCI states across all CCs”. Following the Rel-17 solution, this would be a component in FG 40-1-1. “FFS: separate row for TRP-specific BFR with unified TCI framework”. This is captured in FG 40-1-5. The FFS can be removed. “FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH.” It is unclear what this means. The FFS can be removed. 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; - maximum number of configured joint TCI states per CC per BWP - maximum number of activated joint TCI states across all CCs. Support two default beams. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; - The maximum number of configured joint TCI state per BWP per CC in a band. - Two MAC-CE activated joint TCI-states per CC in a band. - MAC-CE based TCI-state indication for two active TCI-states. - The maximum number of MAC-CE activated joint TCI-states across all CC(s) in a band.; Component 1 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate value {2, 4, 8, 16, 32}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: separate row for TRP-specific BFR with unified TCI framework FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH; 23-1-1, 16-2b-1 or 16-2b-2 or 16-2b-3 or 16-2b-4 or 16-2b-5; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-1-1 - Components: - We think support of the following in the FFS can be separate FGs. We added our reason in the sub-bullet. - Intra-cell and inter-cell multi-TRP - Reason: In Rel.17, there are separate UE capability between intra-cell and inter-cell. We don’t think inter-cell should be a basic UE capability in Rel.18. - Additional support of DCI format 1_1 and 1_2 configured with TCI state selection field - Reason: In Rel.15-17, DCI-level dynamic TCI indication is separate UE capability. Although our preference is to make TCI state selection field as a basic FG, we can accept to have a separate UE capability. - two default beams - Reason: In Rel.16-17, two default beam is separate UE capability. We don’t think there is strong need to make the two default beam as a basic UE capability in Rel.18. - TRP-specific BFR with unified TCI framework - Reason: In Rel.17, TRP-specific BFR is optional UE capability. We don’t think there is strong need to make it as a basic UE capability in Rel.18. - Regarding to the last FFS, we think the motivation is to ensure that one UE can assume that TCI state of the scheduling DCI can be the same as one of the TCI states of the scheduled PDSCH. We suggest updating the FFS to have the separate two FGs. - FG1 (optional): Support TCI state selection field. UE assumes TCI state of the scheduling PDCCH is the same as one of the TCI states of the scheduled PDSCH. - FG2 (optional): Support TCI state selection field. UE assumes TCI state of the scheduling PDCCH can be the same or different as one of the TCI states of the scheduled PDSCH. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; 1. Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured joint TCI states per BWP per CC in a band 3. Maximum of two MAC-CE activated joint TCI states per CC in a band 4. MAC CE based TCI state indication for a single activated TCI codepoint 5. The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band; At least one of 16-2b-1, 16-2b-2, 16-2b-3, 16-2b-4, 16-2b-5, 23-2-1, 23-3-1, 23-3-2, 23-3-3, 23-6-1, 23-6-2, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4; Yes; Per band; N/A; N/A; N/A; Component 2 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 5 candidate value {2, 4, 6, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| MediaTek Inc. [17] | What to do/signal about two default beams In Rel-16, there is UE capability (FG 16-2b-0) for “Two default beams for single-DCI based multi-TRP”. In our view, the same UE capability can be reused for Rel-18 unified TCI for single-DCI based multi-TRP. Proposal 2: Reuse the legacy UE capability (FG 16-2b-0) for “Two default beams for single-DCI based multi-TRP” Separate row for TRP-specific BFR with unified TCI framework In Rel-17 unified TCI framework, a separate UE capability is introduced for “SCell BFR with unified TCI framework” (FG 23-1-1c). Similarly, a separate row can be introduced for “TRP-specific BFR with unified TCI framework”. Proposal 3: Introduce a separate row for “TRP-specific BFR with unified TCI framework” 40. NR_MIMO_evo_DL_UL; 40-1-1; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; - Support joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC - Support joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC - Per aperiodic CSI-RS resource/resource set configuration for TCI selection; Optional with UE capability Component-3 candidate values: {resource, resource set, both}. 40. NR_MIMO_evo_DL_UL; 40-1-5; TRP-specific BFR with unified TCI framework. |

| 40. NR_MIMO_evo_DL_UL | 40-1-1a | Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated joint TCI states per CC |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS that whether separate rows/FGs are needed for intra-cell mTRP and inter-cell mTRP case. To our understanding, new rows are not needed. These FGs are just used to report that UE support unified TCI for mTRP operation. As for whether the unified TCI support is for intra-cell mTRP and inter-cell mTRP, it is subject to whether UE support intra-cell mTRP and inter-cell mTRP. If a UE support both unified TCI for mTRP and intra-cell mTRP operation, it implies that the UE support unified TCI for intra-cell mTRP. Similarly, if a UE support both unified TCI for mTRP and inter-cell mTRP operation, it implies that the UE support unified TCI for inter-cell mTRP. Proposal 2-1: For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is no need to introduce separate rows for intra-cell mTRP and inter-cell mTRP case. For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS on whether a separate row is needed for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. We think that a UE that support unified TCI for single-DCI based mTRP should support the [TCI selection field] in the DCI by default and a separate row is not needed. Proposal 2-2: For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is no need to introduce a row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS on what to do/signal about two default beams. In our view, a new FG should be introduced for supporting two default beams under unified TCI framework since a UE supporting FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a does not necessarily support two default beams. We have added this as FG 40-1-14 in the updated table at the end of this section. Proposal 2-3: Introduce a new FG for the support of two default beams for a UE that supports FG 40-1-1, FG 40-1-1a, FG 40-1-2 or FG 40-1-2a. Proposal 2-4: Support the following components for the maximum number of configured/activated TCI states: - For FG 40-1-1a, support following components: - The maximum number of activated TCI state per BWP per CC - Note: The maximum number of configured joint TCI states per BWP per CC is not needed for this FG, as its prerequisite FG is FG 40-1-1. 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated joint TCI states per CC 1. maximum number of activated TCI state per BWP per CC; 23-1-1b, 40-1-1; Per band; n/a; n/a; n/a; Component 1 candidate values: {2, 3, 4, 5, 6, 7, 8, 16}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-1-1a - FFS1: We recommend having a single row for intra-cell and inter-cell multi-TRP - FFS2: We recommend having a single row for additional support of DCI format 1_1 and 1_2 configured with TCI selection field - FFS3: What to do/signal about two default beams - decisions pending from WI, to be addressed later - FFS4: Candidate values for the maximum number of activated joint TCI states per CC {1, 2, 4, 8, 16} 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field FFS: what to do/signal about two default beams FFS: maximum number of activated joint TCI states per CC – candidate values {1, 2, 4, 8, 16}. |
| ZTE [4] | 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: Maximum number of activated joint TCI states per CC; 40-1-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; Support of DCI format 1_1 and 1_2 configured with [TCI selection field] Prerequisite 23-1-1b FFS: separate rows intra-cell and inter-cell multi-TRP Comment: inter-cell single-DCI based multi-TRP is not supported in Rel-18. FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] Comment: no need. Support of configuration with [TCI selection field] should be mandatory. FFS: what to do/signal about two default beams Comment: can reuse the 16-2c and 16-2b-0. FFS: maximum number of activated joint TCI states per CC Comment: reuse the values in Rel-17 unified TCI framework 23-1-1.. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra and inter-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated joint TCI states per CC. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; 1. Maximum number of activated joint TCI states per CC. |
| CATT [9] | 40-1-1a: - The maximum number of activated joint TCI states per CC has been specified in Rel-17 with candidate value {2, 3, 4, 5, 6, 7, 8}. Considering Rel-18 mTRP, the candidate values could be {4, 6, 8, 10, 12, 14, 16}. |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. Assuming that we follow the structure from Rel-17, this FG does not indicate support of DCI-based TCI state indication. DCI-based joint TCI state indication is a separate FG, added as FG 40-1-1b. “FFS: separate rows intra-cell and inter-cell multi-TRP”. In Rel-17, the FG that describes support of multiple activated TCI states also include inter-cell operation. Following this design, there is no need for a separate row: support for inter-cell operation is inherent. “FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]”. The separate FG proposed under FG 40-1-1 can be reused. “FFS: what to do/signal about two default beams.” This is captured in FG 40-1-6. This FFS can be removed. “FFS: maximum number of activated joint TCI states per CC.” Following the Rel-17 solution, this would be a component in FG 40-1-1a. The formulation from Rel-17 is “Support K additional MAC-CE activated joint TCI states per CC in a band”. - Following the Rel-17 solution, one additional component is needed in FG 40-1-1a: Support K additional activated joint TCI states across all CCs. 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; - Support K additional MAC-CE activated joint TCI states per CC - Support K additional activated joint TCI states across all CCs.. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated joint TCI states per CC in a band. - The minimum beam application time in Y symbols per SCS; Component 2 candidate value {2, 4, 8, 10, 12, 14, 16} Component 3 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated joint TCI states per CC; 23-1-1, 16-2b-1 or 16-2b-2 or 16-2b-3 or 16-2b-4 or 16-2b-5; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-1-1a/40-1-2a - Components: - We think support of the following in the FFS can be separate FGs, because of the same reason as FG40-1-1. - Intra-cell and inter-cell multi-TRP. - Additional support of DCI format 1_1 and 1_2 configured with TCI state selection field. - two default beams. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated joint TCI states per CC in a band; 40-1-1; Yes; Per band; N/A; N/A; N/A; Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be indicated in FR2 Component 3 candidate values: {3, 4, 5, 6, 7, 8} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH; Optional with capability signaling. |
| MediaTek Inc. [17] | What to do/signal about two default beams In Rel-16, there is UE capability (FG 16-2b-0) for “Two default beams for single-DCI based multi-TRP”. In our view, the same UE capability can be reused for Rel-18 unified TCI for single-DCI based multi-TRP. Proposal 2: Reuse the legacy UE capability (FG 16-2b-0) for “Two default beams for single-DCI based multi-TRP” 40. NR_MIMO_evo_DL_UL; 40-1-1a; Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; - Support joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC - Support joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC - Per aperiodic CSI-RS resource/resource set configuration for TCI selection; Optional with UE capability Component-3 candidate values: {resource, resource set, both}. |

| 40. NR_MIMO_evo_DL_UL | 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated DL TCI states per CC FFS: maximum number of activated UL TCI states per CC FFS: separate row for TRP-specific BFR with unified TCI framework |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS that whether separate rows/FGs are needed for intra-cell mTRP and inter-cell mTRP case. To our understanding, new rows are not needed. These FGs are just used to report that UE support unified TCI for mTRP operation. As for whether the unified TCI support is for intra-cell mTRP and inter-cell mTRP, it is subject to whether UE support intra-cell mTRP and inter-cell mTRP. If a UE support both unified TCI for mTRP and intra-cell mTRP operation, it implies that the UE support unified TCI for intra-cell mTRP. Similarly, if a UE support both unified TCI for mTRP and inter-cell mTRP operation, it implies that the UE support unified TCI for inter-cell mTRP. Proposal 2-1: For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is no need to introduce separate rows for intra-cell mTRP and inter-cell mTRP case. For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS on whether a separate row is needed for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. We think that a UE that support unified TCI for single-DCI based mTRP should support the [TCI selection field] in the DCI by default and a separate row is not needed. Proposal 2-2: For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is no need to introduce a row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS on what to do/signal about two default beams. In our view, a new FG should be introduced for supporting two default beams under unified TCI framework since a UE supporting FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a does not necessarily support two default beams. We have added this as FG 40-1-14 in the updated table at the end of this section. Proposal 2-3: Introduce a new FG for the support of two default beams for a UE that supports FG 40-1-1, FG 40-1-1a, FG 40-1-2 or FG 40-1-2a. Proposal 2-4: Support the following components for the maximum number of configured/activated TCI states: - For FG 40-1-2, support following components: - The maximum number of configured DL TCI states per BWP per CC - The maximum number of configured UL TCI states per BWP per CC - The maximum number of activated DL TCI states cross all CC - The maximum number of activated UL TCI states cross all CC For FG 40-1-2, there are two FFSs on “maximum number of activated DL TCI states per CC” and “maximum number of activated UL TCI states per CC”. However, this FG corresponds to the UE with a single activated TCI codepoint per CC and, therefore, these two FFSs should be removed. Proposal 2-5: For FG 40-1-2, remove “FFS: maximum number of activated DL TCI states per CC” and “FFS: maximum number of activated UL TCI states per CC” form the components column. For FG 40-1-1 and FG 40-1-2, there is an FFS on whether to have a separate row/FG for the supporting of TRP-specific BFR under unified TCI framework. In our view, a UE support unified TCI framework and TRP-specific BFR should naturally support TRP-specific BFR under unified TCI framework. Proposal 2-6: For FG 40-1-1 and FG 40-1-2, there is no need to introduce a separate row for supporting of TRP-specific BFR under unified TCI framework. 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated DL TCI states per CC FFS: maximum number of activated UL TCI states per CC FFS: separate row for TRP-specific BFR with unified TCI framework 1. maximum number of configured DL TCI states per BWP per CC 2. maximum number of configured UL TCI states per BWP per CC 3. maximum number of activated DL TCI states cross all CC 4. maximum number of activated UL TCI states cross all CC; 23-10-1; Per band; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-1-2 - FFS1: We recommend having separate rows for inter-cell and intra-cell m-TRPs - FFS2: We recommend having separate row for additional support of DCI format 1_1 and 1_2 configured with TCI selection field - FFS3: What to do/signal about two default beams - decisions pending from WI, to be addressed later - FFS4: Candidate values for the maximum number of activated DL TCI states per CC {2, 4, 8, 16, 32} - FFS5: Candidate values for the maximum number of activated DL TCI states per CC {2, 4, 8, 16, 32} - FFS6: We recommend having separate rows for TRP-specific BFR with the unified TCI framework 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated DL TCI states per CC – candidate values {2, 4, 8, 16, 32} FFS: maximum number of activated UL TCI states per CC– candidate values {2, 4, 8, 16, 32} FFS: separate row for TRP-specific BFR with unified TCI framework. 40. NR_MIMO_evo_DL_UL; 40-1-2b; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC in intra-cell scenario. 40. NR_MIMO_evo_DL_UL; 40-1-2c; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC in inter-cell scenario. 40. NR_MIMO_evo_DL_UL; 40-1-2d; Additional support for DCI format 1_1 and 1_2 with unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC. 40. NR_MIMO_evo_DL_UL; 40-1-2e; Support for TRP-specific BFR with unified TCI with separate DL/UL TCI. |
| ZTE [4] | - We are fine to have the following separate rows for the following. But it should be noticed, for 40-1-2, we should have ‘maximum number of activated joint TCI states per CC’, i.e., for multi-activated-TCI-state case. - Maximum number of configured joint TCI states per CC per BWP - Maximum number of activated joint TCI states across all CCs - TRP-specific BFR with unified TCI framework 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams 1. Support of unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC. FFS: 2. Maximum number of configured DL TCI states per CC per BWP FFS: 3. Maximum number of configured UL TCI states per CC per BWP 4. Maximum number of activated DL TCI states across all CCs 5. Maximum number of activated UL TCI states across all CCs FFS: separate row for TRP-specific BFR with unified TCI framework; 40-1-1. |
| Spreadtrum Communications [5] | For FG 40-1-1 and FG 1-2, a common FFS is whether to separate the intra-cell and inter-cell MTRP into two rows. We think the UE features in Rel-17 [2] can be referred. The same structure could be used, i.e., merge the intra-cell and inter-cell MTRP into a same row. Proposal 1: Do not separate the intra-cell and inter-cell MTRP into two rows in FG 40-1-1 and FG 40-1-2. As for the issues on the maximum number of the TCI states, since there is no consensus on change the maximum number of joint/separate TCI states configured in each CC/BWP or across all CCs, the candidate values of which should be keep the same as those in Rel-17. Proposal 2: The candidate values of the maximum number of joint/separate TCI states configured in each CC/BWP or across all CCs should be keep the same as those in Rel-17. For s-DCI based PDSCH MTRP, two default beams were agreed as a UE capability in Rel-18 and the information should be reported for PDSCH reception. Refer to the UE features in Rel-16, “FG 16-2b-0 Two default beams for single-DCI based multi-TRP” can be reused. Proposal 3: FG 10-2b-0: two default beams for single-DCI based multi-TRP in Rel-16 can be referred. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; Support of DCI format 1_1 and 1_2 configured with [TCI selection field] Prerequisite 23-10-1 FFS: separate rows intra-cell and inter-cell multi-TRP Comment: inter-cell single-DCI based multi-TRP is not supported in Rel-18. FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] Comment: no need. Support of configuration with [TCI selection field] should be mandatory. FFS: what to do/signal about two default beams Comment: can reuse the 16-2c and 16-2b-0. FFS: maximum number of activated DL TCI states per CC Comment: reuse the values in Rel-17 unified TCI framework 23-10-1. FFS: maximum number of activated UL TCI states per CC Comment: reuse the values in Rel-17 unified TCI framework 23-10-1. FFS: separate row for TRP-specific BFR with unified TCI framework Comment: yes.. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra and inter-cell multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated DL TCI states per CC FFS: maximum number of activated UL TCI states per CC FFS: separate row for TRP-specific BFR with unified TCI framework. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; 1. Maximum number of configured DL TCI states per CC per BWP 2. Maximum number of configured UL TCI states per CC per BWP 3. Maximum number of total configured DL and UL TCI states per CC per BWP. |
| CATT [9] | 40-1-2: - With single activated TCI codepoint, for Rel-18 mTRP, the candidate values for the maximum number of activated DL or UL TCI states per CC could be {1, 2}. |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. The input on FG 40-1-2 essentially mirrors the input on FG 40-1-1. Note that FG 40-1-1 will be a pre-requisite for FG 40-1-2. “FFS: separate rows intra-cell and inter-cell multi-TRP”. Although single-DCI inter-cell multi-TRP is not agreed, inter-cell beam management is supported. However, inter-cell beam management requires the support of more than one activated DL TCI state. Since only one activated DL TCI state is supported in FG 40-1-2, inter-cell beam management is not possible. In our understanding, inter-cell support is included in FG 40-1-2a. “FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]”. The separate FG proposed under FG 40-1-1 can be reused. “FFS: what to do/signal about two default beams.” This is captured in FG 40-1-6. This FFS can be removed. “FFS: maximum number of activated DL TCI states per CC”. This looks like a typo. Following FG 40-1-1, and the Rel-17 solution, this should be a “maximum number of activated DL TCI states across CCs”, which should be a component in FG 40-1-2. “FFS: maximum number of activated UL TCI states per CC”. This looks like a typo. Following FG 40-1-1, and the Rel-17 solution, this should be a “maximum number of activated UL TCI states across CCs”, which should be a component in FG 40-1-2. “FFS: separate row for TRP-specific BFR with unified TCI framework“. This is captured in FG 40-1-5. The FFS can be removed. - Following the Rel-17 solution, two additional components are needed in FG 40-1-2: maximum number of configured DL TCI states per CC per BWP maximum number of configured UL TCI states per CC per BWP 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; - maximum number of configured DL TCI states per CC per BWP - maximum number of configured UL TCI states per CC per BWP - maximum number of activated DL TCI states across all CCs. maximum number of activated UL TCI states across all CCs. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC; - The maximum number of configured separate DL TCI state per BWP per CC in a band. - The maximum number of configured separate UL TCI state per BWP per CC in a band. - Two MAC-CE activated DL TCI-states per CC in a band. - Two MAC-CE activated UL TCI-states per CC in a band. - MAC-CE based TCI-state indication. - The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band. - The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band; Component 1 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 6 candidate value {2, 4, 8, 16, 32} Component 7 candidate value {2, 4, 8, 16, 32}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated DL TCI states per CC FFS: maximum number of activated UL TCI states per CC FFS: separate row for TRP-specific BFR with unified TCI framework; 23-10-1, 16-2b-1 or 16-2b-2 or 16-2b-3 or 16-2b-4 or 16-2b-5; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-1-2 - Components: - We think support of the following in the FFS can be separate FGs, because of the same reason as FG40-1-1. - Intra-cell and inter-cell multi-TRP. - Additional support of DCI format 1_1 and 1_2 configured with TCI state selection field. - two default beams. - TRP-specific BFR with unified TCI framework |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; 1. Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured DL TCI states per BWP per CC 3. The maximum number of configured UL TCI states per BWP per CC 4. The maximum number of MAC-CE activated DL TCI states per CC in a band 5. The maximum number of MAC-CE activated UL TCI states per CC in a band 6. MAC CE based TCI state indication for a single TCI codepoint 7. The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band 8. The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band; 40-1-1; Yes; Per band; N/A; N/A; N/A; Component 2 candidate value {4, 8, 12, 16, 24, 32, 48, 64, 128} Component 3 candidate value {4, 8, 12, 16, 24, 32, 48, 64} Component 4 candidate value {1, 2} Component 5 candidate value {1, 2} Component 7 candidate value {1, 2, 4, 8, 16} Component 8 candidate value {1, 2, 4, 8, 16} Note: UE should support candidate value of 2 for at least one of Component 4 and 5; Optional with capability signaling. |
| MediaTek Inc. [17] | What to do/signal about two default beams In Rel-16, there is UE capability (FG 16-2b-0) for “Two default beams for single-DCI based multi-TRP”. In our view, the same UE capability can be reused for Rel-18 unified TCI for single-DCI based multi-TRP. Proposal 2: Reuse the legacy UE capability (FG 16-2b-0) for “Two default beams for single-DCI based multi-TRP” Separate row for TRP-specific BFR with unified TCI framework In Rel-17 unified TCI framework, a separate UE capability is introduced for “SCell BFR with unified TCI framework” (FG 23-1-1c). Similarly, a separate row can be introduced for “TRP-specific BFR with unified TCI framework”. Proposal 3: Introduce a separate row for “TRP-specific BFR with unified TCI framework” 40. NR_MIMO_evo_DL_UL; 40-1-2; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC; - Support separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC - Support separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC - Per aperiodic CSI-RS resource/resource set configuration for TCI selection; Optional with UE capability Component-3 candidate values: {resource, resource set, both}. |

| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS that whether separate rows/FGs are needed for intra-cell mTRP and inter-cell mTRP case. To our understanding, new rows are not needed. These FGs are just used to report that UE support unified TCI for mTRP operation. As for whether the unified TCI support is for intra-cell mTRP and inter-cell mTRP, it is subject to whether UE support intra-cell mTRP and inter-cell mTRP. If a UE support both unified TCI for mTRP and intra-cell mTRP operation, it implies that the UE support unified TCI for intra-cell mTRP. Similarly, if a UE support both unified TCI for mTRP and inter-cell mTRP operation, it implies that the UE support unified TCI for inter-cell mTRP. Proposal 2-1: For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is no need to introduce separate rows for intra-cell mTRP and inter-cell mTRP case. For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS on whether a separate row is needed for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. We think that a UE that support unified TCI for single-DCI based mTRP should support the [TCI selection field] in the DCI by default and a separate row is not needed. Proposal 2-2: For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is no need to introduce a row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. For FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a, there is an FFS on what to do/signal about two default beams. In our view, a new FG should be introduced for supporting two default beams under unified TCI framework since a UE supporting FG 40-1-1, FG 40-1-1a, FG 40-1-2 and FG 40-1-2a does not necessarily support two default beams. We have added this as FG 40-1-14 in the updated table at the end of this section. Proposal 2-3: Introduce a new FG for the support of two default beams for a UE that supports FG 40-1-1, FG 40-1-1a, FG 40-1-2 or FG 40-1-2a. Proposal 2-4: Support the following components for the maximum number of configured/activated TCI states: - For FG 40-1-2a, support following components: - The maximum number of activated DL TCI state per BWP per CC - The maximum number of activated UL TCI state per BWP per CC - Note: The maximum number of configured DL/UL TCI states per BWP per CC is not needed for this FG, as its prerequisite FG is FG 40-1-2. 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams 1. maximum number of activated DL TCI state per BWP per CC 2. maximum number of activated UL TCI state per BWP per CC; 23-10-1b, 40-1-2; Per band; n/a; n/a; n/a; Component 1 candidate values: {2, 3, 4, 5, 6, 7, 8, 16} Component 2 candidate values: {2, 3, 4, 5, 6, 7, 8, 16}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-1-2a - FFS1: We recommend having a single row for intra-cell and inter-cell multi-TRP - FFS2: We recommend having a single row for additional support of DCI format 1_1 and 1_2 configured with TCI selection field - FFS3: What to do/signal about two default beams - decisions pending from WI, to be addressed later 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; Support of DCI format 1_1 and 1_2 configured with [TCI selection field] Prerequisite 23-10-1b FFS: separate rows intra-cell and inter-cell multi-TRP Comment: inter-cell single-DCI based multi-TRP is not supported in Rel-18. FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] Comment: no need. Support of configuration with [TCI selection field] should be mandatory. FFS: what to do/signal about two default beams Comment: can reuse the 16-2c and 16-2b-0.. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra and inter-cell multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams. 40. NR_MIMO_evo_DL_UL; 40-1-2b; Support of two default joint/DL TCI states for single-DCI multi-TRP; Note: Applies to FR2. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; 1. Maximum number of activated DL TCI states 2. Maximum number of activated UL TCI states 3. Maximum number of total activated DL and UL TCI states. |
| CATT [9] | 40-1-2a: - The maximum number of activated DL TCI states per CC has been specified in Rel-17 with candidate value {2, 3, 4, 5, 6, 7, 8}. Considering Rel-18 mTRP, the candidate values could be {4, 6, 8, 10, 12, 14, 16}. - The maximum number of activated UL TCI states per CC has been specified in Rel-17 with candidate value {2, 3, 4, 5, 6, 7, 8}. Considering Rel-18 mTRP, the candidate values could be {4, 6, 8, 10, 12, 14, 16}. |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. The input on FG 40-1-2a essentially mirrors the input on FG 40-1-1a. Still, support for DCI-based TCI state indication is not implied by FG 40-1-2a. This is described by the added FG 40-1-2b. Note that FG 40-1-1a will be a pre-requisite for FG 40-1-2a. “FFS: separate rows intra-cell and inter-cell multi-TRP”: In Rel-17, the FG that describes support of multiple activated DL TCI states also include inter-cell operation. Following this design, there is no need for a separate row: support for inter-cell operation is implied for a UE that supports FG 40-1-2a FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field]. The separate FG proposed under FG 40-1-1 can be reused. This FFS can be removed. FFS: what to do/signal about two default beams. This is captured in FG 40-1-6. This FFS can be removed. Following the Rel-17 solution, the following additional components would be needed in FG 40-1-2a: Support K additional MAC-CE activated DL TCI states per CC. Support K additional activated DL TCI states across all CCs. Note that FG 40-1-2a only introduces support for inter-cell beam management. In this case, there is no need to support multiple activated UL TCI states, since the additional activated TCI states are needed only to receive broadcast. 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; - Support K additional MAC-CE activated DL TCI states per CC - Support K additional activated DL TCI states across all CCs.. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC; - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated DL TCI states per CC in a band. - The maximum number of MAC-CE activated UL TCI states per CC in a band. - The minimum beam application time in Y symbols per SCS; Component 2 candidate value {2, 4, 8, 10, 12, 14, 16} Component 3 candidate value {2, 4, 8, 10, 12, 14, 16} Component 4 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams; 23-10-1, 16-2b-1 or 16-2b-2 or 16-2b-3 or 16-2b-4 or 16-2b-5; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-1-1a/40-1-2a - Components: - We think support of the following in the FFS can be separate FGs, because of the same reason as FG40-1-1. - Intra-cell and inter-cell multi-TRP. - Additional support of DCI format 1_1 and 1_2 configured with TCI state selection field. - two default beams. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated DL TCI states per CC in a band 4. The maximum number of MAC-CE activated UL TCI states per CC in a band; 40-1-2; Yes; Per band; N/A; N/A; N/A; Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be indicated in FR2 Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH Note: UE should support candidate value of at least 3 for at least one of Component 3 and 4; Optional with capability signaling. |
| MediaTek Inc. [17] | What to do/signal about two default beams In Rel-16, there is UE capability (FG 16-2b-0) for “Two default beams for single-DCI based multi-TRP”. In our view, the same UE capability can be reused for Rel-18 unified TCI for single-DCI based multi-TRP. Proposal 2: Reuse the legacy UE capability (FG 16-2b-0) for “Two default beams for single-DCI based multi-TRP” 40. NR_MIMO_evo_DL_UL; 40-1-2a; Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC; - Support separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC - Support separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC - Per aperiodic CSI-RS resource/resource set configuration for TCI selection; Optional with UE capability Component-3 candidate values: {resource, resource set, both}. |

| 40. NR_MIMO_evo_DL_UL | 40-1-3 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP | FFS: separate rows for resource set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 40-1-3, in order for the UE to report whether it support per resource TCI selection configuration or per resource set TCI selection configuration, one component should be introduced. In our view, there is no need to introduce two independent FGs for the support of per resource and per resource set TCI selection configuration and the FFS can be removed. Proposal 2-8: For FG 40-1-3, remove the FFS in the fourth column and introduce the following component: - Supported TCI selection granularity for aperiodic CSI-RS, which has three candidate value {per resource, per resource set, both} 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; FFS: separate rows for resource set 1. Supported TCI selection granularity for aperiodic CSI-RS; 23-1-1i; Per band; n/a; n/a; n/a; Component 1 candidate values: {per resource, per resource set, both}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-1-3 - FFS1: We recommend having a single row for per CSI-RS resource set, per CSI-RS resource, or both 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; FFS: separate rows for resource set. |
| ZTE [4] | - Besides for S-DCI based MTRP, we still need another FGs for M-DCI based MTRP. Then, per resource or per resource set should be assumed as separate FGs. Then, in our views, ‘per CSI-RS resource set’ should be assumed as a basic UE features for both M-DCI and S-DCI. 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; FFS: separate rows for resource set Support of per aperiodic CSI-RS resource configuration for TCI selection in S-DCI based MTRP; 40-1-1. 40-1-3a; Per aperiodic CSI-RS resource set configuration for TCI selection in S-DCI based MTRP; Support of per aperiodic CSI-RS resource set configuration for TCI selection in S-DCI based MTRP; 40-1-1. 40-1-3b; Per aperiodic CSI-RS resource configuration for TCI selection in M-DCI based MTRP; Support of per aperiodic CSI-RS resource configuration for TCI selection in M-DCI based MTRP; 40-1-7. 40-1-3c; Per aperiodic CSI-RS resource set configuration for TCI selection in M-DCI based MTRP; Support of per aperiodic CSI-RS resource set configuration for TCI selection in M-DCI based MTRP; 40-1-7. |
| Spreadtrum Communications [5] | For FG 40-1-3, the UE capability of RRC-configured TCI selection for each resource/ resource set should be signaled. A FFS is that whether to separate rows for resource set. In our view, the RRC configuration for each resource set can be regarded as a default capability, and the RRC configuration for each resource can be defined as one of candidate values. Thus, there is no need to specifically separate rows for resource set. Proposal 4: Do not separate rows for resource set. Proposal 5: The component candidate values of FG-40-1-3 candidate values are {per resource, per resource set, both}. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; FFS: separate rows for resource set Comment: no need for a separate row.. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; FFS: separate rows for resource set. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; 1. Granularity for CSI-RS for configuration of TCI selection (candidate value: resource, resource set or both). |
| CATT [9] | 40-1-3: - According to the following agreement, two separate FGs corresponding to per CSI-RS resource set and per CSI-RS resource should be introduced for aperiodic CSI-RS TCI selection. Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability. Proposal 5: For aperiodic CSI-RS, FG 40-1-3 should be separated into two FGs as follows: 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource configuration for TCI selection in S-DCI based MTRP. 40. NR_MIMO_evo_DL_UL; 40-1-Z; Per aperiodic CSI-RS resource set configuration for TCI selection in S-DCI based MTRP. |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. “FFS: separate rows for resource set”. Prefer to add a separate row for the resource set. This is added as FG 40-1-3b - This needs to be added separately for multi-DCI based multi-TRP. 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; - Supported configuration of mode, on unified TCI framework extension for single-DCI based multi-TRP, RRC configuration in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState configures the UE to apply the first or the second indicated joint/DL TCI state per CSI-RS resource or per CSI-RS resource set if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state.; Component 1 candidate value: {per CSI-RS resource set, per CSI-RS resource, both }; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP; FFS: separate rows for resource set; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-1-3 - Components: - We believe separate row is not necessary for configuration “per CSI-RS resource” or “per CSI-RS resource set”. If UE reports FG40-1-3, UE should support both configurations. If we have separate UE capability, gNB will need to implement both configurations. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-3; Association between joint/DL TCI state and AP CSI-RS based on per CSI-RS resource for single-DCI based multi-TRP; Support of RRC configuration per CSI-RS resource to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource; 40-1-1, 40-1-2; Yes; Per band; N/A; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-1-3 To support unified TCI update for AP CSI-RS (for BM and CSI), at least the UE needs to support either resource-based configuration or resource-set-based configuration. Thus, we don’t think this should be put in a separate row, which means support of unified TCI update for AP CSI-RS is a UE optional feature. Instead, this should be a basic component in FG 40-1-1/40-1-1a/40-1-2/40-1-2a, and the UE can report a value from the candidate {resource, resource set, both}. Proposal 5: Remove FG 40-1-3 and include it as a component in FG 40-1-1/40-1-1a/40-1-2/40-1-2a 40. NR_MIMO_evo_DL_UL; 40-1-3; Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP. |

| 40. NR_MIMO_evo_DL_UL | 40-1-4 | Support of two TCI states for CJT Tx scheme for PDSCH | FFS: how to signal QCL subset |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of two TCI states for CJT Tx scheme for PDSCH; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-1-4 - FFS1: how to signal QCL subset : it is not clear, want to clarify. To be addressed later 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of two TCI states for CJT Tx scheme for PDSCH; FFS: how to signal QCL subset. |
| ZTE [4] | - While having two TCI states for CJT Tx, the supported list of QCL subset can be indicated based on the following agreement. 40-1-4; Support of two TCI states for CJT Tx scheme for PDSCH; FFS: how to signal QCL subset Support of one or both of the following QCL mode: {Mode-1, Mode-2, both} - Mode-1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Mode-2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state; 40-1-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of two TCI states for CJT Tx scheme for PDSCH; FFS: how to signal QCL subset. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of two TCI states for CJT Tx scheme for PDSCH; 1. QCL-TypeA indication scheme for the second TCI states (candidate value: QCL-TypeA or QCL-TypeA except for {Doppler shift, Doppler spread}). |
| CATT [9] | 40-1-4: - According to the following agreement of the last meeting, if UE supports two TCI states for CJT-based PDSCH, a UE capability on which alternative(s) is supported should be introduced. Therefore, two additional FGs corresponding to Alt1 and Alt2 should be added. Agreement On unified TCI framework extension for S-DCI based MTRP, the following two alternatives are supported for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state Introduce a UE capability on which alternative(s) is supported, and either one of above alternatives can be configured by RRC according to the UE capability Note: In Rel-18, RAN1 has no consensus to support Alt3 - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB. Proposal 6: For PDSCH-CJT, two additional FGs (40-1-4a, 40-1-4b), shown as follows, should be added. 40. NR_MIMO_evo_DL_UL; 40-1-4a; Use full QCL information. 40. NR_MIMO_evo_DL_UL; 40-1-4b; Use subet of QCL information. |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. “FFS: how to signal QCL subset”. It is proposed to signal this in the same way as for HST, i.e., via two separate FGs: “CJT scheme A” and “CJT scheme B”. FG 40-1-4b is added to handle CJT scheme B. 40-1-4; Support of two TCI states for CJT Tx scheme A for PDSCH. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of two TCI states for CJT (Coherent Joint Transmission) Tx scheme for PDSCH; - Support of unified TCI with two joint/DL TCI states update for CJT (Coherent Joint Transmission) PDSCH transmission scheme - Supported QCL configuration modes. FFS: 1 TCI state for CJT, whether it is Rel-17 UE transparent for uTCI, e.g., Rel-17 uTCI framework; Component 1 candidate value: {Joint UL/DL, Separate UL/DL, both} Component 2 candidate value: {mode 1, mode 2, both} Note: Mode 1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA Mode 2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of two TCI states for CJT Tx scheme for PDSCH; FFS: how to signal QCL subset; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of two TCI states for CJT Tx scheme for PDSCH; Support of two indicated joint TCI states for PDSCH-CJT per BWP per CC in a band; 40-1-1; Yes; Per band; N/A; N/A; FR1 only; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-1-4 Since there are two QCL assumptions can be supported for PDSCH-CJT with two indicated joint TCI states, e.g., cjtSchemeA and cjtSchemeB, and a UE can support any of them if it supports two PDSCH-CJT with two joint TCI states. Proposal 6: Introduce a component “Support two indicated joint TCI states for CJT Tx scheme for PDSCH” in FG 40-1-4 - Candidate value for the component: {cjtSchemeA, cjtSchemeB, both} 40. NR_MIMO_evo_DL_UL; 40-1-4; Support of two TCI states for CJT Tx scheme for PDSCH; - Support two indicated joint TCI states for CJT Tx scheme for PDSCH; Optional with UE capability Component-1 candidate values: {cjtSchemeA, cjtSchemeB, both}. |

| 40. NR_MIMO_evo_DL_UL | 40-1-7 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Proposal 2-9: Introduce the following components for the maximum number of configured/activated TCI states: - For FG 40-1-7, support following components: - maximum number of configured joint TCI states per BWP per CC - maximum number of activated joint TCI states cross all CC 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. maximum number of configured joint TCI states per BWP per CC 2. maximum number of activated joint TCI states across all CCs; 23-1-1; Per band; n/a; n/a; n/a; Component 2 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {1, 2, 4, 8, 16}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - FG 40-1-7 should be assumed as a basic FG for multi-DCI based multi-TRP operation in terms of unified TCI extension. Then, the following should be as components of the FG. - Support of a single default beam for PDSCH with scheduling offset less than a threshold - Maximum number of configured joint TCI states per CC per BWP - Maximum number of activated joint TCI states across all CCs 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of intra-cell mTRP operation for M-DCI 2. Support of a single default beam for PDSCH with scheduling offset less than a threshold 3. Maximum number of configured joint TCI states per CC per BWP 4. Maximum number of activated joint TCI states across all CCs. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for intra-cell multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; Prerequisite 23-1-1 Comment: separate rows are needed for intra-cell multi-DCI based multi-TRP and inter-cell multi-DCI based multi-TRP.. 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for inter-cell multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; Comment: separate rows are needed for intra-cell multi-DCI based multi-TRP and inter-cell multi-DCI based multi-TRP.. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured joint TCI states per CC per BWP. |
| CATT [9] |  |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. Following the Rel-17 solution, these components are needed in FG 40-1-7: Maximum number of configured joint TCI states per CC per BWP. Maximum number of activated joint TCI states across all CCs. 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; - maximum number of configured joint TCI states per CC per BWP - maximum number of activated joint TCI states across all CCs. Support two default beams. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based intra-cell and inter-cell multi-TRP with single activated TCI codepoint per coresetPoolIndex per CC; - Support unified TCI with joint DL/UL TCI update for multi-DCI intra-cell and inter-cell multi-TRP operation - The maximum number of configured joint TCI state per BWP per CC in a band. - One MAC-CE activates one joint TCI-states per CC in a band for a TRP assocaited with a ‘coresetPoolIndex’ value. - The maximum number of MAC-CE activated joint TCI-states across all CC(s) in a band per ‘coresetPoolIndex’ value.; Component 1 candidate values {Intra-cell, intra-cell and inter-cell} Component 2 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate value {1, 2, 4, 8, 16}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 23-1-1, 16-2a; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured joint TCI states per BWP per CC in a band 3. Maximum of two MAC-CE activated joint TCI states per CC in a band 4. MAC CE based TCI state indication for a single activated TCI codepoint per CORESETPoolIndex 5. The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band; 16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4; Yes; Per band; N/A; N/A; N/A; Component 2 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 5 candidate value {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions; Optional with capability signaling. |
| MediaTek Inc. [17] | Separate rows for S-DCI/M-DCI/Intra-Cell/Inter-Cell MTRP In current NR, whether a UE can support S-DCI and/or M-DCI based MTRP can be reported by separate capability signaling corresponding to different MTRP schemes. Also, whether a UE can support inter-cell MTRP can be reported by a corresponding Rel-17 capability signaling. Therefore, we don’t see the need to introduce separate rows for TCI update for different MTRP schemes since the unified TCI framework extension is “unified” for all MTRP schemes. Proposal 1: No need to introduce separate rows for TCI update for S-DCI/M-DCI/Intra-Cell/Inter-Cell MTRP operation - Combine FG 40-1-7 into FG 40-1-1 40. NR_MIMO_evo_DL_UL; 40-1-7; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC. |

| 40. NR_MIMO_evo_DL_UL | 40-1-7a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Proposal 2-9: Introduce the following components for the maximum number of configured/activated TCI states: - For FG 40-1-7a, support following components: - maximum number of activated joint TCI state per CORESETPoolIndex per BWP per CC - Note: maximum number of configured joint TCI states per BWP per CC is not needed for this FG, as its prerequisite FG is FG 40-1-7. 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. maximum number of activated joint TCI states per CORESETPoolIndex per BWP per CC; 23-1-1b 40-1-7; Per band; n/a; n/a; n/a; Component 1 candidate values: {2, 3, 4, 5, 6, 7, 8}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-1-7a, we suggest to capture ‘maximum number of activated joint TCI states per CORESETPoolIndex per CC’ as its component. 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; Maximum number of activated joint TCI states per CORESETPoolIndex per CC; 40-1-7. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-7ab; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; Prerequisite 23-1-1b Comment: this feature is applicable for only intra-cell multi-DCI based multi-TRP or both intra-cell and inter-cell multi-DCI based multi-TRP depends on the support of former two rows.. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. Maximum number of activated joint TCI states per CC. |
| CATT [9] |  |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. Following the Rel-17 solution, these components are needed in FG 40-1-8: Support K additional MAC-CE activated joint TCI states per CC Support K additional activated joint TCI states across all CCs. Support for DCI-based TCI state indication is not implied by FG 40-1-7a. This is described by the added FG 40-1-7b. 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; - Support K additional MAC-CE activated joint TCI states per CC - Support K additional activated joint TCI states across all CCs.. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based intra-cell and inter-cell multi-TRP with multiple activated TCI codepoints per coresetPoolIndex per CC; - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated joint TCI states per CC in a band per ‘coresetPoolIndex’ value. - The minimum beam application time in Y symbols per SCS; Component 2 candidate value {1, 2, 4, 8 } Component 3 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 23-1-1, 16-2a; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated joint TCI states per CC in a band; 40-1-7; Yes; Per band; N/A; N/A; N/A; Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be indicated in FR2 Component 3 candidate values: {3, 4, 5, 6, 7, 8} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH; Optional with capability signaling. |
| MediaTek Inc. [17] | Separate rows for S-DCI/M-DCI/Intra-Cell/Inter-Cell MTRP In current NR, whether a UE can support S-DCI and/or M-DCI based MTRP can be reported by separate capability signaling corresponding to different MTRP schemes. Also, whether a UE can support inter-cell MTRP can be reported by a corresponding Rel-17 capability signaling. Therefore, we don’t see the need to introduce separate rows for TCI update for different MTRP schemes since the unified TCI framework extension is “unified” for all MTRP schemes. Proposal 1: No need to introduce separate rows for TCI update for S-DCI/M-DCI/Intra-Cell/Inter-Cell MTRP operation - Combine FG 40-1-7a into FG 40-1-1a 40. NR_MIMO_evo_DL_UL; 40-1-7a; Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC. |

| 40. NR_MIMO_evo_DL_UL | 40-1-9 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Proposal 2-9: Introduce the following components for the maximum number of configured/activated TCI states: - For FG 40-1-9, support following components: - maximum number of configured DL TCI states per BWP per CC - maximum number of configured UL TCI states per BWP per CC - maximum number of activated DL TCI states cross all CC - maximum number of activated UL TCI states cross all CC 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. maximum number of configured DL TCI states per BWP per CC 2. maximum number of configured UL TCI states per BWP per CC 3. maximum number of activated DL TCI states cross all CC 4. maximum number of activated UL TCI states cross all CC; 23-10-1; Per band; n/a; n/a; n/a; Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 1 candidate values: {1, 2, 4, 8, 16} Component 1 candidate values: {1, 2, 4, 8, 16}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - Then, FG 40-1-7 should be assumed as a prerequisite for FG 40-1-9, and then we provide the following as its components - Maximum number of activated DL TCI states per CORESETPoolIndex per CC - Maximum number of activated UL TCI states per CORESETPoolIndex per CC 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Support of unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC. 2. Maximum number of configured DL TCI states per CC per BWP 3. Maximum number of configured UL TCI states per CC per BWP 4. Maximum number of activated DL TCI states across all CCs 5. Maximum number of activated UL TCI states across all CCs; 40-1-7. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for intra-cell multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; Prerequisite 23-10-1 Comment: separate rows are needed for intra-cell multi-DCI based multi-TRP and inter-cell multi-DCI based multi-TRP.. 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for inter-cell multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; Prerequisite 23-10-1b Comment: separate rows are needed for intra-cell multi-DCI based multi-TRP and inter-cell multi-DCI based multi-TRP.. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Maximum number of configured DL TCI states per CC per BWP 2. Maximum number of configured UL TCI states per CC per BWP 3. Maximum number of total configured DL and UL TCI states per CC per BWP. |
| CATT [9] |  |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. Following the Rel-17 solution, these components are needed in FG 40-1-9: maximum number of configured DL TCI states per CC per BWP maximum number of configured UL TCI states per CC per BWP 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based intra-cell and inter-cell multi-TRP with single activated TCI codepoint per coresetPoolIndex per CC; - Support unified TCI with separate DL/UL TCI update for multi-DCI intra-cell and inter-cell multi-TRP operation - The maximum number of configured separate DL TCI state per BWP per CC in a band. - The maximum number of configured separate UL TCI state per BWP per CC in a band. - One MAC-CE activated DL TCI-state per CC in a band for a TRP assocaited with a ‘coresetPoolIndex’ value. - One MAC-CE activated UL TCI-state per CC in a band for a TRP assocaited with a ‘coresetPoolIndex’ value. - The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band per ‘coresetPoolIndex’ value. - The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band per ‘coresetPoolIndex’ value.; Component 1 candidate values {Intra-cell, intra-cell and inter-cell} Component 2 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 3 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 6 candidate value {1, 2, 4, 8, 16} Component 7 candidate value {1, 2, 4, 8, 16}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 23-10-1, 16-2a; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC; 1. Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured DL TCI states per BWP per CC 3. The maximum number of configured UL TCI states per BWP per CC 4. Maximum of two MAC-CE activated DL TCI states per CC in a band 5. The maximum number of MAC-CE activated UL TCI states per CC in a band 6. MAC CE based TCI state indication for a single activated TCI codepoint per CORESETPoolIndex 7. The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band 8. The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band; 40-1-7; Yes; Per band; N/A; N/A; N/A; Component 2 candidate value {4, 8, 12, 16, 24, 32, 48, 64, 128} Component 3 candidate value {4, 8, 12, 16, 24, 32, 48, 64} Component 5 candidate value {1, 2} Component 7 candidate value {2, 4, 6, 8, 16} Component 8 candidate value {1, 2, 4, 6, 8, 16}; Optional with capability signaling. |
| MediaTek Inc. [17] | Separate rows for S-DCI/M-DCI/Intra-Cell/Inter-Cell MTRP In current NR, whether a UE can support S-DCI and/or M-DCI based MTRP can be reported by separate capability signaling corresponding to different MTRP schemes. Also, whether a UE can support inter-cell MTRP can be reported by a corresponding Rel-17 capability signaling. Therefore, we don’t see the need to introduce separate rows for TCI update for different MTRP schemes since the unified TCI framework extension is “unified” for all MTRP schemes. Proposal 1: No need to introduce separate rows for TCI update for S-DCI/M-DCI/Intra-Cell/Inter-Cell MTRP operation - Combine FG 40-1-9 into FG 40-1-2 40. NR_MIMO_evo_DL_UL; 40-1-9; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC. |

| 40. NR_MIMO_evo_DL_UL | 40-1-9a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Proposal 2-9: Introduce the following components for the maximum number of configured/activated TCI states: - For FG 40-1-9a, support following components: - maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC - maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC - Note: maximum number of configured DL/UL TCI states per BWP per CC is not needed for this FG, as its prerequisite FG is FG 40-1-9. 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 2. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC; 23-10-1b 40-1-9; Per band; n/a; n/a; n/a; Component 1 candidate values: {2, 3, 4, 5, 6, 7, 8} Component 1 candidate values: {2, 3, 4, 5, 6, 7, 8,}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - After that, for FG 40-1-9/9a, the UE can separately indicate the corresponding configured/activated DL/UL TCI state. 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. Maximum number of activated DL TCI states per CORESETPoolIndex per CC 2. Maximum number of activated UL TCI states per CORESETPoolIndex per CC; 40-1-9. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-9ab; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; Comment: this feature is applicable for only intra-cell multi-DCI based multi-TRP or both intra-cell and inter-cell multi-DCI based multi-TRP depends on the support of former two rows.. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. Maximum number of activated DL TCI states 2. Maximum number of activated UL TCI states 3. Maximum number of total activated DL and UL TCI states. |
| CATT [9] |  |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. Following the Rel-17 solution, these components are needed in FG 40-1-9a: Support K additional MAC-CE activated DL TCI states per CC in a band. Support K additional activated DL TCI states across all CCs. - Support for DCI-based TCI state indication is not implied by FG 40-1-9a. This is described by the added FG 40-1-9b. 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based intra-cell and inter-cell multi-TRP with multiple activated TCI codepoints per coresetPoolIndex per CC; - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated DL TCI states per CC in a band per ‘coresetPoolIndex’ value. - The maximum number of MAC-CE activated UL TCI states per CC in a band per ‘coresetPoolIndex’ value. - The minimum beam application time in Y symbols per SCS; Component 2 candidate value {1, 2, 4, 8} Component 3 candidate value {1, 2, 4, 8} Component 4 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 23-10-1, 16-2a; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC; 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated DL TCI states per CC in a band 4. The maximum number of MAC-CE activated UL TCI states per CC in a band; 40-1-9; Yes; Per band; N/A; N/A; N/A; Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be indicated in FR2 Component 3 candidate values: {2, 3, 4, 5, 6, 7, 8} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH Note: UE should support candidate value of at least 3 for at least one of Component 3 and 4; Optional with capability signaling. |
| MediaTek Inc. [17] | Separate rows for S-DCI/M-DCI/Intra-Cell/Inter-Cell MTRP In current NR, whether a UE can support S-DCI and/or M-DCI based MTRP can be reported by separate capability signaling corresponding to different MTRP schemes. Also, whether a UE can support inter-cell MTRP can be reported by a corresponding Rel-17 capability signaling. Therefore, we don’t see the need to introduce separate rows for TCI update for different MTRP schemes since the unified TCI framework extension is “unified” for all MTRP schemes. Proposal 1: No need to introduce separate rows for TCI update for S-DCI/M-DCI/Intra-Cell/Inter-Cell MTRP operation - Combine FG 40-1-9a into FG 40-1-2a 40. NR_MIMO_evo_DL_UL; 40-1-9a; Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC. |

| 40. NR_MIMO_evo_DL_UL | 40-1-12 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 23-1-1f and at least one of 40-1-1, 40-1-1a, 40-1-2 or 40-1-2a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - Sharing TCI state pool across different CC should be supported as a basic feature, and then a separate row may NOT be needed. 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; 40-1-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; Prerequisite 23-1-1f. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; - Common multi-CC TCI state ID update and activation with joint TCI upate for single-DCI based multi-TRP. - Maximum number of configured CC lists per cell group for common multi-CC TCI state ID activation/update.; Component 2 candidate value: {1, 2, ,3, 4 }; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP; Common multi-CC TCI state ID update and activation; 40-1-1, 40-1-2; Yes; Per band; N/A; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-1-12/40-1-13 We also don’t see the need to introduce different rows for common multi-CC TCI state ID update for S-DCI and M-DCI based MTRP operations in this case since the unified TCI framework extension is “unified” for all MTRP schemes. Proposal 7: No need to introduce separate rows for common multi-CC TCI state ID update for S-DCI and M-DCI based MTRP operations - Combine FG 40-1-12 and FG 40-1-13 into one FG 40. NR_MIMO_evo_DL_UL; 40-1-12; Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP operation. |

| 40. NR_MIMO_evo_DL_UL | 40-1-13 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 23-1-1f and at least one of 40-1-7, 40-1-7a, 40-1-9 or 40-1-9a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - Sharing TCI state pool across different CC should be supported as a basic feature, and then a separate row may NOT be needed. 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; 40-1-7. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; Prerequisite 23-1-1f. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | Proposal: As baseline, the UE features related to the multi-TRP extension of the unified TCI framework are provided per band. 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; - Common multi-CC TCI state ID update and activation with joint TCI upate for multi-DCI based multi-TRP. - Maximum number of configured CC lists per cell group for common multi-CC TCI state ID activation/update.; Component 2 candidate value: {1, 2, ,3, 4 }; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; Common multi-CC TCI state ID update and activation; 40-1-7, 40-1-9; Yes; Per band; N/A; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-1-12/40-1-13 We also don’t see the need to introduce different rows for common multi-CC TCI state ID update for S-DCI and M-DCI based MTRP operations in this case since the unified TCI framework extension is “unified” for all MTRP schemes. Proposal 7: No need to introduce separate rows for common multi-CC TCI state ID update for S-DCI and M-DCI based MTRP operations - Combine FG 40-1-12 and FG 40-1-13 into one FG 40. NR_MIMO_evo_DL_UL; 40-1-13; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP. |

| 40. NR_MIMO_evo_DL_UL | 40-2-1 | Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement | FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 40-2-1 and FG 40-2-2, there is an FFS on whether to have separate rows for joint TCI framework and separate TCI framework. In our view, this is not needed, these two FGs are just used to indicate whether two TA enhancement is supported for mTRP. Whether UE support two TA enhancement under joint TCI framework or separate TCI framework can be implied by whether UE support joint or separate TCI framework. In particular, if a UE support two TA enhancement and joint TCI framework, then it supports joint TCI based two TA enhancement. Similarly, if a UE support two TA enhancement and separate TCI framework, then it supports separate TCI based two TA enhancement. Proposal 2-10: For FG 40-2-1 and FG 40-2-2, there is no need to introduce separate rows for two TA enhancement under joint and separate TCI framework. For FG 40-2-1 and FG 40-2-2, there is an FFS on whether to have a separate row/FG for the supporting of absolute TA command MAC-CE. In our view, a UE support two TA enhancement may not support absolute TA command MAC-CE. So, a new FG should be introduced for it. Proposal 2-11: Introduce a new FG for the supporting of absolute TA command MAC-CE. 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: 1. Maximum number of n-TimingAdvanceOffset value per serving cell; Per band; n/a; n/a; n/a; Component 1 candidate values: {1, 2}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-2-1 - FFS1: We wouldn’t recommend splitting joint TCI states and separating DL/UL TCI states into two rows/FGs - FFS2: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH - decisions pending from WI, to be addressed later - FFS3: what to do/signal about absolute TA command MAC CE- decisions pending from WI, to be addressed later - FFS4: Maximum number of n-TimingAdvanceOffset values per serving cell: May want to wait for RAN4 inputs. Conclusion (RAN1#109e) For multi-DCI multi-TRP operation with two TAs, the decision on the maximum uplink timing difference is left up to RAN4. • Send an LS to RAN4 asking them the maximum uplink timing difference RAN1 can assume between the two TAs for multi-DCI multi-TRP operation. The LS is endorsed in R1-2205593.. 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell Support for PDCCH order from one TRP for cross-TRP RACH process. |
| ZTE [4] | - For FG 40-2-1, - Regarding the first FFS, note that it was agreed that each of two TAGs is associated with UL/joint TCI state, it is not needed to split joint/separate TCI state as two sub-cases. Instead, it is sufficient to generally capture unified TCI state framework for two TAs enhancement in MDCI MTRP operation. - Regarding the second FFS, it can be common understanding that MDCI based STxMP PUSCH+PUSCH is in scope of MDCI based intra-cell MTRP operation, hence this component is not needed. - Regarding the third FFS, it should be removed due to there is no related agreement reached so far in the discussion of AI 9.1.1.2. - Regarding the forth FFS, it is proper to specify the case of “two n-TimingAdvanceOffset value per serving cell” as a child FG, which is added as below, i.e. FG 40-2-9. - Besides, the following components should be added: - Support of configuring two TAGs within a serving cell - Support of joint/separate DL/UL TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG - Support of Rx timing difference between the two DL reference timings is no larger than CP length - Support of RAR-based solution where RAR is only received from a TRP that is associated with Type 1 CSS - Last, its prerequisite should include FG 40-1-7, FG 40-1-7a, FG 40-1-9, FG 40-1-9a. 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell - Support of configuring two TAGs within a serving cell - Support of joint/separate DL/UL TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG - Support of Rx timing difference between the two DL reference timings is no larger than CP length - Support of RAR-based solution where RAR is only received from a TRP that is associated with Type 1 CSS; 40-1-7, 40-1-7a, 40-1-9, 40-1-9a. |
| Spreadtrum Communications [5] | Regarding whether to combine two TA with joint TCI states and separate DL/UL TCI states separately, we have not seem the necessity, and there is no any additional complexity for this issue. Proposal 6: Suggest to remove FFS parts related to ‘split joint TCI states and separate DL/UL TCI states’. Proposal 8: Suggest to add one component into FG 40-2-1 and 40-2-2 - Component x: two UL transmissions are expected not to overlap when UE does not support U STxMP Regarding the combination with multi-DCI based STxMP PUSCH+PUSCH, it is observed that it seems only to be paid special attention to the issue on overlapping handling, and other FGs/components can be reused. In our mind, if UE supports STxMP and two TA, it is natural for UE to be capable of supporting two overlapping transmissions. Thus, we suggest to add one component in FG 40-2-1 and 40-2-2 to state that two UL transmissions can be overlapped when UE supports STxMP (i.e., 40-6-1, 40-6-3a) Proposal 9: Suggest to add one component into FG 40-2-1 and 40-2-2 to state that two UL transmissions can be overlapped when UE supports STxMP. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs Comments: no need to split. FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH Comments: candidate values can be listed for 2TA basic feature: {for mDCI-based TDM PUSCH, for STx2P, for both} FFS: what to do/signal about absolute TA command MAC CE Comments: no need to consider. FFS: Maximum number of n-TimingAdvanceOffset value per serving cell Comments: candidate value {1,2} for the number of TimingAdvanceOffset value is required.. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement 1. Maximum number of n-TimingAdvanceOffset values per serving cell 2. Whether the overlapping duration of the later of the two UL transmissions is reduced; Note: If STxMP PUSCH+PUSCH is configured, component 2 is not applicable. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; 1. TCI type associated with a TAG (Candidate value: UL TCI, joint TCI or both). |
| CATT [9] | 40-2-1: - According to the following agreement, for multi-DCI based multi-TRP operation, TAG ID can be associated with UL TCI state or joint TCI state. Considering not all UEs can work in both joint TCI state and separate DL/UL TCI states mode, two separate FGs corresponding to joint TCI states and separate DL/UL TCI states should be introduced for associating TAG to TCI-state. Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG. Proposal 7: For multi-DCI based intra-cell Multi-TRP operation with two TA enhancement, FG 40-2-1 should be separated into two FGs as follows: 40. NR_MIMO_evo_DL_UL; 40-2-1; Multi-DCI based intra-cell Multi-TRP operation with two TA enhancement for joint TCI state. 40. NR_MIMO_evo_DL_UL; 40-2-X; Multi-DCI based intra-cell Multi-TRP operation with two TA enhancement for separate DL/UL TCI state. |
| Ericsson [10] | This FG contains several FFSs. We will address them one by one: “FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs”: Support for joint TCI and separate DL/UL TCI states need to be reported separately. Hence, there is a need for a separate FG for separate DL/UL TCI states. This is added as FG 40-2-1a. “FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH”: We do not see that STxMP needs special treatment. The FFS can be removed. “FFS: what to do/signal about absolute TA command MAC CE”. RAN1 made the following agreements Agreement (RAN1#110bis) For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI Conclusion (RAN1#112bis) For multi-DCI based Multi-TRP operation with two TA enhancement, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2: - One of two TAG IDs configured in the SpCell can be indicated Hence, RAN1 has agreed to support an enhanced absolute TAC MAC CE, and a new feature group is needed. We propose to add that as FG 40-2-3. “FFS: Maximum number of n-TimingAdvanceOffset value per serving cell”. We do not see that it is possible to define more than one n-TimingAdvanceOffset value per serving cell, since this is a cell-specific quantity: it determines the offset between UL and DL and is the same for all UEs in a cell. For now, this FFS can remain. 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement for joint TCI states; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell. 40-2-1a; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement for separate DL/UL states; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; - Support of 2 TA/TAG within a serving cell for intra-cell multi-DCI multi-TRP - Rx timing difference between different TRP is no larger than CP length - The UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to one TAG; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; 1. Support two TAs for multi-DCI based intra-cell multi-TRP. 2. Support configuring two TAGs belonging to a serving cell. 3. Support two DL reference timings. 4. Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE. 5. Supported maximum number of n-TimingAdvanceOffset value per serving cell.; 16-2a; yes; n/a; Per Band; n/a; n/a; n/a; Component 5 candidate values: {1,2}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-2-1 - Components: - This FG can be common for joint TCI state and separate TCI state and does not need to be split. - A separate row/FG is not needed for STxMP. Support of two TA in STxMP is same as support of two TA in single panel transmission. - A separate FG is needed for absolute TA command MAC CE. Absolute TA command MAC CE is for 2-step RACH. UE can support this feature only if it supports 2-step RACH. - Support of up to two n-TimingAdvanceOffset should be a component of the basic feature group. - There is a working assumption to support PDCCH order sent by one TRP triggers RACH procedure towards a different TRP in intra-cell M-TRP, which should also be included in the basic feature group. - To summarize, following components should be included in basic feature of intra-cell M-TRP TA. - Support of configuring two TAGs belonging to a serving cell. - Support of configuring two n-TimingAdvanceOffset values per serving cell. - Support of two DL reference timings. - PDCCH order sent by one TRP triggers RACH procedure towards a different TRP (This is WA) - Prerequisite FG: FG23-1-1 (unified TCI) |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-2-1; Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement; Support of two TAGs in a same CC for intra-cel multi-DCI based multi-TRP FFS: what to do/signal about absolute TA command MAC CE; 16-2a; Yes; N/A; FS; No; No; N/A; Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP; Optional with capability signaling. |
| MediaTek Inc. [17] | Split joint TCI states and separate DL/UL TCI states into two rows/FGs We see UE implementation to support two TAs for MTRP operation is the same for both joint and separate TCI update modes, thus there is no need to split joint TCI states and separate DL/UL TCI states into two rows. Proposal 9: No need to introduce separate rows for joint TCI states and separate DL/UL TCI states |

| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 40-2-1 and FG 40-2-2, there is an FFS on whether to have separate rows for joint TCI framework and separate TCI framework. In our view, this is not needed, these two FGs are just used to indicate whether two TA enhancement is supported for mTRP. Whether UE support two TA enhancement under joint TCI framework or separate TCI framework can be implied by whether UE support joint or separate TCI framework. In particular, if a UE support two TA enhancement and joint TCI framework, then it supports joint TCI based two TA enhancement. Similarly, if a UE support two TA enhancement and separate TCI framework, then it supports separate TCI based two TA enhancement. Proposal 2-10: For FG 40-2-1 and FG 40-2-2, there is no need to introduce separate rows for two TA enhancement under joint and separate TCI framework. For FG 40-2-1 and FG 40-2-2, there is an FFS on whether to have a separate row/FG for the supporting of absolute TA command MAC-CE. In our view, a UE support two TA enhancement may not support absolute TA command MAC-CE. So, a new FG should be introduced for it. Proposal 2-11: Introduce a new FG for the supporting of absolute TA command MAC-CE. 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: 1. Maximum number of n-TimingAdvanceOffset value per serving cell; Per band; n/a; n/a; n/a; Component 1 candidate values: {1, 2}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-2-2 - FFS1: We wouldn’t recommend splitting joint TCI states and separate DL/UL TCI states into two rows/FGs - FFS2: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH - decisions pending from WI, to be addressed later - FFS3: what to do/signal about absolute TA command MAC CE- decisions pending from WI, to be addressed later - FFS4: Maximum number of n-TimingAdvanceOffset value per serving cell: Same comment as FG 40-2-1 FFS4 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell Support for PDCCH order from one TRP for cross-TRP RACH process. |
| ZTE [4] | - For FG 40-2-2, - Regarding the first FFS, similar to that of FG 40-2-1, it is sufficient to generally capture unified TCI state framework for two TAs enhancement in MDCI MTRP operation. - Regarding the second FFS, MDCI based STxMP PUSCH+PUSCH is also in scope of MDCI based inter-cell MTRP operation, hence this component is not needed. - Regarding the third FFS, it should be removed due to there is no related agreement reached so far in the discussion of AI 9.1.1.2. - Regarding the forth FFS, it is proper to specify the case of “two n-TimingAdvanceOffset value per serving cell” as a child FG, which is added as below, i.e. FG 40-2-9. - Besides, the following components should be added: - Support of configuring two TAGs within a serving cell - Support of joint/separate DL/UL TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG - Support of Rx timing difference between the two DL reference timings is no larger than CP length - Support of RAR-based solution where RAR is only received from a TRP that is associated with Type 1 CSS - Last, its prerequisite should include FG 40-1-7, FG 40-1-7a, FG 40-1-9, FG 40-1-9a. 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell - Support of 2 TAG per serving cell - Support of joint/separate DL/UL TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG - Support of Rx timing difference between the two DL reference timings is no larger than CP length - Support of RAR-based solution where RAR is only received from a TRP that is associated with Type 1 CSS; 40-1-7, 40-1-7a, 40-1-9, 40-1-9a. |
| Spreadtrum Communications [5] | Regarding whether to combine two TA with joint TCI states and separate DL/UL TCI states separately, we have not seem the necessity, and there is no any additional complexity for this issue. Proposal 6: Suggest to remove FFS parts related to ‘split joint TCI states and separate DL/UL TCI states’. Proposal 8: Suggest to add one component into FG 40-2-1 and 40-2-2 - Component x: two UL transmissions are expected not to overlap when UE does not support U STxMP Regarding the combination with multi-DCI based STxMP PUSCH+PUSCH, it is observed that it seems only to be paid special attention to the issue on overlapping handling, and other FGs/components can be reused. In our mind, if UE supports STxMP and two TA, it is natural for UE to be capable of supporting two overlapping transmissions. Thus, we suggest to add one component in FG 40-2-1 and 40-2-2 to state that two UL transmissions can be overlapped when UE supports STxMP (i.e., 40-6-1, 40-6-3a) Proposal 9: Suggest to add one component into FG 40-2-1 and 40-2-2 to state that two UL transmissions can be overlapped when UE supports STxMP. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs Comments: no need to split. FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH Comments: candidate value can be listed for 2TA basic feature: {for mDCI-based TDM PUSCH, for STx2P, for both} FFS: what to do/signal about absolute TA command MAC CE Comments: no need to consider. FFS: Maximum number of n-TimingAdvanceOffset value per serving cell Comments: candidate value {1,2} for the number of TimingAdvanceOffset value is required.. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement 1. Maximum number of n-TimingAdvanceOffset values per serving cell 2. Whether the overlapping duration of the later of the two UL transmissions is reduced; Note: If STxMP PUSCH+PUSCH is configured, component 2 is not applicable. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; 1. TCI type associated with a TAG (Candidate value: UL TCI, joint TCI or both). |
| CATT [9] |  |
| Ericsson [10] | This FG contains several FFSs. We will address them one by one: “FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs.”: Support for joint TCI and separate DL/UL TCI states need to be reported separately. Hence, there is a need for a separate FG for separate DL/UL TCI states. This is added as FG 40-2-2a. “FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH.” We do not see that STxMP needs special treatment. The FFS can be removed. “FFS: what to do/signal about absolute TA command MAC CE”. A new FG is needed. FG 40-2-3 introduced earlier can be reduced also for inter-cell – there is no difference. “FFS: Maximum number of n-TimingAdvanceOffset value per serving cell” We do not see that it is possible to define more than one n-TimingAdvanceOffset value per serving cell, since this is a cell-specific quantity: it determines the offset between UL and DL and is the same for all UEs in a cell. For now, this FFS can remain. 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement for joint TCI states; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell. 40-2-2a; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement for separate UL/DL TCI states; FFS: Maximum number of n-TimingAdvanceOffset value per serving cell. 40-2-3; Support TAG Id in absolute TAC MAC CE. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; - Support of 2 TA/TAG within a serving cell for inter-cell multi-DCI multi-TRP - Rx timing difference between different TRP is no larger than CP length - The UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to one TAG; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; 1. Support two TAs for multi-DCI based inter-cell multi-TRP. 2. Support configuring two TAGs belonging to a serving cell. 3. Support two DL reference timings. 4. Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE. 5. Supported maximum number of n-TimingAdvanceOffset value per serving cell.; 16-2a; yes; n/a; Per Band; n/a; n/a; n/a; Component 5 candidate values: {1,2}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-2-2 - Components: The components can be similar as 40-2-1. In addition, for inter-cell M-TRP, support of PRACH configuration associated with non-serving cell should be included. Following components should be included in basic feature of inter-cell M-TRP TA. - Support of configuring two TAGs belonging to a serving cell. - Support of configuring two n-TimingAdvanceOffset values per serving cell. - Support of two DL reference timings. - Support of PRACH configuration associated with PCI different from serving cell PCI. - Support of PDCCH order sent by one TRP triggers RACH procedure to different TRP - Prerequisite FG: FG23-1-1a (unified TCI for inter-cell M-TRP) |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-2-2; Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement; 1. Support of two TAGs in a same CC inter-cel multi-DCI based multi-TRP 2. The maximum number of additional PRACH configurations corresponding to different additional PCIs FFS: what to do/signal about absolute TA command MAC CE; 23-4; Yes; N/A; FS; No; No; N/A; Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP Component 2 candidate values: {1,2,…,7}; Optional with capability signaling. |
| MediaTek Inc. [17] | Split joint TCI states and separate DL/UL TCI states into two rows/FGs We see UE implementation to support two TAs for MTRP operation is the same for both joint and separate TCI update modes, thus there is no need to split joint TCI states and separate DL/UL TCI states into two rows. Proposal 9: No need to introduce separate rows for joint TCI states and separate DL/UL TCI states |

| 40. NR_MIMO_evo_DL_UL | 40-2-4 | Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA | FFS: merge with 40-2-2 FFS: inter-/intra-cell |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-2-4 - FFS1: we recommend merging FG 40-2-4 with 40-2-1 and 40-2-2 - FFS2: we recommend merging FG 40-2-4 with 40-2-1 and 40-2-2 for inter-cell and intra-cell scenarios 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; FFS: merge with 40-2-2 FFS: inter-/intra-cell. |
| ZTE [4] | - For FG 40-2-4, - It should be a child FG that independent with FG 40-2-2, so its prerequisite should include FG 40-2-2. - According to the agreements reached so far, at least the case of inter-cell MTRP should be captured. By comparison, the case of inter-cell MTRP is still pending as WA. - Besides, the following components should be added: - PDCCH scheduling RAR is always received from serving cell - Support of one additional PRACH configuration is supported for each configured additional PCI different from the PCI of the serving cell when multi-DCI based intra-cell Multi-TRP operation - Support of indicating which PRACH configuration to be used in the RACH procedure in the PDCCH order - Support of inter-cell [and intra-cell] multi-DCI based MTRP operation 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; FFS: merge with 40-2-2 FFS: inter-/intra-cell - PDCCH scheduling RAR is always received from serving cell - Support of one additional PRACH configuration is supported for each configured additional PCI different from the PCI of the serving cell when multi-DCI based intra-cell Multi-TRP operation - Support of indicating which PRACH configuration to be used in the RACH procedure in the PDCCH order - Support of inter-cell [and intra-cell] multi-DCI based MTRP operation.; [40-2-1], 40-2-2. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; FFS: merge with 40-2-2 Comments: no need to merge FFS: inter-/intra-cell Comments: separate rows for inter-intra-cell.. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG contains two FFSs. We will address them one by one: “FFS: merge with 40-2-2”. In our view, this cannot be merged with 40-2-2. “FFS: inter-/intra-cell.” This needs to be separated into intra-cell/inter-cell. FG 40-2-4a is introduced for the inter-cell case. 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for the intra-cell case; This is a WA. 40-2-4a; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for the inter-cell case. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; - For multi-DCI based Multi-TRP operation with two TA, support of a PDCCH order sent by TRP X triggers RACH procedure towards a different TRP Y; Component 1 candidate value: {Intra-cel, inter-cell, both}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; 1. Support of a PDCCH order sent by one TRP triggers RACH procedure towards a different TRP for inter-cell multi-DCI. 2. FFS: Support of a PDCCH order sent by one TRP triggers RACH procedure towards a different TRP for intra-cell multi-DCI.; yes; n/a; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-2-4 - It should be basic feature and merged to 40-2-1 and 40-2-2. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-2-4; Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA; Support of cross-TRP PDCCH order based on CFRA for inter-cell multi-DCI based mTRP Note: Intra-cell case is a WA; yes; N/A; band; No; No; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-2-5 | TCI states of one CORESET Pool associated to both TAGs | This is a WA FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one CORESET Pool associated to both TAGs; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-2-5 - FFS1: We recommend removing “ This is a WA” and splitting joint TCI states and separate DL/UL TCI states into two rows/FGs 40. NR_MIMO_evo_DL_UL; 40-2-5; Joint TCI states of one CORESET Pool associated to both TAGs; This is a WA FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs. 40. NR_MIMO_evo_DL_UL; 40-2-5a; Separate DL/UL TCI states of one CORESET Pool associated to both TAGs. |
| ZTE [4] | - For FG 40-2-5, - It should be removed as of now due to there is no related agreement reached so far to confirm this WA. 40-2-5; TCI states of one CORESET Pool associated to both TAGs; This is a WA FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs. |
| Spreadtrum Communications [5] | Regarding whether to combine two TA with joint TCI states and separate DL/UL TCI states separately, we have not seem the necessity, and there is no any additional complexity for this issue. Proposal 6: Suggest to remove FFS parts related to ‘split joint TCI states and separate DL/UL TCI states’. |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one CORESET Pool associated to both TAGs; 1. TCI type associated with a TAG (Candidate value: UL TCI, joint TCI or both). |
| CATT [9] |  |
| Ericsson [10] | This contains one FFS. This is a WA “FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs”. Support for joint TCI and separate DL/UL TCI states need to be reported separately. Hence, there is a need for a separate FG for separate DL/UL TCI states. This is added as FG 40-2-5a. 40-2-5; TCI states of one CORESET Pool associated to both TAGs for joint TCI states; This is a WA. 40-2-5a; TCI states of one CORESET Pool associated to both TAGs for separate UL/DL TCI states; This is a WA. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one coresetPoolIndex associated to both TAGs; - Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one CORESET Pool associated to both TAGs; 1. FFS: Support of TCI states associated to one CORESETPoolIndex correspond to both TAGs.; yes; n/a; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-2-5 - Component: This FG can be common for joint TCI state and separate TCI state and does not need to be split. - Prerequisite FG: FG 40-2-1 or 40-2-2 |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-2-5; TCI states of one CORESET Pool associated to both TAGs; This is a WA FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs. |
| MediaTek Inc. [17] | Split joint TCI states and separate DL/UL TCI states into two rows/FGs We see UE implementation to support two TAs for MTRP operation is the same for both joint and separate TCI update modes, thus there is no need to split joint TCI states and separate DL/UL TCI states into two rows. Proposal 9: No need to introduce separate rows for joint TCI states and separate DL/UL TCI states |

| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-2-6, - The following components should be added: - Maximum value of Rx timing difference - Its prerequisite should include FG 40-2-1, FG 40-2-2. 40-2-6; Rx timing difference larger than CP length; - Maximum value of Rx timing difference; 40-2-1, 40-2-2. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; 1. Supported mTRP operation with Rx timing difference larger than CP length (Candidate value: intra-cell mTRP, inter-cell mTRP or both). |
| CATT [9] |  |
| Ericsson [10] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; - Support of maximum Rx timing difference between the two DL reference timings larger than CP; Optional with capability signalling. |
| Apple [11] |  |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; 1. Support of the Rx timing difference between the two DL reference timings is larger than CP length.; 16-2a; yes; n/a; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-2-6 - Prerequisite FG: FG 40-2-1 or 40-2-2 |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-2-6; Rx timing difference larger than CP length; Maximum receive timing difference between the DL transmissions from two TRPs can exceed a CP for multi-DCI based mTRP operation; 16-2a; Yes; N/A; FSPC; No; No; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-2-8 | The maximum number of TAGs all CCs |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; 1. The maximum number of TAGs all CCs; Per band; n/a; n/a; n/a; Component 2 candidate values: {1, 2, 3, 4}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-2-8, - Its prerequisite should include FG 40-2-1, FG 40-2-2. 40-2-8; The maximum number of TAGs all CCs; 40-2-1, 40-2-2. 40-2-9; Two n-TimingAdvanceOffset value in serving cell; 40-2-1, 40-2-2. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; 1. Maximum number of TAGs across all CCs 2. Maximum number of TAGs across all CCs configured with two TA. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; - Maximum number of TAGs that can be supported across all intra-frequency and inter-frequency CCs; Component 1 candidate value: { 2, 3, 4[, 5, 6, 7, 8]} Note: the UE only supports the configuration where all UL CCs of the same frequency band are configured with up to 2 Timing Advance Group ID.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs all CCs; 1. Supported maximum number of TAGs per cell group.; yes; n/a; Per Band; n/a; n/a; n/a; Component 1 candidate values: {4,8}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-2-8 - Prerequisite FG: FG 40-2-1 or 40-2-2 - Candidate values: {1,2,3,4} |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-2-8; The maximum number of TAGs across all CCs; Supported maximum number of TAGs across all CCs when at least one CC is configured with 2 TAGs.; Yes; N/A; Band combination; No; No; N/A; {2,3,4} Note: The same description of “supportedNumberTAG” in 38.306 applies to this FG as well.; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-1 | Support of mode 2 for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only A-CSI only FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For CJT CSI codebook, similar to legacy Type I single panel codebook in FG 2-36, Type I multi panel codebook in FG 2-40, Type II codebook in FG 2-41 and etc., a list of supported capability combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC should be introduced. The gNB can configure multiple CSI measurement according to the above listed combinations. For example, if UE reports {16, 8, 128}, then gNB can configure CSI report #1 and CSI report #2 based on {16, 4, 64} and {16, 4, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, CSI report #3, and CSI report #4 based on {16, 2, 64}, {16, 2, 64}, {16, 2, 64} and {16, 2, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, and CSI report #3 based on {16, 2, 64}, {16, 2, 64}, {16, 4, 64}, respectively. However, it is quite complicated for UE preforming CJT CSI measurement on 4 CSI-RS resources than 2 CSI-RS resources. Therefore, to avoid the configuration of the base station exceeding UE capacity, a list of supported combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement should be introduced. In RAN1#113, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-1-1 for Rel-16-based CJT and FG 40-3-1-5 for Rel-17-based CJT are endorsed, where “A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously” and “FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement” are both FFS. Thus we have the following Proposal: Proposal 3-1: Introduce following components in FG 40-3-1-1, 40-3-1-1a, 40-3-1-5 and 40-3-1-5a: - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only A-CSI only FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-1 - This FG should include all the basic components for Rel-16 based Type-II-CJT, and support of mode-2 is one of the basic components. 1. Support of N=N_TRP only 2. Support of N_L=1 only 3. Support of codebook mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, with R=1: a) Max # of Tx ports in one resource b) Max # of resources c) total # of Tx ports 5. A list of supported combinations in one NZP CSI-RS resource set, with R=1: a) # of Tx ports in one resource b) Max # of resources 6. Support of parameter combinations with L1=…=LN_TRP, L1<6 and {pv, beta} ≠{{1/2,1/2,1/2,1/2}, 1/2} 7. Support of rank 1,2 - Both aperiodic and semi-persistent CSI reporting on PUSCH should be supported as basic feature - Agreement (RAN1#110bis-e) - For the Rel-18 Type-II codebook refinement for CJT mTRP, following legacy, support both aperiodic and semi-persistent CSI reporting on PUSCH.. - FFS1: the triplets {max # ports, max # resources, total # ports} should be provided across all CCs per band and per BC. - FFS2: a UE can indicate a list of supported combinations in one CSI-RS resource set: {max # ports, max # resources}. This component limits the max value of NTRP that can be configured for a given max. number of ports per resource. No need to indicate “total # of ports” because all the resources in a resource set have the same number of ports. - Support of parameter combinations {L1,…,LNTRP} and {pυ,β} with L1<6 and pυ,β≠{{1/2,1/2,1/2,1/2},1/2} - Support of rank 1,2 is a basic feature inherited from Rel-16 Type-II 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Basic features of Rel-16-based CJT type-II codebook; 1. Support of N=N_TRP only 2. Support of N_L=1 only 3. Support of codebook mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, with R=1: a) Max # of Tx ports in one resource b) Max # of resources c) total # of Tx ports 5. A list of supported combinations in one NZP CSI-RS resource set, with R=1: a) Max # of Tx ports in one resource b) Max # of resources 6. Support of parameter combinations with L1=…=LN_TRP, L1<6 and {pv, beta} ≠{{1/2,1/2,1/2,1/2}, 1/2} 7. Support of rank 1,2 Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG; Yes; Rel-16-based CJT CSI reporting is not supported; Per band and per BC; Component 4 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {8 to 256} Component 5 candidate values: a) {4,8,12,16,24,32} b) {2,3,4}. |
| ZTE [4] | - Regarding CPU occupation, considering that it’s the necessary parameter for enabling this CJT CSI, we can introduce a component for FG 40-3-1-1. 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; - Support of N=N_TRP only - Support of N_L=1 only - Support of A-CSI report only - FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously - FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement - Value of X for CPU occupation (OCPU = X.NTRP) Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only A-CSI only A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously (per band and per BC) Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI-ReportConfig Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; - Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. - Support of parameter combinations with L=2,4 - Support of rank 1,2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode 2, i.e., common freqencyt basis selection among different TRPs - Supported NL=1 list for spatial basis selection - Supported N=NTRP for multi-TRP selection; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1,2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only A-CSI only FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG; 16-3a; Yes; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1; Support of mode 2 for Rel-16-based CJT type-II codebook; • Support of rank 1,2 • {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support regular eType-II-CJT for R=1 • No mandatary support of p_v != {1/2, 1/2, 1/2, 1/2} and beta=1/2 • Support of parameter combinations satisfying - N_TRP <= [2], and - L_total <= [8], and - p_v != [{1/8, 1/8, 1/16, 1/16}]; 2-35; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource set: {8,...,128} - Max # resource sets: {1 to 16} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-1a | Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For CJT CSI codebook, similar to legacy Type I single panel codebook in FG 2-36, Type I multi panel codebook in FG 2-40, Type II codebook in FG 2-41 and etc., a list of supported capability combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC should be introduced. The gNB can configure multiple CSI measurement according to the above listed combinations. For example, if UE reports {16, 8, 128}, then gNB can configure CSI report #1 and CSI report #2 based on {16, 4, 64} and {16, 4, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, CSI report #3, and CSI report #4 based on {16, 2, 64}, {16, 2, 64}, {16, 2, 64} and {16, 2, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, and CSI report #3 based on {16, 2, 64}, {16, 2, 64}, {16, 4, 64}, respectively. However, it is quite complicated for UE preforming CJT CSI measurement on 4 CSI-RS resources than 2 CSI-RS resources. Therefore, to avoid the configuration of the base station exceeding UE capacity, a list of supported combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement should be introduced. In RAN1#113, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-1-1 for Rel-16-based CJT and FG 40-3-1-5 for Rel-17-based CJT are endorsed, where “A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously” and “FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement” are both FFS. Thus we have the following Proposal: Proposal 3-1: Introduce following components in FG 40-3-1-1, 40-3-1-1a, 40-3-1-5 and 40-3-1-5a: - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-1a/2 - Instead of integer/fraction frequency offset, which is not a terminology used in the specs description or RRC parameter description, we suggest using “oversampling factor”, which is the actual parameter value used in the specs description to configure mode1, i.e., O4=1 and O4=4, respectively 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with O3=1 basis selection integer frequency offset. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; - Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. - Support of parameter combinations with L=2,4 - Support of rank 1,2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode 1, i.e., common freqencyt basis selection among different TRPs with FD basis selection integer frequency offset; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1,2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-1a; Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-2 | Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1 | Prerequisite 40-3-1-1a |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-1a/2 - Instead of integer/fraction frequency offset, which is not a terminology used in the specs description or RRC parameter description, we suggest using “oversampling factor”, which is the actual parameter value used in the specs description to configure mode1, i.e., O4=1 and O4=4, respectively 40. NR_MIMO_evo_DL_UL; 40-3-1-2; Support of mode 1 for Rel-16-based CJT type-II codebook with O3=4 for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1; Prerequisite 40-3-1-1a. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-2; Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1; - Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-16 eType-II based CJT codebook; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-2; Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1; Prerequisite 40-3-1-1a. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-2; Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1; 40-3-1-1a; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-3 | Support R=2 for Rel-16-based CJT codebook | FFS: split for mode 1/2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-3 - component 1: triplets for active resources/ports across CCs, for R=2 - component 2: {#ports, max # resources} per resource set, for R=2 - FFS: no need to split between mode 1 and 2, to avoid over complication in the reporting of max number of resources/ports across CCs and per resource set 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2 2. {Max # of Tx ports in one resource, Max # of resources}, in one NZP CSI-RS resource set, with R=2; Component 1 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {8 to 256} Component 2 candidate values: a) {4,8,12,16,24,32} b) {2,3,4}. |
| ZTE [4] | - We do not identify the necessity of having separate rows for mode1/2 for supporting R=2 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; FFS: split for mode ½ Support R=2 for Rel-16-based CJT codebook; 40-3-1-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; For mode 1 (if supported) and mode 2.. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; 1. Supported mode for R=2 (candidate value: mode 1, mode 2 or both). |
| CATT [9] |  |
| Ericsson [10] | Regarding FFS on whether to “split [this row] for mode 1/2”. As UEs supporting mode 1/2 will have different pre-requisites, we support to split this row into two rows (one for mode 1 and another for mode 2). 40-3-1-3; Support R=2 for Rel-16-based CJT codebook for mode 1; FFS: split for mode 1/2. 40-3-1-3a; Support R=2 for Rel-16-based CJT codebook for mode 2. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; - Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1,2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; FFS: split for mode 1/2. |
| NTT DOCOMO, INC. [13] | 40-3-1-3/40-3-1-4/40-3-1-7/40-3-1-8 - Components: For each FG, support to split for mode1 and mode2. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-3; Support R=2 for Rel-16-based CJT codebook; {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support regular eType-II-CJT for R=2; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource set: {8,...,128} - Max # resource sets: {1 to 16} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-4 | Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook | FFS: split for mode 1/2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-4 - FFS: no need to split between mode 1 and mode 2 40. NR_MIMO_evo_DL_UL; 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; Support of parameter combinations with pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook. |
| ZTE [4] | - We do not identify the necessity of having separate rows for the case of ‘pv={1/2,1/2,1/2,1/2} and beta=1/2’. 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; FFS: split for mode ½ Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; 40-3-1-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; For mode 1 (if supported) and mode 2.. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; 1. Supported mode for pv={1/2,1/2,1/2,1/2} and beta=1/2 (candidate value: mode 1, mode 2 or both). |
| CATT [9] |  |
| Ericsson [10] | Regarding FFS on whether to “split [this row] for mode 1/2”. As UEs supporting mode 1/2 will have different pre-requisites, we support to split this row into two rows (one for mode 1 and another for mode 2). 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook for mode 1; FFS: split for mode 1/2. 40-3-1-4a; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook for mode 2. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; - Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination pv={1/2,1/2,1/2,1/2} and beta=1/2; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; FFS: split for mode 1/2. |
| NTT DOCOMO, INC. [13] | 40-3-1-3/40-3-1-4/40-3-1-7/40-3-1-8 - Components: For each FG, support to split for mode1 and mode2. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-4; Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-5 | Support of mode 2 for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only A-CSI only FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For CJT CSI codebook, similar to legacy Type I single panel codebook in FG 2-36, Type I multi panel codebook in FG 2-40, Type II codebook in FG 2-41 and etc., a list of supported capability combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC should be introduced. The gNB can configure multiple CSI measurement according to the above listed combinations. For example, if UE reports {16, 8, 128}, then gNB can configure CSI report #1 and CSI report #2 based on {16, 4, 64} and {16, 4, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, CSI report #3, and CSI report #4 based on {16, 2, 64}, {16, 2, 64}, {16, 2, 64} and {16, 2, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, and CSI report #3 based on {16, 2, 64}, {16, 2, 64}, {16, 4, 64}, respectively. However, it is quite complicated for UE preforming CJT CSI measurement on 4 CSI-RS resources than 2 CSI-RS resources. Therefore, to avoid the configuration of the base station exceeding UE capacity, a list of supported combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement should be introduced. In RAN1#113, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-1-1 for Rel-16-based CJT and FG 40-3-1-5 for Rel-17-based CJT are endorsed, where “A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously” and “FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement” are both FFS. Thus we have the following Proposal: Proposal 3-1: Introduce following components in FG 40-3-1-1, 40-3-1-1a, 40-3-1-5 and 40-3-1-5a: - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only A-CSI only FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-5 - Similar proposed description and basic components as 40-3-1-1, but for Rel-17-based CJT. 7 proposed components 1. Support of N=N_TRP only 2. Support of N_L=1 only 3. Support of codebook mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, with M=1 and R=1: a) Max # of Tx ports in one resource b) Max # of resources c) total # of Tx ports 5. A list of supported combinations in one NZP CSI-RS resource set, with M=1 and R=1: a) max # of Tx ports in one resource b) max # of resources 6. Support of parameter combinations with M=1 7. Support of rank 1,2 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 forBasic features of Rel-17-based CJT type-II codebook; 1. Support of N=N_TRP only 2. Support of N_L=1 only 3. Support of codebook mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, with M=1 and R=1: a) Max # of Tx ports in one resource b) Max # of resources c) total # of Tx ports 5. A list of supported combinations in one NZP CSI-RS resource set, with M=1 and R=1: a) Max # of Tx ports in one resource b) Max # of resources 6. Support of parameter combinations with M=1 7. Support of rank 1,2 Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG. |
| ZTE [4] | - After that, regarding the restriction of NTRP supported by a UE, it can be implied by ‘{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}’. 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; - Support of N=N_TRP only - Support of N_L=1 only - Support of A-CSI report only - FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously - FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement - Value of X for CPU occupation (OCPU = X.NTRP) Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only A-CSI only A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously (per band and per BC) Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI-ReportConfig Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. - Support of parameter combinations with M=1 - Support of rank 1,2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode 2, i.e., common freqencyt basis selection among different TRPs - Supported NL=1 list for spatial basis selection - Supported N=NTRP for multi-TRP selection; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {1, 2,3,4 … 64} c) {4, …, 256} d) {1,2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; Support of N=N_TRP only Support of N_L=1 only A-CSI only FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5; Support of mode 2 for Rel-17-based CJT type-II codebook; • Support of rank 1,2 • {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support regular eType-II-CJT for M=1 and R=1; 2-35; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-5a | Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For CJT CSI codebook, similar to legacy Type I single panel codebook in FG 2-36, Type I multi panel codebook in FG 2-40, Type II codebook in FG 2-41 and etc., a list of supported capability combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC should be introduced. The gNB can configure multiple CSI measurement according to the above listed combinations. For example, if UE reports {16, 8, 128}, then gNB can configure CSI report #1 and CSI report #2 based on {16, 4, 64} and {16, 4, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, CSI report #3, and CSI report #4 based on {16, 2, 64}, {16, 2, 64}, {16, 2, 64} and {16, 2, 64}, respectively; or the gNB can configure CSI report #1, CSI report #2, and CSI report #3 based on {16, 2, 64}, {16, 2, 64}, {16, 4, 64}, respectively. However, it is quite complicated for UE preforming CJT CSI measurement on 4 CSI-RS resources than 2 CSI-RS resources. Therefore, to avoid the configuration of the base station exceeding UE capacity, a list of supported combinations where each combination contains {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement should be introduced. In RAN1#113, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-1-1 for Rel-16-based CJT and FG 40-3-1-5 for Rel-17-based CJT are endorsed, where “A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously” and “FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement” are both FFS. Thus we have the following Proposal: Proposal 3-1: Introduce following components in FG 40-3-1-1, 40-3-1-1a, 40-3-1-5 and 40-3-1-5a: - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement 40. NR_MIMO_evo_DL_UL; 40-3-1-5a; Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset; A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement. |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5a; Support of mode 1 for Rel-17-based CJT type-II codebook with O3=1 FD basis selection integer frequency offset. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5a; Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset; - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. - Support of parameter combinations with M=1 - Support of rank 1,2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode 1, i.e., common freqencyt basis selection among different TRPs with FD basis selection integer frequency offset; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {1, 2,3,4 … 64} c) {4, …, 256} d) {1,2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5a; Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-5a; Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset; 40-3-1-5; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-6 | Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1 | Prerequisite 40-3-1-5a |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-1-6; Support of mode 1 for Rel-17-based CJT type-II codebook with O3=4for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1; Prerequisite 40-3-1-5a. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-6; Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1; - Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-17 FeType-II port selection based CJT codebook; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-6; Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1; Prerequisite 40-3-1-5a. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-6; Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1; 40-3-1-5a; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-7 | Support of M=2 and R=1 for Rel-17-based CJT codebook | FFS: split for mode ½ |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with M=2 and R=1 2. {Max # of Tx ports in one resource, Max # of resources}, in one NZP CSI-RS resource set, with M=2 and R=1 3. Support of parameter combinations with M=2. |
| ZTE [4] | 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; FFS: split for mode ½ Support of M=2 and R=1 for Rel-17-based CJT codebook; 40-3-1-5. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; For mode 1 (if supported) and mode 2.. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; 1. Supported mode for M=2 and R=2 (candidate value: mode 1, mode 2 or both). |
| CATT [9] |  |
| Ericsson [10] | Regarding FFS on whether to “split [this row] for mode 1/2”. As UEs supporting mode 1/2 will have different pre-requisites, we support to split this row into two rows (one for mode 1 and another for mode 2). 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook for mode 1; FFS: split for mode ½. 40-3-1-7a; Support of M=2 and R=1 for Rel-17-based CJT codebook for mode 2. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1,2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; FFS: split for mode ½. |
| NTT DOCOMO, INC. [13] | 40-3-1-3/40-3-1-4/40-3-1-7/40-3-1-8 - Components: For each FG, support to split for mode1 and mode2. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-7; Support of M=2 and R=1 for Rel-17-based CJT codebook; {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support regular eType-II-CJT for M=2 and R=1; 40-3-1-5; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-8 | Support R=2 for Rel-17-based CJT codebook | FFS: split for mode 1/2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2 2. {Max # of Tx ports in one resource, Max # of resources}, in one NZP CSI-RS resource set, with R=2. |
| ZTE [4] | 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; FFS: split for mode ½ Support R=2 for Rel-17-based CJT codebook; 40-3-1-5. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; For mode 1 (if supported) and mode 2.. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; 1. Supported mode for R=2 (candidate value: mode 1, mode 2 or both). |
| CATT [9] |  |
| Ericsson [10] | Regarding FFS on whether to “split [this row] for mode 1/2”. As UEs supporting mode 1/2 will have different pre-requisites, we support to split this row into two rows (one for mode 1 and another for mode 2). 40-3-1-8; Support R=2 for Rel-17-based CJT codebook for mode 1; FFS: split for mode 1/2. 40-3-1-8a; Support R=2 for Rel-17-based CJT codebook for mode 2. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1,2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; FFS: split for mode 1/2. |
| NTT DOCOMO, INC. [13] | 40-3-1-3/40-3-1-4/40-3-1-7/40-3-1-8 - Components: For each FG, support to split for mode1 and mode2. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-8; Support R=2 for Rel-17-based CJT codebook; {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support regular eType-II-CJT for R=2; 40-3-1-5; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-9 | Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2] | FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-9 - No need for separate FG, this can be included in components 4 and 5 of 40-3-1-1 (Rel16-based CJT) and 40-3-1-5 (Rel17-based CJT) 40. NR_MIMO_evo_DL_UL; 40-3-1-9; Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; No need for separate FG, this can be included in components 4 and 5 of 40-3-1-1 (Rel16-based CJT) and 40-3-1-5 (Rel17-based CJT). |
| ZTE [4] | 40-3-1-9; Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Maximum value of 2NN1N2 >32 supported; 40-3-1-1, 40-3-1-5. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-9; Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-9; Support for 2NN1N2 >32 for Rel-16-based CJT type-II codebook Support for 2NN1N2 >32 for Rel-17-based CJT type-II codebook; A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously (per band and per BC). |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-1-9; Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI-ReportConfig. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-9; Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; - Maximum number of ports across all TRPs in the same CSI report setting for multi-TRP CJT based on Rel-16 eType-II codenbook - Maximum number of ports across all TRPs in the same CSI report setting for multi-TRP CJT based on Rel-17 FeType-II port selection codenbook; Component 1 candidate values: {32, 64, 96, 128} Component 2 candidate values: {32, 64, 96, 128}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-9; Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement. |
| NTT DOCOMO, INC. [13] | 40-3-1-9 - Support this FG to be conditioned upon support for 40-3-1-1 and/or 40-3-1-2. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-9; Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; 40-3-1-1, or 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-10 | Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-10 - This can be removed, see 40-3-1-15/16 40. NR_MIMO_evo_DL_UL; 40-3-1-10; Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; This can be removed, see 40-3-1-15/16. |
| ZTE [4] | 40-3-1-10; Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; Additional support for dynamic selection of N<N_TRP; 40-3-1-1, 40-3-1-5. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-1-10; Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-1-10; Additional support for dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook. Additional support for dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook.. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-1-10; Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-10; Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-10; Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]. |
| NTT DOCOMO, INC. [13] | 40-3-1-10 - Support to delete FG 40-3-1-10. - Reason: Dynamic selection of N<N_TRP is supported for both Rel-16-based CJT CSI and Rel-17-based CJT CSI. There are FGs 40-3-1-15 and 40-3-1-16 defined for such features. Thus, FG 40-3-1-10 could be deleted. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-10; Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2]; 40-3-1-1, or 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-11 | Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot | analogous to 23-9-5 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination analogous to 23-9-5. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot; analogous to 23-9-5. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 40-3-1-1,40-3-1-5,2-36,2-40; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-12 | Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-12; Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook; - Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with rank 3,4; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-12; Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-11; Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-13 | Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-13; Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook; - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with rank 3,4; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-13; Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-12; Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook; 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-14 | Support of Support of L=6 for Rel-16-based CJT type-II codebook | Only for N_TRP=1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-1-14; Support of Support of L1=6 for Rel-16-based CJT type-II codebook; Support of parameter combinations with L1=6 for Rel-16-based CJT codebook Only for N_TRP=1. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-14; Support of Support of L=6 for Rel-16-based CJT type-II codebook; - Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination with L=6.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-14; Support of Support of L=6 for Rel-16-based CJT type-II codebook; Only for N_TRP=1. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-13; Support of Support of L=6 for Rel-16-based CJT type-II codebook; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-15 | dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook | Update 40-3-1-1 with “support of N=N_TRP only” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG supports dynamic selection of N<N_TRP in addition to the features supported in FG 40-3-1-1. Hence, FG 40-3-1-1 shall be the pre-requisite for this FG. 40-3-1-15; dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook; Update 40-3-1-1 with “support of N=N_TRP only” Pre-requisite FG 40-3-1-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-15; dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook; - Support of slection of N < N_TRP CSI-RS resource by UE for multi-TRP CJT based on Rel-16 eType-II codenbook; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-15; dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook; Update 40-3-1-1 with “support of N=N_TRP only”. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-14; dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-16 | dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook | Update 40-3-1-5 with “support of N=N_TRP only” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG supports dynamic selection of N<N_TRP in addition to the features supported in FG 40-3-1-5. Hence, FG 40-3-1-5 shall be the pre-requisite for this FG. 40-3-1-16; dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook; Update 40-3-1-5 with “support of N=N_TRP only” Pre-requisite FG 40-3-1-5. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-16; dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook; - Support of slection of N < N_TRP CSI-RS resource by UE for multi-TRP CJT based on Rel-17 FeType-II port selection codenbook; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-16; dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook; Update 40-3-1-5 with “support of N=N_TRP only”. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-15; dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook; 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-17 | Number of N_L>1 combinations across for Rel-16-based CJT type-II codebook | Update 40-3-1-1 with “support of N_L=1 only” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG supports N_L > 1 combinations in addition to the features supported in FG 40-3-1-1. Hence, FG 40-3-1-1 shall be the pre-requisite for this FG. 40-3-1-17; Number of N_L>1 combinations across for Rel-16-based CJT type-II codebook; Update 40-3-1-1 with “support of N_L=1 only” Pre-requisite FG 40-3-1-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-17; Number of N_L>1 combinations across for Rel-16-based CJT type-II codebook; - Maximum number of lists for spatial basis selection, i.e., NL, for multi-TRP CJT based on Rel-16 eType-II codenbook; Component 1 candidate values: {2, 4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-17; Number of N_L>1 combinations across for Rel-16-based CJT type-II codebook; Update 40-3-1-1 with “support of N_L=1 only”. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-16; Number of N_L>1 combinations across for Rel-16-based CJT type-II codebook; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: N_L = {2,[3],4}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-18 | Number of N_L>1 combinations across for Rel-17-based CJT type-II codebook | Update 40-3-1-5 with “support of N_L=1 only” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG supports N_L > 1 combinations in addition to the features supported in FG 40-3-1-5. Hence, FG 40-3-1-5 shall be the pre-requisite for this FG. 40-3-1-18; Number of N_L>1 combinations across for Rel-17-based CJT type-II codebook; Update 40-3-1-5 with “support of N_L=1 only” Pre-requisite FG 40-3-1-5. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-18; Number of N_L>1 combinations across for Rel-17-based CJT type-II codebook; - Maximum number of lists for spatial basis selection, i.e., NL, for multi-TRP CJT based on Rel-17 FeType-II port selection codenbook; Component 1 candidate values: {2, 4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-18; Number of N_L>1 combinations across for Rel-17-based CJT type-II codebook; Update 40-3-1-5 with “support of N_L=1 only”. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-17; Number of N_L>1 combinations across for Rel-17-based CJT type-II codebook; 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: N_L = {2,[3],4}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-19 | Support of SP CSI for Rel-16-based CJT type-II codebook | Update 40-3-1-1 with “A-CSI only” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG supports SP CSI feedback in addition to the features supported in FG 40-3-1-1. Hence, FG 40-3-1-1 shall be the pre-requisite for this FG. 40-3-1-19; Support of SP CSI for Rel-16-based CJT type-II codebook; Update 40-3-1-1 with “A-CSI only” Pre-requisite FG 40-3-1-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-19; Support of SP CSI for Rel-16-based CJT type-II codebook; - Support of semi-persitent CSI report for Rel-16 eType-II codebook refinement for multi-TRP CJT; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-19; Support of SP CSI for Rel-16-based CJT type-II codebook; Update 40-3-1-1 with “A-CSI only”. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-18; Support of SP CSI for Rel-16-based CJT type-II codebook; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-20 | Support of SP CSI for Rel-17-based CJT type-II codebook | Update 40-3-1-5 with “A-CSI only” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG supports SP CSI feedback in addition to the features supported in FG 40-3-1-5. Hence, FG 40-3-1-5 shall be the pre-requisite for this FG. 40-3-1-20; Support of SP CSI for Rel-17-based CJT type-II codebook; Update 40-3-1-5 with “A-CSI only” Pre-requisite FG 40-3-1-5. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-20; Support of SP CSI for Rel-17-based CJT type-II codebook; - Support of semi-persitent CSI report for Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-20; Support of SP CSI for Rel-17-based CJT type-II codebook; Update 40-3-1-5 with “A-CSI only”. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-19; Support of SP CSI for Rel-17-based CJT type-II codebook; 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-21 | Active CSI-RS resources and ports in the presence of mixed codebooks | FFS: which codebooks |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - Regarding ‘Active CSI-RS resources and ports in the presence of mixed codebooks’, the existing codebook combination should be considered fully. 40-3-1-21; Active CSI-RS resources and ports in the presence of mixed codebooks; FFS: which codebooks Note: The existing CSI codebook (TypeI (Rel-15, Rel-17 for NCJT)/Type-II/eTypeII/FD-TypeII) should be considered; 40-3-1-1, 40-3-1-5. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-21; Active CSI-RS resources and ports in the presence of mixed codebooks; - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-21; Active CSI-RS resources and ports in the presence of mixed codebooks; FFS: which codebooks. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-20; Active CSI-RS resources and ports in the presence of mixed codebooks; (Dupulicated as 40-3-1-11). |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-22 | Max supported offset between CSI-RS (from different TRPs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-22 - No need for this FG. The slot offset X∈{1,2} between the K resources of a resource set is the same as for NCJT CSI in Rel-17, for which we did not have a UE capability Agreement (RAN1#112bis-e) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding CSI calculation and measurement, - For the configured NTRP CSI-RS resources comprising the CMR, the restriction specified for Rel-17 NCJT CSI is fully reused, i.e. the configured NTRP CSI-RS resources are located either in the same slot or two consecutive slots. |
| ZTE [4] | - Finally, regarding FG 40-3-1-22, based on the current agreement, the UE assumes that there are the same CSI-RS ports for different TRPs under CJT CSI measurement. Therefore, we suggest to remove the FG once one clear motivation is identified. 40-3-1-22; Max supported offset between CSI-RS (from different TRPs). 40-3-1-22; Z/Z’ for CJT type-II codebook; For NTRP>1, support one of capabilities for Z/Z’ for CJT type-II codebook: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: For NTRP=1: reuse legacy Z/Z’ values; 40-3-1-1, 40-3-1-5. 40-3-1-22a; r for determining Z/Z’ for CJT type-II codebook once capability 2 is indicated.; The value of r for determining Z/Z’ for CJT type-II codebook once capability 2 is indicated.; 40-3-1-22. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-22; Max supported offset between CSI-RS (from different TRPs); - The maximum supported offset between different CSI-RS for multi-TRP CJT CSI; Component 1 candidate values: {1, 2, 3, 4} slots; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-22; Max supported offset between CSI-RS (from different TRPs). |
| NTT DOCOMO, INC. [13] | 40-3-1-22 - Support to delete FG 40-3-1-22. - Reason: Only two values of slot offsets are supported for up to 4 CSI-RS resources. There is no need to introduce additional UE capability on the supported slot offset value. Thus, FG 40-3-1-22 could be deleted. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-21; Max supported offset between CSI-RS (from different TRPs); 40-3-1-1, or 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-1-23 | Unequal number of spatial basis selection for multi-TRP CJT |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-1-23 - Support of parameter combinations with unequal values of L1,…,LN_TRP for Rel-16-based CJT codebook. Only for N_L =1 40. NR_MIMO_evo_DL_UL; 40-3-1-23; Unequal number of spatial basis selection for Rel-16-based CJT codebook multi-TRP CJT; Support of parameter combinations with unequal values of L1,…,LN_TRP for Rel-16-based CJT codebook Only for N_L =1. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-1-23; Unequal number of spatial basis selection for multi-TRP CJT; - Support of unequal number of spatial basis selection for multi-TRP CJT including Rel-16 eType-II codebook and Rel-17 FeType-II port seletion codebook refinement; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-1-23; Unequal number of spatial basis selection for multi-TRP CJT. |
| NTT DOCOMO, INC. [13] | 40-3-1-23 - Support to delete FG 40-3-1-23. - Reason: The configuration of N_L>1 SD basis combinations has been defined in FG 40-3-1-17/40-3-1-18, there is no need to further introduce a separate UE capability on unequal number of SD basis. Thus, FG 40-3-1-23 could be deleted. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-1-22; Unequal number of spatial basis selection for multi-TRP CJT; (Question: Duplicated as 40-3-1-16 and 40-3-1-17?). |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-1 | Support of Rel-16-based doppler measurement | - Support X=1 CQI based on one slot - Support R=1 - Support parameter combinations with L=2,4 - Support for rank = 1,2 FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In RAN1#113, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-2 is endorsed for Rel-16-based doppler codebook and Rel-17-based doppler codebook. And whether to support a list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement is FFS. In our view, for doppler CSI codebook, multiple CSI-RS occasions measurement, UE-side prediction and PMI calculation in three dimensions would be performed, which increase calculation complexity and the requirement of UE memory. Obviously, the value of N4 affects the calculation complexity of the UE. Therefore, the configuration of supported combinations should be considered together with the value of N4. Therefore, we propose: Proposal 3-2: Add the following components into FG 40-3-2-1, 40-30-2-4: - {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC. 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler measurement; - Support X=1 CQI based on one slot - Support R=1 - Support parameter combinations with L=2,4 - Support for rank = 1,2 - FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-2-1 - Measurement support for UE-side CSI prediction. We suggest separating the measurement support for UE-side prediction from the basic features of Rel-16-based Doppler codebook and Rel-17-based Doppler codebook 1. Number, N4, of consecutive slot intervals associated to the predicted precoder matrices 2. {Max # of Tx ports in one resource, Max # of resources}, in one AP- CSI-RS resource set 3. Separation m=1,2 slots separation between 2 consecutive AP-CSI-RS resources 4. Support of time unit d=1,m slots for AP-CSI-RS resources and d=periodicity for P- and SP- CSI-RS resources 5. First slot l = n + delta, with delta=0,1,2, and where n=slot associated with CSI report - For FG 40-3-2-1a - Basic features of Rel-16-based Doppler codebook. Prerequisite 40-3-2-1 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=1 2. Support of parameter combinations 1-6 3. Support of rank 1-2 4. Support X=1 CQI based on one slot (TDCQI=’1-1’) 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Measurement support for UE-side CSI prediction; 1. Number, N4, of consecutive slot intervals associated to the predicted precoder matrices 2. {Max # of Tx ports in one resource, Max # of resources}, in one AP- CSI-RS resource set 3. Separation m=1,2 slots separation between 2 consecutive AP-CSI-RS resources 4. Support of time unit d=1,m slots for AP-CSI-RS resources and d=periodicity for P- and SP- CSI-RS resources 5. First slot l = n + delta, with delta=0,1,2, and where n=slot associated with CSI report; Component 1 candidate values: {1,2,4,8} Component 2 candidate values: {4,8,12,16,24,32} {4,8,12}. 40. NR_MIMO_evo_DL_UL; 40-3-2-1a; Support of Basic features of Rel-16-based Doppler measurementcodebook; 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=1 2. Support of parameter combinations 1-6 3. Support of rank 1-2 4. Support X=1 CQI based on one slot (TDCQI=’1-1’) Prerequisite 40-3-2-1. |
| ZTE [4] | - As for CSI triplet, we are fine to use the legacy one, but having additional max# of time unit does not make sense and may introduce the backward compatibility issues while mixing with other legacy CSI codebook. 40-3-2-1; Support of Rel-16-based doppler measurement; - Support X=1 CQI based on one slot - Support R=1 - Support parameter combinations with L=2,4 - Support for rank = 1,2 - Support for the size of DD-basis, N4=1 - Value of Y for CPU occupation (OCPU = YN4), when P/SP-CSI-RS is configured for CMR - Value of Y for CPU occupation (OCPU = Y.K), when P/SP-CSI-RS is configured for CMR - Note: When N4=1, OCPU =4 - Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR - Value of KP for P/SP-CSI-RS active resource counting FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. - Regarding UE feature/capability to support only a subset of Parameter Combinations, we think that, for L=6, the same restriction and UE optionality as legacy apply. Similarly, we have additional UE feature for RANK =3, 4. 40-3-2-10; Support of L=6 for Rel-16-based doppler codebook; Support of L=6 for Rel-16-based doppler codebook; 40-3-2-1. 40-3-2-11; Support of RANK =3 and 4 for Rel-16-based doppler codebook; Support of RANK =3 and 4 for Rel-16-based doppler codebook; 40-3-2-1. - Regarding N4>1, we have an individual UE feature of supporting the maximum number of N4 (with candidate value of {1, 2, 4, 8}), and then the feature of N4=1 should be assumed as a basic feature. 40-3-2-9; Additional support of the size of DD-basis; Besides for N4=1, additional support of a list of values of N4 - Note: the candidate value for N4 is {2, 4, 8}; 40-3-2-1,. - Regarding K (the number of AP-CSI-RS resources for the CMR), we have an individual UE feature of supporting K=12. 40-3-2-8; Support of the number of AP-CSI-RS resources for the CMR, i.e., K, equal to 12; For the number of AP-CSI-RS resources for the CMR, i.e., K, we have an individual UE feature of supporting K=12.; 40-3-2-1, 40-3-2-4. - Regarding the start of CSI window is l=n+, the maximum number of  is supported as an individual UE feature and can be supported with candidate values of {0, 1, 2}. 40-3-2-7a; Support of for determining the start of CSI window of l=n+; Regarding the start of CSI window is l=n+, the maximum number of  is supported. - Note: the candidate values for  are {0, 1, 2}.; 40-3-2-7. - Finally, for Z/Z’, we may introduce the corresponding UE feature after the corresponding solution is stable. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler measurement; - Support X=1 CQI based on one slot - Support R=1 - Support parameter combinations with L=2,4 - Support for rank = 1,2 - Support N4=1 - Support d=CSI-RS periodicity for P/SP CSI-RS or d=m for AP CSI-RS - Support l = slot (n+δ) where δ ≥ 0 A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler measurement; - Support X=1 CQI based on one slot - Support R=1 - Support parameter combinations with L=2,4 - Support for rank = 1,2 FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously (per band and per BC). |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler measurement; - Support X=1 CQI based on one slot - Support R=1 - Support parameter combinations with L=2,4 - Support for rank = 1,2 - 1. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 2. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} per CSI-ReportConfig. |
| CATT [9] | 40-3-2-1: - According to the following agreement, one separate FG corresponding to parameter combinations with L=6 should be introduced for Rel-16-based doppler measurement. Agreement For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, at least the following Parameter Combinations are supported 4; 1/4; 1/4; 1/4. 4; 1/4; 1/4; 1/2. 4 (*); 1/2; 1/4; 1/2. 4 (*); 1/4; 1/4; 3/4. 6 (*); 1/4; --; 1/2. 6 (*); 1/4; --; 3/4. (*) Note: From legacy. For L=6, the same restriction and UE optionality as legacy apply. Proposal 29: For Rel-16-based doppler measurement, one additional FG, shown as follows, should be added. 40. NR_MIMO_evo_DL_UL; 40-3-2-x; Support parameter combinations with L=6. - According to the following agreement, one separate FG corresponding to the parameter K=12 should be introduced for Rel-16-based doppler measurement. Agreement For the Type-II codebook refinement for high/medium velocities, the parameter K (the number of AP-CSI-RS resources for the CMR) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {4, 8} - FFS: If additional candidate value(s) of K are supported, e.g. 5, 12, 16, also taking into account other use cases (e.g. for training filter coefficients, prediction or performance monitoring) and TDD Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR), optionally support only K=12 as an additional candidate value. Proposal 30: For Rel-16-based doppler measurement, one additional FG, shown as follows, should be added. 40. NR_MIMO_evo_DL_UL; 40-3-2-x; Support K=12 for Rel-16-based doppler codebook. Proposal 31: For Rel-16-based doppler measurement, 40-3-2-1 is modified as follows. 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler measurement; - Support X=1 CQI based on one slot - Support R=1 - Support parameter combinations with L=2,4 - Support for rank = 1,2 - Support for K=4,8 - FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. - According to the following agreement, rank=1,2,3,4 is supported. Same to Rel-16 and Rel-17 Type II codebook, rank=1,2 is basic feature and rank=3,4 is optional subject to UE capability. Therefore, one separate FG corresponding to rank=3,4 should be introduced for Rel-16-based doppler measurement. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support RI={1,2,3,4}.. Proposal 32: For Rel-16-based doppler measurement, one additional FG, shown as follows, should be added. 40. NR_MIMO_evo_DL_UL; 40-3-2-x; Support rank = 3,4 for Rel-16-based doppler codebook. - According to the following agreement, rank=1,2,3,4 is supported. Same to Rel-16 and Rel-17 Type II codebook, rank=1,2 is basic feature and rank=3,4 is optional subject to UE capability. Therefore, two separate FG corresponding to rank=3,4 should be introduced for CJT Type II codebook. Agreement For the Rel-18 Type-II codebook for CJT mTRP, support RI={1,2,3,4}.. Proposal 33: For CJT Type II codebook, two additional FG, shown as follows, should be added. 40. NR_MIMO_evo_DL_UL; 40-3-1-x; Support rank = 3,4 for Rel-16-based CJT type-II codebook. 40. NR_MIMO_evo_DL_UL; 40-3-1-x; Support rank = 3,4 for Rel-17-based CJT type-II codebook. |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based predicted PMI codebok; - Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1. - Support of parameter combinations with L=2,4 - Support of rank 1,2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources - Number of predicted CSI (N4) - Maximum numner of aperiodic CSI-RS resoruces that can be configured in the same CSI report setting - Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1, 2, 4, 8} Component 5 candidate values: {4, 8, 12}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler measurement; 1. Support X=1 CQI based on one slot 2. Support R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Rel-16-based doppler; 2-35; Yes; Rel-16 based Type II doppler codebook is not supported; per band per BC; n/a; n/a; n/a; Candidate values for component 5: - Max# of time unit: - {1,2,4,8,16} for aperoidc CSI-RS - {4 to 8T} for periodic or semi-persisten CSI-RS, where T is period of CSI-RS. - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-3-2-1 - Adding a component on Y value for CPU occupation capability, reflecting the following agreement in May Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 [+4] when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused. - Adding a component on r value for Z/Z’ capability, reflecting the following agreement in May. We are open to either 1) a component in this FG can report Capability 1 or Capability 2, or 2) a component in FG just reports Capability2, and defining new FG for the support of Capability#1 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’. - Adding a component on KP value for P/SP-CSI-RS active resource counting among {1, 2, 4}, reflecting the following agreement Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, the value of KP for P/SP-CSI-RS active resource counting is determined based on UE capability, where the candidate values are {1, 2, 4}.. |
| OPPO [14] |  |
| Samsung [15] | FG 40-3-2-1 and FG 40-3-2-4: for consistency across different FGs, it is preferred to use the one wording ‘Doppler measurement’ or ‘Doppler codebook’. Our preference is to use the latter since this enhancement is mainly about Doppler codebook. - Replace ‘Doppler measurement’ with ‘Doppler codebook’ |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-2-1; Support of Rel-16-based doppler measurement; Basic components: • {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} to support regular • Support X=1 CQI based on one slot • Support R=1 • Support parameter combinations with L=2,4 • Support for rank = 1,2; 2-35; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-2 | Support R=2 for Rel-16-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based Doppler codebook; {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based predicted PMI codebook; - Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subbands R=2. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources - Number of predicted CSI (N4); Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1, 2, 4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based doppler codebook; Max #of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support R=2 for Rel-16-based doppler codebook; 40-3-2-1; Yes; per band per BC; n/a; n/a; n/a; Candidate values for component 1: - Max# of time unit: - {1,2,4,8,16} for aperoidc CSI-RS - {4 to 8T} for periodic or semi-persisten CSI-RS, where T is period of CSI-RS. - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-2-2; Support R=2 for Rel-16-based doppler codebook; {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} to support regular eType-II-Doppler for R=2; 40-3-2-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-3 | Support X=1 based on averaging first and last slot of WCSI, for Rel-16-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-2-3; Support X=1 based on averaging first and last slot of WCSI, for Rel-16-based Doppler codebook; Support of TDCQI = ‘1-2’. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-2-3; Support X=1 based on averaging first and last slot of WCSI, for Rel-16-based predicted PMI codebook; - Support X=1 CQI associated with the first and the last slot of the CSI reporting window for Rel-16-based predicted PMI codebook; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-2-3; Support X=1 based on averaging first and last slot of WCSI, for Rel-16-based doppler codebook; Support X=1 CQI based on the first slot of WCSI and the assocated first of N4 PMI, and last slot of WCSI and the assocated last of N4 precoding matrixes, where N4 is the number of precoding matrixes.; 40-3-2-1; Yes; per band per BC; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-2-3; Support X=1 based on averaging first and last slot of WCSI , for Rel-16-based doppler codebook; 40-3-2-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-3 | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-2-3; Support X=2 CQI based on 2 slots for Rel-16-based Doppler codebook; Support of TDCQI = ‘2’. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-2-3a; Support X=2 CQI based on 2 slots for Rel-16-based predicted PMI codebook; - Support X=2 CQI for Rel-16-based predicted PMI codebook - The 1st CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI, and - The 2nd CQI is associated with the middle slot of the CSI reporting window and the predicted PMI in the middle; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-2-3; Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook; - Support the first CQI based on the first slot of WCSI and the assocated first of N4 precoding matrixes. Support the first CQI based on the middle slot of WCSI and the assocated N4/2 precoding matrixes.; 40-3-2-1; Yes; per band per BC; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | FG 40-3-2-3: two rows can be merged into one, and it needs to be clarified that X is the number of time-domain CQIs - Support X=1 time-domain CQI based on averaging first and last slot of WCSI, for Rel-16-based doppler codebook - Support X=2 time-domain CQIs based on 2 slots for Rel-16-based doppler codebook |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-2-3; Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook; 40-3-2-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-4 | Support of Rel-17-based doppler measurement | - Support of M = 1 - Support for rank = 1,2 FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In RAN1#113, FGs for CJT CSI enhancement in [1] were agreed to be adopted. FG 40-3-2 is endorsed for Rel-16-based doppler codebook and Rel-17-based doppler codebook. And whether to support a list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement is FFS. In our view, for doppler CSI codebook, multiple CSI-RS occasions measurement, UE-side prediction and PMI calculation in three dimensions would be performed, which increase calculation complexity and the requirement of UE memory. Obviously, the value of N4 affects the calculation complexity of the UE. Therefore, the configuration of supported combinations should be considered together with the value of N4. Therefore, we propose: Proposal 3-2: Add the following components into FG 40-3-2-1, 40-30-2-4: - {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CC. 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; - Support of M = 1 - Support for rank = 1,2 - FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-2-4 - Basic features of Rel-17-based Doppler codebook. Prerequisite 40-3-2-1 with N4=1 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously with M=1 and R=1 2. Support of parameter combinations with M=1 3. Support for rank = 1,2 40. NR_MIMO_evo_DL_UL; 40-3-2-4; SupportBasic features of Rel-17-based Doppler codebook measurement; 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously with M=1 and R=1 2. Support of parameter combinations with M=1 3. Support for rank = 1,2 Prerequisite 40-3-2-1 with N4=1. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; - Support of M = 1 - Support for rank = 1,2 A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; - Support of M = 1 - Support for rank = 1,2 FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement (per band and per BC). |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; - Support of M = 1 - Support for rank = 1,2 - 1. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 2. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} per CSI-ReportConfig. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based predicted PMI codebook; - Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1. - Support of parameter combinations with M=1 - Support of rank 1,2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources - Maximum numner of aperiodic CSI-RS resoruces that can be configured in the same CSI report setting - Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI; Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {1, 2,3,4 … 64} c) {4, …, 256} Component 5 candidate values: {4, 8, 12}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; - Support of M = 1 - Support for rank = 1,2 {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement; 2-35; Yes; Rel-17 based Type II doppler codebook is not supported; per band per BC; n/a; n/a; n/a; Candidate values for component 3: - Max# of time unit: - {1,2} for aperoidc CSI-RS - {T} for periodic or semi-persisten CSI-RS, where T is period of CSI-RS. - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | FG 40-3-2-1 and FG 40-3-2-4: for consistency across different FGs, it is preferred to use the one wording ‘Doppler measurement’ or ‘Doppler codebook’. Our preference is to use the latter since this enhancement is mainly about Doppler codebook. - Replace ‘Doppler measurement’ with ‘Doppler codebook’ |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-2-4; Support of Rel-17-based doppler measurement; Basic components: • Support of M = 1 and R=1 • Support for rank = 1,2 • Support for N4=1; 2-35; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-5 | Support of M=2 and R=1 for Rel-17-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-2-5; Support of M=2 and R=1 for Rel-17-based predicted PMI codebook; - Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with M=2 and PMI subband R=1. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-2-5; Support of M=2 and R=1 for Rel-17-based doppler codebook; Support of M=2 and R=1; 40-3-2-4; Yes; per band per BC; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-2-5; Support of M=2 and R=1 for Rel-17-based doppler codebook; {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support Port selection FeType-II-Doppler for M=2 and R=1; 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-6 | Support R=2 for Rel-17-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-2-6; Support R=2 for Rel-17-based predicted PMI codebook; - Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subbands R=2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources; Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-2-6; Support R=2 for Rel-17-based doppler codebook; {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support R=2 for Rel-17-based doppler codebook; 40-3-2-4; Yes; per band per BC; n/a; n/a; n/a; Candidate values for component 1: - Max# of time unit: - {1,2} for aperoidc CSI-RS - {T} for periodic or semi-persisten CSI-RS, where T is period of CSI-RS. - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-2-6; Support R=2 for Rel-17-based doppler codebook; 1. Support of R = 2 for FeType-II-Doppler 2.{Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support Port selection FeType-II-Doppler for M=2 and R=2; 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-2-7 | support of l = (n – nCSI,ref ) for CSI reference slot |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-2-7; Support of first slot l = (n – nCSI,ref ) for CSI reference slot. |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-2-7; support of l = (n – nCSI,ref ) for CSI reference slot; - Support of CSI reference resource as time domain location, i.e., l = (n – nCSI,ref ), for predicted PMI based on Rel-16 eType-II codebook and predicted PMI based on Rel-17 FeType-II port seletion codebook; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-2-7; support of l = (n – nCSI,ref ) for CSI reference slot; - Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 Support of l = (n – nCSI,ref ) for CSI reference slot when N4>1; 40-3-2-4; Yes; per band per BC; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-2-7; support of l = (n – nCSI,ref ) for CSI reference slot; 40-3-2-1, or 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; n is the slot of report PUSCH; nCSI,ref is according to Section 5.2.2.5 of 38.214; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-1 | TDCP (Time Domain Channel Properties) report | 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - Regarding phase report and value of Y, we prefer to have to separate UE features, i.e., one FG of indicating whether the UE supports phase report, and another FG of indicating the maximum number of Y (with candidate value of [1, 2, 3, 4]). - As mentioned in the agreement, Y=1 with delay ≤ Dbasic symbols, only wideband quantized normalized amplitude is reported as a basic feature. 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X). |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. The maximum number of active resources Agreement For the Rel-18 TRS-based TDCP reporting, when Y delay(s) are configured - OCPU=(Y+1).X where X≥1 is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X can depend on the value of D, and whether phase reporting is switched ON - Reuse legacy Z2/Z2’ values - To count active resources used for TDCP reporting, reuse the legacy number counting mechanism for CSI-RS resources UE reports the maximum number of active resources for TDCP in UE capability reporting.. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; - Support of TDCP (Time Domain Channel Properties) report based on TRS measurement - Support of Y=1 delay value for TDCP report - Support basic delay value, D_basic = 1 slot - Support of amplitude report; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report; 2-35; Yes; gNB does not know TDCP measured by TRS; per band per BC; n/a; n/a; n/a; For component 3, 4 bis is used to quantization amplitude; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1; TDCP (Time Domain Channel Properties) report; 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report; 2-35; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-2 | Number of delay values | Number Y>1 of delay values |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | 40-3-3-2; Number of delay values to be reported; Number Y>1 of delay values Note: candidate value is {1, 2, 3, 4}; 40-3-3-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; - For TDCP report, maximum number of delay value, i.e., Y, that can be configured; Component 1 candidate values: {2,3,4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; Number Y>1 of delay values; 40-3-3-1; Yes; per band per BC; n/a; n/a; n/a; The number Y>1 of delay values is 10, and the delay value D={4 symbols, 1 slot, 2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots when SCS >=30kHz }; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | FG 40-3-3-2: according to the RAN1 agreements, the following need to be included as candidate values for Y. - Y = {2,3,4} - Y=1 and delay > D_basic = 1 slot |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; Number Y>1 of delay values; 40-3-3-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values: {2,3,4}; Optional with capability signaling. |
| MediaTek Inc. [17] | The Number of delay values with phase reporting: The “Number of delay values” and “Phase report” are two other FGs agreed for TDCP. The current agreement allows the UE to signal capability for phase reporting for all TDCP coefficients, or for none of them. We propose allowing more flexibility in the capability report, where UEs can signal the number of delay values with phase reporting switched on in a separate FG. Proposal 11: Add a new component for the FG “Number of delay values” (FGI 40-3-3-2) to indicate the number of supported delay values with phase reporting switched ON: 40. NR_MIMO_evo_DL_UL; 40-3-3-2; Number of delay values; - Number Y>1 of delay values - Number Y≥1 of delay values with phase reporting switched on; Optional with UE capability. |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-1a | delay values larger than D_basic | Note: candidate component values cover both range 1 values and range 2 values |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - Regarding maximum value of supported D, we may have two individual FGs of: FG-1 {4 symbols, 1 slot, 2 slots, 3 slots, 4 slots, 5 slots}; FG-2 additional D value of {6 slots or 10 slots} 40-3-3-1a; delay values larger than D_basic; Note: candidate component values cover both range 1 values and range 2 values Support of delay value of {range-1, range-2, or both} besides for D_basic =1 slot - Range-1 value is {2 slots, 3 slots, 4 slots, 5 slots}; - Range-2 value is {6 slots, 10 slots}; 40-3-3-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | The candidate values for this FG should include 2 slots, 3 slots, 4 slots, 5 slots, 6 slots, and 10 slots. 40-3-3-1a; delay values larger than D_basic; Note: candidate component values cover both range 1 values and range 2 values Candidate component values: { 2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots}. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1a; delay values larger than D_basic; - For TDCP report, maximum delay value that can be configured; Component 1 candidate values: {2,3,4,5,6,10} slots Note: 10 slots can only be configured for SCS >= 30kHz; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1a; delay values larger than D_basic; Note: candidate component values cover both range 1 values and range 2 values; 40-3-3-1; Yes; per band per BC; n/a; n/a; n/a; The delay value D={2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots when SCS >=30kHz }; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-3-1a; delay values larger than D_basic; Note: candidate component values cover both range 1 values and range 2 values; 40-3-3-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values: {2,3,4,5,6,10} slots; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-4 | Phase report | Support of phase report |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; Support of phase report for Y>1. |
| ZTE [4] | 40-3-3-4; Phase report; Support of phase report; 40-3-3-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; - For TDCP report, support of phase report; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; Support of phase report; 40-3-3-1; Yes; per band per BC; n/a; n/a; n/a; 4 bis is used to quantization phase; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | FG 40-3-3-4: according to the RAN1 agreement, the phase reporting applies to the case when Y>1. This condition needs to be added. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-3-4; Phase report; Support of phase report; 40-3-3-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-5 | Number of CSI-RS resources for TDCP | FFS: Per-CC, across all CCs, and, simultaneous across all CCs (analogous to 2-33 or 16-1g) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-3-5 - This should indicate the maximum number of TRS resource sets simultaneously active for TDCP calculation, across all CCs. This capability should be per band and per BC. 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of TRS resource sets CSI-RS resources for TDCP; Max # of TRS resource sets across all CCs simultaneously (per band and per BC) FFS: Per-CC, across all CCs, and, simultaneous across all CCs (analogous to 2-33 or 16-1g). |
| ZTE [4] | - Then, regarding the number of CSI-RS resources for TDCP, we suggest to use the similar rule as 2-33 or 16-1g, i.e., per-CC, across all CCs, and, simultaneous across all CCs. 40-3-3-5; Number of CSI-RS resources for TDCP; FFS: Per-CC, across all CCs, and, simultaneous across all CCs (analogous to 2-33 or 16-1g) - Maximum number of configured CSI-RS resources for TDCP per CC; - Maximum number of configured CSI-RS resources for TDCP across all CCs; - Maximum number of simultaneously activated CSI-RS resources for TDCP across all CCs;; 40-3-3-1. |
| Spreadtrum Communications [5] | Regarding the FG 40-3-3-5 and FG 40-3-3-6, based on our understanding, they are introduced for the same purpose which is used to control the UE complexity. Besides, FG 40-3-3-5 is more aligned with agreement above. The only change is to replace ‘CSI-RS’ with ‘TRS’. Proposal 13: Suggest to change FG 40-3-3-5 as ‘Number of TRS resources for TDCP’, and remove FG 40-3-3-6. |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Maximum number of CSI-RS resources for TDCP Per-CC 2. Maximum number of CSI-RS resources for TDCP across all CCs. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; - For TDCP report, maximum number of CSI-RS resoruces configured in each CSI report setting.; Component 1 candidate values: {1, 2, 4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; - Support of 4∙ KTRS CSI-RS resouces across CCs for TDCP , KTRS≥1; 40-3-3-1; Yes; per band per BC; n/a; n/a; n/a; - KTRS =1 for aperiodic TRS resource set - KTRS≥1 for periodic TRS resource set; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-3-5; Number of CSI-RS resources for TDCP; 1. Supported max # of configured CSI-RS resources for TDCP per CC 2. Supported max # of configured CSI-RS resources for TDCP across all CCs 3. Supported max # of CSI-RS resources for simultaneous TDCP measurement across all CCs; 40-3-3-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for component 1: {1,...,Y+1} Candidate values for component 2: {2,4,8} Candidate values for component 3 <= value configured for componenet 2; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-3-3-6 | Maximum number of TRS resource sets in a report configuration |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-3-3-6 - This should indicate the maximum number of TRS resource sets configurable in a single CSI-RS Resource Setting linked to a TRS Report Setting 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Max # of TRS resource sets in a single CSI-RS resource setting. |
| ZTE [4] | 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Maximum number of TRS resource sets in a report configuration; 40-3-3-1. |
| Spreadtrum Communications [5] | Regarding the FG 40-3-3-5 and FG 40-3-3-6, based on our understanding, they are introduced for the same purpose which is used to control the UE complexity. Besides, FG 40-3-3-5 is more aligned with agreement above. The only change is to replace ‘CSI-RS’ with ‘TRS’. Proposal 13: Suggest to change FG 40-3-3-5 as ‘Number of TRS resources for TDCP’, and remove FG 40-3-3-6. |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; 1. Maximum number of TRS resource sets in a CSI-ReportConfig. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; - For TDCP report, maximum number of TRS resource set configured in each CSI report setting.; Component 1 candidate values: {1, 2}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; - Support of KTRS≥1 TRS resouces sets across CCs for TDCP reporting; 40-3-3-1; Yes; per band per BC; n/a; n/a; n/a; - KTRS =1 for aperiodic TRS resource set - KTRS≥1 for periodic TRS resource set; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | FG 40-3-3-6: the maximum value of KTRS is an open point. In legacy CSI reporting setting, only S=1 CSI resource set can be configured for CSI/beam report, and S=2 CSI resource sets can be configured for group-based beam report, i.e., there is no legacy CSI report configuration with S≥2 CSI resource sets. In legacy TRS configuration, the maximum number of TRS resource sets can be larger than 2 (relevant text from 38.331 is copied below). Allowing a value KTRS >2 therefore would require enhancing legacy CSI report configuration. Besides, the need for supporting KTRS >2 is unclear considering that we only support Y∈{1,2,3,4} and can have up to 4 CSI-RS resources in a TRS resource set. Finally, a large value such as KTRS >2 is also undesired from UE complexity perspective as it would require more CSI calculation time and occupy more CPUs. We therefore propose to support KTRS =1,2 only, and the value KTRS =2 should be subject to UE capability. SIB17-IEs-r17 ::= SEQUENCE { trs-ResourceSetConfig-r17 SEQUENCE (SIZE (1..maxNrofTRS-ResourceSets-r17)) OF TRS-ResourceSet-r17, maxNrofTRS-ResourceSets-r17 INTEGER ::= 64 -- Maximum number of TRS resource sets CSI-RS-ForTracking ::= SEQUENCE { maxBurstLength INTEGER (1..2), maxSimultaneousResourceSetsPerCC INTEGER (1..8), maxConfiguredResourceSetsPerCC INTEGER (1..64), maxConfiguredResourceSetsAllCC INTEGER (1..256) }. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-3-3-6; Maximum number of TRS resource sets in a report configuration; Number<=Y+1; 40-3-3-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values: {1,2,3,4,5}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] [4) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In RAN1#113, FG 40-4-1 are endorsed for the basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A, and whether to treat component 4, e.g. 1 FL DMRS and 2 additional DMRS symbols for at least one port, as a separate FG is FFS. In our view, component 4 mainly applies to medium/high mobility scenario and contradicts with the target scenario of Rel.18 DMRS. Besides that, the higher channel estimation complexity of Rel.18 DMRS and more than 1 additional DMRS symbol makes their combination unacceptable. As a result, component 4 should be a non-basic feature. Proposal 4-2: Regarding FG 40-4-1: - Support component 4 “Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.” as a separate UE feature. 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] [4) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.]. 40. NR_MIMO_evo_DL_UL; 40-4-1h; Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-4-1 - We also support 1 symbol FL DMRS without additional symbol(s) - We also support 1 symbol FL DMRS and 1 additional DMRS symbol - We recommend adding a separate row for DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4. Added FG 40-4-1h - We also support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.]. 40. NR_MIMO_evo_DL_UL; 40-4-1h; DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4. |
| ZTE [4] | - For FG 40-4-1, - Component 3 should be removed due to there is no agreement on DL PTRS RE mapping for Rel-18 PDSCH DMRS with rank 1-4. - Component 4 needs to be kept because the case of “1-symbol FL + 2 additional DMRS symbols” should be the part of FG 40-4-1 as the legacy. Besides, the wording “for at least one port” in component 4 is redundant and should be removed. 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping scheduling type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] [4) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.]. |
| Spreadtrum Communications [5] | Based on the above agreement, it is clear that FGs 40-4-1/1a/1b/1c/1d/1e/1f/1g and FGs 40-4-6/6a/6b/6c/6d/6e/6f are defined analogous to the structure of Rel-15 DMRS Type-I/II for DL and UL respectively. While the other FGs are defined considering the additional UE complexity. Regarding component 4 in FG 40-4-1, similar component was defined for Rel.15 DMRS. For Rel.18 DMRS, the length-4 FD-OCC based Rel.18 DMRS requires higher channel estimation complexity. Therefore, we think it’s reasonable to move component 4 as separate FG. Thus the UE supporting Rel.18 DMRS for PDSCH for mapping type A requires to support 1 symbol FL DMRS with 1 additional DMRS symbol or without additional DMRS symbol. For FG 40-4-6, we prefer to have the same structure as FG 40-4-1. In other words, we suggest to remove component 3 in FG 40-4-6 and define it as separate FG. Proposal 14: Suggest to remove component 4 in FG 40-4-1 and component 3 in FG 40-4-6, while define them as separate FGs. Regarding component 3 in FG 40-4-1 and component 4 in FG 40-4-6, based on our understanding, the intention is to make sure PTRS RE mapping is always supported as long as UE declares supporting Rel.18 DMRS. However, it looks weird since the capability of PTRS RE mapping is defined within the capability of Rel.18 DMRS. Alternatively, we think it’s better to clarify the intention in RAN1 spec such as TS 38.214. Therefore, we proposal to remove component 3 in FG 40-4-1 and component 4 in FG 40-4-6. Proposal 15: Suggest to remove component 3 in FG 40-4-1 and component 4 in FG 40-4-6. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] Comments: it’s better to make it a separate row as shown in 40-4-2 for rank 1-8 [4) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.] Comments: support, it is useful for high-speed case. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-4-1 and 40-4-6: - For 1 symbol FL DMRS without additional symbol(s) or with 1 additional symbol, the number of antenna ports is up to eight. Therefore, Rel.18 PTRS RE mapping is needed to determine the resource of PTRS port(s), and it shall be the basic feature of Rel.18 DMRS. However, there is no need to restrict the rank of PDSCH/PUSCH in PTRS RE mapping, since no restriction is made for DMRS. - In Rel-15 PDSCH DMRS, support of 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is basic feature of DL DMRS. In Rel-15 PUSCH DMRS, support of 1 symbol FL DMRS and 2 additional DMRS symbols is basic feature of UL DMRS. Similarly, these features can also be reused as basic features of Rel-18 DMRS and the bracket of 4) are removed. Proposal 14: Feature 40-4-1 and 40-4-6 are modified as follows. 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping scheduling type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] [4) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.]. |
| Ericsson [10] | This is the basic FG for Rel.18 DMRS PDSCH enhancement for mapping type A. Rel.15 DMRS basic FG 2-5 for PDSCH mapping Type A shall be prerequisite FG for FG 40-4-1. 1 symbol FL DMRS and 2 additional DMRS symbols shall be added to follow same components in FG 2-5. FG 40-4-1 is prerequisite FG for FG40-4-1b/1c/1d/1e/1f/1g. 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] [4) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.] Prerequisite FG 2-5. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-1; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A; - Support of Rel.18 enhanced DMRS ports with FD-OCC4 for PDSCH for mapping type A - Support 1 symbol FL DMRS without additional symbol(s) - Support 1 symbol FL DMRS and 1 additional DMRS symbol - Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] | 40-4-1 - Components: - We believe 3) DL PTRS RE mapping should be a basic FG, otherwise UE cannot determine PTRS REs. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-1; Downlink DMRS for mapping type A; 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] [4) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.]; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1a | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-4-1a - we also support 1 symbol FL DMRS without additional symbol(s) - we also support 1 symbol FL DMRS and 1 additional DMRS symbol 40. NR_MIMO_evo_DL_UL; 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol. |
| ZTE [4] | 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping scheduling type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This is the basic FG for Rel.18 DMRS PDSCH enhancement for mapping type B. Rel.15 DMRS basic FG 2-6 for mapping Type B shall be prerequisite FG for FG 40-4-1a. 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol Prerequisite FG 2-5. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-1a; Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B; - Support of Rel.18 enhanced DMRS ports with FD-OCC4 for PDSCH for mapping type B - Support 1 symbol FL DMRS without additional symbol(s) - Support 1 symbol FL DMRS and 1 additional DMRS symbol; Optional with capability signalling. |
| Xiaomi [12] | 40-4-1a - Components: - We believe 3) DL PTRS RE mapping should be added as a basic FG, otherwise UE cannot determine PTRS REs. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-1a; Downlink DMRS for mapping type B; 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1b | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-1b, - It is duplicated with the revised component 4 in FG 40-4-1, hence it should be removed. 40-4-1b; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-1b; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; Prerequisite FG 40-4-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-1b; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; - Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH - Support 1 symbol FL DMRS without additional symbol(s) - Support 1 symbol FL DMRS and 1 additional DMRS symbol; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-1b; Support 1+2 DMRS (downlink); 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1c | Support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-1c, - FG 40-4-1 and FG 5-28 should be the prerequisite FGs. 40-4-1c; Support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1 and 5-28. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG supports LTE CRS rate matching in addition to FG 40-4-1. FG 5-28 (Rate-matching around LTE CRS) and FG 40-4-1 shall be the prerequisite FG for FG 40-4-1c. 40-4-1c; Support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Prerequisite FG 40-4-1 and FG 5-28. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-1c; Support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; - Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-1c; Support alternative additional DMRS location; Support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 2-5 and 5-28; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1d | Support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-1d/1e/1f, - FG 40-4-1 should be the prerequisite FG. 40-4-1d; Support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-1d; Support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Prerequisite FG 40-4-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-1d; Support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; - Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-1d; Supported 2 symbols front-loaded DMRS(downlink); 1. Support 2 symbols FL-DMRS; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1e | Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-1d/1e/1f, - FG 40-4-1 should be the prerequisite FG. 40-4-1e; Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-1e; Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Prerequisite FG 40-4-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-1e; Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; - Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-1e; Supported 2 symbols front-loaded +2 symbols additional DMRS(downlink); 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1f | Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-1d/1e/1f, - FG 40-4-1 should be the prerequisite FG. 40-4-1f; Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-1f; Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Prerequisite FG 40-4-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-1f; Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-1f; Support 1+3 DMRS symbols(downlink); 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-1g | Support enhanced DMRS {type 1, both type 1 and type 2} for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-1g, - The wording “type 1” and “type 2” should be revised to “eType1” and “eType2” to literally keep inline with Rel-18 enhanced DMRS types. - FG 40-4-1 should be the prerequisite FG. 40-4-1g; Support Rel-18 enhanced DMRS {teType 1, both teType 1 and teType 2} for PDSCH; 40-4-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; Support enhanced DMRS {type 1, both type 1 and type 2} for PDSCH Support DMRS type for Rel.18 enhanced DMRS ports for PUSCH; Support DMRS {etype 1, both etype 1 and etype 2}. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-1g; Support enhanced DMRS {type 1, both type 1 and type 2} for PDSCH; Prerequisite FG 40-4-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; Support enhanced DMRS {etype 1, both etype 1 and etype 2} for PDSCH; - Support of Rel.18 enhanced DMRS ports with FD-OCC4; Component 1 candidate values: {etype 1, both etype 1 and etype 2} }; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] | 40-4-1g - Components: - We propose to add candidate values: Supported DMRS type(s) {eType1, both eType1 and eType2}. We believe eType1 should be a basic FG of R18 DMRS ports. - We believe UE must report FG40-4-1g if UE reports at least one of FG40-4-1 or FG40-4-1a. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-1g; Support DMRS type (downlink); 1. Supported DMRS types; YES; N/A; Per FS; NO; NO; N/A; Candidate values for component 1 = {type 1, type 2, both type 1 and type 2}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-1h; Suport DL PTRS with Rel-18 DMRS; 1.DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-4 | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-4, - FG 40-4-1 should be the prerequisite FG. 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 40-4-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; 1. Support the PDSCH without the following scheduling restrictions: - The number of consecutively scheduled PRBs for PDSCH is even. - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; - Support of Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Note: If this feature is not supported. UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even. 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; - Support of PDSCH scheduling without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS; 40-4-1; Yes; - gNB shall apply the following scheduling restrictions for the PDSCH scheduling other than M-TRP PDSCH with FDM 2a or FDM 2b scheme - The number of consecutively scheduled PRBs for PDSCH is even - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even - FFS: Restriction on scheduling of different UEs in case of MU-MIMO - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme; Per UE; N/A; N/A; N/A; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-4-4a; Reception of S-TRP PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; - Support of S-TRP PDSCH scheduling without the scheduling restriction for Rel.18 eType 1 DMRS; 40-4-1,; Yes; - gNB shall apply the following scheduling restrictions for the PDSCH scheduling - The number of consecutively scheduled PRBs for PDSCH is even - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even - FFS: Restriction on scheduling of different UEs in case of MU-MIMO; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-4-4b; Reception of MTRP-PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; - Support of MTRP-PDSCH scheduling without the scheduling restriction for Rel.18 eType 1 DMRS; 40-4-1, 40-4-5, 40-4-5b; Yes; - gNB shall apply the following scheduling restrictions for MTRP PDSCH scheduling - The number of consecutively scheduled PRBs for PDSCH is even - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even - FFS: Restriction on scheduling of different UEs in case of MU-MIMO - FFS: Other scheduling restrictions for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | In RAN1#110b-e, a UE capability handling orphan RE problem (e.g., if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) was introduced, and the corresponding agreement is as follows: Agreement in RAN1#110b-e For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. - The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - 1) The number of consecutively scheduled PRBs for PDSCH is even. - 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. - 3) FFS: Restriction on scheduling of different UEs in case of MU-MIMO. - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - Note1: Up to UE how to implement DMRS channel estimation. - Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction. - Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded.. As in Table. 2, FG 40-4-4 was defined and discussed for supporting “orphan RE” handling. More details are needed to be discussed as follows: - For pre-requisite, our view is that at least an FG supporting Rel-18 DMRS eType-I is needed. Hence, FG 40-4-1g is appropriate for pre-requisite. - For type, similar with other DMRS related FGs, it can be reported by per FS. - For note, the scheduling restriction in the above agreement if this FG is not reported can be captured. Also, since single-DCI based multi-TRP scheme is also scheduled by using Rel-18 DMRS eType-I, we can define additional scheduling restriction for single-DCI based multi-TRP PDSCH transmission with FDM scheme A or B by extending such scheduling restrictions as per each of TRPs or each of indicated TCI states. Since pre-requisite FG would be different, if this extension on single-DCI based multi-TRP with FDM scheme A or B is defined, then separate FG can be defined. Proposal 11: Support the FGs considering single-TRP PDSCH and single-DCI based FDMSchemeA or FDMSchemeB whether a UE can handle orphan RE problem or not. 40-4-4; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports; Support reception of PDSCH without the scheduling restriction for Rel.18 enhanced DMRS type 1; 40-4-1g; Per FS; If this FG is not reported, UE expects that gNB shall apply the following frequency domain scheduling restrictions for PDSCH for Rel-18 DMRS eType-I. - The number of consecutively scheduled PRBs for PDSCH is even. - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.. 40-4-4a; Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports with single-DCI based FDMSchemeA or FDMSchemeB; Support reception of PDSCH without the scheduling restriction for Rel.18 enhanced DMRS type 1 with single-DCI based FDMSchemeA or FDMSchemeB; 16-2b-2, and/or 16-2b-3, 40-4-1g; Per FS; If this FG is not reported, UE expects that gNB shall apply the following frequency domain scheduling restrictions for PDSCH for Rel-18 DMRS eType-I. - The number of consecutively scheduled PRBs per TCI state for PDSCH is even. - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) per TCI state is even.. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-4; Even RB restriction for type 1 DMRS; 1. Support type 1 DMRS with or without even RB restriction; YES; N/A; Per FS; NO; NO; N/A; Candidate values for component 1 = {Type 1 DMRS with even RB restrictions, Type 1 DMRS without even RB restrictions} Note: Even RB restrictions are the following. 1. The number of consecutively scheduled PRBs for PDSCH is even. 2. The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-4a; Even RB restriction for type 2 DMRS; 1. Support type 2 DMRS with or without even RB restriction; YES; N/A; Per FS; NO; NO; N/A; Candidate values for component 1 = {Type 2 DMRS with even RB restrictions, Type 2 DMRS without even RB restrictions} 1. The number of consecutively scheduled PRBs for PDSCH is even. 2. The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-5 | Support Rel-18 DL DMRS with single DCI based M-TRP | FFS: separate row for Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Regarding FG 40-4-5, it is FFS whether separate row for additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP is needed. Similar to FG 16-2b-1b for single-DCI SDM scheme for PDSCh, separate row is needed for additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP. Proposal 4-3: Support separate row for additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP in FG 40-4-5. 40. NR_MIMO_evo_DL_UL; 40-4-5; Support Rel-18 DL DMRS with single DCI based M-TRP; FFS: separate row for Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-4-5a - FFS1: We recommend additional row(s) of Rel.18 DMRS ports for single-DCI-based M-TRP 40. NR_MIMO_evo_DL_UL; 40-4-5; Support Rel-18 DL DMRS with single DCI based M-TRP; FFS: Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP. 40. NR_MIMO_evo_DL_UL; 40-4-5a; Support additional row(s) of Rel-18 DL DMRS ports with single DCI based M-TRP. |
| ZTE [4] | - For FG 40-4-5, - It is not needed due to this can be inferred by UE capability reporting of FG 40-4-5 and FG 16-2b-1b. 40-4-5; Support Rel-18 DL DMRS with single DCI based M-TRP; FFS: separate row for Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP; 40-4-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-5; Support Rel-18 DL DMRS with single DCI based M-TRP; FFS: separate row for Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP Comments: separate row for additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP is not needed.. |
| Intel Corporation [7] |  |
| Google [8] | 40-4-5: - In Rel-16, support of new DMRS port entry {0, 2, 3} is separate feature of the feature of single-DCI based SDM scheme. Similarly, additional row(s) of Rel.18 DMRS can be treated in the same way. Proposal 15: 40-4-5 is modified as follows. 40. NR_MIMO_evo_DL_UL; 40-4-5; Support Rel-18 DL DMRS with single DCI based M-TRP; FFS: separate row for Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP. 40. NR_MIMO_evo_DL_UL; 40-4-5a; Support of additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP. |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-5; Support Rel-18 DL DMRS with single DCI based M-TRP; - Support of Rel-18 DL DMRS with single DCI based M-TRP PDSCH operation; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] | 40-4-5 - Components: - We don’t believe the FFS is necessary. We believe UE should support the additional row for sDCI mTRP if UE supports FG40-4-5. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-5; Support Rel-18 DL DMRS with S-DCI based M-TRP; 1. Support Rel-18 DL DMRS with S-DCI based M-TRP PDSCH; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-5a; Support new DMRS port entry {0,2,3} for S-DCI based SDM for M-TRP PDSCH; 1.Support DMRS port entry {0,2,3} for S-DCI based SDM for M-TRP PDSCH; 40-4-11; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-5b | Support Rel-18 DL DMRS with M-DCI based M-TRP |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-4-5b The numbering of the FG needs to be fixed. It could be numbered as 40-4-6 |
| ZTE [4] | - For FG 40-4-5b, - It should be removed unless any agreement can be reached from AI 9.1.3.1. 40-4-5b; Support Rel-18 DL DMRS with M-DCI based M-TRP. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-5b; Support Rel-18 DL DMRS with M-DCI based M-TRP; - Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-5b; Support Rel-18 DL DMRS with M-DCI based M-TRP; 1. Support Rel-18 DL DMRS with M-DCI based M-TRP; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-6 | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols] [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols] [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4]. 40. NR_MIMO_evo_DL_UL; 40-4-6g; Support 1 symbol FL DMRS and 2 additional DMRS symbols. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-4-6 - We also support 1 symbol FL DMRS and 1 additional DMRS symbol - We also support 1 symbol FL DMRS and 2 additional DMRS symbols - We recommend adding a separate row for DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols] [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4]:. 40. NR_MIMO_evo_DL_UL; 40-4-6g; UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4. |
| ZTE [4] | - For FG 40-4-6, similar to FG 40-4-1, - Component 3 needs to be kept. - Component 4 should be removed. 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols] [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4]. |
| Spreadtrum Communications [5] | Based on the above agreement, it is clear that FGs 40-4-1/1a/1b/1c/1d/1e/1f/1g and FGs 40-4-6/6a/6b/6c/6d/6e/6f are defined analogous to the structure of Rel-15 DMRS Type-I/II for DL and UL respectively. While the other FGs are defined considering the additional UE complexity. Regarding component 4 in FG 40-4-1, similar component was defined for Rel.15 DMRS. For Rel.18 DMRS, the length-4 FD-OCC based Rel.18 DMRS requires higher channel estimation complexity. Therefore, we think it’s reasonable to move component 4 as separate FG. Thus the UE supporting Rel.18 DMRS for PDSCH for mapping type A requires to support 1 symbol FL DMRS with 1 additional DMRS symbol or without additional DMRS symbol. For FG 40-4-6, we prefer to have the same structure as FG 40-4-1. In other words, we suggest to remove component 3 in FG 40-4-6 and define it as separate FG. Proposal 14: Suggest to remove component 4 in FG 40-4-1 and component 3 in FG 40-4-6, while define them as separate FGs. Regarding component 3 in FG 40-4-1 and component 4 in FG 40-4-6, based on our understanding, the intention is to make sure PTRS RE mapping is always supported as long as UE declares supporting Rel.18 DMRS. However, it looks weird since the capability of PTRS RE mapping is defined within the capability of Rel.18 DMRS. Alternatively, we think it’s better to clarify the intention in RAN1 spec such as TS 38.214. Therefore, we proposal to remove component 3 in FG 40-4-1 and component 4 in FG 40-4-6. Proposal 15: Suggest to remove component 3 in FG 40-4-1 and component 4 in FG 40-4-6. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port] Comments: support, it is useful for high-speed case [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4] Comments: it’s better to take it as an optional feature as shown in 40-4-12a for rank 1-8. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-4-1 and 40-4-6: - For 1 symbol FL DMRS without additional symbol(s) or with 1 additional symbol, the number of antenna ports is up to eight. Therefore, Rel.18 PTRS RE mapping is needed to determine the resource of PTRS port(s), and it shall be the basic feature of Rel.18 DMRS. However, there is no need to restrict the rank of PDSCH/PUSCH in PTRS RE mapping, since no restriction is made for DMRS. - In Rel-15 PDSCH DMRS, support of 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is basic feature of DL DMRS. In Rel-15 PUSCH DMRS, support of 1 symbol FL DMRS and 2 additional DMRS symbols is basic feature of UL DMRS. Similarly, these features can also be reused as basic features of Rel-18 DMRS and the bracket of 4) are removed. Proposal 14: Feature 40-4-1 and 40-4-6 are modified as follows. 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols] [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4]. |
| Ericsson [10] | This is the basic FG for Rel.18 DMRS PUSCH enhancement for mapping type A. Rel.15 DMRS basic FG 2-16 for PUSCH mapping Type A shall be prerequisite FG for FG 40-4-1. 1 symbol FL DMRS with 0, 1 and 2 additional DMRS symbols shall be added to follow same components in FG 2-16. FG 40-4-6 is prerequisite FG for FG40-4-4b/4c/4d/4e/4f. 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols] [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4] Prerequisite FG 2-16. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports; - Support of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports - Support 1 symbol FL DMRS without additional symbol(s) - Support 1 symbol FL DMRS and 1 additional DMRS symbols - Support 1 symbol FL DMRS and 2 additional DMRS symbols; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-6; Uplink DMRS for scheduling type A; 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols] [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4]; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-6a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-4-6a - We also support 1 symbol FL DMRS without additional symbol(s) - We also support 1 symbol FL DMRS and 1 additional DMRS symbol 40. NR_MIMO_evo_DL_UL; 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol. |
| ZTE [4] | 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This is the basic FG for Rel.18 DMRS PUSCH enhancement for mapping type B. Rel.15 DMRS basic FG 2-16a for mapping Type B shall be prerequisite FG for FG 40-4-6a. FG 40-4-6a shall be prerequisite FG for 40-4-6b. In RAN1#110bis-e meeting we had following agreement: Agreement For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only. According to above agreement, for more than 4 layers SU-MIMO PUSCH, a UE support Rel-18 DMRS shall also support Rel-15 Type1/Type2 DMRS ports. Therefore, the candidate “Rel.18 DMRS” for 40-4-10 shall be removed. 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol Prerequisite FG 2-16a. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-6a; Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports; - Support of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports - Support 1 symbol FL DMRS without additional symbol(s) - Support 1 symbol FL DMRS and 1 additional DMRS symbols; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-6a; Uplink DMRS for scheduling type B; 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-6b | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-6b, - It is duplicated with the revised component 3 in FG 40-4-6, hence it should be removed. 40-4-6b; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-6b; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH; Prerequisite FG 40-4-6 and FG 40-4-6a. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-6b; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH; - Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-6b; Support 1+2 DMRS (uplink); 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-6c | Support DMRS type (uplink) for Rel.18 enhanced DMRS ports for PUSCH | Support DMRS {type 1, both type 1 and type 2} |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-4-6c - We also support DMRS {type 1, both type 1 and type 2} as components 40. NR_MIMO_evo_DL_UL; 40-4-6c; Support DMRS type (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support DMRS {type 1, both type 1 and type 2}. |
| ZTE [4] | - For FG 40-4-6c, - The wording “type 1” and “type 2” should be revised to “eType1” and “eType2” to literally keep inline with Rel-18 enhanced DMRS types. - The wording “one of” should be added to the component. - FG 40-4-6 should be the prerequisite FG. 40-4-6c; Support DMRS type (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support DMRS: one of {teType 1, both teType 1 and teType 2}; 40-4-6. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-6c; Support DMRS type (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support DMRS {etype 1, both etype 1 and etype 2}. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-6c; Support DMRS type (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Support DMRS {type 1, both type 1 and type 2} Prerequisite FG 40-4-6. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-6c; Support DMRS type (uplink) for Rel.18 enhanced DMRS ports for PUSCH; - Supported DMRS type (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Component 1 candidate values: {etype 1, both etype 1 and etype 2}; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] | 40-4-6c - Components: - We propose to add candidate values: Supported DMRS type(s) {eType1, both eType1 and eType2}. We believe eType1 should be a basic FG of R18 DMRS ports. - We believe UE must report FG40-4-6c if UE reports at least one of FG40-4-6 or FG40-4-6a. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-6c; Support DMRS type (uplink); 1. Supported DMRS types; YES; N/A; Per FS; NO; NO; N/A; Candidate values for component 1 = {type 1, type 2, both type 1 and type 2}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-6d | Supported 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-6d/6e/6f/10, - FG 40-4-1 should be the prerequisite FG. 40-4-6d; Supported 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-6d; Supported 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Prerequisite FG 40-4-6. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-6d; Supported 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; - Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-6d; Supported 2 symbols front-loaded DMRS (uplink); 1. Support 2 symbols FL-DMRS; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-6e | Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-6d/6e/6f/10, - FG 40-4-1 should be the prerequisite FG. 40-4-6e; Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-6e; Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; Prerequisite FG 40-4-6. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-6e; Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; - Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-6e; Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink); 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-6f | Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-6d/6e/6f/10, - FG 40-4-1 should be the prerequisite FG. 40-4-6f; Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-6f; Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; Prerequisite FG 40-4-6. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-6f; Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; - Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-6f; Support 1+3 uplink DMRS symbols(uplink); 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-10 | DMRS port configuration for PUSCH with rank 5-8 | Candidate values: {Rel. 15 DMRS, Rel. 18 DMRs, borh} |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-6-10 - For DMRS port configuration for PUSCH with rank 5-8, the rel 15 DMRS, and both the Rel 15 and 18 DMRS can be considered as components based on the agreement made in RAN1#110bis-e. Agreement (RAN1#110bis-e) For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.. 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH with rank 5-8; Candidate values: {Rel. 15 DMRS, Rel. 18 DMRs, borh Rel 15 and rel 18 DMRS}. |
| ZTE [4] | - For FG 40-4-6d/6e/6f/10, - FG 40-4-1 should be the prerequisite FG. 40-4-10; DMRS port configuration for PUSCH with rank 5-8; Candidate values: {Rel. 15 DMRS, Rel. 18 DMRs, borth}; 40-4-6. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH with rank 5-8; Candidate values: {Rel. 15 DMRS, Rel. 18 DMRS, both}. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | 40-4-10; DMRS port configuration for PUSCH with rank 5-8; Candidate values: {Rel. 15 DMRS, Rel. 18 DMRs, both Rel.15 DMRS and Rel.18 DMRS}. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-10; DMRS port configuration for PUSCH with rank 5-8; - Supported DMRS port configuration for PUSCH with rank 5-8; Component 1 candidate values: {Rel.15 DMRS, Rel.18 DMRS, both}; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] | 40-4-10 - Components: - We suggest updating candidate values to {Rel. 15 DMRS, both} or {Rel. 18 DMRS, both}. We believe at least one of R15 DMRS ports or R18 DMRS ports should be a basic FG of 8Tx, otherwise it causes market fragmentation. |
| OPPO [14] |  |
| Samsung [15] | DMRS for UL 8TX For DMRS up to 8 layers for supporting UL 8TX, since supported DMRS type is one of parts for supporting UL 8TX, this could be required with having pre-requisite of a basic feature of supporting UL 8TX, which could be discussed and defined in AI 9.1.4.2. Also, in RAN1#110b-e, it was agreed that which DMRS type(s) can be used for PUSCH with more than 4 layers as follows. Agreement in RAN1#110b-e For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.. Since a UE supporting Rel-18 DMRS types could be indicated with either Rel-15 or Rel-18 DMRS types, and a UE not supporting Rel-18 DMRS types could be indicated Rel-15 DMRS types only, our view is that a new FG supporting Rel-15 DMRS types with up to 8 layers could be defined as a basic feature for UL 8TX DMRS and can be reported by the UE. On top of that, Rel-18 DMRS types could be additionally reported from the UE, then gNB can indicate either of Rel-15 or Rel-18 DMRS types for supporting up to 8 layers for the UE. Hence, candidate values can include Rel.15 DMRS only, and both Rel.15 and Rel.18 DMRS, and Rel.18 DMRS only is not needed. Proposal 15: Support the following table of DMRS port configuration for PUSCH with rank 5-8. 40-4-10; DMRS port configuration for PUSCH with rank 5-8; Per FS; Candidate values: {Rel.15 DMRS only, both Rel.15 DMRS and Rel.18 DMRS}. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-4-10; Rank >4 PUSCH for a Rel-18 UE configured with Rel-15 DMRS ports by RRC DMRS port configuration for PUSCH with rank 5-8; Support Rank >4 PUSCH for a Rel-18 UE configured with Rel-15 DMRS ports by RRC DMRS port configuration for PUSCH with rank 5-8; YES; N/A; Per FS; NO; NO; N/A; Candidate values: {Rel. 15 DMRS, Rel. 18 DMRs, both}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-4-12 | UL PTRS for PUSCH with rank 5-8 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-4-12, - It is not needed due to it can be inferred by the legacy FG 2-47 and FG 2-48. 40-4-12; UL PTRS for PUSCH with rank 5-8. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-4-12： - We propose to delete FG 40-4-12 since it is nature for an UE supports UL PTRS for PUSCH with rank 1-4 to support UL PTRS for PUSCH with rank 5-8. Proposal 16: Remove FG 40-4-12 for PUSCH transmission with rank 5-8. |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-4-12; UL PTRS for 8 Tx PUSCH operation; - Maximum number of UL PTRS ports for 8Tx PUSCH operation; Component 1 candidate values: {1,2}; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | Regarding FG 40-4-12 supporting UL PTRS for PUSCH with rank 5-8, our view is that the maximum number of PTRS ports which can be either 1 or 2 can be a component, and FG 2-47 which is a basic PTRS feature and FG 40-4-10 which is DMRS port configuration for PUSCH with rank 5-8 can be pre-requisites. Proposal 16: Support the following table of UL PTRS for PUSCH with rank 5-8. 40-4-12; UL PTRS for PUSCH with rank 5-8; 1. Maximum number of PTRS ports; 2-47, 40-4-10; Per FS; Candidate values of Component 2: {1, 2}. |
| Qualcomm Incorporated [16] |  |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-1 | SRS comb offset hopping |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; 1. Supported time-domain behavior for the SRS (candidate value: aperiodic/semi-persistent/periodic). |
| CATT [9] | 40-5-1/40-5-2: - In RAN1 #113 meeting, it was agreed that SRS comb offset hopping and cyclic shift hopping can be configured for an SRS resource at the same time as a separate UE capability, therefore a new FG should be added for supporting configuring SRS comb offset hopping and cyclic shift hopping simultaneously for an SRS resource. Proposal 26: A new FG is added for supporting configuring SRS comb offset hopping and cyclic shift hopping simultaneously for an SRS resource: 40. NR_MIMO_evo_DL_UL; 40-5-x; SRS with comb offset hopping and cyclic shift hopping configured simultaneously. |
| Ericsson [10] | This is the basic FG for supporting SRS comb offset hopping. All FGs introducing features for SRS comb offset hopping shall have this as a prerequisite. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; - Support of SRS comb offset hopping; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; - Support of comb value {2, 4, 8} for comb offset hopping - Support of configuring asubset of comb offsets; 2-53; Yes; SRS comb offset hopping is not supported.; per band; n/a; n/a; n/a; For component 2, the size of subset of comb offsets is TBD; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | For supporting SRS cyclic shift hopping and comb-offset hopping, separate FG for each hopping scheme has been agreed as in Table. 3. Further, simultaneous configuration of both comb-offset hopping and cyclic shift hopping has been agreed in RAN1#113 based on a separate UE capability and no joint hopping scheme is supported. Hence, three separate FGs are needed as follows. For all three FGs, the applicable SRS is for periodic, semi-persistent, and aperiodic SRS with usage of antenna switching. Also, basic feature for SRS as FG 2-52 is needed as a pre-requisite. Proposal 17: Support the following table of hopping schemes. 40-5-1; SRS comb offset hopping; 2-52; Per band; Applicable for periodic, semi-persistent, and aperiodic SRS with usage of antenna switching. 40-5-2; SRS cyclic shift hopping; 2-52; Per band; Applicable for periodic, semi-persistent, and aperiodic SRS with usage of antenna switching. 40-5-XX; Simultaneous configuration of both SRS comb-offset hopping and SRS cyclic shift hopping; SRS comb offset hopping and cyclic shift hopping can be configured for a SRS resource at the same time; 2-52, 40-5-1, 40-5-2; Per band; No joint scheme between SRS comb-offset and SRS cyclic shift hopping scheme is supported.. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-5-1; SRS comb offset hopping; Support of comb offset hopping for SRS; Yes; N/A; band; No; No; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-2 | SRS cyclic shift hopping |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; 1. Supported time-domain behavior for the SRS (candidate value: aperiodic/semi-persistent/periodic). |
| CATT [9] | 40-5-1/40-5-2: - In RAN1 #113 meeting, it was agreed that SRS comb offset hopping and cyclic shift hopping can be configured for an SRS resource at the same time as a separate UE capability, therefore a new FG should be added for supporting configuring SRS comb offset hopping and cyclic shift hopping simultaneously for an SRS resource. Proposal 26: A new FG is added for supporting configuring SRS comb offset hopping and cyclic shift hopping simultaneously for an SRS resource: 40. NR_MIMO_evo_DL_UL; 40-5-x; SRS with comb offset hopping and cyclic shift hopping configured simultaneously. |
| Ericsson [10] | This is the basic FG for supporting SRS cyclic shift hopping. All FGs introducing features for SRS cyclic shift hopping shall have this as a prerequisite. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; - Support of cyckic shift hopping; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; - Support of nSRScs,max for cyclic shift hopping - Support of configuring a subset of cyclic shifts - Support of K∙nSRScs,max for cyclic shift hopping when a subset for cyclic shifts is not configured; 2-53; Yes; SRS cyclic shift hopping is not supported; per band; n/a; n/a; n/a; For component 3, the value of K is TBD.; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-5-2; SRS cyclic shift hopping; Supoort of cyclic shift hopping for SRS; Yes; N/A; band; No; No; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-1b | SRS comb offset hopping combined with legacy group/sequence hopping |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | 40-5-1b; SRS comb offset hopping combined with legacy group/sequence hopping; 40-5-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | In RAN1#112-bis-e, it was agreed that comb-offset hopping can be combined with legacy group/sequence hopping, depending on UE capability. This FG indicates support for such combination of different hopping schemes. FG 40-5-1 is a prerequisite for this FG. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-5-1b; SRS comb offset hopping combined with legacy group/sequence hopping; - Support of SRS comb offset hopping combined with group / sequence hopping on an SRS resource.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-5-1b; SRS comb offset hopping combined with legacy group/sequence hopping; - Support of SRS comb offset hopping combining with one of group /seuqnece hopping; 40-5-1; Yes; per band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-5-1b; SRS comb offset hopping combined with legacy group/sequence hopping; Support of being configured with both comb offset hopping and legacy group/sequence hopping for the same SRS resource; Yes; N/A; band; No; No; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-1a | Comb offset hopping time-domain behavior when repetition factor R>1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | 40-5-1a; Comb offset hopping time-domain behavior when repetition factor R>1 and TDM scheme is configured; Support at least one of the following mode: - Mode 1: support the comb offset hopping time-domain behavior depends on the symbols index l’ of each symbol across the R repetition symbols or the m TDMed symbols; - Mode 2: support the comb offset hopping time-domain behavior depends on the symbol index l’ of the first symbol across the R repetition symbols or m TDMed symbols; - Mode 3: support the comb offset hopping time-domain behavior depends on the symbol index l’ of each symbol or the first symbol across the R repetition symbols or m TDMed symbols; Notes: - The candidate values of R are {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} The candidate values of m are {2, 4, 8, 10, 12, 14}; 40-5-1 40-5-4 40-5-5. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG is needed for UE to indicate whether it supports one or both of per-symbol or per-repetition comb-offset hopping. FG 40-5-1 is a prerequisite for this FG. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-5-1a; Comb offset hopping time-domain behavior when repetition factor R>1; - Support of different SRS ports mapped onto different OFDM symbols for 8 port SRS; Component 1 candidate value: {pattern 1, pattern 2, both} Note: Pattern 1: within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol. Pattern 2: within a slot, the time-domain hopping behavior depends on the first symbol across the R repetitions.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-5-1a; Comb offset hopping time-domain behavior when repetition factor R>1; - Support of time-domain hopping behaviour depends on the OFDM symol index of each symbol or the first symbol across the R repetitions based on RRC configuratoin; 40-5-1; Yes; per band; n/a; n/a; n/a; FFS configuration details.; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-2a; SRS cyclic shift hopping combined with legacy group/sequence hopping; - Support of SRS cyclic shfit hopping combining with one of group /seuqnece hopping; 40-5-2; Yes; per band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-5-1a; Comb offset hopping time-domain behavior when repetition factor R>1; Supported comb offset hopping granularity in time when repetition factor R>1 is configured; Yes; N/A; band; No; No; N/A; Candidate values: {‘per SRS symbol’,’per R SRS symbols’, ‘both’}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS with 8 Tx ports—antenna switching | FFS: separate row for comb8 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-5-4 - FFS1: For SRS with 8 Tx ports—antenna switching, we propose having a separate row for comb8 mapping 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS with 8 Tx ports—antenna switching; FFS: separate row for comb8. 40. NR_MIMO_evo_DL_UL; 40-5-4b; SRS with 8 Tx ports and Comb8 mapping —antenna switching. |
| ZTE [4] | 40-5-4; SRS with 8 Tx ports—antenna switching; FFS: separate row for comb8 Support TDM factor s = 2, the number of TDMed symbols m = {2, 4, 8, 10, 12, 14}. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS with 8 Tx ports—antenna switching; FFS: separate row for comb8 Comments: no need. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-5-4/40-5-4a/40-5-6/40-5-7: - A UE supports 8Tx SRS with comb-2 and comb-4 may not support 8Tx SRS with comb-8. Therefore separate row of comb-8 are needed for 40-5-4/40-5-4a/40-5-6/40-5-7. - A UE supports SRS with TDMed 8Tx ports for codebook may not support SRS with TDMed 8Tx ports for antenna switching, therefore we prefer not to merge FG 40-5-7 with FG 40-5-6. Proposal 27: Separate row of comb-8 are needed for FG 40-5-4/40-5-4a/40-5-6/40-5-7. |
| Ericsson [10] | This is the FG for supporting an 8-port SRS resource in an SRS resource set with usage ‘antennaSwitching’. In our view, there is no need for a separate FG for comb 8. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-5-4; SRS with 8 Tx ports—antenna switching; - Support of 8T8R SRS antenna switching. - Report the entry number of the first-listed band with UL in the band combination that affects this DL. - Report the entry number of the first-listed band with UL in the band combination that switches together with this UL.; Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: Compnent 1 indicates the supported xTyR in addition to 8T8R. Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} Note: component 2 and Component 3 are optional. If reported, the reported values for component 2 and component 3 are not valid for the same values of xTyR in component 1 reported with Rel-15/16/17 UE capability reporting; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | Regarding 8-port SRS, non-TDMed port mapping and TDMed port mapping related FGs have been adopted as in Table. 3. For both non-TDMed and TDMed port mapping, our view is that separate rows for Comb-8 is not needed and it could be as a component of FG 40-5-4 and FG 40-5-4a. For TDMed port mapping, our view is that merging FG 40-5-6 and FG 40-5-7 is not needed and we can keep separate rows for antenna switching and codebook. Proposal 21: Define FGs for SRS with 8Tx ports by non-TDMed and TDMed port mapping. 40-5-4; SRS with 8 Tx ports—antenna switching; 1. Support 8T8R 2. Whether to support comb-8; 2-53, 2-55; Per band; Candidate values of component 2: {support, not support}. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-5-3; 8-ports SRS without TDM for antenna switch; 1. Support 8-ports SRS without TDM for antenna switch, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS without TDM for antenna switch, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS without TDM for antenna switch, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS without TDM for antenna switch, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot; Yes; N/A; Per FSPC; No; No; N/A; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-3a; 8-port nonTDMed SRS with srs-AntennaSwitching2SP-1Periodic; 1.Support of maximum 2 SP 8-port SRS resource sets and maximum 1 periodic 8-port SRS resource set for SRS antenna switching without TDM; Yes; N/A; Per FSPC; No; No; N/A; Note1: - Applies for all supported xTyR where y<=8. - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS. - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. - The two SP-SRS resource sets are not activated at the same time.; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-4a | SRS with 8 Tx ports—codebook | FFS: separate row for comb8 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-5-4a - FFS1: For SRS with 8 Tx ports—codebook, we propose having a separate row for comb8 mapping 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS with 8 Tx ports—codebook; FFS: separate row for comb8. 40. NR_MIMO_evo_DL_UL; 40-5-4c; SRS with 8 Tx ports and Comb8 mapping—codebook. |
| ZTE [4] | 40-5-4a; SRS with 8 Tx ports—codebook; FFS: separate row for comb8 Support TDM factor s = 2, the number of TDMed symbols m = {2, 4, 8, 10, 12, 14}. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS with 8 Tx ports—codebook; FFS: separate row for comb8 Comments: no need. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-5-4/40-5-4a/40-5-6/40-5-7: - A UE supports 8Tx SRS with comb-2 and comb-4 may not support 8Tx SRS with comb-8. Therefore separate row of comb-8 are needed for 40-5-4/40-5-4a/40-5-6/40-5-7. - A UE supports SRS with TDMed 8Tx ports for codebook may not support SRS with TDMed 8Tx ports for antenna switching, therefore we prefer not to merge FG 40-5-7 with FG 40-5-6. Proposal 27: Separate row of comb-8 are needed for FG 40-5-4/40-5-4a/40-5-6/40-5-7. |
| Ericsson [10] | This is the FG for supporting an 8-port SRS resource in an SRS resource set with usage ‘codebook’. In our view, there is no need for a separate FG for comb 8. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS with 8 Tx ports—codebook; - Support of 8 port SRS for codebook based UL operation; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | Regarding 8-port SRS, non-TDMed port mapping and TDMed port mapping related FGs have been adopted as in Table. 3. For both non-TDMed and TDMed port mapping, our view is that separate rows for Comb-8 is not needed and it could be as a component of FG 40-5-4 and FG 40-5-4a. For TDMed port mapping, our view is that merging FG 40-5-6 and FG 40-5-7 is not needed and we can keep separate rows for antenna switching and codebook. Proposal 21: Define FGs for SRS with 8Tx ports by non-TDMed and TDMed port mapping. 40-5-4a; SRS with 8 Tx ports—codebook; 2-52; Per band. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-5-4; 8-ports SRS without TDM for codebook PUSCH; 1. Support 8-ports SRS without TDM for codebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS without TDM for codebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS without TDM for codebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS without TDM for codebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot; N/A; Per FSPC; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-6 | SRS with TDMed 8 Tx ports—antenna switching | FFS: separate row for comb8 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-5-6 - FFS1: For SRS with TDMed 8 Tx ports—antenna switching, we propose having a separate row for comb8 mapping as well 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; FFS: separate row for comb8. 40. NR_MIMO_evo_DL_UL; 40-5-6a; SRS with TDMed 8 Tx ports with Comb8 mapping—antenna switching and codebook. |
| ZTE [4] | 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; FFS: separate row for comb8 Support TDM factor s = 2, the number of TDMed symbols m = {2, 4, 8, 10, 12, 14}; 40-5-4. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; FFS: separate row for comb8 Comments: no need. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-5-4/40-5-4a/40-5-6/40-5-7: - A UE supports 8Tx SRS with comb-2 and comb-4 may not support 8Tx SRS with comb-8. Therefore separate row of comb-8 are needed for 40-5-4/40-5-4a/40-5-6/40-5-7. - A UE supports SRS with TDMed 8Tx ports for codebook may not support SRS with TDMed 8Tx ports for antenna switching, therefore we prefer not to merge FG 40-5-7 with FG 40-5-6. Proposal 27: Separate row of comb-8 are needed for FG 40-5-4/40-5-4a/40-5-6/40-5-7. Proposal 28: Not to merge FG 40-5-7 with FG 40-5-6. |
| Ericsson [10] | This FG is for additionally supporting TDM for an 8-port SRS resource in an SRS resource set with usage ‘antennaSwitching’. In our view, this can be merged with FG 40-5-7. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; - Support of different SRS ports mapped onto 2 OFDM symbols, each OFDM symbol carries 4 SRS ports, for 8 port SRS for 8T8R SRS antenna switching.; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | Regarding 8-port SRS, non-TDMed port mapping and TDMed port mapping related FGs have been adopted as in Table. 3. For both non-TDMed and TDMed port mapping, our view is that separate rows for Comb-8 is not needed and it could be as a component of FG 40-5-4 and FG 40-5-4a. For TDMed port mapping, our view is that merging FG 40-5-6 and FG 40-5-7 is not needed and we can keep separate rows for antenna switching and codebook. Proposal 21: Define FGs for SRS with 8Tx ports by non-TDMed and TDMed port mapping. 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; 1. Support 8T8R 2. Whether to support comb-8; 2-53, 2-55; Per band; Candidate values of component 2: {support, not support} Only 2 TDMed subsets of ports are supported.. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-5-5; 8-ports SRS with TDM for antenna switch; 1. Support 8-ports SRS with TDM for antenna switch, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS with TDM for antenna switch, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS with TDM for antenna switch, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS with TDM for antenna switch, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Support number of SRS symbols m; Yes; N/A; Per FSPC; No; No; N/A; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-5a; 8-port TDMed SRS with srs-AntennaSwitching2SP-1Periodic; 1.Support of maximum 2 SP 8-port SRS resource sets and maximum 1 periodic 8-port SRS resource set for SRS antenna switching with TDM; Yes; N/A; Per FSPC; No; No; N/A; Note1: - Applies for all supported xTyR where y<=8. - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS. - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. - The two SP-SRS resource sets are not activated at the same time.; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-5-7 | SRS with TDMed 8 Tx ports—codebook | FFS: merge with antenna switching |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-5-7 - FFS1: For SRS with TDMed 8 Tx ports—codebook, we propose having a common row for antenna switching and codebook 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; FFS: merge with antenna switching:. |
| ZTE [4] | 40-5-7; SRS with TDMed 8 Tx ports—codebook; FFS: merge with antenna switching Support TDM factor s = 2, the number of TDMed symbols m = {2, 4, 8, 10, 12, 14}; 40-5-4a. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; FFS: merge with antenna switching Comments: it’s better to keep the separate row since they are different usages for PUSCH and PDSCH. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-5-4/40-5-4a/40-5-6/40-5-7: - A UE supports 8Tx SRS with comb-2 and comb-4 may not support 8Tx SRS with comb-8. Therefore separate row of comb-8 are needed for 40-5-4/40-5-4a/40-5-6/40-5-7. - A UE supports SRS with TDMed 8Tx ports for codebook may not support SRS with TDMed 8Tx ports for antenna switching, therefore we prefer not to merge FG 40-5-7 with FG 40-5-6. Proposal 27: Separate row of comb-8 are needed for FG 40-5-4/40-5-4a/40-5-6/40-5-7. Proposal 28: Not to merge FG 40-5-7 with FG 40-5-6. |
| Ericsson [10] | This FG is for additionally supporting TDM for an 8-port SRS resource in an SRS resource set with usage ‘codebook. In our view, this can be merged with FG 40-5-6. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; - Support of different SRS ports mapped onto 2 OFDM symbols, each OFDM symbol carries 4 SRS ports, for 8 port SRS for codebook based UL operation.; Optional with capability signalling. |
| Xiaomi [12] |  |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] | Regarding 8-port SRS, non-TDMed port mapping and TDMed port mapping related FGs have been adopted as in Table. 3. For both non-TDMed and TDMed port mapping, our view is that separate rows for Comb-8 is not needed and it could be as a component of FG 40-5-4 and FG 40-5-4a. For TDMed port mapping, our view is that merging FG 40-5-6 and FG 40-5-7 is not needed and we can keep separate rows for antenna switching and codebook. Proposal 21: Define FGs for SRS with 8Tx ports by non-TDMed and TDMed port mapping. 40-5-7; SRS with TDMed 8 Tx ports—codebook; 2-52; Per band; Only 2 TDMed subsets of ports are supported.. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-5-6; 8-ports SRS with TDM for codebook PUSCH; 1. Support 8-ports SRS with TDM for codebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS with TDM for codebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS with TDM for codebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS with TDM for codebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Support number of SRS symbols m; Yes; N/A; Per FSPC; No; No; N/A; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-6-1 | Single-DCI based STx2P SDM scheme for PUSCH—codebook | FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In RAN1#113, FG 40-6-1 and FG 40-6-1a were agreed for codebook-based SDM and non-codebook-based SDM schemes, respectively. Whether separate rows are needed for “2 PTRS ports for SDM scheme” and “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” are FFS. In our view, a UE can support SDM scheme without necessarily supporting 2 PTRS ports. Thus, a separate row is needed for “2 PTRS ports for SDM scheme”. In addition, sTRP transmission is a basic UE transmission scheme and a UE that supports SDM scheme should also support sTRP scheme. Therefore, a separate row is not needed for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP”. In addition, based on FG 2-47, supporting 1 PTRS port is “Mandatory with capability signalling for FR2” for sTRP transmission from Rel-15. Similarly, we think 1 PTRS port must also be supported for codebook/non-codebook based SDM scheme. Therefore, a new component of 1 PTRS port for codebook/non-codebook based SDM scheme should be added to 40-6-1 and 40-6-1a. Based on the above discussion, we have the following proposal: Proposal 5-1: Regarding FG 40-6-1 and FG 40-6-1a: - Separate row/FG is needed for 2 PTRS ports for codebook-based and non-codebook based SDM scheme - Separate row is not needed for dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP. - Add 1 PTRS port for codebook/non-codebook based SDM scheme as a component of 40-6-1 and 40-6-1a 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “ - Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP - 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-1c; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-6-1 - FFS1: We recommend a separate row for “2 PTRS ports for SDM scheme” (2-48) - FFS2: We recommend a separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP”. 40. NR_MIMO_evo_DL_UL; 40-6-1c; Single-DCI based STx2P with 2 PTRS ports for SDM scheme for PUSCH—codebook. 40. NR_MIMO_evo_DL_UL; 40-6-1d; Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook. |
| ZTE [4] | - For FG 40-6-1, - Given that there is no agreement reached so far with regards to “2 PTRS ports for STxMP SDM scheme”, the first FFS should be removed as of now. In the meanwhile, it is worth noting that there is no such UE capability for Rel-17 MTRP TDM PUSCH repetition, where we fail to see the differentiation between Rel-18 STxMP SDM scheme and Rel-17 MTRD TDM repetition in terms of the number of PTRS ports. - In general, dynamic switching by DCI between STxMP SDM scheme and sTRP scheme is the inherence of Rel-18 STxMP SDM scheme, which is similar to that of Rel-17 MTRP TDM PUSCH repetition. Hence, it is not needed to take this as a separate row of this FG. - Besides, the components of the number of SRS resource sets, the supported number of SRS resources in one SRS resource set and the maximal number of MIMO layers of PUSCH associated each SRS resource set should be added. - FG 2-14 should be adopted as the prerequisite FG. 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” - Support of single-DCI based STx2P SDM scheme for codebook based PUSCH - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in one SRS resource set - Maximum number of MIMO layers of each SRS resource set for CB PUSCH with SDM scheme; 2-14. |
| Spreadtrum Communications [5] | For single-DCI based STx2P, the feature “2 PTRS ports for SDM scheme” is only used for SDM scheme and these two features, i.e. 2 PTRS ports and SDM scheme, are tightly coupled. In other word, an UE supporting SDM scheme must support 2 PTRS ports. Thus, the feature “2 PTRS ports for SDM scheme” can be included in the rows of FGs 40-6-1 and 40-6-1a. Proposal 20: Support including “2 PTRS ports for SDM scheme” in 40-6-1 and 40-6-1a. The feature “Dynamic switching by DCI 0_1/0_2 between single-DCI STx2P SDM and sTRP” is common for 40-6-1 and 40-6-1a and can be a separate row. Similarly, the feature “Dynamic switching by DCI 0_1/0_2 between single-DCI STx2P SFN and sTRP” is also common for 40-6-2 and 40-6-2a and can be a separate row. Proposal 21: Support the separate rows for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” for codebook-based PUSCH and non-codebook-based PUSCH. Proposal 22: Support the separate rows for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” for codebook-based PUSCH and non-codebook-based PUSCH. For SDM scheme, the maximum number of layers, two SRS resource sets and the number of SRS resources in one SRS resource set should be included in 40-6-1 and 40-6-1a, where the supported maximum number of layers can be per UE panel or the layer combination. For SFN scheme, the maximum number of layers, two SRS resource sets and the number of SRS resources in one SRS resource set should be included in 40-6-2 and 40-6-2a. Proposal 23: Support including the following features in 40-6-1 and 40-6-1a: supported maximum number of layers associated with one SRS resource set/supported layer combinations, two SRS resource sets, the number of SRS resources in one SRS resource set. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; FFS: separate row for “2 PTRS ports for SDM scheme” Comments: support separate row for maximum number of PTRS ports for Stx2P. FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” Comments: it should be the mandatory feature. Additional components to be added 1. Support of two SRS resource sets with usage set to 'codebook' 2. Supported number of SRS resources in one SRS resource set 3. supported layer combinations. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Maximum number of layers per TPMI. |
| CATT [9] | 40-6-1 and 40-6-1a: - In RAN1#110-bis e-meeting and RAN1#112 meeting, following agreements have been achieved on PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: Agreement Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: - For 2 PTRS ports, study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM. Agreement When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field.. As it was agreed in Rel-18 that up to 2 PTRS ports for SDM is supported for single-DCI based STxMP PUSCH transmission, it is our view that such capability could be optional for UE based on UE implementation choice. Besides, the interpretations of the 2 bits are stated without ambiguity. It is better to list separate rows for the “2 PTRS ports for SDM scheme”. This applies for both CB and NCB cases. Proposal 8: For 40-6-1 and 40-6-1a, support listing “PTRS ports for SDM scheme” in separate rows. - In RAN1#112-bis e-meeting and RAN1#113 meeting, following agreements have been achieved on the codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP: Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP transmission are interpreted and the SRI/TPMI fields are designed as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. For SRI/TPMI field design, support Alt1: - Alt1: The DCI has two SRI fields and two TPMI fields. The first SRI field and first TPMI field are associated the first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. The second SRI field and second TPMI fields are reserved. - The codepoints 10 indicate SDM transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the second SRS resource set. - FFS: The codepoint 11 is reserved.. There is no difference between the design of DCI format 0_1 and DCI format 0_2 nor the design on TPMI/SRI fields. UE should support the dynamic switching between STxMP SDM and sTRP as it is essential to the operation of STxMP. Thus, it is our view that not to support listing separate rows for“Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP”. This applies for both CB and NCB cases. Proposal 9: For 40-6-1 and 40-6-1a, do not support listing “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” in separate rows. |
| Ericsson [10] | This is the FG for supporting codebook-based STxMP SDM scheme. There are two FFSs, which we address one by one: - “FFS: separate row for “2 PTRS ports for SDM scheme””: We support separate row for 2 PTRS ports for STxMP scheme. There is no need to have separate capabilities for codebook-based and non-codebook-based operation. - “FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP””: For the STxMP SDM (and SFN) scheme, an SRS resource set indicator field is present in the DCI scheduling PUSCH to allow dynamic switching between multi-panel and single-panel transmission. Whether single-panel or multi-panel transmission is preferred depends on the current channel conditions and, hence, in our view, a UE supporting STxMP needs to also support such dynamic switching. Hence, a separate row is not needed. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; - Support of Single-DCI STx2P (Simultaneous Transmission across 2 Panels) with SDM for codebook based PUSCH. - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of SRS antenna ports for each SRS resource in each SRS resource set - Maximum number of layers of each panel for Single-DCI STx2P with SDM - Maximuml total number of layers across both panels for Single-DCI STx2P with SDM; FR2 only; Component 3 candidate values: {1, 2 ,4} Component 4 candidate values: {1, 2 [,4]} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” - Supported S-DCI based codebook PUSCH MIMO with SDM scheme - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in each SRS resource set - Maximuml total number of layers across both panels for Single-DCI STxMP SDM scheme - Support of 2 PTRS ports - Support of the dynamic switching between STxMP SDM scheme and sTRP for DCI formats 0_1/0_2; 2-14; Yes; Single-DCI based STx2P SDM scheme for codebook based PUSCHis not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-1 - Component - Similar as R17 MTRP PUSCH repetition, dynamic switching can be basic feature and separate FG is not needed, - Support of 2 PTRS ports is an optional UE feature. However, existing FG 2-48 can be sufficient. New FG may not be needed. - Similar as R17 MTRP PUSCH repetition, support of two SRS resource sets, supported number of SRS resources in one SRS resource set should be included. - Supported maximal number of layers associated with each SRS resource set should be reported. - In summarize, following components can be included. - Support of STx2P SDM scheme for codebook based PUSCH. - Support of two SRS resource sets with usage set to ‘codebook’. - Supported number of SRS resources in one SRS resource set. Candidate values: {1,2,4} - Supported maximal number of layers associated with one SRS resource set. Candidate values: {1,2} - Prerequisite FG: 2-14 (CB based PUSCH) |
| OPPO [14] |  |
| Samsung [15] | In the previous RAN1 meeting, RAN1 agreed to support up to two PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission and an additional RRC parameter was introduced to indicate the max number of PTRS ports for SDM scheme. Agreement (RAN1#110b-e) Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: - For 2 PTRS ports, study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM scheme Agreement (RAN1#112) When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field. - Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource. FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission Agreement (RAN1#113) Introduce an additional RRC parameter for the max number of PTRS ports for STxMP SDM scheme.. The main difference of two PTRS ports for SDM scheme is each PTRS port is associated with the first TPMI (for only CB PUSCH)/SRI field or second TPMI (for only CB PUSCH)/SRI field. It is similar to Rel-17 PTRS-DMRS association for mTRP TDM PUSCH repetition scheme. However, the followings are different between association rule for STxMP SDM PUSCH and rule for mTRP TDM PUSCH repetition: - For mTRP TDM PUSCH repetition, - When maxRank=2, MSB indicates association between PTRS port(s) and DMRS port(s) corresponding the first TPMI/SRI and LSB indicates association between PTRS port(s) and DMRS port(s) corresponding the second TPMI/SRI. - PTRS port can be associated with any TPMI/SRI, i.e. the certain PTRS port is associated with either the first TPMI/SRI field or second TPMI/SRI field, i.e. PTRS port 0 can be associated with any TPMI/SRI and PTRS port1 can be associated with any TPMI/SRI also. - For STxMP SDM PUSCH - Regardless of maxRank for SDM scheme and the total number of layers for both PUSCH transmission via two panels, MSB indicates association between PTRS port(s) and DMRS port(s) corresponding the first TPMI/SRI and LSB indicates association between PTRS port(s) and DMRS port(s) corresponding the second TPMI/SRI. - There is the fixed rule that PTRS port 0 should be associated with the first TPMI/SRI and PTRS port 1 should be associated with the second TPMI/SRI. As shown above, different UE feature is required to support two PTRS ports for STxMP SDM PUSCH scheme. In the previous RAN1#113 meeting, it is remained whether to make separate FG for “2 PTRS ports for SDM scheme” as below: 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP”. According to the agreement for PTRS in RAN1#110b-e, PTRS ports for STxMP SDM PUSCH can be supported up to two. This means the number of supported PTRS port by UE can be less than two. Therefore, this feature is not the basic component of FG40-6-1 and the FG for 2 PTRS ports for STxMP SDM PUSCH scheme should be separate. Proposal 22: Separate the FG for 2 PTRS ports for STxMP SDM PUSCH. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; 1. Support of single-DCI based SDM STxMP for codebook-based PUSCH 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers associated with one SRS resource set for SDM scheme 5. Supported number of used PUSCH ports associated with one SRS resource set for SDM scheme; 2-14; Yes; N/A; Per FSPC; No; No; N/A; Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not used.; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-1-3; 2 PTRS ports for SDM scheme; Support of 2 PTRS ports for single-DCI based SDM STxMP PUSCH; Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-6-1/40-6-1a First, we don’t see the need to introduce a separate row for “2 PTRS ports for SDM scheme” since there is already a legacy capability (FG 2-48) for reporting whether a UE can support 2 ports of PTRS. If a UE supports both FG 40-6-1/40-6-1a and FG 2-48, it is natural for the UE to support 2 PTRS ports for SDM scheme. Proposal 21: No need to introduce a separate row for “2 PTRS ports for SDM scheme” Meanwhile, in FG 40-6-1 and FG 40-6-1a, we propose to include “The combination of maximal number of layers for single-DCI based STx2P SDM scheme” as the component of FG 40-6-1/40-6-1a, and a UE can report a combination from the candidates {1+1, 1+2, 2+2}. Proposal 22: Include a component “The combination of maximal number of layers for single-DCI based STx2P SDM scheme” in FG 40-6-1 and FG 40-6-1a - Candidate value for the component: {1+1, 1+2, 2+2} 40. NR_MIMO_evo_DL_UL; 40-6-1; Single-DCI based STx2P SDM scheme for PUSCH—codebook; - The combination of maximal number of layers for single-DCI based STx2P SDM scheme; Optional with UE capability Component-1 candidate values: {1+1, 1+2, 2+2}. |

| 40. NR_MIMO_evo_DL_UL | 40-6-1a | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook | FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In RAN1#113, FG 40-6-1 and FG 40-6-1a were agreed for codebook-based SDM and non-codebook-based SDM schemes, respectively. Whether separate rows are needed for “2 PTRS ports for SDM scheme” and “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” are FFS. In our view, a UE can support SDM scheme without necessarily supporting 2 PTRS ports. Thus, a separate row is needed for “2 PTRS ports for SDM scheme”. In addition, sTRP transmission is a basic UE transmission scheme and a UE that supports SDM scheme should also support sTRP scheme. Therefore, a separate row is not needed for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP”. In addition, based on FG 2-47, supporting 1 PTRS port is “Mandatory with capability signalling for FR2” for sTRP transmission from Rel-15. Similarly, we think 1 PTRS port must also be supported for codebook/non-codebook based SDM scheme. Therefore, a new component of 1 PTRS port for codebook/non-codebook based SDM scheme should be added to 40-6-1 and 40-6-1a. Based on the above discussion, we have the following proposal: Proposal 5-1: Regarding FG 40-6-1 and FG 40-6-1a: - Separate row/FG is needed for 2 PTRS ports for codebook-based and non-codebook based SDM scheme - Separate row is not needed for dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP. - Add 1 PTRS port for codebook/non-codebook based SDM scheme as a component of 40-6-1 and 40-6-1a 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “ - Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP - 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-1d; 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-6-1a - FFS1: We recommend a separate row for “2 PTRS ports for SDM scheme” - FFS2: We recommend a separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP”. 40. NR_MIMO_evo_DL_UL; 40-6-1e; Single-DCI based STx2P with 2 PTRS ports for SDM scheme for PUSCH—noncodebook. 40. NR_MIMO_evo_DL_UL; 40-6-1f; Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook. |
| ZTE [4] | - For FG 40-6-1a, - Similar to the above analysis of FG 40-6-1, both the first FFS on PTRS ports number and the second FFS on dynamic switching by DCI between STxMP SDM scheme and sTRP scheme should be removed. - Besides, the components of the number of SRS resource sets, the supported number of SRS resources in one SRS resource set and the maximal number of MIMO layers of PUSCH associated each SRS resource set should be added. - In particular, according to the discussion in R1-2306614, the components of maximum number of simultaneously transmitted SRS resources across two SRS resource sets and maximum number of simultaneously transmitted SRS resources from one SRS resource sets should be added. Furthermore, the candidate values of the first component can include {1, 2, 3, 4}, and the candidate values of the first component can include {1, 2}. - FG 2-15 should be adopted as the prerequisite FG. 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” - Support of single-DCI based STx2P SDM scheme for non-codebook based PUSCH - Support of two SRS resource sets with usage set to 'nonCodebook' - Maximum number of SRS resources in one SRS resource set - Maximum number of MIMO layers of each SRS resource set for NCB PUSCH with SDM scheme - Maximum number of simultaneous transmitted SRS resources at one symbol - Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 2-15. |
| Spreadtrum Communications [5] | For single-DCI based STx2P, the feature “2 PTRS ports for SDM scheme” is only used for SDM scheme and these two features, i.e. 2 PTRS ports and SDM scheme, are tightly coupled. In other word, an UE supporting SDM scheme must support 2 PTRS ports. Thus, the feature “2 PTRS ports for SDM scheme” can be included in the rows of FGs 40-6-1 and 40-6-1a. Proposal 20: Support including “2 PTRS ports for SDM scheme” in 40-6-1 and 40-6-1a. The feature “Dynamic switching by DCI 0_1/0_2 between single-DCI STx2P SDM and sTRP” is common for 40-6-1 and 40-6-1a and can be a separate row. Similarly, the feature “Dynamic switching by DCI 0_1/0_2 between single-DCI STx2P SFN and sTRP” is also common for 40-6-2 and 40-6-2a and can be a separate row. Proposal 21: Support the separate rows for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” for codebook-based PUSCH and non-codebook-based PUSCH. Proposal 22: Support the separate rows for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” for codebook-based PUSCH and non-codebook-based PUSCH. For SDM scheme, the maximum number of layers, two SRS resource sets and the number of SRS resources in one SRS resource set should be included in 40-6-1 and 40-6-1a, where the supported maximum number of layers can be per UE panel or the layer combination. For SFN scheme, the maximum number of layers, two SRS resource sets and the number of SRS resources in one SRS resource set should be included in 40-6-2 and 40-6-2a. Proposal 23: Support including the following features in 40-6-1 and 40-6-1a: supported maximum number of layers associated with one SRS resource set/supported layer combinations, two SRS resource sets, the number of SRS resources in one SRS resource set. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” Comments: same as for 40-6-1. Additional components to be added 1. Support of two SRS resource sets with usage set to 'nonCodebook' 2. Supported number of SRS resources in one SRS resource set 3. supported layer combinations. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-6-1 and 40-6-1a: - In RAN1#110-bis e-meeting and RAN1#112 meeting, following agreements have been achieved on PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: Agreement Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: - For 2 PTRS ports, study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM. Agreement When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field.. As it was agreed in Rel-18 that up to 2 PTRS ports for SDM is supported for single-DCI based STxMP PUSCH transmission, it is our view that such capability could be optional for UE based on UE implementation choice. Besides, the interpretations of the 2 bits are stated without ambiguity. It is better to list separate rows for the “2 PTRS ports for SDM scheme”. This applies for both CB and NCB cases. Proposal 8: For 40-6-1 and 40-6-1a, support listing “PTRS ports for SDM scheme” in separate rows. - In RAN1#112-bis e-meeting and RAN1#113 meeting, following agreements have been achieved on the codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP: Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP transmission are interpreted and the SRI/TPMI fields are designed as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. For SRI/TPMI field design, support Alt1: - Alt1: The DCI has two SRI fields and two TPMI fields. The first SRI field and first TPMI field are associated the first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. The second SRI field and second TPMI fields are reserved. - The codepoints 10 indicate SDM transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the second SRS resource set. - FFS: The codepoint 11 is reserved.. There is no difference between the design of DCI format 0_1 and DCI format 0_2 nor the design on TPMI/SRI fields. UE should support the dynamic switching between STxMP SDM and sTRP as it is essential to the operation of STxMP. Thus, it is our view that not to support listing separate rows for“Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP”. This applies for both CB and NCB cases. Proposal 9: For 40-6-1 and 40-6-1a, do not support listing “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” in separate rows. |
| Ericsson [10] | This is the FG for supporting non-codebook-based STxMP SDM scheme. There are two FFSs, which we address one by one: - “FFS: separate row for “2 PTRS ports for SDM scheme””: See above, we support separate row. - “FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP””: For the same reason as for codebook-based STxMP SDM, we don’t think a separate row is needed for non-codebook-based STxMP SDM. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; - Support of Single-DCI STx2P (Simultaneous Transmission across Multiple Panel) with SDM for noncodebook based PUSCH. - Support of two SRS resource sets with usage set to 'nonCodebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of simultaneous transmitted SRS at one symbol per SRS resource set - Maximum number of layers of each panel for Single-DCI STx2P with SDM - Maximuml total number of layers across both panels for Single-DCI STx2P with SDM; FR2 only; Component 3 candidate values: {1, 2 ,3, 4} Component 4 candidate values: {1, 2, 3, 4} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” - Supported S-DCI based non-codebook based PUSCH MIMO with SDM scheme - Support of two SRS resource sets with usage set to 'non-codebook' - Maximum number of simultaneous transmitted SRS resources at one symbol - Maximuml total number of layers across both panels for Single-DCI STxMP SDM scheme - Support of 2 PTRS ports - Support of the dynamic switching between STxMP SDM scheme and sTRP for DCI formats 0_1/0_2; 2-15; Yes; Single-DCI based STx2P SDM scheme for non-codebook based PUSCHis not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-1a - Component: Similar as 40-6-1, following components can be included. - Support of STx2P SDM scheme for non-codebook based PUSCH. - Support of two SRS resource sets with usage set to “non-codebook” - Supported number of SRS resources in one SRS resource set. Candidate values: {1,2,3,4} - Supported maximal number of layers associated with one SRS resource set. Candidate values: {1,2} - Prerequisite FG: 2-15 (NCB based PUSCH). |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; 1. Support of single-DCI based SDM STxMP for non-codebook-based PUSCH 2. Support of two SRS resource sets with usage set to 'nonCodebook' 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers associated with one SRS resource set for SDM scheme; 2-15; Yes; N/A; Per FSPC; No; No; N/A; Component 3 candidate values: {1,2,4} Component 4 candidate values: {1,2}; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-6-1/40-6-1a First, we don’t see the need to introduce a separate row for “2 PTRS ports for SDM scheme” since there is already a legacy capability (FG 2-48) for reporting whether a UE can support 2 ports of PTRS. If a UE supports both FG 40-6-1/40-6-1a and FG 2-48, it is natural for the UE to support 2 PTRS ports for SDM scheme. Proposal 21: No need to introduce a separate row for “2 PTRS ports for SDM scheme” Meanwhile, in FG 40-6-1 and FG 40-6-1a, we propose to include “The combination of maximal number of layers for single-DCI based STx2P SDM scheme” as the component of FG 40-6-1/40-6-1a, and a UE can report a combination from the candidates {1+1, 1+2, 2+2}. Proposal 22: Include a component “The combination of maximal number of layers for single-DCI based STx2P SDM scheme” in FG 40-6-1 and FG 40-6-1a - Candidate value for the component: {1+1, 1+2, 2+2} 40. NR_MIMO_evo_DL_UL; 40-6-1a; Single-DCI based STx2P SDM scheme for PUSCH—noncodebook; - The combination of maximal number of layers for single-DCI based STx2P SDM scheme; Optional with UE capability Component-1 candidate values: {1+1, 1+2, 2+2}. |

| 40. NR_MIMO_evo_DL_UL | 40-6-1b | Associated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-6-1b, - Basically, this FG is needed due to two SRS resource sets are introduced for single-DCI based STxMP SDM scheme for NCB based PUSCH transmission. - Note that this feature is technically similar to FG 23-3-1-2a for Rel-17 MTRP TDM PUSCH repetition, FG 23-3-1-2a can be taken as the reference of this FG. Furthermore, the newly suggested FG 40-6-1c should be added to keep alignment with FG 23-3-1-2b. - FG 2-15a and FG 40-6-1a should be adopted as the prerequisite FGs. 40-6-1b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH; Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH; 2-15a, 40-6-1a. 40-6-1c; CSI-RS processing framework for SRS with two associated CSI-RS resources; - Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 4. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 5. UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously; 40-6-1b. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH; Comments: Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based sTx2p should be considered.. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This FG supports associating CSI-RS resources with SRS for non-codebook-based STxMP SDM scheme (FG 2-15 supports associating a CSI-RS resource with SRS for non-codebook-based sTRP PUSCH). FG 40-6-1a should be the prerequisite for this FG. 40-6-1b; Association between CSI-RS resources and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH; - Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook single-DCI based STx2P SDM scheme for PUSCH - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.; FR2 only; Component 2 candidate value: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: { 2 to 256}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-1c; Dynamic switching between Single-DCI STx2P SDM scheme and Single-TRP scheme - codebook based; Support of dynamic switching between Single-DCI STx2P SDM scheme and Single-TRP scheme for codebook based PUSCH by DCI formats 0_1/0_2; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-1d; Dynamic switching between Single-DCI STx2P SDM scheme and Single-TRP scheme - non-codebook based; Support of dynamic switching between Single-DCI STx2P SDM scheme and Single-TRP scheme for non-codebook based PUSCH by DCI formats 0_1/0_2; FR2 only; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH; Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH; 2-15a; Yes; Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-1b - Component: following components can be included. - Support of up to two CSI-RS resources associated with two SRS resource sets - Prerequisite FG: 40-6-1a, 2-15a (Association between CSI-RS and SRS) |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-1b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based SDM scheme 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; Yes; N/A; Per band; No; No; N/A; Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2}; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-6-1b/40-6-2b/40-6-3b-1 Similar UE capability (FG 23-3-1-2a) has been introduced for Rel-17 Multi-TRP PUSCH repetition. Thus, we think it can be also introduced for each Rel-18 STxMP scheme. However, current wording for these FGs is a bit unclear, and we suggest the following changes: Proposal 23: Support FG 40-6-1b/40-6-2b/40-6-3b-1 with the following changes: - FG 40-6-1b: Two Aassociated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH - FG 40-6-2b: Two Aassociated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH - FG 40-6-3b-1: Two Aassociated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH 40. NR_MIMO_evo_DL_UL; 40-6-1b; Two Aassociated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH. |

| 40. NR_MIMO_evo_DL_UL | 40-6-1-2 | New DMRS port entry for single-DCI based SDM scheme |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-6-1-2, - One row should be added to point out the exact new DMRS port entry {0, 2, 3} as agreed in AI 9.1.4.1. - FG 40-6-1 should be adopted as the prerequisite FG 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; Support of new DMRS port entry {0, 2, 3}; 40-6-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | In RAN1#112, it was agreed to support new DMRS port entry for STxMP SDM scheme as a UE-optional feature, which motivates the need for this FG. 40-6-1-2; New DMRS port entry for single-DCI based STx2P SDM scheme. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; - Support of DMRS ports {0, 2, 3} for Single-DCI STxMP SDM scheme.; FR2 only; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; Support of new DMRS port entry {0, 2, 3}; 40-6-1,40-6-1a; Yes; New DMRS port entry for single-DCI based SDM scheme is not supported; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-1-2 - Component: following components can be included. - Support new DMRS port entry {0,2,3} - Prerequisite FG: 40-6-1 or 40-6-1a |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-1-2; New DMRS port entry for single-DCI based SDM scheme; Support of new DMRS port entry {0,2,3} for single-DCI based SDM STxMP PUSCH; Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-6-2 | Single-DCI based STx2P SFN scheme for PUSCH—codebook | FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | FG 40-6-2, and FG 40-6-2a were agreed for codebook-based SFN and non-codebook-based SFN scheme, respectively. Similar to the SDM scheme, a separate row is not needed for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP”, and 1 PTRS port for codebook/non-codebook based SFN scheme should be added to 40-6-2 and 40-6-2a. Also, a UE can support SFN scheme without necessarily supporting 2 PTRS ports. Therefore, 2 new FGs should be defined for the support of 2 PTRS ports for single-DCI based STx2P SFN scheme for CB-based and NCB-based PUSCH. Based on the above discussion, we have the following proposal: Proposal 5-1: Regarding FG 40-6-2 and FG 40-6-2a: - Separate row/FG is needed for 2 PTRS ports for codebook-based and non-codebook based SFN scheme - Separate row is not needed for dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP. - Add 1 PTRS port for codebook/non-codebook based SFN scheme as a component of 40-6-2 and 40-6-2a 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; - FFS: separate row for Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP - 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-2c; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-6-2 - FFS: We recommend a separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP”. 40. NR_MIMO_evo_DL_UL; 40-6-2c; Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP for PUSCH—codebook. |
| ZTE [4] | - For FG 40-6-2, - Similarly, dynamic switching by DCI between STxMP SFN scheme and sTRP scheme is the inherence of Rel-18 STxMP SDM scheme, it is not needed to take this as a separate row of this FG as Rel-17 MTRP TDM PUSCH repetition. - Besides, the components of the number of SRS resource sets, the supported number of SRS resources in one SRS resource set and the maximal number of MIMO layers of PUSCH associated each SRS resource set should be added. - FG 2-14 should be adopted as the prerequisite FG. 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” - Support of single-DCI based STx2P SFN scheme for codebook based PUSCH - Support of two SRS resource sets with usage set to 'codebook' - Supported number of SRS resources in one SRS resource set - Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SDM scheme; 2-14. |
| Spreadtrum Communications [5] | Proposal 24: Support including the following features in 40-6-2 and 40-6-2a: supported maximum number of layers, two SRS resource sets, the number of SRS resources in one SRS resource set. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” Comments: it should be the mandatory feature. Additional components to be added 1. Support of two SRS resource sets with usage set to 'Codebook' 2. Supported number of SRS resources in one SRS resource set 3. supported maximum number of layers. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Maximum number of layers per TPMI. |
| CATT [9] | 40-6-2 and 40-6-2a: - In RAN1#112bis e-meeting, following agreement has been achieved on he codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP: Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP transmission are interpreted and the design of SRI/TPMI fields are as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. - For the design of SRI/TPMI fields, re-use the design that is decided for dynamic switching between STxMP SDM and sTRP transmission. - The codepoint 10 indicates STxMP SFN transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the second SRS resource set and they indicate the precoder(s)/SRI for the second SRS resource set (the rank is indicated by the first SRI field for NCB or the first TPMI field for CB) - FFS: The codepoint 11 is reserved.. Same as the statement for SDM cases, the design for dynamic switching between SFN and sTRP and the interpretations for codepoints between TPMI field and SRI field are the same. It is necessary for UE to support such feature. This applies for both CB and NCB cases. Proposal 10: For 40-6-2 and 40-6-2a, do not support listing “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” in separate rows. |
| Ericsson [10] | This is the FG for supporting codebook-based STxMP SFN scheme. There is an FFS: - “FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP””: For the same reason as for codebook-based STxMP SDM, we don’t think a separate row is needed for codebook-based STxMP SFN. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; - Support of Single-DCI STx2P (Simultaneous Transmission across 2 Panels) with SFN for codebook based PUSCH. - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of SRS antenna ports for each SRS resource in each SRS resource set - Maximum number of layers for Single-DCI STx2P with SFN; FR2 only; Component 3 candidate values: {1, 2 ,4} Component 4 candidate values: {1, 2 [,4]} Component 5 candidate values: {1, 2} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” - Supported S-DCI based codebook PUSCH MIMO with SFN scheme - Support of two SRS resource sets with usage set to 'Codebook' - Support of the dynamic switching between STxMP SFN scheme and sTRP for DCI formats 0_1/0_2 - Maximum number of SRS resources in each SRS resource set - Maximum number of layers for Single-DCI STxMP SFN scheme; 2-14; Yes; Single-DCI based STx2P SFN scheme for codebook based PUSCHis not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-2 - Component: Similar as 40-6-1, following components can be included. - Support of STx2P SFN scheme for codebook based PUSCH. - Support of two SRS resource sets with usage set to ‘codebook’. - Supported number of SRS resources in one SRS resource set. Candidate values: {1,2,4} - Supported maximal number of layers associated with one SRS resource set. Candidate values: {1,2} - Prerequisite FG: 2-14 |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-2; Single-DCI based STx2P SFN scheme for PUSCH—codebook; 1. Support of single-DCI based SFN STxMP for codebook-based PUSCH 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers associated with one SRS resource set for SFN scheme 5. Supported number of used PUSCH ports associated with one SRS resource set for SFN scheme; 2-14; Yes; N/A; Per FSPC; No; No; N/A; Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not used.; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook | FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | FG 40-6-2, and FG 40-6-2a were agreed for codebook-based SFN and non-codebook-based SFN scheme, respectively. Similar to the SDM scheme, a separate row is not needed for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP”, and 1 PTRS port for codebook/non-codebook based SFN scheme should be added to 40-6-2 and 40-6-2a. Also, a UE can support SFN scheme without necessarily supporting 2 PTRS ports. Therefore, 2 new FGs should be defined for the support of 2 PTRS ports for single-DCI based STx2P SFN scheme for CB-based and NCB-based PUSCH. Based on the above discussion, we have the following proposal: Proposal 5-1: Regarding FG 40-6-2 and FG 40-6-2a: - Separate row/FG is needed for 2 PTRS ports for codebook-based and non-codebook based SFN scheme - Separate row is not needed for dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP. - Add 1 PTRS port for codebook/non-codebook based SFN scheme as a component of 40-6-2 and 40-6-2a 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; - FFS: separate row for Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP - 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-2d; 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-6-2a - FFS: We recommend a separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP”. 40. NR_MIMO_evo_DL_UL; 40-6-2d; Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP for PUSCH—noncodebook. |
| ZTE [4] | - For FG 40-6-2a, - Similar to FG 40-6-2, it is not needed to take this as a separate row of this FG as Rel-17 MTRP TDM PUSCH repetition. - Besides, the components of the number of SRS resource sets, the supported number of SRS resources in one SRS resource set and the maximal number of MIMO layers of PUSCH associated each SRS resource set should be added. - In particular, the components of maximum number of simultaneously transmitted SRS resources across two SRS resource sets and maximum number of simultaneously transmitted SRS resources from one SRS resource sets should be added. Furthermore, the candidate values of the first component can include {1, 2, 3, 4}, and the candidate values of the first component can include {1, 2}. - FG 2-14 should be adopted as the prerequisite FG. 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” - Support of single-DCI based STx2P SDM scheme for non-codebook based PUSCH - Support of two SRS resource sets with usage set to 'nonCodebook' - Supported number of SRS resources in one SRS resource set - Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SDM scheme - Maximum number of simultaneous transmitted SRS resources at one symbol - Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 2-15. |
| Spreadtrum Communications [5] | Proposal 24: Support including the following features in 40-6-2 and 40-6-2a: supported maximum number of layers, two SRS resource sets, the number of SRS resources in one SRS resource set. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” Comments: it should be the mandatory feature. Additional components to be added 1. Support of two SRS resource sets with usage set to 'nonCodebook' 2. Supported number of SRS resources in one SRS resource set 3. supported maximum number of layers. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1. Maximum number of SRS resources per SRI(s). |
| CATT [9] | 40-6-2 and 40-6-2a: - In RAN1#112bis e-meeting, following agreement has been achieved on he codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP: Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP transmission are interpreted and the design of SRI/TPMI fields are as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. - For the design of SRI/TPMI fields, re-use the design that is decided for dynamic switching between STxMP SDM and sTRP transmission. - The codepoint 10 indicates STxMP SFN transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the second SRS resource set and they indicate the precoder(s)/SRI for the second SRS resource set (the rank is indicated by the first SRI field for NCB or the first TPMI field for CB) - FFS: The codepoint 11 is reserved.. Same as the statement for SDM cases, the design for dynamic switching between SFN and sTRP and the interpretations for codepoints between TPMI field and SRI field are the same. It is necessary for UE to support such feature. This applies for both CB and NCB cases. Proposal 10: For 40-6-2 and 40-6-2a, do not support listing “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” in separate rows. |
| Ericsson [10] | This is the FG for supporting non-codebook-based STxMP SFN scheme. There is an FFS: - “FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP””: For the same reason as for non-codebook-based STxMP SDM, we don’t think a separate row is needed for non-codebook-based STxMP SFN. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; - Support of Single-DCI STx2P (Simultaneous Transmission across 2 Panels) with SFN for noncodebook based PUSCH. - Support of two SRS resource sets with usage set to 'nonCodebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of simultaneous transmitted SRS at one symbol per SRS resource set Maximum number of layers for Single-DCI STx2P with SFN; FR2 only; Component 3 candidate values: {1, 2 ,3, 4} Component 4 candidate values: {1, 2, 3, 4} Component 5 candidate values: {1, 2}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” - Supported S-DCI based codebook PUSCH MIMO with SFN scheme - Support of two SRS resource sets with usage set to 'nonCodebook' - Maximum number of simultaneous transmitted SRS resources at one symbol - Support of the dynamic switching between STxMP SFN scheme and sTRP for DCI formats 0_1/0_2; 2-15; Yes; Single-DCI based STx2P SFN scheme for non-codebook based PUSCHis not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-2a - Component: Similar as 40-6-1a, following components can be included. - Support of STx2P SFN scheme for non-codebook based PUSCH. - Support of two SRS resource sets with usage set to “non-codebook” - Supported number of SRS resources in one SRS resource set. Candidate values: {1,2,3,4} - Supported maximal number of layers associated with one SRS resource set. Candidate values: {1,2} - Prerequisite FG: 2-15. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-2a; Single-DCI based STx2P SFN scheme for PUSCH—noncodebook; 1. Support of single-DCI based SFN STxMP for non-codebook-based PUSCH 2. Support of two SRS resource sets with usage set to 'nonCodebook' 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers associated with one SRS resource set for SFN scheme; 2-15; Yes; N/A; Per FSPC; No; No; N/A; Component 3 candidate values: {1,2,4} Component 4 candidate values: {1,2}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-6-2b | Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-6-2b, - Basically, this FG is needed due to two SRS resource sets are introduced for single-DCI based STxMP SFN scheme for NCB based PUSCH transmission. - Note that this feature is technically similar to FG 23-3-1-2a for Rel-17 MTRP TDM PUSCH repetition, then FG 23-3-1-2a can be taken as the reference of this FG. Furthermore, the newly suggested FG 40-6-2c should be added to keep alignment with FG 23-3-1-2b. - FG 2-15a and FG 40-6-2a should be adopted as the prerequisite FGs. 40-6-2b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH; Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH; 2-15a, 40-6-2a. 40-6-2c; CSI-RS processing framework for SRS with two associated CSI-RS resources; - Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP - Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP - Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP - UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS - UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously; 40-6-2b. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH; Comments: Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based STx2P should be considered.. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This is the equivalent of FG 40-6-1-2 for non-codebook-based STxMP SFN scheme. FG 40-6-2a should be the prerequisite for this FG. 40-6-2b; Association between CSI-RS resources and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH; - Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook single-DCI based STx2P SFN scheme for PUSCH - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.; FR2 only; Component 2 candidate value: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: { 2 to 256}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH; Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH; 2-15a; Yes; Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-2b - Component: Similar as 40-6-1b, following components can be included. - Support of up to two CSI-RS resources associated with two SRS resource sets - Prerequisite FG: 40-6-1a, 2-15a |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-2b; Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based SFN scheme 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; Yes; N/A; Per band; No; No; N/A; Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2}; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-6-1b/40-6-2b/40-6-3b-1 Similar UE capability (FG 23-3-1-2a) has been introduced for Rel-17 Multi-TRP PUSCH repetition. Thus, we think it can be also introduced for each Rel-18 STxMP scheme. However, current wording for these FGs is a bit unclear, and we suggest the following changes: Proposal 23: Support FG 40-6-1b/40-6-2b/40-6-3b-1 with the following changes: - FG 40-6-1b: Two Aassociated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH - FG 40-6-2b: Two Aassociated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH - FG 40-6-3b-1: Two Aassociated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH 40. NR_MIMO_evo_DL_UL; 40-6-2b; Two Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3a | codebook multi-DCI based STx2P PUSCH+PUSCH | FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Regarding FG 40-6-3a, e.g., codebook multi-DCI based STx2P PUSCH+PUSCH, separate rows for DG-PUSCH+DG-PUSCH, CG-PUSCH +CG-PUSCH, DG-PUSCH +CG-PUSCH are needed since the requirement is different for UE implementation. Similarly, separate rows for DG-PUSCH+DG-PUSCH, CG-PUSCH +CG-PUSCH, DG-PUSCH +CG-PUSCH are needed in FG 40-6-3b. Regarding the overlapping in frequency and time of FG 40-6-3a and FG 40-6-3b, similar to FG 16-2a, support of fully/partial overlapping PUSCH in time and non-overlapping in frequency should remain as components of FG 40-6-3a and FG 40-6-3b. In addition, similar to FG 16-2a-0, separate row is needed for supporting fully overlapping PUSCH in time and frequency and similar to FG 16-2a-1, separate row is needed for supporting partial overlapping PUSCH in time and frequency. Proposal 5-2: Regarding FG 40-6-3a and FG 40-6-3b: - Support separate rows for DG-PUSCH+DG-PUSCH, CG-PUSCH +CG-PUSCH, DG-PUSCH +CG-PUSCH - Support fully/partial overlapping PUSCH in time and non-overlapping in frequency as a component of each of the above three FGs; - Separate row is needed for supporting fully overlapping PUSCH in time and frequency; - Separate row is needed for supporting partial overlapping PUSCH in time and frequency. 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P DG-PUSCH+DG-PUSCH; - Support for fully/partial overlapping PUSCH in time and non-overlapping in frequency FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time partially or non-overlapping in frequency, partially or fully overlapping in time; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3a-2; codebook multi-DCI based STx2P CG-PUSCH+CG-PUSCH; - Support for fully/partial overlapping PUSCH in time and non-overlapping in frequency; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3a-3; codebook multi-DCI based STx2P CG-PUSCH+DG-PUSCH; - Support for fully/partial overlapping PUSCH in time and non-overlapping in frequency; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3a-4; codebook multi-DCI based STx2P PUSCH+PUSCH with fully overlapping PUSCH in time and frequency; At least one of 40-6-3a, 40-6-3a-2, or 40-6-3a-3; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3a-5; codebook multi-DCI based STx2P PUSCH+PUSCH with partial overlapping PUSCH in time and frequency; At least one of 40-6-3a, 40-6-3a-2, or 40-6-3a-3; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-6-3a - FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH - may be addressed in the future meetings - FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time - decisions pending from WI, to be addressed later 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time. |
| ZTE [4] | - For FG 40-6-3a, - For the first FFS part on the grant type combination between DG-PUSCH and CG-PUSCH, its necessity is agnostic. Besides, there is no agreement reached so far on supporting to take this feature as UE optional. Hence, this should be removed. - For the second FFS part on the overlapping type of STxMP PUSCH+PUSCH, it is sufficient to generally specify that two PUSCHs are fully/partially-overlapped in time domain and fully/partially/non-overlapped in frequency domain, due to it is over-demanded to gNB to take care of the overlapping level especially for non-ideal backhaul assumption of MDCI MTRP operation. - In addition, the components of the number of SRS resource sets, the supported number of SRS resources in one SRS resource set and the maximal number of MIMO layers of PUSCH associated each SRS resource set should be added. - FG 2-14 and FG 16-2a should be adopted as the prerequisite FGs. 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time - Support of multi-DCI based STx2P PUSCH+PUSCH in CB scheme - Support of STx2P PUSCH+PUSCH are fully/partially-overlapped in time domain and fully/partially/non-overlapped in frequency domain - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in one SRS resource set - Maximum number of MIMO layers of each SRS resource set for CB PUSCH with SDM scheme; 16-2a, 2-14. |
| Spreadtrum Communications [5] | In the FGs for multi-DCI based PUSCH+PUSCH, the supported maximum number of layers for each of two PUSCHs can be included in 40-6-3a and 40-6-3b. Proposal 25: Support including the supported maximum number of layers for each of two PUSCHs in 40-6-3a and 40-6-3b. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH Comments: OK with separate rows, DG-PUSCH+DG-PUSCH should be basic feature. FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time Comments: no need to signal. Additional components to be added 1. Support of two SRS resource sets with usage set to 'Codebook' 2. Supported number of SRS resources in one SRS resource set. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH Supports one or more of the following choices: a)fully/partially overlapping PUSCHs in time and non-overlapping in frequency b)fully overlapping in both time and frequency c)partially or non-overlapping in frequency, partially or fully overlapping in time. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; 1. Maximum number of layers per PUSCH 2. Maximum number of layers across the overlapped PUSCHs. |
| CATT [9] | 40-6-3a and 40-6-3b: - In Rel-17, support of CG PUSCH transmission towards multi-TRPs using a single CG configuration is not mandatory for the UE supporting multi-TRP PUSCH repetition. Similarly, DG PUSCH + DG PUSCH can be mandatory feature of multi-DCI based STx2P PUSCH+PUSCH, and DG PUSCH + CG PUSCH and CG PUSCH + CG PUSCH can be optional feature. - In Rel-16, support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency is basic UE feature of multi-DCI based multi-TRP feature, and support of fully overlapping and partially overlapping in frequency and time are sub-features of multi-DCI based multi-TRP feature. Different from PDSCH reception, transmitting PUSCHs in an overlapping manner or in a non-overlapping require similar UE capabilities. Therefore, we propose that different overlapping cases are basic features of 40-6-3a or 40-6-3b, and no other separate features are needed. With regarding to the wording of overlapping cases, “partial-overlapping PUSCHs in time and non-overlapping in frequency” is included in the case of “partially or non-overlapping in frequency, partially or fully overlapping in time”. Besides, the description of “overlapping PUSCHs in time” is unclear. Therefore, we propose to reword the overlapping cases. Proposal 11: For 40-6-3a and 40-6-3b, support of DG-PUSCH+CG-PUSCH and support of CG-PUSCH+CG-PUSCH are listed in separate rows. Proposal 12: For 40-6-3a and 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; 1.Support of DG-PUSCH+DG-PUSCH 2. Support of fully/partially overlapping PUSCHs in time and fully/partially/non-overlapping in frequency FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time. |
| Ericsson [10] | This is the FG for supporting codebook based multi-DCI STxMP, There are some FFSs that needs to be addressed: “FFS: separate rows for DG-PUSCH+CG-PUSCH, CG-PUSCH+CG-PUSCH, DG-PUSCH+DG-PUSCH”: Separate rows are not needed: a UE that supports 40-6-3a and CG will support all three combinations. “FFS: how to signal, Partial-overlapping PUSCHs in time and non-overlapping in frequency, Overlapping PUSCHs in time and partially overlapping in frequency, fully overlapping in both frequency and time, fully overlapping in frequency, partially overlapping in time, partially or non-overlapping in frequency, partially or fully overlapping in time”. Here we propose to use the same three options as for PDSCH overlap for multi-DCI multi-TRP (FG 16-2a-x): partial/ full overlap in time + no overlap in frequency, full overlap in time + full overlap in frequency and partial overlap in time + partial overlap in frequency. 40-6-3a; Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG, CG+DG, CG+CG; - Support full/partial overlap in time and no overlap in frequency - Support full overlap in time and full overlap in frequency Support partial overlap in time and partial overlap in frequency. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; - Support of Multi-DCI STx2P (Simultaneous Transmission across 2 Panels) for codebook based PUSCH with fully overlapping in both frequency and time. - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of SRS antenna ports for each SRS resource in each SRS resource set - Maximum number of layers of each overlapping PUSCH - Maximuml total number of layers across two overlapping PUSCH; FR2 only; Component 1 candidate values: a combination from the set {DG PUSCH, Type I CG PUSCH, Type II CG PUSCH} Component 3 candidate values: {1, 2 ,4} Component 4 candidate values: {1, 2 [,4]} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time partially or non-overlapping in frequency, partially or fully overlapping in time - Supported M-DCI based codebook PUSCH+PUSCH; 2-14; Yes; Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-3a - Component: Similar components as 40-6-1 should be included. DG+CG and CG+CG should be separate FG, since they can be supported only if CG is supported. Following components can be included. - Support of multi-DCI based STx2P PUSCH+PUSCH for codebook based PUSCH. - Support of two SRS resource sets with usage set to ‘codebook’. - Supported number of SRS resources in one SRS resource set. Candidate values: {1,2,4} - Supported maximal number of layers associated with one SRS resource set. Candidate values: {1,2} - Prerequisite FG: 2-14 |
| OPPO [14] |  |
| Samsung [15] | For multi-DCI based STxMP PUSCH scheme, following overlapping cases between scheduling types for PUSCH can be supported: Agreement (RAN1#110b-e) Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations: - DG-PUSCH + DG-PUSCH - CG-PUSCH + DG-PUSCH Agreement (RAN1#111) Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission.. In the previous RAN1 meeting, whether to separate DG+CG and CG+CG from FG 40-6-3a and 40-6-3b was remained as FFS. In our view, DG-PUSCH + DG-PUSCH is only basic feature of FG 40-6-3a and 40-6-3b and others are optional features because additional capabilities to manage additional configurations for STxMP CG PUSCH may be required. Therefore, RAN1 needs to define separate FG for ‘DG-PUSCH+CG-PUSCH’ and ‘CG-PUSCH+CG-PUSCH’. For the separate FG, ‘DG-PUSCH+CG-PUSCH’ and ‘CG-PUSCH+CG-PUSCH’ can be candidate values, for example {DG+CG, CG+CG, both}. Proposal 23: Define a separate FG for ‘DG-PUSCH+CG-PUSCH’ and ‘CG-PUSCH+CG-PUSCH’ and ‘DG-PUSCH+CG-PUSCH’ and ‘CG-PUSCH+CG-PUSCH’ can be candidate values for newly defined FG, for example {DG+CG, CG+CG, both}. If the UE can support multi-DCI based STxMP PUSCH scheme, two PUSCHs which are overlapped in time domain can be scheduled to the UE. RAN1 discussed possible overlapping cases and the following agreement were endorsed: Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain. In our view, some of overlapping case can require further enhanced UE capability, e.g. the case with partially overlapping in time domain. If two PUSCHs are partially overlapped in time domain, it can effect that the UE determines uplink power control (total transmit power or each transmit power per panel etc.) and prepares two overlapped PUSCHs. On the other hand, we think the fully overlapping case in time domain can be kept in main FG 40-6-3a and 40-6-4b because less capability can be required than partially overlapping cases. Therefore, we suggest making separate a FG to support two partially overlapping PUSCHs in time domain. Proposal 24: Separate a FG to support two partially overlapping PUSCHs in time domain |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for codebook-based PUSCH: Support fully-overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolInde values 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Supported number of used PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain 6. Maximum number of PUSCHs per CORESETPoolIndex per slot; 2-14; Yes; N/A; Per FSPC; No; No; N/A; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not used. Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-6-3a/40-6-3b First, we don’t see the need to introduce separate rows for “DG-PUSCH+CG-PUSCH” and “CG-PUSCH+CG-PUSCH”, i.e., these combinations should be baseline as “DG-PUSCH+DG-PUSCH”. Proposal 24: No need to introduce separate rows for “DG-PUSCH+CG-PUSCH” and “CG-PUSCH+CG-PUSCH” Second, we don’t see the need to signal the UE can support what types of overlapping in time/frequency domain, i.e., any type of overlapping/non-overlapping should be supported as baseline. There is no such capability for Rel-16 M-DCI based PDSCH+PDSCH. Proposal 25: No need to signal the followings, i.e., any type of overlapping/non-overlapping should be supported as baseline - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - Fully overlapping in both frequency and time - Fully overlapping in frequency, partially overlapping in time - Partially or non-overlapping in frequency, partially or fully overlapping in time Meanwhile, we propose to include the component “The combination of maximal number of layers for each PUSCH when two PUSCHs are overlapped in at least one OFDM symbol” in FG 40-6-3a and FG 40-6-3b, and a UE can report a combination from the candidates {1+1, 1+2, 2+2}. Note that for the case if two PUSCHs are not overlapped in time domain, legacy capability XX-XX is used for the maximal number of layers for each PUSCH Proposal 26: Include a component “The combination of maximal number of layers for each PUSCH when two PUSCHs are overlapped in at least one OFDM symbol” in FG 40-6-3a and FG 40-6-3b - Candidate value for the component: {1+1, 1+2, 2+2} - Note: For the case if two PUSCHs are not overlapped in time domain, legacy capability FG 2-14/2-15 is used for the maximal number of layers for each PUSCH 40. NR_MIMO_evo_DL_UL; 40-6-3a; codebook multi-DCI based STx2P PUSCH+PUSCH; - The combination of maximal number of layers for each PUSCH when two PUSCHs are overlapped in at least one OFDM symbol; For the case if two PUSCHs are not overlapped in time domain, legacy capability FG 2-14 is used for the maximal number of layers for each PUSCH; Optional with UE capability Component-1 candidate values: {1+1, 1+2, 2+2}. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3b | noncodebook multi-DCI based STx2P PUSCH+PUSCH | FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time partially or non-overlapping in frequency, partially or fully overlapping in time |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Regarding FG 40-6-3a, e.g., codebook multi-DCI based STx2P PUSCH+PUSCH, separate rows for DG-PUSCH+DG-PUSCH, CG-PUSCH +CG-PUSCH, DG-PUSCH +CG-PUSCH are needed since the requirement is different for UE implementation. Similarly, separate rows for DG-PUSCH+DG-PUSCH, CG-PUSCH +CG-PUSCH, DG-PUSCH +CG-PUSCH are needed in FG 40-6-3b. Regarding the overlapping in frequency and time of FG 40-6-3a and FG 40-6-3b, similar to FG 16-2a, support of fully/partial overlapping PUSCH in time and non-overlapping in frequency should remain as components of FG 40-6-3a and FG 40-6-3b. In addition, similar to FG 16-2a-0, separate row is needed for supporting fully overlapping PUSCH in time and frequency and similar to FG 16-2a-1, separate row is needed for supporting partial overlapping PUSCH in time and frequency. Proposal 5-2: Regarding FG 40-6-3a and FG 40-6-3b: - Support separate rows for DG-PUSCH+DG-PUSCH, CG-PUSCH +CG-PUSCH, DG-PUSCH +CG-PUSCH - Support fully/partial overlapping PUSCH in time and non-overlapping in frequency as a component of each of the above three FGs; - Separate row is needed for supporting fully overlapping PUSCH in time and frequency; - Separate row is needed for supporting partial overlapping PUSCH in time and frequency. 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P DG-PUSCH+DG-PUSCH; - Support for fully/partial overlapping PUSCH in time and non-overlapping in frequency FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time partially or non-overlapping in frequency, partially or fully overlapping in time; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3b-2; noncodebook multi-DCI based STx2P CG-PUSCH+CG-PUSCH; - Support for fully/partial overlapping PUSCH in time and non-overlapping in frequency; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3b-3; noncodebook multi-DCI based STx2P CG-PUSCH+DG-PUSCH; - Support for fully/partial overlapping PUSCH in time and non-overlapping in frequency; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3b-4; noncodebook multi-DCI based STx2P PUSCH+PUSCH with fully overlapping PUSCH in time and frequency; At least one of 40-6-3b, 40-6-3b-2, or 40-6-3b-3; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-3b-5; noncodebook multi-DCI based STx2P PUSCH+PUSCH with partial overlapping PUSCH in time and frequency; At least one of 40-6-3b, 40-6-3b-2, or 40-6-3b-3; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-6-3b - FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH - may be addressed in the future meetings - FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time - decisions pending from WI, to be addressed later 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time. |
| ZTE [4] | - For FG 40-6-3b, similar to the update of FG 40-6-3a, - The first FFS part should be removed. - For the first FFS part, it is sufficient to generally specify that two PUSCHs are fully/partially-overlapped in time domain and fully/partially/non-overlapped in frequency domain. - In addition, the components of the number of SRS resource sets, the supported number of SRS resources in one SRS resource set and the maximal number of MIMO layers of PUSCH associated each SRS resource set should be added. - FG 2-15 and FG 16-2a should be adopted as the prerequisite FGs. 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time - Support of multi-DCI based STx2P PUSCH+PUSCH in NCB scheme - Support of STx2P PUSCH+PUSCH are fully/partially-overlapped in time domain and fully/partially/non-overlapped in frequency domain - Support of two SRS resource sets with usage set to 'nonCodebook' - Supported number of SRS resources in one SRS resource set - Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SDM scheme - Maximum number of simultaneous transmitted SRS resources at one symbol - Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol; 16-2a, 2-15. |
| Spreadtrum Communications [5] | In the FGs for multi-DCI based PUSCH+PUSCH, the supported maximum number of layers for each of two PUSCHs can be included in 40-6-3a and 40-6-3b. Proposal 25: Support including the supported maximum number of layers for each of two PUSCHs in 40-6-3a and 40-6-3b. |
| Vivo [6] |  |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH Supports one or more of the following choices: a)fully/partially overlapping PUSCHs in time and non-overlapping in frequency b)fully overlapping in both time and frequency c)partially or non-overlapping in frequency, partially or fully overlapping in time. |
| Google [8] |  |
| CATT [9] | 40-6-3a and 40-6-3b: - In Rel-17, support of CG PUSCH transmission towards multi-TRPs using a single CG configuration is not mandatory for the UE supporting multi-TRP PUSCH repetition. Similarly, DG PUSCH + DG PUSCH can be mandatory feature of multi-DCI based STx2P PUSCH+PUSCH, and DG PUSCH + CG PUSCH and CG PUSCH + CG PUSCH can be optional feature. - In Rel-16, support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency is basic UE feature of multi-DCI based multi-TRP feature, and support of fully overlapping and partially overlapping in frequency and time are sub-features of multi-DCI based multi-TRP feature. Different from PDSCH reception, transmitting PUSCHs in an overlapping manner or in a non-overlapping require similar UE capabilities. Therefore, we propose that different overlapping cases are basic features of 40-6-3a or 40-6-3b, and no other separate features are needed. With regarding to the wording of overlapping cases, “partial-overlapping PUSCHs in time and non-overlapping in frequency” is included in the case of “partially or non-overlapping in frequency, partially or fully overlapping in time”. Besides, the description of “overlapping PUSCHs in time” is unclear. Therefore, we propose to reword the overlapping cases. Proposal 11: For 40-6-3a and 40-6-3b, support of DG-PUSCH+CG-PUSCH and support of CG-PUSCH+CG-PUSCH are listed in separate rows. Proposal 12: For 40-6-3a and 40-6-3b, support of fully/partially overlapping PDSCHs in time and fully/partially/non-overlapping in frequency is basic feature. 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; 1.Support of DG-PUSCH+DG-PUSCH 2. Support of fully/partially overlapping PUSCHs in time and fully/partially/non-overlapping in frequency FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - partially or non-overlapping in frequency, partially or fully overlapping in time. |
| Ericsson [10] | This is the FG for supporting non-codebook based multi-DCI STxMP, There are some FFSs that needs to be addressed: “FFS: separate rows for DG-PUSCH+CG-PUSCH, CG-PUSCH+CG-PUSCH, DG-PUSCH+DG-PUSCH”: Separate rows are not needed: a UE that supports 40-6-3b and CG will support all three combinations. “FFS: how to signal, Partial-overlapping PUSCHs in time and non-overlapping in frequency, Overlapping PUSCHs in time and partially overlapping in frequency, fully overlapping in both frequency and time, fully overlapping in frequency, partially overlapping in time, partially or non-overlapping in frequency, partially or fully overlapping in time”. Here we propose to use the same three options as for PDSCH overlap for multi-DCI multi-TRP (FG 16-2a-x): partial/ full overlap in time + no overlap in frequency, full overlap in time + full overlap in frequency and partial overlap in time + partial overlap in frequency. 40-6-3b; Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG, CG+DG, CG+CG; - Support full/partial overlap in time and no overlap in frequency - Support full overlap in time and full overlap in frequency Support partial overlap in time and partial overlap in frequency. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; - Support of Multi-DCI STx2P (Simultaneous Transmission across 2 Panels) for non-codebook based PUSCH with fully overlapping in both frequency and time. - Support of two SRS resource sets with usage set to 'nonCodebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of simultaneous transmitted SRS at one symbol per SRS resource set - Maximum number of layers of each overlapping PUSCH - Maximuml total number of layers across two overlapping PUSCH; FR2 only; Component 1 candidate values: a combination from the set {DG PUSCH, Type I CG PUSCH, Type II CG PUSCH} Component 3 candidate values: {1, 2 ,3, 4} Component 4 candidate values: {1, 2, 3, 4} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time partially or non-overlapping in frequency, partially or fully overlapping in time - Supported codebook based PUSCH MIMO with maximal number of supported layers - Maximum number of simultaneous transmitted SRS resources at one symbol; 2-15; Yes; Non-codebook multi-DCI based STx2P PUSCH+PUSCH is not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-3b - Component: Similar components as 40-6-1a should be included. DG+CG and CG+CG should be separate FG, since they can be supported only if CG is supported. Following components can be included. - Support of multi-DCI based STx2P PUSCH+PUSCH for non-codebook based PUSCH. - Support of two SRS resource sets with usage set to “non-codebook” - Supported number of SRS resources in one SRS resource set. Candidate values: {1,2,3,4} - Supported maximal number of layers associated with one SRS resource set. Candidate values: {1,2} - Prerequisite FG: 2-15. |
| OPPO [14] |  |
| Samsung [15] | For multi-DCI based STxMP PUSCH scheme, following overlapping cases between scheduling types for PUSCH can be supported: Agreement (RAN1#110b-e) Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations: - DG-PUSCH + DG-PUSCH - CG-PUSCH + DG-PUSCH Agreement (RAN1#111) Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission.. In the previous RAN1 meeting, whether to separate DG+CG and CG+CG from FG 40-6-3a and 40-6-3b was remained as FFS. In our view, DG-PUSCH + DG-PUSCH is only basic feature of FG 40-6-3a and 40-6-3b and others are optional features because additional capabilities to manage additional configurations for STxMP CG PUSCH may be required. Therefore, RAN1 needs to define separate FG for ‘DG-PUSCH+CG-PUSCH’ and ‘CG-PUSCH+CG-PUSCH’. For the separate FG, ‘DG-PUSCH+CG-PUSCH’ and ‘CG-PUSCH+CG-PUSCH’ can be candidate values, for example {DG+CG, CG+CG, both}. Proposal 23: Define a separate FG for ‘DG-PUSCH+CG-PUSCH’ and ‘CG-PUSCH+CG-PUSCH’ and ‘DG-PUSCH+CG-PUSCH’ and ‘CG-PUSCH+CG-PUSCH’ can be candidate values for newly defined FG, for example {DG+CG, CG+CG, both}. If the UE can support multi-DCI based STxMP PUSCH scheme, two PUSCHs which are overlapped in time domain can be scheduled to the UE. RAN1 discussed possible overlapping cases and the following agreement were endorsed: Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain. In our view, some of overlapping case can require further enhanced UE capability, e.g. the case with partially overlapping in time domain. If two PUSCHs are partially overlapped in time domain, it can effect that the UE determines uplink power control (total transmit power or each transmit power per panel etc.) and prepares two overlapped PUSCHs. On the other hand, we think the fully overlapping case in time domain can be kept in main FG 40-6-3a and 40-6-4b because less capability can be required than partially overlapping cases. Therefore, we suggest making separate a FG to support two partially overlapping PUSCHs in time domain. Proposal 24: Separate a FG to support two partially overlapping PUSCHs in time domain |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; 1. Support of multi-DCI based STxMP PUSCH+PUSCH for non-codebook-based PUSCH: Support fully overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'nonCodebook' associated with two coresetPoolInde values 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of PUSCHs per CORESETPoolIndex per slot; 2-15; Yes; N/A; Per FSPC; No; No; N/A; Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-6-1b/40-6-2b/40-6-3b-1 Similar UE capability (FG 23-3-1-2a) has been introduced for Rel-17 Multi-TRP PUSCH repetition. Thus, we think it can be also introduced for each Rel-18 STxMP scheme. However, current wording for these FGs is a bit unclear, and we suggest the following changes: Proposal 23: Support FG 40-6-1b/40-6-2b/40-6-3b-1 with the following changes: - FG 40-6-1b: Two Aassociated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH - FG 40-6-2b: Two Aassociated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH - FG 40-6-3b-1: Two Aassociated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH FG 40-6-3a/40-6-3b First, we don’t see the need to introduce separate rows for “DG-PUSCH+CG-PUSCH” and “CG-PUSCH+CG-PUSCH”, i.e., these combinations should be baseline as “DG-PUSCH+DG-PUSCH”. Proposal 24: No need to introduce separate rows for “DG-PUSCH+CG-PUSCH” and “CG-PUSCH+CG-PUSCH” Proposal 25: No need to signal the followings, i.e., any type of overlapping/non-overlapping should be supported as baseline - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - Fully overlapping in both frequency and time - Fully overlapping in frequency, partially overlapping in time - Partially or non-overlapping in frequency, partially or fully overlapping in time Meanwhile, we propose to include the component “The combination of maximal number of layers for each PUSCH when two PUSCHs are overlapped in at least one OFDM symbol” in FG 40-6-3a and FG 40-6-3b, and a UE can report a combination from the candidates {1+1, 1+2, 2+2}. Note that for the case if two PUSCHs are not overlapped in time domain, legacy capability XX-XX is used for the maximal number of layers for each PUSCH Proposal 26: Include a component “The combination of maximal number of layers for each PUSCH when two PUSCHs are overlapped in at least one OFDM symbol” in FG 40-6-3a and FG 40-6-3b - Candidate value for the component: {1+1, 1+2, 2+2} Note: For the case if two PUSCHs are not overlapped in time domain, legacy capability FG 2-14/2-15 is used for the maximal number of layers for each PUSCH 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; - The combination of maximal number of layers for each PUSCH when two PUSCHs are overlapped in at least one OFDM symbol; For the case if two PUSCHs are not overlapped in time domain, legacy capability FG 2-15 is used for the maximal number of layers for each PUSCH; Optional with UE capability Component-1 candidate values: {1+1, 1+2, 2+2}. |

| 40. NR_MIMO_evo_DL_UL | 40-6-3b-1 | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-6-3b-1, similar to the update of FG 40-6-3a, - Basically, this FG is needed due to two SRS resource sets are introduced for multi-DCI based STxMP PUSCH+PUSCH in NCB scheme. - Note that this feature is technically similar to FG 23-3-1-2a for Rel-17 MTRP TDM PUSCH repetition, then FG 23-3-1-2a can be taken as the reference of this FG. Furthermore, the newly suggested FG 40-6-3b-1a should be added to keep alignment with FG 23-3-1-2b. - FG 2-15a and FG 40-6-3b should be adopted as the prerequisite FGs. 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH; 2-15a, 40-6-3b. 40-6-3b-1a; CSI-RS processing framework for SRS with two associated CSI-RS resources; - Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP - Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP - Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP - UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS - UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously; 40-6-3b-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This is the equivalent of FG 40-6-1-2 for multi-DCI based STxMP. FG 40-6-3b should be the prerequisite for this FG. 40-6-3b-1; Association between CSI-RS resources and SRS for noncodebook multi-DCI based STx2P PUSCH+PUSCH. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; - Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook multi-DCI based STx2P PUSCH+PUSCH - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.; FR2 only; Component 2 candidate value: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: { 2 to 256}; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook multi-DCI based STxMP SFN scheme for PUSCH; 2-15a; Yes; Associated CSI-RS resources for non-codebook multi-DCI based STxMP PUSCH+PUSCH is not supported; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-3b-1 - Component: Similar as 40-6-1b, following components can be included. - Support of up to two CSI-RS resources associated with two SRS resource sets. - Prerequisite FG: 40-6-1a, 2-15a |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based multi-DCI based STxMP 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously; Yes; N/A; Per band; No; No; N/A; Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3c; Partial-overlapping PUSCHs in time and non-overlapping in frequency; Support partial-overlapping PUSCHs in time and non-overlapping in frequency; Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3d; Overlapping PUSCHs in time and fully overlapping in frequency and time; Support PUSCHs with fully overlapping REs, i.e. the allocated REs for PUSCH associated with CORESETPoolIndex = 0 and PUSCH associated with CORESETPoolIndex = 1 are exactly the same REs; Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3e; Overlapping PUSCHs in time and partially overlapping in frequency; Support PUSCHs with partially overlapping REs, i.e. the allocated REs for PUSCH associated with CORESETPoolIndex = 0 and PUSCH associated with CORESETPoolIndex = 1 are partially overlapped, with at least one RE; Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3f; Multi-DCI based STx2P for CG+CG; Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH; Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3g; Multi-DCI based STx2P for DG+CG; Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH; Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. |
| MediaTek Inc. [17] | 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Two Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH. |

| 40. NR_MIMO_evo_DL_UL | 40-6-4 | Single-DCI based STx2P SFN scheme for PUCCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-6-4, - Given that STxMP PUCCH transmission is enabled by the PUCCH resource activated with two TCI stats (where unified TCI framework extension is assumed as discussed in AI 9.1.1.1) that aiming for FR2, it is needed to added one component to capture this. - Its prerequisite should include FG 23-3-2d. 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; - Support of single-DCI based STx2P SFN scheme for PUCCH - Support of up to two TCI states per PUCCH resource; 23-3-2d. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] | This is the FG for supporting STxMP SFN scheme for PUCCH, which is needed. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; - Support of Single-DCI STx2P (Simultaneous Transmission across 2 Panels) with SFN for PUCCH.; FR2 only; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; - Support of single-DCI based STxMP SFN scheme for PUCCH - Supported PUCCH formats for STxMP SFN scheme; Yes; Single-DCI based STx2P SFN scheme for PUCCH is not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH; 1. Support of Single-DCI based SFN STxMP scheme for PUCCH 2. Supported PUCCH formats for this scheme; Yes; N/A; Per FS; No; No; N/A; Component 2 candidate values: {PF0/2, PF1/3/4, PF0-4}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-6-5 | Support grouped-based beam reporting for STx2P |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | - For FG 40-6-5, - Note that two Alts were adopted to enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission and each of them is subject to UE capability, hence it is needed to capture this in the part of component. - Its prerequisite should include FG 23-5-1. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. 40-6-5; Support grouped-based beam reporting for STx2P; Support of one of {Mode 1, Mode 2, both Mode 1 and Mode 2} - Note: Mode 1 is that the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Note: Mode 2 is that the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously.; 23-5-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; 1. Supported mode for group-based beam reporting (Candidate value: jointULandDL, ULOnly or both) 2. Maximum number of SSB/CSI-RS resources per set 3. Maximum number of reported beam groups. |
| CATT [9] | 40-6-5: - In RAN1#113 e-meeting, following agreement has been achieved on he codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP: Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. As it was agreed that supporting Alt1 and/or Alt2 is subject to UE capability, it is our view that UE should report such capability FG with 40-6-5. Proposal 13: For FG 40-6-5, follow modifications are suggested: 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; 1) Support Alt1 for group-based beam L1-RSRP reporting. 2) Support Alt2 for group-based beam L1-RSRP reporting. 3) Support Alt1 and Alt2 group-based beam L1-RSRP reporting.. |
| Ericsson [10] | RAN1 agreed two alternatives for group-based beam reporting for STx2P. For Alt1, simultaneous UL transmission is possible. For Alt 2, simultaneous UL transmission and simultaneous DL reception is possible. There is a need for separate rows. We propose to add a row FG 40-6-5a. 40-6-5; Support UL-only group-based beam reporting for STx2P; 1. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets (Follow FG 23-5-1). 40-6-5a; Support UL and DL group-based beam reporting for STx2P; 1. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets (Follow FG 23-5-1). 40-6-6; Two PTRS ports for single-DCI based STx2P SDM scheme. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; - Support grouped-based beam reporting for STx2P; FR2 only; Component 1 candidate value: {mode 1, mode 2, both} Note: Mode 1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. Mode 2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously.; Optional with capability signalling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; - Support of the UL Tx spatial filters determined from the reported pair of CRI/SSBRIs can be applied simultaneously; - Support of the UL Tx spatial filters determined from the reported pair of CRI/SSBRIs can be applied simultaneously and received simultaneously; - Max number N of beam pairs in a single L1-RSRP reporting instance based on measurement on two CMR resource sets; 23-5-1; Yes; Group-based beam reporting for STx2P is not supported; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-6-5 - Component: similar components as Rel-17 group based beam reporting can be included. Following components can be included. - Max number of beam groups in a beam reporting. Candidate values: {1,2,3,4}. - Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs. Candidate values: {2,3,4,8,16,32,64} - Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs. Candidate values: {8,16,32,64,128} - Prerequisite FG:40-6-1 or 40-6-1 or 40-6-2 or 40-6-2a or 40-6-32 or 40-6-3b or 40-6-4 |
| OPPO [14] |  |
| Samsung [15] | RAN1 agreed enhance group-based beam reporting to support UL STxMP schemes. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. In the agreement, Alt1 (the reported beam pair(s) can be used for both UL simultaneous transmission and DL simultaneous reception) and/or Alt2 (the reported beam pair(s) can be used for only UL simultaneous transmission) are supported depending on UE capability reporting. Therefore, if the UE supports UL STxMP scheme(s), the UE should report the possible scheme of group based beam reporting. To report the possible reporting scheme, we suggest following updates of FG 40-6-5 with reference to the above agreement and introduced Rel-17 FG 23-5-1 (Group based L1 RSRP reporting enhancements). As additional FFS point, we think RAN1 needs to discuss on prerequisite for FG 40-6-5, especially, whether FG 23-5-1 can be prerequisite or not. 40-6-5; Support grouped-based beam Group based L1-RSRP reporting for STx2P; 1. Support group based L1-RSRP reporting for STxMP based transmission - Component 1 candidate values: {DL and UL, UL only} 2. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 3. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 4. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs. |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-6-5; Support group-based beam reporting for STx2P; 1. Supported mode of group-based beam reporting for STx2P 2. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 3. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 4. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs; Yes; N/A; Per band; No; No; N/A; Component 1 candidate values: {JointULandDL, ULOnly, both} Component 2 candidate values: {1,2,3,4} Component 3 candidate values: {2,3,4,8,16,32,64} Component 4 candidate values: {8, 16, 32, 64, 128} Note: component 3 and 4 are also counted in FG 16-1g, 16-1g-1, and 23-5-1.; Optional with capability signaling. |
| MediaTek Inc. [17] | FG 40-6-5 In this FG, based on the following agreement, supporting Alt1 and/or Alt2 is subject to UE capability. Thus, a UE can report a value from the candidate {JointULandDL, ULOnly, both JointULandDL and ULOnly}. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. Proposal 27: Support the following candidate value for the component “Support grouped-based beam reporting for STx2P” in FG 40-6-5 - {JointULandDL, ULOnly, both JointULandDL and ULOnly} 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P; - Grouped-based beam reporting for STx2P; Optional with UE capability Component-1 candidate values: {JointULandDL, ULOnly, both JointULandDL and ULOnly }. |

| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | FFS: Full-coherent codebook parameters as separate row or component |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; Full-coherent codebook parameters. 40. NR_MIMO_evo_DL_UL; 40-7-1a; (N1,N2) for coherent codebook; Support of (N1,N2); Candidate value {(4, 1), (2, 2), both (4,1) and (2,2)}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-7-1 - A FG for basic features of codebook-based 8TX PUSCH is necessary. Thus, 40-7-1 and 40-7-5 could be combined to make a single FG. Separate FGs could be created to include the parameter values, including Antenna group Ng =1 (fully coherent), Ng = 2,4 (partially coherent), and Ng = 8 (non-coherent), and antenna panel sizes N1, N2. Additional FGs 40-7-1a, 40-7-1b, 40-7-1c were created. 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; FFS: Full-coherent codebook parameters as separate row or component. 40. NR_MIMO_evo_DL_UL; 40-7-1a; coherent codebook; Coherent Ng=1 components (N1, N2) ={ (2,2) and (4,1)}. 40. NR_MIMO_evo_DL_UL; 40-7-1b; Partially coherent; Ng = {2, 4}. 40. NR_MIMO_evo_DL_UL; 40-7-1c; Non coherent; Ng=8. |
| ZTE [4] |  |
| Spreadtrum Communications [5] | For FG 40-7-1 for enabling 8TX UL transmission, the full-coherent codebook parameters, the partial-coherent codebook parameters and the non-coherent codebook parameters can be included in the basic features for codebook-based 8TX PUSCH, where the full-coherent codebook parameters can include (N1, N2). For FG 40-7-5, we think this feature means the support of Ng value and should be also included in FG 40-7-1 as the component of basic feature for codebook-based 8TX PUSCH. For example, the supported Ng value(s) is included in the partial-coherent codebook parameters and can be 2 and/or 4. Proposal 26: Support the full-coherent codebook parameters as the component in FG 40-7-1 and delete FG 40-7-1a, where (N1, N2) is included in full-coherent codebook parameters. Proposal 27: Support including the Ng value in FG 40-7-1 and deleting 40-7-5. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH Component1: support of full-coherent codebook Component2: support of partial-coherent codebook, Ng=2 Component3: support of partial-coherent codebook, Ng=4 Component4: support of non-coherent codebook; FFS: Full-coherent codebook parameters as separate row or component Component1: {support, not support} Component2: {support, not support} Component3: {support, not support} Component4: {support, not support}. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-7-1: - FG 40-7-1 is the basic feature for codebook based UL 8Tx PUSCH transmission. Therefore if a UE supports FG 40-7-1, it supports UL 8Tx PUSCH transmission. - In Rel-17, for codebook based PUSCH transmission, FG 2-14 is used to report the following two components: 1) Supported codebook based PUSCH MIMO with maximal number of supported layers and 2) Supported max number of SRS resource per set (SRS set use is configured as for codebook) . The FG also should be applied to codebook based UL 8Tx PUSCH transmission, i.e., if a UE supports FG 40-7-1, it shall report FG 2-14 as a prerequisite FG, and the UE supports codebook based 8Tx PUSCH transmission with the number of layers no more than the value of component-1 in FG 2-14 (i.e., maxNumberMIMO-LayersCB-PUSCH), and the number of SRS resource per set for codebook based UL 8Tx should be no more than the value of component-2 in FG 2-14(i.e., maxNumberSRS-ResourcePerSet). |
| Ericsson [10] | We have the following agreement from RAN1#113: Agreement For an 8TX UE, Option 1 is supported, - Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value, based on which, gNB may RRC configure UE with a codebook corresponding to only one of the supported Ng values. From this agreement, UEs can be configured with different codebooks according to different numbers of coherent ports, but each codebook must only contain precoders with one number of coherent ports. We also see that the Rel-18 draft for 38.211 contains the non-coherent and fully coherent (N1, N2) = (4, 1) and (N1, N2) = (4, 1) codebooks, labelled as ‘Type A’, ‘Type B’, and ‘Type C’, respectively. Rel-15 UL MIMO uses coherence as the basic UE feature for codebook-based MIMO, and builds the codebooks on top of that, as can be seen below for UE features 2-13 and 2-14. Index; Feature group; Components; Prerequisite feature groups. 2-13; PUSCH codebook coherency subset; Supported codebook coherency subset type; 2-12. 2-14; Codebook based PUSCH MIMO transmission; 1) Supported codebook based PUSCH MIMO with maximal number of supported layers 2) Supported max number of SRS resource per set (SRS set use is configured as for codebook).; 2-13. However, since coherence is now directly tied to each codebook, coherence should be merged with the codebook. Therefore, we can have something like the following: Index; Feature group; Components. 40-7-1; Basic features for Codebook-based 8Tx PUSCH; FFS: Full-coherent codebook parameters as separate row or component Codebooks Type(s) supported (at least one), e.g. ‘Type A’, ‘Type B’, etc.. 40-7-1a; (N1,N2) for coherent codebook. 40-7-5; PUSCH [for codebook coherency subset]. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; - Maximum number of supported layers for codebook based PUSCH operation with 8 Tx. - Maximum number of SRS resources in each SRS resource set with usage set to 'codebook’. - Support of 2 codewords if the maximum number of supported layers is larger than 4; Component 1 candidate values: {1, 2, [3,] 4, [5, 6, 7,] 8} Component 2 candidate values: {1, 2}; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Basic features for Codebook-based 8Tx PUSCH; FFS: Full-coherent codebook parameters as separate row or component 1. Supported maximum number of SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH. 2. Supported maximum number of codewords for codebook-based 8Tx PUSCH.; 2-14; yes; n/a; Per Band; n/a; n/a; n/a; Component 1 candidate values: {1,2} Component 2 candidate values: {1,2}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-7-1 - Components: - For basic features for codebook-based 8Tx PUSCH, UE should support configuration of 1 SRS resource set containing up to 2 8-port SRS resources. - In addition, for supported codebook coherency subset, one of them should be supported by default. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-1; Codebook based PUSCH; 1.Supported maximal PUSCH MIMO layers for codebook based PUSCH; YES; N/A; Per FSPC; NO; NO; NO; Component 1 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-2 | Basic features for Non-Codebook-based 8Tx PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] |  |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-7-2: - Similar as that for FG 40-7-1, FG 2-15 should be a prerequisite FG for FG 40-7-2, with the components in FG 2-15 applied to non-codebook based 8Tx PUSCH transmission. |
| Ericsson [10] | 8 Tx non-codebook based PUSCH is a simple extension of 4 Tx non-codebook based PUSCH, and so we can essentially reuse the Rel-15 feature. Index; Feature group; Components. 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; 1) Maximal number of supported layers (non-codebook transmission scheme) (range: 1 to 8) 2) Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). (range: 1 to 8) 3) Maximum number of simultaneous transmitted SRS resources at one symbol (range: 1 to 8). |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; - Maximum number of supported layers for non-codebook based PUSCH operation with 8 Tx. - Maximum number of SRS resources in each SRS resource set with usage set to 'nonCodebook’. - Maximum number of simultaneous transmitted SRS at one symbol; Component 1 candidate values: {1, 2, [3, ]4, [5, 6, 7, ]8} Component 2 candidate values: {5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8}; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-2; Basic features for Non-Codebook-based 8Tx PUSCH; 1. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ for non-codebook-based 8Tx PUSCH. 2. Supported maximum number of codewords for non-codebook-based 8Tx PUSCH.; 2-15; yes; n/a; Per Band; n/a; n/a; n/a; Component 1 candidate values: {1,2,3,4,5,6,7,8} Component 2 candidate values: {1,2}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-7-2 - Components: - For basic features for non-codebook-based 8Tx PUSCH, UE should support configuration of up to 8 single-port SRS resources. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-2; Noncodebook based PUSCH; 1.Supported maximal PUSCH MIMO layers for noncodebook based PUSCH; YES; N/A; Per FSPC; NO; NO; NO; Component 1 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-2a | Association between CSI-RS and SRS for non-codebook case |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-7-2a - We don’t have a strong opinion on having this FG 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and SRS for non-codebook case; Note: propose to remove the FG. |
| ZTE [4] | 40-7-2a; Association between CSI-RS and SRS for non-codebook case; 40-7-2. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-7-2a : - Besides FG 40-7-2, FG 2-15a can be a prerequisite FG for FG 40-7-2a, with the components in FG 2-15a applied to non-codebook based 8Tx PUSCH transmission. |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and SRS for non-codebook case; - Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation - A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.; Component 2 candidate value: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: { 2 to 256}; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and SRS for non-codebook case; 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH.; 2-15a; yes; n/a; Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-7-2a - Not sure why this feature group is needed. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-2a; Association between CSI-RS and 8Tx SRS; 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRSresoureset" 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously.; YES; N/A; Per FSPC; NO; NO; NO; Component-2: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-3 | Maximum number of layers [for rank 5-8] | FFS: split for codebook and noncodebook is in this row or separate FFS: CBG based PUSCH with rank >4 whether component or new row |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-7-3; Maximum number of layers [for rank 5-8]; 1. Support of maximal number of layers for CB and NCB; Candidate value {4,8}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-7-3a; CBG based PUSCH with rank >4. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-7-3 - We propose to confirm ”for rank 5-8” - FFS1: we propose having separate rows for codebook and non codebook. - FFS2: In addition, we are of the opinion that we don’t need a separate FG for dual CW PUSCH operation in CBG-based transmission when the rank > 4 40. NR_MIMO_evo_DL_UL; 40-7-3; Maximum number of layers [for rank 5-8] - codebook; FFS: split for codebook and noncodebook is in this row or separate FFS: CBG based PUSCH with rank >4 whether component or new row. 40. NR_MIMO_evo_DL_UL; 40-7-3a; Maximum number of layers [for rank 5-8] -non codebook. |
| ZTE [4] | - Regarding maximum number of layers, we suggest to have separate FGs for codebook and non-codebook based transmission. Then, we are okay to have a separate FG for supporting CBG based PUSCH transmission with RANK>4. 40-7-3b; Support of CBG based PUSCH with rank >4; Support of CBG based PUSCH with rank >4; 40-7-1, 40-7-2. - For additional maximum number of ranks, we can have a UE feature group of indicating maximum number of ranks for codebook and non-codebook based 8-Tx PUSCH. 40-7-3; Maximum number of layers for Codebook-based 8Tx PUSCH [for rank 5-8]; FFS: split for codebook and noncodebook is in this row or separate FFS: CBG based PUSCH with rank >4 whether component or new row; 40-7-1. 40-7-3a; Maximum number of layers for Non-Codebook-based 8Tx PUSCH; 40-7-2. |
| Spreadtrum Communications [5] | For FG 40-7-3, the maximum number of layers for rank 1~8 for codebook-based PUSCH and non-codebook-based PUSCH needs to be split into 40-7-1 and 40-7-2 separately, similar to Rel-15. Proposal 28: Split 40-7-3 into 40-7-1 and 40-7-2, i.e. include the maximum number of layers for rank 1~8 for codebook-based PUSCH and non-codebook-based PUSCH separately |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-3; Maximum number of layers [for rank 5-8]; FFS: split for codebook and non-codebook is in this row or separate Comments: no need for separate rows for codebook and non-codebook FFS: CBG based PUSCH with rank >4 whether component or new row Comments: separate row for CBG based PUSCH with rank>4. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-7-3; Maximum number of layers [for rank 5-8]; 1. Maximum number of layers for codebook based transmission 2. Maximum number of layers for non-codebook based transmission. |
| CATT [9] | 40-7-3: - For a given UE, the maximum number of layers for rank>4 for codebook based UL 8Tx transmission and non-codebook based UL 8Tx transmission should be two separate FGs, therefore FG 40-7-2 should be split to two separate FGs for codebook and non-codebook separately. - For the FG of maximum number of layers for rank >4 for codebook based UL 8Tx transmission, FG 40-7-1 should be the prerequisite FG. For the FG of maximum number of layers for rank >4 for non-codebook based UL 8Tx transmission, FG 40-7-2 should be the prerequisite FG. |
| Ericsson [10] | Regarding the maximum number of layers, we think the 8 Tx codebook should not be restricted to 5 or more layers, given the precoding gain with coherent transmission with 1 to 4 layers. Moreover, it seems reasonable to use the Rel-15 approach that identifies the maximum number of layers and SRS resources. Note that we should say ‘8 port’ SRS resources, to avoid confusion with UL full power mode 2. We also see no need to tie the number of layers for codebook and non-codebook as this was not needed for Rel-15. Index; Feature group; Components; Prerequisite feature groups. 40-7-3; Maximum number of layers [for rank 5-8] (1 to 8) 1) Supported maximal number of supported layers for codebook based 8Tx PUSCH (range: 1 to 8) 2) Supported max number of 8 port SRS resources per set (SRS set use is configured as for codebook) for codebook based 8Tx PUSCH (range: 1 to 2); FFS: split for codebook and noncodebook is in this row or separate FFS: CBG based PUSCH with rank >4 whether component or new row; 40-7-1. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-3; Maximum number of layers [for rank 5-8]. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-3; Maximum number of layers [for rank 5-8]; FFS: split for codebook and noncodebook is in this row or separate FFS: CBG based PUSCH with rank >4 whether component or new row 1. Supported maximum number of layers for 8Tx PUSCH. 2. Support CBG-based transmission for dual CW PUSCH operation.; 2-14, 2-15; yes; n/a; Per Band; n/a; n/a; n/a; Component 1 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-7-3 Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e The above applies only with regards to the work scope of this agenda item. Feature group: For the max number of layers, we have made following agreements. So up to X=4 and up to X=8 should be separate UE capabilities. - Components: - Support to split it as separate rows for codebook-based and non-codebook-based UL transmission. - Support to split it as new row for CBG based PUSCH transmission. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-3; CBG based 2 CWs PUSCH; 1.Support CBG based transmission for 2 CWs PUSCH; YES; N/A; Per FSPC; NO; NO; NO; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-4 | UL full power transmission mode 0 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| ZTE [4] | 40-7-4; UL full power transmission mode 0; 40-7-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] |  |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] | 40-7-4: - For the FG of UL full power transmission mode 0 for 2Tx and 4Tx, FG 2-13 and FG 2-14 are the prerequisite FGs. Similarly, for FG 40-7-4, FG 40-7-5 and FG 40-7-1 should be prerequisite FGs. |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-4; UL full power transmission mode 0; - Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-4; UL full power transmission mode 0; 1. Support UL full power transmission mode 0 for 8Tx PUSCH.; 16-5a; yes; n/a; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-4; Full power mode 0 for 8 Tx PUSCH; 1. Support full power mode 0 for 8 Tx PUSCH; YES; N/A; Per FSPC; NO; NO; NO; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-4a | UL full power transmission mode 1 | This is a WA |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-7-4a - We recommend removing the ”This is a WA” 40. NR_MIMO_evo_DL_UL; 40-7-4a; UL full power transmission mode 1; This is a WA. |
| ZTE [4] | 40-7-4a; UL full power transmission mode 1; This is a WA; 40-7-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-4a; UL full power transmission mode 1; This is a WA Comments: confirm the WA. |
| Intel Corporation [7] |  |
| Google [8] |  |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-4a; UL full power transmission mode 1; - Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-4a; UL full power transmission mode 1; 1. Support UL full power transmission mode 1 for 8Tx PUSCH.; 16-5b; yes; n/a; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-4a; Full power mode 1 for 8 Tx PUSCH; 1. Support full power mode 1 for 8 Tx PUSCH; YES; N/A; Per FSPC; NO; NO; NO; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-4b | UL full power transmission mode 2 | This is a WA |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-7-4b - We recommend removing the ”This is a WA” 40. NR_MIMO_evo_DL_UL; 40-7-4b; UL full power transmission mode 2; This is a WA. |
| ZTE [4] | 40-7-4b; UL full power transmission mode 2; This is a WA; 40-7-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-4b; UL full power transmission mode 2; This is a WA Comments: confirm the WA. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-7-4b; UL full power transmission mode 2; 1. Precoder group(s) for UL full power mode 2 2. Number of SRS ports for indicated SRI for PUSCH for UL full power operation (Candidate value: 2, 4 or both). |
| CATT [9] |  |
| Ericsson [10] |  |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-4b; UL full power transmission mode 2; - Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2; Component 1 candidate values: {1, 2, 4}; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-4b; UL full power transmission mode 2; 1. Support UL full power transmission mode 2 for 8Tx PUSCH. 2. Supported TPMI groups which delivers full power. 3. Supported maximum number of SRS resources per SRS resource set with usage set to 'codebook’ for mode 2; 16-5c; yes; n/a; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-4b; Full power mode 2 for 8 Tx PUSCH; 1. Support full power mode 2 for 8 Tx PUSCH FFS how to indicate full power precoders under full power mode 2.; YES; N/A; Per FSPC; NO; NO; NO; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-5 | PUSCH [for codebook coherency subset] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-7-5; PUSCH [for codebook coherency subset]; Support of codebook coherency; Candidate value {Ng=1 or Ng=2 or Ng=4 or Ng=8 }; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | - For FG 40-7-5 - It is merged with FG 40-7-1 40. NR_MIMO_evo_DL_UL; 40-7-5; PUSCH [for codebook coherency subset]; NG=1, 2, 4, 8 ( 2, 4 partially coherent, 8 non coherent) …. |
| ZTE [4] | 40-7-5; PUSCH [for codebook coherency subset]. |
| Spreadtrum Communications [5] | For FG 40-7-1 for enabling 8TX UL transmission, the full-coherent codebook parameters, the partial-coherent codebook parameters and the non-coherent codebook parameters can be included in the basic features for codebook-based 8TX PUSCH, where the full-coherent codebook parameters can include (N1, N2). For FG 40-7-5, we think this feature means the support of Ng value and should be also included in FG 40-7-1 as the component of basic feature for codebook-based 8TX PUSCH. For example, the supported Ng value(s) is included in the partial-coherent codebook parameters and can be 2 and/or 4. Proposal 26: Support the full-coherent codebook parameters as the component in FG 40-7-1 and delete FG 40-7-1a, where (N1, N2) is included in full-coherent codebook parameters. Proposal 27: Support including the Ng value in FG 40-7-1 and deleting 40-7-5. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-5; PUSCH [for codebook coherency subset]; Comments: merge this row with 40-7-1 as shown above. 40. NR_MIMO_evo_DL_UL; 40-7-5; PUSCH [for codebook coherency subset]; Comments: merge this row with 40-7-1 as shown above. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-7-5; PUSCH [for codebook coherency subset]; 1. Supported codebook coherency subset (Candidate value (bit-map): fully-coherent, partially-coherent with Ng=2, partially coherent with Ng=4, non-coherent). |
| CATT [9] | 40-7-5: - In Rel-17, the FG for PUSCH codebook coherency subset (i.e., FG 2-13) is a feature in IE MIMO-ParametersPerBand. Similarly, FG 40-7-5 can be a feature in IE MIMO-ParametersPerBand. - In Rel-17, FG 2-13 is a prerequisite FG for the FG for codebook based PUSCH MIMO transmission(i.e., FG 2-14). Similarly, FG 40-7-5 can be a prerequisite FG for FG 40-7-1. - In RAN1 #113 meeting, it was agreed that subject to its capability, an 8TX UE may report more than one Ng value. It is our views that any combination of Ng = {1, 2, 4, 8} shall be allowed to be reported by UE. Therefore any combination of Ng = {1, 2, 4, 8} shall be allowed for FG 40-7-5. |
| Ericsson [10] | 40-7-5; PUSCH [for codebook coherency subset]. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-5; Codebook coherent subset for 8Tx codebook based PUSCH; - Supported codebook coherency subset type when UE is configured with 8Tx codebook based PUSCH operation.; Component 1 candidate values: a combination from the set {non-coherent Ng=8, partial coherent Ng=4, partial coherent Ng=2, full-coherent Ng=1} Note: (1) non-coherent Ng=8 (2) partial-coherent Ng=4, i.e., 4 coherent port groups with 2 ports each group, while non-coherent between different port groups (3) partial-coherent Ng=2, i.e., 2 coherent port groups with 4 ports each group, while non-coherent between different port groups (4) full-coherent Ng=1. Note: For codebook based PUSCH operation, the number of Tx is the maximum number of SRS ports of SRS resource(s) configured in the SRS resource set with usage set to “codebook”.; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-5; PUSCH [for codebook coherency subset]; 1.Supported codebook coherency subset types for codebook-based 8Tx PUSCH.; 2-13; yes; n/a; Per Band; n/a; n/a; n/a; Component 1 candidate values: {non-coherent Ng=8, partially-coherent Ng=4, partially-coheren Ng=2, fully-coherent Ng=1}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-7-5 - Components: - For codebook coherent subset configuration, we have made following agreements. So four coherent subsets can be considered for this feature. Agreement For codebook-based 8TX PUSCH transmission, Alt3 is supported, where - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1 - A partially-coherent UE, with Ng =2, can only use precoders considered for Ng =2 - A partially-coherent UE, with Ng =4, can only use precoders considered for Ng =4 - A non-coherent UE, with Ng =8, can only use precoders considered for Ng = 8 Agreement For an 8TX UE, Option 1 is supported, - Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value, based on which, gNB may RRC configure UE with a codebook corresponding to only one of the supported Ng values. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-5; Supported codebook; Supported codebook for 8 Tx PUSCH PUSCH [for codebook coherency subset]; YES; N/A; Per FSPC; NO; NO; NO; Candidate values of Ng = any subset of {1,2,4,8} Notes: Ng=1: full coherent codebook Ng=2: partial coherent codebook with 2 antenna groups. Ng=4: partial coherent codebook with 4 antenna groups. Ng=8: non coherent codebook.; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

| 40. NR_MIMO_evo_DL_UL | 40-7-1a | (N1,N2) for coherent codebook |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; (N1,N2) for coherent codebook; Support of (N1,N2); Candidate value {(4, 1), (2, 2), both (4,1) and (2,2)}; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; (N1,N2) for coherent codebook; Ng=1 componenets (2,2) and (4,1). |
| ZTE [4] | - Regarding UE antenna architecture, we think that the value of supporting “Ng and antenna architecture (i.e., N1, N2 for Ng=1)’ should be supported as in an FG-a. - For instance, a UE can indicate the support of Ng={1, 2, 8}, and then for Ng=1, the UE support (N1, N2) = (2, 2). 40-7-1a; (N1,N2) for coherent codebook when Ng=1 codebook type is supported; Support of (N1,N2) for coherent codebook: {(4,1), (2,2), both}; 40-7-1. |
| Spreadtrum Communications [5] |  |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; (N1,N2) for coherent codebook; Supported values {(4,1, (2,2)}. |
| Intel Corporation [7] |  |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; (N1,N2) for coherent codebook; 1. Supported (N1, N2) for coherent codebook (candidate value: (2, 2), (4, 1)). |
| CATT [9] | 40-7-1a: - FG 40-7-5 should be the prerequisite FG for FG 40-7-1a since FG 40-7-1a is used to describe the codebook subset for Ng =1 in FG 40-7-5. - In RAN1 #112 meeting, it has been agreed that the following combinations of (N1, N2) for Ng =1 are supported: (N1, N2) = (4, 1) and (N1, N2) = (2, 2). There is no motivation for an UE to report both of (4, 1) and (2, 2) for (N1, N2). Therefore reporting only one of (4, 1) and (2, 2) by FG 40-7-1a can be considered, i.e., the two candidate values of FG 40-7-1a correspond to (4, 1) and (2, 2), respectively. |
| Ericsson [10] | 40-7-1a; (N1,N2) for coherent codebook. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; (N1,N2) for coherent codebook; - Supported full-coherent codebook structure.; Component 1 candidate values: {N1=4 and N1=1, N1=2 and N2=2} Note: UE reports FG40-7-1a must report ‘full-coherent Ng=1” for component 1 in FG40-7-5; Optional with capability signaling. |
| Xiaomi [12] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; (N1,N2) for coherent codebook; 1. Supported number of antenna ports in vertical and horizontal dimensions (N1,N2).; 40-7-1; yes; n/a; Per Band; n/a; n/a; n/a; Component 1 candidate values: {(4,1),(2,2)}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 40-7-1a - This feature is needed for fully-coherent codebook based UL transmission only. |
| OPPO [14] |  |
| Samsung [15] |  |
| Qualcomm Incorporated [16] | 40. NR_MIMO_evo_DL_UL; 40-7-1a; (N1,N2) for coherent codebook; (N1,N2) for coherent codebook; YES; N/A; Per FSPC; NO; NO; NO; Candidate values = {(4,1) only, (2,2) only, both (4,1) and (2,2)}; Optional with capability signaling. |
| MediaTek Inc. [17] |  |

Other

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 40. NR_MIMO_evo_DL_UL; 40-1-14; Support of two default beams for unified TCI; At least one of 40-1-1, 40-1-1a, 40-1-2 or 40-1-2a; Per band; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-2-9; Support of absolute TA command MAC-CE; Per band; n/a; n/a; n/a; Optional with capability signalling. Considering the higher channel estimation complexity brought by length-4-FD-OCC-based Rel.18 DMRS, the support of the combination of Rel.18 DMRS and other features with high timing requirement/channel estimation complexity should be enabled by separate UE features if supported. First, for PDSCH processing time, there are two capabilities defined in current spec., i.e., capability 1 and capability 2. Capability 2 with higher timing requirement is mainly used for URLLC scenario, which requires higher robustness and lower latency compared with eMBB scenario, while Rel.18 DMRS targets at supporting higher MU-MIMO layers under eMBB scenario. Furthermore, both the lower channel estimation performance and higher channel estimation complexity of length-4 FD-OCC makes Rel.18 DMRS inappropriate under URLLC scenario. As a result, PDSCH processing capability 2 should not be simultaneously supported with Rel.18 DMRS. Next, due to the high complexity of multi-CDM-group channel estimation, a UE capability indicating the simultaneous support of cross-CDM-group Rel.18 DMRS port combination for one codeword should be introduced. Proposal 4-1: Following Rel.18-DMRS-related UE capabilities should be introduced: - UE capability 1: indicate whether UE can simultaneously support Rel.18 DMRS and PDSCH processing capability 2. - UE capability 2: indicate whether UE can support cross-CDM-group Rel.18 DMRS port combinations for one codeword. 40. NR_MIMO_evo_DL_UL; 40-4-1i; Support PDSCH processing capability 2. 40. NR_MIMO_evo_DL_UL; 40-4-1j; Support cross-CDM-group Rel.18 DMRS port combinations for one codeword. |
| Nokia/Nokia Shanghai Bell [3] | - The following agreement about TCI state(s) selection for PUSCH, including dynamic switching from single-TRP to multi-TRP is not really explicitly covered in any FG. This indication leverage existing field but the operation itself would need to be considered as a UE feature Agreement (RAN1#112 Athens) On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission. Proposal: Add a new FG for UTCI framework extension for S-DCI-based MTRP when two SRS resource sets for CB/NCB are configured to support PUSCH transmission scheduled by DCI format 0_1/0_2. - For FG 40-3-1-24 - Minimum CSI calculation delay requirements for AP-CSI reporting for Rel-16/17-based CJT codebooks. Additional delay parameter r applicable to Z2 and Z'2. FFS: candidate values Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’. 40. NR_MIMO_evo_DL_UL; 40-3-1-24; Minimum CSI calculation delay requirements for AP-CSI reporting for Rel-16/17-based CJT codebooks; Additional delay parameter r applicable to Z2 and Z'2; FFS: candidate values. - For FG 40-3-1-25 - Number of CPUs occupied by a CMR for Rel-16/17-based CJT codebooks. It follows from the agreement Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1. 40. NR_MIMO_evo_DL_UL; 40-3-1-25; Number of CPUs occupied by a CMR for Rel-16/17-based CJT codebooks; A CMR occupies X CPUs. - For FG 40-3-2-8 - Number of CPUs occupied by an AP-CMR, for Rel-16/17-based Doppler codebooks. Each of the K AP-CMRs occupies Y1 CPUs 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Number of CPUs occupied by an AP-CMR, for Rel-16/17-based Doppler codebooks; Each of the K AP-CMRs occupies Y1 CPUs. - For FG 40-3-2-9 - Number of CPUs occupied by a P- or SP-CMR, with N4>1, for Rel-16/17-based Doppler codebooks. A P- or SP-CMR occupies Y2*N4 CPUs, for N4>1 40. NR_MIMO_evo_DL_UL; 40-3-2-9; Number of CPUs occupied by a P- or SP-CMR, with N4>1, for Rel-16/17-based Doppler codebooks; A P- or SP-CMR occupies Y2*N4 CPUs, for N4>1. 40-3-2-8/9 follow from the agreement on the CPU occupancy: Agreement(RAN1#113) Previous agreement is revised as follows For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 [+4] when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR. - For FG 40-3-2-10 - Multiplying factor, Kp, for counting active P- or SP-CSI-RS resources/ports, for Rel-16/17-based Doppler codebooks Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for high/medium velocities, the value of KP for P/SP-CSI-RS active resource counting is determined based on UE capability, where the candidate values are {1, 2, 4}.. 40. NR_MIMO_evo_DL_UL; 40-3-2-10; Multiplying factor, Kp, for counting active P- or SP-CSI-RS resources/ports, for Rel-16/17-based Doppler codebooks; An active P- or SP-CSI-RS resource is counted Kp times; Candidate values: {1,2,4}. - For FG 40-3-2-11 - Additional Z2 delay for AP-CSI reporting on AP-CSI-RS resources, for Rel-16/17-based Doppler codebooks. Additional delay parameter m applicable to Z2. FFS: candidate values 40. NR_MIMO_evo_DL_UL; 40-3-2-11; Additional Z2 delay for AP-CSI reporting on AP-CSI-RS resources, for Rel-16/17-based Doppler codebooks; Additional delay parameter m applicable to Z2; FFS: candidate values. - For FG 40-3-2-12 - Additional Z2 delay for AP-CSI reporting on P- or SP-CSI-RS resources, for Rel-16/17-based Doppler codebooks. Additional delay parameter w applicable to Z2. FFS: candidate values 40. NR_MIMO_evo_DL_UL; 40-3-2-12; Additional Z2 delay for AP-CSI reporting on P- or SP-CSI-RS resources, for Rel-16/17-based Doppler codebooks; Additional delay parameter w applicable to Z2; FFS: candidate values. - For FG 40-3-2-13 - Additional Z’2 delay for AP-CSI reporting with N4>1, for Rel-16/17-based Doppler codebooks. Additional delay parameter r applicable to Z’2. FFS: candidate values 40. NR_MIMO_evo_DL_UL; 40-3-2-13; Additional Z’2 delay for AP-CSI reporting with N4>1, for Rel-16/17-based Doppler codebooks; Additional delay parameter r applicable to Z’2; FFS: candidate values. 40-3-2-11/12/13 follow from the agreements: Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Agreement(RAN1#113) For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’. - For FG 40-4-4-7 - Parameter X for the number of CPUs occupied by a TDCP calculation. A TDCP report occupies (Y+1)*X CPUs 40. NR_MIMO_evo_DL_UL; 40-3-3-7; Parameter X for the number of CPUs occupied by a TDCP calculation; A TDCP report occupies (Y+1)*X CPUs. |
| ZTE [4] | - Finally, as for S-DCI based mTRP operation, supporting of two default TCI state and TRP-specific BFR procedure for M-DCI based mTRP can be separately indicated with individual FGs. Meanwhile, we can have an individual FG for supporting inter-cell mTRP. 40-1-9b; Two default beams for PDSCH with scheduling offset less than a threshold; Support of two default beams for PDSCH(s) corresponding to respective CORESETPoolIndex with scheduling offset less than a threshold; 40-1-7. 40-1-9c; TRP-specific BFR with unified TCI framework; Support of TRP-specific BFR with unified TCI framework under M-DCI based MTRP operation; 40-1-7. 40-1-9e; Unified TCI indication for M-DCI based inter-cell mTRP; Support of unified TCI indication for inter-cell mTRP in M-DCI; 40-1-7. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value of SRS transmission power equally across the SRS ports configured on each OFDM symbol, if the UE is capable of transmitting at per OFDM symbol with 8/s ports, where is specified in the current specifications. - Note: This may be captured in the specification in a few different but equivalent ways, and it is up to the editor to decide. Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value on an OFDM symbol. - This is a UE-optional feature. Agreement For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., , where can be randomly chosen from at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. Agreement Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design. Agreement SRS comb offset hopping and cyclic shift hopping can be configured for a SRS resource at the same time as a separate UE capability. No joint hopping scheme is supported.. 40-5-1c; SRS comb offset hopping within a configured comb offset subset; 40-5-1. 40-5-2a; SRS cyclic shift hopping combined with legacy group/sequence hopping; 40-5-2. 40-5-2b; SRS cyclic shift hopping with finer granularity; Candidate values of the parameter K indicating the hopping granularity are TBD; 40-5-2. 40-5-2c; SRS cyclic shift hopping within a configured cyclic shift subset; 40-5-2. 40-5-8; Joint configuration of SRS cyclic shift hopping and comb offset hopping; 40-5-1 40-5-2. |
| Spreadtrum Communications [5] | In RAN1#113 meeting, after extensive discussion and compromise, we have the following agreement on how to handle overlapping issue between two UL transmissions hen the UE does not support UL STxMP transmission. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, - for the baseline feature, the UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) - as an optional feature, the overlapping duration of the later of the two UL transmissions is reduced. - FFS: for the optional feature, whether or not the overlapping duration needs to be specified as 1 (in case 2) or 2 (in case 1) OFDM symbols where - Case 1 applies when UE is capable of supporting MRTD > CP, SCS=60 kHz and frequency range is FR1. - Case 2 applies in all other cases. Thus, we suggest to add one new FG to reflect the agreement: 40. NR_MIMO_evo_DL_UL; 40-2-xx; Reduce the overlapping duration of the latter slot of the two UL transmission when the UE does not support UL STxMP. Proposal 7: Suggest to add one new FG: 40. NR_MIMO_evo_DL_UL; 40-2-xx; Reduce the overlapping duration of the latter slot of the two UL transmission when the UE does not support UL STxMP. During the last meeting, RAN1 has achieved the following agreement on how to count the number of CPUs for CJT type-II codebook. Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1 Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’. Based on the agreement, when NTRP>1, the scaling factor X for counting the number of CPUs and the CSI computation time Z/Z’ are both based on UE capability. Therefore, we think it is straightforward to introduce two new FGs. Proposal 10: Suggest to add two new FGs as follows: - Scaling factor for counting the number of CPUs occupied by a CJT type-II codebook - CSI computation time for CJT type-II codebook During the last meeting, RAN1 has achieved the following agreement on how to count the number of CPUs for doppler codebook. Agreement Previous agreement is revised as follows For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 [+4] when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, the value of KP for P/SP-CSI-RS active resource counting is determined based on UE capability, where the candidate values are {1, 2, 4}.. Based on the agreement above, when N4>1, the scaling factor Y for counting the number of CPUs is based on UE capability. Further, the scaling factor Y can be different when the CMR is P/SP-CSI-RS or AP-CSI-RS. Besides, the CSI computation time Z/Z’ are also based on UE capability. Furthermore, when counting the number of active resources for P/SP CSI-RS, one resource is counted as KP active resources where KP is determined based on UE capability. In summary, we have the following proposal. Proposal 11: Suggest to add the following new FGs: - Scaling factor for counting the number of CPUs occupied by a doppler codebook - Scaling factor Y1 when P/SP-CSI-RS is configured for CMR - Scaling factor Y2 when AP-CSI-RS is configured for CMR - CSI computation time for doppler measurement - Number of active resources for each P/SP CSI-RS resource During the last meeting, RAN1 has achieved the following agreement on how to count the number of CPUs for a TDCP report. Agreement For the Rel-18 TRS-based TDCP reporting, when Y delay(s) are configured - OCPU=(Y+1).X where X≥1 is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X can depend on the value of D, and whether phase reporting is switched ON - Reuse legacy Z2/Z2’ values - To count active resources used for TDCP reporting, reuse the legacy number counting mechanism for CSI-RS resources - UE reports the maximum number of active resources for TDCP in UE capability reporting.. Based on the agreement, the scaling factor X for counting the number of CPUs is based on UE capability. However, it has not been decided whether the supported value(s) of X is independently reported or depend on the value of D. In our views, since the dependency maybe different for different UE implementation, we prefer to introduce a new FG for UE to report the value of X directly. Proposal 12: Suggest to add a new FG: Scaling factor for counting the number of CPUs occupied by a TDCP report. Since the length-4 FD-OCC based Rel.18 DMRS requires higher channel estimation complexity, the simultaneous support of Rel.18 DMRS and other features with high timing requirement may need further discussion, and additional UE features may need to be defined. For example, Proposal 16: Further discuss whether/how to define UE capability on simultaneous support of Rel.18 DMRS and other features. Similar to FG40-5-1b, in last meeting it was agreed that SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design. We suggest to add a new FG40-5-1c to explain this capability. Proposal 17: Suggest to add a new FG: SRS cyclic shift hopping can be combined with one of group / sequence hopping. As agreed, finer time-delay-domain granularity of cyclic shift is supported which can be list as a separate FG. Proposal 18: Suggest to add a new FG: Cyclic shift with finer time-delay-domain granularity. Besides, it was also agreed to configure a subset of comb offsets when comb offset hopping is configured. We are fine to set this feature as a separate FG or as a component of FG40-5-1. Proposal 19: Suggest to add a new FG or a component of FG40-5-1: A subset of comb offsets for comb offset hopping. |
| Vivo [6] | 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Support of N4>1 for Rel-16-based Doppler codebook; Comments: Based on previous RAN1 study and evaluation results, most of the gain for CSI enhancement for high/medium UE velocities comes from CSI prediction. Further time domain compression (N4>1) provides limited gain compared with prediction, while compression causes extra UE complexity. Hence we suggest to make the support of N4>1 optional.. 40. NR_MIMO_evo_DL_UL; 40-3-2-9; Support of d=1 for AP CSI-RS if m>1; Comments: If m>1, support of d=1 requires UE to perform extra interpolation to get the channel for each DD unit. Hence we support to make it optional due to its high complexity.. 40. NR_MIMO_evo_DL_UL; 40-3-2-10; Support of rank>2 for Rel-16-based Doppler codebook; Comments: Following legacy principles. 40. NR_MIMO_evo_DL_UL; 40-3-2-11; Support of rank>2 for Rel-17-based Doppler codebook; Comments: Following legacy principles. 40. NR_MIMO_evo_DL_UL; 40-3-2-12; Support of L=6 for Rel-16-based Doppler codebook; Comments: Following legacy principles. 40. NR_MIMO_evo_DL_UL; 40-3-2-13; Scaling factor Y for CPU counting when N4>1 for P/SP CSI-RS; Agreement Previous agreement is revised as follows For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR. 40. NR_MIMO_evo_DL_UL; 40-3-2-14; Scaling factor Y for CPU counting when N4>1 for AP CSI-RS; Agreement Previous agreement is revised as follows For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR. 40. NR_MIMO_evo_DL_UL; 40-3-2-15; Applied Z/Z’ values when N4>1; Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’. 40. NR_MIMO_evo_DL_UL; 40-3-2-16; Scaling factor Kp of P/SP active resource counting for Rel-16 based Doppler codebook; Candidate values of Kp are {1, 2, 4} Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, - for AP CSI -RS, reuse legacy definition and counting mechanism for active resources - for P/SP CSI-RS, one resource is counted as occupying KP ≥1 active resource(s) - TBD: the value of KP , e.g. N4, fixed value, or according to UE capability Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, the value of KP for P/SP-CSI-RS active resource counting is determined based on UE capability, where the candidate values are {1, 2, 4}.. 40. NR_MIMO_evo_DL_UL; 40-3-2-17; Scaling factor Kp of P/SP active resource counting for Rel-17 based Doppler codebook; Candidate values of Kp are {1, 2, 4} Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, - for AP CSI -RS, reuse legacy definition and counting mechanism for active resources - for P/SP CSI-RS, one resource is counted as occupying KP ≥1 active resource(s) - TBD: the value of KP , e.g. N4, fixed value, or according to UE capability Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, the value of KP for P/SP-CSI-RS active resource counting is determined based on UE capability, where the candidate values are {1, 2, 4}.. 40. NR_MIMO_evo_DL_UL; 40-3-1-24; Scaling factor X for CPU counting when NTRP>1 for Rel-16 based CJT codebook; Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1. 40. NR_MIMO_evo_DL_UL; 40-3-1-25; Applied Z/Z’ when NTRP>1 for Rel-16 based CJT codebook; Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’. 40. NR_MIMO_evo_DL_UL; 40-3-1-26; Scaling factor X for CPU counting when NTRP>1 for Rel-17 based CJT codebook; Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1. 40. NR_MIMO_evo_DL_UL; 40-3-1-27; Applied Z/Z’ when NTRP>1 for Rel-17 based CJT codebook; Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’. 40. NR_MIMO_evo_DL_UL; 40-3-7; Scaling factor X for CPU counting of TDCP; Agreement For the Rel-18 TRS-based TDCP reporting, when Y delay(s) are configured - OCPU=(Y+1).X where X≥1 is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X can depend on the value of D, and whether phase reporting is switched ON - Reuse legacy Z2/Z2’ values - To count active resources used for TDCP reporting, reuse the legacy number counting mechanism for CSI-RS resources - UE reports the maximum number of active resources for TDCP in UE capability reporting.. 40. NR_MIMO_evo_DL_UL; 40-4-2; Support DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8. 40. NR_MIMO_evo_DL_UL; 40-4-12a; UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-8. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Support a subset of comb offsets when comb offset hopping is configured. 40. NR_MIMO_evo_DL_UL; 40-5-2a; SRS cyclic shift hopping combined with legacy group/sequence hopping. 40. NR_MIMO_evo_DL_UL; 40-5-2b; SRS cyclic shift hopping combined with comb offset hopping. 40. NR_MIMO_evo_DL_UL; 40-5-2c; Support a subset of cyclic shifts when cyclic shift hopping is configured. 40. NR_MIMO_evo_DL_UL; 40-5-2d; Support finer time-delay-domain granularity for SRS cyclic shift hopping; Candidate values of K: {2}. 40. NR_MIMO_evo_DL_UL; 40-5-8; SRS transmitted at PCMAX per OFDM symbol for SRS with TDMed 8 Tx ports-. 40. NR_MIMO_evo_DL_UL; 40-6-3b; noncodebook multi-DCI based STx2P PUSCH+PUSCH; FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time Comments: same as in row 40-6-3b. Additional components to be added 1. Support of two SRS resource sets with usage set to 'nonCodebook' 2. Supported number of SRS resources in one SRS resource set. 40. NR_MIMO_evo_DL_UL; 40-6-3b-1; Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH; Comments: Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based STx2P should be considered.. 40. NR_MIMO_evo_DL_UL; 40-6-4; Single-DCI based STx2P SFN scheme for PUCCH. 40. NR_MIMO_evo_DL_UL; 40-6-5; Support grouped-based beam reporting for STx2P. 40. NR_MIMO_evo_DL_UL; 40-7-4b-1; UL full power transmission mode 2 - SRS resources. 40. NR_MIMO_evo_DL_UL; 40-7-4b-2; UL full power transmission mode 2 - full power TPMI groups. |
| Intel Corporation [7] | 40. NR_MIMO_evo_DL_UL; 40-1-4a; Support of PDSCH DMRS ports QCLed with DL RSs of both indicated TCI states with respect to QCL-Type A. 40. NR_MIMO_evo_DL_UL; 40-1-4b; Support of PDSCH DMRS ports QCL with a subset of QCL-Type A parameters for second indicated TCI state. |
| Google [8] | 40. NR_MIMO_evo_DL_UL; 40-1-14; Two default beams for PDSCH for intra-cell mTRP; 1. Support of two default beams for PDSCH with scheduling offset below the threshold. 40. NR_MIMO_evo_DL_UL; 40-1-14a; Two default beams for PDSCH for inter-cell mTRP; 1. Support of two default beams for PDSCH with scheduling offset below the threshold. 40. NR_MIMO_evo_DL_UL; 40-1-15; TCI update during a PDSCH/PUCCH/PUSCH transmission with inter-slot based TDM transmission scheme; 1. Support of TCI update during a transmission with inter-slot based TDM transmission scheme. 40. NR_MIMO_evo_DL_UL; 40-3-1-12; Support of different TCI states for CSI-RSs configured as CMR in a CSI-ReportConfig for CJT; 1. Maximum number of different TCI states for the CSI-RSs configured as CMR in a CSI-ReportConfig for CJT. 40. NR_MIMO_evo_DL_UL; 40-3-1-13; Minimum CSI computation time for CJT CSI report; 1. Minimum CSI computation time for CJT CSI report (candidate value: (Z2, Z2’) or (Z2+r, Z2’+r) ). 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Minimum CSI computation time for CSI report with doppler codebook; 1. Minimum CSI computation time for CSI report with doppler codebook (candidate value: capability 1 or capability 2). 40. NR_MIMO_evo_DL_UL; 40-4-5; Support of MU-MIMO operation from different DMRS types; 1. Support of MU-MIMO operation for current UE and co-scheduling UEs with different DMRS types (Candidate value: Type1+eType1, eType1+Type1, Type2+eType2, eType2+Type2). 40. NR_MIMO_evo_DL_UL; 40-5-1c; Comb offset hopping based on configured subset of comb offsets. 40. NR_MIMO_evo_DL_UL; 40-5-2a; Cyclic shift hopping based on configured subset of cyclic shifts. 40. NR_MIMO_evo_DL_UL; 40-5-3; Support of comb offset hopping and cyclic hopping for the same SRS resource. 40. NR_MIMO_evo_DL_UL; 40-5-8; Support of UL full power per symbol for SRS with TDMed 8Tx. 40. NR_MIMO_evo_DL_UL; 40-5-9; Support of UL full power for SRS configured with <8Tx; 1. Supported number of SRS ports for UL full power for SRS with <8Tx (Candidate value: 2Tx, 4Tx, 2Tx and 4Tx). 40. NR_MIMO_evo_DL_UL; 40-5-10; Support of TDMed SRS with more than 2 symbols based on cyclic mapping. 40. NR_MIMO_evo_DL_UL; 40-6-1-3; Number of PTRS ports for SDM scheme; 1. Number of PTRS ports for SDM scheme (Candidate value: 1, 2 or both). 40. NR_MIMO_evo_DL_UL; 40-6-2c; Support of 2 PTRS ports for SFN scheme. 40. NR_MIMO_evo_DL_UL; 40-6-3-1; Support of the same HARQ process for the partially/fully overlapped PUSCHs associated with different CORESETPoolIndex. 40. NR_MIMO_evo_DL_UL; 40-6-3-2; Support of UCI multiplexing when on PUCCH overlaps with PUSCHs associated with different CORESETPoolIndex. 40. NR_MIMO_evo_DL_UL; 40-7-6; Support of UCI multiplexing on PUSCH with 2 codeword. |
| CATT [9] |  |
| Ericsson [10] | In our understanding, the following feature groups from the Rel-17 TCI framework can be reused: 23-1-1c: SCell BFR 23-1-1d: Per BWP TCI state configuration 23-1-1e: Support of reference CC for TCI state configuration 23-1-1g: Beam misalignment 23-1-1h: Association between PC parameters and TCI state 23-1-1i, 23-1-1m, 23-1-1j: Configuration of a single Rel-17 TCI state for PDCCH, CSI-RS, SRS and CORESET#0 In our understanding, none of the above FGs are related to multi-TRP operation, or multiple indicated TCI states, and hence the Rel-17 FGs can be reused. In RAN1#113, RAN1 made the following agreement: Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, - for the baseline feature, the UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) - as an optional feature, the overlapping duration of the later of the two UL transmissions is reduced. - FFS: for the optional feature, whether or not the overlapping duration needs to be specified as 1 (in case 2) or 2 (in case 1) OFDM symbols where - Case 1 applies when UE is capable of supporting MRTD > CP, SCS=60 kHz and frequency range is FR1. Case 2 applies in all other cases We propose to add the new FG 40-2-9 according to this agreement: Allow scheduling overlapping UL transmissions for UEs not supporting STxMP. In RAN1#113, it was agreed to support hopping over a subset of cyclic shifts and comb offsets, as a UE-optional feature: Agreement (RAN1#113) Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature. Two new FGs are needed for the above agreement, one for comb offset hopping and one for cyclic shift hopping, with FG 40-5-1 and 40-5-2 as prerequisites, respectively. In RAN1#113, it was further agreed to support finer-granularity cyclic-shift hopping, as a UE-optional feature: Agreement (RAN1#113) For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. A new FG is needed to support finer-granularity cyclic-shift hopping, with FG 40-5-2 as a prerequisite. In RAN1#113, it was also agreed that cyclic-shift hopping can be combined with legacy group/sequence hopping, depending on UE capability: Agreement (RAN1#113) Whether SRS cyclic shift hopping can be combined with one of group/sequence hopping on an SRS resource depends on UE feature/capability design. A new FG is needed to indicate if cyclic-shift hopping can be combined with legacy group/sequence hopping, with FG 40-5-2 as a prerequisite. Finally, in RAN1#113, the following was agreed: Agreement (RAN1#113) SRS comb offset hopping and cyclic shift hopping can be configured for an SRS resource at the same time as a separate UE capability. No joint hopping scheme is supported. A new FG is needed to support comb offset and cyclic shift hopping at the same time. Both FG 40-5-1 and 40-5-2 are prerequisites for this FG. 40-1-b; Unified TCI with joint DL/UL DCI-based TCI update for single-DCI based multi-TRP; - The maximum number of MAC-CE activated joint TCI states per CC Minimum beam application time. 40-1-1d; Support [TCI selection] field for DCI format 1_1 and 1_2; Minimum time duration between PDCCH and PDSCH for switching between single-TRP and multi-TRP. 40-1-2b; Unified TCI with separate DL/UL DCI-based TCI update for single-DCI based multi-TRP; - The maximum number of MAC-CE activated DL TCI states per CC - The maximum number of MAC-CE activated UL TCI states per CC - Minimum beam application time. 40-1-3b; Per aperiodic CSI-RS resource set configuration for TCI selection in S-DCI based MTRP. 40-1-4b; Support of two TCI states for CJT Tx scheme B for PDSCH. 40-1-5; TRP-specific BFR with unified TCI framework for single-DCI based multi-TRP. 40-1-7b; Unified TCI with joint DL/UL DCI-based TCI update for multi-DCI based multi-TRP; - The maximum number of MAC-CE activated joint TCI states per CC - Minimum beam application time. 40-1-9b; Unified TCI with separate DL/UL DCI-based TCI update for multi-DCI based multi-TRP; - The maximum number of MAC-CE activated DL TCI states per CC - The maximum number of MAC-CE activated UL TCI states per CC - Minimum beam application time. 40-1-15; TRP-specific BFR with unified TCI framework for multi-DCI based multi-TRP. 40-1-16; Per aperiodic CSI-RS resource set configuration for TCI selection in M-DCI based MTRP. 40-1-17; Per aperiodic CSI-RS resource set configuration for TCI selection in M-DCI based MTRP. 40-1-18; Multi-TRP schemes supported with the unified TCI framework; - Single-DCI NC-JT - Multi-DCI NC-JT - PDCCH repetition - PUCCH repetition - PUSCH repetition - HST-SFN. 4-2-9; Allow scheduling overlapping UL transmissions for UEs not supporting STxMP. 40-5-6; SRS with TDM. 40-5-8; Hopping within a subset for SRS comb offset hopping. 40-5-9; Hopping within a subset for SRS cyclic shift hopping. 40-5-10; Finer-granularity SRS cyclic shift hopping. 40-5-11; SRS cyclic shift hopping combined with legacy group/sequence hopping. 40-5-12; Support comb offset hopping and cyclic shift hopping at the same time. |
| Apple [11] | 40. NR_MIMO_evo_DL_UL; 40-1-5; For unified TCI framework for single-DCI multi-TRP, spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission; - For unified TCI framework for single-DCI multi-TRP, support of spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission that is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission. Note: This is still an FFS since RAN1#112. Depends on whether this is supproted by Rel-18, needs further discussion; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-6; Unified TCI framework for single-DCI multi-TRP, DCI format 1_1/1_2 configured with or without the [TCI selection field]; - For unified TCI framework for single-DCI multi-TRP, support of DCI format 1_1/1_2 configured with or without the [TCI selection field]; Component 1 candidate value: {without the [TCI selection field], with the [TCI selection field], both}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-2-3; Overlapping UL transmission reduction; - Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions, when the UE does not support UL STx2P (Simultaneous Transmission cross 2 Panels), for multi-DCI based Multi-TRP operation with two TA enhancement; Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions); Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-2-7; Maximum number of n-TimingAdvanceOffset value per serving cell; - Maximum number of n-TimingAdvanceOffset value supported per serving cell; Component 1 candidate value: {1, 2 }; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-1-24; CPU counting for multi-TRP CJT; - Scaling factor X for CPU occupation of multi-TRP CJT CSI based on Type-II codebook; Component 1 candidate values: {1, 2, 3, 4} Note: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU =X * NTRP; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-1-25; Aperidoic CSI report timing relaxation for multi-TRP CJT; - Aperidoic CSI report timing relaxation, r, for multi-TRP CJT; Component 1 candidate values: UE reports candiate value independently for each SCS in unit of symbols For 15kHz SCS: {0, 4, 8, 16, 32} For 30kHz SCS: {0, 8, 16, 32, 64} For 60kHz SCS: {0, 16, 32, 64, 128} For 120kHz SCS: {0, 16, 32, 64, 128} For 480kHz SCS: {0, 64, 128, 256, 512} For 960kHz SCS: {0, 128, 256, 512, 1024} Note: Z/Z’ required for multi-TRP CJT CSI is Legacy Z/Z’ values + r where Legacy Z/Z’ values are defined as Z2/Z'2 in Table 5.4-2 in TS38.214; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-2a; Support Rel-16-based predicted PMI codebook with parameter combination with L=6; - Support of Rel-16 eType-II regular codebook refinement for predicted PMI with parameter combination with L=6; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-2b; Support Rel-16-based predicted PMI codebook with rank 3,4; - Support of Rel-16 eType-II regular codebook refinement for predicted PMI with rank 3,4; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Support Rel-17-based predicted PMI codebook with rank 3,4; - Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with rank 3,4; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-9; Support X=1 based on averaging first and last slot of WCSI, for Rel-17-based predicted PMI codebook; - Support X=1 CQI associated with the first and the last slot of the CSI reporting window for Rel-17-based predicted PMI codebook; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-9a; Support X=2 CQI based on 2 slots for Rel-17-based predicted PMI codebook; - Support X=2 CQI for Rel-17-based predicted PMI codebook - The 1st CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI, and - The 2nd CQI is associated with the middle slot of the CSI reporting window and the predicted PMI in the middle; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-10; CPU counting for predicted PMI codebook based on Type-II codebook enhancement; - Scaling factor Y for CPU occupation of predicted PMI codebook based on Type-II codebook when P/SP CSI-RS is configured for CMR - Scaling factor Y for CPU occupation of predicted PMI codebook based on Type-II codebook when AP CSI-RS is configured for CMR; Component 1 candidate values: {2, 4, 6, 8} Component 2 candidate values: {1, 2, 4, 8, N4} Note: When N4=1, OCPU =4 OCPU ≥ 4 when P/SP CSI-RS is configured for CMR OCPU = Y * N4 when P/SP CSI-RS is configured for CMR OCPU = Y * K when AP CSI-RS is configured for CMR N4 = {1,2,4,8}, is the number of predicted PMI K = {4,8,12}, is the number of AP CSI-RS resources for the CMR in a CSI report setting; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-11; Aperidoic CSI report timing relaxation for predicted PMI codebook based on Type-II codebook; - Aperidoic CSI report timing relaxation, r, for predicted PMI codebook based on Type-II codebook - Aperidoic CSI report timing relaxation, w, for predicted PMI codebook based on Type-II codebook - Aperidoic CSI report timing relaxation, Z’, for predicted PMI codebook based on Type-II codebook; Component 1 candidate values: UE reports candiate value, r, independently for each SCS in unit of symbols For 15kHz SCS: {0, 4, 8, 16, 32} For 30kHz SCS: {0, 8, 16, 32, 64} For 60kHz SCS: {0, 16, 32, 64, 128} For 120kHz SCS: {0, 16, 32, 64, 128} For 480kHz SCS: {0, 64, 128, 256, 512} For 960kHz SCS: {0, 128, 256, 512, 1024} Component 2 candidate values: UE reports candiate value, w, independently for each SCS in unit of symbols For 15kHz SCS: {0, 4, 8, 16, 32} For 30kHz SCS: {0, 8, 16, 32, 64} For 60kHz SCS: {0, 16, 32, 64, 128} For 120kHz SCS: {0, 16, 32, 64, 128} For 480kHz SCS: {0, 64, 128, 256, 512} For 960kHz SCS: {0, 128, 256, 512, 1024} Component 3 candidate values: {CAP1, CAP2} Note: Z’ = Legacy Z’ values + r For CAP1 in component 3 1) For AP CSI-RS: Z=legacy Z+14*(K–1)*m 2) For P/SP CSI-RS: Z= legacy Z+w For CAP2 in component 3 1) For AP CSI-RS: Z= legacy Z+14*(K–1)*m + r 2) For P/SP CSI-RS: Z= legacy Z+w+r where Legacy Z/Z’ values are defined as Z2/Z'2 in Table 5.4-2 in TS38.214 K = {4,8,12}, is the number of AP CSI-RS resources for the CMR in a CSI report setting M = {1,2}, is the offset between two adjacent AP CSI-RS resources for the CMR in slots; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-2-12; Acctive P/SP CSI-RS resource counting relaxation for predicted PMI codebook based on Type-II codebook; - Acctive P/SP CSI-RS resource counting relaxation, Kp for predicted PMI codebook based on Type-II codebook; Cmponent 1 candidate values: {1,2,4} Note: For P/SP CSI-RS, one resource is counted as occupying KP ≥1 active resource(s); Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-3-7; CPU counting for TDCP report; - Scaling factor X for CPU occupation of TDCP report; Component 1 candidate values: {1,2,3,4} Note: OCPU=(Y+1) * X Y = {1,2,3,4} is number of configured delay values; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-3-3-8; CSI-RS for TDCP report; - Supported max number of simultaneous NZP-CSI-RS resources configured to measure TDCP within a slot in active BWPs across all CCs - Supported max number of simultaneous NZP-CSI-RS resources configured to measure TDCP within a slot in active BWP per CC; Component 1 candidate values: {2,4,6,8,12,16,20, 24,28,32,40,48,56,64} Component 2 candidate values: {2,4,6,8,12,16,20,24,28,32}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-4-1i; [Place Holder] MU-MIMO with Rel.18 DMRS ports for PDSCH when antenna ports are in more than one CDM group; Support of MU-MIMO between Rel.18 DMRS ports and Rel.15/Rel.18 DMRS ports for PDSCH when antenna ports are in more than one CDM group; Depending on whether explicit restriction would be agreed in the spefiicaiton; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2a; SRS cyclic shift hopping combined with group/sequence hopping on an SRS resource; - Support of SRS cyclic shift hopping combined with group / sequence hopping on an SRS resource.; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Configuration of subset of comb offsets for comb offset hopping; - Support of configuration of a subset of comb offsets when comb offset hopping is configured; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-2b; Configuration of subset of cyclic shifts for cyclic shift hopping; - Support of configuration of a subset of cyclic shifts when cyclic shift hopping is configured; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; Finer time-delay-domain granularity for SRS cyclic shift hopping; - Support of finer time-delay-domain granularity for SRS cyclic shift hopping; Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-6-2c; Dynamic switching between Single-DCI STx2P SFN scheme and Single-TRP scheme - codebook based; - Support of dynamic switching between Single-DCI STx2P SFN scheme and Single-TRP scheme for codebook based PUSCH by DCI formats 0_1/0_2; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-2d; Dynamic switching between Single-DCI STx2P SDM scheme and Single-TRP scheme - non-codebook based; - Support of dynamic switching between Single-DCI STx2P SFN scheme and Single-TRP scheme for non-codebook based PUSCH by DCI formats 0_1/0_2; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3c; Multi-DCI STx2P scheme – fully overlapping in frequency, partially overlapping in time for codebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs fully overlapping in frequency and partially overlapping in time for codebook based PUSCH; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3d; Multi-DCI STx2P scheme – partially or non-overlapping in frequency, partially or fully overlapping in time for codebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and partially or fully overlapping in time for codebook based PUSCH; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3e; Multi-DCI STx2P scheme – fully overlapping in frequency, partially overlapping in time for noncodebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs fully overlapping in frequency and partially overlapping in time for noncodebook based PUSCH; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-6-3f; Multi-DCI STx2P scheme – partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook based PUSCH; - Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and partially or fully overlapping in time for noncodebook based PUSCH; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-4c; UL full power transmission mode of fullpowerMode2 for 8 Tx - SRS resource; - The supported SRS configurations with different number of antenna ports per SRS resource for full power Mode 2 when UE is capable of and configured with 8 Tx codebook based PUSCH operation; Component (1) candidate values: {1_8, 1_2_8, 1_4_8, 1_2_4_8} 1st state (1_8): each SRS resource can be configured with 1 port or 8 ports 2nd state (1_2_8): each SRS resource can be configured with 1 port or 2 ports or 8 ports 3rd state (1_4_8): each SRS resource can be configured with 1 port or 4 ports or 8 ports 4th state (1_2_4_8): each SRS resource can be configured with 1 port or 2 port or 4 ports or 8 ports; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-4d; UL full power transmission mode of fullpowerMode2 for 8 Tx - full power TPMI groups; - TPMI group(s) which delivers full power when UE is capable of and configured with 8 Tx codebook based PUSCH operation; FFS; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-4e; UL full power transmission mode of fullpowerMode2; FFS How to support downgrade configuration, i.e., UE is configured with 2/4 Tx while UE supports 8Tx Whether and how to use the legacy FG16-5c, FG16-5c-2, FG16-5c-3. 40. NR_MIMO_evo_DL_UL; 40-7-1a-1; PUSCH codebook coherency subset when an 8 Tx capable UE is configured with 2 Tx codebook based PUSCH operation; - Supported codebook coherency subset type when an 8 Tx capable UE is configured with 2 Tx codebook based PUSCH.; Component 1 candidate values: {non-coherent, non-coherent and full-coherent}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-1a-2; PUSCH codebook coherency subset when an 8 Tx capable UE is configured with 4 Tx codebook based PUSCH operation; - Supported codebook coherency subset type when an 8 Tx capable UE is configured with 4 Tx codebook based PUSCH operation.; Component 1 candidate values: {non-coherent, non-coherent and partial coherent, non-coherent and partial coherent and full-coherent} FFS: whether to resuse legacy FG2-13 assuming UE reports that UE supports 4Tx in legacy; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-7-6; CBG-based re-transmission for UL using CBGTI for 2 CW; - Support of CBG-based re-transmission for UL using CBGTI when NW configures a maximum rank larger than 4 for PUSCH operation.; Optional with capability signaling. |
| Xiaomi [12] | Refer to TS 38.306 [2], the following two UE feature were defined that UE need to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by TCI state field in DCI formats 1_1 and 1_2. But in Rel-18, a new DCI field, i.e., [TCI selection field] will be introduced into DCI formats 1_1 and 1_2 to indicate the TCI state for PDSCH. Thus it is necessary to define a new UE capability to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by [TCI selection field] in DCI formats 1_1 and 1_2. sfn-SchemeA-DynamicSwitching-r17 Indicates whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1 and 1_2. The UE supporting this feature shall indicate sfn-SchemeA-r17 or sfn-SchemeA-PDSCH-only-r17.; FS; No; N/A; N/A. sfn-SchemeB-DynamicSwitching-r17 Indicates whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme B by TCI state field in DCI formats 1_1 and 1_2. The UE supporting this feature shall indicate sfn-schemeB-r17 or sfn-schemeB-PDSCH-only-r17.; FS; No; N/A; N/A. Proposal 2-2: define a new UE capability to indicate whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A/B by [TCI selection field] in DCI formats 1_1 and 1_2. According to the following agreement, the number of supported SD basis combination can be reported based on UE capability. Agreement (RAN1-112 meeting) On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement FFS: Whether the total number of Ln is a UE capability NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. Proposal 4-1: define a UE capability to indicate the total number of supported SD basis combinations for CJT. According to the following agreement, the supported subset of linkages can be reported based on UE capability. Proposal 4-2: define a UE capability to indicate the supported subset of linkages for CJT. Agreement (RAN1-112b e-meeting) On the Parameter Combination of Type-II codebook refinement for CJT mTRP, only the following linkages are supported (marked ‘x’), for Rel-16 eType-II based - For NTRP =1, - fully reuse seven out of the eight Parameter Combinations from Rel-16 eType-II as indicated in the table below - FFS (by RAN1#112bis-e): whether to add one more Parameter Combination for L=4 based on the legacy Rel-16 eType-II FD combo {½, ½, ¼, ¼; ½} or the agreed FD combo {½, ½, ½, ½; ½}, or not to add from the indicated seven below - For NTRP >1, only the following linkages are supported (marked ‘x’) - Note: Configured linkage(s) are associated with the configured value of NTRP, regardless whether the dynamic TRP selection (the dynamic change of N given NTRP) is configured. Also, the configured linkage(s) are valid for any dynamically selected SD basis and/or any dynamically selected CSI-RS resource (TRP). - FFS: UE feature/capability to support only a subset of linkages NTRP; SD combo; FD combo {pv},. {1/8, 1/8, 1/16, 1/16}, ¼; {1/8, 1/8, 1/16, 1/16}, ½; {1/4, ¼, 1/8, 1/8}, ¼; {1/4, ¼, 1/8, 1/8}, ½; {1/4, ¼, ¼, ¼}, ¾; {1/2, ½, ½, ½}, ½. 1; 2; x; x. 4; x; x; x. 6 w/ restriction; x; x. 2; {2,2}; x. {2,4} {4,2}; x. {4,4}; x; x; x. 3; {2,2,2}; x; x. {2,2,4} {2,4,2} {4,2,2}; x; x. {4,4,4}; x; x; x; x; x; x. 4; {2,2,2,2}; x; N/A. {2,2,2,4}; x; N/A. {2,2,4,4}; x; x; N/A. {4,4,4,4}; x; x; x; N/A. Agreement(RAN1-113 meeting) For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ According to the following agreement, the supported value of Z/Z’ can be reported based on UE capability for NTRP>1.Proposal 4-3: define a UE capability to indicate the supported value of Z/Z’ for NTRP>1 for CJT. 40. NR_MIMO_evo_DL_UL; 40-5-4a; SRS with 8 Tx ports—codebook; FFS: separate row for comb8 Support of non-TDMed 8 port SRS for codebook; ,2-52,23-8-5,23-8-8; Yes; Non-TDMed 8-port SRS for Codebook is not supported; per band; n/a; n/a; n/a; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-6; SRS with TDMed 8 Tx ports—antenna switching; FFS: separate row for comb8 - Support of TDMed 8 port SRS for antenna switching - Supported number of subset s=2; 2-53，2-55，23-8-5，23-8-8; Yes; TDMed 8-port SRS for AS is not supported; per band; n/a; n/a; n/a; Component candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8,t2r8,t4r8,t8r8}; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-5-7; SRS with TDMed 8 Tx ports—codebook; FFS: merge with antenna switching - Support of TDMed 8 port SRS for codebook - Supported number of subset s=2; 2-52，2-55，23-8-5，23-8-8; Yes; Non-TDMed 8-port SRS for Codebook is not supported; per band; n/a; n/a; n/a; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | New FG 40-2-9: TAG ID in absolute TAC MAC CE - Component: Support of indication of TAG ID in absolute MAC CE. - Prerequisite FG: FG 40-2-1 or 40-2-2, FG 9-1 (2-step RACH) New FG - The following agreement (especially the part of Note on L=6) should be reflected. FG16-3a-2 in Rel-16 eType-II CSI can be a good reference. Agreement For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, at least the following Parameter Combinations are supported L; pυ; β. υ ∈1,2; υ ∈3,4. 4; 1/4; 1/4; 1/4. 4; 1/4; 1/4; 1/2. 4 (*); 1/2; 1/4; 1/2. 4 (*); 1/4; 1/4; 3/4. 6 (*); 1/4; --; 1/2. 6 (*); 1/4; --; 3/4. (*) Note: From legacy. For L=6, the same restriction and UE optionality as legacy apply - Select at most 3 additional Parameter Combinations from the list below L; pυ; β. υ ∈1,2; υ ∈3,4. 2; 1/8; 1/16; 1/4. 2; 1/8; 1/16; 1/2. 2(*); ¼; 1/8; ¼. 2 (*); ¼; 1/8; ½. 4; 1/8; 1/16; 1/4. 4 (*); ¼; 1/8; 1/4. (*) Note: From legacy. - FFS: UE feature/capability to support only a subset of Parameter Combinations. 16-3a-2; Support of parameter combinations 7-8; 1. Support of parameter combinations 7-8; 16-3a; paramComb7-8-r16; CodebookParametersAddition-r16; n/a; n/a; Optional with capability signaling. New FG/components - The following agreement (especially the sub-bullet in the 3rd main bullet, i.e., the maximum number of active resources for TDCP) should be reflected. In our view, this can be defined as another component of TDCP basic feature. Agreement For the Rel-18 TRS-based TDCP reporting, when Y delay(s) are configured - OCPU=(Y+1).X where X≥1 is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X can depend on the value of D, and whether phase reporting is switched ON - Reuse legacy Z2/Z2’ values - To count active resources used for TDCP reporting, reuse the legacy number counting mechanism for CSI-RS resources - UE reports the maximum number of active resources for TDCP in UE capability reporting.. - The following agreement also needs to be reflected. That is, for D=6 and D=10, they are supported and defined as a separate capability - Note that the part of WA has been confiemed already, with a change that the support of D=10 is only for the operation with 30 kHz SCS or larger. Agreement For the Rel-18 TRS-based TDCP reporting, - Support the following D (delay) values: 4 symbols, 1 slot, 2 slots, 3 slots, 4 slots, 5 slots - Working assumption: Support the following D (delay) values in a separate UE Feature Group: 6 slots, 10 slots FFS: The value of Dbasic FFS: Applicability of each D value candidate for different SCS values and/or other parameters (e.g. Y, quantization). New FG - We believe we need to add the following FG based on the following RAN1 agreement. We understand FG 40-6-1-2 for STxMP is for Rel.15 DMRS ports. - FG40-4-X: New DMRS port entry for single-DCI based SDM STxMP for Rel.18 DMRS ports Agreement (RAN1#113) For Rel.18 eType 1/eType 2 DMRS with maxLength= 1/2 for PUSCH, additionally support the following rows. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). Y; 2; 0,2,3. Table 7.3.1.1.2-14-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. Y; 2; 0,2,3; 1. Table 7.3.1.1.2-14-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). Y; 2; 0,2,3. Table 7.3.1.1.2-22-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. Y; 2; 0,2,3; 1.. New FG - According to the following agreements, new FGs seem to be necessary for below: - For the support of subset configuration for comb offset hopping - 40-5-1 should be a prerequisite FG - For the support of subset configuration for cyclic shift hopping - 40-5-2 should be a prerequisite FG - For the support of finer time-delay-domain granularity for cyclic shift hopping - 40-5-2 should be a prerequisite FG - For the support of SRS cyclic shift hopping combined with legacy group/sequence hopping Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature. Agreement For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. Agreement Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.. New FG 40-6-1c: CSI-RS processing framework for SRS with two associated CSI-RS resources - Similar as FG 23-3-1-2b in Rel-17 MTRP PUSCH repetition, following FG related to CSI-RS processing framework for SRS with two associated CSI-RS resources should be included. - Component: following components can be included. - Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}. - Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}. - Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. Candidate value {0 to 8}. - UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. Candidate value {1 to 16}. - UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously. Candidate value {1, 2}. - Prerequisite FG: 40-6-1b New FG 40-6-2c: CSI-RS processing framework for SRS with two associated CSI-RS resources - Component: Similar as 40-6-1c, following components can be included. - Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}. - Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}. - Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. Candidate value {0 to 8}. - UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. Candidate value {1 to 16}. - UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously. Candidate value {1, 2}. - Prerequisite FG: 40-6-2b New FG 40-6-3a-1: support of CG transmission for codebook based multi-DCI based STx2P PUSCH+PUSCH - Component: following components can be included. - Support of DG PUSCH+CG PUSCH - Support of CG PUSCH+CH PUSCH - Prerequisite FG:40-6-3a New FG 40-6-3b-2: CSI-RS processing framework for SRS with two associated CSI-RS resources - Component: Similar as 40-6-1c, following components can be included. - Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}. - Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}. - Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. Candidate value {0 to 8}. - UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. Candidate value {1 to 16}. - UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously. Candidate value {1, 2}. - Prerequisite FG: 40-6-3b New FG 40-6-3b-3: support of CG transmission for non-codebook based multi-DCI based STx2P PUSCH+PUSCH - Component: following components can be included. - Support of DG PUSCH+CG PUSCH - Support of CG PUSCH+CH PUSCH - Prerequisite FG:40-6-3b |
| OPPO [14] | In the achieved progress, the following UE capabilities were discussed and agreed. - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - RRC configuration on aperiodic CSI-RS for CSI/BM to apply the first or second indicated DL/joint TCI state per CSI-RS resource set or per CSI-RS resource is up to UE capability In RAN1#113 meeting, a few of progress on unified TCI framework extension have been made. Hence we update the UE capabilities accordingly which are to be discussed as listed below. - For S-DCI MTRP, support of dynamic switching between Rel-17 TCI framework and Rel-18 TCI framework - For PDSCH-CJT, the support of PDSCH DMRS new QCL rule, i.e. QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} is up to UE capability - Support of UE determines two UL Tx power values for PUCCH/PUSCH STxMP based on two UE-configured maximum output power (if per panel or per indicated UL/joint TCI states maximum output power is supported) Proposal 1: The above information can be considered and reported via UE capability (if supported) for Rel-18 unified TCI state extension. In previous agreement, the following UE capabilities were discussed and agreed. - For the case of two DL reference timing, support of Rx timing difference between two DL reference timings can be assumed as larger than a CP length is up to UE capability For intra-cell MTRP and inter-cell MTRP, the following UE capabilities should be discussed for two TA enhancement. In RAN1#113, a few of progress has been made and we update the items according to it. - For both inter-cell and intra-cell MDCI MTRP, support of two TAGs associated with one serving cell - Inter-cell and intra-cell MDCI MTRP should be separate features - Support of associated TAG to UL/joint TCI state - No split between joint TCI state and UL TCI state, as one row/FG) - For inter-cell M-DCI MTRP, support of cross-TRP RACH triggering by PDCCH order - Support of PRACH configuration per additionalPCI - Support of reducing the overlapped duration in case the UE does not support STxMP Proposal 2: The above information can be considered and reported via UE capability (if supported) for Rel-18 two TA enhancement. It was agreed that a set of NL combinations of values for {L1, ..., LNTRP} could be gNB-configured via RRC signaling for SD basis selection, and candidate combinations of {Ln} were also agreed. Based on the agreed combinations, the total number of Ln can be up to 16 for one layer, which is significantly lager than the value with S-TRP. Considering the UE complexity would be increased accordingly, the supported total number of SD basis across all CSI-RS resources should be reported via UE capability, and then gNB can configure a proper combination based on the reporting. Furthermore, to support CJT CSI, some fundamental features should also be reported by UE, e.g. the supported number of TRP for CJT, the supported number of total antenna ports for CJT, supported number of CSI-RS ports per TRP. For Rel-18 medium/high mobility codebook enhancement, expect the agreed UE features, some other parameters should also be considered. For example, for the values of N4, it was agreed in RAN1#111 that the candidate values supported by UE are reported via UE capability. At least UE can report the supported maximal value of N4 with candidate values of {1,2,4,8}. The support of the values of K larger than 4 should also be reported as UE capability, which is highly related to the UE complexity. Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), support 8 as an additional candidate value - FFS (by RAN1#112): Whether any of the following additional candidate values are supported: 3, 5, 16, 32 - The candidate values supported by UE are reported via UE capability (details can be discussed in UE feature).. Proposal 3: The following information can be additionally reported via UE capability for Rel-18 CJT codebook enhancement: - The supported total number of SD basis Ltot across CSI-RS resources for one layer - The supported number of TRPs NTRP for CJT (e.g. 2,3,4) - Supported number of CSI-RS ports per TRP for CJT Proposal 4: The following information can be reported via UE capability for Rel-18 medium/high mobility codebook enhancement: - Supported maximal values of N4 lager than 1 (e.g. 2,4,8) for Rel-16 etypeII codebook - Support of the values of K larger than 4 (e.g. 8) - Supported values of δ>0 (e.g.,1,2) In currently agreed UE feature, supporting Rel-18 DL DMRS for S-DCI based mTRP and M-DCI based mTRP can be reported as separate UE capability. Similarly, supporting UL DMRS for sTxMP should also be a optional UE feature since the two features need different UE processing capability. Whether single DCI based STxMP (or even SDM with Rel-18 DMRS and SFN with Rel-18 DMRS) and multi-DCI based STxMP need separate capabilities can be further discussed. Similar to Rel-16, DMRS port {0,2,3} was also agreed for Rel-18 DLDMRS to support S-DCI based m-TRP. For a UE supporting Rel-18 DL DMRS for single DCI based M-TRP, ports {0,2,3} can be naturally supported since it is a basic feature for SDM transmission. No additional UE feature for the port combination is needed in addition to FG 40-4-5 and a corresponding note can be added to FG40-4-5. Proposal 5: Support Rel-18 UL DMRS for STxMP can be reported as UE capablity. - FFS whether single DCI based STxMP and multi-DCI based STxMP need separate capabilities. Proposal 6: For a UE supporting Rel-18 DL DMRS for single DCI based M-TRP, ports {0,2,3} is naturally supported, and no additional row for ports {0,2,3} is needed. - Add a note to FG 40-4-5 that ports {0,2,3} is supported by this FG. Based on the agreements in RAN1#113 meeting, some capabilities can be introduced for Rel-18 SRS enhancement in addition to currently agreed FGs. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, if the UE is capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications. - Note: This may be captured in the specification in a few different but equivalent ways, and it is up to the editor to decide. Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature. Agreement For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., , where nSRScs,offset can be randomly chosen from at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. Agreement Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design. Agreement SRS comb offset hopping and cyclic shift hopping can be configured for a SRS resource at the same time as a separate UE capability. No joint hopping scheme is supported.. Proposal 7: The following information can be additionally reported via UE capability for Rel-18 SRS enhancement: - Whether UE is capable of transmitting at PCMAX in each OFDM symbol for s=2 - Support configuring a subset of comb offsets for comb offset hopping - Support finer time-delay-domain granularity for cyclic shift hopping - SRS cyclic shift hopping combined with legacy group/sequence hopping - Configuration of both comb offset hopping and cyclic shift hopping for a SRS resource It is agreed that the maximal number of layers for SDM/SFN scheme can be different from legacy PUSCH transmission, a separate UE capability to report the supported maximal number of layers for SDM/SFN scheme can be discussed. For multi-DCI based STxMP PUSCH transmission, it is agreed that the maximal number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain can be 1 or 2 subjects to UE capability. Besides, fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain are supported for multi-DCI based STxMP PUSCH transmission. The overlapping types can be reflected in different UE capabilities: - Partial-overlapping in time domain and non-overlapping in frequency domain - Fully overlapping in time domain and partially overlapping in frequency domain - Fully overlapping in both frequency domain and time domain - Fully overlapping in frequency domain, partially overlapping in time domain - partially or non-overlapping in frequency domain, partially or fully overlapping in time domain Based on the agreements in RAN1#113 meeting, following capability can be introduced to support group-based beam reporting. Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability. Proposal 8: The following information can be reported via UE capability for Rel-18 STxMP transmission. - For single-DCI based STxMP PUSCH transmission, - Supported maximal number of layers for SDM/SFN scheme. - For multi-DCI based STxMP PUSCH transmission, - Maximal number of transmission layers for STxMP PUSCH overlapping in time domain can be 1 or 2 subjects to UE capability. - Support of overlapping types, including: fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - Support Alt1 and/or Alt2 for group-based beam reporting for STxMP. In the past meeting, it was agreed that a 8Tx UE can report one or more supported Ng values for gNB configuration. It was also agreed in LS to RAN2 that RRC signaling can indicate one of Ng={1,2,4,8} which correspond to four different codebooks. In this case, the UE feature can use different FGs or different rows to indicate supported different Ng values. Agreement For an 8TX UE, Option 1 is supported, - Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value, based on which, gNB may RRC configure UE with a codebook corresponding to only one of the supported Ng values.. Furthermore, full power mode 2 was agreed in RAN1#113 meeting. Similar to legacy full power mode for 4Tx, the SRS resources for full power mode 2 and supported precoder groups with full power should be reported as UE capability. which depends on the output in 9.1.4.2. Proposal 9: The following information can be additionally reported via UE capability for codebook based 8Tx transmission: - Supported one or more codebook type(s) corresponding to Ng={1,2,4,8} - The maximum number of SRS resources in one SRS resource set with usage set to ‘codebook’ for 8Tx full power Mode 2 - The SRS configuration with different number of antenna ports per SRS resource for 8Tx full power Mode 2, e.g. {1_8, 1_2_8,1_4_8, 1_2_4_8} - Supported precoder groups for 8Tx full power Mode 2 |
| Samsung [15] | Dynamic TRP(s) selection/switching Two FFSs in the above table related to dynamic TRP(s) selection/switching for single-DCI based MTRP need to be resolved. They are: - FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] - FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH The related RAN1 agreements corresponding to the above two FFSs are provided below: Agreement (PDCCH) On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement (PDSCH) On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value Agreement On unified TCI framework extension for S-DCI based MTRP, the presence of the [TCI selection field] can be RRC-configured per DL BWP - FFS: Whether the presence of the [TCI selection field] can be configured individually for DCI format 1_1 and DCI format 1_2 in the same DL BWP Agreement (PUCCH) On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement (PUSCH) On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission. According to the RAN1 agreements presented above, as different signalling mediums/methods – RRC configurations for control channels and DCI indicators for shared channels – are applied for different channels, same or different beams can be used for PDCCH/PDSCH receptions and PUCCH/PUSCH transmissions. For instance, a RRC configuration is used to inform the UE which of the indicated TCI states/beams to use for PDCCH reception, while a [TCI selection field] in DCI format 1_1/1_2 is used to inform the UE which of the indicated TCI states/beams to use for PDSCH reception (similar settings for uplink transmissions). As the DCI indicator field(s) for shared channels can be absent in the corresponding DCI format(s), e.g., a 0-bit field based on network’s configuration, it would make sense to (i) support the DCI indicator field(s) based on UE capability and (2) differentiate the scenarios of applying the same or different beams for different channels. Based on the above analysis, we propose to address the two FFSs in the above table using two FGs. Proposal 1: support the following two FGs (separate rows from the basic FGs) for Rel-18 eUTCI - FG 40-1-x: support for TCI selection field in DCI format 1_1/1_2 and SRS resource set indicator in DCI format 0_1/0_2 when dedicated PDCCH shares a same TCI state(s) as at least one of the indicated TCI states for PDSCH/PUCCH/PUSCH. - FG 40-1-y: support for TCI selection field in DCI format 1_1/1_2 and SRS resource set indicator in DCI format 0_1/0_2 for other cases (note: UE supporting this FG also supports FG 40-1-x). Maximum number of configured or activated TCI states In Rel-17, following UE capabilities are defined to share UE’s capability on number of supported TCI state: - unifiedJointTCI-r17 - maxConfiguredJointTCI-r17 :indicates the maximum number of configured joint TCI states per BWP per CC in a band - maxActivatedTCIAcrossCC-r17 :indicates the maximum number of MAC-CE activated joint TCI states across all CC(s) in a band - unifiedSeparateTCI-r17 - maxConfiguredDL-TCI-r17 :indicates the maximum number of configured DL TCI states per BWP per CC - maxConfiguredUL-TCI-r17 : indicates the maximum number of configured UL TCI states per BWP per CC - maxActivatedDL-TCIAcrossCC-r17 : indicates the maximum number of MAC-CE activated DL TCI states across all CC(s) in a band - maxActivatedUL-TCIAcrossCC-r17 : indicates the maximum number of MAC-CE activated UL TCI states across all CC(s) in a band The parameters above define the number of TCI state UE supports when UE is connected in sTRP mode. Though RAN1 did not make final decision yet on supported number unified TCI state for mTRP mode operation, it should be a common sense that UE may need more of TCI states to be activated and possibly to be configured. Unless the number of supported active TCI state is reduced per connected TRP. Observation 1: Support of mTRP operation via unified TCI framework may request UE to support more of TCI state in total, not to reduce number of supported TCI states per TRP. There should be two different ways of defining total number of supported TCI states based on Rel-17 capability. First, the total number of supported TCI state of UE is informed as UE capability and network should define number of configured or activated TCI states per TRP separately. Or as second approach, network understand the reported UE capability as a number of TCI states supported per TRP. Considering that no clear motivation to support more of active TCI state per TRP, compared with Rel-17, we prefer 2nd approach. The 1st approach may also need more cooperation between TRPs in case of MDCI based operation. If different numbers of activated TCI state are supported for sTRP and mTRP, RAN1 may define two separated UE capability for each case, or RAN1 may define new UE capability informing number of supported TCI states with clear notification whether the number is a total or per TRP. Proposal 2: RAN1 increase the maximum number of activated TCI state per CC either by defining larger value for the current UE capability, or by defining the capability as per TRP Beam application latency In Rel-18 design, UE has two of indicated beam, 1st TCI state and 2nd TCI state. In case of MPUE, UE has one indicated TCI state per panel, and application of indicated TCI state may mean switching of UE panel, not changing of UE shaped beam. In that sense, the latency at UE beam application ordered by Rel-18 eUTCI could be different with the case ordered by Rel-17 UTCI. Based on this observation, there were discussion whether to introduce new timing threshold defining UE beam switching latency. But as captured above, the actual latency should depend on required UE operation which could be sometime blinded to gNB. Therefore, we propose not to define new UE capability on UE beam switching or UE panel switching latency Proposal 3: RAN1 does not define new UE capability for Re-18 UE beam switching latency or panel switching latency. Application of TCI state for CSI-RS In RAN1 #112b-e, it is agreed that CSI-RS can be associated with 1st or 2nd TCI state either per resource or per resource set depending on UE’s capability: Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability. As agreed, it is up to UE capability whether applicable TCI state is configured per CSI-RS resource or CSI-RS resource set. In our perspective, per resource is more general way actually covering per resource set, and pre resource set is allowed to release more simplified UE operation. So we propose to define per resource set as default mode of UE operation claying easier UE operation: Proposal 4: As a default UE capability, application of 1st or 2nd indicated TCI state is configured per CSI-RS resource set. Application of TCI state for CJT As agreed in RAN1 #110b-e, we need new UE capability informing whether 1 or 2 joint TCI state can be applied for CJT. Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1. In the discussion, RAN1 defined alternatives on how UE understand the indicated TCI state, and it seems supporting of two TCI states should be the major case: Agreement On unified TCI framework extension for S-DCI based MTRP, the following two alternatives are supported for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state Introduce a UE capability on which alternative(s) is supported, and either one of above alternatives can be configured by RRC according to the UE capability Note: In Rel-18, RAN1 has no consensus to support Alt3 - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB. So we propose to define support of 2 TCI states as default UE capability, if UE is supporting Rel-18 CJT. Proposal 5: Define UE capability support of Rel-18 CJT including number of supported TCI states for CJT - If number of supported TCI state is not reported, network assumes 2 TCI states as UE’s support RAN1 has agreed to support Alt1 and/or Alt2 as UE’s capability, and discuss whether one of the alternative is defined as a default UE capability. Proposal 6: Define UE capability indicating which of listed alternatives supported on UE’s application of QCL of indicated two TCI states for CJT operation. Reference timing For the number of reference timing, two alternatives were agreed and alt 1 was final down selection Agreement For multi-DCI multi-TRP operation with two TAs, study the following alternatives: - Alt 1: two reference timings are considered - Alt 2: one reference timing is considered Note: reference timing above is the timing of the DL reception Agreement For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length - FFS: the maximum Rx timing difference (could be up to RAN4) - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected. Therefore, we do not see there is a UE supporting only one reference timing while supporting multiple TA for serving cell. We propose not to define separated UE capability on supported number of reference timing. Proposal 7: RAN1 does not define separated UE capability on number of supported reference timing - Network assumes two reference timing is supported for a UE reporting capability of multiple TA/TAG for serving cell. As aligned approach with another agreement, we propose to define a new capability of supporting timing difference larger than CP. We also propose to define a default UE capability of not supporting timing difference large than CP. Proposal 8: Define a new UE capability of supporting Rx timing difference larger than CP - As a default, Rx timing difference larger than CP is not supported, if parameter is not reported CJT: A new feature group (e.g., FG 40-3-1-A) for OCPU and Z/Z’ is needed based on the agreements below. Our preference is to have X∈1,2,3,4×NL for OCPU = X.NTRP, based on the analysis in our tdoc on 9.1.2 [2], where the X is defined based on UE capabilities and determined by the UE. - Support X∈1,2,3,4×NL if OCPU=XNTRP, or X∈1,2,3,4 if OCPU=XNLNTRP Regarding Z/Z’, based on the agreements below, Capability 1 and Capability 2 need to be supported via another FG. [113] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1 [113] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’. (New) FG 40-3-2-A: Support of K=12 aperiodic NZP CSI-RS resources (according to the agreement below) [112] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR), optionally support only K=12 as an additional candidate value. (New) FG 40-3-2-B: Support of L=6 (according to the agreement below) [112bis-e] Agreement For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, at least the following Parameter Combinations are supported … Note: From legacy. For L=6, the same restriction and UE optionality as legacy apply. A new feature group (e.g., FG 40-3-2-C) for OCPU and Z/Z’ is needed based on the agreements below. Our preference is to have Y∈2,3,4 for OCPU = Y.N4 when P/SP-CSI-RS is configured for CMR, and Y∈1,2,3,4 for OCPU = Y.K, when AP-CSI-RS is configured for CMR, based on the analysis in our tdoc on 9.1.2 [2], where the Y is defined based on UE capabilities and determined by the UE. - Support Y∈2,3,4 when P/SP CSI-RS is configured for CMR, and - Support Y∈1,2,3,4 when AP CSI-RS is configured for CMR. Regarding Z/Z’, based on the agreements below, Capability 1 and Capability 2 need to be supported via another FG. [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’. A new feature group (e.g., FG 40-3-3-A) for Ocpu and Z/Z’ is needed based on the agreements below. Our preference is to have X∈1 when Y delay values ∈{4 symbols, 1 slot}, and X∈2,3 when Y delay values ∈{2,3,4,5,6,10} (in slots), based on the analysis in our tdoc on 9.1.2 [2], where the X is defined based on UE capabilities and determined by the UE. - Support X∈1 when delay values ∈{4 symbols, 1 slot} and - Support X∈2,3 when delay values ∈{2,3,4,5,6,10} (in slots). [113] Agreement For the Rel-18 TRS-based TDCP reporting, when Y delay(s) are configured - OCPU=(Y+1).X where X≥1 is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X can depend on the value of D, and whether phase reporting is switched ON - Reuse legacy Z2/Z2’ values - To count active resources used for TDCP reporting, reuse the legacy counting mechanism for CSI-RS resources - UE reports the maximum number of active resources for TDCP in UE capability reporting.. Basic structure on Rel-18 DMRS eType-I/II for PDSCH As we proposed, the basic structure of UE features for Rel-18 DMRS eType-I/II for PDSCH are totally reused by Rel-15 DMRS Type-I/II for PDSCH, at least for FGs and components. Hence, we think that corresponding pre-requisite, type, and note can be also reused as much as possible. Having said that, we would like to propose the following FGs with pre-requisite, type, and note. In addition, we would like to show our views on each FG as follows: - FG 40-4-1: Basic feature of Rel.18 enhanced DMRS for PDSCH for mapping type A - Regarding component 3) highlighted as yellow above, we support to define as separate FG for supporting DL PTRS for Rel-18 enhanced DMRS since there is no such component in basic feature of Rel-15 DMRS for PDSCH mapping type A and PTRS related feature is defined separately. - Regarding component 4) highlighted as yellow above, we support to capture as a component in FG 40-4-1 which is same as basic feature of Rel-15 DMRS for PDSCH mapping type A. Proposal 10: Support the following table of Rel-18 DMRS eType-I/II for PDSCH by re-using the structure of Rel-15 DMRS Type-I/II for PDSCH as much as possible. Index; Feature group; Components; Pre-requisite; Type; Note. 40-4-1; Basic feature of Rel.18 enhanced DMRS for PDSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.; Per FS. 40-4-1a; Basic feature of Rel.18 enhanced DMRS for PDSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; Per FS. 40-4-1b; Support 1+2 symbols Rel.18 enhanced DMRS for PDSCH; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Per FS. 40-4-1c; Support alternative additional DMRS position for Rel.18 enhanced DMRS for PDSCH; Support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1, 5-28; Per FS. 40-4-1d; Support 2 symbols front-loaded Rel.18 enhanced DMRS for PDSCH; Support 2 symbols FL DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Per FS. 40-4-1e; Support 2 symbols front-loaded + 2 symbols additional Rel.18 enhanced DMRS for PDSCH; Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Per FS. 40-4-1f; Support 1+3 symbols Rel.18 enhanced DMRS symbols for PDSCH; Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH; 40-4-1; Per FS. 40-4-1g; Support Rel.18 enhanced DMRS type for PDSCH; Per FS; Candidate value: {type1, both type 1 and type 2}. In addition to the basic structure of Rel-18 DMRS eType-I/II, based on UE feature structure for Rel-15 DMRS, we think that at least the following UE features would be defined. Additional rows for S-DCI & M-DCI based M-TRP In RAN1#112 and RAN1#112b-e, one additional row including DMRS ports as {0, 2, 3}, and number of DMRS CDM group(s) without data as {2} in DMRS port indication table was agreed for Rel-18 DMRS eType-I with maxLength = 1 and 2, and eType-II with maxLength = 1 and 2. Since the related FG including the exact same combination of DMRS ports was agreed for Rel-16 single-DCI based multi-TRP as in FG 16-2b-1b as follows. Index; Feature group; Components; Pre-requisite; Type; Note. 16-2b-1b; Single-DCI based SDM scheme – Support of new DMRS port entry; 1. Support of new DMRS port entry {0, 2, 3}; 16-2b-1; Per band. Our view is that if two different FGs are reported from a UE, then it does not imply that the combined functionality of the two FGs from the UE is always supported. Hence, we think that a dedicate FG supporting certain combinations of DMRS ports at least including DMRS ports {0, 2, 3} for Rel-18 DMRS eType-I/II. Since additional rows for S-DCI based M-TRP are still discussing, we can merge in a single FG supporting additional rows for S-DCI based M-TRP based on Rel-18 DMRS eType-I/II. Proposal 12: Support a UE capability introducing additional rows of Rel-18 DMRS eType-I/II for supporting multi-TRP schemes as follows. Index; Feature group; Components; Pre-requisite; Reporting granularity; Note. 40-4-5; Support Rel-18 DL DMRS with single DCI based M-TRP; 16-2b-1, 16-2b-2, 16-2b-3, 16-2b-4, or 16-2b-5; Per FS. 40-4-5a; Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP; Support of new DMRS port entry {0,2,3}; 16-2b-1, 40-4-5; Per band. 40-4-5b; Support Rel-18 DL DMRS with M-DCI based M-TRP; 16-2a; Per FSPC. The maximum number of different DMRS types for all DCI formats Since Rel-18 DMRS eType-I/II are introduced, there are 4 different types of DMRSs: Rel-15 DMRS Type-I/II, and Rel-18 DMRS eType-I/II. Given supportable types of DMRS for each different DCI format defined in NR, a UE shall prepare up to 4 different DMRS types whenever the UE receives same or different DCI formats, which could be burden to UE. Hence, our view is that defining the maximum number of different DMRS types considering for all DCI formats as a UE capability could inform to gNB the processing limit of the UE. Proposal 13: Support a UE capability introducing the maximum number of different DMRS types considering for all DCI formats. Index; Feature group; Components; Pre-requisite; Type; Note. 40-4-XX; Maximum number of different DMRS types for all DCI formats; 40-4-1g; Per FS; Candidate values: {1, 2, 3, 4}. Basic structure on Rel-18 DMRS eType-I/II for PUSCH As we proposed, the basic structure of UE features for Rel-18 DMRS eType-I/II for PUSCH are totally reused by Rel-15 DMRS Type-I/II for PUSCH, at least for FGs and components. Hence, we think that corresponding pre-requisite, type, and note can be also reused as much as possible. Having said that, we would like to propose the following FGs with pre-requisite, type, and note. In addition, we would like to show our views on each FG as follows: - FG 40-4-6: Basic feature of Rel.18 enhanced DMRS for PUSCH for mapping type A - Regarding component 1) ~ 3) highlighted as yellow above, we support to capture as a component in FG 40-4-6 which are same as basic feature of Rel-15 DMRS for PUSCH mapping type A. - Regarding component 4) highlighted as yellow above, we support to define as separate FG for supporting UL PTRS for Rel-18 enhanced DMRS with rank 1-4 since there is no such component in basic feature of Rel-15 DMRS for PUSCH mapping type A and PTRS related feature is defined separately. Proposal 14: Support the following table of Rel-18 DMRS eType-I/II for PUSCH by re-using the structure of Rel-15 DMRS Type-I/II for PUSCH as much as possible. Index; Feature group; Components; Pre-requisite; Type; Note. 40-4-6; Basic feature of Rel.18 enhanced DMRS for PUSCH for mapping type A; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.; Per FS. 40-4-6a; Basic feature of Rel.18 enhanced DMRS for PUSCH for mapping type B; 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol; Per FS. 40-4-6b; Support 1+2 symbols Rel.18 enhanced DMRS for PUSCH; Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6, 40-4-6a; Per FS. 40-4-6c; Support Rel.18 enhanced DMRS type for PUSCH; Per FS; Candidate value: {type1, both type 1 and type 2}. 40-4-6d; Support 2 symbols front-loaded Rel.18 enhanced DMRS for PUSCH; Support 2 symbols FL DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Per FS. 40-4-6e; Support 2 symbols front-loaded + 2 symbols additional Rel.18 enhanced DMRS for PUSCH; Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Per FS. 40-4-6f; Support 1+3 symbols Rel.18 enhanced DMRS symbols for PUSCH; Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH; 40-4-6; Per FS. Comb offset hopping time-domain behavior when repetition factor R>1 In RAN1#112b-e, it was agreed the UE behavior of comb-offset hopping within R symbols if repetition factor R>1 is configured, as follows. The intention of the agreement was compromise of two alternatives by RRC configuration. - Hopping behavior 1: depends on the OFDM symbol index l' of each symbol - Hopping behavior 2: depends on the first symbol across the R repetitions Agreement in RAN1#112b-e For a SRS resource configured with comb offset hopping, if the repetition factor R > 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on RRC configuration, and FFS configuration details. - UE can indicate whether it supports one or both the options. Details to be discussed in UE feature.. In legacy SRS, since frequency domain resources are maintained during R symbols if repetition factor R>1 is configured, we think that the UE feature design shall reflect such legacy UE behavior. In other words, the Hopping behavior 2 above (depends on the first symbol across the R repetitions) can be a default hopping behaviour of comb-offset hopping by a UE, and on top of this, the UE can additionally support Hopping behaviour 1. Hence, we can define a UE feature indicating whether Hopping behavior 2 is only supported, or both Hopping behaviour 1 and 2 can be supported. Proposal 18: Define a FG supporting hopping behavior of comb-offset hopping with repetition factor R>1. 40-5-1a; Comb offset hopping time-domain behavior when repetition factor R>1; 1. Support comb-offset hopping time-domain behavior; 2-52, 40-5-1; Per band; Hopping behaviour 1: hopping depends on the OFDM symbol index l' of each symbol Hopping behaviour 2: hopping depends on the 1st symbol across the R repetitions. Candidate value: {hopping behaviour 2 only, both hopping behaviour 1 and 2}. Subset of comb-offset hopping and cyclic shift hopping In RAN1#113, a concept of a subset has been agreed to limit the hopping region of comb-offsets for comb-offset hopping and cyclic shifts for cyclic shift hopping, as follows: Agreement in RAN1#113 Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature.. Since FGs for comb-offset hopping and cyclic shift hopping are defined separately as in Table. 3, our view is that FGs for subset of comb-offsets and cyclic shifts for comb-offset hopping and cyclic shift hopping, respectively, can be defined separately as well. Hence, we support the following table for supporting subset of comb-offsets and cyclic shifts for comb-offset hopping and cyclic shift hopping, respectively. Proposal 19: Define a FG supporting subset of comb-offsets and cyclic shifts for comb-offset hopping and cyclic shift hopping, respectively. 40-5-XX; Subset of comb-offsets for comb-offset hopping; FFS; 2-52, 40-5-1; Per band; FFS. 40-5-XX; Subset of cyclic shifts for cyclic shift hopping; FFS; 2-52, 40-5-2; Per band; FFS This feature and cyclic shift hopping with finer granularity are not configured with a UE at the same time.. Finer granularity of cyclic shift hopping In RAN1#113, a concept of finer granularity for cyclic shift hopping has been agreed as follows: Agreement in RAN1#113 For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature.. Based on the above agreement, an optional UE feature is needed to be defined, hence we would like to suggest the following FG. Proposal 20: Define a FG supporting finer granularity of cyclic shift hopping. 40-5-XX; Cyclic shift hopping with finer granularity; FFS; 2-52, 40-5-1; Per band; FFS This feature and a subset of cyclic shifts for cyclic shift hopping are not configured with a UE at the same time.. In Rel.15 NR MIMO, the following UE feature groups (FGs) are supported regarding PUSCH transmission from a UE with 1, 2, or 4 antenna ports. - FG 2-12: Basic PUSCH transmission - single port codebook-based PUSCH transmission - FG 2-13: PUSCH codebook coherency subset - one of non-coherent, partial/non-coherent, full/partial/non-coherent - FG 2-14: Codebook based PUSCH MIMO transmission - maximum number of supported layers from {1,2,4} - maximum number of SRS resource per set (with usage = codebook) from {1,2} - FG 2-15: Non-codebook based PUSCH transmission - maximum number of supported layers from {1,2,4} - maximum number of SRS resource per set (with usage = non-codebook) from {1,2,3,4} - Maximum number of simultaneous transmitted SRS resources at one symbol from {1,2,3,4} In Rel.16 eNR MIMO, the following UE feature groups (FGs) are specified regarding full power transmission for non-coherent or partial-coherent precoders. - FG 16-5a: UL full power transmission mode of fullpower - Support for full power based on all full-rated PAs - FG 16-5b: UL full power transmission fullpowerMode1 - Support of full power based on a full-coherent precoder - FG 16-5c: UL full power transmission fullpowerMode2 - Support of full power based on SRS virtualization or full power TPMI groups - FG 16-5c-2: UL full power transmission fullpowerMode2 – SRS resources - Support of full power based on virtualized SRS ports (combing PAs before SRS transmission) - FG 16-5c-3: UL full power transmission fullpowerMode2 – full power TPMI groups - Support of full power based on TPMIs that can achieve full using at least one (but not all) full-rated PA or all non-full-power PAs For 8Tx PUSCH transmission, the UE features can be similar to the legacy (above-mentioned) FGs. Based on the agreements and working assumptions that far, the UE features as shown in Table. 4 can be supported, wherein a brief explanation of each feature is as follows. - FG 40-7-1: basic FG for the support of codebook-based 8Tx PUSCH transmission. Similar to legacy Rel.15 FG, it includes component 1 for the supported codebook parameter(s) (Ng,N1,N2), component 2 for the maximum number of supported layers, and component 3 for the maximum number of SRS resources in a set - FGs 40-7-5 and FG 40-7-1a are not needed, hence should be deleted. - FG 40-7-2: basic FG for support of non-codebook-based 8Tx PUSCH transmission. Similar to legacy Rel.15 FG, it includes component 1 for the maximum number of supported layers, component 2 for the maximum number of SRS resources in a set, and component for the maximum number of simultaneously transmitted SRS resources at one symbol - FG 40-7-3: support for a maximum of 8 layers including component 2 for one or both of the codebook-based and non-codebook-based transmission schemes - FG 40-7-4b: the value = 8 needs to be included in addition to legacy values {1,2,4} - (New) FG 40-7-A: support of a first sub-feature of mode2 that is based on SRS virtualization - (New) FG 40-7-B: support of a second sub-feature of mode2 that is based on full power TPMI group(s) FG 40-7-1: Basic features for Codebook-based 8Tx PUSCH; 1. one or multiple values of (Ng, N1, N2) 2. Supported codebook based 8Tx PUSCH MIMO with maximal number of supported layers 3. Supported max number of SRS resource per set (SRS set use is configured as for codebook).; Component 1: (1,4,1), (1,2,2), (2,-,-), (4,-,-), (8,-,-) Component-2: Candidate value: {1, 2, 4} Component-3 Candidate value: {1, 2}. FG 40-7-2: Basic features for Non-Codebook-based 8Tx PUSCH; 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 3. Maximum number of simultaneous transmitted SRS resources at one symbol; Optional with UE capability Component-1 candidate values: {1, 2, 4} Component-2 Candidate value: {1,2,3,4,5,6,7,8} Component-3 Candidate value: TBD (legacy {1,2,3,4}). FG 40-7-3: Support for up to 8 layers; 1. Maximum number of layers = 8 2. Supported transmission scheme; Candidate value-2: {codebook-based, non-codebook, both}. FG 40-7-4b: UL full power transmission 8TxfullpowerMode2; 1.The maximum number of SRS resources in one SRS resource set with usage set to ‘codebook’ for Mode 2: {1, 2, 4, 8}. (New) FG 40-7-A UL full power transmission 8TxfullpowerMode2 – SRS resources; 1. The SRS configuration with different number of antenna ports per SRS resource for Mode 2; Component 1: TBD. (New) FG 40-7-B UL full power transmission 8TxfullpowerMode2 – full power TPMI groups; 1. TPMI group(s) which delivers full power; Component 1: TBD. |
| Qualcomm Incorporated [16] | We note the following regarding UE features for two TAs for multi-DCI: - Regarding “FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH” in FGs 40-2-1/2, we think a separate FG is needed here. This is because a UE may choose to support 2TAs enhancement only when it supports STxMP feature (multi-DCI), and not for the case of TDM. This is because with STxMP, separate panels are used, and hence, supporting 2 TAs becomes easier from implementation point of view à FG 40-2-3 is proposed below for this purpose - Note that multi-DCI STxMP feature itself is separate (40-6-3a or 40-6-3b), but it does not mean UE also supports 2 TAs. - Regarding “FFS: Maximum number of n-TimingAdvanceOffset value per serving cell” in FGs 40-2-1/2, we also think that a separate FG is needed given that such feature is not essential for supporting two TAGs in the same CC à FG 40-2-7 below is proposed for this purpose - Regarding “FFS: what to do/signal about absolute TA command MAC CE” in FGs 40-2-1/2, it may be ok to wait for further progress in RAN1 given that this feature is agreed for 2-step RACH only so far. - Regarding “FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs” in FGs 40-2-1/2, we are ok either with or without such separation. - For FG 40-2-2, a component on maximum number of additional PRACH configurations is required. This is because even though the max number of such additional RACH configurations is 7, this should not be mandatory for a UE that supports 2 TAs for inter-cell mTRP due to additional memory/complexity requirements. - For FG 40-2-4, the case of intra-cell cross-TRP PDCCH order is a WA in MIMO (only inter-cell case is agreed so far), and it can be simply noted until resolved in the work item. Also, there is no need to merge this FG with 40-2-2 as this is an additional feature, and not part of the baseline. - For FG 40-2-8, the same description of legacy FG “supportedNumberTAG” in 38.306 applies to this FG as well, with the difference that FG 40-2-8 is applicable when at least one CC is configured with 2 TAGs. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, - for the baseline feature, the UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) - as an optional feature, the overlapping duration of the later of the two UL transmissions is reduced. - FFS: for the optional feature, whether or not the overlapping duration needs to be specified as 1 (in case 2) or 2 (in case 1) OFDM symbols where - Case 1 applies when UE is capable of supporting MRTD > CP, SCS=60 kHz and frequency range is FR1. - Case 2 applies in all other cases Lastly, a new FG 40-2-9 is needed to capture the following agreement in the previous RAN1 meeting: - The new FGs 40-5-1c (for comb offset hopping) and 40-5-2c (for cyclic shift hopping) below are added as a result of the following agreement: Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature. Agreement For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission. - Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured. Above is a UE optional feature. The new FG 40-5-2a (for cyclic shif hopping) is added as a result of the following agreement Agreement Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design. The new FG 40-5-2b (for cyclic shif hopping) is added as a result of the following agreement Agreement SRS comb offset hopping and cyclic shift hopping can be configured for a SRS resource at the same time as a separate UE capability. No joint hopping scheme is supported. The new FG 40-5-3 (for combination of comb offset hopping and cyclic shif hopping) is added as a result of the following agreement For STxMP UE feature details, we note the following: - Regarding “FFS: separate row for “2 PTRS ports for SDM scheme”” in FG 40-6-1 / 40-6-1a, we think a separate FG is required which is captured by proposed FG 40-6-1-3. This is because supporting 2 PTRS ports for SDM should not be mandatory for the UE (even though it may be a typical implementation) since UE may use different sub-arrays / sub-modules to effectively form two analog beams, where the LO may be common between the two sub-modules. - Regarding “FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP”” in FGs 40-6-1 / 40-6-1a / 40-6-2 / 40-6-2a, we are ok either with or without a separate FG. - For the detailed components and candidate values of FGs 40-6-1 / 40-6-1a / 40-6-1b / 40-6-2 / 40-6-2a / 40-6-2b: The proposed components are similar to the related Rel-15 FGs 2-14 / 2-15 / 2-15a / 2-15b as well as the related Rel-17 FGs 23-3-1 / 23-3-1-2 / 23-3-1-2a / 23-3-1-2b. In addition, we note the following: - “Supported number of layers associated with one SRS resource set” and “number of used PUSCH ports associated with one SRS resource set” are additionally needed due to simultaneous transmission. - For FGs 40-6-1 / 40-6-1a / 40-6-2 / 40-6-2a, the reporting type should be “per FSPC” given that number of layers is indicated (which is always reported per FSPC given the RF / baseband complexity associated with it). - For FGs 40-6-1b and 40-6-2b, the reporting type can be “per band”. - Regarding the FFS on DG+CG, CG+CG, and DG+DG in FGs 40-6-3a / 40-6-3b (multi-DCI), we think separate FGs are needed for DG+CG and CG+CG as captured by proposed FGs 40-6-3f and 40-6-3g, while DG+DG should be assumed for the baseline FGs 40-6-3a / 40-6-3b. - Regarding the FFS on partial/full overlap in time/frequency in FGs 40-6-3a / 40-6-3b (multi-DCI), similar structure as in FG 16-2a family on DL multi-DCI (16-2a, 16-2a-0, 16-2a-1) can be considered here. In addition, given the additional complexity / RF requierements in case of UL transmissions of partially-overlapping PUSCHs in time domain, we propose the following: - The basic FGs 40-6-3a / 40-6-3b include fully-overlapping PUSCHs in time and non-overlapping in frequency. - Proposed FG 40-6-3c includes partial-overlapping PUSCHs in time and non-overlapping in frequency. - Proposed FG 40-6-3d includes fully-overlapping PUSCHs in frequency and time (similar to Rel-16 FG 16-2a-0). - Proposed FG 40-6-3e includes overlapping PUSCHs in time and partially overlapping in frequency (similar to Rel-16 FG 16-2a-1). - For the detailed components and candidate values of FGs 40-6-3a / 40-6-3b / 40-6-3b-1 (multi-DCI): The proposed components are analoguous to that of FGs 40-6-1 / 40-6-1a / 40-6-1b (for single-DCI) by additionally considering the structure of Rel-16 FG 16-2a (DL multi-DCI), i.e., for the component “Maximum number of PUSCHs per CORESETPoolIndex per slot”. - The reporting type for FGs 40-6-3a / 40-6-3b should be “per FSPC” consistent with the corresponding Rel-16 FG 16-2a for DL multi-DCI. - Regarding the detailed description / components of FG 40-6-4 (SFN PUCCH): The proposed components are similar to the related Rel-17 FGs 23-3-2 / 23-3-3 (TDM PUCCH). Agreement To enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, support the system to configure the UE to report one of the followings: - Alt1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Supporting Alt1 and/or Alt2 is subject to UE capability Regarding the componenet 1 candidate values of FG 40-6-5, the proposed values capture the following RAN1 agreement. Components 2/3/4, their candidate values, and the note are consistent with Rel-17 FG 23-5-1 (Rel-17 group-based L1-RSRP). 40. NR_MIMO_evo_DL_UL; 40-1-14; Two default beams for single-DCI based multi-TRP; Support of default QCL assumption with two TCI states; 40-1-1, 40-1-2; Yes; Per band; N/A; N/A; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-15; Association between joint/DL TCI state and AP CSI-RS based on per CSI-RS resource set for single-DCI based multi-TRP; Support of RRC configuration per CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource; 40-1-1, 40-1-2; Yes; Per band; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-16; [TCI selection field] for single DCI based multi-TRP; Support of [TCI selection] field in DCI format 1_1 and 1_2; 40-1-1, 40-1-2; Yes; Per band; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-17; Indication/configuration of R18 joint/DL TCI states for aperiodic CSI-RS, PDCCH, PDSCH for single-DCI based multi-TRP; Support of indication/configuration of R18 joint/DL TCI states for aperiodic CSI-RS, PDCCH, PDSCH (except for TRS) reusing the Rel-15/16 signaling/configuration design(s); 40-1-1, 40-1-2; Yes; Per band; N/A; N/A; N/A; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-18; Indication/configuration of R18 joint/UL TCI states for SRS for single-DCI based multi-TRP; Support of indication/configuration of R17 TCI states for SRS (except for periodic/semi-persistent SRS for BM) reusing the Rel-15/16 signaling/configuration design(s); 40-1-1, 40-1-2; Yes; Per band; N/A; N/A; N/A; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-19; Two default beams for multi-DCI based multi-TRP; Support of default QCL assumption with two TCI states; 40-1-7, 40-1-9; Yes; Per band; N/A; N/A; FR2 only; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-20; Per-TRP BFR with unified TCI framework for multi-DCI based multi-TRP; Support of per-TRP BFR with unified TCI framework; 40-1-7, 40-1-9; Yes; Per band; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-21; Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP; Common multi-CC TCI state ID update and activation; 40-1-7, 40-1-9; Yes; Per band; N/A; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-1-22; Indication/configuration of R18 joint/DL TCI states for aperiodic CSI-RS, PDCCH, PDSCH for multi-DCI based multi-TRP; Support of indication/configuration of R18 joint/DL TCI states for aperiodic CSI-RS, PDCCH, PDSCH (except for TRS) reusing the Rel-15/16 signaling/configuration design(s); 40-1-7, 40-1-9; Yes; Per band; N/A; N/A; N/A; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-1-23; Indication/configuration of R18 joint/UL TCI states for SRS for multi-DCI based multi-TRP; Support of indication/configuration of R17 TCI states for SRS (except for periodic/semi-persistent SRS for BM) reusing the Rel-15/16 signaling/configuration design(s); 40-1-7, 40-1-9; Yes; Per band; N/A; N/A; N/A; Optional with capability signalling. 40. NR_MIMO_evo_DL_UL; 40-2-3; Two TAs for multi-DCI based STxMP PUSCH+PUSCH; Support of two TAGs in a same CC for Rel-18 multi-DCI based STxMP PUSCH+PUSCH; Yes; N/A; FS; No; No; N/A; Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-2-7; Two n-TimingAdvanceOffset values per CC; Support of two n-TimingAdvanceOffset values per CC; Yes; N/A; band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-2-9; Partial dropping in case of overlap; Support of reducing the overlapping duration of the later of the two UL transmissions, when two logically TDMed UL transmissions overlap due to different TAs; Yes; N/A; band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-3-7; Aperiodic CSI-RS resource; DCI triggered aperiodic CSI-RS resource /resource set; 40-3-3; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-x; Resources for TDCP, beam management, pathloss measurement, BFD, RLM and new beam identification; 1. The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure simultaneously across all CCs in FR1 for any of TDCP measurement, L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification 2. The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in FR1 for any of TDCP measurement, L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification; 16-1g, 40-3-3; Yes; N/A; Per-band Per-BC; No; Yes; Yes; FR1-only Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1a-2a; Support of parameter combinations other than basic components; Support of parameter combinations satisfying N_TRP = {3,4}; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1a-2b; Support of parameter combinations other than basic components; Support of parameter combinations satisfying L_total > 8; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1a-2c; Support of parameter combinations other than basic components; Support of parameter combinations satisfying p_v != {1/8,1/8, 1/16, 1/16}; 40-3-1-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1a-5; Number of CPUs for regular eType-II-CJT CSI, or for port selection FeType-II-CJT CSI; O_CPU counting parameter X>=1: For N_TRP>1, O_CPU = X*N_TRP;; 40-3-1-1, or 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; FFS candidate values of X>=1; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-1a-7; Timeline for regular eType-II-CJT CSI, or for port selection FeType-II-CJT CSI; Timeline relaxation parameter: r (for both Z' and Z);; 40-3-1-1, or 40-3-1-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; FFS candidate values of r; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-8; Support of rank 3,4; Support of rank 3,4; 40-3-2-1; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2-9; Support of rank 3,4; Support of rank 3,4; 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2a-5; Number of CPUs for regular eType-II-Doppler CSI, or for FeType-II-Doppler CSI; O_CPU counting parameter Y: For periodic CSI-RS, O_CPU = Y*N4; For aperiodic CSI-RS, O_CPU = Y*K; 40-3-2-1, or 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; FFS candidate values of Y; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2a-6; Active CSI-RS resources/ports counting for regular eType-II-Doppler CSI, or for FeType-II-Doppler CSI; Counting number Kp for periodic CSI-RS; 40-3-2-1, or 40-3-2-5; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values: Kp={1,2,4}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-2a-7; Timeline for regular eType-II-Doppler CSI, or for FeType-II-Doppler CSI; Timeline relaxation parameter: r (for both Z' and Z); w (for Z); 40-3-2-1, or 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; FFS candidate values of r, w; Optional with capability signaling. 41. NR_MIMO_evo_DL_UL; 40-3-2-10; Number of aperiodic CSI-RS resources K>4, or for FeType-II-Doppler CSI; 40-3-2-1, or 40-3-2-4; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Candidate values for K: {8,12}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-3-x; Active CSI-RS resources and ports for mixed Type-II-Doppler codebook types in any slot; 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; 40-3-2-1,40-3-2-4,2-36,2-40; Yes; N/A; Per-band Per-BC; No; N/A; N/A; Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-Doppler R=1, NULL), (eType-II-Doppler R=2, NULL), (FeType-II-Doppler PS R=1 M=1, NULL), (FeType-II-Doppler PS R=1 M=2, NULL), (FeType-II-Doppler PS R=2 M=2, NULL), (FFS CB combos…)}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-11; New DMRS port entry for SDM scheme for PUSCH; Support of new DMRS port entry {0,2,3} for single-DCI based SDM STxMP PUSCH with Rel-18 DMRS; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-11a; Support Rel-18 UL DMRS with M-DCI based M-TRP; 1. Support Rel-18 UL DMRS with M-DCI based M-TRP; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-11b; Support Rel-18 UL DMRS with M-DCI based M-TRP; 1. Support Rel-18 UL DMRS with M-DCI based M-TRP; YES; N/A; Per FS; NO; NO; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-4-12; PTRS enhancement for S-TRP based PUSCH; 1. Support 1 port PTRS for full coherent PUSCH with more than 4 layers 2. Support 2 port PTRS for partial coherent PUSCH with more than 4 layers 3. Support 2 port PTRS for non coherent PUSCH with more than 4 layers 4. Supported PTRS power boost configurations 5. UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4; YES; N/A; Per FS; NO; NO; N/A; Candidate values for component 4 = {ptrs-Power configures 00, ptrs-Power configures 01, both ptrs-Power configures 00 and 01}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-1c; Comb offset hopping within a subset; Support of being configured with a subset of comb offsets for comb offset hopping; Yes; N/A; band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2a; Smaller cyclic shift granularity for cyclic shift hopping; Max value of “cyclicShiftHoppingFinerGranularity” for cyclic shift hopping with smaller granularity; Yes; N/A; band; No; No; N/A; Candidate values: {2,4,FFS}; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2b; SRS cyclic shift hopping combined with legacy group/sequence hopping; Support of being configured with both cyclic shift hopping and legacy group/sequence hopping for the same SRS resource; Yes; N/A; band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-2c; Cyclic shift hopping within a subset; Support of being configured with a subset of cyclic shifts for cyclic shift hopping; Yes; N/A; band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-3; SRS comb offset hopping combined with SRS cyclic shift hopping; Support of being configured with both comb offset hopping and cyclic shift hopping for the same SRS resource; Yes; N/A; band; No; No; N/A; Optional with capability signaling. 40. NR_MIMO_evo_DL_UL; 40-5-7; 8-ports SRS for noncodebook PUSCH; 1. Support 8-ports SRS for noncodebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS for noncodebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS for noncodebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS for noncodebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot 7. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 8. Maximum number of simultaneous transmitted SRS resources at one OFDM symbol; Yes; N/A; Per FSPC; No; No; N/A; Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14} Candidate value for component 7 = {1,2,3,4,5,6,7,8} Candidate value for component 8 = {1,2,4,8}; Optional with capability signaling. |
| MediaTek Inc. [17] | What to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH In Rel-17 unified TCI framework, at least for intra-cell case, it is baseline that a UE supports PDCCH, PDSCH, PUSCH, and PUCCH share a same unified TCI state, and it is optional that some PDCCHs and respective PDSCHs can be configured with different TCI state(s) from the unified one. The same principle can be kept in Rel-18 unified TCI extension at least for PDSCH reception, i.e., if a UE is scheduled by a PDCCH with a PDSCH reception, using the same indicated joint/DL TCI state(s) for the scheduling PDCCH and the scheduled PDSCH should be the baseline. Whether a UE can support using an indicated joint/DL TCI state for the scheduled PDSCH that is not used for scheduling PDCCH can be an optional UE feature. Proposal 4: Introduce a separate row for “Using different indicated joint/DL TCI states for the scheduled PDSCH reception and the scheduling PDCCH reception” The Number of TDCP reports: In RAN1#113, an FG for the “Number of CSI-RS resources for TDCP” is agreed. This includes both per-CC and across-CC resources, as well as configured and simultaneous resources. The configured resources can be used in different ways to create various TDCP reports. Hence, we propose adding a separate FG to indicate the maximum number of active (simultaneous) and configured TDCP reports that the UE supports, for both per-CC and across-CC. Proposal 10: Add an FG to indicate the “Maximum number of TDCP reports” per-CC and across-CC, with indicators for both the total number of configured reports and for active reports in each case: Feature; Index; Feature group; Components; Note; Mandatory/Optional. 40. NR_MIMO_evo_DL_UL; 40-3-3-x; Maximum number of TDCP reports; - Maximum number of configured TDCP reports per-CC. - Maximum number of configured TDCP reports across-CC. - Maximum number of active TDCP reports per-CC. - Maximum number of active TDCP reports across-CC.. A-TRS based TDCP reporting: Since aperiodic TRS (A-TRS) is an optional UE feature, we propose adding a separate FG to indicate whether UE supports TDCP reporting based on A-TRS. Proposal 12: Add an FG for “Aperiodic-TRS-based” to indicate whether the UE supports TDCP reporting based on Aperiodic-TRS: Feature; Index; Feature group; Components; Note; Mandatory/Optional. 40. NR_MIMO_evo_DL_UL; 40-3-3-x; Aperiodic-TRS-based; - A Flag to indicate support of aperiodic-TRS-based TDCP reporting.; Optional with UE capability. RAN1# 112 Agreement On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1. - FFS: Additional restriction(s) depending on the configured value for NTRP In RAN1# 112-bis, there was no consensus to add or remove any value from the agreed list of NL ={1,2,4}. With NL =1 as part of the basic feature, we have the following proposal for the additional feature: Proposal 13: Introduce the feature on maximum number of supported spatial domain bases combination for Rel-18 Type II codebook for CJT. RAN1# 113 Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation: OCPU = X.NTRP where - X≥1 when NTRP>1, is defined based on UE capabilities and determined by the UE - FFS: Whether the supported value(s) of X are common or can depend on the value of NTRP, NL, total sum of {Ln}, and/or other CJT features (e.g. dynamic TRP selection) - The legacy specification on CPU pools is fully reused - Note: When NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1 Based on this agreement, we have the following proposal: Proposal 14: Introduce the capability on number of CPUs occupied per active CSI-RS resource for Rel-18 Type II codebook for CJT. RAN1# 113 Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Based on this agreement, we have the following proposal: Proposal 15: Introduce the UE capability on CSI computation time for Rel-18 Type II codebook for CJT. In summary, we suggest the following changes to the UE feature table for Rel-18 Type II codebook for mTRP CJT: Proposal 16: Support the following addition to FG 40-3-1-X Feature; Feature Group; Components; Mandatory/Optional. 40. NR_MIMO_evo_DL_UL; Number of spatial domain bases combination; Maximum number of supported spatial domain bases combination; Optional with capability signaling. Candidate values: {1,2,4}. 40. NR_MIMO_evo_DL_UL; Basic features of CSI enhancement for mTRP CJT – number of CPUs; Number of CPUs occupied per active CSI-RS resource; Optional with capability signalling Candidate values {1,2,3,4,5}. 40. NR_MIMO_evo_DL_UL; Basic features of CSI enhancement for mTRP CJT – CSI computation time; Capability index on CSI computation time; Optional with capability signalling Candidate values {1,2}. RAN1# 113 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Based on this agreement, we have the following proposal: Proposal 17: Introduce the following UE features on CPU rules for CSI enhancement for high/medium UE velocities: - Number of CPU units occupied per Rel-18 Type II doppler codebook time unit with P/SP-CSI-RS - Number of CPU units occupied per active AP-CSI-RS for Rel-18 Type II doppler codebook RAN1# 113 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Based on this agreement, we have the following proposal: Proposal 18: Introduce the UE capability on CSI computation time for Rel-18 Type II doppler codebook. RAN1# 113 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, the value of KP for P/SP-CSI-RS active resource counting is determined based on UE capability, where the candidate values are {1, 2, 4}. Based on this agreement, we have the following proposal: Proposal 19: Introduce the UE capability on counting the number of active CSI-RS resources per instance of P/SP-CSI-RS In summary, we suggest the following changes to the UE feature table for Rel-18 Type II doppler codebook: Proposal 20: Support the following addition to FG 40-3-2-X Feature; Feature Group; Components; Mandatory/Optional. 40. NR_MIMO_evo_DL_UL; Basic features of CSI enhancement for high/medium UE velocities – number of CPUs; - Number of CPUs occupied per codebook time unit for P/SP-CSI-RS - Number of CPUs occupied per active AP-CSI-RS resource; Optional with capability signalling Component 1 and 2 candidate values {2,3,4,5}. 40. NR_MIMO_evo_DL_UL; Basic features of CSI enhancement for high/medium UE velocities – CSI computation time; Capability index on CSI computation time; Optional with capability signalling Candidate values {1,2}. 40. NR_MIMO_evo_DL_UL; Basic features of CSI enhancement for high/medium UE velocities – counting active resources for P/SP-CSI-RS; Number of active resources per instance of P/SP-CSI-RS; Optional with capability signalling Candidate values {1,2,4}. |

# Conclusion

Agreements reached during RAN1 #114 as part of this agenda item are summarized in [18].

# References

- R1-2306223, Updated RAN1 UE features list for Rel-18 NR after RAN1#113, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2306540,1 UE features for NR MIMO evolution, Huawei/HiSilicon

- R1-2306583, On UE features for Rel. 18 MIMO evolution WI, Nokia/Nokia Shanghai Bell

- R1-2306618, Discussion on UE features for NR MIMO evolution, ZTE

- R1-2306671, Discussion on UE features for NR MIMO evolution, Spreadtrum Communications

- R1-2306776, Discussion on Rel-18 MIMO UE features, vivo

- R1-2306852, UE features for NR-MIMO evolution, Intel Corporation

- R1-2306965, UE feature on NR MIMO evolution, Google

- R1-2307068, On UE features for NR MIMO evolution, CATT

- R1-2307230, Discussion on UE features for Rel18 MIMO, Ericsson

- R1-2307309, Views on UE features for Rel-18 NR MIMO evolution, Apple

- R1-2307363, Discussion on UE features for MIMO evolution, xiaomi

- R1-2307497, Discussion on UE features for NR MIMO evolution, NTT DOCOMO, INC.

- R1-2307519, UE features for Rel-18 MIMO evolution, OPPO

- R1-2307709, UE features for Rel-18 MIMO, Samsung

- R1-2307959, UE features for NR MIMO evolution, Qualcomm Incorporated

- R1-2308066, UE Features for NR MIMO Evolution, MediaTek Inc.

- R1-2308574, Session Notes of AI 9.16.1, Ad-Hoc Chair (AT&T)