3GPP TSG RAN WG1 #114		R1-2308055

Toulouse, France, August 21th – 25th, 2023

Agenda Item: 9.2.3.2

Source: MediaTek Inc.

Title:	Other aspect on AI/ML for beam management

Document for: Discussion & Decision

# Introduction

A new Study Item (SI) to study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface had been approved in RAN Plenary RP#94 meeting. AI/ML-based Beam management has been identified as one of the three use cases for investigation and evaluation. It is also mentioned to identify the potential specification impact required to enable AI/ML techniques for the air-interface. In RAN1 109e the SI on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface has been initiated. The SI includes “Other aspects on AI/ML for beam management” under the agenda 9.3.2.2. Over the last RAN1 meeting, i.e., RAN1 109e, the study identified “AI/ML for Beam Management in Spatial Domain” and “AI/ML for Beam Management in Temporal Domain” as the basic use-cases and made some agreements on this aspect. As mentioned in RAN1 109e, the spatial domain and temporal domain beam predictions are defined as:

•	Spatial-domain beam prediction for Set A of beams based on measurement results of Set B of beams.

•	Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams.

In this contribution we discuss the way forward considering the sub use-cases for beam management (BM) and the potential specification impacts.

# Discussion

In RAN1#113, the following agreements are made for BM Case-1 and BM Case-2.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding performance monitoring, study potential spec impact(s) from the following aspects in addition to those included in previous agreements: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - FFS: definition of an event and the performance metric(s) used to identify it - Indication from NW for UE to do LCM operations Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding performance monitoring, study the necessity and potential spec impact(s) of the mechanism that facilitate UE to detect whether the functionality/model is suitable or no longer suitable. Conclusion For the study of DL beam pair prediction of BM-Case1 and BM-Case2 with a UE-side AI/ML model, RAN1 has no consensus to support the reporting of the predicted Rx beam(s) (e.g., Rx beam ID, Rx beam angle information, etc) from UE to network. Agreement For BM-Case2, study necessity, benefit(s) and potential specification impact from the following additional aspects for AI model inference: - Reporting information about measurements of multiple past time instances in one reporting instance for BM-Case2 - Note: only applicable to network-side AI/ML model - Note: The potential performance gains of measurement reporting should be justified by considering UCI payload overhead Agreement For BM-Case1 and BM-Case2, study necessity, benefit(s) and potential specification impact from the following additional aspects for AI model inference: - How to perform beam indication of beams in Set A not in Set B - Note: the legacy mechanism may be sufficient Agreement Regarding data collection for BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the benefits, necessity and potential specification impact of the following aspect on top of those we have agreed in previous meeting: - Assistance information from NW to UE for UE data collection for categorizing the data for the purpose of differentiating characteristics of data - The assistance information should preserve privacy/proprietary information. Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity and potential BM-specific conditions/additional conditions for functionality(ies) and/or model(s) at least from the following aspects: - information regarding model inference - Set A / Set B configuration - performance monitoring - data collection - assistance information

In this contribution, we further discuss the remaining issues that are left during the RAN1#113 offline discussion and some further details of BM Case-1 and BM Case-2 based on the previous agreements.

## Sub-use case of BM Case-1 and BM Case-2

Based on the discussions of RAN1#111, a proposal regarding the type of beams is left for further discussion.

Proposal: For the sub use case BM-Case1 and BM-Case2, focus on Alt.1 and Alt.3 for the predicted beams for further study Note: Alt.1 and Alt.3 were agreed in RAN1#110 meeting as below Alt.1: DL Tx beam prediction Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam)

There are two alternatives that are proposed for further study, DL Tx beam prediction (Alt.1) and Beam pair prediction (Alt.3). We agree to further study DL Tx beam prediction as the problem formulation is clear. For example, the model input and output can be clearly defined. However, for Beam pair prediction, the output may consist of a DL Tx beam and a corresponding DL Rx beam. Although the DL Tx beam part of the beam pair output can be explicitly determined as gNB and its antenna are fixed and facing the same direction, the DL Rx beam part is hard to define. UE and its antenna are frequently changing their orientation across time. Thus, the Rx beam ID and beam angle to gNB are no longer a one-to-one mapping, which makes the potential number of different Rx beams arbitrarily large. Even by assuming the AI/ML model can predict the best Rx beam angle to gNB, UE needs to frequently track its orientation so that it can derive the corresponding Rx beam to use, which tremendously increases the UE side implementation complexity. Based on the above discussion, we have the following proposal,

Besides the feasibility issue, in our latest evaluation result [2], we have observed no performance drop when we use specific Rx beam/Quasi-optimal Rx beam to measure for model input for DL TX beam prediction. Note that by using the specific Rx beam/Quasi-optimal Rx beam, the overhead for sweeping each available Rx can be avoid, leading to the same level of RS overhead reduction as DL beam pair prediction. Since we believe the motivation for the group to introduce beam pair prediction is to reduce measurement overhead due to Rx beam sweeping, we didn’t see the benefit of studying beam pair prediction by addressing all the feasibility issues while we already have solutions on the table to achieve the same overhead reduction performance without any UE implementation/UE rotation concern. Therefore, we propose to deprioritize the beam pair prediction study at this stage.

Proposal 1: For the sub use case BM-Case1 and BM-Case2, focus on Alt.1 (i.e., DL Tx beam prediction) and deprioritize Alt.3 for the predicted beams for further study.

## Data collection

### Rx beam assumption

In RAN1#112 bis-e Agenda 9.2.3.1, one agreement was made to evaluate the performance of DL Tx beam prediction, considering different Rx beam assumptions.

Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Companies report how to select the “best” Rx beam(s) - Option 2: Measurements of specific Rx beam(s) - Companies report how to select specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample Other options are not precluded and can be reported by companies.

For option2, some companies have provided results of choosing the specific Rx by using quasi-optimal methods. For example, the specific Rx beam can be chosen based on previous measurements (exhaustive Rx beam sweeping of a previous P3 procedure). The resulting beam prediction accuracy is comparable to Option1, where the quasi-optimal Rx beam can provide less RS overhead than finding the best Rx measurement. In [1] (Section 2.4.1.5), we have also shown that the following three Rx beam assumptions can provide the best beam prediction accuracy among other tested Rx beam assumptions:

- Best-Rx (i.e., Option 1), exhaustive Rx beam sweeping, where the model input is the best measurement among measurements from all Rx for each Tx beam.

- Case4B (a special case of Option2), the best Rx beam searched for one Tx beam, which was the best Set A Tx beam in the previous RS period, is used for measuring all other Set B beams

- Case5 (quasi-optimal case for Option2), Rx beam of the previous best Tx/Rx pair, is used for measuring all other Set B beams

Moreover, in some evaluated cases (e.g., when SetB/SetA = 8/32), Case4B and Case5 can have better performance than Best-Rx. Judging from the fact that Case4B and Case5 have lower RS overhead than Best-Rx, we believe it is valuable to study the benefit and spec impact of using different Rx beam assumptions for AI/ML-based beam management.

Proposal 2:  Study potential specification impact of using different Rx beam assumptions for AI/ML model inference for DL beam prediction.

In RAN1#113, we have the following proposal remained for discussion for facilitating different Rx beam assumptions when using AI/ML-based beam management:

Proposal: Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, study necessity, and potential specification impact on the following aspect - Mechanism to ensure the common understanding between NW and UE on Rx beam assumption for reported measurement result(s)

The discussion about this issue can be two-fold. First, the necessity, whether there is any performance degradation if a NW-model that is trained with one Rx beam assumption is inferred with a different Rx beam assumption. Second, is there any potential specification impact for NW and UE to align such Rx beam assumption.

From the perspective of beam prediction accuracy performance, in [1] (Section 2.4.4.4) we have evaluated AI/ML models’ generalization performance when trained with one Rx beam assumption and tested with another Rx beam assumption. The AI/ML model generalized well when trained with (1) Best-Rx beam assumptions and tested with Case4B or Case5. In fact, we have seen that the performance increases when the model is tested with Case4B or Case5 with Set B size = 4 or 8 and Set A=32. The corresponding performance is almost the same as training and testing the model with Case4B or Case5. On the other hand, if the model is trained with Case4B or Case5 and tested with Best-Rx, the performance will drop, but the resulting performance is almost the same as training and testing the model with Best-Rx.

As a conclusion, the AI/ML model performance is directly impacted by the Rx assumptions that UE uses during inference regardless the Rx assumptions used for model training. If UE has the capability of using Case4B and Case 5 to measure the Set B of beams, UE can decide to use these Rx assumptions for better performance without NW changing the AI/ML model.

Note that for each Rx beam assumption, the corresponding RS and reporting configuration from NW to UE will be different. For example, for Best-Rx, NW may configure one reporting with Periodic or Semi Persistent RS resources for UE to sweep all its Rx beams. For Case 4B, NW may configure two reports, the first report has corresponding Aperiodic RS resources with repetition on for UE to sweep its Rx beams and the second report has corresponding Aperiodic RS resources with repetition off for UE to measure the Set B of beams. For Case 5, NW may configure one report, with Aperiodic RS resources with repetition off. Therefore, such discrepancy in RS and reporting configurations for different Rx beam assumptions can help UE to determine which Rx beam assumption is used by the NW-side model. In this way, UE preserves the right to decide by its own which Rx beam assumption to use.

Proposal 3:  UE can determine its own Rx beam assumption used for measurement, regardless NW-side model’s Rx beam assumption.

Proposal 4:  NW-side model’s Rx beam assumption can be implicitly indicated to UE through the corresponding reporting and RS configurations for data collection.

### UE-side model

In RAN1#112bis-e, we have the following agreement for data collection mechanism for AI/ML model training at UE side:

Agreement Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact of UE reporting to network from the following aspect - Supported/preferred configurations of DL RS transmission - Other aspect(s) is not precluded Agreement Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact (if any) to initiate/trigger data collection from RAN1 point of view by considering the following options as a starting point - Option 1: data collection initiated/triggered by configuration from NW - Option 2: request from UE for data collection - FFS: details

There are three factors that will impact the UE’s supported/preferred configurations of DL RS transmission for a UE-side model, (1) the supported Set A and Set B of beams, (2) the supported observation/prediction window sizes for BM-Case2 (3) the supported Rx beam assumptions.

Proposal 5: study the potential specification impact of UE reporting to network the supported/preferred configurations of DL RS transmission through reporting the following aspect:

- the supported Set A and Set B of beams

- the supported observation/prediction window sizes for BM-Case2

- the supported Rx beam assumptions

Regarding how to initiate/trigger data collection from RAN1 point of view, we think both Option1(data collection initiated/triggered by configuration from NW) and Option2 (request from UE for data collection) can be considered. Option1 may have smaller spec impact than Option2, as NW already has CSI measurement and report configurations for each UE measurement instance.

Proposal 6:  Study the spec impact of configuring Option1 (data collection initiated/triggered by configuration from NW) by the CSI measurement and report configurations.

On the other hand, Option2 can be used when UE needs to collect data for model training or monitoring. Considering it is a UE-side model, NW has limited understanding on how many data is left for collection for training or monitoring this model. Option2 can provide UE an opportunity to align with NW whether any more data collection is required. For data collection of model monitoring and training at UE side, measurements on both Set A and Set B are required. If Set B is a subset of Set A, basically the data collection mechanism is the same as legacy beam management. That is, UE measures Set A, reports the measured best beam(s) and formulates a training/monitoring data sample for the UE-side model. On the contrary, if Set B is not in Set A, the data collection mechanism is the same as model monitoring by using prediction accuracy as the performance metric. In both Set A and Set B relationship cases, the mechanism is related to UE-side model LCM procedures. In fact, during RAN1#112bis-e, we have the following agreement,

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring

Therefore, we believe Option2 can be studied together with the procedures of facilitating UE-side performance monitoring. Note that, the above discussions are for data collection for UE-side model training and monitoring. For model inference, Option1 is more suitable since the purpose of UE-side model inference is to prepare the beam report that is configured by NW (naturally initiated/triggered by configuration from NW).

Proposal 7: Option2 (request from UE for data collection) can be studied together with UE-side performance monitoring procedures.

### NW-side model

#### Contents of collected data

In RAN1#112bis-e, we have the following agreement for the content of data collection for AI/ML model at NW side:

Agreement Regarding data collection for NW-side AI/ML model, study the following options (including the combination of options) for the contents of collected data, - Opt.1: M1 L1-RSRPs (corresponding to M1 beams) with the indication of beams (beam pairs) based on the measurement corresponding to a beam set, where M1 can be larger than 4, if applicable - FFS: the range of M1 - Opt.2: M2 L1-RSRPs (corresponding to M2 beams) based on the measurement corresponding to a beam set, where M2 can be larger than 4, if applicable - FFS: the range of M2 - Opt.3: M3 beam (beam pair) indices based on the measurement corresponding to a beam set, where M3 can be larger than 4, if applicable - FFS: the range of M3 - FFS: How to select the M1/M2/M3 beam(s) or beam pair(s) - Note: Overhead, UE complexity and power consumption should be considered for the above options

For different cases, different options of the content of collected data can be used. For Opt.1, it can be used for NW-side model inference where UE reports the measurement and the resource indication of the Set B of beams.

However, note that it is not mandatory for UE to report resource indication of the Set B of beams with measurement. Assuming for model inference, NW-side only configures Set B of beams in the resource set of the current reporting instance, there is no need to include the indication of beams in the beam report since all the measurement for all the resources in the resource set will be reported. Consequently, using Opt.2 for model inference can be more beneficial since it can further reduce the reporting overhead. Note that Opt.2 cannot indicate which beam has the best RSRP (since no best CRI/SSBRI is in the report), hence, the method for quantizing the RSRP report in Opt.2 needs to be carefully designed. Otherwise, the reporting overhead will increase if Opt.2 use 7 bits (i.e., the method that current spec uses) to quantize each RSRP measurement. Another possible method is to use Opt.1 but only report the indication of the beam which has the best RSRP measurement, while the rest of the RSRP measurements are reported as differential RSRP and are reported in the same order as the corresponding order of Set B of beams in the configured resource set. Table 1 shows an example of the resulting reporting overhead for one reporting instance of model inference, when NW configures 8 beams as Set B for UE measurement. We reuse the beam report format of the current spec and increase the number of beams in the beam report to 8. For Opt.2, we assume each reported RSRP value is quantized by the same method that the current spec uses to quantize the best RSRP (i.e., 7 bits for each beam). Table 1 shows that both Opt.2 and Opt.1 (only report best RI) can reduce the report overhead compared to using Opt.1 with the current reporting spec format. While Opt.2 can provide 5% report overhead reduction, using Opt.1 with reporting only the indication for the best beam in Set B can provide 36% report overhead reduction. Note that the potential of using Opt.2 in this case may be underestimated since we are using the quantization method which uses the highest number of bits for each RSRP measurement.

Table 1: Reporting overhead for one reporting instance of model inference by using different Options of the content of collected data

|  | RSRP field | CRI/SSBRI field | Total (bits) | Report overhead reduction |
| --- | --- | --- | --- | --- |
| Opt.1 | 7bits*1beam + 4bits*7beams = 35 | 3 bits*8beams = 24 | 59 | 0 % |
| Opt.1 (only report best RI) | 7bits*1beam + 4bits*7beams = 35 | 3 bits*1 beams = 3 | 38 | 36% |
| Opt.2 | 7bits*8beam = 56 | - | 56 | 5 % |

In addition to model inference, Opt.1 can also be used for data collection for NW-side model training. Besides the method where Opt.1 is used for UE to report all the measurement and resource indication of Set A of beams. Another method, where UE reports the measurement of the Set B of beams and the ground truth label (e.g., the indication of the best beam(s)), can save more reporting overhead. Following the same logic as above, since NW and UE already align the indication of Set B of beams, UE doesn’t need to include the indication of Set B of beams in the report. Instead, UE can include the indication of the best beam(s) in Set A in the beam report (as label). The number of Set B of beams and the number of reported best beams(s) in Set A (depends on the output design of the NW-side model) can be different. Therefore, for both model inference and training, we have observed a beneficial way to use Opt.1 by separating the beams of the reported RSRP and the beams of the reported indication of beams. We have the following proposal to cover different ways of using Opt.1,

Proposal 8:  For Opt.1 of the content of collected data for NW-side model, consider the following cases between the reported RSRP and indication of beams

- The beams of the reported M1 L1-RSRPs are the same as the beams of the reported indication

- The beams of the reported M1 L1-RSRPs are not the same as the beams of the reported indication

  - M1 and the number of the indication of beams can be different

Proposal 9: For Opt.2 of the content of collected data for NW-side model, study the method and spec impact to quantize the reported M2 RSRP values.

Proposal 10: For the content of collected data for model inference of NW-side model, consider NW configures only Set B of beams for measurement and UE doesn’t report the indication for all the Set B of beams.

Proposal 11: For the content of collected data for model training of NW-side model, study mechanisms and spec impact of UE reports only the RSRP measurement of Set B of beams and label, without reporting the resource indication of Set B and/or Set A of beams.

Note that there is one case where UE need to report the indication of Set B of beams in the content of collected data, that is, when data filtering/omitting technique is used. Since not all the measurement of Set B of beams are reported, UE need to include the indication of Set B of beams in the report to align with NW which beams in Set B is reported. Consequently, for model inference/training/monitoring purposes, we can see that the corresponding optimal content of reporting the collected data can be different. Even for Opt.1, the optimal way to configure the content in the reporting container can be different for different data collection purposes. Using the wrong format will create huge reporting overhead (as shown in Table 1). According to our observation in [1] (Section 2.4.1.6), some SetA/SetB ratios are suitable for using the data filtering/omitting technique, some ratios are not, in terms of the beam prediction accuracy performance. Hence, even for different SetA/SetB scenarios, the optimal way to configure the content in the reporting container can be different too.

Proposal 12: For the content of collected data for model inference of NW-side model, if data filtering/omitting is used, UE should use Opt.1 to report the measurement and the corresponding indication of the reported beams.

Proposal 13: For the content of collected data NW-side model, separately define the content for reporting the collected data for different data collection purposes and different SetA/SetB scenarios.

Finally, in the agreement of the content of collected data for NW-side model, the values of M1, M2, and M3 have not been determined. The values of M1, M2, and M3 can be determined by at least the following factors: (1) Set A and Set B sizes, (2) the data collection purpose, (3) model output design (e.g., beam ID/RSRP, label sizes), and (4) the overhead reduction strategies. First, it is clear that the number of reported RSRP and beam indications in the report will be impacted by the number of beams in Set A and Set B. Second, for different data collection purposes and model output design, M1, M2, and M3 can be different. For example, for data collection for model training by using Opt.1, if model output is Top-1 best beam ID in Set A, M1 can be the number of beams in Set B plus the 1 additional beam as the label. If model output is the beam RSRPs for all the Set A of beams, M1 should be the number of beams in Set A. On the other hand, for data collection for model inference by using Opt.1, M1 should be the number of beams in Set B. Finally, the overhead reduction strategies, there have been some discussion in the previous few meetings about data filtering/omitting, where UE can filter out some RSRP measurements and report a subset of all the measurements. Consequently, the values of M1, M2, and M3 will be determined by UE and can be different for each reporting instance. However, note that in the current spec for beam report, NW needs to configure the number of beams for reporting. If the values of M1, M2, and M3 are determined by UE, there will be challenges and spec impacts on how NW can decode the CSI report without knowing the number of beams beforehand.

Proposal 14: For the content of collected data for NW-side model, study benefit and spec impact of UE determining M1,M2,M3 values for each reporting instance.

Proposal 15: For the content of collected data for NW-side model, further study defining the value of M1, M2, and M3 based on the following factors,

- Set A and Set B sizes

- data collection purposes

- model output design

- overhead reduction strategies

#### Mechanism related to the reporting

In RAN1#112bis-e, we have the following agreement for data collection mechanism for AI/ML model at NW side:

Agreement Regarding data collection for NW-side AI/ML model, study necessity, benefits and beam-management-specific potential specification impact from RAN1 point of view on the following additional aspects - Mechanism related to the reporting - Additional information for content of the reporting - FFS: Information associated with or configured for the reported data samples, e.g., timestamps, SNR, data quality, etc. - Reporting overhead reduction - Note1: non-3GPP based solution is a separate issue. - Note2: The framework corresponding to higher layer(s) are up to the associated WG(s) - Note 3: Overhead, UE complexity and power consumption should be considered

From the discussion in the previous section, both of the optimal content of collected data for NW-side model, and the number of reported beams in each reporting instance can be different for different (1) Set A and Set B sizes, (2) the data collection purpose, (3) model output design (e.g., beam ID/RSRP, label sizes), and (4) the overhead reduction strategies. Since it is a NW-side model, for UE to prepare the corresponding reported NW ‘s desire content, NW needs to align with UE the above conditions. For example, UE needs to know whether the current reporting instance is for model inference or model training. If it is for model training, UE needs to include labels in the beam report in addition to the RSRP measurement for Set B of beams. NW can align these conditions with UE explicitly by configuring these conditions to UE or implicitly by configuring the reporting container formats (different content options may have different formats).

Proposal 16: For the data collection reporting mechanism for NW-side model, study how NW aligns with UE the following conditions for UE to report the corresponding content of data,

- Set A and Set B sizes

- data collection purposes

- model output design (e.g., label sizes, beam ID/RSRP)

- overhead reduction strategies (e.g., w/wo data omitting)

On the other hand, in RAN1#113, we have the following agreement for data collection mechanism for BM-Case2 for AI/ML model at NW side.

Agreement For BM-Case2, study necessity, benefit(s) and potential specification impact from the following additional aspects for AI model inference: - Reporting information about measurements of multiple past time instances in one reporting instance for BM-Case2 - Note: only applicable to network-side AI/ML model - Note: The potential performance gains of measurement reporting should be justified by considering UCI payload overhead

In our opinion, if UE report the measurements of multiple past time instances in one reporting instance, UE may need to include the corresponding additional time information for each measurement in that report. However, UE is not necessarily to report the measurements of multiple past time instances in one reporting instance for BM-Case2. For instance, NW can configure separate reports for each time instance in the observation window. One benefit of separate reporting is that UE does not need to report temporal information as additional information in the report since NW knows the mapping of each report to each time instance in the observation window. Therefore, we propose to study the benefit and spec impact of both options of data collection for NW-side model inference/training/monitoring for BM-Case2.

Proposal 17: For the data collection for NW-side model for BM-Case2, it is not necessarily for UE to report information about measurements of multiple past time instances in one reporting instance for BM-Case2

Proposal 18: For the data collection for NW-side model inference/training/monitoring for BM-Case2, study the benefit(s) and potential spec impact from the following two options of UE reporting as a starting point,

- Option1: NW configures separate report for each time instance in the observation window

- Option2: NW configures one report for all the time instances in the observation window

#### Quantization for Data collection

In the following discussion, we will study the requirement of each data sample for data collection for NW-side AI/ML models. NW-side AI/ML model requires UE to report the measured L1-RSRP values or beam ID for training. However, the quality of the reported values has not been discussed yet. In RAN1#112, an observation regarding quantization the L1-RSRP with the current spec [2] has been drawn:

Observation - At least for BM-Case1 for inference of DL Tx beam with L1-RSRPs of all beams in Set B, existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) causes [a minor loss x%~y%, if applicable] in beam prediction accuracy compared to unquantized L1-RSRPs of beams in Set B.

In the current spec, the L1-RSRP is quantized to integer dBm levels for UE reporting. The largest L1-RSRP in one report is mapped to dBm levels, which is represented by 7 bits (i.e., 128 integer dBm levels). For the rest of the L1-RSRP values, UE maps their difference to the maximum L1-RSRP values (in dB) by Table 10.1.6.1-2 in [3]. As a result, it takes 19 bits for UE to report L1-RSRP for 4 beams.

On the other hand, we have shown (Section 2.4.1.5.2 in [1]) that by using a simple quantization method, we can achieve the same level of accuracy performance as by using the FP16 quantization method, while using the same number of bits in one UE report as the current spec uses. Furthermore, we have shown that with lower number of bits per beam’s L1-RSRP, the studied quantization method can achieve the same level of accuracy performance for model training and testing.

Therefore, we believe it is valuable to study the possibility of using lower precision quantization method for beam RSRP report other than the current spec, for supporting AI/ML model inference and data collection for NW-side models. Also, we can further consider the feasibility of using different quantizing methods, including different bits used for quantization and quantized quantity (linear or dBm), for different Set B designs.

Proposal 19:  For data collection for NW-side AI/ML model, study spec impact of reporting overhead reduction with quantizing L1-RSRP and/or normalized L1-RSRP measurement with lower number of bits than the current spec.

## Model inference

### Beam indication

In RAN1#113, we have the following agreement for model inference:

Agreement For BM-Case1 and BM-Case2, study necessity, benefit(s) and potential specification impact from the following additional aspects for AI model inference: - How to perform beam indication of beams in Set A not in Set B - Note: the legacy mechanism may be sufficient

In our opinion, beam indication should not be performed when the indicated beam is not measured by UE. If the predicted beam(s) is not in Set B, NW can configure additional DL RS for UE to measure these beams. Note that the resulting RS overhead is not necessarily larger than beam indication without configuring additional DL RS for UE measurements. Additional DL RS configured for UE to measure these beams enables UE to find the actual best Rx beam for each predicted beam(s). This will allow UE to use different Rx beam assumptions, like the quasi-optimal Rx beams, that can reduce the RS overhead more significantly during the model inference phases (i.e., during the P1 procedure).

Proposal 20: Deprioritize the study of performing beam indication of beams in Set A not in Set B

Proposal 21: Considering performing beam indication of beams in Set A not in Set B by legacy mechanism where NW configures additional DL RS for UE measurement.

### Irregular beam report for BM-Case2

Unlike conventional periodic beam report, in BM-Case2, UE is measuring only for observation window and stops the measurement during the prediction window. In the meantime, NW can just configure RS resources in the observation window and save the RS resources in the prediction window. This will create a “duty cycle” behavior (i.e., not always ON) if such RS resources and reports are configured as periodic, which may have spec impact. However, this kind of RS configuration and measurement pattern can be achieved by NW configuring separate reports for each observation window (i.e., each report instance).

Proposal 22:  For BM-Case2, instead of configuring periodic beam report, NW can configure separate reports for each BM-Case2 model inference instance.

### NW-side model

In RAN1#111, we have the following agreement for NW-side model,

Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on the following L1 reporting enhancement for AI/ML model inference - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered

The beam RSRP report quantization method in the current specification, as described above, can be easily expanded, and used for higher number of beams (>4). However, in our current evaluation results (i.e., Section 2.4.1.5.2 in [2]), we have shown that for Set B size = 8 or 16, the reporting precision of the spec and the resulting model performance might not be optimal, there may be possibilities to use even lower precision of L1-RSRP values or quantizing with lower bits on normalized L1-RSRP values. Moreover, different Set B designs will have different sensitivity to the beam RSRP report’s precision. We have shown the possibility and benefit of using different reporting quantization methods for L1-RSRP report for different Set B designs in Section 2.4.1.5.2 in [2].

On the other hand, we have observed that if a model is trained with FP32 samples while its inference is conducted with another quantization method samples, it will perform worse than training with samples from the same quantization method that is being used for inference. Therefore, the UE and the network should align the quantization method that is being used for both model training and inference.

Proposal 23: To facilitate AI model inference for a NW-side AI/ML model, study spec impact of reporting overhead reduction with quantizing L1-RSRP and/or normalized L1-RSRP measurement with lower number of bits than the current spec.

### UE-side model

In RAN1#110bis-e and RAN1#112 meeting, we have the following agreement for the reporting of inference output of the UE-side model.

Agreement For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW - The beam(s) that is based on the output of AI/ML model inference - FFS: Predicted L1-RSRP corresponding to the beam(s) - FFS: other information Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network: - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) - FFS: Definition/content of confidence/probability information - Note: At least the performance and spec impact should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

Regarding UE reporting the beam(s) that is based on the output of AI/ML model inference, evaluation results from most of the companies have shown that the beam prediction accuracy will increase when the value K of the Top-K predicted beams increases. It is reasonable that when UE reporting more beams based on its AI/ML model inference output, more chances on that the genie-aided best beam is among the reported beams. However, it also means the corresponding UCI payload overhead and the following RS overhead for a P2 procedure (to find the best beam) will increase.

On the other hand, if a UE is very confident on its AI/ML model inference output, it is beneficial to UE to report fewer number of beams as its AI/ML model inference output to save the UCI payload overhead and RS overhead in P2 procedure. For example, if the AI/ML model output is the predicted L1-RSRP values and UE is going to report the beams that have higher predicted L1-RSRP values than other beams, UE can estimate the correctness of its inference result by comparing the prediction error between the predicted L1-RSRP values and the measured K1-RSRP values of certain beams. If the prediction error is very low, UE can determine to report fewer number of beams as its AI/ML model inference output since it has higher confidence on its output. In the current spec, the number of beams for UE to report is configured by the NW with the nrofReportedRS parameter, UE is required to report nrofReportedRS number of beams in the corresponding beam report. There will be spec impact if UE can determine lesser number of beams than the configured nrofReportedRS in one beam report. Therefore, we propose the following:

Proposal 24: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study spec impact to facilitate UE to report various number of Top-K beams in one beam report (K ≤ nrofReportedRS) as AI/ML model output.

## Model monitoring

### UE-side model

During the offline discussion in RAN1#112be, one proposal regarding performance monitoring for UE-side model to facilitate functionality-based LCM has been discussed.

Proposal 4.3.2: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when functionality-based LCM is applicable, support NW-side performance monitoring to facilitate functionality related LCM operations (i.e., activation/deactivation/fallback/switching of AI/ML functionality) - FFS: UE-side performance monitoring

We understand the proposal is based on the agreement that has been made in Agenda 9.2.1 as following:

Agreement in 9.2.1 For UE-side models and UE-part of two-sided models: … - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM

All the agreements in this agenda and 9.2.1 regarding NW-side performance monitoring only mentions that “Network indicates activation/deactivation/fallback/switching of AI/ML functionality to UE”. We believe this sentence does not exclude the possibility that UE can initiate a request of LCM operations for certain AI/ML functionalities from NW. After NW receives the request, NW can determine whether to grant the request and indicates the corresponding LCM operations to UE. The above cases should be covered by the current agreements. For example, when the considered AI/ML functionality is the observation window and prediction window length (i.e., the number of measurement and prediction instances F and K, respectively) of BM Case2. When UE detects the increase of its speed from 30 km/h to 60 km/h, UE can then request to change the observation/prediction window length to shorter number of instances for more frequent beam prediction, instead of reporting its speed to NW. Note that it is still NW’s decision on whether to take the requested operation and hence, NW will indicate the LCM operation to UE, which satisfies the current agreement.

However, we understand that some AI/ML functionalities may not be feasible for UE to initiate a LCM operation request. Therefore, as a starting point, we can first identify the AI/ML functionalities for beam management that are feasible/not feasible for UE initiating a LCM operation request.

Proposal 25: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when functionality-based LCM is applicable, study a performance monitoring method with UE initiating LCM operation request and NW indicating/granting the corresponding activation/deactivation/fallback/switching LCM operation of AI/ML functionality to UE.

Proposal 26: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when functionality-based LCM is applicable, identify for each AI/ML functionality whether it is feasible for UE to initiate LCM operation requests.

In RAN1#113, the following agreements have been made about performance monitoring for UE-side AI/ML model.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding performance monitoring, study potential spec impact(s) from the following aspects in addition to those included in previous agreements: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - FFS: definition of an event and the performance metric(s) used to identify it - Indication from NW for UE to do LCM operations Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding performance monitoring, study the necessity and potential spec impact(s) of the mechanism that facilitate UE to detect whether the functionality/model is suitable or no longer suitable.

From the first agreement, for performance monitoring of a UE-side model, UE can report an event to NW based on the monitored performance metric(s). How UE can detect such event can be facilitated by the second agreement. To study the mechanism to facilitate UE to detect such event, the definition of an event should be aligned between NW and UE first. For example, an event can be one monitoring instance of missed beam prediction, a certain number of consecutive instances of missed beam prediction, or an observation that the beam prediction accuracy is less than a threshold after a sequence of monitoring instances. Meanwhile, the event can also be when the RSRP prediction error is larger than a threshold from one or more monitoring instances. It is also important to define how frequently that UE can collect the monitoring samples and how many number of monitoring samples is required for model monitoring.

Note that different definitions/configurations (like different performance metrics, threshold, required number of samples, and monitoring frequency) of an event may have different monitoring accuracy and overhead. For example, beam prediction accuracy may have the same RS overhead as Set A, but it can 100% examine the model output’s correctness. On the other hand, RSRP prediction error can be calculated only among a subset of Set A of beams, but it is not a direct KPI for reflecting the beam prediction accuracy and therefore cannot 100% examine the model output’s correctness.

Based on the monitoring history or current condition, NW can have different events, each with different performance metrics, threshold, required number of samples, and monitoring frequency, configured for UE to monitor and report.

Proposal 27:  For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding the event that UE reports for performance monitoring, defining the event at least from the following aspects,

- The performance metrics monitored for the event

- The threshold of the performance metrics for determining the occurrence of the event

- The number of samples of the occurrence instances required for determining the occurrence of the event, where the occurrence instances are the monitoring samples that the monitored metrics falls below a threshold

- The number of monitoring samples required for determining the occurrence of the event

- The frequency of each monitoring samples

Proposal 28:  For BM-Case1 and BM-Case2 with a UE-side AI/ML model, model, to facilitate UE to detect a monitoring event for performance monitoring, study the necessity and potential spec impact(s) of NW signaling to UE the following aspects,

- The performance metrics monitored for the event

- The threshold of the performance metrics for determining the occurrence of the event

- The number of samples of the occurrence instances required for determining the occurrence of the event, where the occurrence instances are the monitoring samples that the monitored metrics falls below a threshold

- The number of monitoring samples required for determining the occurrence of the event

- The frequency of each monitoring samples

Meanwhile, we also need to study the container for UE to report the monitoring results. The spec impact for UE reporting the beam measurement and model output may be the smallest. However, the corresponding reporting overhead will be very high. For UE to report the observed performance metrics or event, we can study if the current beam report format can be used or if new container format is required.

Proposal 29:  Study the necessity and spec impact of the content of collected data that UE reports for performance monitoring for BM-Case1 and BM-Case2 with a UE-side AI/ML model, including at least the following contents,

- Performance metrics

- Beam measurements

- Events for monitoring

## Functionality/Model-ID based LCM

In RAN1#113, the following agreement has been made for listing the possible conditions for Functionality/Model-based LCM.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity and potential BM-specific conditions/additional conditions for functionality(ies) and/or model(s) at least from the following aspects: - information regarding model inference - Set A / Set B configuration - performance monitoring - data collection - assistance information

For the first bullet regarding the conditions about the “information regarding model inference” for UE-side AI/ML model, the possible conditions can be:

- conditions the number of predicted best beams (e.g., value of K for Top-K predicted beams)

- conditions the model output (e.g., predicted beam ID/confidence score of each beam/beam RSRP)

- conditions the available Rx beam assumptions

, where the first condition is important for NW to configure the corresponding nrofReportedRS parameter in the CSI-report configuration. It also indicates whether a second beam sweeping of the Top-K predicted beams is required. The second condition describes the condition of the model output. At the UE side, UE may have multiple models providing different types of model output, such as beam ID or beam RSRP. Furthermore, there are some discussions during the last meeting about UE deciding how many beams to report as model output based on the confidence score of each beam. However, some models predicting the beam ID may allow UE to extract the confidence score of each predicted beam while some models may not. This condition can help in categorizing each UE-side model based on whether the confidence score or beam RSRP can be used as assistance information for improving the mechanism of UE-side model inference. Finally, in Section 2.2.1, we have discussed the benefit of UE using different Rx beam assumptions for model inference. From the perspective of RS overhead reduction performance, if a UE can use quasi-optimal Rx beam assumptions for a UE-side model, NW can configure a smaller number of RS than Best-Rx for data collection and further reduce the RS overhead. This condition helps NW to configure the corresponding RS resources, based on whether UE and its AI/ML model can use any specific Rx beam assumptions for measurement.

For the third bullet regarding the conditions about the “performance monitoring” for UE-side AI/ML model, the possible conditions can be:

- conditions on performance metrics

- conditions on the detectable events

, where the first condition is to describe the type of performance metrics that a UE and a UE-side AI/ML model can support. For example, if it is a classification model which predicts beam ID, then it is not applicable to use the error between measured RSRP and predicted RSRP as the performance metrics. For the scenario when UE reports an event to NW for performance monitoring, the second conditions of the event that UE can detect is useful for NW to find the best monitoring strategies (e.g., choosing between the event-report based monitoring or performance-metrics-report based monitoring).  Therefore, for the BM-specific conditions of a UE-side AI/ML model, we have the following proposal:

Proposal 30: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for the BM-specific conditions regarding “information regarding model inference”, study the necessity at least for the following sub-conditions,

- conditions on the number of predicted best beams (e.g., value of K for Top-K predicted beams)

- conditions on the model output (e.g., predicted beam ID/confidence score of each beam/beam RSRP)

- conditions on the available Rx beam assumptions

Proposal 31: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for the BM-specific conditions regarding “performance monitoring”, study the necessity at least for the following sub-conditions,

- conditions on performance metrics

- conditions on the detectable events

Also, in Section 2.2.3.1, we have discussed that under different data collection purposes (inference/training/monitoring) and different SetA/SetB scenario, the suitable content that UE reports for NW-side model, in terms of better reporting overhead performance can be different. In the meantime, some of the UEs may not support the corresponding suitable content/reporting format for each type of NW-side models. Therefore, for the NW-side AI/ML model, it is also important to define the corresponding BM-specific conditions/additional conditions for UE to align its capability of such conditions.

On the other hand, some of the NW-side AI/ML model may have assistance information as model input, while some of the UEs may not support reporting of such additional information. Therefore, we believe it is important to have a separate study on the BM-specific conditions/additional conditions of UE for a NW-side AI/ML model,

Proposal 32: For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the necessity and potential BM-specific conditions/additional conditions of UE for functionality(ies) and/or model(s) at least from the following aspects:

- data collection (e.g., conditions on data omitting/filtering methods, conditions on the reporting contents)

- assistance information

# Conclusion

In summary, based on the above discussion we have the following observations and proposals:

Proposal 1: For the sub use case BM-Case1 and BM-Case2, focus on Alt.1 (i.e., DL Tx beam prediction) and deprioritize Alt.3 for the predicted beams for further study.

Proposal 2:  Study potential specification impact of using different Rx beam assumptions for AI/ML model inference for DL beam prediction.

Proposal 3:  UE can determine its own Rx beam assumption used for measurement, regardless NW-side model’s Rx beam assumption.

Proposal 4:  NW-side model’s Rx beam assumption can be implicitly indicated to UE through the corresponding reporting and RS configurations for data collection.

Proposal 5: study the potential specification impact of UE reporting to network the supported/preferred configurations of DL RS transmission through reporting the following aspect:

- the supported Set A and Set B of beams

- the supported observation/prediction window sizes for BM-Case2

- the supported Rx beam assumptions

Proposal 6:  Study the spec impact of configuring Option1 (data collection initiated/triggered by configuration from NW) by the CSI measurement and report configurations.

Proposal 7: Option2 (request from UE for data collection) can be studied together with UE-side performance monitoring procedures.

Proposal 8:  For Opt.1 of the content of collected data for NW-side model, consider the following cases between the reported RSRP and indication of beams

- The beams of the reported M1 L1-RSRPs are the same as the beams of the reported indication

- The beams of the reported M1 L1-RSRPs are not the same as the beams of the reported indication

  - M1 and the number of the indication of beams can be different

Proposal 9: For Opt.2 of the content of collected data for NW-side model, study the method and spec impact to quantize the reported M2 RSRP values.

Proposal 10: For the content of collected data for model inference of NW-side model, consider NW configures only Set B of beams for measurement and UE doesn’t report the indication for all the Set B of beams.

Proposal 11: For the content of collected data for model training of NW-side model, study mechanisms and spec impact of UE reports only the RSRP measurement of Set B of beams and label, without reporting the resource indication of Set B and/or Set A of beams.

Proposal 12: For the content of collected data for model inference of NW-side model, if data filtering/omitting is used, UE should use Opt.1 to report the measurement and the corresponding indication of the reported beams.

Proposal 13: For the content of collected data NW-side model, separately define the content for reporting the collected data for different data collection purposes and different SetA/SetB scenarios.

Proposal 14: For the content of collected data for NW-side model, study benefit and spec impact of UE determining M1,M2,M3 values for each reporting instance.

Proposal 15: For the content of collected data for NW-side model, further study defining the value of M1, M2, and M3 based on the following factors,

- Set A and Set B sizes

- data collection purposes

- model output design

- overhead reduction strategies

Proposal 16: For the data collection reporting mechanism for NW-side model, study how NW aligns with UE the following conditions for UE to report the corresponding content of data,

- Set A and Set B sizes

- data collection purposes

- model output design (e.g., label sizes, beam ID/RSRP)

- overhead reduction strategies (e.g., w/wo data omitting)

Proposal 17: For the data collection for NW-side model for BM-Case2, it is not necessarily for UE to report information about measurements of multiple past time instances in one reporting instance for BM-Case2

Proposal 18: For the data collection for NW-side model inference/training/monitoring for BM-Case2, study the benefit(s) and potential spec impact from the following two options of UE reporting as a starting point,

- Option1: NW configures separate report for each time instance in the observation window

- Option2: NW configures one report for all the time instances in the observation window.

Proposal 19:  For data collection for NW-side AI/ML model, study spec impact of reporting overhead reduction with quantizing L1-RSRP and/or normalized L1-RSRP measurement with lower number of bits than the current spec.

Proposal 20: Deprioritize the study of performing beam indication of beams in Set A not in Set B

Proposal 21: Considering performing beam indication of beams in Set A not in Set B by legacy mechanism with NW configures additional DL RS for UE measurement.

Proposal 22:  For BM-Case2, instead of configuring periodic beam report, NW can configure separate reports for each BM-Case2 model inference instance.

Proposal 23: To facilitate AI model inference for a NW-side AI/ML model, study spec impact of reporting overhead reduction with quantizing L1-RSRP and/or normalized L1-RSRP measurement with lower number of bits than the current spec.

Proposal 24: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study spec impact to facilitate UE to report various number of Top-K beams in one beam report (K ≤ nrofReportedRS) as AI/ML model output.

Proposal 25: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when functionality-based LCM is applicable, study a performance monitoring method with UE initiating LCM operation request and NW indicating/granting the corresponding activation/deactivation/fallback/switching LCM operation of AI/ML functionality to UE.

Proposal 26: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when functionality-based LCM is applicable, identify for each AI/ML functionality whether it is feasible for UE to initiate LCM operation requests.

Proposal 27:  For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding the event that UE reports for performance monitoring, defining the event at least from the following aspects,

- The performance metrics monitored for the event

- The threshold of the performance metrics for determining the occurrence of the event

- The number of samples of the occurrence instances required for determining the occurrence of the event, where the occurrence instances are the monitoring samples that the monitored metrics falls below a threshold

- The number of monitoring samples required for determining the occurrence of the event

- The frequency of each monitoring samples

Proposal 28:  For BM-Case1 and BM-Case2 with a UE-side AI/ML model, model, to facilitate UE to detect a monitoring event for performance monitoring, study the necessity and potential spec impact(s) of NW signaling to UE the following aspects,

- The performance metrics monitored for the event

- The threshold of the performance metrics for determining the occurrence of the event

- The number of samples of the occurrence instances required for determining the occurrence of the event, where the occurrence instances are the monitoring samples that the monitored metrics falls below a threshold

- The number of monitoring samples required for determining the occurrence of the event

- The frequency of each monitoring samples

Proposal 29:  Study the necessity and spec impact of the content of collected data that UE reports for performance monitoring for BM-Case1 and BM-Case2 with a UE-side AI/ML model, including at least the following contents,

- Performance metrics

- Beam measurements

- Events for monitoring

Proposal 30: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for the BM-specific conditions regarding “information regarding model inference”, study the necessity at least for the following sub-conditions,

- conditions on the number of predicted best beams (e.g., value of K for Top-K predicted beams)

- conditions on the model output (e.g., predicted beam ID/confidence score of each beam/beam RSRP)

- conditions on the available Rx beam assumptions

Proposal 31: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for the BM-specific conditions regarding “performance monitoring”, study the necessity at least for the following sub-conditions,

- conditions on performance metrics

- conditions on the detectable events

Proposal 32: For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the necessity and potential BM-specific conditions/additional conditions of UE for functionality(ies) and/or model(s) at least from the following aspects:

- data collection (e.g., conditions on data omitting/filtering methods, conditions on the reporting contents)

- assistance information

# References

- R1-2308054, Evaluation on AI/ML for beam management, MediaTek, RAN WG1 #114

- TS 38.331, “5G NR Radio Resource Control (RRC); Protocol specification,” 3GPP Release 16, Jul. 2020.

- TS 38.133, “5G NR Requirements for support of radio resource management,” 3GPP Release 15, Oct. 2018.