3GPP TSG RAN WG1 #114		                                             R1-2307555

Toulouse, France, August 21st - 25th, 2023

Agenda Item:	9.11.1

Source: 	OPPO

Title:	Evaluation for lower power wake-up signal

Document for:	Discussion & Decision

Introduction

In RAN1#113, the following agreements of the evaluation on low power WUS have been made [1].

Agreement Use the same channel specific assumptions as defined in TR38.830 for reference PUSCH for message3, i.e., Parameter; Value. Frequency hopping; w/ or w/o frequency hopping. Number of UE transmit chains; 1, 2 (optional). Number of DMRS symbol; w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop. Waveform; DFT-s-OFDM. SCS; 30kHz for TDD, 15kHz for FDD.. HARQ configuration; Whether HARQ is adopted is reported by companies.. PUSCH duration; 14 OS. Number of PRBs; 2. TBS; 56 bits. Other parameters; Reported by companies.. Agreement For reference setting for further study on LP-SS performance and resource overhead (including sync and/or measurement), companies to report the following used in their evaluations - the number of slots or symbols per period - the periodicty - the functionality of the LP-SS Agreement ----------------------------TP start for TR38.869 v0.1.0------------------------------------------- 6.3.2 Power model for LP-WUR (LR) The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off[1]; 0.001 / 0.02/ 1% of ON Power value 0.1/0.2/0.3, only for 10/20/30, for 0.1, [oscillator option 3/4] are not used for envelope detection based receiver; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On[2]; 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation - For evaluation, 10/20/30 for LP-WUR ON power state are not used for envelope detection based receiver for LP-WUS monitoring. - For evaluation, 10/20/30 for LP-WUR ON power state are used for OFDM receiver when noise figure is less than [MR noise figure + 2.5dB], [0.2/0.5/1/2/4] for LP-WUS can be assumed for other NF values larger than [MR noise figure + 2.5dB] - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: - For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; - [For WUR Off value 0.02, only oscillator option 1, 2 can be assumed and only RTC can beis maintained; ] - [For other WUR Off value, oscillator option 1,2,3,4 can be assumed.] - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization. ----------------------------TP End------------------------------------------- Agreement Confirm the following WA with the following changes Working Assumption The following for usage of the clock is assumed for LP-WUR OFF/ON Assumption on LP-WUR OFF power; Assumptions on the clock usage. 0.001; When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 [Note2] (cumulating based on the frequency drift and not exceed maximum frequency error) - The initial frequency offset when LP-WUR switches on can be set to the [FFS: maximum frequency error or a random value within the maximum frequency error] following the parameters of the oscillator option 1/2/3/4[Note2]. - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done.. TBD: value(s) >0.001; For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 or option 3/4[Note2] (cumulating based on the second value in the value pair and not exceed maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration. [Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power] FFS: Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD2, option 1/2 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD1 Note3: The clock error (of both RTC and LO) could be improved to be less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble. Agreement Observations: For RRM with duty-cycled LP-WUS monitoring, the following observations are made with the assumption that - MR in ultra-deep sleep - Effective per UE paging arrival rate <=1% - LP-WUR duty cycle ratio <=2% - MR ramp-up time/transition energy option 1 (i.e., 400ms, 15000) - RRM relaxation is assumed for both serving and neighbouring cells Compared with i-DRX, LP-WUS operation with - No RRM relaxed - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([-301%~-569%]) - MR relaxed < 8 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([-10%~7%)) - 8 times<= MR relaxed <=16 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([31%~60%]) - RRM relaxed > 16 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([60~92%]) - RRM offload RRM to LR - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([76%~92%]) Note: The ‘Effective per UE paging arrival rate’ is defined as (without taking FAR into account) - Per UE paging probability RE if LP-WUS is per UE paging - Per group paging probability RG = 1 – (1 – RE)N, if LP-WUS is per group paging (N is the number of UEs in the group). There will be another observation for continuous monitoring case

In this contribution, we provide some evaluation results and analysis on low power WUS.

Power consumption evaluation

Evaluation Assumptions

In this section, some evaluation results on power consumption and latency are provided, we consider the following assumptions and the power models.

Table 1: UE Power Consumption Model for Main Radio and LP-WUR

| Main Radio |  |  |
| --- | --- | --- |
| Power State | Characteristics | Relative Power |
| Ultra-deep sleep | Power state for main radio of UEs with LP-WUS receiver. | 0.015 |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| LP-WUR |  |  |
| Power State | Characteristics | Relative Power |
| LP-WUR ‘off’ | The LP-WUR does not perform monitoring. | 0.001/0.02/0.1 |
| LP-WUR ‘on’ | The LP-WUR performs monitoring. | 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 |

Table 2: UE Power Consumption for Main Radio during the state transition

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Ultra-deep sleep | 15000(1) | 400ms (2) |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| (1) Ramp-down and ramp-up without sync/re-sync transition energy. (2) The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the ramp-up time. Energy consumption for sync/re-sync is separately calculated. * Immediate transition is assumed for power saving study purpose from or to a non-sleep state. |  |  |

Table 3: Simulation assumptions (based TR 38.840)

| Parameter |  | Values |
| --- | --- | --- |
| Frequency |  | 4GHz |
| Subcarrier spacing |  | 15 kHz |
| System bandwidth |  | 100MHz |
| Link Adaptation (MCS select) |  | Off |
| Retransmission |  | Off |
| i-DRX cycle length |  | 1.28(s) |
| Number of POs in Paging Frame |  | 1 |
| Number of SSB before PO / PEI |  | 1, 2, 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) |
| Number of SSBs for sync/re-sync for MR |  | 2/4/6 |
| LP-WUS monitoring |  | Option1: Continuously monitoring Option2: Discontinuously monitoring, with [64] ms as the period for complete an on-and-off cycle, and [64] ms as the active time for monitoring LP-WUS every cycle. And ramp-up time is 10ms. |
| Traffic model | Y | 1.28s |
|  | Per UE paging rate RE | 1% within duration Y |
|  | The mean arrival time P | P = Y/RE = 128s |
|  | The number of UEs in the group | 10 |
|  | Per group paging probability RG | RG = 1 – (1 – RE)N=9.56% |
| LP-WUS duration |  | 1ms |
| LP-WUS monitoring duty cycle |  | 128ms |
| The miss-detection rate (MDR) of LP-WUS |  | 1% |
| The false-alarm rate (FAR) of LP-WUS |  | 0.1% |

Evaluation Results

- Processing Timeline

Rel-17 UE paging reception procedure with PEI is used as baseline, which is shown in Figure 1. The number of SSB bursts processed, PEI-O monitoring duration, PO monitoring duration, and intermediate sleep states are shown for three different SNR cases (low, medium, and high) when PEI is configured.

Figure 1: Rel-17 UE paging reception procedure with PEI (baseline)

Assume UE monitors one paging occasion in a paging cycle for I-DRX cycle length of 1.28s after waking up from deep sleep. The power consumption in a paging cycle for UE operations can be calculated as follows.

Table 4: Power consumption of Rel-17 UE operations in a paging cycle

| UE operations in a paging cycle | Duration (ms) | Relative Power (unit) | Relative Power Consumption (unit*ms) |
| --- | --- | --- | --- |
| Not Paged |  |  |  |
| (1) SSB Processing | 2 | 100 | 100*2 = 200 |
| (2) PEI-O Monitoring | 4 | 100 | 100*4 = 400 |
| (3) Deep Sleep | 1274 | 1 | 1*1274 + 450 Transition Energy = 1724 |
| Total | 1280 | - | 2324 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 2324 / 1280 = 1.8156 |
| Low SINR case (Paged) |  |  |  |
| (1) SSB Processing | 2 | 100 | 100*2 = 200 |
| (2) PEI-O Monitoring | 4 | 100 | 100*4 = 400 |
| (3) Light Sleep | 14 | 20 | 20*14 + 100Transition Energy = 380 |
| (4) SSB Processing | 2 | 100 | 100*2 = 200 |
| (5) Light Sleep | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (6) SSB Processing | 2 | 100 | 100*2 = 200 |
| (7) Light Sleep | 8 | 20 | 20*8 + 100 Transition Energy = 260 |
| (8) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (9) Deep Sleep | 1226 | 1 | 1*1226 + 450 Transition Energy = 1676 |
| Total | 1280 | - | 4976 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 4976 / 1280 = 3.8875 |
| Medium SINR case (Paged) |  |  |  |
| (1) SSB Processing | 2 | 100 | 100*2 = 200 |
| (2) PEI-O Monitoring | 4 | 100 | 100*4 = 400 |
| (3) Light Sleep | 14 | 20 | 20*14 + 100Transition Energy = 380 |
| (4) SSB Processing | 2 | 100 | 100*2 = 200 |
| (5) Deep Sleep | 28 | 1 | 1*28 + 450 Transition Energy = 478 |
| (6) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (7) Deep Sleep | 1226 | 1 | 1*1226 + 450 Transition Energy = 1676 |
| Total | 1280 | - | 4534 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 4534 / 1280 = 3.5422 |
| High SINR case (Paged) |  |  |  |
| (1) SSB Processing | 2 | 100 | 100*2 = 200 |
| (2) PEI-O Monitoring | 4 | 100 | 100*4 = 400 |
| (3) Deep Sleep | 44 | 1 | 1*44 + 450 Transition Energy = 494 |
| (4) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (5) Deep Sleep | 1226 | 1 | 1*1226 + 450 Transition Energy = 1676 |
| Total | 1280 | - | 3970 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 3970 / 1280 = 3.1016 |

Rel-18 UE paging reception procedure with LP-WUR is shown in Figure 2. The MR is in ultra-deep sleep state before the trigger from the LP-WUR. We assume ramp-up time is 400ms, and MR need consider SSB search and synchronization after wake-up. In our assumption, the SSB search time is 20ms, while the time of (re-)synchronization is 120ms (6 SSB bursts) in Low SINR case.

Figure 2: Rel-18 UE paging reception procedure with LP-WUR

Assume UE monitors one paging occasion in a paging cycle for I-DRX cycle length of 1.28s after waking up from ultra-deep sleep. The power consumption in a paging cycle for UE operations can be calculated as follows.

Table 5: Power consumption of Rel-18 UE with LP-WUR operations in a paging cycle

| UE operations in a paging cycle |  | Duration (ms) | Relative Power (unit) | Relative Power Consumption (unit*ms) |
| --- | --- | --- | --- | --- |
| Not Paged |  |  |  |  |
| MR Power Consumption: |  |  |  |  |
| (1) Ultra-deep sleep |  | 1280 | 0.015 | 1280*0.015 = 19.2 |
| LP-WUR Power Consumption: (continuously monitoring) |  |  |  |  |
| (1) On state |  | 1280 | 0.5 | 1280 * 0.5 = 640 |
| Total |  | - | - | 19.2MR + 640LP-WUR =659.2 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  |  | 659.2 / 1280 = 0.515 |
| LP-WUR Power Consumption: (discontinuously monitoring) |  |  |  |  |
| Duty Cycle (128ms, 50%) | (1) On state | 64 (Transition time = 10ms) | 0.5 | 64 * 0.5 + 2.495 Transition Energy = 34.495 |
|  | (2) Off state | 64 | 0.001 | 64 * 0.001 = 0.064 |
|  | (3) cycle of (1)&(2) | 10 times | 34.495+0.064 = 34.559 | 34.559 * 10 = 345.59 |
| Duty Cycle (128ms, 0.9375%) | (1) On state | 1 (Transition time = 10ms) | 0.5 | 1 * 0.5 + 2.495 Transition Energy = 2.995 |
|  | (2) Off state | 127 | 0.001 | 127*0.001 = 0.127 |
|  | (3) cycle of (1)&(2) | 10 times | 2.995 + 0.127 = 3.122 | 3.122 * 10 = 31.22 |
| Duty Cycle (128ms, 50%) |  | Total |  | 19.2MR + 345.59LP-WUR = 364.79 |
|  |  | Average power consumption |  | 364.79 / 1280 = 0.2850 |
| Duty Cycle (128ms, 0.9375%) |  | Total |  | 19.2MR + 31.22LP-WUR = 50.42 |
|  |  | Average power consumption |  | 50.42 / 1280 = 0.0394 |
| Low SINR case (Paged) |  |  |  |  |
| (1) SSB Search |  | 20 | 100 | 100*20 = 2000 |
| (2) Freq. sync |  | 60 (3 SSB bursts) | 200 + 460 = 660 | 660*3 = 1980 |
| (3) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (4) Light Sleep |  | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (5) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (6) Light Sleep |  | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (7) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (8) Light Sleep |  | 8 | 20 | 20*8 + 100 Transition Energy = 260 |
| (9) PO Monitoring |  | 4 | 300 | 300*4 = 1200 |
| (10) Ultra-deep Sleep |  | 1146 | 0.015 | 0.015*1146 + 15000 Transition Energy = 15017.19 |
| Total (continuously monitoring) |  | 1280 | - | 21977.19MR + 640LP-WUR = 22617.19 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  |  | 22617.19 / 1280 = 17.6697 |
| Duty Cycle (128ms, 50%) |  | Total (duty cycle monitoring) |  | 21977.19MR + 345.59LP-WUR = 22322.78 |
|  |  | Average power consumption |  | 22322.78 / 1280 = 17.4397 |
| Duty Cycle (128ms, 0.9375%) |  | Total (duty cycle monitoring) |  | 21977.19MR + 31.22LP-WUR = 22008.41 |
|  |  | Average power consumption |  | 22063.3 / 1280 = 17.1941 |
| Medium SINR case (Paged) |  |  |  |  |
| (1) SSB Search |  | 20 | 100 | 100*20 = 2000 |
| (2) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (3) Light Sleep |  | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (4) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (5) Light Sleep |  | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (6) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (7) Light Sleep |  | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (8) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (9) Light Sleep |  | 8 | 20 | 20*8 + 100 Transition Energy = 260 |
| (10) PO Monitoring |  | 4 | 300 | 300*4 = 1200 |
| (11) Ultra-deep Sleep |  | 1186 | 0.015 | 0.015*1186 + 15000 Transition Energy = 15017.79 |
| Total (continuously monitoring) |  | 1280 | - | 20657.79MR + 640LP-WUR = 21297.79 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  |  | 21297.79 / 1280 = 16.6389 |
| Duty Cycle (128ms, 50%) |  | Total (duty cycle monitoring) |  | 20657.79MR + 345.59LP-WUR = 21003.38 |
|  |  | Average power consumption |  | 21003.38 / 1280 = 16.4089 |
| Duty Cycle (128ms, 0.9375%) |  | Total (duty cycle monitoring) |  | 20657.79MR + 31.22LP-WUR = 20689.01 |
|  |  | Average power consumption |  | 20743.9 / 1280 = 16.1633 |
| High SINR case (Paged) |  |  |  |  |
| (1) SSB Search |  | 20 | 100 | 100*20 = 2000 |
| (2) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (3) Light Sleep |  | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (4) SSB Processing |  | 2 | 100 | 100*2 = 200 |
| (5) Light Sleep |  | 8 | 20 | 20*8 + 100 Transition Energy = 260 |
| (6) PO Monitoring |  | 4 | 300 | 300*4 = 1200 |
| (7) Ultra-deep Sleep |  | 1226 | 0.015 | 0.015*1226 + 15000 Transition Energy = 15018.39 |
| Total (continuously monitoring) |  | 1280 | - | 19338.39MR + 640LP-WUR = 19978.39 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  |  | 19978.39 / 1280 = 15.6082 |
| Duty Cycle (128ms, 50%) |  | Total (duty cycle monitoring) |  | 19338.39MR + 345.59LP-WUR = 19683.98 |
|  |  | Average power consumption |  | 19683.98 / 1280 = 15.3781 |
| Duty Cycle (128ms, 0.9375%) |  | Total (duty cycle monitoring) |  | 19338.39MR + 31.22LP-WUR = 19370.61 |
|  |  | Average power consumption |  | 19424.5 / 1280 = 15.1333 |

MR performs measurement every 5 IDRX cycles

Different SINR levels, e.g. low, medium and high could be assumed to evaluation. UE need different number of SSB bursts to acquire synchronization. LP-WUR can monitor LP-WUS under “continuously monitoring” manner or “duty cycle” manner. We assume the length of “duty cycle” is 128ms, i.e. there are 10 cycles in one paging cycle. In this contribution, we evaluate the power consumption at the assumption of “LP-WUS duty cycle ration” with 50% and 0.78%. We assume RRM measurement performed by “LR+MR”, the measurement period of MR is 6400ms, i.e. MR performs measurement every 5 IDRX cycles, and the measurement period of LR is 1280ms. The RRM measurement period of PEI is 1280ms.

Table 5: Power Saving Gain of LP-WUS compared to DRX with PEI (Per UE Paging)

Additional transition energy from ultra-deep sleep is 15000

| DRX settings | SINR | Paging Rate (Per UE) | Scheme | Relative Power for LP-WUR on | Average power (Relative Power of LP-WUR off) |  |  | PS gain (%) (Relative Power of LP-WUR off) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 0.001 | 0.02 | 0.1 | 0.001 | 0.02 | 0.1 |
| I-DRX = 1.28s (107 Cycle) (FAR = 0.1%) | Low | 1% | Rel-17 UE with PEI | - | 3.3806 |  |  |  |  |  |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 2.8425 | 2.8424 | 2.8424 | 15.9195 | 15.9222 | 15.9207 |
|  |  |  |  | 0.05 | 2.8825 | 2.8824 | 2.8824 | 14.7363 | 14.7390 | 14.7375 |
|  |  |  |  | 0.1 | 2.9325 | 2.9324 | 2.9324 | 13.2573 | 13.2600 | 13.2585 |
|  |  |  |  | 0.2 | 3.0325 | 3.0324 | 3.0324 | 10.2992 | 10.3020 | 10.3005 |
|  |  |  |  | 0.5 | 3.3325 | 3.3324 | 3.3324 | 1.4252 | 1.4279 | 1.4264 |
|  |  |  |  | 1 | 3.8325 | 3.8324 | 3.8324 | -13.3650 | -13.3622 | -13.3637 |
|  |  |  |  | 2 | 4.8325 | 4.8324 | 4.8324 | -42.9452 | -42.9425 | -42.9439 |
|  |  |  |  | 4 | 6.8325 | 6.8324 | 6.8324 | -102.1056 | -102.1031 | -102.1044 |
|  |  |  |  | 10 | 12.8325 | 12.8324 | 12.8324 | -279.5870 | -279.5849 | -279.5859 |
|  |  |  |  | 20 | 22.8325 | 22.8324 | 22.8324 | -575.3892 | -575.3878 | -575.3882 |
|  |  |  |  | 30 | 32.8325 | 32.8324 | 32.8324 | -871.1914 | -871.1907 | -871.1906 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 2.8382 | 2.8469 | 2.8839 | 16.0443 | 15.7878 | 14.6936 |
|  |  |  |  | 0.05 | 2.8598 | 2.8685 | 2.9055 | 15.4065 | 15.1500 | 14.0558 |
|  |  |  |  | 0.1 | 2.8867 | 2.8954 | 2.9324 | 14.6092 | 14.3527 | 13.2585 |
|  |  |  |  | 0.2 | 2.9407 | 2.9493 | 2.9863 | 13.0147 | 12.7581 | 11.6639 |
|  |  |  |  | 0.5 | 3.1024 | 3.1110 | 3.1480 | 8.2310 | 7.9744 | 6.8803 |
|  |  |  |  | 1 | 3.3719 | 3.3806 | 3.4176 | 0.2582 | 0.0016 | -1.0925 |
|  |  |  |  | 2 | 3.9110 | 3.9196 | 3.9566 | -15.6874 | -15.9440 | -17.0381 |
|  |  |  |  | 4 | 4.9891 | 4.9978 | 5.0348 | -47.5786 | -47.8352 | -48.9293 |
|  |  |  |  | 10 | 8.2235 | 8.2321 | 8.2691 | -143.2522 | -143.5089 | -144.6029 |
|  |  |  |  | 20 | 13.6141 | 13.6228 | 13.6598 | -302.7083 | -302.9650 | -304.0589 |
|  |  |  |  | 30 | 19.0047 | 19.0134 | 19.0504 | -462.1643 | -462.4212 | -463.5149 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 2.8339 | 2.8519 | 2.9281 | 16.1732 | 15.6403 | 13.3870 |
|  |  |  |  | 0.05 | 2.8358 | 2.8538 | 2.9299 | 16.1178 | 15.5848 | 13.3315 |
|  |  |  |  | 0.1 | 2.8381 | 2.8561 | 2.9323 | 16.0484 | 15.5155 | 13.2622 |
|  |  |  |  | 0.2 | 2.8428 | 2.8608 | 2.9370 | 15.9098 | 15.3769 | 13.1235 |
|  |  |  |  | 0.5 | 2.8568 | 2.8749 | 2.9510 | 15.4938 | 14.9609 | 12.7076 |
|  |  |  |  | 1 | 2.8803 | 2.8983 | 2.9745 | 14.8005 | 14.2676 | 12.0143 |
|  |  |  |  | 2 | 2.9272 | 2.9452 | 3.0213 | 13.4139 | 12.8810 | 10.6277 |
|  |  |  |  | 4 | 3.0209 | 3.0389 | 3.1151 | 10.6408 | 10.1079 | 7.8546 |
|  |  |  |  | 10 | 3.3022 | 3.3202 | 3.3963 | 2.3214 | 1.7884 | -0.4649 |
|  |  |  |  | 20 | 3.7709 | 3.7889 | 3.8651 | -11.5444 | -12.0773 | -14.3307 |
|  |  |  |  | 30 | 4.2397 | 4.2577 | 4.3338 | -25.4101 | -25.9431 | -28.1964 |
|  | Medium |  | Rel-17 UE with PEI | - | 3.3768 |  |  | - |  |  |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 2.8311 | 2.8310 | 2.8311 | 16.1608 | 16.1632 | 16.1620 |
|  |  |  |  | 0.05 | 2.8711 | 2.8710 | 2.8711 | 14.9762 | 14.9787 | 14.9774 |
|  |  |  |  | 0.1 | 2.9211 | 2.9210 | 2.9211 | 13.4956 | 13.4980 | 13.4968 |
|  |  |  |  | 0.2 | 3.0211 | 3.0210 | 3.0211 | 10.5342 | 10.5367 | 10.5354 |
|  |  |  |  | 0.5 | 3.3211 | 3.3210 | 3.3211 | 1.6502 | 1.6526 | 1.6513 |
|  |  |  |  | 1 | 3.8211 | 3.8210 | 3.8211 | -13.1566 | -13.1542 | -13.1554 |
|  |  |  |  | 2 | 4.8211 | 4.8210 | 4.8211 | -42.7701 | -42.7677 | -42.7689 |
|  |  |  |  | 4 | 6.8211 | 6.8210 | 6.8211 | -101.9971 | -101.9948 | -101.9960 |
|  |  |  |  | 10 | 12.8211 | 12.8210 | 12.8211 | -279.6782 | -279.6760 | -279.6771 |
|  |  |  |  | 20 | 22.8211 | 22.8210 | 22.8211 | -575.8133 | -575.8113 | -575.8122 |
|  |  |  |  | 30 | 32.8211 | 32.8210 | 32.8211 | -871.9485 | -871.9467 | -871.9474 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 2.8269 | 2.8356 | 2.8726 | 16.2857 | 16.0287 | 14.9335 |
|  |  |  |  | 0.05 | 2.8485 | 2.8571 | 2.8941 | 15.6471 | 15.3902 | 14.2949 |
|  |  |  |  | 0.1 | 2.8754 | 2.8841 | 2.9211 | 14.8489 | 14.5920 | 13.4968 |
|  |  |  |  | 0.2 | 2.9293 | 2.9380 | 2.9750 | 13.2526 | 12.9957 | 11.9004 |
|  |  |  |  | 0.5 | 3.0910 | 3.0997 | 3.1367 | 8.4635 | 8.2066 | 7.1113 |
|  |  |  |  | 1 | 3.3606 | 3.3692 | 3.4062 | 0.4818 | 0.2248 | -0.8704 |
|  |  |  |  | 2 | 3.8996 | 3.9083 | 3.9453 | -15.4818 | -15.7387 | -16.8340 |
|  |  |  |  | 4 | 4.9778 | 4.9864 | 5.0234 | -47.4088 | -47.6658 | -48.7610 |
|  |  |  |  | 10 | 8.2121 | 8.2208 | 8.2578 | -143.1901 | -143.4471 | -144.5423 |
|  |  |  |  | 20 | 13.6028 | 13.6114 | 13.6484 | -302.8255 | -303.0825 | -304.1776 |
|  |  |  |  | 30 | 18.9934 | 19.0021 | 19.0390 | -462.4608 | -462.7179 | -463.8130 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 2.8225 | 2.8406 | 2.9167 | 16.4148 | 15.8810 | 13.6252 |
|  |  |  |  | 0.05 | 2.8244 | 2.8424 | 2.9186 | 16.3593 | 15.8255 | 13.5697 |
|  |  |  |  | 0.1 | 2.8268 | 2.8448 | 2.9210 | 16.2899 | 15.7560 | 13.5002 |
|  |  |  |  | 0.2 | 2.8314 | 2.8495 | 2.9256 | 16.1511 | 15.6172 | 13.3614 |
|  |  |  |  | 0.5 | 2.8455 | 2.8635 | 2.9397 | 15.7346 | 15.2008 | 12.9450 |
|  |  |  |  | 1 | 2.8689 | 2.8870 | 2.9631 | 15.0406 | 14.5067 | 12.2509 |
|  |  |  |  | 2 | 2.9158 | 2.9338 | 3.0100 | 13.6524 | 13.1186 | 10.8628 |
|  |  |  |  | 4 | 3.0096 | 3.0276 | 3.1038 | 10.8762 | 10.3423 | 8.0865 |
|  |  |  |  | 10 | 3.2908 | 3.3088 | 3.3850 | 2.5474 | 2.0135 | -0.2423 |
|  |  |  |  | 20 | 3.7596 | 3.7776 | 3.8538 | -11.3340 | -11.8678 | -14.1236 |
|  |  |  |  | 30 | 4.2283 | 4.2463 | 4.3225 | -25.2153 | -25.7492 | -28.0050 |
|  | High |  | Rel-17 UE with PEI | - | 3.3720 |  |  | - |  |  |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 2.8198 | 2.8197 | 2.8197 | 16.3766 | 16.3788 | 16.3778 |
|  |  |  |  | 0.05 | 2.8598 | 2.8597 | 2.8597 | 15.1904 | 15.1926 | 15.1915 |
|  |  |  |  | 0.1 | 2.9098 | 2.9097 | 2.9097 | 13.7076 | 13.7098 | 13.7087 |
|  |  |  |  | 0.2 | 3.0098 | 3.0097 | 3.0097 | 10.7420 | 10.7441 | 10.7431 |
|  |  |  |  | 0.5 | 3.3098 | 3.3097 | 3.3097 | 1.8451 | 1.8473 | 1.8463 |
|  |  |  |  | 1 | 3.8098 | 3.8097 | 3.8097 | -12.9829 | -12.9807 | -12.9818 |
|  |  |  |  | 2 | 4.8098 | 4.8097 | 4.8097 | -42.6390 | -42.6368 | -42.6379 |
|  |  |  |  | 4 | 6.8098 | 6.8097 | 6.8097 | -101.9512 | -101.9489 | -101.9500 |
|  |  |  |  | 10 | 12.8098 | 12.8097 | 12.8097 | -279.8878 | -279.8853 | -279.8866 |
|  |  |  |  | 20 | 22.8098 | 22.8097 | 22.8097 | -576.4488 | -576.4459 | -576.4475 |
|  |  |  |  | 30 | 32.8098 | 32.8097 | 32.8097 | -873.0098 | -873.0065 | -873.0084 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 2.8156 | 2.8242 | 2.8612 | 16.5016 | 16.2442 | 15.1475 |
|  |  |  |  | 0.05 | 2.8371 | 2.8458 | 2.8828 | 15.8621 | 15.6047 | 14.5080 |
|  |  |  |  | 0.1 | 2.8641 | 2.8728 | 2.9097 | 15.0628 | 14.8054 | 13.7087 |
|  |  |  |  | 0.2 | 2.9180 | 2.9267 | 2.9636 | 13.4642 | 13.2068 | 12.1101 |
|  |  |  |  | 0.5 | 3.0797 | 3.0884 | 3.1254 | 8.6682 | 8.4108 | 7.3141 |
|  |  |  |  | 1 | 3.3492 | 3.3579 | 3.3949 | 0.6750 | 0.4176 | -0.6791 |
|  |  |  |  | 2 | 3.8883 | 3.8970 | 3.9340 | -15.3115 | -15.5689 | -16.6656 |
|  |  |  |  | 4 | 4.9664 | 4.9751 | 5.0121 | -47.2845 | -47.5419 | -48.6386 |
|  |  |  |  | 10 | 8.2008 | 8.2095 | 8.2465 | -143.2034 | -143.4607 | -144.5575 |
|  |  |  |  | 20 | 13.5914 | 13.6001 | 13.6371 | -303.0682 | -303.3255 | -304.4224 |
|  |  |  |  | 30 | 18.9820 | 18.9907 | 19.0277 | -462.9331 | -463.1903 | -464.2872 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 2.8112 | 2.8292 | 2.9054 | 16.6310 | 16.0961 | 13.8371 |
|  |  |  |  | 0.05 | 2.8131 | 2.8311 | 2.9073 | 16.5754 | 16.0405 | 13.7815 |
|  |  |  |  | 0.1 | 2.8154 | 2.8335 | 2.9096 | 16.5059 | 15.9710 | 13.7120 |
|  |  |  |  | 0.2 | 2.8201 | 2.8381 | 2.9143 | 16.3669 | 15.8320 | 13.5730 |
|  |  |  |  | 0.5 | 2.8342 | 2.8522 | 2.9284 | 15.9499 | 15.4150 | 13.1560 |
|  |  |  |  | 1 | 2.8576 | 2.8756 | 2.9518 | 15.2548 | 14.7199 | 12.4609 |
|  |  |  |  | 2 | 2.9045 | 2.9225 | 2.9987 | 13.8647 | 13.3298 | 11.0708 |
|  |  |  |  | 4 | 2.9982 | 3.0163 | 3.0924 | 11.0844 | 10.5495 | 8.2905 |
|  |  |  |  | 10 | 3.2795 | 3.2975 | 3.3737 | 2.7436 | 2.2088 | -0.0502 |
|  |  |  |  | 20 | 3.7482 | 3.7663 | 3.8424 | -11.1577 | -11.6925 | -13.9515 |
|  |  |  |  | 30 | 4.2170 | 4.2350 | 4.3112 | -25.0590 | -25.5938 | -27.8528 |

Observation 1: Under the assumption of RRM measurement performed by “LR+MR”, while the measurement period of MR is 6400ms, and the measurement period of LR is 1280ms. When compared with legacy measurement period is 1280ms. For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging. [Relative Power for LP-WUR off is 0.001]

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 1.43%~15.92% power saving gain, while have 1.65%~16.16% power saving gain under Medium SINR case, and have 1.84%~16.37% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 1/2/4/10/20/30, LP-WUR monitor LP-WUS under “continuously monitoring” manner have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% can have 0.26%~16.04% power saving gain under Low SINR case, while have 0.48%~16.28% power saving gain under Medium SINR case, and have 0.67%~16.50% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 2/4/10/20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/4/10, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 0.78% can have 2.32%~16.17% power saving gain under Low SINR case, while have 2.54%~16.41% power saving gain under Medium SINR case, and have 2.74%~16.63% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 0.78% have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 2: Relative Power of LP-WUR off takes small influence on power saving gain of LP-WUS.

Assume the number of UEs in the group is 10, when RE is 1%, the per group paging probability RG is 9.56%. Evaluation results on power saving gain is summarized in Table 6.

Table 6: Power Saving Gain of LP-WUS compared to DRX with PEI (Per UE Group Paging)

Additional transition energy from ultra-deep sleep is 15000

Relative Power for LP-WUR off is 0.001

| DRX settings | SINR | Paging Rate (Per Group) | Scheme | Relative Power for LP-WUR on | Average power | PS gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
| I-DRX = 1.28s (107 Cycle) (FAR = 0.1%) | Low | 9.56% | Rel-17 UE with PEI | - | 3.4267 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 4.1455 | -20.9775 |
|  |  |  |  | 0.05 | 4.1855 | -22.1448 |
|  |  |  |  | 0.1 | 4.2355 | -23.6039 |
|  |  |  |  | 0.2 | 4.3355 | -26.5221 |
|  |  |  |  | 0.5 | 4.6355 | -35.2769 |
|  |  |  |  | 1 | 5.1355 | -49.8682 |
|  |  |  |  | 2 | 6.1355 | -79.0507 |
|  |  |  |  | 4 | 8.1355 | -137.4157 |
|  |  |  |  | 10 | 14.1355 | -312.5109 |
|  |  |  |  | 20 | 24.1355 | -604.3362 |
|  |  |  |  | 30 | 34.1355 | -896.1615 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 4.1414 | -20.8564 |
|  |  |  |  | 0.05 | 4.1630 | -21.4856 |
|  |  |  |  | 0.1 | 4.1899 | -22.2722 |
|  |  |  |  | 0.2 | 4.2438 | -23.8453 |
|  |  |  |  | 0.5 | 4.4055 | -28.5647 |
|  |  |  |  | 1 | 4.6751 | -36.4303 |
|  |  |  |  | 2 | 5.2141 | -52.1615 |
|  |  |  |  | 4 | 6.2923 | -83.6239 |
|  |  |  |  | 10 | 9.5266 | -178.0111 |
|  |  |  |  | 20 | 14.9173 | -335.3232 |
|  |  |  |  | 30 | 20.3079 | -492.6353 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 4.1372 | -20.7341 |
|  |  |  |  | 0.05 | 4.1391 | -20.7888 |
|  |  |  |  | 0.1 | 4.1414 | -20.8572 |
|  |  |  |  | 0.2 | 4.1461 | -20.9940 |
|  |  |  |  | 0.5 | 4.1602 | -21.4044 |
|  |  |  |  | 1 | 4.1836 | -22.0884 |
|  |  |  |  | 2 | 4.2305 | -23.4563 |
|  |  |  |  | 4 | 4.3242 | -26.1922 |
|  |  |  |  | 10 | 4.6055 | -34.3997 |
|  |  |  |  | 20 | 5.0742 | -48.0790 |
|  |  |  |  | 30 | 5.5430 | -61.7583 |
|  | Medium |  | Rel-17 UE with PEI | - | 3.3919 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 4.0415 | -19.1538 |
|  |  |  |  | 0.05 | 4.0815 | -20.3331 |
|  |  |  |  | 0.1 | 4.1315 | -21.8073 |
|  |  |  |  | 0.2 | 4.2315 | -24.7555 |
|  |  |  |  | 0.5 | 4.5315 | -33.6002 |
|  |  |  |  | 1 | 5.0315 | -48.3413 |
|  |  |  |  | 2 | 6.0315 | -77.8236 |
|  |  |  |  | 4 | 8.0315 | -136.7882 |
|  |  |  |  | 10 | 14.0315 | -313.6820 |
|  |  |  |  | 20 | 24.0315 | -608.5049 |
|  |  |  |  | 30 | 34.0315 | -903.3279 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 4.0374 | -19.0315 |
|  |  |  |  | 0.05 | 4.0590 | -19.6673 |
|  |  |  |  | 0.1 | 4.0859 | -20.4619 |
|  |  |  |  | 0.2 | 4.1398 | -22.0512 |
|  |  |  |  | 0.5 | 4.3015 | -26.8190 |
|  |  |  |  | 1 | 4.5711 | -34.7654 |
|  |  |  |  | 2 | 5.1101 | -50.6582 |
|  |  |  |  | 4 | 6.1883 | -82.4438 |
|  |  |  |  | 10 | 9.4226 | -177.8006 |
|  |  |  |  | 20 | 14.8133 | -336.7286 |
|  |  |  |  | 30 | 20.2039 | -495.6566 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 4.0332 | -18.9079 |
|  |  |  |  | 0.05 | 4.0351 | -18.9632 |
|  |  |  |  | 0.1 | 4.0374 | -19.0323 |
|  |  |  |  | 0.2 | 4.0421 | -19.1705 |
|  |  |  |  | 0.5 | 4.0562 | -19.5851 |
|  |  |  |  | 1 | 4.0796 | -20.2761 |
|  |  |  |  | 2 | 4.1265 | -21.6580 |
|  |  |  |  | 4 | 4.2202 | -24.4220 |
|  |  |  |  | 10 | 4.5015 | -32.7139 |
|  |  |  |  | 20 | 4.9702 | -46.5337 |
|  |  |  |  | 30 | 5.4390 | -60.3535 |
|  | High |  | Rel-17 UE with PEI | - | 3.3474 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 3.9375 | -17.6294 |
|  |  |  |  | 0.05 | 3.9775 | -18.8243 |
|  |  |  |  | 0.1 | 4.0275 | -20.3180 |
|  |  |  |  | 0.2 | 4.1275 | -23.3054 |
|  |  |  |  | 0.5 | 4.4275 | -32.2676 |
|  |  |  |  | 1 | 4.9275 | -47.2045 |
|  |  |  |  | 2 | 5.9275 | -77.0784 |
|  |  |  |  | 4 | 7.9275 | -136.8261 |
|  |  |  |  | 10 | 13.9275 | -316.0693 |
|  |  |  |  | 20 | 23.9275 | -614.8079 |
|  |  |  |  | 30 | 33.9275 | -913.5466 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 3.9334 | -17.5054 |
|  |  |  |  | 0.05 | 3.9549 | -18.1496 |
|  |  |  |  | 0.1 | 3.9819 | -18.9548 |
|  |  |  |  | 0.2 | 4.0358 | -20.5652 |
|  |  |  |  | 0.5 | 4.1975 | -25.3963 |
|  |  |  |  | 1 | 4.4671 | -33.4483 |
|  |  |  |  | 2 | 5.0061 | -49.5522 |
|  |  |  |  | 4 | 6.0842 | -81.7599 |
|  |  |  |  | 10 | 9.3186 | -178.3832 |
|  |  |  |  | 20 | 14.7092 | -339.4220 |
|  |  |  |  | 30 | 20.0999 | -500.4608 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 3.9292 | -17.3800 |
|  |  |  |  | 0.05 | 3.9311 | -17.4361 |
|  |  |  |  | 0.1 | 3.9334 | -17.5061 |
|  |  |  |  | 0.2 | 3.9381 | -17.6461 |
|  |  |  |  | 0.5 | 3.9522 | -18.0662 |
|  |  |  |  | 1 | 3.9756 | -18.7664 |
|  |  |  |  | 2 | 4.0225 | -20.1667 |
|  |  |  |  | 4 | 4.1162 | -22.9674 |
|  |  |  |  | 10 | 4.3975 | -31.3694 |
|  |  |  |  | 20 | 4.8662 | -45.3728 |
|  |  |  |  | 30 | 5.3350 | -59.3762 |

Observation 3: Under the assumption of RRM measurement performed by “LR+MR”, while the measurement period of MR is 6400ms, and the measurement period of LR is 1280ms. When compared with legacy measurement period is 1280ms. For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging.

- LP-WUR monitor LP-WUS under “continuously monitoring” and “duty cycle” manner both have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Proposal 1: For I-DRX cycle, the different value of “additional transition energy from ultra-deep sleep” will cause different conclusion of whether LP-WUR has power saving gain compared to I-DRX with PEI or not. Prioritize the case ‘LP-WUR on state’ is smaller than 1.

Proposal 2: Both “continuously monitoring” and “discontinuously monitoring” manner should be further studied.

Observation 4: “LP-WUS duty cycle ration” will cause different power saving gain. Smaller “LP-WUS duty cycle ration” will have more power saving gain.

MR performs measurement every 16 IDRX cycles

We assume RRM measurement performed by “LR+MR”, the measurement period of MR is 20480ms, i.e. MR performs measurement every 16 IDRX cycles, and the measurement period of LR is 1280ms. The RRM measurement period of PEI is 1280ms.

Table 5: Power Saving Gain of LP-WUS compared to DRX with PEI (Per UE Paging)

Additional transition energy from ultra-deep sleep is 15000

Relative Power for LP-WUR off is 0.001

| DRX settings | SINR | Paging Rate (Per UE) | Scheme | Relative Power for LP-WUR on | Average power | PS gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
| I-DRX = 1.28s (107 Cycle) (FAR = 0.1%) | Low | 1% | Rel-17 UE with PEI | - | 3.3806 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 1.0350 | 69.3845 |
|  |  |  |  | 0.05 | 1.0750 | 68.2012 |
|  |  |  |  | 0.1 | 1.1250 | 66.7222 |
|  |  |  |  | 0.2 | 1.2250 | 63.7642 |
|  |  |  |  | 0.5 | 1.5250 | 54.8901 |
|  |  |  |  | 1 | 2.0250 | 40.1000 |
|  |  |  |  | 2 | 3.0250 | 10.5197 |
|  |  |  |  | 4 | 5.0250 | -48.6409 |
|  |  |  |  | 10 | 11.0250 | -226.1227 |
|  |  |  |  | 20 | 21.0250 | -521.9257 |
|  |  |  |  | 30 | 31.0250 | -817.7286 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 1.0307 | 69.5125 |
|  |  |  |  | 0.05 | 1.0522 | 68.8747 |
|  |  |  |  | 0.1 | 1.0792 | 68.0774 |
|  |  |  |  | 0.2 | 1.1331 | 66.4828 |
|  |  |  |  | 0.5 | 1.2948 | 61.6991 |
|  |  |  |  | 1 | 1.5643 | 53.7263 |
|  |  |  |  | 2 | 2.1034 | 37.7807 |
|  |  |  |  | 4 | 3.1815 | 5.8893 |
|  |  |  |  | 10 | 6.4159 | -89.7846 |
|  |  |  |  | 20 | 11.8065 | -249.2412 |
|  |  |  |  | 30 | 17.1972 | -408.6977 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 1.0264 | 69.6382 |
|  |  |  |  | 0.05 | 1.0283 | 69.5827 |
|  |  |  |  | 0.1 | 1.0306 | 69.5134 |
|  |  |  |  | 0.2 | 1.0353 | 69.3747 |
|  |  |  |  | 0.5 | 1.0494 | 68.9588 |
|  |  |  |  | 1 | 1.0728 | 68.2655 |
|  |  |  |  | 2 | 1.1197 | 66.8789 |
|  |  |  |  | 4 | 1.2135 | 64.1058 |
|  |  |  |  | 10 | 1.4947 | 55.7863 |
|  |  |  |  | 20 | 1.9635 | 41.9205 |
|  |  |  |  | 30 | 2.4322 | 28.0548 |
|  | Medium |  | Rel-17 UE with PEI | - | 3.3768 |  |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 1.0237 | 69.6855 |
|  |  |  |  | 0.05 | 1.0637 | 68.5009 |
|  |  |  |  | 0.1 | 1.1137 | 67.0203 |
|  |  |  |  | 0.2 | 1.2137 | 64.0589 |
|  |  |  |  | 0.5 | 1.5137 | 55.1749 |
|  |  |  |  | 1 | 2.0137 | 40.3681 |
|  |  |  |  | 2 | 3.0137 | 10.7546 |
|  |  |  |  | 4 | 5.0137 | -48.4725 |
|  |  |  |  | 10 | 11.0137 | -226.1537 |
|  |  |  |  | 20 | 21.0137 | -522.2891 |
|  |  |  |  | 30 | 31.0137 | -818.4245 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 1.0194 | 69.8134 |
|  |  |  |  | 0.05 | 1.0409 | 69.1748 |
|  |  |  |  | 0.1 | 1.0679 | 68.3767 |
|  |  |  |  | 0.2 | 1.1218 | 66.7803 |
|  |  |  |  | 0.5 | 1.2835 | 61.9912 |
|  |  |  |  | 1 | 1.5530 | 54.0095 |
|  |  |  |  | 2 | 2.0921 | 38.0459 |
|  |  |  |  | 4 | 3.1702 | 6.1188 |
|  |  |  |  | 10 | 6.4046 | -89.6626 |
|  |  |  |  | 20 | 11.7952 | -249.2981 |
|  |  |  |  | 30 | 17.1858 | -408.9337 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 1.0151 | 69.9395 |
|  |  |  |  | 0.05 | 1.0170 | 69.8840 |
|  |  |  |  | 0.1 | 1.0193 | 69.8146 |
|  |  |  |  | 0.2 | 1.0240 | 69.6758 |
|  |  |  |  | 0.5 | 1.0381 | 69.2593 |
|  |  |  |  | 1 | 1.0615 | 68.5653 |
|  |  |  |  | 2 | 1.1084 | 67.1771 |
|  |  |  |  | 4 | 1.2021 | 64.4009 |
|  |  |  |  | 10 | 1.4834 | 56.0721 |
|  |  |  |  | 20 | 1.9521 | 42.1907 |
|  |  |  |  | 30 | 2.4209 | 28.3094 |
|  | High |  | Rel-17 UE with PEI | - | 3.3720 |  |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 1.0123 | 69.9779 |
|  |  |  |  | 0.05 | 1.0523 | 68.7916 |
|  |  |  |  | 0.1 | 1.1023 | 67.3088 |
|  |  |  |  | 0.2 | 1.2023 | 64.3432 |
|  |  |  |  | 0.5 | 1.5023 | 55.4464 |
|  |  |  |  | 1 | 2.0023 | 40.6184 |
|  |  |  |  | 2 | 3.0023 | 10.9623 |
|  |  |  |  | 4 | 5.0023 | -48.3498 |
|  |  |  |  | 10 | 11.0023 | -226.2862 |
|  |  |  |  | 20 | 21.0023 | -522.8468 |
|  |  |  |  | 30 | 31.0023 | -819.4074 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 1.0080 | 70.1056 |
|  |  |  |  | 0.05 | 1.0296 | 69.4662 |
|  |  |  |  | 0.1 | 1.0566 | 68.6669 |
|  |  |  |  | 0.2 | 1.1105 | 67.0682 |
|  |  |  |  | 0.5 | 1.2722 | 62.2723 |
|  |  |  |  | 1 | 1.5417 | 54.2790 |
|  |  |  |  | 2 | 2.0808 | 38.2926 |
|  |  |  |  | 4 | 3.1589 | 6.3197 |
|  |  |  |  | 10 | 6.3933 | -89.5990 |
|  |  |  |  | 20 | 11.7839 | -249.4636 |
|  |  |  |  | 30 | 17.1745 | -409.3281 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 1.0038 | 70.2323 |
|  |  |  |  | 0.05 | 1.0056 | 70.1767 |
|  |  |  |  | 0.1 | 1.0080 | 70.1072 |
|  |  |  |  | 0.2 | 1.0127 | 69.9681 |
|  |  |  |  | 0.5 | 1.0267 | 69.5511 |
|  |  |  |  | 1 | 1.0502 | 68.8560 |
|  |  |  |  | 2 | 1.0970 | 67.4659 |
|  |  |  |  | 4 | 1.1908 | 64.6857 |
|  |  |  |  | 10 | 1.4720 | 56.3449 |
|  |  |  |  | 20 | 1.9408 | 42.4436 |
|  |  |  |  | 30 | 2.4095 | 28.5423 |

Observation 5: Under the assumption of RRM measurement performed by “LR+MR”, while the measurement period of MR is 20480ms, and the measurement period of LR is 1280ms. When compared with legacy measurement period is 1280ms. For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging.

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/2, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 10.52%~69.38% power saving gain, while have 10.75%~69.69% power saving gain under Medium SINR case, and have 10.96%~69.98% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 4/10/20/30, LP-WUR monitor LP-WUS under “continuously monitoring” manner have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/2/4, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% can have 5.88%~69.51% power saving gain under Low SINR case, while have 6.12%~69.81% power saving gain under Medium SINR case, and have 6.31%~70.11% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 10/20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/4/10/20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 0.78% can have 28.05%~69.64% power saving gain under Low SINR case, while have 28.31%~69.94% power saving gain under Medium SINR case, and have 28.54%~70.23% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Assume the number of UEs in the group is 10, when RE is 1%, the per group paging probability RG is 9.56%. Evaluation results on power saving gain is summarized in Table 6.

Table 6: Power Saving Gain of LP-WUS compared to DRX with PEI (Per UE Group Paging)

Additional transition energy from ultra-deep sleep is 15000

Relative Power for LP-WUR off is 0.001

| DRX settings | SINR | Paging Rate (Per Group) | Scheme | Relative Power for LP-WUR on | Average power | PS gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
| I-DRX = 1.28s (107 Cycle) (FAR = 0.1%) | Low | 9.56% | Rel-17 UE with PEI | - | 3.4267 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 2.5023 | 26.9757 |
|  |  |  |  | 0.05 | 2.5423 | 25.8084 |
|  |  |  |  | 0.1 | 2.5923 | 24.3493 |
|  |  |  |  | 0.2 | 2.6923 | 21.4310 |
|  |  |  |  | 0.5 | 2.9923 | 12.6762 |
|  |  |  |  | 1 | 3.4923 | -1.9150 |
|  |  |  |  | 2 | 4.4923 | -31.0976 |
|  |  |  |  | 4 | 6.4923 | -89.4627 |
|  |  |  |  | 10 | 12.4923 | -264.5580 |
|  |  |  |  | 20 | 22.4923 | -556.3836 |
|  |  |  |  | 30 | 32.4923 | -848.2091 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 2.4982 | 27.0968 |
|  |  |  |  | 0.05 | 2.5197 | 26.4675 |
|  |  |  |  | 0.1 | 2.5467 | 25.6810 |
|  |  |  |  | 0.2 | 2.6006 | 24.1078 |
|  |  |  |  | 0.5 | 2.7623 | 19.3885 |
|  |  |  |  | 1 | 3.0319 | 11.5229 |
|  |  |  |  | 2 | 3.5709 | -4.2084 |
|  |  |  |  | 4 | 4.6490 | -35.6708 |
|  |  |  |  | 10 | 7.8834 | -130.0581 |
|  |  |  |  | 20 | 13.2740 | -287.3704 |
|  |  |  |  | 30 | 18.6647 | -444.6826 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 2.4942 | 27.2126 |
|  |  |  |  | 0.05 | 2.4961 | 27.1579 |
|  |  |  |  | 0.1 | 2.4984 | 27.0895 |
|  |  |  |  | 0.2 | 2.5031 | 26.9527 |
|  |  |  |  | 0.5 | 2.5172 | 26.5423 |
|  |  |  |  | 1 | 2.5406 | 25.8584 |
|  |  |  |  | 2 | 2.5875 | 24.4904 |
|  |  |  |  | 4 | 2.6812 | 21.7546 |
|  |  |  |  | 10 | 2.9625 | 13.5470 |
|  |  |  |  | 20 | 3.4312 | -0.1323 |
|  |  |  |  | 30 | 3.9000 | -13.8116 |
|  | Medium |  | Rel-17 UE with PEI | - | 3.3919 |  |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 2.3983 | 29.2917 |
|  |  |  |  | 0.05 | 2.4383 | 28.1124 |
|  |  |  |  | 0.1 | 2.4883 | 26.6383 |
|  |  |  |  | 0.2 | 2.5883 | 23.6901 |
|  |  |  |  | 0.5 | 2.8883 | 14.8454 |
|  |  |  |  | 1 | 3.3883 | 0.1043 |
|  |  |  |  | 2 | 4.3883 | -29.3780 |
|  |  |  |  | 4 | 6.3883 | -88.3427 |
|  |  |  |  | 10 | 12.3883 | -265.2365 |
|  |  |  |  | 20 | 22.3883 | -560.0595 |
|  |  |  |  | 30 | 32.3883 | -854.8826 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 2.3942 | 29.4140 |
|  |  |  |  | 0.05 | 2.4157 | 28.7783 |
|  |  |  |  | 0.1 | 2.4427 | 27.9837 |
|  |  |  |  | 0.2 | 2.4966 | 26.3944 |
|  |  |  |  | 0.5 | 2.6583 | 21.6266 |
|  |  |  |  | 1 | 2.9279 | 13.6802 |
|  |  |  |  | 2 | 3.4669 | -2.2126 |
|  |  |  |  | 4 | 4.5450 | -33.9982 |
|  |  |  |  | 10 | 7.7794 | -129.3551 |
|  |  |  |  | 20 | 13.1700 | -288.2831 |
|  |  |  |  | 30 | 18.5607 | -447.2112 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 2.3902 | 29.5314 |
|  |  |  |  | 0.05 | 2.3921 | 29.4761 |
|  |  |  |  | 0.1 | 2.3944 | 29.4070 |
|  |  |  |  | 0.2 | 2.3991 | 29.2688 |
|  |  |  |  | 0.5 | 2.4132 | 28.8542 |
|  |  |  |  | 1 | 2.4366 | 28.1632 |
|  |  |  |  | 2 | 2.4835 | 26.7812 |
|  |  |  |  | 4 | 2.5772 | 24.0173 |
|  |  |  |  | 10 | 2.8585 | 15.7254 |
|  |  |  |  | 20 | 3.3272 | 1.9055 |
|  |  |  |  | 30 | 3.7960 | -11.9143 |
|  | High |  | Rel-17 UE with PEI | - | 3.3474 |  |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 2.2943 | 31.4595 |
|  |  |  |  | 0.05 | 2.3343 | 30.2646 |
|  |  |  |  | 0.1 | 2.3843 | 28.7709 |
|  |  |  |  | 0.2 | 2.4843 | 25.7835 |
|  |  |  |  | 0.5 | 2.7843 | 16.8213 |
|  |  |  |  | 1 | 3.2843 | 1.8844 |
|  |  |  |  | 2 | 4.2843 | -27.9894 |
|  |  |  |  | 4 | 6.2843 | -87.7371 |
|  |  |  |  | 10 | 12.2843 | -266.9802 |
|  |  |  |  | 20 | 22.2843 | -565.7187 |
|  |  |  |  | 30 | 32.2843 | -864.4572 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 50%) | 0.01 | 2.2902 | 31.5834 |
|  |  |  |  | 0.05 | 2.3117 | 30.9393 |
|  |  |  |  | 0.1 | 2.3387 | 30.1341 |
|  |  |  |  | 0.2 | 2.3926 | 28.5237 |
|  |  |  |  | 0.5 | 2.5543 | 23.6925 |
|  |  |  |  | 1 | 2.8239 | 15.6406 |
|  |  |  |  | 2 | 3.3629 | -0.4633 |
|  |  |  |  | 4 | 4.4410 | -32.6710 |
|  |  |  |  | 10 | 7.6754 | -129.2942 |
|  |  |  |  | 20 | 13.0660 | -290.3329 |
|  |  |  |  | 30 | 18.4567 | -451.3717 |
|  |  |  | LP-WUR discontinuously monitoring (LP-WUS duty cycle ration is 0.78%) | 0.01 | 2.2862 | 31.7026 |
|  |  |  |  | 0.05 | 2.2881 | 31.6466 |
|  |  |  |  | 0.1 | 2.2904 | 31.5766 |
|  |  |  |  | 0.2 | 2.2951 | 31.4365 |
|  |  |  |  | 0.5 | 2.3092 | 31.0164 |
|  |  |  |  | 1 | 2.3326 | 30.3163 |
|  |  |  |  | 2 | 2.3795 | 28.9159 |
|  |  |  |  | 4 | 2.4732 | 26.1152 |
|  |  |  |  | 10 | 2.7545 | 17.7132 |
|  |  |  |  | 20 | 3.2232 | 3.7098 |
|  |  |  |  | 30 | 3.6920 | -10.2935 |

Observation 6: Under the assumption of RRM measurement performed by “LR+MR”, while the measurement period of MR is 20480ms, and the measurement period of LR is 1280ms. When compared with legacy measurement period is 1280ms. For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging.

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 12.67%~26.98% power saving gain, while have 14.85%~29.29% power saving gain under Medium SINR case, and have 16.82%~31.46% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 2/4/10/20/30, LP-WUR monitor LP-WUS under “continuously monitoring” manner have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% can have 11.52%~27.10% power saving gain under Low SINR case, while have 13.68%~29.41% power saving gain under Medium SINR case, and have 15.64%~31.58% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 2/4/10/20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/4/10/20, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 0.78% can have -0.1323%~27.21% power saving gain under Low SINR case, while have 1.91%~29.53% power saving gain under Medium SINR case, and have 3.71%~31.70% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Proposal 3: RRM measurement of MR would have a great influence on power saving gain of LP-WUS. RRM measurement relax for MR should be studied.

LP-WUS coverage evaluation

Evaluation Assumptions

- Evaluation methodologies

In RAN1#110-bis-e, it was agreed to study coverage of WUS based on methodology and assumptions in R17 CovEnh SI.

Agreement For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline. - MIL is used as the metric for LP-WUS coverage evaluation - urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded. Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830. Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

Thus, according to the agreement, we need to study the coverage of LP-WUS in terms of MIL margin.

- General Evaluation Assumption

Table 7: General Evaluation Assumption for LP-WUS Coverage Evaluation

| Number of RX chains at the UE’s MR | 2 Rx |
| --- | --- |
| Number of RX chains for LP-WUR | 1 Rx |
| Scenario and frequency | Urban: 2.6GHz (TDD) |
| Reference data rates for MR | Urban: PDSCH 10Mbps, PUSCH 1Mbps |
| Reference PDCCH configuration | SCS; 15kHz/30kHz. Aggregation level; 8. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. BLER; 1% BLER,. |
| Pathloss model (select from LoS or NLoS) | NloS |
| Bandwidth | 100MHz for 2.6GHz. |
| Channel model for link-level simulation | TDL-C |
| Delay spread | 300ns |
| UE velocity | 3km/h |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 TXRUs for 700 MHz - 64TxRUs for 2.6 and 4 GHz. |

- Evaluation Assumption

Table 8: Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| Waveform | OOK-1 / OOK-2 / OOK-4 / FSK-1 |
| Channel structure | Option 3: Payload (48bits)+CRC(16bits) |
| SCS of OFDM generator for NR signal | 30kHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, o Option 1b: M =1 for SCS = 30 kHz (same as NR signal) |
|  | o Option 2b: M =8 for SCS = 30 kHz (same as NR signal) |
|  | o Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 |
|  | Other options are up to companies to report |
| WUS duration | Number of OFDM symbols: 8/64 symbols |
| MDR/FAR assumption |  The miss-detection rate (MDR) of LP-WUS 1%, |
|  |  The false-alarm rate (FAR) of LP-WUS |
|  | o 0.1% |
|  |  Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold |
|  | o Companies to report the selected reference time duration values and the associated number of WUS attempts/trials |
| Code scheme | Manchester code, code rate (1/2) |
| gNB Channel BW | 20MHz |
| LP-WUS BW | Option 1: |
|  | - 5MHz including subcarriers for guard band |
|  | - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS |
|  | GB is symmetrically placed on each side of LP-WUS |
| Filter | 5-th Order filter (e.g. Butterworth, Chebyshev, …) with 4.32 MHz bandwidth, |
|  | Companies to report any other assumptions if needed |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; |
|  | EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB |
| Sampling Rate | - 3.84MHz |
| ADC bit width | 8-bit |
| Channel Model | TDL-C, Delay Spread = 300 ns, UE velocity = 3 km/h |

Evaluation Results

Figure 1 Coverage performance of LP-WUS

We provide the coverage comparison between LP-WUS and legacy NR signals. The results are summary as following.

Table 9: Coverage performance for LP-WUS and legacy NR signals

|  | PDCCH AL8 | PUSCH | OOK1 | OOK2 | OOK4 | FSK1 |
| --- | --- | --- | --- | --- | --- | --- |
| Urban (2.6GHz) | 163.54 dB | 135.88 dB | 143.78 | 133.03 | 141.49 | 139.40 |

Observation 7: The coverage performance of LP-WUS is worse than PDCCH.

Proposal 4: Channel structure and Waveform should be determined.

Proposal 5: The data rate and payload of LP-WUS should be determined.

LP-WUS overhead evaluation

In this section, the LP-WUS part overhead is provided in the Table 10. We consider the LP-WUS number in time and given the estimated resourced occupied by WUS. The 1 beam case is generally used.

Table 10: System overhead for LP-WUS for RRC IDLE/INACTIVE

| Signal | # of info bits carried by signal | Overhead (%) | T resources per beam | F resources | SCS | number of paged UE per second or signal periodicity | number of UE per group | Number of LP-WUS transmissions per second | System configuration, e.g., BW and etc. | the target channel | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP-WUS | 48 | 0.0720% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 10 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.2808% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 39 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.3600% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 50 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.5760% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 10 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 2.2464% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 39 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 2.8800% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 50 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0144% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 10 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0562% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 39 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0720% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 50 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.1152% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 10 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.4493% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 39 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.5760% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 50 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0072% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 1 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0281% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 3.9 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0360% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 5 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0576% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 1 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.2246% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 3.9 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.2880% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 5 | 20MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0014% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 1 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0056% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 3.9 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0072% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 5 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0115% | 48 symbols | 5.04MHz | 30kHz | 10 | 1 | 1 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0449% | 48 symbols | 5.04MHz | 30kHz | 39 | 1 | 3.9 | 100MHz, 1beam | Msg.3 |  |
| LP-WUS | 48 | 0.0576% | 48 symbols | 5.04MHz | 30kHz | 50 | 1 | 5 | 100MHz, 1beam | Msg.3 |  |

Conclusion

In this contribution, we provide some evaluation results on power consumption and coverage performance. We have following observations and proposals:

Observation 1: Under the assumption of RRM measurement performed by “LR+MR”, while the measurement period of MR is 6400ms, and the measurement period of LR is 1280ms. When compared with legacy measurement period is 1280ms. For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging.

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 1.43%~15.92% power saving gain, while have 1.65%~16.16% power saving gain under Medium SINR case, and have 1.84%~16.37% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 1/2/4/10/20/30, LP-WUR monitor LP-WUS under “continuously monitoring” manner have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% can have 0.26%~16.04% power saving gain under Low SINR case, while have 0.48%~16.28% power saving gain under Medium SINR case, and have 0.67%~16.50% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 2/4/10/20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/4/10, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 0.78% can have 2.32%~16.17% power saving gain under Low SINR case, while have 2.54%~16.41% power saving gain under Medium SINR case, and have 2.74%~16.63% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 0.78% have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 2: Relative Power of LP-WUR off takes small influence on power saving gain of LP-WUS.

Observation 3: Under the assumption of RRM measurement performed by “LR+MR”, while the measurement period of MR is 6400ms, and the measurement period of LR is 1280ms. When compared with legacy measurement period is 1280ms. For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging.

- LP-WUR monitor LP-WUS under “continuously monitoring” and “duty cycle” manner both have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Proposal 1: For I-DRX cycle, the different value of “additional transition energy from ultra-deep sleep” will cause different conclusion of whether LP-WUR has power saving gain compared to I-DRX with PEI or not. Prioritize the case ‘LP-WUR on state’ is smaller than 1.

Proposal 2: Both “continuously monitoring” and “discontinuously monitoring” manner should be further studied.

Observation 4: “LP-WUS duty cycle ration” will cause different power saving gain. Smaller “LP-WUS duty cycle ration” will have more power saving gain.

Observation 5: Under the assumption of RRM measurement performed by “LR+MR”, while the measurement period of MR is 20480ms, and the measurement period of LR is 1280ms. When compared with legacy measurement period is 1280ms. For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging.

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/2, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 10.52%~69.38% power saving gain, while have 10.75%~69.69% power saving gain under Medium SINR case, and have 10.96%~69.98% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 4/10/20/30, LP-WUR monitor LP-WUS under “continuously monitoring” manner have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/2/4, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% can have 5.88%~69.51% power saving gain under Low SINR case, while have 6.12%~69.81% power saving gain under Medium SINR case, and have 6.31%~70.11% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 10/20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% have no power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/4/10/20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 0.78% can have 28.05%~69.64% power saving gain under Low SINR case, while have 28.31%~69.94% power saving gain under Medium SINR case, and have 28.54%~70.23% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 6: Under the assumption of RRM measurement performed by “LR+MR”, while the measurement period of MR is 20480ms, and the measurement period of LR is 1280ms. When compared with legacy measurement period is 1280ms. For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging.

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 12.67%~26.98% power saving gain, while have 14.85%~29.29% power saving gain under Medium SINR case, and have 16.82%~31.46% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 2/4/10/20/30, LP-WUR monitor LP-WUS under “continuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% can have 11.52%~27.10% power saving gain under Low SINR case, while have 13.68%~29.41% power saving gain under Medium SINR case, and have 15.64%~31.58% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 2/4/10/20/30, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 50% have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

- When relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.2/0.5/1/4/10/20, LP-WUR monitor LP-WUS under “duty cycle” manner with LP-WUS duty cycle ration is 0.78% can have -0.1323%~27.21% power saving gain under Low SINR case, while have 1.91%~29.53% power saving gain under Medium SINR case, and have 3.71%~31.70% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Proposal 3: RRM measurement of MR would have a great influence on power saving gain of LP-WUS. RRM measurement relax for MR should be studied.

Observation 7: The coverage performance of LP-WUS is worse than PDCCH.

Proposal 4: Channel structure and Waveform should be determined.

Proposal 5: The data rate and payload of LP-WUS should be determined.

Reference

- Chair’s notes RAN1#113 eom