3GPP TSG-RAN WG1 Meeting #114bis	R1-230xxxx

# Xiamen, China, October 9th - October 13th, 2023

Agenda Item:	6

Source:	Moderator (Lenovo)

Title:	Summary on HARQ timing for CEMode B

Document for:	Discussion and decision

Introduction

This document summarizes the discussions during RAN1#114bis agenda 6 on the following CR.

R1-2309791	Draft CR on HARQ timing for CEMode B	Lenovo

R1-2309887	Clarification on UL timing for CE Mode B	Ericsson

Discussion

  - Summary of the CR

As observed by [Lenovo, Ericsson], for FDD, the HARQ timing for multiple TBs scheduling for UE configured with CEMode B is missing. The corresponding CRs are proposed.

TP1a from Lenovo in R1-2309791

TS36.213

- 10.2	Uplink HARQ-ACK timing

For TDD or for FDD-TDD and primary cell frame structure type 2 or for FDD-TDD and primary cell frame structure type 1, if a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell, "UL/DL configuration" of the serving cell in Clause 10.2 refers to the UL/DL configuration given by the parameter eimta-HARQ-ReferenceConfig-r12 for the serving cell unless specified otherwise.

<Unchanged parts are omitted>

For FDD, if a BL/CE UE is configured with CEModeA, the UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, or if the UE is configured with CEModeB and multiple TB are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.2.1 in subframe(s) sb+ki with b=0,1,⋯NTB-1,  i =0,1, …, N-1, where

-	is the number of scheduled TB determined in the corresponding DCI;

-	if the UE is not configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config and the UE is not in half-duplex FDD operation

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	otherwise

-	s0=max n0+4,  nL+2, sb=max {nb+4,  sb-1+Nb-1}, b≠0

<Unchanged parts are omitted>

TP2a from Ericsson in R1-2309887

TS36.213

------------------------------------------- Text Start --------------------------------------------------------------

- 10.2	Uplink HARQ-ACK timing

For TDD or for FDD-TDD and primary cell frame structure type 2 or for FDD-TDD and primary cell frame structure type 1, if a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell, "UL/DL configuration" of the serving cell in Clause 10.2 refers to the UL/DL configuration given by the parameter eimta-HARQ-ReferenceConfig-r12 for the serving cell unless specified otherwise.

------------------------------------------ Text Omitted ---------------------------------------------------------

For FDD, if a BL/CE UE is configured with CEModeA or CEModeB, and if the UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.2.1 in subframe(s) sb+ki with b=0,1,⋯NTB-1,  i =0,1, …, N-1, where

  - Discussion

Question 1: Do you agree with the intention of the CR?

| Company | Comments |
| --- | --- |

Question 2: Do you agree with the any of CR above? If not, what is your proposal?

| Company | Comments |
| --- | --- |

- References

R1-2309791	Draft CR on HARQ timing for CEMode B	Lenovo

R1-2309887	Clarification on UL timing for CE Mode B	Ericsson