3GPP TSG RAN WG1 #115	R1-2310896

Chicago, USA. 13th – 17th November 2023

Agenda item:		8.16.8

Source:	Nokia, Nokia Shanghai Bell

Title:	UE features for Further NR coverage enhancements

WI code:	NR_cov_enh2

Document for:		Discussion and Decision

# Introduction

Some progress was made on the Coverage Enhancement UE feature groups for Rel-18 in RAN1 #114bis [1]. Three FGs have been agreed as shown in the appendix. They are.

- FG 54-1 PRACH coverage enhancements

- FG 54-3 Dynamic waveform switching

- FG 54-3a PHR enhancement for dynamic waveform switching

Several component values and their details are still FFS. In this contribution, we are providing our inputs to address those open issues.

# Discussion

As the PRACH coverage enhancement feature operates on a cell-specific configuration before the UE capabilities are known to the network, it is highly desirable that there is just a single monolithic FG that defines a binary supported/not supported UE capability. Thus, RAN1 should avoid adding any further granularity to the FG 54-1 PRACH coverage enhancements FG. We are addressing the open items in FG and proposing the following components.

- For FG 54-1

  - FFS1: whether to separate this FG for CBRA and CFRA. We don’t see a need for separate FGs for CBRA and CFRA and recommend keeping them as components.

  - FFS2: the granularity of the FG is “Per band”

For the Dynamic waveform switching FG 54-3, there are several open issues. We address them as follows

- For FG 54-3

  - FFS1: Support of dynamic waveform switching for DCI format 0_1/0_2[/0_3]. Including DCI format 0_3 would require the format to first support the DWS bit. Hence for now we’d need to leave FG 0_3 in square brackets and either confirm or remove DCI 0_3 based on whether the functional support is specified for it or not.

  - FFS2: whether to separate this FG for multi-PUSCH scheduling – we recommend including multi-PUSCH as a component.

  - FFS3: whether to separate this FG for single-carrier case and multiple-carrier case – We would like to keep this as an FFS for the time being.

  - FFS4: The granularity of the FG is “per FS”

- For FG 54-3a

  - FFS1: The granularity of the FG is “Per FS’’

Table 1.  Coverage enhancements UE Features

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-1 | PRACH coverage enhancements | Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx spatial filter. Support for CBRA and CFRA |  | Yes | N/A | UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter. | Per band | N/A | N/A | N/A | RACH capability {CBRA, CFRA, Both CBRA and CFRA} | Optional with capability signalling. |
| 54. NR_cov_enh2 | 54-3 | Dynamic waveform switching | Support of dynamic waveform switching for DCI format 0_1/0_2[/0_3]. Support for multi-PUSCH scheduling FFS whether/how to separate this FG for single-carrier case and multiple-carrier case |  | Yes | N/A | Dynamic waveform switching is not supported | Per FS | N/A | N/A | N/A | If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2 | Optional with capability signaling. |
| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH | 54-3 | Yes | N/A | PHR enhancement for dynamic waveform switching is not supported | Per FS | N/A | N/A | N/A |  | Optional with capability signaling. |

# Conclusion

In this contribution, we make the following proposals:

Proposal 1: Consider our proposals and recommendations in section 2 on PRACH coverage enhancement and Dynamic waveform switching.

Proposal 2: Adopt the Table 1 for UE features for Further NR coverage enhancements.

# References

- R1-2310635	Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis	Moderators (AT&T, NTT DOCOMO, INC.), October. 2023

- RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom, June 2022

Appendix:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-1 | PRACH coverage enhancements | Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx spatial filter. FFS whether to separate this FG for CBRA and CFRA |  | Yes |  | UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter. |  | N/A | N/A | N/A |  | Optional with capability signalling. |
| 54. NR_cov_enh2 | 54-3 | Dynamic waveform switching | Support of dynamic waveform switching for DCI format 0_1/0_2[/0_3]. FFS whether to separate this FG for multi-PUSCH scheduling FFS whether/how to separate this FG for single-carrier case and multiple-carrier case |  | Yes |  | Dynamic waveform switching is not supported | FFS | N/A | N/A | N/A | If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2 | Optional with capability signaling. |
| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH | 54-3 | Yes | N/A |  |  |  |  |  |  | Optional with capability signaling. |