3GPP TSG RAN WG1#115	                                   R1-2311028

Chicago, USA, November 13th – November 17th, 2023

Title:  	Discussion on UE features for MC enhancements

Source:  	ZTE

Agenda item:  	8.16.12

Document for:	Discussion/Decision

Introduction

After RAN1#112 meeting, the WI “multi-carrier enhancements” has been completed from RAN1 perspective. In this contribution, we provide our views on UE features for multi-cell scheduling.

UE features for multi-cell scheduling

For the FG 49-1/1a/1b and FG 49-2/2a/2b, some components are the same as shown in the appendix. The same component is discussed together.

- Whether a scheduling cell of a set of cells can be scheduled by another cell (FG 49-1/FG 49-2)

[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]

In the current spec, a cell can only be either scheduled cell or scheduling cell for legacy cross cell scheduling. When the scheduling cell is outside the set of cells, if it is configured with another cell to monitor its PDCCH, it means that it is a scheduling cell for a set and the scheduled cell scheduled by another cell as well. It is not in line with the legacy principle. It should not be supported at this stage.

Proposal 1: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell.

- Separate FG for the specific case (FG 49-1 and FG 49-2)

FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells

If the scheduling cell and the scheduled cell have the same SCS, there is no fundamental difference between the case of scheduling cell in the set and the case of scheduling cell outside the set. In addition, it was agreed that the reference can be any one of the scheduled cells. It can also be the scheduling cell if scheduling cell is included in the set. The only difference is the search space configuration, which is also the same as the legacy. Therefore, there is no need to separate this FG for the case when scheduling cell is not included in the set and/or when scheduling cell is not the reference cell for the set.

Proposal 2: There should be no need to separate this FG for the case when scheduling cell is not included in the set and/or when scheduling cell is not the reference cell for the set.

- The number of unicast DCI (FG 49-1/1b and FG 49-2/2b)

Regarding the DCI processing, the following was agreed in RAN1#114bis.

Agreement For 49-1: - [At least for the case if UE does not support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 according to 49-1;] - If the scheduling cell is in the set and is the reference cell; - One unicast SC-DCI (for self scheduling) or one DCI 1_3 for the set of cells that schedules at least the scheduling cell per slot of scheduling cell, or; - One unicast SC-DCI (for self scheduling) and one DCI format 1_3 for the set of cells that schedules other than the scheduling cell per slot of scheduling cell - If the scheduling cell is NOT in the set; - One DCI 1_3 per slot of scheduling cell for the set of cells - FFS when the scheduling cell is in the set but is NOT the reference cell; - FFS one DCI 1_3 per slot of scheduling cell for the set of cells - FFS No SC-DCI is configured for the scheduling cell

The DCI format 0_3/1_3 are used for unicast scheduling. It is also agreed that it is counted only on the reference cell. Therefore, compared with the legacy DCI format, the only difference is that it may have a larger size than the legacy DCI from the PDCCH monitoring perspective. So, it is just a specific DCI for the reference cell. For the number of unicast DCI that can be processed by the UE for the set of cells, the legacy UE capability is enough. From the scheduled cell perspective, there is at most one PDSCH in a scheduled cell. Therefore, the UE can process at most N DCIs including at most 1 MC-DCI, where N is the number of scheduled cells in the set, and the MC-DCI and SC-DCI cannot schedule the PDSCH in the same slot in the same cell simultaneously.

In addition, monitoring the DCI format 0_3/1_3 for a set of cells and the legacy DCI formats for the cells in the set should be supported by default since the DCI size budget and monitoring budget keeps unchanged after introducing the multi-cell scheduling.

Proposal 3: The UE can process at most N DCIs including at most 1 MC-DCI in a PDCCH monitoring occasion, where N is the number of scheduled cells in the set, and the MC-DCI and SC-DCI cannot schedule the PDSCH in the same slot in the same cell simultaneously.

Proposal 4: Monitoring the DCI format 0_3/1_3 for a set of cells and the legacy DCI formats for the cells in the set should be supported by default.

- FG 49-1a and 49-2a

As discussed above, there is no fundamental difference between the of scheduling cell in the set and the case of scheduled outside the set. FG 49-1a should be merged into FG 49-1 and FG 49-2a should be merged into FG 49-2.

Proposal 5: FG 49-1a should be merged into FG 49-1 and FG 49-2a should be merged into FG 49-2.

- FG 49-3: Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell

As discussed above, DCI format 0_3/1_3 is a specific unicast DCI. Even though DCI format 0_3/1_3 is introduced, the UE capability on the PDCCH monitoring does not change, including BD/CCE budget and DCI size budget. Therefore, monitoring legacy DCI formats and DCI format 0_3/1_3 on the same serving cell should be the basic feature for multi-cell scheduling.

Proposal 6: Monitoring legacy DCI formats and DCI format 0_3/1_3 on the same serving cell should be the basic feature for multi-cell scheduling

- FG 49-5a/FG49-5b

Tyep-3 codebook construction is not related to the DCI formats. Introducing multi-cell scheduling has no impact to Type-3 codebook. Regarding the indication, there is no difference between legacy DCI format and DCI format 1_3. If the UE support both Type-3 codebook and multi-cell scheduling, it means that Type-3 codebook can be triggered by DCI format 1_3. If the UE does not support Type-3 codebook but multi-cell scheduling, it means that gNB cannot configure Type-3 codebook indication in any DCI format.

Proposal 7: FG 49-5a or FG 49-5b is not needed.

Conclusion

In this contribution, we provide our analysis for UE capabilities for Rel-18 MC WI and we have the following proposals.

Proposal 1: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell.

Proposal 2: There should be no need to separate this FG for the case when scheduling cell is not included in the set and/or when scheduling cell is not the reference cell for the set.

Proposal 3: The UE can process at most N DCIs including at most 1 MC-DCI in a PDCCH monitoring occasion, where N is the number of scheduled cells in the set, and the MC-DCI and SC-DCI cannot schedule the PDSCH in the same slot in the same cell simultaneously.

Proposal 4: Monitoring the DCI format 0_3/1_3 for a set of cells and the legacy DCI formats for the cells in the set should be supported by default.

Proposal 5: FG 49-1a should be merged into FG 49-1 and FG 49-2a should be merged into FG 49-2.

Proposal 6: Monitoring legacy DCI formats and DCI format 0_3/1_3 on the same serving cell should be the basic feature for multi-cell scheduling

Proposal 7: FG 49-5a or FG 49-5b is not needed.

Appendix

| 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type: value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] FFS whether to report max number of sets of cells supported by UE across PUCCH groups 7) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field 10) The number of unicast DL DCI to process [for a set of cells] configured for multi-cell PDSCH scheduling by a DCI format 1_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD/TDD scheduling cell - FFS whether to count DCI format 1_3 only or both legacy DCI formats and DCI format 1_3 [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- |
| 49-1a | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) HARQ feedback based on Type 1 HARQ codebook 7) FDRA field based co-scheduled cell indication |
| 49-1b | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1b 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type:value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields 9) The number of unicast UL DCI to process [for a set of cells] configured for multi-cell PUSCH scheduling by a DCI format 0_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD scheduling cell - Two unicast DCI per slot of scheduling cell [for the set of cells] for TDD scheduling cell - FFS whether to count DCI format 0_3 only or both legacy DCI formats and DCI format 0_3 [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| 49-2a | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) FDRA field based co-scheduled cell indication |
| 49-2b | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| 49-3 | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell |
| 49-3x | Advanced UE capability for larger number of unicast DL DCI | Details FFS |
| 49-3y | Advanced UE capability for larger number of unicast UL DCI | Details FFS |
| 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 |
| 49-4b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 |
| 49-4c | Configurable Type-1A fields for DCI format 0_3/1_3 | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 |
| 49-4d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 |
| 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 |
| 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 |