3GPP TSG RAN WG1 #115	R1-2311048

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.14.2

Source: 	Fujitsu

Title: 	    Views on specification impact for CSI feedback enhancement

Document for:	Discussion/Decision

Introduction

Channel state information (CSI) feedback enhancement using AI/ML-based method is agreed to be one of the use cases in this study item [1]. In the previous RAN WG1 #114bis meeting, the following aspects of the potential specification impacts were discussed [2] for the CSI compression using two-sided AI/ML models (Fig. 1):

- Training collaborations.

- Dataset delivery for Type 3 training.

- Quantization alignment.

- CSI reporting format.

Fig. 1: The two-sided AI/ML model [1].

In this paper, we further share our views on potential specification impacts on various topics for both two sub use cases in CSI feedback enhancement using AI/ML methods. We also share our views on the recommendation of the normative work of CSI feedback enhancement in the coming release.

CSI Prediction Using UE-Side Model

The scope for the study of the potential specification impacts on the sub use case of CSI prediction using UE-side AI/ML model is determined in RAN #100.

Proposal 1 (RAN #100 RP-231481): RAN tasks RAN WGs to study a subset of the specification impacts of CSI prediction limited to the following aspects:

- data collection procedures reusing as much as possible what is defined for UE side use cases,

- monitoring procedure and associated fallback mechanism to legacy CSI reporting.

In the RAN1 #114 meeting, three types of performance monitoring for functionality-based LCM proposed by companies are summarized. In this paper, we further study three types of performance monitoring in the agreement, as well as the data collection for the AI/ML-based CSI prediction.

Agreement For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculate the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculate the performance metric(s) - UE report performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/ deactivation/switching what is defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report. - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

Among the three types of AI/ML monitoring for CSI prediction, UE-side monitoring is preferred. The main reason is that both the output of the AI/ML model and the ground-truth CSI are available at the UE side. As a result, a UE is able to accurately measure the performance of the AI/ML model for the CSI prediction, without the need of the help from the NW. So, the type 1 and type 3 monitoring are preferred.

On the other hand, the performance metrics are calculated at NW in type 2 performance monitoring. Our view is that it is not necessary. The reasons are two-folded. First, as mentioned above, a UE is able to compute the performance metric perfectly. Second, the computation of the performance metrics, e.g., SGCS or NMSE, does not require a high complexity and power consumption, and the UE is able to implement it with low cost. So, we find that it is not necessary to let the NW compute the performance metric in AI/ML-based CSI prediction. Furthermore, a large overhead and delay is caused by sending the output of the AI/ML model and the ground-truth CSI to the NW, which makes type 2 monitoring less practical.

In summary, UE-side performance monitoring is preferred for the CSI prediction sub use case. The similar view was also shared by a number of companies in the previous meeting [2]. We have the following proposal.

Proposal-1: In CSI prediction using UE-side AI/ML model sub use case, prioritize Type 1 and Type 3 functionality-based LCM for performance monitoring.

On the issue of UE-side data collection for AI/ML model training, the assistance information is worth to be studied. We take the time stamp as an example to illustrate the reasons. First of all, since the CSI prediction is supposed to be done based on historical CSI from multiple slots, the necessity of time stamps on the data is straightforward. Furthermore, taking the channel aging issue into account, the data should be marked time stamps to reflect the properties of the channel in the past, now, and future. We have the following proposal.

Proposal-2: In CSI prediction using UE-side AI/ML model sub use case, study assistance information needed for data collection at UE-side.

CSI Compression Using Two-Sided Models

In this section, we present our views on the potential specification impacts on the sub use case of CSI compression using two-sided AI/ML models. The topics on AI/ML model alignment, inference-related STD impacts, AI/ML model performance monitoring, and data collection are covered.

AI/ML Model Alignment

The AI/ML-based CSI generation part is often equipped at the UE side, which maps the channel information into a bit sequence as a part of the CSI. The AI/ML-based CSI reconstruction part is equipped at the NW side, which recovers the channel information. At the NW side, a gNB is expected to support various AI/ML-based models manufactured by a number of vendors. On the other hand, however, only a subset of AI/ML-based models may be supported by a UE. The knowledge of the AI/ML-based models supported by a UE may not be assumed to be available at the NW. For this reason, it is difficult for the NW to configure the information of proper AI/ML-based CSI generation part(s) to a UE. As a result, the AI/ML model alignment should be done before configurations and AI/ML-based CSI reporting.

Our view is that the NW and UE should align the models they support before communications in multi-vendor collaboration. In the RAN1 #113 meeting [4], the feasibility and procedures of the AI/ML model alignment is agreed to be further studied. In the RAN1 #114 meeting [3], a total of six options are observed for defining the pairing information.

Observation

In CSI compression using two-sided model use case, at least the following options have been proposed by companies to define the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB:

- Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use.

- Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use.

- Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID.

- Option 4: The pairing information is in the forms of by the dataset ID during type 3 sequential training.

- Option 5: The pairing information is in the forms of a training session ID to a prior training session (e.g., API) between NW and UE.

- Option 6: The pairing information is up to UE/NW offline co-engineering alignment, transparent to 3GPP specification.

- Note: the disclosure of the vendor information during the model pairing procedure and model identification procedure should be considered.

- Note: If each UE side model is compatible with all NW side model, the information is not needed for the UE.

- Note: Above does not imply there is a need for a central entity for defining/storing/maintaining the IDs.

A proposal is also made for the AI/ML model alignment during the previous meeting as follows [2]. We are supportive of the NW and UE interactions in general. Regarding the highlighted bullets, we prefer to keep the first one, i.e., “Pairing information can be considered as model ID”, since only model ID is agreed to be reported in a functionality.

Proposal 2-3-1(v1): In CSI compression using two-sided model use case, in order to enable the UE to select a CSI generation model compatible with the CSI reconstruction model used by the gNB, the following aspects have been proposed: - UE report the supported AI/ML based CSI feedback features/FGs in capability report. - Additional information, if needed, to align the pairing information: - UE initiated: UE reports the pairing information in capability report for NW confirmation. - NW initiated: NW indicates the pairing information supported for UE confirmation. - [Pairing information can be considered as model ID]. - [Model ID can be used to represent the paring information as a starting point]

For the additional information for aligning the pairing information, we support both the UE initiated and NW initiated approaches. In fact, both the two methods have their own merit. Specifically, for the UE initiated approach, the NW sends request for UE capability for the supported pairing information, and UE reports what is requested. For the NW initiated approach, the NW sends request for UE capability as well as the pairing information supported by the NW. The UE only reports the pairing information within the scope of what the NW indicates. This potentially reduces the amount of information that the UE sends via uplink. As a result, both the two approaches are worth to be investigated.

Proposal-3: In CSI compression using two-sided model use case, support the NW and UE interaction for AI/ML model alignment.

  - UE initiated: UE reports the pairing information for NW confirmation.

  - NW initiated: NW indicates the pairing information supported for UE confirmation.

  - Pairing information can be considered as model ID.

We take Option 2 in the “Observation” as an example to describe a possible procedure of the model alignment. UE reports to the NW the IDs of the AI/ML models it supports at first. The NW, based on the AI/ML models it owns, assigns the IDs of the available AI/ML models to the UE. In this way, the UE and NW align their supported AI/ML models, and they are ready to communicate.

In the RAN2 #121 meeting, it is agreed that the model ID is assumed to be unique “globally”. The global model ID can be regarded as an identification for each one of the AI/ML models in use.

RAN2 #121 agreement [5] RAN2 assumes that Model ID is unique “globally”, e.g., in order to manage test certification each retrained version need to be identified.

In the following, we give an example to illustrate the AI/ML model alignment using global model IDs. As the setup, the AI/ML models supported by the UE and NW are given in the first and second columns of Table 1. The AI/ML models deployed at the NW-side are Model #A, #B, #C, #D, and #E. A UE supports Model #A, #C, and #X. As the first step, the UE reports to the NW these three AI/ML models it supports. Upon receiving the report from UE, the NW looks up the list of its available AI/ML models and finds out that the Model #X is beyond its reach. The NW then decides to assign Model #A and #C to this UE.

Table 1: AI/ML model alignment between UE and NW.

| NW-side supported AI/ML models | UE-side report | NW-side can assign (Paired model ID) |
| --- | --- | --- |
| Model #A | Model #A | Model #A |
| Model #B | Model #C | Model #C |
| Model #C | Model #X |  |
| Model #D |  |  |
| Model #E |  |  |

In the example above, the IDs, i.e., #A, #B, …, can be regarded as global model IDs. A UE reports to the NW, possibly through UE capability report, the global model IDs of the AI/ML models available for some scenario and configurations. Since the global model IDs may result in a large overhead, the NW may assign local model IDs to the UE to inform the outcome of the AI/ML model alignment. Compared to the global model IDs, local model IDs are only assigned to a subset of the set of available AI/ML models. The overhead of the former may be significantly smaller than that of the latter. We have the following proposal.

Proposal-4: In the CSI compression using two-sided AI/ML models sub use case, study using local model IDs in AI/ML model operations and CSI configuration/reporting after model alignment between UE and NW, which reduces the overhead compared to global model IDs.

In the previous meetings, the notion of pairing ID was proposed by companies in the agenda 9.2.1 for general framework, which may be used in model alignment in the CSI compression using two-sided models. It was suggested that the necessity of pairing ID can be discussed in each of the use cases. In the sub use case of CSI compression using two-sided AI/ML models, our view is that the global model ID is enough for model alignment, and there is no need to introduce pairing IDs.

The reasons are two folded. First, the global model IDs may carry the pairing information for two-sided AI/ML models. So global model IDs serve the purpose of model alignment. Second, pairing IDs do not reduce the overhead of doing model alignment. It is not clear the benefit of introducing the notion of pairing ID at least for model alignment.

Proposal-5: For the sub use case of CSI compression using two-sided AI/ML models, global model ID is sufficient for model alignment, and there is no need to introduce pairing IDs.

Inference-Related Specification Impacts

CSI Configurations and Reports

Progress has been made in RAN1 #113 in the topic of CSI configurations and reports, and the following agreement is reached. In particular, the contents of the CSI configurations and reports are discussed. In this paper, we further analyze the CSI configurations and CSI reporting formats for various AI/ML model settings.

Agreement In CSI compression using two-sided model use case, further study the applicability and potential specification impact for CSI configuration and report: - For network to indicate CSI reporting related information, gNB can indicate the UE with the one or more of following information: - Information indicating CSI payload size - Information indicating quantization method/granularity. - Rank restriction - Other payload related aspects - For UE determination/reporting of the actual CSI payload size, UE reports related information as configured by the NW.

The AI/ML model setting refers to how AI/ML models are deployed in spatial layers for possible reporting rank values. In the previous meetings in the sub-agenda 9.2.2.1, the combinations of layer common/specific and rank common/specific are evaluated and discussed. There are two potential directions for studying the CSI configurations required and CSI reporting formats from the perspective of AI/ML model settings. One is to design a separate configuration and reporting format for each of the possible settings. An alternative approach is to specify a unified configuration and reporting format that adapts to various settings.

Each of the possible AI/ML model settings may be subject to a featured CSI configuration and reporting format. Take layer common/specific as an example, the configurations may indicate whether the CSI is generated in a layer common/specific manner. For instance, the NW may configure to the UE only one (non-scalable) AI/ML model. The CSI generation may be restricted to layer common. However, in case the NW configures an AI/ML model for each layer, and the models are not the same, then the AI/ML model setting is layer specific for multi-layer CSI feedback. Furthermore, the AI/ML model settings configured may specify the CSI reporting formats. Specifically, for UE determination/reporting the actual CSI payload size, a UE may only need to report a single AI/ML model related information if configured “layer common”. This apparently helps reduce the overhead in multi-layer CSI reporting because it is not necessary to report the same AI/ML model related information multiple times for each layer, considering that the NW knows the setting of “layer common” it configures. However, multiple AI/ML models have to be reported to the NW if a UE is configured “layer specific”. As a result, a separate CSI configuration and reporting format should be studied for each AI/ML model setting.

On the other hand, for the sake of simplicity, a unified CSI configuration and CSI reporting format can be specified for all possible AI/ML model settings of interest. Specifically, the configuration and reporting format will be the same regardless of whether the AI/ML models are deployed in a layer common/specific or rank common/specific manner. The price to pay is that the overhead may be slightly larger than that in the case of AI/ML-model-setting-specific configurations and reporting, considering that we may have to report the same AI/ML model related information repeatedly for each layer in the case of “layer common”.

For the purpose of reducing the workload, our view is to down select one from the above two approaches, and study the CSI configurations and reporting formats. The outcome may serve as the input for the coming Rel-19, in which AI for air interface will be in the work item phase.

Proposal-6: In CSI compression using two-sided AI/ML models sub use case, further study the configurations and CSI reporting formats required for various AI/ML model settings. In particular, down select one from the following for the purpose of reducing the workload in normative phase.

- AI/ML-model-setting-specific CSI configurations and CSI reporting formats.

- A configuration and CSI reporting format adapting to various possibilities, including at least

- layer specific and rank common.

- layer specific and rank specific.

- layer common and rank common.

- layer common and rank specific.

Legacy CSI Reporting Principles

In the RAN1#113 meeting, the UCI format for the CSI generated by AI/ML method is discussed. It is agreed that the legacy CSI reporting principle is considered as a starting point. In this paper, we further analyze the UCI format and CSI reporting priority based on the agreement achieved [4].

Agreement

In CSI compression using two-sided model use case, for the study of UCI format, consider the legacy CSI reporting principle with CSI Part 1 and Part 2 as a starting point, where Part 1 has a network configured fixed size and Part 2 size is dynamic, determined by information in Part 1.

The bitwidth of a CSI report varies depending on the NW configuration and the channel. Specifically, the size of PMI depends on the rank number reported as indicated in RI. In legacy CSI reporting principles, the RI belongs to Part 1 CSI, and PMI is in Part 2 CSI. This enables the NW to estimate the payload size of the CSI, so as to configure reasonable payload size for the CSI report. From our point of view, the RI should also be in Part 1 (AI/ML-based) CSI, and the precoding-matrix-related information should be in Part 2.

In the case of insufficient uplink resources, a portion of the Part 2 CSI generated by the AI/ML-based method may be omitted according to some priority order, as what is done in the legacy codebook-based CSI reporting. In the legacy CSI reporting, the priority order is set either in the frequency domain, i.e., the subband indices, or by the components of the PMI, etc. However, in the AI/ML-based CSI, the frequency-domain correlation is exploited for compression, and the CSI generated by AI/ML-based method does not have the structure similar to the PMI indices. As a result, there is a need to study the rule for the CSI priority for AI/ML-based CSI feedback. As an example, the significance among spatial layers is an option for setting the CSI priority in the CSI generated by AI/ML-based method.

Proposal-7: For the sub use case of CSI compression using two-sided AI/ML models, study the rules for setting the CSI priority. As an example, the spatial layer indicator can be an option.

RI Determination

Rank indicator (RI) is an important component of CSI for channel measurement in legacy CSI reporting. It indicates the number of layers for downlink communication. It also helps estimate the size of the Part 2 CSI if PMI is reported, as mentioned above. As a result, RI is also an important component to be reported in the AI/ML-based CSI feedback, if precoding matrix is the input-CSI-NW and output-CSI-UE. Since the RI is determined by the channel estimation and rank restriction (if configured), no AI/ML-based CSI compression operation is involved. So, the legacy RI determination approach can be reused as a starting point in AI/ML-based CSI feedback.

Proposal-8: In the CSI compression using two-sided AI/ML models sub use case, the approach of RI determination in legacy CSI reporting can be used as a starting point.

AI/ML Model Performance Monitoring

In the RAN1 #112 meeting, the agreement of studying the potential specification impacts on intermediate KPI-based AI/ML model performance monitoring is achieved. The monitoring can be performed at UE side or NW side [6].

Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side. - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded. Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

In this paper, we study the intermediate-KPI-based AI/ML model performance monitoring. Both the UE-side and NW-side AI/ML monitoring are considered in this section. The follow-up mechanism after the AI/ML monitoring is also studied, including the mechanism of fallback to legacy codebook-based CSI reporting.

UE-Side Monitoring

In the previous meetings, the AI/ML model performance monitoring at UE side is proposed. Specifically, a proxy model is suggested to be equipped at the UE, which outputs some intermediate KPI, such as squared generalized cosine similarity (SGCS), which measures the performance of the AI/ML model in use. In this section, we analyze and present our views on the feasibility of this approach.

First of all, a careful evaluation and justification is needed before concluding that the cost of UE-side AI/ML model performance monitoring is small. Specifically, the issues of computational complexity, power consumption, as well as any additional assistance information needed for using the UE-side proxy model(s) should be taken into account. To reduce the computational complexity and power consumption, a proxy model is not expected to be complicated in structure nor large in size. This, however, looks contradict to a well performed AI/ML model by deep learning intuitively. In particular, the generalization capability of the said proxy model is not strong, i.e., a proxy model outputs reasonable prediction of the model performance in one scenario, may not work well in another scenario. To solve this issue, multiple proxy models are needed to cover various scenarios that a UE meets, e.g., an indoor factory area, city center with a large number of tall buildings, and the peak of a mountain, etc. However, additional complexity for a UE to manage multiple proxy models is inevitable. What is more, the mechanism of choosing the right proxy model is not clear without further assistance information, and this results in extra overhead. Consequently, the cost in the aforementioned various aspects should be carefully studied before concluding the feasibility of the UE-side AI/ML model performance monitoring.

Observation-1: In UE-side AI/ML model performance monitoring using a proxy model, the expectation of a simple structure and small size contradicts to the needs of a strong generalization capability for a proxy model to work well in various scenarios that a UE meets.

Observation-2: Using multiple proxy models for UE-side AI/ML model performance monitoring results in additional burden for model management, as well as potential additional overhead because of the assistance information required for choosing a right proxy model among multiple ones.

Furthermore, the reliability of the UE-side monitoring results may be in doubt because of a lack of a convincing mechanism for verification. In other words, it is not clear how to monitor the performance of the proxy model(s), as well as the cost for monitoring the proxy model(s). Similar to the necessity of monitoring a pair of well-trained AI/ML-based CSI generation and reconstruction parts, the proxy model(s), although possibly be well-trained, should also be monitored because it is also obtained in a data-driven approach. In the case of operating the proxy model(s) at collaboration level x, the NW does not control, or even have no idea about the performance of the proxy model. So, it is difficult for the NW to trust the reliability of the monitoring results for the decision of any follow-up actions.

In the case of other collaboration levels, additional assistance information is needed, which consumes uplink and/or downlink resources. We note that the additional cost introduced by monitoring the performance of proxy model(s) should be taken into consideration for concluding the necessity and feasibility of the UE-side AI/ML model performance monitoring.

Considering the aforementioned technical issues for UE-side AI/ML model performance monitoring, especially the risk of a lack of reliability of the monitoring results, the feasibility of UE-side monitoring is not obvious. Hence, we suggest carefully study the feasibility of UE-side AI/ML model performance monitoring using a proxy model(s) before discussing any further related specification impacts, such as triggering and means for reporting the monitoring metrics discussed in the RAN1 #112bis-e meeting [7].

Agreement In CSI compression using two-sided model use case, for UE-side monitoring, further study potential specification impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE.

Proposal-9: In the CSI compression using two-sided AI/ML models sub use case, the feasibility, reliability, and generalization capability of the UE-side AI/ML model performance monitoring using proxy model(s) should be evaluated and concluded before any further discussion on the related specification impacts.

On the other hand, NW-side AI/ML model performance monitoring has its own merit. Specifically, although reporting target CSI is needed in the NW-side monitoring, the monitoring results are reliable. In addition, as shown in our simulation evaluation [8], the overhead for reporting the target CSI can be greatly reduced with minor performance degradation by using the Rel-16 type II-like codebook quantization with new parameter values. In the next subsection, we study the NW-side AI/ML model performance monitoring.

NW-Side Monitoring

In this section, we study the AI/ML model performance monitoring at NW side. To simplify the presentation, we assume that the target CSI is the true right singular vectors of the channel matrix from realistic channel estimation.

Agreement In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring, including: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - RRC signaling and/or L1 signaling procedure to enable fast identification of AI/ML model performance. - Aperiodic/semi-persistent or periodic ground-truth CSI report.

Target CSI is needed to be reported to the NW in the AI/ML model performance monitoring at NW side. The issue of overhead should be considered since high-resolution ground-truth CSI is needed to guarantee an accurate monitoring result. Scalar quantization of the target CSI, e.g., using floating point numbers, may be an option of the ground-truth CSI, however, the overhead may be prohibitive. To reduce the overhead, the target CSI can be quantized using codebook-based quantization approach. One of the examples is the Rel-16 e-type II-like codebook with new parameter values. It is shown in our companion contribution [8] that Rel-16 e-type II-like codebook with specified or new parameter values offer high accuracy, which significantly reduces the overhead of reporting the ground-truth CSI. Specifically, link-level simulations are performed with the parameters summarized in Table 2. The monitoring error can be less than 0.04 in terms of SGCS under 90% monitoring accuracy by using Rel-16 e-type II-like codebook with PC Set B, which consumes 730 bits (< 92 bytes). However, a number of 3328 bytes is required to quantize the right singular vectors using floating point numbers (float32) according to the antenna configuration and the number of subbands as summarized in Table 3. This means that only 2.8% of the overhead for scalar quantization is consumed for the codebook-based quantization method. As a result, Rel-16 e-type II-like codebook with new parameter values should be regarded as a prioritized quantization approach for reporting ground-truth CSI in the NW-side AI/ML model performance monitoring.

Proposal-10: In the NW-side AI/ML model performance monitoring for CSI compression, prioritize the study of using the codebook-based quantization method to obtain the ground-truth CSI, e.g., Rel-16 e-type II-like codebook. Besides, adding new parameter values to legacy codebook for higher resolution ground-truth CSI should be studied.

Table 2. Simulation parameters.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| BS Antenna Element Number (Nt) | 32: (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ |
| UE Antenna Element Number (Nr) | 4: (1,2,2,1,1,1,2), (dH, dV) = (0.5, 0.5)λ |
| Channel Estimation | Realistic Channel Estimation |
| Channel Model | CDL-C |
| Bandwidth | 10 MHz |
| RB Number | 52 |
| Sub-Band Number | 13 |
| Carrier Frequency | 2 GHz |
| Sub-Carrier Spacing | 15 kHz |
| Delay Spread | 30/300 ns |
| Doppler shift | 100/200/400 Hz |

Table 3. New parameter values for Rel-16 e-type II-like codebook for NW-side AI/ML model performance monitoring.

| Parameter configuration | L | pv | β | N3 | R | Reference Amplitude(bits) | Difference Amplitude (bits) | Phase (bits) | Overhead (bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC Set A (PC 6 in e-type II) | 6 | 0.5 | 0.5 | 13 | 1 | 4 | 4 | 4 | 449 |
| PC Set B | 10 | 0.5 | 0.5 | 13 | 1 | 4 | 4 | 4 | 730 |
| PC Set C | 10 | 0.6 | 0.5 | 13 | 1 | 4 | 4 | 4 | 830 |
| PC Set D | 12 | 0.95 | 0.5 | 13 | 1 | 4 | 4 | 4 | 1579 |

The method of initiating the AI/ML model performance monitoring should be studied. In the AI/ML monitoring at NW side, the ground-truth CSI is needed at NW side so as to compute the squared generalized cosine similarity (SGCS) of the recovered CSI by the AI/ML-based CSI reconstruction part and the ground-truth CSI. As a result, triggering the reporting of the ground-truth CSI can be the method of initiating the NW-side AI/ML monitoring.

However, triggering the reporting of the ground-truth CSI may also mean initiating the NW-side data collection for model training. To distinguish it with reporting ground-truth CSI for NW-side AI/ML monitoring, different signaling may be used for these two purposes. Specifically, the physical layer signaling may be used for reporting the ground-truth CSI for NW-side AI/ML monitoring. Higher layer signaling may be used for transmitting the ground-truth CSI to the NW for model training. The reasons are two folded. First, since the latency of signaling in physical layer is much smaller than that in higher layers, the physical layer signaling works better than the other for obtaining the AI/ML monitoring results timely. Second, the fact that only a moderate amount of data is needed for AI/ML monitoring enables the use of physical layer signaling. On the other hand, a relatively large amount of data may be required for model training, so higher layer signaling may be more suitable than L1 signaling for model training. In this way, there is no confusion if different signaling are used for different purposes of reporting the ground-truth CSI. So, triggering the reporting of the ground-truth CSI can be an option for initiating the NW-side AI/ML monitoring.

Proposal-11: For the sub use case of CSI compression using two-sided AI/ML models, study the procedures needed for initiating the NW-side AI/ML model performance monitoring.

In the NW-side monitoring, an existing CSI feedback scheme may be used as a reference, as agreed in the RAN1 #112bis-e meeting [7].

Agreement In CSI compression using two-sided model use case, for NW-side monitoring, further study the necessity, feasibility and potential specification impact to enable performance monitoring using an existing CSI feedback scheme as the reference. - The association between AI/ML scheme and existing CSI feedback scheme for monitoring - Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI. - Other aspects are not precluded.

To monitor the performance of AI/ML models, the performance of the AI/ML-based CSI reporting may be compared with that obtained from the legacy codebook-based approach. In the intermediate KPI-based AI/ML model performance monitoring at NW side, this can be done by comparing the SGCS(AI) and the SGCS(codebook), where SGCS(AI) is the SGCS of the recovered CSI by the AI/ML-based CSI reconstruction part and the ground-truth CSI reported by a UE, and the SGCS(codebook) is the SGCS of the recovered CSI from the legacy PMI and the ground-truth CSI. For the computation of the SGCS(codebook), one approach is that a UE reports the PMI to the NW, who will then compute the SGCS(codebook). The second approach is to compute the SGCS(codebook) at UE side, which is then reported to the NW. The second approach is feasible because the recovered CSI from legacy PMI can be computed at the UE side. The advantage is that the uplink overhead may be reduced by reporting the SGCS(codebook), instead of the PMI. The third approach is to let NW selects PMI and computes SGCS based on the ground-truth CSI from a UE. The advantage is that the uplink overhead may be further reduced compared to the second approach, because the SGCS(codebook) will not be reported. However, the accuracy of the PMI selected by the NW is not guaranteed, because the accuracy of the ground-truth CSI used by NW for PMI selection may be lower than that of the target CSI used by UE for the PMI computation. In summary, we have the following proposal.

Proposal-12: In the CSI compression using two-sided AI/ML models sub use case, for the NW-side AI/ML model performance monitoring using an existing CSI feedback scheme as a reference, study the potential specification impacts for the following two options:

- Option-1: UE selects and reports PMI to the NW.

- Option-2: UE computes and reports the intermediate KPI for the reference scheme, e.g., the SGCS of the recovered CSI from PMI and the ground-truth CSI.

- Option-3: NW selects the PMI based on the ground-truth CSI reported by a UE.

Follow-up Mechanism

In the RAN1 #110bis-e meeting, it is agreed to study the specification impact related to the co-existence of the AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode, as well as the fallback mechanisms from the former to the latter [9].

Agreement In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

Under the framework of the coexistence of AI/ML-based and legacy non-AI/ML-based CSI feedback modes, follow-up actions after AI/ML monitoring are needed. In particular, the NW and/or UE may decide whether the CSI feedback mode is fallen back to the non-AI/ML or is kept in the AI/ML mode, depending on the monitoring result. It is likely that falling back to the legacy non-AI/ML-based approach is needed if a poor performance is indicated by monitoring results for the AI/ML model in use, and vice versa. The related signaling and procedures needed for the follow-up mechanism should be studied.

The decision for fall back can be made according to the comparison of some threshold with the SGCS of the ground-truth CSI and the recovered CSI from an AI/ML approach. The signaling needed for triggering fall back may be either physical layer signaling, such as DCI, or higher layer signaling, such as RRC. We have the following proposal.

Proposal-13: For the CSI compression using two-sided AI/ML models sub use case, study the procedures and signaling needed for the follow-up actions after the AI/ML model performance monitoring, including falling back to legacy codebook-based CSI reporting from AI/ML-based methods.

The mechanism for the further follow-up actions after falling back to non-AI/ML-based CSI reporting should also be studied. One option is to stay on the non-AI/ML-based CSI reporting. Another choice is to re-activate the AI/ML-based CSI reporting when appropriate. Specifically, the CSI reporting mode can be switched back again to AI/ML, if it outperforms the legacy codebook-based mode. The mechanism to determine which of the two is superior should be studied. The procedures and signaling for the AI/ML model performance monitoring may be re-used as a starting point.

The activation of the AI/ML-based CSI reporting may also be needed at the initial stage. The instance that a UE is turned on is an example of the initial stage. The idea is that the CSI reporting mode, which outperforms the other, should be used, instead of assigning by will. For example, the performance of the AI/ML model(s) to be used may be tested before deciding the use of AI/ML-based CSI reporting mode. The procedures and the signaling required in the case of AI/ML performance monitoring discussed above may be re-used in this scenario as a starting point. We have the following proposal.

Proposal-14: For the AI/ML model performance monitoring in the sub use case of CSI compression using two-sided AI/ML models, study the potential specification impacts on monitoring the performance of an AI/ML model in inactivate mode, taking at least the following cases into consideration.

- Initial activation of an AI/ML model.

- Re-activation of an AI/ML model.

Data Collection

In this section, we study the issue of NW-side data collection for AI/ML model training and monitoring. The contents and means of the data collection are discussed.

Delivering Ground-Truth CSI

The demand for data collection at NW side may vary depending on application scenarios, e.g., AI/ML model training and performance monitoring. On one hand, the latency requirements are different. Specifically, it is expected that the results of the AI/ML performance monitoring reflect timely the performance of the AI/ML model. Because of this, the requirement of low latency may be imposed on the data collection for monitoring. However, a low-latency data collection is not compulsory for the purpose of AI/ML model training. Specifically, the NW may only collect data for AI/ML model training whenever the traffic is not busy, considering the fact that the channel characteristics may not vary rapidly. On the other hand, the overhead issue varies in these two scenarios. In particular, the amount of data required for AI/ML performance monitoring is less than that required in the AI/ML model training. As a result, the data collection mechanism should be studied separately for different purposes and application scenarios.

Furthermore, the potential specification impacts may vary for data collection in the scenarios of AI/ML model training and performance monitoring. One of the aspects is the uplink signaling used for collecting the ground-truth CSI. Between the two options, i.e., physical-layer signaling and higher-layer signaling, our view is that the former is suitable for AI/ML model performance monitoring, while the latter may work well in AI/ML model training. Specifically, for AI/ML model performance monitoring, the amount of data needed may not be large, which makes collecting the data through UCI feasible. In addition, L1 signaling offers a lower latency than higher-layer one, which makes it a better choice for data collection for AI/ML model monitoring. For AI/ML model training, on the other hand, RRC signaling may be a better choice than L1 one because a large amount of data should be collected to guarantee a well-trained model. In addition, the latency requirement is relatively loose for AI/ML model training, which makes it feasible to be used in collecting data for model training. In summary, both L1 and higher layer signaling are options for data collection at NW, and the choice depends on the application scenario of interest. We have the following proposal.

Proposal-15: On the issue of NW-side data collection of ground-truth CSI in CSI compression, the container used should vary depending on the purpose of data collection.  Specifically, study the following methods:

- Option 1: physical layer signaling, e.g., UCI, for NW-side AI/ML model performance monitoring.

- Option 2: higher layer signaling, e.g., RRC, for AI/ML model training.

NW-Side Data Collection For AI/ML Model Training

In this section, we study the NW-side data collection for AI/ML model training. To reduce the overhead for sending ground-truth CSI from UE to NW, codebook-based quantization method is demonstrated promising based on the evaluation results from companies. As a result, it is agreed in RAN1 #112bis-e meeting [7] that the STD impacts on the codebook-based quantization for ground-truth CSI for NW-side data collection for AI/ML model training should be studied. In particular, parameter set enhancement of existing e-type II codebook should be studied based on evaluation results in the sub-agenda 9.2.2.1.

Agreement In CSI compression using two-sided model use case, further study the necessity and potential specification impact of the following aspects related to the ground truth CSI format for NW side data collection for model training: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization, based on evaluation results in 9.2.2.1 - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - Number of layers for which the ground truth data is collected. And whether UE or NW determine the number of layers for ground-truth CSI data collection.

In the previous meeting, it is suggested to further evaluate whether the enhanced parameter set beyond specified ones should be used, e.g., PC6 & PC 8 in Rel-16 e-type II codebook. In this paper, we present our view that enhanced parameter set is needed because significant training gains can be obtained.

Agreement For the evaluation of the R16 eType II-like codebook based high resolution quantization of the ground-truth CSI in the CSI compression for AI/ML training, regarding the evaluation of new values of eType II parameters, consider the legacy values of PC6&PC8 as the baseline/lower-bound of performance comparison. - Note: it has been agreed that Float32 is adopted as the baseline/upper-bound of performance comparison.

There are two candidate methods to introduce new parameter sets.

- Option 1: Changing the value(s) of at least one parameter in the existing parameter set(s), offering a higher-accuracy quantization than specified ones.

- Option 2: Enhancing existing parameter sets PC7 and/or PC8 by adding value(s) in pv, v>2.

In this section, we demonstrate that both the two options are useful for the training performance enhancement.

In our companion contribution [10], we show that the dataset of the ground-truth CSI obtained by quantizing the right singular vectors of the channel matrix by PC set D (Table 3) offers significant gain in training performance than that from PCs specified in 3GPP TS 38.214 [11]. Specifically, it is shown in Table 4 that an increment of 2~3% SGCS gain is achieved compared to PC6, and at least 1.5% SGCS gain compared to PC8. The details of the simulation are presented in [10]. It is demonstrated in our simulations that new parameter sets help reduce the degradation of the training performance caused by the inaccuracy of quantizing the target CSI using specified PCs.

Table 4. The loss in SGCS for the CSI generated by AI/ML models trained using ground-truth CSI quantized by Rel-16 e-type II-like codebook with various sets of parameter values, compared to that trained using ground-truth CSI quantized by Float32 scalar quantization.

| Feedback overhead(bits) | 56 | 84 | 102 | 156 | 214 | 268 |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 (3GPP TS 38.214) | -6.8% | -6.0% | -6.7% | -5.0% | -5.5% | -4.2% |
| PC6 (3GPP TS 38.214) | -3.0% | -3.3% | -3.2% | -3.1% | -2.9% | -1.7% |
| PC8 (3GPP TS 38.214) | -1.5% | -1.5% | -1.7% | -1.7% | -1.9% | -1.2% |
| PC Set D (in Table 3) | -0.2% | -0.5% | -0.3% | -0.1% | -0.1% | 0.5% |

Observation-3: A significant AI/ML model training performance enhancement is obtained by using the training dataset composed by Rel-16 e-type II-like codebook with enhanced parameter values, compared to that with specified parameter values, such as PC6 and PC8.

In the case of rank>2, the specified PC7 and PC8 may not be used to construct the training dataset. The main reason is that the codebook-based ground-truth CSI is missing for the third spatial layer and above, which causes training performance degradation in both layer common and layer specific settings. PC6, however, may not offer enough quantization resolution for high performance as shown in Table 4. As a result, new parameter sets are needed. As presented above, option 2 is a candidate solution.

Proposal-16: On collecting ground-truth CSI at NW side for AI/ML model training in CSI compression, enhancing parameter values of Rel-16 e-type II-like codebook offers significant gains, and hence should be specified.

Views on the Recommendation of the Normative Work

In this section, we share our views on the recommendation of the normative work for the use case of CSI feedback enhancement using AI/ML method in the coming Release 19.

Two sub use cases are studied in this SI, i.e., CSI compression using two-sided AI/ML models, and CSI prediction using UE-side AI/ML model. Extensive simulation works were performed to evaluate the benefits of the AI/ML method for the two sub use cases. It is observed that good performance gains are achieved in both two sub use cases, in terms of both intermediate KPI (e.g. SGCS) and eventual KPI (e.g., UPT). Since benefits are clear, both the two sub use cases should be included in the normative work.

In the sub use case of CSI compression using two-sided AI/ML models, potential specification impacts on various technical issues were discussed in this SI with a number of agreements as outcome. We are supportive to the normative work on data collection, AI/ML model alignment, inference, and LCM operations. We also support performance monitoring, and issues in training collaborations in some circumstances. In particular, we support the normative work of AI/ML model performance monitoring at NW side, but not at UE side. The reasons are presented in Sec. 3.3 in this paper. We also support the signaling and mechanisms to facilitate Type 1 and Type 3 training, e.g., dataset delivery. However, since Type 2 training is agreed to be deprioritize, it should not be included in the normative work.

In the sub use case of CSI prediction using UE-side AI/ML model, our recommendation is to follow the proposal in RAN #100 meeting, i.e., to do normative work on data collection and performance monitoring, and associated fallback mechanism to legacy CSI reporting.

We have the following proposal.

Proposal-17: For the use case of CSI feedback enhancement using AI/ML methods, at least the following are recommended for normative work from RAN1 perspective:

- Both the sub use cases of

  - CSI compression using two-sided AI/ML models.

  - CSI prediction using UE-side AI/ML model.

- For CSI compression using two-sided AI/ML models, normative works on

  - Signaling and mechanisms to facilitate data collection, model inference at both UE and NW sides.

  - Signaling and mechanisms to facilitate AI/ML performance monitoring at NW side.

  - Signaling and mechanisms to facilitate the alignment of UE and NW part models.

  - Signaling and mechanisms to facilitate training collaborations Type 1 and Type 3.

  - Signaling and mechanisms to facilitate LCM operations.

- For CSI prediction using UE-side AI/ML model, normative works on

  - Signaling and mechanisms to facilitate data collection.

  - Signaling and mechanisms to facilitate AI/ML performance monitoring and associated fallback mechanism to legacy CSI reporting.

For the sub use case of CSI compression, initial evaluations by companies show further gains for temporal-spatial-frequency-domain CSI compression on top of the spatial-frequency-domain CSI compression using two-sided AI/ML models. This is also a valuable direction in Rel-19. We have the following proposal.

Proposal-18: To further enhance the performance gain of CSI compression using two-sided AI/ML models, temporal-spatial-frequency-domain CSI compression can be considered in the normative work.

Conclusions

Observation-1: In UE-side AI/ML model performance monitoring using a proxy model, the expectation of a simple structure and small size contradicts to the needs of a strong generalization capability for a proxy model to work well in various scenarios that a UE meets.

Observation-2: Using multiple proxy models for UE-side AI/ML model performance monitoring results in additional burden for model management, as well as potential additional overhead because of the assistance information required for choosing a right proxy model among multiple ones.

Observation-3: A significant AI/ML model training performance enhancement is obtained by using the training dataset composed by Rel-16 e-type II-like codebook with enhanced parameter values, compared to that with specified parameter values, such as PC6 and PC8.

Proposal-1: In CSI prediction using UE-side AI/ML model sub use case, prioritize Type 1 and Type 3 functionality-based LCM for performance monitoring.

Proposal-2: In CSI prediction using UE-side AI/ML model sub use case, study assistance information needed for data collection at UE-side.

Proposal-3: In CSI compression using two-sided model use case, support the NW and UE interaction for AI/ML model alignment.

  - UE initiated: UE reports the pairing information for NW confirmation.

  - NW initiated: NW indicates the pairing information supported for UE confirmation.

  - Pairing information can be considered as model ID.

Proposal-4: In the CSI compression using two-sided AI/ML models sub use case, study using local model IDs in AI/ML model operations and CSI configuration/reporting after model alignment between UE and NW, which reduces the overhead compared to global model IDs.

Proposal-5: For the sub use case of CSI compression using two-sided AI/ML models, global model ID is sufficient for model alignment, and there is no need to introduce pairing IDs.

Proposal-6: In CSI compression using two-sided AI/ML models sub use case, further study the configurations and CSI reporting formats required for various AI/ML model settings. In particular, down select one from the following for the purpose of reducing the workload in normative phase.

- AI/ML-model-setting-specific CSI configurations and CSI reporting formats.

- A configuration and CSI reporting format adapting to various possibilities, including at least

- layer specific and rank common.

- layer specific and rank specific.

- layer common and rank common.

- layer common and rank specific.

Proposal-7: For the sub use case of CSI compression using two-sided AI/ML models, study the rules for setting the CSI priority. As an example, the spatial layer indicator can be an option.

Proposal-8: In the CSI compression using two-sided AI/ML models sub use case, the approach of RI determination in legacy CSI reporting can be used as a starting point.

Proposal-9: In the CSI compression using two-sided AI/ML models sub use case, the feasibility, reliability, and generalization capability of the UE-side AI/ML model performance monitoring using proxy model(s) should be evaluated and concluded before any further discussion on the related specification impacts.

Proposal-10: In the NW-side AI/ML model performance monitoring for CSI compression, prioritize the study of using the codebook-based quantization method to obtain the ground-truth CSI, e.g., Rel-16 e-type II-like codebook. Besides, adding new parameter values to legacy codebook for higher resolution ground-truth CSI should be studied.

Proposal-11: For the sub use case of CSI compression using two-sided AI/ML models, study the procedures needed for initiating the NW-side AI/ML model performance monitoring.

Proposal-12: In the CSI compression using two-sided AI/ML models sub use case, for the NW-side AI/ML model performance monitoring using an existing CSI feedback scheme as a reference, study the potential specification impacts for the following two options:

- Option-1: UE selects and reports PMI to the NW.

- Option-2: UE computes and reports the intermediate KPI for the reference scheme, e.g., the SGCS of the recovered CSI from PMI and the ground-truth CSI.

- Option-3: NW selects the PMI based on the ground-truth CSI reported by a UE.

Proposal-13: For the CSI compression using two-sided AI/ML models sub use case, study the procedures and signaling needed for the follow-up actions after the AI/ML model performance monitoring, including falling back to legacy codebook-based CSI reporting from AI/ML-based methods.

Proposal-14: For the AI/ML model performance monitoring in the sub use case of CSI compression using two-sided AI/ML models, study the potential specification impacts on monitoring the performance of an AI/ML model in inactivate mode, taking at least the following cases into consideration.

- Initial activation of an AI/ML model.

- Re-activation of an AI/ML model.

Proposal-15: On the issue of NW-side data collection of ground-truth CSI in CSI compression, the container used should vary depending on the purpose of data collection.  Specifically, study the following methods:

- Option 1: physical layer signaling, e.g., UCI, for NW-side AI/ML model performance monitoring.

- Option 2: higher layer signaling, e.g., RRC, for AI/ML model training.

Proposal-16: On collecting ground-truth CSI at NW side for AI/ML model training in CSI compression, enhancing parameter values of Rel-16 e-type II-like codebook offers significant gains, and hence should be specified.

Proposal-17: For the use case of CSI feedback enhancement using AI/ML methods, at least the following are recommended for normative work from RAN1 perspective:

- Both the sub use cases of

  - CSI compression using two-sided AI/ML models.

  - CSI prediction using UE-side AI/ML model.

- For CSI compression using two-sided AI/ML models, normative works on

  - Signaling and mechanisms to facilitate data collection, model inference at both UE and NW sides.

  - Signaling and mechanisms to facilitate AI/ML performance monitoring at NW side.

  - Signaling and mechanisms to facilitate the alignment of UE and NW part models.

  - Signaling and mechanisms to facilitate training collaborations Type 1 and Type 3.

  - Signaling and mechanisms to facilitate LCM operations.

- For CSI prediction using UE-side AI/ML model, normative works on

  - Signaling and mechanisms to facilitate data collection.

  - Signaling and mechanisms to facilitate AI/ML performance monitoring and associated fallback mechanism to legacy CSI reporting.

Proposal-18: To further enhance the performance gain of CSI compression using two-sided AI/ML models, temporal-spatial-frequency-domain CSI compression can be considered in the normative work.

- References

[1] 3GPP SR RP-221347: “Status report for study on AI/ML for NR air interface”, Jun. 2022.

[2] 3GPP “RAN1 chair’s notes (v17)”, RAN WG1 #114bis, Oct. 2023.

[3] 3GPP “RAN1 chair’s notes (eom4)”, RAN WG1 #114, Aug. 2023.

[4] 3GPP “RAN1 chair’s notes (eom)”, RAN WG1 #113, May 2023.

[5] 3GPP “RAN2 chair’s notes (eom)”, RAN WG2 #121, Athens, Greece, Feb. 2023.

[6] 3GPP “RAN1 chair’s notes (eom4)”, RAN WG1 #112, Athens, Greece, Feb. 2023.

[7] 3GPP “RAN1 chair’s notes (v15)”, RAN WG1 #112bis-e, Apr. 2023.

[8] 3GPP R1-2304764: “Evaluations on AI/ML for CSI feedback enhancement,” Fujitsu, May 2023.

[9] 3GPP “RAN1 chair’s notes (v17)”, RAN WG1 #110bis-e, Oct. 2022.

[10] 3GPP R1-2307155: “Evaluations on AI/ML for CSI feedback enhancement,” Fujitsu, Aug. 2023.

[11] 3GPP TS 38.214, v17.6.0, Jun. 2023.