3GPP TSG RAN WG1 #115	                                              	                 R1-2311109

Chicago, USA, November 13th - November 17th, 2023

Source:	vivo

Title:	Discussions on remaining issues of dynamic waveform switching

Agenda Item:	8.8.3

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss remaining issues of supporting dynamic waveform switching (DWS) between DFT-s-OFDM and CP-OFDM as approved in [1], including following aspects:

- DWS of PUSCH scheduled by DCI 0-3,

- PHR enhancement for DWS,

- Other issues,

- RRC parameter list .

# Discussions

## 2.1 DWS of PUSCH scheduled by DCI 0-3

In previous RAN1 meetings, following agreements and working assumptions have been made on the signaling of dynamic waveform indication. The working assumption is not confirmed and instead the single bit signaling is only agreed for indicating DWS of PUSCH transmissions with single TB.

Agreement For DCI based solution, - For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion Note: It does not imply that the waveform switching indication applies to other transmission or not - Indication from non-UL scheduling DCI is not supported. Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed Working Assumption Support new 1-bit field for dynamic waveform indication from UL scheduling DCI - Note: no change of the current size alignment procedure between UL DCI and DL DCI Agreement - For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. - Note: no change of the current size alignment procedure between UL DCI and DL DCI. Agreement - For UE configured with multi-PUSCH scheduling in time domain in a carrier (i.e. pusch-TimeDomainAllocationListForMultiPUSCH), DCI format 0_1 supports 1-bit field for dynamic waveform switching indication. - When configured, 1-bit field indicates waveform for all scheduled PUSCH transmissions. Agreement - Configuration of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2.

The remaining issue is on whether PUSCH transmissions in multiple carriers should be indicated with DWS independently. In our view, for the case when multiple PUSCH transmissions are scheduled on different cells by single DCI, there’s no need to restrict the PUSCH transmissions to have same waveform in different cells as the legacy waveforms for those PUSCH transmissions are already independently configured by RRC. If there’s concern on the overhead of DCI due to one additional bit being required for indicating DWS in each serving cell, at least for inter-band serving cells, independent DWS signalings are needed.

Regarding whether this work should be up to MCE topic or CE topic, in our understanding, it would be good to discuss in CE topic on whether DWS for DCI0_3 scheduled PUSCH is supported or not, since this has RRC impact, i.e. it requires one RRC parameter for the enabling or disabling of support of DWS for PUSCH scheduled by DCI0_3 format. Based on the decision in CE topic, if this feature is supported, MCE topic can discuss the detailed field type for DWS indication in DCI0_3.

Therefore, we have following proposal.

Proposal 1:

- Multiple bits are used to indicate DWS for PUSCH transmissions on different serving cells (at least for inter-band serving cells) scheduled by DCI0_3, and detailed DWS indication field type in DCI0_3 can be up to MCE topic to discuss during NR Rel-18 maintenance stage.

## 2.2 PHR enhancement for DWS

PHR enhancement for DWS was agreed to be supported based on following agreement made in RAN1 #114 meeting.

Agreement Support following enhancement to assist the scheduler in determining waveform switching: - Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH. - Note: Any MAC CE related design is up to RAN2 - Subject to separate UE capability - Details FFS. Agreement For reporting of power headroom information for assumed PUSCH using target waveform different from waveform of actual PUSCH, support the following: - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - DWS field needs to be configured for at least one DCI format for the BWP of the actual PUSCH, otherwise power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH contains: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - If UE reports power headroom information for assumed PUSCH in a PUSCH transmission, legacy PHR is also reported in the same PUSCH transmission. - No consensus in RAN1 if the following applies or not: if UE reports legacy PHR in a PUSCH transmission, power headroom information for assumed PUSCH is also reported. - Note: RAN endorsed the following at RAN#100: “RAN2 will not work on PHR triggering procedure for dynamic waveform switching in Rel-18 UL Coverage enh WI” [RP-231498]. Send LS to RAN2 to inform above agreement.

As can be seen, it has been agreed that “RAN2 will not work on PHR triggering procedure for dynamic waveform switching in Rel-18 UL Coverage enh WI” which means no new PHR triggering procedure is expected from RAN2 perspective. Therefore, whenever a legacy PHR is triggered and when DWS is enabled for the DCI scheduling a PUSCH carrying the PHR, the PHR of a different waveform should be calculated and reported in the same PUSCH transmission.

As is known, in current NR spec., PHR procedure is used to provide the serving gNB with a Type 1 power headroom, i.e. the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell.

According to section 7.7.1 of TS 38.213 v18.0.0, this Type 1 power headroom can be based on actual PUSCH transmission, which is calculated according to following formula,

[dB],

where PCMAX,f,c(i), PO_PUSCH,b,f,c(j), MRB,b,f,cPUSCH(i), αb,f,cj, PLb,f,c(qd), ∆TF,b,f,ci and fb,f,ci,l are defined in clause 7.1.1 of TS 38.213 v18.0.0.

Type 1 power headroom report can also be based on a reference format. In this case the PHR is virtual PHR, and for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, it is computed as

[dB]

where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. ΔTC = 0 dB. MPR, A-MPR, P-MPR and ΔTC are defined in TS 38.101-1, TS 38.101-2 and TS 38.101-3.

As can be seen, for actual PHR, it’s based on actual waveform since the PCMAX,f,c(i) is derived according to the actual waveform dependent power reduction values defined in 38.101. While for virtual PHR, it does not depend on waveform since power reduction related parameters are assumed to be 0 dB for determination of PCMAX,f,c(i) in the formula. Therefore, no matter how RAN2 defines the detailed MAC-CE for report of 2 PHRs of different waveforms, the RAN1 specification impact to support DWS is mainly to introduce the calculation of two Type 1 power headroom reports assuming both CP-OFDM and DFT-s-OFDM respectively when a Type 1 PHR reported is actual PHR, DWS is enabled for the scheduling DCI and the enhanced PHR report is enabled	. TP#1 in Table 1 is provided to capture this.

Proposal 2:

- Introduce the calculation of two Type 1 power headroom reports assuming CP-OFDM and DFT-s-OFDM waveforms respectively when a Type 1 PHR reported is actual PHR, DWS is enabled for the DCI scheduling the PUSCH and the enhanced PHR report is enabled, according to TP #1 provided in table 1.

Table 1. TP #1 to 3GPP TS 38.213 v18.0.0 for capturing PHR enhancement when DWS is enabled

Reason for change： Capturing PHR enhancement when DWS is enabled according to RAN1 agreement. Summary of change: Capturing PHR enhancement when DWS is enabled according to RAN1 agreement in section 7.7.1 of 38.213. Consequences if not approved: PHR enhancement when DWS is enabled is not captured in RAN1 specification and is not supported, which is against RAN1 agreement. <----------------------------------------------------------- Start of TP -------------------------------------------------------------> ### 7.7.1 Type 1 PH report If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as PHtype1,b,f,ci,j,qd,l= PCMAX,f,ci-PO_PUSCH,b,f,cj+10log102μ⋅MRB,b,f,cPUSCH(i)+αb,f,cj⋅PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dB] where PCMAX,f,c(i), PO_PUSCH,b,f,c(j), MRB,b,f,cPUSCH(i), αb,f,cj, PLb,f,c(qd), ∆TF,b,f,ci and fb,f,ci,l are defined in clause 7.1.1. If the actual PUSCH transmission is scheduled by a DCI format with a “Transform precoder indicator” field of size not equal to 0 bit and if the UE is configured with the higher layer parameter assumedPUSCHInfo set to 'enabled ', two Type 1 power headroom reports shall be calculated as described above assuming that transform precoding is applied and not applied respectively. If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration μ1 on active UL BWP b1 of carrier f1 of serving cell c1 is smaller than a SCS configuration μ2 on active UL BWP b2 of carrier f2 of serving cell c2, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP b1 that overlaps with multiple slots on active UL BWP b2, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP b2 that fully overlaps with the slot on active UL BWP b1. If a UE is configured with multiple cells for PUSCH transmissions, where a same SCS configuration on active UL BWP b1 of carrier f1 of serving cell c1 and active UL BWP b2 of carrier f2 of serving cell c2, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP b1, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the slot on active UL BWP b2 that overlaps with the slot on active UL BWP b1. If a UE is configured with multiple cells for PUSCH transmissions and provides a Type 1 power headroom report in a PUSCH transmission with PUSCH repetition Type B having a nominal repetition that spans multiple slots on active UL BWP b1 and overlaps with one or more slots on active UL BWP b2, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the one or more slots on active UL BWP b2 that overlaps with the multiple slots of the nominal repetition on active UL BWP b1. For a UE configured with EN-DC/NE-DC and capable of dynamic power sharing, if E-UTRA Dual Connectivity PHR [14, TS 36.321] is triggered, the UE provides power headroom of the first PUSCH, if any, on the determined NR slot as described in clause 7.7. If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP b1 of carrier f1 of serving cell c1, a second PUSCH transmission on active UL BWP b2 of carrier f2 of serving cell c2 that overlaps with the first PUSCH transmission if - the second PUSCH transmission is scheduled by a DCI format in a PDCCH received in a second PDCCH monitoring occasion, and - the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format scheduling an initial transmission of a transport block after a power headroom report was triggered or - the second PUSCH transmission is after the first uplink symbol of the first PUSCH transmission minus T'proc,2=Tproc,2 where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1 =1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant if the first PUSCH transmission is on a configured grant after a power headroom report was triggered. If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as PHtype1,b,f,ci,j,qd,l= PCMAX,f,c(i)-PO_PUSCH,b,f,cj+αb,f,cj⋅PLb,f,cqd+fb,f,ci,l [dB] where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in clause 7.1.1 and, if ul-powerControl is not provided, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0. If ul-powerControl is provided, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the indicated TCI-State or TCI-UL-State. <-------------------------------- unchanged text omitted ------------------------------->

In an LS from RAN2 [3], RAN1 is asked to inform RAN2 if a UE reporting PCMAX for actual and assumed PUSCH to support the DC/CA scenario has any impact to RAN1’s design in addition to that of the single carrier case. In our understanding, PCMAX actually used is transparent in RAN1 specification, the only RAN impact due to this PHR enhancement feature is that different waveforms should be assumed in RAN1 spec. for the PHR calculation as proposed in TP#1. And there’s no specific RAN1 impact due to PCMAX reporting for both waveforms, which can be informed to RAN2.

According to above, a draft LS reply [4] is prepared to be sent to RAN2.

Proposal 3:

- Send a reply LS to RAN2 to inform that there’s no RAN1 impact to support PCMAX report for both waveforms.

## 2.4 Other issues

One remaining issue is about the pi/2 BPSK modulation type determination when DWS is enabled. As is known, pi/2 BPSK is only supported for DFT-s-OFDM. When DWS is enabled, even if RRC configured waveform is DFT-s-OFDM, a CP-OFDM waveform may be indicated in DCI, in this case pi/2 BPSK cannot be assumed in the MCS tables Table 6.1.4.1-1 and Table 6.1.4.1-2 in 38.214 v18.0.0. However, in current spec., only RRC parameter is considered for determining whether pi/2 BPSK should be assumed in the MCS tables, which is wrong.

According to above, UE behaviour of pi/2 BPSK modulation type determination should be updated and we have following proposal.

Proposal 4:

- Specify that UE only assumes q = 1 in the related MCS tables for PUSCH transmission dynamically switched to DFT-s-OFDM when when Pi/2 BPSK modulation is applied, according to TP#2.

Table 2. TP #2 to 3GPP TS 38.214 v18.0.0 for pi/2 BPSK determination when DWS is configured

Reason for change： When DWS is enabled, even if RRC configured waveform is DFT-s-OFDM, a CP-OFDM waveform may be indicated in DCI, in this case pi/2 BPSK cannot be assumed in the MCS tables Table 6.1.4.1-1 and Table 6.1.4.1-2. However, in current spec., only RRC parameter is considered for determining whether pi/2 BPSK should be assumed in the MCS tables, which is wrong. Summary of change: Capture that if higher layer parameter tp-pi2BPSK is configured and if "Transform precoder indicator" field has 0 bit or indicates that transform precoder is enabled when the PUSCH transmission is scheduled by the DCI format with DWS enabled. Consequences if not approved: pi/2BPSK will be assumed in the MCS tables Table 6.1.4.1-1 and Table 6.1.4.1-2 as long as the RRC parameter tp-pi2BPSK is configured, which is wrong for case when DWS enabled and a CP-OFDM is indicated in DCI. <----------------------------------------------------------- Start of TP -------------------------------------------------------------> #### 6.1.4.1 Modulation order and target code rate determination <-------------------------------- unchanged text omitted -------------------------------> For Table 6.1.4.1-1 and Table 6.1.4.1-2, if higher layer parameter tp-pi2BPSK is configured and if "Transform precoder indicator" field has 0 bit or indicates that transform precoder is enabled when the PUSCH transmission is scheduled by the DCI format according to [5, TS 38.212], q = 1 otherwise q=2. <-------------------------------- unchanged text omitted ------------------------------->

## 2.5 RRC parameter list

On top of the agreed consolidated RRC parameter list sent to RAN2 in the LS [2] agreed in RAN1 #114bis meeting, following RRC parameter provided in Table 3 is needed for supporting dynamic waveform switching of PUSCH transmission scheduled by DCI 0_3 according proposal 1.

Proposal 5:

- RAN1 to update the RRC parameter list for supporting DWS of PUSCH transmission scheduled by DCI0_3 via including the RRC parameter provided in Table 3.

Table 3. New RRC parameter for supporting DWS of PUSCH scheduled by DCI0_3

| Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Description |
| --- | --- | --- | --- | --- |
| Dynamic waveform switching | PUSCH-Config | dynamicTransformPrecoderIndicationDCI-0-3 | New | Configures whether the field "Transform precoder indicator" is present or not in DCI format 0_3. If the field is set to enabled, DWS field is present in DCI format 0_3 and UE follows DWS field. If the field is set to disabled, DWS field is not present in DCI format 0_3 and UE follows legacy parameter (transformPrecoder) when scheduled using DCI format 0_3. |

# Conclusion

In this contribution, we discuss remaining issues of supporting dynamic waveform switching between DFT-s-OFDM and CP-OFDM waveform, and have following proposals:

Proposals 1 to 4:

- Multiple bits are used to indicate DWS for PUSCH transmissions on different serving cells (at least for inter-band serving cells) scheduled by DCI0_3, and detailed DWS indication field type in DCI0_3 can be up to MCE topic to discuss during NR Rel-18 maintenance stage.

- Introduce the calculation of two Type 1 power headroom reports assuming CP-OFDM and DFT-s-OFDM waveforms respectively when a Type 1 PHR reported is actual PHR, DWS is enabled for the DCI scheduling the PUSCH and the enhanced PHR report is enabled, according to TP #1 provided in table 1.

- Specify that UE only assumes q = 1 in the related MCS tables for PUSCH transmission dynamically switched to DFT-s-OFDM when when Pi/2 BPSK modulation is applied, according to TP#2.

- RAN1 to update the RRC parameter list for supporting DWS of PUSCH transmission scheduled by DCI0_3 via including the RRC parameter provided in Table 3.

# References

- RP-221858, Revised WID on Further NR coverage enhancements, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6th - 9th, 2022.

- R1-2310694, LS on Rel-18 higher-layers parameter list, 3GPP TSG RAN WG1 Meeting #114bis, Xiamen, China, October 9th - 13th, 2023.

- R2-2311616, LS to RAN1 on PHR reporting, 3GPP TSG-RAN WG2 Meeting #123bis, Xiamen, China, October 9th - 13th, 2023.

- R1-2311068, Draft reply LS to RAN2 on PHR reporting, 3GPP TSG RAN WG1 Meeting #115, November 13th - November 17th, 2023.