3GPP TSG RAN WG1 #115	R1-2311207

Chicago, USA, November 13th – November 17th, 2023

Source:	ZTE

Title:	Discussion on UE features for IoT-NTN

Agenda Item:	8.16.13

Document for:    Discussion

- Introduction

The latest version of UE features list for IoT-NTN is provided in [1]. In this contribution, our views on some features are elaborated with corresponding updates.

- Discussion on UE feature for IoT-NTN

As listed in  Table 1, the prerequisite [Rel. 18 2-3a] and [Rel. 18 2-3b] should be removed from FG 2-4a and FG2-4b, respectively. The triggered method and autonomous method can work independently. It is not preferred to couple the two methods when defining FGs.

Moreover, for the column of type, “per UE” is preferred as it is used in Rel-17 IoT-NTN UE feature. No benefit is observed by changing it to “per band”.

And for the column of note, the note “[Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible]” is preferred to be removed. In Rel-18 IoT-NTN, there is no differentiation of scenarios when discussing the enhancements. UE can just report whether to support a feature independent to the satellite orbit, and network perform corresponding actions. Therefore, no need of a signaling to differentiate GSO/NGSO.

In conclusion, the updates on UE features for IoT-NTN can be found in Table 1.

Proposal-1: The updates on the UE features for IoT-NTN listed in Table 1 should be supported.

Table 1 UE features for Rel-18 IoT-NTN

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-1a-1 | Semi-static HARQ feedback disabling for eMTC CE mode B in single TB case | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback in single TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in CE mode B Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1b-1 | Dynamic HARQ feedback disabling by DCI-based direct indication for eMTC CE mode B in single TB case | 1. UE receives DCI indication to directly indicate disabling HARQ feedback | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback in single TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 in CE mode B Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1c-1 | Dynamic HARQ feedback disabling by DCI-based overridden indication for eMTC CE mode B in single TB case | 1. UE receives DCI indication to override RRC configuration for disabling HARQ feedback | Rel.17 2-1, Rel-18 2-1a-1, 2-1b-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback in single TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 + Option 3 in CE mode B Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1a-2 | Semi-static HARQ feedback disabling for eMTC CE mode B in multi TB case | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel-16 1-11, Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback in multi TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in CE mode B Note: this applies to multi-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1b-2 | Dynamic HARQ feedback disabling by DCI-based direct indication for eMTC CE mode B in multi TB case | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel-16 1-11, Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback in multi TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 in CE mode B Note: this applies to multi-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1c-2 | Dynamic HARQ feedback disabling by DCI-based overridden indication for eMTC CE mode B in multi TB case | 1. UE receives DCI indication to override RRC configuration for disabling HARQ feedback | Rel-16 1-11, Rel.17 2-1, Rel-18 2-1a-2, 2-1b-2 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback in multi TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 + Option 3 in CE mode B Note: this applies to multi-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1d-1 | Semi-static HARQ feedback disabling for eMTC CE mode A in single TB case | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode A cannot disable HARQ feedback in single TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in CE mode A Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1d-2 | Semi-static HARQ feedback disabling for eMTC CE mode A in multi TB case | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel-16 1-10, Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode A cannot disable HARQ feedback in multi TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in CE mode A Note: this applies to multi-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-2 | Semi-static HARQ feedback disabling for SPS PDSCH for eMTC CE Mode A | UE reports ACK/NACK for the first SPS PDSCH after activation if enabled, and follow per-process HARQ feedback enabled/disabled configuration otherwise | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE Mode A cannot disable HARQ feedback for SPS PDSCH | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1e-1 | Semi-static HARQ feedback disabling for NB-IoT in single TB case | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback in single TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1f-1 | Dynamic HARQ feedback disabling by DCI-based direct indication for NB-IoT in single TB case | 1. UE receives DCI indication to directly indicate disabling HARQ feedback | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback in single TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1g-1 | Dynamic HARQ feedback disabling by DCI-based overridden indication for NB-IoT in single TB case | 1. UE receives DCI indication to override RRC configuration for disabling HARQ feedback 2. For single TB scheduled by single DCI, UE follows NPDCCH monitoring behavior for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI | Rel. 17 2-1, Rel-18 2-1e-1, 2-1f-1 | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback in single TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 + Option 3 Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1e-2 | Semi-static HARQ feedback disabling for NB-IoT in multi TB case | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | At least one of {Rel-16 2-6, 2-7}, Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback in multi TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 Note: this applies to multi-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1f-2 | Dynamic HARQ feedback disabling by DCI-based direct indication for NB-IoT in multi TB case | 1. UE receives DCI indication to directly indicate for disabling HARQ feedback | At least one of {Rel-16 2-6, 2-7}, Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback in multi TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 Note: this applies to multi-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1g-2 | Dynamic HARQ feedback disabling by DCI-based overridden indication for NB-IoT in multi TB case | 1. UE receives DCI indication to override RRC configuration for disabling HARQ feedback 2. For single TB scheduled by single DCI, UE follows NPDCCH monitoring behavior for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI | At least one of {Rel-16 2-6, 2-7}, Rel. 17 2-1, Rel-18 2-1e-2, 2-1f-2 | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback in multi TB case | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 + Option 3 Note: this applies to multi-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-3a | GNSS position fix in RRC Connected state for eMTC—triggered | 1. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode via RRCConnectionReestablishmentComplete, RRCConnectionReestablishmentComplete-NB and RRCConnectionReconfigurationComplete for HO case 2. UE receives eNB GNSS measurement trigger 4. UE re-acquires GNSS position fix within a configured gap 5. UE reports the remaining GNSS validity duration with MAC CE in connected mode | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE cannot get triggered GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] Note: This applies to non-DRX | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-4a | GNSS position fix in RRC Connected state for eMTC—autonomous | 1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode via RRCConnectionReestablishmentComplete, RRCConnectionReestablishmentComplete-NB and RRCConnectionReconfigurationComplete for HO case 3. UE reports the remaining GNSS validity duration with MAC CE in connected mode | [Rel. 18 2-3a] Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE cannot get autonomous GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-3b | GNSS position fix in RRC Connected state for NB-IoT—triggered | 1. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode via RRCConnectionReestablishmentComplete, RRCConnectionReestablishmentComplete-NB and RRCConnectionReconfigurationComplete for HO case 2. UE receives eNB GNSS measurement trigger 4. UE re-acquires GNSS position fix within a configured gap 5. UE reports the remaining GNSS validity duration with MAC CE in connected mode | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot get triggered GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-4b | GNSS position fix in RRC Connected state for NB-IoT—autonomous | 1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode via RRCConnectionReestablishmentComplete, RRCConnectionReestablishmentComplete-NB and RRCConnectionReconfigurationComplete for HO case 3. UE reports the remaining GNSS validity duration with MAC CE in connected mode | [Rel. 18 2-3b], Rel. 17 2-1b |  |  | Release 18 NB-IoT UE cannot get autonomous GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |

Reference

- R1-2310632, Updated RAN1 UE features list for Rel-18 LTE after RAN1#114bis, RAN1#114bis