3GPP TSG RAN WG1 #115                                                     R1-2311316

Chicago, USA, November 13th – November 17th, 2023

Source: 	CATT

Title:	Maintenance on CSI enhancement

Agenda Item:	8.1.2

Document for:	Discussion and Decision

Introduction

In this contribution, text proposals and views on some remaining issues on Type II codebook refinement for high/medium mobility and coherent-JT are provided.

Text Proposals and discussion

NZP CSI-RS number for interference measurement

Reason for change:

In Rel-15/16/17, if interference measurement is performed on NZP CSI-RS, there is no restriction of “only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement”, i.e., more than one resource in an NZP CSI-RS resource set for interference measurement can be configured. It is reasonable. Because the ports number of NZP CSI-RS for interference measurement may not be supported by only one resource. The supported ports number of an NZP CSI-RS resource is 1, 2, 4, 8, 12, 16, 24, and 32. Up to 18 NZP CSI-RS ports can be configured in an NZP CSI-RS resource set for interference measurement as specified in TS 38.214. One NZP CSI-RS port corresponds to one interference layer. For example, if 18 interference layers are to be measured, it is not possible to configure 18 ports with only one resource. One resource with 16 ports and another resource with 2 ports in an NZP CSI-RS resource set can be configured to support 18 NZP CSI-RS ports.

According to following agreement, legacy specification is fully reused for interference measurement, including the configuration for NZP CSI-RS or CSI-IM in relation to the configured CMR. Based on above discussion, legacy specification supports configuring more than one NZP CSI-RS resource in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. To fully reuse legacy specification, more than one NZP CSI-RS resource should be supported in the corresponding NZP-CSI-RS-ResourceSet for interference measurement for Rel-18 CJT and TypeII Doppler.

[112bis-e] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding CSI calculation and measurement, - The number of CSI-RS ports is the same for all the K configured CSI-RS resources comprising the CMR and the antenna ports for the same antenna port index across the K CSI-RS resources are the same. - All the K configured CSI-RS resources comprising the CMR share the same BW and RE locations - For interference measurement, legacy specification is fully reused, including the configuration for NZP CSI-RS for interference measurement or CSI-IM in relation to the configured CMR, i.e. only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured irrespective of the value of K ……

Summary of change:

Remove the text of “If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement.” in 38.214.

Consequence if not approved:

Configuration of NZP CSI-RS for interference measurement is restricted.

Proposal-1:

- For CJT and TypeII Doppler, the following TP is adopted in 5.2.1.4.1 of 38.214.

##### 5.2.1.4.1 Resource Setting configuration <Unchanged text is omitted> A UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' is expected to be configured with 1≤K≤4 CSI-RS resources in a resource set for channel measurement. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. A UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI', is expected to be configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement. For an aperiodic CSI-RS resource set for channel measurement, the K CSI-RS resources are triggered by the same triggering instance and the separation between two consecutive CSI-RS resources is m∈{1,2} slots, which is configured by higher layer parameter in the NZP-CSI-RS-ResourceSet. The UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. <Unchanged text is omitted>

NZP-CMR restriction if interference measurement is performed on NZP CSI-RS

Reason for change:

According to the following text, there is also restriction on Rel-18 CJT and TypeII Doppler that only one NZP CSI-RS for channel measurement can be configured when the interference measurement is performed on NZP CSI-RS. It is not correct for Rel-18 CJT and TypeII Doppler. When interference measurement is performed on NZP CSI-RS, K>=1 NZP CSI-RS resources in a NZP CSI-RS resource set should be supported for Rel-18 CJT and TypeII Doppler.

5.2.1.4.1 Resource Setting configuration <Unchanged text is omitted> Except for L1-SINR, if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement. <Unchanged text is omitted>

Summary of change:

Remove the restriction that only one NZP-CSI-RS for channel measurement can be configured when the interference measurement is performed on NZP CSI-RS for Rel-18 CJT and TypeII Doppler.

Consequence if not approved:

NZP CSI-RS for interference measurement is not supported if K>1 NZP CSI-RS resources in a NZP CSI-RS resource set are configured for Rel-18 CJT and TypeII Doppler.

Proposal-2:

- For CJT and TypeII Doppler, the following TP is adopted in 5.2.1.4.1 of 38.214.

5.2.1.4.1 Resource Setting configuration <Unchanged text is omitted> Except for L1-SINR, a CSI-ReportConfig configured with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18', or 'typeII-Doppler-PortSelection-r18', if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement. Except for L1-SINR, the UE configured with the higher layer parameter nzp-CSI-RS-ResourcesForInterference may expect no more than 18 NZP CSI-RS ports configured in a NZP CSI-RS resource set. <Unchanged text is omitted>

restrictedCMR-Selection for NTRP=1

In latest TS 38.214 and higher layer parameters list(R1-2310692), there is no description on the supported values of higher layer parameter restrictedCMR-Selection for NTRP=1, and no clear agreement about this. If the supported value of restrictedCMR-Selection can be “OFF” for NTRP=1, the NTRP-bit bitmap will be reported according to current TS 38.214 which is not align with the following agreement.

Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, support …… - if NTRP =1, that the NTRP-bit bitmap (for dynamic TRP selection) is not reported

Proposal-3:

- For CJT, clarify the supported value of restrictedCMR-Selection when NTRP=1.

We can either clarify that restrictedCMR-Selection can only be “ON” for NTRP=1, or clarify that restrictedCMR-Selection can be “ON” and “OFF” for NTRP=1.

If the value of restrictedCMR-Selection can only be “ON” for NTRP=1, we suggest to add the restriction in 38.214. Otherwise, if the value of restrictedCMR-Selection can be “ON” and “OFF” for NTRP=1, we suggest to correct current spec to ensure that the NTRP-bit bitmap for dynamic TRP selection is not reported.

Proposal-4:

- For CJT, adopt the following TP in 5.2.2.2.8 of 38.214, or add the following comment for higher parameter, if the value of restrictedCMR-Selection can only be “ON” for NTRP=1.

##### 5.2.2.2.8 Enhanced Type II codebook for CJT <Unchanged text is omitted> - The UE may be configured with higher layer parameter restrictedCMR-Selection. When NTRP=1, restrictedCMR-Selection is always configured. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N0 selected CSI-RS resources corresponds to the nonzero bit with lowest index. <Unchanged text is omitted>

| Parameter name in the spec | Description | Value range | Comment |
| --- | --- | --- | --- |
| restrictedCMR-Selection | enabling/disabling CSI-RS resource (TRP) restriction ON: N=NTRP OFF: dynamic TRP selection, i.e. N out of NTRP CSI-RS resources selected and reported by UE | {ON,OFF} | Only “ON” can be configured for NTRP=1. |

- For CJT, adopt the following TP in 5.2.2.2.8 of 38.214, if value of restrictedCMR-Selection can be “ON” and “OFF” for NTRP=1.

5.2.2.2.8 Enhanced Type II codebook for CJT <Unchanged text is omitted> - The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and if NTRP>1, the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N0 selected CSI-RS resources corresponds to the nonzero bit with lowest index. <Unchanged text is omitted>

Text Proposal to 38.212

The priority function is different for N4=1 and N4>1. For N4=1, the legacy priority function Pril,i,f is reused. For N4>1, the priority function Pril,i,f,j is newly defined. The related descriptions in 38.214 are shown as following. Therefore, we suggest to correct it according to 38.214.

### 5.2.3 CSI reporting using PUSCH <Unchanged text is omitted> - For Enhanced Type II and Enhanced Type II for predicted PMI configured with higher layer parameter N4=1, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where <Unchanged text is omitted> - For Enhanced Type II for predicted PMI configured with N4>1, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l,i, f and j, is associated with a priority value Pril,i,f,j=2L⋅υ⋅Mν⋅j+2L⋅υ⋅f+υ⋅i+l, for l=1,…,υ, i=0,1,…,2L-1, f=0,1,…,Mυ-1 and j=0,1. The element with the highest priority has the lowest associated value Pril,i,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where <Unchanged text is omitted>

Proposal-5:

- For Type II Doppler, the following TP is adopted in 6.3.2.1.2 of 38.212.

6.3.2.1.2 CSI Table 6.3.2.1.2-5F: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType= typeII-Doppler CSI report number; CSI fields. CSI report #n CSI part 2, group 0; PMI fields X1, from left to right as in Table 6.3.2.1.2-1C, if reported; The second time-domain wideband CQI as in Table 6.3.1.1.2-8B, if present and reported. CSI report #n CSI part 2, group 1; The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,10,l:l=1,…,υ, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυQ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q Pril,i,f for N4 = 1 or Pril,i,f, j for N4 > 1 defined in clause 5.2.3 of TS 38.214 [6], if present and if reported; The second time-domain subband differential CQI of all even subbands with increasing order of subband number, as in Table 6.3.1.1.2-8B, if present and if reported. CSI report #n CSI part 2, group 2; The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q Pril,i,f for N4 = 1 or Pril,i,f, j for N4 > 1 defined in clause 5.2.3 of TS 38.214 [6], if reported; The second time-domain subband differential CQI of all odd subbands with increasing order of subband number, as in Table 6.3.1.1.2-8B, if present and if reported.

The cited Table for typeII-Doppler-PortSelection in Table 6.3.2.1.2-5B is incorrect. The Tables 6.3.2.1.2-2D are newly provided for the PMI bitwidth of codebookType=typeII-Doppler-PortSelection, so we suggest to add the citation in Table 6.3.2.1.2-5B.

Proposal-6:

- For Type II Doppler, the following TP is adopted in 6.3.2.1.2 of 38.212.

6.3.2.1.2 CSI Table 6.3.2.1.2-5B: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-PortSelection-r17 or typeII-Doppler-PortSelection CSI report number; CSI fields. CSI report #n CSI part 2, group 0; PMI fields X1, from left to right as in Tables 6.3.2.1.2-2B/2D, if reported. CSI report #n CSI part 2, group 1; The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-2B/2D: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*K1M-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported. CSI report #n CSI part 2, group 2; The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-2B/2D: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported.

The cited Table for typeII-Doppler in Table 6.3.2.1.2-5F is incorrect. The Table 6.3.2.1.2-8B are newly provided for the RI/CQI bitwidth of codebookType=typeII-Doppler, so we suggest to modify the citation in Table 6.3.2.1.2-5F.

Proposal-7:

- For Type II Doppler, the following TP is adopted in 6.3.2.1.2 of 38.212.

6.3.2.1.2 CSI Table 6.3.2.1.2-5F: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType= typeII-Doppler CSI report number; CSI fields. CSI report #n CSI part 2, group 0; PMI fields X1, from left to right as in Table 6.3.2.1.2-1C, if reported; The second time-domain wideband CQI as in Table 6.3.12.1.2-8B, if present and reported. CSI report #n CSI part 2, group 1; The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,10,l:l=1,…,υ, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυQ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q defined in clause 5.2.3 of TS38.214, if present and if reported; The second time-domain subband differential CQI of all even subbands with increasing order of subband number, as in Table 6.3.12.1.2-8B, if present and if reported. CSI report #n CSI part 2, group 2; The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q defined in clause 5.2.3 of TS38.214, if reported; The second time-domain subband differential CQI of all odd subbands with increasing order of subband number, as in Table 6.3.12.1.2-8B, if present and if reported.

Conclusions

In this contribution, we provide the following proposal:

Proposal-1:

- For CJT and TypeII Doppler, the following TP is adopted in 5.2.1.4.1 of 38.214.

##### 5.2.1.4.1 Resource Setting configuration <Unchanged text is omitted> A UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' is expected to be configured with 1≤K≤4 CSI-RS resources in a resource set for channel measurement. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. A UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI', is expected to be configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement. For an aperiodic CSI-RS resource set for channel measurement, the K CSI-RS resources are triggered by the same triggering instance and the separation between two consecutive CSI-RS resources is m∈{1,2} slots, which is configured by higher layer parameter in the NZP-CSI-RS-ResourceSet. The UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. <Unchanged text is omitted>

Proposal-2:

- For CJT and TypeII Doppler, the following TP is adopted in 5.2.1.4.1 of 38.214.

5.2.1.4.1 Resource Setting configuration <Unchanged text is omitted> Except for L1-SINR, a CSI-ReportConfig configured with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18', or 'typeII-Doppler-PortSelection-r18', if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement. Except for L1-SINR, the UE configured with the higher layer parameter nzp-CSI-RS-ResourcesForInterference may expect no more than 18 NZP CSI-RS ports configured in a NZP CSI-RS resource set. <Unchanged text is omitted>

Proposal-3:

- For CJT, clarify the supported value of restrictedCMR-Selection when NTRP=1.

Proposal-4:

- For CJT, adopt the following TP in 5.2.2.2.8 of 38.214, or add the following comment for higher parameter, if the value of restrictedCMR-Selection can only be “ON” for NTRP=1.

##### 5.2.2.2.8 Enhanced Type II codebook for CJT <Unchanged text is omitted> - The UE may be configured with higher layer parameter restrictedCMR-Selection. When NTRP=1, restrictedCMR-Selection is always configured. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N0 selected CSI-RS resources corresponds to the nonzero bit with lowest index. <Unchanged text is omitted>

| Parameter name in the spec | Description | Value range | Comment |
| --- | --- | --- | --- |
| restrictedCMR-Selection | enabling/disabling CSI-RS resource (TRP) restriction ON: N=NTRP OFF: dynamic TRP selection, i.e. N out of NTRP CSI-RS resources selected and reported by UE | {ON,OFF} | Only “ON” can be configured for NTRP=1. |

- For CJT, adopt the following TP in 5.2.2.2.8 of 38.214, if value of restrictedCMR-Selection can be “ON” and “OFF” for NTRP=1.

##### 5.2.2.2.8 Enhanced Type II codebook for CJT <Unchanged text is omitted> - The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and if NTRP>1, the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N0 selected CSI-RS resources corresponds to the nonzero bit with lowest index. <Unchanged text is omitted>

Proposal-5:

- For Type II Doppler, the following TP is adopted in 6.3.2.1.2 of 38.212.

6.3.2.1.2 CSI Table 6.3.2.1.2-5F: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType= typeII-Doppler CSI report number; CSI fields. CSI report #n CSI part 2, group 0; PMI fields X1, from left to right as in Table 6.3.2.1.2-1C, if reported; The second time-domain wideband CQI as in Table 6.3.1.1.2-8B, if present and reported. CSI report #n CSI part 2, group 1; The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,10,l:l=1,…,υ, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυQ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q Pril,i,f for N4 = 1 or Pril,i,f, j for N4 > 1 defined in clause 5.2.3 of TS 38.214 [6], if present and if reported; The second time-domain subband differential CQI of all even subbands with increasing order of subband number, as in Table 6.3.1.1.2-8B, if present and if reported. CSI report #n CSI part 2, group 2; The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q Pril,i,f for N4 = 1 or Pril,i,f, j for N4 > 1 defined in clause 5.2.3 of TS 38.214 [6], if reported; The second time-domain subband differential CQI of all odd subbands with increasing order of subband number, as in Table 6.3.1.1.2-8B, if present and if reported.

Proposal-6:

- For Type II Doppler, the following TP is adopted in 6.3.2.1.2 of 38.212.

6.3.2.1.2 CSI Table 6.3.2.1.2-5B: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-PortSelection-r17 or typeII-Doppler-PortSelection CSI report number; CSI fields. CSI report #n CSI part 2, group 0; PMI fields X1, from left to right as in Tables 6.3.2.1.2-2B/2D, if reported. CSI report #n CSI part 2, group 1; The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-2B/2D: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*K1M-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported. CSI report #n CSI part 2, group 2; The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-2B/2D: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported.

Proposal-7:

- For Type II Doppler, the following TP is adopted in 6.3.2.1.2 of 38.212.

6.3.2.1.2 CSI Table 6.3.2.1.2-5F: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType= typeII-Doppler CSI report number; CSI fields. CSI report #n CSI part 2, group 0; PMI fields X1, from left to right as in Table 6.3.2.1.2-1C, if reported; The second time-domain wideband CQI as in Table 6.3.12.1.2-8B, if present and reported. CSI report #n CSI part 2, group 1; The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,10,l:l=1,…,υ, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυQ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q defined in clause 5.2.3 of TS38.214, if present and if reported; The second time-domain subband differential CQI of all even subbands with increasing order of subband number, as in Table 6.3.12.1.2-8B, if present and if reported. CSI report #n CSI part 2, group 2; The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q defined in clause 5.2.3 of TS38.214, if reported; The second time-domain subband differential CQI of all odd subbands with increasing order of subband number, as in Table 6.3.12.1.2-8B, if present and if reported.

References

- 3GPP RAN1#109 e-meeting, Chairman’s notes.

- 3GPP RAN1#114bis meeting, Chairman’s notes.