3GPP TSG RAN WG1 #115						               R1-2311501

Chicago, USA, November 13th – November 17th, 2023

Agenda item:	8.16.1

Title:	UE features for NR MIMO evolution

Source:	CMCC

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we provide our views on Rel-18 UE features for NR MIMO evolution.

# 2. UE feature for unified TCI framework extension for multi-TRP

Agreement@RAN1#114 On unified TCI framework extension for S-DCI based MTPR, a UE capability is used as the threshold for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2 if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2 - Note: Whether to reuse the legacy UE capability (timeDurationForQCL) as the threshold and corresponding candidate values are discussed in Rel-18 UE feature AI

In RAN1#114 meeting [1], it has been agreed that a UE capability is used as the threshold for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2 if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2. If the UE doesn’t support the capability of two default beams, UE must determine a default beam to buffer PDSCH before the threshold, the threshold should include the time for both DCI decoding and beam switching. In Rel-15, the TCI state of PDSCH is indicated in the DCI, before DCI decoding, UE does not know the TCI state of PDSCH, so a larger time is needed for preparing the Rx beam for PDSCH reception. In Rel-18, the TCI state for PDSCH reception is one or both TCI states that UE applied for unified TCI, which are already known for UE. After DCI decoding, UE only needs to determine whether one or both TCI states are used for PDSCH reception, so the time for PDSCH Rx beam preparing time is smaller than Rel-15. It has been agreed that before the threshold, the first indicated joint/DL TCI state is used for PDSCH reception. If some advanced UE could support a threshold smaller than legacy UE capability (timeDurationForQCL), it will be beneficial for gNB to choose a suitable TRP for PDSCH transmission when the scheduling gap is small. So, A new UE capability smaller than legacy UE capability (timeDurationForQCL) is introduced as the threshold for PDSCH reception. Similarly, a new UE capability smaller than the legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) can be also introduced as the threshold for AP CSI-RS reception.

Proposal 1: A new UE capability smaller than legacy UE capability (timeDurationForQCL) is introduced as the threshold for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2.

Proposal 2: A new UE capability smaller than legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) is introduced as the threshold for AP CSI-RS reception.

# 3. UE feature for CSI enhancement

For CJT CSI enhancement

In RAN1#114 meeting [1], we have the following agreement on the CPU occupation for Rel-18 CJT codebook:

Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation, - X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability

So, we suggest that the component of value of X for CPU occupation should be introduced for FG 40-3-1-1/40-3-1-5 and the candidate value could be {1, 1.5, 2}:

Proposal 3: For FG 40-3-1-1/40-3-1-1, support to introduce the component of value of X for CPU occupation:

| Features | Index | Feature group | Components | Prerequisite feature groups | Note |
| --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1 | Support of mode 2 for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs The value of X for CPU occupation | [2-35] | Candidate value X for CPU occupation {1, 1.5, 2} Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5 | Support of mode 2 for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs The value of X for CPU occupation | [2-35] | Candidate value X for CPU occupation {1, 1.5, 2} Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG |

# 4. Conclusion

Proposal 1: A new UE capability smaller than legacy UE capability (timeDurationForQCL) is introduced as the threshold for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2.

Proposal 2: A new UE capability smaller than legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) is introduced as the threshold for AP CSI-RS reception.

Proposal 3: For FG 40-3-1-1/40-3-1-1, support to introduce the component of value of X for CPU occupation:

| Features | Index | Feature group | Components | Prerequisite feature groups | Note |
| --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1 | Support of mode 2 for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs The value of X for CPU occupation | [2-35] | Candidate value X for CPU occupation {1, 1.5, 2} Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5 | Support of mode 2 for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs The value of X for CPU occupation | [2-35] | Candidate value X for CPU occupation {1, 1.5, 2} Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG |

# 5. References

- 3GPP RAN1#114, RAN1 Chairman’s Notes, August 2023.