3GPP TSG RAN WG1 #115	R1-2311618

Chicago, USA, November 13th – November 17th, 2023

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on 8TX UL transmission

Agenda Item:	8.1.4.2

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. One from the seven objectives is to study 8 TX UL operation. In this contribution, we discuss the remaining issues on 8TX UL transmission.

- TPMI tables

In TS 38.211, for the new TPMI tables for 8 antenna ports for different codebook types, separate tables are introduced for different layers transmissions. For the tables for larger than one layer transmission, ‘with transform precoding disabled’ is added in the title of each table. For the tables for one layer transmission, there is no description related to transform precoding. We understand the intention is to apply the tables for both cases of with DFT precoding and without DFT precoding. However, to make the spec. clear, we suggest adding ‘with transform precoding enabled or with transform precoding disabled’ in the title of those tables for one layer transmission.

Proposal 1

- For TPMI tables for one layer transmission for eight antenna ports in TS 38.211, suggest adding ‘with transform precoding enabled or with transform precoding disabled’ in the title of each table.

  - Adopt following TP for TS 38.211.

#### 6.3.1.5 Precoding […] Table 6.3.1.5-9: Precoding matrix W for codebook1=ng1n4n1 and single-layer transmission using eight antenna ports with transform precoding enabled or with transform precoding disabled. […] Table 6.3.1.5-17: Precoding matrix W for codebook1=ng1n2n2 and single-layer transmission using eight antenna ports with transform precoding enabled or with transform precoding disabled. […] Table 6.3.1.5-29: Intermediate precoding matrix W' for codebook2 and single-layer transmission using eight antenna ports with transform precoding enabled or with transform precoding disabled. […] Table 6.3.1.5-39: Intermediate precoding matrix W' for codebook3 and single-layer transmission using eight antenna ports with transform precoding enabled or with transform precoding disabled. […]

- TRI/TPMI indication in DCI for full power modes

For full power mode 2, it has been agreed not supported for Ng=4 and Ng=8 cases. Full power mode 2 is supported for Ng=2 case only. Thus, fullpowerMode2 could not be configured for Ng=4 and Ng=8 cases and corresponding update is needed. On the other hand, if the conclusion could be reverted in next meeting, and full power mode 2 could be supported for Ng=4 case, we’re also supportive and the following TP can be further updated. Note that following TP is provided for DCI format 0_1. Similar revision should be applied for DCI format 0_2.

Proposal 2

- Adopt following TP regarding DCI format 0_1 for full power mode 2 for TS 38.212 (if the conclusion on full power mode 2 is not reverted).

#### 7.3.1.1.2 Format 0_1 […] - 8 bits according to Table 7.3.1.1.2-5F for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower. - 10 bits according to Table 7.3.1.1.2-5I for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank; - 10 bits according to Table 7.3.1.1.2-5K for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank; - 6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, or 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1. - 6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3, 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1; - 5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3, or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank; - 8 or 9 bits according to Table 7.3.1.1.2-5Q for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 10 bits according to Table 7.3.1.1.2-5R for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 10 bits according to Table 7.3.1.1.2-5S for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank =5, 6, 7, or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; Table 7.3.1.1.2-5F: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookType=Codebook4, and ul-FullPowerTransmission is not configured or configured to fullpower Table 7.3.1.1.2-5G: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 2, 3 or 4, CodebookType=Codebook4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower Table 7.3.1.1.2-5H: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 if transform is disabled, CodebookType=Codebook4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower Table 7.3.1.1.2-5K: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookType=Codebook3, and ul-FullPowerTransmission is not configured or configured to fullpower Table 7.3.1.1.2-5L: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookType=Codebook3, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

- CG PUSCH

For CG PUSCH, a conclusion has been made that there is no consensus to support CG transmission with dual CW PUSCH by an 8TX UE. However, it is not clear for us if no specification impact is necessary even with the conclusion. For example, for following spec. text in TS 38.213, if DCI format 0_1/0_2 is used for Type 2 CG activation, and two sets of NDI/RV/MCS fields exist in the DCI due to configuration of up to 8-layer, it is better to revise the description of special fields for RV as ‘for the enabled transport block: set to all ‘0’s’, similar as that for DCI format 1_1.

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- |
| HARQ process number (if present) | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version (if present) | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- |
| Redundancy version (if present) | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Thus, following TP is proposed.

Proposal 3

- For CG PUSCH, support following TP for TS 38.213.

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- | --- |
| HARQ process number (if present) | set to all '0's | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version (if present) | set to all '0's | For the enabled transport block: set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- | --- |
| Redundancy version (if present) | set to all '0's | For the enabled transport block: set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Based on current specification, the parameter of max rank for CG PUSCH is provided by pusch-Config, not configuredGrantConfig. However, if the max rank is configured to be larger than 4, how to understand the max rank for CG PUSCH is not clarified in specification yet. Thus, we suggest following proposal.

Proposal 4

- For CG PUSCH, support following TP for TS 38.214.

#### 6.1 UE procedure for transmitting the physical uplink shared channel […] For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. The maximum number of layers for configured grant PUSCH is given by the minimum of 4 and maxRank. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubsetDCI-0-2, maxRankDCI-0-2, scaling of UCI-OnPUSCH, resourceAllocationType1GranularityDCI-0-2 provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in clause 6.1.4 for the PUSCH transmission corresponding to a configured grant.

- DCI fields for transport block 2

In current TS 38.212, the presence of DCI fields for transport block 2 is described as ‘only present if maxRank > 4 or maxMIMO-Layers > 4’, where maxRank is per BWP and applicable to CB-based PUSCH transmission, and maxMIMO-Layers is per cell and applicable to NCB-based PUSCH transmission.

#### 7.3.1.1.2 Format 0_1 […] For transport block 2 (only present if maxRank > 4 or maxMIMO-Layers > 4): - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 […] - SRS resource indicator – number of bits determined by the following: - bits according to Tables 7.3.1.1.2-28/28A/29/29B/30/30B/31/31B/31C/31D/31E/31F if the higher layer parameter txConfig = nonCodebook, where

However, for NCB-based PUSCH transmission, whether transport block 2 is present or not is also related to the configuration of NSRS (the number of configured SRS resources in the SRS resource set for NCB-based PUSCH) in addition to maxMIMO-Layers. In current spec., there is no configuration restriction between maxMIMO-Layers and NSRS. From the description for SRS resource indicator, it can be seen that if configured values of NSRS and Lmax (maxMIMO-Layers) are not aligned, minimal value between them is used to determine the bit size of SRS resource indicator. However, the presence of MCS/NDI/RV for TB2 is determined by maxMIMO-Layers only. Thus, it is possible that gNB configures maxMIMO-Layers > 4 with  NSRS <= 4. In this case, the presence of MCS/NDI/RV for transport block 2 is a waste of DCI resources. To resolve this issue, there are two possible solutions.

- Option 1: Add configuration restriction between maxMIMO-Layers and NSRS, e.g., UE is not expected to be configured with maxMIMO-Layers > 4 and NSRS <= 4 at the same time.

- Option 2: Revise the condition for presence of transport block 2 as ‘only present if maxRank > 4 or maxMIMO-Layers > 4 with NSRS > 4’ without introducing configuration restriction between maxMIMO-Layers and NSRS.

Either Option can work in our views. Option 1 is slightly preferred as such configuration is not reasonable and can be avoided.

Proposal 5

- Add configuration restriction between maxMIMO-Layers and NSRS to avoid unnecessary DCI overhead.

  - Adopt following TP for TS 38.212.

#### 7.3.1.1.2 Format 0_1 […] - SRS resource indicator – number of bits determined by the following: - bits according to Tables 7.3.1.1.2-28/28A/29/29B/30/30B/31/31B/31C/31D/31E/31F if the higher layer parameter txConfig = nonCodebook, where - is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present, - NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI, - otherwise NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList and associated with the higher layer parameter usage of value 'nonCodeBook', and - if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, - Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSdm} if maxMIMO-LayersforSdm is configured - Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSfn} if maxMIMO-LayersforSfn is configured - Lmax is given by maxMIMO-Layers otherwise - otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation. - UE is not expected to be configured with maxMIMO-Layers > 4 and NSRS <= 4 at the same time.

- Conclusion

In this contribution, we discussed the remaining issues on 8TX UL transmission. We have following proposals and TPs.

Proposal 1

- For TPMI tables for one layer transmission for eight antenna ports in TS 38.211, suggest adding ‘with transform precoding enabled or with transform precoding disabled’ in the title of each table.

  - Adopt following TP for TS 38.211.

#### 6.3.1.5 Precoding […] Table 6.3.1.5-9: Precoding matrix W for codebook1=ng1n4n1 and single-layer transmission using eight antenna ports with transform precoding enabled or with transform precoding disabled. […] Table 6.3.1.5-17: Precoding matrix W for codebook1=ng1n2n2 and single-layer transmission using eight antenna ports with transform precoding enabled or with transform precoding disabled. […] Table 6.3.1.5-29: Intermediate precoding matrix W' for codebook2 and single-layer transmission using eight antenna ports with transform precoding enabled or with transform precoding disabled. […] Table 6.3.1.5-39: Intermediate precoding matrix W' for codebook3 and single-layer transmission using eight antenna ports with transform precoding enabled or with transform precoding disabled. […]

Proposal 2

- Adopt following TP regarding DCI format 0_1 for full power mode 2 for TS 38.212 (if the conclusion on full power mode 2 is not reverted).

#### 7.3.1.1.2 Format 0_1 […] - 8 bits according to Table 7.3.1.1.2-5F for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower. - 10 bits according to Table 7.3.1.1.2-5I for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank; - 10 bits according to Table 7.3.1.1.2-5K for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank; - 6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, or 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1. - 6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3, 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1; - 5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3, or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank; - 8 or 9 bits according to Table 7.3.1.1.2-5Q for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 10 bits according to Table 7.3.1.1.2-5R for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 10 bits according to Table 7.3.1.1.2-5S for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank =5, 6, 7, or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; Table 7.3.1.1.2-5F: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookType=Codebook4, and ul-FullPowerTransmission is not configured or configured to fullpower Table 7.3.1.1.2-5G: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 2, 3 or 4, CodebookType=Codebook4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower Table 7.3.1.1.2-5H: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 if transform is disabled, CodebookType=Codebook4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower Table 7.3.1.1.2-5K: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookType=Codebook3, and ul-FullPowerTransmission is not configured or configured to fullpower Table 7.3.1.1.2-5L: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookType=Codebook3, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

Proposal 3

- For CG PUSCH, support following TP for TS 38.213.

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- | --- |
| HARQ process number (if present) | set to all '0's | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version (if present) | set to all '0's | For the enabled transport block: set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 0_1/0_2 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- | --- |
| Redundancy version (if present) | set to all '0's | For the enabled transport block: set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Proposal 4

- For CG PUSCH, support following TP for TS 38.214.

#### 6.1 UE procedure for transmitting the physical uplink shared channel […] For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. The maximum number of layers for configured grant PUSCH is given by the minimum of 4 and maxRank. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubsetDCI-0-2, maxRankDCI-0-2, scaling of UCI-OnPUSCH, resourceAllocationType1GranularityDCI-0-2 provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in clause 6.1.4 for the PUSCH transmission corresponding to a configured grant.

Proposal 5

- Add configuration restriction between maxMIMO-Layers and NSRS to avoid unnecessary DCI overhead.

  - Adopt following TP for TS 38.212.

#### 7.3.1.1.2 Format 0_1 […] - SRS resource indicator – number of bits determined by the following: - bits according to Tables 7.3.1.1.2-28/28A/29/29B/30/30B/31/31B/31C/31D/31E/31F if the higher layer parameter txConfig = nonCodebook, where - is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present, - NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI, - otherwise NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList and associated with the higher layer parameter usage of value 'nonCodeBook', and - if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, - Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSdm} if maxMIMO-LayersforSdm is configured - Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSfn} if maxMIMO-LayersforSfn is configured - Lmax is given by maxMIMO-Layers otherwise - otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation. - UE is not expected to be configured with maxMIMO-Layers > 4 and NSRS <= 4 at the same time.

References

- 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.