3GPP TSG RAN WG1 #115			R1-2311640

Chicago, USA, November 13th – November 17th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on AI/ML for CSI feedback enhancement

Agenda Item:	8.14.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of potential specification impacts as following.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements.

- Discussion on CSI compression

At the RAN1#109-e meeting, spatial-frequency domain CSI compression using two-sided AI models, as known as auto-encoder of CSI feedback was agreed to be a representative sub use case as following [2].

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

- Note: Study of other sub use cases is not precluded.

- Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Fig. 1 illustrates the framework of CSI compression with two-sided models. As shown in Fig. 1, UE is equipped with an AI/ML encoder to compress CSI into encoded bits, while the corresponding AI/ML decoder is deployed on gNB to reconstruct CSI from encoded bits. In CSI compression with two-sided models, UE calculates downlink CSI, such as channel matrix or precoding matrix, and feeds the CSI into the encoder for compression. After the AI/ML encoder extracts essential features and outputs the encoded bits, UE reports the encoded bits to gNB where CSI can be reconstructed from encoded bits with the AI/ML decoder. In this contribution, the inputs of encoder and outputs of decoder are assumed to be the same.

Figure 1. The framework of spatial-frequency domain CSI compression with two-sided models.

With this AI/ML-based compression technique, accuracy improvements under a certain overhead of CSI reports and overhead reduction for CSI reports achieving a certain performance are observed [3].

  - Model training for CSI compression

At the RAN1#112 meeting, the aspects to study for each training procedure were agreed as follows [4].

Conclusion

In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects:

- Whether model can be kept proprietary

- Requirements on privacy-sensitive dataset sharing

- Flexibility to support cell/site/scenario/configuration specific model

- gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware

- Model update flexibility after deployment

- feasibility of allowing UE side and NW side to develop/update models separately

- Model performance based on evaluation in 9.2.2.1

- Whether gNB can maintain/store a single/unified model

- Whether UE device can maintain/store a single/unified model

- Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use

- Whether training data distribution can be matched to the device that will use the model for inference

- Whether device capability can be considered for model development

- Other aspects are not precluded

- Note: training data collection and dataset/model delivery will be discussed separately

Also, it was agreed at the RAN1#113 meeting to analyze the aspects separately for known model structure and unknown model structure in type 1 training, and to analyze the aspects separately for simultaneous training and sequential training in type 2 training [5].

Agreement

In CSI compression using two-sided model use case, for discussion of training collaboration type 1,

- Create separate table with separate columns for both known model structure, and unknown model structure separately for NW-sided and UE-sided, respectively.

Agreement

  - Type 2 Joint training of the two-sided model at network side and UE side, respectively.

    - Note: Joint training includes both simultaneous training and sequential training, in which the pros and cons could be discussed separately

    - Note: Sequential training includes starting with UE side training, or starting with NW side training

In Table 1 and Table 2, characteristics of type 1, type 2, and type 3 training procedures are analysed in terms of the aspects that was agreed to study. In the tables, the differences from the latest one in the FL summary [6] are highlighted with yellow color, while green highlighted parts were already agreed at the RAN1#114bis [6].

In type 1 training procedure, a model is delivered from the training entity to the other side. After model delivery, the model at training entity side can be updated assuming that the other side still uses the delivered model. Hence, if the NW side or UE side is the training entity, a new NW side or UE side model can be developed/updated separately from the other side, respectively. In the same manner, it is possible to train a new NW side or UE side model compatible with the other side model in use, as long as the other side model in use is the delivered model.

Feasibility of allowing UE side and NW side to develop/update models separately for type 2 sequential training is controversial. Type 2 sequence training requires dataset sharing between UE side and NW side as well as type 3 training. In addition, type 2 sequence training requires gradient exchange between UE and NW to update UE side part model. Hence, type 2 sequential training can be said to be feasible for allowing UE side to develop/update model separately with gradient exchange.

In type 3 training procedure, it was proposed to add the note mentioning a combined solution of NW first type 3 training and UE first type 3 training in extendibility aspects. However, a combined solution of training types has not considered in other aspects. Considering the consistency with other characteristics, it is better not to consider a combined solution in these tables.

Proposal 1: Conclude characteristics of type 1, type 2, and type 3 training procedure based on Table 1 and Table 2.

Table 1. Characteristics of type 1 training procedure.

| Training types Characteristics | Type1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at NW | Known model structure at NW |
| Whether model can be kept proprietary | No | No | No | No |
| Whether require privacy-sensitive dataset sharing | No (note 3) | No (note 3) | No (note 3) | No (note 3) |
| Flexibility to support cell/site/scenario/configuration specific model | Flexible except for UE defined scenarios. Not flexible for UE defined scenarios unless UE assistance information is supported and available. | Flexible except for UE defined scenarios. Not Flexible for UE defined scenarios unless UE assistance information is supported and available. | Flexible except for NW defined scenarios. Not flexible for NW defined scenarios unless NW assistance information is supported and available. | Flexible except for NW defined scenarios. Not flexible for NW defined scenarios unless NW assistance information is supported and available. |
| Whether gNB/device specific optimization is allowed | gNB: Yes UE: No | gNB: Yes UE: less flexible compared to UE side | gNB: No UE: Yes | UE: Yes gNB: less flexible compared to NW side |
| Model update flexibility after deployment | Flexible only if UE supports the new structure | Flexible for parameter update | Flexible less flexible than Type 1 NW side | Flexible for parameter update. less flexible than Type 1 NW side |
| Feasibility of allowing UE side and NW side to develop/update models separately | Feasible only for NW side model | Feasible only for NW side model | Feasible only for UE side model | Feasible only for UE side model |
| Whether gNB can maintain/store a single/unified CSI reconstruction model over different UE vendors (note x3) | Yes | Yes Performance refers to observations in “1 NW part model to M>1 UE part models” of Section 6.2.2.4, TR38.843 (note x5) | No | No |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (note x4) | No | No | Yes | Yes Performance refers to observations in “1 UE part model to N>1 NW part models” of Section 6.2.2.4, TR38.843 (note x5) |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Support | Support | Support, if NW-side model in use is delivered by UE | Support, if NW-side model in use is delivered by UE |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Support, if UE-side model in use is delivered by NW | Support, if UE-side model in use is delivered by NW | Support | Support |
| Whether training data distribution can match the inference device | Limited | Limited | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No for UE | Yes | No for NW | Yes |
| Model performance based on evaluation in 9.2.2.1 | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations |

Table 2. Characteristics of type 2 training procedure and type 3 training procedure.

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first (note 1) | NW first | UE first |
| Whether model can be kept proprietary | Yes (note 2) | Yes (note 2) | Yes (note 2) | Yes (note 2) |
| Whether require privacy-sensitive dataset sharing | No (Note 3) | No (Note3) | No (Note 3) | No (Note 3) |
| Flexibility to support cell/site/scenario/configuration specific model | No consensus | No consensus | [Semi] flexible except for UE defined scenarios. (note x1) [Semi] flexible for UE defined scenarios if UE assistance information is supported and available. | [Semi] flexible except for NW defined scenarios (note x1). [Semi] flexible for NW defined scenarios if NW assistance information is supported and available. |
| Whether gNB/device specific optimization is allowed | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment (note 4) | Not flexible | No consensus. | Semi-flexible | Semi-flexible. |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Feasible with gradient exchange | Feasible. | Feasible |
| Whether gNB can maintain/store a single/unified CSI reconstruction model over different UE vendors (note x3) | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” and “1 UE part model to N>1 NW part models” of Section 6.2.2.4, TR38.843 | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of Section 6.2.2.4, TR38.843 | Yes. Performance refers to observations in “NW first training, 1 NW part model to 1 UE part model, same backbone”, and “NW first training, 1 NW part model to 1 UE part model, different backbones” of Section 6.2.2.5, TR38.843 | Yes. Performance refers to observations in “UE first training, M>1 UE part models to 1 NW part model” of Section 6.2.2.5, TR38.843 |
| Whether UE device can maintain/store a single/unified CSI generation model over different NW vendors (note x4) | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” and “1 UE part model to N>1 NW part models” of Section 6.2.2.4, TR38.843 | Yes. Performance refers to observations in “1 NW part model to M>1 UE part models” of Section 6.2.2.4, TR38.843 | Performance refers to observations in “NW first training, 1 UE part model to N>1 NW part models” of Section 6.2.2.5, TR38.843 | Yes. Performance refers to observations in “UE first training, 1 NW part model to 1 UE part model, same backbone”, and “UE first training, 1 NW part model to 1 UE part model, different backbones” of Section 6.2.2.5, TR38.843 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | Not support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not Support | Not support | Support |
| Whether training data distribution can match the inference device | No consensus | Yes for UE-part model, Limited for NW-part model. | Limited | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Compatible | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations |

  - Pairing mechanism for CSI compression

At the RAN1#114 meeting, the following observation, proposal, and agreement regarding the paring mechanism and model ID were made [7] [8].

Observation

In CSI compression using two-sided model use case, at least the following options have been proposed by companies to define the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB:

- Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use.

- Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use.

- Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID.

- Option 4: The pairing information is in the forms of by the dataset ID during type 3 sequential training.

- Option 5: The pairing information is in the forms of a training session ID to a prior training session (e.g., API) between NW and UE.

- Option 6: The pairing information is up to UE/NW offline co-engineering alignment, transparent to 3GPP specification.

- Note: the disclosure of the vendor information during the model pairing procedure and model identification procedure should be considered.

- Note: If each UE side model is compatible with all NW side model, the information is not needed for the UE.

- Note: Above does not imply there is a need for a central entity for defining/storing/maintaining the IDs.

Proposal:

In CSI compression using two-sided model use case, in order to enable the UE to select a CSI generation model compatible with the CSI reconstruction model used by the gNB.

- UE report the supported the pairing information in UE capability report as a starting point.

- When multiple pairing information are supported through UE capability report, additional NW and UE interaction is needed.

Agreement

Once models are identified via Type A, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point.

- FFS: Using a procedure other than UE capability report

- Note: The support and applicability of model identification Type A is a separate discussion.

In the observation, there are six options as paring information. In our view, paring information in almost all options are in form of model ID. Apparently, Option 1-3 assumes the pairing info in form of ID representing model. In Option 4 and 5, the paring information is in form of dataset or training session. As the model can be defined based on what dataset is used for training and/or how the model is trained, the paring information in Option 4 and 5 can be viewed as model ID in a sense.

Also, it was agreed to report the supported model ID in UE capability as a starting point. This agreed procedure is very similar to the proposed pairing mechanism in the above proposal, except for that paring information is reported instead of model ID. For those reasons, we think model ID can be used as paring information and the model ID reporting in UE capability can be starting point even for pairing of two-sided model. In that case, there is no need to introduce additional ID for pairing.

Proposal 2: Model ID can be used as pairing information for two-sided model.

To use model ID as paring information, model identification is necessary. If a model identification procedure is applicable to training procedures for two-sided model, this model identification procedure can be said to be applicable with two-sided model use cases. As discussed in our companion contribution, the following potential model identification procedures are considered [8]

- Model identification type A

Step 1: UE side or/and NW side (re-)trains a new model with offline coordination between UE and NW

Step 2: UE deploys a new model

Step 3: UE reports the supported/applicable ID

- Model identification type B1

Step 1: UE side (re-)trains and deploys a new model

Step 2: UE reports the existence of the new model with additional condition over the air

Step 3: NW indicates the model ID assigned to the reported model

- Model identification type B2-1 (model transfer-based)

Step 1: UE reports the supportable model information

Step 2: NW side (re-)trains a new model

Step 3: NW transfers the new model to UE with model ID

- Model identification type B2-2 (time duration/area ID-based)

Step 1: NW indicates time duration/area ID associated with specific NW additional condition

Step 2: UE side (re-)trains a new model

Step 3: UE reports the supported/applicable ID

- Model identification type B2-3 (dataset transfer-based)

Step 1: NW side transfers the dataset to UE side and the associated ID

Step 2: UE side (re-)trains the model based on the transferred dataset

Step 3: UE reports the supported/applicable ID

Among the above model identification procedures, we think at least the following model identification procedures are applicable to two-sided model use cases.

- Model identification type A

Model training can be performed with offline coordination between NW and UE. Type 1, type 2, and type 3 training procedure can be performed with this offline coordination.

- Model identification type B2-1

Model is trained by NW and transferred to UE. If NW trains both UE side part model and NW side part model and transfer the UE side part model to UE, the paired model can be deployed at UE and NW. This procedure is aligned with type 1 training procedure.

- Model identification type B2-3

UE side trains a model based the transferred dataset from NW. If NW transfers the dataset generated by NW side part model, UE side can train the UE side part model applicable to the corresponding NW side part model. This procedure is aligned with NW first type 3 training procedure.

Observation 1: The following model identification procedures are applicable with two-sided model use cases.

- Model identification type A

It is applicable to type 1, type 2, and type 3 training procedure

- Model identification type B2-1 (model transfer-based model identification)

It is applicable to type 1 training procedure

- Model identification type B2-3 (dataset transfer-based model identification)

It is applicable to NW first type 3 training procedure

- Discussion on CSI prediction

At the RAN#100 meeting, it was agreed that RAN WGs study specification impacts of CSI prediction regarding data collection procedures and monitoring procedures as follows [9].

Agreement

- RAN tasks RAN WGs to study a subset of the specification impacts of CSI prediction limited to the following aspects:

  - data collection procedures reusing as much as possible what is defined for UE side use cases

  - monitoring procedure and associated fallback mechanism to legacy CSI reporting

- The RAN WGs spec impact work on this use case shall not affect progress on the on-going work for other use cases.

  - Performance monitoring

Agreement

For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM:

- Type 1:

  - UE calculate the performance metric(s)

  - UE reports performance monitoring output that facilitates functionality fallback decision at the network

    - Performance monitoring output details can be further defined

    - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed).

  - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

- Type 2:

  - UE reports predicted CSI and/or the corresponding ground truth

  - NW calculates the performance metrics.

  - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

- Type 3:

  - UE calculate the performance metric(s)

  - UE report performance metric(s) to the NW

  - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

- Functionality selection/activation/ deactivation/switching what is defined for other UE side use cases can be reused, if applicable.

- Configuration and procedure for performance monitoring

  - CSI-RS configuration for performance monitoring

- Performance metric including at least intermediate KPI (e.g., NMSE or SGCS)

- UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report.

- Note: down selection is not precluded.

Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

At the RAN1#114 meeting, the performance monitoring of CSI prediction was categorized into three types only for functionality-based LCM as the above agreement [6]. Given that the functionality-based LCM and model ID-based LCM are usually discussed together without any prioritization, the same discussion should be triggered in model ID-based LCM. In our view, the similar categorization can be applied even to model ID-based LCM in CSI prediction. Specifically, the following categorization can be considered for model ID-based LCM in CSI prediction.

・Type 1

- UE calculate the performance metric(s)

- UE reports performance monitoring output that facilitates model ID decision at the network

  - Performance monitoring output details can be further defined

  - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed).

- NW makes decision(s) of model selection/activation/ deactivation/switching.

・Type 2

- UE reports predicted CSI and/or the corresponding ground truth

- NW calculates the performance metrics.

- NW makes decision(s) of model selection/activation/ deactivation/switching.

・Type 3

- UE calculate the performance metric(s)

- UE report performance metric(s) to the NW

- NW makes decision(s) of model selection/activation/ deactivation/switching.

Proposal 2: Model ID can be used as pairing information for two-sided model.

Proposal 3: For model ID-based LCM in CSI prediction using UE side model, the following monitoring procedure can be considered.

・Type 1

- UE calculate the performance metric(s)

- UE reports performance monitoring output that facilitates model ID decision at the network

  - Performance monitoring output details can be further defined

  - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed).

- NW makes decision(s) of model selection/activation/ deactivation/switching.

・Type 2

- UE reports predicted CSI and/or the corresponding ground truth

- NW calculates the performance metrics.

- NW makes decision(s) of model selection/activation/ deactivation/switching.

・Type 3

- UE calculate the performance metric(s)

- UE report performance metric(s) to the NW

- NW makes decision(s) of model selection/activation/ deactivation/switching.

As can be seen in Fig.2, one model/functionality of CSI prediction can predict CSI associated with multiple prediction time offsets (time gaps between the measurement and predicted CSI). As the prediction time offset gets large, the prediction difficulty gets increased. Then, it is preferrable to calculate the performance metric per prediction time offset for the performance analysis. In this manner, NW can discern reliability of the predicted value corresponding to each prediction time offsets for NW operation.

Proposal 4: Consider performance metric calculation per prediction time offset for monitoring CSI prediction.

Figure 2. Performance monitoring of model/functionality predicting CSI associated with multiple time instances.

- Discussion on recommendation for Rel-19 AI/ML air interface WI

In Rel-18 SI, performance gains compared to baseline approaches are observed for both CSI compression and CSI prediction. Also, we think intensive studies were conducted for the specification impacts in both sub use cases, although RAN4 aspects may not be sufficiently studied for two-sided model use cases. For those reasons, we recommend normative work on both CSI compression and CSI prediction in Rel-19. Regarding CSI compression, it can also be considered to further study CSI compression especially in RAN4 aspects if there are strong views that it is still premature for normative work.

Proposal 5: Recommend normative work for both CSI compression and CSI prediction in Rel-19. If there are still concerns about the normative work on CSI compression, further study of CSI compression especially on RAN4 aspects can be considered in Rel-19 instead.

- Conclusion

In this contribution, we discussed the sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements. Based on the discussion we made the following observations and proposals.

Observation 1: The following model identification procedures are applicable with two-sided model use cases.

- Model identification type A

It is applicable to type 1, type 2, and type 3 training procedure

- Model identification type B2-1 (model transfer-based model identification)

It is applicable to type 1 training procedure

- Model identification type B2-3 (dataset transfer-based model identification)

It is applicable to NW first type 3 training procedure

Proposal 1: Conclude characteristics of type 1, type 2, and type 3 training procedure based on Table 1 and Table 2.

Proposal 2: Model ID can be used as pairing information for two-sided model.

Proposal 3: For model ID-based LCM in CSI prediction using UE side model, the following monitoring procedure can be considered.

・Type 1

- UE calculate the performance metric(s)

- UE reports performance monitoring output that facilitates model ID decision at the network

  - Performance monitoring output details can be further defined

  - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed).

- NW makes decision(s) of model selection/activation/ deactivation/switching.

・Type 2

- UE reports predicted CSI and/or the corresponding ground truth

- NW calculates the performance metrics.

- NW makes decision(s) of model selection/activation/ deactivation/switching.

・Type 3

- UE calculate the performance metric(s)

- UE report performance metric(s) to the NW

- NW makes decision(s) of model selection/activation/ deactivation/switching.

Proposal 4: Consider performance metric calculation per prediction time offset for monitoring CSI prediction.

Proposal 5: Recommend normative work for both CSI compression and CSI prediction in Rel-19. If there are still concerns about the normative work on CSI compression, further study of CSI compression especially on RAN4 aspects can be considered in Rel-19 instead.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Moderator (Apple), R1-2205556, “Email discussion on other aspects of AI/ML for CSI enhancement”, May. 2022.

- NTT DOCOMO, INC., R1-2305591, “Discussion on evaluation on AI/ML for CSI feedback enhancements”, May. 2023.

- Moderator (Apple), R1-2301912, “Email discussion on other aspects of AI/ML for CSI enhancement”, Feb. 2023.

- Moderator (Apple), R1-2306047, “Final summary on other aspects of AI/ML for CSI enhancement”, May. 2023.

- Moderator (Apple), R1-2310317, “Final summary on other aspects of AI/ML for CSI enhancement”, Oct. 2023.

- Moderator (Apple), R1-2308247, “Summary #4 on other aspects of AI/ML for CSI enhancement”, Aug. 2023.

- Moderator (Qualcomm), R1- 2308292, “Final Summary of General Aspects of AIML Framework”, Aug. 2023

- NTT DOCOMO, INC., R1-2311639, “Discussion on general aspects of AI/ML framework”, Nov. 2023.

- Moderator (Qualcomm), RP-231481, “AI/ML for Air Interface Moderator Summary”, June. 2023.