3GPP TSG RAN WG1 #115			R1-2301684

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.3.1.3

Source:	Apple Inc.

Title:	Remaining Issues On Resource allocation for SL positioning reference signal

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss resource allocation and grouping for the SL positioning reference signal.

# Discussion

In summary, the overall design is stable with just a few low priority outstanding issues based on discussions in [1].

## Inter UE Coordination

In the work item, the following agreements have been made:

Agreement For the shared resource pool, reuse the existing IUC signaling of both Scheme 1 and Scheme 2. - SL-PRS transmissions are treated as any other legacy transmission for SL communication when considering IUC information exchanges. Conclusion For Rel-18 sidelink positioning: - For the dedicated resource pool, IUC signalling is not supported Do not support that a UE can reserve a SL-PRS resource for the transmission of another UE

##### In 38.331, we have the following:

sl-TriggerConditionRequest

##### Indicates the trigger condition of an explicit request from UE-B to UE-A. Value 0 means the explicit request is triggered by UE-B's implementation. Value 1 means the explicit request can be triggered only when UE-B has data to be transmitted to UE-A.

Proposal 1: For support of IUC in shared SL PRS resource pool update 38.331 to the following,

- value 1 of parameter sl-TriggerConditionRequest means the explicit request can be triggered only when UE-B has data or SL PRS to be transmitted to UE-A.

### 1-symbol PSCCH

In RAN1 #113, the following agreement was made:

Agreement In a dedicated resource pool, with regards to the PSCCH, reuse the PSCCH channel structure of SL communications, at least with regards to the following aspects: - The first PSCCH symbol is mapped to the 2nd symbol available for SL transmissions in a slot - Note: 1st symbol available for SL transmissions in a slot is for PSCCH AGC similar to legacy - PSCCH DM-RS in the slot is being reused from legacy - The number of PSCCH symbol(s) is (pre-)configured to (down-select at RAN1#114): - Alt. 1: 2 or 3 symbols (same as legacy) - Alt. 3: 1, or 2 or 3 symbols - The number of PRBs is (pre-)configured using the legacy values - FFS: reconsider if 1-symbol PSCCH is supported

The last FFS has not be concluded and we propose that we conclude 1-symbol PSSCH is not supported.

Conclusion: 1-symbol PSSCH is not supported for Rel-18

### Congestion Restriction

In the WI, the following agreements/conclusions have been made:

Agreement In Scheme 2, - For a dedicated resource pool for positioning, - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority: - Maximum SL PRS transmission power - Maximum Number of SL PRS (re-)transmissions - Discuss further the following four SL PRS transmission parameters: - Minimum Periodicity of SL PRS - Maximum Number of SL PRS resources in a slot - Maximum comb-size of a SL PRS resource in a slot - Maximum Number of OFDM symbols of a SL PRS resource in a slot - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool - Note: Similar to SL communication how to achieve the CR limit is left to UE implementation. - For a shared resource pool for positioning, the SL PRS can share the same restriction of PSSCH without specific enhancement in addition to what is already specified. Conclusion For a dedicated resource pool, no more discussion on potential restriction by SL PRS-CBR and priority for the following SL PRS transmission parameters: - Maximum Number of SL PRS resources in a slot - Maximum comb-size of a SL PRS resource in a slot - Maximum Number of OFDM symbols of a SL PRS resource in a slot

This leaves the parameter “Minimum Periodicity of SL PRS” undecided. The agreement should be updated as follows:

Proposal 2:

In Scheme 2,

- For a dedicated resource pool for positioning,

  - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority:

    - Maximum SL PRS transmission power

    - Maximum Number of SL PRS (re-)transmissions

    - Minimum Periodicity of SL PRS

  - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool

# Conclusion

In this contribution, the following observations and proposals were made:

Proposal 1: For support of IUC in shared SL PRS resource pool update 38.331 to the following,

- value 1 of parameter sl-TriggerConditionRequest means the explicit request can be triggered only when UE-B has data or SL PRS to be transmitted to UE-A.

Conclusion: 1-symbol PSSCH is not supported for Rel-18

Proposal 2:

In Scheme 2,

- For a dedicated resource pool for positioning,

  - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority:

    - Maximum SL PRS transmission power

    - Maximum Number of SL PRS (re-)transmissions

    - Minimum Periodicity of SL PRS

  - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool

# Reference

- R1-2310601 Moderator Summary #3 on resource allocation for SL PRS	Moderator (Qualcomm), October 2023