3GPP TSG RAN WG1 #115			R1-2311703

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.13.4

Source:	Apple

Title:	Remaining issues on improved GNSS operations for IoT NTN

Document for:	Discussion/Decision

# Introduction

The Release-18 IoT NTN enhancement work item was approved in RAN#94e meeting [1] and updated in RAN#96e meeting [2].

One of the RAN1 objectives is to study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption.

The following agreements were reached in the RAN1#114 bis meeting [3].

Agreement When timeAlignmentTimer is infinity, the duration X is equal to Y. Network can configure Y via a 3-bit field at least with component values [sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240]. FFS: whether there is a new value Agreement The feature of “UL transmission after original validity duration expires with duration X” can be enabled/disabled by network via RRC signalling. Agreement For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, the start time of the gap should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled and X2 is a predefined value, down select - Alt- A: X2 = 1ms - Alt- B: X2 = 2ms - Alt- C: X2 = 3ms - Alt- E: X2 = 1ms for NB-IoT, X2 = 4ms for eMTC Agreement New texts for GNSS measurement gaps for NB-IoT and eMTC in TS36.213 should be added to capture the determination of the start time of GNSS measurement gap triggered by MAC CE. Agreement For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, the start time of the gap should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled and X2 is predefined value, where X2 = 2ms. Agreement Endorsed the TP below for TS36.213, and leave it to the spec editor whether to create new clauses for this TP and if so to decide the title of the new clauses.

In this contribution, we provide our views on the remaining issues of improved GNSS operations.

# Discussion

For the extended UL transmissions, it is enabled by closed loop time correction from the network, which can guarantee that the frequency error and timing error are within requirements. Regarding whether GNSS validity duration can be extended by duration X, from our understanding, it is not crucial to explicitly agree whether the GNSS validity duration is extended from UE point of view as a GNSS validity duration timer would not run at UE. Our view is it is sufficient enough to say the GNSS is valid during X.

Another point that GNSS validity duration should not be manipulated is during handover the original validity duration which comes from RRC layer should be used and the extension duration X should be cancelled because UE moves to a different eNB and the MAC layer is re-set. If the validity duration is constantly changing, it would be very complex for UE to re-derive its original validity duration by deducting the extension X upon handover.

Agreement From RAN1 perspective, down select one for the duration X: - Alt-3: when timeAlignmentTimer is not infinity, X is equal to remaining timeAlignmentTimer; when timeAlignmentTimer is infinity, X is equal to Y; - FFS: whether X can be used to extend the original GNSS validity duration - Y is a configured value. Note 1: The feature can be enabled/disabled by network Note 2 (as already agreed): The duration X is where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition.

Therefore, UE considers its GNSS is valid when any condition below is met, instead of extending the GNSS validity duration:

- Condition 1: GNSS validity duration

- Condition 2: Extension duration Y

- Condition 3: During TAT according to network indication

Proposal 1: During duration X where UL transmission is allowed, UE considers the UL synchronization is valid and there is no need to extend the GNSS validity duration.

Another open issue is how to calculate the timing advance during the duration X. According to current specification, the UL transmission timing is determined according to the below formula.

TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs Except for the cases mentioned in Table 8.1-1, Table 8.1-2 and Table 8.1-3, the range of is: . For frame structure type 1 and for frame structure type 2 unless stated otherwise in [4]. Note that not all slots in a radio frame may be transmitted. One example hereof is TDD, where only a subset of the slots in a radio frame is transmitted. is defined in different ranges depending on the UE configuration according to Table 8.1-1, Table 8.1-2 and Table 8.1-3. In case of subslot based transmission (Table 8.1-2 and Table 8.1-3), the UE is configured by higher layer signalling a processing timeline and an associated range of timing advance. The quantity NTA,adjcommon is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured (see Clause 4.2.3 in TS 36.213 [4]), otherwise NTA,adjcommon=0. The quantity NTA,adjUE is computed by the UE based on UE position and serving satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

In the time duration X, the UE GNSS position fix may not be valid, but the UL synchronization can still be kept via the time error correction. However, to calculate the timing advance, one component is NTA, adjUE, which is determined by UE position and satellite ephemeris info.

If NTA, adjUE is not adjusted in the timing advance calculation, it means only depending on NTA to accumulate the TA command, it’s not enough to compensate for the timing error due to fast movement from the satellite. Otherwise, NTA, adjUE should be adjusted in the timing advance determination, then the latest applied GNSS position fix should be used to calculate NTA, adjUE with the reasonable implementation. From this perspective, the current specification is enough, no special handling is needed to determine the timing advance within duration X.

Proposal 2: No special handling is needed to determine the timing advance within duration X.

# Conclusion

In this contribution, we provided our views on improved GNSS operations for IoT NTN. Our proposals are as follows:

Proposal 1: During duration X where UL transmission is allowed, UE considers the UL synchronization is valid and there is no need to extend the GNSS validity duration.

Proposal 2: No special handling is needed to determine the timing advance within duration X.

# References

- RP-213596, “New WID on IoT NTN enhancements,” Dec. 2021

- RP-221806, “Revised WID on IoT NTN enhancements,” Jun. 2022

- R1-2310781, “Report of RAN1#114bis meeting”, ETSI MCC