3GPP TSG RAN WG1 Meeting #115			    			             R1-2311751

Chicago, USA, November 13th – November 17th, 2023

# Source:	ETRI

# Title:		Remaining issues on channel access mechanism for sidelink on unlicensed spectrum

# Agenda item:	8.2.1.1

# Document for:	Discussion/Decision

Introduction

Rel-18 WI, “NR sidelink evolution” has been completed in the last meeting. However, there are several remaining issues that still need to be clarified and resolved. In this contribution, we introduce our views on the remaining issues on channel access mechanism for sidelink on unlicensed spectrum taking into account the previous agreements [1].

Channel access mechanism

Contention window (CW) adjustment

For SL-U Type 1 channel access procedure will be used for initial channel acquisition as well as sidelink transmission including PSCC/PSSCH, PSFCH, S-SSB, and so on similar as NR-U. For Type 1 channel access procedure, CW (contention window) size is adjusted based on HARQ feedback in a SL reference duration. For CW adjustment, the agreements for unicast and groupcast option 2 were made in the previous RAN1#112bis-e meeting [2]. In case of groupcast HARQ feedback option 1 (NACK-only), it is not supported in Rel-18 SL-U. Since it was failed to reach a consensus on CW adjustment. However, there is no such a conclusion. Even though it seems that not supporting groupcast HARQ feedback option 1 in Rel-18 is the common understanding, it would be necessary to make a clear conclusion on the issue in order to avoid ambiguity on SL-U operation.

Proposal 1: Groupcast HARQ feedback option 1 is not supported in Rel-18 SL-U.

Multi-channel access procedures

Regarding multi-channel access procedure, the following three issues should be resolved. The first one is Type A and Type B multi-channel access procedures for PSFCH transmissions. In the current TS 37.213, there are still brackets in the procedures for determining CWp for channel ci in Type A and Type B multi-channel access procedures for PSFCH transmission. It was discussed in the last meeting, however failed to reach a consensus. Type A, B1, and B2 are different channel access procedures. Therefore, different procedures for CW size adjustment should applied considering the corresponding channel access procedures. Among the options in the last meeting [3], we think option 5 is well aligned with the NR-U principles.

Proposal 2: Regarding the CW size adjustments for Type A and Type B multi-channel access procedures for PSFCH transmissions, it is proposed to adopt the following text proposal.

- Reason for change: In the current specification, there are still square brackets for determining the CWp in Type A and Type B multi-channel access procedures for PSFCH transmissions.

- Summary of change: The determinations of CWp for Type A and Type B are clarified.

- Consequences if not approved: The determination of CWp on multiple channels is still unclear in Type A and Type B multi-channel access procedures for PSFCH transmissions.

< Start of text proposal > ### 4.5.6 Channel access procedures for transmission(s) on multiple channels <Unchanged part omitted> #### 4.5.6.1 Type A multi-channel access procedures for PSFCH transmissions A UE can access multiple channels on which only PSFCH transmissions are performed, according to the procedures described in this clause. A UE shall perform channel access on each channel ci∈C, according to the procedures described in clause 4.5.1, where C is a set of channels on which the UE intends to transmit, and i=0,1,…q-1, and q is the number of channels on which the UE intends to transmit. The counter N described in clause 4.5.1 is determined for each channel ci and is denoted as Nci. Nci is maintained according to clause 4.5.6.1.1 or 4.5.6.1.2. For determining CWp for channel ci, any PSSCH that fully or partially overlaps with channel ci, is used in the procedures described in clause 4.5.4. <Unchanged part omitted> ##### 4.5.6.2.1 Type B1 multi-channel access procedure A single CWp value is maintained for the set of channels C. For determining CWp forci the set of channels C, any PSSCH that fully or partially overlaps with any channel ci∈C, is used in the procedures described in clause 4.5.4. ##### 4.5.6.2.2 Type B2 multi-channel access procedure A CWp value is maintained independently for each channel ci∈C using the procedure described in clause 4.5.4. For determining CWp for channel ci, any PSSCH that fully or partially overlaps with any channel ci∈C, is used in the procedures described in clause 4.5.4. For determining Ninit for channel cj, CWp value of channel cj1∈C is used, where cj1 is the channel with largest CWp among all channels in set C. <End of text proposal>

The second issue is that multi-channel access procedure when a UE is configured without intra-cell guard band(s) on an SL bandwidth part. In the last meeting, multi-channel access procedures in Clause 4.5.6.3 of TS 37.213 can be applied to S-SSB and PSFCH transmissions. Therefore, the paragraph within the square brackets would be necessary to prevent unintended SL transmissions when the intra-cell guard band(s) is not configured.

Proposal 3: Regarding the multi-channel access procedure for a UE is configured without intra-cell guard band(s) on an SL bandwidth part, it is proposed to adopt the following text proposal.

- Reason for change: In the current specification, it is unclear whether it is necessary to describe the behaviour where a UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an SL bandwidth part, and the UE fails to access any of the channels of the SL bandwidth part.

- Summary of change: The square brackets in the related part of the specification text are removed.

- Consequences if not approved: It remains unclear whether to support the behavior where a UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an SL bandwidth part, and the UE fails to access any of the channels of the SL bandwidth part.

< Start of text proposal > ### 4.5.6 Channel access procedures for transmission(s) on multiple channels <Unchanged part omitted> #### 4.5.6.3 Multi-channel access procedures for SL transmissions The procedures described in this clause are applied for PSCCH/PSSCH/S-SSB transmission(s) and can be applied for PSFCH transmission. A UE can access multiple channels on which SL transmissions are performed, according to the procedures described in this clause. If a UE intends to transmit SL transmissions on a set of channels C, the following is applicable: - if Type 1 channel access procedure is used for SL transmissions on the set of channels C, - the UE may transmit on channel ci∈C using Type 2A channel access procedure as described in clause 4.5.2.1, - if the channel frequencies of the set of channels C is a subset of the sets of channel frequencies defined in clause X.X in [2X], and - if Type 2A channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and - if the UE has accessed channel cj using Type 1 channel access procedure as described in clause 4.5.1, - where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C. - the UE may transmit on channel ci∈C using Type 1 channel access procedure as described in clause 4.5.1 - the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured with SL resources. - the UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an SL bandwidth part as described in clause X of [8], and the UE fails to access any of the channels of the SL bandwidth part. After a UE successfully performs a multi-channel access procedure for a set of RB sets, a channel occupancy is initiated for the set of RB sets and the UE can use the initiated channel occupancy for own subsequent transmissions (including any of S-SSB, PSFCH or PSCCH/PSSCH). <End of text proposal>

The last one is whether multi-channel access procedures in Clause 4.5.6.3 of TS 37.213 can be applied to simultaneous PSFCH transmission(s). In the last meeting, it was agreed that the multi-channel access procedures in Clause 4.5.6.3 of TS 37.213 can be applied to S-SSB transmission(s) and PSFCH transmission in addition to PSCCH/PSSCH transmission(s). According to the multi-channel access procedures in the clause, it does not allow multiple PSFCH transmissions. However, multiple PSFCH transmissions can be performed as long as the conditions in the clause are satisfied.

Proposal 4: Regarding whether to apply multiple PSFCH transmissions using the multi-channel procedures based on NR-U UL mechanism, it should be allowed as long as the conditions for the multi-channel procedures are satisfied and it is proposed to adopt the following text proposal.

- Reason for change: In the current specification, it is unclear whether the multi-channel access procedures based on NR-U UL mechanism is supported for multiple PSFCH transmission(s).

- Summary of change: It is clarified that the multi-channel access procedures based on NR-U UL mechanism is supported for multiple PSFCH transmission(s).

- Consequences if not approved: It is still unclear whether the multi-channel access procedures based on NR-U UL mechanism is supported for multiple PSFCH transmission(s).

< Start of text proposal > ### 4.5.6 Channel access procedures for transmission(s) on multiple channels <Unchanged part omitted> #### 4.5.6.3 Multi-channel access procedures for SL transmissions The procedures in this clause are applied for SL PSCCH/PSSCH/S-SSB transmission(s) and can be applied for PSFCH transmission(s). A UE can access multiple channels on which SL transmissions are performed, according to the procedures described in this clause. <End of text proposal>

Summary

In this contribution, we made the following proposals on the remaining issues for channel access mechanism for sidelink on unlicensed spectrum.

Proposal 1: Groupcast HARQ feedback option 1 is not supported in Rel-18 SL-U.

Proposal 2: Regarding the CW size adjustments for Type A and Type B multi-channel access procedures for PSFCH transmissions, it is proposed to adopt the following text proposal.

- Reason for change: In the current specification, there are still square brackets for determining the CWp in Type A and Type B multi-channel access procedures for PSFCH transmissions.

- Summary of change: The determinations of CWp for Type A and Type B are clarified.

- Consequences if not approved: The determination of CWp on multiple channels is still unclear in Type A and Type B multi-channel access procedures for PSFCH transmissions.

< Start of text proposal > ### 4.5.6 Channel access procedures for transmission(s) on multiple channels <Unchanged part omitted> #### 4.5.6.1 Type A multi-channel access procedures for PSFCH transmissions A UE can access multiple channels on which only PSFCH transmissions are performed, according to the procedures described in this clause. A UE shall perform channel access on each channel ci∈C, according to the procedures described in clause 4.5.1, where C is a set of channels on which the UE intends to transmit, and i=0,1,…q-1, and q is the number of channels on which the UE intends to transmit. The counter N described in clause 4.5.1 is determined for each channel ci and is denoted as Nci. Nci is maintained according to clause 4.5.6.1.1 or 4.5.6.1.2. For determining CWp for channel ci, any PSSCH that fully or partially overlaps with channel ci, is used in the procedures described in clause 4.5.4. <Unchanged part omitted> ##### 4.5.6.2.1 Type B1 multi-channel access procedure A single CWp value is maintained for the set of channels C. For determining CWp forci the set of channels C, any PSSCH that fully or partially overlaps with any channel ci∈C, is used in the procedures described in clause 4.5.4. ##### 4.5.6.2.2 Type B2 multi-channel access procedure A CWp value is maintained independently for each channel ci∈C using the procedure described in clause 4.5.4. For determining CWp for channel ci, any PSSCH that fully or partially overlaps with any channel ci∈C, is used in the procedures described in clause 4.5.4. For determining Ninit for channel cj, CWp value of channel cj1∈C is used, where cj1 is the channel with largest CWp among all channels in set C. <End of text proposal>

Proposal 3: Regarding the multi-channel access procedure for a UE is configured without intra-cell guard band(s) on an SL bandwidth part, it is proposed to adopt the following text proposal.

- Reason for change: In the current specification, it is unclear whether it is necessary to describe the behaviour where a UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an SL bandwidth part, and the UE fails to access any of the channels of the SL bandwidth part.

- Summary of change: The square brackets in the related part of the specification text are removed.

- Consequences if not approved: It remains unclear whether to support the behavior where a UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an SL bandwidth part, and the UE fails to access any of the channels of the SL bandwidth part.

< Start of text proposal > ### 4.5.6 Channel access procedures for transmission(s) on multiple channels <Unchanged part omitted> #### 4.5.6.3 Multi-channel access procedures for SL transmissions The procedures described in this clause are applied for PSCCH/PSSCH/S-SSB transmission(s) and can be applied for PSFCH transmission. A UE can access multiple channels on which SL transmissions are performed, according to the procedures described in this clause. If a UE intends to transmit SL transmissions on a set of channels C, the following is applicable: - if Type 1 channel access procedure is used for SL transmissions on the set of channels C, - the UE may transmit on channel ci∈C using Type 2A channel access procedure as described in clause 4.5.2.1, - if the channel frequencies of the set of channels C is a subset of the sets of channel frequencies defined in clause X.X in [2X], and - if Type 2A channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and - if the UE has accessed channel cj using Type 1 channel access procedure as described in clause 4.5.1, - where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C. - the UE may transmit on channel ci∈C using Type 1 channel access procedure as described in clause 4.5.1 - the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured with SL resources. - the UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an SL bandwidth part as described in clause X of [8], and the UE fails to access any of the channels of the SL bandwidth part. After a UE successfully performs a multi-channel access procedure for a set of RB sets, a channel occupancy is initiated for the set of RB sets and the UE can use the initiated channel occupancy for own subsequent transmissions (including any of S-SSB, PSFCH or PSCCH/PSSCH). <End of text proposal>

Proposal 4: Regarding whether to apply multiple PSFCH transmissions using the multi-channel procedures based on NR-U UL mechanism, it should be allowed as long as the conditions for the multi-channel procedures are satisfied and it is proposed to adopt the following text proposal.

- Reason for change: In the current specification, it is unclear whether the multi-channel access procedures based on NR-U UL mechanism is supported for multiple PSFCH transmission(s).

- Summary of change: It is clarified that the multi-channel access procedures based on NR-U UL mechanism is supported for multiple PSFCH transmission(s).

- Consequences if not approved: It is still unclear whether the multi-channel access procedures based on NR-U UL mechanism is supported for multiple PSFCH transmission(s).

< Start of text proposal > ### 4.5.6 Channel access procedures for transmission(s) on multiple channels <Unchanged part omitted> #### 4.5.6.3 Multi-channel access procedures for SL transmissions The procedures in this clause are applied for SL PSCCH/PSSCH/S-SSB transmission(s) and can be applied for PSFCH transmission(s). A UE can access multiple channels on which SL transmissions are performed, according to the procedures described in this clause. <End of text proposal>

References

- Chairman’s notes, RAN1#114bis, October 2023.

- Chairman’s notes, RAN1#112bis-e, April 2023.

- R1-2310293, “FL summary for AI 8.2.1.1: SL-U channel access mechanism (EOM),” Moderator (OPPO)